GEOLOGICAL REPORT

ON RECONNAISSANCE SURVEY (G4) FOR REE AND ASSOCIATED MINERALS IN MADDUR

AREA (160 Sq. Km) NARAYANPET DISTRICT, TELANGANA.

(UNDER NMEDT PROGRAMME) TEXT, ANNEXURES AND PLATES

1

2

3

4

5

6

CRITICAL MINERAL TRACKERS

(NOTIFIED PRIVATE EXPLORATION AGENCY)

DATE: JULY – 2026

CERTIFICATION

This is to certify that geological report has been prepared in respect of Reconnaissance Survey (G-4) for REE and Associated minerals in Maddur Block, District Narayanpet, State: Telangana by CRITICAL MINERAL TRACKERS on behalf of National Mineral Exploration and Development Trust (NMEDT). The report has been prepared in accordance with the Minerals (Evidence of Mineral Contents) Rule 2015 specified under Mineral Auction Rule, 2015 and amended up to 2021.

NAME: K. MAHENDER REDDY

DESIGNATION: Technical Area Expert

CONTENT

COVER PAGE PHOTOGRAPHS:

- Grey biotite granite

- Alkali Feldspar granite

- Photomicrograph showing allanite

- Photomicrograph showing allanite, apatite, biotite,

- porphyritic granite

- Quartz reef (sheared)

| No | Title | Page No |
| --- | --- | --- |
|  | CHAPTER - I | 9-17 |
| 1.0.0 | काययकारी सारााांश | 3-8 |
| 1.0.0 | EXECUTIVE SUMMARY |  |
| 1.1.0 | Background information | 9-10 |
| 1.2.0 | Location and accessibility of the block | 10-11 |
| 1.3.0 | Geology | 11-12 |
| 1.4.0 | Previous Exploration | 12 |
| 1.5.0 | Present Exploration | 12-15 |
| 1.6.0 | Conclusion | 15-16 |
| 1.7.0 | Recommendation | 17 |
|  | CHAPTER- II |  |
| 2.0.0 | DETAILS OF THE QUALIFIED PERSON(S)/ EXPLORATION AGENCY | 18 |
| 2.1.0 | Details of qualification and experience of persons associated with various aspects of exploration assessment of resources and reserves. | 19 |
|  | CHAPTER- III |  |
| 3.0.0 | TITLE AND OWNERSHIP |  |
| 3.1.0 | Title of The Report | 20 |
| 3.2.0 | Details of period of Prospecting | 20-21 |
|  | CHAPTER- IV |  |
| 4.0.0 | DETAILS OF AREA UNDER STUDY |  |

GEOLOGICAL REPORT ON RECONAISSANCE SURVEY (G4) FOR REE AND ASSOCIATED MINERALS IN MADDUR     AREA, NARAYANPET DISTRICT, TELANGANA

| No | Title | Page No |
| --- | --- | --- |
| 4.1.0 | Location of the Block | 22 |
| 4.2.0 | Accessibility to the Block | 22-23 |
| 4.3.0 | Land Use Pattern | 24-25 |
| 4.4.0 | Block area marked on the Digital Elevation Map (56O) | 26 |
|  | CHAPTER- V |  |
| 5.0.0 | PHYSIOGRAPHY AND ENVIRONMENT |  |
| 5.1.0 | Topography | 27 |
| 5.2.0 | Location and Toposheet Reference | 27-28 |
| 5.3.0 | Outcrop Distribution and Geological Framework | 29-30 |
| 5.4.0 | Connectivity | 31 |
| 5.5.0 | Population | 31 |
| 5.6.0 | Socio Demographic profile | 31 |
| 5.7.0 | Historical / Archaeological Site | 31 |
| 5.8.0 | Forests, sanctuaries, national park and wildlife sanctuaries | 32 |
| 5.9.0 | Flora and Fauna within and nearby | 32 |
| 5.10.0 | Drainage Pattern | 32 |
| 5.11.0 | Climatic Condition | 32-33 |
| 5.12.0 | Any other factor having potential to affect the viability of the project and assessment of resources and reserves | 33 |
|  | CHAPTER- VI |  |
| 6.0.0 | INFRASTRUCTURE |  |
| 6.1.0 | Local infrastructure | 34 |
|  | CHAPTER- VII |  |

LIST OF CONTENTS

| No | Title | Page No |
| --- | --- | --- |
| 7.0.0 | GEOLOGY |  |
| 7.1.0 | Regional Geology | 35-43 |
| 7.2.0 | Geology of the block | 44-45 |
| 7.3.0 | Structure of the block | 45-48 |
| 7.4.0 | Description of lithology | 49-63 |
| 7.5.0 | Sem studies | 64-69 |
| 7.6.0 | Geochemistry and Mineralization | 70-120 |
|  | CHAPTER- VIII |  |
| 8.0.0 | PREVIOUS EXPLORATION | 121 |
| 8.1.0 | General | 121 |
|  | CHAPTER- IX |  |
| 9.0.0 | AERIAL OR GROUND GEOPHYSICAL OR GEOCHEMICAL SURVEY | 122 |
| 9.1.0 | Aerial or Ground Geophysical Survey | 122 |
| 9.2.0 | Geochemical Survey | 122 |
|  | CHAPTER- X |  |
| 10.0.0 | EXPLORATION UNDERTAKEN DURING CURRENT INVESTIGATION | 123 |
| 10.1.0 | Background | 123 |
| 10.2.0 | Exploration Methodology | 123-127 |
|  | CHAPTER- XI |  |
| 11.0.0 | Location of Data Point | 128 |
| 11.1.0 | Accuracy and Quality of Survey | 128 |
| 11.2.0 | Quality and adequacy of topographic control | 128 |
|  | CHAPTER- XII |  |
| 12.0.0 | Sampling | 129 |
| 12.1.0 | Sampling Technique | 129-130 |
| 12.2.0 | Nature, quality and appropriateness of sample preparation | 130 |

| No | Title | Page No |
| --- | --- | --- |
|  | technique |  |
| 12.3.0 | Quality control procedures adopted during sampling | 130 |
| 12.4.0 | Measures taken to ensure that the sampling is representative of the in-situ material collected | 130 |
| 12.5.0 | Appropriateness of gran size | 130-131 |
|  | CHAPTER- XIII |  |
| 13.0.0 | Drilling Technique and drill sampling employed | 131 |
|  | CHAPTER- XIV |  |
| 14.0.0 | SUB-SAMPLING TECHNIQUES AND SAMPLE PREPARATION | 132 |
|  | CHAPTER- XV |  |
| 15.0.0 | QUALITY OF ASSAY DATA AND LABORATORY TEST |  |
| 15.1.0 | The nature, quality and appropriateness of Chemical analysis | 133 |
| 15.2.0 | Quality control procedure | 133 |
| 15.3.0 | External Check Analyses | 134 |
| 15.4.0 | Security and chain of control of samples | 134-179 |
|  | CHAPTER- XVI |  |
| 16.0.0 | MOISTURE | 180 |
| 16.1.0 | Method of determination of the moisture content. | 180 |
|  | CHAPTER- XVII |  |
| 17.0.0 | BULK DENSITY | 181 |
| 17.1.0 | Method of determination and result | 181 |
|  | CHAPTER- XVIII |  |
| 18.0.0 | BENEFICIATION STUDY | 182 |

| No | Title | Page No |
| --- | --- | --- |
| 18.1.0 | Details of beneficiation studies | 182 |
|  | CHAPTER- XIX |  |
| 19.0.0 | RESOURCE ESTIMATION TECHNIQUES | 183 |
|  | CHAPTER- XX |  |
| 20.0.0 | REPORTING OF RESOURCES | 184 |
|  | CHAPTER- XXI |  |
| 21.0.0 | SUMMARY AND RECOMMENDATIONS |  |
| 21.1.0 | Summary | 185-187 |
| 21.2.0 | Recommendations | 188 |
| 21.3.0 | Location Index | 189 |
|  | CHAPTER- XXII |  |
| 22.0.0 | PLATES AND MAPS | 190 |
| 22.1.0 | List of Plates | 190-191 |
|  | CHAPTER- XXIII |  |
| 23.0.0 | ANNEXURES OR ENCLOSURES TO THE REPORT |  |
| 23.1.0 | List of Annexure | 192 |
|  | CHAPTER- XXIV |  |
| 24.0.0 | ANY OTHER INFORMATION | 193 |
| 24.1.0 | Any other information as may be available or required by any authority as prescribed | 193 |
| 24.1.1 | References | 193 |
|  | CHAPTER- XXV |  |
| 25.0.0 | CERTIFICATION | 194 |

GEOLOGICAL REPORT ON RECONNAISSANCE SURVEY (G4) FOR REE AND ASSOCIATED MINERALS IN MADDUR AREA, NARAYANPET  DISTRICT, TELANGANA

LIST OF TABLES

| Table No | Title | Page No |
| --- | --- | --- |
| 1.1 | The Coordinates of Maddur area in Narayanpet districts of Telangana. | 11 |
| 1.2 | Summarized Table showing Component wise proposed quantum Vs. Quantum Achieved | 13 |
| 2.1 | Details of Qualified Person(S) / Exploration Agency | 18 |
| 2.2 | Details of Personnel Associated with The Exploration work of Maddur area Narayanpet District. | 19 |
| 4.1 | Coordinates of cardinal points of Maddur Narayanpet Districts of Telangana. | 22 |
| 7.1 | Regional Stratigraphic Sequence | 36 |
| 7.2 | Comparison of Major Oxide values of Granite | 72 |
| 7.3 | Summarized Table showing range of elemental concentration in Granite’s variants. | 79 |
| 10.1 | Summarized Table showing Component wise proposed quantum Vs. Quantum Achieved. | 124 |

RECONNAISSANCE SURVEY (G4) FOR REE AND ASSOCIATED MINERALS IN MADDUR AREA, NARAYANPET DISTRICT, TELANGANA

LIST OF TEXT FIGURES

| Text fig No | Title | Page No |
| --- | --- | --- |
| 1. | Location map of Maddur block in parts of Narayanpet district in Telangana | 23 |
| 2. | Digital Elevation Model of the Area –(56H/09) | 26 |
| 3. | Geological map of undivided Mahabubnagar district showing the block area. | 42 |
| 4 | SEM \ EDX – Image and Graph (TS - 47) | 65 |
| 5 | SEM \ EDX – Image and Graph (TS - 40) | 65 |
| 6 | SEM \ EDX – Image and Graph (TS - 25) | 66 |
| 7 | SEM \ EDX – Image and Graph (TS - 60) | 66 |
| 8 | SEM \ EDX – Image and Graph (TS - 67) | 67 |
| 9 | SEM \ EDX – Image and Graph (TS - 35) | 67 |
| 10 | SEM \ EDX – Image and Graph (TS - 61) | 68 |
| 11 | SEM \ EDX – Image and Graph (TS - 57) | 68 |
| 12 | SEM \ EDX – Image and Graph (TS - 17) | 69 |
| 13 | SEM \ EDX – Image and Graph (TS - 7) | 69 |

| Text fig No | Title | Page No |
| --- | --- | --- |
| 14 | Feldspar triangle (O’ Connor 1965) | 72 |
| 15 | QAP streckeinsen | 72 |
| 16 | TAS diagram (M.J. Le Bas and R.W. Le Maitre) | 73 |
| 17 | Th TAS (Cox et al.1979) | 73 |
| 18 | Middlemost diagram (1994) | 74 |
| 19 | R1-R2 plot (De la Roche et al. 1980) | 75 |
| 20 | Granite tectonic discrimination – Frost and Frost (2008) | 75 |
| 21 | Granite tectonic discrimination – Maniar and Piccoli (1989) | 76 |
| 22 | SiO2 – K2O plot (Peccerillo and Taylor 1976) | 76 |
| 23 | R1 - R2 (Batchelor + Bowden 1985) | 77 |
| 24 | Spider plot – REE chondrite (Boynton 1984) (Lakkayapalli Zone) | 91 |
| 25 | Spider plot – REE chondrite (Boynton 1984) (Pallera zone) | 99 |
| 26 | Spider plot – REE chondrite (Boynton 1984) (Nandipad zone -A) | 101 |
| 27 | Spider plot – REE chondrite (Boynton 1984) Zone (Pedripad) | 104 |
| 28 | Spider plot – REE chondrite (Boynton 1984) Zone (Jadavarao palli) | 106 |
| 29 | Chondrite normalized concentration | 108 |
| 30 | Chondrite normalized concentration | 108 |
| 31 | Eu – Ce anomalies | 109 |
| 32 | Y- Ho relationship | 110 |
| 33 | REE vs Zr | 110 |
| 34 | REE vs Hf | 111 |
| 35 | REE vs Th TAS (Cox et al.1979) | 111 |
| 36 | LREE -HREE | 112 |
| 37 | Frequency distribution of REE | 114 |
| 38 | Long transformed distribution of REE | 114 |
| 39 | Cumulative Frequency curve to TREE | 115 |
| 40 | Long normal probability plot of TREE | 115 |
| 41 | Box - and whisker plot of TREE | 116 |
| 42 | Single component gaussian mixture model | 116 |
| 43 | Correlation of major oxide with precious metal | 119 |
| 44 | Scatter Plot for Primary Vs. Check (External) sample Analysis for La (ppm) | 164 |
| 45 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Ce (ppm) | 165 |
| 46 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Pr (ppm) | 166 |
| 47 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Nd (ppm) | 167 |
| 48 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Sm (ppm) | 168 |
| 49 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Eu (ppm) | 169 |
| 50 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Gd (ppm) | 170 |
| 51 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Tb (ppm) | 171 |
| 52 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Dy (ppm) | 172 |
| 53 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Ho (ppm) | 173 |
| 54 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Er (ppm) | 174 |
| 55 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Tm (ppm) | 175 |
| 56 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Yb (ppm) | 176 |
| 57 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Lu (ppm) | 177 |
| 58 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Sc (ppm) | 178 |
| 59 | Scatter Plot for Primary Vs. Check (External) sample Analysis for Y (ppm) | 179 |

RECONAISSANCE SURVEY (G4) FOR REE AND ASSOCIATED MINERALS IN MADDUR AREA, NARAYANPET DISTRICT, TELANGANA.

LIST OF PHOTOGRAPHS

| Photograph No | Title | Page No |
| --- | --- | --- |
| 7.3.0-1 | Field Photograph showing migmatite structure (1) outcrop exposed at NW of Nandipad | 46 |
| 7.3.0-2-3 | Field Photograph showing Sheared Quartz outcrop exposed at the Northern part Maddur around Lakkayapalli | 47 |
| 7.3.0-4 | Photomicrographs of sheared amphibolite | 48 |
| 7.4.0- 1.1-1.2 | Field Photograph showing grey biotite granite outcrop exposed at the Central part Maddur Block | 49 |
| 1.3 | Photomicrographs of Grey biotite Granite | 51 |
| 2.1 -2.3 | Field Photograph showing Alkali feldspar Granite 2.1,2.2,2.3 exposed at West of Lakkayapalli village | 52 |
| 2.4 | Photomicrographs of alkali feldspar granite | 55 |
| 3.1 | Field Photograph showing Leucogranite outcrop exposed at southern part of Maddur block | 55 |
| 3.2 | Photomicrographs of Leucogranite | 56 |
| 4.1 | Field Photograph showing Porphyritic Granite, outcrop located East of Abhangapur | 57 |
| 4.2 | Photomicrographs of Porphyritic Granite | 58 |
| 5.1 | Field Photograph showing Meta Gabbro, outcrop located South of Bandakonda village | 59 |
| 5.2 | Photomicrographs of meta gabbro | 60 |
| 6.1 | Field Photograph showing Pegmatite, outcrop located NE of Jadavaraopalle village | 61 |
| 7.1 | Field Photograph showing Quartz Vein outcrop exposed at Northern part of the block | 61 |
| 8.1 | Field Photograph showing dolerite dyke located at SW of Monapuram of the block (10) | 62 |
| 10.2.0 | Pit photograph (10.2.0- 1.1-1.4) | 126 |

TEXT

अध्याय – I

1.0.0कार्यकारीसारांश
1.1.0पृष्ठभूमि

लैंथेनाइड तत्वों को दो समूहों में विभाजित किया जाता है। हल्के दुर्लभ मृदा तत्व (Light Rare Earth Elements - LREE) वे तत्व हैं जिनकी परमाणु संख्या 57 से 62 तक होती है, अर्थात् लैंथेनम (La), सेरियम (Ce), प्रसीओडिमियम (Pr), नियोडिमियम (Nd), प्रोमेथियम (Pm) तथा समेरियम (Sm)। भारी दुर्लभ मृदा तत्व (Heavy Rare Earth Elements - HREE) वे तत्व हैं जिनकी परमाणु संख्या 63 से 71 तक होती है, जिनमें यूरोपियम (Eu), गैडोलिनियम (Gd), टर्बियम (Tb), डिस्प्रोसियम (Dy), होल्मियम (Ho), एर्बियम (Er), थुलियम (Tm), यिटरबियम (Yb) तथा ल्यूटेशियम (Lu) शामिल हैं। इसके अतिरिक्त इट्रियम (Y) और स्कैंडियम (Sc) को भी प्रायः इसी समूह में सम्मिलित किया जाता है। दुर्लभ मृदा तत्व (Rare Earth Elements - REE) 17 धात्विक तत्वों का एक समूह है, जो आधुनिक प्रौद्योगिकियों की एक विस्तृत श्रृंखला के लिए अत्यंत महत्वपूर्ण हैं। अपने नाम के विपरीत, ये पृथ्वी की पर्पटी में अपेक्षाकृत प्रचुर मात्रा में पाए जाते हैं; तथापि, इनका आर्थिक रूप से व्यवहार्य तरीके से निष्कर्षण एवं पृथक्करण कठिन तथा महंगा होता है। यही कारण है कि इन्हें दुर्लभमानाजाताहै।

17 दुर्लभ मृदा तत्वों के समूह में 15 लैंथेनाइड तत्व शामिल हैं: लैंथेनम (La), सेरियम (Ce), प्रसीओडिमियम (Pr), नियोडिमियम (Nd), प्रोमेथियम (Pm), समेरियम (Sm), यूरोपियम (Eu), गैडोलिनियम (Gd), टर्बियम (Tb), डिस्प्रोसियम (Dy), होल्मियम (Ho), एर्बियम (Er), थुलियम (Tm), यिटरबियम (Yb) तथा ल्यूटेशियम (Lu)। इनके अतिरिक्त स्कैंडियम (Sc) और इट्रियम (Y) भी प्रायः लैंथेनाइड तत्वों के साथ पाए जाते हैं तथा उनके समान गुण प्रदर्शित करतेहैं।

दुर्लभ मृदा तत्वों की विशेषता उनका उच्च घनत्व, उच्च गलनांक, उच्च विद्युत चालकता तथा उच्च तापीय चालकता है। इनके विशिष्ट विद्युत, धातुकर्मीय, उत्प्रेरक, नाभिकीय, चुंबकीय एवं प्रकाशीय गुण इन्हें विभिन्न उभरती हुई उच्च-स्तरीय और महत्वपूर्ण तकनीकी अनुप्रयोगों के लिए अनिवार्य बनाते हैं। ये भारत की ऊर्जा सुरक्षा से संबंधित क्षेत्रों, जैसे स्वच्छ ऊर्जा, रक्षा, नागरिक उपयोग, पर्यावरण संरक्षण तथा आर्थिक विकास में महत्वपूर्णभूमिकानिभातेहै।

दुर्लभ मृदा तत्व प्राकृतिक रूप से अपने शुद्ध रूप में नहीं पाए जाते, बल्कि विभिन्न खनिजों में अंतर्निहित रूप से विद्यमान रहते हैं, जो मुख्यतः सिलिकेट, ऑक्साइड, कार्बोनेट अथवा फॉस्फेट समूहों से संबंधित होते हैं। सामान्यतः हल्के दुर्लभ मृदा तत्व (LREE) पृथ्वी की पर्पटी में अधिक मात्रा में पाए जाते हैं तथा भारी दुर्लभ मृदा तत्वों (HREE) की तुलना में अपेक्षाकृत आसानी से निष्कर्षित किए जा सकते हैं। इन खनिजों की दुर्लभता (जिन्हें पहले “अर्थ्स” कहा जाता था) के कारण ही इन्हें “दुर्लभ मृदा तत्व” नाम दिया गया। इस प्रकार का पहला खोजा गया खनिज गैडोलिनाइट (Gadolinite) था, जो सेरियम, इट्रियम, लौह, सिलिकॉन तथा अन्य तत्वों का एक यौगिक है। यह खनिज स्वीडन के यिटरबी (Ytterby) नामक गाँव की एक खदान से प्राप्त किया गया था। दुर्लभ मृदा तत्वों में से अनेक के नाम इसी स्थान से व्युत्पन्न किए गए हैं।

महत्वपूर्ण खनिज (Critical Minerals) वे खनिज हैं जो आर्थिक विकास तथा राष्ट्रीय सुरक्षा के लिए अत्यावश्यक होते हैं। जब किसी खनिज की आवश्यकता उसकी उपलब्ध संसाधन क्षमता से अधिक हो जाती है, तब वह महत्वपूर्ण खनिज की श्रेणी में आ जाता है। वर्तमान वैश्विक अर्थव्यवस्था लिथियम, ग्रेफाइट, कोबाल्ट, टाइटेनियम तथा दुर्लभ मृदा तत्वों जैसे खनिजों की उपलब्धता एवं उनके निष्कर्षण की प्रौद्योगिकी पर आधारित है। ये खनिज ऑटोमोबाइल उद्योग, पवन टर्बाइन, उत्प्रेरक (Catalysts), फ्लोरोसेंट एवं एलईडी प्रकाश व्यवस्था, काँच उद्योग, सैन्य अनुप्रयोगों, चिकित्सा उपकरणों तथा एयरोस्पेस जैसे क्षेत्रों की प्रगति के लिए अत्यंत महत्वपूर्ण हैं।

देश में संसाधन/भंडार की स्थिति, उत्पादन, आयात पर निर्भरता, भविष्य की प्रौद्योगिकी एवं स्वच्छ ऊर्जा में उपयोग तथा कृषि प्रधान अर्थव्यवस्था में उर्वरक खनिजों की आवश्यकता जैसे महत्वपूर्ण मानकों को ध्यान में रखते हुए समिति ने 30 महत्वपूर्ण खनिजों की पहचान की है। इनमें एंटिमनी  बेरिलियम , बिस्मथ, कोबाल्ट, तांबा , गैलियम जर्मेनियम), ग्रेफाइट हैफ्नियम इंडियम लिथियम मोलिब्डेनम नियोबियम निकेल पीजीई (PGE), फॉस्फोरस पोटाश), दुर्लभ मृदा तत्व (REE), रेनियम सिलिकॉन, स्ट्रोंशियम, टैंटलम , टेल्यूरियम टिन टाइटेनियम टंग्स्टन वैनेडियम , जिरकोनियम, सेलेनियम

तथा कैडमियम शामिल  हैं।(संदर्भ: क्रिटिकल मिनरल्स फॉर इंडिया, महत्वपूर्ण खनिजों की पहचान संबंधी समिति की रिपोर्ट,खान मंत्रालय, जून 2023) MMDR अधिनियम में हाल ही में किए गए संशोधन के पश्चात भारत सरकार द्वारा महत्वपूर्ण खनिजों के अन्वेषण को सर्वोच्च प्राथमिकता दी जा रही है। चूँकि दुर्लभ मृदा तत्व (REE) अधिसूचित महत्वपूर्ण खनिजों में सम्मिलित हैं, इसलिए वर्तमान अन्वेषण कार्यक्रम को क्षेत्र में भारतीय भूवैज्ञानिक सर्वेक्षण (GSI) द्वारा पूर्व में किए गए कार्यों के आधार पर तैयार किया गया। प्रस्ताव प्रस्तुत करने से पूर्व क्षेत्र का भ्रमण किया गया, कुछ ट्रैवर्स लिए गए, आधारशिला (Bedrock) नमूने एकत्रित

किए गए तथा उनका रासायनिक विश्लेषण किया गया। क्षेत्रीय भ्रमण के दौरान उत्तर-पश्चिम से दक्षिण-पूर्व (NW-SE) दिशा में विस्तृत एक प्रमुख भ्रंश/शीयर ज़ोन का अवलोकन किया गया, जो कई किलोमीटर तक फैला हुआ है। इस शीयर ज़ोन के साथ एक क्वार्ट्ज रीफ अंतःस्थापित पाई गई, जो ब्लॉक क्षेत्र के उत्तरी भाग से होकर गुजरती है। इस क्वार्ट्ज रीफ से एक नमूना भी एकत्रित किया गया तथा उसका स्वर्ण (Gold), आर्सेनिक (Arsenic), एंटिमनी (Antimony), बिस्मथ (Bismuth) एवं रजत (Silver) के लिए विश्लेषण किया गया, जिसमें उत्साहजनक मान प्राप्त हुए। इसी कारण इन तत्वों के लिए क्वार्ट्ज रीफ को लक्ष्य क्षेत्र के रूप में चयनित किया गया तथा प्रस्ताव में सम्मिलित किया गया। स्वर्ण एक अत्यंत मूल्यवान धातु है, जिसका उपयोग बुलियन, आभूषण निर्माण, औषधीय प्रयोजनों तथा विभिन्न औद्योगिक उपयोगों में किया जाताहै।

देश में संसाधन/भंडार की स्थिति, उत्पादन, आयात पर निर्भरता, भविष्य की प्रौद्योगिकी एवं स्वच्छ ऊर्जा में उपयोग तथा कृषि प्रधान अर्थव्यवस्था में उर्वरक खनिजों की आवश्यकता जैसे महत्वपूर्ण मानकों को ध्यान में रखते हुए समिति ने 30 महत्वपूर्ण खनिजों (Critical Minerals) की पहचान की है। इनमें एंटिमनी (Antimony), बेरिलियम (Beryllium), बिस्मथ (Bismuth), कोबाल्ट (Cobalt), तांबा (Copper), गैलियम (Gallium), जर्मेनियम (Germanium), ग्रेफाइट (Graphite), हैफ्नियम (Hafnium), इंडियम (Indium), लिथियम (Lithium), मोलिब्डेनम (Molybdenum), नियोबियम (Niobium), निकेल (Nickel), पीजीई (PGE), फॉस्फोरस (Phosphorous), पोटाश (Potash), दुर्लभ मृदा तत्व (REE), रेनियम (Rhenium), सिलिकॉन (Silicon), स्ट्रोंशियम (Strontium), टैंटलम (Tantalum), टेल्यूरियम (Tellurium), टिन (Tin), टाइटेनियम (Titanium), टंग्स्टन (Tungsten), वैनेडियम (Vanadium), जिरकोनियम (Zirconium), सेलेनियम (Selenium) तथा कैडमियम (Cadmium)शामिलहैं। (संदर्भ: क्रिटिकल मिनरल्स फॉर इंडिया, महत्वपूर्ण खनिजों की पहचान संबंधी समिति की रिपोर्ट, खान मंत्रालय, जून2023)MMDR अधिनियम में हाल ही में किए गए संशोधन के पश्चात भारत सरकार द्वारा महत्वपूर्ण खनिजों के अन्वेषण को सर्वोच्च प्राथमिकता दी जा रही है। चूँकि दुर्लभ मृदा तत्व (REE) अधिसूचित महत्वपूर्ण खनिजों में सम्मिलित हैं, इसलिए वर्तमान अन्वेषण कार्यक्रम को क्षेत्र में भारतीय भूवैज्ञानिक सर्वेक्षण (GSI) द्वारा पूर्व में किए गए कार्यों के आधार पर तैयार किया गया। प्रस्ताव प्रस्तुत करने से पूर्व क्षेत्र का भ्रमण किया गया, कुछ ट्रैवर्स लिए गए, आधारशिला (Bedrock) नमूने एकत्रित किए गए तथा उनका रासायनिक विश्लेषण किया गया। क्षेत्रीय भ्रमण के दौरान उत्तर-पश्चिम से दक्षिण-पूर्व (NW-SE) दिशा में विस्तृत एक प्रमुख भ्रंश/शीयर ज़ोन का अवलोकन किया गया, जो कई किलोमीटर तक फैला हुआ है। इस शीयर ज़ोन के साथ एक क्वार्ट्ज रीफ अंतःस्थापित पाई गई, जो ब्लॉक क्षेत्र के उत्तरी भाग से होकर गुजरती है। इस क्वार्ट्ज रीफ से एक नमूना भी एकत्रित किया गया तथा उसका स्वर्ण (Gold), आर्सेनिक (Arsenic), एंटिमनी (Antimony), बिस्मथ (Bismuth) एवं रजत (Silver) के लिए विश्लेषण किया गया, जिसमें उत्साहजनक मान प्राप्त हुए।

इसी कारण इन तत्वों के लिए क्वार्ट्ज रीफ को लक्ष्य क्षेत्र के रूप में चयनित किया गया तथा प्रस्ताव में सम्मिलित किया गया। स्वर्ण एक अत्यंत मूल्यवान धातु है, जिसका उपयोग बुलियन, आभूषण निर्माण, औषधीय प्रयोजनों तथा विभिन्न औद्योगिक उपयोगों में किया जाता है। एंटिमनी (Antimony) और बिस्मथ (Bismuth) महत्वपूर्ण खनिजों (Critical Minerals) की सूची में शामिल हैं तथा इनके महत्वपूर्ण औद्योगिक उपयोग भी हैं। आर्सेनिक (Arsenic) और रजत (Silver), जो स्वर्ण (Gold) के साथ संबद्ध तत्व हैं, उन्हें भी प्रस्ताव में सम्मिलित किया गया।

इसके पश्चात इस प्रस्ताव को NMEDT की TCC-I की 78वीं बैठक में मूल्यांकन हेतु प्रस्तुत किया गया। समिति ने प्रस्ताव का मूल्यांकन किया और यह मत व्यक्त किया कि यह क्षेत्र दुर्लभ मृदा तत्वों (REE) तथा उनसे संबंधित खनिजों की संभावित उपस्थिति के लिए अनुकूल है।

अतः प्रस्ताव को EC, NMEDT से अनुमोदन हेतु अनुशंसित किया गया। EC, NMEDT की 43वीं बैठक, जो 14 अगस्त 2025 को आयोजित की गई थी, में इस प्रस्ताव को ₹1,31,12,292/- की अनुमानित लागत के साथ अनुमोदित किया गया। इसकी सूचना पत्र संख्या F. No. 23/648/2025-NMET/350 दिनांक 04 सितम्बर 2025 के माध्यम से प्रदान की गई। तदनुसार, CMT ने 09 अक्टूबर 2025 को क्षेत्रीय कार्य प्रारम्भ किया तथा क्षेत्रीय कार्य के प्रथम चरण को पूर्ण करने हेतु अप्रैल 2026 के अंतिम सप्ताह तक कार्य सम्पन्न किया। इस चरण में भूवैज्ञानिक मानचित्रण, आधारशिला (Bedrock) नमूनों का संग्रह, रेगोलिथ (गड्ढा) नमूनों का संग्रह तथा बेडरॉक ग्रैब नमूनों का विश्लेषण शामिल था।

इन सभी कार्यों के आधार पर भूवैज्ञानिक प्रतिवेदन (Geological Report) तैयार किया गया, जिसे 03 जुलाई 2026 तक NMEDT को प्रस्तुत किया जाना निर्धारित किया गया।

मद्दूर ब्लॉक, जिसका क्षेत्रफल 160 वर्ग किमी है, 16.752° उत्तर से 16.951° उत्तर अक्षांश तथा 77.596° पूर्व से 77.663° पूर्व देशांतर के बीच स्थित है। यह तेलंगाना राज्य के नारायणपेट जिले के मद्दूर मंडल का एक भाग है। ब्लॉक के भीतर एवं इसके आसपास के प्रमुख स्थानों में मद्दूर, नंदीपाड, पल्लेरला, लक्कायापल्ली, जादवरावपल्ली, दोरेपल्ली, मोनापुरम, रेनुवटला, अप्पारेड्डीपल्ली, चेन्नाराम, अभंगापुर, बंडाकोंडा तथा कोटाकोंडा शामिल हैं। यह ब्लॉक क्षेत्र टोपोशीट 56H/09 के मध्य भाग में स्थित है तथा इसकी सीमा निर्देशांक नीचे तालिका 1.1 में दिए गए हैं।

तालिका संख्या 1.1

तेलंगाना के नारायणपेट जिले में स्थित मद्दूर ब्लॉक के निर्देशांक

| बिंदु | अक्षांश | देशांतर | ईस्टिंग (मी.) | नॉर्थिंग (मी.) | क्षेत्रफल |
| --- | --- | --- | --- | --- | --- |
| A | 16.95 | 77.596 | 776478.86 | 1875850.73 | 160 वर्ग किमी |
| B | 16.951 | 77.659 | 783190.96 | 1876051.27 |  |
| C | 16.752 | 77.663 | 366976.59 | 1935306.34 |  |
| D | 16.752 | 77.596 | 359650.24 | 1935355.85 |  |

महबूबनगर रेलवे स्टेशन ब्लॉक क्षेत्र से लगभग 49 किमी दक्षिण-दक्षिणपूर्व दिशा में स्थित है और यह दक्षिण-मध्य रेलवे नेटवर्क का हिस्सा है। इसके अतिरिक्त नारायणपेट एवं महबूबनगर क्षेत्र सड़क नेटवर्क से भी अच्छी तरह जुड़े हुए हैं। रेल परिवहन के साथ-साथ सड़क परिवहन सुविधाएँ भी पर्याप्त रूप से उपलब्ध हैं।

CHAPTER –I

1.0.0	EXECUTIVE SUMMARY

1.1.0	Background information

The Rare earth elements (REE) are a Group of 17 metallic elements crucial for a vast array of modern technologies. Despite their name, they are relatively abundant in the Earth's crust; however, they are difficult and costly to extract and separate in an economically viable way. This is why they are considered rare. The group of 17 rare earth elements includes the 15 lanthanides: lanthanum (La), cerium (Ce), praseodymium (Pr), neodymium (Nd), promethium (Pm), samarium (Sm), europium (Eu), gadolinium (Gd), terbium (Tb), dysprosium (Dy), holmium (Ho), erbium (Er), thulium (Tm), ytterbium (Yb), and lutetium (Lu). Besides these Scandium (Sc) and yttrium (Y), are often found alongside lanthanides and share similar properties.

Rare earth elements are characterized by high density, high melting point, high conductivity and high thermal conductance with distinctive electrical, metallurgical, catalytic, nuclear, magnetic and luminescent properties make them indispensable for a variety of emerging high end and critical technology applications which are relevant to India’s energy security i.e., clean energy, defense, civilian application, environment and economic areas.

The lanthanide elements are divided into two groups. The light rare earth elements (LREE) are those with atomic numbers 57 through 62 (La, Ce, Pr, Nd, Pm, Sm) and the heavy rare earth elements (HREE) with atomic numbers from 63 to 71 (Eu, Gd, Tb, Dy, Ho, Er, Tm, Yb and Lu) and Y, Sc. Rare earth elements are not found in their pure form naturally but are embedded within various minerals, primarily belonging to the silicate, oxide, carbonate, or phosphate groups. Generally, the light rare earth elements (LREE) are more abundant in the earth’s crust and easily extracted than heavy rare earth elements (HREE). It was the very scarcity of these minerals (previously called "earths") that led to the term "rare earth". The first such mineral discovered was gadolinite, a compound of cerium, yttrium, iron, silicon and other elements. This mineral was extracted from a mine in the village of Ytterby in Sweden; several of the rare earth elements bear names derived from this location.

Critical minerals are those minerals that are essential for economic development and national security. The minerals become critical when the requirement is more than the availability of their resources. The global economy is presently based on the availability of minerals such as lithium, graphite, cobalt, titanium, and rare earth elements and their extraction technology. These are vital for the advancement of sectors such as in automobiles, wind turbines, catalysts, fluorescent LED lights, glass, military applications, medical equipment and aerospace, etc.

Exploration for critical minerals is being given top priority by the Government of India, after the recent amendment in the MMDR Act. Keeping in view that REE being a listed critical mineral, the present exploration program was planned on the basis of the previous work carried out in the area, and the proposal was put up for evaluation in the 78thmeeting of TCC-I, NMEDT. The committee evaluated the proposal and opined that the area has potential for hosting REE and associated minerals. and it was recommended for approval to EC, NMEDT. In the 43rd meeting of EC, NMEDT held on 14th August 2025, approved the proposal with an estimated cost of Rs.1,31,12,292/- and communicated vide letter F. No 23/648/2025-NMET/350 dated 04th September 2025. Accordingly, CMT initiated the field work on 09 October 2025 and completed it in the last week of April 2025 to conclude the first phase of field work, including mapping and collection of bedrock samples, Regolith (Pit) samples and bedrock grab samples analyzed and prepared the Geological Report to submit by 03 July 2026 to NMEDT.

1.2.0 	Location and accessibility of the block

The Maddur block, having an area of 160 sq. km, by latitudes 16.752°N to 16.951° N and longitudes 77.596°E to 77.663° E, forms a part of Maddur Mandal, in Narayanpet district of Telangana state. The main localities in and adjoining the block are Maddur, Nandipad, Pallerla, Lakkayapalli, Jadavaraopalli, Dorepalli, Monapuram, Renuvatla, Appareddipally, Chennaram, Abhangapur, Bandakonda, kotakonda, the block area forms the central part of the toposheet 56H/09 and the co-ordinates of the block boundary are given below in Table No 1.1

Table No-1.1

The Co-ordinates of Maddur block in Narayanpet district of Telangana.

| Cardinal Points | Geographic Coordinate System in Degree Decimal (WGS 1984) |  | UTM (WGS 1984, Zone 44N) |  | Area in Sq. Km |
| --- | --- | --- | --- | --- | --- |
|  | Latitude | Longitude | Easting (m) | Northing (m) |  |
| A | 77.596 | 16.95 | 776478.86 | 1875850.73 | 160 sq.km |
| B | 77.659 | 16.951 | 783190.96 | 1876051.27 |  |
| C | 77.663 | 16.752 | 366976.59 | 1935306.34 |  |
| D | 77.596 | 16.752 | 359650.24 | 1935355.85 |  |

The Mahabubnagar Railway station is 49 kms south- southeast to the block area and all these are part of the South-Central Railway network. Besides this, Narayanpet, Mahabubnagar area are well connected by road network. Apart from railways, road transport services are also good

1.3.0	Geology

Undivided Mahbubnagar District forms a part of the stable Dharwar Craton of South Indian shield. It exposes rocks of Peninsular Gneissic Complex, Dharwar Supergroup, Cuddapah Supergroup, Kurnool and Bhima Groups and also of Deccan Traps. The Peninsular Gneissic Complex, which covers most of the area, comprises granites, gneisses and migmatites with undigested patches of older metamorphic rocks. The Dharwar Supergroup rocks are represented mainly by Gadwal Schist Belt, running in NW-SE direction between Narayanpet and Manyapadu. It exposes basic, intermediate and acid volcanic rocks associated with agglomerate and thin bands of banded iron formation. The rocks of Peninsular Gneissic Complex and Dharwar Supergroup are intruded by younger granitoids of tonalite - trondhjemite granodiorite composition and diabase dykes. These dykes are tholeiitic to sub alkalic in composition and intrude along E-W, NW-SE and NE-SW directions. Radiometric dating by Sm-Nd, Rb-Sr and Pb-Pb methods indicated the age of the dyke systems to be around 2200 Ma. (Pandey eta, 1997) In recent years, a total of 16 kimberlite pipe occurrences have been reported by GSI from Kotakonda, Maddur and Narayanpet areas of the district. In the southern part of the district, Proterozoic sedimentary rocks, belonging to Cuddapah Supergroup and Kurnool Group having 1100-600 million years age are exposed.

The Cuddapah sediments, deposited non- conformably over peninsular gneisses and Dharwars, comprise conglomerate, quartzite, limestone, dolomite and shale. The Kurnool Group of rocks overlies Cuddapah rock formations and consists mainly of limestone, shale and quartzite.

A thin sequence of Bhima Group of rocks (equivalents of Kurnool Group of rocks), comprising shale and sandstone are exposed in northwestern part of the district in Korangal taluk. Trap rocks of basaltic composition occur over the peninsular gneisses and Bhima Group of rocks in Korangal and Shadnagar taluks of the undivided Mahabubnagar district.

1.4.0 	Previous Exploration

Geological mapping on 1:50K scale was carried out in FS 1972-73 by. B.K. Ranganath, in   FS 197374 by S. Narsimha and S.K Bhattacharya.

In FS  1984-85 Geological mapping on 1:50 K scale was carried out by S.S. Nayak and Kasi Viswanathan and brought out the various geological units including the Kimberlite bodies. The entire area was rigorously searched for Kimberlites and a total of 37 bodies were identified and demarcated. Several clusters were identified and the belt was named as Narayanpet Kimberlite Belt. Out of these six Kimberlites of Maddur cluster fall in the block area.

No other exploration work has been done in the proposed block.

1.5.0    Present Exploration

Maddur Block of Narayanpet District in Telangana. was proposed for G-4 level of exploration for REE and associated Minerals. The Exploration has been carried out as per Minerals (Evidence of Mineral Contents) Amendment Rules, 2021 to achieve the following objective to carry out Geological mapping on 1:12500 scale for delineation of different variants of granites, identification of mineralized areas of REE and associated minerals and their host rocks and controls mineral concentration. To collect surface bedrock samples for analyses of REE and associated minerals/elements such as Zr, Hf, Rb, Sr and As, Sb, Bi, Ag, and Au along the major shear zone occupied by the quartz reef in order to decide further course of exploration program. To collect Regolith samples from Pits (1m*1m*1m) cu.m to know the concentration of REE and associated minerals/elements such as Zr, Hf, Rb, Sr in the regolith within the target area.

1.5.1	Exploration Methodology:

The G-4 stage of exploration for REE and associated minerals/elements in Maddur Block is to achieve the objective cited above in accordance with MEMC rule 2015(amended in 2021) as approved by NMEDT is furnished in table no 1.2 and the field component wise description has been given in subsequent paragraphs.

Table No 1.2

Summarized Table showing Component wise proposed quantum Vs. Quantum Achieved

| Sl. No. | Item of Work | Unit | Target | Achievement |
| --- | --- | --- | --- | --- |
| 1 | Geological Mapping (on 1:12,500 Scale) | Sq. km | 160 | 160 |
| 2 | Geochemical Sampling |  |  |  |
|  | Bedrock Sample | Nos | 115 | 115 |
| 3 | Exploratory Mining |  |  |  |
|  | Pitting (1 cu.m each) collection of samples | 1*1*1 | 150 | 150 |
| 4 | Laboratory Studies |  |  |  |
|  | BRS: ICP-MS (including 10% check samples) | Nos | 127 | 127 |
|  | Pitting sample: ICP-MS (including 10% check samples) | Nos | 165 | 165 |
| 7 | Preparation of standard thin section | Nos | 30 | 30 |
| 8 | Complete Petrographic/Mineragraphic Studies | Nos | 30 | 30 |
| 9 | EPMA/SEM studies for mineral phase analysis | Nos | 10 | 10 |
| 10 | Report Preparation (5 Hard copies with a soft copy) | Nos. | 1 | 5 |

1.5.2	Geological Mapping

Large-scale geological mapping (LSM) was completed over an area of 160sq km in the Survey of India toposheet No 56H/09 on 1:12,500 scale. The toposheet was used to understand the area's topography, roads, and drainage, serving as the base map for the mapping effort. Handheld GPS device (GARMIN - GPS 12H) was used to record precise sample locations and conduct the geological survey with coordinates Degree, minutes, seconds, (WGS 1984 datum). The mapping systematically documented variations in lithologies and structural elements.

1.5.3	Pitting

Pitting in the present exploration work was carried out to collect 150 no’s samples from pits of 1 cu.m which were dug by JCB as per the terrain and requirement. Surface Sampling In accordance with the approved NQT, the present G-4 stage exploration was carried out which includes collection of 115 bed rock samples.

1.5.4	Geochemical Analysis

All the 265 Nos of primary samples (115 Nos of Bed rock, 150 Nos of pit sample) subjected to analyses of trace elements REEs (La, Ce, Pr, Nd, Sm, Eu, Gd, Tb, Dy, Ho, Er, Tm, Yb, Lu) and associated elements/minerals Zr, Hf, Sr, Rb, Th, U by ICPMS instrument and As, Sb, Bi, Ag, Au by Fire Assay method at LUCID Laboratory Hyderabad.

Moreover, the external check samples of 27 Nos have been analysed for same elements from SHIVA Laboratory, Bengaluru. Further, 12Nos of selected granitic samples were subjected to analyses of major oxides, i.e., SiO₂, Al₂O₃, Fe₂O₃, TiO₂, MnO, CaO, MgO, Na₂O, K₂O, P₂O₅, V₂O₅, Loss on Ignition (LOI).

The ΣREE+Sc+Y value in pit sample varies from 23.06 ppm to 1606.07 ppm with an average value of 279.24 ppm. The ΣREE+Sc+Y value in bed rock samples analysed up to 1682 ppm with average of 294.94 ppm within the entire block area.

1.5.5	Petrographic & Mineralogical Study

Petrographic and Mineralogical study has been carried out on 30 Nos of samples collected from different variants of granites. Ten (10 no’s) Sections were Studied under SEM.

1.5.6	Preparation of Geological Report

Geological Report has been prepared in Head office of Critical Mineral Trackers, Hyderabad. Integration of geological and geochemical data was done using QGIS software for preparation of various maps and GCD kit module of R programming language has been used for generating geochemical plots. The report has been written using Microsoft word and excel.

1.6.0	Conclusions

The study area forms a part of the Eastern Dharwar Craton and predominantly occupied by variants of granites of peninsular Gneissic Complex-II and is an integral part of Narayanpet kimberlite belt in Maddur area Narayanpet district of Telangana.

The area forms a part of the Toposheet 56H/09 between latitude 16.752°N to 16.951° N, longitude 77.596°E to 77.663°E,

The lithological assemblage of the area is predominately composed of grey biotite granite pink alkali feldspar granite, porphyritic granite, leucogranite, of PGC -II which are of Archean to Paleoproterozoic. Thise are traversed by basic intrusive such as gabbro, dolerite, acid intrusive like pegmatite and quartz, five kimberlite bodies are located in the central part of the block area within the grey biotite granite. In the northern part of the block area there in major shear trending NW-SE cuts across the block and runs for several kilometers. Within the shear zone a quartz reef is emplaced which is highly sheared in nature.

The pink alkali feldspar granite which has almost negligible mafics is seen bordering this shear zone and forms a positive topographic. The granite of the area retrograde metamorphism in the form of plagioclase altering to epidote and sericite. The overall metamorphic grade varies from lower amphibolite to green schist facies.

The nature of the granitoids highly heterogenous, has gradational contacts and lacks metamorphic aureole. Stages of Granitization in the form of fragmentation and transformation of pre-existing lithounits into granitoids is evident.

Clouding of Feldspars and pink pigmentation of Feldspars suggest late-stage hydrothermal activity. Presence of myrmekitic borders to plagioclase suggest their metamorphic origin. Occurrence of Epidote indicates low temperature transformation due to hydrothermal activity. High alkali nature of these Granites is a clear indication of K-metasomatism. These Granites are Granodiorite to Quartzolite in chemical composition.

And interpreted to have formed as a result of a series of processes like assimilation, reconstitution and migmatization, caused by injection and metasomatism. when plotted in R1-R2 diagram of Bachelor and Bowden (1985). They fall in mantle fractionates field The chemical analysis, petrographic, and SEM-EDX studies complement each other in confirming the mineral assemblage like Albite, Microcline, Biotite, Muscovite, monazite, Apatite, Calcite kaolinite and illite which are the favorable minerals for hosting the Rare Earth Elements.

The geochemical anomaly threshold of the block area 240 ppm because it is lies above the       dataset median.

The Geochemical classes of the block area followed are follow

Geochemical background:      <120 ppm TREE

Weakly anomalous:                120–240 ppm TREE

Geochemically anomalous:    >240 ppm TREE

High-priority REE anomaly:  >500 ppm TREE

Exceptional REE anomaly:    >1000 ppm TREE

1.7.0    Recommendations

Five zones of REE Mineralization and one zone of precious minerals such as arsenic, antimony, gold, silver, bismuth have been identified and demarcated out of which the Pallerla zone and Lakkayapalli zone are more potential ones within the block which have given the TREE values above 1600 ppm with the maximum of value 1682. Among the REE besides Lanthanum, Cerium, Neodymium, certain uncommon elements like, Praseodymium, Samarium, Gadolinium, have analyzed higher values. Pallerla zone has the TREE   maximum value is 1682 ppm   and in Lakkayapalli zone the maximum value is more than 1600 ppm0.847Sq. km in aerial extent.

Since these are values recorded in an younger intrusive alkali feldspar granite, seen bordering the major shear zone and as per the Bachelor and Bowden 1985 plot they are considered as mantle fractionates which is also substantiated by the petrographic studies.

Therefore, it is strongly opined that one borehole in Pallerla zone and one bore hole in Lakkayapalli zone may be drilled to ascertain the depth persistence of the mineralization.

Besides these along the quartz reef which has emplaced within the major shear zone / fault zone trending NW-SE direction is observed to be highly sheared. The bed rock samples of these quartz analyzed gave for arsenic, Au, As, Ag, Bi, Sb, the maximum value of arsenic as 82.2ppm, the maximum value of gold (Au)– 1.4ppm, the maximum value (Bi) -1.3ppm, and maximum value of antimony (Sb) 19.5ppm, and the maximum value of silver (Ag) 43.5ppm.

The exceptional TREE anomaly zones more than 1000 ppm are recommended next level   exploration

CHAPTER – II

2.0.0	DETAILS OF THE QUALIFIED PERSON(S) / EXPLORATION   AGENCY

(To be provided separately for all the qualified persons signing off the report)

Table No 2.1

Details of Qualified Person(S) / Exploration Agency

| (a) Name: | CRITICAL MINERAL TRACKERS (An Exploration Agency, NPEA) (recognized Ministry of Mines, Govt. of India) |
| --- | --- |
| (b) Address: | Concourse, No. 306, Greenland's Main Road, opp. Lal Bungalow, Greenlands, Begumpet, Hyderabad, Telangana 500016 |
| (c) Contact Mobile No: | +91 7893847742 +91 9849574333 |
| (d) E-Mail id: | criticalmineraltrackers@gmail.com |
| (e) Qualification: | Professionals with M. Sc (Geology) Qualification |
| (f) Experience: | Professionals with experience more than 35years |
| (g) Affiliation to any organization/company, if yes, specify the name of the organization or company. | An Exploration Agency, NPEA Recognized Ministry of Mines, Govt. of India |

2.1.0.  Details of qualification and experience of persons associated with various aspects of exploration assessment of resources and reserves.

The Details of personnel associated with the Exploration in Maddur block Narayanpet & of Telangana, are given in following Table No 2.2.

Table No 2.2

Details Of Personnel Associated with The Exploration Maddur Area in Narayanpet Rural Districts Telangana.

| 1 | Overall Planning, Co-ordination & Overall supervision |  | Dr. K. Mahender Reddy Former Director of GSI |
| --- | --- | --- | --- |
| 2 | Project management & Field Operation | : | Shri Devigari Prem Kumar Geologist |
|  |  | : | Shri Venne Naveen Kumar Geologist |
| 3 | Chemical Laboratory | : | LUCID (NABL -Accredited) |
| 5 | Petrological Laboratory | : | GSISR Petrology Division (Outsourced) |
| 6 | Data Processing & Documentation | : | Shri Devigari Prem Kumar Geologist |
|  |  | : |  |
| 7 | Reprography & Printing | : | CMT Head office |

CHAPTER – III

3.0.0.	TITLE AND OWNERSHIP

3.1.0.	Title of the Report

GEOLOGICAL REPORT ON RECONNAISSANCE SURVEY (G-4 STAGE) FOR REE AND ASSOCIATED MINERALS IN MADDUR BLOCK, NARYANPET DISTRICTS IN TELANGANA

Ownership: Government of Telangana

Name of Prospector: (CRITICAL MINERAL TRACKERS) – NPEA (Recognized by the Ministry of Mines, Government of India)

Address of Prospector: Concourse, No. 306, Greenland's Main Road, opp. Lal Bungalow, Greenlands, Begumpet, Hyderabad, Telangana 500016.

E-mail of Prospector: criticalmineraltrackers@gmail.com

Telephone numbers of Prospector: + 91 7893847742, + 91 9849574333

3.2.0.	Details of period of Prospecting

Reconnaissance Survey (G4) for REE and Associated in Maddur block in Narayanpet districts of Telangana. Maddur Block was recommended in 78th meeting on 14th August, 2025 Technical cum Cost Committee (TCC) of NMEDT was approved in 43rdh meeting of EC, NMET (24th February 2025) and the same was intimated to CMT vide MoM letter No. F.NO. 23/648/2025-NMEDT/350 dated 04th September 2025.

Accordingly, CMT initiated Project on 04 September 2026, and the fieldwork was successfully carried out. The field activities included geological mapping on a scale of 1:12500 scale, collection of 115 bedrock samples, 150 pit samples, and along with 30 Chip Samples for petrographic studies.

The fieldwork was completed in the 2nd week of April 2025. Subsequently, Geological maps were finalized and digitized. Petrographic Studies were conducted at GSI, followed by SEM analysis. All the samples (Bedrock, Pit (Regolith) were submitted to lucid chemical laboratory for analysis of REE and associated elements. The analytical data received was interpreted and plotted on the map for delineation of the mineralized zones. The petrographic data received along with photomicrographs was studied and corroborated with the chemical data and interpreted for identifying the mineralized zones. The work was reviewed by the TCC-1 Committee on 26th May 2025 and advised to submit the Geological Report to NMEDT before the timeline i.e., 03 July 2025.

.

CHAPTER - IV

4.0.0. 	DETAILS OF THE AREA UNDER STUDY

4.1.0   	Location of the Block

The Maddur block, having an area of 160 sq. km, by latitudes 16.752°N to 16.951° N and longitudes 77.596°E to 77.663° E, forms a part of Maddur Mandal, in Narayanpet districts of Telangana state. The main localities in and adjoining the block are Maddur, Nandipad, Pallela, lakkayapalli, Jadavaraopalli, Dorepalli, Monapuram, Renuvatla, Appareddipally, Chennaram, Abhangapur, Bandakonda, kotakonda, the block is an area forms the central part of the toposheet 56H/09 and the co-ordinates of the block boundary are given below in Table No 1.1

Table No-4.1

Coordinates of cardinal points of Maddur block in

Narayanpet Districts of Telangana.

| Cardinal Points | Geographic Coordinate System in Degree Decimal (WGS 1984) |  | UTM (WGS 1984, Zone 44N) |  | Area in Sq. Km |
| --- | --- | --- | --- | --- | --- |
|  | Latitude | Longitude | Easting (m) | Northing (m) |  |
| A | 77.596 | 16.95 | 776478.86 | 1875850.73 | 160 sq.km |
| B | 77.659 | 16.951 | 783190.96 | 1876051.27 |  |
| C | 77.663 | 16.752 | 366976.59 | 1935306.34 |  |
| D | 77.596 | 16.752 | 359650.24 | 1935355.85 |  |

4.2.0.  	Accessibility to the Block

The Mahabubnagar Railway station is 49 kms south- southeast to the block area and all these are part of the South-Central Railway network. Besides this, Narayanpet, Mahabubnagar area are well connected by road network. Apart from railways, road transport services are also good.

TEXT FIG No: 1

Location map of Maddur Block in parts of Narayanpet district in Telangana.

4.3.0.	Land use land cover pattern

As reported by GSI from the land use /landcover map of the study area part of toposheet no.56 H/09 is occupied by cultivated lands, settlements, barren lands occupied by rock exposures, mango gardens and water bodies. Approximately 50 % of the area is covered by agricultural lands, Pedda Vagu a tributary of River Krishna and its stream network drain the area. Besides this several lakes are also present within the area. The landuse/landcover map of the area covered by toposheet 56H/09 is shown below in Fig No. 2 under Mineral(s)investigation or granted license or lease of Maddur Block was explored for the occurrences of Rare Earth Elements (REE) and Associated minerals.

PLATE No: 1

Landuse Landcover Map of Maddur Block

4.3.0. 	Digital Elevation Model of the Block Area marked on 56H

The Maddur block area which is part of 56H/09 has been marked on the Digital Elevation Model (DEM) map of 56H. This map represents the elevation variation across parts of Gurmatkal, Yadgir, Maddur, Wadagera, Honwada Mahktal, Magnur, upperu, Raichur,  Atmakur, Kadgchur, gabbur, districts within the NGCM toposheet 56 H. Elevation ranges from about 264m to 718 m, classified into five colour-coded categories from low (light green) to high (red). Higher elevation zones occur mainly in the north-Eastern region near Gurmatkal, Honwada Maddur, while lower elevations dominate the south-eastern areas around Atmakur, Gadwal, and. Major localities such as Gadwal, Makhtal, Gurmatkal, Maddur, Raichur, and are marked to provide spatial references. Overall, the map helps in understanding terrain relief, geomorphological patterns and the influence of elevation on regional geochemical variability.

TEXT FIG No: 2

CHAPTER – V

5.0.0.   PHYSIOGRAPHY AND ENVIRONMENT

5.1.0.     Topography

The area forms an undulating topography with scattered hillocks and mounds.  within the vast Pediplain. Overall drainage pattern is dendritic to sub dendritic and is mainly controlled by joints.  There is a major NW-SE trending Fault/Shear zone in 56H/09 toposheet which is occupied by a quartz reef and bordered by pink granite ridge. The other landforms observed are pediment, dissected plateau denudation hills, flood plain, residual hill, channel fill, which is part of the Eastern Dharwar Craton and exposes a variety of granites and under Peninsular Gneissic Complex-II.

5.2.0    Location and Toposheet Reference

The presented toposheet of the Maddur Block (part of Toposheet No. 56H/09) provides a detailed base map for geological mapping and field investigations in Narayanpet districts of Telangana. The sheet accurately depicts the spatial distribution of settlements, drainage networks, transport routes, tanks, and minor geomorphological features, which serve as important reference points during geological traverses. The red demarcated boundary highlights the designated study area, enabling precise planning of traverses, sample collection, and lithological boundary identification. This toposheet forms a fundamental preliminary tool for understanding the terrain, assessing accessibility, and correlating geological features with surface morphology within the Maddur Block.

Plate No -2

5.3.0     Outcrop Distribution and Geological Framework

The outcrop map of the Maddur Block provides a detailed representation of all exposed bedrock occurrences within the mapped area, forming an essential basis for geological fieldwork and interpretation. The distribution of outcrops, shown through shaded polygons, highlights the areas where lithological units are accessible for direct observation, sampling, and structural measurements. The block boundary is clearly demarcated in red, ensuring accurate spatial control during geological traverses. Surrounding villages such as lakkayapalli, Pallela, Nandipaad, Jadavaraoplli, Manipura, Abhangapur and Bandakonda Appareddipalli serve as important reference points for navigation and planning of field operations. This map facilitates a clear understanding of surface geological.

Plate No: 03

5.4.0.	Connectivity

The Mahabubnagar railway station is 49 kms south east of Maddur Besides this the block area is well connected by road network. Road transport are also good. Narayanpet district, Mahabubngar are well connected to Maddur by road. Apart from railway, road transport service is also good.

#### 5.5.0.	Population

As per the Census data, 2011, Narayanpet district is approximately 41,756, out of which male and female were 20,697 and 21,055. respectively.

#### 5.6.0.	Socio Demographic profile

Maddur district of Telangana State had sex ratio of 1009 females for every 1000 male. In 2011 census, child sex ratio was 916 girls per 1000 boys. The literacy rate of the district was 49.93%. It has been observed from census data; 8.51% of the population were residing in the urban area whereas rest of the population were residing in rural areas. According to the 2011 census, 82.65% of the population in the district speaks Telugu, Hindi- 0.52%, Urdu- 4.96% and 9.40% speaks Lamani/Lambadi and 1.68% of Banjari.

#### 5.7.0.	Historical / Archaeological Site

Narayanpet in Telangana is a historically significant district known for its ancient Megalithic sites and religious landmarks. Key sites include the 3,500–4,000-year-old Mudumal Megalithic Astro/ archeological site (recently shortlisted by UNESCO’ for decongestion list), the Narayanpet Fort from the Nawabi era, and ancient temples like the Padamati Anjaneya Swamy Temple

Narayanpet Fort: A structure dating back to the Nawabi era, reflecting the area's history under the Nizam and Qutb Shahi dynasties. Tangidi Village (Maga Noor Mandal): Features significant Megalithic Cist burials on the banks of the Krishna River, indicating early Iron Age settlements

5. 8.0.	 Forests, sanctuaries, national park and wildlife sanctuaries

The block contains approximately 1.7 sq km of forest area And There are no sanctuaries, national park and wildlife sanctuaries. Within this block area.

5.9.0.	 Flora and Fauna within and nearby

The Narayanpet region lies in the semi-arid zone of southern Telangana and is characterized by dry deciduous vegetation and scrub forests. The ecology is influenced by low to moderate rainfall, rocky terrain, and red sandy soils, supporting hardy plant species adapted to drought conditions. The flora mainly consists of thorny shrubs and trees such as neem (Azadirachta indica), acacia (Acacia nilotica), and banyan (Ficus benghalensis), along with grasses and seasonal herbs. The fauna includes common wildlife like Indian hare (Lepus nigricollis), fox, mongoose, and reptiles such as lizards and snakes. Bird species like peafowl, partridges, and pigeons are also found in the area. Overall, the ecological setting of Narayanpet reflects a typical Deccan plateau environment with moderate biodiversity adapted to semi-arid conditions. Sanctuaries key hotspots for conservation within the region.

5.10.0.   Drainage Pattern

There is no major drainage and the only rivulet that flows through the area from north to south is Peddavagu, a minor tributory of the river Krishna. Other streamlets and nalas that join the Peddavagu are very minor ones and often found dry. The drainage pattern in the area is generally dendritic to sub-dendritic and mainly controlled by joints.

5.11.0. Climatic Condition

Climate of the area is a typical hot semi-arid climate pattern, featuring intense summers (March–May), a monsoon season (June–October), and a mild winter (November–February).

Summer (March – May): High temperatures consistently exceeded 350 C with peak,400 C – 450 C dry heat in April and May often reaching).

Monsoon (June – October): June brought relief with pre-monsoon showers, with rainfall increasing significantly in July and August. Temperatures typically hovered around 300 C during these months.

Winter (Nov – Feb): Pleasant and dry conditions prevailed, with daytime highs around 280C and cooler nights during December and January

5.12.   Any other factor having potential to affect the viability of the   project and     assessment of resources and reserves.

CHAPTER – VI

6.0.0.	  INFRASTRUCTURE

6.1.0.	  Local infrastructure Connectivity and facilities

The block has a very good road connectivity with other towns and Headquarters by road. It takes an hour to reach Narayanpet by The Maddur block is part of the Narayanpet districts. The mandal headquarters Maddur is central of the block. District administrative offices are located in Narayanpet. Major Banks have their branches in Maddur. Post office and medical centers are available in these two towns.

The Mahabubnagar railway station 49 kms south -south east to the Maddur block and besides this, the block area is well connected by road network. And Maddur area has good road connectivity Narayanpet district.

.

CHAPTER – VII

7.0.0	 GEOLOGY

7.1.0	 Regional Geology

The geological set of the area undivided Mahabubnagar district is predominately composed of variety of granites and gneissis with enclaves of mafic and ultramafic lithounits, schistose patches, BIF, intrusives both basic and acidic.  It forms a part of the stable Dharwar craton of South Indian shield. It exposes rocks of Peninsular Gneissic Complex, Dharwar Supergroup, Cuddapah Supergroup, Kurnool and Bhima Groups and also of Deccan Traps. The Peninsular Gneissic Complex, which covers most of the area, comprises granites, gneisses and migmatites with undigested patches of older metamorphic rocks. The Dharwar Supergroup rocks are represented mainly by Gadwal Schist Belt, running in NW-SE direction between Narayanpet and Manyapadu. It exposes basic, intermediate and acid volcanic rocks associated with agglomerate and thin bands of banded iron formation. The rocks of Peninsular Gneissic Complex and Dharwar Supergroup are intruded by younger granitoids of tonalite - trondhjemite granodiorite composition and diabase dykes. These dykes are tholeiitic to sub alkalic in composition and intrude along E-W, NW-SE and NE-SW directions. A prominent quartz reef is seen along a major fault /shea zone trending in NW-SE direction extending more than 30 km. Radiometric dating by Sm-Nd, Rb-Sr and Pb-Pb methods indicated the age of the dyke systems to be around 2200 Ma. (Pandey etal, 1997). GSI reported 37 kimberlites from Kotakonda, Maddur and Narayanpet areas of the district. Which is consider to be Narayanpet kimberlite belt in the southern part of the district, Proterozoic sedimentary rocks, belonging to Cuddapah Supergroup and Kurnool Group having 1100-600 million years age are exposed.

The Cuddapah sediments, deposited non- conformably over peninsular gneisses and Dharwars, comprise conglomerate, quartzite, limestone, dolomite and shale. The Kurnool Group of rocks overlies Cuddapah rock formations and consists mainly of limestone, shale and quartzite. A thin sequence of Bhima Group of rocks (equivalents of Kurnool Group of rocks), comprising shale and sandstone are exposed in northwestern part of the district in Korangal taluk. Trap rocks of basaltic composition occur over the peninsular gneisses and Bhima Group of rocks in Korangal and Shadnagar taluks of the district

Table No. 7.1

REGIONAL STRATIGRAPHIC SEQUENCE

Structural Features

Foliation

It is very prominent in the Gneisses with a general ranging from N25W-S25E to N40W-S40E with steep dips of 50 to 70 degrees mostly due easterly. But however minor variation in strike and dips of 50 to 70 degrees mostly due Easterly. But however minor variation in strike and dip of foliation is commonly seen due to folding and flexuring.

Lineation

This feature in the form of mineral lineation was often seen in these Granitoids. Generally, it is seen on the horizontal surface mainly marked by the preferred orientation of K-feldspar phenocrysts in porphyritic granodiorite whose longer axis is parallel to the regional foliation of the gneisses. Apart from this groove lineation is also seen in gneisses. These are well preserved in the magnetite/hematite quartzites near Vanchanagiri and Rangapuram in the form of meso-folds. At Rangapuram a very good meso-fold was observed which has a plunge of 50 degrees due to N400W, and is tight Isoclinal in nature. The regional foliation also is in the same direction. In a quartzite hill near Vanchanagiri village, it was observed to have folded, with its axial plane trending in NNE-SSW direction, which is again refolded along NW-SE direction. This fold has a plunge with an angle of 50 degrees, due to NE direction. From the data collected from the area, it bears the imprints of at least 2 phases of deformation.

These are the most common secondary structures noticed in rocks of the area. Several sets of joints having a trend in almost all direction, but however on analysis it is found that the most prominent joint directions are N-S, E-W, NE-SW/N30W-S30E, N60E—S60W. Besides these a few sheet joints resulting in the blocky nature of the outcrops were also observed.

Lineaments

Several Lineaments were interpreted in the area which trend in NE-SW, NW-SE and N-S directions.

Fault

A major fault/shear extending in NW -SE direction

along Gundumal, Nandipad and Allapur was inferred. Along this fault zone quartz reef is seen emplaced and it’s highly sheared in nature.

Migmatitic Structures

The migmatite structures observed in the area are stromatic or layered, Agmatic, Schlieren, Folded, Pygmatic, Ophthalumic and Nebulous structures. These represent the different stages of migmatization noticed in area. Among the above-mentioned structures, layered and Agmatic structures are the most common ones.

Metamorphism

The Granitoids of the undivided Mahabubnagar show imprints of retrograde metamorphism in the form of garnet altering to biotite, biotite to chlorite, plagioclase feldspar to epidote and sericite in gneisses and granites, pyroxene to amphibole in pyroxenites.

The rocks of the area at present comprise the mineral assemblage characteristic of Almandine Amphibolite facies to green schist facies. The amphibolites and schist enclaves and metasediments show the effect of regional metamorphism. Neither cordierite nor sillimanite was noticed in the rocks suggesting a low grade of metamorphism.

The occurrence of clots of garnet in grey granodiorite and gneisses mostly occupying the central core portion of the ferromagnesian minerals suggests that they could be met amorphic residues and hence it would be proper to explain their development as a result of argillaceous sediments. This is evidenced by the presence of Almandine garnet surrounded by biotite. Which suggests that it has undergone retrograde metamorphism. The granodioritic rocks which are considered to be there constituted Dharwar rocks are often schistose and present some interesting metamorphic effects on them. The feldspars are highly epidotised and sericitised, segregation of quartz resulting in recrystallization, the biotite shows fragmentation and some of the porphyritic varieties of aplitic facies exhibit development of K - feldspar augens.

In spite of the presence of enclaves of gneisses and schistose rocks in the Granodiorites the following points suggest that the granitoids are not of intrusive origin but are a product of metasomatism and migmatisation. The dykes of the area are in general fresh and unaltered/ but they have caused some effects along the borders of the country rock into which they are intruded.

Geochemistry

The granites of undivided Mahabubnagar district and its adjoining areas are formed to be highly heterogeneous in nature. The selected samples of these granitoids were analyzed for major and trace element concentration. Their analytical data were plotted various geochemical diagrams to arrive at their classification, source and nature of magma and their tectonic environment in which they got emplaced. When plotted the analytical data in QAPF diagram they plotted mostly in Granodiorite and Tonalite fields and few in Monzo granite. The feldspar triangle of O Connor (1965) indicated them to be Granodiorite to Quartz monzonite. The R1—R2 plot of De La Roche et al (1980) suggested these to be granodiorite to tonalite in composition. The data plotted on Middlemost (1994) diagram reflected them to be granodiorite to Quartz monzonite. TAS diagram of Cox et. al. (1979) indicated them to predominantly Granodiorite in composition. They’re taking into consideration all the above plots these granites can be geochemically classified as Granodiorite- tonalite - Quartz monzonite suite (TGM).

The SiO₂, K₂O plot of Peccerillo and Taylor (1976) indicates that these granites are predominantly high K-Calc alkaline in nature. AFM plot of Irvine and Baragar (1971) also suggests them to be Calc- alkaline in nature.

As per the tectonic discrimination diagram of Frost and Frost these granites are magnesian, Calc-alkali strongly peraluminous to marginally metaluminous and silica saturated in nature. The Si O₂ – K₂O plot suggests it to be S-type due to high potash whereas the Frost diagram suggests it to be I-type, probably these are affected by K metasomatism. As per the tectonic discrimination diagram of Maniar and Piccolli (1989) these granitoids fall in the IAG-CAG-CCG field and are more peraluminous and marginally metaluminous, highly magnesian, Calc-alkaline and silica saturated as per Frost and Frost (2008) tectonic discrimination diagram. As per the R₁—R₂ plot of Batchelor and Bowden (1985), these are post orogenic and pre-plate collision emplaced granites.

REE Mineralization

The Granitic variants within the GQM 56H map which forms part of the mahabubnagar district and ngcm data of REE reported TREE ranging from the 21.21 to 2572.35ppm. Among these REE, concentration of LREE specifically Ce (1418.52) maximum, La (533.60PPM max) Nd(214.24max), Eu (41.40 max) Pr (108.26max) Sm (40.83 maxPPM) Gd (24.83ppm max), Dy (20.23ppm max) granodiorites of the area gave 2 to 2 1/2 times more than the background values when a Geiger Muller Counter was operated the chemical analysis of the random samples collected from these granites gave significant concentration of LREE specifically Cerium (900-1200 ppm) and Lanthanum (200-400 ppm). Systematic studies and close spaced sample collection and their analysis might result in locating Cerium and Lanthanum deposits since these elements are sought after these days.

AGE

The granites of undivided Mahabubnagar district are considered to be equivalent of Closepet Granite which is 2500 Ma and the field relations substantiate this since they are a part of the Younger Gneissic Complex which is stratigraphically referred to as Peninsular Gneissic Complex -II and is considered to be of Archaean to Paleoproterozoic age. block location has been shown in the Text Figure 3 below

Text Fig No: 03

Plate No - 4

7.2.0.	 GEOLOGY OF THE BLOCK

The Maddur block area is located in Narayanpet district of Telangana and geologically part of the Narayanpet Kimberlite belt within the Eastern Dharwar Craton. The block area is 160 sq. kms and is part of the toposheet 56 H/09. The block area is composed of grey biotite granite occupying the central part of the block which is mostly a Pediment- Pediplain complex with ground level outcrops whereas the northern part and the southern part of the block is occupied by fine grained pink granite which is more aplitic in nature with almost negligible mafics and forms hills extending in NW-SE direction. Porphyritic granite is located east of Abhangapur village in the southern part of the block. Leucogranite is exposed in the southern part of the block area south of Basannapalli and east of Bommannapadu where few rocky knobs are present. Meta Gabbro is located south of Banda Konda village. Dolerite dyke located south west Monapuram. Pegmatite vein are located North of Monapuram and northeast of Jadavaraopalli These trend in NW-SE direction in the northern part of the block a major shear/fault zone passes through the block in a NW-SE direction which is occupied by the Quartz reef.

The Geological Survey of India (GSI) has identified 37 kimberlite pipes within the Narayanpet Kimberlite Belt. Of these, six kimberlite bodies, namely MK-2, MK-3, MK-4, MK-5, MK-6, and KK-3, are located within the Maddur Block area which are mostly covered by soil no outcrops were seen

Plate No: 05

7.3.0 Structures of the block

Joints

The lithounits which comprise predominantly granitic variants and are well jointed and are the most common secondary structures noticed in rocks of the area. Several sets of joints having a trend in almost all directions, but however on analysis it is found that the most prominent joint directions are N-S, E-W, NW- SE, SW- SW.

Lineaments

Prominent lineaments observed in the area are the major fractures which control the drainage network, the directions in which they are oriented are in NW- SE, NE-SW and N-S directions but among these the NW-SE trending lineaments are very significant. Along some of these lineaments dolerite dykes are seen emplaced and along some others stream courses are noticed.

Migmatitic Structures

The migmatite structures observed in the area

Field photograph No: 7.3.0- 1

are stromatic or layered, Agmatic, Schlieren, and Nebulous structures. These represent the different stages of migmatisation noticed in the area among the above-mentioned structures, Agmatic, Schlieren and Nebulous structures are the most common ones.

Shear Zone

A major shear zone extending in NW -SE direction along Gundumal, Nandipad and Allapur was inferred. Along this fault zone quartz reef is seen emplaced and it’s highly sheared in nature.

Fault

there are faults trending in NNW-SSE-SSE direction located north of Renuvatla which are are parallel to each other these are smaller areas extending for about are kilometer these are in the central

Field photograph No: 7.3.0-3

Field photograph No: 7.3.0-2

Field photograph of sheared quartz seen along the major shear/fault     zone trending NW-SE travesing the northern part of the block area

Near Nandipad evidences for ductile deformation was observed in the form of mylonitization where Feldspar often forms rounded to augen shaped clasts with thin pressure shadow tails. At most of the place’s sericite and epidote replace the feldspar. Epidote also formed along the mylonite zones probably due to fluid movement. Titanite grains are completely skeletal in shape and formed over other grains indicating its late-stage development. Alteration of amphibolite-rich and feldspar-quartz rich bands define banding parallel to mylonitic foliation. Often zone of epidotization cut across the amphibolites and are parallel to the mylonitization. Based on the petrographic study, it is inferred that the original amphibolite rocks was affected by high temperature ductile deformation and shearing resulted into thin mylonite zone where grain size drastically reduced and S-C fabric was formed. The shearing was accompanied by intrusion of epidote veins. This rock represents a zone of high shear strain.

Micro photograph No: 7.3.0 -4

7.4.0. Description of lithology

Grey Biotite Granodiorite

Field Photograph No: 7.4.0 -1.1

This Lithounit is medium to coarse-grained, grey in colour, meso to melanocratic, inequigranular and is primarily composed of Quartz, Plagioclase felspar, K-felspar, Biotite and under the microscope, perthitic alkali feldspar, microcline, plagioclase (Plg), and biotite (Bt), along with accessory phases such as epidote, apatite, allanite, titanite (Ttn),

zircon (Zrn), have been identified. This unit is seen occupying the central part of the block area north and south of Maddur and is the predominant lithounit within this block area. It’s observed that this lithounit has at places enclaves

of the parent rock which are mostly mafic rich and

Field Photograph No: 7.4.0 -1.2

are considered to be restites which are predominantly. Amphibolite, The Biotite in this lithounit appears as subhedral flakes with moderate to strong pleochroism

and is commonly associated with accessory minerals. Accessory phases are well represented. Apatite occurs as small prismatic inclusions within biotite and feldspar, indicating early magmatic crystallization. Allanite is observed as high relief, brownish grains, often associated with altered places. Zircon occurs as small, euhedral grains with high relief and bright interference colors, typically enclosed within biotite, indicating early crystallization from the magma.

Allanite is dark brown in color and are associated with chlorite and epidote. Biotites are pleochroic in shades of yellow to brown and most of the places are replaced by chlorite along cleavages. Biotite and chlorites   mostly occur as interstitial grains. Plagioclase grains are identified by its lamellar twinning and tiny epidote grains were formed over it. A late thin epidote vein cut across the rockThe presence of epidote (Ep) as high-relief, brightly birefringent grains associated with plagioclase indicates saussuritization under hydrothermal conditions.

Biotite (Bt) appears as pleochroic, flaky grains with moderate to high interference colors, often hosting accessory phases. Allanite (Aln), identified by its high relief and strong interference colors, occurs in association with biotite. Apatite (Ap) is present as small prismatic inclusions, commonly enclosed within biotite or feldspar.

Orthoclase is mostly altered to sericite indicated by its dusty appearance. Microcline grains have formed later as it has numerous inclusions of quartz and plagioclase.

Zircon is a secondary mineral which occurs as euhedral to subhedral accessory grains with high relief and bright interference colors, typically included within biotite or quartz, indicating a primary magmatic origin (Fig. 15F). The assemblage of epidote, sericite, and altered feldspars suggests fluid-assisted metasomatism, overprinting the original magmatic textures.

Micro photograph No: 7.4.0 -1.3

Alkali Feldspar Granite

This lithounit is located in the northern part of the block area forming a ridge bordering a major fault/shear zone trending NW-SE  extending for several k Microscopic study reveals that the rock exhibits a coarse-grained, inequigranular texture. The modal mineral assemblage is dominated by K-feldspar (microcline and perthite), plagioclase, quartz, and accessory phases apatite, allanite and zircon.

Field Photograph No: 7.4.0 -2.1

Photograph no: 4.b showing View

of the pink alkali feldspar granite

ridge around Lakkayapalli in the

northern part of the block area

Field Photograph No: 7.4.0 -2.2

Accessory minerals include titanite (Ttn), recognized by its high relief and wedge shaped morphology, and allanite (Aln), which appears as brownish, high-relief grains often associated with alteration halos.

Field Photograph No: 7.4.0 -2.3

The spatial association of epidote, calcite, and allanite with feldspar alteration zones indicates significant fluid-mediated metasomatism during the late to post-magmatic stages. Based on the petrographic features suggest crystallization from an evolved, silica-rich magma followed by extensive sub solidus re-equilibration and hydrothermal alteration. Porphyritic rock consisting of large alkali feldspar phenocrysts within quartz –biotite rich matrix. Alkali feldspar grains are characterized by tapered cross hatched twinning and flame perthite. Myrmekite intergrowth of albite-quartz along the feldspars boundaries is also conspicuous. Often coarsening of myrmekite form tiny grains along the boundaries. Alteration of alkali feldspar to sericite at places give the dusty appearance of feldspar.Epidote, sericite and chlorite are the secondary mineral assemblages formed during late hydrothermal alteration of the granitic rock. Alteration of orthoclase feldspar to sericite give a dusty appearance to it.

micro photograph No: 7.4.0 -2.4

brown coloured, anhedral allanite(?) are present with an epidotrim. Biotite flakes are present as small mafic clusters. It is pleochroic from pale yellow to brown. At most of the places, biotite has been replaced by pale green coloured chlorite. Epidote also occur as veins passing through a brecciated zone. This zone consist of broken feldspar grains and interstices are filled up by epidote.

Leucogranite

Field Photograph No: 7.4.0 -3.1

This lithounit is leucocratic, medium to fine grained exhibiting an inequigranular texture and in primarily composed of plagioclase feldspar and quartz with minor amount of biotite oriented and forms a thin streak at places. This unit is exposed in the southern part of the block area south of Basnnapalli and east of Bommannapadu where few rocky knobs are present. Under the microscope besides plagioclase, quartz, biotite accessory minerals like zircon and secondary minerals such as epidote and sericite have also been observed. Plagioclase is subhedral and is characterized by its distinct thin and closely spaced lamellar twinning indicating, indicative of an albite (Na)-rich

Micro photograph No: 7.4.0 -3.2

porphyritic granite

Porphyritic granite is located west of Abhangapur village in the southern part of the block

Field Photograph No: 7.4.0 – 4.1

It is coarse-grained and inequigranular, displaying a distinct porphyritic texture. The predominant mineral assemblage is dominated by quartz and plagioclase in the groundmass with phenocrysts of K-felspars mostly orthoclase with   minor amounts of microcline (Fig. 31A).  under the microscope it’s observed that biotite occurs as segregated clusters, commonly surrounding K-felspar phenocrysts and occupying the interstitial spaces of the groundmass. The principal accessory mineral identified is zircon, while opaque phases are represented mainly by magnetite with inclusions of ilmenite

Quartz is generally medium-grained and locally exhibits sub-grain formation adjacent to plagioclase. The plagioclase crystals display well-developed polysynthetic twinning (albite/carlsbad type), though in places the twinning is partially obliterated due to alteration. Grain boundaries are diffuse and irregular, reflecting recrystallization and secondary alteration processes (Fig. 31B).

Biotite is relatively common, forming subhedral to euhedral, elongated, foliated flakes. It shows distinct pleochroism from light to dark brown and frequently encloses accessory minerals. A characteristic feature observed is the development of myrmekite texture, manifested as micrographic intergrowths of vermicular quartz along the margins of plagioclase feldspar (Fig. 31C).

Zircon typically occurs as inclusions within biotite (Fig. 31D). Additional accessory minerals include rutile and magnetite (Fig. 31E). Minor sulphides are also recorded, occurring as fine   disseminated grains of pyrite and chalcopyrite (Fig. 31F). the precipitation of secondary minerals such as epidote, and sericite. The rock represents a plutonic, albite-rich felsic lithology, initially crystallized from an evolved magma and subsequently overprinted by tectonc deformation and hydrothermal fluid infiltration.

Micro photograph No: 7.4.0 – 4.2

Meta Gabbro

Meta Gabbro is located south of Bandakonda village and it is coarse grained, melanocratic mainly composed of Clinopyroxene, Amphibole, Plagioclase, Titanite Alkali Feldspar with Secondary minerals: Sericite Epidote and sphene showing a granoblastic aggregate. Amphiboles are green to dark green in pleochroism and associated closely with clinopyroxene. Clinopyroxene mostly occur as subhedral grains as well as inclusions in amphiboles. Clinopyroxenes are characterized by pale green colour, one set of cleavage, higher order interference colour and inclined extinction... Secondary minerals such as sericite and epidote were formed by replacing plagioclase. Epidote grains are subhedral and mostly forming over plagioclase and often near amphiboles. Titanite, a dark yellowish brown coloured mineral formed interstitially. Inclusions of other minerals are also present within this late brown-coloured minerals.

Field Photograph No: 7.4.0 – 5.1

Micro Photograph No: 7.4.0 – 5.2

Intrusives

The entire block area comprising of different variants of granites are traversed by a number of basic and acid intrusives. The basic intrusives are predominantly Dolerites  with few Gabbro exposures. The acid intrusives include the Quartz and Pegmatite. The area being an integral part of the Narayanpet Kimberlite Belt where a number of clusters   have been identified by GSI and five kimberlites of Maddur clusterare located within this area and all of them are within the grey biotite granite. The basic and acid intrusives are of Paleoproterozoic age whereas the kimberlites are of Mesoproterozoic age. The details of these are given below.

Pegmatite

Pegmatite vein are located North

of Monapuram

and northeast of Jadavaraopalli

These trend in NW-SE direction

and are about 350 to 400 m long

and 150 to 200 m wide

Field Photograph No: 7.4.0 – 6.1

Quartz reef/vein

In the northern part of the block a

major shear/fault zone

passes through the block in a NW-SE direction

along which a Quartz reef is seen emplaced.

Field Photograph No: 7.4.0 – 7.1

Dolerite dyke

View of the pink alkali feldspar granite ridge located south west Monapuram dolerite cutting across the alkali feldspar granite

Field Photograph No: 7.4.0 – 8.1

Kimberlite

Kimberlites were located by GSI around Maddur and Narayanpet areas in undivided Mahabubnagar district, Telangana, and it has been referred as Narayanpet Kimberlite field (NKF). In this field 37 kimberlite pipes were identified which got emplaced during Mesoproterozoic, having age range of 1000-1300 my. The kimberlites of this field are small bodies except the Kotakonda and Vinjamur Kimberlites (KK-1 and KK-2) which are about 2 km in length with width varying from 20m to 100m. The outcrops are very few and

sparse and are generally of 1 m x 2m in size. The kimberlites are oval, elliptical and linear in shape The morphological, textural and mineralogical characters of the pipe rock indicate that they are eroded to near root zone level. The kimberlites are usually weathered and altered with a thin layer of calcrete/ or covered by colluvium. In this block area the cluster of this kimberlites have been named by GSI as Maddur cluster of which six pipe rocks are marked around Appireddipalli, Abhangapur, and Nidjinta, Monapuram but no outcrops are seen covered by regolith within the cultivated lands. As per GSI literature these rocks are hard, compact, dark blue, porphyritic enclosing numerous xenoliths of country rocks. The mineral assemblage of kimberlites reported are olivine, perovskite phlogopite, richterite, serpentine and carbonates along with xenocrysts of chrome diopside. Chrome-spinel, picro-ilmenite are also recorded in varying proportions in different pipes.

7.5.0 SEM Studies:

The lithounits which analysed for significant values of REE were probed under SEM-EDX are found that the minerals, such as Monazite, Allanite, biotite, ilmenite,  muscovite, Albite, Microcline, Calcite, Apatite, Bastnasite, Thorite, Titanite zircon which are the favorable minerals which host the rare earth elements, such as cerium, lanthanum, neodymium, samarium, europium, gadolinium, praseodymium and other rare earth elements. However, the Alkali feldspar granite samples analyzed significant concentration of REE (up to 1682 ppm) in Pallerla zone-and up to 1600 ppm in Lakkayapalli zone. (Cerium-lanthanum-Phosphate), Allanite-a cerium epidote, biotite, Muscovite are potassium, Albite and Microcline are alkali feldspars presence of Monazite Allanite, Bastnasite also contributes to the REE concentration since Monazite is Cerium lanthanum- Phosphate Hence, the mineralogical assemblage of the lithounit alkali feldspar granite, which is geochemically Quartzolite in composition, is hosting these REE minerals This lithounit is seen bordering the major shear zone Neither side granodiorite in composition, is hosting these REE minerals. When the chemical data was plotted in the R1-R2 plot of Bachelor and Bowden (1985). It fell in the Mantle fractionate field.

Thin section Sample No: –MN-47

Rock Type: Alkali feldspar granite

Back Scattered image (BSE) Showing Bastnasite, Allanite, K- feldspar, Quartz

Text Fig: No: 4

Thin section Sample No: MN-40

Rock Type: Alkali Feldspar Granite

Back Scattered image (BSE) Showing Zircon, iron oxide, Argentite, amphibole, biotite,

Text Fig: No: 5

Thin section Sample No: MN-25

Rock Type: Grey Biotite Granite

Text Fig: No: 6

Thin Section Sample No:  MN-60

Rock Name: Alkali Feldspar granite

Back Scattered image (BSE) Showing

Zirconium, Plagioclase

Text Fig: No: 7

Back Scattered image (BSE) Showing

Plagioclase

Thin section Sample No: MN- 67

Rock type: Alkali feldspar granite

Back Scattered image (BSE) Showing

Apatite, vanadium with cerium, apatite, zircon,amphibolite

Text Fig: No: 8

Thin section Sample No: MN- 35

Rock Type: Grey Biotite Granite

Back Scattered image (BSE) Showing

Thorite, Iron oxide, K- feldspar, Plagioclase, Allanite, Zircon

Text Fig: No: 9

Thin section Sample No: MN – 61

Rock Type: Grey Biotite Granite

Back Scattered image (BSE)

Showing thorite, ilmenite, iron oxide, albite,

Titanite, k-felspar

Text Fig No: 10

Thin section Sample No: MN-57

Rock Type: Alkali Feldspar Granite

Back Scattered image (BSE)

Showing k- feldspar, biotite, apatite, allanite, thorite

Text Fig No : 11

Thin section Sample No: MN-17

Rock Type: Alkali Feldspar Granite

Back Scattered image (BSE)Showing Thorite, Iron Oxide, Allanite, Albite, Zircon

Text Fig No: 12

Thin Section Sample no: MN-7

Rock Type: Alkali feldspar granite

Back Scattered image (BSE)Showing iron oxide, biotite thorite, k-feldspar,muscovite, epidote

Text Fig No: 13

#### 7.6.0 Geochemistry

The geochemical evolution of granitic system is governed by progressive magmatic differentiation, late-stage fluid evolution, and hydrothermal overprinting. Major-element chemistry provides the first-order constraints on magma evolution, whereas trace elements including high field strength elements (HFSE), and rare earth elements (REE) preserve information on incompatible-element enrichment, accessory-mineral crystallization, and fluid–rock interaction. Integration of these datasets is therefore

essential for understanding the petrogenesis of REE systems and assessing their mineralization potential. REE commonly represent the final products of extreme fractional crystallization. During this process, early crystallization of quartz, feldspar, and ferromagnesian minerals progressively removes compatible elements from the melt, leading to concentration of incompatible elements in the residual liquid. Once saturation of accessory phases such as zircon, monazite, xenotime, allanite, apatite, and columbite-group minerals is reached, the geochemical behavior of HFSE and REE becomes increasingly controlled by these minerals rather than by major rock-forming phases. Subsequent hydrothermal activity may substantially modify the major-element composition of the host rocks through silicification, alkali depletion, sericitization, chloritization, and carbonatization. In contrast, many REE-bearing accessory minerals remain comparatively resistant to alteration, allowing the primary magmatic REE signature to be preserved even where major oxides have been extensively redistributed. Consequently, interpretation based solely on major oxides may underestimate the degree of magmatic evolution unless integrated with REE and HFSE geochemistry. The objective of this chapter is therefore to integrate the major-element, HFSE, and REE datasets into a coherent petrogenetic model describing magma evolution, hydrothermal modification, and REE and precious metals enrichment.

The analyzed samples are characterized by exceptionally high silica contents ranging from approximately 76 to 85 wt.% SiO₂, indicating that they represent highly evolved felsic rocks or intensely silicified equivalents. In contrast, Na₂O and K₂O contents are uniformly low, whereas Al₂O₃ varies moderately and Fe₂O₃, MgO, TiO₂, and CaO display localized enrichment in a limited number of samples.”

The granitic variants of Maddur block are found to be highly heterogeneous in nature and the selected samples of these were analyzed for major oxides and trace elements such as REE and associated elements including the precious elements As Sb Bi within the quartz reef emplaced along the major fault zone trending NW- SE be targeted along the evaluating their concentration.

Twelve (12) Nos of granitic samples were Chemically analysed for major oxides. The details are shown on the table below:

#### Table No: 7.2

#### Comparison of Major Oxide values of Granite

| Major Oxide | Range of Values for granite |  |  |
| --- | --- | --- | --- |
|  | Min % | Max % | Average |
| SiO2 | 76.08 | 85.1 | 82.41 |
| Al2O3 | 10.6 | 15.58 | 12.33 |
| Fe2O3 | 0.62 | 2.61 | 1.64 |
| MnO | 0.01 | 0.04 | 0.02 |
| MgO | 0.03 | 0.61 | 0.28 |
| Cao | 0.72 | 3.6 | 1.45 |
| Na2O | 0.19 | 0.39 | 0.26 |
| K2O | 0.04 | 0.31 | 0.23 |
| TiO2 | 0.02 | 0.33 | 0.18 |
| P2O5 | 0.02 | 0.11 | 0.06 |
| V2O5 | 0.37 | 0.01 | 0.73 |
| Loss on Ignition (LOI) | 0.01 | 1.46 | 0.01 |

Their analytical data were plotted on various geochemical diagrams to arrive at their classification, source and nature of magma and their tectonic environment in which they got emplaced.

The QAP diagram of albert streckeisen with modified CIPW normative calculations indicate overwhelmingly Quartz rich Granitoid field with one sample after the IUGS classification. The dominance of normative quartz reflex exceptionally high silica contents combined with strong depletion of normative felspar components resulting from the low Na2O and K2O concentrations.

The QAP diagram demonstrate that silica saturation remained extremely throughout the evolution of the system. Such quartz dominating residual compositions are consisted with late-stage granitic different ions and provide favourable conditions for the concentration of incompatible elements within Residual melts. The TAS diagram of M.J.Le Bas, Maitre placed all the analysed samples in the high silica but extremely low alkali portion of the classification field although silica contents correspond to evolved.

Felsic compositions, total alkalis significantly lower than those of unaltered rhyolites all granites. Such a position is typical for fresh igneous rock and these interpreted to reflect extensive alkali deflation rather than primary magma chemistry.

Hydrothermal fluids capable of destroying feldspars and promoting quartz enrichment commonly produced similar chemical signatures through removal of Na and K while leaving silica relatively immobile.

Consequently, the TAS classification should be regarded as an alteration sensitive indicator rather than a representation of the original magma composition. The silica rich alkali poor nature of the samples supports extensive hydrothermal modification of an evolved felsic precursor.

As per Shands classification the ASI graph where the A/NK, - A/CNK diagram indicates strongly peraluminous compositions. In fresh granite A/CNK values generally indicate S-type or strongly peraluminous magmas. In the present case the indices are more reasonably interpreted as evidence of hydrothermal alteration involving feldspar detraction and residual alumina enrichment.

Text Fig: No :15

Text Fig: No :14

Text Fig: No :16

Text Fig: No :17

Text Fig: No :18

Text Fig: 19

Text Fig: 20

Text Fig: 22

Text Fig: 21

Text Fig: No :23

When plotted the analytical data in QAP diagrams they plotted predominantly in quartzolite and Granodiorite fields The feldspar triangle of O Connor (1965) indicated them to be Granodiorite to tonalite in composition.

QAP diagram for by modified CIPW normative calculations indicating overwhelmingly quartz-rich normative compositions. Normalized QAP proportions place most samples within the quartz-rich granitoid field with one sample After the IUGS classification The dominance of normative quartz reflects the exceptionally high silica contents combined with strong depletion of normative feldspar components resulting from low Na₂O and K₂O concentrations. The QAP diagram demonstrates  that silica saturation remained extremely high throughout the evolution of the system. Such quartz-dominated residual compositions are consistent with late-stage granitic differentiation and provide favorable conditions for the concentration of incompatible elements within residual melts.

R1—R2 plot of De La Roche et al (1980) suggested these to be predominantly granodiorite and couple them of granite comosition. Which are highy alkali rich. The data plotted on Middlemost (1994) diagram reflected them to be predominately quartzolite except one plot fell granite field. TAS Diagram of Coxeet al (1979 indicated them to be highly acidic and predominately granite in composition except one which ploted in the marginal zone of granite – granodiorite.

Taking into consideration all above plots these granitic variants can be           geochemically classified as Granite -granodiorite – quartzolite suits (GGQ) The sio2 -k20 plot of percirillo and Taylor 1976 indicates that these granitic variants are calk alkaline to tholeiitic in nature. As per the tectonic Discrimination diagram of Frost – frost diagram (2008) these are ferromagnesian, calcic, strongly peraluminous, and silica saturation in nature, as per the tectonic discrimination diagram of manior and piccolo (1989) these grnitic varients fall in the borderline of IAG-CAG-CCD and RRG- CEUG. According to the R1-R2 plot of Batchelor and bowden (1985) These are mantle fractionates

As per the tectonic discrimination diagram of Maniar and Piccolli (1989) these granitic variants fall in the IAG-CAG-CCG field whereas they are more peraluminous and marginally metaluminous, ferro-magnesian, Calcic and silica saturated as per Frost and Frost (2008) tectonic discrimination diagram. According to the R₁—R₂ plot of Batchelor (1985), these are mantle fractionates.

Summarized Table showing range of elemental concentration in Granitic Variants

Table No: 7.3

| Lithology | Granite |  |  |
| --- | --- | --- | --- |
| Element | Minimum Value ( ppm) | Maximum Value (ppm) | Mean Value (ppm) |
| La | 0.25 | 447 | 66.48 |
| Ce | 0.8 | 753.2 | 126.33 |
| Pr | 0.25 | 78.2 | 12.60 |
| Nd | 0.25 | 250.7 | 41.71 |
| Sm | 0.25 | 39.8 | 6.90 |
| Sc | 0.25 | 66.8 | 7.69 |
| Eu | 0.25 | 10.6 | 1.31 |
| Gd | 0.25 | 30.6 | 5.31 |
| Tb | 0.25 | 3.9 | 0.98 |
| Dy | 0.25 | 21 | 3.67 |
| Ho | 0.25 | 3.8 | 0.97 |
| Er | 0.1 | 11.9 | 1.99 |
| Tm | 0.2 | 1.5 | 0.52 |
| Yb | 0.25 | 8.8 | 1.81 |
| Lu | 0.25 | 1.2 | 0.47 |
| Y | 0.25 | 110.1 | 19.32 |
| U | 0.5 | 32 | 6.16 |
| Th | 0.5 | 385.6 | 52.10 |
| Hf | 0.3 | 10.7 | 3.34 |
| Rb | 1.4 | 305 | 144.53 |
| Sr | 1.4 | 1140.2 | 189.40 |
| Ta | 0.4 | 8.3 | 1.33 |
| Nb | 1.2 | 197.7 | 11.01 |
| Zr | 4.8 | 433.9 | 114.52 |

Mineralization

The present exploration and object to delineated the mineralized zones  for REE   within the granite  variants Precius elements Au, Ag, As, Sb, Bi, along the major  zone occupied by quartz reef was caried out, in the process  collected and chemically analysed the 115 bed rock samples, 150 pit samples, latter carried out petrographic and SEM  studies and based on the this data interpreted and delineated six potential zones of REE , And are zone of precious elements these zones are marked on the the geological map the block area and their individual zone details are

The analytical data was interpreted and the mineralized zones have been demarcated and details of each are follows six mineralized zones have been delineated within the block area, namely Lakkayapalli Zone, Pallerla Zone, Nandipad zone -A, Nandipad zone – B, Pedripad, Jadavaraopalli, The REE values recorded in Pallerla Zone-A range from 150 ppm to 1,682 (ppm). Lakkayapalli zone – 0.845Sq.km – TREE- 359.1-1600 (ppm)Pallerla zone - 0.837Sq.km – TREE – 462 -1682 ppm

Nandipad zone – A (2.8 – TREE – 141 – 1182.8ppm)

Nandipad zone – B (Precious metals)

As – 1.1 to 82.2ppm, Au – 0.02 – to 1.4 ppm

Sb -1.3 – to 9.4 ppm

Bi - <0.5 to 1.3 ppm

Ag- 0.4to 43.5

Pedripad zone - 0.233 Sq.km – TREE - 89.7 -1529ppm

Jadavaraopalli Zone – 1.05 Sq. km – TREE196.1 to 1426 ppm

Petrographic studies thin section 30 thin sections of different granitic variants pertaining to the block area was carried out by GSI, SR petrological division which rewied presence of allanite apatite biotite alkali feldspars which host rare earth elements. Beside these zircon was also identified which host zirconium and hafnium, Titanite a titanium bearing mineral was also identified and reported. Ten (10) thin sections were selected submitted to lucid laboratories for SEM studies for mineral characterizes the results show presence of allanite, biotite apatite alkali feldspars, zircon, and titanite, which corroboted with analytical data and petrographic studies where high values of TREE 1682 ppm which includes values are Ce, La, Nd, Pr, Gd, Sm, Dy.

PLATE NO: 6

Plate No 7

Plate No 8

PLATE N0- 9

Plate No: 10

Plate No: 11

Plate No: 12

Plate No: 13

Plate No: 14

Plate No: 15

Lakkayapalli zone Most of the REE data in this plot shows negative Europium anomaly indicating late crystallization of feldspar in magma melt.

Text fig- 24

Plate No: 16

Plate No: 17

Plate No: 18

Plate No: 19

Plate No: 20

Plate No: 21

Plate No: 22

Pallerla zone Most of the REE data in this plot shows negative Europium anomaly indicating late crystallization of feldspar in magma melt.

Text fig- 25

Plate No: 23

Nandipad – A zone Most of the REE data in this plot shows negative Europium anomaly indicating late       crystallization of feldspar in magma melt.

Text fig No: 26

Plate No: 24

Plate No: 25

perdripad  zone Most of the REE data in this plot shows negative Europium anomaly indicating late crystallization of feldspar in magma melt.

Text fig -27

Plate No: 26

Jadavarao palli, The REE data in this plot shows nagative Europium anomaly indicating early and late crystallization feldspar in magma melt.

Text fig -28

Values reported below detection limits were replaced by one-half of the stated detection limit

for calculations (for example, <0.5 was treated as 0.25).

REE concentrations were normalized to CI-chondrite values commonly attributed to Sun and McDonough (1989): La 0.237, Ce 0.613, Pr 0.0928, Nd 0.457, Sm 0.148, Eu

0.0563, Gd 0.199, Tb 0.0361, Dy 0.246, Ho 0.0546, Er 0.160, Tm 0.0247, Yb 0.161

and Lu 0.0246 ppm. Calculated parameters:

  - Eu/Eu* = EuN / (SmN x GdN)^0.5

  - Ce/Ce* = CeN / (LaN x PrN)^0.5

  - (La/Yb)N and (Gd/Yb)N for LREE-HREE fractionation

  - Y/Ho, Zr/Hf, Th/U and LREE/HREE ratios

Following table shows the descriptive statistics;

| Parameter | Minimum | Q1 | Median | Mean | Q3 | Maximum |
| --- | --- | --- | --- | --- | --- | --- |
| Total REE | 8.05 | 116 | 175 | 295 | 344 | 1.68e+03 |
| Eu/Eu* | 0.225 | 0.527 | 0.696 | 0.75 | 0.853 | 3.05 |
| Ce/Ce* | 0.53 | 1.01 | 1.07 | 1.16 | 1.19 | 3 |
| Y/Ho | 0.25 | 26 | 29.3 | 31.1 | 36.1 | 94 |
| (La/Yb)N | 0.679 | 13.7 | 21.1 | 30.9 | 38.2 | 177 |
| (Gd/Yb)N | 0.405 | 1.85 | 2.21 | 2.63 | 2.79 | 10.4 |
| Zr/Hf | 12.2 | 32.5 | 35.2 | 35.8 | 38 | 82.2 |

| Th/U | 0.021 | 4.51 | 7.25 | 9.79 | 12.7 | 46.5 |
| --- | --- | --- | --- | --- | --- | --- |

| Group | n | Median ΣREE | Median Eu/Eu* | Median Ce/Ce* | Median Y/Ho | Median (La/Yb)N |
| --- | --- | --- | --- | --- | --- | --- |
| BRS-MN | 93 | 187 | 0.663 | 1.06 | 27.5 | 36.8 |
| PIT-MN | 150 | 168 | 0.706 | 1.09 | 30.5 | 17.6 |

Median chondrite-normalized REE patterns

Text fig -29

Both groups are LREE-enriched relative to HREE. BRS-MN has a higher median (La/Yb)N than PIT-MN, indicating stronger LREE-HREE fractionation. The interquartile envelopes show substantial within-group variability, so neither sample prefix represents a single homogeneous geochemical population.

Text fig: 30

The progressive increase in normalized REE abundance with ΣREE is broadly coherent rather than restricted to one or two individual REEs. This pattern is compatible with varying proportions of REE-bearing accessory phases. Crossing or strongly irregular patterns would be more diagnostic of selective redistribution

Text fig -31

Negative Eu anomalies dominate, while most Ce anomalies cluster near unity. The Eu pattern is compatible with feldspar fractionation or alteration; the lack of widespread strong Ce anomalies argues against pervasive redox-driven Ce mobility.

.

## Y-Ho relationship

Text fig -32

Many samples lie near a coherent Y-Ho trend, but significant deviations occur. Values outside the approximate near-chondritic range can flag fluid-rock interaction, although low-level Ho censoring and mineral-specific partitioning must be considered.

Text fig -33

Total REE versus Zr

The moderate positive correlation (ρ=0.55) indicates partial zircon or zircon-associated control, but zircon alone does not explain the full REE variation.

Text fig -34

Total REE versus Hf

The moderate correlation (ρ=0.53) parallels the Zr relationship and supports a contribution from zircon or evolved Zr-Hf-bearing mineral assemblages.

Text fig -35

Total REE versus Th

The strong positive relationship (ρ=0.65) is compatible with control by Th-bearing LREE minerals, particularly monazite and/or allanite. Mineralogical confirmation is essential.

## LREE-HREE fractionation

Text fig -36

This plot separates variation in overall LREE/HREE fractionation from middle-to-heavy REE fractionation. Broad scatter may reflect multiple protoliths, accessory-mineral proportions, fractional crystallization and/or later fluid modification.

From all these plots, the most defensible interpretation is that the dataset records strong primary mineralogical and magmatic controls, with possible superimposed fluid-related fractionation in a subset of samples.

Evidence supporting primary/accessory-mineral control: coherent normalized REE patterns; strong ΣREE-Y and ΣREE-Th relationships; moderate ΣREE-Zr-Hf correlations.

Possible hydrothermal indicators: anomalous Y/Ho in a substantial subset; wide variation in Eu/Eu*; and dispersion away from simple fractionation trends.

Evidence presently lacking: demonstrable mass gain or loss relative to a known parent rock, systematic alteration zoning, mineral replacement textures, fluid-sensitive major-element indices, and direct association with F-, P- or carbonate-bearing alteration.

Thus, in conclusion it can be stated that the dataset demonstrates marked LREE enrichment, widespread negative Eu anomalies and substantial variation in Y/Ho and LREE/HREE fractionation. The dominant statistical signal links total REE with Y and Th, with secondary links to Zr-Hf-Nb, indicating strong control by accessory minerals. A hydrothermal contribution is plausible for samples with non-chondritic Y/Ho and unusual anomaly patterns, but cannot be established conclusively without fresh-parent comparisons, alteration mineralogy and mass-balance constraints.

Text fig -37

This classification is statistically derived from block dataset and is appropriate for

Exploration. defensible statistical basis for selecting the threshold.

Text fig -38

Text fig -39

Text fig -40

Text fig -41

Text fig -42

### Summary statistics

Parameter Value

Number of samples	243

Minimum TREE 8.05 ppm Maximum TREE                   1682.70 ppm

| Median TREE | 174.60 ppm |
| --- | --- |
| Mean TREE | 294.94 ppm |
| 25th percentile | 116.50 ppm |
| 75th percentile | 343.55 ppm |
| 90th percentile | 624.20 ppm |
| 95th percentile | 1066.03 ppm |

### Statistical natural cut-off

Using clustering of the log-transformed TREE values, the dataset separates naturally into:

- Background to moderately enriched population centered around 115 ppm.

- Enriched population centered around 499 ppm.

The transition between these two populations occurs at approximately:

Natural TREE cut-off ≈ 240 ppm

This value represents the statistical break between the background and enriched populations in the dataset.

### exploration classes

the following classification:

TREE (ppm)	Interpretation

<120	           Background

120–240	Weak enrichment

240–500	Moderate enrichment (anomalous)

500–1000	 High enrichment

>1000      Very high enrichment / exceptional targets

Interpretation

The result suggests that:

  - TREE values form a continuous geochemical population.

There is no statistically distinct background and anomalous population.

  - Enrichment appears to occur gradually through geological processes such as magmatic differentiation, rather than through two discrete populations.

This is common in granitoid and pegmatitic REE systems

Geological interpretation

A TREE cut-off of ~240 ppm because:

  - It lies above the dataset median (174.6 ppm).

  - It corresponds to a natural statistical break rather than an arbitrary threshold.

  - It separates the main background population from a distinctly enriched population.

This 240-ppm value is a geochemical anomaly threshold.

This are classes of anomaly of the block:

  - Geochemical background: <120 ppm TREE

  - Weakly anomalous: 120–240 ppm TREE

  - Geochemically anomalous: >240 ppm TREE

  - High-priority REE anomaly: >500 ppm TREE

  - Exceptional REE anomaly: >1000 ppm TREE

Main results

### Method            TREE threshold

Lower probability-plot breakpoint	67.7 ppm Upper probability-plot breakpoint 238.5 ppm Tukey upper outlier fence	684.1 ppm

Median TREE	174.6 ppm

Mean TREE	294.9 ppm

The most defensible natural geochemical screening cut-off is approximately 240 ppm TREE, based on the upper breakpoint in the segmented log-normal probability plot.

An important result is that the Bayesian Information Criterion favours a single-component Gaussian model (the last figure). Therefore, the samples do not form two sharply separated statistical populations. The approximately 240 ppm threshold should consequently be described as a pragmatic geochemical anomaly threshold, rather than a mathematically unique population boundary or an economic cut-off

Accordingly, I have prepared a contour map , you may build a map like this to delineate the actual interesting zones logically (not as shown by you in the given LSM actual scale map) where you can clearly demarcate the Geochemically anomalous: >240 ppm TREE and High-priority REE anomaly: >500 ppm TREE. My suggestion is that you prepare your own contour map demarcating all the zones as per TREE thresholds at GIS. Also calculate the estimated areas. The map should be given at original scale.

### Significant Anomalous Samples Sample	Anomalous Element Value

BRS-MN-91 Au	0.18 ppm

BRS-MN-59 Ag	43.5 ppm

BRS-MN-123 Sb	9.5 ppm

BRS-MN-21 Bi	1.3 ppm

The analytical data indicate:

- Overall low precious-metal background.

- Presence of isolated hydrothermal anomalies in Au, Ag, Sb, and Bi.

- Lack of a strong multi-element geochemical association suggests that mineralization is likely localized rather than widespread.

- Elevated Sb and Bi indicate possible hydrothermal fluids, while the Ag anomaly may represent a separate mineralizing event or a silver-rich vein.

The Au anomaly at BRS-MN-91 is the most prospective sample for follow-up. This represents a weak but significant gold anomaly. The correlation matrix shows that generally weak to moderate correlations, implying that mineralization may not be controlled by a single hydrothermal event.

Correlation of major oxides with precious-metal dataset   suggests only weak but distinct hydrothermal anomalies.

Text fig -43

Au anomaly is probably due to the transportation of gold Quartz- rich hydrothermal fluids

The combined datasets indicate: Quartz-rich hydrothermal fluids

Hydrothermal alteration preceded or accompanied mineralization.

High CIA values and alkali depletion indicate pervasive fluid-rock interaction.

Gold occurs in silica-rich altered zones.

The Au anomaly (0.18 ppm) is consistent with quartz-vein or silicified alteration rather than disseminated mineralization.

Sb and Bi are useful pathfinder elements.

Elevated Sb and Bi are typical of late hydrothermal systems and may mark fluid conduits.

Silver behaves independently.

The isolated Ag anomaly (~43.5 ppm) probably reflects a local Ag-bearing vein or sulphide concentration rather than the main Au event.

## CHAPTER - VIII

8.0.0.  PREVIOUS EXPLORATION

8.1.0	 General

Geological mapping on 1:50K scale was carried out in FS 1972-73 by. B.K. Ranganath, in   FS 197374 by S. Narsimha and S.K Bhattacharya.

In FS  1984-85 Geological mapping on 1:50 K scale was carried out by S.S. Nayak and Kasi Viswanathan and brought out the various geological units including the Kimberlite bodies. The entire area was rigorously searched for Kimberlites and a total of 37 bodies were identified and demarcated. Several clusters were identified and the belt was named as Narayanpet Kimberlite Belt. Out of these six Kimberlites of Maddur cluster fall in the block area.

NGCM was carried out in this area covered by toposheet 56H/09 by Parvathy. R. Krishnan and Swetha Rath in FS 2014-15 and reported TREE up to a maximum of 2572.35 ppm. Apart from the work cited above, no other exploration work has been done in the proposed block.

CHAPTER - IX

9.0.0	AERIAL OR GROUND GEOPHYSICAL OR GEOCHEMICAL SURVEY

#### 9.1.0 Aerial or Ground Geophysical Survey

Areal or ground geophysical survey was not carried out during the present         exploration.

#### 9.2.0 Geochemical Survey

Surface geochemical survey has been carried out in the block with collection of 115 no’s Bed rock samples and 150 no regolith (Pit) samples by analysed of REE elements (La, Ce, Pr, Nd, Sm, Eu, Gd, Tb, Dy, Ho, Er, Tm, Yb, Lu,) and Associated Elements (Zr, Hf, Th, U, Sr, Rb) by ICP-MS instrument. All these analyses have been carried out in LUCID Laboratory in Hyderabad. The details of chemical analyses are given in Annexure (I to III).

The element wise geochemical distribution maps manifest the Bed rock, Regolith (Pit), geochemistry to understand in the best possible way. Elemental distribution maps in the form of classed post maps and contour maps were prepared by using inverse power weightage method of Interpolation module of QGIS software. During the preparation of geochemical contours, the contour intervals were kept depending upon the minimum and maximum values in order to cover the entire range of data. In case of elemental values below the detection limit, new values have been assigned which are smaller than the detection limit to show the decreasing trend of the contours. The geochemical distribution map has been prepared for all the trace elements and ΣREE. The element wise interpretation is furnished below.

CHAPTER - X

10.0.0 EXPLORATION UNDERTAKEN DURING CURRENT     INVESTIGATION

#### Background

Maddur Block was proposed for G-4 level of exploration of REE and associated elements. The Exploration has been carried out as per Minerals (Evidence of Mineral Contents) Amendment Rules, 2021 to achieve the following objective

      - To carry out Geological & Structural mapping on 1:12500 scale for identification of REE and associated elements bearing formation (host rock) with the structural features to identify the surface manifestation and lateral disposition of the mineralized zones.

      - To collect Bedrock samples for analyses of REE and associated elements. to decide further courses of exploration program.

      - Pitting to know the concentration of REE and associated elements in the regolith within the target area.

#### 10.2.0      Exploration Methodology

The components of G-4 level of exploration for REE and associated elements. in Madddur Block to full fill the above-mentioned objective in accordance with MEMC rule 2015(amended till 2021) as approved by NMET is furnished in table no 10.1 and the field component wise description has been given in subsequent paragraphs

Table No 10.1

Summarized Table showing Component wise proposed quantum Vs. Quantum Achieved

| Sl. No. | Item of Work | Unit | Target | Achievement |
| --- | --- | --- | --- | --- |
| 1 | Geological Mapping (on 1:12,500 Scale) | Sq. km | 160 | 160 |
| 2 | Geochemical Sampling |  |  |  |
|  | A. Bedrock samples | No | 115 | 115 |
| 3 | Exploration |  |  |  |
|  | Excavation (Pitting) in 150 Pits | Cu.m | 150 | 150 |
|  | Pit samples | Nos | 150 | 150 |
| 4 | Laboratory Studies |  |  |  |
|  | 23 Elements by ICPMS of Surface Primary Samples | Nos | 275 | 275 |
|  | Elements by ICPMS of Surface Check Samples | Nos | 27 | 27 |
| 7 | Petrological Samples (Rock Samples) | Nos | 30 | 30 |
| 8 | EPMA/SEM Studies (Rock Samples) | Nos | 10 | 10 |
| 9 | XRD Mineral phase analysis | Nos | 5 | 5 |
| 10 | Report Preparation (5 Hard copies with a soft copy) | Nos. | 1 | 5 |

Geological Mapping

Large scale Geological mapping (LSM) completed in the total area of160 sq. km on 1:12500 scale in the part of Survey of India Toposheet No 56H/09. Survey of India toposheet was used to understand the topography, roads, and drainage of the mapped area and was also used as a base map for the LSM. For recording precise sample location and to carry out a geological survey, handheld GPS of GARMIN 12H has been used. The coordinates had been recorded degree, minutes, Seconds coordinate system with WGS 1984 datum. Variation of lithologies along with the structural elements were systematically recorded and prepared the final geological map as presented in Plate No- 05.

During mapping, different variants of granites belonging to the Peninsular Gneissic Complex (PGC) were demarcated. These granitic variants are characterized by the presence of enclaves of amphibolite, and These are traversed by intrusives such as dolerite, gabbro and. Acid intrusive are quartz reef, pegmatite, The detailed chapter mentions the above

#### Pitting

Pitting in the present exploration work has been carried out to collect the Regolith Samples.by JCB as per requirement and the approval of the landowner. Pits were dug with a dimension of 1 cu.m (1m*1m*1m). Hence a total of 150 cu.m of pitting has been carried out. Few photographs of the pits are the place the below and the entire list of pits locations mentioned in the Annexures.

Images showing the pit study area in Maddur Block

Photograph No: 10.2.0-1.1

Photograph No: 10.2.0-1.2

Photograph No: 10.2.0-1.4

Photograph No: 10.2.0-1.3

Chemical Analysis

All the 165 Nos of primary samples (115 Nos of Bed rock, pit (REGOLITH) subjected to analyses of REE (La, Ce, Pr, Nd, Sm, Eu, Gd, Tb, Dy, Ho, Er, Tm, Yb, Lu) and associated elements (Zr, Hf, Th, U, Sr, Rb) by ICP- MS instrument. The samples from quartz were analysed for As, Au, Sb, Bi by fire assay method The results of these samples have been discussed with the description in the block geology under sub head geochemistry.

Moreover, the external check samples of 27 Nos have been analysed for same elements from SHIVA Laboratory, Bengaluru.

Further, 12 no of selected granitic samples were subjected to analyses of major oxides, i.e., SiO₂, Al₂O₃, Fe₂O₃, TiO₂, MnO, CaO, MgO, Na₂O, K₂O, P₂O₅, V₂O₅, Loss on Ignition (LOI) by XRF instrument. The results of these samples have been discussed with the description in the block geology under sub head geochemistry.

Physical Analysis

30 Nos of samples have been studied under microscope for complete petrographic Study. Ten samples have been subjected to SEM-EDX and Five The study report has been discussed under the description of lithology in chapter-VII.

Preparation of Geological Report:

Geological Report has been prepared in Head Office, CMT, Hyderabad by integration of geological and geochemical data. QGIS software has been used for preparation of various maps and GCD kit module of R programming language has been used for generating geochemical plots. The report has been written using Microsoft word and excel 2019 versions

CHAPTER - XI

#### 11.0.0 Location of Data Point

11.1.0 Accuracy and Quality of Survey

The current exploration for REE and Associated minerals was conducted at the G-4 stage. To accurately record sample locations and perform geological surveys, a handheld GPS device from GARMIN – GPS 12H was utilized. The coordinates were recorded using the degree, minutes, seconds (WGS 1984 datum).

#### 11.2.0 Quality and adequacy of topographic control

The topographic survey was out of the scope of present G-4 stage exploration.

CHAPTER - XII

#### 12.0.0 Sampling

Sampling is a systematic process for collection of material from any media which can provide the best representation. In the present study in the Maddur Block, sampling includes bed rock sampling (BRS), and pit sample (PT).

A total of 115 Nos. of BRS, 150 NoS (PT) Pit samples and 30 no thin section has been collected.

#### Sampling techniques:

- Bedrock Sampling:

A total of 115 Nos. of bed rock samples (BRS) were collected systematically from different lithounits exposed in the study area. The samples were collected by chipping exposed rock by sledgehammer (5kg) and chisel. The sample locations are plotted on geological map. First, each BRS sample of 2-3 kgs collected from the field was crushed. After crushing, the crushed samples were mixed thoroughly and reduced the sample size to 500 gm by conning and quartering. These samples were powdered and completely passed through (-) 200 mesh size sieves from which 100 g sample packed in polythene sample pouch and sent for chemical analysis for REE and Associated elements by ICPMS from LUCID Laboratory, Hyderabad and 12 selected samples of granitic rock were also analysed for major oxides by XRF. The remaining 400 g sample has been preserved for future reference.

#### Pit Regolith sampling:

      - Methodology for Collection of pit regolith samples: pit sample comprises of material collected from within the block area About 2 Kg samples was collected was dried in natural sunlight and They were marked systematically with suffix PIT- MN. These samples were preserved in plastic bags, and one set of samples were transported to LUCID Laboratory for analysis, Hyderabad and another set preserved in the CMT Headquarters.

      - Processing of Collected Samples: The 2 kg samples were passed through available sieve to get uniform natural fractions of the samples. Then these portions were pounded and passed through (-) 200 mesh sized sieve from which 500 g sample has been collected by coning and quartering. From the 500 g sample, 100 g sample packed in polythene sample pouch and sent for chemical analysis for 34 elements by ICPMS from LUCID Laboratory for analysis, Hyderabad and another set were preserved in the CMT Headquarters.

#### 12.2.0	Nature, quality and appropriateness of sample preparation technique

Sampling methodology adopted for bedrock, Pit sample is different from each other. The detailed description of sampling methods for each type of sample has been discussed in subsequent paragraphs.

#### 12.3.0	Quality control procedures adopted during sampling

The samples have been prepared under the supervision of a geologist and a qualified sampling technician. All the stream-sediment samples collected from the field were dried under natural sunlight, packed properly in the polythene sample bag with proper tag. Then the entire samples were transported to central sample processing unit located in Hyderabad.

#### 12.4.0	Measures taken to ensure that the sampling is representative

It is very important to submit the representative sample of collected material for geochemical analyses. As discussed in the previously, initial sieving has been carried out on the regolith to collect the natural fraction. Further after pounding and passing samples of regolith and through (-)200 mesh sieve and crushing, grinding and passing 200 mesh sieves for Bed rock samples, quantity of the samples has been reduced to 500 gm by coning and quartering method after thoroughly mixing to maintain the homogeneity of the samples.

#### 12.5.0	Appropriateness of grain size

In accordance with the standard sampling procedures, it has also been observed that smaller the particle size, higher the homogeneity of the sample as well as higher the dissolvability during the chemical analysis. As per the standard practice, samples are generally pounded to (-)200 mesh size for analysis of REE and Associated elements through ICPMS.

CHAPTER - XIII

13.0.0	DRILLING TECHNIQUE AND DRILL SAMPLING EMPLOYED

Drilling work was not in the scope of the present level of exploration work.

CHAPTER - XIV

14.0.0	SUB-SAMPLINGTECHNIQUES AND SAMPLE PREPARATION

As drilling was not in the scope of present exploration work, hence drill core samples have not been generated in the present project. The sampling technique of Pit sample, thin section and bedrock has been discussed in Chapter XII.

CHAPTER - XV

15.0.0	QUALITY OF ASSAY DATA AND LABORATORY TEST

#### 15.1.0	The nature, quality and appropriateness of Chemical analysis

A total 165 Nos of surface samples which includes 30 Nos of Thin sections, 115 Nos of Bed rock samples and 150 Nos of pit samples have been analysed for La, Ce, Pr, Nd, Sm, Eu, Sc, Gd, Tb, Dy, Ho, Er, Tm, Yb, Lu, Y, Th, U, Sr, Hf, Zr, Rb and REE and Associated elements by ICP-MS instrument and by WD-XRF instrument.12 no of samples were analysed for major oxides, i.e. SiO2, Al2O3, Fe2O3, TiO2, MnO, CaO, MgO, Na2O, K2O, P2O5, V2O5, Loss on Ignition (LOI).

#### Methodology for analysing REE and Associated elements:

ICP-MS instrument model (Agilent 7850) and the sensitivity is measured by external solutions (Tuning solution provided by Agilent), with surrogate calibration for 23 elements (La, Ce, Pr, Nd, Sm, Eu, Sc, Gd, Tb, Dy, Ho, Er, Tm, Yb, Lu, Y, Th, U, Sr, Hf, Zr, Rb). Matrix correction is made by using standard CRM (Certified reference material).

#### Methodology of Chemical Analyses by XRD/XRF

Chemical Laboratory, SHIVA, Bengaluru make 1.60 KW POWDER X RAY DIFRACTOMETER METHOD (XRD/XRF) instrument for analyses of major oxides.

First pellets of the powdered samples are made using hydraulic press. Then the XRD/XRF instrument is calibrated using suitable matrix matching CRMs. After calibration of the instrument the samples are analysed and the values of,

i.e., SiO2, Al2O3, Fe2O3, TiO2, MnO, CaO, MgO, Na2O, K2O, P2O5, V2O5,

Loss on Ignition (LOI) Are obtained from the software.

#### 15.2.0	Quality control procedure

The standard procedure of quality control has been adopted during the chemical analysis in NABL accredited Chemical laboratory, LUCID, Hyderabad

### 15.3.0.	External Check Analysis

In accordance with the standard practice of quality assurance and quality control, 27 Check samples were analysed at SHIVA Analytical in Bengaluru, Karnataka as external check samples. The analytical results of external check samples have been compared with the corresponding analysis of primary samples and have been presented as Annexure No II- III

The statistical analysis of primary and external check analysis data has been carried out on REE and Associated elements, i.e., La, Ce, Pr, Nd, Sm, Eu, Sc, Gd, Tb, Dy, Ho, Er, Tm, Yb, Lu, Y, Th, U, Sr, Hf, Zr, Rb by ICP-MS and furnished in Annexure No (II to IV).

The analytical data of check samples have been compared with the data of primary samples and it was found that most of the samples are strongly correlatable. However, couple of bedrock samples and pit samples showed some variation.

Further the Scatter plot of the principal Vs external check assay (Text Fig 27- 43) in respect of above-mentioned elements shows scatter of values are closely following the line slightly below the bisector. The primary and check data set are close to each other. Thus, it can be inferred that the chemical analysis of the samples shows the repeatability in inter laboratory setting and hence the reliability of the analysis carried out is also established. Further it also proves the homogeneity of the samples prepared.

#### 15.4.0 Security and chain of control of samples

The samples have been prepared at centralized mechanized sampling unit with proper labeling and tag and sent to chemical laboratory in supervision of qualified sampling technician.

The samples have been transported from field to sample processing unit with proper sampling of sampling bags and the same has been verified in the sampling unit before opening the bags.

At the sampling unit, standard procedure has been followed, and all the precautionary measures have been taken to avoid the contamination. Further, the remaining samples are properly preserved with sample tags for any future reference under the custody of the company.

Annexure - I

Major oxides analytical Results

Summarized table showing statistical analyses of BED ROCK (BRS) samples of REE and

Associated elements

Annexure - II

Annexure -III

Summarized table showing statistical analyses of regolith (Pit) samples of REE and

Associated elements

Summarised table showing statistical analyses of primary Vs. external check samples of REE Elements

Annexure -IV

Text fig - 44

Text fig - 45

Text fig - 46

Text fig - 47

Text fig - 48

Text fig - 49

Text fig - 50

Text fig - 51

Text fig - 52

Text fig - 53

Text fig - 54

Text fig - 55

Text fig - 56

Text fig - 57

4

Text fig - 58

Text fig - 59

CHAPTER - XVI

16.0.0  MOISTURE

16.1.0  Method of determination of the moisture content.

All the analysis has been carried out with natural moisture. Moisture content has not been determined.

CHAPTER - XVII

17.0.0	 BULK DENSITY

17.1.0 	 Method of determination and result

The present exploration has been carried out at G-4 level, i.e., reconnaissance survey with an objective to identify prospective areas of further mineral exploration. Hence, only surface sampling has been carried out. Drilling of scout boreholes was not approved by the TCC therefore it was not carried out. As a result, bulk density or specific gravity has not been determined.

CHAPTER – XVIII

18.0.0  BENEFICIATION STUDIES AS MAY BE REQUIRED

#### 18.1.0	 Details of beneficiation studies

Beneficiation studies have not been carried out in the present exploration as it was out of the scope of the approved quantum of work.

CHAPTER – XIX

19.0.0   RESOURCE ESTIMATION TECHNIQUES

The test drilling was not approved by the TCC therefore resource estimation was not done. However, Lakkayapalli, Pallerla zones for REE and Nandipad zone for As, Au, Sb, Bi can be explored further as they have the potential. The details of the zones and their potential have been discussed in mineralization (Chapter VII).

CHAPTER – XX

20.0.0	 REPORTING RESOURCE

As mentioned in the Chapter – XIX, resource has not been estimated i  the present stage of exploration

CHAPTER - XXI

21.0.0	 SUMMARY AND RECOMMENDATIONS

21.1.0	 Summary

Geological mapping on 1:12,500 scale was initiated in the 1st week of October 2025 in Maddur block area and by the end of April 2026. AND the assigned target 160 sq km and completed. Collection of 115 bed Nos bed rock samples 150 pit samples were collected and submitted to chemical lab for analysis.

The analytical data of 115 bed rock samples and 150 pit samples received end of the may. The interpreted litho contacts have been drawn and the analytical data of bed rock samples and pit samples received has plotted on the map, the lithological contacts and structural data has been digitized.

The Maddur block area is located in Narayanpet district of Telangana and geologically part of the Narayanpet Kimberlite belt within the Eastern Dharwar Craton. The block area is 160 sq.kms and is part of the toposheet 56 H/09.

The block area is composed of grey biotite granite occupying the central part of the block which is mostly a Pediment- Pedi plain complex with ground level outcrops whereas the northern part and the southern part of the block is occupied by fine grained pink granite which is more aplitic in nature with almost negligible mafics and forms hills extending in NW-SE direction. In the northern part of the block a major shear/fault zone passes through the block in a NW-SE direction which is occupied by the Quartz reef. One sample of this collected prior to submission of proposal analysed 82 ppm As, 7.5ppm Sb and 0.29 ppm Au and the granitic variants gave encouraging values for REE.

The whole rock analysis was done for 12 samples of the granitic variants and the data was plotted on the variation diagrams which revealed that they are Granodiorite to quartzolite suits of rocks, Silica saturated, ferro magnesian and mantle fractionates, calk alkaline to tholeiitic, in nature.

When plotted the analytical data in QAPF diagrams they plotted predominantly in quartzolite and Granodiorite fields The feldspar triangle of O Connor (1965) indicated them to be Granodiorite to tonalite in composition. The R1—R2 plot of De La Roche et al (1980) suggested these to be predominantly granodiorite and couple of them granite composition. Which are highly alkali rich. The data plotted on Middlemost (1994) diagram reflected them to be predominately quartzolite except one plot fell granite field.

TAS Diagram of Cox et al. (1979 indicated them to be highly acidic and predominately granite in composition except one which plotted in the marginal zone of granite – granodiorite. Taking into consideration all above plots these granitic variants can be geochemically classified as Granite -granodiorite – quartzolite suits, (GGQ)

The sio2 -k20 plot of peccerillo and Taylor 1976 indicates that these granitic variants are calk alkaline to tholeiitic in nature.

As per the tectonic discrimination diagram of Frost – frost diagram (2008) these are ferromagnesian, calcic, strongly peraluminous, and silica saturation in nature,

As per the tectonic discrimination diagram of Major and piccolo (1989) these granitic variants fall in the borderline [ of IAG-CAG-CCD and RRG- CEUG. According to the R1-R2 plot of Batchelor and bowden (1985) These are mantle fractionates   in nature.

As per the tectonic discrimination diagram of Maniar and Piccolli (1989) these granitic variants fall in the IAG-CAG-CCG field whereas they are more peraluminous and marginally metaluminous, ferro-magnesian, Calcic and silica saturated.

as per Frost and Frost (2008) tectonic discrimination diagram. According to the R₁—R₂ plot of Batchelor (1985), these are mantle fractionates.

The analytical data was interpreted and the mineralized zones have been demarcated and details of each are follows six mineralized zones have been delineated within the block area, namely Lakkayapalli Zone, Pallerla Zone, Nandipad zone -A, Nandipad zone – B, Pedripad, Jadavaraopalli, The REE values recorded in Pallerla Zone-A range from 150 ppm to 1,682 (ppm). Lakkayapalli zone – 0.845Sq.km – TREE- 359.1-1600 (ppm)Pallerla zone - 0.837Sq.km – TREE – 462 -1682 ppm

Nandipad zone – A (2.8 – TREE – 141 – 1182.8ppm)

Nandipad zone – B (Precious metals)

As – 1.1 to 82.2ppm, Au – 0.02 – to 1.4 ppm

Sb -1.3 – to 9.4 ppm

Bi - <0.5 to 1.3 ppm

Ag- 0.4to 43.5

Pedripad zone - 0.233 Sq.km – TREE - 89.7 -1529ppm

Jadavaraopalli Zone – 1.05 Sq. km – TREE196.1 to 1426 ppm

Petrographic studies thin section 30 thin sections of different granitic variants pertaining to the block area was carried out by GSI, SR petrological division which rewied presence of allanite apatite biotite alkali feldspars which host rare earth elements. Beside these zircon was also identified which host zirconium and hafnium, Titanite a titanium bearing mineral was also identified and reported. Ten (10) thin sections were selected submitted to lucid laboratories for SEM studies for mineral characterizes the results show presence of allanite, biotite  apatite alkali feldspars, zircon, and titanite, Which corroboted with analytical data and petrographic studies where high values of TREE 1682 ppm which includes values are Ce, La, Nd, Pr, Gd, Sm, Dy, The Project work was reviewed by the TCC-I committee on 26th   May,2026 and during the review the entire data pertaining to the geological setup of the block, the analytical data of bed rock and pit samples, petrographic study results photomicrographs the mineral characterization corroborated by the SEM study were presented The members after detailed discussion did not approve test drilling and asked us to submit the Geological Report before the completion of the timeline

### 21.2.0   Recommendations:

Five zones of REE Mineralization and one zone of precious minerals such as arsenic, antimony, gold, silver, bismuth have been identified and demarcated out of which the Pallerla zone and Lakkaypalli zone are more potential ones within the block which have given the TREE values above 1600 ppm with the maximum of value 1682. Among the REE besides Lanthanum, Cerium, Neodymium, certain uncommon elements like, Praseodymium, Samarium, Gadolinium, have analyzed higher values. Pallerla zone has the TREE   maximum value is 1682 ppm   and in Lakkaypalli zone the maximum value is more than 1600 ppm--n aerial extent.

Since these are values recorded in an younger intrusive alkali feldspar granite, seen bordering the major shear zone and as per the Bachelor and Bowden 1985 plot they are considered as mantle fractionates which is also substantiated by the petrographic studies.

Therefore, it is strongly opined that one borehole in Pallerla zone and one bore hole in Lakkayapalli zone may be drilled to ascertain the depth persistence of the mineralization.

Besides these along the quartz reef which has emplaced within the major shear zone / fault zone trending NW-SE   direction is observed to be highly sheared. The bed rock samples of these quartz analyzed gave for arsenic, Au, As, Ag, Bi, Sb, the maximum value of arsenic as 82.2ppm, the maximum value of gold (Au)– 1.4ppm, the maximum value (Bi) -1.3ppm, and maximum value of antimony (Sb) 19.5ppm, and the maximum value of silver (Ag) 43.5ppm.

| S. NO | villages | Latitude | Longitude |
| --- | --- | --- | --- |
| 1 | Lakkayapalli | 16.929596 | 77.620625 |
| 2 | Nandipad | 16.934755 | 77.638315 |
| 3 | Pallerla | 16.912098 | 77.617186 |
| 4 | jadavarao palli | 16.897487 | 77.640232 |
| 5 | Hanmanayak Tanda | 16.895931 | 77.651504 |
| 6 | Chintaldinne | 16.885594 | 77.619646 |
| 7 | Dorepalli | 16.873192 | 77.650299 |
| 8 | Dorepalli Tanda | 16.865911 | 77.642717 |
| 9 | Nagireddipalli | 16.886523 | 77.596876 |
| 10 | Maddur | 16.862924 | 77.614449 |
| 11 | Bhimpur | 16.853786 | 77.598332 |
| 12 | Erragutta Tanda | 16.845828 | 77.624177 |
| 13 | Rallabai | 16.827426 | 77.623299 |
| 14 | Renuvatla | 16.825944 | 77.608211 |
| 15 | Achchampalli | 16.817007 | 77.629537 |
| 16 | Appareddipalli | 16.821375 | 77.631621 |
| 17 | Chennaram | 16.811371 | 77.650236 |
| 18 | Bandakonda | 16.796308 | 77.64514 |
| 19 | Basannapalli | 16.791789 | 77.631031 |
| 20 | Abhangapur | 16.771831 | 77.633592 |
| 21 | Nemillapalli | 16.768868 | 77.616395 |
| 21 | Kotakonda | 16.757797 | 77.645366 |
| 22 | Narsampalli | 16.755236 | 77.659475 |
| 23 | Monapuram | 16.804748 | 77.614211 |

## LOCATION INDEX

#### 21.3.0                                                   Table No: 21.1

CHAPTER - XXII

22.0.0	PLATES AND MAPS

#### 22.1.0	List of Plates

| Plate No | Title | Scale |
| --- | --- | --- |
| 1. | Land use land cover Map of Madur block, Madur block | Not To Scale |
| 2. | Toposheet of Madur block in Madur block District of Telangana. | 1:12500 |
| 3. | Outcrop Map of Madur block Madur block District. | 1:12500 |
| 4. | Regional Geological Map of Madur block Madur block District. | 1:50000 |
| 5. | Geological Map of Madur block Madur block Districts Telangana | 1:12500 |
| 6. | Geological Map of Madur block and Madur block Districts Telangana. (Delineated Zones) | 1:12500 |
| 7. | Map showing, Delineated Zones – Lakkayapalli (TREE) | - |
| 8. | Map showing distribution of Lanthanum (La) Lakkayapalli - Zone) | - |
| 9. | Map showing distribution of Cerium (Ce) Lakkayapalli -Zone) | - |
| 10. | Map showing distribution of Praseodymium (Pr) Lakkayapalli - Zone) | - |
| 11. | Map showing distribution of Neodymium (Nd) Lakkayapalli - Zone) |  |

| 12. | Map showing distribution of Samarium (Sm) Lakkayapalli - Zone) | - |
| --- | --- | --- |
| 13. | Map showing distribution of Europium (Eu) Lakkayapalli - Zone) | - |
| 14. | Map showing distribution of Gadolinium (Gd) Lakkayapalli - Zone) | - |
| 15. | Map showing, Delineated Zones – Pallerla Zone (TREE) | - |
| 16. | Map showing distribution of lanthanum (La) Pallerla Zone | - |
| 17. | Map showing distribution of Cerium (Ce) Pallerla Zone | - |
| 18. | Map showing distribution of Praseodymium (Pr) Pallerla Zone | - |
| 19. | Map showing distribution of Neodymium Nd) Pallerla Zone | - |
| 20. | Map showing distribution of Samarium (Sm) Pallerla Zone | - |
| 21. | Map showing distribution of Gadolinium (Gd) (Pallerla Zone | - |
| 22. | Map showing, Delineated Zones, (Nandipad Zone- A), (TREE) | - |
| 23. | Map showing, Delineated Zones, (Nandipad Zone- B) (TREE) | - |
| 24. | Map showing, Delineated Zones, (Pedripad), (TREE) | - |
| 25. | Map showing, Delineated Zones, (Jadavarao palli), (TREE) | - |
| 26. |  | - |

CHAPTER - XXIII

ANNEXURES OR ENCLOSURES TO THE REPORT

#### 23.1.0	List of Annexure

| Annexure No | Title | Page No |
| --- | --- | --- |
| I | Major oxides analytical Results | 115 |
| II | Statement showing details of 100 no of Bed rock chip samples with analysis of REE and Associated elements Maddur block, in Narayanpet District. | 116- 126 |
| III | Statement showing details of 150 Nos Pit samples with analysis of REE and Associated elements Maddur block, District. | 127 - 137 |
| V | Summarized table showing statistical analyses of primary Vs. external Check samples of REE Elements | 138- 143 |

CHAPTER - XXIV

24.0.0	        ANY OTHER INFORMATION

#### Any other information may be available or required by any authority as prescribed.

    - References

GSI’s online data portal – Bhukosh (for NGCM raw data source, Block Geology and Regional Geology)

Geological quadrangle map 1:50K scale  FS 1972-73 by. B.K. Ranganath, in   FS 197374 by S. Narsimha and S.K Bhattacharya.

systematic geological mapping in parts of mahabubnagar district, andhra pradesh (unpublished geological report of GSI for the F, S 1984– 85)

by S. S. Nayak, geologist (jr) and C. V. Kasiviswanathan, Asstt. geologist

NGCM report for the toposheet 56H/09 by Parvathy. R. Krishnan and Swetha Rath in FS 2014-15 and reported TREE

District Resources Map of Undivided Mahabubnagar (Published in 2000, 56H/9, BY k. Mahender Reddy Geologist and A. Bhopal Reddy Geologist

principles and applications of photogeology by Pandey S.N 1987.

CHAPTER- XXV

25.0.0    CERTIFICATION

This is to certify that geological report has been prepared in respect of Reconnaissance Survey (G-4) for REE and Associated minerals in Maddur Block, Narayanpet, District, Telangana State by CRITICAL MINERAL TRACKERS on behalf of National Mineral Exploration and Development Trust (NMEDT). The report has been prepared in accordance with the Minerals (Evidence of Mineral Contents) Rule 2015 specified under Mineral Auction Rule, 2015 and amended up to 2021.

NAME: Dr. K. MAHENDER REDDY

DESIGNATION: Technical Area Expert