Reconnaissance Survey (G4) for Glauconite and associated mineralization In Dhamni-Piparwasi Simliya area, Sheopur district, Madhya Pradesh

Litholog DPS/BH/02

| Agency | Maheshwari Mining Pvt Ltd |
| --- | --- |
| Date of Commence | 09-05-2025 |
| Date of Closing | 13-05-2025 |
| Easting | 733524.788 |
| Northing | 2866738.401 |
| RL | 282.1039 |
| Lattitude | 25:54:01.72442 |
| Longitude | 77:19:51.86654 |
| BH NO | DPS/BH-02 |
| Azimuth | Vertical |
| Inclination | Vertical |

| Lithology From_Depth (m) | Lithology To_Depth (m) | Lithology contact angle | Lithology True Width (m) | Lithology Name | Lithology Description | Structures | Stratigraphic Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 9.5 | NA | 9.5 | Loose Material | Green to Brown color soil, mainly composed of clay and silt particles loose soil material also have broken pieces of sandstone of 1 to 2 inches and cores of boulders of up to inches. |  | Govindgarh Formation Rewa Group Vindhyan Supergroup |
| 9.5 | 12.5 | 90⁰ | 3 | Sandstone with intercalated Shale | White sandstone interbedded with weathered shale. The sandstone is fine- to medium-grained, quartz-dominated, occurring in thick, massive beds exceeding 10 inches. Grains are sub-rounded to rounded and moderately sorted. The sandstone is very compact, with a Mohs hardness >7, indicating high induration. Intercalated shale units are greenish-grey, weathered, and range from 2 to 3 inches in thickness. Overall, the unit exhibits alternating hard, resistant sandstone beds with softer, weathered shale layers | Thick bedding and Laminations |  |
| 12.5 | 18 | 90⁰ | 5.5 | Shale | Black, fresh, evenly laminated shale with partings ranging from <1 inch to 8 inches. Occasional discontinuous light green laminations are present, likely associated with clay minerals. The shale is calcareous, exhibiting mild effervescence with dilute HCl, corroborated by geochemical analysis indicating 10–19% CaO. Secondary mineral fillings are common within partings, predominantly concordant to lamination planes, though some occur at oblique angles. | Lamminations |  |
| 18 | 20.5 | 90⁰ | 2.5 | Sandstone | White sandstone, fine-grained, with sub-rounded grains that are poorly to moderately sorted. The unit is quartz-dominated. Sandstone exhibits a hardness >7 on the Mohs scale, indicating high induration. The rock mass shows frequent fractures, some of which are open or partially healed with secondary mineral infillings. The sandstone is compact, with minimal matrix and low porosity, suggesting a mature sedimentary provenance and diagenetic overprint. | bedding |  |
| 20.5 | 22.5 | 90⁰ | 2 | Shale | Black color, evenly laminated, fresh shale with partings at less than one to eight inches, secondary fillings are common in partings, mostly concordant but some are at angle with lamination plane. | laminations |  |
| 22.5 | 27 | 90⁰ | 4.5 | Sandstone | Light grey sandstone, fine-grained, compact, and well-indurated with a Mohs hardness >7. Grains are sub-rounded and poorly to moderately sorted. The rock mass exhibits numerous random fractures, many of which are filled with secondary minerals. Some fractures also contain visible sulfide mineralization, indicating late-stage hydrothermal activity or diagenetic processes. Overall, the sandstone shows low porosity and a tightly packed grain framework | Bedding |  |
| 27 | 30 | 90⁰ | 3 | Shale intercalated with sandstone | Dark grey shale with laminations is intercalated with grey color sandstone, sandstone is fine grained. | Laminations and bedding |  |