DPS/BH02 Petrography report

| Sample id | Depth From(m) | Depth To(m) | Borehole no. |
| --- | --- | --- | --- |
| DPS-BH2-PG4 | 7.2m | 7.35m | BH2 |
| DPS-BH2-PG5 | 23.40m | 23.50m | BH2 |

Sample: DPS-BH2-PG4

Rock Type: Glauconite bearing quartz arenite

The thin section comprises a quartz arenite with tightly packed medium- to fine-grained quartz grains. Quartz shows sub-rounded boundaries and displays uniform extinction under XPL. One to two glauconite grains are visible, appearing as isolated rounded green grains under PPL. The framework is grain-supported with silica cement, and no significant feldspar or lithic grains are present. The texture indicates high compositional and textural maturity, consistent with reworking and prolonged transport.

Fig. 9. Microphotographs of thin section DPS-BH2-PG4 show medium- to fine-grained quartz grains with sub-rounded grain boundaries and isolated glauconite grains under PPL in (a) and XPL in (b).

Sample: DPS-BH2-PG5

Rock Type: Glauconite bearing Quartz arenite

This thin section is a quartz arenite composed of medium- to fine-grained quartz grains exhibiting sub-rounded boundaries and point contacts. One to two glauconite grains are present and show characteristic green color and rounded shape under PPL. Muscovite or other micaceous minerals are occasionally observed intergrown with the quartz framework. Minor feldspar grains and small lithic fragments are also present, reflecting a slightly less mature composition than PG4. The rock is silica-cemented with minimal matrix.

Fig. 10. Microphotographs of thin section DPS-BH2-PG5 show (a, c) quartz grains with minor glauconite, feldspar, and micaceous minerals under PPL, (b, d) quartz with sub-rounded contacts and occasional lithic fragments under XPL, indicating a mature framework with slightly more compositional variation.