GEOLOGICAL REPORT

RECONNAISSANCE SURVEY (G4) FOR Ni, Co AND PGE

IN BOTAD BLOCK (99.6 Sq km area) - BOTAD DISTRICT

GUJARAT (UNDER NMEDT PROGRAMME)

Critical Mineral Trackers

(NOTIFIED PRIVATE EXPLORATION AGENCY)

March 2026

CERTIFICATION

This is to certify that geological report has been prepared in respect of “Reconnaissance Survey (G-4) for Ni, Co and PGE in Botad Block, Botad District and Gujarat” by CRITICAL MINERAL TRACKERS on behalf of National Mineral Exploration and Development Trust (NMEDT). The report has been prepared in accordance with the Minerals (Evidence of Mineral Contents) Rule 2015 specified under Mineral Auction Rule, 2015 and amended up to 2021.

| MEKALA CHANDU Team Member (TM) | S. RAMA MURTHY Technical Area Expert (TAE) & Principal Investigator |
| --- | --- |

Reconnaissance Survey (G4) for Ni, Co and PGE in Botad area,  Botad District,

Gujarat

Project Code:    23/607/2025-NMET/1009

Under NMDET

By

S. Rama Murthy

M. Chandu

| S.no | CONTENTS | Page No |
| --- | --- | --- |
|  | CHAPTER-1 | 1-5 |
| 1 | SUMMARY (HINDI & ENGLISH) | 6 |
| 2 | CHAPTER-2 | 6 |
|  | INTRODUCTION | 11 |
| 2.1 | Details of the project | 11 |
| 2.2 | Investigating agency | 12 |
| 2.3 | Objectives of investigation | 12 |
| 2.4 | Basis for taking up investigation | 12 |
| 2.5 | Nature and quantum of work Approved vs Achievement | 13-15 |
| 2.6 | Personnel involved | 16 |
| 2.7 | Mode of operation of different work components and associated agency | 16 |
| 2.8 | Acknowledgement | 16 |
|  | CHAPTER-3 | 12 |
| 3.1 | Property description | 17 |
| 3.2 | Location and Accessibility | 18 |
| 3.3 | Climate | 18 |
| 3.4 | Flora / fauna | 18 |
| 3.5 | Land use / Land cover | 19 |
|  | CHAPTER-4 | 16 |
| 4 | Previous work | 16 |
| 5 | CHAPTER-5 | 18 |
|  | GEOLOGY OF THE AREA | 18 |
| 5.1 | Regional Geology | 18 |
| 5.2 | Block Geology | 21 |
|  | CHAPTER-6 | 22 |
| 6 | Activity During the period (Geo science Investigation) | 22 |
| 6.1 | Large scale mapping | 22 |
| 6.1.1 | Delineation of Basaltic Flows | 27 |
| 6.1.2 | Basaltic Flows | 27 |
| 6.1.3 | Geological cross section of Botad block | 33 |
| 6.1.4 | Outcrop Map of Botad Block | 45 |
| 6.1.5 | Description of rock types | 46 |
| 6.1.6 | Petrological, petrochemical studies (SEM-EDX, EPMA), whole rock and trace element analysis | 61 |
| 6.1.7 | Structure | 163 |
| 6.1.8 | Pitting | 164 |
| 6.1.9 | Sampling | 266 |
| 6.2 | Geo chemical Exploration | 269 |
| 7 | CHAPTER-7 | 274 |
| 7.1 | Integration of Geology, geophysics (with available aero geophysical data) and geochemical exploration data and the interpretation | 278 |
| 8 | CHAPTER-8 | 280 |
|  | MINERAL PROSPECT |  |
| 9 | CHAPTER-9 | 280 |
|  | EXPLORATION BY SCOUT DRILLING | 280 |
| 10 | CHAPTER-10 | 281 |
|  | RESOURCE ESTIMATION | 281 |
| 11 | CHAPTER-11 | 282 |
|  | CONCLUSION AND RECOMMENDATION | 282 |
| 12 | CHAPTER-12 | 284 |
|  | EXPENDITURE | 284 |
| 13 | CHAPTER-13 | 289 |
|  | REFERENCES | 289 |
| 14 | CHAPTER-14 | 290 |
|  | LOCALITY INDEX | 290 |

List of Photographs

| Photographs | Description | Pg no |
| --- | --- | --- |
| Photograph 1 | Picritic basalt with Vesicular nature near RTO office road, Botad | 23 |
| Photograph 2 | Red bole bed in massive olivine basalt | 25 |
| Photograph 3 | Amygdaloidal basalt Exposure near Khas Road | 25 |
| Photograph 4 | Exposure of Olivine altered to Iddingsite in Khas Road | 25 |
| Photograph 5 | Olivine crystals in green and yellow (Fresh to partially altered) near Khas Road | 25 |
| Photograph 6 | Exposure of Red bole bed (Act as Marker horizon) | 26 |
| Photograph 7 | picritic basaltic sample near the village senthali. | 28 |
| Photograph 8 | picritic basaltic sample near the village senthali. | 29 |
| Photograph 9 | olivine basalt showing porphyritic texture. | 29 |
| Photograph 10 | Olivine basalt in green color. | 29 |
| Photograph 11 | Fine-grained olivine basalt in Sherthali village | 30 |
| Photograph 12 | Fine-grained olivine basalt in Khas village | 36 |
| Photograph 13 | Flow 3 and flow 4 and in between there is a red bole bed | 40 |
| Photograph 14 | Zeolite | 40 |
| Photograph 15 | Red bole bed at ARTO Botad | 42 |
| Photograph 16 | Picritic Quarry area along sherthali village | 42 |
| Photograph 17 | Quarrie area showing picritic basalt. | 46 |
| Photograph 18 | Picrite sample along khas road | 46 |
| Photograph 19 | Olivine basalt along Tajpar near sub station | 47 |
| Photograph 20 | Olivine basalt along Tajpar near sub station | 47 |
| Photograph 21 | Fine grained olivine basalt along khas road | 51 |
| Photograph 22 | Fine grained olivine basalt along khas road | 51 |
| Photograph 23 | Amygdaloidal basalt along khas canal road | 52 |
| Photograph 24 | Massive basalt along khas road | 54 |
| Photograph 25 | Massive basalt along Bhambhan road | 54 |
| Photograph 26 | Red bole bed along Bhambhan road | 54 |
| Photograph 27 | Red bole bed is located near Rto road office, botad having the Elevation of 86m. | 54 |
| Photograph 28 | serpentine along Bhambhan road | 55 |
| Photograph 29 | serpentine along Bhambhan road | 55 |
| Photograph 30 | Massive basaltic dyke along Tajpar road | 57 |
| Photograph 31 | Quarry 1 area showing picritic basalt. | 59 |
| Photograph 32 | Quarry 2 area showing picritic basalt. | 59 |
| Photograph 33 | Quarry 3 area showing picritic basalt. | 59 |
| Photograph 34 | Quarry 4 area showing picritic basalt. | 60 |
| Photograph 35 | Quarry 5 area showing picritic basalt. | 60 |
| Photograph 36 | Quarry 6 area showing picritic basalt. | 60 |
| Photograph 37 | worksheet of pit details | 165 |
| Photograph 38A | Pitting photographs showing pit walls | 166 |
| Photograph 38B | Pitting photographs showing pit walls | 166 |
| Photograph 38C | Pitting photographs showing pit walls | 166 |
| Photograph 38D | Pitting photographs showing pit walls | 166 |

List Of Photomicrographs

| Photomicrographs | Description | Pg no |
| --- | --- | --- |
| Photomicrograph 1 | Large phenocrysts of olivine and clinopyroxene in very fine basaltic matric showing porphyritic texture; under plane polarized light; 2X magnification | 49 |
| Photomicrograph 2 | Large phenocrysts of olivine and clinopyroxene in very fine basaltic matric showing porphyritic texture; under cross polarized light; 2X | 49 |
| Photomicrograph 3 | Clusters of phenocrysts within basaltic matrix showing Glomeroporphyritic texture; under cross polarized light at 2X magnification | 49 |
| Photomicrograph 4 | A closer view of Clusters of phenocrysts of olivine and Cpx within basaltic matrix showing Glomeroporphyritic texture; under cross polarized light at 5X magnification. | 49 |
| Photomicrograph 5 | Bimodal distribution of phenocrysts size; very sharp-edged phenocrysts of Olivine in a fine-grained basaltic matrix | 50 |
| Photomicrograph 6 | Olivine in plagioclase-pyroxene rich matrix; showing Fractured small magnetites and rutiles are present in the matrix; under 10X magnification | 50 |
| Photomicrograph 7 | Intergranular texture defined by: showing plagioclase laths and clinopyroxenes in basalt; magnetites are the only opaques present in the rock; under 10X magnification | 50 |
| Photomicrograph 8 | Olivine phenocrysts replaced by late biotite along fractures. | 50 |
| Photomicrograph 9 | Small anhedral magnetites disseminated in matrix; under reflected light and 20X magnification | 51 |
| Photomicrograph 10 | Small anhedral magnetites disseminated in matrix; under reflected light and 20X magnification | 51 |
| Photomicrograph 11 | Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) in fine grained ground mass under transmitted light XPL (5X) | 62 |
| Photomicrograph 12 | Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) phenocrysts in fine grained ground mass under transmitted light XPL (5X) | 62 |
| Photomicrograph 13 | Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) phenocrysts in fine grained ground mass under transmitted light XPL (2X) | 62 |
| Photomicrograph 14 | Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) in fine grained ground mass under transmitted light XPL (20X) | 62 |
| Photomicrograph 15 | Photomicrograph showing presence of Clinopyroxene (Cpx) phenocrysts in fine grained ground mass under transmitted light XPL (2X). | 63 |
| Photomicrograph 16 | Photomicrograph showing presence of Clinopyroxene (Cpx) phenocrysts in fine grained ground mass under transmitted light XPL (2X) | 63 |
| Photomicrograph 17 | Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) in fine grained ground mass under transmitted light XPL (10X) | 64 |
| Photomicrograph 18 | Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) in fine grained ground mass under transmitted | 64 |
| Photomicrograph 19 | Olivine and clinopyroxene phenocrysts in very fine-grained plagioclase-pyroxene rich basaltic matrix; porphyritic texture; also note the fusion of two phenocrysts; under cross polarized light and at 2X magnification | 65 |
| Photomicrograph 20 | Compositional zoning in olivine phenocrysts; note the plagioclase rich matrix; under 20X magnification | 65 |
| Photomicrograph 21 | Bimodal distribution of sizes of phenocrysts; euhedral phenocrysts often broken and subsequently filled up by basaltic crystallization; 2X | 65 |
| Photomicrograph 22 | Glomeroporphyritic texture defined by clusters of olivine crystals in a basaltic matrix; 2X magnification | 65 |
| Photomicrograph 23 | Secondary alteration of olivine and pyroxene by biotite and serpentinite and chlorite; note the close association of olivine and magnetite; under plane polarized light and 10X magnification | 66 |
| Photomicrograph 24 | Close association of magnetite and olivine; showing Close magnetite also disseminated in matrix; under 2X magnification | 66 |
| Photomicrograph 25 | Fractured and altered outer rim of Olivine phenocrysts showing syn crystallization deformation | 66 |
| Photomicrograph 26 | Undulose extinction in clinopyroxene indicating grain scale ductile deformation. | 66 |
| Photomicrograph 27 | Anhedral, skeletal shaped large magnetite intergrown with Olivine phenocrysts; small magnetite as dissemination within matrix | 67 |
| Photomicrograph 28 | Disseminated magnetite within alteration zones of the basalt. | 67 |
| Photomicrograph 29 | Porphyritic texture with olivine phenocrysts in fine grained plagioclase clinopyroxene rich basaltic matrix; under 2X magnification | 68 |
| Photomicrograph 30 | Euhedral phenocrysts of Clinopyroxene and olivine in basaltic matrix; note the cluster of phenocrysts defining glomero-porphyritic texture; under 2X | 68 |
| Photomicrograph 31 | Microlites of plagioclase oriented haphazardly and interstitial spaces filled by pyroxene defining intergranular texture; also note the rounded vesicles. | 68 |
| Photomicrograph 32 | Sub-rounded vesicles within basaltic matrix, filled up by quartz crystals grew perpendicular to the wall of vesicles; | 68 |
| Photomicrograph 33 | Anhedral magnetite occurring within basalt and closely associated with phenocrysts; under reflected light and 10X magnification | 69 |
| Photomicrograph 34 | Anhedral and small magnetite as dissemination within basalt; under reflected light and 10X | 69 |
| Photomicrograph 35 | Fractured olivine phenocrysts with alteration along margin and fracture in a fine-grained magnetite rich basaltic matrix | 70 |
| Photomicrograph 36 | Bimodal distribution of phenocrysts size in basalt; under cross polarized light and 2X magnification | 70 |
| Photomicrograph 37 | Clinopyroxene and olivine phenocrysts within basalt; note the exsolution lamellae within clinopyroxene | 71 |
| Photomicrograph 38 | Small, rounded filled up vesicles within basaltic matrix; quartz crystals have filled up the vesicles; under 5X magnification | 71 |
| Photomicrograph 39 | Olivine pseudomorph; secondary serpentine and a very few biotite replace the olivine phenocrysts completely, keeping intact its boundary and shape; also note the matrix rich in small magnetite and rutile. | 72 |
| Photomicrograph 40 | Olivine pseudomorph; secondary serpentine and a very few biotite replace the olivine phenocrysts completely, keeping intact its boundary and shape; also note the matrix rich in small magnetite and rutile under XPL. | 72 |
| Photomicrograph 41 | Large anhedral clinopyroxene phenocrysts intergrown with coarse plagioclase laths indicating eutectic crystallization. Under 5X magnification. | 72 |
| Photomicrograph 42 | Large anhedral clinopyroxene phenocrysts intergrown with coarse plagioclase laths indicating eutectic crystallization. Under 5X Magnification under XPL. | 72 |
| Photomicrograph 43 | Olivine pseudomorph; secondary serpentine replaces the olivine phenocrysts completely, keeping intact its boundary and shape; also note the magnetite rich matrix | 73 |
| Photomicrograph 44 | Olivine pseudomorph; secondary serpentine replaces the olivine phenocrysts completely, keeping intact its boundary and shape; also note the magnetite rich matrix. | 73 |
| Photomicrograph 45 | Matrix rich in plagioclase laths with interstitial clinopyroxene and magnetite; under 10X magnification | 73 |
| Photomicrograph 46 | Disseminated and tiny magnetite in basalt; under reflected light | 73 |
| Photomicrograph 47 | Clinopyroxene and olivine phenocrysts; small magnetite disseminated in matrix | 73 |
| Photomicrograph 48 | Porphyritic basalt with phenocrysts of olivine associated with large magnetite clusters. | 74 |
| Photomicrograph 49 | Fractured phenocrysts of olivine in magnetite rich basaltic matrix; note few large magnetite closely associated with olivine phenocrysts. | 75 |
| Photomicrograph 50 | Bimodal distribution of size of phenocrysts; smaller sized phenocrysts of plagioclase and pyroxene in basaltic matrix; under 5X magnification. | 75 |
| Photomicrograph 51 | Bimodal distribution of size of phenocrysts; smaller sized phenocrysts of plagioclase and pyroxene in basaltic matrix; under 5X magnification under crossed polar. | 75 |
| Photomicrograph 52 | Zoning and exsolution lamellae and twinning in clinopyroxene phenocrysts; under 5X magnification. | 75 |
| Photomicrograph 53 | Compositional zoning in olivine showing phenocrysts. | 76 |
| Photomicrograph 54 | Olivine phenocrysts and anhedral magnetite intergrown with olivine; under reflected light 20x magnification. | 76 |
| Photomicrograph 55 | Intergrown phase; under reflected light with silicate as and 10X magnification | 76 |
| Photomicrograph 56 | Olivine phenocrysts surrounded by anhedral magnetite; under reflected light and 5X magnification. | 76 |
| Photomicrograph 57 | Fractured phenocrysts of large olivine in basaltic matrix; note the bimodal distribution of size of phenocrysts. | 77 |
| Photomicrograph 58 | Angular subhedral phenocrysts of clinopyroxene and relatively small olivine phenocrysts in microlites rich basaltic matrix | 77 |
| Photomicrograph 59 | Small rounded vesicles in basalt; also note the fractured olivine phenocrysts. | 77 |
| Photomicrograph 60 | Under cross polarised light; vesicles are filled up with quartz; note the microlites of plagioclase and glass defining the hyalopilitic texture. | 77 |
| Photomicrograph 61 | Higher magnification of the vesicles filled up with quartz. | 78 |
| Photomicrograph 62 | Higher magnification of the vesicles filled up with quartz. | 78 |
| Photomicrograph 63 | Porphyritic and hyalopilitic textures of the rock. | 78 |
| Photomicrograph 64 | Tiny magnetite disseminated in matrix; under reflected light and 10X magnification. | 78 |
| Photomicrograph 65 | Euhedral and fractured olivine phenocrysts in fine Grained basaltic matrix defining porphyritic texture. | 79 |
| Photomicrograph 66 | Compositional zoning in olivine | 79 |
| Photomicrograph 67 | Large and fractured olivine phenocrysts showing Euhedral, with compositional zoning in fine grained basaltic matrix; under 2X magnification | 80 |
| Photomicrograph 68 | Plagioclase laths and interstitial clinopyroxene define the intergranular texture of basalt; under cross polarized light and at 10X magnification | 80 |
| Photomicrograph 69 | Chloritic alteration of the matrix; note the anhedral magnetite grains disseminated in matrix | 80 |
| Photomicrograph 70 | Anhedral magnetite intergrown with silicates; under reflected light and at 20X magnification | 80 |
| Photomicrograph 71 | Anhedral magnetite intergrown with silicates; under reflected light and at 10X magnification | 81 |
| Photomicrograph 72 | Anhedral magnetite intergrown with silicates; tiny magnetites disseminated in matrix; under reflected light and at 20X magnification | 81 |
| Photomicrograph 73 | Large euhedral and fractured olivine phenocrysts; note the bimodal phenocrysts size distribution; magnetite rich matrix and phenocrysts of clinopyroxene forming star shaped cluster. | 7782 |
| Photomicrograph 74 | Large euhedral and fractured olivine phenocrysts; note the bimodal phenocrysts size distribution; magnetite rich matrix and phenocrysts of clinopyroxene forming star shaped cluster under cross polarised light | 82 |
| Photomicrograph 75 | Compositional oscillatory zoning in clinopyroxene basal section indicating different pulses of magma influx | 82 |
| Photomicrograph 76 | Compositional oscillatory zoning in clinopyroxene basal section indicating different pulses of magma influx | 82 |
| Photomicrograph 77 | Bimodal distribution in phenocryst’s size; note the large olivine phenocrysts which crystallized early; relatively smaller pyroxene phenocrysts disseminated in microlite and glassy matrix of basalt. | 83 |
| Photomicrograph 78 | Bimodal distribution in phenocryst’s size; note the large olivine phenocrysts which crystallized early; relatively smaller pyroxene phenocrysts disseminated in microlite and glassy matrix of basalt. | 83 |
| Photomicrograph 79 | Alteration of clinopyroxene phenocrysts and matrix phases by chlorite and biotite; note the magnetite concentration in matrix. | 83 |
| Photomicrograph 80 | Olivine phenocryst altered along fractures by biotite; biotite also alters the matrix phases. | 83 |
| Photomicrograph 81 | Rounded nodular part with coarser clinopyroxene and plagioclase grains; magnetite is completely absent in this coarser part. | 84 |
| Photomicrograph 82 | Rounded nodular part with coarser clinopyroxene and plagioclase grains; magnetite is completely absent in this coarser part under cross polarised light. | 84 |
| Photomicrograph 83 | Rounded nodular part with coarser clinopyroxene and plagioclase grains; magnetite is completely absent in this coarser part under cross polarised light. | 84 |
| Photomicrograph 84 | Clusters of phenocrysts within microlite and glass enriched matrix | 84 |
| Photomicrograph 85 | Clusters of phenocrysts within microlite and glass enriched matrix | 85 |
| Photomicrograph 86 | Small magnetites as disseminate | 85 |

List of Annexures

| SNO | List of Annexures |
| --- | --- |
| 1 | Analytical results of Bed rock samples |
| 2 | Analytical results of Pit samples |
| 3 | Major oxide test results |
| 5 | BRS sample test results |
| 4 | Pit sample test results |

List of Figures

| SNO | List of Figures | Pg no |
| --- | --- | --- |
| 1 | TAS Plot | 122 |
| 2 | R1-R2 plot | 123 |
| 3 | Pearce et al plot | 125 |
| 4 | Mullen plot | 126 |

| SNO | List of Tables | Pg no |
| --- | --- | --- |
| 1 | Details of the Project | 6 |
| 2 | Investigation Agency | 7 |
| 3 | NQT | 8-10 |
| 3 | Persons Involved | 11 |
| 4 | Lithostratigraphic succession of the Botad area | 19-20 |
| 5 | Block Geology | 21 |
| 6 | Details of the Pitting | 160-163 |
| 7 | Major Oxides | 270 |
| 8 | Expenditure | 277-281 |
| 9 | Locality Index | 283 |

List of Tables

| Plate no List of Plates |  | Pg no |
| --- | --- | --- |
| Plate no 1 | Location map | 12 |
| Plate no 2 | Land use / Land cover | 15 |
| Plate no 3 | Regional Geological map | 22 |
| Plate no 4 | Botad block boundary on Toposheet | 24 |
| Plate no 5 | Outcrop map | 27 |
| Plate no 6 | Red bole bed locations on Toposheet. | 31 |
| Plate no 7 | Red bole bed locations | 32 |
| Plate no 8 | Elevation map of Botad block with pit locations | 34 |
| Plate no 9 | Geological Cross section of the Botad block | 39 |
| Plate no 10 | Lithological Map of Botad block | 40 |
| Plate no 11 | Lithological Map of Botad block with pit locations | 164 |
| Plate no 12 | Pit locations | 258 |
| Plate no 13 | BRS Outcrop locations on Toposheet | 260 |
| Plate no 14 | BRS Outcrop locations on Geological map | 261 |
| Plate no 15 | Distribution of Nickel in Botad Block | 263 |
| Plate no 16 | Distribution of Niobium in Botad Block | 264 |
| Plate no 17 | Distribution of Chromium in Botad Block | 265 |
| Plate no 18 | Distribution of Cobalt in Botad Block | 266 |
| Plate no 19 | Delineation of Basaltic Flows | 35 |

Critical Mineral Trackers

(NOTIFIED PRIVATE EXPLORATION AGENCY)

Project Title: Reconnaissance Survey (G4) for Ni, Co and PGE in Botad area,

Botad District, Gujarat.

Project Code:    23/607/2025-NMET/1009

Project Timeline: 10 Months (From 13.03.2025 to 12.01.2026)

Project Location: Botad, Gujarat

Funding Organisation

National Mineral Exploration and Development Trust (NMEDT), Ministry of Mines

Room No. 325 & 326, Wing-F, Udyog Bhawan, Rafi Ahmed Kidwai Marg,

Rajpath Area, Central Secretariat

New Delhi-110011

Phone: 011 2307 1006

Email Id: nmet-mines@gov.in

Project executed as NPEA:

Critical Minerals Trackers

#Concourse, No 306,7-1-58/CC/306

Opp Lal Bungalow, Greenland’s,

Hyderabad-500016 INDIA.

Ph no:7893847742

www.criticalmineraltrackers.co.in

Team Involved:

S. Rama Murthy

Principal Investigator

Mekala Chandu

Team Member (Geologist)

Dr A Ajay Kumar

Team Member (Geologist)

### CHAPTER -1

### "सारांश"

निकेल–कोबाल्ट–पीजीई (Ni–Co–PGE) की संभाव्यता के आकलन हेतु बोटाद क्षेत्र, बोटाद जिला, गुजरात में पुनर्ज्ञान स्तर (G4) की खोज कार्यवाही NMET द्वारा वित्तपोषित परियोजना के अंतर्गत M/s Critical Minerals Trackers (आगे बोटाद परियोजना के रूप में संदर्भित) द्वारा निष्पादित की गई। अन्वेषण क्षेत्र का कुल विस्तार 99.6 वर्ग किलोमीटर है, जो भारतीय सर्वेक्षण विभाग (Survey of India) की टोपोशीट संख्या 41N/12 एवं 41N/16 में स्थित है तथा 22°03′45″ उ. से 22°14′30″ उ. अक्षांश एवं 71°38′15″ पू. से 71°46′15″ पू. देशांतर के मध्य विस्तृत है।

इस परियोजना का मुख्य उद्देश्य दक्कन लार्ज इग्नियस प्रांत (DLIP) में पाई जाने वाली पिक्राइट-आश्रित महाकाय PGE–Ni–Cu–(Co) निक्षेपों के अनुरूप मैग्मैटिक सल्फाइड प्रकार की PGE–Cu–Ni–Co खनिजीकरण संभाव्यता का आकलन करना था।

कार्य-घटक में 1:25,000 पैमाने पर भूवैज्ञानिक मानचित्रण, पेट्रोग्राफिक अध्ययन, पिटिंग कार्य, तथा बेडरॉक एवं पिट नमूनों का रासायनिक विश्लेषण सम्मिलित था। अध्ययन क्षेत्र मुख्यतः दक्कन ट्रैप अनुक्रमों पर विकसित पिक्राइटिक बेसाल्ट से आच्छादित है। क्षेत्र में दक्कन ट्रैप अनुक्रम का प्रतिनिधित्व ऊपरी क्रिटेशस से निचले पैलियोसीन आयु के पोरफिरिटिक एवं अमिग्डुलर बेसाल्ट प्रवाहों द्वारा होता है, जिन्हें पिक्राइट एवं बेसाल्टिक पिक्राइट के सिल एवं डाइक अंतर्भेदित करते हैं।

अध्ययन क्षेत्र में चार प्रमुख बेसाल्ट प्रवाहों की पहचान की गई है, जिन्हें रेड बोले परतों द्वारा अलग किया गया है। ये रेड बोले परतें सतही अनावृत्तियों तथा खुदे हुए कुओं (dug wells) के खंडों में स्थान-स्थान पर देखी गईं:

| प्रवाह | समुद्र तल से ऊँचाई | सापेक्ष आयु |
| --- | --- | --- |
| प्रवाह–4 | लगभग 90 मीटर | सबसे नवीन |
| प्रवाह–3 | 80–90 मीटर |  |
| प्रवाह–2 | 65–80 मीटर |  |
| प्रवाह–1 | 65 मीटर से कम | सबसे प्राचीन |

इन प्रवाहों को पिक्राइट एवं बेसाल्टिक पिक्राइट के सिल असंगत (discordant) संबंध के साथ अंतर्भेदित करते हैं। पिक्राइटिक शैलें लगभग पूरे 99.6 वर्ग किलोमीटर क्षेत्र में फैली हुई हैं और यही वर्तमान अन्वेषण का मुख्य केंद्र रहीं। विस्तृत मानचित्रण के माध्यम से पिक्राइट एवं बेसाल्टिक पिक्राइट के ज्ञात अनावृत्त क्षेत्र में उल्लेखनीय विस्तार

किया गया है। इसके अतिरिक्त, कई खुदे हुए कुओं में बेसाल्ट प्रवाहों के नीचे पिक्राइट सिलों की उपस्थिति भी दर्ज की गई है।

कुल 182 नमूनों का पेट्रोग्राफिक अध्ययन किया गया, जिनमें सतही एवं पिट नमूने सम्मिलित हैं। अध्ययन का उद्देश्य शैल प्रकारों की पहचान, बनावट (texture), संचयी (cumulus) बनावट की उपस्थिति/अनुपस्थिति, सल्फाइड या ऑक्साइड अयस्क खनिजों की उपस्थिति, तथा हाइड्रोथर्मल परिवर्तन का आकलन करना था। क्षेत्र में पहचानी गई प्रमुख शैल इकाइयों में पिक्राइट, बेसाल्टिक पिक्राइट, एफैनिटिक बेसाल्ट एवं पोरफिरिटिक बेसाल्ट शामिल हैं। पिक्राइट में ओलिविन प्रमुख खनिज है, जो प्रायः क्लिनोपाइरॉक्सीन एवं अल्प मात्रा में फेल्डस्पार के साथ सह-अस्तित्व में पाया जाता है। अनेक नमूनों में ओलिविन का सर्पेन्टाइन एवं/या इडिंग्साइट में परिवर्तन पाया गया है, जो कुछ क्षेत्रों में हाइड्रोथर्मल क्रियाओं की उपस्थिति का संकेत देता है। पिक्राइट एवं बेसाल्टिक पिक्राइट में मध्यम-दानेदार, होलोक्रिस्टलाइन से पोरफिरिटिक बनावट पाई जाती है तथा रिक्तिकाएँ (vugs) प्रायः जिओलाइट से भरी होती हैं। बेसाल्ट में मुख्यतः प्लैजियोक्लेज फिनोक्रिस्ट पाए जाते हैं, जिनके साथ ग्राउंडमास में अल्प मात्रा में ओलिविन एवं क्लिनोपाइरॉक्सीन उपस्थित हैं।

1:25,000 पैमाने पर भूवैज्ञानिक मानचित्र अंतिम रूप से तैयार कर लिया गया है तथा सभी पिट स्थलों को उसमें अंकित किया गया है। 95 पिट नमूनों के रासायनिक विश्लेषण पूर्ण हो चुके हैं तथा Ni, Cr, Co एवं Nb के तत्वीय वितरण मानचित्र तैयार किए गए हैं। इन तत्वों के वितरण पैटर्न के आधार पर यह पाया गया है कि प्रवाह–3 सर्वाधिक संभावनाशील है तथा कुछ सीमा तक प्रवाह–4 भी आशाजनक प्रतीत होता है।

पिट नमूनों में तत्वीय सांद्रण निम्न प्रकार है:

- Cr: 122–1280 पीपीएम

- Co: 8–90 पीपीएम

- Ni: 87–1116 पीपीएम

- Nb: 250–1395 पीपीएम; जबकि प्रवाह–3 के उत्तरी भाग में नमूना संख्या 94 में 39,850 पीपीएम (3.985%) Nb का असाधारण मान प्राप्त हुआ है।

20 पिट नमूनों के प्रमुख ऑक्साइड आंकड़ों को TAS एवं R1–R2 आरेखों पर निरूपित किया गया, जिनके अनुसार शैलें पिक्राइटिक बेसाल्ट एवं बेसाल्ट प्रकृति की हैं। Pearce एवं Mullen त्रिकोणीय आरेख के अनुसार इन शैलों की उत्पत्ति महासागरीय रिज/महासागरीय तल परिवेश से संबंधित है।

इसके अतिरिक्त, 77 बेडरॉक नमूनों (BRS) का भी रासायनिक विश्लेषण किया गया, जिनमें तत्वीय मान निम्न सीमाओं में पाए गए:

- Co: 2.9–80.3 पीपीएम

- Cr: 12.4–1411.6 पीपीएम

- Ni: 23.6–1105.1 पीपीएम

- Nb: 4.4–1214.6 पीपीएम

इस प्रकार, पिट एवं बेडरॉक नमूनों सहित कुल 182 नमूनों का रासायनिक विश्लेषण किया गया। 10 नमूनों को पेट्रोलॉजी डिवीजन, GSI, साउदर्न रीजन में पेट्रोग्राफिक अध्ययन हेतु भेजा गया, जिनकी रिपोर्ट प्राप्त हो चुकी है तथा संलग्न की गई है। इसके अतिरिक्त, 5 नमूनों का XRD तथा 4 नमूनों का SEM विश्लेषण किया गया है।

### Summary

A reconnaissance stage (G4) exploration programme for Ni–Co–PGE was carried out in the Botad area, Botad District, Gujarat, under NMET funding, and executed by M/s Critical Minerals Trackers (hereafter referred to as the Botad Project). The total investigation area covers 99.6 sq. km, falling within Survey of India toposheets 41N/12 and 41N/16, bounded by latitudes 22°03′45″ N to 22°14′30″ N and longitudes 71°38′15″ E to 71°46′15″ E.

The principal objective of the project was to assess the prospectivity of magmatic sulphide–type PGE–Cu–Ni–Co mineralization within the Deccan Large Igneous Province (DLIP), analogous to picrite-hosted supergiant PGE–Ni–Cu deposits documented elsewhere.

The scope of work included large scale geological mapping at 1:25,000 scale, petrographic studies, pitting, and chemical analysis of bedrock and pit samples. The area is dominantly underlain by picritic basalts developed over the Deccan Trap sequences. The Deccan Traps in the area comprise Upper Cretaceous–Lower Paleocene porphyritic and amygdaloidal basalt flows, intruded by picrite and olivine basaltic sills and dykes.

Four basalt flows were delineated, separated by intermittently developed red bole horizons, observed both in surface exposures and dug-well sections:

| Flow | Elevation above MSL | Relative age |
| --- | --- | --- |
| Flow 4 | ~90 m | Youngest |
| Flow 3 | 80–90 m |  |
| Flow 2 | 65–80 m |  |
| Flow 1 | <65 m | Oldest |

Picrite and basaltic picrite sills intrude these flows with discordant relationships. Olivine rich basalts/Picritic rocks are mostly confined to Flow 3 and Flow 4 in the study area and formed the main focus of investigation. Detailed mapping has significantly extended the known outcrop distribution of picrite and basaltic picrite, including their occurrence as sills encountered in dug wells beneath basalt flows.

A total of 182 samples  including surface and pit samples for chemical analysis by ICPMS to identify lithological variations, textural features, presence/absence of sulphides or oxide minerals, and hydrothermal alteration. Lithologies identified include picrite, olivine basalt, aphanitic basalt, and

porphyritic basalt. Picrites are characterized by olivine with frequent clinopyroxene and minor feldspar, occurring as phenocrysts and groundmass. Olivine alteration to serpentine and iddingsite in several samples suggests localized hydrothermal activity. Picrite and basaltic picrite exhibit medium-grained holocrystalline to porphyritic textures, with vugs commonly filled by zeolites. Basalts are dominated by plagioclase phenocrysts with minor olivine and clinopyroxene in the groundmass.

A 1:25,000 scale geological map has been finalized, incorporating all pit locations. Chemical analysis of 95 pit samples has been completed, and elemental distribution maps for Ni, Cr, Co, and Nb were prepared. Based on geochemical patterns, Flow 3 appears most promising, followed by Flow 4.

Major oxide data from 20 pit samples, plotted on TAS and R1–R2 diagrams, classify the rocks as picritic basalt and basalt. Pearce and Mullen discrimination diagrams indicate an oceanic ridge/oceanic floor tectonic affinity.

In addition, 77 bedrock samples (BRS) were analysed, yielding the following ranges:

- Co: 2.9–80.3 ppm

- Cr: 12.4–1411.6 ppm

- Ni: 23.6–1105.1 ppm

- Nb: 4.4–1214.6 ppm

Overall, 182 samples (pit + BRS) were chemically analysed. Ten samples were submitted to the Petrology Division, GSI, Southern Region for petrographic studies, and the report has been received and appended. Additionally, five samples were analysed by XRD and four samples by SEM.

CHAPTER-2

INTRODUCTION

M/S Critical Mineral Trackers, Hyderabad, a Ministry of Mines Notified Private Exploration Agency (NPEA) was awarded the NMET funded project, entitled “Reconnaissance Survey (G4) for Ni, Co and PGE Prospect Block in Botad area, Botad District, Gujarat. The majority of the rocks in the research region are from the Tertiary Deccan Trap, which also contains Quaternary Miliolite Formation, sedimentary inter-trappeans, tholeiites, and picritic basalt. Worldwide picritic rocks associated with Large Igneous Provinces (LIP) host many rich Ni, Co and PGE deposits. The Botad Project area covers an area of 99.6 sq.km in the Botad district of Gujarat. This project report outlines all the activities undertaken during the project from 13.03.2025 to 12.01.2026, which includes large-scale geological mapping (1: 25000), Sample collection, pitting, petrographic studies, geochemical analysis (XRF & ICPMS), XRD and SEM analysis.

2.1 Details of the Project:

| 1 | Project name | Reconnaissance Survey (G4) for Ni, Co and PGE Prospect Block in Botad area, Botad District, Gujarat |
| --- | --- | --- |
| 2 | Project code | 23/607/2025-NMET/1009 |
| 3 | Commodity | Ni, Co and PGE |
| 4 | Tehsil | Botad, Gadhada, Barvala, and Ranpur |
| 5 | Nearby villages | Sherthali, Amapar, Samadhiyala No1, Khas Road, Tajpar, Bhambhan |
| 6 | State | Gujarat |
| 7 | Project area | 99.6 sq km (100 sq km) |
| 8 | Survey of India Toposheet No | 41 N/12 & 16 |
| 9 | Date of commencement of project | 13 March 2025 |
| 10 | Boundary coordinates of the project area | Boundary coordinates Latitude (DD) Longitude (DD) A 22.167 71.574 B 22.282 71.634 C 22.185 71.763 D 22.081 71.701 |

2.2 Investigating agency:

| Name | Critical Minerals Trackers |
| --- | --- |
| Postal Address | H.O #Concourse, No 306,7-1-58/cc/306, Opp Lal Bungalow, Greenland, Hyderabad-500016, INDIA |
| Telephone No (Office) | 9666975499 |
| Mobile No | 9849574333 |
| E-Mail address | criticalmineraltrackers@gmail.com |
| Date of accreditation granted by QCI-NABET | May 20,2024 |
| Date of expiry of Accreditation | May 15,2027 |
| Telephone No (Residence) |  |
| Category of the NPEA | Category A Exploration Agency |

2.3   Objectives of investigation:

Search and assessment of Nickel, Cobalt & PGE resources in Botad Block, Botad District, Gujarat.

2.4 Basis for taking up Investigation:

About 99.6 sq. km area around Botad- Hadadad-Senthali are known for picrite flows/ sills/ dykes within tholeiitic basalt flows. The area was mapped on scale 1:25,000 about 30% of the area is covered by picritic flows/sills and dykes which are 10 m to 20 m thick. Sulphide related PGE-Ni-Co mineralization is usually localized at the basal part of thick picritic sills and flows. Weathered lateritic areas that developed lover picritic basalts should also be tested for possible Ni rich zones. Some of these picritic hocks are very high in Ni concentration (over 1000 ppm). MgO content of these picritic hocks goes up to 20%. Co and PGE content of these picritic rocks are not analysed. High concentration of thick picritic flows sills and dykes with high MgO content along with isolated high Ni concentration in these rocks make them a good target for Ni-Co-PGE mineralization. Especially the basal parts of thick picritic flows may host Sulphur rich Ni-Co-PGE mineralization However, Ghevariya et al. (1996) analysed 33 basaltic samples (picrite and other

basalts) from the area of which nine samples yielded over 500 ppm Ni, highest concentration being 0.1% Ni.

All the above observations of Ghevariya et al., 1996 are positive indicators for presence of sulphide related PGE-Ni mineralization in the area.

In view of the above observations, a G4 exploration program for Ni-PGE in picritic basalts of the    area is proposed.

2.5     Table 1: Nature and quantum of work Approved vs Achievement:

| S. No. | Nature of work* | Approved Quantum | Achieved Quantum | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Aerial Reconnaissance: PGRS studies (1:25/50,000) (sq km) | 100 sq km | 100 sq km | Using Google imagery and Landsat OLI imagery |
| 2 | Geological Survey- (DGPS/Total Station Survey) (sq km) | Nil | Nil |  |
| 3 | Geological Mapping (LSM on 1:12500) (sq km) | 100 sq km | 100 sq km |  |
| 4 | Geophysical Surveys (Gravity, Magnetic Stn), Nos | Nil | Nil |  |
| 5 | Technological |  |  |  |
|  | - Surface exploration- Pitting (cu m) | 100 | 93 | Remaining 7 could not be done because of objection from the local villagers |
|  | (b) Sub Surface exploration- Drilling (m) | 300 | Nil | Not recommended by the TCC II |
| 6 | Geochemical survey |  |  |  |
|  | (a) Bed Rock Sample (Nos.) | Nil | Nil |  |
|  | - Stream Sediment Sample (Nos.) | Nil | Nil |  |
|  | (c) Core Sample (Nos.) | 300 | Nil |  |
|  | (d) PT sample (Nos) | 100 | 95 (93 pit samples and 2 bed rock samples) |  |
| 7 | Petrographic/ Mineragraphic/ other studies |  |  |  |
|  | Pitting sample: 100*1=100 nos) by fire assay (ICPMS Ni-S) for PGE | 25 | Nil | TCC II recommended not to go of PGE analysis as Ni values are less than 0.2% |
|  | Core drilling samples 3*100=300 Total depth 100m each, samples will be collected at every 1.0m interval by ICPMS method | 50 | Nil | No drilling was done as per the recommendation of TCC II |
|  | Trace element analysis (Ni, Cu, As, Mo, Re, In, Te & Co) 14 elements by ICPMS | 245 | 182 | Remaining samples are earmarked for drilling, hence not done |
|  | Major oxides by XRF technique (whole rock analysis) | 40 | 20 | Remaining samples are earmarked for drilling, hence not done |
|  | Complete Petrographic/ Ore Microscopic/Minerographic studies (Nos.) | 10 | 10 | Nil |
|  | XRD analysis for identification of minerals (random) | 10 | 5 | Since drilling is not recommended by TCC II, only 5 samples have been sent for XRD |
|  | EPMA/SEM | 10 | 4 | Since drilling is not recommended by TCC II, only 4 samples have been sent for SEM |
| 8 | Chemical Analysis |  |  |  |
|  | (a) Internal Check samples (5% of Primary samples) | 0 | Nil | Nil |
|  | (c) External Check sample (10 % of Primary samples) | 0 | Nil | Nil |
|  | (d) Composite samples | Nil | Nil | Nil |

2.6     Personnel Involved

| Sno | Team Members | Role |
| --- | --- | --- |
| 1. | Soma Ramamurthy | Principle Investigator |
| 2. | Mekala chandu | Team Member (Geologist) |
| 3. | A. Ajay kumar | Team Member (Geologist) |

2.7   Mode of operation of different working components and associated agency

All field components, encompassing geological mapping, Pitting have been conducted using in-house resources of Critical Minerals Trackers. Geochemical analyses were carried out in collaboration with the Lucid Laboratories, Hyderabad. Samples for XRD are analyzed at the Shiva Analytical in Bengaluru, Karnataka. Additional analyses, including SEM (Scanning Electron Microscope) and preparation of thin section and petrographic studies were carried out at Petrology Divn of Geological Survey of India, Southern Region, Hyderabad.

2.8 Acknowledgements:

This project is funded by National Mineral Exploration Trust (NMET), Ministry of Mines, Government of India, New Delhi (vide project sanction File No: 23/607/2025-NMET/1009 dated 13th March, 2025). The project is technically designed and also reviewed time to time by the Technical and Cost Committee (TCC) followed by the Executive Committee (EC) of NMET. We express our sincere gratitude to EC, Chairman and members of TCC and HOD of NMET for awarding this project. Technical and financial review by the CGM and GMRDS (Government of Gujarat) Gandhinagar are sincerely acknowledged. Critical Minerals Trackers (Hyderabad) expresses sincere thanks to all including local administration and villagers of the block area for their invaluable supports in executing the project in and around Botad-Khas-Samadhiyala area, Gujarat.

CHAPTER-3

3.1 Property description

The Botad Block having an area of 99.6 sq. km and nearest villages names include Botad, Samadhiyala no 1, Khas, Tajpar, Senthali. The Botad block comes under Botad district, Gujarat State. The total investigation area covers 99.6 sq. km, falling within Survey of India toposheets 41N/12 and 41N/16, bounded by latitudes 22°03′45″ N to 22°14′30″ N and longitudes 71°38′15″ E to 71°46′15″ E.

PLATE-1

3.2   Location and Accessibility

The Area is connected to Ahmedabad and other major cities of Gujarat both by rail and road. Paliyad-Botad and other major villages pass through the area. Botad is well connected to Ahmedabad, Vadodara, Bhavnagar, Rajkot and Surendranagar by road network. There are three major roads passing through Botad town namely NH-51 (Dwarka to Bhavnagar), SH-38 (Sarangpur road) and SH-21(Paliyad road). Botad is the Sub-divisional and Tehsil headquarters. Paliyad and Gadhada are other important towns and pilgrimage centers. Almost all the villages in the area are connected to Botad and Paliyad by roads.

Physiography and Drainage

The northern, eastern and southeastern parts of the area show almost flat ground with few low mounds. Major part of the area is covered by cultivated fields. The area shows a general east to southeast slope. The average elevation of the area is around 120m in northern part, 130m in western part and around 100m in central and northeastern part and around 80m in eastern part. The A171 spot height near Paliyad Jaga forms the highest point in the area. Drainage pattern is mainly dendritic to sub dendritic with major streams flowing towards east. There are two ephemeral streams flowing through the Botad block namely Utavali Nadi in the north, Bhalagama Nadi in the south. The reservoirs generally remain empty due to severe drought conditions.

3.3  Climate

It has the Tropical climate. The average rainfall of the area has 616 mm. The Minimum temperature during January is 17.4 degree Celsius and maximum temperature during May 42.43 degree Celsius.

Topography and Land use and Land cover

The block falling under the Toposheet Number 41N/12, 41N/16. The morphology of the area is undulatory to gently undulating plains, small hillocks, pediment & Pediplains.

3.4   Flora and Fauna

Major parts of the area in east and north are under cultivation except some wastelands, stream courses and low mounds. The hilly parts are mostly barren with scanty bushes along stream courses and along peripheral parts of the reservoir areas. Babul (Acacia indica and Acasia arabica), Cactus, fern are the important

vegetation. Shady trees like Pipal, Neem and Banyan are scattered mostly around populated areas and cultivated lands near villages. Groundnut, cotton, bazra, jowar, wheat are the important seasonal crops. Northwestern part of the area has scanty vegetation (Babuls and cactus) with a number of wildlife like deer and neelgai, fox, jackal etc.

3.5     Land use/Landcover:

This is a Land Use and Land Cover Map of Botad Block. It shows how different types of land are distributed      in the area using colours. Most of the area is agricultural land (yellow), indicating farming is the dominant activity. Settlements (red) appear in patches, showing villages or built-up areas. Small areas of barren/waste land (purple) and water bodies (blue) are also present, along with some trees/plants (green) in scattered locations. The black boundary line marks the limits of the Botad block.

PLATE-2

CHAPTER-4

4.Previous Work

- Fedden (1884) first carried out geological mapping of this area as part of his monumental work on Deccan Traps of Saurashtra area. He has described intrusive dykes from this area and from adjacent areas like Chogat Chamardi with acid intrusive. He also suggested that majority of the flows were derived from the fissure eruption. He proposed that the dykes belong to different stages and some of them are post-trappean in age and also are of hypabyssal phase.

- West (1958) studied different lava flows from bore hole logs and gave an account in detail in his work on lava flows from Saurashtra. West also described some picrite basaltic flows from boreholes in Dhanduka and Botad areas. He described 4 picrite basalt flows in Botad area. Gopalan et at (1971) have studied picritic basalt from a quarry near Botad.

- Agrawal and Dey (1985) have mapped these areas and have described porphyritic and fine-grained basalts from Botad, Turkha, and Dhuphamia areas. They have also described iron rich basalt with fine banding 4 km north of Dhakhaniya village. Though they have described olivine in petrographic description, they did not mention about the picritic basalts. There is also no mention about the presence of interstratified, cherty and silty fossiliferous sedimentary/bole beds in their report.

- Mr. Z. G. Ghevariya and Dr B.K. Sahu (1996) representing the Geological Survey of India, conducted extensive mapping of approximately 224 sq. km in the Botad-Hadadad-Senthali region.   Their work primarily focused on identifying the unique volcanic sequences within the Deccan Volcanic Province of Saurashtra.   He is credited with the significant discovery of thick picritic basalt flows and sills, which are rare magnesium-rich volcanic rocks. Ghevariya’s mapping revealed that these picritic units make up nearly 30% of the local geology, suggesting a high-temperature mantle source. Through geochemical analysis, they identified high concentrations of Nickel (Ni), with some samples exceeding 500 ppm. His findings provided the first concrete evidence that the Botad region holds high potential for Platinum Group Element (PGE) mineralization.  They hypothesized that these valuable minerals are likely concentrated at the base of the thick volcanic sills through a process of sulphide settling. Beyond mineralogy, his structural studies highlighted how regional faults and crustal doming controlled the movement of magma in the area.  His broader research also connected these volcanic events to the K-Pg boundary, helping to date the extinction-era lava flows. Ultimately, Ghevariya’s work transformed Botad from a standard volcanic landscape into a high-priority target for strategic mineral exploration in India.

- Naushad Md of Geological Survey of India during field seasons 2014–16 had investigated mafic and picritic lava flows and intrusive of the Deccan Volcanic Province in Kachchh, Saurashtra and East Cambay regions to evaluate their origin and Pt–Pd fertility. The basic plugs of Kachchh are porphyritic with dominant olivine and Ti-augite phenocrysts showing primitive compositions and Cr-spinel inclusions, indicating mantle-derived magma. Globular clinopyroxene textures and quartz xenocrysts suggest low-pressure crystallization and magma–crust interaction. Carbonate inclusions within Ti-augite phenocrysts point to carbonate-rich fluid activity during magma evolution. Picritic basalts of Saurashtra are rich in olivine and clinopyroxene and contain sulphide phases such as pentlandite and chalcopyrite, indicating sulphide immiscibility. Alkaline intrusive of Kanesara and nearby areas show gabbro–pyroxenite compositions with sulphide mineralization. Mantle xenoliths with highly forsteritic olivine confirm a primitive mantle source. A microscopic gold grain in lamprophyre from Kanesara marks the first such report from Gujarat Deccan alkaline rocks. Geochemically, strong Cr–Ni correlation reflects olivine–chromite control, while Pd shows no correlation with MgO, suggesting limited sulphide fractionation control. Although Pt–Pd values are generally low, sulphur saturation and sulphide occurrences indicate localized potential for Ni–Cu–PGE mineralization.

CHAPTER-5

5.0	Geology of the area

5.1	Regional Geology:

The exposed rocks in the region are Miliolite limestone, sedimentary intertrappean, and members of the Deccan Trap suite. The predominant rock type in the region is picrite basalt flows. FGB, or fine-grained basalt, is the primary rock type in the south region of the station on Paliyad Road. Basaltic, doleritic, and gabbroic dykes have penetrated these rocks.

Within the picrite basalt flows at different locations, the intertrappean sedimentary strata are found as thin, interstratified, roughly continuously traceable horizons. These can be easily recognized by their characteristic sub horizontal spread of outcrops in the cultivated fields, at the edges of the low mounds, and in the nalla cuts, as well as their unique hue. Along the higher reaches of the nallahs, the hillslopes, and the communities, the miliolitic limestone is found in thin sheets covering older rocks.

Cultivated fields cover a large portion of the northern and central regions. Road and nalla cuttings, solitary mounds, partially covered wastelands, minor nalla courses, and dam spillways and the faces of Quarry’s. Different flows could not be distinguished independently throughout the region because of scanty exposures, the discontinuous nature of partially exposed sedimentary intertrappean/red-bole beds, and a lack of adequate physiographic expressions.

Sections with weathered top and bottom surfaces, zones of highly altered amygdular and vesicular surfaces containing pipe amygdules, red bole/intertrappean beds, and unique mineralogical characteristics like crude banded nature and distinct physical layering were used to identify distinct flows whenever possible. In certain sections, changing coloring was also helpful in differentiating various flows.

5.1.1         Table: Lithostratigraphic succession of the Botad area

(Modified after Ghevariya and Sahu)

| Age | Lithological Units | Remarks | Thickness (m) |
| --- | --- | --- | --- |
| Recent |  | Top soil and sand cover | - |
| Quaternary | Miliolitic limestone | Dirty white, friable, stratified sandy limestone with grit, trap fragments and pebble boulders | 1–2 |
|  | Limestone | White, compact, well bedded, burrowed limestone with cherty and ashy partings; conglomerate at base | - |
| …………………………………Unconformityy…………………………………………… |  |  |  |
| Deccan Traps | Intrusives | Dolerite, basalt and gabbro dykes |  |
|  | Flow 8b | Vesicular picrite basalts with comparatively fresh olivine and pyroxene phenocrysts |  |
|  | Flow 8a | Weathered picrite basalt with Alternate fine-grained basalt; olivine phenocrysts altered |  |
|  | Intertrappeans (160 m RL) | Red bole bed-Baked reddish clay | 0.3–0.5 |
|  | Flow 7 | Dark greenish grey porphyritic picrite with Olivine and pyroxene phenocrysts with distinct amygdaloidal and vesicular zones at top and base in two alternations | 0.5 -0.8 |
|  | Intertrappeans (140 m RL) | Greenish, purple and red cherty fossiliferous beds Containing plant fossils, laterally changing to red bole bed in simtalavadi area. |  |
|  | Flow 6 | Porphyritic olivine basalt with large phenocrysts of pyroxene and olivine with amygdaloidal zone at the base |  |
|  | Intertrappeans (130–132 m RL) | Black fossiliferous chert, Grey to pink shale, silt stone, red clay with Plant fossils; laterally passing into red bole beds | 2 to 2.5 |
|  | Flow 5 (128–130 m RL) | Highly amygdaloidal Grey porphyritic olivine basalt with megaphenocrysts of pyroxene, amphibole and olivine | 8 to 10 |
|  | Intertrappeans (120 m RL) | Red and chocolate baked fossiliferous mud, chert, volcanoclastic sandy clay, marl with megaplants and micro-invertebrate fossils | 3–3.5 |
|  | Flow 4 | Picrite basalt -coarse porphyritic with large phenocrysts of olivine and pyroxene | 10 |
|  | Intertrappeans | Pink and grey baked clay (Bhadravadi area) | 1 to 1.5 |
|  | Flow 3 | Coarse porphyritic picrite basalt flows as sub-crop in Bhadravadi area | 8 TO 10 |
|  | Intertrappeans | Chocolate coloured baked and burrowed sandy clay and mud (sub crop in Bhadravadi area) | 0.5 to 0.8 |
|  | Flow 2 (90m RL) | Coarse porphyritic olivine basalt (picrite) with crude crystal layering. | 20 |
|  | Intertrappeans (71-72 m RL) | Red and greenish volcanic clays with baked top lensoid cherty layer exposed near bhambhan and lathidad. |  |
|  | Flow 1 (70m RL) | Weathered coarse porphyritic basalt flow in Lathidad and Bhambhan area;Subcrop at Aniyali |  |

5.2    Block Geology:

The Rock types located in Botad block, covering the villages- Senthali, Samadhiyala, Khas Road, Arto           road, Amapar, Tajpar etc. The outcrops are widely exposed and the different basaltic flows and red bole beds are observed in the different villages of the botad block. The Red bole beds in the area are seen as  a marker horizon and it is differentiating the basaltic flows.

The below are the Flows delineated:

Flow 		 Elevation range above MSL

- Flow 4                   90 m above	(youngest)

- Flow 3                  80 to 90 m

- Flow 2                  65 to 80m

- Flow 1                  Below 65m (oldest flow)

5.2.2    Table showing Block Geology:

| Rock type | Formation | Age | Flows |
| --- | --- | --- | --- |
| Olivine Basalt (Picritic basalt) | Junagarh volcanics | Late cretaceous-paleocene | Flow 4 |
| Red bole | Junagarh volcanics | Late cretaceous-paleocene | - |
| Porphyritic Olivine Basalt(Picritic Basalt) | Junagarh volcanics | Late cretaceous-paleocene | Flow 3 |
| Red bole | Junagarh volcanics | Late cretaceous-paleocene | - |
| Massive Basalt (Unclassified) | Junagarh volcanics | Late cretaceous-paleocene | Flow 2 |
| Red bole | Junagarh volcanics | Late cretaceous-paleocene | - |
| Amygdular Basalt, Fine grained olivine basalt | Junagarh volcanics | Late cretaceous-paleocene | Flow 1 |

CHAPTER-6

### Activity during the period (Geo-science Investigation)

##### 6.1    Large Scale Mapping (LSM):

Geologically, the area has variants of basalts like Fine-grained olivine basalt, Amygdaloidal basalt, massive basalt, Porphyritic olivine basalt, Picritic basalt. All these basaltic flows belong to Deccan Trap Group and of Saurashtra Supergroup and it belongs to late-cretaceous to Paleocene age. And mostly picritic basalts are vesicular in nature.

PLATE NO-3

A scale of 1: 25,000 has been used for Large Scale Mapping (LSM). For the working base map of the block, the 1:50,000 scale India toposheet (41N/12&16) was enlarged to a 1:25,000 size. The exposed rocks in the region are sedimentary intertrappean, and Deccan Trap suite. The predominant rock type in the region is made up of picrite basalt flows. FGB or fine-grained basalt, is the primary sort of rock located south of the Paliyad Road station. Doleritic, gabbroic, and basaltic dykes intrude into these rocks.

The thin inter stratified sedimentary strata are found more or less continually differentiable horizons at different locations within the picrite basalt flows. These are primarily distinguished by their unique coloring and a characteristic sub-horizontal outcrop distribution in the cultivated fields, at the edges of the low mounds, and in the nalla cuts.

Identifiable horizons at different locations within the picrite basalt flows. These are primarily distinguished by their unique coloring and a characteristic sub-horizontal outcrop distribution in the cultivated fields, at the edges of the low mounds, and in the nalla cuts.

Photograph 1: Picritic basalt with vesicular nature near RTO office road, Botad

Photograph 1: Exposure of picritic basalt with Vesicular nature near RTO office road,Botad

On the basis of weathered top and bottom surfaces, zones of highly altered amygdular (Fig 3) and vesicular surfaces (Fig 1) with pipe amygdules, red bole/intertrappean beds (Fig 2), and unique mineralogical characters like crude banded nature and distinct physical layering, various flows were identified in section.

.

PLATE NO-4

Photograph 2: Red bole bed in massive olivine basalt

Photograph 2 :Exposure of Red bole bed in massive olivine basalt near ARTO Botad

Photograph 3: Amygdaloidal basalt Exposure near Khas Road

Photograph 3: Exposure of Amygdaloidal basalt near Khas Village.

Photograph 4: Exposure of Olivine altered to Iddingsite in Khas Road

Photograph 5: Olivine crystals in green and yellow (Fresh to partially altered) near Khas Road

Photograph 5: Exposure of Olivine crystals in green and yellow (Fresh to partially in Khas road

Red bole bed is helpful as guides for demarcation of different lava flows acts as marker horizon (Fig 6)]

Picritic basalt show porphyritic texture with segregations of phenocrysts into layers of olivine, clinopyroxene, feldspar. In most basaltic rocks, Olivine and pyroxenes are altered to Iddingsite (Fig 4).

Massive, dark grey to greenish black, picritic basalt is dotted with olivine green to bottle green phenocrysts and varying numbers of yellowish aged peanut-sized nodules and the same rock contains both fresh and heavily changed olivine variants (Fig 5)

Photograph 6: Exposure of Red bole bed (Act as Marker horizon)

Photograph 6: Exposure of Red bole bed (Act as Marker horizon) near ARTO,Botad

##### 6.1.1   DELINEATION OF BASALTIC FLOWS:

Basaltic flows:

Supergroup: Saurashtra Group: Deccan Traps

Formation: Junagarh Volcanics

Age: Late cretaceous to Paleocene

### Red bole beds:

Plate no 5

During our Field Investigation, Red bole beds are identified at different locations.

- Indicates Flow Boundaries

Red bole beds form only at the top of a cooling lava flow. Their presence marks the end of one eruptive phase and the beginning of the next. Therefore, each red bole horizon represents a distinct flow interface.

- Helps in Vertical Flow Classification

Successive lava flows stack vertically. A red bole bed between two basalts confirms the separation of two different flow units.

- Laterally Continuous Marker

They provide a correlation reference across mapping areas.

- Field Recognition Aid

Easily recognizable due to:  Reddish to purple color, Soft, clayey texture, Presence of hematite and alteration minerals Useful when basalt textures are similar and difficult to differentiate.

### Red bole bed contact photographs:

Photograph 7: This red bole bed is located near Samadhiyala no1.Above the red bole, there is a massive olivine basalt. This red bole bed has the thickness of 2m. Excellent lateral continuity; red bole horizon extends continuously with sharp flow contacts.

Photograph 8: This red bole bed is located near Rto road office botad having the elevation of 86m.This red bole bed has good lateral continuity and it is traceable across the exposed section. Above the red bole bed. There is a massive olivine basalt.

Width (Thickness): 40–45 cm thick.

Photograph 9: This red bole is located near the Tajpar road near railway track having the elevation of 89m.Between the red bole, there is massive basalt and the dyke cutting across indicating is a different flow. This red bole bed has moderate lateral continuity but locally disturbed by vegetation and erosion. There is a calcite venations are visible.

Width (Thickness): Around 30–35 cm thick.

Photograph 10: This red bole bed is located near to the Bhambhan road. The lateral continuity of this red bole is clearly traceable. Above there is a vesicular olivine basalt. This red bole bed is located towards the south east of the botad block.

Width (Thickness): Approximately 25–30 cm thick.

Photograph 11: This red bole is located near the Amapar road. The lateral continuity of red bole bed is discontinuous in places due to slope wash and human disturbance, but identifiable in patches. Width (Thickness):

Around 20–25 cm thick.

These Basaltic flows are often sheet-like and horizontal to sub-horizontal at the time of emplacement. Successive flows stack vertically, so the oldest flow is generally at the lowest elevation, and youngest flows are at the highest levels. Although erosion may modify surfaces, the relative elevation differences remain fairly uniofrm across the field. In regions with uniform dip, contour spacing also shows gentle tilting of older flows.

PLATE NO 6

## 2. Contour Elevation

Below are the Flows delineated: With the help of Handy GPS, took the elevation of the area.

Flow	Elevation range above MSL

Flow 4   90 m above (youngest flow)

Flow 3   80 to 90 m

Flow 2  65 to 80m

Flow 1  Below 65m	(oldest flow)

Plate no:7

### Characteristics of the rock:

During the field investigation, four distinct basaltic flows were identified and differentiated based on macroscopic rock characteristics, including texture, phenocryst content, vesicularity, and mineralogical features. The observed characteristics allowed classification of the flows into Picritic, Amygdaloidal, Porphyritic, and Unclassified Massive basalt units.

## 6.1.2   Geological Cross section of  Botad Block:

Basaltic lava flows commonly occur as multiple stacked units formed during successive volcanic eruptions. When these flows have similar lithology and mineral composition, elevation becomes one of the most reliable criteria, besides the occurrence of red bole horizon in between the flows, to distinguish them.

Using the SRTM elevation data (30m resolution) topographic profile has been drawn along the section line A-B in the NW-SE direction and also drawn geological cross section across the delineated various flows. Since, most of the areas, the flows are horizontal to sub horizontal in nature and taking the red bole beds as contact between the flows, the following elevation range of flows are deciphered

The contour elevation in the range of -

FLOW 4 80 to 100 m

FLOW 3 70-80 m

FLOW 2 60-70 m

FLOW 1  <60m

Therefore, based on red bole beds, Contour elevation and megascopic characteristics of the basalts, the flows have been identified. Out of 4 flows, the Flow 4 is much thicker than the other flows.

Plate no 8

Fig showing A-B cross section of the block

Basaltic flows Profile:

Contour

interval in (m)

1KM

Distance (in km)

| Index |  |
| --- | --- |
|  | Flow 4 |
|  | Flow 3 |
|  | Flow 2 |
|  | Flow 1 |

Plate no:19

4 basaltic/picritic basalt flows have been demarcated in the area.

# 6.1.3 Basaltic Flows:

##### FLOW 1: (Fine grained olivine basalt & Amygdaloidal basalt)

Photograph 12: Fine-grained olivine basalt in Khas village

Photograph 12: showing Fine grained olivine basalt near khas village location.

- Sub horizontal in nature

- The Flow 1 has the elevation range of < 65m MSL.

- It is fine grained olivine basalts and the amygdular (amygdaloidal) basalts where the vesicles are filled with the secondary minerals such as calcite & zeolites.

- The fine-grained olivine basalts are very fine-grained olivine crystals which are aphanitic in nature.

| ANALYTICAL VALUES (PPM) OF BED ROCK AND PIT SAMPLES COLLECTED FROM FLOW 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No | Sample ID | Latitude | Longitude | Ni | Cr | Co | Nb | Sc |
| 1 | P41/BT8/2025 | 22.14236 | 71.72028 | 202.9 | 266.7 | 45.2 | 517.7 | 13.7 |
| 2 | CMT/011/01/2025 | 22.16286 | 71.73026 | 792.8 | 427.6 | 46.2 | 733.9 | 30.6 |
| 3 | P42/BTB/2025 | 22.14559 | 71.71484 | 417.4 | 474.6 | 131 | 771.6 | 27.9 |
| 4 | P36/BTB/2025 | 22.15806 | 71.72417 | 150 | 396.4 | 34.1 | 360.5 | 14.7 |
| 5 | CMT/011/71/2025 | 22.14027 | 71.72448 | 175.9 | 292.8 | 27.2 | 25.9 | 22.7 |
| 6 | CMT/011/08/2025 | 22.16778 | 71.72174 | 490.2 | 759.2 | 43 | 871.4 | 36.3 |
| 7 | P37/BTB/2025 | 22.15389 | 71.72778 | 140.9 | 250.4 | 24 | 251.6 | 10.8 |
| 8 | P38/BTB/2025 | 22.15557 | 71.7254 | 530.8 | 546.8 | 50 | 386.4 | 19.1 |
| 9 | CMT/011/02/2025 | 22.18014 | 71.74058 | 150.7 | 244.3 | 22 | 791.9 | 24.3 |
| 10 | P39/BTB/2025 | 22.15944 | 71.72222 | 266.3 | 604.5 | 98.9 | 857.3 | 31.6 |
| 11 | P77/BTB/2025 | 22.13575 | 71.7183 | 283.2 | 369.6 | 23.3 | 601.9 | 16.7 |
| 12 | CMT/011/60/2025 | 22.2073 | 71.7219 | 502.5 | 952.5 | 47.3 | 30.7 | 33.8 |
| 13 | P32/BTB/2025 | 22.16923 | 71.71355 | 164.8 | 484.1 | 37 | 410.1 | 18.4 |
| 14 | P33/BTB/2025 | 22.16806 | 71.71389 | 87.2 | 150.6 | 20.3 | 254.8 | 8.9 |
| 15 | P34/BTB/2025 | 22.1675 | 71.72167 | 88.1 | 144.3 | 20.3 | 259.3 | 9 |
| 16 | CMT/011/59/2025 | 22.2073 | 71.7219 | 668.6 | 823.5 | 53.3 | 26.4 | 30.6 |
| 17 | P35/BTB/2025 | 22.16139 | 71.72694 | 93.1 | 164 | 65.2 | 1084.3 | 27.1 |
| 18 | P87/BTB/2025 | 22.10028 | 71.68389 | 1025.3 | 918 | 81 | 418.4 | 19.3 |
| 19 | P67/BTB/2025 | 22.09417 | 71.70583 | 392.6 | 759.6 | 28.4 | 612.5 | 21.8 |
| 20 | CMT/011/03/2025 | 22.17953 | 71.74162 | 481.6 | 1036.2 | 48.9 | 665.2 | 34.9 |
| 21 | P61/BTB/2025 | 22.21417 | 71.71111 | 175.1 | 424.3 | 46.9 | 670.8 | 23.1 |
| 22 | CMT/011/61/2025 | 22.2073 | 71.71899 | 110.1 | 309.7 | 31.1 | 889.3 | 33.3 |
| 23 | CMT/011/41/2025 | 22.2073 | 71.7219 | 665.8 | 1040.3 | 53.9 | 724 | 31.8 |
| 24 | CMT/011/07/2025 | 22.17987 | 71.7418 | 88.3 | 67.7 | 24.3 | 1214.6 | 33 |
| 25 | P40/BTB/2025 | 22.14118 | 71.72764 | 315.3 | 406.9 | 111.2 | 921.7 | 27.4 |
|  |  |  |  | Ni | Cr | Co | Nb | Sc |
|  |  |  | Minimum | 87.2 | 67.7 | 20.3 | 25.9 | 8.9 |
|  |  |  | Maximum | 1025.3 | 1040.3 | 131 | 1214.6 | 36.3 |
|  |  |  | Mean | 338.38 | 492.584 | 48.56 | 574.088 | 24.032 |
|  |  |  |  | 15 PIT SAMPLES |  |  |  |  |
|  |  |  |  | 10 BED ROCK SAMPLES |  |  |  |  |
|  |  |  |  | TOTAL 25 SAMPLES FROM FLOW 1 |  |  |  |  |

##### FLOW 2: MASSIVE BASALT (Aphanitic nature)

  - Occurs in the elevation range of > 65m to 70 m. MSL

  - Fine grained and massive in nature with phenocrysts of olivine in it.

  - Fine grained basalt having the olivine crystals very small & are in aphanitic in nature, crystals in green color observed through hand lens. This basaltic flow is very hard and massive.

  - The flow 2 is overlaying on the flow 1 of the fine-grained and Amygdaloidal basalt. This basaltic flow is well exposed at the Khas village near canal road and located towards and North east -North west of the Botad block.

| ANALYTICAL VALUES (PPM) OF BED ROCK AND PIT SAMPLES COLLECTED FROM FLOW 2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No | Sample ID | Latitude | Longitude | Ni | Cr | Co | Nb | Sc |
| 1 | CMT/011/68/2025 | 22.09993 | 71.68057 | 140.3 | 253.9 | 29.6 | 35.6 | 32.1 |
| 2 | CMT/011/24/2025 | 22.12693 | 71.72386 | 23.6 | 47.2 | 3.8 | 4.4 | 3.4 |
| 3 | P44/BTB/2025 | 22.15197 | 71.69891 | 667.2 | 741.2 | 79.3 | 542.3 | 20 |
| 4 | P45/BTB/2025 | 22.1594 | 71.68962 | 121 | 130.7 | 62.6 | 1186.6 | 26.3 |
| 5 | CMT/011/52/2025 | 22.18248 | 71.71603 | 180.5 | 397.7 | 36.5 | 33.1 | 27.2 |
| 6 | CMT/011/50/2025 | 22.12656 | 71.66904 | 688.5 | 986.9 | 56.1 | 37.6 | 30.4 |
| 7 | PS7/BTB/2025 | 22.11806 | 71.67722 | 641 | 680.7 | 64.9 | 525.2 | 21.9 |
| 8 | CMT/011/66/2025 | 22.1266 | 71.66917 | 56.6 | 826.3 | 51.8 | 20 | 28.2 |
| 9 | CMT/011/57/2025 | 22.16211 | 71.71542 | 113.6 | 326.7 | 31.6 | 327.6 | 38.2 |
| 10 | P60/BTB/2025 | 22.20407 | 71.70715 | 309.5 | 387 | 144.7 | 842.9 | 26.6 |
| 11 | P54/BTB/2025 | 22.10361 | 71.67917 | 740.7 | 576.6 | 86.9 | 693.8 | 29.9 |
| 12 | PS5/BTB/2025 | 22.1225 | 71.6675 | 660.4 | 535.9 | 130.2 | 634.1 | 22.1 |
| 13 | P73/BTB/2025 | 22.13111 | 71.70556 | 563.1 | 810.4 | 44.1 | 883 | 31.5 |
| 14 | PS6/BTB/2025 | 22.12417 | 71.67111 | 602.8 | 557.4 | 148.4 | 707.8 | 23 |
| 15 | P74/BTB/2025 | 22.13226 | 71.70188 | 417.3 | 776.6 | 39.9 | 674.6 | 28.6 |
| 16 | CMT/011/51/2025 | 22.18248 | 71.71603 | 72.3 | 114.7 | 19.5 | 36.5 | 21.2 |
| 17 | CMT/011/44/2025 | 22.11475 | 71.65999 | 496.6 | 590 | 46.9 | 40.9 | 30 |
| 18 | P68/BTB/2025 | 22.09833 | 71.69167 | 933.4 | 728.7 | 76.2 | 415.2 | 20.3 |
| 19 | CMT/011/28/2025 | 22.12877 | 71.71457 | 349.1 | 482.3 | 34.6 | 310.2 | 26.1 |
| 20 | CMT/011/05/2025 | 22.19059 | 71.7305 | 520.9 | 868.2 | 48.8 | 835.8 | 34.4 |
| 21 | P81/BTB/2025 | 22.12278 | 71.71528 | 501.1 | 591.3 | 50.5 | 787 | 25.6 |
| 22 | P78/BTB/2025 | 22.12667 | 71.72361 | 432.1 | 757.3 | 39.7 | 945.6 | 33.1 |
| 23 | CMT/011/26/2025 | 22.12859 | 71.72004 | 538.8 | 514.4 | 48.5 | 830.5 | 28.3 |
| 24 | P79/BTB/2025 | 22.13167 | 71.71778 | 526.7 | 787.3 | 46.7 | 754.2 | 29.7 |
| 25 | CMT/011/69/2025 | 22.1274 | 71.71592 | 723 | 44.2 | 4.9 | 593.3 | 23.4 |
| 26 | P80/BTB/2025 | 22.12722 | 71.71583 | 530.3 | 791.5 | 45.6 | 643.1 | 30.3 |
| 27 | CMT/011/25/2025 | 22.12693 | 71.72386 | 28.9 | 971.4 | 59 | 479 | 12.2 |
| 28 | P90/BTB/2025 | 22.10974 | 71.67326 | 290.2 | 605.3 | 27.1 | 732.3 | 31.3 |
| 29 | P91/BTB/2025 | 22.11306 | 71.66694 | 873 | 804.9 | 65.5 | 633.2 | 20.9 |
| 30 | CMT/011/31/2025 | 22.20385 | 71.7045 | 889.6 | 911.5 | 60.4 | 576.7 | 26.6 |
| 31 | P85/BTB/2025 | 22.1125 | 71.67833 | 776.8 | 835.6 | 59.9 | 856.9 | 32.7 |
| 32 | PB6/BTB/2025 | 22.11142 | 71.67882 | 415.3 | 706.3 | 29.5 | 681.7 | 26.7 |
| 33 | CMT/011/15/2025 | 22.11544 | 71.66057 | 144.5 | 886.3 | 57.3 | 742.8 | 20.5 |
| 34 | P89/BTB/2025 | 22.10194 | 71.6725 | 467.6 | 640.8 | 37.7 | 943 | 31.3 |
| 35 | BRS94/BTB/2025 | 22.19417 | 71.71944 | 270.5 | 431.4 | 33.4 | 39850 | 29.1 |
| 36 | CMT/011/04/2025 | 22.12877 | 71.71457 | 43.2 | 144.8 | 28.4 | 249.4 | 6.9 |
| 37 | CMT/011/05/2025 | 22.19059 | 71.7305 | 894.4 | 994.5 | 67.9 | 645.8 | 30.3 |
| 38 | CMT/011/56/2025 | 22.13442 | 71.70184 | 969.3 | 946.3 | 80.3 | 578.3 | 29.5 |
| 39 | P22/BTB/2025 | 22.17 | 71.71083 | 383.8 | 656.5 | 30.8 | 771.9 | 249 |
| 40 | CMT/011/06/2025 | 22.19059 | 71.7305 | 294.5 | 719.5 | 47.7 | 1085.1 | 33.3 |
| 41 | CMT/011/55/2025 | 22.13442 | 71.70185 | 368.1 | 521.2 | 39.8 | 907.6 | 29.9 |
| 42 | P43/BTB/2025 | 22.14391 | 71.7055 | 313.5 | 473.3 | 120.9 | 984.5 | 28.9 |
| 43 | CMT/011/49/2025 | 22.09569 | 71.69467 | 442.6 | 685.8 | 47 | 878.8 | 33.6 |
| 44 | CMT/011/27/2025 | 22.12877 | 71.71457 | 196.4 | 458.7 | 38.9 | 32 | 25.2 |
|  |  |  |  | Ni | Cr | Co | Nb | Sc |
|  |  |  | Min | 23.6 | 44.2 | 3.8 | 4.4 | 3.4 |
|  |  |  | Max | 969.3 | 994.5 | 148.4 | 39850 | 249 |
|  |  |  | Mean | 445.7409 | 606.8 | 54.63182 | 1476.589 | 31.58409 |
|  |  |  |  | 21 PIT SAMPLES |  |  |  |  |
|  |  |  |  | 23 BED ROCK SAMPLES |  |  |  |  |
|  |  |  |  | 44 TOTAL SAMPLES FROM FLOW 2 |  |  |  |  |

##### FLOW 3: PICRITIC BASALT

  - The Flow 3 occurs in the elevation range of 80 to 90 m MSL

  - Most of the places it is vesicular basalt which are porphyritic in nature.

  - High amount of olivine crystals, which constitute more than 30% and the olivine crystals are in green, red, yellow.

  - The olivine crystals which are partially to highly altered. Calcite venations are very prominent in this flow, besides few secondary minerals like calcite, zeolite is also noticed at several places.

  - The olivine which seems to have altered to Iddingsite which is red in color at. In this particular flow.

Photograph 13: showing Flow 3 and flow 4 and in between there is a red bole bed

Photograph 13: Red bole bed emplaced between flow 3 and flow 4 basaltic flows

Photograph 14: Zeolite

Photograph 14: Zeolite

| ANALYTICAL VALUES (PPM) OF BED ROCK AND PIT SAMPLES COLLECTED FROM FLOW 3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No | Sample ID | Latitude | Longitude | Ni | Cr | Co | Nb | Sc |
| 1 | P49/BTB/2025 | 22.13609 | 71.66419 | 262.7 | 506.3 | 78.3 | 955.2 | 32.8 |
| 2 | P50/BTB/2025 | 22.11444 | 71.65889 | 355.9 | 522.1 | 117.7 | 837 | 30.2 |
| 3 | PS1/BTB/2025 | 22.11639 | 71.66 | 320.9 | 457.1 | 115.6 | 834.3 | 30.1 |
| 4 | CMT/011/35/2025 | 22.19193 | 71.71452 | 308.6 | 736.9 | 45.5 | 26.4 | 33.9 |
| 5 | P46/BTB/2025 | 22.14944 | 71.67194 | 420 | 500.4 | 102.3 | 783.6 | 22.5 |
| 6 | P47/BTB/2025 | 22.14667 | 71.67333 | 435.5 | 823 | 126.8 | 878.4 | 25.4 |
| 7 | CMT/011/65/2025 | 22.14014 | 71.66176 | 46.5 | 20.9 | 29.3 | 42.4 | 17.1 |
| 8 | P58/BTB/2025 | 22.12056 | 71.6775 | 919.6 | 771.5 | 99 | 351 | 17.7 |
| 9 | P59/BTB/2025 | 22.13364 | 71.67476 | 732.5 | 609.1 | 81.9 | 553.5 | 23.2 |
| 10 | PS2/BTB/2025 | 22.11667 | 71.66083 | 303.7 | 391.9 | 103.3 | 711.8 | 25.8 |
| 11 | CMT/011/33/2025 | 22.19382 | 71.71593 | 529.8 | 816.7 | 45.7 | 34.7 | 35.3 |
| 12 | P65/BTB/2025 | 22.1189 | 71.69273 | 1025.4 | 853.4 | 81.5 | 437.4 | 21 |
| 13 | P66/BTB/2025 | 22.12139 | 71.69778 | 1024.6 | 966.9 | 77.6 | 455.6 | 20.4 |
| 14 | P75/BTB/2025 | 22.13528 | 71.70056 | 857.1 | 968.5 | 59.9 | 575.9 | 25.4 |
| 15 | P76/BTB/2025 | 22.13444 | 71.69722 | 830.5 | 842.3 | 60.5 | 707.4 | 27.6 |
| 16 | P69/BTB/2025 | 22.12398 | 71.69963 | 989.3 | 1218.7 | 68.1 | 573.8 | 23.8 |
| 17 | P70/BTB/2025 | 22.12417 | 71.70139 | 1073.5 | 1162.4 | 74.8 | 493.8 | 27.6 |
| 18 | P62/BTB/2025 | 22.11389 | 71.68667 | 647.2 | 645.7 | 60 | 602.8 | 27.9 |
| 19 | P71/BTB/202 | 22.12778 | 71.70167 | 676.8 | 1282.1 | 50.9 | 794.9 | 30.6 |
| 20 | P63/BTB/2025 | 22.11667 | 71.68639 | 645.6 | 532.9 | 52.1 | 667.9 | 28.2 |
| 21 | P72/BTB/202 | 22.12806 | 71.69917 | 633 | 991.9 | 47.3 | 738.7 | 29.5 |
| 22 | CMT/011/53/2025 | 22.16424 | 71.68434 | 1056.2 | 1052.3 | 69.8 | 544.2 | 27.2 |
| 23 | P64/BTB/2025 | 22.11889 | 71.68806 | 989.4 | 825.8 | 71.9 | 444.7 | 20 |
| 24 | P83/BTB/2025 | 22.14015 | 71.66178 | 251.7 | 515.1 | 43.1 | 747.3 | 22.2 |
| 25 | CMT/011/23/2025 | 22.13491 | 71.66442 | 539.7 | 638.2 | 46.8 | 631 | 32.7 |
| 26 | P10/BTB/2025 | 22.17417 | 71.69003 | 543.7 | 691.5 | 43.7 | 840.4 | 292.3 |
| 27 | P84/BTB/2025 | 22.13592 | 71.67362 | 179.3 | 254 | 34.7 | 497.7 | 34.5 |
| 28 | CMT/011/54/2025 | 22.13239 | 71.70182 | 247.1 | 515.1 | 33 | 1057.7 | 32.1 |
| 29 | P17/BTB/2025 | 22.18272 | 71.69231 | 412.9 | 539.7 | 33.6 | 1207.6 | 273.7 |
| 30 | P92/BTB/2025 | 22.11861 | 71.65278 | 805.5 | 758.4 | 74.3 | 430.4 | 25.9 |
| 31 | P93/BTB/2025 | 22.11806 | 71.6475 | 866.1 | 936.2 | 65.7 | 741.9 | 18.1 |
| 32 | P19/BTB/2025 | 22.18091 | 71.702 | 757.1 | 729.6 | 51.5 | 569.9 | 278 |
| 33 | CMT/011/39/2025 | 22.18454 | 71.70008 | 800 | 1028.6 | 53.2 | 694.5 | 30.5 |
| 34 | CMT/011/73/2025 | 22.19294 | 71.71702 | 550 | 678.3 | 51 | 674.1 | 34.7 |
| 35 | P25/BTB/2025 | 22.16642 | 71.67894 | 371.5 | 564.7 | 27.7 | 684.3 | 217.9 |
| 36 | P26/BTB/2025 | 22.16239 | 71.67985 | 351.9 | 530.2 | 33.3 | 1164.8 | 309.3 |
| 37 | P27/BTB/2025 | 22.16441 | 71.68432 | 179.1 | 424.8 | 28.8 | 1004.9 | 260.7 |
| 38 | P20/BTB/2025 | 22.17875 | 71.69849 | 751.8 | 770.1 | 52.1 | 455.9 | 239.7 |
| 39 | BRS95/BTB/2025 | 22.19028 | 71.72028 | 244.8 | 382.6 | 32.1 | 905 | 28.7 |
| 40 | P21/BTB/2025 | 22.17863 | 71.69358 | 195.1 | 340.3 | 24.3 | 848.5 | 226 |
| 41 | CMT/011/37/2025 | 22.1848 | 71.71879 | 246.5 | 355.7 | 32 | 1001.5 | 33.7 |
| 42 | CMT/011/11/2025 | 22.12601 | 71.69898 | 446.8 | 505.1 | 42 | 818.5 | 35.4 |
| 43 | CMT/011/32/2025 | 22.19416 | 71.71169 | 614.2 | 483.9 | 38.3 | 865.3 | 36.2 |
| 44 | CMT/011/09/2025 | 22.11637 | 71.70405 | 542.6 | 922.3 | 70.9 | 843.9 | 34.9 |
| 45 | P31/BTB/2025 | 22.12089 | 71.69607 | 746.9 | 1136.1 | 51.5 | 666.1 | 26.7 |
| 46 | CMT/011/36/2025 | 22.19193 | 71.71452 | 123.1 | 159.1 | 27 | 1143.8 | 30.4 |
| 47 | CMT/011/34/2025 | 22.19274 | 71.71526 | 35.4 | 12.4 | 2.9 | 15.6 | 1.2 |
| 48 | CMT/011/38/2025 | 22.18454 | 71.72228 | 194.7 | 326.8 | 32.2 | 33.2 | 34.2 |
|  |  |  |  | Ni | Cr | Co | Nb | Sc |
|  |  |  | Minimum | 35.4 | 12.4 | 2.9 | 15.6 | 1.2 |
|  |  |  | Maximum | 1073.5 | 1282.1 | 126.8 | 1207.6 | 309.3 |
|  |  |  | Mean | 543.3708 | 660.3667 | 57.19792 | 658.1083 | 66.51875 |
|  |  |  | 32 PIT SAMPLES |  |  |  |  |  |
|  |  |  | 16 BED ROCK SAMPLES |  |  |  |  |  |
|  |  |  | TOTAL 48 SAMPLES FROM FLOW 3 |  |  |  |  |  |

##### FLOW 4: PORPHYRITIC OLIVINE BASALT

  - In this particular flow one can see massive olivine basalt having porphyritic texture, where olivine crystals in green to red, having high picritic nature. Olivine phenocrysts are abundant in volume in comparison to the other three flows in the study area hence it is considered as picritic basalt. The outcrops are located near Arto Road, Amapar village and Senthali. Compared to all the basaltic flows, this flow 4 is very prominent and having higher thickness. Several stone Quarry’s for building stones are reported in this flow.

Photograph 15: Red bole bed at ARTO Botad

Photograph 15: showing Red bole bed Between Flow3 and Flow4

Photograph 16: Picritic Quarry area along Sherthali village

Photograph 16: showing Quarrie area picritic basalt.

| ANALYTICAL VALUES (PPM) OF BED ROCK AND PIT SAMPLES COLLECTED FROM FLOW 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No | Sample ID | Latitude | Longitude | Ni | Cr | Co | Nb | Sc |
| 1 | CMT/011/17/2025 | 22.14825 | 71.664815 | 19.9 | 101.9 | 9.7 | 10 | 2.8 |
| 2 | P6/BTB/2025 | 22.17784 | 71.685027 | 160 | 218.4 | 20.6 | 933.8 | 250.1 |
| 3 | CMT/011/43/2025 | 22.17936 | 71.688045 | 70.6 | 69.9 | 9.7 | 35.6 | 14.5 |
| 4 | P7/BTB/2025 | 22.17939 | 71.68802 | 434.7 | 658.2 | 34.3 | 1097.1 | 317.7 |
| 5 | CMT/011/20/2025 | 22.14564 | 71.665273 | 429.5 | 730.6 | 46.4 | 809.9 | 31 |
| 6 | P16/BTB/2025 | 22.18492 | 71.692865 | 653.5 | 1156.9 | 47.9 | 788.4 | 329 |
| 7 | CMT/011/21/2025 | 22.14564 | 71.665273 | 190.5 | 238 | 26 | 761.7 | 29.5 |
| 8 | P18/BTB/2025 | 22.18188 | 71.695478 | 260.3 | 271.2 | 27.5 | 1074.9 | 259.9 |
| 9 | CMT/011/12/2025 | 22.14632 | 71.679646 | 111.6 | 163.9 | 11.6 | 1023.1 | 26.7 |
| 10 | P12/BTB/2025 | 22.1708 | 71.689153 | 449.8 | 446.5 | 40 | 796.4 | 251.4 |
| 11 | P13/BTB/2025 | 22.19621 | 71.6982 | 598 | 507.7 | 64.1 | 661 | 202.6 |
| 12 | P48/BTB/2025 | 22.14589 | 71.679947 | 393.2 | 813.5 | 86.4 | 767.9 | 31.5 |
| 13 | P14/BTB/2025 | 22.19256 | 71.692855 | 285.8 | 344.4 | 17.6 | 252.3 | 117 |
| 14 | P15/BTB/2025 | 22.18726 | 71.691693 | 404.4 | 606.2 | 37.6 | 747.6 | 328 |
| 15 | CMT/011/19/2025 | 22.14564 | 71.66527 | 629.6 | 1312.3 | 69.1 | 21.4 | 35.4 |
| 16 | P24/BTB/2025 | 22.1688 | 71.684002 | 497.2 | 715.8 | 39.3 | 1104.6 | 323.8 |
| 17 | P82/BTB/2025 | 22.14528 | 71.665278 | 300.4 | 466.4 | 31.6 | 1024.3 | 30.3 |
| 18 | CMT/011/45/2025 | 22.18084 | 71.688229 | 581.3 | 903.5 | 52.4 | 680.1 | 31.1 |
| 19 | CMT/011/77/2025 | 22.14634 | 71.680033 | 158.7 | 391.9 | 38.3 | 40.1 | 39.5 |
| 20 | P1/BTB/2025 | 22.1718 | 71.68595 | 297.2 | 468 | 31.2 | 1262.6 | 291 |
| 21 | P2/BTB/2025 | 22.1753 | 71.686443 | 405.4 | 374.3 | 32 | 1188.7 | 287.4 |
| 22 | P3/BTB/2025 | 22.17594 | 71.686137 | 315.3 | 433.4 | 29.9 | 1219.6 | 299.6 |
| 23 | P23/BTB/2025 | 22.16953 | 71.685037 | 468.1 | 735.5 | 38.4 | 1020.4 | 307.6 |
| 24 | P28/BTB/2025 | 22.17548 | 71.681255 | 425.2 | 765.9 | 36.9 | 1053.2 | 312.2 |
| 25 | P29/BTB/2025 | 22.18403 | 71.695361 | 561.9 | 0.25 | 438.6 | 1000.1 | 313.2 |
| 26 | CMT/011/22/2025 | 22.14564 | 71.665273 | 82.2 | 214.1 | 65.3 | 1098.3 | 32 |
| 27 | P30/BTB/2025 | 22.16865 | 71.685003 | 468.4 | 852.7 | 39.3 | 1051.4 | 314.6 |
| 28 | CMT/011/46/2025 | 22.17426 | 71.689048 | 407 | 600.3 | 43.8 | 965.2 | 30.5 |
| 29 | CMT/011/42/2025 | 22.18504 | 71.692137 | 659.7 | 1015 | 52.6 | 726 | 36.5 |
| 30 | CMT/011/63/2025 | 22.17705 | 71.68412 | 795.2 | 404.5 | 33.7 | 585.3 | 25.5 |
| 31 | P8/BTB/2025 | 22.17983 | 71.688527 | 223.5 | 452 | 32.2 | 838.5 | 232.5 |
| 32 | P9/BTB/2025 | 22.18217 | 71.687272 | 511.3 | 377.8 | 48.1 | 536.2 | 178.8 |
| 33 | P11/BTB/2025 | 22.17224 | 71.689718 | 346.3 | 399.1 | 34.8 | 1059.3 | 261.3 |
| 34 | P4/BTB/2025 | 22.17693 | 71.686795 | 139.5 | 122.2 | 23 | 1395.9 | 316.9 |
| 35 | CMT/011/18/2025 | 22.14825 | 71.664815 | 194.7 | 46.7 | 3.7 | 5 | 2.1 |
| 36 | P5/BTB/2025 | 22.17802 | 71.686268 | 209.3 | 269.8 | 18.2 | 660.4 | 178.3 |
|  |  |  |  | Ni | Cr | Co | Nb | Sc |
|  |  |  | MINIMUM | 19.9 | 0.25 | 3.7 | 5 | 2.1 |
|  |  |  | MAXIMUM | 795.2 | 1312.3 | 438.6 | 1395.9 | 329 |
|  |  |  | MEAN | 114.6 | 135.025 | 10.95 | 332.7 | 90.2 |
|  |  |  |  |  | 23 PIT SAMPLES |  |  |  |
|  |  |  |  |  | 13 BED ROCK SAMPLES |  |  |  |
|  |  |  |  |  | 36 TOTAL SAMPLES |  |  |  |

Lithological Map of the Botad Block

PLATE NO-9

## 6.1.4   Outcrop Map of the Botad Block:

Outcrop map of the botad block showing different variants of basalts- Amygdaloidal basalt, olivine basalt, picritic basalt, fine grained olivine basalt, massive olivine basalt.

Plate no 10

## 6.1.5   Description of the rock types:

There are different types of basaltic flows found in the botad block. These includes-Porphyritic olivine- bearing picrite basalt, porphyritic basalt, fine-grained basalt, sandstone, tuff, ash, and green, black, and red bole beds (Deccan) are among the significant rock types in the region.

##### Picritic Basalt:

Photograph 17: Quarrie area showing picritic basalt.

Photograph 17 : showing Quarrie area of picritic basalt near Senthali village.

Photograph 18: Picrite sample along khas road

Photograph 18: showing picritic basaltic sample near the village Sen  thali.

To classify it as a picritic basalt, it should have-

  - 12 wt% MgO

  - 52 wt% SiO2

  - Alkali content (Na2O+K2O): 3%

The eastern, northern, and central regions of the region are dominated by picrite basalt. Few out crops are also noticed to northeast of Paliyad Road. Other significant locations of well exposures of Picritic basalt include near Botad, Sherthali, Samadhiyala No1, Khas Road, Bhambhan, Amapar road.

Two of the four demarcated flows that are found in the area are picritic in origin. The thickness of these varies. In the Botad area, there is a rise in thickness. Megascopically, the rock

is enormous, dark grey to greenish black, and contains olivine phenocrysts that range in color

from olive green to bottle green as well as varying numbers of yellowish aged peanut-sized nodules. Sometimes the same rock contains both fresh and significantly changed forms of olivine.

The rock's worn surface has a faint pitted appearance because some of the olivine phenocrysts have been removed. Large phenocrysts of pyroxenes, measuring one to two and a half centimeters, are found in picrite basalt. Most of these picritic basalts are vesicular in nature and most olivine are in green, yellow, red. The olivine got altered to Iddingsite.

##### Porphyritic olivine Basalt:

Photograph 19:  Olivine basalt along Tajpar near sub station

Photograph 19: showing porphyritic olivine basalt located near Tajpar village.

Photograph 20: showing olivine basalt

located	near	Tajpar

Porphyritic olivine basalt is a mafic volcanic rock distinguished by conspicuous olivine phenocrysts set within a fine-grained to microcrystalline groundmass of plagioclase and clinopyroxene. The porphyritic texture indicates a two-stage cooling history: initial slow cooling at depth allowing olivine crystals to grow, followed by rapid cooling during eruption, producing the fine groundmass. Olivine phenocrysts are typically green to yellow, rounded to subhedral, and may show alteration rims of iddingsite. The rock is dark grey to black, massive, and may contain minor vesicles. Groundmass textures are typically intergranular to intersertal.

Under Microscope, it is observed that-

1.  Sample no: CMT/011/05/2025

Mineral assemblage: Porphyritic phases: Clinopyroxene + Olivine Matrix phases: Clinopyroxene + Plagioclase + Magnetite + pyrite

Texture: The rock represents fine grained porphyritic basalt having inequigranular texture. Large olivine and clinopyroxene phenocrysts occur within very fine-grained basaltic matrix showing intergranular texture. Phenocrysts are angular to subhedral shaped, often clusters of phenocrysts of olivine define glomeroporphyritic texture. Olivine phenocrysts are identified by their high refractive index, colorless, fractured nature and by high birefringence color. Clinopyroxene phenocrysts are identified by high refractive index (< than olivine), one set of perfect cleavage with inclined extinction. Often basal section with two sets of perfect cleavage at 90 angle is also noted. Exsolution lamellae and zoning are the common features of clinopyroxene phenocrysts. Thin brownish colored rim representing alteration zones were formed over clinopyroxene and olivine. Often fusing of two phenocrysts are also observed representing high temperature crystallization. The matrix consists of very fine-grained lath shaped plagioclase occurring randomly. Very small clinopyroxenes are present in the interstitial spaces of plagioclase laths defining intergranular texture. Very small magnetite and very few pyrite occur as dissemination within the matrix.

Photomicrograph 1: showing large phenocrysts of olivine and clinopyroxene in very fine basaltic matric showing porphyritic texture; under plane polarized light; 2X magnification

Photomicrograph 2: showing large phenocrysts of olivine and clinopyroxene in very fine basaltic matric showing porphyritic texture; under cross polarized light.

Photomicrograph 3: showing Clusters of phenocrysts within basaltic matrix showing Glomeroporphyritic texture; under cross polarized light at 2X magnification.

Photomicrograph 4: showing A closer view of Clusters of phenocrysts of olivine and Cpx within basaltic matrix showing Glomeroporphyritic texture; under cross polarized light at 5X magnification.

Photomicrograph 5: showing Bimodal distribution of phenocrysts size; very sharp-edged phenocrysts of Olivine in a fine-grained basaltic matrix

Photomicrograph 6: showing Fractured olivine in plagioclase-pyroxene rich matrix; small magnetites and rutiles are present in the matrix; under 10X magnification

Photomicrograph 7: showing Intergranular texture defined by plagioclase laths and clinopyroxenes in basalt; magnetites are the only opaques present in the rock; under 10X magnification

Photomicrograph 8: showing Olivine phenocrysts replaced by late biotite along fractures.

Photomicrograph 9: showing Small anhedral magnetites disseminated in matrix; under reflected light and 20X magnification

Photomicrograph 10: showing Small anhedral magnetites disseminated in matrix; under reflected light and 20X magnification

##### Fine Grained Olivine Basalt:

Photograph 21: Fine grained olivine basalt along Khas road

Photograph 21: showing fine-grained olivine basalt in sherthali village

Photograph 22: Fine grained olivine basalt along Khas road

Photograph 22: showing fine-grained olivine basalt in khas village.

Fine-grained olivine basalt is a mafic volcanic rock composed predominantly of plagioclase and clinopyroxene, with dispersed fresh or altered olivine phenocrysts. The groundmass is fine-grained due to rapid cooling of basaltic lava at or near the surface, producing a compact, dark grey to black matrix. The fine-grained olivine basalt is observed along few villages named Samadhiyala, khas, shenthali, Tajpar road.

The texture is typically massive, though minor vesicles may occur. Weathering commonly results in reddish or brown alteration halos around olivine grains due to iddingsite formation.

##### Amygdaloidal Basalt:

Amygdaloidal basalt is a fine-grained volcanic rock characterized by the presence of rounded to oval mineral-filled cavities known as amygdales. These cavities originate as gas bubbles (vesicles) trapped within basaltic lava during eruption and cooling. After solidification, circulating hydrothermal fluids deposit secondary minerals into the vesicles, converting them into amygdales. Typical infilling minerals include calcite, zeolites, chlorite.

      - This amygdaloidal basalt are found along the northern side of the block and mostly occur along the canal road of Khas village.

Photograph 23: Amygdaloidal basalt along khas canal road

Photograph 23: showing Amygdaloidal Basalt along Khas road village.

##### Massive Basalt:

Massive basalt is dark grey to black, fine-grained. Megascopically, it appears dense, compact, and hard, with an aphanitic texture in which individual mineral grains are not visible to the naked eye. The rock shows a massive structure, lacking vesicles, cavities, or layering, which distinguishes it from vesicular or amygdaloidal varieties of basalt. Fresh surfaces are typically dark and compact. The main mineral constituents, include plagioclase feldspar and pyroxene.

This massive basalt predominantly found in the Flow 2 of the Botad block. Olivine is conspicuously absent in this variant of basalt. Vertical to sub-vertical joints are present.

Photograph 24: Massive basalt along khas road

Photograph 24: showing massive basalt along Northern part of the block, Khas village.

Photograph 25: Massive basalt along Bhambhan road

Photograph 25: showing massive basalt along Northern part of the block, Khas village.

##### Red bole bed:

Red bole beds are thin, reddish to brick-red colored horizons occur between successive lava flows. Megascopically, the rock appears soft, fine-grained, earthy and compact, with a clayey texture. It is usually laminated. The color ranges from red, reddish brown to chocolate brown due to the presence of iron oxides formed during weathering. Red bole beds in the study area composed of clay minerals along with fine volcanic ash (?) and altered underlying basaltic material. The thickness of red bole beds usually varies from a few centimeters to about a meter, Geologically, red bole beds in the Botad area are significant as they indicate pauses in volcanic activity and provide important stratigraphic markers for correlating individual lava flows within the basalt sequence.

Photograph 26: Red bole bed along Bhambhan road

Photograph 26: showing Red bole bed at Samadhiyala -1 village

Photograph 27: Red bole bed along ARTO road

Photograph 27:showing Red bole bed at ARTO road, Botad.

The red bole bed locations include RTO road, Samadiyala 1, Khas Road, Amapar road, Tajpar road near substation,

##### Serpentization:

During the course of mapping, serpentinization in basalt and olivine bearing basalt/picrite is observed as an alteration process affecting the original mafic minerals, particularly olivine and pyroxene. This may be due to the action of hydrothermal fluids emplaced later. The affected rocks usually show a distinct change in colour from the original dark grey or black basalt or picrite to green, dark green, or greenish-black shades. The rock surface looks mottled, patchy, or streaked, indicating partial to intense alteration. In picritic basalts, where olivine is abundant, serpentinization is more prominent and the olivine crystals are often replaced by soft, green serpentine minerals. The rock may become comparatively softer and sometimes displays a smooth or slightly waxy feel on freshly broken surfaces.  Serpentinites zones mostly noticed along fractures, joints.  At places, thin veinlets and patches of serpentine minerals fill cracks and cavities, and small magnetite grains may also be present as a by-product of the alteration process.

Photograph 28: serpentine along Bhambhan road

Photograph 28: showing serpentization along Bhambhan village.

Photograph 29:  serpentine along Bhambhan road

Photograph 29: showing serpentization and fibrous textures along Bhambhan village.

Serpentization: Hydration of ultramafic rocks (rich in olivine and pyroxenes) A GREENISH TINT

FIBROUS TEXTURES

VOLUME EXPANSION FRACTURES AND HYDRATION VEINS

- This serpentization is found along the low-lying areas which is located along southwest of the block, near to the streams of the Botad block.

##### Massive Basaltic Dyke:

A massive basalt dyke occurs at the contact between Flow 3 and Flow 4, both identified as picritic basalts.

The dyke is medium- to fine-grained, dark grey to black, and shows a dense, non-vesicular, sharply bounded intrusive fabric. Its massive character and cross-cutting relation indicate emplacement after the solidification of Flow 3 but before or during the early stage of Flow 4 formation.

Flow 3 exhibits calcite-filled fractures and venations, reflecting post-emplacement hydrothermal circulation and low-temperature alteration. The dyke truncates or offsets these calcite veins, confirming its younger intrusive age relative to the alteration event in Flow 3.

Flow 4, also picritic, lies above the dyke and does not show similar vein development, suggesting either limited fluid access or later sealing by the dyke activity.

Photograph 30: Massive basaltic dyke along Tajpar road

Photograph 30: There is a massive basaltic dyke (Vertical) between flow 3 and flow 4 along Tajpar village.

### Quarry:

In the western part of the Botad block, different Quarry near Sherthali and ARTO road are observed. Most of these Quarry located in the Flow 4 which is of picritic composition and having the elevation around 95 to 110 m MSL. Two sets of vertical to sub vertical joints and one sub horizontal joint set are very common in these Quarry. The olivine porphyritic crystals are very common these basalts exposed in the quarry faces. Calcite veins are present along the fractured zones. More than 10 Quarry are located in the Flow 4. The thickness of flow 4 is more compared to other flows. The quarry consists of fracture zones where the fractures are filled with calcite minerals.

These are few photographs of Quarrys taken in the field

ge 15

Few Photographs showing Quarry Area locations:

Quarry-1

Photograph 31: showing Quarrie 1 outcrop location along senthali village, having the elevation of 110m, and having the height of 3m. It is a picritic basalt and there is a calcite venation filling in the outcrop.

Joints:

Strike

Dip amount Dip Direction

2100

300

1200

Quarrie-3

Photograph 33: showing Quarry 3 outcrop location along senthali village, having the elevation of 97m. It is a picritic basalt and there is a calcite venation filling in the outcrop. There are vertical joints are present.

Joints:

Strike

2600

Dip direction

Dip amount

1700

200

Photograph 32: showing Quarry 2 outcrop location along Senthali village, having the elevation of 110m, and having the height of 2.5 m. It is a picritic basalt and there is a calcite venation filling in the outcrop. Vertical to sub-vertical joints can be seen.

Quarry-2

##### Quarry-4

Photograph 34: showing Quarry4 outcrop location along Senthali village, having the elevation of 105m. It is a picritic basalt and there is a calcite venation filling in the outcrop. There are vertical joints are present. Here, there is a water bodies in depression areas.

Photograph 35: showing Quarry 5 outcrop location along Senthali village, having the elevation of 93m. It is a picritic basalt and there is a calcite venation filling in the outcrop. There are vertical joints are present. It is having the height of 12m.

Joints:

Strike

Dip direction Dip amount

3400

2500

100

Quarry-5

Quarry-6

Photograph 36: showing Quarry 6 outcrop location along Senthali village, having the elevation of 111m. It is a picritic basalt and there is a calcite venation filling in the outcrop. There are vertical joints are present. Here, there is a water bodies in depression.

### 6.1.6  PETROGRAPHIC STUDIES:

### petrographic studies (Thin sections):

- Sample code: CMT/011/7

Microscopic study reveals that the rock exhibits a fine- to medium-grained dominated by plagioclase (Plg) and clinopyroxene (Cpx) as major mineral constituents. The overall texture indicates a hypidiomorphic to sub ophitic intergrowth between plagioclase laths and interstitial clinopyroxene, suggesting a crystallization sequence typical of mafic to intermediate igneous rocks.

Plagioclase occurs as lath-shaped to tabular crystals exhibiting polysynthetic twinning under crossed nicols. The grains display moderate to high birefringence colors, implying a composition within the labradorite to andesine range. Clinopyroxene appears as subhedral to anhedral grains with high interference colors and distinct cleavage traces at nearly 90°, characteristic of augite. Some grains exhibit alteration along cleavage planes and marginal chloritization, indicating incipient secondary alteration. groundmass shows a fine interlocking mosaic of plagioclase microlites and subordinate clinopyroxene, representing rapid crystallization from a melt. In some portions, plagioclase microlites form a trachytic texture, whereas in others, an intergranular to intersertal texture is observed with plagioclase and clinopyroxene forming the main constituents of the matrix. The dominance of plagioclase microlites over clinopyroxene in parts of the sample reflects variable cooling rates during the final stages of solidification.

The mineral assemblage and textures collectively indicate a magmatic origin, likely from a basaltic magma, crystallized under subvolcanic to shallow intrusive conditions. The fine-grained groundmass and presence of euhedral to subhedral phenocrysts of plagioclase and clinopyroxene suggest that the rock represents a basaltic lava that experienced partial crystallization before eruption.

Photomicrograph 11: showing Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) in fine grained ground mass under transmitted light XPL (5X).

Photomicrograph 12: showing Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) phenocrysts in fine grained ground mass under transmitted light XPL (5X).

Photomicrograph 13: showing Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) phenocrysts in fine grained ground mass under transmitted light XPL (2X).

Photomicrograph 14: showing Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) in fine grained ground mass under transmitted light XPL (20X)

Rock Name: Based on the mineral and textural characteristics, it is a porphyritic basalt.

- Sample code: CMT/011/64

Microscopic study reveals that the rock exhibits a fine- to medium-grained characterized by the predominance of clinopyroxene (Cpx) and plagioclase (Plg) as the major mineral constituents (Fig. 2A-2D). The overall texture indicates a sub ophitic to intergranular relationship between plagioclase laths and interstitial clinopyroxene.

Clinopyroxene occurs as large, subhedral to anhedral grains with high first- to second-order interference colors, showing well-defined cleavages intersecting nearly at 90°. The grains occasionally display faint zoning and marginal alteration to chlorite or opaque minerals along cleavage planes, suggesting minor post-crystallization alteration. Some clinopyroxene crystals exhibit fracturing and inclusions of fine plagioclase microlites, indicating simultaneous growth during early magmatic crystallization.

Plagioclase is abundant in the groundmass, forming lath-shaped microlites and interlocking textures with clinopyroxene. Under crossed nicols, it displays characteristic polysynthetic twinning and low birefringence colors. The plagioclase groundmass is generally fine-grained, reflecting rapid cooling of the residual melt. In some areas, plagioclase and clinopyroxene coexist as an intergranular mosaic, while in others, plagioclase dominates the matrix forming a trachytic texture.

The textural relations suggest crystallization began with clinopyroxene, followed by the growth of plagioclase during the later stages of solidification. The absence of olivine and opaque mineral abundance indicates a moderately evolved basaltic magma.

Photomicrograph 15: showing Photo micro               graph showing presence of Clinopyroxene     (Cpx) phenocrysts in fine grained ground mass under transmitted light XPL (2X)

Photomicrograph 16: Fig showing

Photomicrograph showing presence of

Clinopyroxene (Cpx) phenocrysts in fine grained

ground mass under transmitted light XPL (2X)

Photomicrograph 17: showing Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) in fine grained ground mass under transmitted light XPL (10X)

Photomicrograph 18: showing Photomicrograph showing presence of plagioclase (Plg) and Clinopyroxene (Cpx) in fine grained ground mass under transmitted light XPL (10X)

Rock/Mineral Name: Based on the mineral and textural characteristics, it is a porphyritic basalt.

- Sample no: CMT/011/10/2025 Mineral assemblage:

Phenocrystic phases: Olivine + Clinopyroxene

Matrix phases: Clinopyroxene + Plagioclase + Magnetite Secondary mineral: Serpentine + biotite

Texture: It is a very fine-grained rock showing inequigranular texture. Large, rounded, euhedral to subhedral phenocrysts of olivine and smaller clinopyroxene phenocrysts occur within very fine-grained basaltic matrix. Oscillatory compositional zoning and large size of the phenocrysts of olivine indicate early and slow magmatic crystallization with. Olivine phenocrysts have thin rim of fractures as well as growth rims containing tiny inclusions of another mineral. Often phenocrysts of olivine form clusters defining glomeroporphyritic texture.

Twin crystals, undulous extinctions are also observed among the phenocrysts. Small olivine phenocrysts developed over larger olivine crystal indicating change in rate of crystallization. Biotite and serpentine replace olivine along the fractures. Clinopyroxene phenocrysts are relatively small in size than olivine representing late crystallization.

The matrix consists of lath shaped plagioclase and clinopyroxene defining intergranular texture. Presence of microlites of feldspar also define the hyalopilitic texture indicating crystallization from volcanic glass. Often at places, the grain size of the matrix became coarser. Small Magnetite occur as disseminated grains within the matrix but coarser grains are closely associated with olivine phenocrysts indicating synchronous crystallization.

Photomicrograph 19: showing Olivine and clinopyroxene phenocrysts in very fine-grained plagioclase-pyroxene rich basaltic matrix; porphyritic texture; also note the fusion of two phenocrysts; under cross polarized light and at 2X magnification

Photomicrograph 20: showing Compositional zoning in olivine phenocrysts; note the plagioclase rich matrix; under 20X magnification

Photomicrograph 21: showing Bimodal distribution of sizes of phenocrysts; euhedral phenocrysts often broken and subsequently filled up by basaltic crystallization; 2X magnification

Photomicrograph 22: showing Glomeroporphyritic texture defined by clusters of olivine crystals in a basaltic matrix; 2X magnification

Photomicrograph 23: showing Secondary alteration of olivine and pyroxene by biotite and serpentinite and chlorite; note the close association of olivine and magnetite; under plane polarized light and 10X magnification

Photomicrograph 24: showing Close association of magnetite and olivine; magnetite also disseminated in matrix; under 2X magnification

Photomicrograph 25: showing Fractured and altered outer rim of Olivine phenocrysts showing syn crystallization deformation

Photomicrograph 26: showing Undulous extinction in clinopyroxene indicating grain scale ductile deformation.

Photomicrograph 27: showing Anhedral, skeletal shaped large magnetite intergrown with Olivine phenocrysts; small magnetite as dissemination within matrix.

Photomicrograph 28: showing Disseminated magnetite within alteration zones of the basalt.

Rock name: Porphyritic olivine basalt.

- Sample no: CMT/011/25/2025

Mineral assemblage: Phenocrystic phases: Olivine + Clinopyroxene Matrix phases: Plagioclase + Pyroxene + magnetite

Vesicular phases: Zeolite (quartz) Texture:

Phenocrysts of olivine and clinopyroxene occurring in very fine-grained basaltic matrix define the porphyritic texture. Olivine phenocrysts are mortly rounded, colorless and fractured. Clinopyroxene phenocrysts are relatively smaller in size and euhedral in shape. The matrix consists of very fine-grained plagioclase microlites and clinopyroxene defining typical basaltic intergranular and hyalopilitic texture. At places volcanic glass is preserved though devitrified. Tiny, anhedral to skeletal shaped magnetites occur as dissemination within the matrix. Small rounded shaped part represents filled up vesicles. Mostly quartz crystals, growing perpendicular to the wall of the vesicles represent the zeolite. Undulous extinction of quartz crystals of vug filling is also noted.

Photomicrograph 29 showing Porphyritic texture with olivine phenocrysts in fine grained plagioclase clinopyroxene rich basaltic matrix; under 2X magnification

Photomicrograph 30 showing Euhedral phenocrysts of Clinopyroxene and olivine in basaltic matrix; note the cluster of phenocrysts defining glomero-porphyritic texture; under 2X magnification

Photomicrograph 31: showing Note the microlites of plagioclase oriented haphazardly and interstitial spaces filled by pyroxene defining intergranular texture; also note the rounded vesicles.

Photomicrograph 32: showing Sub-rounded vesicles within basaltic matrix, filled up by quartz crystals grew perpendicular to the wall of vesicles; under 5X magnification.

Photomicrograph 33: showing Anhedral magnetite occurring within basalt and closely associated with phenocrysts; under reflected light and 10X magnification

Name of the rock: Porphyritic olivine basalt

Photomicrograph 34: showing Anhedral and small magnetite as dissemination within basalt; under reflected light and 10X magnification

- Sample no: CMT/011/31/2025

Mineral assemblage: Phenocrystic phases: Olivine + Clinopyroxene Matrix phases: Plagioclase + Pyroxene + magnetite

Secondary mineral: Serpentine; Vesicular phases: Quartz

Texture: The rock represents fine grained porphyritic basalt having inequigranular texture. Large olivine and clinopyroxene phenocrysts occur within very fine-grained basaltic matrix showing porphyritic texture. Olivine phenocrysts are identified by their high refractive index, colorless, fractured nature and by high birefringence color.

Often, olivine phenocrysts become altered and serpentinite and only pseudomorph of olivine is noted. Clinopyroxene phenocrysts are identified by high refractive index (< than olivine), one set of perfect cleavage with inclined extinction. At places, Clinopyroxene and olivine phenocrysts have fused grain boundaries. Exsolution lamellae and zoning are the common features of clinopyroxene phenocrysts. Two distinct modes of size of phenocrysts is noted. At places large anhedral clinopyroxene show intergrowth with laths of plagioclase feldspar showing eutectic crystallization. The matrix consists of very fine-grained lath shaped plagioclase microlites occurring randomly.

Very small clinopyroxene are present in the interstitial spaces of plagioclase laths defining intergranular texture. Very small magnetite and very few pyrite occur as dissemination within the matrix. Small rounded shaped part represents filled up vesicles. Mostly quartz crystals, growing perpendicular to the wall of the vesicles represent the zeolite.

Photomicrograph 35: showing Fractured       olivine phenocrysts with alteration along margin and fracture in a fine-grained magnetite rich basaltic matrix

Photomicrograph 36: showing Bimodal distribution of phenocrysts size in basalt; under cross polarized light and 2X magnification

Photomicrograph 38: Small, rounded filled up vesicles within basaltic matrix; quartz crystals have filled up the vesicles; under 5X magnification.

Photomicrograph 37: showing Clinopyroxene and olivine phenocrysts within basalt; note the exsolution lamellae within clinopyroxene

Photomicrograph 39: showing Olivine pseudomorph; secondary serpentine and a very few biotite replace the olivine phenocrysts completely, keeping intact its boundary and shape; also note the matrix rich in small magnetite and rutile.

Photomicrograph 40: showing same as left side one under cross polarized light and 10X magnification.

Photomicrograph 41: showing Large anhedral clinopyroxene phenocrysts intergrown with coarse plagioclase laths indicating eutectic crystallization. Under 5X magnification.

Photomicrograph 42: showing Same as left side one under cross polarized light and at 5X magnification.

Photomicrograph 43: showing Olivine pseudomorph; secondary serpentine replaces the olivine phenocrysts completely, keeping intact its boundary and shape; also note the magnetite rich matrix.

Photomicrograph 44: showing Olivine pseudomorph; secondary serpentine replaces the olivine phenocrysts completely, keeping intact its boundary and shape; also note the magnetite rich matrix

Photomicrograph 45: Showing Matrix rich in plagioclase laths with interstitial clinopyroxene and magnetite; under 10X magnification

Photomicrograph 46: showing Disseminated and tiny magnetite in basalt; under reflected light

Name of the rock: Porphyritic olivine basalt

- Sample no: CMT/011/62/2025

Mineral assemblage: Phenocrystic phases: Olivine + Clinopyroxene + Magnetite Matrix phases: Plagioclase + Pyroxene + magnetite

Texture: It is a very fine-grained rock showing inequigranular texture. Large, rounded, euhedral to subhedral phenocrysts of olivine, clinopyroxene and smaller magnetite phenocrysts occur within very fine-grained basaltic matrix. Magnetite and olivine phenocrysts were crystallized simultaneously which is evident from the coexistence of olivine and magnetite all along the rock. Magnetites are associated surrounding the olivine phenocrysts. Oscillatory compositional zoning in clinopyroxene phenocrysts is frequently noted indicating change in magmatic composition. Tiny magnetites occur as dissemination within matrix. Coarse magnetites are anhedral and intergrown silicate phases indicating synchronous crystallization. The matrix consists of very fine-grained lath shaped plagioclase occurring randomly. Very small clinopyroxene grains are present in the interstitial spaces of plagioclase laths defining intergranular texture.

Photomicrograph 47: showing Clinopyroxene and olivine phenocrysts; small magnetite disseminated in matrix

Photomicrograph 48: showing Porphyritic basalt with phenocrysts of olivine associated with large magnetite clusters

Photomicrograph 49: showing Fractured phenocrysts of olivine in magnetite rich basaltic matrix; note few large magnetite closely associated with olivine phenocrysts.

Photomicrograph 50: showing Bimodal distribution of size of phenocrysts; smaller sized phenocrysts of plagioclase and pyroxene in basaltic matrix; under 5X magnification.

Photomicrograph 51: showing Bimodal distribution of size of phenocrysts; smaller sized phenocrysts of plagioclase and pyroxene in basaltic matrix; under 5X magnification under crossed polar

Photomicrograph 52: showing Zoning and exsolution lamellae and twinning in clinopyroxene phenocrysts; under 5X magnification.

Photomicrograph 53: showing  compositional  zoning in olivine phenocrysts.

Photomicrograph 54: showing Olivine phenocrysts and anhedral magnetite  intergrown with olivine; under reflected light and 20X magnification

Photomicrograph 55: with silicate as intergrown phase; under reflected light and 10X magnification

Photomicrograph 56: showing Olivine phenocrysts surrounded by anhedral magnetite; under reflected light and 5X magnification.

- Name of the rock: Porphyritic olivine basalt

- Sample no: CMT/011/70/2025

Mineral assemblage: Phenocrystic phases: Olivine + Clinopyroxene Matrix phases: Plagioclase + Pyroxene + magnetite

Vesicular phases: Zeolite (quartz)

Texture: Phenocrysts of olivine and clinopyroxene occurring in very fine-grained basaltic matrix define the porphyritic texture. Olivine phenocrysts are sub-rounded and extensively fractured whereas clinopyroxene phenocrysts are anhedral to subhedral in shape and smaller in size.

Plagioclase microlites and devitrified glass represent hyalopilitic texture suggesting volcanic origin. Small rounded shaped part represents filled up vesicles. Mostly quartz crystals, growing perpendicular to the wall of the vesicles represent the zeolite. Undulous extinction of quartz crystals is also noted. The matrix consists of very fine-grained plagioclase and clinopyroxene defining typical basaltic texture. Tiny magnetites occur as dissemination within the matrix.

Photomicrograph 57: showing Fractured phenocrysts of large olivine in basaltic matrix; note the bimodal distribution of size of phenocrysts

Photomicrograph 58: Angular subhedral phenocrysts of clinopyroxene and relatively small olivine phenocrysts in microlites rich basaltic matrix.

Photomicrograph 59: showing Small rounded vesicles in basalt; also note the fractured olivine phenocrysts.

Photomicrograph 60: under cross polarised light; vesicles are filled up with quartz; note the microlites of plagioclase and glass defining the hyalopilitic texture.

Photomicrograph 61: vesicles filled with

quartz

Photomicrograph 61: showing higher magnification of the vesicles filled up with quartz.

Photomicrograph 63: showing Porphyritic and hyalopilitic textures of the rock

Photomicrograph 64: showing tiny magnetite disseminated in matrix; under reflected light and 10X magnification.

- Name of the rock: Porphyritic olivine basalt

- Sample no: CMT/011/73/2025

Mineral assemblage: Phenocrystic phases: Olivine Matrix phases: Plagioclase + Pyroxene + magnetite Secondary mineral: Serpentine + Chlorite

Texture: It is a very fine grained rock showing inequigranular texture. Large, rounded, euhedral to subhedral phenocrysts of olivine occur within very fine-grained basaltic matrix. Olivine phenocrysts are mostly fractured and have high birefringence color under cross polarized light. Compositional zoning in olivine is also noted. Chlorite and serpentine replaced and altered olivine along the fractures as secondary minerals. The matrix consists of lath shaped plagioclase and clinopyroxene defining intergranular texture. Magnetite are mostly small in size and anhedral in shape. They occur as disseminated grains within the matrix.

Photomicrograph 65: showing Euhedral and fractured olivine phenocrysts in fine grained basaltic matrix defining porphyritic texture.

Photomicrograph 66:is Same as left one; under cross polarized light; also note the compositional zoning in olivine.

Photomicrograph 67: showing Euhedral, large and fractured olivine phenocrysts with compositional zoning in fine grained basaltic matrix.

Photomicrograph 68: showing Plagioclase laths and interstitial clinopyroxene define the intergranular texture of basalt; under cross polarized light and at 10X magnification

Photomicrograph 69: showing Chloritic alteration of the matrix; note the anhedral magnetite grains disseminated in matrix

Photomicrograph 70: showing Anhedral magnetite intergrown with silicates; under reflected light and at 20X magnification.

Photomicrograph 71: showing Anhedral magnetite intergrown with silicates; under reflected light and at 10X magnification

Photomicrograph 72: showing Anhedral magnetite intergrown with silicates; tiny magnetites disseminated in matrix; under reflected light and at 20X magnification.

- Name of the rock: Porphyritic olivine basalt.

- Sample no: CMT/011/75/2025

Mineral assemblage: Phenocrystic phases: Olivine + Clinopyroxene + Plagioclase Matrix phases: Plagioclase + Pyroxene + magnetite

Secondary mineral: Biotite + Chlorite

Texture: This rock is very fine-grained basaltic rock having spectacular porphyritic texture with star shaped twin crystals of clinopyroxene and plagioclase. The volume percentage of olivine phenocrysts is more than that of clinopyroxene. Glomeroporphytic texture is defined by the clusters of olivine phenocrysts within basaltic matrix. Plagioclase phenocrysts are identified by their low refractive index and lamellar twinning. Often clinopyroxene and plagioclase phenocrysts show intergrowth texture. Compositional oscillatory zoning in clinopyroxene basal section is also noted indicating different pulses of magma influx. Bimodal distribution of grain size of phenocrysts indicates different stages of crystallization of mineral phases. At places the basalt became relatively coarser grained and magnetite is completely absent in that part. In rest of the matrix, tiny magnetites occur as disseminations. Small biotite and chlorites occur as secondary minerals within the matrix probably replacing the mafic phases.

Photomicrograph 73: showing Large euhedral and fractured olivine phenocrysts; note the bimodal phenocrysts size distribution; magnetite rich matrix and phenocrysts of clinopyroxene forming star shaped cluster.

Photomicrograph 74: Large euhedral and fractured olivine phenocrysts; note the bimodal phenocrysts size distribution; magnetite rich matrix and phenocrysts of clinopyroxene forming star shaped cluster under cross polarised light

Photomicrograph 75: showing Compositional oscillatory zoning in clinopyroxene basal section indicating different pulses of magma influx

Photomicrograph 76: showing Compositional oscillatory zoning in clinopyroxene basal section indicating different pulses of magma influx

Photomicrograph 77: showing Bimodal  distribution in phenocryst’s size; note the large olivine phenocrysts which crystallized early; relatively smaller pyroxene phenocryst disseminated in microlite and glassy matrix of basalt.

Photomicrograph 78: showing Bimodal distribution in phenocryst’s size; note the large olivine phenocrysts which crystallized early; relatively smaller pyroxene phenocrysts disseminated in microlite and glassy matrix of basalt.

Photomicrograph 79: showing Alteration of clinopyroxene phenocrysts and matrix phases by chlorite and biotite; note the magnetite concentration in matrix.

Photomicrograph 80: showing Olivine phenocryst altered along fractures by biotite; biotite also alters the matrix phases.

Photomicrograph 81: showing Rounded nodular part with coarser clinopyroxene and plagioclase grains; magnetite is completely absent in this coarser part.

Photomicrograph 82: showing Rounded nodular part with coarser clinopyroxene and plagioclase grains; magnetite is completely absent in this coarser part under cross polarised light.

Photomicrograph 83: showing Rounded nodular part with coarser clinopyroxene and plagioclase grains; magnetite is completely absent in this coarser part under cross polarised light.

Photomicrograph 84: showing Clusters of phenocrysts within microlite and glass enriched matrix

Photomicrograph 85: showing Clusters of phenocrysts within microlite and glass enriched matrix

Name of the rock: Porphyritic olivine basalt.

Photomicrograph 86: showing Small magnetites as disseminate

XRD Analysis:

Five samples were collected from the BRS Samples were subjected to  XRD-XRF analysis for mineral characterisation.The analytical results of 5 samples indicate a silicate-rich composition dominated by SiO₂ along with Al₂O₃, Fe₂O₃, CaO, and MgO. XRD mineralogical analysis shows diopside as the dominant crystalline phase (~30–35%) along with plagioclase feldspars like andesine, labradorite and Albite (~15–20%) and minor mica and oxide minerals like aegirine, augite, jadeite, forsterite, aegirine-augite. The high proportion of diopside (pyroxene) indicates a pyroxene-rich mafic mineral assemblage. Overall, the samples represent mafic igneous rocks with basaltic to doleritic affinity showing minor alteration effects.

### Whole rock analysis (Major oxides):

Several geochemical plots are created using GCD Kit software to understand the chemical variation patterns within the cogenetic volcanic rock series. These diagrams aided in identifying the systematic changes in the chemical data, which, in turn, provide significant insights into the causes regulating the chemical variation. Major oxide analysis of twenty pit samples shows the compositional diversity of the variants of basalts in the area. Major Oxides resulted are attached in annexures.

##### TAS Diagram:

The Total Alkali-Silica (TAS) diagram analysis demonstrates that all volcanic rocks plotted within the basaltic field. Petrographic study of the picrite to basaltic picrite samples demonstrated widespread alteration of olivine to iddingsite, an iron-rich alteration product. This modification likely impacted the bulk chemistry of picrite-basaltic picrite, causing them to be categorized as basalts on the TAS diagram instead of appearing in the anticipated basaltic picrite field.

## R1-R2 Plot:

The R₁–R₂ discrimination diagram is a geochemical classification tool used to identify and group volcanic and plutonic rocks based on their major oxide chemistry. The plot transforms major elements (Si, Al, Fe, Mg, Ca, Na, K, Ti) into two linear indices (R₁ and R₂), which effectively separate rock types even when mineralogical information is limited.

The diagram shown helps in:

- Classifying Igneous Rocks

- Distinguishing Alkaline vs. Subalkaline Trends

- Understanding Magmatic Evolution

- Identifying Special Rock Types

Fields like picritic rock, ankaratrite, hawaiite, mugearite help classify transitional or uncommon mafic compositions.

- The plotted samples classify mainly as basalt, andesite, and dacite, forming a sub alkaline, tholeiitic to calc-alkaline trend, reflecting typical fractional crystallization in a volcanic arc or subduction-related tectonic environment. No alkaline or highly evolved silicic rocks are present.

### Feo-Mgo-Al2o3 plot:

According to pearce et al, the samples plot mainly between the Ocean Island, Mid-Ocean Ridge, and Orogenic fields, showing a transitional geochemical signature. This reflects a mantle-derived basaltic magma with both enriched (OIB-type) and depleted (MORB-type) components, consistent with a rifted, back-arc, or transitional arc tectonic environment rather than a single pure tectonic setting.

10MnO–TiO₂–10P₂O₅ (Mullen 1983) plot:

The 10MnO – TiO₂ – 10P₂O₅ ternary diagram, proposed by Mullen (1983), is a geochemical discrimination tool used to identify the tectonic setting of basaltic rocks using three oxides:

TiO₂ – indicates mantle enrichment vs depletion MnO – related to melting depth and source properties P₂O₅ – sensitive to enriched mantle (OIB) signatures

  - Samples plot mainly in the Ocean Island Alkali Basalt (OIA) field, indicating an enriched mantle source and intraplate/alkaline affinity, with a minor tendency toward calc-alkaline signature. This suggests a transitional tectonic setting influenced by mantle plume-type magmatism and mild subduction-related overprints.

Gist of the above plots:

Whole-rock major oxide data of twenty pit samples were analysed using GCD Kit software to understand chemical variation within the volcanic rock suite. The results indicate compositional diversity among the basaltic variants present in the study area.

The TAS (Total Alkali–Silica) diagram shows that all samples plot within the basalt field. Petrographic observations reveal that olivine in picrite and basaltic picrite samples is widely altered to iddingsite, which likely modified the bulk chemistry and caused these rocks to plot as basalts instead of their expected picritic compositions.

The R₁–R₂ discrimination diagram classifies the samples mainly as basalt, andesite, and dacite, forming a sub-alkaline tholeiitic to calc-alkaline trend. This pattern reflects magmatic differentiation through fractional crystallization, with no strong evidence of highly alkaline or evolved silicic rocks.

In the FeO–MgO–Al₂O₃ plot, the samples fall between Ocean Island Basalt (OIB), Mid-Ocean Ridge Basalt (MORB), and orogenic fields, suggesting a transitional geochemical signature derived from mantle magma containing both enriched and depleted components.

The 10MnO–TiO₂–10P₂O₅ ternary diagram indicates that most samples plot in the Ocean Island Alkali Basalt field, pointing to an enriched mantle source and intraplate affinity, with minor calc-alkaline influence. This suggests mantle plume-related magmatism possibly modified by mild subduction-related processes.

In nutshell the studied volcanic rocks are predominantly basaltic, showing sub-alkaline to mildly alkaline characteristics and a transitional tectonic setting, likely derived from mantle-sourced magma influenced by both plume-type enrichment and minor arc-related processes.

Scanning Electorn Microscope (SEM) report

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

The following images and photographs represent four basalt/picritic basalt samples that were submitted to the Geological Survey of India (GSI), Southern Region for Scanning Electron Microscope (SEM) studies. The detailed analytical report and interpretation of the SEM study given below:

Normalized stoichiometric concentration [%]

Spectrum	OxygenAl2O3TiO2 FeO

CMT/011/07  0.00	2.85  17.06 80.09

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgO Al2O3SiO2 CaO  FeO

CMT/011/07 0.00	12.66 3.47  50.56 25.34 7.97

124

CXXIV

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenNa2OAl2O3SiO2 K2OCaO

CMT/011/070.00	4.79 24.82  54.62 1.09 14.69

125

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgO SiO2 FeO  MnO

CMT/011/07 0.00	28.63 33.20 37.41 0.76

126

127

128

129

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	Oxygen Al2O3 TiO2  MnO FeO

CMT/011/62   0.00	2.32   25.9  21.54 70.22

131

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgO  Al2O3  SiO2  CaO  FeO

CMT/011/62   0.00	11.99  4.01   49.62 25.99 8.39

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	Oxygen Na2O Al2O3  SiO2  CaO

CMT/011/62  0.00	3.69  27.56  52.82  15.93

132

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	 Sulfur Copper

CMT/011/62 21.48  78.52

133

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

| Spectrum OxygenAl2O3TiO2 Cr2O3FeO |  |  |  |
| --- | --- | --- | --- |
| CMT/011/620.00 | 2.42 | 25.892.44 | 69.25 |

134

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgO Al2O3  SiO2   CaO   FeO

CMT/011/6   20.00	13.5  32.52  50.81 28.6  44.50

135

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgO SiO2 FeO

CMT/011/620.00	36.3944.1119.51

136

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenCr2O3FeO

CMT/011/620.00	89.64 10.36

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgOAl2O3TiO2Cr2O3FeO

CMT/011/640.00	3.81 8.99  1.50 48.96 36.74

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

normalized stoichiometric concentration [%]

Spectrum	OxygenMgO Al2O3SiO2 CaO  FeO

CMT/011/640.00	13.485.04  50.3624.117.01

141

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenNa2OAl2O3SiO2 CaO

CMT/011/640.00	4.34 27.69 51.7216.26

142

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgO SiO2 FeO

CMT/011/640.00	33.4041.8724.72

143

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgO Al2O3 TiO2    FeO

CMT/011/640.00	2.41 2.40  31.4163.78

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgOAl2O3TiO2Cr2O3FeO

CMT/011/700.00	8.26 13.09 1.71 47.36 29.57

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenAl2O3TiO2 MnOFeO

CMT/011/700.00	1.61  32.063.13 63.20

147

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgO Al2O3SiO2 CaO FeO

CMT/011/700.00	14.743.27  51.0825.805.10

148

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgO SiO2 FeO

CMT/011/700.00	38.2448.3713.39

149

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgO SiO2 CaO FeO Al2O3

CMT/011/700.00	25.2562.481.359.141.77

150

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenNa2OAl2O3 SiO2   K2O   CaO

CMT/011/700.00	4.53 26.88 56.422.789.40

151

EM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgOTiO2 MnOFeO

CMT/011/700.00	2.83 43.770.96 52.43

152

SEM-EDS Lab, Palaeontology Division, Geological Survey of India (SR)

Hyderabad

Normalized stoichiometric concentration [%]

Spectrum	OxygenMgOAl2O3SiO2TiO2 FeO  MnO

Normalized stoichiometric concentration [%]

Spectrum	Copper

CMT/011/70100.00

CMT/011/700.00	4.73 2.08  4.6926.0261.770.

153

Conclusion:

The mineralogical suite identified across all samples is characteristic of a layered 	 mafic to ultramafic igneous assemblage (likely Gabbroic or Peridotitic).

Primary Silicates: The rock is dominated by high-magnesium/calcium phases, specifically Diopside (clinopyroxene), Olivine, and calcic Plagioclase (Labradorite-Bytownite).

Oxide Mineralization: There is significant enrichment of Chromite (up to 89% Cr2o3)  Ilmenite/Titaniferous Magnetite. This suggests potential for chromium and iron-titanium ore deposits.

Metallic/Sulfide Phases: The detection of Copper sulfides (Chalcocite) and Native Copper indicates a late-stage hydrothermal or magmatic sulfide mineralizing event.

The samples represent a high-temperature magmatic environment with strong evidence of Cr-Fe-Ti oxide crystallization and localized copper mineralization. The presence of fresh pyroxene and olivine alongside these metallic phases suggests the samples originate from an economically promising mafic-ultramafic complex

Summary:   (AI interpreted)

A reconnaissance-level (G4) mineral investigation was carried out on basalt/picritic basalt samples from the Botad area using SEM–EDS analysis at GSI, Hyderabad to evaluate their mineralogical composition and assess potential for Ni–Co–PGE and associated mineralization.

The SEM–EDS results indicate that the samples belong to a mafic to ultramafic igneous assemblage, characterized by a primary mineral suite of olivine, clinopyroxene (diopside/augite), and calcic plagioclase (labradorite–bytownite). The high MgO and FeO contents suggest a primitive, mantle-derived magma, indicative of high-temperature crystallization conditions.

Significant oxide mineralization has been identified in the form of:

- Chromite with Cr₂O₃ values up to ~89%

- Fe–Ti oxides such as ilmenite and titaniferous magnetite

These features indicate oxide segregation during magmatic differentiation, suggesting potential for chromium and Fe–Ti mineralization.

Importantly, sulfide and metallic phases have also been detected, including:

- Chalcocite (Cu₂S)

- Native copper

This points to late-stage magmatic sulfide segregation and/or hydrothermal activity, which is a key indicator for Ni–Cu–PGE mineral systems.

Overall, the mineral assemblage, geochemical signatures, and presence of oxide and sulfide phases suggest that the study area represents a fertile mafic–ultramafic magmatic system, possibly related to a layered intrusion or cumulate zone.

### The following favourable indicators for Ni–Co–PGE mineralization are observed:

- Ultramafic affinity (olivine-rich system)

- Strong chromite association

- Presence of sulfide mineralization (Cu phase)

- High MgO magma (primitive nature)

The Botad block area exhibits promising geological conditions for magmatic sulfide and oxide mineralization, with potential for:

- Chromite (Cr)

- Fe–Ti oxides

- Cu–Ni–Co sulfides

- PGE mineralization

6.1.7   Structure:

The following are the common structures noticed in the mapped area include:

Joints, including vertical, horizontal, and irregular joints that divide the basalt into blocks and facilitate weathering and groundwater movement. These joints trend N-S (vertical), E-W, and NE-SW (vertical to subvertical), along with horizontal sheet joints.

Vesicular and amygdaloidal structures are also widely present in the upper parts of lava flows, where gas bubbles formed cavities (vesicles) that were later filled by secondary minerals such as calcite, quartz, or zeolites to form amygdales.

Sheet joints and spheroidal weathering are also commonly observed in exposed basalt, producing rounded blocks due to chemical weathering along joint planes. These structural features are significant as they control groundwater movement, mineralisation, and the geomorphology of basaltic terrains.

Dyke intrusions are another important structural feature, where vertical or steeply inclined sheets of basaltic magma intruded along fractures and served as feeders for the lava flows.

6.1.8   Pitting

Pitting is an early-stage exploration method used to expose soil and weathered bedrock by excavating shallow pits. It is important because it provides direct access to subsurface materials, enabling geologists to observe lithology, alteration more accurately than by surface mapping alone. Pitting also facilitates reliable geochemical sampling from relatively undisturbed soil horizons and helps in understanding the local weathering profile and subsurface continuity of lithological units.

Initially, pits were planned along Flow 3 and Flow 4, as these represent picritic basalt flows.  Preliminary BRS samples from these zones yielded encouraging values, with Nickel (Ni) concentrations exceeding 1200 ppm. Based on these results, exploration efforts were initially focused on these flows and subsequently extended to the other flows in the area. Eventually, all the flows were investigated and the entire block was systematically covered.93 pits and 2BRS samples have been excavated in the mapped zone, each measuring 1 x 1 x 1 cubic meters (approx.) in dimension.

A total of 95 samples were systematically collected from the pits, representing each of the distinct layers observed in the pit sections. These include the duri-crust layer, red bole bed layer, and the olivine basaltic layer containing calcareous material.

However, seven pits could not be excavated due to obstruction from local villagers. Subsequently, 20 pit samples were analysed using XRF, while all 95 samples were subjected to ICP–MS chemical analysis. The subsurface observations from the pits have been documented and demarcated as A (along the north direction), B, C, D, E, and so on for systematic representation and interpretation.

Pitting revealed the continuation of picritic basalt in the subsurface layers. Within these layers, olivine crystals were observed, displaying colors ranging from green to reddish. In several instances, the olivine shows alteration to iddingsite, indicating secondary alteration processes within the basalt.

Photograph 37: showing worksheet of pit details

Few Photographs related to Pitting:

Photograph 38B: Showing pitting

Photograph 38A: Showing pitting

Photograph 38D: Showing pitting

Photograph 38C: Showing pitting

Table: Details of Pitting

| S. No | Sample Id | Latitude | Longitude | Dimension (in m) | Pit Volume (cu m) |
| --- | --- | --- | --- | --- | --- |
| 1 | P1/BTB/2025 | 22.171803 | 71.68595 | 1*1*1 | 1 |
| 2 | P2/BTB/2025 | 22.1753 | 71.686443 | 1*1*1 | 1 |
| 3 | P3/BTB/2025 | 22.175935 | 71.686137 | 1*1*1 | 1 |
| 4 | P4/BTB/2025 | 22.17693 | 71.686795 | 1*1*1 | 1 |
| 5 | P5/BTB/2025 | 22.178015 | 71.686268 | 1*1*1 | 1 |
| 6 | P6/BTB/2025 | 22.177838 | 71.685027 | 1*1*1 | 1 |
| 7 | P7/BTB/2025 | 22.179392 | 71.68802 | 1*1*1 | 1 |
| 8 | P8/BTB/2025 | 22.17983 | 71.688527 | 1*1*1 | 1 |
| 9 | P9/BTB/2025 | 22.182173 | 71.687272 | 1*1*1 | 1 |
| 10 | P10/BTB/2025 | 22.174173 | 71.690027 | 1*1*1 | 1 |
| 11 | P11/BTB/2025 | 22.1722445 | 71.689718 | 1*1*1 | 1 |
| 12 | P12/BTB/2025 | 22.1708 | 71.689153 | 1*1*1 | 1 |
| 13 | P13/BTB/2025 | 22.196212 | 71.6982 | 1*1*1 | 1 |
| 14 | P14/BTB/2025 | 22.192557 | 71.692855 | 1*1*1 | 1 |
| 15 | P15/BTB/2025 | 22.187255 | 71.691693 | 1*1*1 | 1 |
| 16 | P16/BTB/2025 | 22.184922 | 71.692865 | 1*1*1 | 1 |
| 17 | P17/BTB/2025 | 22.182715 | 71.692312 | 1*1*1 | 1 |
| 18 | P18/BTB/2025 | 22.181883 | 71.695478 | 1*1*1 | 1 |
| 19 | P19/BTB/2025 | 22.180908 | 71.701997 | 1*1*1 | 1 |
| 20 | P20/BTB/2025 | 22.178752 | 71.698485 | 1*1*1 | 1 |
| 21 | P21/BTB/2025 | 22.178628 | 71.693582 | 1*1*1 | 1 |
| 22 | P22/BTB/2025 | 22.17 | 71.710833 | 1*1*1 | 1 |
| 23 | P23/BTB/2025 | 22.169528 | 71.685037 | 1*1*1 | 1 |
| 24 | P24/BTB/2025 | 22.1688 | 71.684002 | 1*1*1 | 1 |
| 25 | P25/BTB/2025 | 22.166423 | 71.678943 | 1*1*1 | 1 |
| 26 | P26/BTB/2025 | 22.162388 | 71.679848 | 1*1*1 | 1 |
| 27 | P27/BTB/2025 | 22.164405 | 71.684322 | 1*1*1 | 1 |
| 28 | P28/BTB/2025 | 22.175482 | 71.681255 | 1*1*1 | 1 |
| 29 | P29/BTB/2025 | 22.184031 | 71.695361 | 1*1*1 | 1 |
| 30 | P30/BTB/2025 | 22.16865 | 71.685003 | 1*1*1 | 1 |
| 31 | P31/BTB/2025 | 22.120888 | 71.696067 | 1*1*1 | 1 |
| 32 | P32/BTB/2025 | 22.169232 | 71.713553 | 1*1*1 | 1 |
| 33 | P33/BTB/2025 | 22.168056 | 71.713889 | 1*1*1 | 1 |
| 34 | P34/BTB/2025 | 22.1675 | 71.721667 | 1*1*1 | 1 |
| 35 | P35/BTB/2025 | 22.161389 | 71.726944 | 1*1*1 | 1 |
| 36 | P36/BTB/2025 | 22.158056 | 71.724167 | 1*1*1 | 1 |
| 37 | P37/BTB/2025 | 22.153889 | 71.727778 | 1*1*1 | 1 |
| 38 | P38/BTB/2025 | 22.15557 | 71.725395 | 1*1*1 | 1 |
| 39 | P39/BTB/2025 | 22.159444 | 71.722222 | 1*1*1 | 1 |
| 40 | P40/BTB/2025 | 22.141183 | 71.727643 | 1*1*1 | 1 |
| 41 | P41/BT8/2025 | 22.142363 | 71.720277 | 1*1*1 | 1 |
| 42 | P42/BTB/2025 | 22.145588 | 71.71484 | 1*1*1 | 1 |
| 43 | P43/BTB/2025 | 22.143911 | 71.705497 | 1*1*1 | 1 |
| 44 | P44/BTB/2025 | 22.151969 | 71.698906 | 1*1*1 | 1 |
| 45 | P45/BTB/2025 | 22.159397 | 71.689622 | 1*1*1 | 1 |
| 46 | P46/BTB/2025 | 22.149444 | 71.671944 | 1*1*1 | 1 |
| 47 | P47/BTB/2025 | 22.146667 | 71.673333 | 1*1*1 | 1 |
| 48 | P48/BTB/2025 | 22.145886 | 71.679947 | 1*1*1 | 1 |
| 49 | P49/BTB/2025 | 22.136086 | 71.664194 | 1*1*1 | 1 |
| 50 | P50/BTB/2025 | 22.114444 | 71.658889 | 1*1*1 | 1 |
| 51 | PS1/BTB/2025 | 22.116389 | 71.66 | 1*1*1 | 1 |
| 52 | PS2/BTB/2025 | 22.116667 | 71.660833 | 1*1*1 | 1 |
| 53 | P53/BTB/2025 | 22.128056 | 781.6675 | 1*1*1 | 1 |
| 54 | P54/BTB/2025 | 22.103611 | 71.679167 | 1*1*1 | 1 |
| 55 | PS5/BTB/2025 | 22.1225 | 71.6675 | 1*1*1 | 1 |
| 56 | PS6/BTB/2025 | 22.124167 | 71.671111 | 1*1*1 | 1 |
| 57 | PS7/BTB/2025 | 22.118056 | 71.677222 | 1*1*1 | 1 |
| 58 | P58/BTB/2025 | 22.120556 | 71.6775 | 1*1*1 | 1 |
| 59 | P59/BTB/2025 | 22.133644 | 71.674756 | 1*1*1 | 1 |
| 60 | P60/BTB/2025 | 22.204069 | 71.707147 | 1*1*1 | 1 |
| 61 | P61/BTB/2025 | 22.214167 | 71.711111 | 1*1*1 | 1 |
| 62 | P62/BTB/2025 | 22.113889 | 71.686667 | 1*1*1 | 1 |
| 63 | P63/BTB/2025 | 22.116667 | 71.686389 | 1*1*1 | 1 |
| 64 | P64/BTB/2025 | 22.118889 | 71.688056 | 1*1*1 | 1 |
| 65 | P65/BTB/2025 | 22.118903 | 71.692731 | 1*1*1 | 1 |
| 66 | P66/BTB/2025 | 22.121389 | 71.697778 | 1*1*1 | 1 |
| 67 | P67/BTB/2025 | 22.094167 | 71.705833 | 1*1*1 | 1 |
| 68 | P68/BTB/2025 | 22.098333 | 71.691667 | 1*1*1 | 1 |
| 69 | P69/BTB/2025 | 22.12398 | 71.69963 | 1*1*1 | 1 |
| 70 | P70/BTB/2025 | 22.124167 | 71.701389 | 1*1*1 | 1 |
| 71 | P71/BTB/202 | 22.127778 | 71.701667 | 1*1*1 | 1 |
| 72 | P72/BTB/202 | 22.128056 | 71.699167 | 1*1*1 | 1 |
| 73 | P73/BTB/2025 | 22.131111 | 71.705556 | 1*1*1 | 1 |
| 74 | P74/BTB/2025 | 22.132264 | 71.701878 | 1*1*1 | 1 |
| 75 | P75/BTB/2025 | 22.135278 | 71.700556 | 1*1*1 | 1 |
| 76 | P76/BTB/2025 | 22.134444 | 71.697222 | 1*1*1 | 1 |
| 77 | P77/BTB/2025 | 22.13575 | 71.7183 | 1*1*1 | 1 |
| 78 | P78/BTB/2025 | 22.126667 | 71.723611 | 1*1*1 | 1 |
| 79 | P79/BTB/2025 | 22.131667 | 71.717778 | 1*1*1 | 1 |
| 80 | P80/BTB/2025 | 22.127222 | 71.715833 | 1*1*1 | 1 |
| 81 | P81/BTB/2025 | 22.122778 | 71.715278 | 1*1*1 | 1 |
| 82 | P82/BTB/2025 | 22.145278 | 71.665278 | 1*1*1 | 1 |
| 83 | P83/BTB/2025 | 22.140148 | 71.66178 | 1*1*1 | 1 |
| 84 | P84/BTB/2025 | 22.135923 | 71.673618 | 1*1*1 | 1 |
| 85 | P85/BTB/2025 | 22.1125 | 71.678333 | 1*1*1 | 1 |
| 86 | PB6/BTB/2025 | 22.111418 | 71.678823 | 1*1*1 | 1 |
| 87 | P87/BTB/2025 | 22.100278 | 71.683889 | 1*1*1 | 1 |
| 88 | P88/BTB/2025 | 22.088611 | 71.69 | 1*1*1 | 1 |
| 89 | P89/BTB/2025 | 22.101944 | 71.6725 | 1*1*1 | 1 |
| 90 | P90/BTB/2025 | 22.10974 | 71.673258 | 1*1*1 | 1 |
| 91 | P91/BTB/2025 | 22.113056 | 71.666944 | 1*1*1 | 1 |
| 92 | P92/BTB/2025 | 22.118611 | 71.652778 | 1*1*1 | 1 |
| 93 | P93/BTB/2025 | 22.118056 | 71.6475 | 1*1*1 | 1 |
|  |  |  |  | Total Volume | 93 cubic meter |

MAP SHOWING BASALTIC FLOWS ALONG WITH PIT LOCATIONS:

PLATE NO:11

#### Pit Profiles:

On the basis of pit worksheet (eg: Photograph 37), we have created the pit profiles showing the subsurface rock types and described about each wall and made pit profiles where each wall of the pit is 1m.

Ni: 297.2

Cr : 468

Co: 31.2

Nb: 1262.6

Pit 1 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P1/BTB/2025 |  |
| --- | --- |
| Location: 22.171803,71.68595 | Elevation: 100m |
| Pit top measurement: a. Length: 1m b.Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 2 profile:

Ni: 405.4

Cr : 374.3

Co: 32

Nb: 1188.7

| Name of the investigation: Ni, Co and PGE Pit no: P2/BTB/2025 |  |
| --- | --- |
| Location:22.175300,71.686 | Elevation:92m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 315.3

Cr : 433.4

Co: 29.9

Nb: 1219.6

Pit 3 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P3/BTB/2025 |  |
| --- | --- |
| Location: 22.175935,71.686137 | Elevation: 98m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 4 profile:

Ni: 139.5

Cr : 122.2

Co: 23

Nb: 1395.9

| Name of the investigation: Ni, Co and PGE Pit no: P4/BTB/2025 |  |
| --- | --- |
| Location: 22.176930,71.686795 | Elevation: 95 m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 209.3

Cr : 269.8

Co: 18.2

Nb: 660.4

Pit 5 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P5/BTB/2025 |  |
| --- | --- |
| Location: 22.178015,71.686268 | Elevation: 88m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: RED BOLE BED |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 160

Cr : 218.4

Co: 20.6

Nb:933.8

Pit 6 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P6/BTB/2025 |  |
| --- | --- |
| Location: 22.177838,71.685027 | Elevation: 100m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 7 profile:

Ni:434.7

Cr :658.2

Co: 34.3

Nb:1097.1

| Name of the investigation: Ni, Co and PGE Pit no: P7/BTB/2025 |  |
| --- | --- |
| Location: 22.179392,71.688020 | Elevation: 88m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 8 profile:

Ni: 223.5

Cr :452

Co: 32.2

Nb:838.5

| Name of the investigation: Ni, Co and PGE Pit no: P8/BTB/2025 |  |
| --- | --- |
| Location: 22.179830,71.688527 | Elevation: 89m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 9 profile:

Ni: 511.3

Cr : 377.8

Co: 48.1

Nb:536.2

| Name of the investigation: Ni, Co and PGE Pit no: P9/BTB/2025 |  |
| --- | --- |
| Location: 22.182173,71.687272 | Elevation: 84m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 10 profile:

Ni: 543.7

Cr :691.5

Co: 43.7

Nb: 840.4

| Name of the investigation: Ni, Co and PGE Pit no: P10/BTB/2025 |  |
| --- | --- |
| Location: 22.174173,71.690027 | Elevation: 85m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 11 profile:

Ni: 346.3

Cr : 399.1

Co: 34.8

Nb: 1059.3

| Name of the investigation: Ni, Co and PGE Pit no: P11/BTB/2025 |  |
| --- | --- |
| Location: 22.172245,71.689718 | Elevation: 102m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 12 profile:

Ni: 449.8

Cr : 446.5

Co: 40

Nb: 786.4

| Name of the investigation: Ni, Co and PGE Pit no: P12/BTB/2025 |  |
| --- | --- |
| Location: 22.170800,71.689153 | Elevation: 89m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 13 profile:

Ni: 598

Cr : 507.7

Co: 64.1

Nb: 661

| Name of the investigation: Ni, Co and PGE Pit no: P13/BTB/2025 |  |
| --- | --- |
| Location: 22.196212,71.698200 | Elevation: 93m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 14 profile:

Red bole bed

Ni: 285.8

Cr : 344.4

Co: 17.6

Nb: 252.3

| Name of the investigation: Ni, Co and PGE Pit no: P14/BTB/2025 |  |
| --- | --- |
| Location: 22.192557,71.692855 | Elevation: 91m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 15 profile:

Ni: 404.4

Cr : 606.2

Co: 37.6

Nb: 747.6

| Name of the investigation: Ni, Co and PGE Pit no: P15/BTB/2025 |  |
| --- | --- |
| Location: 22.187255,71.691693 | Elevation: 91m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 16 profile:

Ni: 653.5

Cr : 1156.9

Co: 47.9

Nb: 788.4

| Name of the investigation: Ni, Co and PGE Pit no: P16/BTB/2025 |  |
| --- | --- |
| Location: 22.184922,71.692865 | Elevation: 99m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 17 profile:

Ni: 412.9

Cr : 539.7

Co: 33.6

Nb: 1207.6

| Name of the investigation: Ni, Co and PGE Pit no: P17/BTB/2025 |  |
| --- | --- |
| Location: 22.182715,71.692312 | Elevation: 87 |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 18 profile:

Ni: 260.3

Cr : 271.2

Co: 27.5

Nb: 1074.9

| Name of the investigation: Ni, Co and PGE Pit no: P18/BTB/2025 |  |
| --- | --- |
| Location: 22.181883,71.695478 | Elevation: 86m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 19 profile:

Ni: 757.1

Cr : 729.6

Co: 51.5

Nb: 569.9

| Name of the investigation: Ni, Co and PGE Pit no: P19/BTB/2025 |  |
| --- | --- |
| Location: 22.180908,71.701997 | Elevation: 80m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 20 profile:

Ni: 751.8

Cr : 770.1

Co: 52.1

Nb: 455.9

| Name of the investigation: Ni, Co and PGE Pit no: P20/BTB/2025 |  |
| --- | --- |
| Location: 22.178752,71.698485 | Elevation: 83m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 21 profile:

Ni: 195.1

Cr : 340.3

Co: 24.3

Nb: 848.5

| Name of the investigation: Ni, Co and PGE Pit no: P21/BTB/2025 |  |
| --- | --- |
| Location: 22.178628,71.693582 | Elevation: 81m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 22 profile:

Ni: 383.8

Cr : 656.530.8

Co: 771.9

Nb:

| Name of the investigation: Ni, Co and PGE Pit no: P22/BTB/2025 |  |
| --- | --- |
| Location: 22.170000,,71.71833 | Elevation: 87m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Amygdaloidal basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

| Name of the investigation: Ni, Co and PGE Pit no: P23/BTB/2025 |  |
| --- | --- |
| Location: 22.169528,71.685037 | Elevation: 104m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 23 profile:

Ni: 468.1

Cr : 735.5

Co: 38.4

Nb: 1020.4

Pit 24 profile:

Ni: 497.2

Cr : 715.8

Co: 39.3

Nb: 1104.6

| Name of the investigation: Ni, Co and PGE Pit no: P24BTB/2025 |  |
| --- | --- |
| Location: 22.168800,71.684002 | Elevation: 101m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 25 profile:

Ni: 371.5

Cr : 564.7

Co: 27.7

Nb: 684.3

| Name of the investigation: Ni, Co and PGE Pit no: P25/BTB/2025 |  |
| --- | --- |
| Location: 22.166423,71.678943 | Elevation: 84m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 26 profile:

Ni: 351.9

Cr : 530.2

Co: 33.3

Nb: 1164.8

| Name of the investigation: Ni, Co and PGE Pit no: P26/BTB/2025 |  |
| --- | --- |
| Location: 22.162388,71.679848 | Elevation: 90m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 27 profile:

Ni: 179.1

Cr : 424.8

Co: 28.8

Nb: 1004.9

| Name of the investigation: Ni, Co and PGE Pit no: P27/BTB/2025 |  |
| --- | --- |
| Location: 22.164405,71.684322 | Elevation: 81m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 28 profile:

Ni: 425.2

Cr : 765.9

Co: 36.9

Nb: 1053.2

| Name of the investigation: Ni, Co and PGE Pit no: P28/BTB/2025 |  |
| --- | --- |
| Location: 22.175482,71.681255 | Elevation: 94m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 29 profile:

Ni: 561.9

Cr : 438.6

Co: 36.6

Nb: 1001.1

| Name of the investigation: Ni, Co and PGE Pit no: P29/BTB/2025 |  |
| --- | --- |
| Location: 22.184031,71.695361 | Elevation: 89m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 30 profile:

Ni: 468.4

Cr : 852.7

Co: 39.3

Nb: 1051.4

| Name of the investigation: Ni, Co and PGE Pit no: P30/BTB/2025 |  |
| --- | --- |
| Location: 22.168650,71.685003 | Elevation: 99m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 31 profile:

Ni: 746.9

Cr : 1136.1

Co: 51.5

Nb: 666.1

| Name of the investigation: Ni, Co and PGE Pit no: P31/BTB/2025 |  |
| --- | --- |
| Location: 22.120888,71.696067 | Elevation: 86m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 32 profile:

Ni: 164.8

Cr : 484.1

Co: 37

Nb: 410.1

| Name of the investigation: Ni, Co and PGE Pit no: P32/BTB/2025 |  |
| --- | --- |
| Location: 22.169232,71.3553 | Elevation: 76m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 33 profile:

Ni: 87.2

Cr : 150.6

Co: 20.3

Nb: 254.8

| Name of the investigation: Ni, Co and PGE Pit no: P33/BTB/2025 |  |
| --- | --- |
| Location: 22.168056,71.713889 | Elevation: 85m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 34 profile:

Ni: 88.1

Cr : 144.3

Co: 20.3

Nb: 259.3

| Name of the investigation: Ni, Co and PGE Pit no: P34/BTB/2025 |  |
| --- | --- |
| Location: 22.167500,71.721667 | Elevation: 85m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Fine grained olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 35 profile:

Ni: 93.1

Cr : 164

Co: 65.2

Nb: 1084.3

| Name of the investigation: Ni, Co and PGE Pit no: P35/BTB/2025 |  |
| --- | --- |
| Location: 22.161389,71.726944 | Elevation: 82m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Fine grained olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 36 profile:

Ni: 150

Cr : 396.4

Co: 34.1

Nb: 360.5

| Name of the investigation: Ni, Co and PGE Pit no: P36/BTB/2025 |  |
| --- | --- |
| Location: 22.158056,71.724167 | Elevation: 81m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay Kumar, Geologists |  |
| Lithology details: Fine grained olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 140.9

Cr : 250.4

Co: 24

Nb: 251.6

Pit 37 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P37/BTB/2025 |  |
| --- | --- |
| Location: 22.153889,71.727778 | Elevation: 83m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Fine grained olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 38 profile:

Ni: 530.8

Cr : 546.8

Co: 50

Nb: 386.4

| Name of the investigation: Ni, Co and PGE Pit no: P38/BTB/2025 |  |
| --- | --- |
| Location: 22.155570,71.725395 | Elevation: 100m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Fine grained olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 39 profile:

Ni: 266.3

Cr : 604.5

Co: 98.9

Nb: 857.3

| Name of the investigation: Ni, Co and PGE Pit no: P39/BTB/2025 |  |
| --- | --- |
| Location: 22.159444,71.722222 | Elevation: 84m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar , Geologists |  |
| Lithology details: Fine grained olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 40 profile:

Ni: 315.3

Cr : 406.9

Co: 111.2

Nb: 921.7

| Name of the investigation: Ni, Co and PGE Pit no: P40/BTB/2025 |  |
| --- | --- |
| Location: 22.141183,71.727643 | Elevation: 57m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Fine grained olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 41 profile:

Ni: 202.9

Cr : 266.7

Co: 45.2

Nb: 517.7

| Name of the investigation: Ni, Co and PGE Pit no: P41/BTB/2025 |  |
| --- | --- |
| Location: 22.142363,71.720277 | Elevation: 68m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Fine grained olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 42 profile:

Ni: 417.4

Cr : 474.6

Co: 131

Nb: 771.6

| Name of the investigation: Ni,Co and PGE Pit no:P42/BTB/2025 |  |
| --- | --- |
| Location: 22.145588,71.714840 | Elevation: 69m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Fine grained olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 43 profile:

Ni: 313.5

Cr : 473.3

Co: 120.9

Nb: 984.5

| Name of the investigation: Ni,Co and PGE Pit no:P43/BTB/2025 |  |
| --- | --- |
| Location: 22.143911,71.705497 | Elevation: 96m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 44 profile:

Ni: 667.2

Cr : 741.2

Co: 79.3

Nb: 542.3

667.2741.2

| Name of the investigation: Ni, Co and PGE Pit no: P44/BTB/2025 |  |
| --- | --- |
| Location: 22.151969,71.698906 | Elevation: 75m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 45 profile:

Ni: 121

Cr : 130.7

Co: 62.6

Nb: 1186.6

| Name of the investigation: Ni, Co and PGE Pit no: P45/BTB/2025 |  |
| --- | --- |
| Location: 22.159397,71.689622 | Elevation: 77m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 46 profile:

Ni: 420

Cr : 500.4

Co: 102.3

Nb: 783.6

| Name of the investigation: Ni, Co and PGE Pit no: P46/BTB/2025 |  |
| --- | --- |
| Location: 22.149444,71.671944 | Elevation: 89m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by:Mekala Chandu and Ajay kumar , Geologists |  |
| Lithology details: Olivine basalt in red bole bed |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 47 profile:

Ni: 435.5

Cr : 823

Co: 126.8

Nb: 878.4

| Name of the investigation: Ni, Co and PGE Pit no: P47/BTB/2025 |  |
| --- | --- |
| Location: 22.146667,71.673333 | Elevation: 101m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt in red bole bed |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 48 profile:

Ni: 393.2

Cr : 813.5

Co: 86.4

Nb: 767.9

| Name of the investigation: Ni,Co and PGE Pit no:P48/BTB/2025 |  |
| --- | --- |
| Location: 22.145886,71.679947 | Elevation: 91m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 49 profile:

Ni: 262.7

Cr : 506.3

Co: 78.3

Nb: 955.2

| Name of the investigation: Ni,Co and PGE Pit no:P49/BTB/2025 |  |
| --- | --- |
| Location: 22.136086,71.664194 | Elevation: 79m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 50 profile:

Ni: 355.9

Cr : 522.1

Co: 117.7

Nb: 837

| Name of the investigation: Ni, Co and PGE Pit no: P50/BTB/2025 |  |
| --- | --- |
| Location: 22.114444,71.658889 | Elevation: 89m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 51 profile:

Ni: 320.9

Cr : 457.1

Co: 115.6

Nb: 834.3

320.9457.1115.6834.3

| Name of the investigation: Ni, Co and PGE Pit no: P51/BTB/2025 |  |
| --- | --- |
| Location: 22.116389,71.660000 | Elevation: 89m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 52 profile:

Ni: 303.7

Cr : 391.9

Co: 103.3

Nb: 711.8

| Name of the investigation: Ni, Co and PGE Pit no: P52/BTB/2025 |  |
| --- | --- |
| Location: 22.116667,71.660833 | Elevation: 89m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 53 profile:

Ni: 407.6

Cr : 478.2

Co: 80.6

Nb: 707.3

| Name of the investigation: Ni, Co and PGE Pit no: P53/BTB/2025 |  |
| --- | --- |
| Location: 22.128056,71.667500 | Elevation: 79m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 54 profile:

Ni: 740.7

Cr : 576.6

Co: 86.9

Nb: 693.8

| Name of the investigation: Ni, Co and PGE Pit no: P54/BTB/2025 |  |
| --- | --- |
| Location: 22.103611,71.679167 | Elevation: 92m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt in red bole bed |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 55 profile:

Ni: 660.4

Cr : 535.9

Co: 130.2

Nb: 634.1

| Name of the investigation: Ni, Co and PGE Pit no: P55/BTB/2025 |  |
| --- | --- |
| Location: 22.122500,71.667500 | Elevation: 98m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt in red bole bed |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 56 profile:

Ni: 602.8

Cr : 557.4

Co: 148.4

Nb: 707.8

| Name of the investigation: Ni, Co and PGE Pit no: P56/BTB/2025 |  |
| --- | --- |
| Location: 22.124167,71.671111 | Elevation: 97m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 57 profile:

Ni: 641

Cr : 680.7

Co: 64.9

Nb: 525.2

| Name of the investigation: Ni, Co and PGE Pit no: P57/BTB/2025 |  |
| --- | --- |
| Location: 22.118056,71.677222 | Elevation: 73m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 58 profile:

Ni: 919.6

Cr : 771.5

Co: 99

Nb: 351

| Name of the investigation: Ni, Co and PGE Pit no: P58/BTB/2025 |  |
| --- | --- |
| Location: 22.120556,71.677500 | Elevation: 76m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 59 profile:

Ni: 732.5

Cr : 609.1

Co: 81.9

Nb: 553.5

| Name of the investigation: Ni, Co and PGE Pit no: P59/BTB/2025 |  |
| --- | --- |
| Location: 22.133644,71.674756 | Elevation: 84m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 60 profile:

Ni: 309.5

Cr : 387

Co: 144.7

Nb: 842.9

| Name of the investigation: Ni, Co and PGE Pit no: P60/BTB/2025 |  |
| --- | --- |
| Location: 22.204069,71.707147 | Elevation: 73m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt in red bole bed |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 61 profile:

Ni: 175.1

Cr : 424.3

Co: 46.9

Nb: 670.8

| Name of the investigation: Ni, Co and PGE Pit no: P61/BTB/2025 |  |
| --- | --- |
| Location: 22.214167,71.711111 | Elevation: 68m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Amygdaloidal basalt (precipitated form) |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 62 profile:

Ni: 647.2

Cr : 645.7

Co: 60

Nb: 602.8

| Name of the investigation: Ni, Co and PGE Pit no: P62/BTB/2025 |  |
| --- | --- |
| Location: 22.113889,71.686667 | Elevation: 82m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 63 profile:

Ni: 645.6

Cr : 532.9

Co: 52.1

Nb: 667.9

| Name of the investigation: Ni, Co and PGE Pit no: P63/BTB/2025 |  |
| --- | --- |
| Location: 22.116667,71.686389 | Elevation: 84m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 64 profile:

Ni: 989.4

Cr : 825.8

Co: 71.9

Nb: 447.7

| Name of the investigation: Ni, Co and PGE Pit no: P64/BTB/2025 |  |
| --- | --- |
| Location: 22.118889,71.688056 | Elevation: 85m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 65 profile:

Ni: 1025.4

Cr : 853.4

Co: 81.5

Nb: 437.4

| Name of the investigation: Ni, Co and PGE Pit no: P65/BTB/2025 |  |
| --- | --- |
| Location: 22.118903,71.692731 | Elevation: 100m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 66 profile:

Ni: 1024.6

Cr : 966.9

Co: 77.6

Nb: 455.6

| Name of the investigation: Ni, Co and PGE Pit no: P66/BTB/2025 |  |
| --- | --- |
| Location: 22.121389,71.697778 | Elevation: 86m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 67 profile:

Ni: 392.6

Cr : 759.6

Co: 28.4

Nb: 612.5

| Name of the investigation: Ni, Co and PGE Pit no: P67/BTB/2025 |  |
| --- | --- |
| Location: 22.094167,71.705833 | Elevation: 70m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 68 profile:

Ni: 933.4

Cr : 728.7

Co: 76.2

Nb: 415.2

| Name of the investigation: Ni, Co and PGE Pit no: P68/BTB/2025 |  |
| --- | --- |
| Location: 22.098333,71.691667 | Elevation: 69m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 69 profile:

Ni: 989.3

Cr : 1218.7

Co: 68.1

Nb: 573.8

| Name of the investigation: Ni, Co and PGE Pit no: P69/BTB/2025 |  |
| --- | --- |
| Location: 22.123980,71.699630 | Elevation: 87m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 70 profile:

Ni: 1073.5

Cr : 1162.4

Co: 74.8

Nb: 493.8

| Name of the investigation: Ni, Co and PGE Pit no: P70/BTB/2025 |  |
| --- | --- |
| Location: 22.124167,71.701389 | Elevation: 88m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 71 profile:

Ni: 676.8

Cr : 1282.1

Co: 50.9

Nb: 784.9

| Name of the investigation: Ni, Co and PGE Pit no: P71/BTB/2025 |  |
| --- | --- |
| Location: 22.127778,71.701667 | Elevation: 85m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 72 profile:

Ni: 633

Cr : 991.9

Co: 47.3

Nb: 738.7

| Name of the investigation: Ni, Co and PGE Pit no: P72/BTB/2025 |  |
| --- | --- |
| Location: 22.128056,71.699167 | Elevation: 86m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 73 profile:

Ni: 563.1

Cr : 810.4

Co: 44.1

Nb: 883

| Name of the investigation: Ni, Co and PGE Pit no: P73/BTB/2025 |  |
| --- | --- |
| Location: 22.131111,71.705556 | Elevation: 83m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 74 profile:

Ni: 417.3

Cr : 776.6

Co: 39.9

Nb: 674.6

| Name of the investigation: Ni, Co and PGE Pit no: P74/BTB/2025 |  |
| --- | --- |
| Location: 22.132264,71.68595 | Elevation: 79m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 75 profile:

Ni: 857.1

Cr : 968.5

Co: 59.9

Nb: 575.9

| Name of the investigation: Ni,Co and PGE Pit no:P75/BTB/2025 |  |
| --- | --- |
| Location: 22.135278,71.700556 | Elevation: 90m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 830.5

Cr : 842.3

Co: 60.5

Nb: 707.4

Pit 76 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P76/BTB/2025 |  |
| --- | --- |
| Location: 22.134444,71.697222 | Elevation: 92m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 283.2

Cr : 369.6

Co: 23.3

Nb: 609.1

Pit 77 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P77/BTB/2025 |  |
| --- | --- |
| Location: 22.135750,71.718300 | Elevation: 67m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 78 profile:

Ni:432.1

Cr : 757.3

Co: 39.7

Nb: 945.6

| Name of the investigation: Ni, Co and PGE Pit no: P78/BTB/2025 |  |
| --- | --- |
| Location: 22.126667,71.723611 | Elevation: 77m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by:Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 526.7

Cr : 787.3

Co: 46.7

Nb: 754.2

Pit 79 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P79/BTB/2025 |  |
| --- | --- |
| Location: 22.131667,71.717778 | Elevation: 100m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 80 profile:

Ni: 530.3

Cr : 791.5

Co: 45.6

Nb: 643.1

| Name of the investigation: Ni, Co and PGE Pit no: P80/BTB/2025 |  |
| --- | --- |
| Location: 22.127222,71.715833 | Elevation: 79m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 81 profile:

Ni: 501.1

Cr : 591.3

Co: 50.5

Nb: 787

| Name of the investigation: Ni, Co and PGE Pit no: P81/BTB/2025 |  |
| --- | --- |
| Location: 22.122778,71.715278 | Elevation: 76m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 300.4

Cr : 466.4

Co: 31.6

Nb: 1024.3

Pit 82 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P82/BTB/2025 |  |
| --- | --- |
| Location: 22.145278,71.665278 | Elevation: 89m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni:251.7

Cr : 515.1

Co: 43.1

Nb: 747.3

Pit 83 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P83/BTB/2025 |  |
| --- | --- |
| Location: 22.140148,71.661780 | Elevation: 89m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 179.3

Cr : 254

Co: 34.7

Nb: 497.7

Pit 84 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P84/BTB/2025 |  |
| --- | --- |
| Location: 22.135923,71.673618 | Elevation: 86m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 85 profile:

Ni: 776.8

Cr : 835.6

Co: 59.9

Nb: 856.9

| Name of the investigation: Ni, Co and PGE Pit no: P85/BTB/2025 |  |
| --- | --- |
| Location: 22.112500,71.678333 | Elevation: 74m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 86 profile:

Ni: 415.3

Cr : 706.3

Co: 29.5

Nb: 681.7

| Name of the investigation: Ni, Co and PGE Pit no: P86/BTB/2025 |  |
| --- | --- |
| Location: 22.111418,71.678823 | Elevation: 78m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 1025.3

Cr : 918

Co: 81

Nb: 418.4

Pit 87 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P87/BTB/2025 |  |
| --- | --- |
| Location: 22.100278,71.683889 | Elevation: 71m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 1116.3

Cr : 909.1

Co: 91

Nb: 313.2

Pit 88 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P88/BTB/2025 |  |
| --- | --- |
| Location: 22.088611,71.690000 | Elevation: 72m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Picritic basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Ni: 467.6

Cr : 640.8

Co: 37.7

Nb: 943

Pit 89 profile:

| Name of the investigation: Ni, Co and PGE Pit no: P89/BTB/2025 |  |
| --- | --- |
| Location: 22.101944,71.672500 | Elevation: 100m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Red bole bed |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 90 profile:

Ni: 290.2

Cr : 605.3

Co: 27.1

Nb: 732.3

290.2

| Name of the investigation: Ni, Co and PGE Pit no: P90/BTB/2025 |  |
| --- | --- |
| Location: 22.109740,71.673258 | Elevation: 70m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 91 profile:

Ni: 873

Cr : 804.9

Co: 65.5

Nb: 633.2

| Name of the investigation: Ni, Co and PGE Pit no: P91/BTB/2025 |  |
| --- | --- |
| Location: 22.113056,71.666944 | Elevation: 84m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 92 profile:

Ni: 805.5

Cr : 758.4

Co: 74.3

Nb: 430.4

| Name of the investigation: Ni, Co and PGE Pit no: P92/BTB/2025 |  |
| --- | --- |
| Location: 22.118611,71.652778 | Elevation: 86m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Pit 93 profile:

Ni: 866.1

Cr : 936.2

Co: 65.7

Nb: 741.9

866.1

| Name of the investigation: Ni, Co and PGE Pit no: P93/BTB/2025 |  |
| --- | --- |
| Location: 22.118056,71.647500 | Elevation: 81m |
| Pit top measurement: - Length: 1m - Breadth: 1m | Pit bottom measurement: - Length: 1m - Breadth: 1m - Depth: 1m |
| Recorded by: Mekala Chandu and Ajay kumar, Geologists |  |
| Lithology details: Olivine basalt |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |

Plate No 12:  showing Lithological Map of Botad block with Pit locations

LITHOLOGICAL MAP SHOWING PIT LOCATIONS:

6.1.9   Sampling

The region has been methodically sampled for bedrock during the geological fieldwork. For analysis, picritic basaltic samples are also gathered from the sections. When necessary, sampling has been done; fresh rock samples from outcrops are preferred. To guarantee that every sample is numbered sequentially, a sequential numeric sample numbering scheme has been used. Both the field diary and the sample registry or database provide explicit documentation of the sample type. Furthermore, the sample register/database and the field journal both carefully record GPS locations and sample numbers. A clear picture of the sample bag with the sample number on it, as well as pictures taken during sampling and sample packing, must be included in the photographic documentation.

Bed Rock Sampling

A total of 77 bedrock samples from all exposed outcrops have been gathered, mostly from picrite-basaltic picrite and a small number from amygdular and porphyritic basalt. The regions of Sherthali, Tajpar, Amapar road, samadhiyala 1, Bhambhan are the main locations for picrite-basaltic picrite exposures. Sample collection has been carried out based on the restricted availability of picrite basalt exposures, open mines, and excavated dumps from wells,1.5 kg of material are collected for every bedrock sample. The samples collected from each different basaltic layers and few samples also collected from red bole beds.

PLATE-13

PLATE-14

6.2   Geochemical Exploration:

Surface Samples:

During Field work 77 BRS samples have collected from all the areas, covering different rock types and different flows.

77 BRS samples has been analysed. The chemical analysis of this samples in the range of:

- Co values, in the block, are in the range of 2.9 ppm to 80.3 ppm

- Cr values, in the block, are in the range of 12.4 ppm to 1411.6 ppm

- Ni values, in the block, are in the range of 23.6 ppm to 1105.1 ppm

- Nb values, in the block, are in the range of 4.4 ppm to 1214.6 ppm.

Sub-surface samples:

Pitting work is complete and all the pit locations have been plotted on the geological map of the area. Chemical analysis of the 95 pit samples has been received from the lab. Elemental distribution maps for Ni, Cr, Co and Nb have been prepared. Based on the distribution patterns of the above elements, it is observed that Flow 3 appears to be more promising and to some extent Flow 4 also

- Cr values, in the block, are in the range of 122 ppm to 1280 ppm

- Co values, in the block, are in the range of 8 ppm to 90 ppm

- Ni values, in the block, are in the range of 87 ppm to 1116 ppm

- Nb values, in the block, are in the range of 250 ppm to 1395 ppm but sample number 94 in northern part of the block in the flow no 3 analysed more 39850 ppm (3.985%).

- Major Oxides of 20 Pit samples have been plotted. According to TAS and R1-R2 Plot, the samples were of picritic basalt and basaltic nature. As per the Pearce and Mullet triangular plot, the rocks were originated in Oceanic ridge and oceanic floor.

Map showing Elemental Distribution values of Nickel (Ni), Niobium (Nb), Chromium (Cr), Cobalt (Co):

PLATE-15

NICKEL:

420ppm is 5 times the crustal abundance

Results: In the range of- 425.2 to 1116.3 ppm

Out of 93+2 pit samples, 47 pit samples met the values of crustal abundance

NIOBIUM:

100ppm is 5 times the crustal abundance

Results: In the range of- 251 to 39850ppm

Out of 95 pit samples, every sample met the values of crustal abundance

PLATE-16

CHROMIUM:

510ppm is 5 times the crustal abundance

Results: In the range of- 515 to 1218 ppm

Out of 95 pit samples, 56 Pit samples met the values of crustal abundance.

PLATE-17

COBALT:

120ppm is 5 times the crustal abundance

Results:< 120ppm

PLATE-18

# Geochemical Interpretation of the analytical data: 
The integrated geochemical dataset comprising 172 samples collected through Bed rock (BRS) and pit (PITS) sampling methods defines a predominantly mafic to ultramafic geochemical environment with evidence of secondary hydrothermal overprinting and localized high field strength element (HFSE) enrichment, particularly Niobium (Nb). The overall elemental distribution pattern suggests a polyphase event dominant geochemical signature is characterized by elevated Nickel (Ni), Chromium (Cr), and Cobalt (Co), with strong positive inter-element correlations indicating derivation from olivine and pyroxene-rich parental lithologies. Nickel concentrations reaching values above 800 ppm and Chromium exceeding 1000 ppm in several samples strongly support an ultramafic source, likely representing serpentinite peridotites or picritic compositions. The coherence between Ni–Cr–Co suggests primary magmatic control with limited secondary remobilization for these elements. Chromium is likely hosted in spinel or chromite phases, whereas Nickel may occur within olivine, secondary serpentine phases, or minor disseminated sulfides. The persistence of this association across bed rock sample intervals indicates a lithologically controlled geochemical regime rather than structurally focused mineralization.

Major oxide data reinforces this interpretation. Silica values ranging broadly within the ultramafic to mafic field (approximately 30–44% SiO₂) confirm the low-silica character of the host rocks. Elevated MgO inferred from the compositional balance and high Loss on Ignition (LOI) values—reaching up to approximately 18% in some samples—indicate extensive serpentinization and hydration. Such alteration processes may have modified primary mineral assemblages, potentially redistributed mobile elements while leaving relatively immobile components intact. High LOI values point toward secondary mineral formation such as serpentine, talc, and carbonate phases, suggesting post-emplacement hydrothermal alteration or low-grade metamorphism.

Niobium presents a distinct geochemical behaviour compared to Ni and Cr. It shows weak correlation with ultramafic-associated elements and instead exhibits moderate affinity toward titanium and likely zirconium, consistent with its classification as a high field strength element. This decoupling from the mafic-ultramafic suite indicates that Nb is not controlled by primary mantle-derived mineral phases such as olivine or pyroxene. Instead, it is likely hosted in accessory minerals including titaniferous oxides, titanite, or potentially columbite-type phases if present. The distribution pattern suggests either late-stage magmatic differentiation or minor alkaline affinity within the system.

Superimposed on this primary ultramafic signature is a localized hydrothermal component evidenced by strong Copper (Cu) anomalies and moderate Molybdenum (Mo) enrichment. Copper shows highly skewed distribution patterns with isolated high values, indicating structurally controlled or vein-type mineralization rather than uniform lithological dispersion. The association of Cu with Mo in certain surface samples suggests a magmatic-hydrothermal overprint, potentially related to late-stage intrusive activity. This may represent minor porphyry-style mineralization or fracture-controlled sulphide deposition. The hydrothermal system appears spatially restricted and does not significantly disrupt the overall ultramafic geochemical coherence.

The Nb behaviour implies the presence of localized differentiation pockets within the mafic-ultramafic framework. If Nb enrichment corresponds spatially with elevated TiO₂ or Fe-oxide concentrations, this may indicate concentration within titanomagnetite or ilmenite-bearing zones. Alternatively, discrete Nb enrichment could reflect subtle alkaline intrusive influence not directly evident from major oxide data alone. The absence of strong correlation between Nb and Ni–Cr further supports the interpretation that Nb enrichment represents a separate geochemical event or phase within the broader magmatic evolution.

The dataset collectively defines three geochemical domains within the study area. The first domain represents the ultramafic/picrite core characterized by coherent Ni–Cr–Co enrichment and low silica content. The second domain reflects hydrothermal influence marked by localized Cu–Mo anomalies. The third domain comprises HFSE enrichment zones defined by Nb variability and association with titanium-bearing phases. These domains likely represent sequential geological processes: primary mantle-derived magmatic emplacement, subsequent serpentinization and alteration, late-stage magmatic-hydrothermal fluid activity, and minor differentiation-related HFSE concentration.

From a metallogenic perspective, the system is multi-commodity in nature. The ultramafic signature supports potential for nickel and chromium resources, particularly if sulphide segregation or cumulate layering is present at depth. Copper and molybdenum anomalies warrant structural and geophysical follow-up to determine continuity and economic concentration. Niobium, although not clearly indicative of high-grade mineralization at this stage, introduces the possibility of HFSE-enriched accessory phases that require mineralogical confirmation through heavy mineral separation and advanced microanalytical techniques.

In summary, the geochemical dataset reveals a complex, multi-stage evolutionary history. The primary ultramafic signature is robust and regionally consistent, while hydrothermal and HFSE signatures are localized and likely represent later superimposed events. The presence of Niobium adds a critical dimension to the interpretation, suggesting magmatic differentiation or subtle alkaline influence within an otherwise mafic–ultramafic geological framework.

| Sno | Sample ID | SiO2 | Al2O3 | Major oxides in % Fe2O3 | MgO | CaO | MnO | Na2O | K2O | TiO2 | P2O5 | V2O5 | LOI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P61/BTB/2025 | 30.11 | 13.29 | 9.357 | 4.703 | 20.576 | 0.123 | 0.633 | 0.491 | 1.641 | 0.182 | <0.01 | 18.67 |
| 2 | P60/BTB/2025 | 41.259 | 10.878 | 9.371 | 14.4 | 9.514 | 0.18 | 0.668 | 0.695 | 1.304 | 0.174 | <0.01 | 11.17 |
| 3 | P12/BTB/2025 | 44.029 | 10.998 | 11.271 | 16.875 | 9.812 | 0.171 | 2.068 | 0.708 | 1.291 | 0.331 | <0.01 | 1.75 |
| 4 | P6/BTB/2025 | 45.174 | 14.598 | 11.373 | 10.291 | 10.022 | 0.173 | 3.051 | 1.096 | 1.879 | 0.452 | <0.01 | 1.51 |
| 5 | P16/BTB/2025 | 46.443 | 17.254 | 9.851 | 6.206 | 9.391 | 0.153 | 4.243 | 1.724 | 1.971 | 0.387 | <0.01 | 2.07 |
| 6 | P26/BTB/2025 | 24.308 | 4.879 | 3.537 | 15.026 | 17.886 | <0.01 | 0.407 | 0.142 | 0.516 | <0.01 | <0.01 | 32.78 |
| 7 | P62/BTB/2025 | 44.659 | 15.055 | 11.554 | 9.573 | 10.08 | 0.175 | 3.492 | 0.853 | 1.857 | 0.385 | <0.01 | 1.89 |
| 8 | P53/BTB/2025 | 43.848 | 12.682 | 10.685 | 12.182 | 9.209 | 0.176 | 1.878 | 0.76 | 1.132 | 0.295 | <0.01 | 6.73 |
| 9 | P86/BTB/2025 | 42.473 | 9.803 | 11.555 | 15.897 | 4.59 | 0.16 | 0.162 | <0.01 | 1.512 | <0.01 | <0.01 | 13.16 |
| 10 | P69/BTB/2025 | 43.504 | 13.153 | 10.458 | 12.743 | 8.088 | 0.171 | 1.121 | 0.745 | 1.555 | 0.272 | <0.01 | 7.87 |
| 11 | P75/BTB/2025 | 37.083 | 10.137 | 7.269 | 10.719 | 13.042 | <0.01 | 1.266 | 0.58 | 1.302 | 0.103 | <0.01 | 18.05 |
| 12 | P78/BTB/2025 | 40.882 | 7.439 | 10.558 | 16.757 | 9.768 | 0.166 | 1.25 | 0.15 | 0.633 | 0.124 | <0.01 | 11.79 |
| 13 | P80/BTB/2025 | 42.359 | 10.92 | 12.808 | 20.169 | 7.674 | 0.199 | 1.945 | 0.504 | 0.961 | 0.272 | <0.01 | 1.44 |
| 14 | P37/BTB/2025 | 42.912 | 11.176 | 12.518 | 19.403 | 7.428 | 0.196 | 2.184 | 0.283 | 0.987 | 0.264 | <0.01 | 2.07 |
| 15 | P35/BTB/2025 | 44.178 | 14.244 | 11.329 | 11.457 | 10.275 | 0.175 | 3.395 | 0.518 | 1.411 | 0.37 | <0.01 | 2.16 |
| 16 | P54/BTB/2025 | 44.002 | 14.376 | 11.531 | 11.795 | 9.689 | 0.189 | 2.261 | 0.411 | 0.925 | 0.233 | <0.01 | 4.19 |
| 17 | P68/BTB/2025 | 33.37 | 11.741 | 9.643 | 8.903 | 15.195 | 0.114 | 1.678 | 0.52 | 1.31 | 0.134 | <0.01 | 16.92 |
| 18 | P43/BTB/2025 | 38.955 | 6.523 | 12.046 | 14.019 | 11.312 | 0.247 | 0.508 | 0.183 | 0.565 | 0.1 | <0.01 | 15.05 |
| 19 | BRS95/BTB/2025 | 49.517 | 13.896 | 11.146 | 9.334 | 7.625 | 0.147 | 2.477 | 1.426 | 1.399 | 0.303 | <0.01 | 2.29 |
| 20 | BRS94/BTB/2025 | 49.688 | 14.242 | 10.959 | 8.866 | 7.764 | 0.163 | 2.752 | 1.512 | 1.456 | 0.336 | <0.01 | 1.79 |

CHAPTER 7

7.1 Integration of Geology, geophysics (with available aero geophysical data) and geochemical exploration data and the interpretation

Creation of geological (lithological & structural), geophysical, geochemical and outcrop maps on true scale, overlay studies in GIS platform. Discuss on the interpretation and results and attach the soft copy of the same in shape file

Shape files in GIS format for the themes lithology, Oriented structures, Outcrop map, sample locations, pit locations and various elemental distribution maps viz Ni, Cr, Co and Nb etc have been prepared and discussed in the CHAPTER 6.

No geophysical component is in the scope of the present investigation.

No aero geophysical data is available in the public domain for the block area

Drilling is not recommended by the TCC-II of NMDET in view of the non-encouraging analytical results of bed rock and pit samples collected from the study area.

CHAPTER-8

Mineral prospect

Olivine bearing/Mg rich basaltic flows have been delineated during the large-scale geological mapping.  Also indicated the locations of Red bole beds (though not mappable on this scale) have identified and incorporated in the geological map as point locations. Whole rock analysis of the bed rock and pit samples indicates some consistent high values of Ni, Cr, Co and Nb in the flow 3 and flow 4 in the area.

Unusually high values of Nb (3.8%) in one of the bed rock samples collected in the north eastern part of the block is worth mentioning for future studies if needed.

Analytical data of the bed rock and pit samples of the are presented to TCC-II and the honourable members of the TCC-II, after detailed scrutiny of the data, recommended not to go for drilling and submit the Geological report.

Note: Number of rock Quarry’s of different variants of basalts are reported from the mapped area for construction and building material.  Red bole material can be used for agricultural purpose

CHAPTER-9

Exploration by scout Drilling

Based on the analytical data of both bed rock and pit samples of the area presented to the TCC-II of NMDET, drilling is not recommended by the TCC-II. Hence the Drilling was not undertaken in the area.

CHAPTER-10

Resource estimation

As there is no potential area for the commodities like Cr, Ni, Co and PGE in the area, there is no scope for resource estimation

CHAPTER-11

Conclusion and Recommendation

A reconnaissance stage (G4) exploration programme for Ni–Co–PGE was carried out in the Botad area, Botad District, Gujarat, under NMET funding, and executed by M/s Critical Minerals Trackers (hereafter referred to as the Botad Project). The total investigation area covers 99.6 sq. km, falling within Survey of India toposheets 41N/12 and 41N/16, bounded by latitudes 22°03′45″ N to 22°14′30″ N and longitudes 71°38′15″ E to 71°46′15″ E.

Large scale geological mapping on 1: 12500 scale reveals that the area is dominantly occupied by Olivine rich basalts / picritic basalts. The Deccan Traps in the area comprise Upper Cretaceous–Lower Paleocene porphyritic and amygdaloidal basalt flows, intruded by picrite and olivine basaltic sills and dykes. Four basalt flows were delineated, separated by intermittently developed red bole horizons, observed both in surface exposures and dug-well sections.

Lithologies identified include picrite basalt, aphanitic basalt, and porphyritic basalt. Picrites are characterized by olivine with frequent clinopyroxene and minor feldspar, occurring as phenocrysts and groundmass. Olivine alteration to serpentine and iddingsite in several samples suggests localized hydrothermal activity. Picrite and basaltic picrite exhibit medium-grained holocrystalline to porphyritic textures, with vugs commonly filled by zeolites. Basalts are dominated by plagioclase phenocrysts with minor olivine and clinopyroxene in the groundmass. Four flows were delineated, separated by intermittently developed red bole horizons.

Major oxide data from 20 pit samples, plotted on TAS and R1–R2 diagrams classify the rocks as picritic basalt and basalt. Pearce and Mullen discrimination diagrams indicate an oceanic ridge/oceanic floor tectonic affinity.

95 pits (1x1x1m dimension) have been excavated in the soil cover areas to expose the bed rock and bed rock samples have been collected and got them analysed for major oxides and Cr, Ni, Co, Nb and other elements. Based on the geochemical patterns, Flow 3 appear to be the most promising, followed by Flow 4 in respect of the spatially consistent anomalous values of Cr, Ni, Co and Nb.

Salient points of chemical analysis of pit samples:

Ni values range from 425.2 ppm to 1116.3 ppm

Cr value ranges from 515 ppm to 1218 ppm

Co value ranges from 12 ppm to 95 ppm

Nb value ranges from 251 ppm to 39850 ppm (a nugget value)

Salient points of chemical analysis of bed rock samples:

Ni ranges from 23.6 ppm to 1105.1 ppm

Cr ranges from 12.4 ppm to 1411.6 ppm

Co ranges from 2.9 ppm to 80.3 ppm

Nb ranges from 4.4 ppm to 1214.6 ppm

CHAPTER-12

Expenditure

The total expenditure incurred for execution of the project “Reconnaissance survey (G4) for Nickel, Cobalt and PGE in Botad area, Botad District, Gujarat” is Rs. 46.44 lakhs (Rupees forty-six lakhs forty-four thousand) only as detailed below. This has been approved by the 20th TCC-2 held on 2nd & 5th Jan 2026 and also in the 4th PSC held on 13th Jan 2026 of NMDET (Vide No. 117/1/2025/NMET/739, dated : 20.1.2026

CHAPTER-13

REFERENCES

- Arndt N., Lesher C.M. and Czamanske G.K., 2006. Mantle-derived magmas and magmatic Ni-Cu-(PGE) deposits. Economic Geology, 100th Anniversary volume, pp.5- 24, 2005. hal-00016864.

- Barnes S-J, Couture JF, Sawyer EW and Bouchaib C (1993) Nickel-copper occurrences in the Belleterre-Angliers belt of the Pontiac Sub province and the use of Cu-Pd ratios in interpreting platinum-group element distributions. Econ Geol 88(6):1402–1418.

- Des Raj, Banerjee A.C., Roychowdhury M., 1998. Study of picrite basalt and associated variants of Deccan Traps of Gujarat (T/S NO. 46K/1, 46 G/13 AND 41F/8) (Progress report for the Field Season 1994-95).

- Fedden F., 1884. The geology of the Kathiawar peninsula in Gujarat. Mem. Geol. Survey of India, Vol. XXI, Pt. 2.

- Ghevariya Z.H., and Sahu B.K., Study of picrite basalts and associated variants of Deccan Traps in parts of Bhavnagar, Rajkot and Ahmedabad districts of Gujarat (unpub Prog. Report of GSI, for the Field Season 1995-'96).

- Krishnamurthy P, 2015. Chalcophile Element Depletion in Lower Deccan Trap Formations and Implications for Cu-Ni-PGE Sulphide Mineralization in the Deccan Traps, India akin to those of Norilsk-Talnakh, Siberian Traps, Russia. Journal of Geological Society of India, Vol.85, pp.411-418

- M.J LE BAS IUGS Reclassification of the High-Mg and picritic volcanic rocks.

- Naldrett A.J., 1999. World-class Ni-Cu-PGE deposits: key factors in their genesis. Mineralium Deposita, Vol. 34, pp. 227-240.

- Naushad Md, 2016… “Study of mafic rich/ picritic lava flows/ lobes and intrusive from the Deccan Volcanic Province to understand their origin and to comment on their fertility in terms of Pt-Pd mineralization”, un pub prog report, GSI

CHAPTER-14

LOCALITY INDEX

| Sno | Name | Latitude (DD) | Longitude (DD) |
| --- | --- | --- | --- |
| 1 | Amapar | 22.14825400 | 71.66537100 |
| 2 | ARTO road | 22.18100600 | 71.68811600 |
| 3 | Bhambhan | 22.10825500 | 71.67504200 |
| 4 | Bhambhan (SW) | 22.18272700 | 71.72457200 |
| 5 | Khas | 22.18992700 | 71.72917200 |
| 6 | Khas road canal | 22.17916667 | 71.74472222 |
| 7 | Samadhiyala No 1 | 22.13238700 | 71.70181500 |
| 8 | Sherthali/senthali | 22.18819000 | 71.72824600 |
| 9 | Tajpar | 22.12338900 | 71.70025700 |
| 10 | Tajpar road | 22.14601800 | 71.67920300 |

List of Annexures: Analytical Results

BRS Test Results:

z

Pits Test Results: