Statement showing details of the Pit along with logging in Mudhan-Khatiya Block for Glauconite, District- Kachchh, Gujarat

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 25.08.2025 |
| 3. Pit no.: | MP-01 |
| 4. Location geo-coordinates: | Easting: 490211, Northing: 2627777 Elevation: 35m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.20 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/ Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.80 | Soil cover |
|  | 0.80 | 2.20 | Yellowish to grayish color shale and clay bands |

Other details:

1. Volume of the final pit: 2.20 cu.m.

2. Volume of the total excavated material: 2.20 cu.m.

3. Number and details of the channel samples: (1 nos.)  MP/01/01 – 0.81 m to 2.20m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 25.08.2025 |
| 3. Pit no.: | MP-02 |
| 4. Location geo-coordinates: | Easting: 491874 Northing: 2627964 Elevation: 47m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.10 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.27 | Soil cover |
|  | 0.27 | 1.25 | Brown sandstone (fine to medium grained) |
|  | 1.25 | 2.10 | Fine grained Sandstone |

Other details:

1. Volume of the final pit: 2.10 cu.m.

2. Volume of the total excavated material: 2.10 cu.m.

3. Number and details of the channel samples: (2 nos.) MP/02/01 – 0.27 m to 1.25 m

MP/02/01 – 1.25 m to 2.1 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 25.08.2025 |
| 3. Pit no.: | MP- 03 |
| 4. Location geo-coordinates: | Easting: 494113 Northing: 2628053 Elevation: 32m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/ Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.30 | Soil cover |
|  | 0.30 | 1.00 | Brownish color sandstone (friable) |
|  | 1.00 | 2.00 | Reddish to light greenish color sandstone with shale bands |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (1 nos.)  MP/03/01 – 1.0 m to 2.0m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 26.08.2025 |
| 3. Pit no.: | MP-04 |
| 4. Location geo-coordinates: | Easting: 494899 Northing: 2627711 Elevation: 25m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/ Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.24 | Soil cover |
|  | 0.24 | 0.74 | Yellowish to reddish colored sandstone with shale bands |
|  | 0.74 | 0.94 | Reddish sandstone |
|  | 0.94 | 1.13 | Yellowish colored friable sandstone |
|  | 1.13 | 2.00 | Reddish sandstone |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (2 nos.) MP/04/01 – 0.24 m to 1.00 m

MP/04/02 – 1.00 m to 2.00

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 26.08.2025 |
| 3. Pit no.: | MP- 05 |
| 4. Location geo-coordinates: | Easting: 494886 Northing: 2626513 Elevation: 25m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 3.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 1.00 | Soil cover |
|  | 1.00 | 2.00 | Soil cover with very rounded minor grains of sandstone |

Other details:

1. Volume of the final pit: 3.00 cu.m.

2. Volume of the total excavated material: 3.00 cu.m.

3. Number and details of the channel samples: No sample taken negative pit

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 26.08.2025 |
| 3. Pit no.: | MP-06 |
| 4. Location geo-coordinates: | Easting: 494923 Northing: 2626188 Elevation: 27 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.25 | Soil cover |
|  | 0.25 | 0.52 | Yellow to gray Shale and clay layer |
|  | 0.52 | 1.19 | Yellowish brown sandstone |
|  | 1.19 | 2.00 | Shale band |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (1 nos.) MP/06/01 – 1.19 to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 26.08.2025 |
| 3. Pit no.: | MP-07 |
| 4. Location geo-coordinates: | Easting: 495661 Northing: 2622877 Elevation: 58 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 3.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.70 | Soil cover |
|  | 0.70 | 3.00 | Yellow to gray Shale and clay layer |

Other details:

1. Volume of the final pit: 3.00 cu.m.

2. Volume of the total excavated material: 3.00 cu.m.

3. Number and details of the channel samples: (2 nos.) MP/07/01 – 0.70 m to 1.85 m

MP/07/02 – 1.85 m to 3.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 27.08.2025 |
| 3. Pit no.: | MP-08 |
| 4. Location geo-coordinates: | Easting: 495720 Northing: 2621955 Elevation: 58 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 3.10 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.20 | Soil cover |
|  | 0.20 | 0.35 | Gray to greenish color clay band |
|  | 0.35 | 0.63 | Yellowish sandstone |
|  | 0.63 | 0.84 | Gray to greenish color clay band |
|  | 0.84 | 1.10 | Red sandstone |
|  | 1.10 | 3.10 | Gray to greenish color clay band |

Other details:

1. Volume of the final pit: 3.10 cu.m.

2. Volume of the total excavated material: 3.10 cu.m.

3. Number and details of the channel samples: (3 nos.) MP/08/01 – 0.20 m to 1.10 m

MP/08/01 – 1.10 m to 2.10 m

MP/08/03 – 2.10 m to 3.10 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 27.08.2025 |
| 3. Pit no.: | MP-09 |
| 4. Location geo-coordinates: | Easting: 495653 Northing: 2621513 Elevation: 65 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.30 | Soil cover |
|  | 0.30 | 2.00 | Yellow sandstone |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (1 nos.) MP/09/01 – 1.00 m to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 27.08.2025 |
| 3. Pit no.: | MP- 10 |
| 4. Location geo-coordinates: | Easting: 496003 Northing: 2620820 Elevation: 77m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.12 | Soil cover |
|  | 0.12 | 0.27 | Yellowish to greenish colored sandstone |
|  | 0.27 | 0.50 | Friable sandstone |
|  | 0.50 | 1.00 | Yellowish clay band |
|  | 1.00 | 2.00 | Clay/marl band |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (1 nos.) MP/10/01 – 0.50 to 1.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 27.08.2025 |
| 3. Pit no.: | MP-11 |
| 4. Location geo-coordinates: | Easting: 496574 Northing: 2620922 Elevation: |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.50 | Gray clay bed |
|  | 0.50 | 1.00 | Yellowish sandstone |
|  | 1.00 | 2.00 | Fine grained sandstone |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (2 nos.) MP/11/01 – 0.00 to 1.00 m

MP/11/02 – 1.00 to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 28.08.2025 |
| 3. Pit no.: | MP-12 |
| 4. Location geo-coordinates: | Easting: 493942 Northing: 2620814 Elevation: |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.10 | Soil cover |
|  | 0.10 | 0.33 | Dark gray shale band |
|  | 0.33 | 0.41 | Brown to yellow coloured shale bands |
|  | 0.41 | 0.50 | Dark gray shale band |
|  | 0.50 | 2.00 | Alternating band of brown to yellow shale bands and dark gray shale bands |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (2 nos.) MP/12/01 – 0.10 m to 1.00 m

MP/12/02 – 1.00 m to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 28.08.2025 |
| 3. Pit no.: | MP-13 |
| 4. Location geo-coordinates: | Easting: 496435 Northing: 2619859 Elevation: 110 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.75 | Soil cover |
|  | 0.75 | 2.00 | Green to yellowish colored sandstone |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (1 nos.) MP/13/01 – 0.75 m to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 28.08.2025 |
| 3. Pit no.: | MP-14 |
| 4. Location geo-coordinates: | Easting: 496809 Northing: 2619547 Elevation: 96m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.18 m |
| --- | --- | --- |

8. Recorded by: Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.30 | Glauconitic sandstone |
|  | 0.30 | 0.90 | Dark gray shale |
|  | 0.90 | 1.22 | Yellow sandstone with fine shale bands |
|  | 1.22 | 1.89 | Dark gray shale |
|  | 1.89 | 2.18 | Glauconitic sandstone |

Other details:

1. Volume of the final pit: 2.18 cu.m.

2. Volume of the total excavated material: 2.18 cu.m.

3. Number and details of the channel samples: (4 nos.) MP/14/01 – 0.00 m to 0.30 m

MP/14/02 – 0.30 m to 1.22 m

MP/14/03 – 1.22 m to 1.89 m

MP/14/04 – 1.89 m TO 2.18 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 28.08.2025 |
| 3. Pit no.: | MP-15 |
| 4. Location geo-coordinates: | Easting: 497196 Northing: 2618542 Elevation: |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.04 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.29 | Soil cover |
|  | 0.29 | 0.88 | Weathered sandstone |
|  | 0.88 | 1.34 | Yellowish green sandstone (Glauconitic) |
|  | 1.34 | 2.04 | Gray to yellowish color intercalated shale |

Other details:

1. Volume of the final pit: 2.04 cu.m.

2. Volume of the total excavated material: 2.04 cu.m.

3. Number and details of the channel samples: (2 nos.) MP/15/01 – 0.88 m to 1.34 m

MP/15/02 – 1.34 m to 2.04 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 29.08.2025 |
| 3. Pit no.: | MP-16 |
| 4. Location geo-coordinates: | Easting: 498071 Northing: 2617086 Elevation: 116 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 3.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 1.15 | Soil cover |
|  | 1.15 | 3.00 | Friable sandstone |

Other details:

1. Volume of the final pit: 3.00 cu.m.

2. Volume of the total excavated material: 3.00 cu.m.

3. Number and details of the channel samples: (2 nos.) MP/16/01 – 1.15 m to 2.08 m

MP/16/02 – 2.08 m to 3.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 29.08.2025 |
| 3. Pit no.: | MP-17 |
| 4. Location geo-coordinates: | Easting: 496737 Northing: 2616515 Elevation:12 |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth:2m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.43 | Soil cover |
|  | 0.43 | 0.73 | White to yellowish clay band |
|  | 0.73 | 1.13 | Shaly sandstone |
|  | 1.13 | 1.43 | Ferruginous sandstone |
|  | 1.43 | 2.00 | Fine grained sandstone |

Other details:

1. Volume of the final pit: 2 cu.m.

2. Volume of the total excavated material: 2 cu.m.

3. Number and details of the channel samples: (2 nos.)  MP/17/01 – 0.43 to 1.00 m

MP/17/02 – 1.00 to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 29.08.2025 |
| 3. Pit no.: | MP-18 |
| 4. Location geo-coordinates: | Easting: 497600 Northing: 2615690 Elevation:104m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth:2.02 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.20 | Ferruginous shale |
|  | 0.20 | 0.30 | clay band |
|  | 0.30 | 0.40 | Ferruginous shale or marl |
|  | 0.40 | 0.50 | Fine to medium-grained Red sandstone |
|  | 0.50 | 0.90 | Fine to medium-grained yellow sandstone |
|  | 0.90 | 1.17 | Carbonaceous shale |
|  | 1.17 | 1.37 | Ferruginous shale |
|  | 1.37 | 1.67 | Carbonaceous shale |
|  | 1.67 | 1.87 | Ferruginous shale |
|  | 1.87 | 2.02 | Carbonaceous shale |

Other details:

1. Volume of the final pit: 2.02 cu.m.

2. Volume of the total excavated material: 2.02 cu.m.

3. Number and details of the channel samples: (2 nos.)  MP/18/01 – 0.00 to 1.00 m

MP/18/02 – 1.00 to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 30.08.2025 |
| 3. Pit no.: | MP-19 |
| 4. Location geo-coordinates: | Easting: 498540 Northing: 2615269 Elevation: 102 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.20 | Soil cover |
|  | 0.20 | 2.00 | Gray to greenish colored shale and clay bands |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (2 nos.) MP/19/01 – 0.20 m to 1.10m

MP/19/02 – 1.10 m to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 30.08.2025 |
| 3. Pit no.: | MP-20 |
| 4. Location geo-coordinates: | Easting: 498291 Northing: 2614893 Elevation: 95 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 3.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 1.00 | Dark gray to red colored shale bands |
|  | 1.00 | 2.00 | Dark gray to red colored shale bands |
|  | 2.00 | 3.00 | Dark gray to red colored shale bands |

Other details:

1. Volume of the final pit: 3.00 cu.m.

2. Volume of the total excavated material: 3.00 cu.m.

3. Number and details of the channel samples: (3 nos.) MP/20/01 – 0.00 m to 1.00m

MP/20/02 – 1.00 m to 2.00 m

MP/20/03 – 2.00 m to 3.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 30.08.2025 |
| 3. Pit no.: | MP-21 |
| 4. Location geo-coordinates: | Easting: 499492 Northing: 2615021 Elevation: 98 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.18 | Friable sandstone |
|  | 0.18 | 0.58 | Friable sandstone with minor greenish colored grain |
|  | 0.58 | 0.83 | Red colored friable sandstone |
|  | 0.83 | 2.00 | Friable clay |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (2 nos.) MP/21/01 – 0.18 m to 0.58 m

MP/21/02 – 0.83 m to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 30.08.2025 |
| 3. Pit no.: | MP-22 |
| 4. Location geo-coordinates: | Easting: 499329 Northing: 2615473 Elevation: 114 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 1.00 | Glauconitic shale/ clay |
|  | 1.00 | 2.00 | Dark green colored very fine-grained shale/ clay |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (2 nos.) MP/22/01 – 0.00 m to 1.00 m

MP/22/02 – 1.00 m to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 30.08.2025 |
| 3. Pit no.: | MP- 23 |
| 4. Location geo-coordinates: | Easting: 500618 Northing: 2615598 Elevation: 135m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth:2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.19 | Soil cover |
|  | 0.19 | 0.68 | Reddish color sandstone |
|  | 0.68 | 2.00 | Yellowish brown sandstone hard and compact |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (2 nos.)  MP/23/01 – 0.2 m to 0.71 m

MP/23/02 - 0.71 m to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 30.08.2025 |
| 3. Pit no.: | MP- 24 |
| 4. Location geo-coordinates: | Easting: 500414 Northing: 2616382 Elevation:123m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth:1.80 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.10 | Soil cover |
|  | 0.10 | 0.50 | Weathered basalt |
|  | 0.50 | 1.80 | Basalt |

Other details:

1. Volume of the final pit: 1.80 cu.m.

2. Volume of the total excavated material: 1.80 cu.m.

3. Number and details of the channel samples: No sample taken waterlogged

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 30.08.2025 |
| 3. Pit no.: | MP- 25 |
| 4. Location geo-coordinates: | Easting: 502113 Northing: 2616452 Elevation: |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth:2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.06 | Glauconitic sandstone |
|  | 0.06 | 1.30 | Grayish to yellowish brown shale bands |
|  | 1.30 | 2.00 | Carbonaceous shale |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (2 nos.)  MP/25/01 – 0.00 m to 1.00 m

MP/25/02 - 1.00 m to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 01.09.2025 |
| 3. Pit no.: | MP- 26 |
| 4. Location geo-coordinates: | Easting: 500788 Northing: 2618392 Elevation:131m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth:2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.17 | Soil cover |
|  | 0.17 | 0.36 | Shale |
|  | 0.36 | 0.66 | Glauconitic shale |
|  | 0.66 | 0.88 | Ferruginous shale |
|  | 0.88 | 1.15 | Yellowish sandstone |
|  | 1.15 | 2.00 | Fine grained red sandstone |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (2 nos.)  MP/26/01 – 0.18 m to 1.15 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 01.09.2025 |
| 3. Pit no.: | MP-27 |
| 4. Location geo-coordinates: | Easting: 501145 Northing: 2619897 Elevation: 129 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.90 | Brown sandstone |
|  | 0.90 | 2.00 | Marl band |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (1 nos.) MP/27/01 – 0.90 to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 01.09.2025 |
| 3. Pit no.: | MP-28 |
| 4. Location geo-coordinates: | Easting: 500119 Northing: 2619605 Elevation: 115 m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 3.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.40 | Brown sandstone |
|  | 0.40 | 1.00 | Shale/clay band |
|  | 1.00 | 2.00 | Glauconitic clay / shale |
|  | 2.00 | 3.00 | Grayish to olive green colored clay or shale bands |

Other details:

1. Volume of the final pit: 3.00 cu.m.

2. Volume of the total excavated material: 3.00 cu.m.

3. Number and details of the channel samples: (3 nos.) MP/28/01 – 0.40 to 1.00 m

MP/28/02 – 1.00 to 2.00 m

MP/28/03 – 2.00 to 3.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 01.09.2025 |
| 3. Pit no.: | MP-29 |
| 4. Location geo-coordinates: | Easting: 494991 Northing: 2619984 Elevation: |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth: 2.00 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.05 | Soil cover |
|  | 0.05 | 0.19 | Yellow sandstone |
|  | 0.19 | 0.41 | Red to white color shale and clay bands |
|  | 0.41 | 0.74 | Whitish color marl band |
|  | 0.74 | 0.93 | White to yellow sandstone hard and compact |
|  | 0.93 | 2.00 | White to yellow sandstone friable |

Other details:

1. Volume of the final pit: 2.00 cu.m.

2. Volume of the total excavated material: 2.00 cu.m.

3. Number and details of the channel samples: (2 nos.) MP/29/01 – 0.05 m to 1.00 m

MP/29/02 – 1.00 m to 2.00 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 01.09.2025 |
| 3. Pit no.: | MP- 30 |
| 4. Location geo-coordinates: | Easting: 495348 Northing: 2614892 Elevation:160m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth:2.80m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 1.80 | Soil cover |
|  | 1.80 | 2.20 | Sandstone and shale mix |
|  | 2.20 | 2.40 | Yellow sandstone |
|  | 2.40 | 2.80 | Fine-grained white shale/ marl band |

Other details:

1. Volume of the final pit: 2.80 cu.m.

2. Volume of the total excavated material: 2.80 cu.m.

3. Number and details of the channel samples: (1 nos.)  MP/30/01 – 1.80 to 2.80 m

| 1. Name of the investigation: | Mudhan – Khatiya block for Glauconitic sandstone |
| --- | --- |
| 2. Date of commencement: | 01.09.2025 |
| 3. Pit no.: | MP-31 |
| 4. Location geo-coordinates: | Easting: 495185 Northing: 2617283 Elevation:155m |
| 5. Date of completion: |  |

6. Pit top measurement:

| (a) Length: 1m | (b) Breadth: 1m |
| --- | --- |

7. Pit bottom measurement:

| (a) Length: 1m | (b) Breadth:1m | (c) Depth:2.05 m |
| --- | --- | --- |

8. Recorded by:

Comments:

9. Pit Data

| Pit Face/Orientation | From (m) | To (m) | Lithology & Structural Details |
| --- | --- | --- | --- |
| Face-1 (360°) All faces have the same lithology | 0 | 0.85 | Soil cover |
|  | 0.85 | 1.15 | Yellow to red sandstone (friable) |
|  | 1.15 | 2.05 | Ferruginous sandstone |

Other details:

1. Volume of the final pit: 2.05 cu.m.

2. Volume of the total excavated material: 2.05 cu.m.

3. Number and details of the channel samples: (1 nos.)  MP/31/01 – 1.10 m to 2.05 m