PEER REVIEW COMMENTS ON “GEOLOGICAL REPORT ON RECONNAISSANCE SURVEY (G-4 STAGE) FOR GLAUCONITIE SANDSTONE IN MUDHAN-KHATIYA BLOCK (129 Sq Km) DISTRICT-KACHCHH, GUJARAT”

| Sl. No. | Peer Review Comments | MECL Compliance |
| --- | --- | --- |
| 1 | Mention owner’s name on the Cover Page as given at 3.1.0 | Geological report has been prepared as per the MEMC rule 2015 (Reporting of mineral resources- PART IVA format) |
| 2 | Maintain unique Title of the report / Block name, terminology for Text, Annexures and Plates. | Attended-Title of the block has been uniformed |
| 3 | Maintain Unique symbology and Colour combination for same Physical features, Formations, Ore body etc.. | Attended- Maintained |
| 4 | All Photographs should be geodetic photograph. | Attended – Coordinates and village name provided wherever applicable |
| 5 | A plate showing boreholes location, pit and sample location with physical feature only. | Attended- Provided in Plate- III & IV |
| 6 | In Chapter- 1, deliberate a para on field investigation, strike &dip and also on geological formation observations made during field visit. Based on field investigation and geochemical analysis, a note on mineralisation of Glauconite and other important minerals mineralised in the block may be incorporated, in-term of percent with respect to area of the block. | Attended – Added para 1.1.14 & 1.1.15 and the geochemical data already provided in para 1.1.17 and 1.1.19 |
| 7 | In Chapter- 4, elaborate on block boundary delineation and mention the source of the cardinal points of the block in table 4.1. | Attended – Added para 4.1.1 |
| 8 | In Chapter- 5, Para 5.2.2. is missing. | Attended – Formatting error corrected |
| 9 | Content of Chapter-6 can be explained in Chapter-5. No need of separate chapter. | Geological report has been prepared as per the MEMC rule 2015 (Reporting of mineral resources- PART IVA format) |
| 10 | In Chapter- 7, at Parar 7.4.5 it is mentioned that “GSI data is presented in table below” instead write that GSI data is presented at table No. 7.2. | Attended- Formatting error corrected |
| 11 | Text Figure- 7.3 is the copy of plate 03, shall be edited and delete plate details and give details of figure. | Attended- Updated |
| 12 | Before 7.5.0: Detail description on field investigation may be made with traverse line/path with photographs, under Field Study heading. | Attended- Complied in Plate-IV |
| 13 | 7.5.0 DESCRIPTION OF ROCK TYPES PRESENT IN MUDHAN KHATIYA BLOCK: and repetition of 7.5.0 for LITHOLOGIES BELONGING TO BHUJ FORMATION and also repeated in subsequent sub-headings/paras. Correction may be made for LITHOLOGIES BELONGING TO BHUJ FORMATION and subsequent sub-headings/paras. More geodetic photographs shall be included for rock types present in the block under report. Photographs given in this chapter should be referred properly in the text. | Attended- Name of the villages and co-ordinates of the places has been provided |
| 14 | At 7.5.3 it is mentioned as a Deccan Basalt; It may be written as Basalt. | Attended- Updated |
| 15 | Mention and explain the location of photograph 7.14. | Attended- Village name added |
| 16 | At 7.7.8 Interpretation of Bivariate Plots have been done. It is not clear from the description that the data considered for bivariate are discrete in nature or continuous in nature. What is distribution pattern of the data (normal, positive bell shaped or negative bell-shaped curve)? As in general geological data set never be distributed normal with same mean and mode with proper normal curve distribution. Apart from mentioning correlation coefficient and maturity of chemical components, also mention the skewness and kurtosis of the data set and find out the factor which is responsible for mineralisation. | Attended – added para 7.7.12 & 7.7.13 |
| 17 | NGCM- Geochemical anomaly map of K2O data plotted and interpreted for the block under report in Chapter-8, the same may also be tabulated in the chapter. Geochemical anomaly map of K2O data of present Pits, bed rocks and boreholes along with NGCM data shall be prepared and report in this chapter with findings. | Attended- Updated Geochemical anomaly map showing K2O distribution added as Text Figure.10.1 |
| 18 | Details of Exploratory Work carried out by MECL in Mudhan-Khatiya Block, Kachchh District, Gujarat is quite less than the sanctioned quantity in particular to surface geochemical sampling, pitting, DGPS survey for fixation of boreholes, Drilling (m) and Surface samples (Primary and check samples) as per Table No 10.1. Item wise reason thereof should be given in table No.10.1, with proper explanation. | Attended – added in para 10.1.6 |
| 19 | Photo 10.1. shall be upgraded with geodetic photo. | Attended – Location identity has been mentioned |
| 20 | Include a map showing Field traverse line and prominent features and villages etc. | Attended- Complied in Plate-IV |
| 21 | structural observations (strike and dip) are quite less as compared to area of the block (129 sq.km.) (ref. PLATE-IV). It should have been much more than this. | Attended- The structural observation has been recorded based on the availability of the surface exposures, the weathering profile and the soil cover of the area |
| 22 | At 10.2.13, it is mentioned that 32 pits were excavated, whereas the data of 31 pits are tabulated in Table 10.2. | Attended- Typing error |
| 23 | The resource estimation for K₂O-rich glauconite mineralisation in the Mudhan–Khatiya Block has been carried out considering the bedded nature of the deposit and the level of geological confidence achieved through Reconnaissance Stage (G-4) exploration and the resources are classified as Reconnaissance Mineral Resources (UNFC 334), estimated at 142.96 million tonnes with an average grade of 4.10 % K₂O at 3% K₂O cut-off grade. The resources calculated by drawings polygons around boreholes only, but from the field data and geochemical analysis of surface samples and pits, it is evident that the area left out is also enriched with glauconite also. As this is for Reconnaissance Stage (G-4) exploration, resource may be calculated for left out area with different cut-ff grades. | Attended – The geochemical analysis of surface samples and pit samples has been considered for the planning of boreholes and incorporated in the resource estimation. Resource estimation has been done as per the MEMC rule 2015 |
| 24 | Maintain unique Title / Block name and terminology for all the Annexures. | Attended – Updated |
| 25 | ANNEXURE IA- Include the source of the cardinal point co-ordinates. | Attended – Updated |
| 26 | ANNEXURE IB – In the header of the table, mention title of each column or mention etc. after total depth. Angle of each borehole should be -90.0 as each boreholes drilled vertically, from surface to below the surface. | Attended – Updated |
| 27 | ANNEXURE II – Correct the header to ANNEXURE II, instead of ANNEXURE IIIA. Data provided under the column with heading “Thickness REC.” is the same as true thickness/extrapolated thickness, misleads with recovered thickness. May like to delete the column. | Attended- The annexure has been maintained according to the standard reporting format |
| 28 | ANNEXURE IVA: Results of all samples are not included; the reason thereof may be mentioned in the note. | Attended- Result of all the samples have been provided |
| 29 | ANNEXURE IVB - Different analysis results observed for different samples of the same point having the same co-ordinates, for example P-02, P-08, P-11, P-14, P-15, P-20, P-21, etc. Results may be verified and explained, as data having >3% of K2O% was considered for resource estimation. P-05 data is not given in the Annexure, but the location is given in Plate-III. Insert in the Annexure with a remark for non-consideration. | Attended- The depth-wise sampling data has been updated in Annexure IVB. The P-05 contains only soil cover and feldspathic sandstone, which is why it's not analysed and considered as a negative. A detailed interpretation is given in the plate-VI |
| 30 | ANNEXURE IVA & B may be analysed together as most of the P point data are not matching with MK data (e.g. P-08, MK-1321, 1326) | Attended- The “P” data points represent pit samples collected from varying depths, whereas the “MK” data correspond to bedrock samples obtained from the surface. This difference in sampling depth accounts for the observed variation in geochemical values. |
| 31 | ANNEXURE IVD – Not legible. | Attended- Updated |
| 32 | For all the plates, maintain Unique symbology and Colour Index for same Physical features, Formations, Ore body etc.. | Attended- Updated |
| 33 | Include Plate details at the bottom left corner of the plate and make a unique format for all the plate for example: - Customer/Owner: - Title of the Report: - Subject: - Prepared by : Signature: - Checked by : Signature: - Approved by : Signature: - Drawing No. Plate No.I/II/III/IV/V | Attended- It is a standard format for map details (monogram), typically placed at the bottom-right corner of the plate. Therefore, no changes have been made to the plate details, as suggested. |
| 34 | In Plate-II, Basalt, Basalt Unclassified and Olivine Basalt are having same colour index, difficult to differentiate. It should be only Basalt and use only one colour. Give Plate details as mentioned above. | Attended- Updated |
| 35 | Write PLATE-III/IV instead of PLATE-03/04. | Attended- Updated |