1.0 EXECUTIVE SUMMARY

The present investigation was carried out at G3 level of exploration for Bauxite/Aluminous Laterite, Gallium (Ga), Vanadium (V), and Rare Earth Elements (REE) in Naredi block. The block is located in Abdasa taluk of Kachchh district, Gujarat.

This project was awarded by the Commissioner of Geology and Mines (CGM), Gujarat, to the NPEA Geo Marine Solutions Private Limited, Mangaluru, to upgrade and develop the existing mineral resources in the Naredi region under the programme “Gujarat’s Mineral Wealth – A Responsible Exploration and Development Paradigm”.

Geologically, the area comprises of sediments belonging to the Katrol and the Bhuj formations of Mesozoic age, overlain by volcanic rocks representing the Deccan Volcanics; the laterite represented by the Matanomadh Formation of Palaeocene; The supratrappean Matanomadh Formation which is of Palaeocene age, comprises white, soft aluminous clays, lavender cellular ash, hard laterite with concretionary surfaces and bauxite. The Fulra Formation overlying the Matanomadh Formation comprises nummulitic limestone, gypseous shale and variegated clay.

detailed geological mapping revealed that >90% of the block was covered with soil, with very small outcrops of laterite, bauxite & limestone in the north central, north eastern & eastern part of the block occurs in the form of pockets and lenses outside the eastern margin of the block. However, within the block the area is predominantly covered by soil, with sporadic exposures of laterite and limestone.

First phase of drilling boreholes were to be drilled on 400m X 400m spacing. Drilling was taken up from the eastern margin of the block. First 06 boreholes were drilled on a 200mX400m, the results of chemical analysis of 13 samples from all the runs in one of the six initially drilled boreholes and 10 representative samples from 5 other boreholes, showed anomalous values of Sc and Ga in them, and since laterite/ bauxite was only encountered in one run of half meter length.

Based on the above results the TCC suggested that the project may be re-oriented to explore the block for clay hosted Sc, Ga , REE and associated critical elements. The committee also suggested to cover the entire block on 400m X 400m grid.

In total, twenty-seven (27) boreholes were drilled, achieving an aggregate meterage of 567 m. The obtained drill core lithologies were broadly classified as Soil, Limestone, Laterite/bauxite, Lateritic Clay, Clay.

168 Samples from 27 boreholes (23 laterite, 7 limestones and 136 clay& or lateritic clay samples and 02 soil samples) were analyzed for major oxides and LOI by XRF. For analysis of minor, trace and REE by ICP OES/MS, 342 Samples from 27 boreholes (23 no of laterite, 9 limestones and 307 clay& or lateritic clay and 2 soil) were analyzed.

The detailed chemical analysis showed that as per the CGPB Threshold values (64th CGPB Meeting, 2025) TREEO+SC2O3+Y2O3 was more than 350 ppm in 11 out 27 boreholes. Ga enrichment above 30 ppm This REE & Ga enrichment is seen in the immature clays that are seen in the eastern side of the block, Sc distribution does not show any pattern.

As the project is re oriented to clay hosted REE,Ga & Sc the resource was estimated for the same using Geovia Surpac software. For Resource Estimation of (TREEO+Sc2O3+Y2O3) the cut-off grade of above 350 ppm (0.035%) has been considered.	For Ga, two cut of grades of 30 ppm (Industrial cut-off) and 50 ppm were used. Similarly, for Sc 20ppm (Industrial cut-off) and 50ppm has been used.

The estimated resources indicate there is a total resource of 5.07195 MMT at a weighted avg. grade of 798.912 ppm. of TREEO+ Sc2O3+Y2O3. Sc resource of 18.9002 MMT is seen at 20ppm cut off and 3.374 MMT is seen at 50ppm cut off. The Ga resource of 11.5534 MMT is seen at 30 ppm cutoff and 1.5391MMT is seen at 50PPM cut off. The details of the estimated resources for different commodities is given in the following tables.

- The resource estimation table for (TREEO+Sc2O3+Y2O3)>350ppm for the whole block(3.92Sq.km).

| Table 1.1 The resource estimation table for (TREEO+Sc2O3+Y2O3)>350ppm. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Cut off | Volume (Cu.m) | BD | Tonnes | Avg Grade (ppm) |
| Clay | above 350ppm (TREEO+ Sc2O3 Y2O3) | 3649843.75 | 1.22 | 4452809.375 | 639.7288 |
| Laterite |  | 107187.5 | 1.69 | 181146.875 | 1280.6437 |
| Limestone |  | 339531.25 | 1.29 | 437995.3125 | 2217.9942 |
| Grand Total |  |  |  | 5071951.563 | 798.91261 |
| Total resource of TREEO+ Sc2O3+Y2O3 is 5.07195 MMT at a weighted avg. grade of 798.912ppm. |  |  |  |  |  |

- Total Resource Estimation of Sc at Cut-Off 20ppm & 50ppm (For whole block 3.92Sq.km).

| Table 1.2 Total Resource Estimation of Scandium > 20 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 13667656.25 | 1.22 | 16674540.63 | 16.6745 | 36.5273 |
| Laterite | 940937.5 | 1.69 | 1590184.375 | 1.5902 | 32.5888 |
| Limestone | 492656.25 | 1.29 | 635526.562 | 0.6355 | 37.6435 |
|  |  |  |  | 18.9002 |  |

| Table 1.3 Total Resource Estimation of Scandium > 50 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 2469843.75 | 1.22 | 3013209.375 | 3.0132 | 60.1948 |
| Laterite | 74687.5 | 1.69 | 126221.875 | 0.1262 | 55.6245 |
| Limestone | 181875 | 1.29 | 234618.75 | 0.2346 | 54.4255 |
|  |  |  |  | 3.374 |  |

- Total Resource Estimation of Gallium Cut-Off 30ppm & 50ppm (For whole block 3.92Sq.km).

| Table 1.4 Total Resource Estimation of Gallium > 30 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 7496562.5 | 1.22 | 9145806 | 0.914806 | 38.813 |  |
| Laterite | 1222812.5 | 1.69 | 2066553 | 2.066553 | 48.72 |  |
| Limestone | 264375 | 1.29 | 341043.8 | 0.0340438 | 39.823 |  |
|  |  |  |  | 3.0154028 |  |  |

| Table 1.5 Total Resource Estimation of Gallium> 50 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 713125 | 1.22 | 870012.5 | 0.8700125 | 56.359 |  |
| Laterite | 395937.5 | 1.69 | 669134.4 | 0.6691344 | 56.77 |  |
| Limestone | 0 | 1.29 | 0 | 0 | NA |  |
|  |  |  |  | 1.5391469 |  |  |

Based on the Geological mapping, drilling, chemical analysis of drilled samples and the resource estimation, a zone has been delineated as high potential zone(Fig 1.1 shown below)

Fig 1.1  Map of  delineated zone (TREEO+SC2O3+Y2O3)>350 ppm

Resource estimated for the above delineated zone is shown in the table below.

| Table 1.6 Total Estimated TREEO+ Sc2O3+Y2O3 resource in the block delineated zone recommended for auction block |  |  |
| --- | --- | --- |
| Lithology | TREEO+ Sc2O3+Y2O3 | Tonnes (MMT) |
| Total resource Estimated in the Naredi block | > 350 ppm | 5.07 |
| Laterite | > 350 ppm | 0.18 |
| Clay | > 350 ppm | 4.22 |
| limestone | > 350 ppm | 0.34 |
| Total resource in the demarcated block |  | 4.74 |
| The demarcated zone accounts for 93.52% of the total estimated resource. |  |  |

As can be seen from the above table, the delineated block has an estimated (TREEO+ Sc2O3+Y2O3) resource of 4.74 MMT out of a total resource of 5.07 MMT. This accounts for 93.52% of the total estimated resource. This zone has been recommended as the block for auction. The estimated resources for Scandium and Gallium for the delineated block are given in the tables below.

| Table 1.7 Estimated Resource of Scandium > 20 ppm in Delineated zone(0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 9667656.25 | 1.22 | 11794540.63 | 11.795 | 40.8819 | 39.84932821 |
| Laterite | 939843.75 | 1.69 | 1588335.938 | 1.588 | 32.594 |  |
| Limestone | 432031.25 | 1.29 | 557320.312 | 0.557 | 38.6744 |  |
| Total |  |  |  | 13.94 |  |  |

| Table 1.8 Estimated Resource of Scandium > 50 ppm in Delineated zone(0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 2405625 | 1.22 | 2934862.5 | 2.935 | 60.2331 | 59.6662740147007 |
| Laterite | 74687.5 | 1.69 | 126221.875 | 0.126 | 55.6245 |  |
| Limestone | 175000 | 1.29 | 225750 | 0.226 | 54.5571 |  |
| Total |  |  |  | 3.287 |  |  |

| Table 1.9 Estimated Resource of Gallium > 30 ppm in Delineated zone(0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 6808906.25 | 1.22 | 8306865.625 | 8.307 | 39.2304 | 41.0880128 |
| Laterite | 1221718.75 | 1.69 | 2064704.688 | 2.065 | 48.7225 |  |
| Limestone | 249687.5 | 1.29 | 322096.875 | 0.322 | 40.0572 |  |
| Total |  |  |  | 10.694 |  |  |

| Table 1.10 Estimated Resource of Gallium > 50 ppm in Delineated zone(0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 707656.25 | 1.22 | 863340.625 | 0.863 | 56.3721 | 56.54561931 |
| Laterite | 395937.5 | 1.69 | 669134.375 | 0.669 | 56.7695 |  |
| Limestone | 0 | 1.29 | 0 | 0 | NA |  |
| Total |  |  |  | 1.532 |  |  |

Further it is concluded that the block does not have any potential for bauxite, laterite & limestone.

1.0 कार्यकारी सारांश

वर्तमान अन्वेषण कार्य नरेडी ब्लॉक में बॉक्साइट/एल्यूमिनस लेटराइट, गैलियम (Ga), वैनेडियम (V) एवं रेयर अर्थ एलिमेंट्स (REE) हेतु G3 स्तर पर किया गया। यह ब्लॉक गुजरात राज्य के कच्छ जिले के अब्दासा तालुका में स्थित है।

यह परियोजना “गुजरात की खनिज संपदा – उत्तरदायी अन्वेषण एवं विकास प्रतिमान” कार्यक्रम के अंतर्गत आयुक्त, भूविज्ञान एवं खनन (CGM), गुजरात द्वारा एनपीईए जियो मरीन सॉल्यूशन्स प्रा. लि., मंगलुरु को प्रदान की गई।

भूवैज्ञानिक दृष्टि से यह क्षेत्र मेसोज़ोइक युग की कट्रोल एवं भुज संरचनाओं के अवसादी शैलों से निर्मित है, जिनके ऊपर डेक्कन ज्वालामुखीय शैल अध्यारोपित हैं। इनके ऊपर पेलियोसीन युग की मातानोमाध संरचना की लेटराइट विकसित है। सुप्राट्रैपियन मातानोमाध संरचना में श्वेत, कोमल एल्यूमिनस क्ले, लैवेंडर रंग की कोशिकीय राख, कंक्रीशनयुक्त कठोर लेटराइट एवं बॉक्साइट पाए जाते हैं। इसके ऊपर फुलरा संरचना स्थित है, जिसमें न्यूम्युलिटिक चूना पत्थर, जिप्सीयस शेल एवं बहुरंगी क्ले सम्मिलित हैं।

विस्तृत भूवैज्ञानिक मानचित्रण से ज्ञात हुआ कि ब्लॉक का >90% भाग मृदा से आच्छादित है तथा लेटराइट, बॉक्साइट एवं चूना पत्थर के अत्यंत सीमित अनावृत भाग उत्तर-मध्य, उत्तर-पूर्व एवं पूर्वी भाग में पॉकेट एवं लेंस के रूप में पाए जाते हैं। ब्लॉक के भीतर अधिकांश क्षेत्र मृदा से आच्छादित है, जबकि कहीं-कहीं लेटराइट एवं चूना पत्थर के विरल अनावृत भाग देखे गए।

प्रथम चरण में ड्रिलिंग 400मी × 400मी ग्रिड पर प्रस्तावित थी। प्रारंभिक 06 बोरहोल 200मी × 400मी ग्रिड पर पूर्वी सीमा से ड्रिल किए गए। इन बोरहोल से प्राप्त नमूनों के रासायनिक विश्लेषण से Sc एवं Ga के असामान्य मान प्राप्त हुए, जबकि लेटराइट/बॉक्साइट केवल 0.5 मीटर मोटाई के एक रन में पाया गया।

उक्त परिणामों के आधार पर TCC ने परियोजना को क्ले-होस्टेड Sc, Ga, REE एवं संबद्ध क्रिटिकल तत्वों के अन्वेषण हेतु पुनः अभिमुखित करने की अनुशंसा की तथा संपूर्ण ब्लॉक को 400मी × 400मी ग्रिड पर कवर करने का सुझाव दिया।

कुल 27 बोरहोल ड्रिल किए गए, जिनकी कुल ड्रिलिंग 567 मीटर रही। ड्रिल कोर लिथोलॉजी को मृदा, चूना पत्थर, लेटराइट/बॉक्साइट, लेटरिटिक क्ले एवं क्ले में वर्गीकृत किया गया।

27 बोरहोल से 168 नमूनों का प्रमुख ऑक्साइड एवं LOI हेतु XRF द्वारा विश्लेषण किया गया तथा 342 नमूनों का सूक्ष्म, ट्रेस एवं REE तत्वों हेतु ICP-OES/MS द्वारा विश्लेषण किया गया।

रासायनिक विश्लेषण से ज्ञात हुआ कि CGPB (64वीं बैठक, 2025) के अनुसार TREEO+Sc₂O₃+Y₂O₃ का मान 350 ppm से अधिक 27 में से 11 बोरहोल में पाया गया। 30 ppm से अधिक गैलियम का संवर्धन ब्लॉक के पूर्वी भाग की अपरिपक्व क्ले में देखा गया, जबकि Sc का वितरण किसी स्पष्ट पैटर्न का अनुसरण नहीं करता है।

परियोजना के पुनः अभिमुखन के पश्चात संसाधन का आकलन Geovia Surpac सॉफ्टवेयर द्वारा किया । सम्पूर्ण ब्लॉक (3.92 वर्ग किमी) के लिए अनुमानित संसाधन नीचे दी गई सारणियों में प्रस्तुत हैं।

| सारणी 1.1:(TREEO+Sc₂O₃+Y₂O₃) > 350 ppm हेतु संसाधन आकलन सारणी |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| लिथोलॉजी | कट-ऑफ | आयतन (घन मी.) | BD | टन | औसत ग्रेड (ppm) |
| क्ले | >350 ppm | 3649843.75 | 1.22 | 4452809.375 | 639.7288 |
| लेटराइट |  | 107187.5 | 1.69 | 181146.875 | 1280.6437 |
| चूना पत्थर |  | 339531.25 | 1.29 | 437995.3125 | 2217.9942 |
| कुल योग |  |  |  | 5071951.563 | 798.91261 |

| सारणी 1.2: Sc संसाधन (>20 ppm) – सम्पूर्ण ब्लॉक (3.92 वर्ग किमी) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| लिथोलॉजी | आयतन | BD (g/cc) | टन | MMT | औसत Sc (ppm) |
| क्ले | 13667656.25 | 1.22 | 16674540.63 | 16.6745 | 36.5273 |
| लेटराइट | 940937.5 | 1.69 | 1590184.375 | 1.5902 | 32.5888 |
| चूना पत्थर | 492656.25 | 1.29 | 635526.562 | 0.6355 | 37.6435 |
| कुल |  |  |  | 18.9002 |  |

| सारणी 1.3: Sc संसाधन (>50 ppm) – सम्पूर्ण ब्लॉक (3.92 वर्ग किमी) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| लिथोलॉजी | आयतन | BD | टन | MMT | औसत Sc (ppm) |
| क्ले | 2469843.75 | 1.22 | 3013209.375 | 3.0132 | 60.1948 |
| लेटराइट | 74687.5 | 1.69 | 126221.875 | 0.1262 | 55.6245 |
| चूना पत्थर | 181875 | 1.29 | 234618.75 | 0.2346 | 54.4255 |
| कुल |  |  |  | 3.374 |  |

| सारणी 1.5: Ga संसाधन (>50 ppm) – सम्पूर्ण ब्लॉक (3.92 वर्ग किमी) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| लिथोलॉजी | आयतन | BD | टन | MMT | औसत Ga (ppm) |
| क्ले | 713125 | 1.22 | 870012.5 | 0.8700125 | 56.359 |
| लेटराइट | 395937.5 | 1.69 | 669134.4 | 0.6691344 | 56.77 |
| चूना पत्थर | 0 | 1.29 | 0 | 0 | NA |
| कुल |  |  |  | 1.5391469 |  |

| सारणी 1.4: Ga संसाधन (>30 ppm) – सम्पूर्ण ब्लॉक (3.92 वर्ग किमी) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| लिथोलॉजी | आयतन | BD | टन | MMT | औसत Ga (ppm) |
| क्ले | 7496562.5 | 1.22 | 9145806 | 0.914806 | 38.813 |
| लेटराइट | 1222812.5 | 1.69 | 2066553 | 2.066553 | 48.72 |
| चूना पत्थर | 264375 | 1.29 | 341043.8 | 0.0340438 | 39.823 |
| कुल |  |  |  | 3.0154028 |  |

भूवैज्ञानिक मानचित्रण, ड्रिलिंग, ड्रिल नमूनों के रासायनिक विश्लेषण तथा संसाधन आकलन के आधार पर एक क्षेत्र को उच्च संभाव्यता क्षेत्र (High Potential Zone) के रूप में सीमांकित किया गया है (चित्र 1.1)।

चित्र 1.1: सीमांकित क्षेत्र का मानचित्र (TREEO+Sc₂O₃+Y₂O₃ > 350 ppm)

उपरोक्त सीमांकित क्षेत्र के लिए अनुमानित संसाधन नीचे दी गई सारणी में प्रदर्शित है।

| सारणी 1.6: चिन्हित क्षेत्र हेतु संसाधन |  |  |
| --- | --- | --- |
| लिथोलॉजी | TREEO+Sc₂O₃+Y₂O₃ | टन (MMT) |
| नरेडी ब्लॉक का कुल संसाधन | >350 ppm | 5.07 |
| लेटराइट | >350 ppm | 0.18 |
| क्ले | >350 ppm | 4.22 |
| चूना पत्थर | >350 ppm | 0.34 |
| चिन्हित ब्लॉक कुल |  | 4.74 |

उपरोक्त सारणी से स्पष्ट है कि सीमांकित ब्लॉक में (TREEO+Sc₂O₃+Y₂O₃) का अनुमानित संसाधन 5.07 MMT के कुल संसाधन में से 4.74 MMT है, जो कुल अनुमानित संसाधन का 93.52% है। इस क्षेत्र को नीलामी हेतु ब्लॉक के रूप में अनुशंसित किया गया है।

सीमांकित ब्लॉक के लिए अनुमानित संसाधन नीचे दी गई सारणी में प्रस्तुत हैं:

| सारणी 1.7: Sc संसाधन (>20 ppm) – चिन्हित क्षेत्र (0.89 वर्ग किमी) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| लिथोलॉजी | आयतन | BD | टन | MMT | औसत Sc (ppm) | कट-ऑफ ग्रेड |
| क्ले | 9667656.25 | 1.22 | 11794540.63 | 11.795 | 40.8819 | 39.8493 |
| लेटराइट | 939843.75 | 1.69 | 1588335.938 | 1.588 | 32.594 |  |
| चूना पत्थर | 432031.25 | 1.29 | 557320.312 | 0.557 | 38.6744 |  |
| कुल |  |  |  | 13.94 |  |  |

| सारणी 1.8: Sc संसाधन (>50 ppm) – चिन्हित क्षेत्र (0.89 वर्ग किमी) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| लिथोलॉजी | आयतन | BD | टन | MMT | औसत Sc (ppm) | कट-ऑफ |
| क्ले | 2405625 | 1.22 | 2934862.5 | 2.935 | 60.2331 | 59.6663 |
| लेटराइट | 74687.5 | 1.69 | 126221.875 | 0.126 | 55.6245 |  |
| चूना पत्थर | 175000 | 1.29 | 225750 | 0.226 | 54.5571 |  |
| कुल |  |  |  | 3.287 |  |  |

| सारणी 1.9: Ga संसाधन (>30 ppm) – चिन्हित क्षेत्र (0.89 वर्ग किमी) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| लिथोलॉजी | आयतन | BD | टन | MMT | औसत Ga (ppm) | कट-ऑफ |
| क्ले | 6808906.25 | 1.22 | 8306865.625 | 8.307 | 39.2304 | 41.088 |
| लेटराइट | 1221718.75 | 1.69 | 2064704.688 | 2.065 | 48.7225 |  |
| चूना पत्थर | 249687.5 | 1.29 | 322096.875 | 0.322 | 40.0572 |  |
| कुल |  |  |  | 10.694 |  |  |

| सारणी 1.10: Ga संसाधन (>50 ppm) – चिन्हित क्षेत्र (0.89 वर्ग किमी) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| लिथोलॉजी | आयतन | BD | टन | MMT | औसत Ga (ppm) | कट-ऑफ |
| क्ले | 707656.25 | 1.22 | 863340.625 | 0.863 | 56.3721 | 56.5456 |
| लेटराइट | 395937.5 | 1.69 | 669134.375 | 0.669 | 56.7695 |  |
| चूना पत्थर | 0 | 1.29 | 0 | 0 | NA |  |
| कुल |  |  |  | 1.532 |  |  |

अतः यह निष्कर्ष निकाला गया है कि उक्त ब्लॉक में बॉक्साइट, लेटराइट एवं चूना पत्थर के लिए कोई संभाव्यता नहीं है।

2.0   Introduction

Gujarat is endowed with a wide variety of mineral resources, underpinned by its complex and varied geology that spans from ancient Precambrian basement to recent sedimentary and coastal deposits. The state is also a significant producer of bauxite, with lateritic deposits developed over Deccan Trap basalt in Kachchh, Jamnagar, and Junagadh, suitable for alumina and aluminium production as well as refractories; in Kachchh, sizeable occurrences are reported in areas such as Abdasa and Mandvi talukas.

Kachchh is one of Gujarat’s most mineral-rich districts, hosting significant deposits of lignite, bauxite, gypsum, limestone, china clay, silica sand, and bentonite, contributing substantially to the state’s mineral output. Lignite fields such as Panandhro and Matanomadh are of economic importance for thermal power generation.

. 	Lateritic bauxite, developed by in situ weathering of Deccan basalts, occurs extensively across the district, while limestone from Jurassic and younger formations supports cement production. The laterite and bauxite formed on the cretaceous deccan basalts are potential hosts for rare earth elements, vanadium, gallium, nickel and cobalt.

Geologically, the area comprises of sediments belonging to the Katrol and the Bhuj formations of Mesozoic age, overlain by volcanic rocks representing the Deccan Volcanics; the laterite represented by the Matanomadh Formation of Palaeocene; The supratrappean Matanomadh Formation which is of Palaeocene age, comprises white, soft aluminous clays, lavender cellular ash, hard laterite with concretionary surfaces and bauxite. The Fulra Formation overlying the Matanomadh Formation comprises nummulitic limestone, gypseous shale and variegated clay. The Gaj Formation rests unconformably on the Fulra limestone and forms extensive plains in the western part.

The earlier works of Commissioner of Geology and Mines(CGM), Gujarat in the areas around Naredi village reported, occurrence of bauxite with average Al2O3 content of 49.77% and average SiO2 is 2.49% and estimated bauxite resource of 27.85 MT. To cater the present-day requirements the CGM government decided to explore and upgrade the resources in the region and hence a G3 level of exploration work was recommended by the state government in the present block.

Naredi block  is situated towards the south west of Bhuj city and has an area of 3.92 Sq.km, the block has flat cultivated lands covered by soil cover on the surface with very less outcrops.

2.1 Details of the Project

This project was initiated by Commissioner of Geology and Mines (CGM), Gujarat State. This project is based on the earlier work carried out by them in the field seasons 1964-65 (the details of their studies is described in detail; in the section 4.1 of this report). The current project was initially aimed at the estimation of Bauxite, Gallium, Vanadium and also the associated REE elements, which are reportedly present in the block, and to expand this prospect to a deposit scale. However, as we discussed in chapter 9, during the course of the project it was observed that the block was covered with extensive clay sediments; hence the project was reoriented to explore for clay hosted scandium, gallium, REE and associated critical elements.

2.2 Investigating Agency

M/s. Geo Marine Solutions Pvt. Ltd (CIN U74900KL2011PTC28241) is a Notified Private Exploration Agency (NPEA), Science and Technology Services Company with a vision to take proactive role in nation building by providing latest scientific knowledge and technological services in the field of Earth System Science. The company draws its inspiration and ability to undertake challenging projects from team leaders in different disciplines of Earth Sciences who are former scientists from prestigious organizations like Geological Survey of India, National institute of Oceanography etc. Each team leader of Geo Marine Solutions is a leading scientist in his / her area of expertise (domain experts) and brings in more than three decades of experience in Offshore- and Onshore-Geo Scientific Survey and Exploration.

Name and address of the NPEA

Geo Marine Solutions Pvt Ltd

15-17-909/9, 5th Cross, Shivabagh Kadri, Mangalore-575005, Karnataka, INDIA

Mobile No: +91-9038534560, +91-8277363919

E-Mail id: mail@geomarinesolutions.in, oed@geomarinesolutions.in

2.3 Objectives of the present investigation are as follows

- To carryout integrated topographical and geological survey on 1:4000 scale and

delineate the rock types present in the block.

- Core drilling is undertaken to understand the vertical continuity of Bauxite up to a depth of 20 meters.

- To understand the geochemical characteristics of Bauxite, Clay minerals, limestone

and other associated Trace and Ree using XRF and ICP-MS.

- Block modelling will be carried out and the resource estimation will be done for Bauxite, Gallium, Vanadium and REE (whichever is present in economically viable quantities).

2.4 Basis for taking up investigation

CGM Gujarat explored Naredi area by means of pitting, trenching, and drilling and had estimated a resource of 27.85 MT. Chemical analysis data of the bauxite shows high Al2O3 (average 49.77%) with low SiO2 percentage (average 2.49%). As the present block is situated at about 500m to the west of existing bauxite leases of GMDC, CGM recommended for the upgradation of resources in the block by carrying out a G3 level of exploration.

Following the CGM’s recommendation, a reconnaissance field work was conducted in Naredi block by Geo Marine Solutions Pvt Ltd in 30th -31st October 2024. Two grab samples were collected– one of the samples was from an active Bauxite quarry of GMDC which is situated at about 1.0 km from the southern margin of the block. The other sample was collected within the block, from Fulra Formation. The aluminous laterite sample from the GMDC quarry analyzed 35.5% Al2O3; Gallium was 47ppm which is above industry cut off. The other sample was a limestone from Fulra formation with CaO 45.39% and SiO2 10.94% which may qualify as a low-grade limestone for cement industry.

2.5 The Nature and Quantum of Work Proposed and Carried Out

The nature and quantum of work proposed and carried out is given in the table below

| Table 2.1: Nature and Quantum of work proposed and carried out |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl. No. | Nature of work* | Target assigned | Target Achieved | Remarks |
| 1 | DGPS Survey-(1:4000 Scale) (DGPS/Total Station Survey) (sq. km) | 3.92 sq. km. | 3.92 sq.km. | Topographic survey completed |
| 2 | Geological Mapping-(1:4000 Scale) (LSM/DM) (sq. km) | 3.92 sq.km. | 3.92 sq.km. | Geological mapping completed |
| 3 | Geophysical Surveys (Gravity, Magnetic Stn.), Nos | NA | NA | NA |
| 4 | (a) Surface exploration- Pitting (20 pit up to 2 m) | NA | NA | Not Executed |
|  | (b) Sub Surface exploration- Drilling (m) | 910 M | 567 M | Drilling of 567m for 27 Boreholes completed |
| 5 | (a) Preparation of Thin sections (Nos.) | Not Executed | Not Executed | Not Executed |
|  | (b) Preparation of polished sections (Nos.) | Not Executed | Not Executed | Not Executed |
|  | (d) Petrographic/Ore Microscopic study (Nos.) | Not Executed | Not Executed | Not Executed |
|  | (e) XRD | 10 | 06 | 06 Samples of XRD completed |
|  | (f) Sp. Gravity | - | 14 | 12 clay, 1 laterite and 1 Limestone sample Completed |
| 6 | (a) Primary +10% Check Sample for Oxides an LOI by XRF technique | 333 | 168 | 168 Primary samples completed. |
|  | (b) Primary + check Samples including composite samples for REE, Ga, V and Other Trace elements: by ICPMS | 333 | 342 | 342 Primary samples completed |
|  | (C) Composite samples | Not Executed | Not Executed | Not Executed |
| 7 | Core Preservation | 910 M | 567 M | Recovered core from 567m drilling is preserved |
| 8 | Geological Report Writing | 5 Hard copies with soft copy | - | Completed |

  - Technical personnel involved in the project

Technical personnel involved in the project is given in the table 2.2

| Table 2.2: Details of Technical persons engaged in exploration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl. no | Name | Qualification | Designation | Main contributions |
| 1 | Dr. Fareeduddin | PhD in Geology with 41 years of experience in Mineral Exploration and Geological mapping | Former director in GSI, Principal Consultant- Geo Marine Solutions Pvt Ltd. and NABET approved Project coordinator | Guidance throughout the field season, sample processing, analysis and report writing |
| 2 | Dr. A. V. Keshava Prasad | PhD in Geology with 35 years of experience in Mineral Exploration and Geological mapping | Chief Geologist (Lead) Exploration, Geo Marine Solutions Pvt. Ltd. | Supervision and guidance in geological mapping, topographic survey, core drilling, selection of samples for chemical analysis, and interpretation of data, map preparation, ore modelling analysis and report writing. |
| 3 | Dr. B. M. Thanmaya | PhD. in Earth Science, with 06 years of experience | Geologist, M/s. Geo Marine Solutions Pvt. Ltd. | core drilling, selection of samples for chemical analysis, and interpretation of data, map preparation, ore modelling analysis and report writing. |
| 4 | Nagarjun Patil | M.Sc. Applied Geology with 01-year experience | Geologist, M/s. Geo Marine Solutions Pvt. Ltd. | Geological mapping, topographic survey, core drilling, selection of samples for chemical analysis, and interpretation of data, map preparation, ore modelling analysis and report writing. |
| 5 | Guruprasad S. | M.Sc. Geology with 06 years of experience | Geologist, M/s. Geo Marine Solutions Pvt. Ltd. | Interpretation of data, map preparation, ore modelling analysis. |
| 6 | Abhilash A Divatagi | M.Sc. Applied Geology with 01-year experience | Geologist, M/s. Geo Marine Solutions Pvt. Ltd. | Geological mapping, topographic survey, core drilling, selection of samples for chemical analysis, and interpretation of data, map preparation, ore modelling analysis and report writing. |
| 7 | Rishi Rajgor | M.Sc. Geology with 03 years of Experience | Geologist, M/s. Geo Marine Solutions Pvt. Ltd. | Geological mapping, topographic survey, core drilling, selection of samples for chemical analysis. |

2.7 Mode of operation of different work components and associated agencies

During this investigation, the work was executed in three different stages, namely pre-field studies, field studies and post-field studies. The different components and procedures followed in each stage are discussed below.

- Pre-field studies: This stage included preliminary desktop research of available literature in the different data repositories like GSI Portal, NGDR, NMEDT and Bhukosh on laterite and bauxite exploration projects; compiling a database of toposheet(41E/04), geological maps, and previous reports related to the geology.

- Field studies: The field studies were undertaken between the period from March 2025 to October 2025.

  - Reconnaissance survey was conducted initially to assess the actual conditions of roads, accessibility in different parts of the area, communication links therein, hydrography (drainage pattern and nearby streams) and climatic conditions.

  - Topographic survey in the study area was carried out with three sets of Geomate Real Time Kinematic–Global positioning system (RTK-GPS) receivers.

  - A total of 3.92 sq.km area was covered by systematic Geological mapping on 1:4000 scale.

  - Core drilling of boreholes on a 400x400m grid pattern as per MEMC (2021) rule was carried out and the total meterage achieved was 567m.

  - Detailed core logging of all the 27 Boreholes

- Post field studies: The post field study included (i) data compilation, (ii) Core log data analysis (iii) sample preparation for XRF, ICP-MS, bulk density (Lab- based) (iv) processing and interpretation of the laboratory result data & QA-QC (v) preparation of resource Model, Block modelling and estimation of Resource using Surpac module, vi) finalization of maps and preparation of report.

Other agencies involved in the project

All field components like geological mapping, topographic survey, core drilling, sample collection as well as post field components like, sample preparation, resource estimation, block modelling and petrographic studies were done in-house. To carry out the laboratory analysis such as XRF, ICP-MS, Bulk density, other agencies were used. The details are given below.

- Chemical analysis of primary samples:

M/s. Bureau of Veritas, Inspectorate Griffith India Pvt. Ltd., Plot No 73, Sector 11, GIDC Gandhidham-370201, Kutch, Gujarat.

IGI is a NABL accredited analytical laboratory, offers biological, chemical and mechanical testing services along with validations, research and failure investigations. Bureau Veritas is a world leading inspection and testing organization for the growing commodities industry. Inspectorate Griffith India is the core of the Bureau Veritas Commodities Division in India.

Chemical analysis of 168 samples for major oxide using WDXRF followed by lithium- borate fusion bead, and 342 samples by ICP-MS/OES for niobium, Gallium, scandium, REE and other trace element analysis were carried out.

- Chemical analysis of check samples and XRD:

Shiva Analyticals (India) Private Ltd., Hoskote, Bangalore-562114 (NABL accredited)

Founded in 1997, Shiva Analyticals is NABL accredited company and is a leading analytical testing laboratory in India, based in Bangalore. Specializing in diverse sectors such as pharmaceuticals, food & agriculture, ores & minerals, petroleum, environmental, and materials testing, Shiva Analytical offers comprehensive solutions using advanced instrumentation and expert analysis. With a focus on regulatory compliance, quality control, and innovative testing methods, the lab supports clients in ensuring product safety and efficacy, while also contributing valuable insights to the mining industry and addressing environmental and industrial hygiene concerns. As part of the Cotecna Group (Switzerland), Shiva Analytical is committed to enhancing service delivery and customer satisfaction in a dynamic market.

- Bulk density determination:

Matter Material Testing & Research Laboratory (P) Ltd. (NABL accredited), Mini Bypass Road, Thiruvannur Nada (PO), Thiruvannur, Kozhikode Kerala, 673029.

Matter Laboratory is exceptionally well equipped with a group of Engineers and Technical Professionals committed to provide best in class Engineering & Technical solutions across the region for construction and other industries through material testing, inspection and consultancy services.

Activities of Matter Laboratory include wide range of construction materials testing, chemical analysis &microbiological examination of water, food & food products, environmental testing, metallurgical (steel) testing, non-destructive concrete testing and forensic investigation of concrete structures.

Bulk density determination for 01 laterite, 01 limestone and 12 clay samples were carried out by Matter Material Testing and Research laboratory using Test Method BS: 1377(Part 1):1990.

3.0 PROPERTY DESCRIPTION

3.1 Details of the area (Village name, District, State)

| Block name | Naredi block |
| --- | --- |
| Villages | Naredi |
| Taluk | Abdasa Taluk |
| District | Kachchh |
| State | Gujarat |

The block covers an area of 3.92 Sq. km, location map of the study area is shown in Annexure-2

The block area under investigation is dominantly flat area on the western part and undulated at the eastern part. Majority of the area is covered by soil on the top with pebbles of bauxite, laterite, limestone, shell limestone and sandstone were observed. Small outcrops of laterite & limestone beds are exposed in the eastern margin of the block.

3.2 Survey of India toposheet No.

The Naredi block is situated in Kachchh district, Abdasa taluk and falls in toposheet No. 41E/04.

Fig 3.1 Block area on Toposheet no 41E/04

Fig 3.2 Block area on Google earth imagery

| Table 3.1. Boundary coordinates of Naredi block, Gujarat |  |  |
| --- | --- | --- |
| Sl. No. | Longitude | Latitude |
| 1 | 69°10’49.81” E | 23°10’50.69” N |
| 2 | 69°10’8.48” E | 23°11’46.48” N |
| 3 | 69°11’26.98” E | 23°12’9.32” N |
| 4 | 69°11’56.68” E | 23°11’22.75” N |
| 5 | 69°10’58.24” E | 23°11’8.18” N |

3.3 Location and Accessibility

Naredi block is situated towards the northern part of Naredi village. The nearest railway station is Mothara railway station 10km from the Naredi block, the nearest passenger railway station is Bhuj railway station situated 60kms from the block. Nearest airport is Bhuj airport located at Bhuj city, which is at 60km from the block.

Fig 3.3 Accessibility map of Naredi Block

3.4 Cadastral details, land use/ cover, forest with type of forest, free hold/lease hold details

The block contains both government and private lands. There are no records of leasehold area in the block. The block doesn’t have any forest area.

| Table 3.2: Details of land ownership inside the Naredi block |  |  |  |
| --- | --- | --- | --- |
| Sl. No | Survey No. | Village | Owner Type |
| 1 | 446 | Naredi | Govt |
| 2 | 446 | Naredi | Govt |
| 3 | 96 | Naredi | Pvt |
| 4 | 70/1 | Naredi | Pvt |
| 5 | 449 | Naredi | Govt |
| 6 | 449 | Naredi | Govt |
| 7 | 449 | Naredi | Govt |
| 8 | 451 | Naredi | Govt |
| 9 | 451 | Naredi | Govt |
| 10 | 49 | Naredi | Pvt |
| 11 | 48 | Naredi | Govt |
| 12 | 483 | Naredi | Govt |
| 13 | 95 | Naredi | Pvt |
| 14 | 94 | Naredi | Govt |
| 15 | 71/1 | Naredi | Pvt |
| 16 | 449 | Naredi | Govt |
| 17 | 73 | Naredi | Pvt |
| 18 | 449 | Naredi | Govt |
| 19 | 74 | Naredi | Pvt |
| 20 | 52 | Naredi | Govt |
| 21 | 51/1 | Naredi | Pvt |
| 22 | 47 | Naredi | Pvt |
| 23 | 485 | Naredi | Govt |
| 24 | 98 | Naredi | Pvt |
| 25 | 82 | Naredi | pvt |
| 26 | 94 | Naredi | Govt |
| 27 | 449 | Naredi | Govt |
| 28 | 77 | Naredi | Pvt |
| 29 | 76 | Naredi | Pvt |
| 30 | 449 | Naredi | Govt |
| 31 | 2 | Naredi | Govt |
| 32 | 51/1 | Naredi | Pvt |
| 33 | 28 | Naredi | Pvt |
| 34 | 485 | Naredi | Govt |
| 35 | 485 | Naredi | Govt |
| 36 | 100 | Naredi | Pvt |
| 37 | 101 | Naredi | Pvt |
| 38 | 93 | Naredi | Pvt |
| 39 | 79 | Naredi | Pvt |
| 40 | 76 | Naredi | Pvt |
| 41 | 75 | Naredi | Pvt |
| 42 | 26 | Naredi | Pvt |
| 43 | 26 | Naredi | Pvt |
| 44 | 25 | Naredi | Pvt |
| 45 | 485 | Naredi | Govt |
| 46 | 485 | Naredi | Govt |
| 47 | 102 | Naredi | Pvt |
| 48 | 82 | Naredi | Pvt |
| 49 | 82 | Naredi | Pvt |
| 50 | 80 | Naredi | Pvt |
| 51 | 22 | Naredi | Pvt |
| 52 | 23 | Naredi | Pvt |
| 53 | 24 | Naredi | Pvt |
| 54 | 437 | Naredi | Govt |
| 55 | 84 | Naredi | Pvt |
| 56 | 21 | Naredi | Pvt |
| 57 | 20 | Naredi | Pvt |
| 58 | 18 | Naredi | Pvt |
| 59 | 116 | Naredi | Pvt |
| 60 | 444/1 | Naredi | Pvt |
| 61 | 2 | Naredi | Govt |
| 62 | 447 | Naredi | Govt |
| 63 | 89 | Naredi | Pvt |
| 64 | 85 | Naredi | Pvt |
| 65 | 8 | Naredi | Pvt |
| 66 | 10 | Naredi | Pvt |
| 67 | 15 | Naredi | Pvt |
| 68 | 115 | Naredi | Pvt |
| 69 | 114 | Naredi | Pvt |
| 70 | 443 | Naredi | Govt |
| 71 | 437 | Naredi | Govt |
| 72 | 106 | Naredi | Pvt |
| 73 | 85 | Naredi | Pvt |
| 74 | 6 | Naredi | Pvt |
| 75 | 12 | Naredi | Pvt |
| 76 | 15 | Naredi | Pvt |
| 77 | 365 | Naredi | Pvt |
| 78 | 362 | Naredi | Pvt |
| 79 | 366 | Naredi | Pvt |

3.5 Climate

Naredi village is a village in Abdasa taluk of Kachchh district. It experiences a semi-arid to arid climate characteristic of the Kutch region, with extreme temperatures, low annual rainfall, and high evaporation. The Climate in this region is very dry and hot during summer and cold during the winter. The region receives average rainfall of 1257 mm annually.

3.6 Flora/Fauna

Agriculture and animal husbandry are the predominant economic activities in the district. Major Crops being produced in Kachchh district are groundnut, castor seeds, mustard seeds, bajra, jowar, cotton, wheat, millets and pulses. Kachchh is a chief producer of psyllium (Isabgul), cumin and coriander. Around one thousand cotton producers in Rapar taluka of Kachchh have been granted ‘organic’ certification by Agrocel (an agricultural services provider). Bajra is an important food grain of the district. Fruits grown in the district are mango, banana, papaya, chikoo, date palms, Sapota, Lemon and Pomegranate Vegetable grown in the district are brinjal, cucurbits, tomato, and okra Kachchh is a major producer of Fodder Grass. Varieties of reptiles, mammals and fishes are found in Bhuj and its adjoining areas. Crocodile, spiny tailed lizard, monitor lizard, Kachchh Rock Gecho, Black Cobra, Black Krait, Sand Boa, Royal Snake, Sand Boa, Python etc. are some of the reptiles found here. Varieties of toads and frogs are found in Bhuj. The mammals you can get to see in Bhuj and its surrounding areas are Neelgai, Wild Boar, Chinkara, Indian wolf, Jackal, pangolin. (Kachchh.nic)

3.7 Regional Geomorphology

Physiographically, Kachchh landmass is surrounded by low-lying Ranns in north and east, the Indus River delta zones in north-western parts and Gulf of Kachchh in south.

The Kachchh mainland is a central high land with a core plateau zone, which is dissected in the north, west and east and associated with coastal plain in south. All these physiographic features, comprising unique saline wasteland forms of Ranns and associated plains, hill ranges, gently sloping peripheral coastal tract, dissected coastal erosional plain fringed successively by younger deltaic plains, tidal flats, spits and marginal accretionary zones etc, can broadly be divided into four characteristic units,

a)	Central High land and the Upland in the Rann Area.

b)	Central Plain in the southern part.

c)	Little and Great Rann areas, and the

d)	Banni plains.

The mainland of Kachchh is an undulating country with rugged broken ground and broad plains and is marked by hill ranges and isolated peaks. There are three hill ranges in the main land namely Dhinodhar, Jura and Vavar, which rise to 387m and 274m msl. The other uplands in the main land area are in Wagad area in the east forming part of Bhachau and Rapar Talukas. The Pachham, Khadir and Bela islands in the Great Rann in the north also form highlands with maximum elevation of 458 m above msl. All the hill ranges follow a general east-west trend. Kachchh district has about 350 km long coastline and its coastal plain is about 35-45 km wide and attains the elevation up to 80 m above msl. The coast is generally flat and broken by small and big creeks, viz., Kori, Boacha and Godia. The Rann forms a unique and conspicuous landform and has been divided into the Great Rann in the north and Little Rann in the east. The Rann mainly comprises marshy land, salt/mud flats and is devoid of vegetation and habitation and has a very hostile environment. The total area of Rann is about 25000 sq. km. The extensive low-lying area south west of Pachchham island resembles Rann except for some patches of scanty vegetation. It is known as Banni plain and covers an area of about 2000 sq. km. About 777 sq. km of Banni plain is reported to be superior grassland. (CGWB-2022)

Fig 3.4: Geomorphological map of Kachchh

(Aquifer map and management plan, CGWB -2022)

Drainage Pattern

The district is drained by a number of rivers. The main rivers are Bharud, Kali, Suri, Malan, Khari, Kankawati, Mithi, Rukmavati and Bhukhi. All the rivers are ephemeral in nature and have steep gradients. Their flow is flashy depending upon monsoon rainfall. The overall drainage patterns are radial because of central upland which acts as watershed. The lengths of rivers flowing northwards into the Great Rann of Kachchh range from 20 to 47 km and of those flowing southwards into the Arabian Sea, Gulf or Kachchh and Little Rann of Kachchh from 24 to 55 km. On most of the rivers some form of dam has been constructed either under minor or medium irrigation projects (CGWB-2022).

Fig 3.5: Drainage system map of Kachchh

(Aquifer map and management plan, CGWB -2022)

3.8 Local Infrastructure

Naredi village is located in the South of the block. The nearest railway station is Bhuj railway station that is located about 60kms from the block and the nearest bus stop is Naredi bus stop about 1 km from the block. The nearest hospital is the Kothara government hospital situated at Gadhsisa about 20 kms from the block. The nearest police station and post office is located in Kothara. The Tahsildar office is situated 40kms towards the west from the block in Naliya.

3.9 Population

Naredi is a village located in Abdasa Taluka of Kachchh district, Gujarat with total 324 families residing. As per Census the Naredi village has a population of 1537 of which 793 are males and 744 are females.
In Naredi village population of children with age 0-6 is 226 which makes up 14.70 % of total population of village. Average Sex Ratio of Naredi village is 938 which is higher than Gujarat state average of 919. Child Sex Ratio for the Naredi as per census is 837, lower than Gujarat average of 890(Census2011).

Literacy Rate

Naredi village has lower literacy rate compared to Gujarat. In 2011, literacy rate of Naredi village was 71.01 % compared to 78.03 % of Gujarat. In Naredi Male literacy stands at 80.45 % while female literacy rate was 61.15 %(Census2011).

Work Profile

In Naredi village out of total population, 554 were engaged in work activities. 99.10 % of workers describe their work as Main Work (Employment or Earning more than 6 Months) while 0.90 % were involved in Marginal activity providing livelihood for less than 6 months. Of 554 workers engaged in Main Work, 55 were cultivators (owner or co-owner) while 298 were Agricultural labourer (Census2011).

3.10 Archaeological & Historical sites

No historical or archaeological sites are present within the block.

However, there are many historical sites such as Smritivan earthquake memorial and museum (which is a poignant tribute to is to the victims of the devastating 2001 Bhuj earthquake), located in Bhuj about 95km from the block. The Kachchh Museum is is the oldest museum in Gujarat is another historical site present in Bhuj.

Prag Mahal, is a palace commissioned by King Pragmalji in the 1860s, designed by Colonel Henry Saint Wilkins in the Italian Gothic style and built in the middle of Bhuj next to the Aina Mahal (Kachchh. Nic).

Dholavira is an archaeological site at Khadirbet about 220kms away from the block area in Bhachau Taluka of Kachchh District, in the state of Gujarat (kachchh.nic.in).

3.11 National parks and Environmental settings of the area

There are no specified national parks in the area.

4.0 PREVIOUS WORK

4.1 Details of previous exploration/ investigation carried out by other agencies/ parties

Previous exploration in the block area was carried out by the agencies like Commissioner of Geology and Mining, Gujarat and Geological survey of India in the field season 1964-1965. The details of their work are discussed in the section below.

4.2 A very brief note on previous work on Geology, Geophysics, Geochemistry

a) CGM, Gujarat

The document titled “Gujarat's Mineral Wealth: A Responsible Exploration and Development Paradigm,” published by the Commissioner of Geology and Mining, Gujarat, on August 17, 2024, accounts the exploration activities carried out by CGM in the state of Gujarat.

The Geology of the area comprises of Fulra Formation (Clay and marl), Gaj Formation (Limestone and Shale), Matanomadh (Laterite /Bauxite) and Anjar volcanics (Basalt). The topography of the area where laterite occurs is characterized by series NW-SE trending ridges. The laterite follows the slopes of the basalt ridges & mounds and reflects the highly irregular nature of the ground. Bauxite mineralization is bounded by Deccan Traps to east and north and Gaj beds to the west and south. It is suggested that in-situ lateralization and bauxitization of Deccan Traps and associated pyroclastic materials have resulted in the formation of bauxite

CGM studied the area by means of pitting, trenching and drilling with an estimated resource of 27.85 Mt. Chemical analysis of the bauxite from proposed area shows Al2O3 (average 49.77%) and low SiO2 Percentage (average 2.49%).

b) GSI (U.D.G Rao, 1964-1965)

The bauxite deposits of Kotda, Jarjok, Naredi, Nandra and Balachor (N) villages of Kutch district, Gujarat state were taken up for detailed investigation to assess their potentialities during 1964-65.

NAREDI:

As, reported by U.D.G Rao (GSI,1964-1965) the main bauxite pocket is situated two kms roughly north-northeast of Naredi village. Other occurrences are found two to three kms from Naredi, in the north-eastern sector, along the village boundaries of Naredi/Sanosara and Naredi/Nandra.

The village of Naredi, stands on Gaj limestones, and is situated on the northern banks of the river Kankavati two to three kilometres from the village, in the north-eastern sector, an extensive formation of ferruginous laterite is observed covering a low lying plateau with gentle slope to the south (covered by Gaj beds beyond) and steep escarpment to the north (with exposures of trap on low ground beyond the scrap).

The ore is bouldery in nature and the boulders and nodules are exposed on the surface itself, except for some short stretches where they are found under a thin and negligible soil cover. They vary in size from pisolites less than a centimetre in diameter to big boulders as much as a metre or more across. They are hard, compact and massive and predominantly ash-grey of fawn-grey in colour. Others are of various shades of brown, pink, yellow and red. Minor patches of white and yellow coloured boulders are also seen. Thus, in general, the grade of Naredi pockets, appears to be uniformly good. In addition, they constitute the single biggest deposit in Kutch district.

Traps are exposed on the southern banks of the river(Kankavati). It is three kms. roughly north-east of Naredi village. A full section of bauxite formation, from the hard bouldery zone at the top, to the traps exposed below, is seen along the northern scarp margin of the pocket, forming the steep bank of the river here. Reddish and whitish lithomarge bands overlie the traps, succeeded above by the clayey bauxite zone, which in turn passes into the hard bouldery bauxite, seen on the surface. The section is exposed over a length of 215 meters along the river and is limited by Gaj beds, overlying traps to the east and rather abruptly changing to ferruginous laterite to the west. Further west, Gaj beds are again seen to cover the river banks. The pocket has an average width of only 26 meters, being limited by a ferruginous laterite ridge, to the south.

A total Number 130 samples were collected from 155 pits. The mean specific gravity of the bauxite in the area is 2.43. Accordingly, about 748500 tonnes of bauxite have been estimated to be available here.

5.0    GEOLOGY OF THE AREA

5.1 Aerial Reconnaissance

Not Applicable

5.2 Regional geological set up of the area with stratigraphy

The regional geology of the Kachchh Peninsula comprises of rocks ranging in age from Jurassic to Recent with no basement rocks exposed in the area as observed. The Kachchh Basin is an East – West oriented pericratonic rift basin. To the North, the Precambrian basement rocks are exposed in Meruda Takkar and Nagar Parkar hills of Pakistan. To the east, the Precambrian rocks are covered under alluvium, to the west the basin extends across the continental shelf, while the South is occupied by the uplifted Saurashtra platform covered by Late Cretaceous sediments and Deccan Traps. The basin is an asymmetric rift which is bounded in the north by the Nagar Parkar Fault, and in the South by North Kathiawar Fault . To the East the extension of the rift is limited by Radhanpur – Barmer Arc . The sediment fill of the basin has been defined as syn-rift in nature. (Biswas, 1987). The succession of Tertiary rocks is given in the following table 5.1

| Table 5.1: Stratigraphy of Tertiary Group of rocks (after Biswas, S. K. & Khattri, 2001) |  |  |
| --- | --- | --- |
| Age | Formation | Lithology |
| Pliocene | Sandhan Formation | Friable sandstone, siltstone, calcareous clay and conglomerate |
| Lower to Middle Miocene | Gaj Formation | Olive green shale with gypsum and marlite |
| Oligocene to Lower Miocene | Kharinadi Formation | Mottled siltstone, with variegated clay and marlite |
| Oligocene | Maniyara Fort Formation | Sandy limestone, glauconitic clay, siltstone and coralline limestone |
| Middle to Upper Eocene | Fulra Formation | Cream coloured foraminiferal limestone |
| Lower Eocene | Kakdinadi Formation | Greyish to variegated clay, carbonaceous and lignite bearing shale with fossiliferous marlite and limestone |
| Palaeocene | Matanomadh Formation | Lithomargic clay, laterite, and lateritic conglomerate |
| Cretaceous |  | Basalt |

Geologically, the area comprises of sediments belonging to the Katrol and the Bhuj formations of Mesozoic age, overlain by volcanic rocks representing the Deccan Volcanics; the laterite represented by the Matanomadh Formation of Palaeocene; The supratrappean Matanomadh Formation which is of Palaeocene age, comprises white, soft aluminous clays, lavender cellular ash, hard laterite with concretionary surfaces and bauxite. The Fulra Formation overlying the Matanomadh Formation comprises nummulitic limestone, gypseous shale and variegated clay. The Gaj Formation rests unconformably on the Fulra limestone and forms extensive plains in the western part. The Sandhan Formation comprising shale, clay, marl and conglomerate rest unconformably over the Gaj Formation. Quaternary Miliolitic calcarenite is seen along the flanks of some hills and in some nala sections over the Sandhan Formation(GSI 2016).

Stratigraphic succession of the area ( Toposheet 41E/3 & 4) (GSI 2016).

5.3 Geology of the Naredi area

In the Naredi area, the Boulders and nodules are found covering the plateaus and hillocks as the other types but are distributed in an earthy or clayey matrix. The boulders are hard, massive and compact, predominantly ash-grey in colour, (in some places pinkish, reddish boulders are also seen) with smooth, unctous surfaces (where exposed). They exhibit a peculiar ‘glossy-sheen’ and can be seen shimmering in sun-light from considerable distances. The deposits of Naredi, Nandra and Balachor villages are of this type. (UDG Rao 1965).

## 6.0 GEOSCIENCE INVESTIGATION

### 6.1 Geological Mapping and Topographic Survey

Detailed Naredi Block Geological Mapping was carried out in 1:4000 scale as per the MEMC 2015 guidelines. In the present study, the mapping was carried out along with DGPS topographic survey team. Accordingly, east-west traverses were taken along the 40mX40m grid, and field data was collected by making observations at each grid points. Later the field data was processed using the GIS module of QGIS software and the detailed topographic map and geological map (Fig 6.1 and 6.2) are attached as Annexure-3 & Annexure-4 in true scale respectively.

DGPS based topographic survey was carried out in the study area on a prescribed scale of 1:4000 scale as per NMEDT SOP. Three Temporary bench mark stations were setup across the block boundary and the DGPS was kept under observation for 24 hours in order to assure the amount of deflection and to attain the maximum accuracy. To ensure the highest accuracy Rovers (Fig 6.4) were taken along the planned traverses on a 40X40 grid and the topographic mapping was completed by incorporating all the features present within the block. Upon completion of the topographic survey, the obtained data was again correlated with the CORS-Survey of India to ensure the highest accuracy in the survey. The Topographic map with a contour interval of 2.0m was prepared using AutoCAD software.

Fig 6.1: Topographic map prepared by Geo Marine Solutions Pvt Ltd.

Fig 6.2: Detailed Geological map of Naredi Block prepared by Geo Marine Solutions Pvt Ltd

Fig 6.4(a)

Fig 6.4(b)                                          	Fig 6.4(c)

Fig 6.4: Field photographs during Geological mapping and topographic survey.

- Photo of surveyor taking reading and geologist recording field data.

- Photograph of setting TBM using DGPS instrument.

- Photo of surveyor taking reading.

### 6.2 Description of Rock Types

The Naredi block comprises of a dominant Soil cover with pebbles of laterite, limestone, sandstone, and some outcrops of laterite, limestone exposures towards the eastern margin. The soil cover in the block is yellowish to brown in colour, little compact, medium to fine grained. Soil is usually friable, but in north eastern part of the block it is more clayey.

Small outcrops of laterite occur as hard and slightly friable exposures, reddish brown in colour and ferruginous in nature.

Outside the block towards north and eastern side of the block, bauxite observed as whitish and greyish coloured friable slightly hard outcrops. This bauxite is rich in alumina and locally termed as high-grade bauxite.   (Fig 6.5 A).

Limestone occurs as light grey to greyish white bedded outcrops in the eastern part & north central part of the block. The rock is medium grained and weathered. Limestone pebbles were also observed in the soil covered areas in the northern and southern part of the block. (Fig. 6.6).

(Fig 6.5 A)

(Fig 6.5 B)					(Fig 6.5 C)

Fig 6.5 (A) Laterite Exposure.

(B) High grade bauxite observed near NE part of the block in GMDC pit 150m distance   from the block.

(C) Laterite/bauxite sample.

Fig 6.6 Limestone Exposure

Fig 6.7   Cross section lithology visible on the south eastern part of the block

### 6.3 Petrographic Studies:

As a part of NQT, 10 samples were allocated for petrographic studies (10 thin section + 10 ore section). However, the lithology in the block was predominantly clay. So, the petrographic component was removed with approval of TCC.

### 6.4 Structure and Metamorphism:

No structural or metamorphic features were observed in the block.

### 6.5 Pitting and Trenching:

Pitting and Trenching was not part of the approved NQT.

### 6.6 Geophysical Exploration:

Geophysical exploration was not part of the approved NQT.

### 6.7 Geochemical Exploration:

Geochemical exploration was not part of the approved NQT

7.0 Integration of Geology, Geophysics and Geochemical Exploration Data and interpretation

Not required for bauxite and laterite; hence the same was not carried out.

## 8.0 MINERAL PROSPECT

### 8.1 Surface indication

### The Naredi Block was taken up for exploration for bauxite/laterite, gallium, vanadium, and associated rare earth elements (REE). However, the detailed geological mapping revealed that >90% of the block was covered with soil, with very small outcrops of laterite, bauxite & limestone in the north central, north eastern & eastern part of the block occurs in the form of pockets and lenses outside the eastern margin of the block. However, within the block the area is predominantly covered by soil, with sporadic exposures of laterite and limestone.

### Laterite exposures are mainly observed in the north-eastern part of the block. The laterite is hard, friable, pisolitic, and reddish brown in colour.

### Limestone exposures are predominantly present in the eastern part of the block Fig 6.6

### Pebbles of white to greyish bauxite, limestone, and sandstone occur in soil as well.

### 8.2 Mode of occurrence and strike length of the body.

The block is predominantly covered by soil; the presence of laterite/bauxite within the block is minimal to negligible.

### 8.3 Alteration Zones.

No alteration zones are encountered in the area, as the entire area is covered with soil cover and with some patches of limestone and laterite.

### 8.4 Genesis of Mineralized Zones.

### As it will be shown in the next chapter-9, the dominant clay sediments below the soil cover have the potential clay hosted REE, Sc & Ga.

# 9.0 Exploration systematic Drilling

# 9.1 Spacing of Boreholes

As per the approved NQT and MEMC guidelines 2015 (amended, 2021) for G–3 level exploration of laterite/bauxite deposits of pockets/lensoidal nature, a total of 91 boreholes for 910m were initially planned in the Naredi Block on a grid spacing of 200 m × 200 m (with first phase drilling at 400mX400m). The spacing was adopted to ensure uniform coverage of the entire block, considering the irregular and discontinuous occurrence of bauxite pockets in Naredi area.

Fig. 9.1 Map showing 91 proposed boreholes at 200 X 200m grid.

After detailed geological mapping and topographic survey, in the first phase of drilling boreholes were to be drilled on 400m X 400m spacing. First phase of drilling was taken up from the eastern margin of the block. First 06 boreholes were drilled on a 200mX400m (E-W grid) as this part of the block was closest to GMDC bauxite quarries.

Fig. 9.2 Map showing planned Boreholes at 400 x 400 m grid for phase-I

The results of chemical analysis of 13 samples from all the runs in one (BH-13) of the six initially drilled boreholes and 10 representative samples from 5 other boreholes, were presented to TCC for review on 24th July 2025. Since clay was the dominant lithology present in the 6 boreholes, and showed anomalous values of  Sc and Ga in them, and since laterite/ bauxite was only encountered in one run of half meter length, the TCC suggested that the project may be re-oriented to explore the block for clay hosted Sc, Ga, REE and associated critical elements. The committee also suggested to cover the entire block on 400m X 400m grid and present the result of the same to the committee for review. Subsequently, based on the recommendations of the Technical Committee (TCC), Phase–I drilling was revised to cover the rest of the block with a spacing of 400 m × 400 m.

Fig. 9.3 Map showing 27 drilled at 400 m X 400 m spacing.

### 9.2 Methodology of Drilling and types of Drilling

The block consisted of predominantly soil cover on throughout the block very few exposures of limestone and laterite on the eastern part of the block. As per the auction rules diamond core drilling was adopted.

Core drilling in the area was carried out by Voltas-400 core drilling rig. All boreholes were vertical holes, using HQ rods and NX casing. for maximum core recovery, the following steps were taken while drilling:

1. Selecting the type and length the barrel for each run

2. To use water judiciously

These two conditions were monitored for every single run that was drilled in each borehole. Based on the lithology core recovery in each run, the drill conditions were altered accordingly for subsequent runs, to optimize for the maximum recovery.

During drilling, three different types of drill barrels were used -

- Single barrel of 1.0m & 1.5m length.

- Double barrel of 1.0m &1.5m length.

- Triple tube barrel of 1.50m length.

During the initial few boreholes top 2-3 meters were drilled using single & double barrels of  1 meter length, subsequently remaining drilling was carried out by 1.5meter length single and double-barrel to attain maximum recovery.

In general, single tube dry drilling was carried out in most of the boreholes, as the dominant lithology was clay.

Initially it was planned to drill two or three boreholes up to the bedrock basalt. The first 3 boreholes drilled up to 35m, 30m and 28m did not encounter the bedrock. Therefore, after the re-orientation of the project to clay hosted critical elements, depth of the remaining boreholes were drilled up to 20m or2 1m only.

9.3 Borehole Planning

As discussed earlier the drilling up to a depth of 10m was to be carried out in 2 phases; first phase on 400m X 400m spacing, in the second phase it was planned to be drilled at  200m X 200m grid. After the re-orientation of the project, the boreholes were drilled on 400m X 400m grid to a depth of 20m to 21m.

In total, twenty-seven (27) boreholes were drilled, achieving an aggregate meterage of 567 m.

The following table gives the co-ordinates and collar details of the 27 boreholes.

| Table 9.1 Borehole Co-ordinates and Collar details |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl.no. | BH No. | Latitude (DD) | Longitude (DD) | Collar RL (m) |
| 1 | BH73 | 23.188341 | 69.1849157 | 109.609 |
| 2 | BH03 | 23.200777 | 69.1854978 | 120.840 |
| 3 | BH01 | 23.201324 | 69.1905620 | 125.851 |
| 4 | BH13 | 23.199766 | 69.1895724 | 122.060 |
| 5 | BH05 | 23.199706 | 69.1834903 | 122.791 |
| 6 | BH28 | 23.196206 | 69.1821222 | 117.929 |
| 7 | BH30 | 23.194846 | 69.1793721 | 111.958 |
| 8 | BH09 | 23.197652 | 69.1757738 | 111.007 |
| 9 | BH07 | 23.198166 | 69.1793049 | 115.690 |
| 10 | BH32 | 23.193423 | 69.1750878 | 107.161 |
| 11 | BH53 | 23.191927 | 69.1804303 | 109.978 |
| 12 | BH75 | 23.187199 | 69.1835196 | 107.968 |
| 13 | BH55 | 23.189107 | 69.1747429 | 106.136 |
| 14 | BH77 | 23.185532 | 69.1769682 | 105.133 |
| 15 | BH91 | 23.184048 | 69.1810316 | 103.441 |
| 16 | BH57 | 23.192893 | 69.1936371 | 116.209 |
| 17 | BH69 | 23.191104 | 69.1924689 | 113.260 |
| 18 | BH81 | 23.189254 | 69.1920926 | 111.096 |
| 19 | BH67 | 23.192603 | 69.1963219 | 118.395 |
| Sl.no. | BH No. | Latitude (DD) | Longitude (DD) | Collar RL (m) |
| 20 | BH71 | 23.190140 | 69.1885912 | 111.939 |
| 21 | BH47 | 23.194572 | 69.1901213 | 115.425 |
| 22 | BH49 | 23.194856 | 69.1840091 | 115.686 |
| 23 | BH51 | 23.192692 | 69.1829962 | 112.936 |
| 24 | BH23 | 23.198467 | 69.1925539 | 122.550 |
| 25 | BH45 | 23.197156 | 69.1939919 | 122.259 |
| 26 | BH35 | 23.195575 | 69.1916287 | 118.083 |
| 27 | BH26 | 23.196967 | 69.1858279 | 122.295 |

9.3.1 Borehole logging

Drill cores obtained in every run was carefully examined in the field and based on the observations, the following lithocodes were assigned to the lithology to broadly classify the obtained cores.

- Soil

- Limestone

- Laterite/bauxite

- Lateritic Clay

- Clay

Soil: yellow to brownish soil, little compact, medium to fine grained. Soil is usually friable and slightly compact in maximum area, but in north eastern part of the block they are more clayey.

Limestone: This lithocode includes limestone and marl; they appear as weathered, greyish in colour. Some limestone includes shell fragments.

Laterite/bauxite: This appears hard or slightly friable reddish brown material and mostly of ferruginous in nature along with yellowish and whitish fillings.

Lateritic Clay This lithocode includes the reddish brown ferruginous clay.

Clay This lithocodes includes predominantly different colours of clay like yellow, olive green, orange brown and grey colour. The clay slightly friable when dry and sticky when it is wet.

### 9.3.2 Core Recovery

Core recovery was monitored very carefully after each run. Wherever, the core recovery was reduced, in the immediate next run suitable measures were taken to optimize the drilling and to increase the core recovery (the same has been discussed in the section 9.2 of the same chapter).

9.4 Sample Preparation

During borehole logging, the following lithocodes were used to broadly classify

the drill cores for the sampling purpose.

- Soil

- Limestone

- Laterite/ bauxite

- Lateritic Clay

- Clay

Sampling of split cores was based on a combination of run-length and lithology. Accordingly, entire run lengths (of 1.5m/1m as the case may be) all clay horizons, laterite/bauxite, limestone encountered in all boreholes, were sampled for analysis. For Major element and LOI analysis by XRF, only representative samples were prepared.

For sample analysis the drill cores were split into two equal halves using a core splitter and/or using chisel and hammer manually. One portion of the split core was returned to the core box and the other half of the was used for laboratory analysis purpose. Once the core samples splitting was completed, the selected samples for analysis were labelled & packed. The packed samples were submitted to NABL accredited lab for further process of drying, crushing, pulverizing and analysis.

Samples for Analysis of Major oxides and LOI using XRF.

168 Samples from 27 boreholes (22 laterite, 7 limestones and 136 clay& or lateritic clay samples and 02 soil samples) were analyzed for major oxides and LOI by XRF.

Samples for analysis of Minor, trace and REE using ICPMS/OES

For analysis of minor, trace and REE by ICP OES/MS, 342 Samples from 27 boreholes (23 no of laterite, 9 limestones and 307 clay& or lateritic clay) were analyzed.

Check samples

After receiving the analysis results from the primary lab, the samples with the REE enrichment were selected for ICP OES/MS and samples. for XRF the samples were selected in such a way that it represented all the lithologies present in the block.

The samples were drawn from the pulp sample duplicates that were stored in the repository. 33 samples for ICP OES/MS analysis and 16 samples for XRF analysed were prepared.

9.5 Chemical analysis and laboratory Procedure

XRF analysis of the primary samples were carried out at Inspectorate of Griffith lab, Gandhidham (NABL accredited). The procedures adopted for XRF, ICP OES/MS are stated below:

IGI/GDM/LAB/TPM/048 & LOI as per IBM.

Check sample analysis was carried out at Shiva lab, Bangalore (NABL accredited). Procedure adopted for XRF analysis was using fusion beads and SOP/OM/103, 105 was the procedure code, and for ICP OES/MS, the SOP /OM/051,052 was followed.

9.6 Results and Discussion

### 9.6.1 Descriptive Statistics Analysis

Descriptive statistics analysis of the primary samples are discussed below

Descriptive statistical analysis of the laterite samples

The descriptive statistical analysis of 22 laterite samples indicate that the Al2O3 is ranging from 11.35%- 47.77%, Fe2O3 ranges from 13.98%-45.88%, SiO2 ranges from 1.74%-35.49%, TiO2 ranges from 0.78%-4.99% and V2O5 ranges from 0.06%-0.26%. A total of 28.5 meters of laterite samples were identified within the block. Of this, only two samples (total 1.5m was bauxite), eight samples are (total 13m) aluminous laterite and the remaining 13 samples (total of14m) is ferruginous laterite.

| Table 9.2 Descriptive statistics of Laterite |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | Al2O3 | Fe2O3 | SiO2 | TiO2 | V2O5 | LOI |
| Mean | 29.43 | 28.62 | 19.11 | 2.04 | 0.17 | 16.94 |
| Standard Error | 1.96 | 2.18 | 1.92 | 0.20 | 0.01 | 1.04 |
| Median | 27.07 | 30.92 | 20.52 | 2.02 | 0.18 | 15.17 |
| Mode | #N/A | #N/A | #N/A | 2.02 | #N/A | #N/A |
| Standard Deviation | 9.42 | 10.45 | 9.23 | 0.95 | 0.05 | 5.01 |
| Sample Variance | 88.65 | 109.14 | 85.11 | 0.90 | 0.00 | 25.06 |
| Kurtosis | -0.38 | -1.46 | -0.53 | 3.17 | -0.05 | -0.97 |
| Skewness | 0.48 | 0.20 | -0.11 | 1.49 | -0.24 | 0.64 |
| Range | 36.42 | 31.90 | 33.75 | 4.21 | 0.20 | 15.40 |
| Minimum | 11.35 | 13.98 | 1.74 | 0.78 | 0.06 | 11.36 |
| Maximum | 47.77 | 45.88 | 35.49 | 4.99 | 0.26 | 26.75 |
| Count | 23.00 | 23.00 | 23.00 | 23.00 | 23.00 | 23.00 |

Descriptive statistical analysis of the Limestone samples

The descriptive analysis of 7 Limestone samples indicates that the CaO is ranging from 0.79%-36.59%, MgO   ranges from 0.57%-1.36%, SiO2 ranges from 15.62%-61.20%, TiO2 ranges from 0.20%-1.41% and V2O5 ranges from 0.00%-0.10%. A total of 12.5 meters of limestone samples were identified in the block during drilling.

| Table 9.3 Descriptive statistics of limestone |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | CAO | MgO | SiO2 | TIO2 | V2O5 | LOI |
| Mean | 15.35 | 0.93 | 34.67 | 0.72 | 0.03 | 20.20 |
| Standard Error | 4.97 | 0.13 | 5.10 | 0.16 | 0.01 | 2.52 |
| Median | 13.01 | 0.74 | 32.36 | 0.77 | 0.02 | 18.30 |
| Mode | #N/A | #N/A | #N/A | #N/A | 0.00 | #N/A |
| Standard Deviation | 13.15 | 0.35 | 13.50 | 0.43 | 0.04 | 6.66 |
| Sample Variance | 172.83 | 0.12 | 182.16 | 0.19 | 0.00 | 44.32 |
| Kurtosis | -0.54 | -2.32 | 3.42 | -0.80 | -0.01 | 0.79 |
| Skewness | 0.54 | 0.41 | 1.11 | 0.40 | 1.04 | 0.90 |
| Range | 35.79 | 0.79 | 45.58 | 1.21 | 0.09 | 20.31 |
| Minimum | 0.79 | 0.57 | 15.62 | 0.20 | 0.00 | 11.80 |
| Maximum | 36.59 | 1.36 | 61.20 | 1.41 | 0.10 | 32.11 |
| Count | 7.00 | 7.00 | 7.00 | 7.00 | 7.00 | 7.00 |

Descriptive statistical analysis of the Clay samples

The descriptive analysis of 136 Clay samples indicates that the Al2O3 is ranging from 3.49%-56.89%, Fe2O3 ranges from 1.34%-33.76%, K2O ranges from 0.0%-2.54%, Na2O ranges from 0.06%-1.18%, SiO2 ranges from 1.52%-80.77%, TiO2 ranges from 0.19%-4.14% and V2O5 ranges from 0.00%-0.19%. For the purpose of interpretation of chemical analysis results, lateritic clay sample were grouped along with rest of clay samples.

| Table 9.4 Descriptive statistics of Clay |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data | Al2O3 | Fe2O3 | K2O | Na2O | SiO2 | TiO2 | V2O5 | LOI |
| Mean | 17.90 | 11.27 | 0.77 | 0.50 | 46.06 | 1.24 | 0.05 | 14.35 |
| Standard Error | 1.02 | 0.57 | 0.06 | 0.02 | 1.36 | 0.06 | 0.00 | 0.50 |
| Median | 13.99 | 9.49 | 0.84 | 0.43 | 43.29 | 1.05 | 0.03 | 14.10 |
| Mode | 15.38 | 9.26 | 0.00 | 0.92 | 52.62 | 0.88 | 0.03 | 10.47 |
| Standard Deviation | 11.95 | 6.68 | 0.67 | 0.27 | 15.84 | 0.68 | 0.04 | 5.82 |
| Sample Variance | 142.79 | 44.69 | 0.44 | 0.07 | 251.00 | 0.46 | 0.00 | 33.89 |
| Kurtosis | 0.26 | 1.90 | -0.81 | -0.56 | -0.17 | 2.20 | 1.04 | 0.40 |
| Skewness | 1.03 | 1.27 | 0.39 | 0.71 | -0.03 | 1.28 | 1.28 | 0.62 |
| Range | 53.40 | 32.43 | 2.54 | 1.12 | 79.25 | 3.95 | 0.19 | 30.84 |
| Minimum | 3.49 | 1.34 | 0.00 | 0.06 | 1.52 | 0.19 | 0.00 | 3.57 |
| Maximum | 56.89 | 33.76 | 2.54 | 1.18 | 80.77 | 4.14 | 0.19 | 34.41 |
| Count | 136.00 | 136.00 | 136.00 | 136.00 | 136.00 | 136.00 | 136.00 | 136.00 |

Distribution trend in each Borehole

The depth wise major oxide distribution pattern for individual boreholes is discussed below.

The following discussion is based on the results of 168 samples that were analyzed for major oxide using XRF analysis.

Fig 9.4   Vertical distribution pattern of major oxides in BH-01.

Fig 9.5   Vertical distribution pattern of major oxides in BH-23.

Fig 9.6   Vertical distribution pattern of major oxides in BH-35.

Fig 9.7   Vertical distribution pattern of major oxides in BH-45.

Fig 9.8   Vertical distribution pattern of major oxides in BH-57.

Fig 9.9   Vertical distribution pattern of major oxides in BH-13.

Fig 9.10   Vertical distribution pattern of major oxides in BH-67.

Fig 9.11   Vertical distribution pattern of major oxides in BH-69.

Fig 9.12   Vertical distribution pattern of major oxides in BH-71.

Fig 9.13   Vertical distribution pattern of major oxides in BH-47.

Fig 9.14   Vertical distribution pattern of major oxides in BH-09.

Fig 9.15   Vertical distribution pattern of major oxides in BH-26.

Fig 9.16   Vertical distribution pattern of major oxides in BH-28.

Fig 9.17   Vertical distribution pattern of major oxides in BH-81.

Fig 9.18   Vertical distribution pattern of major oxides in BH-75.

Fig 9.19   Vertical distribution pattern of major oxides in BH-53.

Fig 9.20   Vertical distribution pattern of major oxides in BH-77.

Fig 9.21   Vertical distribution pattern of major oxides in BH-55.

Fig 9.22   Vertical distribution pattern of major oxides in BH-51.

Fig 9.23   Vertical distribution pattern of major oxides in BH-91.

Fig 9.24   Vertical distribution pattern of major oxides in BH-07.

Fig 9.25   Vertical distribution pattern of major oxides in BH-03.

Fig 9.26   Vertical distribution pattern of major oxides in BH-49.

Fig 9.27   Vertical distribution pattern of major oxides in BH-30.

Fig 9.28   Vertical distribution pattern of major oxides in BH-05.

Fig 9.29   Vertical distribution pattern of major oxides in BH-73.

Fig 9.30   Vertical distribution pattern of major oxides in BH-32.

Primary and check samples analysis result comparison

The check samples were analysed in the NABL accredited lab at Shiva Analyticals Private Ltd Bangalore and results were compared with the results obtained from the primary lab and the same is discussed below.

Comparison of XRF Results.

The XRF analysis of primary and check samples are shown in the figures below.

Comparative analysis shows that the differences between primary and check samples are well within standard deviation-2 and hence acceptable.

The tables below show the details of  primary and check samples comparison

| Table 9.5 Primary and Check Sample values for Al2O3 (in %) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample ID | Primary Samples | Check Samples | 10 % Difference | Actual Difference | Standard Deviation-2 |
| GEMSNRD1309 | 37.59 | 36.15 | 3.76 | 1.44 | 2.031 |
| GEMSNRD2306 | 20.90 | 18.98 | 2.09 | 1.92 | 2.720 |
| GEMSNRD2307 | 22.61 | 22.96 | 2.26 | 0.35 | 0.498 |
| GEMSNRD2308 | 19.52 | 18.83 | 1.95 | 0.69 | 0.979 |
| GEMSNRD2309 | 14.64 | 14.52 | 1.46 | 0.11 | 0.158 |
| GEMSNRD3508 | 14.11 | 13.43 | 1.41 | 0.67 | 0.949 |
| GEMSNRD4514 | 35.54 | 33.01 | 3.55 | 2.53 | 3.577 |
| GEMSNRD4515 | 30.90 | 30.50 | 3.09 | 0.40 | 0.567 |
| GEMSNRD5708 | 14.36 | 13.38 | 1.44 | 0.98 | 1.392 |
| GEMSNRD5709 | 9.38 | 6.55 | 0.94 | 2.83 | 3.998 |
| GEMSNRD5713 | 28.59 | 27.16 | 2.86 | 1.43 | 2.024 |
| GEMSNRD5714 | 47.88 | 47.55 | 4.79 | 0.33 | 0.471 |
| GEMSNRD5716 | 51.87 | 47.95 | 5.19 | 3.93 | 5.555 |
| GEMSNRD5717 | 38.63 | 43.52 | 3.86 | 4.89 | 6.910 |
| GEMSNRD5718 | 38.83 | 38.93 | 3.88 | 0.10 | 0.148 |
| GEMSNRD101 | 31.28 | 31.06 | 3.13 | 0.22 | 0.310 |

Fig 9.31  Scatter Plot of Primary Vs Check samples for Al2O3 (Values in %).

| Table 9.6 Primary and Check Sample values for Fe2O3 (in %) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample ID | Primary Samples | Check Samples | 10 % Difference | Actual Difference | Standard Deviation-2 |
| GEMSNRD1309 | 1.56 | 1.43 | 0.16 | 0.13 | 0.178 |
| GEMSNRD2306 | 42.76 | 39.70 | 4.28 | 3.07 | 4.339 |
| GEMSNRD2307 | 24.49 | 26.28 | 2.45 | 1.80 | 2.539 |
| GEMSNRD2308 | 32.52 | 36.32 | 3.25 | 3.80 | 5.373 |
| GEMSNRD2309 | 31.67 | 37.64 | 3.17 | 5.96 | 8.434 |
| GEMSNRD3508 | 9.54 | 10.04 | 0.95 | 0.50 | 0.700 |
| GEMSNRD4514 | 10.55 | 12.74 | 1.05 | 2.19 | 3.091 |
| GEMSNRD4515 | 10.23 | 10.84 | 1.02 | 0.61 | 0.858 |
| GEMSNRD5708 | 12.47 | 11.69 | 1.25 | 0.78 | 1.107 |
| GEMSNRD5709 | 13.12 | 10.44 | 1.31 | 2.68 | 3.790 |
| GEMSNRD5713 | 9.38 | 10.28 | 0.94 | 0.90 | 1.270 |
| GEMSNRD5714 | 1.94 | 2.34 | 0.19 | 0.40 | 0.569 |
| GEMSNRD5716 | 1.34 | 1.65 | 0.13 | 0.31 | 0.440 |
| GEMSNRD5717 | 6.74 | 4.31 | 0.67 | 2.43 | 3.434 |
| GEMSNRD5718 | 1.66 | 1.60 | 0.17 | 0.06 | 0.079 |
| GEMSNRD101 | 17.73 | 19.09 | 1.77 | 1.36 | 1.921 |

Fig 9.32  Scatter Plot of Primary Vs Check samples for Fe2O3 (Values in %).

| Table 9.7 Primary and Check Sample values for Sio2 (in %) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample ID | Primary Samples | Check Samples | 10 % Difference | Actual Difference | Standard Deviation-2 |
| GEMSNRD1309 | 42.51 | 40.51 | 4.25 | 2.00 | 2.823 |
| GEMSNRD2306 | 21.80 | 19.15 | 2.18 | 2.65 | 3.742 |
| GEMSNRD2307 | 32.36 | 33.92 | 3.24 | 1.56 | 2.206 |
| GEMSNRD2308 | 26.23 | 27.61 | 2.62 | 1.38 | 1.947 |
| GEMSNRD2309 | 32.31 | 31.56 | 3.23 | 0.75 | 1.059 |
| GEMSNRD3508 | 50.65 | 50.98 | 5.07 | 0.33 | 0.464 |
| GEMSNRD4514 | 36.15 | 36.87 | 3.62 | 0.71 | 1.007 |
| GEMSNRD4515 | 39.61 | 41.32 | 3.96 | 1.72 | 2.427 |
| GEMSNRD5708 | 49.22 | 47.73 | 4.92 | 1.49 | 2.107 |
| GEMSNRD5709 | 40.44 | 46.64 | 4.04 | 6.20 | 8.764 |
| GEMSNRD5713 | 42.44 | 43.15 | 4.24 | 0.71 | 1.001 |
| GEMSNRD5714 | 23.01 | 22.40 | 2.30 | 0.62 | 0.873 |
| GEMSNRD5716 | 18.12 | 21.75 | 1.81 | 3.63 | 5.135 |
| GEMSNRD5717 | 18.76 | 19.05 | 1.88 | 0.29 | 0.413 |
| GEMSNRD5718 | 40.92 | 38.86 | 4.09 | 2.06 | 2.916 |
| GEMSNRD101 | 31.12 | 30.46 | 3.11 | 0.65 | 0.921 |

Fig 9.33  Scatter Plot of Primary Vs Check samples for SiO2 (Values in %).

| Table 9.8 Primary and Check Sample values for Na2O (in %) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample ID | Primary Samples | Check Samples | 10 % Difference | Actual Difference | Standard Deviation-2 |
| GEMSNRD1309 | 0.46 | 0.26 | 0.05 | 0.20 | 0.287 |
| GEMSNRD2306 | 0.08 | 0.08 | 0.01 | 0.00 | 0.001 |
| GEMSNRD2307 | 0.26 | 0.18 | 0.03 | 0.08 | 0.109 |
| GEMSNRD2308 | 0.31 | 0.21 | 0.03 | 0.11 | 0.150 |
| GEMSNRD2309 | 0.41 | 0.32 | 0.04 | 0.10 | 0.139 |
| GEMSNRD3508 | 0.45 | 0.29 | 0.04 | 0.15 | 0.214 |
| GEMSNRD4514 | 0.31 | 0.12 | 0.03 | 0.19 | 0.266 |
| GEMSNRD4515 | 0.43 | 0.32 | 0.04 | 0.11 | 0.150 |
| GEMSNRD5708 | 0.39 | 0.22 | 0.04 | 0.17 | 0.239 |
| GEMSNRD5709 | 0.34 | 0.18 | 0.03 | 0.16 | 0.226 |
| GEMSNRD5713 | 0.44 | 0.29 | 0.04 | 0.15 | 0.215 |
| GEMSNRD5714 | 0.30 | 0.15 | 0.03 | 0.15 | 0.211 |
| GEMSNRD5716 | 0.21 | 0.10 | 0.02 | 0.11 | 0.156 |
| GEMSNRD5717 | 0.17 | 0.08 | 0.02 | 0.09 | 0.122 |
| GEMSNRD5718 | 0.34 | 0.20 | 0.03 | 0.14 | 0.194 |
| GEMSNRD101 | 0.14 | 0.11 | 0.01 | 0.03 | 0.045 |

Fig 9.34  Scatter Plot of Primary Vs Check samples for Na2O  (Values in %).

| Table 9.9 Primary and Check Sample values for K2O (in %) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample ID | Primary Samples | Check Samples | 10 % Difference | Actual Difference | Standard Deviation-2 |
| GEMSNRD1309 | 0.20 | 0.18 | 0.02 | 0.01 | 0.018 |
| GEMSNRD2306 | 0.02 | 0.05 | 0.00 | 0.03 | 0.042 |
| GEMSNRD2307 | 0.04 | 0.05 | 0.00 | 0.01 | 0.014 |
| GEMSNRD2308 | 0.04 | 0.06 | 0.00 | 0.02 | 0.027 |
| GEMSNRD2309 | 0.06 | 0.06 | 0.01 | 0.01 | 0.010 |
| GEMSNRD3508 | 1.47 | 1.48 | 0.15 | 0.01 | 0.018 |
| GEMSNRD4514 | 0.01 | 0.05 | 0.00 | 0.04 | 0.061 |
| GEMSNRD4515 | 0.02 | 0.05 | 0.00 | 0.03 | 0.040 |
| GEMSNRD5708 | 1.52 | 1.40 | 0.15 | 0.12 | 0.174 |
| GEMSNRD5709 | 0.92 | 0.62 | 0.09 | 0.30 | 0.423 |
| GEMSNRD5713 | 1.45 | 1.41 | 0.14 | 0.04 | 0.052 |
| GEMSNRD5714 | 0.24 | 0.24 | 0.02 | 0.00 | 0.001 |
| GEMSNRD5716 | 0.16 | 0.23 | 0.02 | 0.08 | 0.106 |
| GEMSNRD5717 | 0.19 | 0.14 | 0.02 | 0.05 | 0.064 |
| GEMSNRD5718 | 0.03 | 0.05 | 0.00 | 0.02 | 0.031 |
| GEMSNRD101 | 0.07 | 0.07 | 0.01 | 0.01 | 0.011 |

Fig 9.35  Scatter Plot of Primary Vs Check samples for K2O  (Values in %).

| Table 9.10 Primary and Check Sample values for CaO (in %) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample ID | Primary Samples | Check Samples | 10 % Difference | Actual Difference | Standard Deviation-2 |
| GEMSNRD1309 | 0.26 | 2.46 | 0.03 | 2.20 | 3.116 |
| GEMSNRD2306 | 0.65 | 0.52 | 0.06 | 0.12 | 0.174 |
| GEMSNRD2307 | 0.79 | 0.79 | 0.08 | 0.00 | 0.004 |
| GEMSNRD2308 | 1.08 | 1.07 | 0.11 | 0.01 | 0.020 |
| GEMSNRD2309 | 1.10 | 1.03 | 0.11 | 0.07 | 0.099 |
| GEMSNRD3508 | 6.47 | 7.57 | 0.65 | 1.09 | 1.547 |
| GEMSNRD4514 | 0.19 | 0.19 | 0.02 | 0.00 | 0.001 |
| GEMSNRD4515 | 0.42 | 0.44 | 0.04 | 0.02 | 0.021 |
| GEMSNRD5708 | 4.60 | 8.60 | 0.46 | 4.00 | 5.660 |
| GEMSNRD5709 | 14.45 | 16.08 | 1.45 | 1.63 | 2.304 |
| GEMSNRD5713 | 0.64 | 0.88 | 0.06 | 0.24 | 0.345 |
| GEMSNRD5714 | 0.26 | 0.23 | 0.03 | 0.03 | 0.047 |
| GEMSNRD5716 | 0.30 | 0.28 | 0.03 | 0.02 | 0.034 |
| GEMSNRD5717 | 8.05 | 4.44 | 0.80 | 3.61 | 5.100 |
| GEMSNRD5718 | 0.43 | 0.89 | 0.04 | 0.47 | 0.658 |
| GEMSNRD101 | 1.25 | 1.21 | 0.12 | 0.03 | 0.047 |

Fig 9.36  Scatter Plot of Primary Vs Check samples for CaO  (Values in %).

The following discussion is based on the samples which were analyzed for minor & trace elements & REE  using ICP-MS analysis.

The descriptive statistical analysis of all samples (Laterite, limestone, clay) indicate that the Sc is ranging from 3.83ppm–115.52ppm, Ga ranges from 5.94ppm-73.13ppm, Li ranges from 2.94ppm-252.80ppm, Y ranges from 0ppm-1261.66ppm TREE ranges from 1.31ppm-2458.14ppm and TREE+Sc+Y ranges from 14.65ppm-3605.38ppm. Four samples which showed anomalous outlier values for some elements have not been included in descriptive statistics.

| Table 9.11 Descriptive statistics of Minor &Trace |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | Sc (ppm) | Ga (ppm) | Li (ppm) | Y (ppm) | TREE (ppm) | TREE+Sc+Y (ppm) |
| Mean | 21.627 | 23.725 | 41.464 | 33.454 | 163.871 | 218.951 |
| Standard Error | 0.962 | 0.750 | 1.991 | 4.978 | 11.951 | 16.359 |
| Median | 16.182 | 19.689 | 31.281 | 21.981 | 139.532 | 181.177 |
| Mode | 48.139 | 47.550 | #N/A | #N/A | #N/A | #N/A |
| Standard Deviation | 17.693 | 13.796 | 36.604 | 91.518 | 219.712 | 300.756 |
| Sample Variance | 313.028 | 190.329 | 1339.837 | 8375.619 | 48273.500 | 90454.278 |
| Kurtosis | 6.291 | 2.463 | 8.883 | 115.612 | 80.986 | 84.486 |
| Skewness | 2.410 | 1.615 | 2.667 | 10.111 | 8.497 | 8.701 |
| Range | 111.694 | 67.187 | 249.863 | 1261.669 | 2456.837 | 3590.732 |
| Minimum (ppm) | 3.834 | 5.943 | 2.944 | 0.000 | 1.310 | 14.655 |
| Maximum (ppm) | 115.528 | 73.130 | 252.807 | 1261.669 | 2458.147 | 3605.387 |
| Count | 338.000 | 338.000 | 338.000 | 338.000 | 338.000 | 338.000 |

Distribution trend in each Borehole

The depth wise minor & trace elements & REE distribution pattern for individual boreholes is discussed below.

BH-01

In BH-01, two lithologies were identified based on visual observation:  4.5m of Aluminous laterite and 8.5m of Lateritic clay,

- TREEO + Sc₂O₃ + Y₂O₃: in Aluminous laterite ranging from 117.65-330.38 ppm and Lateritic clay ranging from 107.02-170.84 ppm. LREE: HREE ranges from 0.81-0.92.

- Scandium:  in the Aluminous laterite is ranging from 11.22-71.72 ppm and in Lateritic clay is ranging from 32.77-74.99 ppm.

- Gallium: ranges from 45.48-71.60 ppm in Aluminous laterite and 30.09-42.70 ppm in Lateritic clay.

Fig 9.37   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-01.

BH-03

In BH-03, four lithologies were identified based on visual observation:  4.5m of limonitic clay, 9.0 m of clay, 1.5 m of marl/limestone and 1.5 m of variegated clay.

- TREEO + Sc₂O₃ + Y₂O₃: in limonitic clay ranging from 260.49 -300.59 ppm, in clay ranging from 259.83-303.01 ppm, in marl/limestone 246.15 ppm and in variegated clay 321.94 ppm LREE: HREE ranges from 0.89-0.94.

- Scandium:  in limonitic clay is ranging from 0.87-22.46 ppm, in clay is ranging from 18.78-24.61 ppm, in marl/limestone 16.58 ppm and in variegated clay 20.16 ppm.

- Gallium: ranges from 18.90-23.37 ppm in limonitic clay, 19.60-29.21 ppm in clay, 15.54 ppm in marl/limestone and 17.30 ppm in variegated clay.

Fig 9.38 Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-03.

BH-05

In BH-05, three lithologies were identified based on visual observation:  9.0 m of clay, 6.0 m of ferruginous clay/siliceous clay/ limonitic, 3.0m of variegated clay.

- TREEO + Sc₂O₃ + Y₂O₃: in clay ranging from 97.54 -293.14 ppm, in ferruginous clay/siliceous clay/ limonitic clay ranging from 153.54-272.57 ppm, and in variegated clay ranging 222.13-289.70 ppm. LREE: HREE ranges from 0.89-0.94.

- Scandium: in clay ranging from 16.67 -23.19 ppm, in ferruginous clay/siliceous clay/ limonitic clay ranging from 8.77-20.25 ppm, and in variegated clay ranging 23.60-24.56 ppm.

- Gallium: ranges from 16.25-28.28 ppm in clay, 9.52-22.02 ppm in ferruginous clay/siliceous clay/ limonitic clay, and 22.68-26.36 ppm in variegated clay.

Fig 9.39   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-05.

BH-07

In BH-07, only one lithology was identified based on visual observation: Clay 25.5m.

- TREEO + Sc₂O₃ + Y₂O₃: in clay ranging from 179.42–306.19 ppm. LREE: HREE ranges from 0.90–0.94.

- Scandium: in clay ranging from 10.31–22.89 ppm.

- Gallium: in clay ranging from 10.73–29.32 ppm.

Fig 9.40   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-07.

BH-09

In BH-09, two lithologies were identified based on visual observation: 16.5 m of ferruginous clay/siliceous clay/limonitic clay (3.0–16.5 m and 19.5–21.0 m) and 3.0 m of variegated clay (16.5–19.5 m).

- TREEO + Sc₂O₃ + Y₂O₃: in ferruginous clay/siliceous clay/limonitic clay ranging from 96.85–229.63 ppm, and in variegated clay ranging 225.31–256.11 ppm. LREE: HREE ranges from 0.89–0.94.

- Scandium: in ferruginous clay/siliceous clay/limonitic clay ranging from 8.03–14.32 ppm, and in variegated clay ranging 13.75–16.35 ppm.

- Gallium: ranges from 9.40–19.93 ppm in ferruginous clay/siliceous clay/limonitic clay, and 16.57–17.22 ppm in variegated clay.

Fig 9.41   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-09.

BH-13

In BH-13, five lithologies were identified based on visual observation: 6.5 m of marl (1.5–8.0 m), 0.5 m of ferruginous laterite (8.0–8.5 m), 5.0 m of bauxitic clay (8.5–13.5 m), 3.0 m of variegated clay (13.5–16.5 m), and 1.5 m of ferruginous clay (16.5–18.0 m).

- TREEO + Sc₂O₃ + Y₂O₃: in marl ranging from 180.78–340.30 ppm, in ferruginous laterite 166.79 ppm, in bauxitic clay ranging from 171.19–467.85 ppm, in variegated clay ranging from 119.71–214.05 ppm, and in ferruginous clay 124.47 ppm. LREE: HREE ranges from 0.74–0.97.

- Scandium: in marl ranging from 8.49–24.15 ppm, in ferruginous laterite 23.30 ppm, in bauxitic clay ranging from 32.52–40.78 ppm, in variegated clay ranging from 52.32–58.80 ppm, and in ferruginous clay 66.66 ppm.

- Gallium: ranges from 8.51–20.10 ppm in marl, 72.44 ppm in ferruginous laterite, 36.17–54.77 ppm in bauxitic clay, 25.74–33.07 ppm in variegated clay, and 25.14 ppm in ferruginous clay.

Fig 9.42   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-13.

BH-23

In BH-23, 6 types of lithology were identified based on visual observation; lateritic soil- 6 m, aluminous laterite-1 m, ferruginous laterite-4.5 m, shell limestone-3.0 m, green clay-12.0 m and limonitic clay-3.0m.

- TREEO + Sc₂O₃ + Y₂O₃: in lateritic soil ranging from 133.43–228.32 ppm, in aluminous laterite 185.47 ppm, in ferruginous laterite ranging from 177.69–4346.616 ppm (with a very high value of 4346.616 ppm at 12.5–14.0 m), in shell limestone ranging from 889.25–3505.583 ppm, in green clay ranging from 107.87–230.19 ppm, and in limonitic clay ranging from 171.04–178.29 ppm. LREE: HREE ratio ranges from 0.60–0.95.

- Scandium: in lateritic soil ranging from 12.30–27.27 ppm, in aluminous laterite 37.48 ppm, in ferruginous laterite ranging from 23.25–66.27 ppm, in shell limestone ranging from 29.20–62.01 ppm, in green clay ranging from 21.20–34.49 ppm, and in limonitic clay ranging from 23.34–45.96 ppm.

- Gallium: ranges from 44.13–47.55 ppm in lateritic soil, 46.90 ppm in aluminous laterite, 42.12–45.35 ppm in ferruginous laterite, 34.01–44.18 ppm in shell limestone, 15.64–28.03 ppm in green clay, and 15.25–24.65 ppm in limonitic clay.

Fig 9.43   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-23.

BH-26

In BH-26, four lithologies were identified based on visual observation: 9.0 m of clay (1.5–3.0 m; 12.0–19.5 m), 1.5 m of lateritic clay (3.0–4.5 m), 4.5 m of ferruginous clay/siliceous clay/limonitic clay (4.5–6.0 m; 9.0–12.0 m), and 3.0 m of variegated clay (6.0–9.0 m).

- TREEO + Sc₂O₃ + Y₂O₃: in clay ranging from 119.77–269.21 ppm, in lateritic clay 120.70 ppm, in ferruginous clay/siliceous clay/limonitic clay ranging from 71.98–225.53 ppm, and in variegated clay ranging from 89.93–332.51 ppm. LREE: HREE ranges from 0.87–0.93.

- Scandium: in clay ranging from 7.42–15.48 ppm, in lateritic clay 9.30 ppm, in ferruginous clay/siliceous clay/limonitic clay ranging from 6.50–13.46 ppm, and in variegated clay ranging from 13.40–17.81 ppm.

- Gallium: ranges from 10.57–23.86 ppm in clay, 9.26 ppm in lateritic clay, 9.10–18.28 ppm in ferruginous clay/siliceous clay/limonitic clay, and 19.47–19.99 ppm in variegated clay.

Fig 9.44   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-26.

BH-28

In BH-28, five lithologies were identified based on visual observation: 1.5 m of lateritic clay (4.5–6.0 m), 1.5 m of limestone/marl (6.0–7.5 m), 4.5 m of variegated clay (7.5–12.0 m), 1.5 m of clay (13.5–15.0 m), and 6.0 m of ferruginous clay/siliceous clay/limonitic clay (12.0–13.5 m; 15.0–19.5 m).

- TREEO + Sc₂O₃ + Y₂O₃: in lateritic clay 160.85 ppm, in limestone/marl 148.37 ppm, in variegated clay ranging from 66.01–220.80 ppm, in clay 27.07 ppm, and in ferruginous clay/siliceous clay/limonitic clay ranging from 33.49–276.67 ppm. LREE: HREE ranges from 0.84–0.93.

- Scandium: in lateritic clay 9.56 ppm, in limestone/marl 9.85 ppm, in variegated clay ranging from 12.29–19.89 ppm, in clay 6.72 ppm, and in ferruginous clay/siliceous clay/limonitic clay ranging from 11.33–16.22 ppm.

- Gallium: ranges from 10.64 ppm in lateritic clay, 9.59 ppm in limestone/marl, 14.21–19.64 ppm in variegated clay, 18.85 ppm in clay, and 16.95–20.47 ppm in ferruginous clay/siliceous clay/limonitic clay.

Fig 9.45   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-28.

BH-30

In BH-30, three lithologies were identified based on visual observation: 8.0 m of clay (2.0–4.0 m; 7.0–8.0 m; 13.0–17.0 m), 6.0 m of ferruginous clay/siliceous clay/limonitic clay (4.0–7.0 m; 8.0–9.0 m; 12.0–13.0 m), and 6.0 m of variegated clay (9.0–12.0 m; 17.0–20.0 m).

- TREEO + Sc₂O₃ + Y₂O₃: in clay ranging from 161.47–266.63 ppm, in ferruginous clay/siliceous clay/limonitic clay ranging from 137.57–193.43 ppm, and in variegated clay ranging from 144.07–232.85 ppm. LREE: HREE ranges from 0.89–0.94.

- Scandium: in clay ranging from 8.97–19.02 ppm, in ferruginous clay/siliceous clay/limonitic clay ranging from 9.88–14.88 ppm, and in variegated clay ranging from 9.75–18.08 ppm.

- Gallium: ranges from 10.27–20.49 ppm in clay, 9.22–15.16 ppm in ferruginous clay/siliceous clay/limonitic clay, and 8.89–21.42 ppm in variegated clay.

Fig 9.46  Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-30.

BH-32

In  BH-32, five lithologies were identified based on visual observation: 3.0 m of clay (6.0–9.0 m), 4.5 m of ferruginous clay/siliceous clay/limonitic clay (9.0–10.5 m; 12.0–13.5 m; 18.0–19.5 m), 1.5 m of olive & brown clay/siliceous/calcareous clay (10.5–12.0 m), 1.5 m of limonitic clay (13.5–15.0 m), and 3.0 m of variegated clay (15.0–18.0 m).

- TREEO + Sc₂O₃ + Y₂O₃: in clay ranging from 176.78–178.33 ppm, in ferruginous clay/siliceous clay/limonitic clay ranging from 122.20–260.94 ppm, in olive & brown clay/siliceous/calcareous clay 155.61 ppm, in limonitic clay 174.99 ppm, and in variegated clay ranging from 164.40–377.73 ppm. LREE: HREE ranges from 0.90–0.95.

- Scandium: in clay ranging from 10.45–10.81 ppm, in ferruginous clay/siliceous clay/limonitic clay ranging from 6.42–13.12 ppm, in olive & brown clay/siliceous/calcareous clay 6.77 ppm, in limonitic clay 6.26 ppm, and in variegated clay ranging from 9.26–13.10 ppm.

- Gallium: ranges from 14.99–16.38 ppm in clay, 8.96–14.01 ppm in ferruginous clay/siliceous clay/limonitic clay, 9.91 ppm in olive & brown clay/siliceous/calcareous clay, 7.54 ppm in limonitic clay, and 10.00–16.57 ppm in variegated clay.

Fig 9.47   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-32.

BH-35

In BH-35, four lithologies were identified based visual observation: 9.0 m of clay, 1.0 m of white clay, 1.5 m of limonitic clay, 3.0 m of ferruginous clay.

- TREEO + Sc₂O₃ + Y₂O₃: ranging from 187.05-354.45 ppm in clay, in ferruginous clay ranging from 158.11-205.75 ppm, in limonitic clay 283.92 ppm and in white clay 172.76 ppm. LREEO:HREEO ratio is ranging from 0.90-0.94.

- Scandium: in clay ranging from 9.74-16.42 ppm, in ferruginous clay ranging from 7.78-18.18 ppm, in limonitic clay 12.69 ppm, and in white clay 11 ppm.

- Gallium: in clay ranging from 13.06- 20.69 ppm , in ferruginous clay ranging from 18.19-26.18 ppm, in limonitic clay 17.94 ppm, and in white clay 21.16 ppm.

Fig 9.48   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-35.

BH-45

In BH-45, three lithologies were identified based on physical observation, 11.0 m of ferruginous laterite, 7.5 m of ferruginous clay and green clay of 3.0 m.

- TREEO + Sc₂O₃ + Y₂O₃: ranging from 151.72-312.92 ppm in ferruginous laterite, in ferruginous clay ranging from 152.99-892.36 ppm and in green clay 280.46-373.17 ppm. LREEO:HREEO ratio is ranging from 0.79-0.94.

- Scandium: in ferruginous laterite ranging from 15.93-48.73 ppm, in ferruginous clay ranging from 56.07-107.32 ppm and in green clay 64.13-68.80 ppm.

- Gallium: in ferruginous laterite ranging from 40.37-73.13 ppm, in ferruginous clay ranging from 36.54-44.57 ppm and in green clay 20.43-37.45 ppm.

Fig 9.49   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-45.

BH-47

In BH-47, two lithologies were identified based on physical observations; 7.5 m of clay, 9.0m of variegated clay.

- TREEO + Sc₂O₃ + Y₂O₃: in clay ranging from 148.94-310.51 ppm and in variegated clay ranging from 54.45-294.34 ppm. LREEO:HREEO range is 0.83-0.92.

- Scandium: in clay ranging from 11.11-17.79 ppm and in variegated clay ranging from 9.41-15.99 ppm.

- Gallium: in clay ranging from 13.94-20.18 ppm and in variegated clay ranging from 8.60-22.84 ppm.

Fig 9.50   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-47.

BH-49

In BH-49, three lithologies were identified based on the physical observation, 13.5 m of clay, 1.5 m of ferruginous clay/siliceous clay/limonitic clay and in 1.5 m of variegated clay.

- TREEO + Sc₂O₃ + Y₂O₃: in clay ranging from 185.02-333.94 ppm, in ferruginous clay/siliceous clay/limonitic 293.508 ppm and in variegated clay 260.719 ppm. LREEO: HREEO range is 0.89-0.94 .

- Scandium: in clay ranging from 11.10-23.28 ppm, in ferruginous clay/siliceous clay/limonitic 20.06 ppm and in variegated clay 16.14 ppm.

- Gallium: in clay ranging from 12.21-30.02 ppm, in ferruginous clay/siliceous clay/limonitic 19.70 ppm and in variegated clay 19.10 ppm.

Fig 9.51   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-49.

BH-51

In BH-51, three lithologies were identified based on physical observation; 4.5m of variegated clay, 3.0m ferruginous clay/siliceous clay/ limonitic clay and 1.5m of clayey soil.

- TREEO + Sc₂O₃ + Y₂O₃: in variegated clay ranging from 252.34-381.92 ppm, in ferruginous clay/siliceous clay/limonitic ranging from 231.38-284.23 ppm and in clayey soil 132.828 ppm. LREEO: HREEO range is 0.91-0.94.

- Scandium: in variegated clay ranging from 13.72-15.38 ppm, in ferruginous clay/siliceous clay/limonitic ranging from 14.39-22.97 ppm and in clayey soil 13.10 ppm.

- Gallium: in variegated clay ranging from 19.25-22.48 ppm, in ferruginous clay/siliceous clay/limonitic ranging from 13.16-22.30 ppm and in clayey soil 12.19 ppm.

Fig 9.52   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-51.

BH-53
In BH-53, two lithologies were identified based on physical observation; 3.0m of variegated clay, 9.0m ferruginous clay/siliceous clay/ limonitic clay.

- TREEO + Sc₂O₃ + Y₂O₃: in variegated clay ranging from 203.43-211.0 ppm, in ferruginous clay/siliceous clay/limonitic ranging from 204.14-286.76 ppm. LREEO: HREEO range is 0.92-0.94.

- Scandium: in variegated clay ranging from 18.73-22.94 ppm, in ferruginous clay/siliceous clay/limonitic ranging from 9.24-17.12 ppm.

- Gallium: in variegated clay ranging from 17.77-20.20 ppm, in ferruginous clay/siliceous clay/limonitic ranging from 11.59-23.32 ppm.

Fig 9.53   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-53.

BH-55

In BH-55, five lithologies were identified based on physical observation; 4.5 m of olive& brown clay/siliceous/calcareous, 3.0 m of clay/siliceous/calcareous, 1.5 m of limonitic clay, 6.0 m of clay and 1.5 m of ferruginous clay/siliceous clay/limonitic clay.

- TREEO + Sc₂O₃ + Y₂O₃: in olive& brown clay/siliceous/calcareous clay ranging from 137.08-167.31 ppm, in clay/siliceous/calcareous ranging from 133.53-164.62 ppm, in limonitic clay 148.07ppm, in clay 161.26-170.79 ppm and in ferruginous/siliceous/limonitic 184.26 ppm. LREEO: HREEO range is 0.93-.95.

- Scandium: in olive& brown clay/siliceous/calcareous clay ranging from 7.23-8.50 ppm, in clay/siliceous/calcareous ranging from 7.27-8.52 ppm, in limonitic clay 6.74 ppm, in clay 6.20-8.50 ppm and in ferruginous/siliceous/limonitic 8.26 ppm

- Gallium: in olive& brown clay/siliceous/calcareous clay ranging from 10.7-14.68 ppm, in clay/siliceous/calcareous ranging from 12.06-49.91 ppm, in limonitic clay 10.92 ppm, in clay 10.08-12.82 ppm and in ferruginous/siliceous/limonitic 13.63 ppm

Fig 9.54   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-55.

BH-57

In BH-57, eleven lithologies were identified based on physical observation; 4.5 m of bauxitic clay, 1.0 m of bauxitic/ferruginous clay, 2.0 m of bauxitic /siliceous clay, 2.0 m of ferruginous clay, 1.5 m of ferruginous/bauxitic/variegated clay, 3.0 m of ferruginous/silceous/limonitic clay, 1.5 m of ferruginous clay/variegated clay, 1.0 m of ferruginous soil, 2.0 m of ferruginous/lateritic soil, 5.50 m of green/bauxitix clay and 13.0 m of variegated clay.

- TREEO + Sc₂O₃ + Y₂O₃: in bauxitic clay ranges from 124.17-604.12 ppm, bauxitic/ferruginous clay 342.97 ppm, in bauxitic /siliceous clay ranges from 366.32-533.82 ppm, in ferruginous clay ranges from 255.17-279.68 ppm, in ferruginous/bauxitic/variegated clay 3418.83 ppm, in ferruginous/silceous/limonitic clay ranges from 351.79-49.21 ppm, in ferruginous clay/variegated clay 364.89 ppm, in ferruginous soil 140.80 ppm, in ferruginous/lateritic soil ranges from 163.66-190.51 ppm, in green/bauxitix clay Ranges from 123.95-1578.99 ppm and in variegated clay 220.79-743.38 ppm. LREEO: HREEO range is 0.68-0.97.

- Scandium: in bauxitic clay ranges from 13.29-23.76 ppm, bauxitic/ferruginous clay 27.63 ppm, in bauxitic /siliceous clay ranges from 13.81-25.75 ppm, in ferruginous clay ranges from 16.38-38.68 ppm, in ferruginous/bauxitic/variegated clay 53.78 ppm, in ferruginous/silceous/limonitic clay ranges from 43.03-47.86 ppm, in ferruginous clay/variegated clay 41.07 ppm, in ferruginous soil 7.90 ppm, in ferruginous/lateritic soil ranges from 9.10-9.74 ppm, in green/bauxitic clay Ranges from 23.52-34.28 ppm and in variegated clay 10.17-24.29 ppm.

- Gallium: in bauxitic clay ranges from 63.78-66.97 ppm, bauxitic/ferruginous clay 27.86 ppm, in bauxitic /siliceous clay ranges from 37.87-47.55 ppm, in ferruginous clay ranges from 30.19-35.77 ppm, in ferruginous/bauxitic/variegated clay 41.19 ppm, in ferruginous/silceous/limonitic clay ranges from 20.96-30.59 ppm, in ferruginous clay/variegated clay 32.36 ppm, in ferruginous soil 15.25 ppm, in ferruginous/lateritic soil ranges from 19.48-22.15 ppm, in green/bauxitic clay Ranges from 14.37-29.45 ppm and in variegated clay 13.61-47.29 ppm.

Fig 9.55   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-57.

BH-67

In BH-67, four lithologies were identified based on the physical observation; 8.5 m of clay, 8.0 m of ferruginous/siliceous/limonitic clay, 1.0m of lateritic clay and 1.5 m of bauxitic clay/siliceous clay.

- TREEO + Sc₂O₃ + Y₂O₃: in clay ranges from 238.45-339.74 ppm, in ferruginous/siliceous/limonitic clay ranges from 162.58-1099.30 ppm, in lateritic clay 414.31 ppm, in bauxitic/siliceous clay 294.71 ppm.

- Scandium: in clay ranges from 14.36-60.11 ppm, in ferruginous/siliceous/limonitic clay ranges from 16.69-87.19 ppm, in lateritic clay 16.48 ppm, in bauxitic/siliceous clay 61.04 ppm.

- Gallium: in clay ranges from 14.36-60.11 ppm, in ferruginous/siliceous/limonitic clay ranges from 16.69-87.19 ppm, in lateritic clay 16.48 ppm, in bauxitic/siliceous clay 61.04 ppm.

Fig 9.56 Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-67.

BH-69

In BH-69, three lithologies were identified based on physical observation, 3.0 m of lateritic clay, 1.50 m of red ferruginous clay, 1.50 m of clay.

- TREEO + Sc₂O₃ + Y₂O₃: in lateritic clay ranges from 21.98-29.54 ppm, in red ferruginous clay 43.34 ppm and in clay 255.28 ppm. LREEO: HREEO range is 0.54-0.90.

- Scandium: in lateritic clay ranges from 13.34-18.13 ppm, in red ferruginous clay 7.72 ppm and in clay 17.26 ppm.

- Gallium: in lateritic clay ranges from 26.67-28.78 ppm, in red ferruginous clay 20.69 ppm and in clay 20.70 ppm.

Fig 9.57   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-69.

BH-71

In BH-71, three lithologies were identified based on the physical observations, 12.50 m of clay, 3.0 m of limonitic clay, 1.5 m of variegated clay.

- TREEO + Sc₂O₃ + Y₂O₃: in clay ranges from 108.83-289.12 ppm, in limonitic clay ranges from 289.21-325.67 ppm and in variegated clay 186.30ppm. LREEO: HREEO range is 0.8-0.92 .

- Scandium: in clay ranges from 10.07-19.26 ppm, in limonitic clay ranges from 17.82-2.54 ppm and in variegated clay 13.83 ppm.

- Gallium: in clay ranges from 15.77-23.10 ppm, in limonitic clay ranges 18.34-19.10 ppm and in variegated clay 14.03 ppm.

Fig 9.58   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-71.

BH-73

In BH-73, four lithologies were identified based on the physical observation, 4.5 m of clay, 7.5 m of ferruginous/siliceous/limonitic clay, 3.0 m of variegated clay, 1.5 m of clay/siliceous/calcareous.

- TREEO + Sc₂O₃ + Y₂O₃: in clay ranges from 104.06-257.15 ppm, in ferruginous/siliceous/limonitic clay 94.92-376.76 ranges from ppm, in variegated clay ranges from 111.08-341.17 ppm and in clay/siliceous/calcareous 118.97 ppm. LREEO: HREEO range is 0.91-0.94.

- Scandium: in clay ranges from 6.37-14.86 ppm, in ferruginous/siliceous/limonitic clay ranges from 7.17-17.36 ppm, in variegated clay ranges from 11.47-15.77 ppm and in clay/siliceous/calcareous 6.95 ppm.

- Gallium: in clay ranges from 9.40-21.71 ppm, in ferruginous/siliceous/limonitic clay ranges from 8.14-25.39 ppm, in variegated clay ranges from 16.83-25.52 ppm and in clay/siliceous/calcareous 11.08 ppm.

Fig 9.59   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-73.

BH-75

In Bh-75, four lithologies were identified based on physical observation, 9.0 m of ferruginous/siliceous/limonitic clay, 1.5 m of variegated clay, 1.5 m of ferruginous / variegated clay and 1.5m of clay/ferruginous clay.

- TREEO + Sc₂O₃ + Y₂O₃: in ferruginous/siliceous/limonitic clay ranges from 158.57-251.60 ppm, in variegated clay 139.36 ppm, in ferruginous / variegated clay 285.98 ppm and in clay/ferruginous clay 197.67 ppm. LREEO: HREEO range is 0.90-0.94.

- Scandium: in ferruginous/siliceous/limonitic clay ranges from 8.84-14.43 ppm, in variegated clay 11.02 ppm, in ferruginous / variegated clay 16.51 ppm and in clay/ferruginous clay 10.30 ppm.

- Gallium: in ferruginous/siliceous/limonitic clay ranges from 10.36-20.61 ppm, in variegated clay 12.62 ppm, in ferruginous / variegated clay 23.49 ppm and in clay/ferruginous clay 13.36 ppm.

Fig 9.60   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-75.

BH-77

In BH-77, three lithologies were identified based on the physical observation; 90 m of ferruginous/siliceous/limonitic clay, 1.5m of olive & brown clay/siliceous/calcareous, 1.5 m of limonitic clay.

- TREEO + Sc₂O₃ + Y₂O₃: in ferruginous/siliceous/limonitic clay ranges from 114.33-255.86 ppm, in olive & brown clay/siliceous/calcareous 168.10 ppm and in limonitic clay 200.61 ppm. LREEO: HREEO range is 0.90-0.94.

- Scandium: in ferruginous/siliceous/limonitic clay ranges from 5.90-14.87 ppm, in olive & brown clay/siliceous/calcareous 8.38 ppm and in limonitic clay 8.10 ppm.

- Gallium: in ferruginous/siliceous/limonitic clay ranges from 10.29-24.06  ppm, in olive & brown clay/siliceous/calcareous 13.68 ppm and in limonitic clay 12.24 ppm.

Fig 9.61   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-77.

BH-81

In BH-81, four lithologies were identified based on physical observation; 5.5 m of ferruginous clay, 4.0 m of bauxitic/siliceous clay, 3.0 m of clay and 1.0 m of laterite.

- TREEO + Sc₂O₃ + Y₂O₃: in ferruginous clay ranges from 76.66-573.63 ppm, in bauxitic/siliceous clay ranges from 149.81-248.38 ppm, in clay ranges from 108.65-349.59 ppm and in laterite 118.90 ppm. LREEO: HREEO range is 0.55-0.98.

- Scandium: in ferruginous clay ranges from 35.86-115.53 ppm, in bauxitic/siliceous clay ranges from 14.94-29.65 ppm, in clay ranges from 17.31-41.57 ppm and in laterite 19.25 ppm.

- Gallium: in ferruginous clay ranges from 28.18-37.82 ppm, in bauxitic/siliceous clay ranges from 57.84-69.78 ppm, in clay ranges from 39.63-73.10  ppm and in laterite 49.62 ppm.

Fig 9.62   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-81.

BH-91

In BH-91, five lithologies were identified based on physical observation, 3.0 m of ferruginous/siliceous/limonitic clay, 6.0 m of limonitic clay, 4.5 m of variegated clay, 3.0 m of clay/siliceous/calcareous, and 1.5 m of clay.

- TREEO + Sc₂O₃ + Y₂O₃: in ferruginous/siliceous/limonitic clay ranges from 169.33-354.02 ppm, in limonitic clay ranges from 123.81-213.64 ppm, in variegated clay ranges from 254.16-313.26 ppm, in clay/siliceous/calcareous ranges from 124.99-147.01 ppm and in clay 234.26 ppm. LREEO: HREEO range is 0.88-0.94.

- Scandium: in ferruginous/siliceous/limonitic clay ranges from 7.68-11.63 ppm, in limonitic clay ranges from 3.83-7.16 ppm, in variegated clay ranges from 16.49-26.06 ppm, in clay/siliceous/calcareous ranges from 5.85-6.67 ppm and in clay 13.55 ppm.

- Gallium: in ferruginous/siliceous/limonitic clay ranges from 12.66-20.81 ppm, in limonitic clay ranges from 5.94-9.58 ppm, in variegated clay ranges from 21.48-26.34 ppm, in clay/siliceous/calcareous ranges from 8.25-10.10 ppm and in clay 14.12 ppm.

Fig 9.63   Vertical distribution pattern of TREEO+Sc2​O3​+Y2​O3 in BH-91.

Primary and check samples analysis result comparison

The check samples were analysed in the NABL accredited lab at Shiva Analyticals Private Ltd Bangalore and results were compared with the results obtained from the primary lab and the same is discussed below.

Comparison of ICPMS Results.

The ICPMS analysis of primary and check samples are shown in the figures below.

Comparative analysis shows that the differences between primary and check samples are well within standard deviation-2 and hence acceptable.

The tables below show the details of  primary and check samples comparison

| Table 9.12 Primary and Check Sample values for TREE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample ID | Primary Samples | Check Samples | 10 % Difference | Actual Difference | Standard Deviation-2 |
| GEMSNRD101 | 180.54 | 90.19 | 18.05 | 90.35 | 127.775 |
| GEMSNRD1309 | 244.55 | 246.73 | 24.45 | 2.18 | 3.082 |
| GEMSNRD1310 | 232.28 | 233.12 | 23.23 | 0.84 | 1.190 |
| GEMSNRD2306 | 491.31 | 402.92 | 49.13 | 88.40 | 125.013 |
| GEMSNRD2307 | 2172.12 | 1930.52 | 217.21 | 241.59 | 341.666 |
| GEMSNRD2308 | 2295.34 | 1923.06 | 229.53 | 372.28 | 526.483 |
| GEMSNRD2309 | 252.58 | 212.55 | 25.26 | 40.03 | 56.616 |
| GEMSNRD3504 | 197.83 | 175.06 | 19.78 | 22.76 | 32.194 |
| GEMSNRD3508 | 240.11 | 181.42 | 24.01 | 58.69 | 82.997 |
| GEMSNRD3509 | 189.49 | 178.93 | 18.95 | 10.56 | 14.933 |
| GEMSNRD4512 | 107.28 | 76.26 | 10.73 | 31.02 | 43.865 |
| GEMSNRD4514 | 325.20 | 239.21 | 32.52 | 85.99 | 121.612 |
| GEMSNRD4515 | 166.97 | 148.36 | 16.70 | 18.61 | 26.318 |
| GEMSNRD4702 | 188.31 | 208.04 | 18.83 | 19.73 | 27.898 |
| GEMSNRD4707 | 144.78 | 167.68 | 14.48 | 22.90 | 32.391 |
| GEMSNRD5708 | 243.24 | 197.01 | 24.32 | 46.23 | 65.379 |
| GEMSNRD5709 | 294.51 | 182.73 | 29.45 | 111.78 | 158.079 |
| GEMSNRD5713 | 577.74 | 459.61 | 57.77 | 118.12 | 167.052 |
| GEMSNRD5714 | 320.44 | 255.27 | 32.04 | 65.17 | 92.168 |
| GEMSNRD5716 | 463.64 | 502.11 | 46.36 | 38.48 | 54.413 |
| GEMSNRD5717 | 271.76 | 349.96 | 27.18 | 78.19 | 110.584 |
| GEMSNRD5718 | 398.51 | 423.28 | 39.85 | 24.77 | 35.035 |
| GEMSNRD5719 | 231.94 | 216.19 | 23.19 | 15.75 | 22.276 |
| GEMSNRD5722 | 238.22 | 205.34 | 23.82 | 32.88 | 46.499 |
| GEMSNRD5723 | 275.04 | 262.30 | 27.50 | 12.75 | 18.027 |
| GEMSNRD5724 | 186.40 | 216.70 | 18.64 | 30.29 | 42.840 |
| GEMSNRD5725 | 507.89 | 357.51 | 50.79 | 150.39 | 212.679 |
| GEMSNRD6705 | 240.89 | 232.54 | 24.09 | 8.34 | 11.801 |
| GEMSNRD6713 | 274.57 | 180.05 | 27.46 | 94.52 | 133.669 |
| GEMSNRD6902 | 157.28 | 189.99 | 15.73 | 32.71 | 46.254 |
| GEMSNRD8105 | 261.08 | 455.05 | 26.11 | 193.97 | 274.315 |
| GEMSNRD8106 | 209.67 | 299.74 | 20.97 | 90.08 | 127.390 |
| GEMSNRD8111 | 329.89 | 372.07 | 32.99 | 42.19 | 59.661 |

Fig 9.64: Scatter Plot of Primary Vs Check samples for TREE (Values in ppm)

| Table 9.13 Primary and Check Sample values for TREE+Sc+Y |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample ID | Primary Samples | Check Samples | 10 % Difference | Actual Difference | Standard Deviation-2 |
| GEMSNRD101 | 260.00 | 137.48 | 26.00 | 122.51 | 173.26 |
| GEMSNRD1309 | 293.73 | 289.55 | 29.37 | 4.18 | 5.91 |
| GEMSNRD1310 | 280.33 | 280.14 | 28.03 | 0.19 | 0.27 |
| GEMSNRD2306 | 581.43 | 511.11 | 58.14 | 70.32 | 99.45 |
| GEMSNRD2307 | 2925.84 | 2507.60 | 292.58 | 418.24 | 591.48 |
| GEMSNRD2308 | 3605.39 | 2992.27 | 360.54 | 613.12 | 867.08 |
| GEMSNRD2309 | 716.56 | 596.03 | 71.66 | 120.53 | 170.46 |
| GEMSNRD3504 | 244.75 | 224.93 | 24.48 | 19.82 | 28.03 |
| GEMSNRD3508 | 295.39 | 232.93 | 29.54 | 62.46 | 88.33 |
| GEMSNRD3509 | 236.27 | 232.38 | 23.63 | 3.90 | 5.51 |
| GEMSNRD4512 | 248.74 | 216.27 | 24.87 | 32.47 | 45.93 |
| GEMSNRD4514 | 443.46 | 352.31 | 44.35 | 91.15 | 128.90 |
| GEMSNRD4515 | 293.51 | 288.00 | 29.35 | 5.51 | 7.79 |
| GEMSNRD4702 | 256.30 | 270.93 | 25.63 | 14.63 | 20.70 |
| GEMSNRD4707 | 192.76 | 208.70 | 19.28 | 15.94 | 22.55 |
| GEMSNRD5708 | 296.11 | 253.87 | 29.61 | 42.23 | 59.73 |
| GEMSNRD5709 | 363.65 | 234.83 | 36.37 | 128.82 | 182.18 |
| GEMSNRD5713 | 626.10 | 510.95 | 62.61 | 115.14 | 162.84 |
| GEMSNRD5714 | 365.97 | 301.64 | 36.60 | 64.33 | 90.98 |
| GEMSNRD5716 | 510.73 | 565.24 | 51.07 | 54.52 | 77.10 |
| GEMSNRD5717 | 307.38 | 396.12 | 30.74 | 88.74 | 125.50 |
| GEMSNRD5718 | 446.92 | 466.40 | 44.69 | 19.48 | 27.56 |
| GEMSNRD5719 | 282.87 | 262.17 | 28.29 | 20.70 | 29.27 |
| GEMSNRD5722 | 298.09 | 274.07 | 29.81 | 24.02 | 33.97 |
| GEMSNRD5723 | 335.51 | 338.86 | 33.55 | 3.35 | 4.74 |
| GEMSNRD5724 | 282.46 | 310.41 | 28.25 | 27.96 | 39.54 |
| GEMSNRD5725 | 1281.01 | 798.55 | 128.10 | 482.46 | 682.30 |
| GEMSNRD6705 | 342.94 | 310.44 | 34.29 | 32.50 | 45.96 |
| GEMSNRD6713 | 432.56 | 368.63 | 43.26 | 63.93 | 90.41 |
| GEMSNRD6902 | 210.27 | 252.46 | 21.03 | 42.20 | 59.67 |
| GEMSNRD8105 | 292.29 | 508.90 | 29.23 | 216.61 | 306.33 |
| GEMSNRD8106 | 240.25 | 332.18 | 24.02 | 91.93 | 130.01 |
| GEMSNRD8111 | 453.74 | 468.23 | 45.37 | 14.49 | 20.49 |

Fig 9.65: Scatter Plot of Primary Vs Check samples for TREE+Sc+Y (Values in ppm)

| Table 9.14 Primary and Check Sample values for Scandium |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample ID | Primary Samples | Check Samples | 10 % Difference | Actual Difference | Standard Deviation-2 |
| GEMSNRD101 | 71.72 | 42.11 | 7.17 | 29.61 | 41.88 |
| GEMSNRD1309 | 37.55 | 31.52 | 3.76 | 6.03 | 8.52 |
| GEMSNRD1310 | 39.52 | 39.13 | 3.95 | 0.39 | 0.56 |
| GEMSNRD2306 | 66.27 | 86.29 | 6.63 | 20.01 | 28.30 |
| GEMSNRD2307 | 62.01 | 68.06 | 6.20 | 6.05 | 8.55 |
| GEMSNRD2308 | 48.37 | 57.40 | 4.84 | 9.03 | 12.77 |
| GEMSNRD2309 | 29.20 | 34.75 | 2.92 | 5.55 | 7.84 |
| GEMSNRD3504 | 11.10 | 15.33 | 1.11 | 4.24 | 5.99 |
| GEMSNRD3508 | 15.29 | 18.57 | 1.53 | 3.29 | 4.65 |
| GEMSNRD3509 | 12.69 | 16.88 | 1.27 | 4.19 | 5.93 |
| GEMSNRD4512 | 78.39 | 92.44 | 7.84 | 14.05 | 19.87 |
| GEMSNRD4514 | 77.01 | 82.33 | 7.70 | 5.31 | 7.52 |
| GEMSNRD4515 | 68.80 | 88.80 | 6.88 | 20.00 | 28.29 |
| GEMSNRD4702 | 17.79 | 19.39 | 1.78 | 1.60 | 2.26 |
| GEMSNRD4707 | 12.15 | 10.01 | 1.21 | 2.14 | 3.02 |
| GEMSNRD5708 | 15.91 | 20.78 | 1.59 | 4.87 | 6.88 |
| GEMSNRD5709 | 10.42 | 10.50 | 1.04 | 0.08 | 0.12 |
| GEMSNRD5713 | 24.29 | 30.26 | 2.43 | 5.97 | 8.44 |
| GEMSNRD5714 | 23.76 | 28.57 | 2.38 | 4.81 | 6.80 |
| GEMSNRD5716 | 21.53 | 32.86 | 2.15 | 11.33 | 16.02 |
| GEMSNRD5717 | 13.81 | 23.33 | 1.38 | 9.52 | 13.46 |
| GEMSNRD5718 | 25.75 | 21.99 | 2.58 | 3.77 | 5.33 |
| GEMSNRD5719 | 27.63 | 25.37 | 2.76 | 2.26 | 3.20 |
| GEMSNRD5722 | 41.07 | 51.16 | 4.11 | 10.09 | 14.26 |
| GEMSNRD5723 | 43.03 | 58.74 | 4.30 | 15.71 | 22.22 |
| GEMSNRD5724 | 47.86 | 41.12 | 4.79 | 6.74 | 9.53 |
| GEMSNRD5725 | 30.21 | 33.88 | 3.02 | 3.67 | 5.18 |
| GEMSNRD6705 | 16.48 | 13.81 | 1.65 | 2.67 | 3.78 |
| GEMSNRD6713 | 87.19 | 91.71 | 8.72 | 4.52 | 6.39 |
| GEMSNRD6902 | 17.26 | 18.83 | 1.73 | 1.57 | 2.22 |
| GEMSNRD8105 | 18.84 | 29.03 | 1.88 | 10.19 | 14.41 |
| GEMSNRD8106 | 17.31 | 9.81 | 1.73 | 7.50 | 10.61 |
| GEMSNRD8111 | 115.53 | 87.74 | 11.55 | 27.79 | 39.30 |

Fig 9.66: Scatter Plot of Primary Vs Check samples for Scandium (Values in ppm)

| Table 9.15 Primary and Check Sample values for Yttrium |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample ID | Primary Samples | Check Samples | 10 % Difference | Actual Difference | Standard Deviation-2 |
| GEMSNRD101 | 7.73 | 5.18 | 0.77 | 2.55 | 3.61 |
| GEMSNRD1309 | 11.64 | 11.31 | 1.16 | 0.33 | 0.47 |
| GEMSNRD1310 | 8.54 | 7.90 | 0.85 | 0.64 | 0.90 |
| GEMSNRD2306 | 23.84 | 21.90 | 2.38 | 1.94 | 2.74 |
| GEMSNRD2307 | 691.71 | 509.01 | 69.17 | 182.70 | 258.37 |
| GEMSNRD2308 | 1261.67 | 1011.80 | 126.17 | 249.87 | 353.37 |
| GEMSNRD2309 | 434.78 | 348.73 | 43.48 | 86.05 | 121.69 |
| GEMSNRD3504 | 35.83 | 34.54 | 3.58 | 1.30 | 1.83 |
| GEMSNRD3508 | 40.00 | 32.94 | 4.00 | 7.06 | 9.98 |
| GEMSNRD3509 | 34.10 | 36.57 | 3.41 | 2.47 | 3.49 |
| GEMSNRD4512 | 63.07 | 47.56 | 6.31 | 15.51 | 21.93 |
| GEMSNRD4514 | 41.24 | 30.78 | 4.12 | 10.47 | 14.80 |
| GEMSNRD4515 | 57.74 | 50.84 | 5.77 | 6.90 | 9.76 |
| GEMSNRD4702 | 50.19 | 43.50 | 5.02 | 6.69 | 9.46 |
| GEMSNRD4707 | 35.83 | 31.01 | 3.58 | 4.82 | 6.82 |
| GEMSNRD5708 | 36.95 | 36.08 | 3.70 | 0.87 | 1.23 |
| GEMSNRD5709 | 58.73 | 41.60 | 5.87 | 17.12 | 24.22 |
| GEMSNRD5713 | 24.07 | 21.08 | 2.41 | 2.99 | 4.23 |
| GEMSNRD5714 | 21.78 | 17.81 | 2.18 | 3.97 | 5.61 |
| GEMSNRD5716 | 25.56 | 30.27 | 2.56 | 4.71 | 6.66 |
| GEMSNRD5717 | 21.80 | 22.83 | 2.18 | 1.03 | 1.45 |
| GEMSNRD5718 | 22.66 | 21.14 | 2.27 | 1.52 | 2.15 |
| GEMSNRD5719 | 23.30 | 20.62 | 2.33 | 2.68 | 3.79 |
| GEMSNRD5722 | 18.80 | 17.58 | 1.88 | 1.23 | 1.73 |
| GEMSNRD5723 | 17.44 | 17.83 | 1.74 | 0.39 | 0.55 |
| GEMSNRD5724 | 48.20 | 52.60 | 4.82 | 4.40 | 6.23 |
| GEMSNRD5725 | 742.91 | 407.17 | 74.29 | 335.74 | 474.80 |
| GEMSNRD6705 | 85.57 | 64.09 | 8.56 | 21.48 | 30.38 |
| GEMSNRD6713 | 70.80 | 96.86 | 7.08 | 26.07 | 36.86 |
| GEMSNRD6902 | 35.72 | 43.64 | 3.57 | 7.92 | 11.20 |
| GEMSNRD8105 | 12.37 | 24.82 | 1.24 | 12.45 | 17.61 |
| GEMSNRD8106 | 13.27 | 22.63 | 1.33 | 9.36 | 13.23 |
| GEMSNRD8111 | 8.32 | 8.42 | 0.83 | 0.09 | 0.13 |

Fig 9.67: Scatter Plot of Primary Vs Check samples for Yttrium (Values in ppm)

| Table 9.16 Primary and Check Sample values for Gallium |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sample ID | Primary Samples | Check Samples | 10 % Difference | Actual Difference | Standard Deviation-2 |
| GEMSNRD101 | 71.60 | 35.65 | 7.16 | 35.95 | 50.84 |
| GEMSNRD1309 | 44.03 | 41.88 | 4.40 | 2.15 | 3.04 |
| GEMSNRD1310 | 41.96 | 42.45 | 4.20 | 0.50 | 0.70 |
| GEMSNRD2306 | 45.14 | 20.30 | 4.51 | 24.83 | 35.12 |
| GEMSNRD2307 | 44.18 | 22.14 | 4.42 | 22.04 | 31.17 |
| GEMSNRD2308 | 45.35 | 20.84 | 4.54 | 24.51 | 34.67 |
| GEMSNRD2309 | 34.01 | 13.48 | 3.40 | 20.53 | 29.04 |
| GEMSNRD3504 | 16.71 | 13.43 | 1.67 | 3.28 | 4.63 |
| GEMSNRD3508 | 20.69 | 14.61 | 2.07 | 6.08 | 8.60 |
| GEMSNRD3509 | 17.94 | 16.35 | 1.79 | 1.58 | 2.24 |
| GEMSNRD4512 | 42.25 | 33.35 | 4.22 | 8.89 | 12.58 |
| GEMSNRD4514 | 44.57 | 32.89 | 4.46 | 11.67 | 16.51 |
| GEMSNRD4515 | 37.45 | 31.44 | 3.74 | 6.00 | 8.49 |
| GEMSNRD4702 | 20.18 | 15.12 | 2.02 | 5.06 | 7.16 |
| GEMSNRD4707 | 12.66 | 8.78 | 1.27 | 3.88 | 5.48 |
| GEMSNRD5708 | 25.25 | 18.33 | 2.53 | 6.92 | 9.78 |
| GEMSNRD5709 | 17.91 | 8.17 | 1.79 | 9.74 | 13.77 |
| GEMSNRD5713 | 47.29 | 39.31 | 4.73 | 7.98 | 11.28 |
| GEMSNRD5714 | 66.97 | 56.49 | 6.70 | 10.48 | 14.82 |
| GEMSNRD5716 | 63.78 | 57.35 | 6.38 | 6.42 | 9.08 |
| GEMSNRD5717 | 47.55 | 51.84 | 4.76 | 4.29 | 6.07 |
| GEMSNRD5718 | 37.87 | 39.69 | 3.79 | 1.82 | 2.57 |
| GEMSNRD5719 | 27.86 | 27.98 | 2.79 | 0.13 | 0.18 |
| GEMSNRD5722 | 32.36 | 26.43 | 3.24 | 5.93 | 8.39 |
| GEMSNRD5723 | 30.59 | 26.23 | 3.06 | 4.36 | 6.16 |
| GEMSNRD5724 | 20.96 | 20.42 | 2.10 | 0.54 | 0.76 |
| GEMSNRD5725 | 29.45 | 16.11 | 2.95 | 13.34 | 18.86 |
| GEMSNRD6705 | 19.15 | 9.71 | 1.91 | 9.44 | 13.35 |
| GEMSNRD6713 | 37.34 | 27.66 | 3.73 | 9.68 | 13.68 |
| GEMSNRD6902 | 20.70 | 17.08 | 2.07 | 3.62 | 5.12 |
| GEMSNRD8105 | 73.10 | 75.18 | 7.31 | 2.08 | 2.94 |
| GEMSNRD8106 | 64.65 | 65.10 | 6.46 | 0.45 | 0.64 |
| GEMSNRD8111 | 37.82 | 29.56 | 3.78 | 8.26 | 11.69 |

Fig 9.68: Scatter Plot of Primary Vs Check samples for Gallium (Values in ppm)

9.6.2 Discussion

Initially the project in Naredi was aimed at exploring for Bauxite and critical elements. After detailed geological mapping and initially drilling of 7 boreholes, it was established that the lithology in the block was dominated by clay sediments and small pockets of limestone and bauxite/laterite. So this necessitated for re-orientation of the project, with approval from TCC, to explore for clay hosted critical elements and REE. The following exploration threshold values for clay hosted REE and critical elements were used to delineate potential mineralized zones in the block, Based on the values stated in the Report of the committee on threshold values for exploration of critical minerals/elements. (64th CGPB meeting, 2025)

- (TREEO+SC2O3+Y2O3): 350 ppm

- Sc: 20 ppm and 50 ppm.

- Ga: 30 ppm and 50 ppm.

Based on the exploration Threshold values TREEO+SC2O3+Y2O3 was more than 350 ppm in 11 out 27 boreholes.

From the above, a zone of 7 boreholes out of the 11 boreholes in which (TREEO+SC2O3+Y2O3) > 350 ppm, was delineated in the eastern part of the block, accounting for 34.1m out of the total 40.1m REE enriched lithology; 4 isolated small zones are seen in the southern and western part of the block are as shown in the fig below.

Fig. 9.69: Map of (TREEO+SC2O3+Y2O3)>350 ppm.

Table 9.17:  Borehole numbers with (TREEO+SC2O3+Y2O3)>350 ppm.

Similarly, samples from 14 out of 27 boreholes analyzed shows Sc>20 ppm; and samples from 7 out of these 14 boreholes analyzed show Sc>50 ppm. The figure below shows the locations of boreholes in which Scandium enrichment was seen.

Fig. 9.70:  Map of Boreholes with Scandium values >20 ppm and 50 ppm

Further in 8 boreholes out of 27 boreholes, the samples analysed for Ga>30 ppm; out of these, 5 boreholes had samples with Ga>50 ppm. The figure below shows the locations of boreholes in which Scandium enrichment was seen.

Fig. 9.71:  Map of Boreholes with Gallium values >30 ppm and 50 ppm

Characterization of clay

X-Ray diffraction studies on clay.

6 clay samples were drawn from the split cores in such a way that 4 of the samples were the ones which analysed higher values of TREE and 2 samples from clay horizons from two different boreholes.

The details of samples and their results (XRD graphs) are tabulated below.

SAMPLE. No 01

Table 9.18  Physical, Chemical and XRD characteristics of sample no 01

| SAMPLE. NO | DEPTH |  | AS PER BOREHOLE LOGGING | ICP-MS results(ppm) | MINERALS identified in XRD | % | COMPOSITION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GEMSNRD1307 | FROM(m) | TO(m) | laterite/bauxite clay rich sample. | (TREEO+SC2O3+Y2O3) =467.85 | GIBBSITE | 23.31 | Al(OH)3 |
|  | 8.5 | 9.5 |  |  | KAOLINITE | 70.07 | Al2(Si2O5)(OH)4 |
|  |  |  |  | Sc = 32.52 | TARAMITE | 3.89 | Na(CaNa)(Mg3Al2)(Al2Si6O22)(OH)2 |
|  |  |  |  | Ga= 54.77 | BARRINGERITE | 2.74 | (Fe,Ni)2P |

Fig 9.72 XRD graph of sample no 01

SAMPLE. No 02

Table 9.19  Physical, Chemical and XRD characteristics of sample no 02

| SAMPLE. NO | DEPTH |  | AS PER BOREHOLE LOGGING | ICP-MS results(ppm) | MINERALS identified in XRD | % | COMPOSITION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GEMSNRD4513 | FROM(m) | TO(m) | ferruginous purple and white coloured clay. | (TREEO+SC2O3+Y2O3) =892.36 | KAOLINITE | 63.22 | Al2(Si2O5)(OH)4 |
|  | 15.5 | 17.0 |  |  |  |  |  |
|  |  |  |  | Sc = 107.32 | Al2.22 Ca1.88 Fe0.6 H2 Mg3.68 Na 0.75 O 24 Si 6.52 Ti 0.05 | 36.78 | Al2.22 Ca1.88 Fe0.6 H2 Mg3.68 Na 0.75 O 24 Si 6.52 Ti 0.05 |
|  |  |  |  | Ga= 36.54 |  |  |  |

Fig 9.73  XRD graph of sample no 02

SAMPLE. No 03

Table 9.20  Physical, Chemical and XRD characteristics of sample no 03

| SAMPLE. NO | DEPTH |  | AS PER BOREHOLE LOGGING | ICP-MS results(ppm) | MINERALS identified in XRD | % | COMPOSITION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GEMSNRD6714 | FROM(m) | TO(m) | brownish and purplish ferruginous clay with whitish patches and fragments friable. | (TREEO+SC2O3+Y2O3) = 1099.30 | KAOLINITE | 64.51 | Al2(Si2O5)(OH)4 |
|  | 19.5 | 21.0 |  | Sc = 82.38 | Fe3 ((Fe Si)(OH)5)O4 | 11.66 | Fe3 ((Fe Si)(OH)5)O4 |
|  |  |  |  | Ga= 48.53 | Al7.62 Ca18.1 Fe2.55 Mg 0.63 Na0.9 Si 18 Ti 2.2 O 78 | 23.83 | Al7.62 Ca18.1 Fe2.55 Mg 0.63 Na0.9 Si 18 Ti 2.2 O 78 |

Fig 9.74  XRD graph of sample no 03

SAMPLE. No 04

Table 9.21 Physical, Chemical and XRD characteristics of sample no 04

| SAMPLE. NO | DEPTH |  | AS PER BOREHOLE LOGGING | ICP-MS results(ppm) | MINERALS identified in XRD | % | COMPOSITION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GEMSNRD5720 | FROM(m) | TO(m) | ferruginous clay with white clay content inside for 1 meter and next 50cm fully red ferruginous clay. | (TREEO+SC2O3+Y2O3) = 3418.83 | KAOLINITE | 93.88 | Al2(Si2O5)(OH)4 |
|  | 23.0 | 24.5 |  |  |  |  |  |
|  |  |  |  | Sc = 53.78 | GOETHITE | 6.12 | Fe3+O(OH) |
|  |  |  |  | Ga= 41.19 |  |  |  |

Fig 9.75 XRD graph of sample no 04

SAMPLE. No 05

Table 9.22  Physical, Chemical and XRD characteristics of sample no 05

| SAMPLE. NO | DEPTH |  | AS PER BOREHOLE LOGGING | ICP-MS results(ppm) | MINERALS identified in XRD | % | COMPOSITION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GEMSNRD109 | FROM(m) | TO(m) | very fine grained soft clay like dark brown laterite with white patches. | (TREEO+SC2O3+Y2O3) = 107.02 | KAOLINITE | 75.05 | Al2(Si2O5)(OH)4 |
|  | 14.0 | 15.0 |  |  |  |  |  |
|  |  |  |  | Sc = 32.77 | HEMATITE | 24.95 | Fe2O3 |
|  |  |  |  | Ga= 42.70 |  |  |  |

Fig 9.76 XRD graph of sample no 05

SAMPLE. No 06

| SAMPLE. NO | DEPTH |  | AS PER BOREHOLE LOGGING | ICP-MS results(ppm) | MINERALS identified in XRD | % | COMPOSITION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GEMSNRD6707 | FROM(m) | TO(m) | Greyish, ferrugenous/lateritic/bauxitic clay with siliceous/whitish grey spots/patches(slightly harder then other clay) and friable. | (TREEO+SC2O3+Y2O3) = 191.23 | GIBBSITE | 5.63 | Al(OH)3 |
|  | 9.0 | 10.5 |  |  | RICHTERITE | 41.34 | Na(NaCa)Mg5(Si8O22)(OH)2 |
|  |  |  |  | Sc = 16.74 | GOETHITE | 8.23 | Fe3+O(OH) |
|  |  |  |  |  | QUARTZ LOW | 29.93 | SiO2 |
|  |  |  |  | Ga= 28.32 | DELTA-Al(OH)3 | 14.87 | DELTA-Al(OH)3 |

Table 9.23 Physical, Chemical and XRD characteristics of sample no 06

Fig 9.77 XRD graph of sample no 06

From the above it is evident that the clay sediments in naredi block are dominantly rich in kaolinites with other minerals being hematite, goethite, gibbsite and a few other ferromagnesium minerals.

Chemical characterization of clay

Out of a total of 326 clay samples collected from the drilled cores, 136 representative samples drawn from all the 27 boreholes were analysed for major elements and LOI using XRF. Based on the results of these 132 samples, the K₂O:Na₂O ratio was calculated using the formula K₂O/(K₂O + Na₂O). This ratio is used to indicate certain chemical characteristics of the clay, such as maturity and degree of weathering. Of the 132 samples, 83 samples have a ratio of >0.5, indicating that the clay maybe rich in illite; the clays may have experienced more intense weathering. Hence grouped as mature clays.

Remaining 49 samples have a ratio of <0.5, suggesting that these samples, contain higher amounts of Na2O; they may be montmorillonite-rich/ bentonitic clays; the clays may have undergone less intense weathering. Hence these are grouped as are immature clays.

The table below shows the distribution of key indicator elements in the clay samples.

| Table 9.24: 49 samples of immature clay |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample. Id | CaO (%) | K2O (%) | Na2O (%) | Na2O/Cao (%) | K2O/(K2O+Na2O) (%) | Al2O3(%) | SiO2 (%) | clay type |
| GEMSNRD105 | 1.285 | 0.007 | 0.211 | 0.164 | 0.032 | 28.514 | 33.763 | immature clay high in plagioclase feldspar |
| GEMSNRD0106 | 1.793 | 0.002 | 0.190 | 0.106 | 0.010 | 29.836 | 33.884 | immature clay high in plagioclase feldspar |
| GEMSNRD107 | 0.139 | 0.001 | 0.204 | 1.468 | 0.005 | 29.955 | 35.250 | immature clay high in plagioclase feldspar |
| GEMSNRD108 | 0.192 | 0.005 | 0.246 | 1.281 | 0.020 | 30.438 | 34.305 | immature clay high in plagioclase feldspar |
| GEMSNRD109 | 0.133 | 0.009 | 0.229 | 1.722 | 0.038 | 27.430 | 31.506 | immature clay high in plagioclase feldspar |
| GEMSNRD110 | 0.156 | 0.002 | 0.213 | 1.365 | 0.009 | 32.183 | 35.269 | immature clay high in plagioclase feldspar |
| GEMSNRD111 | 0.198 | 0.004 | 0.201 | 1.015 | 0.020 | 30.619 | 33.666 | immature clay high in plagioclase feldspar |
| GEMSNRD112 | 0.223 | 0.007 | 0.205 | 0.919 | 0.033 | 29.501 | 31.680 | immature clay high in plagioclase feldspar |
| GEMSNRD1307 | 0.719 | 0.270 | 0.375 | 0.522 | 0.419 | 42.404 | 29.706 | immature clay high in plagioclase feldspar |
| GEMSNRD1309 | 0.261 | 0.196 | 0.464 | 1.778 | 0.297 | 37.586 | 42.507 | immature clay high in plagioclase feldspar |
| GEMSNRD1310 | 0.215 | 0.146 | 0.472 | 2.195 | 0.236 | 36.813 | 43.255 | immature clay high in plagioclase feldspar |
| GEMSNRD1311 | 0.477 | 0.014 | 0.432 | 0.906 | 0.031 | 36.473 | 43.484 | immature clay high in plagioclase feldspar |
| GEMSNRD1312 | 3.380 | 0.085 | 0.307 | 0.091 | 0.217 | 30.897 | 37.837 | immature clay high in plagioclase feldspar |
| GEMSNRD1313 | 0.137 | 0.003 | 0.286 | 2.088 | 0.010 | 36.246 | 42.501 | immature clay high in plagioclase feldspar |
| GEMSNRD1314 | 0.176 | 0.004 | 0.214 | 1.216 | 0.018 | 25.311 | 31.581 | immature clay high in plagioclase feldspar |
| GEMSNRD2310 | 1.021 | 0.037 | 0.489 | 0.479 | 0.070 | 17.552 | 37.435 | immature clay high in plagioclase feldspar |
| GEMSNRD2311 | 2.891 | 0.058 | 0.542 | 0.187 | 0.097 | 16.188 | 37.303 | immature clay high in plagioclase feldspar |
| GEMSNRD2312 | 1.941 | 0.056 | 0.728 | 0.375 | 0.071 | 15.168 | 43.900 | immature clay high in plagioclase feldspar |
| GEMSNRD2313 | 1.443 | 0.103 | 0.884 | 0.613 | 0.104 | 16.515 | 42.609 | immature clay high in plagioclase feldspar |
| GEMSNRD2314 | 2.089 | 0.186 | 0.953 | 0.456 | 0.163 | 14.030 | 44.560 | immature clay high in plagioclase feldspar |
| GEMSNRD2315 | 2.027 | 0.084 | 1.076 | 0.531 | 0.072 | 18.467 | 46.655 | immature clay high in plagioclase feldspar |
| GEMSNRD2316 | 3.445 | 0.116 | 0.922 | 0.268 | 0.112 | 18.323 | 43.313 | immature clay high in plagioclase feldspar |
| GEMSNRD2317 | 5.709 | 0.153 | 1.049 | 0.184 | 0.127 | 16.533 | 42.222 | immature clay high in plagioclase feldspar |
| GEMSNRD2318 | 3.150 | 0.233 | 1.050 | 0.333 | 0.182 | 17.406 | 46.759 | immature clay high in plagioclase feldspar |
| GEMSNRD2319 | 3.674 | 0.127 | 1.010 | 0.275 | 0.112 | 18.603 | 45.076 | immature clay high in plagioclase feldspar |
| GEMSNRD4510 | 0.387 | 0.002 | 0.178 | 0.460 | 0.011 | 31.729 | 36.204 | immature clay high in plagioclase feldspar |
| GEMSNRD4511 | 0.160 | 0.002 | 0.229 | 1.431 | 0.009 | 33.478 | 38.390 | immature clay high in plagioclase feldspar |
| GEMSNRD4512 | 0.182 | 0.002 | 0.215 | 1.181 | 0.009 | 31.955 | 37.509 | immature clay high in plagioclase feldspar |
| GEMSNRD4513 | 0.190 | 0.005 | 0.261 | 1.374 | 0.019 | 32.706 | 40.310 | immature clay high in plagioclase feldspar |
| GEMSNRD4514 | 0.187 | 0.007 | 0.310 | 1.658 | 0.022 | 35.537 | 36.154 | immature clay high in plagioclase feldspar |
| GEMSNRD4515 | 0.421 | 0.022 | 0.427 | 1.014 | 0.049 | 30.901 | 39.607 | immature clay high in plagioclase feldspar |
| GEMSNRD4516 | 1.138 | 0.040 | 0.656 | 0.576 | 0.057 | 20.777 | 42.751 | immature clay high in plagioclase feldspar |
| GEMSNRD5714 | 0.261 | 0.237 | 0.303 | 1.161 | 0.439 | 47.880 | 23.013 | immature clay high in plagioclase feldspar |
| GEMSNRD5715 | 0.588 | 0.056 | 0.064 | 0.109 | 0.467 | 56.885 | 3.540 | immature clay high in plagioclase feldspar |
| GEMSNRD5716 | 0.300 | 0.155 | 0.211 | 0.703 | 0.423 | 51.873 | 18.121 | immature clay high in plagioclase feldspar |
| GEMSNRD5718 | 0.429 | 0.028 | 0.339 | 0.790 | 0.076 | 38.826 | 40.924 | immature clay high in plagioclase feldspar |
| GEMSNRD5719 | 0.167 | 0.013 | 0.319 | 1.910 | 0.039 | 38.201 | 41.593 | immature clay high in plagioclase feldspar |
| GEMSNRD5720 | 0.316 | 0.019 | 0.314 | 0.994 | 0.057 | 33.133 | 36.692 | immature clay high in plagioclase feldspar |
| GEMSNRD5721 | 0.262 | 0.020 | 0.251 | 0.958 | 0.074 | 25.723 | 29.169 | immature clay high in plagioclase feldspar |
| GEMSNRD5722 | 0.223 | 0.010 | 0.351 | 1.574 | 0.028 | 34.589 | 38.772 | immature clay high in plagioclase feldspar |
| GEMSNRD5723 | 0.573 | 0.063 | 0.513 | 0.895 | 0.109 | 31.502 | 40.355 | immature clay high in plagioclase feldspar |
| GEMSNRD5724 | 0.775 | 0.072 | 0.679 | 0.876 | 0.096 | 24.284 | 41.968 | immature clay high in plagioclase feldspar |
| GEMSNRD6709 | 0.651 | 0.059 | 0.313 | 0.481 | 0.159 | 36.829 | 42.229 | immature clay high in plagioclase feldspar |
| GEMSNRD6710 | 0.235 | 0.024 | 0.302 | 1.285 | 0.074 | 37.547 | 42.019 | immature clay high in plagioclase feldspar |
| GEMSNRD6903 | 3.928 | 0.190 | 0.206 | 0.052 | 0.480 | 24.858 | 17.002 | immature clay high in plagioclase feldspar |
| GEMSNRD6906 | 0.953 | 0.079 | 0.302 | 0.317 | 0.207 | 24.817 | 27.643 | immature clay high in plagioclase feldspar |
| GEMSNRD8108 | 1.876 | 0.103 | 0.214 | 0.114 | 0.325 | 49.106 | 17.664 | immature clay high in plagioclase feldspar |
| GEMSNRD8110 | 1.143 | 0.010 | 0.092 | 0.080 | 0.098 | 38.609 | 1.516 | immature clay high in plagioclase feldspar |
| GEMSNRD8112 | 0.949 | 0.004 | 0.332 | 0.350 | 0.012 | 31.169 | 33.253 | immature clay high in plagioclase feldspar |

Table 9.25:  89 samples of Mature clay.

| sample. Id | CaO (%) | K2O (%) | Na2O (%) | Na2O/ CaO (%) | K2O/ (K2O+ Na2O) (%) | Al2O3 (%) | SiO2 (%) | clay type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEMSNRD0302 | 6.531 | 1.971 | 0.872 | 0.134 | 0.693 | 11.967 | 52.321 | Mature clay, High Illite |
| GEMSNRD0305 | 3.819 | 2.313 | 0.808 | 0.212 | 0.741 | 18.039 | 50.345 | Mature clay, High Illite |
| GEMSNRD0311 | 4.786 | 0.663 | 0.303 | 0.063 | 0.686 | 8.229 | 51.408 | Mature clay, High Illite |
| GEMSNRD0504 | 0.804 | 1.248 | 0.397 | 0.494 | 0.759 | 15.218 | 59.046 | Mature clay, High Illite |
| GEMSNRD0507 | 3.772 | 1.612 | 0.678 | 0.180 | 0.704 | 13.035 | 60.440 | Mature clay, High Illite |
| GEMSNRD0514 | 21.403 | 0.900 | 0.574 | 0.027 | 0.611 | 11.395 | 29.558 | Mature clay, High Illite |
| GEMSNRD0702 | 1.067 | 0.901 | 0.445 | 0.417 | 0.669 | 5.129 | 77.043 | Mature clay, High Illite |
| GEMSNRD0902 | 4.892 | 2.544 | 0.922 | 0.188 | 0.734 | 15.379 | 52.624 | Mature clay, High Illite |
| GEMSNRD0910 | 4.209 | 1.370 | 0.514 | 0.122 | 0.727 | 9.348 | 61.895 | Mature clay, High Illite |
| GEMSNRD2602 | 6.405 | 0.782 | 0.625 | 0.098 | 0.556 | 4.888 | 67.897 | Mature clay, High Illite |
| GEMSNRD2604 | 2.514 | 1.136 | 0.941 | 0.374 | 0.547 | 9.732 | 67.239 | Mature clay, High Illite |
| GEMSNRD2607 | 1.109 | 1.810 | 1.065 | 0.960 | 0.630 | 12.384 | 61.277 | Mature clay, High Illite |
| GEMSNRD2609 | 4.892 | 2.544 | 0.922 | 0.188 | 0.734 | 15.379 | 52.624 | Mature clay, High Illite |
| GEMSNRD2801 | 1.098 | 0.825 | 0.521 | 0.474 | 0.613 | 3.710 | 80.769 | Mature clay, High Illite |
| GEMSNRD2804 | 0.896 | 1.142 | 0.759 | 0.847 | 0.601 | 11.675 | 66.882 | Mature clay, High Illite |
| GEMSNRD2807 | 0.781 | 1.859 | 0.898 | 1.150 | 0.674 | 17.319 | 60.265 | Mature clay, High Illite |
| GEMSNRD2810 | 2.786 | 1.661 | 0.884 | 0.317 | 0.653 | 8.960 | 62.613 | Mature clay, High Illite |
| GEMSNRD3004 | 3.130 | 1.217 | 0.594 | 0.190 | 0.672 | 8.056 | 70.460 | Mature clay, High Illite |
| GEMSNRD3006 | 2.922 | 0.881 | 0.555 | 0.190 | 0.614 | 3.696 | 76.507 | Mature clay, High Illite |
| GEMSNRD3009 | 4.120 | 0.918 | 0.480 | 0.117 | 0.657 | 4.058 | 72.541 | Mature clay, High Illite |
| GEMSNRD3202 | 9.667 | 1.700 | 0.821 | 0.085 | 0.674 | 11.340 | 49.361 | Mature clay, High Illite |
| GEMSNRD3204 | 15.213 | 1.000 | 0.862 | 0.057 | 0.537 | 4.981 | 54.153 | Mature clay, High Illite |
| GEMSNRD3205 | 16.559 | 0.846 | 0.429 | 0.026 | 0.664 | 5.615 | 43.271 | Mature clay, High Illite |
| GEMSNRD3206 | 24.118 | 0.589 | 0.371 | 0.015 | 0.614 | 3.487 | 37.975 | Mature clay, High Illite |
| GEMSNRD3208 | 2.193 | 1.252 | 0.669 | 0.305 | 0.652 | 7.235 | 72.982 | Mature clay, High Illite |
| GEMSNRD3501 | 8.348 | 0.636 | 0.461 | 0.055 | 0.580 | 12.688 | 46.640 | Mature clay, High Illite |
| GEMSNRD3502 | 10.397 | 1.295 | 0.806 | 0.078 | 0.616 | 9.624 | 57.621 | Mature clay, High Illite |
| GEMSNRD3503 | 8.705 | 1.692 | 0.755 | 0.087 | 0.691 | 12.441 | 54.567 | Mature clay, High Illite |
| GEMSNRD3504 | 22.399 | 0.937 | 0.344 | 0.015 | 0.731 | 11.351 | 29.597 | Mature clay, High Illite |
| GEMSNRD3505 | 25.786 | 0.786 | 0.276 | 0.011 | 0.740 | 9.541 | 24.863 | Mature clay, High Illite |
| GEMSNRD3506 | 22.838 | 1.112 | 0.332 | 0.015 | 0.770 | 9.460 | 33.284 | Mature clay, High Illite |
| GEMSNRD3507 | 16.444 | 0.815 | 0.323 | 0.020 | 0.716 | 10.002 | 31.889 | Mature clay, High Illite |
| GEMSNRD3508 | 6.474 | 1.469 | 0.445 | 0.069 | 0.768 | 14.105 | 50.650 | Mature clay, High Illite |
| GEMSNRD3509 | 16.993 | 1.150 | 0.354 | 0.021 | 0.765 | 14.025 | 33.909 | Mature clay, High Illite |
| GEMSNRD3510 | 0.495 | 0.996 | 0.380 | 0.768 | 0.724 | 14.405 | 60.145 | Mature clay, High Illite |
| GEMSNRD3511 | 0.331 | 0.463 | 0.245 | 0.740 | 0.654 | 5.363 | 68.560 | Mature clay, High Illite |
| GEMSNRD4702 | 17.014 | 1.117 | 0.392 | 0.023 | 0.740 | 11.009 | 32.474 | Mature clay, High Illite |
| GEMSNRD4706 | 8.261 | 1.365 | 0.461 | 0.056 | 0.748 | 12.016 | 47.589 | Mature clay, High Illite |
| GEMSNRD4901 | 7.865 | 1.079 | 0.393 | 0.050 | 0.733 | 11.461 | 54.723 | Mature clay, High Illite |
| GEMSNRD4906 | 5.198 | 1.721 | 0.576 | 0.111 | 0.749 | 12.393 | 59.241 | Mature clay, High Illite |
| GEMSNRD4910 | 20.967 | 1.164 | 0.415 | 0.020 | 0.737 | 9.784 | 31.390 | Mature clay, High Illite |
| GEMSNRD5104 | 2.799 | 1.857 | 0.434 | 0.155 | 0.811 | 12.636 | 63.086 | Mature clay, High Illite |
| GEMSNRD5105 | 0.736 | 1.587 | 0.599 | 0.814 | 0.726 | 10.616 | 66.248 | Mature clay, High Illite |
| GEMSNRD5301 | 8.548 | 0.925 | 0.444 | 0.052 | 0.676 | 5.304 | 66.214 | Mature clay, High Illite |
| GEMSNRD5303 | 1.336 | 0.930 | 0.778 | 0.582 | 0.544 | 14.694 | 61.116 | Mature clay, High Illite |
| GEMSNRD5503 | 3.570 | 1.645 | 1.040 | 0.291 | 0.613 | 7.758 | 66.986 | Mature clay, High Illite |
| GEMSNRD5505 | 3.591 | 1.590 | 1.134 | 0.316 | 0.584 | 7.775 | 68.516 | Mature clay, High Illite |
| GEMSNRD5506 | 8.634 | 1.263 | 0.980 | 0.114 | 0.563 | 7.077 | 51.901 | Mature clay, High Illite |
| GEMSNRD5509 | 7.845 | 1.365 | 1.051 | 0.134 | 0.565 | 7.834 | 54.068 | Mature clay, High Illite |
| GEMSNRD5511 | 4.311 | 1.620 | 1.183 | 0.274 | 0.578 | 8.016 | 65.934 | Mature clay, High Illite |
| GEMSNRD5704 | 24.588 | 0.691 | 0.181 | 0.007 | 0.792 | 8.265 | 27.400 | Mature clay, High Illite |
| GEMSNRD5705 | 20.396 | 0.560 | 0.170 | 0.008 | 0.767 | 9.890 | 32.375 | Mature clay, High Illite |
| GEMSNRD5706 | 24.031 | 0.605 | 0.149 | 0.006 | 0.802 | 8.992 | 20.871 | Mature clay, High Illite |
| GEMSNRD5707 | 17.800 | 0.936 | 0.322 | 0.018 | 0.744 | 9.282 | 36.671 | Mature clay, High Illite |
| GEMSNRD5708 | 4.599 | 1.518 | 0.385 | 0.084 | 0.798 | 14.359 | 49.221 | Mature clay, High Illite |
| GEMSNRD5709 | 14.453 | 0.921 | 0.343 | 0.024 | 0.729 | 9.377 | 40.444 | Mature clay, High Illite |
| GEMSNRD5710 | 0.670 | 1.085 | 0.469 | 0.700 | 0.698 | 14.148 | 54.986 | Mature clay, High Illite |
| GEMSNRD5711 | 0.435 | 1.051 | 0.310 | 0.713 | 0.772 | 15.109 | 58.572 | Mature clay, High Illite |
| GEMSNRD5712 | 1.499 | 1.248 | 0.284 | 0.189 | 0.815 | 17.329 | 51.554 | Mature clay, High Illite |
| GEMSNRD5713 | 0.636 | 1.445 | 0.438 | 0.689 | 0.767 | 28.586 | 42.442 | Mature clay, High Illite |
| GEMSNRD5717 | 8.048 | 0.187 | 0.166 | 0.021 | 0.530 | 38.632 | 18.761 | Mature clay, High Illite |
| GEMSNRD6704 | 5.878 | 1.467 | 0.491 | 0.084 | 0.749 | 13.947 | 50.228 | Mature clay, High Illite |
| GEMSNRD6705 | 12.803 | 0.966 | 0.349 | 0.027 | 0.735 | 10.567 | 38.086 | Mature clay, High Illite |
| GEMSNRD6706 | 0.665 | 0.684 | 0.279 | 0.420 | 0.710 | 6.843 | 64.565 | Mature clay, High Illite |
| GEMSNRD6902 | 9.852 | 1.091 | 0.338 | 0.034 | 0.763 | 11.583 | 44.013 | Mature clay, High Illite |
| GEMSNRD7102 | 1.315 | 1.948 | 0.429 | 0.326 | 0.820 | 14.415 | 60.324 | Mature clay, High Illite |
| GEMSNRD7109 | 14.239 | 0.871 | 0.352 | 0.025 | 0.712 | 9.620 | 37.582 | Mature clay, High Illite |
| GEMSNRD7111 | 19.563 | 0.805 | 0.346 | 0.018 | 0.699 | 9.083 | 36.085 | Mature clay, High Illite |
| GEMSNRD7301 | 16.784 | 1.244 | 0.845 | 0.050 | 0.596 | 5.965 | 51.115 | Mature clay, High Illite |
| GEMSNRD7303 | 16.904 | 1.241 | 0.866 | 0.051 | 0.589 | 6.032 | 49.657 | Mature clay, High Illite |
| GEMSNRD7305 | 4.788 | 1.119 | 0.333 | 0.070 | 0.771 | 6.281 | 69.175 | Mature clay, High Illite |
| GEMSNRD7308 | 1.106 | 1.543 | 0.365 | 0.330 | 0.809 | 10.794 | 68.413 | Mature clay, High Illite |
| GEMSNRD7501 | 1.443 | 1.074 | 0.503 | 0.349 | 0.681 | 4.804 | 78.894 | Mature clay, High Illite |
| GEMSNRD7504 | 1.266 | 1.103 | 0.519 | 0.410 | 0.680 | 6.097 | 77.415 | Mature clay, High Illite |
| GEMSNRD7507 | 0.971 | 1.658 | 0.415 | 0.427 | 0.800 | 8.356 | 70.955 | Mature clay, High Illite |
| GEMSNRD7701 | 9.138 | 1.582 | 0.873 | 0.096 | 0.644 | 7.314 | 61.111 | Mature clay, High Illite |
| GEMSNRD7703 | 1.964 | 1.708 | 0.588 | 0.299 | 0.744 | 9.946 | 63.461 | Mature clay, High Illite |
| GEMSNRD7708 | 29.849 | 1.017 | 0.312 | 0.010 | 0.765 | 6.044 | 21.481 | Mature clay, High Illite |
| GEMSNRD9101 | 11.508 | 1.308 | 0.831 | 0.072 | 0.612 | 7.932 | 56.328 | Mature clay, High Illite |
| GEMSNRD9102 | 15.828 | 1.157 | 0.844 | 0.053 | 0.578 | 6.380 | 52.819 | Mature clay, High Illite |
| GEMSNRD9105 | 10.189 | 1.728 | 0.375 | 0.037 | 0.822 | 11.597 | 43.812 | Mature clay, High Illite |
| GEMSNRD9107 | 36.709 | 0.494 | 0.147 | 0.004 | 0.771 | 3.894 | 14.132 | Mature clay, High Illite |
| GEMSNRD9112 | 1.777 | 1.115 | 0.476 | 0.268 | 0.701 | 11.327 | 60.303 | Mature clay, High Illite |

Of the 49 immature clay samples, 18 (Table 9.14) samples have Na2O/Cao ratio >1.0 indicating that they are sodium bentonites; Remaining 31 samples are grouped as calcium bentonites.

| Table 9.26: 18 samples of immature clay with Na2O/Cao > 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample. Id | CaO (%) | K2O (%) | Na2O (%) | Na2O/ Cao (%) | K2O/ (K2O+ Na2O) (%) | Al2O3 (%) | SiO2 (%) | Clay type |
| GEMSNRD107 | 0.139 | 0.001 | 0.204 | 1.468 | 0.005 | 29.955 | 35.250 | immature clay high in plagioclase feldspar |
| GEMSNRD108 | 0.192 | 0.005 | 0.246 | 1.281 | 0.020 | 30.438 | 34.305 | immature clay high in plagioclase feldspar |
| GEMSNRD109 | 0.133 | 0.009 | 0.229 | 1.722 | 0.038 | 27.430 | 31.506 | immature clay high in plagioclase feldspar |
| GEMSNRD110 | 0.156 | 0.002 | 0.213 | 1.365 | 0.009 | 32.183 | 35.269 | immature clay high in plagioclase feldspar |
| GEMSNRD111 | 0.198 | 0.004 | 0.201 | 1.015 | 0.020 | 30.619 | 33.666 | immature clay high in plagioclase feldspar |
| GEMSNRD1309 | 0.261 | 0.196 | 0.464 | 1.778 | 0.297 | 37.586 | 42.507 | immature clay high in plagioclase feldspar |
| GEMSNRD1310 | 0.215 | 0.146 | 0.472 | 2.195 | 0.236 | 36.813 | 43.255 | immature clay high in plagioclase feldspar |
| GEMSNRD1313 | 0.137 | 0.003 | 0.286 | 2.088 | 0.010 | 36.246 | 42.501 | immature clay high in plagioclase feldspar |
| GEMSNRD1314 | 0.176 | 0.004 | 0.214 | 1.216 | 0.018 | 25.311 | 31.581 | immature clay high in plagioclase feldspar |
| GEMSNRD4511 | 0.160 | 0.002 | 0.229 | 1.431 | 0.009 | 33.478 | 38.390 | immature clay high in plagioclase feldspar |
| GEMSNRD4512 | 0.182 | 0.002 | 0.215 | 1.181 | 0.009 | 31.955 | 37.509 | immature clay high in plagioclase feldspar |
| GEMSNRD4513 | 0.190 | 0.005 | 0.261 | 1.374 | 0.019 | 32.706 | 40.310 | immature clay high in plagioclase feldspar |
| GEMSNRD4514 | 0.187 | 0.007 | 0.310 | 1.658 | 0.022 | 35.537 | 36.154 | immature clay high in plagioclase feldspar |
| GEMSNRD4515 | 0.421 | 0.022 | 0.427 | 1.014 | 0.049 | 30.901 | 39.607 | immature clay high in plagioclase feldspar |
| GEMSNRD5714 | 0.261 | 0.237 | 0.303 | 1.161 | 0.439 | 47.880 | 23.013 | immature clay high in plagioclase feldspar |
| GEMSNRD5719 | 0.167 | 0.013 | 0.319 | 1.910 | 0.039 | 38.201 | 41.593 | immature clay high in plagioclase feldspar |
| GEMSNRD5722 | 0.223 | 0.010 | 0.351 | 1.574 | 0.028 | 34.589 | 38.772 | immature clay high in plagioclase feldspar |
| GEMSNRD6710 | 0.235 | 0.024 | 0.302 | 1.285 | 0.074 | 37.547 | 42.019 | immature clay high in plagioclase feldspar |

Interestingly, the above exercise shows the distribution of mature and immature clay sedimentation in the block; all the 49 immature clay samples are from the boreholes on the eastern margin of the block (Fig 9.67). Any further observations on the significance of this kind of distribution of clay sediments in the basin on the depositional environments, like proximal and distil parts of the sedimentary basin is beyond the scope of this project.

Fig 9.78:  Map showing Immature clay zone

Further, following observations are made based on the chemical analysis of the clay samples.

- The (TREEO+SC2O3+Y2O3)>350 ppm enrichment is also in the samples from the boreholes with immature clay on the eastern side. (Fig 9.67).

- 90% of the samples with Ga enrichment above 30 ppm is also seen in the boreholes with immature clay on the eastern side. (Fig. 9.60).

- However, Sc distribution does not show any pattern.

The above attempt of characterization of clay sediments in the block are based on results of major oxides of only representative samples. Therefore, the observations are tentative and for a conclusive characterization of clay sediments in the block, a more detailed study may be necessary in the next stage of exploration. Such a study was beyond the scope of the present investigation.

10.0 GEOTECHNICAL STUDIES ON BOREHOLE CORE SAMPLES OF MINERSLISED ZONE, HANGING WALL AND FOOTWALL SIDE

Not carried out as it wasn’t a part of approved NQT

11.0 RESOURCE ESTIMATION

11.1 Introduction

The total area of Naredi block is 3.92 sq.km. As described in preceding chapters, 90% of the block is covered with soil, with very few outcrops of limestone and laterite/bauxite. The project was initially planned for exploration of Bauxite and associated critical elements. However, after detailed mapping and drilling of initial seven boreholes, the results indicated that, below the soil cover a thick deposition of clay sediments was encountered. Therefore, the project was re-oriented to exploration for clay hosted REE, Sc and Ga. Rest of the block was covered by additional 20 Boreholes on 400X400m grid spacing.

The topographic survey map and Digital Elevation Model created using the survey data reveals that relief is ranging from minimum of 101.31m to maximum of 128.82m. above msl. The minimum depth of bore hole is 14.51m (BH 35) and maximum depth of borehole is 35.0m (BH57). All the boreholes were encountered with ore zone i.e., clay with or without patches of other litho-domains, like Laterite and Limestone. The ICPOES/MS analytical results of 342 no. of samples drawn from all the 27 boreholes were used for resource estimation.

3D geological modelling and resource estimation was carried out using GEOVIA Surpac 2025 software. Sections were created by considering the borehole litholog and collar data. The solid model for each lithology was created and Final resource estimation was done by block modelling using Inverse Distance Weightage (IDW) method. The cut-off values of (TREEO+Sc2O3+Y2O3) and Ga, used for resource estimation were based on report of the committee constituted to deliberate upon and recommend threshold values for exploration of critical minerals. For Scandium, two level cut-off value were used for resource estimation:

- 50ppm cut-off used by Geological Survey of India

- 20ppm cut-off used by mining industry outside of India.

11.2 Detailed description of ore zones

All the bore holes encountered with clay horizon, which is the major lithology present in the study area. Apart from the clay, horizons of limestone and laterite were also encountered in the boreholes. Classification of lithology into three domains viz. CLAY, LIMESTONE and LATERITE were used for resource modelling.

- The litho-domain clay included all the clay horizons of different colors and lateritic /bauxitic clay.

- Similarly, Litho-domain Laterite included bauxite, aluminous laterite and ferruginous laterite.

11.3 Cut-off grade and minimum stopping width consideration

- For Resource Estimation of (TREEO+Sc2O3+Y2O3) the cut-off grade of above 350 ppm (0.035%) has been considered.

- For Ga, two cut of grades of 30 ppm (Industrial cut-off) and 50 ppm were used.

- Similarly for Sc 20ppm (Industrial cut-off) and 50ppm has been used.

Since the resource estimation is carried out in Surpac software with the user block dimension of 100x100x1m and sub block dimension as 25x25x.025m.

No additional stopping width is considered.

11.4 Description and correlation of lodes

All the boreholes are encountered with the clay horizon. The other two litho-domains were correlated based on the occurrence of adjacent boreholes as the case may be.

11.5 Preparation of Sections and Level Plans

The Resource estimation was done using GEOVIA Surpac software (3D modelling and resource estimation software). Nine sections (Fig. 11.1) are created in NW-SE direction. The boreholes falling in each section is given in below table.

Table 11.1: LV sections of Naredi Block

|  | Section Name | Boreholes | No. of BH |
| --- | --- | --- | --- |
| 1 | Section-1 | BH-01, 23, 45 & 67 | 4 |
| 2 | Section-2 | BH-13, 37, 55 | 3 |
| 3 | Section-3 | BH-03, 47, 69 | 3 |
| 4 | Section-4 | BH-05, 26 & 81 | 3 |
| 5 | Section-5 | BH-07, 28, 49, 71 | 4 |
| 6 | Section-6 | BH-09, 30, 53 & 75 | 4 |
| 7 | Section-7 | BH-51, 73 | 2 |
| 8 | Section-8 | BH-32, 91 | 2 |
| 9 | Section-9 | BH-55 & 77 | 2 |
|  |  |  | TOTAL = 27 |

Fig. 11.1 Map showing NW-SE sections

11.6 Specific gravity/bulk density calculation

A total of 12 samples representing clay horizon, one each sample from laterite and limestone were subjected to Bulk density determination at Matter Material Lab, Kozhikode. The density of 12 samples from clay horizon representing different type of clay ranged from 0.93g/cc to 1.59g/cc; hence an average of 1.22 g/cc is used for resource calculation.

Similarly, the bulk density of laterite and limestone is estimated as 1.69g/cc and 1.29g/cc respectively. The details of the bulk density samples are given below.

Table 11.2 Bulk density of Laterite

| Lithology Domain | Sample no. | Density (g/cc) | Average Density (g/cc) |
| --- | --- | --- | --- |
| Clay | 1 | 1.48 | 1.22 |
|  | 2 | 0.96 |  |
|  | 3 | 1.24 |  |
|  | 4 | 1.22 |  |
|  | 5 | 1.22 |  |
|  | 6 | 1.13 |  |
|  | 7 | 1.32 |  |
|  | 8 | 1.08 |  |
|  | 9 | 0.93 |  |
|  | 10 | 1.26 |  |
|  | 11 | 1.59 |  |
|  | 12 | 1.23 |  |
| Limestone | 1 | 1.29 | 1.29 |
| Laterite | 1 | 1.69 | 1.69 |

11.7 Assumption for resource estimation

11.7.1 Dimensions of block

3D block modelling and resource estimation is carried out using Surpac software 2025. During the resource estimation, the unit blocks are considered with dimension of 100mx100m size and 1m vertical thickness, since most of the borehole sampling was done at 400mx400m interval. The unit blocks are further divided in to sub blocks having dimension 25mx25mx0.25m as per standard practice, to get full coverage along the block boundary.

11.7.2 Topography

Topographic digital terrain model (DTM) was created using the elevation data collected during topographic survey. The reference for the elevation is mean sea level (msl). As per the topographic survey data, the lowest elevation within the block is 101.311 m and the highest elevation is 128.821 m with respect to msl. The DTM is considered as one of the constrain during modelling; the estimation was done only below the topographic DTM.

11.7.3 Buffer area/boundary area

The total area of Naredi block is 3.92 sq. km. All the 27 boreholes are spaced with in the boundary. Hence the resource estimation is restricted to 3.92 sq.km itself.

11.7.4 Solid modelling

All the boreholes are terminated in Clay horizon. Hence solid model of each lithology has been prepared viz. Clay, laterite and limestone. In the case of discontinuous horizons of lithologies between adjacent boreholes as well as adjacent sections, an influence of 50% borehole spacing has been considered for solid model creation. In both ends of each section, the solids were extrapolated till 200m (half of grid spacing distance) or till boundary, whichever is closer.

The thickness of orebody is considered till the termination of borehole.

11.7.5 Distribution of mineralization.

The distribution of mineralization viz, clay is assumed to be as continuous and linear, hence inverse distance weightage (IDW) method was adopted for interpolation.

11.8 Resource estimation methodology

In the initial stage of the modelling, the database created were Collar, Survey, Assay and Geology in software format. Later, the connectivity of similar lithologies in adjacent boreholes was established by manual digitization in section view. The nine NW-SE sections were drawn in such a way that the sections covered entire twenty-seven boreholes (section 11.5). Once digitization of all sections was completed, solid model was created for each litho-domains by connecting the respective domains in adjacent section. In both ends the solid were extrapolated till 200m (half of grid spacing distance) or till block boundary, whichever is closer.

During the block modelling, 1/4th of the horizontal grid spacing was used for one block dimension in X & Y direction, whereas vertical dimension is considered as 1.0m. Using the unit blocks having 100mX100mX1m dimension and sub blocks having 25mX25mX0.25m dimension the entire orebody model has been created. IDW method of interpolation has been adopted for resource estimation. The horizontal radius of influence was given as 1200m, 600m and 400m for first, second and third level of interpolation respectively, whereas vertical radius is given as 3.0m, 2.0m & 1.5m for first, second and third level respectively.

The statistical table for the Chemical assay as per the block modelling is given below.

Table 11.3: Statistics chemical assay used for 3D Modelling

| Litho-domain |  | TREE+Sc2O3+y2O3 | Sc | Ga | TREE | HREE | Y | Li |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ppm | ppm | ppm | ppm | ppm | ppm | ppm |
| Clay | Min. | 52.447 | 4.402 | 6.612 | 12.926 | 2.410 | 1.932 | 4.297 |
|  | Max. | 1867.511 | 89.685 | 68.101 | 1317.377 | 164.200 | 396.205 | 229.910 |
|  | Avg. | 244.295 | 18.361 | 20.305 | 155.753 | 19.366 | 27.065 | 37.829 |
| Laterite | Min. | 119.287 | 10.144 | 17.903 | 69.681 | 9.932 | 3.387 | 19.676 |
|  | Max. | 2633.305 | 69.948 | 70.882 | 1532.149 | 338.733 | 640.169 | 127.495 |
|  | Avg. | 282.734 | 28.096 | 47.386 | 169.499 | 27.946 | 33.128 | 72.738 |
| Limestone | Min. | 157.767 | 9.917 | 9.690 | 101.163 | 12.601 | 19.199 | 10.450 |
|  | Max. | 3503.126 | 61.975 | 44.155 | 2170.597 | 387.463 | 691.213 | 44.016 |
|  | Avg. | 831.827 | 23.639 | 19.995 | 495.763 | 90.042 | 171.440 | 24.036 |

Table 11.4: Statistics of lithologic domains of the total block model

| Lithology Domains | Dimension of sub block (m) | No. of blocks | Volume (cu. m) | BD (g/cc) | Tonnes (tonnes) |
| --- | --- | --- | --- | --- | --- |
| Clay | 25x25x0.25 | 404361 | 63181406.25 | 1.22 | 77081315.63 |
| Laterite | 25x25x0.25 | 8321 | 1300156.25 | 1.69 | 2197264.063 |
| Limestone | 25x25x0.25 | 7220 | 1128125 | 1.29 | 1455281.25 |
| Grand Total |  | 419902 | 65609687.5 |  | 80733860.94 |
|  |  | TOTAL |  |  | 80.73 MMT |

Table 11.5: Total Resource Estimation at (TREEO+ Sc2O3+Y2O3) above 350 ppm cutoff

| Lithology | Cut off | Volume (Cu. m) | BD | Tonnes | Avg. Grade (ppm) |
| --- | --- | --- | --- | --- | --- |
| Clay | above 350ppm (TREEO+ Sc2O3 Y2O3) | 3649843.75 | 1.22 | 4452809.375 | 639.7288 |
| Laterite |  | 107187.5 | 1.69 | 181146.875 | 1280.6437 |
| Limestone |  | 339531.25 | 1.29 | 437995.3125 | 2217.9942 |
| Grand Total |  |  |  | 5071951.563 | 798.91261 |
| Total resource of TREEO+ Sc2O3+Y2O3 is 5.07195 MMT at a weighted avg. grade of 798.912 ppm. |  |  |  |  |  |

Table 11.5a: Average grades of other commodities at (TREEO+ Sc2O3+Y2O3) resource shown in table 11.5.

| Lithology | Cut off | TREE | HREE | Sc | Ga | Li | Y |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Clay | above 350ppm (TREEO+ Sc2O3 Y2O3) | 407.1792 | 54.6983 | 42.3446 | 36.3565 | 72.0296 | 78.498 |
| Laterite |  | 719.1079 | 162.61 | 44.7402 | 41.0645 | 54.1485 | 294.9891 |
| Limestone |  | 1303.9219 | 250.77 | 45.2135 | 36.0509 | 32.2864 | 495.4087 |

Table 11.6: Total Resource Estimation of Sc at Cut-Off 20ppm & 50ppm (for whole block 3.92Sq.km)

| Scandium > 20 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 13667656.25 | 1.22 | 16674540.63 | 16.6745 | 36.5273 |
| Laterite | 940937.5 | 1.69 | 1590184.375 | 1.5902 | 32.5888 |
| Limestone | 492656.25 | 1.29 | 635526.562 | 0.6355 | 37.6435 |
| Total |  |  |  | 18.9002 |  |

| Scandium > 50 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 2469843.75 | 1.22 | 3013209.375 | 3.0132 | 60.1948 |
| Laterite | 74687.5 | 1.69 | 126221.875 | 0.1262 | 55.6245 |
| Limestone | 181875 | 1.29 | 234618.75 | 0.2346 | 54.4255 |
| Total |  |  |  | 3.374 |  |

Table 11.7: Total Resource Estimation of Ga at Cut-Off 30ppm & 50ppm (for whole block 3.92Sq.km)

| Gallium > 30 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 7496562.5 | 1.22 | 9145806 | 0.914806 | 38.813 |
| Laterite | 1222812.5 | 1.69 | 2066553 | 2.066553 | 48.72 |
| Limestone | 264375 | 1.29 | 341043.8 | 0.0340438 | 39.823 |
| Total |  |  |  | 3.0154028 |  |

| Gallium> 50 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 713125 | 1.22 | 870012.5 | 0.8700125 | 56.359 |
| Laterite | 395937.5 | 1.69 | 669134.4 | 0.6691344 | 56.77 |
| Limestone | 0 | 1.29 | 0 | 0 | NA |
| Total |  |  |  | 1.5391469 |  |

(Z axis=10x exaggerated)

Litho-domains

Litho-domains

(Z axis=10x exaggerated)

Fig.11.2 The block model in Surpac with litho-domain categories

Fig.11.3: 3D view of the block model in Surpac with Digital Terrain Model

Litho-domains

Litho-domains

Fig.11.4: Top View of Block model in Surpac software.

Implications of Reduced Core Recovery on Resource Estimation

As the core recovery was about 93% in the ore zone, to account for the core loss vis-à-vis volume loss and any other geological uncertainties, a reduction factor of 10% has been considered. The resultant Gross and Net Resource estimates are given in the table below.

Table 11.8: Gross and Net resource estimation (for whole block 3.92Sq.km)

|  |  | Gross Resource | Net Resource |
| --- | --- | --- | --- |
|  |  |  | By considering 10% reduction factor |
| Lithology | Cut off | MMT | MMT |
| Clay | above 350ppm (TREEO+ Sc2O3 Y2O3) | 4.4528 | 4.0075 |
| Laterite |  | 0.1811 | 0.1630 |
| Limestone |  | 0.4380 | 0.3942 |
| Grand Total |  | 5.0720 | 4.5648 |

In addition to the Block Modelling of the ore body, 6 slice plans of 5.0m each below the surface DEM were created to visualize the distribution of different Resource / Litho-Domains, (TREEO+Sc2O3+Y2O3), Sc, Ga.

This exercise reveals that the accumulation of TREEO+Sc2O3+Y2O3, Sc and Ga in various resource domains increases with depth. Maximum concentration is accumulated at 15.0m to 20.0m below ground level. The 6 slice plans are given below.

Fig.11.5: Sliced view at Surface DTM

Fig.11.6: Sliced view below 5.0m Surface DTM

Fig.11.7: Sliced view below 10.0m Surface DTM

Fig.11.8: Sliced view below 15.0m Surface DTM

Fig.11.9: Sliced view below 20.0m Surface DTM

Fig.11.10: Sliced view below 20.0m Surface DTM

11.9 Category of resources as per MEMC 2015 along with UNFC Classification

The resource estimation was carried out based on the ICPOES/MS analysis for 342 samples, which represent the entire orebody in horizontal and vertical extend. Surpac 2025, software is used for the resource estimation. The work carried out is as per UNFC classification under G-3 stage of geological axis and the resource comes under category 333 (For TREEO+Sc2O3+Y2O3, Ga and Sc).

11.10  Resource estimation in the delineated zone of high potential

As discussed in 9.6.2 the zone delineated as high potential zone (Fig 11.11 shown below) was used to carve out as a separate resource block from the whole Naredi resource block. And the resource for this block was estimated separately.

Fig 11.11  Map of  delineated zone (TREEO+SC2O3+Y2O3)>350 ppm

| Table 11.9 Total Estimated TREEO+ Sc2O3+Y2O3 resource in the block delineated zone recommended for auction block |  |  |
| --- | --- | --- |
| Lithology | TREEO+ Sc2O3+Y2O3 | Tonnes (MMT) |
| Total resource Estimated in the Naredi block | > 350 ppm | 5.07 |
| Laterite | > 350 ppm | 0.18 |
| Clay | > 350 ppm | 4.22 |
| limestone | > 350 ppm | 0.34 |
| Total resource in the demarcated block |  | 4.74 |
| The demarcated zone accounts for 93.52% of the total estimated resource. |  |  |

As can be seen from the above table, the delineated block has an estimated (TREEO+ Sc2O3+Y2O3)> 350 ppm resource of 4.74 MMT out of a total resource of 5.07 MMT. This accounts for 93.52% of the total estimated resource.

Details of resource estimation for delineated block are given in below table.

| Table 11.10 Estimated Resource of Scandium > 20 ppm in Delineated zone (0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 9667656.25 | 1.22 | 11794540.63 | 11.795 | 40.8819 | 39.84932821 |
| Laterite | 939843.75 | 1.69 | 1588335.938 | 1.588 | 32.594 |  |
| Limestone | 432031.25 | 1.29 | 557320.312 | 0.557 | 38.6744 |  |
| Total |  |  |  | 13.94 |  |  |

| Table 11.11 Estimated Resource of Scandium > 50 ppm in Delineated zone (0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 2405625 | 1.22 | 2934862.5 | 2.935 | 60.2331 | 59.6662740147007 |
| Laterite | 74687.5 | 1.69 | 126221.875 | 0.126 | 55.6245 |  |
| Limestone | 175000 | 1.29 | 225750 | 0.226 | 54.5571 |  |
| Total |  |  |  | 3.287 |  |  |

| Table 11.12 Estimated Resource of Gallium > 30 ppm in Delineated zone(0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 6808906.25 | 1.22 | 8306865.625 | 8.307 | 39.2304 | 41.0880128 |
| Laterite | 1221718.75 | 1.69 | 2064704.688 | 2.065 | 48.7225 |  |
| Limestone | 249687.5 | 1.29 | 322096.875 | 0.322 | 40.0572 |  |
| Total |  |  |  | 10.694 |  |  |

| Table 11.13 Estimated Resource of Gallium > 50 ppm in Delineated zone(0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 707656.25 | 1.22 | 863340.625 | 0.863 | 56.3721 | 56.54561931 |
| Laterite | 395937.5 | 1.69 | 669134.375 | 0.669 | 56.7695 |  |
| Limestone | 0 | 1.29 | 0 | 0 | NA |  |
| Total |  |  |  | 1.532 |  |  |

12.0 CORE PRESERVATION

The extracted drilled cores were placed in galvanized aluminium core boxes. The cores were arranged in book pattern and directions were marked; by putting an arrow mark from top left to bottom right; indicating the top and bottom directions of the core. Run separators (Gutkhas) were placed in between the two runs by marking the end meterage on them. Whenever there was no recovery then two gutkhas were placed marking the run meterage. The run number, from & to meterage and the recovery percentage is recorded in the form of a table along with the borehole number and box number on the inside part of the lid. Complete title of the project along with toposheet number, borehole number and box number is written on the top surface of the core boxes. The sides of the core boxes are marked with the borehole number and box number. Before the core was taken out for sampling, each core box has been photographed, borehole wise and box wise photograph is arranged and is given in annexure no.09.  After core splitting and sampling, the remaining portion of the cores were, returned to the core box and returned to their original position in each core box.

13.0 Conclusion and Recommendation

Naredi block which is situated towards SW of Bhuj city has an area of 3.92Sq.Km. initially, the present investigation was focussed to assess bauxite, vanadium, gallium and other associated elements in the block area on a G3 level, based on earlier works of CGM, Gujarat in the region.

As per the approved NQT, detailed DGPS based topographical survey and geological mapping were carried out and the respective maps were prepared on 1:4000 scale. However, the detailed geological mapping revealed that >90% of the block was covered with soil, with very small outcrops of laterite/bauxite & limestone in the north central, north eastern & eastern part of the block in the form of pockets.

A total of 91 boreholes of 10m depth each,  for 910m were initially planned in the Naredi Block on a grid spacing of 200 m × 200 m (with first phase drilling at 400mX400m). First phase of drilling was taken up from the eastern margin of the block. First 06 boreholes were drilled on a 200mX400m (E-W grid) because several bauxite GMDC quarries were situated just outside the eastern margin of the block.

Initially the project in Naredi was aimed at exploring for Bauxite and critical elements. After detailed geological mapping and initially drilling of 7 boreholes, it was established that the lithology in the block was dominated by clay sediments and small pockets of limestone and bauxite/laterite. So, this necessitated for re-orientation of the project, with approval from TCC, to explore for clay hosted critical elements and REE.

Accordingly, the remaining part of the block was explored by drilling 20 boreholes on 400mX400m spacing.

168 Samples from all the 27 boreholes (23 laterite, 7 limestones and 136 clay& or lateritic clay samples and 02 soil samples) were analyzed for major oxides and LOI by XRF. For analysis of minor, trace and REE by ICP OES/MS, 342 Samples from 27 boreholes(23 no. of laterite, 09 limestones and 307 clay& or lateritic clay) were analyzed.

The obtained drill core lithologies were broadly classified as Soil, Limestone, Laterite/bauxite, Lateritic Clay, Clay. For the purpose of borehole logging and sampling.

For the interpretation of analysis results of the geochemical samples, the cut-off values of (TREEO+Sc2O3+Y2O3) and Ga, were based on report of the committee constituted to deliberate upon and recommend threshold values for exploration of critical minerals. For Scandium, two level cut-off value were used for resource estimation:

1.	50ppm cut-off used by Geological Survey of India

2.	20ppm cut-off used by mining industry outside of India.

Based on the exploration Threshold values TREEO+SC2O3+Y2O3 was more than 350 ppm in 11 out 27 boreholes. Ga enrichment above 30 ppm This REE & Ga enrichment is seen in the immature clays that are seen in the eastern side of the block. Sc distribution does not show any pattern although.

As the project is re oriented to clay hosted REE, Sc & Ga the resource was estimated for the same using Geovia Surpac software. For Resource Estimation of (TREEO+Sc2O3+Y2O3) the cut-off grade of above 350 ppm (0.035%) has been considered.	For Ga, two cut of grades of 30 ppm (Industrial cut-off) and 50 ppm (used by GSI) were used. Similarly, for Sc 20ppm (Industrial cut-off) and 50ppm (used by GSI) have been used.

The estimated resources are as stated in the following tables

- The resource estimation table for (TREEO+Sc2O3+Y2O3)>350ppm for whole block(3.92 Sq.km).

| Table 13.1 The resource estimation table for (TREEO+Sc2O3+Y2O3)>350ppm. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Cut off | Volume (Cu.m) | BD | Tonnes | Avg Grade (ppm) |
| Clay | above 350ppm (TREEO+ Sc2O3 Y2O3) | 3649843.75 | 1.22 | 4452809.375 | 639.7288 |
| Laterite |  | 107187.5 | 1.69 | 181146.875 | 1280.6437 |
| Limestone |  | 339531.25 | 1.29 | 437995.3125 | 2217.9942 |
| Grand Total |  |  |  | 5071951.563 | 798.91261 |
| Total resource of TREEO+ Sc2O3+Y2O3 is 5.07195 MMT at a weighted avg. grade of 798.912ppm. |  |  |  |  |  |

- The resource estimation table for (TREEO+Sc2O3+Y2O3)>350ppm with 10% reduction factor for whole block(3.92 Sq.km)..

| Table 13.2 The resource estimation table for (TREEO+Sc2O3+Y2O3)>350ppm with reduction factor 10%. |  |  |  |
| --- | --- | --- | --- |
|  |  | Gross Resource | Net Resource |
|  |  |  | By considering 10% reduction factor |
| Lithology | Cut off | MMT | MMT |
| Clay | above 350ppm (TREEO+ Sc2O3 Y2O3) | 4.4528 | 4.0075 |
| Laterite |  | 0.1811 | 0.1630 |
| Limestone |  | 0.4380 | 0.3942 |
| Grand Total |  | 5.0720 | 4.5648 |

- Total Resource Estimation of Sc at Cut-Off 20ppm & 50ppm (For whole block 3.92Sq.km).

| Table 13.3 Total Resource Estimation of Scandium > 20 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume (Cu.m) | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 13667656.25 | 1.22 | 16674540.63 | 16.6745 | 36.5273 |
| Laterite | 940937.5 | 1.69 | 1590184.375 | 1.5902 | 32.5888 |
| Limestone | 492656.25 | 1.29 | 635526.562 | 0.6355 | 37.6435 |
|  |  |  |  | 18.9002 |  |

| Table 13.4 Total Resource Estimation of Scandium > 50 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume (Cu.m) | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 2469843.75 | 1.22 | 3013209.375 | 3.0132 | 60.1948 |
| Laterite | 74687.5 | 1.69 | 126221.875 | 0.1262 | 55.6245 |
| Limestone | 181875 | 1.29 | 234618.75 | 0.2346 | 54.4255 |
|  |  |  |  | 3.374 |  |

- Total Resource Estimation of Gallium Cut-Off 30ppm & 50ppm (For whole block 3.92Sq.km).

| Table 13.5 Total Resource Estimation of Gallium > 30 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume (Cu.m) | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 7496562.5 | 1.22 | 9145806 | 0.914806 | 38.813 |  |
| Laterite | 1222812.5 | 1.69 | 2066553 | 2.066553 | 48.72 |  |
| Limestone | 264375 | 1.29 | 341043.8 | 0.0340438 | 39.823 |  |
|  |  |  |  | 3.0154028 |  |  |

| Table 13.6 Total Resource Estimation of Gallium> 50 ppm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lithology | Volume (Cu.m) | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) |
| Clay | 713125 | 1.22 | 870012.5 | 0.8700125 | 56.359 |  |
| Laterite | 395937.5 | 1.69 | 669134.4 | 0.6691344 | 56.77 |  |
| Limestone | 0 | 1.29 | 0 | 0 | NA |  |
|  |  |  |  | 1.5391469 |  |  |

Recommendation:

The present block at Naredi holds no potential for Bauxite, Laterite and Limestone and no further exploration may be required for the said commodities in future. However, the present blocks holds a very good potential for clay hosted REE, Ga & Sc deposits.

Based on the analytical results a main zone of 0.89sq.km with REE enriched clay has been delineated as shown in the figure below.

Fig 13.1 Map showing 0.89 sq.km (TREEO+Sc2O3+Y2O3) >350ppm rich zone

The demarcated zone is characterized by the following:

- The zone is dominated by immature clay sediments, as characterized by (K2O/K2O+Na2O). (Fig 13.1)

- Samples with (TREEO+Sc2O3+Y2O3) >350ppm, representing 34.1m thick clays (out of 41.1m thick clay sediments) are from this zone.

- 28 samples of 36 m from seven boreholes within this zone have analyzed Sc>50ppm. (Fig 13.2)

- Similarly, 19 samples of 19 m from five boreholes within this zone have analyzed Ga >50ppm. (Fig 13.3)

Therefore, it is recommended that the above demarcated zone may be carved out as an auction block for clay hosted REE+Sc+Ga.

Fig 13.2 Map showing scandium>50 ppm

Fig 13.3 Map showing Gallium>50 ppm

Details of the estimated resource for the delineated zone is given in the tables below.

| Table 13.7 Estimated Resource of Scandium > 20 ppm in Delineated zone(0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume (Cu.m) | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 9667656.25 | 1.22 | 11794540.63 | 11.795 | 40.8819 | 39.84932821 |
| Laterite | 939843.75 | 1.69 | 1588335.938 | 1.588 | 32.594 |  |
| Limestone | 432031.25 | 1.29 | 557320.312 | 0.557 | 38.6744 |  |
| Total |  |  |  | 13.94 |  |  |

| Table 13.8 Estimated Resource of Scandium > 50 ppm in Delineated zone(0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume (Cu.m) | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 2405625 | 1.22 | 2934862.5 | 2.935 | 60.2331 | 59.6662740147007 |
| Laterite | 74687.5 | 1.69 | 126221.875 | 0.126 | 55.6245 |  |
| Limestone | 175000 | 1.29 | 225750 | 0.226 | 54.5571 |  |
| Total |  |  |  | 3.287 |  |  |

| Table 13.9 Estimated Resource of Gallium > 30 ppm in Delineated zone(0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume (Cu.m) | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 6808906.25 | 1.22 | 8306865.625 | 8.307 | 39.2304 | 41.0880128 |
| Laterite | 1221718.75 | 1.69 | 2064704.688 | 2.065 | 48.7225 |  |
| Limestone | 249687.5 | 1.29 | 322096.875 | 0.322 | 40.0572 |  |
| Total |  |  |  | 10.694 |  |  |

| Table 13.10 Estimated Resource of Gallium > 50 ppm in Delineated zone(0.89Sq.km) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lithology | Volume (Cu.m) | BD g/cc | Tonnes | MMT | Avg. Sc (ppm) | Cut-off Grade |
| Clay | 707656.25 | 1.22 | 863340.625 | 0.863 | 56.3721 | 56.54561931 |
| Laterite | 395937.5 | 1.69 | 669134.375 | 0.669 | 56.7695 |  |
| Limestone | 0 | 1.29 | 0 | 0 | NA |  |
| Total |  |  |  | 1.532 |  |  |

14.0 References

- Geological Survey of India (GSI), 2012. Geology and Mineral Resources of Gujarat, Daman & Diu, Misc. Publication No. 30, Part 14.

- Kachchh District Administration. Geology and Mineral Resources of Kachchh, kachchh.nic.in.

- Subramanyam, C., 2017. Stratigraphy and Sedimentary Evolution of Kachchh Basin, ScienceDirect.

- G.R. KULKARNI, GSI, 1965, GEOLOGY OF PARTS OF ABDASA, NAKHATRANA AND MANDVI TALUKS, KUTCH DISTRICT, GUJARAT STATE.

- C.Srikarni, P.Gandhi, 2016, A Final report on Geochemical Mapping of Toposheet 41E3 & 41E/04, Kachchh District, Gujarat.

- S.K. Biswas(1992) ,Tertiary stratigraphy of kutch.

- G. Krishnamurthy (2022), Aquifer map and management plan, Kachchh district, Gujarat, Central ground water board.

- U.D.G Rao(1965) GSI, Report on the Bauxite deposits of Kotda, Jarjok, Naredi, Nanda and Balachod(N) villages, Kutch district, Gujarat.

15.0       LOCALITY INDEX

| Place name | LATITUDE | LONGITUDE |
| --- | --- | --- |
| Sanosara | 23.21682663 | 69.2240086 |
| Mothara | 23.21164861 | 69.1277784 |
| Nandhra mota | 23.19473728 | 69.1991096 |
| Kanakpar | 23.18685058 | 69.0890349 |
| Kotda roha | 23.14819193 | 69.2406566 |