REPORT ON

PRELIMINARY EXPLORATION (G3-STAGE) FOR BAUXITE, GALLIUM, VANADIUM, REE IN NAREDI BLOCK, KACHCHH DISTRICT, GUJARAT.

By

Geo Marine Solutions Pvt Ltd

15-17-909/9, 5th Cross Shivabagh,

Kadri, Mangalore-575005, Karnataka, INDIA

www.geomarine.in

For

National Mineral Exploration and Development Trust (NMEDT) Ministry of Mines

Wing-F, Room No. 325 & 326, Udyog Bhawan,

Rafi Ahmed Kidwai Marg, Rajpath Area, Central Secretariat

New Delhi-110011

Ref. No: 23/566/2025-NMET/846 dated 10th Feb 2025

MARCH-2026

| Sl. No | CONTENTS | Page No. |
| --- | --- | --- |
| 1.0 | EXECUTIVE SUMMARY | 1 |
| 2.0 | Introduction | 16 |
| 2.1 | Details of the Project | 17 |
| 2.2 | Investigating Agency | 17 |
| 2.3 | Objectives of the present investigation | 18 |
| 2.4 | Basis for taking up investigation | 18 |
| 2.5 | The Nature and Quantum of Work Proposed and Carried Out | 19 |
| 2.6 | Technical personnel involved in the project | 20 |
| 2.7 | Mode of operation of different work components and associated agencies | 21 |
| 3.0 | PROPERTY DESCRIPTION | 24 |
| 3.1 | Details of the area (Village name, District, State) | 24 |
| 3.2 | Survey of India toposheet No. | 24 |
| 3.3 | Location and Accessibility | 26 |
| 3.4 | Cadastral details, land use/ cover, forest with type of forest, free hold/lease hold details | 27 |
| 3.5 | Climate | 29 |
| 3.6 | Flora/Fauna | 29 |
| 3.7 | Regional Geomorphology | 29 |
| 3.8 | Local Infrastructure | 32 |
| 3.9 | Population | 32 |
| 3.10 | Archaeological & Historical sites | 33 |
| 3.11 | National parks and Environmental settings of the area | 33 |
| 4.0 | PREVIOUS WORK | 34 |
| 4.1 | Details of previous exploration/ investigation carried out by other agencies/ parties | 34 |
| 4.2 | A very brief note on previous work on Geology, Geophysics, Geochemistry | 34 |
| 5.0 | GEOLOGY OF THE AREA | 37 |
| 5.1 | Aerial Reconnaissance | 37 |
| 5.2 | Regional geological set up of the area with stratigraphy | 37 |
| 5.3 | Geology of the Naredi area | 39 |
| 6.0 | GEOSCIENCE INVESTIGATION | 40 |
| 6.1 | Geological Mapping and Topographic Survey | 40 |
| 6.2 | Description of Rock Types | 43 |
| 6.3 | Petrographic Studies | 45 |
| 6.4 | Structure and Metamorphism | 45 |
| 6.5 | Pitting and Trenching | 45 |
| 6.6 | Geophysical Exploration | 45 |
| 6.7 | Geochemical Exploration | 45 |
| 7.0 | Integration of Geology, Geophysics and Geochemical Exploration Data and interpretation | 46 |
| 8.0 | MINERAL PROSPECT | 47 |
| 8.1 | Surface indication | 47 |
| 8.2 | Mode of occurrence and strike length of the body | 47 |
| 8.3 | Alteration Zones | 47 |
| 8.4 | Genesis of Mineralized Zones | 47 |
| 9.0 | Exploration systematic Drilling | 48 |
| 9.1 | Spacing of Boreholes | 48 |
| 9.2 | Methodology of Drilling and types of Drilling | 50 |
| 9.3 | Borehole Planning | 52 |
| 9.3.1 | Borehole logging | 53 |
| 9.3.2 | Core Recovery | 54 |
| 9.4 | Sample Preparation | 54 |
| 9.5 | Chemical analysis and laboratory Procedure | 55 |
| 9.6 | Results and Discussion | 55 |
| 9.6.1 | Descriptive Statistics Analysis | 55 |
| 9.6.2 | Discussion | 115 |
| 10.0 | GEOTECHNICAL STUDIES ON BOREHOLE CORE SAMPLES OF MINERSLISED ZONE, HANGING WALL AND FOOTWALL SIDE | 137 |
| 11.0 | RESOURCE ESTIMATION | 138 |
| 11.1 | Introduction | 138 |
| 11.2 | Detailed description of ore zones | 138 |
| 11.3 | Cut-off grade and minimum stopping width consideration | 139 |
| 11.4 | Description and correlation of lodes | 139 |
| 11.5 | Preparation of Sections and Level Plans | 139 |
| 11.6 | Specific gravity/bulk density calculation | 140 |
| 11.7 | Assumption for resource estimation | 141 |
| 11.7.1 | Dimensions of block | 141 |
| 11.7.2 | Topography | 141 |
| 11.7.3 | Buffer area/boundary area | 141 |
| 11.7.4 | Solid modelling | 142 |
| 11.7.5 | Distribution of mineralization. | 142 |
| 11.8 | Resource estimation methodology | 142 |
| 11.9 | Category of resources as per MEMC 2015 along with UNFC Classification | 156 |
| 11.10 | Resource estimation in the delineated zone of high potential | 157 |
| 12.0 | CORE PRESERVATION | 160 |
| 13.0 | Conclusion and Recommendation | 161 |
| 14.0 | Reference | 170 |
| 15.0 | Locality Index | 171 |

List of Figures

| Fig. No. | Title | Page No. |
| --- | --- | --- |
| 1.1 | Map of delineated zone (TREEO+SC2O3+Y2O3)>350ppm | 5 |
| 3.1 | Block area on Toposheet no 41E/04 | 25 |
| 3.2 | Block area on Google earth imagery | 25 |
| 3.3 | Accessibility map of Naredi Block | 26 |
| 3.4 | Geomorphological map of Kachchh | 31 |
| 3.5 | Drainage system map of Kachchh | 32 |
| 6.1 | Topographic Map prepared by Geo Marine Solutions Pvt Ltd | 41 |
| 6.2 | Detailed Geological Map prepared by Geo Marine Solutions Pvt Ltd | 41 |
| 6.4 (a) | Photo of surveyor taking reading and geologist recording field data. | 42 |
| 6.4 (b) | Photograph of setting TBM using DGPS instrument. | 42 |
| 6.4 (c) | Photo of surveyor taking reading | 42 |
| 6.5 (A) | Laterite Exposure | 43 |
| 6.5 (B) | High grade bauxite observed near NE part of the block in GMDC pit 150m distance from the block. | 44 |
| 6.5 (C) | laterite/bauxite sample. | 44 |
| 6.6 | Limestone Exposure | 44 |
| 6.7 | cross section lithology visible on the south eastern part of the block | 45 |
| 9.1 | Map showing 91 proposed boreholes at 200 X 200m grid | 48 |
| 9.2 | Map showing planned Boreholes at 400 x 400 m grid for phase-I | 49 |
| 9.3 | Map showing 27 drilled at 400 m X 400 m spacing | 50 |
| 9.4 | Vertical distribution pattern of major oxides in BH-01 | 59 |
| 9.5 | Vertical distribution pattern of major oxides in BH-23 | 59 |
| 9.6 | Vertical distribution pattern of major oxides in BH-35 | 60 |
| 9.7 | Vertical distribution pattern of major oxides in BH-45 | 60 |
| 9.8 | Vertical distribution pattern of major oxides in BH-57. | 61 |
| 9.9 | Vertical distribution pattern of major oxides in BH-13 | 61 |
| 9.10 | Vertical distribution pattern of major oxides in BH-67 | 62 |
| 9.11 | Vertical distribution pattern of major oxides in BH-69. | 62 |
| 9.12 | Vertical distribution pattern of major oxides in BH-71 | 62 |
| 9.13 | Vertical distribution pattern of major oxides in BH-47 | 62 |
| 9.14 | Vertical distribution pattern of major oxides in BH-09 | 63 |
| 9.15 | Vertical distribution pattern of major oxides in BH-26 | 63 |
| 9.16 | Vertical distribution pattern of major oxides in BH-28 | 63 |
| 9.17 | Vertical distribution pattern of major oxides in BH-81 | 64 |
| 9.18 | Vertical distribution pattern of major oxides in BH-75 | 64 |
| 9.19 | Vertical distribution pattern of major oxides in BH-53 | 64 |
| 9.20 | Vertical distribution pattern of major oxides in BH-77 | 64 |
| 9.21 | Vertical distribution pattern of major oxides in BH-55 | 65 |
| 9.22 | Vertical distribution pattern of major oxides in BH-51 | 65 |
| 9.23 | Vertical distribution pattern of major oxides in BH-91 | 65 |
| 9.24 | Vertical distribution pattern of major oxides in BH-07 | 66 |
| 9.25 | Vertical distribution pattern of major oxides in BH-03 | 66 |
| 9.26 | Vertical distribution pattern of major oxides in BH-49 | 66 |
| 9.27 | Vertical distribution pattern of major oxides in BH-30 | 66 |
| 9.28 | Vertical distribution pattern of major oxides in BH-05 | 67 |
| 9.29 | Vertical distribution pattern of major oxides in BH-73 | 67 |
| 9.30 | Vertical distribution pattern of major oxides in BH-32 | 67 |
| 9.31 | ScatterPlotofPrimaryVsChecksamplesforAl2O3(Values in%) | 69 |
| 9.32 | ScatterPlotofPrimaryVsChecksamplesforFe2O3(Values in%) | 70 |
| 9.33 | ScatterPlotofPrimaryVsChecksamplesforSiO2(Values in%) | 71 |
| 9.34 | ScatterPlotofPrimaryVsChecksamplesforNa2O(Values in%). | 72 |
| 9.35 | ScatterPlotofPrimaryVsChecksamplesforK2O(Values in%). | 73 |
| 9.36 | Scatter Plot of Primary Vs Check samples for CaO(Values in%). | 74 |
| 9.37 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-01 | 76 |
| 9.38 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-03 | 77 |
| 9.39 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-05 | 78 |
| 9.40 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-07 | 79 |
| 9.41 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-09 | 80 |
| 9.42 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-13 | 81 |
| 9.43 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-23 | 82 |
| 9.44 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-26 | 83 |
| 9.45 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-28 | 84 |
| 9.46 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-30 | 85 |
| 9.47 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-32 | 86 |
| 9.48 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-35 | 87 |
| 9.49 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-45 | 88 |
| 9.50 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-47 | 89 |
| 9.51 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-49 | 90 |
| 9.52 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-51 | 91 |
| 9.53 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-53 | 92 |
| 9.54 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-55 | 93 |
| 9.55 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-57 | 95 |
| 9.56 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-67 | 96 |
| 9.57 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-69 | 97 |
| 9.58 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-71 | 98 |
| 9.59 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-73 | 99 |
| 9.60 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-75 | 100 |
| 9.61 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-77 | 101 |
| 9.62 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-81 | 102 |
| 9.63 | Vertical distribution pattern of TREEO+Sc2​ O3​ +Y2​ O3 in BH-91 | 103 |
| 9.64 | Scatter Plot of Primary Vs Check samples for TREE (Values in ppm) | 106 |
| 9.65 | Scatter Plot of Primary Vs Check samples for TREE+ Sc+ Y (Values in ppm) | 108 |
| 9.66 | Scatter Plot of Primary Vs Check samples for Scandium (Values in ppm) | 110 |
| 9.67 | Scatter Plot of Primary Vs Check samples for Yttrium (Values in ppm) | 112 |
| 9.68 | Scatter Plot of Primary Vs Check samples for Gallium (Values in ppm) | 114 |
| 9.69 | Map of (TREEO+SC2O3+Y2O3)>350 ppm | 116 |
| 9.70 | Map of Boreholes with Scandium values >20 ppm and 50 ppm | 118 |
| 9.71 | Map of Boreholes with Gallium values >30 ppm and 50 ppm | 119 |
| 9.72 | XRDgraphofsampleno01 | 121 |
| 9.73 | XRDgraphofsampleno02 | 122 |
| 9.74 | XRDgraphofsampleno03 | 123 |
| 9.75 | XRDgraphofsampleno04 | 124 |
| 9.76 | XRDgraphofsampleno05 | 125 |
| 9.77 | XRDgraphofsampleno06 | 126 |
| 9.78 | Map showing Immature clay zone | 136 |
| 11.1 | Map showing NW-SE sections | 140 |
| 11.2 | The block model in Surpac with litho-domain categories | 147 |
| 11.3 | 3D view of the block model in Surpac with Digital Terrain Model | 147 |
| 11.4 | Top View of Block model in Surpac software | 148 |
| 11.5 | Sliced view at Surface DTM | 150 |
| 11.6 | Sliced view below 5.0m Surface DTM | 151 |
| 11.7 | Sliced view below 10.0m Surface DTM | 152 |
| 11.8 | Sliced view below 15.0m Surface DTM | 153 |
| 11.9 | Sliced view below 20.0m Surface DTM | 154 |
| 11.10 | Sliced view below 20.0m Surface DTM | 155 |
| 11.11 | Map of delineated zone(TREEO+SC2O3+Y2O3)>350ppm | 157 |
| 13.1 | Map showing 0.86 sq.km (TREEO+Sc2O3+Y2O3) >350ppm rich zone | 165 |
| 13.2 | Map showing scandium>50 ppm | 166 |
| 13.3 | Map showing Gallium>50 ppm | 167 |

List of Tables

| Sl. No | Title | Page No. |
| --- | --- | --- |
| 1.1 | The resource estimation table for (TREEO+Sc2O3+Y2O3)>350ppm. | 3 |
| 1.2 | Total Resource Estimation of Scandium>20ppm | 3 |
| 1.3 | Total Resource Estimation of Scandium>50ppm | 4 |
| 1.4 | Total Resource Estimation of Gallium>30ppm | 4 |
| 1.5 | Total Resource Estimation of Gallium>50ppm | 5 |
| 1.6 | Total Estimated TREEO+Sc2O3+Y2O3resource in the block Delineated zone recommended for auction block | 6 |
| 1.7 | Estimated Resource of Scandium >20ppminDelineatedzone(0.89Sq.km) | 6 |
| 1.8 | Estimated Resource of Scandium>50ppminDelineated zone(0.89Sq.km) | 7 |
| 1.9 | Estimated Resource of Gallium>30ppminDelineated zone(0.89Sq.km) | 7 |
| 1.10 | Estimated Resource of Gallium > 50 ppm in Delineated zone(0.89Sq.km) | 8 |
| 2.1 | Nature and Quantum of work proposed and carried out | 19 |
| 2.2 | Details of Technical persons engaged in exploration | 20 |
| 3.1 | Boundary coordinates of Naredi block, Gujarat | 26 |
| 3.2 | Details of land ownership inside the Naredi block | 27 |
| 5.1 | Stratigraphy of Tertiary Group of rocks | 37 |
| 9.1 | Borehole Co-ordinates and Collar details | 52 |
| 9.2 | Descriptive statistics of Laterite | 56 |
| 9.3 | Descriptive statistics of limestone | 57 |
| 9.4 | Descriptive statistics of Clay | 58 |
| 9.5 | PrimaryandCheckSamplevaluesforAl2O3(in%) | 69 |
| 9.6 | PrimaryandCheckSamplevaluesforFe2O3(in%) | 70 |
| 9.7 | PrimaryandCheckSamplevaluesforSio2(in%) | 71 |
| 9.8 | PrimaryandCheckSamplevaluesforNa2O(in%) | 72 |
| 9.9 | PrimaryandCheckSamplevaluesforK2O(in%) | 73 |
| 9.10 | Primary and Check Sample values for CaO(in%) | 74 |
| 9.11 | Descriptive statistics of Minor &Trace | 75 |
| 9.12 | Primary and Check Sample values for TREE | 105 |
| 9.13 | Primary and Check Sample values for TREE +Sc +Y | 107 |
| 9.14 | Primary and Check Sample values for Scandium | 109 |
| 9.15 | Primary and Check Sample values for Yttrium | 111 |
| 9.16 | Primary and Check Sample values for Gallium | 113 |
| 9.17 | Borehole numbers with (TREEO+SC2O3+Y2O3)>350 ppm. | 117 |
| 9.18 | Physical, Chemical and XRD characteristics of sample no 01 | 121 |
| 9.19 | Physical, Chemical and XRD characteristics of sample no 02 | 122 |
| 9.20 | Physical, Chemical and XRD characteristics of sample no 03 | 123 |
| 9.21 | Physical, Chemical and XRD characteristics of sample no 04 | 124 |
| 9.22 | Physical, Chemical and XRD characteristics of sample no 05 | 125 |
| 9.23 | Physical, Chemical and XRD characteristics of sample no 06 | 126 |
| 9.24 | 49 samples of immature clay | 127 |
| 9.25 | 89 samples of Mature clay. | 130 |
| 9.26 | 18 samples of immature clay with Na2O/Cao > 1 | 134 |
| 11.1 | LV sections of Naredi Block | 139 |
| 11.2 | Bulk density of Laterite | 141 |
| 11.3 | Statistics chemical assay used for 3D Modelling | 143 |
| 11.4 | Statistics of lithologic domains of the total block model | 143 |
| 11.5 | Total Resource Estimation at (TREEO+ Sc2O3+Y2O3) above 350 ppm cutoff | 144 |
| 11.5a | Average grades of other commodities at (TREEO+ Sc2O3+Y2O3) resource shown in table 11.5 | 144 |
| 11.6 | Total Resource Estimation of Sc at Cut-Off 20ppm & 50ppm (for whole block 3.92Sq.km) | 145 |
| 11.7 | Total Resource Estimation of Ga at Cut-Off 30ppm & 50ppm (for whole block 3.92Sq.km) | 146 |
| 11.8 | Gross and Net resource estimation (for whole block 3.92Sq.km) | 149 |
| 11.9 | TotalEstimatedTREEO+Sc2O3+Y2O3resourceintheblock delineated zone recommended for auction block | 157 |
| 11.10 | Estimated Resource of Scandium>20ppminDelineatedzone(0.89Sq.km) | 158 |
| 11.11 | Estimated Resource of Scandium>50ppminDelineatedzone(0.89Sq.km) | 158 |
| 11.12 | Estimated Resource of Gallium>30ppminDelineatedzone(0.89Sq.km) | 159 |
| 11.13 | Estimated Resource of Gallium>50ppminDelineatedzone(0.89Sq.km) | 159 |
| 13.1 | The resource estimation table for (TREEO+Sc2O3+Y2O3)>350ppm. | 162 |
| 13.2 | The resource estimation table for (TREEO+Sc2O3+Y2O3)>350ppm with reduction factor 10%. | 163 |
| 13.3 | Total Resource Estimation of Scandium>20ppm | 163 |
| 13.4 | Total Resource Estimation of Scandium>50ppm | 163 |
| 13.5 | Total Resource Estimation of Gallium>30ppm | 164 |
| 13.6 | Total Resource Estimation of Gallium>50ppm | 164 |
| 13.7 | Estimated Resource of Scandium>20ppminDelineatedzone(0.89Sq.km) | 168 |
| 13.8 | Estimated Resource of Scandium>50ppminDelineatedzone(0.89Sq.km) | 168 |
| 13.9 | Estimated Resource of Gallium>30ppminDelineatedzone(0.89Sq.km) | 169 |
| 13.10 | Estimated Resource of Gallium>50ppminDelineatedzone(0.89Sq.km) | 169 |

List of Annexures

| Annexure no. | Title | Page No. |
| --- | --- | --- |
| Annexure-1 | Boundary Co-ordinates of Naredi Block | 172 |
| Annexure-2 | Location Map of Naredi Block | 173 |
| Annexure-3 | Topographic Map of Naredi Block | 174 |
| Annexure-4 | Detailed Geological Map of Naredi Block | 175 |
| Annexure-5a | Borehole Locations of Naredi Block | 176 |
| Annexure-5b | Borehole Locations on Detailed Geological Map of Naredi Block | 177 |
| Annexure-6 | Borehole Log | 178 |
| Annexure-7a | List of Samples for XRF Analysis | 207 |
| Annexure-7b | List of Samples for ICPMS Analysis | 209 |
| Annexure-8a | XRF Analysis Results | 213 |
| Annexure-8b | ICPMS Analysis Results | 218 |
| Annexure-9 | Corebox Photos | 253 |
| Annexure-10 | Peer reviewer comments and Action taken | 310 |