GEOLOGICAL REPORT ON RECONNAISSANCE SURVEY

(G4) FOR BAUXITE, Ga, V, Ti & REE

IN

NADAPA AREA

(6.20 Sq. Km)

KACHCHH DISTRICT, GUJARAT

(Under NMEDT Programme)

TEXT, ANNEXURE AND PLATES

CRITICAL MINERAL TRACKERS

(NOTIFIED PRIVATE EXPLORATION AGENCY)

March – 2026

CONTENTS

GEOLOGICAL REPORT ON RECONNAISSANCE SURVEY (G4) FOR BAUXITE, Ga, V, Ti & REE IN NADAPA AREA

KACHCHH DISTRICT, GUJARAT

LIST OF CONTENTS

| No | Title | Page No |
| --- | --- | --- |
|  | CHAPTER-I | 1 - 7 |
| 1.0 | साराांश | 1 - 4 |
| 1.0 | SUMMARY | 5 - 7 |
|  | CHAPTER-II | 8 - 16 |
| 2.0 | INTRODUCTION | 8 |
| 2.1 | - Details of project | 10 |
| 2.2 | - Investigating agency | 11 - 12 |
| 2.3 | - Objectives of investigation | 12 |
| 2.4 | - Basis for Taking up investigation | 13 |
| 2.5 | - Details and nature and quantum of work proposed vs achievement | 13 - 14 |
| 2.6 | - Mode of operation of different work components and associated agency | 15 |
| 2.7 | - Details of personal associated with the exploration work of Nadapa area, District: Kachchh, State: Gujarat | 16 |
|  | CHAPTER-III | 17 - 25 |
| 3.0 | # PROPERTY DESCRIPTION | 17 |
| 3.1 | Location | 17 - 18 |
| 3.2 | Accessibility | 19 |
| 3.3 | Climate | 19 |
| 3.4 | Cadastral Details and Land Status | 19 |
| 3.5 | Land use & Land cover Pattern | 19 - 21 |
| 3.6 | Forest, Sanctuaries, National Park and wildlife sanctuaries | 22 |
| 3.7 | Flora and Fauna within and nearby | 22 |
| 3.8 | Geomorphology | 22 - 23 |
| 3.9 | Local infrastructure | 24 |
| 3.10 | Population | 25 |
| 3.11 | Socio Demographic profile | 25 |
| 3.12 | Historical / Archaeological Site | 25 |
|  | CHAPTER-IV | 26 - 27 |
| 4.0 | # PREVIOUS WORK | 26 - 27 |
|  | CHAPTER-V | 28 - 34 |
| 5.0 | GEOLOGY | 28 |
| 5.1 | Regional Geology | 28 - 31 |
| 5.2 | Regional Structure | 32 - 33 |
| 5.3 | Metamorphism | 33 |
| 5.4 | Host rock for mineralisation | 33 - 34 |
|  | CHAPTER-VI | 35 - 137 |
| 6.0 | ACTIVITY DURING THE PERIOD (Geoscience investigation) | 35 |
| 6.1 | Large-scale Geological mapping | 35 |
| 6.1.1 | Description of Lithology | 36 - 42 |
| 6.1.2 | Description of rock types in outcrop map | 43 - 44 |
| 6.1.3 | Petrographic studies | 44 - 48 |
| 6.1.4 | Structure | 49 - 52 |
| 6.1.5 | Metamorphism | 53 |
| 6.1.6 | Mineralogy of the ore zones and ore textures | 53 |
| 6.1.7 | Pitting and trenching | 53 |
| 6.1.7.1 | Pitting | 53-76 |
| 6.1.7.2 | Trenching | 77 - 83 |
| 6.1.8 | Sampling | 84 - 87 |
| 6.1.9 | Discussion on Geochemistry of pit and trench samples | 88 - 130 |
| 6.1.10 | Ore Zones | 131 |
| 6.2 | Geophysical exploration | 131 |
| 6.3 | Geochemical exploration | 131 - 135 |
| 6.4 | DGPS survey of boundary points | 136 - 137 |
|  | CHAPTER-VII | 138 |
| 7.0 | Integration of geology, geophysics | 138 |
|  | CHAPTER-VIII | 138 |
| 8.0 | Mineral prospect | 138 |
|  | CHAPTER-IX | 138 |
| 9.0 | Exploration by scout drilling | 138 |
|  | CHAPTER-X | 139 |
| 10.0 | Resource estimation | 139 |
|  | CHAPTER - XI | 139 |
| 11.0 | CONCLUSION AND RECOMMENDATION | 139 |
|  | CHAPTER-XII | 140 - 144 |
| 12.0 | EXPENDITURE | 140 - 144 |
|  | CHAPTER-XIII | 145 - 146 |
| 13.0 | REFERENCE | 145 - 146 |
|  | ACKNOWLDGEMENTS | 146 |
|  | CHAPTER-XIV | 147 |
| 14.0 | LOCALITY INDEX | 147 |
|  | LIST OF ANNEXURES & ANNEXURES | 148 - 190 |
|  | LIST OF PLATES | 191 |
| 15.0 | PEER REVIEWER COMMENTS | 192 - 193 |

GEOLOGICAL REPORT ON RECONNAISSANCE SURVEY (G4) FOR BAUXITE, Ga, V, Ti & REE IN NADAPA AREA

KACHCHH DISTRICT, GUJARAT

LIST OF TABLES

| Table No | Title | Page No |
| --- | --- | --- |
| 2.1 | Coordinates & elevation of cardinal points of Nadapa Area, District: Kachchh, Gujarat (as determined by DGPS Survey) | 11 |
| 2.2 | Details of Exploration Agency | 11 - 12 |
| 2.3 | Summarized Table showing Component wise proposed quantum Vs. Quantum Achieved | 13 - 14 |
| 2.4 | Mode of operation of different work component and associated agencies | 15 |
| 2.5 | Details of Personnel Associated with the Exploration work of Nadapa area, Dist.: Kachchh, Gujarat | 16 |
| 3.1 | Details of Coordinates and elevation of 4 cardinal points and two temporary benchmarks as determined by DGPS survey | 17 |
| 5.1 | Generalised Stratigraphic succession of Kachchh mainland basin | 30 |
| 6.1 | The generalized stratigraphic succession of Nadapa area | 36 |
| 6.2 | The location of Pits (measured by GPS) | 54 |
| 6.3 | The location of Trenches (measured by GPS) | 77 |
| 6.4 | Summarized table showing range of major oxides in Katrol formation | 113 |
| 6.5 | Summarized table showing range of REE and Gallium in Katrol formation | 114 |
| 6.6 | Summarized table showing range of major oxides in Bhuj formation | 115 |
| 6.7 | Summarized table showing range of REE and Gallium in Bhuj formation | 117 |
| 6.8 | Lithology wise variation of Major Oxides | 119 |
| 6.9 | Lithology wise variation of REE and Gallium | 120 |
| 6.10 | Summarised table showing statistical analysis of Primary Vs External check sample assay of SiO2, Al2O3, Fe2O3, MnO and MgO | 133 |
| 6.11 | Summarised table showing statistical analysis of Primary Vs External check sample assay of CaO, Na2O, K2O, TiO2 and P2O5 | 133 |
| 6.12 | Statement showing analysis of External check samples for major oxides and its comparison with primary samples of Nadapa area, Kachchh district, Gujarat | 134 - 135 |
| 6.13 | Statement showing analysis of one External check sample for REE and its comparison with primary sample analysis of Nadapa Area, Kachchh District, Gujarat | 135 |
| 6.14 | The details of coordinates and elevation of 4 cardinal points and Two temporary bench marks as determined by DGPS Survey | 137 |
| 12.1 | Details of actual expenditure incurred for Reconnaissance survey (G4) in Nadapa area, Kachchh Dist., Gujarat. | 142 - 144 |
| 14.1 | Locality index | 147 |

GEOLOGICAL REPORT ON RECONNAISSANCE SURVEY (G4) FOR BAUXITE, Ga, V, Ti & REE IN NADAPA AREA

KACHCHH DISTRICT, GUJARAT

LIST OF FIGURES

| Fig No | Title | Page No |
| --- | --- | --- |
| 1 | Details of block boundary, geology, structure, satellite image and justification provided by CGM (Commissioner of Geology and Mining) Gujarat. | 9 |
| 2 | Location Map of Reconnaissance Survey(G4) for Bauxite, Ga, V, Ti & REE Nadapa area, Dist. : Kachchh, Gujarat | 18 |
| 3 | Landuse/landcover map of Nadapa area, Kachchh district, Gujarat | 21 |
| 4 | Topographical map of Nadapa area, Kachchh district, Gujarat | 23 |
| 5 | Regional Geological Map of Kachchh basin(part) with location of Nadapa area, Kachchh district, Gujarat | 31 |
| 6 | Tectonic map of Kachchh (after Shukla, 2011) | 33 |
| 7 | Interpreted large scale Geological map of Nadapa area, Kachchh district, Gujarat | 41 |
| 8 | Outcrop map of Nadapa area, Kachchh district, Gujarat. | 42 |
| 9 | Location of pit & trench of large-scale Geological map of Nadapa area, Kachchh district, Gujarat | 55 |
| 10 | Schematic diagram showing sample preparation procedure | 85 |
| 11 | Nadapa Block: classification of Laterites | 116 |
| 12 | Spider plot of REE | 118 |
| 13 | Distribution map of Al2O3, from pits and trenches samples in Nadapa area, Kachchh district, Gujarat | 122 |
| 14 | Distribution map of Tio2, from pits and trenches samples in Nadapa area, Kachchh district, Gujarat | 124 |
| 15 | Elemental distribution map of V, from pits and trenches samples in Nadapa area, Kachchh district, Gujarat | 126 |
| 16 | Elemental distribution map of Ga, from pits and trenches samples in Nadapa area, Kachchh district, Gujarat | 128 |
| 17 | Elemental distribution map total REE from pits and trenches samples in Nadapa area, Kachchh district, Gujarat | 130 |

GEOLOGICAL REPORT ON RECONNAISSANCE SURVEY (G4) FOR BAUXITE, Ga, V, Ti & REE IN NADAPA AREA

KACHCHH DISTRICT, GUJARAT

LIST OF PHOTOGRAPHS

| Photograph No | Title | Page No |
| --- | --- | --- |
| 1 | Image showing interbedded sequence of sandstone, siltstone of Katrol formation | 37 |
| 2 | Image showing fined grained sandstone with iron concretion of Katrol formation | 37 |
| 3 | Image showing gradational contact between Katrol and Bhuj formations | 37 |
| 4 | Image showing interbedded feldspathic sandstone and siltstone of Bhuj formation | 38 |
| 5 | Image showing thinly laminated siltstone, shale and clay bed at bottom of Bhuj formation | 39 |
| 6 | Image showing ferruginous sandstone and splintery shales of Bhuj formation | 39 |
| 7 | Image showing gritty sandstone with clay lenses | 39 |
| 8 | Image showing laminated fine-grained sandstone with clay, shale and laminae | 39 |
| 9 | Image showing fine grained sandstone interbedded with siltstone | 40 |
| 10 & 11 | Image showing hand specimen medium grained sandstone and corresponding photomicrograph | 44 |
| 12 & 13 | Image showing hand specimen fine grained sandstone and corresponding photomicrograph | 45 |
| 14 & 15 | Image showing hand specimen Ferruginous sandstone and corresponding photomicrograph | 47 - 48 |
| 16 | Image showing ripple marks in Katrol formation | 50 |
| 17 | Image showing burrows structure in Katrol formation | 50 |
| 18 | Image showing cross bedding in fine grained sandstone of Bhuj formation | 51 |
| 19 | Image showing ripples, thin laminations in siltstone of Bhuj formation | 51 |
| 20 | Image showing burrows structure in ferruginous sandstone and laterite of Bhuj formation | 51 |
| 21 | Image showing burrows structure in ferruginous sandstone and laterite of Bhuj formation | 51 |
| 22 | Image showing plant fossil in the shales | 52 |
| 23 | Image showing plant fossil in the siltstone | 52 |
| 24 | Image showing plant fossil in the grey clay beds of Bhuj formation | 52 |
| 25 | Image showing pit in the study area | 56 |
| 26 | Image showing trench in the study area | 78 |
| 27 - 33 | Images showing different activities of sample preparation | 86 - 87 |

GEOLOGICAL REPORT ON RECONNAISSANCE SURVEY (G4) FOR BAUXITE, Ga, V, Ti & REE IN NADAPA AREA

KACHCHH DISTRICT, GUJARAT

LIST OF PHOTOMICROGRAPHS

| No | Title | Page No |
| --- | --- | --- |
| 1 | Photomicrograph showing presence of different grain size of quartz, plagioclase feldspar and biotite under transmitted light XPL (2X,10X). | 44 - 45 |
| 2 | Photomicrograph showing presence of quartz and Plagioclase feldspar, microcline and biotite under transmitted light XPL (10X) | 45 - 46 |
| 3 | Photomicrograph showing presence of angular to sub-angular quartz grains and few mica as detrital material cemented by ferruginous material | 47 - 48 |

CHAPTER - I

सारांश

नडापा क्षेत्र, जिला कच्छ, गुजरात में 6.20 वर्ग किमी क्षेत्रफल में, टोपोशीट संख्या 41E/15 के अंतर्गत, “नडापा क्षेत्र, जिला कच्छ, गुजरात में बॉक्साइट, Ga, V, Ti एवं REE के लिए पुनर्सर्वेक्षण (G4)” शीर्षक NMEDT परियोजना के अंतर्गत सर्वेक्षण कार्य संपन्न किया गया। यह परियोजना NMEDT कार्यक्रम के अंतर्गत Critical Mineral Trackers Pvt. Limited को फाइल सं. 23/569/2025-NMET/849 दिनांक 10 फरवरी 2025 द्वारा प्रदान की गई। तत्पश्चात संशोधित आदेश फाइल सं. 23/570/2025-NMET/901 दिनांक 24 फरवरी 2025 द्वारा Critical Mineral Trackers (An NPEA Company) के नाम जारी किया गया। Critical Mineral Trackers द्वारा 14 फरवरी 2025 से 30 अप्रैल 2025 के बीच 1:12,500 मापनी पर विस्तृत भू-वैज्ञानिक मानचित्रण, गड्ढा (Pitting) एवं ट्रेंचिंग कार्य संपन्न किए गए।

नडापा क्षेत्र 6.20 वर्ग किमी में विस्तृत है और यह सर्वे ऑफ इंडिया टोपोशीट संख्या 41E/15 में स्थित है। यह 23.31999°N से 23.34371°N अक्षांश तथा 69.89864°E से 69.93013°E देशांतर के मध्य स्थित है। यह प्रशासनिक रूप से भुज तहसील, जिला कच्छ, गुजरात के अंतर्गत आता है। क्षेत्र की ऊँचाई लगभग 112 मीटर से 146 मीटर के मध्य है (DGPS सर्वेक्षण के अनुसार)।

| कार्डिनल बिंदु | डिग्री दशमलव में भौगोलिक निर्देशांक प्रणाली (डब्ल्यूजीएस 1984) |  | यूटीएम निर्देशांक (डब्ल्यूजीएस 1984-जोन 42 उत्तर) |  |  | क्षेत्रफल (वर्ग किमी)) |
| --- | --- | --- | --- | --- | --- | --- |
|  | अक्षांश | देशांतर | ऊँचाई (मी) | नाथिंग (मी) | पूरब (मी) |  |
| A | 23.33271300 | 69.90214122 | 112.601 | 2580639.366 | 592227.296 | 6.20 |
| B | 23.32067572 | 69.89864439 | 146.246 | 2579304.447 | 591818.067 |  |
| C | 23.31999622 | 69.93013017 | 136.272 | 2579249.557 | 595097.873 |  |
| D | 23.34371217 | 69.92674692 | 136.913 | 2581873.036 | 594735.121 |  |

यह क्षेत्र सड़क एवं रेल मार्ग से सुगम है। यह 8 किमी लंबे संपर्क मार्ग द्वारा भुज–भचाऊ राज्य राजमार्ग (SH-42) से जुड़ा है। SH-42 आगे चलकर कुकमा के समीप (लगभग 17 किमी पश्चिम) राष्ट्रीय राजमार्ग 341 (अंजार–भुज मार्ग) से जुड़ता है। निकटतम रेलवे स्टेशन रतनाल है, जो लगभग 18 किमी दक्षिण में गांधिधाम–भुज रेलवे लाइन पर स्थित है। भुज हवाई अड्डा लगभग 40 किमी दूरी पर स्थित है।

6.20 वर्ग किमी क्षेत्र में G4 स्तर का अन्वेषण बॉक्साइट एवं संबंधित महत्वपूर्ण खनिजों (Ga, V, Ti एवं REE) की संभाव्यता के मूल्यांकन हेतु किया गया। 1:12,500 मापनी पर किए गए भूवैज्ञानिक मानचित्रण से स्पष्ट हुआ कि क्षेत्र में केवल दो शैल-इकाइयाँ उपस्थित हैं:

- पुरानी कट्रोल संरचना (Late Jurassic से Early Cretaceous)

- युवा भुज संरचना (Early Cretaceous)

अध्ययन क्षेत्र का लगभग 3/4 भाग भुज संरचना से आच्छादित है।

यह संरचना मुख्यतः उत्तरी भाग के पेडीप्लेन क्षेत्र में पाई जाती है। इसमें गुलाबी-भूरे कठोर लौहयुक्त बलुआ पत्थर, पीले-सफेद सिल्टस्टोन, पीले-भूरे अभ्रकीय शेल तथा पीले से गुलाबी कैल्केरियस बलुआ पत्थर सम्मिलित हैं। ये बलुआ पत्थर सामान्यतः कैल्केरियस एवं फेल्सपैथिक प्रकृति के होते हैं।

इनमें मैक्रो जीवाश्मों का अभाव है तथा लेंसाकार स्तरण, रिप्पल मार्क्स एवं मड क्रैक्स जैसे अवसादी लक्षण मिलते हैं, जो उथले समुद्री निक्षेपण वातावरण का संकेत देते हैं।

भुज संरचना कट्रोल संरचना के ऊपर स्थित है और मुख्यतः दक्षिणी भाग में पाई जाती है। यह मध्य भाग में रिज एवं पेडीमेंट रूप में प्रदर्शित होती है। इसमें शीर्ष पर कठोर लौहयुक्त ग्रिट्टी फेल्सपैथिक बलुआ पत्थर, उसके नीचे भूरे शेल, सिल्टस्टोन, भुरभुरे मृत्तिका युक्त बलुआ पत्थर तथा चिकनी मिट्टी की परतें/लेंस पाए जाते हैं।

सिल्टस्टोन एवं चिकनी मिट्टी की परतों में पादप जीवाश्म पाए गए हैं। क्रॉस-बेडिंग, रिप्पल मार्क्स एवं बुरोज जैसे प्राथमिक अवसादी संरचनाएँ यह दर्शाती हैं कि भुज संरचना का निक्षेपण नदीय से डेल्टाई वातावरण में हुआ।

दोनों संरचनाओं में N80°E–S80°W की क्षेत्रीय संरचनात्मक प्रवृत्ति पाई जाती है तथा परतों का झुकाव सामान्यतः 5° से 10° दक्षिण एवं दक्षिण-पूर्व दिशा में है।

विस्तृत मानचित्रण के पश्चात अप्रैल 2025 में 20 गड्ढे (1×1×1 मी., कुल 20 घन मी.) एवं 5 ट्रेंच (10×1×1 मी., कुल 50 घन मी.) JCB मशीन द्वारा खोदे गए। गड्ढों से 20 तथा ट्रेंच से 50 प्रतिनिधि नमूने एकत्र किए गए। कुल 90 नमूनों का प्रमुख ऑक्साइडों हेतु तथा 15 नमूनों का REE हेतु विश्लेषण Lucid Laboratories Pvt. Ltd., हैदराबाद में किया गया। विश्लेषणात्मक परिणामों से स्पष्ट हुआ कि कट्रोल एवं भुज दोनों संरचनाओं में लक्षित तत्वों की सांद्रता समान रूप से निम्न है।

कट्रोल संरचना में:

- Al₂O₃: 0.28–28.81%

- SiO₂: 54.92–73.26%

- TiO₂: 0.39–1.34%

- Ga: 16.33–21.05 ppm

- V: 33.80–103.80 ppm

- कुल REE: 241.65–350.72 ppm

- कुल REE+(Sc+Y): 271.08–381.77 ppm

भुज संरचना में:

- Al₂O₃: 4.83–34.89%

- SiO₂: 44.74–77.97%

- TiO₂: 0.52–2.02%

- Ga: 9.58–31.58 ppm

- V: 15.70–299.18 ppm

- कुल REE: 157.58–338.76 ppm

- कुल REE+(Sc+Y): 176.86–375.99 ppm

लिथोलॉजिकल रूप से ये संरचनाएँ क्लास्टिक अवसादी अनुक्रम हैं। इनमें लैटराइटिक प्रोफाइल, एल्यूमिनस समृद्धि क्षेत्र एवं अनुकूल अपक्षय परिस्थितियों का अभाव है, जो आर्थिक बॉक्साइट निक्षेपों के निर्माण एवं संरक्षण हेतु आवश्यक होते हैं।

पड़ोसी क्षेत्रों में बॉक्साइट खनिजीकरण मातानोमाध संरचना से संबद्ध है, जो इस अध्ययन क्षेत्र में पूर्णतः अनुपस्थित है।

G4 स्तर के अन्वेषण कार्यक्रम से स्पष्ट है कि नडापा क्षेत्र में लैटरिटाइजेशन एवं बॉक्सिटाइजेशन के संकेत अनुपस्थित हैं। सतही एवं उपसतही स्तर पर किसी भी प्रकार की एल्यूमिनस समृद्धि नहीं पाई गई। भू-रासायनिक मान आर्थिक सीमा से निम्न हैं। साथ ही, बॉक्साइट के प्रमुख अधिष्ठान मातानोमाध संरचना का अभाव है।

समेकित भूवैज्ञानिक, संरचनात्मक, उपसतही एवं भू-रासायनिक मूल्यांकन के आधार पर नडापा ब्लॉक में बॉक्साइट अथवा संबंधित महत्वपूर्ण खनिज (Ga, V, Ti, REE) की संभाव्यता नहीं पाई गई।

अतः इस क्षेत्र को अप्रत्याशित (non-prospective) घोषित किया जाता है तथा वर्तमान चरण में आगे किसी अन्वेषण की अनुशंसा नहीं की जाती।

SUMMARY

Reconnaissance survey of 6.20 sq. km area falling under toposheet number 41E/15 was carried out in Nadapa Area, Kachchh district, Gujarat, as part of the NMEDT project entitled “Reconnaissance survey (G4) for Bauxite, Ga, V, Ti & REE in Nadapa area, Kachchh district, Gujarat”. The project was awarded to Critical Mineral Trackers Pvt Limited under the NMEDT Programme vide File no. 23/569/2025-NMET/849 dated 10th February ,2025 and corrected version in the name of Critical Mineral Trackers (An NPEA company) was issued subsequently vide File no:  23/570/2025-NMET/901 dated 24th February, 2025.  Critical Mineral Trackers completed large scale mapping on 1:12500 scale, pitting & trenching works in this area between 14th Feb 2025 to 30th April 2025.

NADAPA Area admeasuring an area of 6.20 sq. km is falling under the Survey of India Toposheet No 41E/15 and bounded by latitudes 23.31999°N to 23.34371 N and longitudes 69.89864°E to 69.93013°E, lies under the administrative boundary of Bhuj tehsil, Kachchh district of Gujarat.

Coordinates & elevation of cardinal points of Nadapa Area,

District: Kachchh, Gujarat (as determined by DGPS Survey)

| Cardinal Points | Geographic Coordinate System in Degree Decimal (WGS 1984) |  | UTM (WGS 1984, Zone 42N) |  |  | Area ( Sq. Km) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Latitude(N) | Longitude(E) | Elevation (m) | Northing (m) | Easting (m) |  |
| A | 23.33271300 | 69.90214122 | 112.601 | 2580639.366 | 592227.296 | 6.20 |
| B | 23.32067572 | 69.89864439 | 146.246 | 2579304.447 | 591818.067 |  |
| C | 23.31999622 | 69.93013017 | 136.272 | 2579249.557 | 595097.873 |  |
| D | 23.34371217 | 69.92674692 | 136.913 | 2581873.036 | 594735.121 |  |

The study area has good road and rail connectivity. It is linked to the Bhuj–Bhachau State Highway (SH-42) via an 8 km approach road. SH-42 further connects to National Highway 341(Anjar- Bhuj highway) near Kukma, 17 km to the west. The nearest railway station is Ratnal, located 18 km towards south on the Gandhidham–Bhuj line of the Western Railway. Bhuj Airport is also accessible, situated about 40 km from the block.

The G4 exploration carried out over 6.20 sq. km in the Nadapa area, Kachchh district, aimed to assess the potential for bauxite and associated critical minerals (Ga, V, Ti and REE). Geological mapping on a 1:12500 scale established the presence of only two litho-units older Katrol Formation (late Jurassic to early cretaceous) and younger Bhuj Formation (early cretaceous). Bhuj formation occupies 3/4th of the study area

The Katrol Formation is occupied by Pediplain area towards north and comprises predominantly of pinkish brown hard ferruginous sandstone, yellowish-white siltstones, yellowish-brown micaceous shales, and yellowish to pinkish calcareous sandstones. The sandstones are typically calcareous and often feldspathic. These units generally lack macrofossils, and shows lenticular bedding, Ripple marks and Mud cracks. These features indicate deposition was under shallow marine environment.

The Bhuj Formation overlies the Katrol Formation and occurs predominantly toward the south side. They are represented by ridge & pediment in the central part and covered by rock outcrops and thorny bushes.  They are composed of hard, resistant ferruginous gritty feldspathic sand stone on the top followed by brown shales, siltstones, friable argillaceous sandstones often micaceous and clay beds/lenses. Siltstones and clay beds contain plant fossils. The observed lithological variability, together with well-preserved primary sedimentary structures such as cross-bedding, ripple marks, burrows indicates that the Bhuj Formation was deposited in a fluvial to deltaic depositional environment.

Both formations exhibit a regional structural trend of N80°E–S80°W, with local variations in NE–SW orientation. The strata generally dip 5° to 10° towards the south and southeast.

After completion of large-scale mapping, Pitting and trenching was carried out in April,2025 and excavated 20 pits of 1*1*1m size (Total 20Cu.m) and 5 trenches of 10*1*1m size (50Cu.m) using JCB and collected 20 representative samples from pits and 50 samples from trenches. All 90 samples were analyzed for major oxides and 15 samples for REE at Lucid Laboratories Pvt Ltd, Hyderabad.

The analytical results indicate uniformly low concentration of targeted elements in both Katrol and Bhuj formations.

In Katrol formation, Al2O3 values varies from 0.28 to 28.81%, SiO2 ( 54.92-73.26%,) TiO2( 0.39-1.34%), Gallium (16.33-21.05ppm) ,Vanadium (33.80-103.80 ppm) Total REE (241.65-350.72ppm) and  TREE+(Sc+Y):271.08-381.77ppm, whereas in Bhuj formation also the concentration of Al2O3 ranges from 4.83-34.89%, SiO2 (44.74-77.97%), TiO2 (0.52-2.02%),Gallium (9.58-31.58ppm),Vanadium (15.70-299.18) , Total REE(157.58-338.76ppm) and TREE+(Sc+Y):176.86-375.99ppm.

Lithologically, these formations represent clastic sedimentary sequences. Such litho-units lack the lateritic profiles, aluminous enrichment zones, and favorable weathering conditions required for the formation and preservation of economic bauxite deposits. In contrast, neighboring bauxite bearing regions are associated with the Matanomadh Formation, which hosts lateritic bauxite horizons. This formation is completely absent within the study area.

The G4 exploration program in the Nadapa area indicates absence of lateritisation and bauxitisation signatures. No surface or subsurface indication of aluminous enrichment are found within the study area. Geochemical values are below economic threshold levels. Absence of the Matanomadh Formation, the principal host of bauxite in adjoining areas.

Based on the integrated geological, structural, subsurface, and geochemical evaluation, the Nadapa block does not exhibit potential for bauxite or associated critical mineral (Ga, V, Ti, REE) mineralization.

The area is therefore considered non-prospective, and no further exploration is recommended at this stage.

CHAPTER – II

2.0	INTRODUCTION

The Bhuj–Kachchh region of Gujarat hosts lateritic and sedimentary bauxite deposits within the Matanomadh-Bhuj stratigraphic intervals. Published studies and exploration results indicate that such bauxite–laterite systems are globally significant hosts of strategic trace elements including gallium (Ga), rare-earth elements (REE) and in specific mineral phases, vanadium (V) and titanium (Ti). Given the strategic importance of these commodities and the potential for low-cost identification of enriched horizons, a G4 reconnaissance stage is both necessary and justified.

The bauxite occurrences of the Bhuj region are developed as lateritic caps and ferricrete horizons overlying Cretaceous–Paleogene sedimentary formations. Lateritic bauxite systems are known to enrich Gallium (Ga) within gibbsite/boehmite phases, while REE enrichment commonly occurs in Fe–Mn oxides, clay-rich basal zones or accessory minerals such as monazite/xenotime. Vanadium(V) and Titanium (TiO2) may concentrate in heavy-mineral fractions (ilmenite, anatase) or Fe-rich profiles. The region’s extensive low-grade bauxite makes it a suitable target for reconnaissance-level resource screening.

Considering important parameters such as resource/ reserve position in the country, production, import dependency, use for future technology/ clean energy, requirement of fertilizer minerals in an agrarian economy, the Committee has identified a set of 30 critical minerals. These are Antimony, Beryllium, Bismuth, Cobalt, Copper, Gallium, Germanium, Graphite, Hafnium, Indium, Lithium, Molybdenum, Niobium, Nickel, PGE, Phosphorous, Potash, REE, Rhenium, Silicon, Strontium, Tantalum, Tellurium, Tin, Titanium, Tungsten, Vanadium, Zirconium, Selenium and Cadmium. (Critical Minerals for India, Report of the Committee on Identification of Critical Minerals, Ministry of Mines, June 2023)

Keeping in view that Gallium, Vanadium, Titanium and REE being a listed critical mineral, the present exploration program may be justified for the above minerals, besides bauxite/laterite.

The Commissioner of Geology & Mining (CGM), Gandhinagar, Gujarat organized one day workshop titled “Gujarat Mineral Wealth: A responsible exploration and development Paradigm” on 17th August, 2024.Critical Mineral Trackers (CMT), an NPEA (Notified private exploration agency) company from Hyderabad had participated in this workshop and expressed interest in certain blocks for exploration. Accordingly, The CGM has granted” No objection certificate for “Reconnaissance (G4) survey for bauxite exploration in NADAPA Area, Kachchh district, Gujarat to Critical mineral trackers vide email dated 11th November,2024. The block boundary coordinates with geology, structure and justification for taking up this block were provided by the CGM, Gandhinagar, Gujarat

& given in Fig No:1

Details of block boundary, geology, structure, satellite image and justification provided by CGM (Commissioner of Geology and Mining), Gujarat.

Fig No: 1

In the outset, CMT has conducted a few field traverses in   Nadapa block during 3rd week of November,2024 and collected representative samples and analysed at Lucid   laboratories Pvt Ltd, Hyderabad (NABL accredited). Subsequently CMT has prepared a proposal titled “Proposal for Bauxite exploration in NADAPA Block, Kachchh District, Gujarat State, Reconnaissance Survey (G4 Stage) under NMET falling under toposheet no; 41E/15, incorporated the analytical results of selected samples and submitted to The Director, National Mineral Exploration Trust (NMET), Ministry of mines on 12th December, 2024 in the designated proforma for necessary sanction/approval. TCC-II, NMET has evaluated the proposal in 4th TCC-II meeting held on 26th &27th December ,2024 and opined that the area has potential for Ga, V, Ti and REE besides bauxite and accordingly the proposal was modified as “Reconnaissance Survey (G4) for Bauxite, Ga, V, Ti and REE in Nadapa area, Kachchh District, Gujarat and submitted to EC, NMET for approval.

The Executive Committee (EC), NMET approved the proposal in its 39th meeting held on 24th January ,2025 with an estimated cost of Rs 53,72,568/- and communicated to M/s Critical Mineral Trackers Pvt Limited vide file no: 23/569/2025-NMET/849 dated 10th February ,2025 and corrected version in the name of Critical Mineral Trackers was issued subsequently vide File no:  23/570/2025-NMET/901 dated 24th February, 2025.

Accordingly, Critical Mineral Trackers initiated field work on 14th Fenruary,2025 and completed large scale mapping in an area of 6.2Sq.km, pitting, trenching and sampling works by 30th April,2025.  After completion of LSM and interpretation of analytical results of pit and trench samples (90 no’s) as received from Lucid laboratories pvt limited, Hyderabad, review was done by 9th TCC-II on 29th May. During the review meeting, TCC-II did not recommend drilling work in this block since the values of Al203, Ga, V, Ti and REE are not encouraging and asked CMT to submit the report. Accordingly, CMT has prepared the geological report and submitted in March-2026

2.1	Details of project

“NADAPA Area” admeasuring an area of 6.20 sq. km is bounded by latitudes 23.31999°N to 23.34371 N and longitudes 69.89864°E to 69.93013°E, lies under the administrative boundary of Bhuj tehsil, Kachchh district of Gujarat. Bhuj is the nearest town and district head-quarter located 35 km towards west. Nadapa village lies at the southern boundary and partially within the study area. But within the block is accessible by cart tracks/ foot tracks by 4*4 vehicle. About 4 China clay processing plants are located within the block.

The Nadapa Area is falling under the Survey of India Toposheet No 41E/15. The coordinates of the cardinal points of the block boundary as determined by DGPS Survey are given below in Table No. 2.1

Table 2.1 Coordinates & elevation of cardinal points of Nadapa area,

District: Kachchh, Gujarat (as determined by DGPS Survey)

| Cardinal Points | Geographic Coordinate System in Degree Decimal (WGS 1984) |  | UTM (WGS 1984, Zone 42N) |  |  | Area ( Sq. Km) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Latitude(N) | Longitude(E) | Elevation (m) | Northing (m) | Easting (m) |  |
| A | 23.33271300 | 69.90214122 | 112.601 | 2580639.366 | 592227.296 | 6.20 |
| B | 23.32067572 | 69.89864439 | 146.246 | 2579304.447 | 591818.067 |  |
| C | 23.31999622 | 69.93013017 | 136.272 | 2579249.557 | 595097.873 |  |
| D | 23.34371217 | 69.92674692 | 136.913 | 2581873.036 | 594735.121 |  |

2.2 	            Details of investigating agency are given in the following table 2.2

# Table 2.2 Details of Exploration Agency

| - (a) Name: | CRITICAL MINERAL TRACKERS (An NPEA company) (Ministry of Mines, Govt. of India) HYDERABAD |
| --- | --- |
| - (b) Address: | #306, Concourse Building, Opp Lal Bungalow, Ameerpet, Hyderabad, Pin Code – 500016. And CMT Geo Solution center, No E5, 3rd Floor, Technology Research Park, Indian Institute of Technology, Hyderabad, Kandi, Sanga Reddy – 502 284 |
| - (c) Contact Mobile No: | +914031531932 |
| - (d) E-Mail id: | criticalmineraltrackers@gmail.com director.ops@criticalmineraltrackers.co.in enquiry@criticalmineraltrackers.co.in |
| - (e) Qualification: | - Professionals with M.Sc. / MSc Tech (Geology) Qualification |
| - (f) Experience: | - Senior Professionals with experience +30 years and young geologists of 2 to 3years experience. |
| Date of Accreditation granted by QCI -NABET | May 20,2024 |
| Date of Expiry of Accreditation | May 15,2027 |
| Date of Notification Under the Proviso to section 4(1) of the MMDR Act | SONO 2379(E) 20.06.2024 |
| Date of Expiry of Notification | 19.06.2027 |
| Category of the Exploration Agency (Category A Or B) under Notification | A |

2.3	Objectives of investigation

Nadapa Area was proposed for G-4 level of exploration for Bauxite, Ga, V, Ti and REE. The exploration has been carried out as per Minerals (Evidence of Mineral contents) Amendment Rules,2021 to achieve the following objectives. To carry out Geological & Structural mapping on 1:12500 scale for identification of Bauxite, Ga, V, Ti & REE bearing formation (host rock) with the structural features to identify the surface manifestation and lateral disposition of the mineralized zones.

2.4	Basis for taking up investigation

As per the compilation of Commissioner of Geology and Mining (CGM), Gandhinagar on mineral resources of Gujarat, identified the NADAPA Area as a potential block for exploration of Bauxite and other associated minerals based on geology, Geophysics, geochemistry and satellite image studies. CGM has awarded this work to Critical mineral trackers (CMT). CMT has undertaken pre-field work during 3rd week of November 2024 and analysed 3 samples for Bauxite &Titanium. The Al2O3 values varies from 17.01% to 40.91% but the respective SiO2(6.24-49.83% and Fe2O3(16.78-32.71%) values are high and TiO2 varies from 0.77 to 1.60%.CMT has prepared a proposal based on these results and submitted to NMET on 12th December .During the review of proposal, TCC-II has opined that this area may be potential for Gallium, Vanadium and REE besides bauxite during 4th TCC-II meeting held on 26th & 27th December,2024 and hence included them in the project for investigation.

2.5       Details of nature and quantum of work proposed vs achievement are given   below:

Table 2.3: Summarized Table showing Component wise proposed quantum Vs. Quantum Achieved

| Sl. No. | Item of Work | Unit | Target | Achievement |
| --- | --- | --- | --- | --- |
| 1 | Geological Mapping (on 1:12,500 Scale) | Sq. km | 6.20 | 6.20 |
| 2 | Exploratory Mining |  |  |  |
| a | Excavation of pits (20 nos), size: 1*1*1 | Cu.m | 20 | 20 |
|  | Pit samples collected | Nos | 20 | 20 |
| b | Excavation of Trenches (5 nos), Size 10*1*1 | Cu.m | 50 | 50 |
|  | Trench samples (14*5=70) collected | Nos | 70 | 70 |
| 3 | DGPS survey (4 block boundary points & 4 Borehole points) | points | 8 | 4(boundary points only) |
| 4 | Scout drilling: 4 boreholes of 30m depth each ( 4*30=120m) | meters | 120 | Not recommended drilling in the review meeting held on 29th May (9th TCC-II) |
| a | Construction of borehole pillar | nos | 4 | -do- |
| b | Compensation for 4 Bhs | nos | 4 | -do- |
| c | Drill core preservation in GI boxes | meters | 120 | -do- |
| 5 | Laboratory Studies |  |  |  |
|  | Major oxides by XRF (Trench-70, pit-20, BH-40 =130 + 13 check samples) | Nos | 143 | 90+9check=99 (40 samples are earmarked for drill core samples) |
|  | REE 14 Elements by ICPMS 33+ 3 check | Nos | 36 | 15+1 check |
| 6 | Combined determination of THA, MHA and Reactive silica | Nos | 4 | 1 |
| 7 | Preparation of polished thin sections | Nos | 10 | 3 * |
| 8 | Complete petrographic//ore2qw3456790-[21-microscopic / mineragraphic studies | ` | 10 | 3 * |
| 9 | XRD Mineral phase analysis | Nos | 4 | 3 |
| 10 | Report Preparation (5 Hard copies with a soft copy) | Nos. | 1 | 5 |

2.6                  Details of mode of operation of different work components

Table 2.4 Mode of operation of different work components and associated agency

| Sr. No | Work component | Agency |
| --- | --- | --- |
| 1 | Large scale Geological mapping (1:12,500 scale) | CMT (In house) |
| 2 | Pitting, trenching, sampling | CMT (In house) |
| 3 | DGPS Survey | CMT (In house) |
| 4 | Chemical Analysis | Lucid laboratories Pvt Ltd, Hyderabad (NABL accredited lab)) |
| 5 | XRD and check sample analysis | Shiva Analyticals India Pvt Ltd, Bengaluru (NABL accredited lab) |
| 6 | Polished thin section & complete petrographic studies. | Petrology division, GSI,Southern Region Hyderabad |

2.7            Table 2.5:  Details of personnel associated with the exploration work of Nadapa area, District: Kachchh, Gujarat

| 1 | Overall Planning, Co-ordination & Overall supervision | S.Uma maheswara rao, Principal investigator & Technical Area Expert S.Rama murthy, Technical area Expert |
| --- | --- | --- |
| 3 | Project management & Field Operation | S.Uma maheswara rao Technical Area Expert |
|  |  | V.Santhosh (Geologist) & TM |
|  |  | B. Mahesh, (Geologist) & TM |
| 5 | Chemical Laboratory | LUCID Laboratories Private Limited, Hyderabad |
|  |  | (NABL Accredited laboratory) |
| 6 | Petrological Laboratory GSI, Hyderabad | Dr. K. Krishnapriya Basak, Director, GSI, petrology division, southern region, Hyderabad |
| 8 | Data Processing & Documentation | S.Uma maheswara rao |
|  |  | S. Rama Murthy |
|  |  | Mr B. Mahesh (Geologist) & TM |
|  |  | Mr. V. Siva Kumar (Geologist) & TM |
| 9 | Reprography & Printing | Mr. B. Mahesh (Geologist) & TM |
|  |  | Mr. V. Siva Kumar (Geologist) & TM |

CHAPTER – III

- Property Description

# 3.1       	Location

NADAPA Block admeasuring an area of 6.20 sq. km is bounded by latitudes 23.31999°N to 23.34371° N and longitudes 69.89864°E to 69.93013°E, lies under the administrative boundary of Bhuj tehsil, Kachchh district of Gujarat. Bhuj is the nearest town and district head-quarter located 35 km towards west. Nadapa village lies at the southern boundary and partially within the study area. About 4 china clay processing plants are located within the block. The location map of the block is furnished on

Fig No-2 as well as Plate No-I.

The Block is falling under the Survey of India Toposheet No 41E/15. The coordinates of the cardinal points of the block boundary and elevation as determined by DGPS survey are given below in Table No. 3.1 and also in Annexure No: 1

Table 3.1: Details of coordinates and elevation of 4 cardinal points and two

temporary benchmarks as determined by DGPS Survey

| Cardinal Points | Geographic Coordinate System in Degree Decimal (WGS 1984) |  | UTM (WGS 1984, Zone 42N) |  |  | Area ( Sq. Km) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Latitude(N) | Longitude(E) | Elevation (m) | Northing (m) | Easting (m) |  |
| A | 23.33271300 | 69.90214122 | 112.601 | 2580639.366 | 592227.296 | 6.20 |
| B | 23.32067572 | 69.89864439 | 146.246 | 2579304.447 | 591818.067 |  |
| C | 23.31999622 | 69.93013017 | 136.272 | 2579249.557 | 595097.873 |  |
| D | 23.34371217 | 69.92674692 | 136.913 | 2581873.036 | 594735.121 |  |
| *TBM-1 | 23.32535850 | 69.90689125 | 163.132 | 2579828.157 | 592718.031 |  |
| *TBM-2 | 23.32518869 | 69.90710933 | 162.493 | 2579809.497 | 592740.447 |  |

Note: *TBM-1 & 2: temporary bench marks established in the block

Fig No: 2

# 3.2		Accessibility

# Nadapa block has good road and rail connectivity. It is linked to the Bhuj–Bachau State Highway (SH-42) via an 8 km approach road. SH-42 further connects to National Highway 341(Anjar- Bhuj highway) near Kukma, 17 km to the west. The nearest railway station is Ratnal, located 18 km towards south on the Gandhidham–Bhuj line of the Western Railway. Bhuj Airport is also accessible, situated about 40 km from the block but within the block, access is limited to cart tracks and foot tracks which are navigable using 4*4 vehicle.

# 3.3         Climate

# Bhuj experiences an arid to semi-arid climate, characterised by hot summers, mild and pleasant winters, and scanty but concentrated monsoonal rainfall. The summer season extends from March to May, during which high temperatures are commonly recorded. The monsoon season begins in June and continues until September, contributing the major share of the annual rainfall. The winter season, from November to February, is generally cool, dry, and pleasant.

Maximum Temperature: 37.8ºC (April)*

Minimum Temperature: 13.1º C (January)*

Average annual Rainfall: 483.00mm (1994-2024) **

*As per IMD data from 1991-2021 from https://en.climate-data.org

** As per Gujarat state disaster management Authority, http://www.gsdma.org

3.4	Cadastral Details and Land Status

The block encompasses the Cadastral Survey Numbers falling within the      revenue limits of Nadapa village (Toposheet No. 41 E/15). The land use pattern within the investigated area is predominantly characterized as Government Waste Land, Barren Rocky terrain, and Agricultural land. A significant portion of the area comprises fallow land utilized for seasonal grazing. There are four small China clay processing plants present within the study area

3.5	Land Use & Land Cover pattern

CMT has prepared a Land Use/Land Cover (LULC) map of the study area based on Google imagery tonal & textural characters and constitutes a part of   Toposheet No. 41E/15. Individual land use/land cover units were digitised, classified, and appropriately labelled.

The majority of the area (55.4%) is occupied by barren land, represented by an east–west trending central ridge and the surrounding pediment areas, characterised by rocky outcrops and sparse vegetation dominated by thorny babul trees. Agricultural land (36.4%) includes both cultivated and fallow fields, where major crops such as castor, bajra, jowar, wheat, groundnut and vegetables are grown. A few patches of pomegranate plantation were also identified and classified under the plantation category. Nadapa Lake, located near the village, along with a small nala at the northern boundary, were mapped under the water-bodies category. Within the block, four China clay processing plants and a small portion of Nadapa village (northern part) were recorded and classified as habitations/infrastructure (7.4%). The residue generated from the China clay processing plants, after separation of the clay and glass-sand fractions, is being disposed of on the surrounding barren lands. Such waste dumps are observed near western part of the area. There is an area of 0.079Sq.km forest land located in the NE corner of the study area is falling under Naliyeri Timbo Reserved Forest. The Land Use/Land Cover map of the study area prepared by CMT falling in T.S. 41E/15, is provided in Fig. No. 3.

Fig No: 3

3.6	Forests, sanctuaries, national park and wild life sanctuaries

Nadapa area covers very small area of 0.079Sq.km under Naliyeri Timbo Reserved Forest in the north-eastern corner of the study area with a variety of trees and plants including neem, banyan, gulmohar, sal, babul, peepal and Aam(mango) The block doesn’t have any sanctuaries, national park and wild life sanctuaries within 10km radius.

3.7	Flora and Fauna within and nearby

Bhuj lies within an arid to semi-arid climatic zone. Its proximity to the Great Rann of Kachchh in the north and the Arabian Sea in the south contributes to a distinct ecological diversity. The dry, hilly terrain supports characteristic vegetation such as peepal, imli, gugal, vad, babul, and various cactus species.

The region hosts a wide range of reptiles, mammals, and aquatic fauna. Common reptiles include the crocodile, spiny-tailed lizard, Kutch rock gecko, black cobra, sand boa, and python. Prominent mammals found in Bhuj and surrounding areas include nilgai, wild boar, chinkara, Indian wolf, and jackal. The Kutch region (Little Raan of Kutch) is also well known for sustaining a significant population of the Indian Wild Ass, a flagship species of the area.

3.8      	Geomorphology

The study area is characterised by a central denudational ridge trending N80°E–S80°W, exhibiting a gentle slope towards the south and a steep slope towards the north. The maximum elevation along the ridge reaches approximately 166 m in the central–western part of the block, gradually decreasing to 142–147 m towards the south and 120–138 m towards the northern flank. The lowest elevation, around 114 m, occurs near the north-western boundary along the nala. Overall, the general slope of the terrain is towards the north and northwest. Central ridge and surrounding pediment areas are covered by rocky outcrops and Acacia trees & other thorny bushes. Agricultural activities are found in the Pediplain areas where crops like castor, bajra, jowar, wheat, groundnut and vegetables are grown. Topographic map of the study area is prepared from SRTM data and is given below in Fig no: 4 and enclosed at plate no: XI

Fig No: 4

3.9	Local infra structure:

Nadapa village is located at the southern boundary of study area. Bhuj is the nearest town and district head-quarter located 35 km towards west. Primary health centre facility is available near Dhaneti village located 8km south of Nadapa.

The Nadapa Area is well connected by road/rail and Air But within the block is accessible by cart tracks/ foot tracks by 4*4 vehicle.

Within the study area, four China clay processing plants are present. These units source China clay from external locations, crush and process the material, and separate it into clay and silica-sand fractions. The remaining residue is disposed of on the adjacent barren lands.

In addition, a cluster of China clay mines and processing plants is located approximately 3.5 km south of Nadapa village (outside the study area), on both sides of the link road connecting Nadapa to SH-42. These small clay mines are located along Gaj formation of Miocene age. Around 10 to 15 operational mines and plants are actively engaged in China clay mining and processing in this area. Prominent operators include:

1. Sandhya Minichem Industries

2. Shree Ram White Clay

3. Sarv Shakti China Clay

4. Kaolin India Pvt. Ltd.

5. Shree Ram Mintech Pvt. Ltd.

6. Shiv Mines

7. Gayatri Kaoline

8.. S.S. China Clay

9. Shree Ram Clay

10. Shree Ram Alumina Products

11. Purusharthi Minerals

12. Umia Minerals

The China clay (kaolinitic clay) produced from this region is widely utilised in ceramics, refractories, paints, pigments, paper, and other industrial applications, reflecting its economic importance.

3.10	Population

Nadapa is a medium size village located in Bhuj Taluka of Kachchh district, Gujarat with total 399 families residing. The Nadapa village has population of 1651 of which 822 are males while 829 are females as per Population Census 2011.

In Nadapa village population of children with age 0-6 is 309 which makes up 18.72 % of total population of village. Average Sex Ratio of Nadapa village is 1009 which is higher than Gujarat state average of 919. Child Sex Ratio for the Nadapa as per census is 1033, higher than Gujarat average of 890.

Nadapa village has lower literacy rate compared to Gujarat. In 2011, literacy rate of Nadapa village was 58.64 % compared to 78.03 % of Gujarat. In Nadapa Male literacy stands at 68.36 % while female literacy rate was 48.96 %. As per constitution of India and Panchyati Raaj Act, Nadapa village is administrated by Sarpanch (Head of Village) who is elected representative of village.

3.11	Socio Demographic profile

Kachchh district of Gujarat is the largest district in India, covering an area of 45,674 sq. km. As per the 2011 Census, the district had a total population of 20,92,371, comprising 10,96,737 males and 9,95,634 females, resulting in a sex ratio of 908 females per 1,000 males. The child sex ratio of the district stands at 921, which is comparatively higher than the overall sex ratio.

The district records a literacy rate of 70.6%. The population density is 46 persons per sq. km, significantly lower than the state average of 308 persons per sq. km, reflecting the sparsely populated nature of the region. About 34.82% of the population resides in urban areas, while the remaining 65.18% lives in rural regions.

The main language spoken in the Kachchh district kachchhi and Gujarati. Other languages are Hindi, Sindhi and Marwari.

3.12	Historical / Archaeological Site

There are no such historical/archaeological monuments present in the Nadapa Area.

CHAPTER – IV

4.0	PREVIOUS WORK

Ghevariya et al mapped the adjoining area of the Nadapa block and delineated Mesozoic rocks belonging to Chari (Jumara), Katrol (Jhuran) and 	Bhuj Formations. As per them, Deccan Trap is represented by interstratified 	volcano sedimentary sequence and lava flows; Tertiary Formations are represented by Madh, Mandaviya and Antarjal Formations and Quaternary is represented by grapestone and Miliolite limestone. B.K. Sahu et al mapped the area during 2004-2005 to study the structural set up, paleo seismicity and geomorphic changes due to 2001 Bhuj earthquake in the area, strip mapping along KMF with a width of 5 	km was done for 300 Sq km on 1:25000 scale

Specialized Thematic Mapping of Inter- Trappean beds of Anjar Volcanics, in an around Anjar, Kachchh District, Gujarat, with Special Investigation on Sedimentology and Vertebrate Palaeontology was carried out by Shreya Basu et al during FS 2021-22. They too reported occurrence of laterite/bauxite in association with lithounits belonging to Matanomadh Formation and Anjar volcanics in the area.

The proposed Nadapa area was not explored previously for bauxite, gallium (Ga), vanadium (V), titanium (Ti), or rare-earth elements (REE) by any exploration Agency. But CGM, Gujarat has investigated in the neighbouring region near Kukma, by pitting, trenching & drilling and reported pocket type of bauxite deposit with an estimated resource of estimated 2342 tons. The results of the chemical analysis of samples show an average Al2O3 to be 51.29% and average SiO2 to be 4.78%.

Geovale Services Pvt. Ltd (an NPEA company) obtained the original KGCMP dataset directly from CGM office and carried out a detailed interpretation. 	Their analysis revealed exceptionally high REE concentrations, with the maximum ∑LREE value of 37,959 ppm and ∑HREE value of 1,965 ppm, based on 182 stream-sediment samples collected within Survey of India Toposheet no 41E/12. These high-grade anomalies are predominantly located along the structurally disturbed Katrol Hill Fault zone and the Median High region located 	south of Bhuj town.

The geochemical anomalies and associated spatial patterns were presented by Geovale in the document titled “Proposal for REE Exploration in Bhuj Clay Prospect Block”, submitted to the National Mineral Exploration Trust (NMET) in August 2023, may be forming a basis for subsequent G4-level exploration for REE in Kachchh area. This could be the reason for giving REE investigation in the present Nadapa area also.

CHAPTER - V

# 5.0	GEOLOGY

5.1	Regional Geology

The proposed Nadapa block forms part of the Kachchh basin, which has been an important site for the deposition of Mesozoic and Cenozoic sediments. These rocks range in age from middle Jurassic to Pliocene. Mesozoics of Kachchh have attained significance on account of its rich fossil record of marine Jurassics in India and are bordered by deccan traps in the south and by saline marsh of the Rann of Kachchh, in the north. The Mesozoic sediments have been classified into Pachham formation of middle Jurassic, the Chari Formation of Middle to late Jurassic, the Katrol Formation of late Jurassic to early Cretaceous and the Bhuj Formation of early Cretaceous period by Waagen et al (1873) subsequently modified by Rajnath (1932), Biswas,ONGC (1971), Krishnan (1982) and Geariya et al,GSI (1984a,1984b) and Ghevariya and Srikarni,GSI (1991) from time to time. The general strike is East-West and with gentle to moderate dipping towards south. SE and SW. Regional geological map of Kachchh basin (part) is given in Fig no:5 and also enclosed on plate no: II.

Pachcham (Jhurio) formation: Pachham comprises of intercalated sequence of siltstone, shale, marl, claystone, Coralline limestone, calcareous sandstone, grey and pink limestone and attained 400m thickness.

Chari (Jumara) formation: Pachhham formation is overlain by rocks of Chari formation. Chari formation is exposed as inliers and lenses along the axis of east-west trending domal anticline ridge (South of Main land fault). They comprise of 350m thick sequence of fossiliferous shale, golden Oolite, fossiliferous limestone, calcareous sandstone, calc and ferruginous nodules

Katrol Formation: Chari formations are overlain by about 400m thick intercalated sequence of gypseous shale with repeated sequence of calcareous sandstone and shales, constituting the Katrol formation.

Bhuj Formation: Bhuj formation conformably overlies the rocks of Katrol and comprises of friable feldspathic, ferruginous sandstone showing graded bedding, cross-bedding, ironstone, clays with many upper Gondwana plant fossils. They have attained a maximum thickness of 1000m in the western mainland Kachchh. They are further divided into Lower & Upper member based on lithology & fossil content.

Deccan Trap: The rocks of Bhuj formation are overlain by Deccan lava flows. Deccan trap is restricted to Kachchh mainland bordering Mesozoic highlands extending from Lakhpat in the west to Anjar in the east. Six major flows have been recorded from late cretaceous to Palaeocene age near Anjar.  Several inter-trappean beds are recorded interstratified with deccan lava flows (Ghevariya &Srikarni, GSI,1987,1990) in Anjar. They attained maximum width of 10km near Anjar and tapering westward. The hard and compact, Deccan trap are be being quarried for road metal in the area near Kukma, Ratnal, Syedpur and Anjar.

Matanomadh Formation: The laterites of the region occur as a narrow, elongated belt running parallel to the Tertiary formations, situated between the underlying Deccan Trap basalts and the overlying younger Tertiary rocks. They are the product of in-situ decomposition of pyroclastic material generated during the Deccan volcanic episode (Shasrabudhe,GSI), which led to the geochemical segregation of silica, alumina, and iron. The laterite horizon is generally 3–4 m thick and consists of porous, pitted, clayey material displaying red, yellow, brown, grey, and mottled colour variations, with considerable textural diversity (P. K. Patel,CGM,Gujarat).

Khari Nadi Formation: Khari Nadi formation overlies the Matanomadh formation and composed of khaki colour gypseous clay with hard marl bands packed with fossils and belongs to oligo-Miocene age.

Gaj Formation: It occurs as an elongated, narrow patch between older Bhuj formation on the north side and younger Sandhan formation on the south. Gaj formation comprises white sandy clay, siltstone and sandstone. Lenticular bodies of white aluminous clays are found at the contact between Mesozoic rocks and Palaeocene rocks at number of places. Such pockets are locally quarried and are being marketed as “China clay”. Prominent China clay deposits are located in south of Nadapa village, north of Ratnal near Ukhamora and Khokra villages and used widely in ceramic, refractory, paints, pigments industries.

Sandhan Formation: Sandhan formation belongs to Pliocene age and comprises of calcarenite, conglomerate, friable sandstone and clay of yellow, pink, brown and variegated colours.

Holocene: Holocene deposits of Kachchh belongs to two categories namely

1.Sediments of the Rann in the north and 2. Coastal mud flats and sandy beaches in the south.

Table 5.1: Generalised stratigraphic succession of Kachchh mainland basin

(after Bhukosh, GSI)

| Age | Group | Sub-group | Formation | Lithology |
| --- | --- | --- | --- | --- |
| Holocene |  |  | Rann | Aeolian sand, clay, marl |
| Pliocene |  |  | Sandhan | Calcarenite, conglomerate, sandstone & clay |
| miocene |  |  | Gaj | Fossiliferous limestone, marl, clay |
| Oligo-miocene |  |  | Khari Nadi | Variegated Gyps shale, siltstone, fossil, marl |
| palaeocene |  |  | Matanomadh | Laterite/ bauxite with clay |
| Late cretaceous-palaeocene | Kachchh | Deccan trap | Anjar volcanics | Basalt (unclassified) |
| Early cretaceous |  |  | Bhuj | Ferruginous, feldspathic sandstone, siltstone, shale, clay, plant fossils |
| Late Jurassic to early cretaceous |  |  | Katrol (Jhuran) | Feldspathic sandstone, shale with ammonite, calcareous sandstone, limestone, siltstone |
| Middle to late jurassic |  |  | Chari (Jumara) | Calc gritty sandstone, gypseous shale, marl, Oolitic Limestone |
| Middle jurassic |  |  | Pachcham (Jhurio) | Shale with micaceous sandstone, limestone |
|  |  |  | Basement not exposed |  |

Fig No: 5

5.2 	Regional Structure:

The Kachchh basin is a western margin peri-cratonic rift basin (basin formed at the boundary of continental crust and oceanic crust) of India bounded by Nagar Parkar uplift in the north and Kathiawar uplift (Saurashtra horst) in the south respectively along Nagar Parkar (NPF) and North Kathiawar faults (NKF) (Fig No: 6). The Radhanpur – Barmer basement arch limits the rift extension to the east. The rift is open to the west, merging with the continental shelf. The graben between them (NPF & NKF) is asymmetric, with a tilt to the south along the North Kathiawar fault accommodating thicker sediments towards the Kathiawar block. The basin is characterised by the development several intra-basinal, sub parallel strike faults forming half grabens. The uplifts are bounded by five parallel faults from north to south. These faults are Nagar Parkar Fault (NPF), Island Belt Fault (IBF), Kachchh Mainland Faul (KMF), Katrol Hill Fault (KHF) and North Kathiawar Faults (NKF). Block tilting along these faults during rift phase extension gave rise to four sub-parallel linear ridges; Nagar Parkar Uplift (NPU), Island Belt Uplift (IBU), Wagad Uplift (WU), and Kachchh Mainland Uplift (KMU). The IBU is broken into four individual uplifts viz. Pachchham (PU), Khadir (KU), Bela (BU) and Chorar (CU), probably by unexposed transverse wrench faults as evidenced by relative displacements and orientations. These uplifts appear as a chain of islands and hence are collectively called “Island Belt”. The KMF marks the northern limit of the Kachchh mainland, beyond this fault Banni plain is present with four islands/uplift and Waged uplift.

The Kachchh rift evolved within the Mid-Proterozoic-Aravalli-Delhi fold belt by reactivation of pre-existing faults along the NE-SW trend of the Delhi fold belt that swings to E-W in Kachchh region. The Kachchh rift was initiated during the Late Triassic breakup of the Gondwanaland. The rifting was aborted during Late Cretaceous pre-collision stage of the Indian plate. During post-collision compressive regime of the Indian plate, the Kachchh rift basin became a shear zone with strike-slip movements along sub-parallel rift faults. The Kachchh Mainland Fault along the rift axis became the active principal fault. All these faults determine the instability of this area and is falling under Zone-V under tectonic zonation map of India (B.K. Sahu, P.K. Singh, GSI, progress report for the FS: 2004-05).

Fig No: 6 Tectonic map of Kachchh (after Shukla, 2011)

5.3	Metamorphism: The area comprises sedimentary rocks only and regional metamorphism is absent in the study area. The litho-units have not undergone significant metamorphic recrystallization.

5.4	Host rock for mineralisation: The Nadapa Area forms a part of the Kachchh Mainland Basin and is represented by two distinct formations—an older Katrol Formation (Late Jurassic to Early Cretaceous) overlain by the younger Bhuj Formation (Early Cretaceous).

Both these formations are devoid of any bauxite enrichment. Furthermore, the      Matanomadh (Madh) Formation, which hosts bauxite/laterite in nearby areas, is completely absent in the present study area.

The Katrol Formation comprises compact, yellowish-brown, fine- to medium-grained calcareous sandstone with shale/siltstone intercalations. The Bhuj Formation consists of brown to reddish, yellowish friable feldspathic sandstone interbedded with laminated siltstone/shale, thin ironstone bands, and grey clay beds that contain plant fossils of the Gondwana Group.

Based on the geochemical analysis of 90 samples collected from pits and trenches within the investigated area, the concentrations of bauxite (Al₂O₃), gallium (Ga), vanadium (V), titanium (TiO₂), and REEs are found to be low and below the threshold required for economic consideration. The obtained values do not indicate any significant enrichment or mineralization of these commodities. Hence, the area does not hold promising potential for bauxite, Ga, V, Ti, or REE mineralization at the explored scale.

Ternary diagram plotting of Al₂O₃–SiO₂–Fe₂O₃ further reveals that all samples     fall within the “bauxitic clay” field rather than the true bauxite domain, confirming that the material represents argillaceous sediments enriched in alumina-bearing clays, not of economic bauxite. Thus, despite localized Al₂O₃ enrichment in clay-rich zones, the absence of low silica and lack of laterite development indicate that the conditions necessary for bauxite formation were not attained in the study area.

CHAPTER – VI

6.0	Activity during the period (Geoscience investigation)

As per NQT Critical Mineral Trackers has undertaken the following activities in an area of 6.20 Sq. Km in Nadapa area, Kachchh district, Gujarat from 14 Feb 2025 and completed all field investigations by 30th April,2025.

- Large scale geological mapping on 1:12500 scale

- Pitting and Trenching

- Sample collection & Preparation

- Chemical analysis

- Complete Petrography study

6.1	Large scale Geological mapping (1:12,500 scale)

Large scale Geological mapping (LSM) completed in the total area of 6.20 sq. km on 1:12500 scale in the part of Survey of India Toposheet No 41E/15. Survey of India toposheet was used to understand the topography, roads, and drainage of the mapped area and was also used as a base map for the LSM.

For recording precise sample location and to carry out a geological survey, handheld GPS (Garmin 12 H model) has been used. The coordinates had been recorded in geographic coordinate system with WGS 1984 datum. Variation of lithologies along with the structural elements were systematically recorded and prepared the final interpreted geological map is enclosed in Plate No-III, and also given in Fig No:7. Out crop map on 1:12500 scale of the study area is also prepared and enclosed in Plate No-IV and also given in Fig no: 8

Different lithological units belonging to the Bhuj and Katrol formations were systematically documented. These include ferruginous gritty sandstones, fine-grained friable sandstones with mica, siltstones, shales, and grey clay horizons. The attitude of beds was measured using a Brunton compass, and several primary sedimentary structures such as cross-bedding, ripple marks, burrows, ironstone concretions, and clay layers containing plant fossil assemblages were recorded. These features provide insights into the palaeo-environmental conditions and depositional processes operating during the formation of these units. A detailed description of the lithological assemblages has been given the subsequent paragraphs.

| Time scale | Formation | Lithology |
| --- | --- | --- |
| Early cretaceous | Bhuj formation | Feldspathic sandstone and gritty Ferruginous, feldspathic sandstone, siltstone with plant fossil, Sandstone shale, conglomerate |
| Late Jurassic to early cretaceous | Katrol Formation | Feldspathic sandstone, shale with ammonite fossils. Fossiliferous calc sandstone, limestone, siltstone Gypseous shale, Calc sandstone with belemnites |

Table 6.1 The generalized stratigraphic succession of Nadapa area

6.1.1	Description of lithology

Katrol Formation: The Katrol Formation in the study area comprises predominantly of pinkish brown hard ferruginous sandstone, yellowish-white siltstones, yellowish-brown micaceous shales, and yellowish to pinkish calcareous sandstones. The sandstones are typically calcareous and often feldspathic, reflecting a moderate degree of chemical and mechanical weathering of the source area.

A major part of the terrain is covered by soil, though excellent exposures occur along nala sections and road cuttings. Compared to the overlying Bhuj Formation, the Katrol rocks are more compact and moderately hard, which makes them locally suitable for building-stone purposes. Intercalations of sandstone and siltstone are common throughout the sequence. These units generally lack macrofossils, and the bedding architecture often displays lenticular bedding, Ripple marks and Mud cracks. These features indicate deposition under shallow marine environment. Structurally, the formation in the study area exhibits a NE–SW strike, with beds dipping 5°–10° towards the southeast, reflecting regional tilting within the Kachchh Mainland.

Description of various lithological units encountered in Katrol formation

with field photographs

| Photo - 1: Near northern boundary of the block. Interbedded sequence of yellowish fine gained sandstone & yellowish-brown siltstone/shale exposed along nala cutting. Fairly hard & compact. Katrol formation. | Photo 2: Near northern boundary along steep gorge (1.5km north of Nadapa village). Yellowish-white fine-grained sandstone with iron concretions. General strike is NE-SW and dipping 6° southeasterly. Katrol Formation. |
| --- | --- |
| Photo 3: NW of the block. There is a narrow and steep gorge of 0.5-2.0m wide & 8-10m height cutting across both Bhuj Formation on the south (gritty sandstone pinkish/yellowish) and Katrol Formation on the north (fine grained yellow calcareous sandstone) with gradational contact. The gorge might be formed due to eroding away of soft siltstones/ clays, leaving hard resistant sandstones on either side as steep walls. Katrol Formation & Bhuj Formation |  |

Bhuj Formation: The Bhuj Formation overlies the Katrol Formation and occurs predominantly toward the southern part of the Nadapa area. In the study area, it is prominently represented by a central ridge trending N80°E–S80°W, with strata dipping 5–10° southward. The ridge shows a gentle southerly slope and a relatively steeper northern slope, reflecting asymmetrical structural geometry. The ridge comprises hard, resistant brown ferruginous sandstone, capped at places by a thin lateritic cover (10–20 cm). The basal portions consist of intercalated sandstone, siltstone, and clay beds.

To the south of the ridge, the lithology includes: Ferruginous sandstone, Brown splintery shale, Yellowish-white friable siltstone, thin brown ironstone bands and grey clay horizons containing plant fossil impressions at the lower levels.

To the north of the ridge, the lithology is dominated by: Friable yellowish to pinkish-white fine- to medium-grained feldspathic sandstone/siltstone, often mica-bearing. The observed lithological variability, together with well-preserved primary sedimentary structures such as cross-bedding, ripple marks, burrows indicates that the Bhuj Formation was deposited in a fluvial to deltaic depositional environment, representing shifting channel–bar complexes, floodplain sediments, and marginal deltaic conditions.

Photo 4: 1.3km NE of Nadapa village: section across the central ridge showing thin lateritic cover on the top. Interbedded feldspathic sandstone/siltstone with thin laminae of shale and clay at the bottom. Strike: N80°E-S80°W and dipping 6 °southerly

| Photo 5: 2.0km NE of Nadapa village: Typical section along south side of the ridge. Splintery shale on the top followed by interbedded siltstone/ shale laminae in the middle and grey clay bed at bottom. Strike: N45°E-S45°W dipping 5° south-easterly | Photo 6: 2.2km NE of Nadapa village: Typical topography of pediment area lying south of the ridge represented by ferruginous sandstone, splintery shale (due to joints) on the top followed by friable sandstone |
| --- | --- |
| Photo 7: 2.30km east of Nadapa village. Hard, compact ferruginous gritty sandstone with lenses of soft clays (eroded away) now leaving void as big as 0.5 to 1m diameter. | Photo 8: 1.0 km north of Nadapa village. North side of the ridge. Very fine to fine grained, laminated, friable sandstones interbedded with kaolinized clay and thin shale laminae |

Photo 9: 1.5km north of Nadapa village. Good section of strata is exposed along big nala located near northern boundary. Fine grained sandstone interbedded with siltstone/ clay/shale laminae.

Interpreted large scale geological map of Nadapa Area (on 1:12500 scale) is given         below in Fig no: 7 and also enclosed on plate no: III

Outcrop map   of the study area is given below in Fig no: 8 and also enclosed in plate no:IV.( on 1:12500 scale)

Fig No: 7

Fig No: 8

6.1.2	Description of rock types in outcrop map: There are quite good number of outcrops have been exposed in nearly 50% of the study area along the central ridge portion and the adjoining pediment area on either side. Few outcrops are also noticed along the nalas in the NW part and NE part. These outcrops have been systematically demarcated and prepared an outcrop map on 1:12,500 scale and enclosed in plate no: IV and also in Fig no:8  .

Brief description of rock types found in outcrops is given below:

The Formations in the study area comprises predominantly reddish brown, hard ferruginous sandstone, yellowish-white siltstones, yellowish-brown micaceous shales/sandstone, and yellowish to pinkish calcareous sandstones, with occasional clay beds. The litho-units are gradational in nature and often intercalated with one another.

- Hard Ferruginous sandstone: The rocks exposed along the central ridge portion of the study area are composed of pinkish brown coarse to gritty ferruginous feldspathic sandstones on the top followed by yellowish white fine-grained argillaceous sandstones interbedded with thin laminae of brown shales & siltstones and white clay laminae. At few places there is a thin lateritic capping of 10-20cm thick on the top. The feldspar if often kaolinized.

- Ferruginous Sandstone, splintery shales, siltstone & clay: South-eastern part of the study area is occupied by coarse grained Ferruginous sandstone, brown splintery shales (spread on ground as broken fragments due to joints) and siltstones on the top followed by grey clay beds. The siltstones and clays often contain plant fossils.

- Splintery shales with ironstone concretions: In a small area towards south exposed brown splintery shales with ironstone concretions on the top followed by grey clay beds.

- Calcareous sandstone and micaceous shale: Fine to medium grained calcareous sandstone interbedded with shale and clay laminae are exposed along the nalas in the NE part. Sandstones are fairly hard and compact.

- Feldspathic gritty sandstone, siltstone, clay laminae: The outcrops exposed along NW part of the study area along big nala on either side are composed of coarse grained to gritty feldspathic pinkish brown sandstones, fine grained yellowish white friable sandstone interbedded with white siltstone and clay formations of Bhuj. Fine to medium grained yellowish white calcareous sandstone interbedded with siltstone/ gypseous shales of Katrol formation

    - Petrographic studies: Ten rock specimens have been sent to petrology division, GSI, southern region, Hyderabad for preparation of thin sections and complete petrographic studies. But they have taken up petrographic studies on 3 specimens only and discarded 7 samples due to soft, friable nature and not suitable for thin section preparation.

Detailed description of the 3 specimens is as follows:

  - Specimen no: NB/TS/P-14

Photo 10 & 11: medium grained sandstone specimen &

corresponding photomicrograph

Microscopic study reveals that the rock displays a medium- to coarse-grained texture, with grain diameters ranging from approximately 250 to 750 micrometers. Grains are poorly sorted, and both angular and subrounded quartz grains coexist, which together indicate rapid deposition or minimal transport. Grain boundaries are irregular, and the grains commonly exhibit point and long contacts, consistent with mechanical compaction during burial. It predominantly shows quartz grains (Qz), which are abundant.

Quartz is present as sub-rounded to angular grains, indicating moderate transport prior to deposition. The presence of plagioclase feldspar (Plg) is signifying a minor but crucial feldspathic component, possibly derived from nearby igneous or metamorphic source rocks. Biotite (Bt) is observed. Its occurrence, though infrequent (Fig. 1A-1D). The study reveals a minor matrix in the form of fine material occupying the intergranular spaces. The degree of visible cementation appears limited, with quartz overgrowths not clearly observed, suggesting that grain-to-grain contacts dominate over load-supporting cement. The matrix may include clay or altered feldspar, contributing to a minor muddy texture. Overall, the study illustrates a quartz-rich, subarkosic sandstone, characterized by poor sorting and angular grains. Such features are indicative of fluvial or proximal alluvial depositional environments, with limited transport and moderate source rock diversity.

Rock/Mineral Name: Based on the mineral and textural characteristics, it is a Sandstone.

- Specimen no: NB/TS/T5

Photo 12 & 13, fine-grained sandstone specimen and  corresponding

photomicrograph

Microscopic study reveals that the rock exhibits a medium to fine grain size.The grains display angular to subangular outlines, indicating limited ttransport and deposition relatively close to their source .Poor sorting is observed with variable grain sizes and shapes, suggesting rapid sedementation possibly by fluvial process or short transport mechanism

Quartz (Qz) is overwhelmingly the dominant mineral phase, showcased by its abundance across rock. The presence of multiple quartz grains, with some displaying undulatory extinction typical of detrital quartz. Plagioclase feldspar (Plg) appears minorly and is identifiable by its characteristic polysynthetic twinning, which suggests an input from igneous or metamorphic source terrain. Additionally, microcline (Mc) is noted, recognized by its cross-hatch twinning, reflecting a granitic provenance. Biotite (Bt) is present as elongate laths and contributes accessory (Fig. 2A-2D). The dark areas in the photomicrographs correspond to matrix material, likely composed of fine-grained clay or altered rock fragments, signifying a minor but present muddy fraction. Visible evidence of cementation is limited, with intergranular contacts prevailing, indicating a predominately clastic-supported framework with minimal authigenic growth.

The dominant quartz content, combined with the presence of feldspar (both plagioclase and microcline) and biotite, classifies the sandstone as sub-arkosic to arkosic, suggesting a mixed provenance from felsic igneous and accessory metamorphic sources. Its textural immaturity and mineral diversity indicate deposition close to uplifted and un-weathered continental or metamorphic-igneous terrains, most likely within fluvial or proximal alluvial environments where rapid sediment supply and limited reworking dominate.

Rock/Mineral Name: Based on the mineral and textural characteristics, it is a Sandstone

- Specimen no: NB/TS/P6

(As received from petrology laboratory, GSI, southern wing, Hyderabad.)

|  | Fig. 3.1 Angular to subangular quartz grains and few micas as detrital material cemented by ferruginous material. | Fig. 3.2 Ferrugenous cement dominated part where angular to subangular quartz grains floated in cement |
| --- | --- | --- |
| Fig. 3.3 Presence of rock fragment and feldspar indicate less weathering and minimal transport | Fig.3.4 Microcline (Mc) and plagioclase (Pl) as frame working mineral indicating igneous provenance. | Fig. 3.5 Feldspar and quartz under X polarized light |

| Fig. 3.6 Cherty fragment in quartzose frame working grains | Fig. 3.7 Magnetite as detrital mineral; ferruginous cement; under reflected light | Fig. 3.8 ferruginous and magnetite rich cement; under reflected light |
| --- | --- | --- |
| Fig. 3.9 Ferruginous cement and the angular quartz grains; under reflected light | Fig. 3.10 Hematite and magnetite as cementing material; detrital magnetite grains along with angular quartz grains |

Photo 14 & 15 : Ferruginous sandstone specimen and  corresponding photomicrograph

Mineral Assemblage: Quartz + Feldspar + Mica + Rock fragment +    Cherty fragment as framework mineral

Cement: Hematite and magnetite,

Name of the rock: Ferruginous sandstone (Bhuj Formation)

Texture: Microscopic study reveals that the rock displays a medium- to coarse-grained texture, with grain diameters ranging from approximately 250 to 750 micrometers. Grains are poorly sorted and are angular to subangular in shape indicating rapid deposition and minimal transport. Grain boundaries are irregular, and the grains commonly exhibit point and long contacts, consistent with mechanical compaction during burial. The rock predominantly consists of quartz grains as framework minerals. Quartz grains are mostly angular, indicating moderate transport prior to deposition. The presence of plagioclase feldspar and microcline is suggestive of igneous source rocks and minimum transportation. Few Mica grains are also present. Two rock fragments of varying size are noted viz., cherty rock fragment and quartzite fragment both indicating metasedimentary provenance. Undulose extinction in quartzite rock fragment also suggest similar provenance.

The rock is extremely cemented by ferruginous material such as hematite and magnetite. At places detrital frame working grains are volumetrically much less than the ferruginous cement and grains are not in contact. Few magnetite occurs as detrital frame working grains.

6.1.4	Structures

The two lithological units present in the study area, namely the Katrol Formation and the Bhuj Formation, exhibit a regional structural trend of N80°E–S80°W, with local variations in NE–SW orientation. The strata generally dip 5° to 10° towards the south and southeast.

The Bhuj Formation shows comparatively steeper dips ranging from 6° to 10°, whereas the older Katrol Formation exhibits gentler dips between 4° and 6°.

Two sets of sub-vertical joins are observed in brown shales towards south. The general trend of Joints is N60°E-S 60°W and N40°W-S40°E. Due to this jointing pattern, the shales were broken into pieces and occur as splintery on the surface

Based on satellite imagery interpretation, six inferred minor faults have been demarcated within the ridge area. The trend of these faults is in NE-SW direction. Minor displacements are observed along these inferred fault traces; however, no clear or recognizable fault-related features viz. fault breccia, silicification etc were identified during field investigations and shown in plate no: III & Fig no:7

The observed lithological variability, together with well-preserved primary sedimentary structures such as cross-bedding, lamination, ripple marks, burrows and plant fossils assemblage indicate that the Bhuj Formation was deposited in a fluvial to deltaic depositional environment.

In contrast, the Katrol formation composed of calcareous sandstones/ siltstones lack of macrofossils, and presence of primary structures viz bedding, Ripple marks, Mud crack indicate that the Katrol formation was deposited under shallow marine environment.

Detailed descriptions of sedimentary structures observed in Katrol and Bhuj formation along with photographic documentation is given below

Photo 16: 1.35 km North side of Nadapa village showing Ripple marks on Yellowish brown shale. Katrol Formation

Photo- 17: 2.28 km NE side of Nadapa village showing Burrows and ironstone concretions within the sandstone. Katrol Formation

| Photo 18: 0.97km North side of Nadapa village showing Cross bedding in fine grained laminated sandstone. Bhuj Formation | Photo 19: 0.45km near Nadapa village showing Ripple marks in siltstoneBhuj Formation |
| --- | --- |
| Photo 20 & 21: 1.0 km NNE of Nadapa village showing burrow structures in the top ferruginous sandstone & lateritic Bhuj formation |  |

| Photo 22: 1.79 km ENE side of Nadapa village plant fossil (Palmoxylin?) in the shales. 2 sets of joints N60°E-S 60°W and N40°W-S40°E found in Bhuj Formation | Photo 24: 1.54 km NE side of Nadapa Village plant fossil Ptylophyllum within grey clay beds of Bhuj Formation |
| --- | --- |
| Photo 23: 1.73 km ENE side of Nadapa Village Plant fossils within siltstone. Bhuj Formation |  |

6.1.5	Metamorphism: The area comprises sedimentary rocks only and regional metamorphism is absent in the study area. The litho-units have not undergone significant metamorphic recrystallization.

6.1.6 	Mineralogy of ore zones and ore textures: During large scale geological mapping, no signatures of Bauxite mineralisation are found on the surface because the bauxite bearing Matanomadh formation is completely absent in the study area.  Further the geochemical analysis of the samples collected from pits and trenches within the investigated area, the concentrations of bauxite (Al₂O₃), gallium (Ga), vanadium (V), titanium (TiO₂), and REEs are found to be low and below the threshold required for economic consideration. The obtained values do not indicate any significant enrichment or mineralization of these commodities. Hence, the area does not hold promising potential for bauxite, Ga, V, Ti, or REE mineralization at the explored scale. Hence, no ore zones are demarcated.

6.1.7 	Pitting and Trenching: After completion of Large-scale geological mapping, Critical mineral trackers has taken up pitting and trenching work and completed 70 pits and 5 trenches during April, 2025.The map showing the locations of pit and trench is enclosed in plate no: V and also given in Fig no: 9

6.1.7.1 Pitting

Pitting during the present exploration work was undertaken to obtain fresh representative samples from a depth of 1 meter. A total of 20 pits were excavated using a JCB, each measuring 1.0 m × 1.0 m × 1.0 m, resulting in an individual volume of 1.0 m³ and a cumulative excavated volume of 20 m³. The pits provided fresh exposures of subsurface lithology and allowed for systematic sampling for subsequent geochemical analysis. Lithological map of pits on 1:100 scale is enclosed in plate no: VII and Assay plan of pits on 1:100 scale is shown in plant no: IX. Analytical results of major oxides, Vanadium, and REE, Gallium of pits are given in Annexure: II, III, IV

Table No: 6.2 The location of Pits (measured by GPS)

| Sr No | Pit No | Coordinate in Degree Decimal Datum WGS-1984 |  | No of samples collected |
| --- | --- | --- | --- | --- |
|  |  | Latitude | Longitude |  |
| 1 | P1/NB/2025 | 23.324488 | 69.917695 | 1 |
| 2 | P2/NB/2025 | 23.323288 | 69.923733 | 1 |
| 3 | P3/NB/2025 | 23.326912 | 69.92718 | 1 |
| 4 | P4/NB/2025 | 23.328038 | 69.928047 | 1 |
| 5 | P5/NB/2025 | 23.32558 | 69.906665 | 1 |
| 6 | P6/NB/2025 | 23.328445 | 69.90643 | 1 |
| 7 | P7/NB/2025 | 23.328882 | 69.910428 | 1 |
| 8 | P8/NB/2025 | 23.32948 | 69.914093 | 1 |
| 9 | P9/NB/2025 | 23.329812 | 69.916072 | 1 |
| 10 | P10/NB/2025 | 23.331522 | 69.918707 | 1 |
| 11 | P11/NB/2025 | 23.339875 | 69.925592 | 1 |
| 12 | P12/NB/2025 | 23.334688 | 69.912072 | 1 |
| 13 | P13/NB/2025 | 23.330597 | 69.904402 | 1 |
| 14 | P14/NB/2025 | 23.321163 | 69.910602 | 1 |
| 15 | P15/NB/2025 | 23.329158 | 69.917623 | 1 |
| 16 | P16/NB/2025 | 23.333752 | 69.916712 | 1 |
| 17 | P17/NB/2025 | 23.322903 | 69.91055 | 1 |
| 18 | P18/NB/2025 | 23.33154 | 69.902135 | 1 |
| 19 | P19/NB/2025 | 23.32382 | 69.90106 | 1 |
| 20 | P20/NB/2025 | 23.32283 | 69.90741 | 1 |

Fig No: 9

Photo No.-25: Image showing Pit in the study area

(Size 1*1*1m)

Brief description of pit profile (lithology) is given below for 20 pits

(not to scale)

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 16/04/2025 |
| --- | --- |
| Pit no: P1/NB/2025 | Date of closer: 16/04/2025 |
| Location: 23.324488°N 69.917695°E | Elevation: 130m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & B. Mahesh, Geologists |  |
| Lithology details: Argil. SST Yellowish white, Clay arenaceous Pinkish white & Dark grey Clay (Bhuj formation). |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P1/NB/2025 Representative sample was collected from bottom E of Dark grey Clay. |  |

Detailed Lithology of Pit no: P1/NB/2025

(not to scale)

Detailed Lithology of Pit no: P2/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 16/04/2025 |
| --- | --- |
| Pit no: P2/NB/2025 | Date of closer: 16/04/2025 |
| Location: 23.323288°N 69.923733°E | Elevation: 121m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & B. Mahesh, Geologists |  |
| Lithology details: Grey clay, Ferruginous SST. Reddish brown (Bhuj formation). |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P2/NB/2025, Representative sample was collected from bottom E of ferruginous sandstone. |  |

(not to scale)

Detailed Lithology of Pit no: P3/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 16/04/2025 |
| --- | --- |
| Pit no: P3/NB/2025 | Date of closer: 16/04/2025 |
| Location: 23.326912°N 69.927180°E | Elevation: 124m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & B. Mahesh, Geologists |  |
| Lithology details: Ferruginous SST., SST argil, Dark grey clay (Bhuj formation). |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P3/NB/2025, representative sample was collected from bottom E of Dark grey Clay. |  |

(not to scale)

Detailed Lithology of Pit no: P4/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 16/04/2025 |
| --- | --- |
| Pit no: P4/NB/2025 | Date of closer: 16/04/2025 |
| Location: 23.328038°N 69.928047°E | Elevation: 124m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & B. Mahesh, Geologists |  |
| Lithology details: Clay stone with mica, Dark grey clay with micaceous interbedded with pink clay bands (Bhuj formation). |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P4/NB/2025, representative sample was collected from bottom E of Dark grey Clay with mica. |  |

(not to scale)

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 17/04/2025 |
| --- | --- |
| Pit no: P5/NB/2025 | Date of closer: 17/04/2025 |
| Location: 23.325580°N 69.906665°E | Elevation: 146m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & B. Mahesh, Geologists |  |
| Lithology details: Ferruginous SST., Fine to medium grained SST micaceous yellow (Bhuj formation). |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P5/NB/2025, representative sample was collected from bottom E of fine to medium grained Sandstone |  |

Detailed Lithology of Pit no: P5/NB/2025

(not to scale)

Detailed Lithology of Pit no: P6/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 17/04/2025 |
| --- | --- |
| Pit no: P6/NB/2025 | Date of closer: 17/04/2025 |
| Location: 23.328445°N 69.906430°E | Elevation: 126m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & B. Mahesh, Geologists |  |
| Lithology details: Fine grained sandstone Y/W, SST. Fine grained with iron concretions, SST. Argillaceous whitish pink. (Bhuj formation). |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P6/NB/2025, Representative sample was collected from bottom E of Argillaceous sandstone. |  |

(not to scale)

Detailed Lithology of Pit no: P7/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 17/04/2025 |
| --- | --- |
| Pit no: P7/NB/2025 | Date of closer: 17/04/2025 |
| Location: 23.328882°N 69.910428°E | Elevation: 125m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & B. Mahesh, Geologists |  |
| Lithology details: Fine grained sandstone yellowish white, Siltstone yellowish white friable (Bhuj formation). |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P7/NB/2025 representative sample was collected from bottom E of siltstone yellowish white |  |

(not to scale)

Detailed Lithology of Pit no: P8/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 17/04/2025 |
| --- | --- |
| Pit no: P8/NB/2025 | Date of closer: 17/04/2025 |
| Location: 23.329480°N 69.914093°E | Elevation: 125m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & B. Mahesh, Geologists |  |
| Lithology details: argillaceous sandstone pinkish white friable, very fine-grained sandstone with mica whitish yellow & friable (Bhuj formation). |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P8/NB/2025 Representative sample was collected from bottom E of very fine-grained sandstone. |  |

(not to scale)

Detailed Lithology of Pit no: P9/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 17/04/2025 |
| --- | --- |
| Pit no: P9/NB/2025 | Date of closer: 17/04/2025 |
| Location: 23.329812°N 69.916072°E | Elevation: 122m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & B. Mahesh, Geologists |  |
| Lithology details: Siltstone yellow friable, Siltstone whitish pink friable (Bhuj formation). |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P9/NB/2025, representative sample was collected from bottom E of siltstone pinkish white |  |

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 17/04/2025 |
| --- | --- |
| Pit no: P10/NB/2025 | Date of closer: 17/04/2025 |
| Location: 23.331522°N 69.918707°Eg | Elevation: 116m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & B. Mahesh, Geologists |  |
| Lithology details: Siltstone yellow, siltstone Variegated with mica (Bhuj formation). |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P10/NB/2025, representative sample was collected from bottom E of siltstone variegated |  |

(not to scale)

Detailed Lithology of Pit no: P10/NB/2025

(not to scale)

Detailed Lithology of Pit no: P11/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 18/04/2025 |
| --- | --- |
| Pit no: P11/NB/2025 | Date of closer: 18/04/2025 |
| Location: 23.339875°N 69.925592°E | Elevation: 110m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & Shiva Kumar Geologists |  |
| Lithology details: SST. fine grained argil. with mica laminae & Calcareous in nature (Katrol formation). |  |
| Structural details Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P11/NB/2025, representative sample was collected from bottom E of fine-grained argillaceous sandstone with mica |  |

(Not to scale)

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 19/04/2025 |
| --- | --- |
| Pit no: P12/NB/2025 | Date of closer: 19/04/2025 |
| Location: 23.334688°N 69.912072°E | Elevation: 114m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & Shiva Kumar, Geologists |  |
| Lithology details: Shale yellow interbedded with pink SST, fine grained laminated sandstone, Fine grained argillaceous sandstone Yellowish white with grey clay intercalations. Calcareous in nature (Katrol formation) |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P12/NB/2025, representative sample was collected from bottom E of fine-grained argillaceous sandstone |  |

Detailed Lithology of Pit no: P12/NB/2025

(not to scale)

Detailed Lithology of Pit no: P13/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 19/04/2025 |
| --- | --- |
| Pit no: P13/NB/2025 | Date of closer: 19/04/2025 |
| Location: 23.330597°N 69.904402°E | Elevation: 107m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & V. Shiva Kumar, Geologists |  |
| Lithology details: Siltstone pinkish, Dark grey clay interbedded with siltstone yellowish pink (Bhuj formation) |  |
| Structural details: Bedding. |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P13/NB/2025, representative sample was collected from bottom E of grey Clay interbedded with siltstone |  |

(not to scale)

Detailed Lithology of Pit no: P14/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 20/04/2025 |
| --- | --- |
| Pit no: P14/NB/2025 | Date of closer: 20/04/2025 |
| Location: 23.321163°N 69.910602°E | Elevation: 127m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & V.S. Kumar, Geologists |  |
| Lithology details: Medium to coarse grained feldspathic sandstone yellowish white. |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P14/NB/2025, representative sample was collected from bottom E of feldspathic sandstone (Bhuj formation) |  |

(not to scale)

Detailed Lithology of Pit no: P15/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 20/04/2025 |
| --- | --- |
| Pit no: P15/NB/2025 | Date of closer: 20/04/2025 |
| Location: 23.329158°N 69.917623°E | Elevation: 127m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & Dr. G. Ravi Kumar, Geologists |  |
| Lithology details: Grey clay interbedded with siltstone and plant fossils (Ptilophyllum). |  |
| Structural details: Bedding. |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P15/NB/2025, representative sample was collected from bottom E of grey clay with siltstone & plant fossils (Ptilophyllum)-Bhuj formation |  |

(not to scale)

Detailed Lithology of Pit no: P16/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 20/04/2025 |
| --- | --- |
| Pit no: P16/NB/2025 | Date of closer: 20/04/2025 |
| Location: 23.333752°N 69.916712°E | Elevation: 120m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & Dr. G. Ravi Kumar, Geologists |  |
| Lithology details: Sandy soil Calcareous in nature (Katrol formation). |  |
| Structural details: Bedding. |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P16/NB/2025, representative sample was collected from bottom E of sandy soil calcareous |  |

(not to scale)

Detailed Lithology of Pit no: P17/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 20/04/2025 |
| --- | --- |
| Pit no: P17/NB/2025 | Date of closer: 16/04/2025 |
| Location: 23.322903°N 69.910550°E | Elevation: 183m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & B. Mahesh, Geologists |  |
| Lithology details: Pinkish grey clay with plant fossils & siltstone hard ferruginous yellowish brown (Bhuj formation). |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P17/NB/2025, representative sample was collected from bottom E of siltstone ferruginous |  |

(not to scale)

Detailed Lithology of Pit no: P18/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 21/04/2025 |
| --- | --- |
| Pit no: P18/NB/2025 | Date of closer: 20/04/2025 |
| Location: 23.331540°N 69.902135°E | Elevation: 93m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & Dr. G. Ravi Kumar, Geologists |  |
| Lithology details: Fine grained yellowish white sandstone, white argillaceous SST., fine grained argillaceous sandstone Pinkish white (Bhuj formation) |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P18/NB/2025, representative sample was collected from bottom E of fine-grained sandstone |  |

(not to scale)

Detailed Lithology of Pit no: P19/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 22/04/2025 |
| --- | --- |
| Pit no: P19/NB/2025 | Date of closer: 22/04/2025 |
| Location: 23.32382°N 69.90106°E | Elevation: 141m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & Dr. G. Ravi Kumar, Geologists |  |
| Lithology details: Yellowish brown fine-grained ferruginous SST., hard and compact (Bhuj formation) |  |
| Structural details: Bedding. |  |
| Log of pit: A, B, C, D are side section and E plan view is bottom of the pit. |  |
| Sample no: P19/NB/2025, representative sample was collected from bottom E of fine to medium grained ferruginous sandstone. |  |

(not to scale)

Detailed Log of Pit no: P20/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti & REE. | Date of start: 22/04/2025 |
| --- | --- |
| Pit no: P20/NB/2025 | Date of closer: 22/04/2025 |
| Location: 23.32283°N 69.90741°E | Elevation: 134m |
| Pit top measurement: a. Length: 1m b. Breadth: 1m | Pit bottom measurement: a. Length: 1m b. Breadth: 1m c. Depth: 1m |
| Recorded by: V. Santosh & Dr G. Ravi Kumar, Geologists |  |
| Lithology details: Grey clay with thin SST. Laminae. |  |
| Structural details: Bedding & lamination |  |
| Log of pit: A, B, C, D are side sections, and E is the plan view is bottom of the pit. |  |
| Sample no: P20/NB/2025, representative sample was collected from bottom E of grey clay with sandstone laminae |  |

6.1.7.2 Trenching

A total of five trenches were excavated in the study area, each measuring 10m in length, 1m in width, and 1m in depth, amounting to a cumulative excavated volume of 50 m³. After excavation, the trench walls and floor were thoroughly cleaned to expose fresh, undisturbed lithology. From each trench, 10 representative channel samples (marked as T3/NB/2025/1 to 10) were collected systematically from the floor (E) at 1-metre intervals. In addition, four wall samples were obtained—one from each exposed face of the trench—labelled as A, B, C, and D (marked as T3/NB/2025/A, B, C, and D). This sampling approach ensured comprehensive coverage of both lateral and vertical lithological variations within the trench.

Lithological map of trenches on 1:100 scale is enclosed in plate no: VIII and Assay plan of trenches on 1:100 scale is shown in plant no: X. Analytical results of major oxides and REE, Ga of trenches are given in Annexure no: II, III,IV

Table No:  6.3 The location of Pits (measured by GPS)

| Sr No | Trench No | Coordinate in Degree Decimal Datum WGS-1984 |  | No of Samples Collected |
| --- | --- | --- | --- | --- |
|  |  | Latitude | Longitude |  |
| 1 | T1/NB/2025 | 23.335140 | 69.924022 | 14 |
| 2 | T2/NB/2025 | 23.331850 | 69.907867 | 14 |
| 3 | T3/NB/2025 | 23.328870 | 69.901747 | 14 |
| 4 | T4/NB/2025 | 23.336027 | 69.912690 | 14 |
| 5 | T5/NB/2025 | 23.323207 | 69.904923 | 14 |

Photo-26: Image showing trench in the study area (Size 10*1*1m)

Brief description of trench profiles is given below for 5 trenches

(Not to scale)

Detailed Lithology of Trench No: T1/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti, REE | Date of commencement: 18/04/2025 |
| --- | --- |
| Trench No: T1/NB/2025 | Date of completion: 18/04/2025 |
| Location: 23.335140°N, 69.924022°E | Elevation: 165m |
| a. Length: 10m b. Breadth: 1m c. Depth: 1m volume: (10Cu.m) |  |
| Recorded by: V. Santosh and B. Mahesh |  |
| Lithology details: yellowish white sandstone argillaceous fine grained, siltstone with mica laminae (Bhuj formation) |  |
| Structural details: bedding and lamination |  |
| Log of Trench: Side walls represented by north by A, east by B, South by C and west by D and bottom of the pit by E. long axis direction of trench is marked. |  |
| Samples: 10 representative samples were collected from bottom of E at 1m interval (T1/NB/2025/01 to T1/NB/2025/10 and 4 samples from four side walls. (T1/NB/2025-A, B, C, D respectively). Total 14 samples were collected from this trench. |  |

| Name of the investigation: Bauxite, Ga, V, Ti, REE | Date of commencement: 19/04/2025 |
| --- | --- |
| Trench No: T2/NB/2025 | Date of completion: 19/04/2025 |
| Location: 23.331850°N, 69.907867°E | Elevation: 108m |
| a. Length: 10m b. Breadth: 1m c. Depth: 1m volume: (10Cu.m) |  |
| Recorded by: V. Santosh and B. Mahesh |  |
| Lithology details: Sandstone thinly laminated yellowish white/pinkish white interbedded with grey clay and pinkish brown siltstone (Bhuj formation) |  |
| Structural details: bedding and lamination |  |
| Log of Trench: Side walls represented by north by A, east by B, South by C and west by D and bottom of the pit by E. long axis direction of trench is marked. |  |
| Samples: 10 representative samples were collected from bottom of E at 1m interval (T2/NB/2025/01 to T2/NB/2025/10 and 4 samples from four side walls. (T2/NB/2025-A, B, C, D respectively). Total 14 samples were collected from this trench. |  |

(not to scale)

Detailed Lithology of Trench no: T2/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti, REE | Date of commencement: 21/04/2025 |
| --- | --- |
| Trench No: T3/NB/2025 | Date of completion: 21/04/2025 |
| Location: 23.328870°N, 69.901747°E | Elevation: 111m |
| a. Length: 10m b. Breadth: 1m c. Depth: 1m volume: (10Cu.m) |  |
| Recorded by: V. Santosh and B. Mahesh |  |
| Lithology details: yellowish white/yellowish-brown fine-grained sandstone (Bhuj formation) |  |
| Structural details: bedding and lamination |  |
| Log of Trench: Side walls represented by north by A, east by B, South by C and west by D and bottom of the pit by E. long axis direction of trench is marked. |  |
| Samples: 10 representative samples were collected from bottom of E at 1m interval (T3/NB/2025/01 to T3/NB/2025/10 and 4 samples from four side walls. (T3/NB/2025-A, B, C, D respectively). Total 14 samples were collected from this trench. |  |
| (not to scale) Detailed Lithology of Trench no: T3/NB/2025 |  |

| Name of the investigation: Bauxite, Ga, V, Ti, REE | Date of commencement: 21/04/2025 |
| --- | --- |
| Trench No: T4/NB/2025 | Date of completion: 21/04/2025 |
| Location: 23.336027°N, 69.912690°E | Elevation: 122m |
| a. Length: 10m b. Breadth: 1m c. Depth: 1m volume: (10Cu.m) |  |
| Recorded by: V. Santosh and B. Mahesh |  |
| Lithology details: calcareous fine to coarse grained sandstone dirty white/pinkish in colour (Katrol formation) |  |
| Structural details: bedding and lamination |  |
| Log of Trench: Side walls represented by north by A, east by B, South by C and west by D and bottom of the pit by E. long axis direction of trench is marked. |  |
| Samples: 10 representative samples were collected from bottom of E at 1m interval (T4/NB/2025/01 to T4/NB/2025/10 and 4 samples from four side walls. (T4/NB/2025-A, B, C, D respectively). Total 14 samples were collected from this trench. |  |
| (not to scale) |  |

Detailed Lithology of Trench no: T4/NB/2025

| Name of the investigation: Bauxite, Ga, V, Ti, REE | Date of commencement: 22/04/2025 |
| --- | --- |
| Trench No: T5/NB/2025 | Date of completion: 22/04/2025 |
| Location: 23.323207°N, 69.904923°E | Elevation: 141m |
| a. Length: 10m b. Breadth: 1m c. Depth: 1m volume: (10Cu.m) |  |
| Recorded by: V. Santosh and B. Mahesh |  |
| Lithology details: Fine grained thinly laminated sandstone and ferruginous shale with sst & mica laminae (Bhuj formation) |  |
| Structural details: bedding and lamination |  |
| Log of Trench: Side walls represented by north by A, east by B, South by C and west by D and bottom of the pit by E. long axis direction of trench is marked. |  |
| Samples: 10 representative samples were collected from bottom of E at 1m interval (T5/NB/2025/01 to T5/NB/2025/10 and 4 samples from four side walls. (T5/NB/2025-A, B, C, D respectively). Total 14 samples were collected from this trench. |  |
| (not to scale) |  |

Detailed Lithology of Trench no: T5/NB/2025

6.1.8 	Sampling

A total of 20 pit samples and 70 trench samples were collected through systematic pitting and trenching activities within the study area. Sampling methodology adopted for pitting and trenching has been discussed in subsequent paragraphs. Thus, a total of 90 samples were collected systematically from pits and trenches in the study area.

Sample collection

A representative sample (2–3 kg) was collected from the pits and trenches using a pickaxe and spade, placed in a properly labelled sample bag, and assigned a unique sample number written clearly with a permanent marker. The lithological details of all the pits and trenches were systematically recorded.

Sample preparation:

The sample preparation followed a multi-stage crushing, grinding, and sieving protocol

- Initial Crushing & Sieving: Mesh No. 8 (2.36 mm) The entire bulk sample was crushed and sieved. The oversize was further crushed to pass through the mesh no:08 completely. The sieved material of (-8 mesh size) was reduced to 1 kg by Coning and Quartering, ensuring homogeneity.

- Second Stage: Mesh No. 80 (177 μm): The 1 kg sample of -8 mesh size was further crushed, ground, and sieved through 80 no mesh. The sieved fraction was reduced to 500 g by Coning and Quartering.

- Final Stage: Mesh No. 200 (74 μm): The 500 g sample of -80 mesh size was again crushed, ground, and passed completely through mesh No. 200.This -200-mesh fraction constituted the final laboratory sample. From this, 100 g of the prepared lab sample was dispatched to the analytical laboratory, while the remaining portion was stored as a reference sample. At every stage of preparation, strict care was taken to maintain sample integrity and homogeneity, in accordance with NQT guidelines

Appropriateness of grain size

In accordance with the standard sampling procedures, it has also been observed that smaller the particle size, higher the homogeneity of the sample as well as higher the dissolvability during the chemical analysis. As per the standard practice, samples are generally pounded to (-)200 mesh size for analysis of trace element through ICPMS.

Fig No: 10

Photos showing different activities of sample preparation: (Photos: 27 to 33)

| Photo No: 27 Sampling shed, CMT, Bhuj-Gujarat | Photo No: 28 Recording the details of sample |
| --- | --- |

| Photo No: 29 Crushing and sieving of samples | Photo No: 30 coning and Quartering |
| --- | --- |

Sieves used:

| Photo No: 31 Sieve no: 8 mesh (2.36mm) | Photo No: 32 Sieve no: 80 mesh (180 micron) |
| --- | --- |
| Photo No: 33 Sieve no: 200 mesh (75 microm) and packing of final sample |  |

6.1.9 		Discussion on Geochemistry of pit and trench samples

Geochemistry of pit and trench Samples:

A total of 90 pit and trench samples were analysed for major oxides, including TiO₂ and V, while 15 selected samples were analysed for Rare Earth Elements (14 elements) and Gallium (Ga). The geochemical results indicate that the concentrations of bauxite indicators (Al₂O₃) as well as Ga, V, TiO₂, and REEs are low and fall below the threshold limits required for economic significance. The analysed samples do not display any anomalous enrichment or geochemical signatures suggestive of mineralisation for the targeted commodities. Therefore, the investigated area does not demonstrate favourable potential for bauxite, Ga, V, Ti, or REE mineralization at the scale explored. The spatial distribution of major oxides and REE elements is presented and interpreted in the following sections through major oxide distribution maps and elemental distribution maps.

Al2O3: Aluminium oxide (Al₂O₃) is the major constituent of bauxite; however, the Katrol and Bhuj formations in the study area comprise predominantly sandstone, siltstone, shale, and clay, with no evidence of lateritization or true bauxite horizons. The analysed samples show Al₂O₃ values ranging from 0.28 to 28.81% in the Katrol Formation and 4.83 to 34.89% in the Bhuj Formation. Although some samples display moderately high Al₂O₃, the corresponding SiO₂ contents are also high, indicating that the aluminium is primarily silicate-bound in clay minerals rather than present as gibbsite or boehmite. Higher Al₂O₃ values correlate strongly with grey clay lithologies, reflecting increased proportions of kaolinite, illite, or smectite.

Ternary diagram plotting of Al₂O₃–SiO₂–Fe₂O₃ further reveals that all samples fall within the “bauxitic clay” field rather than the true bauxite domain, confirming that the material represents argillaceous sediments enriched in alumina-bearing clays, not economic bauxite. Thus, despite localized Al₂O₃ enrichment in clay-rich zones, the absence of low silica and lack of laterite development indicate that the conditions necessary for bauxite formation were not attained in the study area.

The analytical results of major oxides and REE as received from laboratory is given below in Annexures: II, III, IV, V

ANNEXURE – II

| Statement showing analytical results of major oxides (by XRF) of 20 pit samples in Nadapa Area, Dist: Kachchh, Gujarat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S. No | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V | Maturity Index = (K2O/(Na2O+K2O)) |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |  |
| 1 | LL/25-26/001204/01 | P1/ND/2025 | Arenaceous clay | 55.13 | 22.85 | 3.75 | 0.01 | 0.45 | 0.22 | 6.11 | 0.99 | 0.99 | 0.05 | 9.19 | 103.53 | 0.14 | Immaturity |
| 2 | LL/25-26/001204/02 | P2/NB/2025 | Ferru.sandstone+ clay | 54.02 | 34.05 | 1.41 | 0.01 | 0.60 | 0.19 | 0.22 | 0.64 | 1.47 | 0.04 | 7.18 | 82.50 | 0.74 | Maturity |
| 3 | LL/25-26/001204/03 | P3/NB/2025 | d.grey clay | 51.25 | 34.89 | 1.57 | 0.00 | 0.62 | 0.13 | 0.64 | 1.19 | 1.36 | 0.04 | 7.97 | 71.90 | 0.65 | Maturity |
| 4 | LL/25-26/001204/04 | P4/NB/2025 | D.grey clay with mica | 56.89 | 29.95 | 2.48 | 0.00 | 0.74 | 0.05 | 0.82 | 1.67 | 0.94 | 0.05 | 6.25 | 80.55 | 0.67 | Maturity |
| 5 | LL/25-26/001204/05 | P5/NB/2025 | sandstone fgd-mgd y/w | 74.96 | 14.01 | 3.67 | 0.04 | 0.27 | 0.44 | 0.17 | 2.19 | 1.3 I | 0.04 | 2.64 | 52.28 | 0.93 | Maturity |
| 6 | LL/25-26/001204/06 | P6/NB/2025 | Argill.Sst p/w | 58.96 | 17.18 | 15.38 | 0.01 | 0.46 | 0.22 | 0.45 | 1.52 | 0.88 | 0.17 | 4.70 | 168.10 | 0.77 | Maturity |
| 7 | LL/25-26/001204/07 | P7/NB/2025 | Siltstone friable | 66.40 | 17.56 | 5.46 | 0.07 | 1.04 | 0.81 | 0.51 | 1.96 | 0.99 | 0.04 | 5.03 | 77.80 | 0.79 | Maturity |
| 8 | LL/25-26/001204/08 | P8/NB/2025 | SST.vfgd+mica | 70.77 | 19.96 | 2.38 | 0.01 | 0.39 | 0.29 | 0.46 | 2.49 | 0.90 | 0.02 | 2.12 | 15.70 | 0.84 | Maturity |
| 9 | LL/25-26/001204/09 | P9/NB/2025 | Siltstone friable p/w | 72.95 | 10.84 | 6.58 | 0.02 | 0.61 | 1.45 | 0.55 | 1.95 | 0.98 | 0.04 | 3.85 | 41.50 | 0.78 | Maturity |
| 10 | LL/25-26/001204/10 | P10/NB/2025 | siltstone variegated | 60.90 | 24.03 | 3.89 | 0.0 l | 0.72 | 0. 32 | 1.56 | 1.77 | 1.41 | 0.05 | 5.07 | 77.80 | 0.53 | Maturity |
| 11 | LL/25-26/001204/25 | P11/NB/2025 | sst.fgd+mica | 66.54 | 20.83 | 4.57 | 0.02 | 0.42 | 0.41 | 0.15 | 1.52 | 1.34 | 0.07 | 3.96 | 85.75 | 0.91 | Maturity |
| 12 | LL/25-26/001204/40 | P12/NB/2025 | Argill.Sst fgd +mica | 55.40 | 28.81 | 2.81 | 0.03 | 0.45 | 0.70 | 0.23 | 1.09 | 1.21 | 0.04 | 9.09 | 83.28 | 0.83 | Maturity |
| 13 | LL/25-26/001204/41 | P13/NB/2025 | grey clay interbeded with siltstone | 64.50 | 17.57 | 6.44 | 0.02 | 1.09 | 0.2 l | 0.41 | 1.32 | 0.98 | 0.06 | 7. 19 | 95.35 | 0.76 | Maturity |
| 14 | LL/25-26/001204/42 | P14/NB/2025 | Feldpathic Sst | 77.97 | 13.25 | 2.15 | 0.01 | 0.15 | 0.17 | 0.06 | 0.85 | 0.52 | 0.02 | 4.53 | 69.10 | 0.93 | Maturity |
| 15 | LL/25-26/001204/43 | P15/NB/2025 | grey clay interbeded with siltstone | 61.24 | 23.41 | 2.44 | 0.03 | 0.91 | 0.11 | 0.30 | 1.17 | 1.0 1 | 0.03 | 9.15 | 91 .65 | 0.80 | Maturity |
| 16 | LL/25-26/001204/44 | P16/NB/2025 | Calcariuos soil | 54.92 | 0.31 | 4.60 | 0.03 | 1.41 | 17.76 | 0.19 | 0.91 | 0.39 | 0.08 | 19.08 | 60.20 | 0.83 | Maturity |
| 17 | LL/25-26/001204/45 | P17/NB/2025 | siltstone y/brown | 60.98 | 21.10 | 3.10 | 0.03 | 0.24 | 0.11 | 3.74 | 0.68 | 1.19 | 0.03 | 8.50 | 108.53 | 0.15 | Immaturity |
| 18 | LL/25-26/001204/60 | P18/NB/2025 | sandstone with shale laminae | 70.64 | 16.15 | 5.09 | 0.03 | 0.23 | 0.26 | 0.09 | 1.28 | 0.88 | 0.04 | 4.99 | 48.20 | 0.93 | Maturity |
| 19 | LL/25-26/001204/89 | P19/NB/2025 | Hard Ferru.Sst | 44.74 | 9.71 | 29.97 | 0.19 | 0.37 | 2.03 | 0.06 | 0.45 | 0.74 | 0.37 | 11.22 | 299.18 | 0.88 | Maturity |
| 20 | LL/25-26/001204/90 | P20/NB/2025 | Grey clay+ sst. Laminae | 56.11 | 28.55 | 2.27 | 0.03 | 0.54 | 0.10 | 0.35 | 0.64 | 1.03 | 0.03 | 10.05 | 101.58 | 0.65 | Maturity |

ANNEXURE – III

| Statement showing analytical results of major oxides (by XRF) of 70 trench samples in Nadapa Area, Dist: Kachchh, Gujarat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S. No | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V |  | Maturity Index = (K2O/(Na2O+K2O)) |  |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |  |
| 1 | LL/25-26/001204/11 | Tl/NB/2025/01 | siltstone with mica | 65.86 | 20.98 | 3.78 | 0.02 | 0.77 | 0.67 | 0.38 | 2.64 | 0.71 | 0.03 | 3.87 | 30.80 |  | 0.87 |  | Matured |
| 2 | LL/25-26/001204/12 | T1/NB/2025/02 | siltstone with mica | 59.68 | 20.88 | 5.11 | 0.02 | 0.92 | 0.46 | 0.46 | 2.45 | 0.80 | 0.03 | 8.92 | 41.40 |  | 0.84 |  | Matured |
| 3 | LL/25-26/001204/13 | Tl/NB/2025/03 | siltstone with mica | 68.99 | 18.41 | 3.72 | 0.02 | 0.70 | 0.49 | 0.37 | 2.52 | 0.80 | 0.03 | 3.67 | 31.90 |  | 0.87 |  | Matured |
| 4 | LL/25-26/001204/14 | TI/NB/2025/04 | siltstone with mica | 70.24 | 19.10 | 2.78 | 0.01 | 0.64 | 0.19 | 0.41 | 2.78 | 0.70 | 0.03 | 3.0 I | 23.80 |  | 0.87 |  | Matured |
| 5 | LL/25-26/001204/15 | Tl/NB/2025/05 | siltstone with mica | 69.42 | 20.94 | 2.14 | 0.01 | 0.58 | 0.1 I | 0.30 | 2.74 | 0.87 | 0.03 | 2.65 | 25.50 |  | 0.90 |  | Matured |
| 6 | LL/25-26/001204/16 | Tl/NB/2025/06 | siltstone with mica | 68.86 | 21.65 | 2.07 | 0.02 | 0.57 | 0.29 | 0.34 | 2.78 | 0.74 | 0.03 | 2.54 | 22.70 |  | 0.89 |  | Matured |
| 7 | LL/25-26/001204/17 | Tl/NB/2025/07 | siltstone with mica | 68.34 | 20.82 | 2.00 | 0.01 | 0.73 | 0.77 | 0.42 | 2.54 | 1.01 | 0.04 | 3.13 | 26.80 |  | 0.86 |  | Matured |
| 8 | LL/25-26/001204/18 | Tl/NB/2025/08 | siltstone with mica | 68.77 | 19.32 | 2.07 | 0.01 | 0.87 | 1.09 | 0.43 | 2.42 | 0.88 | 0.04 | 3.89 | 29.60 |  | 0.85 |  | Matured |
| 9 | LL/25-26/001204/19 | TI/NB/2025/09 | siltstone with mica | 57.08 | 20.04 | 6.35 | 0.02 | 1.41 | 2.63 | 0.88 | 2.09 | 0.58 | 0.03 | 8.59 | 43.40 |  | 0.70 |  | Matured |
| 10 | LL/25-26/001204/20 | TI/NB/2025/10 | siltstone with mica | 50.08 | 20.57 | 12.45 | 0.03 | 1.71 | 2.08 | 0.99 | 1.75 | 0.87 | 0.05 | 9.22 | 76.80 |  | 0.64 |  | Matured |
| 11 | LL/25-26/001204/21 | TI/NB/2025/A | sandstone+siltstone interbeded with mica | 63.82 | 23.92 | 2.93 | 0.02 | 0.78 | 0.78 | 0.30 | 2.66 | 0.63 | 0.03 | 3.80 | 28.70 |  | 0.90 |  | Matured |
| 12 | LL/25-26/001204/22 | TI/NB/2025/B | sandstone+siltstone interbeded with mica | 63.24 | 20.35 | 4.96 | 0.02 | 1.15 | 1.13 | 0.45 | 2.15 | 0.85 | 0.04 | 5.53 | 50.90 |  | 0.83 |  | Matured |
| 13 | LL/25-26/001204/23 | TI/NB/2025/C | sandstone+siltstone interbeded with mica | 62.01 | 20.10 | 4.74 | 0.01 | 1 .41 | 0.78 | 1.42 | 2.00 | 1.00 | 0.05 | 6.23 | 64.80 |  | 0.58 |  | Matured |
| 14 | LL/25-26/001204/24 | T1/NB/2025/D | sandstone+siltstone interbeded with mica | 61.95 | 22.86 | 3.54 | 0.01 | 1.16 | 1.25 | 0.42 | 2.17 | 0.88 | 0.04 | 5.53 | 51.30 |  | 0.84 |  | Matured |
|  | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V |  | Maturity Index = (K2O/(Na2O+K2O)) |  |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |  |
|  |  | TRENCH - 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | LL/25-26/001204/26 | T2/NB/2025/01 | sandstone+ clay laminae thinly laminated | 64.98 | 19.02 | 4.10 | 0.01 | 1.24 | 0.41 | 1.31 | 1.58 | 1.61 | 0.07 | 5.48 | 108.40 |  | 0.55 |  | Matured |
| 16 | LL/25-26/001204/27 | T2/NB/2025/02 | sandstone+ clay laminae thinly laminated | 65.39 | 20.97 | 4.09 | 0.02 | 0.93 | 0.44 | 0.81 | 1.88 | 1.24 | 0.06 | 4.09 | 56.30 |  | 0.70 |  | Matured |
| 17 | LL/25-26/001204/28 | T2/NB/2025/03 | sandstone+ clay laminae thinly laminated | 62.66 | 4.83 | 18.66 | 0.07 | 0.59 | 4. 19 | 0.24 | 1.58 | 0.91 | 0.13 | 5.94 | 76.90 |  | 0.87 |  | Matured |
| 18 | LL/25-26/001204/29 | T2/NB/2025/04 | siltstone pinkish brown | 63.51 | 5.05 | 18.12 | 0.07 | 0.60 | 3.72 | 0.28 | 1.54 | 0.85 | 0.13 | 6.05 | 67.10 |  | 0.85 |  | Matured |
| 19 | LL/25-26/001204/30 | T2/NB/2025/05 | siltstone ferruginous | 65.80 | 18.75 | 5.33 | 0.02 | 0.75 | 1.41 | 0.49 | 1.96 | 1.33 | 0.07 | 3.77 | 40.50 |  | 0.80 |  | Matured |
| 20 | LL/25-26/001204/31 | T2/NB/2025/06 | siltstone pinkish brown | 56.06 | 25.04 | 5.18 | 0.01 | 0.99 | 0.23 | 0.49 | 1.52 | 1.84 | 0.07 | 8.38 | 125.30 |  | 0.76 |  | Matured |
| 21 | LL/25-26/001204/32 | T2/NB/2025/07 | sandstone+ clay laminae thinly laminated | 57.98 | 20.80 | 6.96 | 0.02 | 1.05 | 1.01 | 0.37 | 1.28 | 1.09 | 0.06 | 9. 10 | 109.30 |  | 0.78 |  | Matured |
| 22 | LL/25-26/001204/33 | T2/NB/2025/08 | sandstone+ clay laminae thinly laminated | 58.94 | 19.21 | 8.56 | 0.03 | 0.99 | 0.70 | 0.32 | 1.31 | 1.23 | 0.07 | 8.32 | 93.70 |  | 0.80 |  | Matured |
| 23 | LL/25-26/001204/34 | T2/NB/2025/09 | sandstone+ clay laminae thinly laminated | 61.33 | 23.02 | 3.86 | 0.01 | 0.97 | 0.11 | 0.31 | 1.37 | 1.46 | 0.05 | 7.27 | 86.00 |  | 0.82 |  | Matured |
| 24 | LL/25-26/001204/35 | T2/NB/2025/10 | sandstone+ clay laminae thinly laminated | 60.91 | 21.96 | 4.71 | 0.03 | 1.01 | 0.21 | 0.35 | 1.31 | 1.52 | 0.05 | 7.70 | 92.30 |  | 0.79 |  | Matured |
| 25 | LL/25-26/001204/36 | T2/NB/2025/A | laminated sandstone | 64.95 | 18.54 | 4.63 | 0.02 | 0.87 | 0.63 | 0.15 | 1.35 | 1.82 | 0.05 | 6.68 | 78.60 |  | 0.90 |  | Matured |
| 26 | LL/25-26/001204/37 | T2/NB/2025/B | interbanded sandstone +clay | 61.99 | 19.24 | 5.19 | 0.02 | 1.04 | 1.19 | 0.36 | 1.34 | 1.68 | 0.06 | 7.67 | 82.00 |  | 0.79 |  | Matured |
| 27 | LL/25-26/001204/38 | T2/NB/2025/C | interbanded sandstone +clay | 61.13 | 18.30 | 8.09 | 0.03 | 1.06 | 0.46 | 0.32 | 1.30 | 1.40 | 0.06 | 7.60 | 93.40 |  | 0.80 |  | Matured |
| 28 | LL/25-26/001204/39 | T2/NB/2025/D | interbanded sandstone +clay | 60.24 | 18.99 | 6.98 | 0.03 | 1.06 | 1.11 | 0.33 | 1.29 | 1.45 | 0.06 | 8.28 | 87.90 |  | 0.80 |  | Matured |
|  | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V |  | Maturity Index = (K2O/(Na2O+K2O)) |  |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |  |
|  |  | TRENCH - 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 | LL/25-26/001204/46 | T3/NB/2025/01 | sandstone yellowish brown | 57.96 | 21.02 | 9.69 | 0.09 | 0.60 | 0.33 | 0.06 | 1.18 | 0.79 | 0.08 | 8.01 | 57.40 |  | 0.95 |  | Matured |
| 30 | LL/25-26/001204/47 | T3/NB/2025/02 | sandstone yellowish brown | 54.39 | 23.13 | 10.40 | 0.08 | 0.68 | 0.24 | 0.06 | 1.11 | 0.83 | 0.06 | 8.73 | 84.00 |  | 0.95 |  | Matured |
| 31 | LL/25-26/001204/48 | T3/NB/2025/03 | sandstone yellowish brown | 53.37 | 24.48 | 8.85 | 0.07 | 0.70 | 0.28 | 0.05 | 1.15 | 0.88 | 0.04 | 9.88 | 99.70 |  | 0.96 |  | Matured |
| 32 | LL/25-26/001204/48 | T3/NB/2025/04 | sandstone yellowish brown | 50.30 | 24.54 | 11.26 | 0.09 | 0.78 | 0.55 | 0.06 | 1.06 | 1.05 | 0.04 | 10.06 | 89.40 |  | 0.95 |  | Matured |
| 33 | LL/25-26/001204/50 | T3/NB/2025/05 | sandstone yellowish brown | 55.39 | 20.17 | 12.95 | 0.1 I | 0.68 | 0.26 | 0.07 | 1.21 | 0. 87 | 0.06 | 8.06 | 85.50 |  | 0.95 |  | Matured |
| 34 | LL/25-26/001204/51 | T3/NB/2025/06 | sandstone yellowish brown | 56.77 | 17.70 | 13.60 | 0.12 | 0.55 | 0.76 | 0.15 | 1.30 | 1.01 | 0.04 | 7.70 | 93.90 |  | 0.90 |  | Matured |
| 35 | LL/25-26/001204/52 | T3/NB/2025/07 | sandstone yellowish brown | 65.72 | 12.59 | 12.03 | 0.08 | 0.37 | 0.38 | 0.10 | 1.37 | 0.90 | 0.03 | 6.22 | 60.90 |  | 0.93 |  | Matured |
| 36 | LL/25-26/001204/53 | T3/NB/2025/08 | sandstone yellowish brown | 57.90 | 22.11 | 4.91 | 0.04 | 0.64 | 2.11 | 0.11 | 1.57 | 1.46 | 0.03 | 8.79 | 76.50 |  | 0.93 |  | Matured |
| 37 | LL/25-26/001204/54 | T3/NB/2025/09 | sandstone yellowish white | 56.99 | 24.13 | 5.25 | 0.05 | 0.57 | 1.15 | 0.10 | 1.55 | 1.07 | 0.03 | 8.87 | 64.80 |  | 0.94 |  | Matured |
| 38 | LL/25-26/001204/55 | T3/NB/2025/10 | sandstone yellowish white | 59.81 | 23.21 | 3.96 | 0.03 | 0.49 | 1.09 | 0.09 | 1.71 | 2.02 | 0.03 | 7.48 | 63.80 |  | 0.95 |  | Matured |
| 39 | LL/25-26/001204/56 | T3/NB/2025/A | sandstone yellowish brown | 60.23 | 18.99 | 9.46 | 0.07 | 0.65 | 0.29 | 0.09 | 1.38 | 1.29 | 0.08 | 7.18 | 89.40 |  | 0.94 |  | Matured |
| 40 | LL/25-26/001204/57 | T3/NB/2025/B | sandstone yellowish brown | 54.90 | 22.92 | 8.55 | 0.07 | 0.67 | 0.81 | 0.08 | 1.35 | 1.43 | 0.04 | 8.97 | 85.60 |  | 0.94 |  | Matured |
| 41 | LL/25-26/001204/58 | T3/NB/2025/C | sandstone yellowish white | 59.24 | 23.20 | 4.57 | 0.05 | 0.69 | 1.33 | 0.08 | 1.46 | 1.66 | 0.04 | 7.41 | 63.70 |  | 0.95 |  | Matured |
| 42 | LL/25-26/001204/59 | T3/NB/2025/D | sandstone yellowish brown | 59.64 | 22.77 | 6.14 | 0.06 | 0.61 | 1.08 | 0.09 | 1.77 | 1.36 | 0.07 | 6.27 | 71.80 |  | 0.95 |  | Matured |
|  | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V |  | Maturity Index = (K2O/(Na2O+K2O)) |  |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |  |
|  |  | TRENCH - 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 43 | LL/25-26/001204/61 | T4/NB/2025/01 | sandstone Argil. | 63.68 | 0.65 | 14.12 | 0.03 | 1.06 | 8.88 | 0.21 | 1.28 | 0.47 | 0.09 | 9.31 | 86.20 |  | 0.86 |  | Matured |
| 44 | LL/25-26/001204/62 | T4/NB/2025/02 | sandstone Argil. | 62.67 | 1.29 | 15.95 | 0.03 | 1.16 | 6.79 | 0.25 | 1.33 | 0.48 | 0.08 | 9.67 | 103.80 |  | 0.84 |  | Matured |
| 45 | LL/25-26/001204/63 | T4/NB/2025/03 | sandstone mgd-cgd | 69.00 | 2.49 | 7.36 | 0.02 | 1.19 | 7.33 | 0.21 | 1.46 | 0.53 | 0.08 | 10.18 | 61.90 |  | 0.87 |  | Matured |
| 46 | LL/25-26/001204/64 | T4/NB/2025/04 | sandstone mgd-cgd | 65.34 | 4.64 | 6.49 | 0.02 | 1.08 | 8.63 | 0.20 | 1.59 | 0.45 | 0.07 | 11.28 | 60.50 |  | 0.89 |  | Matured |
| 47 | LL/25-26/001204/65 | T4/NB/2025/05 | sandstone mgd-cgd | 57.60 | 4.71 | 15.63 | 0.04 | 1.04 | 7.84 | 0.20 | 1.32 | 0.48 | 0.07 | 11.00 | 94.00 |  | 0.87 |  | Matured |
| 48 | LL/25-26/001204/66 | T4/NB/2025/06 | sandstone fgd-mgd | 58.15 | 0.41 | l 6.09 | 0.05 | 0.77 | 10.08 | 0.17 | 1.04 | 0.41 | 0.05 | 12.55 | 84.60 |  | 0.86 |  | Matured |
| 49 | LL/25-26/001204/67 | T4/NB/2025/07 | sandstone fgd-mgd | 73.26 | 1.00 | 11.17 | 0.04 | 0.89 | 3.84 | 0.16 | 1.47 | 0.83 | 0.06 | 7.16 | 70.30 |  | 0.90 |  | Matured |
| 50 | LL/25-26/001204/68 | T4/NB/2025/08 | sandstone mgd-cgd | 69.01 | 0.35 | 7.37 | 0.02 | 0.94 | 9.10 | 0.12 | 1.09 | 0.44 | 0.07 | 11.34 | 33.80 |  | 0.90 |  | Matured |
| 51 | LL/25-26/001204/69 | T4/NB/2025/09 | sandstone mgd-cgd | 68.61 | 0.28 | 5.81 | 0.03 | 1.50 | 9.91 | 0.17 | 1.1 | 0.61 | 0.12 | 11.58 | 42.40 |  | 0.87 |  | Matured |
| 52 | LL/25-26/001204/70 | T4/NB/2025/10 | sandstone mgd-cgd | 69.93 | 0.31 | 8.67 | 0.03 | 0.98 | 7.52 | 0.15 | 1.13 | 0.86 | 0.07 | 10.12 | 50.70 |  | 0.88 |  | Matured |
| 53 | LL/25-26/001204/71 | T4/NB/2025/A | Alluvial soil calcarious | 58.96 | 0.32 | 9.64 | 0.03 | 1.71 | 13.86 | 0.23 | 1.01 | 0.48 | 0.10 | 13.35 | 67.30 |  | 0.81 |  | Matured |
| 54 | LL/25-26/001204/72 | T4/NB/2025/B | Alluvial soil calcarious | 62.01 | 0.29 | 6.61 | 0.04 | 1.78 | 14.02 | 0.35 | 0.92 | 0.53 | 0.10 | 13.26 | 37.10 |  | 0.72 |  | Matured |
| 55 | LL/25-26/001204/73 | T4/NB/2025/C | Alluvial soil calcarious | 63.81 | 1.39 | 6.17 | 0.04 | 1.62 | 11.57 | 0.27 | 1.12 | 0.90 | 0.09 | 12.72 | 62.20 |  | 0.81 |  | Matured |
| 56 | LL/25-26/001204/74 | T4/NB/2025/D | Alluvial soil calcarious | 69.33 | 0.41 | 5.38 | 0.03 | 1.66 | l 1.39 | 0.32 | 0.96 | 0.52 | 0.10 | 9.74 | 39.80 |  | 0.75 |  | Matured |
|  | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V |  | Maturity Index = (K2O/(Na2O+K2O)) |  |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |  |
|  |  | TRENCH - 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 57 | LL/25-26/001204/75 | T5/NB/2025/01 | sandstone Argil. | 54.82 | 13.92 | 20.70 | 0.10 | 0.21 | 0.48 | 0.08 | 1.20 | 0.67 | 0.02 | 7.48 | 51.80 |  | 0.94 |  | Matured |
| 58 | LL/25-26/001204/76 | T5/NB/2025/02 | sandstone Argil. | 46.38 | 16.94 | 18.80 | 0.07 | 0.42 | 3.86 | 0.09 | 1.02 | 0.69 | 0.03 | 11.50 | 60.90 |  | 0.92 |  | Matured |
| 59 | LL/25-26/001204/77 | T5/NB/2025/03 | sandstone mgd-cgd | 51.78 | 9.84 | 25.05 | 0.18 | 0.26 | 1.64 | 0.08 | 1.10 | 0.55 | 0.02 | 9.36 | 49.10 |  | 0.93 |  | Matured |
| 60 | LL/25-26/001204/78 | T5/NB/2025/04 | sandstone mgd-cgd | 53.61 | 9.99 | 25.97 | 0.21 | 0.19 | 0.33 | 0.08 | 1.16 | 0.56 | 0.02 | 7.70 | 54.78 |  | 0.94 |  | Matured |
| 61 | LL/25-26/001204/79 | T5/NB/2025/05 | sandstone mgd-cgd | 46.31 | 24.46 | 16.81 | 0.04 | 0.31 | 0.35 | 0.07 | 1.09 | 0.98 | 0.02 | 9.30 | 82.20 |  | 0.94 |  | Matured |
| 62 | LL/25-26/001204/80 | T5/NB/2025/06 | sandstone fgd-mgd | 44.84 | 23.94 | l 6.91 | 0.05 | 0.33 | 1.34 | 0.07 | 1.11 | 0.96 | 0.02 | 10.27 | 82.40 |  | 0.94 |  | Matured |
| 63 | LL/25-26/001204/81 | T5/NB/2025/07 | sandstone fgd-mgd | 51.00 | 16.55 | 21.57 | 0.09 | 0.22 | 0.17 | 0.08 | 1.33 | 0.72 | 0.02 | 8.17 | 65.00 |  | 0.94 |  | Matured |
| 64 | LL/25-26/001204/82 | T5/NB/2025/08 | sandstone mgd-cgd | 49.81 | 15.27 | 23.14 | 0.09 | 0.29 | 0.74 | 0.08 | 1.13 | 0.60 | 0.02 | 8.73 | 93.60 |  | 0.93 |  | Matured |
| 65 | LL/25-26/001204/83 | T5/NB/2025/09 | sandstone mgd-cgd | 57.97 | 12.10 | 19.02 | 0.08 | 0.45 | 0.88 | 0.08 | 1.01 | 0.61 | 0.03 | 7.55 | 64.60 |  | 0.93 |  | Matured |
| 66 | LL/25-26/001204/84 | T5/NB/2025/10 | sandstone mgd-cgd | 55.86 | 19.77 | 13.00 | 0.04 | 0.45 | 0.42 | 0.08 | 1.05 | 0.95 | 0.03 | 8.04 | 81.0 |  | 0.93 |  | Matured |
| 67 | LL/25-26/001204/85 | T5/NB/2025/A | Alluvial soil calcarious | 53.04 | 17.18 | 7.93 | 0.03 | 0.68 | 6.10 | 0.12 | 0.91 | 0.89 | 0.07 | 12.87 | 61.00 |  | 0.88 |  | Matured |
| 68 | LL/25-26/001204/86 | T5/NB/2025/B | Alluvial soil calcarious | 56.06 | 27.94 | 2.31 | 0.01 | 0.63 | 1.37 | 0.09 | 0.91 | 1.31 | 0.04 | 9.27 | 66.10 |  | 0.91 |  | Matured |
| 69 | LL/25-26/001204/87 | T5/NB/2025/C | Alluvial soil calcarious | 59.78 | 27.00 | 1.94 | 0.01 | 0.67 | 0.61 | 0.07 | 0.83 | 0.88 | 0.03 | 7.96 | 78.05 |  | 0.92 |  | Matured |
| 70 | LL/25-26/001204/88 | T5/NB/2025/D | Alluvial soil calcarious | 58.61 | 26.49 | 2.16 | 0.01 | 0.66 | 1.04 | 0.08 | 0.87 | 1.27 | 0.03 | 8.66 | 63.20 |  | 0.92 |  | Matured |

ANNEXURE – IV

ANNEXURE – V

| Statement showing analytical results of major oxides (by XRF) of Katrol formation in Nadapa Area, Dist: Kachchh, Gujarat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S. No | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V | Maturity Index = (K2O/(Na2O+K2O)) |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |
| 1 | LL/25-26/001204/25 | P11/NB/2025 | sst.fgd+mica | 66.54 | 20.83 | 4.57 | 0.02 | 0.42 | 0.41 | 0.15 | 1.52 | 1.34 | 0.07 | 3.96 | 85.75 | 0.91 | Maturity |
| 2 | LL/25-26/001204/40 | P12/NB/2025 | Argill.Sst fgd +mica | 55.40 | 28.81 | 2.81 | 0.03 | 0.45 | 0.70 | 0.23 | 1.09 | 1.21 | 0.04 | 9.09 | 83.28 | 0.83 | Maturity |
| 3 | LL/25-26/001204/44 | P16/NB/2025 | Calcariuos soil | 54.92 | 0.31 | 4.60 | 0.03 | 1.41 | 17.76 | 0.19 | 0.91 | 0.39 | 0.08 | 19.08 | 60.20 | 0.83 | Maturity |
|  | TRENCH NO - 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | LL/25-26/001204/61 | T4/NB/2025/01 | sandstone Argil. | 63.68 | 0.65 | 14.12 | 0.03 | 1.06 | 8.88 | 0.21 | 1.28 | 0.47 | 0.09 | 9.31 | 86.20 | 0.86 | Matured |
| 5 | LL/25-26/001204/62 | T4/NB/2025/02 | sandstone Argil. | 62.67 | 1.29 | 15.95 | 0.03 | 1.16 | 6.79 | 0.25 | 1.33 | 0.48 | 0.08 | 9.67 | 103.80 | 0.84 | Matured |
| 6 | LL/25-26/001204/63 | T4/NB/2025/03 | sandstone mgd-cgd | 69.00 | 2.49 | 7.36 | 0.02 | 1.19 | 7.33 | 0.21 | 1.46 | 0.53 | 0.08 | 10.18 | 61.90 | 0.87 | Matured |
| 7 | LL/25-26/001204/64 | T4/NB/2025/04 | sandstone mgd-cgd | 65.34 | 4.64 | 6.49 | 0.02 | 1.08 | 8.63 | 0.20 | 1.59 | 0.45 | 0.07 | 11.28 | 60.50 | 0.89 | Matured |
| 8 | LL/25-26/001204/65 | T4/NB/2025/05 | sandstone mgd-cgd | 57.60 | 4.71 | 15.63 | 0.04 | 1.04 | 7.84 | 0.20 | 1.32 | 0.48 | 0.07 | 11.00 | 94.00 | 0.87 | Matured |
| 9 | LL/25-26/001204/66 | T4/NB/2025/06 | sandstone fgd-mgd | 58.15 | 0.41 | l 6.09 | 0.05 | 0.77 | 10.08 | 0.17 | 1.04 | 0.41 | 0.05 | 12.55 | 84.60 | 0.86 | Matured |
| 10 | LL/25-26/001204/67 | T4/NB/2025/07 | sandstone fgd-mgd | 73.26 | 1.00 | 11.17 | 0.04 | 0.89 | 3.84 | 0.16 | 1.47 | 0.83 | 0.06 | 7.16 | 70.30 | 0.90 | Matured |
| 11 | LL/25-26/001204/68 | T4/NB/2025/08 | sandstone mgd-cgd | 69.01 | 0.35 | 7.37 | 0.02 | 0.94 | 9.10 | 0.12 | 1.09 | 0.44 | 0.07 | 11.34 | 33.80 | 0.90 | Matured |
| 12 | LL/25-26/001204/69 | T4/NB/2025/09 | sandstone mgd-cgd | 68.61 | 0.28 | 5.81 | 0.03 | 1.50 | 9.91 | 0.17 | 1.1 | 0.61 | 0.12 | 11.58 | 42.40 | 0.87 | Matured |
| 13 | LL/25-26/001204/70 | T4/NB/2025/10 | sandstone mgd-cgd | 69.93 | 0.31 | 8.67 | 0.03 | 0.98 | 7.52 | 0.15 | 1.13 | 0.86 | 0.07 | 10.12 | 50.70 | 0.88 | Matured |
| 14 | LL/25-26/001204/71 | T4/NB/2025/A | Alluvial soil calcarious | 58.96 | 0.32 | 9.64 | 0.03 | 1.71 | 13.86 | 0.23 | 1.01 | 0.48 | 0.10 | 13.35 | 67.30 | 0.81 | Matured |
| 15 | LL/25-26/001204/72 | T4/NB/2025/B | Alluvial soil calcarious | 62.01 | 0.29 | 6.61 | 0.04 | 1.78 | 14.02 | 0.35 | 0.92 | 0.53 | 0.10 | 13.26 | 37.10 | 0.72 | Matured |
| 16 | LL/25-26/001204/73 | T4/NB/2025/C | Alluvial soil calcarious | 63.81 | 1.39 | 6.17 | 0.04 | 1.62 | 11.57 | 0.27 | 1.12 | 0.90 | 0.09 | 12.72 | 62.20 | 0.81 | Matured |
| 17 | LL/25-26/001204/74 | T4/NB/2025/D | Alluvial soil calcarious | 69.33 | 0.41 | 5.38 | 0.03 | 1.66 | l 1.39 | 0.32 | 0.96 | 0.52 | 0.10 | 9.74 | 39.80 | 0.75 | Matured |

ANNEXURE – VI

| Statement showing analytical results of major oxides (by XRF) of Bhuj formation in Nadapa Area, Dist: Kachchh, Gujarat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S. No | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V | Maturity Index = (K2O/(Na2O+K2O)) |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |
| 1 | LL/25-26/001204/01 | P1/ND/2025 | Arenaceous clay | 55.13 | 22.85 | 3.75 | 0.01 | 0.45 | 0.22 | 6.11 | 0.99 | 0.99 | 0.05 | 9.19 | 103.53 | 0.14 | Immaturity |
| 2 | LL/25-26/001204/02 | P2/NB/2025 | Ferru.sandstone+ clay | 54.02 | 34.05 | 1.41 | 0.01 | 0.60 | 0.19 | 0.22 | 0.64 | 1.47 | 0.04 | 7.18 | 82.50 | 0.74 | Maturity |
| 3 | LL/25-26/001204/03 | P3/NB/2025 | d.grey clay | 51.25 | 34.89 | 1.57 | 0.00 | 0.62 | 0.13 | 0.64 | 1.19 | 1.36 | 0.04 | 7.97 | 71.90 | 0.65 | Maturity |
| 4 | LL/25-26/001204/04 | P4/NB/2025 | D.grey clay with mica | 56.89 | 29.95 | 2.48 | 0.00 | 0.74 | 0.05 | 0.82 | 1.67 | 0.94 | 0.05 | 6.25 | 80.55 | 0.67 | Maturity |
| 5 | LL/25-26/001204/05 | P5/NB/2025 | sandstone fgd-mgd y/w | 74.96 | 14.01 | 3.67 | 0.04 | 0.27 | 0.44 | 0.17 | 2.19 | 1.3 I | 0.04 | 2.64 | 52.28 | 0.93 | Maturity |
| 6 | LL/25-26/001204/06 | P6/NB/2025 | Argill.Sst p/w | 58.96 | 17.18 | 15.38 | 0.01 | 0.46 | 0.22 | 0.45 | 1.52 | 0.88 | 0.17 | 4.70 | 168.10 | 0.77 | Maturity |
| 7 | LL/25-26/001204/07 | P7/NB/2025 | Siltstone friable | 66.40 | 17.56 | 5.46 | 0.07 | 1.04 | 0.81 | 0.51 | 1.96 | 0.99 | 0.04 | 5.03 | 77.80 | 0.79 | Maturity |
| 8 | LL/25-26/001204/08 | P8/NB/2025 | SST.vfgd+mica | 70.77 | 19.96 | 2.38 | 0.01 | 0.39 | 0.29 | 0.46 | 2.49 | 0.90 | 0.02 | 2.12 | 15.70 | 0.84 | Maturity |
| 9 | LL/25-26/001204/09 | P9/NB/2025 | Siltstone friable p/w | 72.95 | 10.84 | 6.58 | 0.02 | 0.61 | 1.45 | 0.55 | 1.95 | 0.98 | 0.04 | 3.85 | 41.50 | 0.78 | Maturity |
| 10 | LL/25-26/001204/10 | P10/NB/2025 | siltstone variegated | 60.90 | 24.03 | 3.89 | 0.0 l | 0.72 | 0. 32 | 1.56 | 1.77 | 1.41 | 0.05 | 5.07 | 77.80 | 0.53 | Maturity |
| 11 | LL/25-26/001204/41 | P13/NB/2025 | grey clay interbeded with siltstone | 64.50 | 17.57 | 6.44 | 0.02 | 1.09 | 0.2 l | 0.41 | 1.32 | 0.98 | 0.06 | 7. 19 | 95.35 | 0.76 | Maturity |
| 12 | LL/25-26/001204/42 | P14/NB/2025 | Feldpathic Sst | 77.97 | 13.25 | 2.15 | 0.01 | 0.15 | 0.17 | 0.06 | 0.85 | 0.52 | 0.02 | 4.53 | 69.10 | 0.93 | Maturity |
| 13 | LL/25-26/001204/43 | P15/NB/2025 | grey clay interbeded with siltstone | 61.24 | 23.41 | 2.44 | 0.03 | 0.91 | 0.11 | 0.30 | 1.17 | 1.0 1 | 0.03 | 9.15 | 91 .65 | 0.80 | Maturity |
| 14 | LL/25-26/001204/45 | P17/NB/2025 | siltstone y/brown | 60.98 | 21.10 | 3.10 | 0.03 | 0.24 | 0.11 | 3.74 | 0.68 | 1.19 | 0.03 | 8.50 | 108.53 | 0.15 | Immaturity |
| 15 | LL/25-26/001204/60 | P I 8/NB/2025 | sandstone with shale laminae | 70.64 | 16.15 | 5.09 | 0.03 | 0.23 | 0.26 | 0.09 | 1.28 | 0.88 | 0.04 | 4.99 | 48.20 | 0.93 | Maturity |
| 16 | LL/25-26/001204/89 | P19/NB/2025 | Hard Ferru.Sst | 44.74 | 9.71 | 29.97 | 0.19 | 0.37 | 2.03 | 0.06 | 0.45 | 0.74 | 0.37 | 11.22 | 299.18 | 0.88 | Maturity |
| 17 | LL/25-26/001204/90 | P20/NB/2025 | Grey clay+ sst. Laminae | 56.11 | 28.55 | 2.27 | 0.03 | 0.54 | 0.10 | 0.35 | 0.64 | 1.03 | 0.03 | 10.05 | 101.58 | 0.65 | Maturity |
|  | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V | Maturity Index = (K2O/(Na2O+K2O)) |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |
|  |  | TRENCH - 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | LL/25-26/001204/11 | Tl/NB/2025/01 | siltstone with mica | 65.86 | 20.98 | 3.78 | 0.02 | 0.77 | 0.67 | 0.38 | 2.64 | 0.71 | 0.03 | 3.87 | 30.80 | 0.87 | Matured |
| 19 | LL/25-26/001204/12 | T1/NB/2025/02 | siltstone with mica | 59.68 | 20.88 | 5.11 | 0.02 | 0.92 | 0.46 | 0.46 | 2.45 | 0.80 | 0.03 | 8.92 | 41.40 | 0.84 | Matured |
| 20 | LL/25-26/001204/13 | Tl/NB/2025/03 | siltstone with mica | 68.99 | 18.41 | 3.72 | 0.02 | 0.70 | 0.49 | 0.37 | 2.52 | 0.80 | 0.03 | 3.67 | 31.90 | 0.87 | Matured |
| 21 | LL/25-26/001204/14 | TI/NB/2025/04 | siltstone with mica | 70.24 | 19.10 | 2.78 | 0.01 | 0.64 | 0.19 | 0.41 | 2.78 | 0.70 | 0.03 | 3.0 I | 23.80 | 0.87 | Matured |
| 22 | LL/25-26/001204/15 | Tl/NB/2025/05 | siltstone with mica | 69.42 | 20.94 | 2.14 | 0.01 | 0.58 | 0.1 I | 0.30 | 2.74 | 0.87 | 0.03 | 2.65 | 25.50 | 0.90 | Matured |
| 23 | LL/25-26/001204/16 | Tl/NB/2025/06 | siltstone with mica | 68.86 | 21.65 | 2.07 | 0.02 | 0.57 | 0.29 | 0.34 | 2.78 | 0.74 | 0.03 | 2.54 | 22.70 | 0.89 | Matured |
| 24 | LL/25-26/001204/17 | Tl/NB/2025/07 | siltstone with mica | 68.34 | 20.82 | 2.00 | 0.01 | 0.73 | 0.77 | 0.42 | 2.54 | 1.01 | 0.04 | 3.13 | 26.80 | 0.86 | Matured |
| 25 | LL/25-26/001204/18 | Tl/NB/2025/08 | siltstone with mica | 68.77 | 19.32 | 2.07 | 0.01 | 0.87 | 1.09 | 0.43 | 2.42 | 0.88 | 0.04 | 3.89 | 29.60 | 0.85 | Matured |
| 26 | LL/25-26/001204/19 | TI/NB/2025/09 | siltstone with mica | 57.08 | 20.04 | 6.35 | 0.02 | 1.41 | 2.63 | 0.88 | 2.09 | 0.58 | 0.03 | 8.59 | 43.40 | 0.70 | Matured |
| 27 | LL/25-26/001204/20 | TI/NB/2025/10 | siltstone with mica | 50.08 | 20.57 | 12.45 | 0.03 | 1.71 | 2.08 | 0.99 | 1.75 | 0.87 | 0.05 | 9.22 | 76.80 | 0.64 | Matured |
| 28 | LL/25-26/001204/21 | TI/NB/2025/A | sandstone+siltstone interbeded with mica | 63.82 | 23.92 | 2.93 | 0.02 | 0.78 | 0.78 | 0.30 | 2.66 | 0.63 | 0.03 | 3.80 | 28.70 | 0.90 | Matured |
| 29 | LL/25-26/001204/22 | TI/NB/2025/B | sandstone+siltstone interbeded with mica | 63.24 | 20.35 | 4.96 | 0.02 | 1.15 | 1.13 | 0.45 | 2.15 | 0.85 | 0.04 | 5.53 | 50.90 | 0.83 | Matured |
| 30 | LL/25-26/001204/23 | TI/NB/2025/C | sandstone+siltstone interbeded with mica | 62.01 | 20.10 | 4.74 | 0.01 | 1 .41 | 0.78 | 1.42 | 2.00 | 1.00 | 0.05 | 6.23 | 64.80 | 0.58 | Matured |
| 31 | LL/25-26/001204/24 | T1/NB/2025/D | sandstone+siltstone interbeded with mica | 61.95 | 22.86 | 3.54 | 0.01 | 1.16 | 1.25 | 0.42 | 2.17 | 0.88 | 0.04 | 5.53 | 51.30 | 0.84 | Matured |
|  | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V | Maturity Index = (K2O/(Na2O+K2O)) |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |
|  |  | TRENCH - 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 | LL/25-26/001204/26 | T2/NB/2025/01 | sandstone+ clay laminae thinly laminated | 64.98 | 19.02 | 4.10 | 0.01 | 1.24 | 0.41 | 1.31 | 1.58 | 1.61 | 0.07 | 5.48 | 108.40 | 0.55 | Matured |
| 33 | LL/25-26/001204/27 | T2/NB/2025/02 | sandstone+ clay laminae thinly laminated | 65.39 | 20.97 | 4.09 | 0.02 | 0.93 | 0.44 | 0.81 | 1.88 | 1.24 | 0.06 | 4.09 | 56.30 | 0.70 | Matured |
| 34 | LL/25-26/001204/28 | T2/NB/2025/03 | sandstone+ clay laminae thinly laminated | 62.66 | 4.83 | 18.66 | 0.07 | 0.59 | 4. 19 | 0.24 | 1.58 | 0.91 | 0.13 | 5.94 | 76.90 | 0.87 | Matured |
| 35 | LL/25-26/001204/29 | T2/NB/2025/04 | siltstone pinkish brown | 63.51 | 5.05 | 18.12 | 0.07 | 0.60 | 3.72 | 0.28 | 1.54 | 0.85 | 0.13 | 6.05 | 67.10 | 0.85 | Matured |
| 36 | LL/25-26/001204/30 | T2/NB/2025/05 | siltstone ferruginous | 65.80 | 18.75 | 5.33 | 0.02 | 0.75 | 1.41 | 0.49 | 1.96 | 1.33 | 0.07 | 3.77 | 40.50 | 0.80 | Matured |
| 37 | LL/25-26/001204/31 | T2/NB/2025/06 | siltstone pinkish brown | 56.06 | 25.04 | 5.18 | 0.01 | 0.99 | 0.23 | 0.49 | 1.52 | 1.84 | 0.07 | 8.38 | 125.30 | 0.76 | Matured |
| 38 | LL/25-26/001204/32 | T2/NB/2025/07 | sandstone+ clay laminae thinly laminated | 57.98 | 20.80 | 6.96 | 0.02 | 1.05 | 1.01 | 0.37 | 1.28 | 1.09 | 0.06 | 9. 10 | 109.30 | 0.78 | Matured |
| 39 | LL/25-26/001204/33 | T2/NB/2025/08 | sandstone+ clay laminae thinly laminated | 58.94 | 19.21 | 8.56 | 0.03 | 0.99 | 0.70 | 0.32 | 1.31 | 1.23 | 0.07 | 8.32 | 93.70 | 0.80 | Matured |
| 40 | LL/25-26/001204/34 | T2/NB/2025/09 | sandstone+ clay laminae thinly laminated | 61.33 | 23.02 | 3.86 | 0.01 | 0.97 | 0.11 | 0.31 | 1.37 | 1.46 | 0.05 | 7.27 | 86.00 | 0.82 | Matured |
| 41 | LL/25-26/001204/35 | T2/NB/2025/10 | sandstone+ clay laminae thinly laminated | 60.91 | 21.96 | 4.71 | 0.03 | 1.01 | 0.21 | 0.35 | 1.31 | 1.52 | 0.05 | 7.70 | 92.30 | 0.79 | Matured |
| 42 | LL/25-26/001204/36 | T2/NB/2025/A | laminated sandstone | 64.95 | 18.54 | 4.63 | 0.02 | 0.87 | 0.63 | 0.15 | 1.35 | 1.82 | 0.05 | 6.68 | 78.60 | 0.90 | Matured |
| 43 | LL/25-26/001204/37 | T2/NB/2025/B | interbanded sandstone +clay | 61.99 | 19.24 | 5.19 | 0.02 | 1.04 | 1.19 | 0.36 | 1.34 | 1.68 | 0.06 | 7.67 | 82.00 | 0.79 | Matured |
| 44 | LL/25-26/001204/38 | T2/NB/2025/C | interbanded sandstone +clay | 61.13 | 18.30 | 8.09 | 0.03 | 1.06 | 0.46 | 0.32 | 1.30 | 1.40 | 0.06 | 7.60 | 93.40 | 0.80 | Matured |
| 45 | LL/25-26/001204/39 | T2/NB/2025/D | interbanded sandstone +clay | 60.24 | 18.99 | 6.98 | 0.03 | 1.06 | 1.11 | 0.33 | 1.29 | 1.45 | 0.06 | 8.28 | 87.90 | 0.80 | Matured |
|  | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V | Maturity Index = (K2O/(Na2O+K2O)) |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |
|  |  | TRENCH - 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 46 | LL/25-26/001204/46 | T3/NB/2025/01 | sandstone yellowish brown | 57.96 | 21.02 | 9.69 | 0.09 | 0.60 | 0.33 | 0.06 | 1.18 | 0.79 | 0.08 | 8.01 | 57.40 | 0.95 | Matured |
| 47 | LL/25-26/001204/47 | T3/NB/2025/02 | sandstone yellowish brown | 54.39 | 23.13 | 10.40 | 0.08 | 0.68 | 0.24 | 0.06 | 1.11 | 0.83 | 0.06 | 8.73 | 84.00 | 0.95 | Matured |
| 48 | LL/25-26/001204/48 | T3/NB/2025/03 | sandstone yellowish brown | 53.37 | 24.48 | 8.85 | 0.07 | 0.70 | 0.28 | 0.05 | 1.15 | 0.88 | 0.04 | 9.88 | 99.70 | 0.96 | Matured |
| 49 | LL/25-26/001204/48 | T3/NB/2025/04 | sandstone yellowish brown | 50.30 | 24.54 | 11.26 | 0.09 | 0.78 | 0.55 | 0.06 | 1.06 | 1.05 | 0.04 | 10.06 | 89.40 | 0.95 | Matured |
| 50 | LL/25-26/001204/50 | T3/NB/2025/05 | sandstone yellowish brown | 55.39 | 20.17 | 12.95 | 0.1 I | 0.68 | 0.26 | 0.07 | 1.21 | 0. 87 | 0.06 | 8.06 | 85.50 | 0.95 | Matured |
| 51 | LL/25-26/001204/51 | T3/NB/2025/06 | sandstone yellowish brown | 56.77 | 17.70 | 13.60 | 0.12 | 0.55 | 0.76 | 0.15 | 1.30 | 1.01 | 0.04 | 7.70 | 93.90 | 0.90 | Matured |
| 52 | LL/25-26/001204/52 | T3/NB/2025/07 | sandstone yellowish brown | 65.72 | 12.59 | 12.03 | 0.08 | 0.37 | 0.38 | 0.10 | 1.37 | 0.90 | 0.03 | 6.22 | 60.90 | 0.93 | Matured |
| 53 | LL/25-26/001204/53 | T3/NB/2025/08 | sandstone yellowish brown | 57.90 | 22.11 | 4.91 | 0.04 | 0.64 | 2.11 | 0.11 | 1.57 | 1.46 | 0.03 | 8.79 | 76.50 | 0.93 | Matured |
| 54 | LL/25-26/001204/54 | T3/NB/2025/09 | sandstone yellowish white | 56.99 | 24.13 | 5.25 | 0.05 | 0.57 | 1.15 | 0.10 | 1.55 | 1.07 | 0.03 | 8.87 | 64.80 | 0.94 | Matured |
| 55 | LL/25-26/001204/55 | T3/NB/2025/10 | sandstone yellowish white | 59.81 | 23.21 | 3.96 | 0.03 | 0.49 | 1.09 | 0.09 | 1.71 | 2.02 | 0.03 | 7.48 | 63.80 | 0.95 | Matured |
| 56 | LL/25-26/001204/56 | T3/NB/2025/A | sandstone yellowish brown | 60.23 | 18.99 | 9.46 | 0.07 | 0.65 | 0.29 | 0.09 | 1.38 | 1.29 | 0.08 | 7.18 | 89.40 | 0.94 | Matured |
| 57 | LL/25-26/001204/57 | T3/NB/2025/B | sandstone yellowish brown | 54.90 | 22.92 | 8.55 | 0.07 | 0.67 | 0.81 | 0.08 | 1.35 | 1.43 | 0.04 | 8.97 | 85.60 | 0.94 | Matured |
| 58 | LL/25-26/001204/58 | T3/NB/2025/C | sandstone yellowish white | 59.24 | 23.20 | 4.57 | 0.05 | 0.69 | 1.33 | 0.08 | 1.46 | 1.66 | 0.04 | 7.41 | 63.70 | 0.95 | Matured |
| 59 | LL/25-26/001204/59 | T3/NB/2025/D | sandstone yellowish brown | 59.64 | 22.77 | 6.14 | 0.06 | 0.61 | 1.08 | 0.09 | 1.77 | 1.36 | 0.07 | 6.27 | 71.80 | 0.95 | Matured |
|  | Lab. No. | Sample No | Lithology | SiO2 | Al2O3 | Fe2O3 | MnO | MgO | CaO | Na2O | K2O | TiO2 | P2O5 | Loss on Ignition (LOI) | V | Maturity Index = (K2O/(Na2O+K2O)) |  |
|  |  |  |  | (%) |  |  |  |  |  |  |  |  |  |  | (ppm) |  |  |
|  |  | TRENCH - 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 60 | LL/25-26/001204/75 | T5/NB/2025/01 | sandstone Argil. | 54.82 | 13.92 | 20.70 | 0.10 | 0.21 | 0.48 | 0.08 | 1.20 | 0.67 | 0.02 | 7.48 | 51.80 | 0.94 | Matured |
| 61 | LL/25-26/001204/76 | T5/NB/2025/02 | sandstone Argil. | 46.38 | 16.94 | 18.80 | 0.07 | 0.42 | 3.86 | 0.09 | 1.02 | 0.69 | 0.03 | 11.50 | 60.90 | 0.92 | Matured |
| 62 | LL/25-26/001204/77 | T5/NB/2025/03 | sandstone mgd-cgd | 51.78 | 9.84 | 25.05 | 0.18 | 0.26 | 1.64 | 0.08 | 1.10 | 0.55 | 0.02 | 9.36 | 49.10 | 0.93 | Matured |
| 63 | LL/25-26/001204/78 | T5/NB/2025/04 | sandstone mgd-cgd | 53.61 | 9.99 | 25.97 | 0.21 | 0.19 | 0.33 | 0.08 | 1.16 | 0.56 | 0.02 | 7.70 | 54.78 | 0.94 | Matured |
| 64 | LL/25-26/001204/79 | T5/NB/2025/05 | sandstone mgd-cgd | 46.31 | 24.46 | 16.81 | 0.04 | 0.31 | 0.35 | 0.07 | 1.09 | 0.98 | 0.02 | 9.30 | 82.20 | 0.94 | Matured |
| 65 | LL/25-26/001204/80 | T5/NB/2025/06 | sandstone fgd-mgd | 44.84 | 23.94 | l 6.91 | 0.05 | 0.33 | 1.34 | 0.07 | 1.11 | 0.96 | 0.02 | 10.27 | 82.40 | 0.94 | Matured |
| 66 | LL/25-26/001204/81 | T5/NB/2025/07 | sandstone fgd-mgd | 51.00 | 16.55 | 21.57 | 0.09 | 0.22 | 0.17 | 0.08 | 1.33 | 0.72 | 0.02 | 8.17 | 65.00 | 0.94 | Matured |
| 67 | LL/25-26/001204/82 | T5/NB/2025/08 | sandstone mgd-cgd | 49.81 | 15.27 | 23.14 | 0.09 | 0.29 | 0.74 | 0.08 | 1.13 | 0.60 | 0.02 | 8.73 | 93.60 | 0.93 | Matured |
| 68 | LL/25-26/001204/83 | T5/NB/2025/09 | sandstone mgd-cgd | 57.97 | 12.10 | 19.02 | 0.08 | 0.45 | 0.88 | 0.08 | 1.01 | 0.61 | 0.03 | 7.55 | 64.60 | 0.93 | Matured |
| 69 | LL/25-26/001204/84 | T5/NB/2025/10 | sandstone mgd-cgd | 55.86 | 19.77 | 13.00 | 0.04 | 0.45 | 0.42 | 0.08 | 1.05 | 0.95 | 0.03 | 8.04 | 81.0 | 0.93 | Matured |
| 70 | LL/25-26/001204/85 | T5/NB/2025/A | Alluvial soil calcarious | 53.04 | 17.18 | 7.93 | 0.03 | 0.68 | 6.10 | 0.12 | 0.91 | 0.89 | 0.07 | 12.87 | 61.00 | 0.88 | Matured |
| 71 | LL/25-26/001204/86 | T5/NB/2025/B | Alluvial soil calcarious | 56.06 | 27.94 | 2.31 | 0.01 | 0.63 | 1.37 | 0.09 | 0.91 | 1.31 | 0.04 | 9.27 | 66.10 | 0.91 | Matured |
| 72 | LL/25-26/001204/87 | T5/NB/2025/C | Alluvial soil calcarious | 59.78 | 27.00 | 1.94 | 0.01 | 0.67 | 0.61 | 0.07 | 0.83 | 0.88 | 0.03 | 7.96 | 78.05 | 0.92 | Matured |
| 73 | LL/25-26/001204/88 | T5/NB/2025/D | Alluvial soil calcarious | 58.61 | 26.49 | 2.16 | 0.01 | 0.66 | 1.04 | 0.08 | 0.87 | 1.27 | 0.03 | 8.66 | 63.20 | 0.92 | Matured |

ANNEXURE – VII

ANNEXURE – VIII

| Statement showing analytical values of REE, Ga of Bhuj formation in Nadapa Area, Kachchh dist., Gujarat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No. | Lab.No. | Sample No | Lithology | La | Ce | Pr | Nd | Pm | Sm | Eu | Gd | Tb | Dy | Ho | Er | Tm | Yb | Lu | Total REE | Sc | Y | Total REE + (Sc+Y) | Th | U | Ga |
|  |  |  |  | (ppm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | LL/25-26/001204/01 | PI/NB/2025 | Arenaceous clay | 36.05 | 73.85 | 9.50 | 69.25 | — | 5.23 | 1.48 | 7.30 | <1.0 | 3.30 | <1.0 | 6.55 | <1.0 | 2.95 | <1.0 | 215.46 | 15.43 | 23.23 | 254.12 | 21.58 | <1.0 | 26.65 |
| 2 | LL/25-26/001204/04 | P4/NB/2025 | D.grey clay with mica | 36.85 | 79.05 | 9.98 | 68.38 | — | 5.50 | 1.25 | 6.33 | <1.0 | 2.35 | <1.0 | 5.45 | <1.0 | 2.80 | <1.0 | 217.94 | 10.93 | 17.93 | 246.80 | 51.93 | <1.0 | 17.23 |
| 3 | LL/25-26/001204/05 | P5/NB/2025 | sandstone fgd-mgd y/w | 26.98 | 59.73 | 7.70 | 61.80 | — | 3.60 | <1.0 | 5.78 | <1.0 | <1.0 | <1.0 | 6.40 | <1.0 | 2.88 | <1.0 | 174.87 | 5.70 | 11.53 | 192.10 | 37.70 | 1.75 | 9.58 |
| 4 | LL/25-26/001204/07 | P7/NB/2025 | Siltstone friable | 31.90 | 63.90 | 10.18 | 63.18 | — | 4.50 | 1.05 | 8.48 | <1.0 | 1.55 | <1.0 | 5.40 | <1.0 | 4.15 | <I.0 | 194.29 | 9.25 | 15.93 | 219.47 | 35.50 | 7.85 | 17.35 |
| 6 | LL/25-26/001204/32 | T2/NB/2025/07 | sandstone+ clay laminae thinly laminated | 30.48 | 64.98 | 10.05 | 66.53 | — | 4.88 | 1.15 | 8.70 | <1.0 | 1.85 | <1.0 | 6.05 | <1.0 | 3.05 | <1.0 | 197.72 | 9.03 | 16.65 | 223.40 | 42.00 | 8.38 | 17.53 |
| g | LL/25-26/001204/41 | P13/NB/2025 | grey clay interbeded with siltstone | 27.70 | 57.50 | 10.85 | 60.70 | — | 4.63 | 1.13 | 10.13 | <1.0 | 1.95 | <1.0 | 5.60 | <1.0 | 3.35 | 1.10 | 184.64 | 12.00 | 18.33 | 214.97 | 42.28 | 18.80 | 20.78 |
| 9 | LL/25-26/001204/43 | P15/NB/2025 | grey clay interbeded with siltstone | 40.83 | 78.20 | 10.4 | 72.88 | — | 5.70 | 1.45 | 7.40 | <1.0 | 2.35 | <1.0 | 6.00 | <l.0 | 3.43 | <1.0 | 218.24 | 12.08 | 19.08 | 249.40 | 33.73 | <1.0 | 24.33 |
| 10 | LL/25-26/001204/45 | P17/NB/2025 | siltstone y/brown | 49.25 | 98.60 | 12.50 | 84.75 | — | 7.13 | 1.70 | 9.25 | <1.0 | 3.50 | <1.0 | 7.38 | <l.0 | 4.18 | <1.0 | 278.24 | 15.63 | 24.88 | 318.75 | 38.23 | <1.0 | 25.80 |
| 11 | LL/25-26/001204/54 | T3/NB/2025/09 | sandstone yellowish white | 28.30 | 59.08 | 6.13 | 62.58 | — | 3.68 | <1.0 | 5.65 | <1.0 | <1.0 | <1.0 | 5.33 | <1.0 | 2.73 | <1.0 | 173.48 | 6.58 | 11.38 | 191.44 | 43.78 | <1.0 | 11.53 |
| 12 | LL/25-26/001204/78 | T5/NB/2025/04 | sandstone mgd-cgd | 18.15 | 36.73 | 22.68 | 36.90 | — | 4.33 | <1.0 | 24.65 | <1.0 | 0.93 | <1.0 | 3.08 | <1.0 | 7.03 | 3.10 | 157.58 | 5.03 | 14.25 | 176.86 | 86.40 | 131.95 | 18.60 |
| 13 | LL/25-26/001204/87 | T5/NB/2025/C | Alluvial soil calcarious | 41.60 | 83.20 | 10.35 | 72.63 | — | 6.63 | 1.98 | 7.28 | <1.0 | 3.80 | <1.0 | 5.93 | <1.0 | 3.15 | <1.0 | 236.55 | 11.23 | 22.33 | 270.11 | 32.05 | <1.0 | 17.95 |
| 14 | LL/25-26/001204/89 | P19/NB/2025 | Hard Ferru.Sst | 46.88 | 96.53 | 48.55 | 64.38 | — | 11.70 | 2.25 | 44.55 | <1.0 | 3.68 | <1.0 | 5.58 | <1.0 | 9.23 | 5.43 | 338.76 | 11.90 | 25.33 | 375.99 | 129.03 | 245.10 | 31.58 |
| 15 | LL/25-26/001204/90 | P20/NB/2025 | Grey clay+ sst. Laminae | 51.35 | 99.08 | 12.53 | 82.38 | — | 7.38 | 1.75 | 8.75 | <1.0 | 3.85 | <1.0 | 6.80 | <1.0 | 3.50 | <1.0 | 277.37 | 15.53 | 26.28 | 319.18 | 26.88 | <1.0 | 23.63 |
|  |  |  | AVERAGE | 35.87 | 73.11 | 13.95 | 66.64 | — | 5.76 | 1.52 | 11.87 | <1.0 | 2.65 | <1.0 | 5.81 | <1.0 | 4.03 | 3.21 | 220.40 | 10.79 | 19.01 | 250.20 | 47.78 | 68.97 | 20.20 |
|  |  |  | MIN | 18.15 | 36.73 | 6.13 | 36.90 | — | 3.60 | 1.05 | 5.65 | <1.0 | 0.93 | <1.0 | 3.08 | <1.0 | 2.73 | 1.10 | 157.58 | 5.03 | 11.38 | 176.86 | 21.58 | 1.75 | 9.58 |
|  |  |  | MAX | 51.35 | 99.08 | 48.55 | 84.75 | — | 11.70 | 2.25 | 44.55 | <1.0 | 3.85 | <1.0 | 7.38 | <1.0 | 9.23 | 5.43 | 338.76 | 15.63 | 26.28 | 375.99 | 129.03 | 245.10 | 31.58 |

ANNEXURE – IX

| Analytical results of THA, MHA and Reactive silica in NADAPA area, Kachchh dist., Gujarat |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S.no | Lab code | Sample no | %THA | %MHA | % Reactive SiO2 |
| 1 | 1204/89 | P19/NB/2025 | 7.15 | 1.48 | 43.66 |

The analytical results show relatively very high values of reactive silica (43.66%) and low values of Trihydrate alumina and monohydrate alumina values. For Bauxite the reactive silica should be ≤ 5 %.

Detailed description of geochemistry of Katrol and Bhuj formations is given below:

Geochemistry of Katrol Formation:

Three pits (P11, P12, P16) and one trench (T4)   were excavated and collected 3 samples from pits and 14 samples from Trench. All samples were analyzed for Major oxides including TiO,2 Vanadium and 2 samples for REE and Gallium. The results are listed in the following table No – 6.4 and also given in Annexure – V.

Table No:  6.4

Summarized table showing range of major oxides in Katrol formation

| Oxides (%)/ Element (ppm) | Minimum value | Maximum values | Mean value |
| --- | --- | --- | --- |
| SiO2 (%) | 54.92 | 73.26 | 64.01 |
| Al2O3(%) | 0.28 | 28.81 | 4.03 |
| Fe2O3(%) | 2.81 | 15.95 | 8.27 |
| MnO (%) | 0.02 | 0.05 | 0.03 |
| MgO (%) | 0.42 | 1.78 | 1.16 |
| CaO (%) | 0.41 | 17.76 | 8.64 |
| Na2O (%) | 0.12 | 0.35 | 0.21 |
| K2O (%) | 0.91 | 1.59 | 1.20 |
| TiO2 (%) | 0.39 | 1.34 | 0.64 |
| P2O5 (%) | 0.04 | 0.12 | 0.08 |
| LOI (%) | 3.96 | 19.08 | 10.91 |
| V (ppm) | 33.80 | 103.80 | 66.11 |

Analytical results from 17 pit and trench samples reveal low Al₂O₃ and high SiO₂ contents consistent with sandstone/siltstone and soil composition of Katrol formation which lack the chemical signatures needed for lateritic/bauxitic enrichment. The Matanomadh (Madh) Formation which is known to host bauxite/laterite in nearby areas, is completely absent in the present study area.

Further Two samples were analyzed for REE, Ga and the analytical results are furnished below in Table no: 6.5 and also given in Annexure – VII.

Table No: 6.5

Summarized table showing range of REE, Ga in Katrol formation

| REE (ppm) | Minimum value(ppm) | Maximum value (ppm) | Mean Value (ppm) |
| --- | --- | --- | --- |
| Lanthanum (La) | 40.35 | 59.85 | 50.10 |
| Cerium (Ce) | 83.6 | 134.75 | 109.18 |
| Praseodymium (Pr) | 11.18 | 16.18 | 13.68 |
| Neodymium (Nd) | 79.3 | 103.43 | 91.37 |
| Promethium (Pm) | - | - | - |
| Samarium (Sm) | 5.68 | 9.55 | 7.62 |
| Europium (Eu) | 1.1 | 1.15 | 1.13 |
| Gadolinium (Gd) | 7.43 | 11.48 | 9.46 |
| Terbium (Tb) | <0.1 | <0.1 | <0.1 |
| Dysprosium (Dy) | 2.2 | 3.28 | 2.74 |
| Holmium (Ho) | <0.1 | <0.1 | <0.1 |
| Erbium (Er) | 6.93 | 7.55 | 7.24 |
| Thulium (Tm) | <0.1 | <0.1 | <0.1 |
| Ytterbium (Yb) | 3.55 | 3.83 | 3.69 |
| Lutetium (Lu) | <0.1 | <0.1 | <0.1 |
| Total REE | 241.65 | 350.72 | 296.19 |
| Scandium (Sc) | 8.85 | 10.60 | 9.73 |
| Yttrium(Y) | 18.83 | 22.20 | 20.52 |
| (TREE+Sc+Y) | 271.08 | 381.77 | 326.43 |
| Gallium (Ga) | 16.33 | 21.05 | 18.69 |
| Uranium (U) | <0.1 | <0.1 | <0.1 |
| Thorium (Th) | 49.05 | 100.05 | 74.55 |

REE and Ga values from two samples of Katrol formation yielded unencouraging values indicating insufficient concentrations for economic interest.  Katrol sandstone/siltstones are basically quartzo-feldspathic in composition with high SiO2 but no elevated Al2O3 or REE and Gallium.

Geochemistry of Bhuj Formation: A total of 17 pits (P1 to P10, P13 to P15 and P17 to P20) and 4 trenches (T1, T2, T3 and T5) were excavated within the Bhuj Formation to assess its geochemical characteristics. From these workings, 17 representative samples were collected from the pits (one from each pit floor), and 56 samples were collected from the trenches (14 samples from each trench), amounting to 73 samples in total. All the samples were analyzed for major oxides (SiO₂, Al₂O₃, Fe₂O₃, CaO, MgO, Na₂O, K₂O, MnO, P₂O₅, TiO₂) and Vanadium (V), Titanium (Ti). Additionally, 13 selected samples were subjected to detailed determination of Rare Earth Elements (REE) and Gallium (Ga) concentrations. The analytical results of major, trace, and REE–Ga geochemistry are presented in Table no 6.6 and also in Annexure - VI

| Oxides (%)/ Element (ppm) | Minimum value | Maximum values | Mean value |
| --- | --- | --- | --- |
| SiO2 (%) | 44.74 | 77.97 | 59.90 |
| Al2O3(%) | 4.83 | 34.89 | 20.01 |
| Fe2O3(%) | 1.41 | 29.97 | 8.07 |
| MnO (%) | 0.00 | 0.21 | 0.04 |
| MgO (%) | 0.15 | 1.71 | 0.68 |
| CaO (%) | 0.05 | 6.10 | 0.88 |
| Na2O (%) | 0.05 | 3.74 | 0.37 |
| K2O (%) | 0.45 | 2.78 | 1.53 |
| TiO2(%) | 0.52 | 2.03 | 1.05 |
| P2O5(%) | 0.02 | 0.37 | 0.05 |
| LOI (%) | 2.12 | 12.87 | 7.11 |
| V(ppm) | 15.70 | 299.18 | 73.33 |

Table no 6.6 Summarized table showing range of major oxides in Bhuj formation

Al2O3, SiO2, Fe2O3and TiO2 values from the dark grey clays, siltstones, and argillaceous sandstones of Bhuj and Katrol formations aligns well with characteristics of the "Bauxitic clay category" in laterite classification. In this classification, lateritic materials are typically categorized based on their chemical composition and mineralogy, with a Ternary plot often used highlighting aluminium oxide (Al2O3), silica (SiO2), and iron oxide (Fe2O3) as key components. The "Bauxitic clay" category is characterized by relatively low alumina (Al2O3) content with high silica (SiO2) values, consistent with observations of Al2O3 ranging 4.83-34.89% and relatively high SiO2(range 44.74-77.97%). The average TiO2 value of 1.05% (range 0.52-2.02%) also fits within expected ranges for such lateritic materials. Hence, the plotting geochemical data and Ternary diagram results indicate that the sampled materials from Bhuj and Katrol formations are best classified as "Bauxitic clay”, reflecting intense weathering concentration of aluminium oxides with significant silica presence and characteristic TiO2 content seen in lateritic soils and clays. Ternary plot of Classification of laterites is given below in

Fig no: 11

Ternary plot (Fig no: 11)

Further 13 samples were analyzed for REE, Ga and the analytical results are furnished in    Table no: 6.7 and also given in Annexure – VIII.

| REE (ppm) | Minimum value(ppm) | Maximum value (ppm) | Mean Value (ppm) |
| --- | --- | --- | --- |
| Lanthanum (La) | 18.15 | 51.35 | 35.87 |
| Cerium (Ce) | 36.73 | 99.08 | 73.11 |
| Praseodymium (Pr) | 6.13 | 48.55 | 14.25 |
| Neodymium (Nd) | 36.90 | 84.75 | 66.64 |
| Promethium (Pm) | - | - | - |
| Samarium (Sm) | 3.60 | 11.70 | 5.76 |
| Europium (Eu) | 1.05 | 2.25 | 1.52 |
| Gadolinium (Gd) | 5.65 | 44.55 | 11.87 |
| Terbium (Tb) | <0.10 | <0.10 | <0.10 |
| Dysprosium (Dy) | 0.93 | 3.85 | 2.65 |
| Holmium (Ho) | <0.10 | <0.10 | <0.10 |
| Erbium (Er) | 3.08 | 7.38 | 5.81 |
| Thulium (Tm) | <0.10 | <0.10 | <0.10 |
| Ytterbium (Yb) | 2.73 | 9.23 | 4.03 |
| Lutetium (Lu) | 1.10 | 5.43 | 3.21 |
| Total REE | 157.58 | 338.76 | 220.40 |
| Scandium (Sc) | 5.03 | 15.63 | 10.79 |
| Yttrium(Y) | 11.38 | 26.28 | 19.01 |
| TREE+(Sc+Y) | 176.86 | 375.99 | 250.20 |
| Gallium (Ga) | 9.58 | 31.58 | 20.20 |
| Uranium (U) | 1.75 | 245.10 | 68.97 |
| Thorium (Th) | 21.58 | 129.03 | 47.78 |

Table 6.7 Summarized table showing range of REE, Ga in Bhuj Formation

REE and Ga values from 13 samples of Bhuj Formation and 2 samples from Katrol Formation yielded unencouraging values indicating insufficient concentrations for economic interest.  Bhuj sandstone/siltstones are basically quartzo-feldspathic in composition with high SiO2 but no elevated Al2O3 or REE and Gallium.

REE values of pit and trench samples from Bhuj formation are plotted in Spider plot in Fig no: 12 				Fig no: 12

The chondrite-normalized REE spider plot of Nadapa block sediments exhibits pronounced LREE enrichment with relatively flat HREE patterns and consistent negative Eu anomalies. The sub-parallel nature of the curves suggests a uniform felsic provenance; most likely derived from upper continental crustal sources. The marked depletion in Eu reflects plagioclase fractionation and intense chemical weathering, which, in conjunction with elevated potash content, indicates enrichment of K-bearing minerals such as illite and K-feldspar. These features collectively point towards compositionally mature, recycled sediments deposited in a stable tectonic setting, typical of intracratonic or passive margin environments. Both Katrol and Bhuj formation show high sediment maturity index (K2O%/(Na2O%+K2O%) ranging from 0.72-0.96 indicate felsic source with strong chemical weathering and Low REE values. (Annexure – V and VI)

Table No :6.8

Lithology-wise variation of major Oxides

|  | Katrol formation |  |  | Bhuj formation |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Oxides (%)/ Element (ppm) | Min. value | Max. values | Mean value | Min. value | Max. values | Mean value |
| SiO2 (%) | 54.92 | 73.26 | 64.01 | 44.74 | 77.97 | 59.90 |
| Al2O3(%) | 0.28 | 28.81 | 4.03 | 4.83 | 34.89 | 20.01 |
| Fe2O3(%) | 2.81 | 15.95 | 8.27 | 1.41 | 29.97 | 8.07 |
| MnO (%) | 0.02 | 0.05 | 0.03 | 0.00 | 0.21 | 0.04 |
| MgO (%) | 0.42 | 1.78 | 1.16 | 0.15 | 1.71 | 0.68 |
| CaO (%) | 0.41 | 17.76 | 8.64 | 0.05 | 6.10 | 0.88 |
| Na2O (%) | 0.12 | 0.35 | 0.21 | 0.05 | 3.74 | 0.37 |
| K2O (%) | 0.91 | 1.59 | 1.20 | 0.45 | 2.78 | 1.53 |
| TiO2(%) | 0.39 | 1.34 | 0.64 | 0.52 | 2.03 | 1.05 |
| P2O5(%) | 0.04 | 0.12 | 0.08 | 0.02 | 0.37 | 0.05 |
| LOI (%) | 3.96 | 19.08 | 10.91 | 2.12 | 12.87 | 7.11 |
| V(ppm) | 33.80 | 103.80 | 66.11 | 15.70 | 299.18 | 73.33 |

Table no: 6.9 Lithology-wise variation of REE and Gallium

| REE (ppm) | Katrol formation |  |  | Bhuj Formation |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Min. value (ppm) | Max. value (ppm) | Mean Value (ppm) | Min. value (ppm) | Max. value (ppm) | Mean Value (ppm) |
| Lanthanum (La) | 40.35 | 59.85 | 50.10 | 18.15 | 51.35 | 35.87 |
| Cerium (Ce) | 83.6 | 134.75 | 109.18 | 36.73 | 99.08 | 73.11 |
| Praseodymium (Pr) | 11.18 | 16.18 | 13.68 | 6.13 | 48.55 | 14.25 |
| Neodymium (Nd) | 79.3 | 103.43 | 91.37 | 36.90 | 84.75 | 66.64 |
| Promethium (Pm) | - | - | - | - | - | - |
| Samarium (Sm) | 5.68 | 9.55 | 7.62 | 3.60 | 11.70 | 5.76 |
| Europium (Eu) | 1.1 | 1.15 | 1.13 | 1.05 | 2.25 | 1.52 |
| Gadolinium (Gd) | 7.43 | 11.48 | 9.46 | 5.65 | 44.55 | 11.87 |
| Terbium (Tb) | <0.1 | <0.1 | <0.1 | <0.10 | <0.10 | <0.10 |
| Dysprosium (Dy) | 2.2 | 3.28 | 2.74 | 0.93 | 3.85 | 2.65 |
| Holmium (Ho) | <0.1 | <0.1 | <0.1 | <0.10 | <0.10 | <0.10 |
| Erbium (Er) | 6.93 | 7.55 | 7.24 | 3.08 | 7.38 | 5.81 |
| Thulium (Tm) | <0.1 | <0.1 | <0.1 | <0.10 | <0.10 | <0.10 |
| Ytterbium (Yb) | 3.55 | 3.83 | 3.69 | 2.73 | 9.23 | 4.03 |
| Lutetium (Lu) | <0.1 | <0.1 | <0.1 | 1.10 | 5.43 | 3.21 |
| Total REE | 241.65 | 350.72 | 296.19 | 157.58 | 338.76 | 220.40 |
| Scandium (Sc) | 8.85 | 10.60 | 9.73 | 5.03 | 15.63 | 10.79 |
| Yttrium(Y) | 18.83 | 22.20 | 20.52 | 11.38 | 26.28 | 19.01 |
| TREE+(Sc+T) | 271.08 | 381.77 | 326.43 | 176.86 | 375.99 | 250.20 |
| Gallium (Ga) | 16.33 | 21.05 | 18.69 | 9.58 | 31.58 | 20.20 |
| Uranium (U) | <0.1 | <0.1 | <0.1 | 1.75 | 245.10 | 68.97 |
| Thorium (Th) | 40.35 | 59.85 | 50.10 | 21.58 | 129.03 | 47.78 |

Based on the geochemical analysis of the samples collected from pits and trenches within the investigated area, the concentrations of bauxite (Al₂O₃), gallium (Ga), vanadium (V), titanium (TiO₂), and REEs are found to be low and below the threshold required for economic consideration. The obtained values do not indicate any significant enrichment or mineralization of these commodities. Hence, the area does not hold promising potential for bauxite, Ga, V, Ti, or REE mineralization at the explored scale.

Elemental Distribution maps of Al2O3, TiO2, Gallium, Vanadium and Total REE are prepared based on analytical results and are described in the following paragraphs. For trench samples, average of 14 sample values was taken for plotting purpose.

Distribution maps of Al2O3%, TiO2%, Vanadium(ppm), Gallium(ppm) and Total REE are given below in fig nos: 13,14,15,16 and 17 respectively.

Al2O3: Aluminium oxide (Al₂O₃) is the major constituent of bauxite; however, the Katrol and Bhuj formations in the study area comprise predominantly sandstone, siltstone, shale, and clay, with no evidence of lateritization or true bauxite horizons. The analysed samples show Al₂O₃ values ranging from 0.28 to 28.81% in the Katrol Formation and 4.83 to 34.89% in the Bhuj Formation. Although some samples display moderately high Al₂O₃, the corresponding SiO₂ contents are also high, indicating that the aluminium is primarily silicate-bound in clay minerals rather than present as gibbsite or boehmite. Higher Al₂O₃ values correlate strongly with grey clay lithologies, reflecting increased proportions of kaolinite, illite, or smectite.

Fig No: 13

TiO2: Titanium, reported as titanium dioxide (TiO₂), shows moderate concentrations across both formations. In the Katrol Formation, TiO₂ values range from 0.39 to 1.34%, while in the Bhuj Formation, they vary from 0.52 to 2.02%. These concentrations fall within the typical range expected for siliciclastic sedimentary rocks, where Ti is commonly associated with heavy minerals such as ilmenite, rutile, and titaniferous magnetite, as well as clay minerals containing structural Ti. Although the values are moderate, they do not indicate any significant Ti enrichment or titanium-bearing mineralization. Instead, the observed TiO₂ levels likely reflect the detrital input from felsic to intermediate provenance sources and the presence of fine-grained clay and ferruginous components within the sedimentary units.

Fig No: 14

Vanadium: Vanadium (V) concentrations in the study area show a modest range. In the Katrol Formation, V varies from 33.80 to 103.80 ppm, while in the Bhuj Formation, it ranges from 15.70 to 299.18 ppm. Although a few samples from the Bhuj Formation exceed the average crustal abundance of ~120 ppm, most values fall within or only slightly above the natural background levels typically observed in siliciclastic sedimentary rocks.

The recorded concentrations therefore do not indicate any significant enrichment or vanadium mineralization, and the values are considered low to marginal from an economic perspective. The observed V content likely reflects the normal geochemical background associated with clay-rich intervals, iron oxide coatings, and detrital minerals, rather than any ore-grade accumulation.

Fig No: 15

Gallium: Gallium (Ga) concentrations in the study area are generally low. In the Katrol Formation, Ga ranges from 16.33 to 21.05 ppm, while in the Bhuj Formation, values vary from 9.58 to 31.58 ppm. Although a few samples marginally exceed the crustal abundance value of ~19 ppm, the majority fall at or below this. These concentrations are considered low and not indicative of any meaningful Gallium enrichment. The Ga present is likely associated with alumino-silicate minerals such as clays and feldspars, reflecting the normal background levels of siliciclastic sediments rather than any economic accumulation. Hence, the Gallium content in both formations is not encouraging from an exploration perspective.

Fig No: 16

Total REE (∑REE) Distribution:

The Rare Earth Elements (REEs), comprising the fifteen lanthanide elements from atomic number 57 to 71 (La, Ce, Pr, Nd, Pm, Sm, Eu, Gd, Tb, Dy, Ho, Er, Tm, Yb and Lu), were summed to derive the Total REE (∑REE) values for each analysed sample. In the Katrol Formation, ∑REE ranges from 241.32 to 350.72 ppm, while in the Bhuj Formation, it varies from 157.58 to 338.76 ppm. Although these concentrations reflect normal REE levels typically associated with felsic-derived siliciclastic sediments, they remain relatively low and do not indicate any significant REE enrichment. The values are well within the background range for clay-rich sandstones and shales and are therefore not encouraging from an economic or mineralization perspective.

Fig No: 17

6.1.10 Ore Zones: Based on the integrated geological, structural, subsurface, and geochemical evaluation, the Nadapa block does not exhibit any potential ore zones for bauxite or associated critical mineral (Ga, V, Ti, REE) mineralization. The area is therefore considered non-prospective, and no further exploration is recommended at this stage.

6.2 	Geophysical Exploration: Geophysical Exploration have not been carried out in the present exploration as it was out of the scope of the approved quantum of work.

6.3 	Geochemical Exploration: During the present exploration pitting work was undertaken to obtain fresh representative samples from a depth of 1 meter. A total of 20 pits were excavated using a JCB, each measuring 1.0 m × 1.0 m × 1.0 m, resulting in an individual volume of 1.0 m³ and a cumulative excavated volume of 20 m³. After cleaning the floor of the pit, one representative sample of 2-3kg was collected from each pit and properly labelled.

Similarly, five trenches were excavated in the study area, each measuring 10m in length, 1m in width, and 1m in depth, amounting to a cumulative excavated volume of 50 m³. After cleaning the floor of the trench, 10 representative samples were collected at an interval of 1m from bottom of the trench and 4 samples from four walls of trench. So total 14 samples from each trench and total 70 samples from five trenches. Each sample weighs 2-3kg.

All 90 samples were sent to field camp at Padhar village. Each sample was crushed/grinded and passed through 3 different sieves to get final laboratory sample of -200 mesh size. Coning and Quartering was done at each stage to reduce the size of sample without losing the homogeneity of sample. Finally, 90 samples were sent to Lucid laboratories Pvt Ltd, Hyderabad (NABL accredited lab) for analysis for major oxides by XRD, REE (14 elements by ICPMS) and MH, TH and reactive silica. Analytical results were given in Annexure-II, III, IV, V and also enclosed in plate no: IX (Assay value map of pits) and plate no: X (Assay value map of trenches).

Nine check samples were sent to Shiva Analyticals India Pvt Ltd, Bengaluru for major oxides and one sample for REE.

The nature, quality and appropriateness of Chemical analysis: A total 90 Nos of surface samples from pits & trenches have been analysed for major oxides by XRF (SiO2, Al2O3, F2O3, MnO MgO, Na2O, K2O, P2O5, TiO2, and vanadium and 15 samples for REEs (La, Ce, Pr, Nd, Sm, Eu, Gd, Tb, Dy, Ho, Er, Tm, Yb, Lu) and Gallium  by ICP-MS instrument at Lucid Laboratories Pvt Ltd (NABL accredited lab) ,Hyderabad. Lucid laboratories Pvt Ltd has used latest equipment for analysis namely WD XRF model Analytical Axios max for major oxides and ICP MS Model: Agilent 7850 for REE and Gallium.

Methodology of Chemical Analyses by ICP-MS: Representative rock/soil samples were dried, crushed, and pulverized to −200 mesh with the help of pestle and mortar and sent to the chemical lab. About 0.1–0.2 g of sample was digested using HF–HNO₃–HClO₄ (silicate samples) / aqua regia (soil) and diluted in 1–2% HNO₃. Multi-element analysis was carried out using ICP-MS under optimized operating conditions. Calibration was performed using certified reference standards with internal standards for drift correction. Quality control was ensured through procedural blanks, CRMs, and duplicate analyses. Analytical precision was generally better than ±5%, with detection limits in the ppb range.

Nine check samples were sent to Shiva Analyticals India Pvt Ltd, Bengaluru for   major oxides and one sample for REE.

Statistical analysis of geochemical data is given below in table nos:6.10, 6.11

Table No 6.10: Summarised table showing statistical analysis of Primary Vs. External Check sample assay of

SiO2, Al2O3, Fe2O3, MnO and MgO

| Sr. No | Comparison Parameter | SiO2(%) |  | Al₂O₃ (%) |  | Fe2O3(%) |  | MnO (%) |  | MgO (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRIMARY | CHECK | PRIMARY | CHECK | PRIMARY | CHECK | PRIMARY | CHECK | PRIMARY | CHECK |
| 1 | No. of sample pairs | 9 |  | 9 |  | 9 |  | 9 |  | 9 |  |
| 2 | Arithmetic mean | 58.83 | 69.28 | 17.87 | 11.99 | 10.54 | 8.07 | 0.06 | 0.14 | 0.68 | 0.29 |
| 3 | Standard Deviation | 9.58 | 11.83 | 9.87 | 4.89 | 12.32 | 10.96 | 0.08 | 0.05 | 0.37 | 0.18 |
| 4 | Standard error of mean | 3.19 | 3.94 | 3.29 | 1.63 | 4.11 | 3.65 | 0.03 | 0.03 | 0.12 | 0.06 |
| 5 | Variance | 91.85 | 139.90 | 97.41 | 23.89 | 151.88 | 120.03 | 0.01 | 0.00 | 0.14 | 0.03 |

Table No 6.11: Summarised table showing statistical analysis of Primary Vs. External Check sample assay of

Cao, Na2O, K2O, TiO2 and P2O5

| Sr. No | Comparison Parameter | CaO (%) |  | Na2O (%) |  | K2O (%) |  | TiO2(%) |  | P2O5(%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRIMARY | CHECK | PRIMARY | CHECK | PRIMARY | CHECK | PRIMARY | CHECK | PRIMARY | CHECK |
| 1 | No. of sample pairs | 9 |  | 9 |  | 9 |  | 9 |  | 9 |  |
| 2 | Arithmetic mean | 1.48 | 0.84 | 0.44 | 0.15 | 1.31 | 2.04 | 1.25 | 1.08 | 0.09 | 0.11 |
| 3 | Standard Deviation | 2.34 | 1.01 | 0.60 | 0.06 | 0.52 | 1.08 | 0.50 | 0.33 | 0.11 | 0.13 |
| 4 | Standard error of mean | 0.78 | 0.34 | 0.20 | 0.03 | 0.17 | 0.36 | 0.17 | 0.11 | 0.04 | 0.05 |
| 5 | Variance | 5.45 | 1.03 | 0.36 | 0.00 | 0.27 | 1.16 | 0.25 | 0.11 | 0.01 | 0.02 |

Table No: 6.12 Statement showing analysis of External check samples for major oxides and its comparison with primary samples of Nadapa area, Kachchh district, Gujarat

| S.no | Reg No | Primary /check | Sample name | SiO2 (%) | Al2O3 (%) | Fe2O3 (%) | MnO (%) | MgO (%) | CaO (%) | Na2O (%) | K2O (%) | TiO2 (%) | P2O5 (%) | LOI (%) | Total (%) | V (ppm) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1204/2 | Primary sample | P2/NB/2025 | 54.021 | 34.051 | 1.417 | 0.011 | 0.603 | 0.191 | 0.225 | 0.642 | 1.471 | 0.044 | 7.18 | 99.86 | 82.5 | Lucid lab Hyderabad |
| 1 | G2296-19 | check sample | P35/NB/2025 | 68.68 | 19.31 | 1.42 | <0.05 | 0.32 | 0.15 | <0.08 | 0.78 | 1.28 | 0.05 | 7.72 | 99.71 |  | Shiva Lab Bangalore |
| 2 | 1204/10 | Primary sample | P10/NB/2025 | 60.901 | 24.03 | 3.899 | 0.012 | 0.728 | 0.324 | 1.568 | 1.771 | 1.41 | 0.055 | 5.07 | 99.77 | 77.8 | Lucid lab Hyderabad |
| 2 | G2296-20 | check sample | P36/NB/2025 | 70.77 | 15.11 | 3.26 | <0.05 | 0.3 | 0.2 | 0.22 | 2.81 | 1.25 | 0.06 | 5.76 | 99.74 |  | Shiva Lab Bangalore |
| 3 | 1204/25 | Primary sample | P11/NB/2025 | 66.543 | 20.831 | 4.572 | 0.02 | 0.422 | 0.412 | 0.158 | 1.526 | 1.34 | 0.071 | 3.96 | 99.86 | 85.75 | Lucid lab Hyderabad |
| 3 | G2296-21 | check sample | P37/NB/2025 | 73.48 | 12.76 | 3.99 | <0.05 | 0.17 | 0.24 | <0.08 | 2.27 | 1.6 | 0.09 | 4.94 | 99.54 |  | Shiva Lab Bangalore |
| 4 | 1204/89 | Primary sample | P19/NB/2025 | 37.646 | 9.713 | 36.973 | 0.199 | 0.373 | 2.031 | 0.064 | 0.451 | 0.74 | 0.37 |  |  |  | Lucid lab Hyderabad |
| 4 | G2296-22 | check sample | P38/NB/2025 | 42.71 | 9.24 | 33.49 | 0.17 | 0.27 | 2 | <0.08 | 0.59 | 0.8 | 0.41 | 9.83 | 99.51 |  | Shiva Lab Bangalore |
| 5 | 1204/23 | Primary sample | T1/NB/2025/C | 62.019 | 20.106 | 4.743 | 0.016 | 1.418 | 0.783 | 1.421 | 2.005 | 1.006 | 0.05 | 6.23 | 99.8 | 64.8 | Lucid lab Hyderabad |
| 5 | G2296-23 | check sample | T30/NB/2025/C | 64.44 | 17.85 | 4.05 | <0.05 | 0.7 | 0.69 | 0.2 | 3.45 | 0.92 | 0.05 | 7.43 | 99.78 |  | Shiva Lab Bangalore |
| 6 | 1204/36 | Primary sample | T2/NB/2025/A | 64.959 | 18.545 | 4.634 | 0.025 | 0.873 | 0.639 | 0.153 | 1.359 | 1.824 | 0.05 | 6.68 | 99.74 | 78.6 | Lucid lab Hyderabad |
| 6 | G2296-24 | check sample | T31/NB/2025/A | 75.98 | 11.49 | 2.71 | <0.05 | 0.37 | 0.52 | 0.1 | 2.9 | 1.16 | 0.06 | 4.45 | 99.74 |  | Shiva Lab Bangalore |
| 7 | 1204/55 | Primary sample | T3/NB/2025/10 | 59.81 | 23.214 | 3.968 | 0.039 | 0.492 | 1.095 | 0.09 | 1.712 | 2.028 | 0.037 | 7.48 | 99.97 | 63.8 | Lucid lab Hyderabad |
| 7 | G2296-25 | check sample | T32/NB/2025/10 | 79.09 | 10 | 1.84 | <0.05 | 0.15 | 0.48 | 0.09 | 3.02 | 1.36 | 0.05 | 3.53 | 99.61 |  | Shiva Lab Bangalore |
| 8 | 1204/70 | Primary sample | T4/NB/2025/10 | 69.933 | 0.316 | 8.677 | 0.03 | 0.988 | 7.524 | 0.15 | 1.134 | 0.86 | 0.078 | 10.12 | 99.81 | 50.7 | Lucid lab Hyderabad |
| 8 | G2296-26 | check sample | T33/NB/2025/10 | 83.68 | 3.76 | 2.86 | <0.05 | 0.24 | 3.08 | 0.11 | 1.13 | 0.71 | <0.05 | 4.06 | 99.63 |  | Shiva Lab Bangalore |
| 9 | 1204/78 | Primary sample | T5/NB/2025/04 | 53.616 | 9.994 | 25.975 | 0.218 | 0.194 | 0.336 | 0.087 | 1.169 | 0.56 | 0.022 | 7.7 | 99.87 | 54.78 | Lucid lab Hyderabad |
| 9 | G2296-27 | check sample | T34/NB/2025/04 | 64.68 | 8.33 | 19 | 0.11 | 0.09 | 0.21 | <0.08 | 1.37 | 0.64 | <0.05 | 5.29 | 99.72 |  | Shiva Lab Bangalore |

Table No: 6.13 Statement showing analysis of one External check sample for REE and its comparison with primary sample analysis of Nadapa Area, Kachchh District, Gujarat

| Primary /check | Sample No | In ppm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | La | Ce | Pr | Nd | Sm | Eu | Gd | Tb | Dy | Ho | Er | Tm | Yb | Lu | Total REE | Sc | Y | Th | U | Ga | V | Ti as Tio2 % by mass |
| Primary sample | P19/NB/2025 | 46.88 | 96.53 | 48.55 | 64.38 | 11.7 | 2.25 | 44.55 | <1.0 | 3.68 | <1.0 | 5.58 | <1.0 | 9.23 | 5.43 | 339 | 11.9 | 25.33 | 129.03 | 245.1 | 31.58 | 299.18 | 1.47 |
| Check sample | P-38/NB/2025 | 63.8 | 130 | 11.7 | 40.4 | 8.66 | 2.87 | 11.2 | 1.17 | 6.9 | 1.14 | 3.63 | <0.5 | 3.2 | <0.5 | 285 | 13.8 | 29.7 | 17.9 | 4.35 | 10.6 | NA | 1.28 |

6.4    	DGPS survey of boundary points

The present exploration for Bauxite, Ga, V, Ti, and REE was carried out at the G-4 stage. To ensure precise documentation of sample locations and geological features, a handheld GPS device (Garmin 12H model) was used for recording the coordinates and elevation of all pit and trench sites, as well as for outcrop mapping across the study area. All spatial data were captured in the UTM coordinate system, referenced to the WGS 1984 datum, enabling accurate integration of field data with GIS-based mapping and interpretation. The Reduced level (RL) was carry forward from Kukma railway station where the MSL value is 122.82m.

The four boundary corner points of the study area were determined using DGPS (E-Survey instruments: E-800 Base and E600-H Rover) to achieve high-precision spatial control. The system provided a static accuracy of Horizontal: 2.5 mm + 0.1 ppm and Vertical: 3.5 mm + 0.4 ppm, while RTK mode offered Horizontal: 8 mm + 1 ppm and Vertical: 15 mm + 1 ppm accuracy. As part of the survey control framework, two temporary benchmarks were established within the study area and designated as NDP-TBM-1 and NDP-TBM-2. These temporary benchmarks may be used as reference points for subsequent mapping and geospatial integration.

Table No: 6.14 The details of coordinates and elevation of 4 cardinal points and

Two temporary bench marks as determined by DGPS Survey

- TBM – 1 and 2: Temporary Bench marks established in Nadapa Area.

CHAPTER - VII

7.0     Integration of Geology, geophysics (with available aero geophysical    data) and geochemical exploration data and the interpretation:

This chapter is not applicable for stratiform type deposits like bauxite. Shape files in GIS format for the themes like lithology, Oriented structures, Outcrop map, sample locations of pits and trenches are prepared and various elemental distribution maps viz Al2O3, G, V, Tio2 and REE etc have been prepared and discussed in the Chapter 6.

CHAPTER - VII

8.0 	Mineral Prospect:

The study area is represented by two major stratigraphic units, namely the Katrol Formation (Late Jurassic to Early Cretaceous) and the Bhuj Formation (Early Cretaceous). Both formations predominantly comprise sandstones, siltstones, shales, and clay beds. Lithologic ally, these formations represent clastic sedimentary sequences deposited in fluvial to deltaic environments. Such litho-units lack the lateritic profiles, aluminous enrichment zones, and favourable weathering conditions required for the formation and preservation of economic bauxite deposits. In contrast, the neighbouring regions where bauxite mineralisation is reported are associated with the Matanomadh Formation, which is known to host lateritic bauxite horizons. This formation is completely absent within the Nadapa study area, eliminating the primary stratigraphic control necessary for bauxite development. Analytical results of pits and trench samples further confirm the absence of economically significant Al₂O₃ enrichment and related critical mineral concentrations.

CHAPTER – 1X

9.0 	Exploration by scout drilling

# Although drilling was proposed in the NQT, during  9th TCC-II review meeting held on 29th May   did not recommend  drilling in this block. This decision was based on the unencouraging analytical results obtained from the pit and trench samples, which demonstrated low and non-economic concentrations of the targeted commodities. Therefore, further subsurface exploration through drilling was not considered warranted at this stage.

# CHAPTER – X

# 10.0 	Resource Estimation:

# Based on the integrated evaluation of geological setting, stratigraphic framework, subsurface investigations, and geochemical data, the Nadapa area does not exhibit any potential for Bauxite mineralization and associated critical minerals (Ga, V, Ti, REE).

CHAPTER – XI

11.0 	Conclusion and recommendation

The G4 exploration carried out over 6.2 sq. km in the Nadapa area, Kachchh district, aimed to assess the potential for bauxite and associated critical minerals (Ga, V, Ti and REE). Geological mapping on a 1:12,500 scale established the presence of only two litho-units—Katrol Formation and Bhuj Formation—neither of which is known to host lateritic or bauxitic enrichment in the region. Subsurface exploration through 20 pits and 5 trenches provided representative samples, and geochemical analysis of 90 samples confirmed uniformly low concentrations of the targeted elements.

No signatures of bauxitisation, lateritisation, or favourable stratigraphy were identified, and the Matanomadh (Madh) Formation, which hosts bauxite deposits in neighbouring areas, is completely absent in the study area.

Based on the combined geological, subsurface, and geochemical evidence, the Nadapa Area does not exhibit any potential for bauxite or related critical mineral mineralisation. Thus, the area is considered non-prospective for further exploration at this stage.

CHAPTER – XII

12.0	EXPENDITURE:

The total expenditure incurred for the execution of the project “Reconnaissance survey (G4) for Bauxite, Ga, V, Ti and REE in Nadapa area, Kachchh Dist., Gujarat is Rs35.41 Lakhs (including GST), Rupees thirty-five lakhs forty-one thousand only against the initial approved cost of 53.73 lakhs as detailed below. This has been approved by 26th TCC-II held on 1st & 2nd April,2026 and also in 8th PSC held on 30th April 2026.

Table No: 12.1 Details of actual expenditure incurred for Reconnaissance survey (G4) in Nadapa area, Kachchh Dist., Gujarat.

CHAPTER – XIII

13.0	REFERENCES

- Agrawal, S.K. (1957). Kutch Mesozoic- A Study of Jurassic of Kutch with Special Reference to Jura dome, Pal.Soc. Ind, V-2, pp. 119-130

- ​Biswas, S.K. (1971). A Note on Geology of Kachchh, Quat. Jour. Geol. Min. Met. Soc. of India, Vol.43 No. (4), pp.223-225

- ​Biswas, S.K. (1974). Mesozoic Rock Stratigraphy of Kutch, Gujarat, Quat. Jourl. Geol. Min. Met. Soc. of India, Vol.49 (3,4), pp.1-54

- ​Biswas, S.K., 1987. Regional tectonic framework, structure and evolution of the western marginal basins of India. Tectonophysics, 135, pp. 307-327.

- ​Ghevariya, Z.G., Sabale, A.B. and Rakshit P. (1984). Geological mapping of the Mesozoic Rocks, Kachchh district, Gujarat. (Unpublished progress Report for the F.S.(1981-82)

- ​Ghevariya, Z.G., Sable, A.B. and Rakshit P. and Shah S.C (1982). Vertebrate fossils from non-marine Lameta sequence from Kachchh, GSI, News Bull.

- ​Ghevariya, Z.G. (1985). Geological Mapping of Mesozoic Rocks around Anjar. Progress Report for the F.S.1984-85 of Geological Survey of India (Unpublished)

- ​Ghevariya, Z.G. (1986). Geological Survey of India, News let.3 (2) pp.8 and 9

- Ghevariya, Z.G. and Srikarni, C. (1986-1987). Report on the systematic geological mapping of the area west of Anjar, Kachchh district, Gujarat. (unpublished progressed report, GSI)

- ​Ghevariya, Z.G. and Srikanri, C. (1987). Report on the systematic geological mapping of the area west of Anjar, Kachchh district. (Parts of toposheet No. 41E/16 and 411/4)

- ​Gregory (1893). Jurassic fauna of Kutch, Pal. Ind. Geol. Surv. of India, The Echinoids Sec. 9 2(1). The corals 2(2), pp.115

- GSI’s online data portal – Bhukosh (for NGCM raw data source, Block Geology and Regional Geology)

- ​Kitchin (1910). A comparative study of cryptogamic spores and their stratigraphic value, Sec. on Palaeo. Palynology and India Stratigraphy, pp.150-155

- ​Kuenen, PH.H. (1953a). Significant features of graded bedding. Bull. Am. Ass. Petrol. Geologists 37, 1044-1066

- Merh, S.S. (1995). Geology Of Gujarat, Geological society of India Bangalore.

- Miscellaneous publication, No. 30, part.14. Geology and mineral resources of Gujarat, Daman Diu, (2012)

- ​Rajnath (1933). Contribution to the Stratigraphy of Kutch district, J1. geol. Min. Met. Soc. India, Vol. IV (4)

- Sahu, B.K. and Singh, P.K. (2006). Special thematic mapping in the area along Kachchh mainland fault, Gujarat.

- ​Spath, L.F. (1933). Ammonite fauna of Kachchh with remarks on their association owing to the beds and probable age. Rec. Geol. Surv. India, Vol.4, Pt.4

- ​Waggen (1876). The Jurassic fauna of Kutch, Pal.Ind. Geol.Surv. India,9(1)

- ​Williams E. (1960). Intrastratal flow and convolute folding Geol.Mag.97,208-214

- ​Wynne, A.B. (1872). Geology of Kutch, Mem. GSI, Vol. IX, pp.1-293

Acknowledgements

Critical Mineral Trackers (CMT) expresses its sincere gratitude to the Commissioner of Geology and Mining (CGM), Gandhinagar, Gujarat for granting the No Objection Certificate (NOC) to undertake the Reconnaissance Survey (G4) for Bauxite, Gallium (Ga), Vanadium (V), Titanium (Ti), and Rare Earth Elements (REE) in the Nadapa area, Kachchh district, Gujarat, and for extending necessary administrative and logistical support during the execution of the field programme.

The company gratefully acknowledges the financial assistance and guidance provided by National Mineral Exploration and Development Trust (NMEDT) for successful implementation of the project.

The support and cooperation extended by the local residents of Nadapa village during field investigations are also sincerely appreciated.

CHAPTER – XIV

14.0	LOCALITY INDEX

Location of villages/ factories/Railway stations/Airport around Nadapa Area

Table no: 14.1

| Locality | Latitude | Longitude |
| --- | --- | --- |
|  | (Degree decimal) | (Degree decimal) |
| Nadapa village | 23.318733° | 69.906531° |
| China clay plants within Nadapa area |  |  |
| China clay plant-1 | 23.325437° | 69.914530° |
| China clay plant-2 | 23.328267° | 69.914493° |
| China clay plant-3 | 23.323118° | 69.901743° |
| China clay plant-4 | 23.328961° | 69.924827° |
| China clay mines & plants located outside Nadapa Area |  |  |
| Sree Ram group of China clay plants | 23.286319° | 69.920692° |
| Bhuj Airport | 23.275506° | 69.663865° |
| Bhuj city | 23.242572° | 69.664388° |
| Bhuj Railway station | 23.265908° | 69.678027° |
| BKT tyre factory | 23.246721° | 69.861985° |
| Dhaneti village | 23.256210° | 69.919325° |
| Kukma village | 23.216733° | 69.777743° |
| Padhar village | 23.240591° | 69.824396° |
| Ratnal Railway station | 23.185004° | 69.903495° |
| Suzlon wind mill factory | 23.246120° | 69.845665° |

kLIST OF ANNEXURES

| Annexure No | Title | Page |
| --- | --- | --- |
| I | Details of coordinates and elevation of 4 cardinal points and two temporary bench marks as determined by DGPS Survey | 17 &137 |
| II | Statement showing analytical results of major oxides (by XRF) of 20 pit samples in Nadapa area, Kachchh district, Gujarat | 89 - 90 |
| III | Statement showing analytical results of major oxides (by XRF) of 70 Trench samples in Nadapa area, Kachchh district, Gujarat | 91 - 97 |
| IV | Statement showing analytical values of REE, Ga, V, Ti from pit and trench samples in Nadapa area, Kachchh district, Gujarat | 98 |
| V | Statement showing analytical results of major oxides (by XRF) of samples in Katrol Formation Nadapa area, Kachchh district, Gujarat | 99 - 100 |
| VI | Statement showing analytical results of major oxides (by XRF) of samples in Bhuj Formation Nadapa area, Kachchh district, Gujarat | 101 - 108 |
| VII | Statement showing analytical values of REE, Ga from Katrol formation in Nadapa area, Kachchh district, Gujarat | 109 |
| VIII | Statement showing analytical values of REE, Ga from Bhuj formation in Nadapa area, Kachchh district, Gujarat | 110 - 111 |
| IX | Analytical results of MHA / THA and Reactive silica in Nadapa area, Kachchh district, Gujarat | 112 |
| X | Statement showing details of 90 Pit & Trench sample analysis of Major oxides in Nadapa area, Kachchh district, Gujarat, (as received from Lucid laboratory, Hyderabad) | 150 - 153 |
| XI | Statement showing details of 15 Pit & Trench sample analysis of REE, Ga in Nadapa area, Kachchh district, Gujarat, (as received from Lucid laboratory, Hyderabad.) | 154 |
| XII | Analytical results of MHA / THA and Reactive silica in Nadapa area, Kachchh district, Gujarat, (as received from Lucid laboratory, Hyderabad) | 155 |
| XIII | Statement showing 9 Check sample analysis of Major oxides, (as received from Shiva Analyticals India Pvt. Ltd, Bangalore) | 156 |
| XIV | Statement showing one Check sample analysis of REE and Gallium (as received from Shiva Analyticals India Pvt. Ltd, Bangalore) | 157 |
| XV | Detail Report on complete Petrographic study carried out on Three bed rock samples Nadapa area, Kachchh district, Gujarat (as received from Petrology division GSI, Southern Region, Hyderabad) | 158 - 165 |
| XVI | Detail report on Minerographic study (XRD) carried out on bed rock samples of Nadapa area, Kachchh district, Gujarat (as received form Shiva Analyticals India Pvt. Ltd, Bangalore) | 166 - 183 |
| XVII | Administrative and Financial approval from NMET to carry out exploration work (G4) in Nadapa Area, Dist: Kachchh, Gujarat | 184 - 190 |

ANNEXURE – X

Statement showing details of 90 Pit & Trench sample analysis of Major oxides in Nadapa area, Kachchh district, Gujarat, (as received from Lucid laboratory, Hyderabad)

ANNEXURE – XI

Statement showing details of 15 Pit & Trench sample analysis of REE, Ga in Nadapa area, Kachchh district, Gujarat, (as received from Lucid laboratory, Hyderabad.)

ANNEXURE – XII

Analytical results of MHA / THA and Reactive silica in Nadapa area, Kachchh district, Gujarat, (as received from Lucid laboratory, Hyderabad)

ANNEXURE – XIII

Statement showing 9 Check sample analysis of Major oxides, (as received from Shiva Analyticals India Pvt. Ltd, Bangalore)

ANNEXURE – XIV

Statement showing one Check sample analysis of REE and Gallium (as received from Shiva Analyticals India Pvt. Ltd, Bangalore)

ANNEXURE – XV

Detailed Report on complete Petrographic study carried out on three bedrock samples, Nadapa area, Kachchh district, Gujarat (as received from Petrology division GSI, Southern Region, Hyderabad)

ANNEXURE – XVI

Statement showing 9 Check sample analysis of Major oxides, (as received form Shiva Analyticals India Pvt. Ltd, Bangalore)

Sample No : P37/NB/2025 (original no : P11/NB/2025)

Sample No: P38/NB/2025(Original no: P19/NB/2025)

Sample No: T34/NB/2025/04( Original no: T5/NB/2025/04)

ANNEXURE - XVII

Administrative and Financial approval from NMET to carry out exploration work (G4) in Nadapa Area, Dist: Kachchh, Gujarat

| Estimate Cost for Reconnaissance Survey (G4) for Bauxite, Ga, V, Ti & REE in Nadapa area Kachchh District, Gujarat |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name of the Exploration Agency Critical Mineral Trackers, Hyderabad |  |  |  |  |  |  |  |
| Total Area -6.203sq.km; No of Boreholes- 4, 120m; Completion Time -10 months, Review 4 months |  |  |  |  |  |  |  |
| S. No | Item of Work | Unit | Rates as per NMET SoC |  | Estimated Cost of the Proposal |  | Remarks |
|  |  |  | SoC- Item-SI No. | Rates as per SOC | Qtm | Total Amount (Rs) |  |
| A | Geological Work |  |  |  |  |  |  |
| 1 | Geological Mapping (1:12500) & sampling Geologist field-days | 6.12 | 1.2 | 11000 | 160 | 17,60,000 | man days |
| 2 | Geologists (HQ)days, pre & post field interpretation 15 +20 days | One Geologist Per Day | 1.2 | 9000 | 35 | 3,15,000 | man days (including Remote sensing studiesman days (including Remote sensing studies |
| 3 | Pitting-20nos each one size 1*1*1m (1 Cu.m each) | Per Cu.m | 2.1.2 | 3800 | 20 | 76,000 | 20 cu.m |
| 4 | Trenching-5 nos, each one size 10*1*1(10 cu.m each) | Per Cu.m | 2.1.1 | 3300 | 50 | 1,65,000 | 50 cu.m |
| 5 | sampler | 45 days | 1.5.2 | 5100 | 23 | 1,17,300 | man days |
| 6 | Labour (2 labour) attached to sampler | 90 labour days | 5.7 | 526 | 92 | 48,392 | labour days |
| 7 | Labour | Per Team of 2 Geologists | 5.7 | 526 | 320 | 1,68,320 | labour days |
|  |  | (2*2=4) |  |  |  |  |  |
|  |  | Labour/Field workers |  |  |  |  |  |
|  | Sub-Total -A |  |  |  |  | 26,50,012 |  |
| B | Survey Work: |  |  |  |  |  |  |
| 1 | Surveyor: Fixation & connection of boundary points (4 nos),4 Bh by Total station/DGPS | One surveyor | 1.6.2 | 19,200 | 8 | 1,53,600 |  |
|  | Sub-total-B |  |  |  |  | 1,53,600 |  |
| C | Core Drilling - Outsourced |  |  |  |  |  |  |

| 1 | Scout drilling(coring) :4 points (each 30m deep) 4*30 | Per meter | 2.2.1.1b | 7,168 | 120 | 8,60,160 | 120m, soft rock- MoC rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | Per pillar | 2.2.7a | 2000 | 0 | 0 | 4 pillars |
| 3 | **Mob & demob drilling machine & iner BH shifting | Per shifting | lumpsum |  |  | 0 | lumpsum |
| 4 | Compensation for 4 Bhs |  | 5.6 | 20,000 | 0 | 0 | 4 BHs |
| 5 | Drill core preservation in GI boxes | Per meter | 5.3 | 1590 | 120 | 1,90,800 | 120m core |
|  | Subtotal-C |  |  |  |  | 10,50,960 |  |
| D | Laboratory Studies |  |  |  |  |  |  |
| 1 | Trench and channel Samples (5*5=25 nos):by AAS method | First five radicals+5 | 4.1.7a &7b | 4181 | 0 | 0 | 90 samples |
| 2 | Pitting Sample: (20*1=20 nos)-AAS method | First five radicals+5 | 4.1.7a &7b | 4181 | 0 | 0 | 60 samples |
| 3 | Core drilling Samples-4*60=240 | First five radicals+5 | 4.1.7a &7b | 4181 | 0 | 0 | 24 samples |
|  | Total depth 30m each, samples will be collected at every 5m interval. |  |  |  |  |  |  |
|  | AAS method |  |  |  |  |  |  |
|  | XRF - Major Oxide |  | 4.1.15a | 4200 | 143 | 6,00,600 | Trench-70, Pit-20, BH-40 (130+13 Check) |
| 4 | Analysis for REE (14 elements/radicals) by ICP- MS+Check Samples | 14 elements/radicals | 4.1.13 | 5380 | 36 | 1,93,680 | 33+3 check |
| 5 | Combined determination of THA, MHA and Reactive silica |  | 4.1.17a | 6700 | 4 | 26,800 | 4 samples |

| 6 | Preparation of Polished thin section | Per sample | 4.3.2 | 1549 | 10 | 15,490 | 4 sections |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | Complete petrographic/ore microscopic/mineral graphic studies | Per sample | 4.3.4 | 4232 | 10 | 42,320 | 4 sections |
| 8 | XRD analysis for identification of minerals(random) | Per sample | 4.5.1 | 4000 | 4 | 16,000 | 4 samples |
|  | Subtotal-D |  |  |  |  | 2,51,970 |  |
| E | Surface Geophysical Survey |  | Not recommended |  |  |  |  |
| 1 | Electrical resistivity | Per Station |  |  |  |  |  |
| 2 | gravity surveys | Per station |  |  |  |  |  |
| 3 | Geophysicist Man days |  |  |  |  |  |  |
|  | (Field Man-days) |  |  |  |  |  |  |
| 4 | Geophysicist Man days (HQ) |  |  |  |  |  |  |
|  | Subtotal-E |  |  |  |  |  |  |
|  | TOTAL (A+B+C+D) |  |  |  |  | 41,06,542 |  |

|  | (5 Hard copies with a soft copy) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G | Geological Report |  | 5.2 | 5% of the Project cost |  | 2,05,327 | 5% of the Project Cost. |
|  | (5 Hard copies with a soft copy) |  |  |  |  |  |  |
|  | Operational Charges |  | 6.0 |  |  | 86,016 |  |
|  | Tender Process Cost |  | 2.3 |  |  | 43,008 |  |
|  | Peer review |  |  |  |  | 30,000 |  |
|  | Additional Copy |  |  | 1000 | 0 | 0 |  |
|  | Project Cost without GST |  |  |  |  | 45,53,024 |  |
|  | 18% GST |  |  |  |  | 8,19,544 |  |
|  | Total Project Cost |  |  |  |  | 53,72,568 |  |
| Note: |  |  |  |  |  |  |  |
| 1 | Strict adherence to the Ministry of Finance's and GFR guidelines is mandatory. Every transaction must adhere to GFR rule 21. |  |  |  |  |  |  |
| 2 | In case of delay/non- performance, the appropriate action will be taken by competent authority against delinquent agency as per prevailing govt. of India rules/guidelines on procurement. |  |  |  |  |  |  |
| 3 | If any part of the project is outsourced, the amount will be reimbursed as per the Paragraph 3 of NMET SoC and Item no. 6 of NMET SoC. In case of execution of the project by NEA on its own, a Certificate regarding non outsourcing of any component/project is required. |  |  |  |  |  |  |
| 4 | Necessary efforts should be made to minimize any adverse impact on the environment during exploration activities. |  |  |  |  |  |  |
| 5 | Any item of work not mentioned above shall be added as per SoC. |  |  |  |  |  |  |
| 6 | All the Geological Reports and data are to be uploaded on NGDR as per MERT template by the agency. |  |  |  |  |  |  |

| Time Schedule/ Action plan for Reconnaissance Survey (G4) for Bauxite, Ga, V, Ti & REE in Nadapa area Kachchh District, Gujarat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S. no | Activity | Unit | Months |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1 | 2 | 3 | 4 |  | 5 | 6 | 7 | 8 | 9 | 10 |
| 1 | Geologist Party Days HQ | Days |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Geologist Party Days Field | Days |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Sampling (Pitting & Trenching) | Days |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Laboratory Studies | Days |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Core Drilling | Days |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Survey Party Days | Days |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Post Field Interpretation | Days |  |  |  |  |  |  |  |  |  |  |  |
| 9 | Report Compilation & Submission | Days |  |  |  |  |  |  |  |  |  |  |  |

Note: CMT has completed field operations in Nadapa area from 14th Feb – 2025 to 13th Apr – 2026 but the report was submitted in March – 2026 due to delay in receiving petrographic studies and Analytical results of check samples.

LIST OF PLATES (ENCLOSED SEPARATLY)

| Plate No | Title | Scale |
| --- | --- | --- |
| I | Location Map of Reconnaissance Survey G4 for Bauxite, Ga, V, Ti & REE in Nadapa Area, Dist : Kachchh, Gujarat | Not To Scale |
| II | Regional Geological Map of Kachchh basin (part) with location of Nadapa Area, Kachchh district, Gujarat | Not To Scale |
| III | Interpreted Large Scale Geological Map of Nadapa Area, Dist: Kachchh, Gujarat | 1:12500 |
| IV | Large Scale Geological Map of outcrops in Nadapa area, Dist: Kachchh, Gujarat | 1:12500 |
| V | Location of Pits & Trench on Large Scale Geological Map of Nadapa Area, Dist: Kachchh, Gujarat | 1:12500 |
| VI | Land use / Land cover map of Nadapa area, Kachchh district, Gujarat | 1:12500 |
| VII | Lithological map of pits in Nadapa area, Kachchh district, Gujarat | 1:100 |
| VIII | Lithological map of Trenches in Nadapa area, Kachchh district, Gujarat | 1:100 |
| IX | Assay Map of pits Nadapa area, Kachchh district, Gujarat | 1:100 |
| X | Assay Map of Trenches Nadapa area, Kachchh district, Gujarat | 1:100 |
| XI | Topographical map of Nadapa area, Kachchh district, Gujarat | 1:12500 |

CHAPTER – XV

PEER RIVIEWER COMMENTS