ANNEXURE-II

| SL NO. | NAME | Mn (%) |  | SAMPLE NO. | CO-ORDINATES | PIT WALL DESCRIPTION IN m |  |  |  |  |  |  |  | LITHOLOGY |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | NORTH |  | EAST | SOUTH |  | WEST |  |  |  |  |  |
| 1 | P-1 |  |  |  | 21° 2' 36.680"N & 84° 9' 38.710"E | 0-1.2 |  | 0-1.2 | 0-1.2 |  | 0-1.2 |  |  | Soil |  |  |
| 2 | P-2 | 29.78 |  | KMPP-08 | 21° 2' 35.810"N & 84° 9' 37.700"E | 0-1.0 |  | 0-0.9 | 0-0.9 |  | 0 |  |  | Soil |  |  |
|  |  |  |  |  |  | 0 |  | 0.9-1.0 | 0.9-1.0 |  | 1.0 |  |  | Manganese |  |  |
| 3 | P-3 | 13.95 |  | KMPP-01 | 21° 2' 35.690"N & 84° 9' 37.070"E | 0-1.5 |  | 0-1.0 | 0-1.0 |  | 0-1.0 |  |  | Soil |  |  |
|  |  |  |  |  |  | 0 |  | 1.0-2.0 | 1.0-2.0 |  | 1.0-2.0 |  |  | Manganese |  |  |
| 4 | P-4 | 27.69 |  | KMPP-07 | 21° 2' 36.230"N & 84° 9' 39.980"E | 0-0.9 |  | 0-0.9 |  |  | 0-0.9 |  |  | Soil |  |  |
|  |  |  |  |  |  | 0.9-0.1 |  | 0.9-0.1 | 0-1.0 |  | 0.9-0.1 |  |  | Manganese |  |  |
| 5 | P-5 |  |  |  | 21° 2' 36.350"N & 84° 9' 37.650"E | 0-1.0 |  | 0-1.0 | 0-1.0 |  | 0-1.0 |  |  | Soil |  | Manganese Boulder was noticed in NW corner |
| 6 | P-6 |  |  |  | 21° 2' 35.203"N & 84° 9' 35.971"E | 0-1.0 |  | 0-1.0 | 0-1.0 |  | 0-1.0 |  |  | Soil |  |  |
| 7 | P-7 | 33.68 |  | KMPP-03 | 21° 2' 30.470"N & 84° 9' 26.510"E | 0-1.5 |  | 0-1.5 | 0-1.5 |  | 0-1.5 |  |  | Soil |  |  |
|  |  |  |  |  |  | 1.5-2.0 |  | 1.5-2.0 | 1.5-2.0 |  | 1.5-2.0 |  |  | Manganese |  |  |
| 8 | P-8 |  |  |  | 21° 2' 31.040"N &84° 9' 24.366"E | 0-1.3 |  | 0-1.3 | 0-1.3 |  | 0-1.3 |  |  | Soil |  |  |
| 9 | P-9 | 24.55 |  | KMPP-04 | 21° 2' 32.310"N & 84° 9' 22.470"E | 0-0.3 |  | 0-0.7 | 0-0.7 |  | 0-0.7 |  |  | Soil |  |  |
|  |  |  |  |  |  | 0.3-1.5 |  | 0.7-1.5 | 0.7-1.5 |  | 0.7-1.5 |  |  | Manganese Boulder |  |  |
| 10 | P-10 |  |  |  | 21° 2' 31.826"N & 84° 9' 18.095"E | 0-0.35 |  | 0-0.35 | 0-0.6 |  | 0-0.4 |  |  | Soil |  |  |
|  |  |  |  |  |  | 0.35-0.70 |  | 0.35-0.65 | 0.6-1.1 |  | 0.4-0.9 |  |  | Manganese Boulder |  |  |
|  |  |  |  |  |  | 0.70-1.2 |  | 0.65-1.6 | 1.1-1.6 |  | 0.9-1.6 |  |  | Khondalites floats with soil |  |  |
| 11 | P-11 |  |  |  | 21° 2' 31.350"N & 84° 9' 21.020"E | 0-1.0 |  | 0-0.5 | 0-0.7 |  | 0-1.0 |  |  | Soil |  | Manganese Boulder was noticed in NE corner |
|  |  |  |  |  |  |  |  | 0.5-1.0 | 0.7-1.0 |  |  |  |  | Manganese Boulder |  |  |
| 12 | P-12 | 13.95 |  | KMPP-02 | 21° 2' 33.838"N & 84° 9' 6.932"E | 0-0.8 |  | 0-0.7 | 0-0.7 |  | 0-0.7 |  |  | Soil |  |  |
|  |  |  |  |  |  | 0.8-1.0 |  | 0.7-1.0 | 0.7-1.0 |  | 0.7-1.0 |  |  | Manganese & Khondalite Boulder |  |  |
| 13 | P-13 | 2.28 |  | KMPP-05 | 21° 2' 32.800"N & 84° 9' 19.600"E | 0-1.2 |  | 0-1.2 | 0-1.2 |  | 0-1.2 |  |  | Manganese Boulder with soil |  |  |
| 14 | P-14 |  |  |  | 21° 2' 29.790"N & 84° 9' 15.050"E | 0-0.3 |  | 0-0.3 | 0-0.3 |  | 0-0.3 |  |  | Soil |  |  |
|  |  |  |  |  |  | 0.3-1.0 |  | 0.3-1.3 | 0.3-1.3 |  | 0.3-1.3 |  |  | Khondalite Boulder with soil |  |  |
| 15 | P-15 |  |  |  | 21° 2' 29.290"N & 84° 9' 9.970"E | 0-0.7 |  | 0-0.3 | 0-0.3 |  | 0-0.3 |  |  | Soil |  |  |
|  |  |  |  |  |  | 0.7-1.1 |  | 0.3-1.3 | 0.3-1.3 |  | 0.3-1.3 |  |  | Khondalite Boulder with soil |  |  |
| 16 | P-16 |  |  |  | 21° 2' 32.560"N & 84° 9' 9.830"E | 0-1.2 |  | 0-1.4 | 0-1.4 |  | 0-1.4 |  |  | Khondalite Boulder with soil |  | Manganese Boulder was noticed in NE corner |
| 17 | P-17 | 34.94 |  | KMPP-06 | 21° 2' 33.550"N & 84° 9' 16.570"E | 0-1.7 |  | 0-1.7 | 0-1.7 |  | 0-1.7 |  |  | Manganese Boulder with soil |  |  |
| 18 | P-18 |  |  |  | 21° 2' 29.330"N & 84° 9' 2.289"E | 0-1.2 |  | 0-1.3 | 0-1.3 |  | 0-1.3 |  |  | Soil |  | A band of Khondalite was noticed in the eastern wall |
| SL NO. | NAME | Mn (%) | SAMPLE NO. |  | CO- ORDINATES |  | TRENCH WALL DESCRIPTION IN M |  |  |  |  |  | LITHOLOGY |  | REMARK |  |
|  |  |  |  |  |  |  | NORTH | EAST |  | SOUTH |  | WEST |  |  |  |  |
| 1 | T-1 |  |  |  | 21° 2' 33.730"N & 84° 9' 33.000"E |  | 0-0.7 | 0-1.1 |  | 0-0.75 |  | 0-1.2 | Soil |  | Granite gneiss boulder was noticed in the base of the western wall |  |
|  |  |  |  |  |  |  |  |  |  | 0.75-1.2 |  |  | Manganese boulder with soil |  |  |  |
|  |  |  |  |  |  |  |  | 1.1-1.6 |  | 1.2-1.9 |  | 1.2-1.9 | Kaolinised Soil |  |  |  |
| 2 | T-2 | 16.97 | KMPT-04 |  | 21° 2' 29.281"N & 84° 9' 24.341"E |  | 0-0.9 | 0-0.4 |  | 0-0.4 |  | 0-0.4 | Soil |  |  |  |
|  |  |  |  |  |  |  |  | 0.4-1.2 |  | 0.4-1.2 |  | 0.4-1.2 | Manganese boulder with soil |  |  |  |
| 3 | T-3 |  |  |  | 21° 2' 31.990"N & 84° 9' 19.760"E |  | 0-1 | 0-1.3 |  | 0-1.3 |  | 0-1.3 | Khondalite boulder with soil |  |  |  |
| 4 | T-4 | 26 | KMPT-01 |  | 21° 2' 34.000"N & 84° 9' 13.160"E |  | 0-0.5 | 0-0.9 |  | 0-0.9 |  | 0-0.9 | Soil |  |  |  |
|  |  |  |  |  |  |  |  | 0.9-1.4 |  | 0.9-1.4 |  | 0.9-1.4 | Manganese |  |  |  |
| 5 | T-5 |  |  |  | 21° 2' 32.930"N & 84° 9' 14.440"E |  | 0-0.6 | 0-1.2 |  | 0-1.2 |  | 0-1.2 | Soil |  | Manganese Boulder was noticed in SE corner |  |
| 6 | T-6 |  |  |  | 21° 2' 30.270"N & 84° 9' 0.500"E |  | 0-0.1 |  |  | 0-0.1 |  | 0-0.1 | Soil |  |  |  |
|  |  |  |  |  |  |  | 0.1-0.4 |  |  | 0.1-0.6 |  | 0.1-0.6 | Manganese boulder |  |  |  |
|  |  |  |  |  |  |  | 0.4-1 | 0-1.1 |  | 0.6-1 |  | 0.6-1 | Khondalites |  |  |  |
| 7 | T-7 |  |  |  | 21° 2' 31.090"N & 84° 9' 1.780"E |  | 0-0.1 | 0-0.1 |  | 0-0.1 |  | 0-0.1 | Soil |  |  |  |
|  |  |  |  |  |  |  | 0.1-0.4 | 0.1-0.4 |  |  |  |  | Manganese boulder with soil |  |  |  |
|  |  |  |  |  |  |  | 0.4-1.0 | 0.4-1.0 |  | 0.1-0.9 |  | 0.1-1 | Khondalites |  |  |  |
| 8 | T-8 | 29.04 | KMPT-02 |  | 21° 2' 27.850"N & 84° 9' 0.770"E |  | 0-0.2 | 0-0.2 |  | 0-0.7 |  | 0-0.2 | Black Soil |  |  |  |
|  |  |  |  |  |  |  | 0.2-1.2 | 0.2-1.2 |  |  |  | 0.2-1.2 | Manganese boulder with soil |  |  |  |
| 9 | T-9 |  |  |  | 21° 2' 27.695"N & 84° 8' 59.545"E |  | 0-1.1 | 0-1.1 |  | 0-1.0 |  | 0-1.1 | Manganese boulder with soil |  |  |  |
| 10 | T-10 | 28.65 | KMPT-03 |  | 21° 2' 28.984"N & 84° 9' 0.283"E |  | 0-0.3 | 0-0.3 |  | 0-0.3 |  | 0-0.3 | Manganese |  |  |  |
|  |  |  |  |  |  |  | 0.3-0.7 | 0.3-0.7 |  | 0.3-0.5 |  | 0.3-0.7 | Manganese boulder with soil |  |  |  |

ANNEXURE –III

Quartz  Samples

ANNEXURE -IV

ANNEXURE –IV b

(Check Sample Analysis)

ANNEXURE –V

ANNEXURE –VI

ANNEXURE –VII

GOVERNMENT OF INDIA GEOLOGICAL SURVEY OF INDIA

Central Petrological Laboratories 15 A & B Kyd Street

Kolkata – 700 016

No.	Pet-Sec-12/PP/CPL/CHQ/2025	12.09.2025

Name: Abhisek Hatui, Geologist Address: DMG, Odisha

Ref: Letter dated 30/05/2025

Petrography

Sample: Five samples, namely, KMP/01, KMP/02, KMP/03, KMP/04 and KMP/05 are manganese- bearing rock. Detail petrographic studies suggest the samples can be classified into two dominant textural domains: (1) an irregular, massive to granular Mn-oxide aggregate and (2) finely rhythmic, parallel laminae up to a few hundred micrometres wide.

# Microscopic description

Massive domain: Sample KMP/01, KMP/02, KMP/04 and KMP/05 are of massive domain. In transmitted light condition of microscope KMP/01 is complete occupied by the opaque manganese ore (Fig.1). In sample no. KMP/02 it is observed that the Mn ores are irregular in shape ranging from rounded to elongated and angular with subhedral to anhedral grain boundaries (Fig. 2). The Mn ores are floating in a mass comprised of heamatite and quartz along with fragments of cryptocrystalline silica (Fig. 2). The cryptocrystalline silica and the Mn ore share corrugated grain boundaries that represent replacement texture (Fig. 3). Sample no. KMP/04 again represents massive Mn ore along with quartz and very small amount of heamatite (Fig. 4) whereas Sample no. KMP/05 contains irregular Mn ore with angular elongated grains having anhedral grain boundaries (Fig. 5). The quartz grains are highly fractured in Sample no. KMP/05 (Fig. 6). In reflected light condition of microscope, it has been observed that the opaque parts of the samples are occupied by a dense, subhomogeneous, silvery-grey to slightly metallic-appearing aggregate (Fig. 7). Grain boundaries are irregular and anastomosing, with Mn-oxide material replacing or filling interstices of a lighter-coloured host (Figs 8 & 9). Individual oxide grains are generally fine to very fine grained, producing a nearly continuous, massive appearance at the scale of the photomicrograph (Fig. 8). Disseminated orangey-brown patches of host/gangue occur locally along the margins and in pockets (Fig. 10).

Banded domain: Sample no. KMP/03 falls under banded domain. In reflected light condition of microscope, a clearly laminated portion displays tightly spaced, laterally continuous light and dark laminae (Fig. 11). Laminae are thin (sub-mm, consistent with the 200 µm scale bar) and parallel to sub- parallel (Fig. 12). Dark laminae are more opaque and finer grained, forming regular rhythmic bands; lighter laminae are slightly coarser and less opaque (Fig. 11).

Contacts and textures: Contacts between Mn-rich bands and lighter bands are sharp and planar at the microscope scale, consistent with rhythmic chemical precipitation or depositional lamination. The massive oxide zones show replacement textures and breccia-like contacts with the host, suggesting post- depositional mobilization and infilling.

# Interpretation

Textural evidence indicates two genetic components in the samples:

- Sedimentary/chemical-precipitation origin for the banded domain. The regular micro-banding is characteristic of rhythmic precipitation of Mn oxides and interlayered silica/carbonate during chemical sedimentation or in a slowly changing depositional environment.

- Superimposed diagenetic or supergene alteration for the massive, replacement-bearing domain. The massive, subhomogeneous oxide aggregate and irregular replacement of host lithology suggest secondary mobilization and concentration of Mn oxides into cavities and along fractures.

Probable mineral phases (optical-based, non-definitive): common Mn-oxide candidates include pyrolusite, cryptomelane/psilomelane-group minerals and hausmannite/braunite in more recrystallized portions. The darker, highly opaque laminae contain greater proportion of silica, carbonate or clay matrix and the lighter laminae are interpreted as Mn-rich oxide/hydroxide phases.

# Confidence and limitations

The petrographic interpretation above is based on reflected/polarized light texture, colour/opacity contrasts and scale of features in the photomicrographs. Optical characters alone do not reliably distinguish many Mn oxide species because they are commonly fine grained, poorly crystalline, and form intergrowths. Also natural manganese oxides comprise many structurally distinct but compositionally similar phases (for example, pyrolusite, ramsdellite, nsutite, cryptomelane, hollandite, birnessite and others). These phases can be polymorphs or form tunnel and layered structures that are not reliably separable by optical properties alone. Final phase-level identification requires complementary mineralogical analyses like XRD, LRS etc.

# Reference

Post, J. E. (1999). The manganese oxide minerals: Crystal structures and economic significance. (Review).

| Fig.1: Photomicrograph of sample no. KMP/01 showing massive opaque Mn ore in transmitted light condition of microscope. | Fig.2: Photomicrograph of sample no. KMP/02 irregular Mn-ore floating in a mass of quartz – heamatite. |
| --- | --- |

| Fig.3: Photomicrograph of Sample no. KMP/02 showing cryptocrystalline quartz replacing Mn ore. | Fig.4: Photomicrograph of sample no. KMP/04 showing massive Mn ore along with quartz and small portion of heamatite. |
| --- | --- |

| Fig.5: Photomicrograph of sample no. KMP/05 showing irregular elongated Mn-ore floating within heamatite. | Fig.6: Photomicrograph of sample no. KMP/05 showing highly fractured quartz grains present with Mn-ore. |
| --- | --- |

| Fig.7: Photomicrograph of sample no. KMP/04 showing a dense, subhomogeneous, silvery-grey to slightly metallic-appearing aggregate of Mn- ore in reflected light condition of microscope. | Fig.8: Photomicrograph of sample no. KMP/01 showing irregular and anastomosing grain boundaries of Mn-ore in reflected light condition of microscope. |
| --- | --- |

| Fig.9: Photomicrograph of sample no. KMP/02 showing irregular and anastomosing grain boundaries of Mn-ore in reflected light condition of microscope with Mn-oxide material replacing or filling interstices of the host. | Fig.10: Photomicrograph of sample no. KMP/05 showing Disseminated orangey-brown patches of host/gangue minerals. |
| --- | --- |

| Fig.11: Photomicrograph of sample no. KMP/03 showing tightly spaced, laterally continuous light and dark laminae. | Fig.12: Photomicrograph of sample no. KMP/03 showing light and dark laminae are thin (sub-mm, consistent with the 200 µm scale bar). |
| --- | --- |

ANNEXURE –VIII

ANNEXURE -IX

ANNEXURE X

| Annexure 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Revised Cost for Reconnaissance Exploration for Manganese around the Kandhal area of Rairakhol Tehsil, Sambalpur District, Odisha |  |  |  |  |  |  |  |  |  |  |  |  |
| Sl. | Item of Work | Unit | Cost Approved by EC |  |  |  | Cost Approved by 81st TCC |  | Cost Revised |  |  | Remark |
| No. |  |  | SoC item Sl. No. | Rate as per SoC (in Rs) | Quantum of Work | Total Amount (Rs.) |  |  | Rate as per SoC (in Rs) | Quantum of Work | Total Amount (Rs.) |  |
| 1 | GEOLOGICAL WORK |  |  |  |  |  |  |  |  |  |  |  |
| i) | Charges for One Geologist - Field | per day | 1.3 | 11,000 | 140 | 1540000.00 | 1540000.00 |  | 11,000 | 100 | 1100000.00 |  |
| ii) | Charges for One Geologist - HQ | per day | 1.3 | 9,000 | 30 | 270000.00 | 270000.00 |  | 9,000 | 30 | 270000.00 |  |
| iii) | Labours (2 Nos.) Base rate- As per rates of state labour commission (Amount will be reimbursed as per notified rates by central labour Commission rates or respective State Govt. Whichever is higher) | per day | 5.7 | 522 | 280 | 146160.00 | 146160.00 |  | 522 | 200 | 104400.00 |  |

| iv) | Channel sample 52 m (7 channels) | per day | 5.7 | 522 |  |  | 0.00* |  | 522 | 26 | 13572.00 | *A difference of ₹13,572/- has been noted in the approved cost of the project submitted by agency during 81st TCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| v) | Charges for one Sampler | per day | 1.5.2 | 5,100 | 17 | 86700.00 | 86700.00 |  | 5,100 | 12 | 61200.00 |  |
| vi) | Labours (4 Nos.) Base rate- As per rates of state labour commission. (Amount will be reimbursed as per notified rates by central labour Commission rates or respective State Govt. Whichever is higher) | per day | 5.7 | 522 | 68 | 35496.00 | 35496.00 |  | 522 | 48 | 25056.00 |  |
| Sub Total-A |  |  |  |  |  | 2078356.00 | 2078356.00 |  |  |  | 1574228.0 0 |  |
| 2 | Survey (Outsourcing) |  |  |  |  |  |  |  |  |  |  |  |

| i) | Survey Party days (One Surveyor Charges) | day | 1.6.1a | 8,300 | 0 | 0.00 | 0.00 |  | 8,300 | 0 | 0.00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ii) | Bore Hole Fixation and determination of co-ordinates & Reduced Level of the boreholes by DGPS | Per Point of observation | 1.6.2 | 19,200 | 4 | 76800.00 | 76800.00 |  | 19,200 | 0 | 0.00 |  |
| iii) | Labours (4 Nos) Base Rate- As per rates of state labour commission Rs. 450*4=1800/- (Amount will be reimbursed as per notified rates by central labour Commission rates or respective State Govt. Whichever is higher) | day | 5.7 | 1,800 | 0 | 0.00 | 0.00 |  | 1,800 | 0 | 0.00 |  |
|  | Sub Total- B |  |  |  |  | 76800.00 | 76800.00 |  |  |  | 0.00 |  |
| 3 | DRILLING (Outsourcing) |  |  |  |  |  |  |  |  |  |  |  |
| i) | Core Drilling | per m | 2.2.1.4 | 11,500 | 200 | 2300000.00 | 2300000.00 | (4 BH:- 40- 50m depth) | 11,500 | 0 | 0.00 |  |
| ii | Construction of concrete pillar(12"x12"x30") | Per Bore hole | 2.2.7a | 2,000 | 0 | 0.00 | 0.00 |  | 2,000 | 0 | 0.00 |  |

| iii) | Monthly Accomodation charges for drilling camp (up to 2 rigs) | Per Months | 2.2.9 | 50,000 | 0 | 0.00 | 0.00 |  | 50,000 | 0 | 0.00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| iv) | Camp setting | Per drill | 2.2.9a | 250000 | 0 | 0.00 | 0.00 |  | 250000 | 0 | 0.00 |  |
| v) | Camp winding | Per drill | 2.2.9b | 250000 | 0 | 0.00 | 0.00 |  | 250000 | 0 | 0.00 |  |
| vi) | Approach road making | Per km | 2.2.10b | 32,200 | 0 | 0.00 | 0.00 |  | 32,200 | 0 | 0.00 |  |
| v) | Drill core preservation | Per m | 5.3 | 1,590 | 80 | 127200.00 | 127200.00 |  | 1,590 | 0 | 0.00 |  |
|  | Sub Total- C |  |  |  |  | 2427200.00 | 2427200.00 |  |  |  | 0.00 |  |
| 4 | PITTING |  |  |  |  |  |  |  |  |  |  |  |
| i) | Pitting | Cu.m | 2.1.2 | 3,800 | 40 | 152000.00 | 152000.00 |  | 3,800 | 40 | 152000.00 |  |
| ii) | Trenching | Cu.m | 2.1.1 | 3,330 | 40 | 133200.00 | 133200.00 |  | 3,330 | 40 | 133200.00 |  |
|  | Sub Total-D |  |  |  |  | 285200.00 | 285200.00 |  |  |  | 285200.00 |  |
| 5 | LABORATORY STUDIES |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Chemical Analysis |  |  |  |  |  |  |  |  |  |  |  |
| i) | XRF analysis of major oxide for 5 radicals (MnO2,Al2O3, SiO2, Fe2O3, and LOI) | Per Sample | 4.1.15a | 4,200 | 125 | 525000.00 | 525000.00 |  | 4,200 | 86 | 361200.00 |  |
| ii) | Check samples Internal for 5 radicals (MnO2,Al2O3, SiO2, Fe2O3, and | Per Sample | 4.1.15a | 4,200 | 13 | 54600.00 | 54600.00 |  | 4,200 | 8 | 33600.00 |  |

|  | LOI) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Physical Analysis |  |  |  |  |  |  |  |  |  |  |  |
| i) | X-RD Studies | Per Sample | 4.5.1 | 4,000 | 5 | 20000.00 | 20000.00 |  | 4,000 | 5 | 20000.00 |  |
| ii) | Preparation of thin section | Per Sample | 4.3.1 | 2,353 | 10 | 23530.00 | 23530.00 |  | 2,353 | 10 | 23530.00 |  |
| iii) | Petrographic studies | Per Sample | 4.3.4 | 4,232 | 10 | 42320.00 | 42320.00 |  | 4,232 | 10 | 42320.00 |  |
| iv) | Preparation of polished section | Per Sample | 4.3.2 | 1,549 | 5 | 7745.00 | 7745.00 |  | 1,549 | 5 | 7745.00 |  |
| v) | Bulk density | Per Sample | 4.1 | 3,540 | 2 | 7080.00 | 7080.00 |  | 3,540 | 2 | 7080.00 |  |
|  | Sub Total-E |  |  |  |  | 680275.00 | 680275.00 |  |  |  | 495475.00 |  |
| Total A+B+C+D+E |  |  |  |  |  | 5547831.00 | 5547831.00 |  |  |  | 2354903.00 |  |
|  | Total Outsourced cost |  |  |  |  | 2504000.00 | 2504000.00 |  |  |  | 0.00 |  |
| F | Tender Preparation, publishing & finalisation | One time | 2.3 |  |  | 50080.00 | 50080.00 |  |  |  | 0.00 |  |
| G | Operational Charge |  | 6.0 | i |  | 250400.00 | 250400.00 |  |  |  | 0.00 |  |
| H | Geological Report Preparation | One no.(5 hard copies,) along with soft copy | 5.2 |  |  | 277391.55 | 278070.15 |  |  |  | 117745.15 |  |
| I | Peer Review |  | As per EC decision |  |  | 30000.00 | 30000.00 |  |  |  | 30000.00 |  |

| J | Proposal preparation |  | 5.1 |  |  | 110956.62 | 111228.06 |  |  |  | 47098.06 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sub Total (F + G + H +I) |  |  |  |  | 718828.17 | 719778.21 |  |  |  | 194843.21 |  |
|  | TOTAL (A+B+C+D+E+ F + G + H +I+J) |  |  |  |  | 6266659.17 | 6281181.21 |  |  |  | 2549746.21 |  |
|  | GST (18%) |  |  |  |  | 1127998.65 | 1130612.62 |  |  |  | 458954.32 |  |
|  | Grand Total |  |  |  |  | 7394657.82 | 7411793.83 |  |  |  | 3008700.53 |  |
|  | Say in lakh |  |  |  |  | 73.94 | 74.11 |  |  |  | 30.08 |  |
|  | Change in amount |  |  |  |  |  | (-) Rs. 43.86 lakhs |  | De-escalation of ~59.32% |  |  |  |
| Note: |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Strict adherence to the Ministry of Finance's and GFR guidelines is mandatory. Every transaction must adhere to GFR rule 21. |  |  |  |  |  |  |  |  |  |  |  |
| 2 | In case of delay/non- performance, the appropriate action will be taken by competent authority against delinquent agency as per prevailing govt. of India rules/guidelines on procurement. |  |  |  |  |  |  |  |  |  |  |  |
| 3 | If any part of the project is outsourced, the amount will be reimbursed as per the Paragraph 3 of NMET SoC and Item no. 6 of NMET SoC. In case of execution of the project by NEA on its own, a Certificate regarding non outsourcing of any component/project is required. |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Necessary efforts should be made to minimize any adverse impact on the environment during exploration activities. |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Any item of work not mentioned above shall be added as per SoC. |  |  |  |  |  |  |  |  |  |  |  |
| 6 | All the Geological Reports and data are to be uploaded on NGDR portal as per Mineral Exploration Reporting Template (MERT) by the agency. |  |  |  |  |  |  |  |  |  |  |  |