STATEMENT SHOWING DESCRIPTIVE LITHOLOGY OF BOREHOLES DRILLED IN WEST OF BORDA & GHONSA -PARSODABLOCK, WARDHA VALLEY COALFIELD, DISTRICT- YAVATMAL, STATE: MAHAARSHTRA

BOREHOLE NO    :CMWWB01              LATITUDE    : 2209645.917M                        DATE OF COMMENCEMENT:19.10.2023

REDUCED LEVEL  : 282.27M             DEPARTURE   : 264858.8494M                        DATE OF COMPLETION: 27.11.2024

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

.00    .20    .10      .00    .20    .10   SOIL,BROWN

SOIL

*************************************************************************************************************************

WEATHERED MANTLE

.20    .60    .10      .20    .60    .10   BASALT,GREY

.80   2.20    .80      .80   2.20    .80   BASALT,GREY,FRACTURE

3.00   1.00    .98     3.00   1.00    .98   BASALT,GREY,FRACTURE

4.00   1.00   1.00     4.00   1.00   1.00   BASALT,GREY                                                              71

5.00   1.50   1.50     5.00   1.50   1.50   BASALT,GREY,FRACTURE                                                     42

****************************************************************************************************************************

TRAP

6.50   3.00   3.00     6.50   3.00   3.00   BASALT,GREY,FRACTURE                                                     30

9.50   3.00   3.00     9.50   3.00   3.00   BASALT,GREY,FRACTURE                                                     66

12.50   3.00   3.00    12.50   3.00   3.00   BASALT,GREY                                                              69

15.50   3.00   3.00    15.50   3.00   3.00   BASALT,GREY                                                              69

18.50   3.00   3.00    18.50   3.00   3.00   BASALT,GREY,FRACTURE                                                     81

21.50   3.00   3.00    21.50   3.00   3.00   BASALT,GREY                                                              97

24.50   3.00   3.00    24.50   3.00   3.00   BASALT,GREY                                                              81

27.50   3.00   3.00    27.50   3.00   3.00   BASALT,GREY                                                              93

30.50   3.00   2.92    30.50   3.00   2.92   BASALT,GREY                                                              69

33.50   3.00   3.00    33.50   3.00   3.00   BASALT,GREY,FRACTURE                                                     50

36.50   3.00   2.99    36.50   3.00   2.99   BASALT,GREY                                                              40

39.50   3.00   3.00    39.50   3.00   3.00   BASALT,GREY                                                              68

42.50   3.00   3.00    42.50   3.00   3.00   BASALT,GREY,FRACTURE                                                     46

45.50   3.00   3.00    45.50   3.00   3.00   BASALT,GREY                                                              79

48.50   3.00   3.00    48.50   3.00   3.00   BASALT,GREY                                                              82

51.50   1.12   1.10    51.50   1.12   1.10   BASALT,GREY,FRACTURE

MOTUR

52.62   1.88   1.83    52.62   1.88   1.83   SILTSTONE,GREY                                                           37

54.50   3.00   2.90    54.50   3.00   2.90   SANDSTONE(FINE GRAINED),BROWN

57.50   1.00    .44    57.50   1.00    .44   CLAYSTONE,BROWN

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

58.50   2.00    .55    58.50   2.00    .55   CLAYSTONE,GREY

60.50   1.50    .85    60.50   1.50    .85   CLAYSTONE,GREY

62.00    .40    .31    62.00    .40    .31   SILTSTONE,GREY

62.40   1.10   1.00    62.40   1.10   1.00   MUDSTONE,REDDISH BROWN

63.50   2.50   2.50    63.50   2.50   2.50   MUDSTONE,REDDISH BROWN

66.00   1.50   1.20    66.00   1.50   1.20   MUDSTONE,REDDISH BROWN

67.50    .50    .17    67.50    .50    .17   BOULDER (UNCONSOLIDATED),GREENISH GREY

68.00   1.50    .47    68.00   1.50    .47   MUDSTONE,YELLOWISHBROWN

69.50   1.10   1.06    69.50   1.10   1.06   MUDSTONE,YELLOWISHBROWN

70.60   1.90   1.84    70.60   1.90   1.84   SANDSTONE(FINE GRAINED),BROWNISH BROWN

72.50    .23    .08    72.50    .23    .08   SILTSTONE,BROWNISH BROWN

72.73   1.27   1.17    72.73   1.27   1.17   SANDSTONE(COARSE GRAINED),YELLOWISHBROWN

74.00   1.50    .53    74.00   1.50    .53   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN

75.50   2.50   1.18    75.50   2.50   1.18   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN

78.00    .50    .38    78.00    .50    .38   SILTSTONE,BROWNISH BROWN

78.50   3.00   2.85    78.50   3.00   2.85   MUDSTONE,BROWNISH BROWN

81.50   3.00   2.98    81.50   3.00   2.98   MUDSTONE,BROWNISH BROWN

84.50   3.00   2.99    84.50   3.00   2.99   MUDSTONE,BROWNISH BROWN,SLICKENSIDED

87.50   3.00   2.99    87.50   3.00   2.99   CLAYSTONE,BROWNISH BROWN

90.50   3.00   3.00    90.50   3.00   3.00   CLAYSTONE,BROWNISH BROWN

93.50   3.00   3.00    93.50   3.00   3.00   SILTSTONE,BROWN

96.50   3.00   3.00    96.50   3.00   3.00   CLAYSTONE,GREY

99.50   3.00   3.00    99.50   3.00   3.00   SILTSTONE,GREY

102.50   3.00   2.50   102.50   3.00   2.50   SILTSTONE,GREY

105.50   3.00    .60   105.50   3.00    .60   CLAYSTONE,BROWNISH BROWN

108.50   3.00   1.70   108.50   3.00   1.70   SANDSTONE(FINE GRAINED),BROWN

111.50   3.00   1.30   111.50   3.00   1.30   SANDSTONE(MEDIUM GRAINED),BROWN

114.50   1.50   1.40   114.50   1.50   1.40   SANDSTONE(MEDIUM GRAINED),WHITE

116.00   1.50   1.35   116.00   1.50   1.35   SANDSTONE(COARSE GRAINED),WHITE

117.50   1.50    .40   117.50   1.50    .40   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

119.00    .97    .82   119.00    .97    .82   SANDSTONE(COARSE GRAINED),BROWN

119.97    .53    .38   119.97    .53    .38   SILTSTONE,BROWN

120.50   3.00   2.20   120.50   3.00   2.20   CLAYSTONE,BROWN

123.50   1.70   1.60   123.50   1.70   1.60   CLAYSTONE,BROWN

125.20   1.30   1.30   125.20   1.30   1.30   SILTSTONE,BROWN

126.50   2.00   1.90   126.50   2.00   1.90   SILTSTONE,GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

128.50   1.00    .87   128.50   1.00    .87   SANDSTONE(MEDIUM GRAINED),WHITE

129.50   3.00   2.40   129.50   3.00   2.40   SANDSTONE(COARSE GRAINED),BROWN

132.50   3.00   1.70   132.50   3.00   1.70   SANDSTONE(COARSE GRAINED),BROWN

135.50   3.00   2.80   135.50   3.00   2.80   CLAYSTONE,REDDISH BROWN

138.50   3.00   2.99   138.50   3.00   2.99   CLAYSTONE,REDDISH BROWN

141.50   1.10   1.10   141.50   1.10   1.10   MUDSTONE,WHITE

142.60   1.90   1.90   142.60   1.90   1.90   SANDSTONE(FINE GRAINED),WHITE

144.50    .87    .87   144.50    .87    .87   SANDSTONE(MEDIUM GRAINED),WHITE                                         100

145.37   2.13   2.13   145.37   2.13   2.13   MUDSTONE,CHOCOLATEBROWN                                                  22

147.50   1.50   1.50   147.50   1.50   1.50   MUDSTONE,CHOCOLATEBROWN

149.00   1.50   1.50   149.00   1.50   1.50   MUDSTONE,CHOCOLATEBROWN

150.50   3.00   3.00   150.50   3.00   3.00   MUDSTONE,BROWN

153.50   3.00   3.00   153.50   3.00   3.00   CLAYSTONE,GREY

156.50   3.00   3.00   156.50   3.00   3.00   SILTSTONE,REDDISH BROWN

159.50   3.00   3.00   159.50   3.00   3.00   SILTSTONE,REDDISH BROWN

162.50   1.38   1.28   162.50   1.38   1.28   SILTSTONE,REDDISH BROWN

163.88    .90    .80   163.88    .90    .80   SANDSTONE(FINE GRAINED),BROWN

164.78    .72    .62   164.78    .72    .62   SILTSTONE,REDDISH BROWN

165.50   1.80   1.80   165.50   1.80   1.80   SILTSTONE,BROWN

167.30   1.20   1.20   167.30   1.20   1.20   SANDSTONE(COARSE GRAINED),BROWN

168.50    .55    .35   168.50    .55    .35   SANDSTONE(COARSE GRAINED),BROWN

169.05   1.25   1.05   169.05   1.25   1.05   SILTSTONE,BROWN

170.30   1.20   1.00   170.30   1.20   1.00   SANDSTONE(FINE GRAINED),BROWN

171.50   2.00   1.70   171.50   2.00   1.70   SANDSTONE(FINE GRAINED),BROWN

173.50   3.00   2.80   173.50   3.00   2.80   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN

176.50   3.00   2.00   176.50   3.00   2.00   SANDSTONE(MEDIUM GRAINED),BROWN                                          28

179.50    .18    .14   179.50    .18    .14   CLAYSTONE,REDDISH BROWN

179.68   2.15   1.93   179.68   2.15   1.93   SANDSTONE(FINE GRAINED),BROWN                                            48

181.83    .67    .53   181.83    .67    .53   CLAYSTONE,REDDISH BROWN

182.50   1.00    .37   182.50   1.00    .37   CLAYSTONE,REDDISH BROWN

183.50   3.00   1.88   183.50   3.00   1.88   CLAYSTONE,REDDISH BROWN

186.50   1.20    .84   186.50   1.20    .84   CLAYSTONE,REDDISH BROWN

187.70    .80    .65   187.70    .80    .65   SANDSTONE(FINE GRAINED),BROWN

188.50   1.00    .15   188.50   1.00    .15   SANDSTONE(COARSE GRAINED),REDDISH BROWN

189.50   2.00    .23   189.50   2.00    .23   CLAYSTONE,BROWN

191.50   1.50    .23   191.50   1.50    .23   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

193.00   1.50    .19   193.00   1.50    .19   CLAYSTONE,BROWNISH RED

194.50   3.00    .55   194.50   3.00    .55   CLAYSTONE,BROWNISH RED

197.50   3.00    .28   197.50   3.00    .28   SANDSTONE(COARSE GRAINED),BROWNISH RED

200.50   3.00    .29   200.50   3.00    .29   SANDSTONE(FINE GRAINED),REDDISH BROWN

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

203.50   2.00    .28   203.50   2.00    .28   SANDSTONE(COARSE GRAINED),BROWNISH RED

205.50   1.00    .17   205.50   1.00    .17   CLAYSTONE,BROWNISH RED

206.50   3.00    .50   206.50   3.00    .50   SANDSTONE(FINE GRAINED),BROWNISH RED

209.50   3.00    .72   209.50   3.00    .72   SANDSTONE(FINE GRAINED),BROWNISH RED

212.50   3.00    .15   212.50   3.00    .15   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

215.50   3.00    .37   215.50   3.00    .37   SANDSTONE(FINE GRAINED),BROWNISH RED

218.50   3.00    .18   218.50   3.00    .18   SANDSTONE(COARSE GRAINED),BROWNISH RED

221.50   3.00    .30   221.50   3.00    .30   SANDSTONE(COARSE GRAINED),BROWNISH RED

224.50   3.00    .23   224.50   3.00    .23   SANDSTONE(COARSE GRAINED),BROWNISH RED

227.50    .52    .30   227.50    .52    .30   SANDSTONE(FINE GRAINED),BROWNISH RED

228.02    .48    .32   228.02    .48    .32   CLAYSTONE,REDDISH BROWN

228.50   1.00    .73   228.50   1.00    .73   CLAYSTONE,REDDISH BROWN                                                  55

229.50   1.00    .83   229.50   1.00    .83   CLAYSTONE,REDDISH BROWN

230.50   3.00    .69   230.50   3.00    .69   CLAYSTONE,REDDISH BROWN

233.50   1.10    .26   233.50   1.10    .26   SANDSTONE(COARSE GRAINED),REDDISH YELLOW

234.60    .90    .24   234.60    .90    .24   CLAYSTONE,REDDISH BROWN

235.50    .20    .13   235.50    .20    .13   SANDSTONE(FINE GRAINED),BROWN

235.70    .30    .28   235.70    .30    .28   SANDSTONE(FINE GRAINED),BROWN

236.00   1.00    .94   236.00   1.00    .94   SANDSTONE(FINE GRAINED),BROWN

237.00   1.50   1.19   237.00   1.50   1.19   SANDSTONE(FINE GRAINED),BROWN

238.50   1.50   1.18   238.50   1.50   1.18   SANDSTONE(FINE GRAINED),REDDISH BROWN

240.00    .30    .21   240.00    .30    .21   CLAYSTONE,REDDISH BROWN

240.30    .45    .30   240.30    .45    .30   SANDSTONE(FINE GRAINED),BROWN

240.75    .75    .65   240.75    .75    .65   CLAYSTONE,BROWNISH RED

241.50   1.50   1.25   241.50   1.50   1.25   SANDSTONE(FINE GRAINED),BROWN

243.00    .15    .13   243.00    .15    .13   SANDSTONE(COARSE GRAINED),BROWNISH RED

243.15    .47    .43   243.15    .47    .43   CLAYSTONE,BROWNISH RED

243.62   1.27   1.18   243.62   1.27   1.18   SANDSTONE(FINE GRAINED),BROWN

244.89   1.11   1.03   244.89   1.11   1.03   CLAYSTONE,BROWNISH RED

246.00   3.00   2.90   246.00   3.00   2.90   CLAYSTONE,BROWNISH RED

249.00    .10    .10   249.00    .10    .10   CLAYSTONE,BROWNISH RED

249.10    .24    .21   249.10    .24    .21   SANDSTONE(FINE GRAINED),BROWNISH RED

249.34    .36    .34   249.34    .36    .34   CLAYSTONE,BROWNISH RED

249.70   2.30   2.13   249.70   2.30   2.13   SANDSTONE(FINE GRAINED),LIGHT BROWN

252.00   3.00   2.90   252.00   3.00   2.90   SANDSTONE(FINE GRAINED),LIGHT BROWN

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

255.00   3.00   2.80   255.00   3.00   2.80   SANDSTONE(FINE GRAINED),LIGHT BROWN

258.00   3.00   2.70   258.00   3.00   2.70   SANDSTONE(FINE GRAINED),LIGHT BROWN

261.00   3.00   2.70   261.00   3.00   2.70   SANDSTONE(FINE GRAINED),LIGHT BROWN

264.00   3.00   2.90   264.00   3.00   2.90   SANDSTONE(FINE GRAINED),LIGHT BROWN

267.00   1.56   1.50   267.00   1.56   1.50   SANDSTONE(FINE GRAINED),LIGHT BROWN

268.56    .74    .71   268.56    .74    .71   CLAYSTONE,BROWNISH RED                                                   38

269.30    .70    .63   269.30    .70    .63   SANDSTONE(COARSE GRAINED),LIGHT BROWN

270.00   3.00   2.90   270.00   3.00   2.90   SANDSTONE(FINE GRAINED),LIGHT BROWN                                      50

273.00   3.00   2.91   273.00   3.00   2.91   SANDSTONE(FINE GRAINED),LIGHT BROWN

276.00   3.00   2.90   276.00   3.00   2.90   SANDSTONE(FINE GRAINED),LIGHT BROWN                                      39

279.00   3.00   2.87   279.00   3.00   2.87   SANDSTONE(FINE GRAINED),BROWNISH RED                                     42

282.00   3.00   2.92   282.00   3.00   2.92   SANDSTONE(MEDIUM GRAINED),BROWNISH RED,FRACTURE                          45

285.00   1.42   1.40   285.00   1.42   1.40   SANDSTONE(COARSE GRAINED),BROWNISH RED                                   26

286.42    .08    .08   286.42    .08    .08   CLAYSTONE,BROWNISH RED

286.50   1.50   1.50   286.50   1.50   1.50   SANDSTONE(FINE GRAINED),BROWNISH RED                                     73

288.00   1.00    .90   288.00   1.00    .90   SANDSTONE(FINE GRAINED),BROWNISH RED

289.00    .20    .18   289.00    .20    .18   SANDSTONE(FINE GRAINED),LIGHT BROWN

289.20    .22    .19   289.20    .22    .19   CLAYSTONE,BROWNISH RED

289.42    .20    .15   289.42    .20    .15   SANDSTONE(FINE GRAINED),BROWNISH RED

289.62    .27    .22   289.62    .27    .22   CLAYSTONE,BROWNISH RED

289.89    .61    .53   289.89    .61    .53   SANDSTONE(FINE GRAINED),LIGHT BROWN                                      48

290.50   1.46   1.23   290.50   1.46   1.23   SANDSTONE(FINE GRAINED),LIGHT BROWN                                      74

291.96   1.54   1.35   291.96   1.54   1.35   SANDSTONE(FINE GRAINED),BROWNISH RED                                     62

293.50   3.00   2.82   293.50   3.00   2.82   SANDSTONE(FINE GRAINED),BROWNISH RED,FRACTURE                            64

296.50   3.00   2.78   296.50   3.00   2.78   SANDSTONE(FINE GRAINED),LIGHT BROWN                                      50

299.50   3.00   2.94   299.50   3.00   2.94   SANDSTONE(FINE GRAINED),REDDISH BROWN,FRACTURE                           47

302.50   1.27   1.23   302.50   1.27   1.23   SANDSTONE(FINE GRAINED),REDDISH BROWN,FRACTURE                           59

303.77    .35    .35   303.77    .35    .35   CLAYSTONE,REDDISH BROWN                                                  77

304.12   1.38   1.36   304.12   1.38   1.36   SANDSTONE(FINE GRAINED),LIGHT BROWN                                      88

305.50   3.00   2.95   305.50   3.00   2.95   SANDSTONE(FINE GRAINED),BROWNISH RED                                     46

308.50   3.00   2.85   308.50   3.00   2.85   SANDSTONE(FINE GRAINED),BROWNISH RED                                     30

311.50   3.00   2.86   311.50   3.00   2.86   SANDSTONE(FINE GRAINED),BROWNISH RED                                     64

314.50   3.00   2.78   314.50   3.00   2.78   SANDSTONE(FINE GRAINED),BROWNISH RED

317.50   2.00   1.78   317.50   2.00   1.78   SANDSTONE(FINE GRAINED),REDDISH GREY

319.50   1.50   1.38   319.50   1.50   1.38   SANDSTONE(FINE GRAINED),REDDISH GREY

321.00   2.00   1.80   321.00   2.00   1.80   SANDSTONE(FINE GRAINED),REDDISH GREY                                    6.5

323.00   2.00   1.80   323.00   2.00   1.80   SANDSTONE(FINE GRAINED),REDDISH GREY

325.00   2.00   1.78   325.00   2.00   1.78   SANDSTONE(FINE GRAINED),REDDISH GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

327.00   2.00   1.77   327.00   2.00   1.77   SANDSTONE(FINE GRAINED),REDDISH GREY

329.00   1.00    .85   329.00   1.00    .85   SANDSTONE(FINE GRAINED),REDDISH GREY

330.00   3.00   2.87   330.00   3.00   2.87   SANDSTONE(FINE GRAINED),REDDISH GREY                                     21

333.00   3.00   2.85   333.00   3.00   2.85   SANDSTONE(FINE GRAINED),REDDISH GREY                                     35

336.00   3.00   2.88   336.00   3.00   2.88   SANDSTONE(FINE GRAINED),BROWNISH RED,FRACTURE                            29

339.00   1.00    .85   339.00   1.00    .85   SANDSTONE(FINE GRAINED),LIGHT GREY                                       12

340.00   1.50   1.37   340.00   1.50   1.37   SANDSTONE(FINE GRAINED),LIGHT GREY                                       33

341.50    .50    .45   341.50    .50    .45   SANDSTONE(FINE GRAINED),LIGHT GREY                                       30

342.00   1.50   1.40   342.00   1.50   1.40   SANDSTONE(FINE GRAINED),LIGHT GREY                                       33

343.50   1.50   1.35   343.50   1.50   1.35   SANDSTONE(FINE GRAINED),LIGHT GREY

345.00   3.00   2.92   345.00   3.00   2.92   SANDSTONE(FINE GRAINED),LIGHT GREY                                       67

348.00   1.50   1.44   348.00   1.50   1.44   SANDSTONE(FINE GRAINED),LIGHT GREY                                       12

349.50   3.00   2.90   349.50   3.00   2.90   SANDSTONE(FINE GRAINED),LIGHT GREY                                       70

352.50   1.50   1.42   352.50   1.50   1.42   SANDSTONE(FINE GRAINED),LIGHT GREY                                       75

354.00   3.00   2.95   354.00   3.00   2.95   SANDSTONE(FINE GRAINED),LIGHT GREY                                       81

357.00   2.86   2.85   357.00   2.86   2.85   SANDSTONE(FINE GRAINED),LIGHT GREY                                       47

359.86    .14    .14   359.86    .14    .14   CLAYSTONE,REDDISH BROWN

360.00   2.27   2.27   360.00   2.27   2.27   CLAYSTONE,BROWNISH RED                                                   70

362.27    .73    .73   362.27    .73    .73   SANDSTONE(FINE GRAINED),BROWNISH RED                                     51

363.00   3.00   2.90   363.00   3.00   2.90   SANDSTONE(FINE GRAINED),BROWNISH RED                                     76

366.00   3.00   2.89   366.00   3.00   2.89   SANDSTONE(FINE GRAINED),BROWNISH RED                                     56

369.00   1.20   1.20   369.00   1.20   1.20   SANDSTONE(FINE GRAINED),BROWNISH RED                                     37

370.20   1.80   1.80   370.20   1.80   1.80   CLAYSTONE,BROWNISH RED                                                   68

372.00    .80    .80   372.00    .80    .80   CLAYSTONE,BROWNISH RED                                                   40

372.80    .98    .98   372.80    .98    .98   MUDSTONE,GREY                                                            51

373.78    .34    .34   373.78    .34    .34   CLAYSTONE,BROWN                                                          44

374.12    .88    .88   374.12    .88    .88   MUDSTONE,GREY

375.00   1.00    .98   375.00   1.00    .98   MUDSTONE,GREY,FRACTURE

376.00   2.00   1.80   376.00   2.00   1.80   MUDSTONE,GREY,FRACTURE                                                  7.5

378.00    .60    .56   378.00    .60    .56   MUDSTONE,GREY,FRACTURE                                                   77

378.60    .26    .24   378.60    .26    .24   CLAYSTONE,BROWN

378.86   1.60   1.49   378.86   1.60   1.49   MUDSTONE,GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

380.46    .54    .48   380.46    .54    .48   SANDSTONE(FINE GRAINED),YELLOWISHGREY                                    89

381.00   2.25   2.25   381.00   2.25   2.25   SANDSTONE(MEDIUM GRAINED),YELLOWISHWHITE                                 95

383.25    .75    .75   383.25    .75    .75   MUDSTONE,GREY,FRACTURE

384.00   3.00   1.36   384.00   3.00   1.36   MUDSTONE,GREY,FRACTURE

387.00   3.00   2.33   387.00   3.00   2.33   MUDSTONE,GREY,FRACTURE

390.00    .22    .22   390.00    .22    .22   MUDSTONE,GREY,FRACTURE

390.22    .28    .28   390.22    .28    .28   CLAYSTONE,REDDISH BROWN

390.50   1.07   1.03   390.50   1.07   1.03   MUDSTONE,GREY

391.57    .43    .42   391.57    .43    .42   CLAYSTONE,REDDISH BROWN

392.00   1.00   1.00   392.00   1.00   1.00   CLAYSTONE,REDDISH BROWN

393.00   1.04   1.04   393.00   1.04   1.04   CLAYSTONE,BROWNISH GREY,FRACTURE                                         20

394.04   1.96   1.96   394.04   1.96   1.96   MUDSTONE,GREY                                                            30

396.00   3.00   2.96   396.00   3.00   2.96   MUDSTONE,GREY

399.00   3.00    .62   399.00   3.00    .62   MUDSTONE,GREY

402.00   3.00   2.98   402.00   3.00   2.98   MUDSTONE,GREY

405.00   3.00   2.75   405.00   3.00   2.75   CLAYSTONE,VARIEGATE

408.00   1.00    .98   408.00   1.00    .98   MUDSTONE,GREY

409.00   3.00   1.30   409.00   3.00   1.30   SANDSTONE(MEDIUM GRAINED),GREY

412.00   3.00   1.30   412.00   3.00   1.30   SANDSTONE(COARSE GRAINED),GREY

415.00   3.00   1.00   415.00   3.00   1.00   SANDSTONE(COARSE GRAINED),GREY

418.00   1.50    .39   418.00   1.50    .39   SANDSTONE(MEDIUM GRAINED),GREY

419.50   1.50    .70   419.50   1.50    .70   CLAYSTONE,GREY                                                           13

421.00   3.00   1.06   421.00   3.00   1.06   CLAYSTONE,BROWN,SLICKENSIDED

424.00   3.00   1.05   424.00   3.00   1.05   CLAYSTONE,GREYISH BROWN                                                  11

427.00   1.50    .20   427.00   1.50    .20   CLAYSTONE,BROWN

428.50   1.50    .80   428.50   1.50    .80   MUDSTONE,GREY

430.00   3.00   1.05   430.00   3.00   1.05   SANDSTONE(FINE GRAINED),GREY,FRACTURE                                    15

433.00   3.00   1.30   433.00   3.00   1.30   CLAYSTONE,GREY,SANDY,FRACTURE                                            13

436.00   3.00   2.10   436.00   3.00   2.10   MUDSTONE,GREY,FRACTURE

439.00    .58    .17   439.00    .58    .17   MUDSTONE,GREY

439.58    .63    .24   439.58    .63    .24   SANDSTONE(FINE GRAINED),GREY

440.21   1.79   1.40   440.21   1.79   1.40   CLAYSTONE,GREY,SLICKENSIDED

442.00   3.00   2.99   442.00   3.00   2.99   CLAYSTONE,VARIEGATE

445.00   3.00   1.50   445.00   3.00   1.50   MUDSTONE,GREY

448.00   3.00   1.50   448.00   3.00   1.50   CLAYSTONE,GREY

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

451.00   3.00   1.80   451.00   3.00   1.80   CLAYSTONE,BROWN,SLICKENSIDED

454.00   3.00   1.50   454.00   3.00   1.50   CLAYSTONE,GREYISH BROWN

457.00   3.00   1.95   457.00   3.00   1.95   CLAYSTONE,GREYISH BROWN

460.00   3.00   2.63   460.00   3.00   2.63   CLAYSTONE,GREYISH BROWN

463.00   3.00   2.98   463.00   3.00   2.98   CLAYSTONE,GREYISH BROWN                                                  45

466.00   1.25   1.09   466.00   1.25   1.09   CLAYSTONE,GREYISH BROWN                                                  78

467.25    .75    .60   467.25    .75    .60   SANDSTONE(FINE GRAINED),GREYISH BROWN                                    81

468.00   3.00   2.98   468.00   3.00   2.98   SANDSTONE(MEDIUM GRAINED),GREYISH BROWN                                  51

471.00   1.97   1.94   471.00   1.97   1.94   SANDSTONE(MEDIUM GRAINED),GREYISH BROWN                                  46

472.97   1.03   1.00   472.97   1.03   1.00   CLAYSTONE,GREYISH BROWN                                                  55

474.00   3.00   2.98   474.00   3.00   2.98   CLAYSTONE,GREY

477.00    .50    .50   477.00    .50    .50   CLAYSTONE,GREYISH BROWN

477.50   1.00   1.00   477.50   1.00   1.00   CLAYSTONE,VARIEGATE

478.50   2.50   2.48   478.50   2.50   2.48   ALTERNATING SHALE AND SST,VARIEGATE                                      46

481.00   3.00   2.99   481.00   3.00   2.99   ALTERNATING SHALE AND SST,VARIEGATE                                      62

484.00   2.00   2.00   484.00   2.00   2.00   CLAYSTONE,VARIEGATE                                                      20

486.00   1.50   1.50   486.00   1.50   1.50   CLAYSTONE,GREYISH BROWN

487.50   3.00   2.50   487.50   3.00   2.50   CLAYSTONE,VARIEGATE

490.50   2.00   1.80   490.50   2.00   1.80   CLAYSTONE,VARIEGATE

492.50   1.00    .98   492.50   1.00    .98   CLAYSTONE,VARIEGATE

493.50    .60    .60   493.50    .60    .60   CLAYSTONE,VARIEGATE

494.10   1.40   1.40   494.10   1.40   1.40   SANDY SHALE,GREY

495.50   1.00    .98   495.50   1.00    .98   ALTERNATING SHALE AND SST,GREY

496.50   1.73   1.70   496.50   1.73   1.70   ALTERNATING SHALE AND SST,GREY

498.23    .27    .23   498.23    .27    .23   SANDSTONE(MEDIUM GRAINED),GREY                                           85

498.50   1.00    .70   498.50   1.00    .70   SANDSTONE(MEDIUM GRAINED),GREY

499.50   1.50   1.20   499.50   1.50   1.20   MUDSTONE,GREY

501.00   1.00    .58   501.00   1.00    .58   CLAYSTONE,GREYISH BROWN

502.00   2.50   2.50   502.00   2.50   2.50   CLAYSTONE,VARIEGATE                                                      61

504.50   2.50   2.50   504.50   2.50   2.50   CLAYSTONE,VARIEGAT,SLICKENSIDED                                          13

507.00   1.16   1.09   507.00   1.16   1.09   CLAYSTONE,VARIEGATE

508.16   1.27   1.20   508.16   1.27   1.20   SANDSTONE(MEDIUM GRAINED),GREY                                           40

509.43    .57    .50   509.43    .57    .50   MUDSTONE,GREY,SLICKENSIDED

510.00    .60    .60   510.00    .60    .60   SANDSTONE(FINE GRAINED),GREY

510.60    .90    .90   510.60    .90    .90   SHALE,GREY,FRACTURE HIGH ANGLE                                           58

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

511.50   1.00    .98   511.50   1.00    .98   CLAYSTONE,GREY,FRACTURE                                                  43

512.50   2.00   1.80   512.50   2.00   1.80   CLAYSTONE,GREY

514.50   1.50   1.50   514.50   1.50   1.50   CLAYSTONE,GREY                                                           29

516.00   3.00   2.98   516.00   3.00   2.98   CLAYSTONE,GREY

519.00   3.00   2.99   519.00   3.00   2.99   CLAYSTONE,GREY

522.00   2.00   2.00   522.00   2.00   2.00   CLAYSTONE,GREY

524.00   3.00   2.98   524.00   3.00   2.98   CLAYSTONE,VARIEGATE

527.00   3.00   2.97   527.00   3.00   2.97   CLAYSTONE,VARIEGATE

530.00   2.00   1.80   530.00   2.00   1.80   CLAYSTONE,BROWN

532.00   1.00   1.00   532.00   1.00   1.00   CLAYSTONE,BROWN

533.00   1.00    .98   533.00   1.00    .98   CLAYSTONE,BROWN                                                          12

534.00   2.00   1.99   534.00   2.00   1.99   CLAYSTONE,BROWN

536.00   2.41   2.40   536.00   2.41   2.40   CLAYSTONE,BROWN

538.41    .34    .33   538.41    .34    .33   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

538.75    .25    .24   538.75    .25    .24   CLAYSTONE,BROWN

539.00   3.00   2.98   539.00   3.00   2.98   CLAYSTONE,BROWNISH GREY

542.00   3.00   2.98   542.00   3.00   2.98   CLAYSTONE,BROWNISH GREY

545.00   1.00    .92   545.00   1.00    .92   CLAYSTONE,BROWN

546.00    .88    .85   546.00    .88    .85   CLAYSTONE,BROWN

546.88    .29    .26   546.88    .29    .26   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY

547.17    .83    .80   547.17    .83    .80   CLAYSTONE,BROWN                                                          36

548.00   1.50   1.50   548.00   1.50   1.50   CLAYSTONE,BROWN                                                          27

549.50   1.50   1.45   549.50   1.50   1.45   CLAYSTONE,BROWN                                                          47

551.00   2.50   2.25   551.00   2.50   2.25   CLAYSTONE,BROWN

553.50   2.00   1.98   553.50   2.00   1.98   CLAYSTONE,BROWN                                                          28

555.50   2.00   1.90   555.50   2.00   1.90   CLAYSTONE,VARIEGATE                                                      72

557.50   3.00   2.75   557.50   3.00   2.75   CLAYSTONE,VARIEGATE                                                       8

560.50   3.00   2.00   560.50   3.00   2.00   CLAYSTONE,GREYISH BROWN

563.50   3.00   2.50   563.50   3.00   2.50   CLAYSTONE,GREYISH BROWN

566.50   2.50   1.90   566.50   2.50   1.90   CLAYSTONE,BROWN

569.00   3.00   2.20   569.00   3.00   2.20   CLAYSTONE,BROWN                                                         3.3

572.00   1.00    .90   572.00   1.00    .90   CLAYSTONE,BROWN                                                          11

573.00   4.50   4.50   573.00   4.50   4.50   CLAYSTONE,BROWN                                                          16

577.50   4.00   3.99   577.50   4.00   3.99   CLAYSTONE,GREYISH BROWN                                                  31

581.50   4.00   4.00   581.50   4.00   4.00   CLAYSTONE,GREYISH BROWN                                                  44

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

585.50   4.00   4.00   585.50   4.00   4.00   CLAYSTONE,GREYISH BROWN

589.50   4.00   4.00   589.50   4.00   4.00   CLAYSTONE,GREYISH BROWN

593.50   2.00   2.00   593.50   2.00   2.00   CLAYSTONE,BROWN                                                          20

595.50   2.00   2.00   595.50   2.00   2.00   CLAYSTONE,BROWN                                                          19

597.50    .53    .53   597.50    .53    .53   SANDSTONE(FINE GRAINED),GREY                                             53

598.03    .47    .47   598.03    .47    .47   CLAYSTONE,GREYISH BROWN

598.50   3.23   3.23   598.50   3.23   3.23   CLAYSTONE,BROWNISH GREY                                                  43

601.73    .57    .57   601.73    .57    .57   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   61

602.30    .70    .70   602.30    .70    .70   CLAYSTONE,BROWN                                                          64

603.00   3.00   2.99   603.00   3.00   2.99   CLAYSTONE,BROWN,FRACTURE                                                 31

606.00   1.52   1.52   606.00   1.52   1.52   CLAYSTONE,BROWN                                                          23

607.52    .46    .45   607.52    .46    .45   CLAYSTONE,GREYISH BROWN                                                  83

607.98   1.11   1.10   607.98   1.11   1.10   SANDSTONE(FINE GRAINED),GREYISH BROWN                                    97

609.09    .91    .90   609.09    .91    .90   SANDSTONE(FINE GRAINED),GREYISH BROWN                                    98

610.00    .91    .80   610.00    .91    .80   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   32

610.91   2.09   1.98   610.91   2.09   1.98   CLAYSTONE,GREYISH BROWN,FRACTURE HIGH ANGLE                              48

613.00   2.00   1.99   613.00   2.00   1.99   CLAYSTONE,BROWN,FRACTURE HIGH ANGLE                                      22

615.00   1.50   1.49   615.00   1.50   1.49   CLAYSTONE,BROWN,FRACTURE HIGH ANGLE                                      16

616.50   2.50   2.40   616.50   2.50   2.40   CLAYSTONE,GREYISH BROWN                                                  68

619.00    .72    .72   619.00    .72    .72   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  96

619.72   1.25   1.25   619.72   1.25   1.25   CLAYSTONE,BROWN,FRACTURE                                                 54

620.97    .30    .30   620.97    .30    .30   SANDSTONE(MEDIUM GRAINED),GREYISH BROWN                                  90

621.27   1.23   1.23   621.27   1.23   1.23   CLAYSTONE,GREYISH BROWN

622.50   3.00   2.99   622.50   3.00   2.99   CLAYSTONE,GREYISH BROWN,SLICKENSIDED

625.50    .34    .33   625.50    .34    .33   CLAYSTONE,BROWN,SLICKENSIDED

625.84   1.36   1.36   625.84   1.36   1.36   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN,                         51

627.20    .30    .30   627.20    .30    .30   CLAYSTONE,GREYISH BROWN,SLICKENSIDED

627.50   2.50   2.50   627.50   2.50   2.50   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                              60

630.00    .56    .52   630.00    .56    .52   SANDSTONE(MED. TO C. GRAINED),GREY,FRACTURE                              25

630.56   2.44   2.40   630.56   2.44   2.40   CLAYSTONE,GREYISH BROWN

633.00   1.03   1.00   633.00   1.03   1.00   CLAYSTONE,BROWN

634.03    .90    .87   634.03    .90    .87   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  69

634.93   1.07   1.05   634.93   1.07   1.05   CLAYSTONE,GREYISH BROWN,SLICKENSIDED

636.00   2.20   2.20   636.00   2.20   2.20   SANDSTONE(MED. TO C. GRAINED),GREY,FRACTURE                              24

638.20    .27    .27   638.20    .27    .27   MUDSTONE,GREY,FRACTURE

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

638.47    .25    .25   638.47    .25    .25   SANDSTONE(MEDIUM GRAINED),GREY                                          100

638.72    .28    .28   638.72    .28    .28   MUDSTONE,GREY,FRACTURE

639.00   2.50   2.40   639.00   2.50   2.40   CLAYSTONE,BROWNISH GREY,FRACTURE HIGH ANGLE                             4.8

641.50   2.00   1.00   641.50   2.00   1.00   CLAYSTONE,GREYISH BROWN

643.50   3.00   2.98   643.50   3.00   2.98   CLAYSTONE,GREYISH BROWN                                                  20

646.50   2.12   2.12   646.50   2.12   2.12   CLAYSTONE,GREYISH BROWN                                                  83

648.62    .26    .26   648.62    .26    .26   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                             100

648.88    .42    .42   648.88    .42    .42   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                             100

649.30    .20    .20   649.30    .20    .20   CLAYSTONE,CHOCOLATEBROWN                                                100

649.50   2.50   2.40   649.50   2.50   2.40   CLAYSTONE,CHOCOLATEBROWN                                                 10

652.00   3.00   2.98   652.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 12

655.00   3.00   3.00   655.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 45

658.00   3.00   2.50   658.00   3.00   2.50   CLAYSTONE,CHOCOLATEBROWN                                                 24

661.00   2.10   2.10   661.00   2.10   2.10   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    34

663.10    .35    .29   663.10    .35    .29   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,

663.45    .55    .55   663.45    .55    .55   CLAYSTONE,CHOCOLATEBROWN

664.00   3.00   2.96   664.00   3.00   2.96   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                       5.7

667.00   3.00   2.94   667.00   3.00   2.94   CLAYSTONE,CHOCOLATEBROWN                                                 18

670.00   3.00   2.97   670.00   3.00   2.97   CLAYSTONE,BROWNISH GREY,FRACTURE                                         24

673.00   2.30   2.30   673.00   2.30   2.30   CLAYSTONE,BROWNISH GREY,FRACTURE                                        7.4

675.30    .23    .23   675.30    .23    .23   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              57

675.53    .34    .34   675.53    .34    .34   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

675.87    .13    .13   675.87    .13    .13   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                             100

676.00    .15    .15   676.00    .15    .15   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                             100

676.15   2.70   2.70   676.15   2.70   2.70   CLAYSTONE,BROWNISH GREY,FRACTURE                                         58

678.85    .15    .15   678.85    .15    .15   CLAYSTONE,GREYISH WHITE                                                  73

679.00    .50    .50   679.00    .50    .50   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              70

679.50   2.50   2.50   679.50   2.50   2.50   CLAYSTONE,BROWNISH GREY                                                  53

682.00   1.66   1.66   682.00   1.66   1.66   CLAYSTONE,BROWNISH GREY                                                  40

683.66    .38    .37   683.66    .38    .37   MUDSTONE,GREY                                                            32

684.04    .96    .96   684.04    .96    .96   CLAYSTONE,CHOCOLATEBROWN                                                 56

685.00   3.00   2.99   685.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        27

688.00   3.00   3.00   688.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    43

691.00   3.00   3.00   691.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        43

694.00   3.00   3.00   694.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        57

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

697.00   3.00   2.99   697.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        28

700.00   3.00   3.00   700.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        46

703.00   3.00   3.00   703.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                         9

706.00   3.00   3.00   706.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    34

709.00   2.50   2.50   709.00   2.50   2.50   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    65

711.50   3.00   2.99   711.50   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    60

714.50   3.00   2.99   714.50   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                     5

717.50   3.00   3.00   717.50   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    30

720.50   3.00   2.98   720.50   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

723.50   3.00   2.98   723.50   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

726.50   3.00   3.00   726.50   3.00   3.00   SANDSTONE(COARSE TO V.C.GRAINED),GREENISH GREY,                          67

729.50   3.00   2.99   729.50   3.00   2.99   SANDSTONE(COARSE TO V.C.GRAINED),GREENISH GREY                           76

732.50   3.00   3.00   732.50   3.00   3.00   SANDSTONE(COARSE TO V.C.GRAINED),GREENISH GREY                           87

735.50   2.45   2.43   735.50   2.45   2.43   SANDSTONE(COARSE TO V.C.GRAINED),GREY                                    74

737.95    .55    .55   737.95    .55    .55   MUDSTONE,GREY,FRACTURE

738.50   1.69   1.65   738.50   1.69   1.65   MUDSTONE,GREY,FRACTURE

740.19    .65    .63   740.19    .65    .63   CLAYSTONE,GREY,SLICKENSIDED

BARAKAR

740.84    .66    .66   740.84    .66    .66   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                         100

741.50   3.00   3.00   741.50   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              88

744.50   1.57   1.57   744.50   1.57   1.57   SANDSTONE(MED. TO C. GRAINED),GREENISH GREY                              90

746.07    .27    .20   746.07    .27    .20   SANDY SHALE,GREY

746.34   1.16   1.16   746.34   1.16   1.16   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          76

747.50   3.00   3.00   747.50   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREENISH GREY                              86

750.50    .08    .08   750.50    .08    .08   SANDSTONE(MED. TO C. GRAINED),GREENISH GREY

750.58    .42    .42   750.58    .42    .42   SHALE,GREY,FRACTURE

751.00   1.38   1.38   751.00   1.38   1.38   SANDY SHALE,GREY,SLICKENSIDED                                            51

752.38   1.12   1.12   752.38   1.12   1.12   SANDSTONE(MEDIUM GRAINED),WHITE                                          76

753.50   3.00   3.00   753.50   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),WHITE,SLICKENSIDED                         93

756.50   2.50   2.50   756.50   2.50   2.50   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          85

759.00   3.00   3.00   759.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              96

762.00   3.00   3.00   762.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         93

765.00   2.65   2.65   765.00   2.65   2.65   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  95

767.65    .35    .30   767.65    .35    .30   SHALE,DARK GREY,SLICKENSIDED

768.00    .36    .35   768.00    .36    .35   SHALE,GREY

768.36   1.38   1.38   768.36   1.38   1.38   SANDY SHALE,GREY,SLICKENSIDED                                            43

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

769.74   1.26   1.26   769.74   1.26   1.26   INTERCALATIONS,GREY                                                      70

771.00   3.00   3.00   771.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          89

774.00    .54    .54   774.00    .54    .54   SANDSTONE(FINE TO MEDIUM GRAINED),GREENISH GREY                          87

774.54   1.46   1.46   774.54   1.46   1.46   SHALE,GREY,SLICKENSIDED

776.00   1.22   1.20   776.00   1.22   1.20   SHALE,DARK GREY,SLICKENSIDED

777.22    .88    .88   777.22    .88    .88   SHALE,LIGHT GREY,SLICKENSIDED                                            43

778.10    .90    .90   778.10    .90    .90   SHALE,GREY,SLICKENSIDED

779.00    .56    .55   779.00    .56    .55   SHALE,GREY,SLICKENSIDED

779.56    .78    .78   779.56    .78    .78   SANDY SHALE,GREY,SLICKENSIDED

780.34   1.66   1.66   780.34   1.66   1.66   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          91

782.00   3.00   3.00   782.00   3.00   3.00   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE,FRACTURE                         80

785.00   3.00   2.98   785.00   3.00   2.98   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  81

788.00   3.00   2.99   788.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              68

791.00   3.00   3.00   791.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              66

794.00   3.00   2.65   794.00   3.00   2.65   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         32

797.00   3.00   2.60   797.00   3.00   2.60   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          44

800.00   1.00    .95   800.00   1.00    .95   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  65

801.00   2.00   1.68   801.00   2.00   1.68   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  75

803.00   3.00   2.85   803.00   3.00   2.85   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  92

806.00   3.00   2.90   806.00   3.00   2.90   SANDSTONE(MED. TO C. GRAINED),WHITE                                      82

809.00   3.00   2.75   809.00   3.00   2.75   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              83

812.00   3.00   2.98   812.00   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              86

815.00    .60    .60   815.00    .60    .60   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                             100

815.60    .37    .37   815.60    .37    .37   COAL                                                            IIA

815.97    .65    .62   815.97    .65    .62   SHALY COAL,FRACTURE                                             IIA

816.62    .60    .59   816.62    .60    .59   COAL                                                            IIA

817.22    .19    .19   817.22    .19    .19   COAL                                                            IIA

817.41    .79    .77   817.41    .79    .77   COAL                                                            IIA

818.20    .25    .24   818.20    .25    .24   COAL                                                            IIA

818.45   2.55   2.52   818.45   2.55   2.52   SANDSTONE(FINE GRAINED)                                                  68

821.00    .75    .70   821.00    .75    .70   INTERCALATIONS                                                           68

821.75   1.00    .99   821.75   1.00    .99   SANDY SHALE

822.75    .19    .16   822.75    .19    .16   SHALY COAL                                                      IIB

822.94    .46    .46   822.94    .46    .46   COAL                                                            IIB

823.40    .28    .27   823.40    .28    .27   COAL                                                            IIB

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

823.68    .32    .32   823.68    .32    .32   COAL                                                            IIB

824.00    .74    .74   824.00    .74    .74   COAL                                                            IIB

824.74    .35    .35   824.74    .35    .35   COAL                                                            IIB

825.09    .65    .65   825.09    .65    .65   COAL                                                            IIB

825.74    .16    .16   825.74    .16    .16   COAL                                                            IIB

825.90    .46    .46   825.90    .46    .46   COAL                                                            IIB

826.36    .24    .23   826.36    .24    .23   COAL,SLICKENSIDED                                               IIB

826.60    .40    .39   826.60    .40    .39   SANDY SHALE                                                              43

827.00   3.00   3.00   827.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED)                                        80

830.00   3.00   2.99   830.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED)                                        94

833.00   3.00   2.99   833.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED)                                            96

836.00    .57    .57   836.00    .57    .57   SANDSTONE(MED. TO C. GRAINED)                                           100

836.57    .36    .35   836.57    .36    .35   SHALY COAL,SLICKENSIDED                                         I

836.93    .39    .39   836.93    .39    .39   SANDY SHALE

837.32    .52    .52   837.32    .52    .52   INTERCALATIONS                                                           23

837.84   1.16   1.16   837.84   1.16   1.16   SANDSTONE(FINE TO MEDIUM GRAINED)                                        74

839.00   3.00   2.98   839.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED)                                        77

842.00   1.00   1.00   842.00   1.00   1.00   SANDSTONE(FINE TO MEDIUM GRAINED)                                        89

BOREHOLE CLOSED AT   843.00 M DEPTH

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

\

BOREHOLE NO    :CMWWB02              LATITUDE    : 2203150.488M                        DATE OF COMMENCEMENT:25.10.2023

REDUCED LEVEL : 250.8714M            DEPARTURE   :271361.8439M                         DATE OF COMPLETION: 14.03.2024

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

.00   3.00   2.98      .00   3.00   2.98   BASALT,GREYISH BROWN

3.00   3.00   2.96     3.00   3.00   2.96   BASALT,GREENISH BROWN

6.00   3.00   2.95     6.00   3.00   2.95   BASALT,GREENISH GREY,VESICULAR

9.00   1.50   1.50     9.00   1.50   1.50   BASALT,GREENISH GREY,VESICULAR

10.50   1.50   1.40    10.50   1.50   1.40   BASALT,GREENISH GREY

12.00   3.00   2.95    12.00   3.00   2.95   BASALT,DARK GREY,VESICULAR                                               73

15.00   3.00   2.99    15.00   3.00   2.99   BASALT,DARK GREY,HARD                                                    64

18.00   3.00   3.00    18.00   3.00   3.00   BASALT,GREY,HARD                                                         70

21.00   3.00   2.98    21.00   3.00   2.98   BASALT,GREY,HARD                                                         90

24.00   3.00   3.00    24.00   3.00   3.00   BASALT,GREY,HARD                                                         81

27.00   3.00   3.00    27.00   3.00   3.00   BASALT,GREY,HARD                                                         93

30.00   3.00   3.00    30.00   3.00   3.00   BASALT,GREY,HARD                                                         91

33.00   1.50   1.44    33.00   1.50   1.44   BASALT,GREY,HARD                                                         81

34.50   1.50   1.17    34.50   1.50   1.17   CLAY,DARK BROWN

36.00   1.00   1.00    36.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN

37.00   1.00    .88    37.00   1.00    .88   CLAYSTONE,CHOCOLATEBROWN

38.00   2.00   1.90    38.00   2.00   1.90   BASALT,DARK GREY                                                         30

40.00   2.00   2.00    40.00   2.00   2.00   BASALT,DARK GREY                                                         57

42.00   3.00   3.00    42.00   3.00   3.00   BASALT,GREY,VESICULAR                                                    81

45.00   3.00   2.95    45.00   3.00   2.95   BASALT,GREY,VESICULAR                                                    52

48.00   3.00   2.90    48.00   3.00   2.90   BASALT,DARK GREY,VESICULAR                                               46

51.00   3.00   2.26    51.00   3.00   2.26   BASALT,GREY,VESICULAR                                                    94

54.00    .40    .38    54.00    .40    .38   BASALT,DARK GREY,VESICULAR                                               85

54.40   1.60   1.58    54.40   1.60   1.58   CLAYSTONE,CHOCOLATEBROWN

56.00    .40    .33    56.00    .40    .33   CLAYSTONE,BROWNISH RED

56.40    .60    .53    56.40    .60    .53   BASALT,GREY,VESICULAR                                                    50

57.00   3.00   3.00    57.00   3.00   3.00   BASALT,DARK GREY,VESICULAR                                               58

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

60.00   3.00   3.00    60.00   3.00   3.00   BASALT,GREY,VESICULAR                                                    64

63.00   3.00   2.90    63.00   3.00   2.90   BASALT,GREY                                                              82

66.00   3.00   2.80    66.00   3.00   2.80   BASALT,GREY                                                              37

69.00   1.10   1.10    69.00   1.10   1.10   BASALT,GREY,VESICULAR                                                    48

TRAP

*************************************************************************************************************************

MOTUR

70.10   1.90   1.90    70.10   1.90   1.90   SANDSTONE(MEDIUM GRAINED),GREY                                           61

72.00   3.00   2.42    72.00   3.00   2.42   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

75.00   2.50   1.85    75.00   2.50   1.85   SANDSTONE(FINE GRAINED),LIGHT BROWN

77.50   2.50   1.92    77.50   2.50   1.92   CLAYSTONE,GREY

80.00   1.00    .80    80.00   1.00    .80   SILTSTONE,BROWNISH BROWN

81.00   2.00   1.75    81.00   2.00   1.75   SILTSTONE,BROWNISH BROWN

83.00   1.50   1.39    83.00   1.50   1.39   SILTSTONE,BROWNISH BROWN

84.50    .53    .50    84.50    .53    .50   SILTSTONE,BROWNISH BROWN

85.03    .97    .97    85.03    .97    .97   SANDSTONE(FINE GRAINED),GREY

86.00   1.90   1.80    86.00   1.90   1.80   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

87.90   1.10    .86    87.90   1.10    .86   CLAYSTONE,BROWNISH BROWN

89.00   3.00   2.70    89.00   3.00   2.70   CLAYSTONE,BROWNISH BROWN

92.00   1.00    .74    92.00   1.00    .74   CLAYSTONE,BROWNISH BROWN

93.00   2.00   1.90    93.00   2.00   1.90   CLAYSTONE,BROWNISH BROWN

95.00   1.80   1.70    95.00   1.80   1.70   CLAYSTONE,BROWNISH BROWN

96.80   1.20   1.10    96.80   1.20   1.10   CLAYSTONE,BROWNISH RED

98.00   1.15    .75    98.00   1.15    .75   CLAYSTONE,BROWNISH BROWN

99.15    .90    .50    99.15    .90    .50   MUDSTONE,GREY

100.05    .95    .55   100.05    .95    .55   SANDSTONE(FINE GRAINED),BROWNISH RED

101.00   3.00   2.86   101.00   3.00   2.86   SILTSTONE,BROWNISH BROWN

104.00    .54    .45   104.00    .54    .45   SILTSTONE,GREY

104.54    .49    .40   104.54    .49    .40   CLAYSTONE,GREY

105.03    .97    .88   105.03    .97    .88   CLAYSTONE,BROWNISH BROWN

106.00   1.00    .97   106.00   1.00    .97   CLAYSTONE,BROWNISH BROWN

107.00   3.00   2.80   107.00   3.00   2.80   CLAYSTONE,BROWNISH BROWN

110.00    .76    .60   110.00    .76    .60   SILTSTONE,BROWNISH BROWN

110.76    .90    .74   110.76    .90    .74   SILTSTONE,GREY

111.66   1.34   1.20   111.66   1.34   1.20   SILTSTONE,BROWNISH RED

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

113.00    .35    .25   113.00    .35    .25   CLAYSTONE,BROWNISH RED

113.35   2.65   2.60   113.35   2.65   2.60   SILTSTONE,BROWNISH RED                                                   57

116.00   1.87   1.75   116.00   1.87   1.75   SILTSTONE,BROWNISH RED                                                   72

117.87    .46    .40   117.87    .46    .40   SANDSTONE(MED. TO C. GRAINED),GREY                                       33

118.33    .67    .60   118.33    .67    .60   CLAYSTONE,BROWNISH RED                                                   40

119.00   3.00   2.80   119.00   3.00   2.80   CLAYSTONE,BROWNISH RED                                                   42

122.00   3.00   2.86   122.00   3.00   2.86   SILTSTONE,GREY                                                           32

125.00   1.60   1.50   125.00   1.60   1.50   SILTSTONE,GREY

126.60   1.40   1.28   126.60   1.40   1.28   SANDSTONE(COARSE GRAINED),GREY

128.00   1.00    .80   128.00   1.00    .80   SANDSTONE(COARSE GRAINED),GREY

129.00   3.00   2.85   129.00   3.00   2.85   CLAYSTONE,GREY

132.00    .50    .48   132.00    .50    .48   SILTSTONE,GREY

132.50   3.00   2.70   132.50   3.00   2.70   SILTSTONE,GREY

135.50   2.00   1.80   135.50   2.00   1.80   SILTSTONE,GREY

137.50   1.00    .85   137.50   1.00    .85   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN

138.50   3.00   1.80   138.50   3.00   1.80   CLAYSTONE,BROWNISH RED

141.50   1.27   1.20   141.50   1.27   1.20   CLAYSTONE,BROWNISH BROWN

142.77   1.73   1.65   142.77   1.73   1.65   SANDSTONE(FINE GRAINED),BROWNISH BROWN

144.50   3.00   1.90   144.50   3.00   1.90   SANDSTONE(FINE GRAINED),GREY

147.50   1.90   1.84   147.50   1.90   1.84   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED

149.40    .32    .24   149.40    .32    .24   SHALE,GREY

149.72    .78    .71   149.72    .78    .71   SANDSTONE(COARSE GRAINED),GREY

150.50   1.50   1.25   150.50   1.50   1.25   SANDSTONE(MED. TO C. GRAINED),GREY

152.00   2.50   2.35   152.00   2.50   2.35   SANDSTONE(MED. TO C. GRAINED),YELLOWISHBROWN

154.50    .70    .60   154.50    .70    .60   SANDSTONE,BROWN

155.20   2.30   2.20   155.20   2.30   2.20   MUDSTONE,BROWN

157.50   3.00   2.85   157.50   3.00   2.85   SILTSTONE,BROWNISH RED

160.50   3.00   2.80   160.50   3.00   2.80   SILTSTONE,BROWNISH RED

163.50   3.00   1.90   163.50   3.00   1.90   SANDSTONE(FINE GRAINED),GREY,FRACTURE                                    65

166.50   3.00   2.70   166.50   3.00   2.70   SILTSTONE,GREY,FRACTURE

169.50   3.00   2.70   169.50   3.00   2.70   SILTSTONE,GREY,FRACTURE

172.50   1.50   1.30   172.50   1.50   1.30   CLAYSTONE,BROWNISH BROWN

174.00   1.50   1.40   174.00   1.50   1.40   CLAYSTONE,BROWNISH BROWN,FRACTURE

175.50   1.50   1.50   175.50   1.50   1.50   MUDSTONE,BROWNISH BROWN

177.00   3.00   2.77   177.00   3.00   2.77   MUDSTONE,BROWNISH RED

180.00   3.00   2.80   180.00   3.00   2.80   SILTSTONE,BROWNISH RED

183.00   3.00   2.83   183.00   3.00   2.83   SILTSTONE,BROWNISH BROWN,FRACTURE

186.00   3.00   2.90   186.00   3.00   2.90   SILTSTONE,GREY,SHALY,FRACTURE

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

189.00    .67    .57   189.00    .67    .57   SILTSTONE,GREY,SHALY

189.67   1.33   1.15   189.67   1.33   1.15   CLAYSTONE,BROWNISH BROWN

191.00   1.50   1.30   191.00   1.50   1.30   CLAYSTONE,BROWNISH BROWN

192.50   3.00   2.88   192.50   3.00   2.88   CLAYSTONE,BROWNISH RED

195.50   1.00    .98   195.50   1.00    .98   CLAYSTONE,BROWNISH RED

196.50    .38    .30   196.50    .38    .30   CLAYSTONE,BROWNISH RED

196.88   1.18   1.10   196.88   1.18   1.10   SILTSTONE,BROWNISH RED

198.06   1.44   1.32   198.06   1.44   1.32   CLAYSTONE,BROWNISH RED

199.50   1.40   1.30   199.50   1.40   1.30   SILTSTONE,BROWNISH RED

200.90   1.60   1.43   200.90   1.60   1.43   SANDSTONE(MED. TO C. GRAINED),BROWN

202.50    .82    .72   202.50    .82    .72   SANDSTONE(MED. TO C. GRAINED),BROWN

203.32   2.18   2.08   203.32   2.18   2.08   CLAYSTONE,BROWNISH RED

205.50   2.00   1.83   205.50   2.00   1.83   CLAYSTONE,BROWNISH RED

207.50   1.00    .70   207.50   1.00    .70   CLAYSTONE,BROWNISH RED

208.50   2.00   1.66   208.50   2.00   1.66   CLAYSTONE,BROWNISH RED

210.50   1.50   1.30   210.50   1.50   1.30   CLAYSTONE,BROWNISH RED

212.00   1.40   1.26   212.00   1.40   1.26   CLAYSTONE,BROWNISH RED

213.40   1.60   1.50   213.40   1.60   1.50   SILTSTONE,BROWNISH GREY

215.00   1.50   1.32   215.00   1.50   1.32   SANDSTONE(FINE GRAINED),BROWNISH GREY

216.50    .11    .08   216.50    .11    .08   SANDSTONE(FINE GRAINED),BROWNISH GREY

216.61    .89    .73   216.61    .89    .73   MUDSTONE,GREY

217.50    .70    .61   217.50    .70    .61   MUDSTONE,GREY

218.20    .30    .20   218.20    .30    .20   CLAYSTONE,BROWNISH RED

218.50   3.00   2.56   218.50   3.00   2.56   CLAYSTONE,BROWNISH RED

221.50   2.00   1.73   221.50   2.00   1.73   CLAYSTONE,BROWNISH RED

223.50   3.00   2.55   223.50   3.00   2.55   CLAYSTONE,BROWNISH RED

226.50   2.00   1.94   226.50   2.00   1.94   CLAYSTONE,BROWNISH RED

228.50   1.00    .92   228.50   1.00    .92   CLAYSTONE,BROWNISH RED

229.50   1.40   1.40   229.50   1.40   1.40   CLAYSTONE,BROWNISH RED

230.90   1.60   1.55   230.90   1.60   1.55   CLAYSTONE,BROWNISH RED

232.50   1.50   1.49   232.50   1.50   1.49   CLAYSTONE,BROWNISH RED

234.00   1.50   1.46   234.00   1.50   1.46   CLAYSTONE,BROWNISH RED

235.50   1.50   1.44   235.50   1.50   1.44   CLAYSTONE,BROWNISH RED

237.00   1.50   1.47   237.00   1.50   1.47   CLAYSTONE,BROWNISH GREY

238.50   1.50   1.48   238.50   1.50   1.48   CLAYSTONE,BROWNISH GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

240.00   1.00   1.00   240.00   1.00   1.00   CLAYSTONE,BROWNISH GREY

241.00   2.00   1.98   241.00   2.00   1.98   CLAYSTONE,BROWNISH GREY

243.00   3.00   2.96   243.00   3.00   2.96   CLAYSTONE,BROWNISH RED

246.00   3.00   2.80   246.00   3.00   2.80   CLAYSTONE,BROWNISH RED

249.00   1.03    .98   249.00   1.03    .98   CLAYSTONE,BROWNISH RED

250.03   1.97   1.87   250.03   1.97   1.87   SANDSTONE(FINE GRAINED),BROWNISH RED                                     61

252.00    .80    .65   252.00    .80    .65   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    79

252.80   2.20   2.04   252.80   2.20   2.04   CLAYSTONE,BROWNISH RED                                                   50

255.00   1.18   1.10   255.00   1.18   1.10   CLAYSTONE,BROWNISH RED                                                   42

256.18    .70    .65   256.18    .70    .65   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    51

256.88    .14    .14   256.88    .14    .14   CLAYSTONE,BROWNISH RED

257.02    .98    .86   257.02    .98    .86   SANDSTONE(FINE GRAINED),BROWNISH RED                                     48

258.00   2.42   2.30   258.00   2.42   2.30   SANDSTONE(FINE GRAINED),BROWNISH RED                                     39

260.42    .20    .15   260.42    .20    .15   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

260.62    .38    .33   260.62    .38    .33   SANDSTONE(FINE GRAINED),BROWNISH RED

261.00   1.50   1.45   261.00   1.50   1.45   CLAYSTONE,BROWNISH GREY                                                  33

262.50   1.50   1.41   262.50   1.50   1.41   CLAYSTONE,BROWNISH GREY

264.00    .07    .07   264.00    .07    .07   CLAYSTONE,BROWNISH GREY

264.07    .11    .11   264.07    .11    .11   SANDSTONE(FINE GRAINED),GREYISH WHITE

264.18   1.32   1.23   264.18   1.32   1.23   CLAYSTONE,BROWNISH RED                                                   11

265.50    .56    .54   265.50    .56    .54   CLAYSTONE,BROWNISH RED                                                   23

266.06    .94    .86   266.06    .94    .86   SANDSTONE(FINE GRAINED),BROWNISH RED                                     67

267.00    .83    .73   267.00    .83    .73   SANDSTONE(FINE GRAINED),BROWNISH RED                                     35

267.83    .35    .32   267.83    .35    .32   MUDSTONE,GREY

268.18    .99    .91   268.18    .99    .91   SILTSTONE,BROWNISH GREY                                                  27

269.17    .83    .78   269.17    .83    .78   SANDSTONE(FINE GRAINED),BROWNISH RED                                     86

270.00   2.37   2.24   270.00   2.37   2.24   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           69

272.37    .18    .15   272.37    .18    .15   CLAYSTONE,BROWNISH RED

272.55    .45    .38   272.55    .45    .38   SANDSTONE(FINE GRAINED),BROWNISH RED                                     44

273.00    .30    .30   273.00    .30    .30   SANDSTONE(FINE GRAINED),BROWNISH RED                                    100

273.30   2.70   2.70   273.30   2.70   2.70   SILTSTONE,BROWNISH RED                                                   64

276.00   1.02    .91   276.00   1.02    .91   SANDSTONE(COARSE GRAINED),GREYISH WHITE                                  58

277.02   1.98   1.87   277.02   1.98   1.87   CLAYSTONE,BROWNISH RED,FRACTURE                                         6.1

279.00   2.53   2.50   279.00   2.53   2.50   CLAYSTONE,BROWNISH RED                                                   13

281.53    .47    .47   281.53    .47    .47   SANDSTONE(FINE GRAINED),BROWNISH WHITE                                   81

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

282.00    .97    .97   282.00    .97    .97   SANDSTONE(FINE GRAINED),BROWNISH RED                                     64

282.97   1.91   1.91   282.97   1.91   1.91   SILTSTONE,BROWNISH GREY                                                  66

284.88    .12    .12   284.88    .12    .12   MUDSTONE,GREY

285.00   3.00   2.97   285.00   3.00   2.97   CLAYSTONE,MEDIUM CRYSTALLINE                                             54

288.00    .72    .69   288.00    .72    .69   CLAYSTONE,MEDIUM CRYSTALLINE                                             57

288.72    .45    .41   288.72    .45    .41   SANDSTONE(COARSE GRAINED),WHITE                                          44

289.17    .33    .30   289.17    .33    .30   CLAYSTONE,BROWNISH RED,FRACTURE

289.50    .54    .51   289.50    .54    .51   SANDSTONE(FINE GRAINED),WHITE

290.04    .96    .92   290.04    .96    .92   CLAYSTONE,BROWNISH RED,FRACTURE

291.00   1.00    .92   291.00   1.00    .92   CLAYSTONE,BROWNISH RED,FRACTURE

292.00   2.00   1.85   292.00   2.00   1.85   CLAYSTONE,BROWNISH RED

294.00   1.00    .92   294.00   1.00    .92   CLAYSTONE,BROWNISH RED

295.00   1.20   1.12   295.00   1.20   1.12   CLAYSTONE,BROWNISH RED

296.20    .80    .80   296.20    .80    .80   CLAYSTONE,BROWNISH RED,FRACTURE

297.00   1.50   1.49   297.00   1.50   1.49   CLAYSTONE,BROWNISH RED

298.50   1.50   1.50   298.50   1.50   1.50   CLAYSTONE,BROWNISH RED

300.00    .52    .52   300.00    .52    .52   CLAYSTONE,BROWNISH RED

300.52    .10    .10   300.52    .10    .10   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

300.62    .30    .30   300.62    .30    .30   MUDSTONE,BROWNISH GREY

300.92   2.08   2.08   300.92   2.08   2.08   CLAYSTONE,BROWNISH RED

303.00   3.00   2.92   303.00   3.00   2.92   CLAYSTONE,BROWNISH RED,FRACTURE                                          10

306.00   2.43   2.43   306.00   2.43   2.43   CLAYSTONE,BROWNISH RED                                                   84

308.43    .47    .47   308.43    .47    .47   SILTSTONE,BROWNISH GREY

308.90    .10    .10   308.90    .10    .10   SANDSTONE(COARSE GRAINED),BROWN

309.00   1.82   1.82   309.00   1.82   1.82   SANDSTONE(COARSE GRAINED),BROWN,FRACTURE                                 51

310.82    .68    .68   310.82    .68    .68   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    74

311.50    .50    .50   311.50    .50    .50   MUDSTONE,GREY

312.00    .11    .10   312.00    .11    .10   MUDSTONE,GREY

312.11    .78    .70   312.11    .78    .70   CLAYSTONE,BROWNISH RED,FRACTURE                                          15

312.89    .11    .10   312.89    .11    .10   SANDSTONE(FINE GRAINED),BROWNISH BROWN

313.00    .42    .42   313.00    .42    .42   SANDSTONE(FINE GRAINED),BROWNISH BROWN                                   62

313.42   1.58   1.58   313.42   1.58   1.58   CLAYSTONE,BROWNISH RED,FRACTURE                                          40

315.00    .93    .83   315.00    .93    .83   CLAYSTONE,BROWNISH RED,FRACTURE                                          16

315.93    .24    .16   315.93    .24    .16   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

316.17   1.83   1.70   316.17   1.83   1.70   SANDSTONE(FINE GRAINED),BROWNISH BROWN                                   77

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

318.00   1.30   1.20   318.00   1.30   1.20   SANDSTONE(FINE GRAINED),BROWNISH BROWN

319.30    .20    .18   319.30    .20    .18   CLAYSTONE,BROWNISH RED

319.50   1.15   1.05   319.50   1.15   1.05   SANDSTONE(FINE GRAINED),BROWNISH RED                                     54

320.65    .35    .26   320.65    .35    .26   CLAYSTONE,BROWNISH BROWN

321.00   2.00   1.85   321.00   2.00   1.85   CLAYSTONE,BROWNISH BROWN

323.00   3.00    .25   323.00   3.00    .25   SANDSTONE(FINE GRAINED),BROWNISH BROWN

326.00   2.00    .65   326.00   2.00    .65   SANDSTONE(FINE GRAINED),BROWNISH RED

328.00   1.50    .78   328.00   1.50    .78   SANDSTONE(FINE GRAINED),BROWNISH RED                                     39

329.50   1.50    .25   329.50   1.50    .25   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

331.00   1.50    .22   331.00   1.50    .22   SANDSTONE(COARSE GRAINED),BROWNISH RED

332.50   1.50    .72   332.50   1.50    .72   SANDSTONE(COARSE GRAINED),BROWNISH RED

334.00   1.50    .62   334.00   1.50    .62   SANDSTONE(COARSE GRAINED),BROWNISH RED

335.50    .15    .12   335.50    .15    .12   CLAYSTONE,BROWNISH BROWN

335.65   1.35    .34   335.65   1.35    .34   SANDSTONE(FINE GRAINED),BROWNISH RED

337.00   1.50    .20   337.00   1.50    .20   SANDSTONE(FINE GRAINED),BROWNISH RED

338.50   1.50    .66   338.50   1.50    .66   SANDSTONE(FINE GRAINED),BROWNISH RED

340.00   1.30    .52   340.00   1.30    .52   SANDSTONE(FINE GRAINED),BROWNISH RED                                     41

341.30    .10    .08   341.30    .10    .08   CLAYSTONE,BROWNISH BROWN

341.40    .10    .06   341.40    .10    .06   SANDSTONE(FINE GRAINED),BROWNISH BROWN

341.50   1.50    .38   341.50   1.50    .38   SANDSTONE(FINE GRAINED),BROWNISH RED                                     11

343.00   2.15    .90   343.00   2.15    .90   CLAYSTONE,BROWNISH BROWN                                                 40

345.15    .85    .68   345.15    .85    .68   SANDSTONE(FINE GRAINED),BROWNISH RED                                     26

346.00   1.50    .26   346.00   1.50    .26   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

347.50   1.50    .34   347.50   1.50    .34   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

349.00   1.00    .40   349.00   1.00    .40   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

350.00   3.00   1.66   350.00   3.00   1.66   CLAYSTONE,BROWNISH BROWN,FRACTURE                                        15

353.00   3.00   1.67   353.00   3.00   1.67   CLAYSTONE,BROWNISH BROWN

356.00   3.00   1.80   356.00   3.00   1.80   CLAYSTONE,BROWNISH BROWN                                                 32

359.00   2.40   1.40   359.00   2.40   1.40   SANDSTONE(FINE GRAINED),GREYISH RED                                      38

361.40    .10    .10   361.40    .10    .10   SANDSTONE(FINE GRAINED),GREYISH RED

361.50   2.00   1.10   361.50   2.00   1.10   SANDSTONE(FINE GRAINED),BROWNISH BROWN

363.50   1.80   1.80   363.50   1.80   1.80   SANDSTONE(FINE GRAINED),BROWNISH BROWN

365.30   3.00   2.60   365.30   3.00   2.60   SANDSTONE(FINE GRAINED),BROWNISH BROWN

368.30   1.45   1.21   368.30   1.45   1.21   CLAYSTONE,BROWNISH BROWN                                                 49

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

369.75    .55    .40   369.75    .55    .40   SANDSTONE(FINE GRAINED),GREYISH RED

370.30   1.70   1.48   370.30   1.70   1.48   SANDSTONE(FINE GRAINED),BROWNISH WHITE                                   19

372.00   1.50   1.25   372.00   1.50   1.25   SANDSTONE(FINE GRAINED),WHITE                                            25

373.50   1.12   1.00   373.50   1.12   1.00   SANDSTONE(FINE GRAINED),BROWN

374.62    .85    .77   374.62    .85    .77   CLAYSTONE,BROWNISH BROWN                                                 89

375.47    .53    .45   375.47    .53    .45   SANDSTONE(FINE GRAINED),BROWN

376.00   3.00   2.80   376.00   3.00   2.80   SANDSTONE(FINE GRAINED),BROWNISH BROWN                                   40

379.00   3.00   2.85   379.00   3.00   2.85   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH BROWN,                        55

382.00    .80    .75   382.00    .80    .75   CLAYSTONE,BROWNISH BROWN                                                 40

382.80    .70    .65   382.80    .70    .65   SANDSTONE(FINE GRAINED),BROWNISH RED,FRIABLE                             34

383.50   1.50   1.30   383.50   1.50   1.30   SANDSTONE(FINE GRAINED),BROWN                                           6.7

385.00   1.00    .80   385.00   1.00    .80   SANDSTONE(FINE GRAINED),BROWN

386.00   1.00    .70   386.00   1.00    .70   SANDSTONE(FINE GRAINED),BROWN                                            10

387.00   2.40   2.15   387.00   2.40   2.15   SANDSTONE(FINE GRAINED),BROWN

389.40    .51    .41   389.40    .51    .41   SANDSTONE(FINE GRAINED),BROWN                                            43

389.91    .82    .80   389.91    .82    .80   CLAYSTONE,BROWNISH BROWN                                                 99

390.73    .27    .25   390.73    .27    .25   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           44

391.00   1.50   1.30   391.00   1.50   1.30   SANDSTONE(FINE TO MEDIUM GRAINED),BROWN

392.50   2.00   1.85   392.50   2.00   1.85   SANDSTONE(FINE TO MEDIUM GRAINED),BROWN                                   8

394.50   1.50   1.36   394.50   1.50   1.36   SANDSTONE(FINE TO MEDIUM GRAINED),BROWN

396.00   2.00   1.85   396.00   2.00   1.85   SANDSTONE(FINE TO MEDIUM GRAINED),BROWN                                 7.5

398.00   1.00    .90   398.00   1.00    .90   SANDSTONE(FINE GRAINED),BROWNISH RED

399.00   1.00    .94   399.00   1.00    .94   SANDSTONE(FINE GRAINED),BROWNISH RED

400.00   2.00   1.86   400.00   2.00   1.86   SANDSTONE(FINE GRAINED),BROWNISH BROWN                                   13

402.00   3.00   2.70   402.00   3.00   2.70   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           36

405.00   2.00   1.82   405.00   2.00   1.82   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH BROWN                         52

407.00   3.00   2.90   407.00   3.00   2.90   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH BROWN                         66

410.00   3.00   2.68   410.00   3.00   2.68   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   24

413.00   1.50   1.40   413.00   1.50   1.40   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                  6.7

414.50   1.06    .96   414.50   1.06    .96   CLAYSTONE,BROWNISH BROWN                                                 70

415.56   1.44   1.25   415.56   1.44   1.25   SANDSTONE(FINE GRAINED),GREY                                             45

417.00   3.00   2.75   417.00   3.00   2.75   SANDSTONE(FINE GRAINED),BROWN

420.00   3.00   2.80   420.00   3.00   2.80   SANDSTONE(FINE GRAINED),BROWN

423.00   1.87   1.83   423.00   1.87   1.83   SANDSTONE(FINE GRAINED),BROWN                                            33

424.87    .13    .13   424.87    .13    .13   CLAYSTONE,BROWNISH BROWN

425.00    .37    .37   425.00    .37    .37   CLAYSTONE,BROWNISH BROWN                                                 86

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

425.37    .63    .63   425.37    .63    .63   SANDSTONE(FINE GRAINED),REDDISH GREY

426.00   2.00   2.00   426.00   2.00   2.00   SANDSTONE(FINE GRAINED),BROWNISH BROWN                                   30

428.00   1.00   1.00   428.00   1.00   1.00   SANDSTONE(FINE GRAINED),BROWN                                            46

429.00    .68    .64   429.00    .68    .64   CLAYSTONE,BROWNISH BROWN                                                 93

429.68   1.52   1.50   429.68   1.52   1.50   SANDSTONE(FINE GRAINED),BROWNISH BROWN                                   59

431.20   1.80   1.80   431.20   1.80   1.80   SANDSTONE(FINE GRAINED),BROWNISH BROWN                                   17

433.00   3.00   2.88   433.00   3.00   2.88   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           28

436.00   3.00   2.90   436.00   3.00   2.90   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH BROWN                         62

439.00   3.00   2.85   439.00   3.00   2.85   SANDSTONE(FINE GRAINED),BROWNISH RED                                     25

442.00   3.00   2.92   442.00   3.00   2.92   SANDSTONE(FINE GRAINED),BROWNISH RED                                     37

445.00   3.00   2.90   445.00   3.00   2.90   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH BROWN

448.00   2.00   1.96   448.00   2.00   1.96   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           21

450.00   3.00   2.88   450.00   3.00   2.88   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           13

453.00   3.00   2.98   453.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           34

456.00   3.00   2.99   456.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),BROWN                                      36

459.00   2.12   2.10   459.00   2.12   2.10   SANDSTONE(MEDIUM GRAINED),BROWN                                          39

461.12    .47    .47   461.12    .47    .47   CLAYSTONE,BROWNISH BROWN                                                 26

461.59    .41    .40   461.59    .41    .40   SANDSTONE(MEDIUM GRAINED),BROWNISH BROWN

462.00   3.00   2.97   462.00   3.00   2.97   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               45

465.00   3.00   2.99   465.00   3.00   2.99   SANDSTONE(COARSE GRAINED),BROWNISH RED                                   51

468.00   3.00   2.95   468.00   3.00   2.95   SANDSTONE(COARSE GRAINED),BROWNISH RED                                   61

471.00   3.00   2.88   471.00   3.00   2.88   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               32

474.00   3.00   2.82   474.00   3.00   2.82   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                              3.3

477.00   3.00   2.90   477.00   3.00   2.90   SANDSTONE(FINE TO COARSE GRAINED),BROWNISH RED                           48

480.00   3.00   2.97   480.00   3.00   2.97   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           36

483.00   3.00   2.91   483.00   3.00   2.91   SANDSTONE(FINE GRAINED),BROWN                                            16

486.00   3.00   2.98   486.00   3.00   2.98   SANDSTONE(COARSE GRAINED),BROWN                                          56

489.00   3.00   3.00   489.00   3.00   3.00   SANDSTONE(COARSE GRAINED),BROWNISH RED                                   65

492.00   3.00   3.00   492.00   3.00   3.00   SANDSTONE(FINE TO COARSE GRAINED),BROWN                                  68

495.00   3.00   2.97   495.00   3.00   2.97   SANDSTONE(FINE TO COARSE GRAINED),BROWN                                  56

498.00   1.50   1.50   498.00   1.50   1.50   SANDSTONE(COARSE GRAINED),BROWNISH RED                                   72

499.50   1.50   1.50   499.50   1.50   1.50   SANDSTONE(COARSE GRAINED),BROWN                                          52

501.00   3.00   3.00   501.00   3.00   3.00   SANDSTONE(COARSE GRAINED),BROWNISH BROWN                                 22

504.00   3.00   2.94   504.00   3.00   2.94   SANDSTONE(MEDIUM GRAINED),BROWNISH RED                                   87

507.00   2.60   2.60   507.00   2.60   2.60   SANDSTONE(MED. TO C. GRAINED),BROWNISH BROWN                             24

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

509.60   1.40   1.37   509.60   1.40   1.37   SANDSTONE(MEDIUM GRAINED),BROWNISH BROWN                                 28

511.00   2.00   2.00   511.00   2.00   2.00   SANDSTONE(MEDIUM GRAINED),BROWNISH BROWN                                 48

513.00   3.00   2.95   513.00   3.00   2.95   SANDSTONE(FINE TO COARSE GRAINED),BROWNISH BROWN                         17

516.00   1.50   1.50   516.00   1.50   1.50   SANDSTONE(FINE GRAINED),BROWNISH BROWN                                   31

517.50   1.50   1.50   517.50   1.50   1.50   SANDSTONE(FINE GRAINED),BROWNISH BROWN                                   31

519.00   1.07   1.06   519.00   1.07   1.06   CLAYSTONE,BROWNISH BROWN                                                 89

520.07    .72    .70   520.07    .72    .70   SANDSTONE(MEDIUM GRAINED),BROWNISH RED                                   92

520.79    .42    .42   520.79    .42    .42   CLAYSTONE,BROWNISH BROWN                                                 95

521.21    .79    .77   521.21    .79    .77   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    71

522.00   3.00   3.00   522.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS                                    59

525.00   3.00   3.00   525.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS                                    32

528.00    .52    .52   528.00    .52    .52   CLAYSTONE,CHOCOLATEBROWN                                                 23

528.52    .54    .53   528.52    .54    .53   SILTSTONE,GREY,SHALY                                                     56

529.06    .94    .90   529.06    .94    .90   MUDSTONE,GREY

530.00   1.00    .97   530.00   1.00    .97   MUDSTONE,GREY

531.00   3.00   2.99   531.00   3.00   2.99   MUDSTONE,GREY

534.00   1.20   1.19   534.00   1.20   1.19   MUDSTONE,GREY

535.20   1.00   1.00   535.20   1.00   1.00   MUDSTONE,GREY

536.20   2.30   2.29   536.20   2.30   2.29   MUDSTONE,GREY

538.50   3.00   2.99   538.50   3.00   2.99   MUDSTONE,GREY

541.50    .98    .96   541.50    .98    .96   MUDSTONE,GREY

542.48   2.02   2.00   542.48   2.02   2.00   SILTSTONE,GREY,SHALY

544.50   2.50   2.00   544.50   2.50   2.00   SILTSTONE,GREY,SHALY

547.00   3.00   2.99   547.00   3.00   2.99   MUDSTONE,VARIEGATE

550.00   2.00   1.98   550.00   2.00   1.98   MUDSTONE,VARIEGATE

552.00   1.50   1.49   552.00   1.50   1.49   MUDSTONE,VARIEGATE

553.50   1.50   1.50   553.50   1.50   1.50   MUDSTONE,GREY,SLICKENSIDED

555.00   3.00   2.99   555.00   3.00   2.99   MUDSTONE,GREY

558.00   2.50   2.50   558.00   2.50   2.50   MUDSTONE,GREY

560.50   2.50   2.49   560.50   2.50   2.49   MUDSTONE,VARIEGATE

563.00   2.50   2.50   563.00   2.50   2.50   MUDSTONE,VARIEGATE

565.50   3.00   1.99   565.50   3.00   1.99   MUDSTONE,GREY

568.50   1.50   1.48   568.50   1.50   1.48   MUDSTONE,GREY

570.00   1.50   1.49   570.00   1.50   1.49   MUDSTONE,GREY

571.50   1.50   1.49   571.50   1.50   1.49   MUDSTONE,GREY

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

573.00   1.00    .99   573.00   1.00    .99   MUDSTONE,GREY

574.00   3.00   2.99   574.00   3.00   2.99   CLAYSTONE,GREY

577.00   1.00   1.00   577.00   1.00   1.00   CLAYSTONE,GREY

578.00   2.00   2.00   578.00   2.00   2.00   CLAYSTONE,GREYISH BROWN

580.00   1.00    .98   580.00   1.00    .98   CLAYSTONE,VARIEGATE

581.00   3.00   2.98   581.00   3.00   2.98   CLAYSTONE,BROWNISH GREY,SLICKENSIDED                                     25

584.00   1.57   1.55   584.00   1.57   1.55   MUDSTONE,GREY                                                           8.3

585.57    .59    .57   585.57    .59    .57   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  46

586.16    .32    .30   586.16    .32    .30   MUDSTONE,GREY

586.48    .52    .50   586.48    .52    .50   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    52

587.00   3.00   2.85   587.00   3.00   2.85   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          72

590.00   2.40   2.40   590.00   2.40   2.40   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                              62

592.40    .60    .58   592.40    .60    .58   CLAYSTONE,BROWN,FRACTURE

593.00   3.00   2.99   593.00   3.00   2.99   CLAYSTONE,BROWNISH GREY,FRACTURE

596.00   1.50   1.48   596.00   1.50   1.48   CLAYSTONE,BROWNISH GREY,FRACTURE

597.50   1.50   1.49   597.50   1.50   1.49   MUDSTONE,GREYISH BROWN

599.00   1.00   1.00   599.00   1.00   1.00   MUDSTONE,GREYISH BROWN                                                   39

600.00   2.08   2.06   600.00   2.08   2.06   MUDSTONE,GREYISH BROWN,FRACTURE                                          17

602.08    .92    .90   602.08    .92    .90   SANDSTONE(FINE GRAINED),GREY                                             34

603.00    .52    .52   603.00    .52    .52   SANDSTONE(FINE GRAINED),GREY                                             35

603.52    .37    .36   603.52    .37    .36   SANDY SHALE,GREY

603.89   2.11   2.10   603.89   2.11   2.10   MUDSTONE,GREYISH BROWN                                                   10

606.00   3.00   3.00   606.00   3.00   3.00   CLAYSTONE,VARIEGATE

609.00   1.50   1.50   609.00   1.50   1.50   CLAYSTONE,VARIEGAT,FRACTURE                                              48

610.50   1.27   1.25   610.50   1.27   1.25   CLAYSTONE,VARIEGAT,SLICKENSIDED

611.77    .23    .23   611.77    .23    .23   SANDSTONE(COARSE GRAINED),GREYISH WHITE                                  52

612.00   1.56   1.55   612.00   1.56   1.55   SANDSTONE(COARSE GRAINED),GREYISH WHITE                                  51

613.56    .95    .94   613.56    .95    .94   SANDY SHALE,GREY                                                         53

614.51    .49    .48   614.51    .49    .48   MUDSTONE,GREY

615.00   3.00   3.00   615.00   3.00   3.00   CLAYSTONE,GREY,SLICKENSIDED                                              51

618.00   3.00   2.98   618.00   3.00   2.98   MUDSTONE,GREY                                                            34

621.00   1.00   1.00   621.00   1.00   1.00   MUDSTONE,GREYISH BROWN                                                   42

622.00   2.00   1.97   622.00   2.00   1.97   CLAYSTONE,GREYISH BROWN,FRACTURE                                         37

624.00   3.00   2.99   624.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                  14

627.00   3.00   2.97   627.00   3.00   2.97   MUDSTONE,GREY                                                            16

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

630.00   2.00   1.99   630.00   2.00   1.99   MUDSTONE,GREYISH BROWN                                                  8.5

632.00    .70    .66   632.00    .70    .66   SHALE,GREY

632.70    .26    .24   632.70    .26    .24   SANDSTONE(MEDIUM GRAINED),GREYISH BROWN                                  92

632.96   1.04   1.00   632.96   1.04   1.00   MUDSTONE,GREY,FRACTURE

634.00   1.00    .97   634.00   1.00    .97   MUDSTONE,GREYISH BROWN                                                   42

635.00   3.00   2.98   635.00   3.00   2.98   MUDSTONE,BROWNISH GREY                                                   37

638.00   3.00   2.99   638.00   3.00   2.99   MUDSTONE,BROWNISH GREY,FRACTURE                                          22

641.00   3.00   2.95   641.00   3.00   2.95   CLAYSTONE,GREYISH BROWN,FRACTURE                                        4.3

644.00   1.00   1.00   644.00   1.00   1.00   ALTERNATING SHALE AND SST,GREYISH BROWN                                  25

645.00   2.00   2.00   645.00   2.00   2.00   ALTERNATING SHALE AND SST,GREYISH BROWN                                  18

647.00   3.00   2.97   647.00   3.00   2.97   CLAYSTONE,GREYISH BROWN                                                  30

650.00   3.00   2.98   650.00   3.00   2.98   CLAYSTONE,GREYISH BROWN                                                  32

653.00   3.00   2.99   653.00   3.00   2.99   ALTERNATING SHALE AND SST,GREYISH BROWN                                  20

656.00   1.00   1.00   656.00   1.00   1.00   MUDSTONE,GREYISH BROWN

657.00   3.00   2.98   657.00   3.00   2.98   SHALE,GREY                                                               30

660.00   3.00   2.99   660.00   3.00   2.99   SHALE,GREY                                                               22

663.00   2.10   2.08   663.00   2.10   2.08   SANDY SHALE,GREY                                                         60

665.10    .47    .46   665.10    .47    .46   SANDSTONE(FINE GRAINED),GREY                                             81

665.57    .43    .41   665.57    .43    .41   SHALE,GREY

666.00   3.00   2.98   666.00   3.00   2.98   SHALE,GREY,SLICKENSIDED                                                  19

669.00   3.00   2.98   669.00   3.00   2.98   MUDSTONE,BROWNISH GREY                                                  5.7

672.00   2.18   2.15   672.00   2.18   2.15   CLAYSTONE,GREYISH BROWN                                                  43

674.18    .82    .80   674.18    .82    .80   SANDSTONE(FINE GRAINED),GREY                                             62

675.00    .54    .53   675.00    .54    .53   MUDSTONE,GREY

675.54   1.55   1.55   675.54   1.55   1.55   CLAYSTONE,BROWNISH GREY                                                  31

677.09    .91    .91   677.09    .91    .91   SANDY SHALE,GREY                                                         54

678.00   3.00   2.99   678.00   3.00   2.99   CLAYSTONE,GREYISH BROWN,SLICKENSIDED                                     27

681.00   1.60   1.60   681.00   1.60   1.60   CLAYSTONE,BROWNISH GREY                                                  32

682.60    .50    .50   682.60    .50    .50   SANDSTONE(MEDIUM GRAINED),GREYISH BROWN                                 100

683.10    .90    .90   683.10    .90    .90   SHALE,GREY

684.00   3.00   2.99   684.00   3.00   2.99   SHALE,GREY                                                               21

687.00   2.00   1.98   687.00   2.00   1.98   CLAYSTONE,GREYISH BROWN                                                  12

689.00   1.00   1.00   689.00   1.00   1.00   CLAYSTONE,GREYISH BROWN

690.00   3.00   2.99   690.00   3.00   2.99   CLAYSTONE,GREYISH BROWN,FRACTURE                                         18

693.00   3.00   2.99   693.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                  26

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

696.00   3.00   2.99   696.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                 5.7

699.00   1.50   1.49   699.00   1.50   1.49   CLAYSTONE,BROWNISH GREY                                                  62

700.50   1.50   1.50   700.50   1.50   1.50   CLAYSTONE,BROWNISH GREY                                                  27

702.00   3.00   2.99   702.00   3.00   2.99   CLAYSTONE,BROWN                                                          24

705.00    .42    .42   705.00    .42    .42   CLAYSTONE,BROWN                                                          86

705.42    .98    .98   705.42    .98    .98   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          71

706.40   1.60   1.60   706.40   1.60   1.60   CLAYSTONE,BROWN                                                          42

708.00   1.20   1.20   708.00   1.20   1.20   CLAYSTONE,BROWN

709.20    .50    .50   709.20    .50    .50   SANDSTONE(MEDIUM GRAINED),GREY                                           90

709.70   1.30   1.30   709.70   1.30   1.30   CLAYSTONE,GREY,FRACTURE

711.00   1.00    .99   711.00   1.00    .99   CLAYSTONE,BROWN                                                          26

712.00   2.00   1.99   712.00   2.00   1.99   CLAYSTONE,BROWN                                                          13

714.00   3.00   2.99   714.00   3.00   2.99   CLAYSTONE,BROWN,SLICKENSIDED

717.00   3.00   2.99   717.00   3.00   2.99   CLAYSTONE,BROWN                                                          17

720.00   3.00   3.00   720.00   3.00   3.00   CLAYSTONE,BROWNISH GREY,SLICKENSIDED                                     41

723.00   3.00   2.98   723.00   3.00   2.98   CLAYSTONE,BROWNISH GREY                                                  44

726.00   3.00   2.98   726.00   3.00   2.98   CLAYSTONE,BROWN                                                          43

729.00   3.00   2.99   729.00   3.00   2.99   CLAYSTONE,BROWN                                                          19

732.00   3.00   2.99   732.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                  49

735.00   3.00   2.97   735.00   3.00   2.97   MUDSTONE,GREY,FRACTURE

738.00   3.00   2.97   738.00   3.00   2.97   MUDSTONE,GREY,FRACTURE                                                   10

741.00   3.00   2.98   741.00   3.00   2.98   CLAYSTONE,GREYISH BROWN                                                  18

744.00   1.00    .98   744.00   1.00    .98   CLAYSTONE,GREYISH BROWN

745.00   2.00   1.98   745.00   2.00   1.98   CLAYSTONE,BROWNISH GREY,FRACTURE

747.00   3.00   2.99   747.00   3.00   2.99   CLAYSTONE,BROWNISH GREY,FRACTURE

750.00   1.50   1.50   750.00   1.50   1.50   CLAYSTONE,BROWNISH GREY                                                  31

751.50   1.50   1.50   751.50   1.50   1.50   SHALE,GREY                                                               29

753.00   3.00   2.99   753.00   3.00   2.99   SHALE,GREY

756.00   3.00   2.98   756.00   3.00   2.98   MUDSTONE,GREY                                                           9.3

759.00   3.00   2.99   759.00   3.00   2.99   MUDSTONE,GREY                                                            15

762.00   3.00   2.98   762.00   3.00   2.98   MUDSTONE,GREY,FRACTURE                                                   29

765.00   3.00   2.98   765.00   3.00   2.98   MUDSTONE,GREY                                                            31

768.00   3.00   2.98   768.00   3.00   2.98   CLAYSTONE,BROWN                                                          37

771.00   3.00   2.99   771.00   3.00   2.99   CLAYSTONE,GREYISH BROWN

774.00   1.00   1.00   774.00   1.00   1.00   CLAYSTONE,GREYISH BROWN                                                  10

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

775.00   2.00   1.98   775.00   2.00   1.98   CLAYSTONE,GREYISH BROWN                                                   5

777.00   3.00   2.98   777.00   3.00   2.98   CLAYSTONE,GREYISH BROWN                                                  15

780.00   3.00   2.98   780.00   3.00   2.98   CLAYSTONE,GREYISH BROWN                                                  27

783.00   1.00    .98   783.00   1.00    .98   CLAYSTONE,BROWN                                                          34

784.00   2.00   1.97   784.00   2.00   1.97   CLAYSTONE,BROWN                                                          12

786.00   3.00   2.98   786.00   3.00   2.98   MUDSTONE,BROWNISH GREY                                                   24

789.00   3.00   2.97   789.00   3.00   2.97   MUDSTONE,GREYISH BROWN                                                   36

792.00   1.00    .99   792.00   1.00    .99   CLAYSTONE,BROWN                                                          65

793.00   2.00   1.98   793.00   2.00   1.98   CLAYSTONE,BROWN                                                          51

795.00   3.00   2.95   795.00   3.00   2.95   CLAYSTONE,BROWN                                                          25

798.00   3.00   2.99   798.00   3.00   2.99   CLAYSTONE,BROWN                                                          25

801.00   1.50   1.48   801.00   1.50   1.48   CLAYSTONE,BROWN                                                          44

802.50   1.50   1.48   802.50   1.50   1.48   CLAYSTONE,BROWN

804.00   3.00   2.99   804.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                  13

807.00   3.00   2.80   807.00   3.00   2.80   CLAYSTONE,BROWNISH GREY                                                  33

810.00   1.50   1.48   810.00   1.50   1.48   CLAYSTONE,BROWNISH GREY                                                  37

811.50   1.50   1.48   811.50   1.50   1.48   CLAYSTONE,GREYISH BROWN

813.00   2.80   2.80   813.00   2.80   2.80   CLAYSTONE,BROWNISH GREY

815.80    .20    .20   815.80    .20    .20   SANDSTONE(MEDIUM GRAINED),GREY                                          100

816.00    .30    .30   816.00    .30    .30   SANDSTONE(MEDIUM GRAINED),GREY                                           50

816.30   2.70   2.70   816.30   2.70   2.70   CLAYSTONE,BROWN,FRACTURE

819.00   1.00    .98   819.00   1.00    .98   CLAYSTONE,BROWN,FRACTURE

820.00   1.05   1.05   820.00   1.05   1.05   SANDSTONE(FINE GRAINED),BROWNISH GREY                                    39

821.05    .67    .67   821.05    .67    .67   CLAYSTONE,GREY                                                           18

821.72    .28    .28   821.72    .28    .28   SANDSTONE(MEDIUM GRAINED),GREY                                           93

822.00   1.60   1.60   822.00   1.60   1.60   CLAYSTONE,GREYISH BROWN                                                  12

823.60    .20    .20   823.60    .20    .20   SANDSTONE(MEDIUM GRAINED),GREY                                          100

823.80   1.20   1.20   823.80   1.20   1.20   CLAYSTONE,BROWN

825.00   3.00   2.98   825.00   3.00   2.98   CLAYSTONE,BROWN                                                          42

828.00   3.00   2.98   828.00   3.00   2.98   CLAYSTONE,BROWN

831.00   3.00   2.99   831.00   3.00   2.99   CLAYSTONE,BROWN

834.00   1.00    .99   834.00   1.00    .99   CLAYSTONE,BROWN

835.00   2.00   1.97   835.00   2.00   1.97   CLAYSTONE,BROWNISH GREY                                                   5

837.00   3.00   2.99   837.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                  19

840.00   2.00   1.95   840.00   2.00   1.95   CLAYSTONE,BROWN

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

842.00   1.00    .98   842.00   1.00    .98   CLAYSTONE,BROWN

843.00   3.00   2.97   843.00   3.00   2.97   CLAYSTONE,BROWNISH GREY                                                   9

846.00   2.85   2.85   846.00   2.85   2.85   CLAYSTONE,BROWNISH GREY                                                  16

848.85    .15    .15   848.85    .15    .15   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY

849.00   1.37   1.17   849.00   1.37   1.17   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   53

850.37   1.07    .82   850.37   1.07    .82   CLAYSTONE,BROWNISH GREY

851.44    .56    .16   851.44    .56    .16   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   30

852.00    .58    .57   852.00    .58    .57   SANDY SHALE,GREY

852.58    .83    .82   852.58    .83    .82   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   61

853.41    .35    .34   853.41    .35    .34   MUDSTONE,GREY,FRACTURE

853.76   1.24   1.23   853.76   1.24   1.23   CLAYSTONE,BROWNISH GREY,FRACTURE                                         15

855.00   3.00   2.98   855.00   3.00   2.98   CLAYSTONE,BROWNISH GREY,FRACTURE

858.00   3.00   2.99   858.00   3.00   2.99   CLAYSTONE,BROWNISH GREY,FRACTURE

861.00   2.00   1.90   861.00   2.00   1.90   CLAYSTONE,BROWNISH GREY,FRACTURE

863.00   1.00    .95   863.00   1.00    .95   CLAYSTONE,BROWNISH GREY,FRACTURE

864.00   1.50   1.45   864.00   1.50   1.45   CLAYSTONE,GREYISH BROWN,FRACTURE

865.50   2.50   2.48   865.50   2.50   2.48   CLAYSTONE,GREYISH BROWN,FRACTURE

868.00   3.00   2.99   868.00   3.00   2.99   CLAYSTONE,BROWNISH GREY,FRACTURE                                          5

871.00   3.00   2.98   871.00   3.00   2.98   CLAYSTONE,GREYISH BROWN,FRACTURE

874.00   3.00   2.95   874.00   3.00   2.95   CLAYSTONE,BROWNISH GREY,FRACTURE                                         49

877.00   3.00   2.95   877.00   3.00   2.95   CLAYSTONE,BROWNISH GREY,FRACTURE                                         16

880.00   2.00   1.98   880.00   2.00   1.98   CLAYSTONE,BROWNISH GREY,FRACTURE                                         26

882.00   3.00   2.60   882.00   3.00   2.60   CLAYSTONE,BROWNISH GREY,FRACTURE

BOREHOLE CLOSED AT   885.00 M DEPTH

---------------------------------------------------------------------------

BOREHOLE NO    :CMWWB03              LATITUDE    : 2208132.208M                  DATE OF COMMENCEMENT : 17.11.2023

REDUCED LEVEL  : 307.94M             DEPARTURE   : 264852.0102M                  DATE OF COMPLETION: 03.02.2024

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

.00   3.00   2.80      .00   3.00   2.80   BASALT,DARK GREY,FRACTURE                                                72

3.00   3.00   3.00     3.00   3.00   3.00   BASALT,DARK GREY,FRACTURE                                                81

6.00    .85    .85     6.00    .85    .85   BASALT,DARK GREY,FRACTURE                                                81

6.85   1.15   1.15     6.85   1.15   1.15   BASALT,GREENISH GREY

8.00   1.00    .94     8.00   1.00    .94   BASALT,GREENISH GREY

9.00    .26    .21     9.00    .26    .21   BASALT,GREENISH GREY

9.26   2.74   2.49     9.26   2.74   2.49   BASALT,GREY                                                              86

12.00   3.00   3.00    12.00   3.00   3.00   BASALT,DARK GREY                                                         64

15.00   3.00   2.96    15.00   3.00   2.96   BASALT,DARK GREY                                                         88

18.00   3.00   3.00    18.00   3.00   3.00   BASALT,GREY                                                             100

21.00   3.00   3.00    21.00   3.00   3.00   BASALT,GREY,FRACTURE                                                     84

24.00   3.00   3.00    24.00   3.00   3.00   BASALT,GREY,FRACTURE                                                     81

27.00   3.00   3.00    27.00   3.00   3.00   BASALT,GREY,FRACTURE                                                     89

30.00   3.00   3.00    30.00   3.00   3.00   BASALT,GREY,FRACTURE                                                     88

33.00   3.00   3.00    33.00   3.00   3.00   BASALT,GREY                                                             100

36.00   3.00   3.00    36.00   3.00   3.00   BASALT,GREY,FRACTURE                                                     47

39.00   3.00   3.00    39.00   3.00   3.00   BASALT,GREY                                                             100

42.00   3.00   3.00    42.00   3.00   3.00   BASALT,DARK GREY                                                         93

45.00   3.00   3.00    45.00   3.00   3.00   BASALT,GREY,FRACTURE                                                     99

48.00   3.00   3.00    48.00   3.00   3.00   BASALT,GREY                                                              99

51.00    .50    .48    51.00    .50    .48   BASALT,GREY                                                              96

51.50    .93    .83    51.50    .93    .83   CLAYSTONE,BROWNISH RED                                                   35

52.43    .57    .52    52.43    .57    .52   BASALT,DARK GREY                                                         84

53.00   1.00   1.00    53.00   1.00   1.00   BASALT,GREY                                                             100

54.00   3.00   3.00    54.00   3.00   3.00   BASALT,DARK GREY                                                         94

57.00   3.00   3.00    57.00   3.00   3.00   BASALT,DARK GREY,FRACTURE                                                98

60.00   3.00   3.00    60.00   3.00   3.00   BASALT,DARK GREY                                                         93

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

63.00   3.00   3.00    63.00   3.00   3.00   BASALT,DARK GREY                                                         76

66.00   3.00   3.00    66.00   3.00   3.00   BASALT,DARK GREY,FRACTURE                                                89

69.00   3.00   2.94    69.00   3.00   2.94   BASALT,DARK GREY,FRACTURE                                                52

72.00   3.00   2.97    72.00   3.00   2.97   BASALT,DARK GREY,FRACTURE                                                65

TRAP

**********************************************************************************************************************

MOTUR

75.00   3.00   2.98    75.00   3.00   2.98   SANDSTONE(V.FINE GRAINED),GREYISH WHITE

78.00   3.00   2.92    78.00   3.00   2.92   SANDSTONE(V.FINE GRAINED),GREYISH WHITE

81.00   1.00    .95    81.00   1.00    .95   CLAYSTONE,REDDISH WHITE                                                  53

82.00   1.85   1.80    82.00   1.85   1.80   SANDSTONE(V.FINE GRAINED),GREY

83.85    .15    .12    83.85    .15    .12   CLAYSTONE,REDDISH BROWN

84.00    .71    .54    84.00    .71    .54   CLAYSTONE,REDDISH BROWN

84.71   2.29   2.12    84.71   2.29   2.12   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH RED

87.00   2.15   2.05    87.00   2.15   2.05   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH RED

89.15    .85    .74    89.15    .85    .74   CLAYSTONE,BROWNISH RED

90.00   3.00   2.82    90.00   3.00   2.82   CLAYSTONE,REDDISH BROWN

93.00   3.00   2.80    93.00   3.00   2.80   CLAYSTONE,REDDISH BROWN

96.00    .65    .63    96.00    .65    .63   CLAYSTONE,REDDISH BROWN

96.65    .68    .66    96.65    .68    .66   MUDSTONE,GREY

97.33   1.67   1.65    97.33   1.67   1.65   CLAYSTONE,BROWNISH RED

99.00    .34    .32    99.00    .34    .32   SANDSTONE(VERY COARSE GRAINED),GREYISH WHITE

99.34   2.66   2.65    99.34   2.66   2.65   CLAYSTONE,BROWNISH RED

102.00   3.00   2.83   102.00   3.00   2.83   CLAYSTONE,REDDISH BROWN

105.00   3.00   2.99   105.00   3.00   2.99   CLAYSTONE,REDDISH BROWN

108.00   2.31   2.30   108.00   2.31   2.30   CLAYSTONE,REDDISH BROWN

110.31    .69    .68   110.31    .69    .68   MUDSTONE,GREY

111.00   3.00   2.98   111.00   3.00   2.98   MUDSTONE,GREY

114.00   3.00   2.76   114.00   3.00   2.76   MUDSTONE,GREY

117.00   3.00   2.85   117.00   3.00   2.85   CLAYSTONE,BROWNISH RED

120.00   3.00   2.80   120.00   3.00   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED

123.00   3.00    .57   123.00   3.00    .57   SANDSTONE(FINE GRAINED),BROWNISH RED

126.00   3.00   1.26   126.00   3.00   1.26   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH RED

129.00   1.00    .26   129.00   1.00    .26   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH RED

130.00   1.00    .28   130.00   1.00    .28   CLAYSTONE,CHOCOLATEBROWN

131.00   1.00    .26   131.00   1.00    .26   SANDSTONE(COARSE GRAINED),BROWNISH RED

132.00    .61    .23   132.00    .61    .23   CLAYSTONE,BROWNISH RED

132.61   2.39   2.00   132.61   2.39   2.00   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN                                 56

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

135.00   3.00   2.90   135.00   3.00   2.90   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHBROWN                          48

138.00   3.00   2.90   138.00   3.00   2.90   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH YELLOW                         51

141.00   3.00   2.95   141.00   3.00   2.95   CLAYSTONE,REDDISH BROWN

144.00   1.39   1.39   144.00   1.39   1.39   MUDSTONE,GREYISH BROWN

145.39    .82    .82   145.39    .82    .82   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN

146.21    .60    .60   146.21    .60    .60   CLAYSTONE,BROWNISH RED

146.81    .19    .18   146.81    .19    .18   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW

147.00   3.00   2.98   147.00   3.00   2.98   SANDSTONE(VERY COARSE GRAINED),BROWNISH YELLOW                           39

150.00    .50    .50   150.00    .50    .50   SANDSTONE(VERY COARSE GRAINED),BROWNISH YELLOW

150.50   2.50   2.48   150.50   2.50   2.48   CLAYSTONE,BROWNISH RED                                                   48

153.00   3.00   2.99   153.00   3.00   2.99   CLAYSTONE,BROWNISH RED                                                   50

156.00   1.62   1.60   156.00   1.62   1.60   SHALE,GREY                                                               31

157.62   1.38   1.37   157.62   1.38   1.37   SANDSTONE(VERY COARSE GRAINED),BROWNISH RED                              82

159.00    .98    .93   159.00    .98    .93   SANDSTONE(VERY COARSE GRAINED),BROWNISH RED

159.98    .41    .35   159.98    .41    .35   CLAYSTONE,BROWNISH RED

160.39   1.61   1.55   160.39   1.61   1.55   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               43

162.00   3.00   2.74   162.00   3.00   2.74   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               49

165.00   3.00   2.80   165.00   3.00   2.80   CLAYSTONE,REDDISH BROWN

168.00   3.00   2.95   168.00   3.00   2.95   CLAYSTONE,BROWNISH RED

171.00   2.00   1.10   171.00   2.00   1.10   SANDSTONE(MEDIUM GRAINED),BROWNISH RED                                   17

173.00   1.48   1.09   173.00   1.48   1.09   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

174.48   1.52   1.15   174.48   1.52   1.15   CLAYSTONE,GREY

176.00   1.00    .70   176.00   1.00    .70   CLAYSTONE,REDDISH BROWN

177.00   3.00   2.30   177.00   3.00   2.30   CLAYSTONE,BROWNISH RED

180.00   1.00    .40   180.00   1.00    .40   CLAYSTONE,REDDISH BROWN

181.00   2.00   1.00   181.00   2.00   1.00   MUDSTONE,BROWNISH GREY

183.00   3.00   2.99   183.00   3.00   2.99   CLAYSTONE,BROWNISH RED

186.00   3.00   2.84   186.00   3.00   2.84   CLAYSTONE,BROWNISH RED

189.00   3.00   2.90   189.00   3.00   2.90   CLAYSTONE,REDDISH BROWN

192.00   2.00   1.95   192.00   2.00   1.95   CLAYSTONE,REDDISH BROWN

194.00   2.00   1.90   194.00   2.00   1.90   MUDSTONE,REDDISH BROWN

196.00   3.00   1.70   196.00   3.00   1.70   CLAYSTONE,REDDISH BROWN

199.00   3.00   2.00   199.00   3.00   2.00   CLAYSTONE,BROWNISH RED

202.00   1.00    .80   202.00   1.00    .80   CLAYSTONE,REDDISH BROWN

203.00   2.00    .90   203.00   2.00    .90   CLAYSTONE,BROWNISH RED

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

205.00   3.00   1.00   205.00   3.00   1.00   CLAYSTONE,REDDISH BROWN

208.00   3.00    .33   208.00   3.00    .33   SANDSTONE(FINE GRAINED),BROWNISH RED

211.00   3.00   1.65   211.00   3.00   1.65   CLAYSTONE,REDDISH BROWN

214.00   3.00    .45   214.00   3.00    .45   SANDSTONE(FINE GRAINED),REDDISH BROWN

217.00   3.00    .41   217.00   3.00    .41   SANDSTONE(FINE GRAINED),REDDISH BROWN

220.00   3.00    .40   220.00   3.00    .40   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

223.00   3.00    .62   223.00   3.00    .62   CLAYSTONE,REDDISH BROWN

226.00   3.00    .34   226.00   3.00    .34   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

229.00   2.00    .29   229.00   2.00    .29   SANDSTONE(FINE GRAINED),REDDISH BROWN

231.00   1.00    .58   231.00   1.00    .58   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

232.00   3.00    .65   232.00   3.00    .65   SANDSTONE(FINE GRAINED),BROWNISH RED

235.00   3.00    .20   235.00   3.00    .20   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

238.00   3.00    .10   238.00   3.00    .10   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

241.00   3.00    .22   241.00   3.00    .22   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

244.00   3.00    .20   244.00   3.00    .20   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

247.00   3.00    .27   247.00   3.00    .27   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

250.00   3.00    .35   250.00   3.00    .35   CLAYSTONE,REDDISH BROWN

253.00   2.00    .47   253.00   2.00    .47   CLAYSTONE,BROWNISH RED

255.00   1.00    .56   255.00   1.00    .56   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED

256.00   3.00   1.10   256.00   3.00   1.10   SANDSTONE(MEDIUM GRAINED),BROWNISH RED

259.00   3.00   1.08   259.00   3.00   1.08   SANDSTONE(MEDIUM GRAINED),BROWNISH RED

262.00   3.00   1.15   262.00   3.00   1.15   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED

265.00   3.00   1.55   265.00   3.00   1.55   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED

268.00   3.00   1.10   268.00   3.00   1.10   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED

271.00   3.00   1.35   271.00   3.00   1.35   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED

274.00   1.50   1.05   274.00   1.50   1.05   SANDSTONE(FINE GRAINED),BROWNISH RED

275.50   1.00    .18   275.50   1.00    .18   CLAYSTONE,REDDISH BROWN

276.50    .50    .12   276.50    .50    .12   SANDSTONE(FINE GRAINED),REDDISH BROWN

277.00   3.00   2.95   277.00   3.00   2.95   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED

280.00   3.00   2.95   280.00   3.00   2.95   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED

283.00   3.00   2.80   283.00   3.00   2.80   SANDSTONE(FINE GRAINED),BROWNISH YELLOW

286.00   3.00   2.80   286.00   3.00   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW

289.00   3.00   2.80   289.00   3.00   2.80   SANDSTONE(FINE GRAINED),BROWNISH RED

292.00   3.00   2.80   292.00   3.00   2.80   SANDSTONE(FINE GRAINED),BROWNISH RED

295.00   3.00   2.80   295.00   3.00   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

298.00   3.00   2.85   298.00   3.00   2.85   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED

301.00    .59    .53   301.00    .59    .53   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED

301.59    .55    .50   301.59    .55    .50   CLAYSTONE,BROWNISH RED

302.14   1.86   1.80   302.14   1.86   1.80   SANDSTONE(FINE GRAINED),BROWNISH RED

304.00   3.00   2.85   304.00   3.00   2.85   SANDSTONE(FINE GRAINED),BROWNISH RED

307.00   3.00   2.85   307.00   3.00   2.85   SANDSTONE(FINE GRAINED),BROWNISH RED

310.00   3.00   2.90   310.00   3.00   2.90   SANDSTONE(FINE GRAINED),BROWNISH YELLOW

313.00   3.00   2.95   313.00   3.00   2.95   SANDSTONE(FINE GRAINED),BROWNISH YELLOW

316.00   3.00   2.80   316.00   3.00   2.80   SANDSTONE(FINE GRAINED),BROWNISH YELLOW

319.00   3.00   2.80   319.00   3.00   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED

322.00   3.00   2.90   322.00   3.00   2.90   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           21

325.00   3.00   2.95   325.00   3.00   2.95   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           41

328.00   3.00   2.84   328.00   3.00   2.84   SANDSTONE(FINE TO MEDIUM GRAINED),REDDISH BROWN                          33

331.00   3.00   2.90   331.00   3.00   2.90   SANDSTONE(MED. TO C. GRAINED),REDDISH BROWN                              44

334.00   1.70   1.50   334.00   1.70   1.50   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

335.70   1.30   1.00   335.70   1.30   1.00   SANDSTONE(FINE GRAINED),BROWNISH YELLOW

337.00   3.00   2.00   337.00   3.00   2.00   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW

340.00   1.60   1.30   340.00   1.60   1.30   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW

341.60   1.40   1.15   341.60   1.40   1.15   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

343.00   3.00   2.97   343.00   3.00   2.97   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW,                           43

346.00   3.00   2.85   346.00   3.00   2.85   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW,                           27

349.00   3.00   2.80   349.00   3.00   2.80   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            41

352.00   3.00   2.68   352.00   3.00   2.68   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            13

355.00   3.00   2.45   355.00   3.00   2.45   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            36

358.00   3.00   2.85   358.00   3.00   2.85   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            42

361.00   3.00   2.80   361.00   3.00   2.80   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                           8.3

364.00   3.00   2.80   364.00   3.00   2.80   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            30

367.00   3.00   2.80   367.00   3.00   2.80   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW,                           44

370.00   3.00   2.95   370.00   3.00   2.95   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               39

373.00   2.32   2.30   373.00   2.32   2.30   CLAYSTONE,REDDISH BROWN                                                  56

375.32    .68    .68   375.32    .68    .68   SANDSTONE(FINE GRAINED),BROWNISH RED                                     85

376.00   1.55   1.50   376.00   1.55   1.50   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  74

377.55   1.45   1.35   377.55   1.45   1.35   CLAYSTONE,REDDISH BROWN                                                  28

379.00   3.00   2.97   379.00   3.00   2.97   CLAYSTONE,VARIEGATE                                                      50

382.00   3.00   2.97   382.00   3.00   2.97   MUDSTONE,VARIEGAT,FRACTURE                                               20

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

385.00   1.50   1.45   385.00   1.50   1.45   MUDSTONE,GREENISH GREY                                                   62

386.50   1.50    .50   386.50   1.50    .50   MUDSTONE,GREENISH GREY                                                   29

388.00   3.00   2.40   388.00   3.00   2.40   MUDSTONE,GREY

391.00   2.30   2.20   391.00   2.30   2.20   MUDSTONE,GREY

393.30   3.00   1.45   393.30   3.00   1.45   MUDSTONE,GREY

396.30   2.70   1.40   396.30   2.70   1.40   MUDSTONE,GREY

399.00   1.00    .98   399.00   1.00    .98   MUDSTONE,GREY

400.00   3.00   2.80   400.00   3.00   2.80   MUDSTONE,DARK GREY

403.00   3.00   2.75   403.00   3.00   2.75   MUDSTONE,DARK GREY,SLICKENSIDED

406.00   2.00   1.90   406.00   2.00   1.90   MUDSTONE,DARK GREY

408.00   3.00   2.35   408.00   3.00   2.35   MUDSTONE,DARK GREY

411.00    .84    .50   411.00    .84    .50   MUDSTONE,GREY

411.84    .83    .63   411.84    .83    .63   SANDY SHALE,GREY                                                         19

412.67   1.33   1.10   412.67   1.33   1.10   MUDSTONE,GREY

414.00   3.00   2.35   414.00   3.00   2.35   MUDSTONE,VARIEGATE

417.00   3.00   2.50   417.00   3.00   2.50   MUDSTONE,VARIEGATE

420.00   3.00   2.50   420.00   3.00   2.50   MUDSTONE,GREY

423.00   3.00   2.75   423.00   3.00   2.75   MUDSTONE,GREY

426.00    .35    .20   426.00    .35    .20   MUDSTONE,GREY

426.35    .39    .25   426.35    .39    .25   SANDSTONE(MEDIUM GRAINED),GREYISH BROWN

426.74   1.16   1.02   426.74   1.16   1.02   CLAYSTONE,VARIEGATE

427.90   1.10    .96   427.90   1.10    .96   MUDSTONE,GREY,SLICKENSIDED

429.00    .91    .66   429.00    .91    .66   CLAYSTONE,GREY

429.91   2.09   1.89   429.91   2.09   1.89   SANDSTONE(FINE GRAINED),GREENISH BROWN                                   44

432.00   3.00   2.70   432.00   3.00   2.70   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           59

435.00   3.00   2.69   435.00   3.00   2.69   SANDSTONE(COARSE TO V.C.GRAINED),GREY                                    63

438.00   1.50   1.20   438.00   1.50   1.20   SANDSTONE(COARSE GRAINED),BROWNISH GREY                                  58

439.50    .79    .71   439.50    .79    .71   SANDSTONE(VERY COARSE GRAINED),BROWNISH GREY                             58

440.29    .71    .63   440.29    .71    .63   CLAYSTONE,BROWNISH GREY                                                  32

441.00   1.50   1.37   441.00   1.50   1.37   CLAYSTONE,BROWNISH GREY

442.50   1.50   1.47   442.50   1.50   1.47   CLAYSTONE,GREY

444.00   2.04   1.97   444.00   2.04   1.97   CLAYSTONE,GREY                                                          5.4

446.04    .96    .90   446.04    .96    .90   SANDSTONE(MEDIUM GRAINED),GREY                                           60

447.00    .18    .16   447.00    .18    .16   SANDSTONE(MEDIUM GRAINED),GREY                                           89

447.18   2.82   2.80   447.18   2.82   2.80   CLAYSTONE,BROWNISH GREY                                                  74

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

450.00   3.00   2.80   450.00   3.00   2.80   CLAYSTONE,GREY                                                           67

453.00    .91    .75   453.00    .91    .75   SHALE,GREY                                                               41

453.91    .43    .26   453.91    .43    .26   SANDSTONE(FINE GRAINED),GREY                                             42

454.34   1.66   1.50   454.34   1.66   1.50   SANDY SHALE,GREY                                                         63

456.00    .90    .83   456.00    .90    .83   SANDY SHALE,GREY                                                         80

456.90   1.66   1.59   456.90   1.66   1.59   SANDSTONE(MED. TO C. GRAINED),GREY                                       72

458.56    .44    .37   458.56    .44    .37   CLAYSTONE,GREY

459.00   1.50   1.15   459.00   1.50   1.15   CLAYSTONE,GREY

460.50   1.50   1.24   460.50   1.50   1.24   CLAYSTONE,GREY

462.00   2.60   2.60   462.00   2.60   2.60   CLAYSTONE,GREY                                                           14

464.60    .40    .38   464.60    .40    .38   CLAYSTONE,BROWN

465.00   1.10   1.10   465.00   1.10   1.10   CLAYSTONE,BROWN                                                          16

466.10   1.32   1.32   466.10   1.32   1.32   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              77

467.42    .58    .58   467.42    .58    .58   CLAYSTONE,BROWN                                                          72

468.00   1.00   1.00   468.00   1.00   1.00   CLAYSTONE,BROWN                                                          76

469.00   2.00   1.80   469.00   2.00   1.80   CLAYSTONE,BROWNISH GREY                                                  39

471.00   2.55   2.54   471.00   2.55   2.54   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              55

473.55    .45    .43   473.55    .45    .43   SHALE,BROWNISH GREY                                                      33

474.00   1.00    .97   474.00   1.00    .97   SHALE,BROWNISH GREY                                                      21

475.00   2.00   1.99   475.00   2.00   1.99   SHALE,GREYISH BROWN                                                      63

477.00   3.00   2.99   477.00   3.00   2.99   SHALE,BROWNISH GREY                                                      75

480.00   3.00   2.99   480.00   3.00   2.99   SHALE,BROWNISH GREY                                                      67

483.00   3.00   2.99   483.00   3.00   2.99   SHALE,GREY                                                               38

486.00   3.00   2.99   486.00   3.00   2.99   SHALE,GREY                                                               49

489.00   2.00   1.99   489.00   2.00   1.99   SHALE,GREY                                                               52

491.00    .39    .39   491.00    .39    .39   SHALE,GREY                                                               85

491.39    .41    .40   491.39    .41    .40   SANDSTONE(FINE GRAINED),GREY                                             59

491.80    .20    .18   491.80    .20    .18   SHALE,GREY

492.00   3.00   2.95   492.00   3.00   2.95   SHALE,DARK GREY,SLICKENSIDED                                             27

495.00    .73    .70   495.00    .73    .70   SANDY SHALE,GREY                                                         63

495.73   2.27   2.26   495.73   2.27   2.26   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   74

498.00    .43    .42   498.00    .43    .42   SHALE,GREY

498.43   1.07   1.06   498.43   1.07   1.06   CLAYSTONE,DARK GREY

499.50   1.50   1.45   499.50   1.50   1.45   CLAYSTONE,DARK GREY

501.00   2.20   2.10   501.00   2.20   2.10   CLAYSTONE,DARK GREY,FRACTURE

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

503.20    .80    .79   503.20    .80    .79   SANDSTONE(VERY COARSE GRAINED),GREY                                      69

504.00   3.00   2.98   504.00   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),GREY                                       87

507.00    .59    .59   507.00    .59    .59   SANDSTONE(COARSE GRAINED),GREY                                           78

507.59   2.41   2.38   507.59   2.41   2.38   SHALE,GREY                                                               15

510.00   1.50   1.49   510.00   1.50   1.49   SHALE,BROWNISH GREY                                                      69

511.50   1.50   1.49   511.50   1.50   1.49   CLAYSTONE,BROWNISH GREY

513.00   2.00   1.45   513.00   2.00   1.45   CLAYSTONE,GREYISH BROWN

515.00   3.00   2.90   515.00   3.00   2.90   CLAYSTONE,DARK GREY

518.00   3.00   2.99   518.00   3.00   2.99   CLAYSTONE,DARK GREY

521.00   3.00   2.99   521.00   3.00   2.99   CLAYSTONE,GREYISH BROWN

524.00   2.50   2.50   524.00   2.50   2.50   CLAYSTONE,GREYISH BROWN

526.50   1.50   1.48   526.50   1.50   1.48   CLAYSTONE,GREYISH BROWN

528.00   1.75   1.72   528.00   1.75   1.72   CLAYSTONE,GREYISH BROWN                                                  40

529.75    .22    .20   529.75    .22    .20   SANDSTONE(COARSE GRAINED),GREY                                           91

529.97   1.03   1.00   529.97   1.03   1.00   CLAYSTONE,GREY                                                           43

531.00   3.00   2.98   531.00   3.00   2.98   CLAYSTONE,DARK GREY                                                      25

534.00   1.00    .99   534.00   1.00    .99   CLAYSTONE,DARK GREY

535.00   3.00   2.99   535.00   3.00   2.99   CLAYSTONE,DARK GREY

538.00   3.00   2.99   538.00   3.00   2.99   CLAYSTONE,DARK GREY

541.00   3.00   2.99   541.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

544.00   1.00    .90   544.00   1.00    .90   CLAYSTONE,BROWNISH GREY

545.00   1.00    .98   545.00   1.00    .98   CLAYSTONE,BROWNISH GREY

546.00   3.00   2.99   546.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

549.00   3.00   2.99   549.00   3.00   2.99   CLAYSTONE,GREYISH BROWN

552.00   3.00   2.99   552.00   3.00   2.99   CLAYSTONE,GREYISH BROWN

555.00   3.00   2.98   555.00   3.00   2.98   CLAYSTONE,GREYISH BROWN

558.00   3.00   2.99   558.00   3.00   2.99   CLAYSTONE,GREYISH BROWN

561.00   2.00   2.00   561.00   2.00   2.00   CLAYSTONE,GREYISH BROWN                                                  31

563.00   3.00   2.99   563.00   3.00   2.99   CLAYSTONE,GREYISH BROWN

566.00   3.00   2.99   566.00   3.00   2.99   CLAYSTONE,GREYISH BROWN

569.00   3.00   2.99   569.00   3.00   2.99   CLAYSTONE,GREYISH BROWN

572.00   1.00   1.00   572.00   1.00   1.00   CLAYSTONE,GREYISH BROWN

573.00   3.00   2.99   573.00   3.00   2.99   CLAYSTONE,GREYISH BROWN                                                 8.3

576.00    .31    .25   576.00    .31    .25   CLAYSTONE,GREY

576.31   1.04   1.00   576.31   1.04   1.00   SANDSTONE(MED. TO C. GRAINED),GREY,FRACTURE                              45

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

577.35    .26    .20   577.35    .26    .20   MUDSTONE,GREY

577.61   1.39   1.33   577.61   1.39   1.33   CLAYSTONE,GREYISH BROWN

579.00   2.00   1.85   579.00   2.00   1.85   CLAYSTONE,GREYISH BROWN

581.00   2.00   1.90   581.00   2.00   1.90   MUDSTONE,GREYISH BROWN

583.00   2.00   1.85   583.00   2.00   1.85   MUDSTONE,GREYISH BROWN

585.00   1.50   1.42   585.00   1.50   1.42   CLAYSTONE,GREYISH BROWN                                                  69

586.50   1.50   1.37   586.50   1.50   1.37   CLAYSTONE,BROWNISH GREY

588.00   1.25   1.16   588.00   1.25   1.16   CLAYSTONE,BROWNISH GREY

589.25   1.75   1.70   589.25   1.75   1.70   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              78

591.00   1.02    .97   591.00   1.02    .97   SANDSTONE(COARSE TO V.C.GRAINED),GREYISH WHITE                           70

592.02    .98    .91   592.02    .98    .91   CLAYSTONE,BROWNISH GREY,FRACTURE

593.00   1.00    .91   593.00   1.00    .91   CLAYSTONE,BROWNISH GREY,FRACTURE

594.00   1.50   1.45   594.00   1.50   1.45   CLAYSTONE,GREYISH BROWN

595.50   1.50   1.49   595.50   1.50   1.49   CLAYSTONE,BROWNISH GREY                                                  13

597.00   2.70   2.64   597.00   2.70   2.64   CLAYSTONE,BROWNISH GREY,FRACTURE

599.70    .30    .27   599.70    .30    .27   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    57

600.00   3.00   2.95   600.00   3.00   2.95   CLAYSTONE,VARIEGATE                                                      63

603.00   3.00   2.98   603.00   3.00   2.98   CLAYSTONE,VARIEGATE                                                      25

606.00   1.50   1.49   606.00   1.50   1.49   CLAYSTONE,VARIEGATE

607.50   1.50   1.50   607.50   1.50   1.50   MUDSTONE,VARIEGAT,FRACTURE

609.00   3.00   2.99   609.00   3.00   2.99   CLAYSTONE,VARIEGATE

612.00    .84    .84   612.00    .84    .84   SANDSTONE(MED. TO C. GRAINED),GREY                                       18

612.84   1.11   1.10   612.84   1.11   1.10   CLAYSTONE,GREY

613.95    .25    .25   613.95    .25    .25   SANDSTONE(FINE GRAINED),GREYISH BROWN

614.20    .30    .30   614.20    .30    .30   CLAYSTONE,BROWN

614.50   3.00   2.98   614.50   3.00   2.98   CLAYSTONE,BROWNISH GREY

617.50   1.70   1.70   617.50   1.70   1.70   CLAYSTONE,BROWNISH GREY                                                  29

619.20    .44    .43   619.20    .44    .43   SANDSTONE(COARSE GRAINED),BROWNISH GREY

619.64    .60    .60   619.64    .60    .60   CLAYSTONE,BROWN

620.24    .26    .25   620.24    .26    .25   SANDSTONE(COARSE GRAINED),BROWNISH GREY                                  77

620.50    .75    .75   620.50    .75    .75   CLAYSTONE,BROWNISH GREY

621.25    .80    .80   621.25    .80    .80   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          33

622.05    .20    .20   622.05    .20    .20   SHALE,BROWNISH GREY

622.25    .42    .42   622.25    .42    .42   CLAYSTONE,DARK GREY                                                      60

622.67    .83    .82   622.67    .83    .82   SANDSTONE(COARSE GRAINED),DARK BROWN                                     66

S/W DEVELOPED BY :

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

623.50    .20    .18   623.50    .20    .18   SANDSTONE(COARSE GRAINED),GREY                                           65

623.70    .24    .24   623.70    .24    .24   MUDSTONE,GREY

623.94   2.56   2.56   623.94   2.56   2.56   CLAYSTONE,BROWNISH GREY                                                  18

626.50   1.92   1.74   626.50   1.92   1.74   CLAYSTONE,VARIEGATE                                                      20

628.42   1.08    .90   628.42   1.08    .90   SANDSTONE(MED. TO C. GRAINED),GREY                                       44

629.50    .20    .20   629.50    .20    .20   SANDSTONE(COARSE GRAINED),GREY

629.70    .58    .58   629.70    .58    .58   CLAYSTONE,GREY

630.28    .72    .70   630.28    .72    .70   SANDSTONE(MED. TO C. GRAINED),GREY                                       79

631.00   1.00   1.00   631.00   1.00   1.00   CLAYSTONE,BROWNISH GREY                                                  52

632.00   2.30   2.30   632.00   2.30   2.30   CLAYSTONE,VARIEGATE                                                      13

634.30    .70    .70   634.30    .70    .70   SANDSTONE(COARSE GRAINED),GREYISH WHITE                                  87

635.00   1.12   1.12   635.00   1.12   1.12   CLAYSTONE,DARK BROWN                                                     35

636.12   1.35   1.35   636.12   1.35   1.35   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   70

637.47    .53    .50   637.47    .53    .50   CLAYSTONE,BROWN                                                          26

638.00    .72    .72   638.00    .72    .72   CLAYSTONE,BROWNISH GREY

638.72    .20    .20   638.72    .20    .20   SANDSTONE(MEDIUM GRAINED),GREY                                           80

638.92    .27    .27   638.92    .27    .27   CLAYSTONE,BROWNISH GREY

639.19    .36    .36   639.19    .36    .36   SANDSTONE(MEDIUM GRAINED),GREY                                           44

639.55    .18    .18   639.55    .18    .18   SANDY SHALE,GREY

639.73   1.27   1.25   639.73   1.27   1.25   CLAYSTONE,DARK BROWN

641.00   3.00   2.95   641.00   3.00   2.95   CLAYSTONE,BROWNISH GREY                                                  13

644.00   1.38   1.37   644.00   1.38   1.37   SANDSTONE(FINE TO COARSE GRAINED),GREY                                   20

645.38   1.62   1.60   645.38   1.62   1.60   CLAYSTONE,GREY

647.00   1.50   1.50   647.00   1.50   1.50   CLAYSTONE,GREY

648.50    .56    .55   648.50    .56    .55   SANDY SHALE,GREY                                                         59

649.06    .94    .92   649.06    .94    .92   CLAYSTONE,BROWN

650.00   1.15   1.15   650.00   1.15   1.15   MUDSTONE,GREYISH BROWN,FRACTURE

651.15    .67    .67   651.15    .67    .67   SANDSTONE(FINE GRAINED),GREY,FRACTURE

651.82    .39    .38   651.82    .39    .38   CLAYSTONE,GREYISH BROWN

652.21    .79    .78   652.21    .79    .78   SANDY SHALE,GREY

653.00    .81    .79   653.00    .81    .79   SANDY SHALE,GREY

653.81    .29    .26   653.81    .29    .26   SANDSTONE(COARSE GRAINED),GREY                                           90

654.10   1.12   1.10   654.10   1.12   1.10   MUDSTONE,GREY

655.22    .78    .76   655.22    .78    .76   CLAYSTONE,GREYISH BROWN

656.00   3.00   2.98   656.00   3.00   2.98   CLAYSTONE,BROWNISH GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

659.00   3.00   2.98   659.00   3.00   2.98   CLAYSTONE,BROWN                                                          17

662.00   1.00   1.00   662.00   1.00   1.00   CLAYSTONE,BROWN

663.00   3.00   2.99   663.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                  54

666.00   3.00   2.99   666.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                  48

669.00   3.00   2.98   669.00   3.00   2.98   CLAYSTONE,BROWN                                                          68

672.00   3.00   2.99   672.00   3.00   2.99   CLAYSTONE,BROWN                                                          51

675.00   3.00   2.99   675.00   3.00   2.99   CLAYSTONE,BROWN                                                          74

678.00   3.00   2.99   678.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 39

681.00    .74    .73   681.00    .74    .73   SANDSTONE(MED. TO C. GRAINED),CHOCOLATEBROWN                             46

681.74   2.26   2.26   681.74   2.26   2.26   CLAYSTONE,CHOCOLATEBROWN                                                 54

684.00   1.00   1.00   684.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 58

685.00   2.00   1.99   685.00   2.00   1.99   CLAYSTONE,CHOCOLATEBROWN                                                 30

687.00   3.00   2.98   687.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 16

690.00   1.00   1.00   690.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN

691.00   2.00   1.97   691.00   2.00   1.97   CLAYSTONE,CHOCOLATEBROWN                                                 27

693.00   2.12   2.12   693.00   2.12   2.12   CLAYSTONE,CHOCOLATEBROWN                                                 41

695.12    .50    .49   695.12    .50    .49   SANDY SHALE,GREY                                                         26

695.62    .38    .38   695.62    .38    .38   CLAYSTONE,CHOCOLATEBROWN                                                 79

696.00   1.90   1.90   696.00   1.90   1.90   CLAYSTONE,CHOCOLATEBROWN                                                 18

697.90    .65    .65   697.90    .65    .65   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  88

698.55    .45    .45   698.55    .45    .45   CLAYSTONE,CHOCOLATEBROWN

699.00   3.00   2.99   699.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                  6

702.00   3.00   2.99   702.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 43

705.00   2.00   1.99   705.00   2.00   1.99   CLAYSTONE,CHOCOLATEBROWN                                                 35

707.00   1.00   1.00   707.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 54

708.00   3.00   2.99   708.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 16

711.00   1.35   1.35   711.00   1.35   1.35   CLAYSTONE,CHOCOLATEBROWN                                                 46

712.35   1.65   1.65   712.35   1.65   1.65   SANDSTONE(MED. TO C. GRAINED),CHOCOLATEBROWN

714.00   3.00   2.99   714.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 21

717.00   3.00   2.99   717.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN

720.00   3.00   2.99   720.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 32

723.00   3.00   2.99   723.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 55

726.00   2.00   2.00   726.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN                                                 56

728.00    .80    .80   728.00    .80    .80   CLAYSTONE,CHOCOLATEBROWN                                                 19

728.80    .20    .20   728.80    .20    .20   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  65

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

729.00    .48    .48   729.00    .48    .48   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  77

729.48   2.52   2.52   729.48   2.52   2.52   CLAYSTONE,CHOCOLATEBROWN                                                 13

732.00    .34    .33   732.00    .34    .33   CLAYSTONE,CHOCOLATEBROWN

732.34   1.40   1.40   732.34   1.40   1.40   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  59

733.74   1.26   1.25   733.74   1.26   1.25   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

735.00   3.00   2.98   735.00   3.00   2.98   CLAYSTONE,VARIEGATE                                                      42

738.00   3.00   2.99   738.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                       6.7

741.00   1.00   1.00   741.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

742.00   2.00   1.97   742.00   2.00   1.97   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

744.00   3.00   2.99   744.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

747.00    .98    .98   747.00    .98    .98   CLAYSTONE,CHOCOLATEBROWN                                                 28

747.98    .24    .24   747.98    .24    .24   SANDSTONE(FINE GRAINED),GREYISH WHITE,FRACTURE                          100

748.22    .78    .77   748.22    .78    .77   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

749.00   1.00   1.00   749.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 27

750.00   3.00   2.97   750.00   3.00   2.97   CLAYSTONE,CHOCOLATEBROWN                                                 47

753.00   3.00   2.99   753.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 39

756.00    .59    .58   756.00    .59    .58   CLAYSTONE,CHOCOLATEBROWN

756.59    .36    .36   756.59    .36    .36   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              42

756.95   2.05   2.05   756.95   2.05   2.05   CLAYSTONE,CHOCOLATEBROWN                                                 56

759.00   3.00   2.99   759.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 39

762.00   3.00   2.99   762.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 39

765.00   2.45   2.45   765.00   2.45   2.45   CLAYSTONE,CHOCOLATEBROWN                                                 14

767.45    .30    .30   767.45    .30    .30   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  47

767.75    .25    .25   767.75    .25    .25   CLAYSTONE,CHOCOLATEBROWN

768.00    .70    .70   768.00    .70    .70   CLAYSTONE,CHOCOLATEBROWN                                                 33

768.70   1.10   1.10   768.70   1.10   1.10   SANDSTONE(FINE TO MEDIUM GRAINED),CHOCOLATEBROWN                         40

769.80   1.20   1.20   769.80   1.20   1.20   CLAYSTONE,CHOCOLATEBROWN

771.00   3.00   2.98   771.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 20

774.00   3.00   2.99   774.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 44

777.00   3.00   2.99   777.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 22

780.00   2.50   2.50   780.00   2.50   2.50   CLAYSTONE,CHOCOLATEBROWN                                                 30

782.50   3.00   2.99   782.50   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 29

785.50   3.00   2.99   785.50   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 41

788.50   3.00   2.99   788.50   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 16

791.50   1.00   1.00   791.50   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 38

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

792.50   2.00   2.00   792.50   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN                                                 32

794.50   2.50   2.45   794.50   2.50   2.45   CLAYSTONE,CHOCOLATEBROWN                                                 38

797.00   3.00   2.99   797.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 19

800.00   1.00   1.00   800.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 11

801.00   1.00   1.00   801.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN

802.00   1.50   1.50   802.00   1.50   1.50   CLAYSTONE,CHOCOLATEBROWN

803.50   3.00   2.99   803.50   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 25

806.50   1.00   1.00   806.50   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 74

807.50   2.00   2.00   807.50   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN                                                 11

809.50   3.00   2.99   809.50   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 12

812.50    .70    .69   812.50    .70    .69   CLAYSTONE,CHOCOLATEBROWN

813.20   1.30   1.30   813.20   1.30   1.30   SANDSTONE(MEDIUM GRAINED),GREY                                           20

814.50   1.00    .99   814.50   1.00    .99   SANDSTONE(MEDIUM GRAINED),GREY                                           20

815.50    .10    .09   815.50    .10    .09   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

815.60   1.60   1.60   815.60   1.60   1.60   CLAYSTONE,CHOCOLATEBROWN

817.20    .72    .68   817.20    .72    .68   CLAYSTONE,CHOCOLATEBROWN

817.92    .58    .55   817.92    .58    .55   SANDSTONE(MEDIUM GRAINED),GREY                                           71

818.50   1.55   1.53   818.50   1.55   1.53   SANDSTONE(MED. TO C. GRAINED),GREY                                       74

820.05    .77    .73   820.05    .77    .73   CLAYSTONE,CHOCOLATEBROWN

820.82    .68    .65   820.82    .68    .65   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          81

821.50   3.00   2.99   821.50   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              75

824.50   3.00   2.97   824.50   3.00   2.97   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              82

827.50   3.00   2.90   827.50   3.00   2.90   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                              69

830.50    .63    .55   830.50    .63    .55   SANDSTONE(MED. TO C. GRAINED),GREENISH GREY                              78

831.13    .37    .28   831.13    .37    .28   CLAYSTONE,DARK BROWN

831.50    .43    .40   831.50    .43    .40   MUDSTONE,DARK GREY

***************************************************************************************************************************

BARAKAR

831.93    .45    .43   831.93    .45    .43   SANDY SHALE,YELLOWISHGREEN

832.38   1.12   1.10   832.38   1.12   1.10   SANDSTONE(FINE GRAINED),BROWNISH YELLOW                                  56

833.50   3.00   2.98   833.50   3.00   2.98   SANDSTONE(FINE TO COARSE GRAINED),BROWNISH GREY                          81

836.50   3.00   2.98   836.50   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),GREY                                       61

839.50   3.00   2.99   839.50   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              81

842.50   1.30   1.30   842.50   1.30   1.30   SANDY SHALE,GREY                                                         82

843.80    .26    .26   843.80    .26    .26   ALTERNATING SHALE AND SST,GREY                                           62

844.06   1.44   1.44   844.06   1.44   1.44   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  98

845.50   3.00   2.99   845.50   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              92

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

848.50   3.00   2.99   848.50   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              71

851.50    .83    .83   851.50    .83    .83   SANDSTONE(MED. TO C. GRAINED),WHITE                                      89

852.33    .03    .03   852.33    .03    .03   COAL                                                            L

852.36    .44    .44   852.36    .44    .44   SANDSTONE(MEDIUM GRAINED),WHITE                                          77

852.80    .90    .90   852.80    .90    .90   SANDY SHALE,GREENISH GREY,FRACTURE                                       40

853.70    .80    .80   853.70    .80    .80   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          90

854.50    .50    .43   854.50    .50    .43   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  44

855.00   3.00   2.60   855.00   3.00   2.60   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              37

858.00   1.97   1.91   858.00   1.97   1.91   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                             5.1

859.97    .33    .28   859.97    .33    .28   SHALY COAL                                                      L1

860.30    .27    .22   860.30    .27    .22   SHALY COAL                                                      L1

860.57    .43    .38   860.57    .43    .38   SHALE,DARK GREY

861.00    .73    .62   861.00    .73    .62   SHALE,GREY,FRACTURE

861.73    .77    .66   861.73    .77    .66   SANDY SHALE,GREY,FRACTURE

862.50    .54    .54   862.50    .54    .54   SANDY SHALE,GREY                                                         19

863.04    .27    .27   863.04    .27    .27   SHALE,GREY

863.31    .69    .68   863.31    .69    .68   SANDY SHALE,GREY

864.00    .26    .20   864.00    .26    .20   SANDY SHALE,GREY

864.26    .26    .19   864.26    .26    .19   SANDSTONE(MEDIUM GRAINED),GREY

864.52   2.48   2.42   864.52   2.48   2.42   SHALE,GREY                                                              9.3

867.00    .47    .42   867.00    .47    .42   SHALE,GREY

867.47   2.53   2.48   867.47   2.53   2.48   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          64

870.00    .78    .78   870.00    .78    .78   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          74

870.78    .22    .21   870.78    .22    .21   SHALE,GREY                                                               55

871.00   2.00   2.00   871.00   2.00   2.00   SANDY SHALE,GREY                                                         46

873.00   3.00   2.99   873.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         92

876.00   1.50   1.49   876.00   1.50   1.49   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          60

877.50   1.50   1.50   877.50   1.50   1.50   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          49

879.00   3.00   2.99   879.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          77

882.00   1.00    .94   882.00   1.00    .94   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          80

883.00   2.00   2.00   883.00   2.00   2.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          34

885.00   3.00   2.99   885.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              76

888.00   2.00   1.99   888.00   2.00   1.99   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  84

890.00   1.00   1.00   890.00   1.00   1.00   SANDSTONE(FINE GRAINED),WHITE                                            96

891.00   2.12   2.12   891.00   2.12   2.12   SANDSTONE(MED. TO C. GRAINED)                                            76

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

893.12    .25    .25   893.12    .25    .25   CARB SHALE LOW                                                  L

893.37    .15    .15   893.37    .15    .15   SHALE,GREY

893.52    .13    .13   893.52    .13    .13   ALTERNATING SHALE AND SST,GREY                                          100

893.65    .35    .35   893.65    .35    .35   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    89

894.00   3.00   2.92   894.00   3.00   2.92   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  88

897.00   1.00    .98   897.00   1.00    .98   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              60

898.00   2.00   1.90   898.00   2.00   1.90   SANDSTONE(FINE TO COARSE GRAINED),GREYISH WHITE                         100

900.00   1.94   1.90   900.00   1.94   1.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              63

901.94    .06    .05   901.94    .06    .05   COAL                                                            L

902.00    .90    .86   902.00    .90    .86   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  89

902.90   2.90   2.90   902.90   2.90   2.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              98

905.80   2.90   2.90   905.80   2.90   2.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              94

908.70   1.92   1.85   908.70   1.92   1.85   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              88

910.62    .98    .90   910.62    .98    .90   COAL                                                            IIA

911.60    .12    .10   911.60    .12    .10   COAL                                                            IIA

911.72   1.59   1.55   911.72   1.59   1.55   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    64

913.31   1.19   1.15   913.31   1.19   1.15   INTERCALATIONS,GREY                                                      79

914.50    .20    .20   914.50    .20    .20   SHALE,GREY

914.70    .26    .26   914.70    .26    .26   CARB SHALE LOW

914.96    .27    .27   914.96    .27    .27   CARB SHALE HIGH

915.23    .27    .27   915.23    .27    .27   CARB SHALE HIGH

915.50    .27    .27   915.50    .27    .27   SHALY COAL                                                      IIB

915.77    .43    .42   915.77    .43    .42   SHALY COAL                                                      IIB

916.20    .30    .30   916.20    .30    .30   SHALY COAL                                                      IIB

916.50    .30    .30   916.50    .30    .30   COAL                                                            IIB

916.80    .30    .30   916.80    .30    .30   COAL                                                            IIB

917.10    .30    .30   917.10    .30    .30   SHALY COAL                                                      IIB

917.40    .60    .60   917.40    .60    .60   COAL                                                            IIB

918.00    .75    .75   918.00    .75    .75   COAL                                                            IIB

918.75    .75    .75   918.75    .75    .75   COAL                                                            IIB

919.50    .80    .80   919.50    .80    .80   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          98

920.30   2.90   2.90   920.30   2.90   2.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              73

923.20   2.90   2.89   923.20   2.90   2.89   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              63

926.10   2.90   2.90   926.10   2.90   2.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              63

929.00    .08    .08   929.00    .08    .08   SANDSTONE(FINE GRAINED),GREYISH WHITE

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

929.08    .28    .28   929.08    .28    .28   SHALY COAL                                                      I

929.36    .28    .28   929.36    .28    .28   SHALY COAL                                                      I

929.64   1.50   1.50   929.64   1.50   1.50   INTERCALATIONS,GREY                                                      67

931.14    .36    .36   931.14    .36    .36   SHALE,GREY

931.50    .31    .30   931.50    .31    .30   SHALE,GREY                                                               42

931.81    .42    .41   931.81    .42    .41   INTERCALATIONS,GREY                                                      90

932.23   2.27   2.26   932.23   2.27   2.26   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          96

BOREHOLE CLOSED AT   934.50 M DEPTH

\

---------------------------------------------------------------------------

BOREHOLE NO    :CMWWB04              LATITUDE    : 2203174.803M                   DATE OF COMMENCEMENT: 30.11.2023

REDUCED LEVEL  : 261.627M            DEPARTURE   : 268127.9779M                   DATE OF COMPLETION: 19.01.2024

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

.00   3.00   2.10      .00   3.00   2.10   BASALT,GREENISH BROWN

3.00   3.00   2.99     3.00   3.00   2.99   BASALT,GREY                                                              74

6.00   3.00   2.99     6.00   3.00   2.99   BASALT,GREY                                                             100

9.00   3.00   2.99     9.00   3.00   2.99   BASALT,GREY                                                              92

12.00   3.00   2.99    12.00   3.00   2.99   BASALT,GREY                                                              95

15.00   3.00   2.99    15.00   3.00   2.99   BASALT,GREY                                                              92

18.00   3.00   2.98    18.00   3.00   2.98   BASALT,GREY,SLICKENSIDED                                                 51

21.00   2.00   2.00    21.00   2.00   2.00   BASALT,GREY,FRACTURE                                                     32

23.00   1.00    .98    23.00   1.00    .98   BASALT,GREY,FRACTURE                                                     52

24.00   3.00   2.98    24.00   3.00   2.98   BASALT,GREY,FRACTURE                                                     67

27.00   3.00   2.97    27.00   3.00   2.97   BASALT,GREY,FRACTURE                                                     38

30.00   3.00   2.99    30.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     74

33.00   1.40   1.39    33.00   1.40   1.39   BASALT,GREY,FRACTURE                                                     66

34.40   1.60   1.58    34.40   1.60   1.58   BASALT,GREY,FRACTURE                                                     84

36.00   3.00   2.99    36.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     92

39.00   3.00   2.99    39.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     85

42.00   1.00    .99    42.00   1.00    .99   BASALT,GREY,FRACTURE                                                     60

43.00   3.00   2.99    43.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     79

46.00   3.00   2.99    46.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     74

49.00   3.00   2.99    49.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     51

52.00   2.00   1.99    52.00   2.00   1.99   BASALT,GREY,FRACTURE                                                     88

54.00   3.00   2.99    54.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     64

57.00   3.00   2.99    57.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     71

60.00   3.00   2.99    60.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     94

63.00   3.00   2.99    63.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     92

66.00   1.09   1.09    66.00   1.09   1.09   BASALT,GREY,FRACTURE                                                     71

67.09   1.91   1.89    67.09   1.91   1.89   BASALT,CHOCOLATEBROWN,FRACTURE

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

69.00    .27    .25    69.00    .27    .25   BASALT,CHOCOLATEBROWN,FRACTURE

69.27   2.73   2.73    69.27   2.73   2.73   BASALT,GREY,FRACTURE                                                     57

72.00   3.00   2.98    72.00   3.00   2.98   BASALT,GREY,FRACTURE                                                     61

75.00   3.00   2.97    75.00   3.00   2.97   BASALT,GREY,FRACTURE                                                     35

78.00   3.00   2.80    78.00   3.00   2.80   BASALT,GREY,FRACTURE                                                     10

81.00   3.00   1.60    81.00   3.00   1.60   BASALT,GREY                                                             9.7

84.00   1.30   1.20    84.00   1.30   1.20   BASALT,GREY                                                              43

85.30   1.70    .60    85.30   1.70    .60   MUDSTONE,GREY

87.00   3.00    .60    87.00   3.00    .60   MUDSTONE,GREY

90.00   3.00    .50    90.00   3.00    .50   MUDSTONE,GREY

93.00   3.00   2.10    93.00   3.00   2.10   BASALT,DARK GREY                                                         45

96.00   3.00   2.99    96.00   3.00   2.99   BASALT,GREY                                                              85

99.00   3.00   2.99    99.00   3.00   2.99   BASALT,GREY                                                              84

102.00   3.00   2.99   102.00   3.00   2.99   BASALT,GREY                                                              78

TRAP

*****************************************************************************************************************************

MOTUR

105.00   3.00    .50   105.00   3.00    .50   CLAYSTONE,CHOCOLATEBROWN

108.00   3.00    .45   108.00   3.00    .45   CLAYSTONE,CHOCOLATEBROWN

111.00   3.00    .35   111.00   3.00    .35   CLAYSTONE,CHOCOLATEBROWN

114.00   3.00    .60   114.00   3.00    .60   CLAYSTONE,VARIEGATE

117.00   3.00    .45   117.00   3.00    .45   CLAYSTONE,VARIEGATE

120.00   2.00    .45   120.00   2.00    .45   CLAYSTONE,VARIEGATE

122.00   1.00    .40   122.00   1.00    .40   CLAYSTONE,CHOCOLATEBROWN

123.00   3.00    .55   123.00   3.00    .55   CLAYSTONE,CHOCOLATEBROWN

126.00   1.52    .15   126.00   1.52    .15   CLAYSTONE,CHOCOLATEBROWN

127.52   1.48    .27   127.52   1.48    .27   SANDSTONE(MEDIUM GRAINED),GREY

129.00   3.00    .55   129.00   3.00    .55   CLAYSTONE,VARIEGATE

132.00   3.00    .70   132.00   3.00    .70   CLAYSTONE,VARIEGATE

135.00   1.00    .15   135.00   1.00    .15   CLAYSTONE,BROWNISH RED

136.00   2.00    .13   136.00   2.00    .13   CLAYSTONE,BROWNISH RED

138.00   1.30    .31   138.00   1.30    .31   CLAYSTONE,BROWNISH RED                                                   12

139.30   1.20    .25   139.30   1.20    .25   SANDSTONE(MEDIUM GRAINED),BROWNISH RED

140.50   1.50    .28   140.50   1.50    .28   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED

142.00    .70    .35   142.00    .70    .35   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN

142.70   1.30    .35   142.70   1.30    .35   CLAYSTONE,BROWNISH RED

144.00   2.00    .25   144.00   2.00    .25   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW

146.00   3.00   1.15   146.00   3.00   1.15   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            16

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

149.00   3.00    .70   149.00   3.00    .70   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH BROWN                         20

152.00    .47    .45   152.00    .47    .45   CLAYSTONE,BROWNISH RED

152.47    .53    .50   152.47    .53    .50   CLAYSTONE,BROWNISH RED

153.00   2.00   1.98   153.00   2.00   1.98   CLAYSTONE,BROWNISH BROWN                                                 38

155.00   1.00    .98   155.00   1.00    .98   CLAYSTONE,BROWNISH BROWN                                                 86

156.00    .58    .50   156.00    .58    .50   SANDSTONE(MEDIUM GRAINED),BROWNISH BROWN

156.58   1.42   1.34   156.58   1.42   1.34   CLAYSTONE,BROWNISH BROWN                                                 13

158.00   1.45   1.30   158.00   1.45   1.30   CLAYSTONE,BROWNISH BROWN,SLICKENSIDED                                    26

159.45   1.55   1.50   159.45   1.55   1.50   SANDSTONE(MEDIUM GRAINED),BROWNISH BROWN                                 38

161.00   3.00   2.75   161.00   3.00   2.75   SANDSTONE(MEDIUM GRAINED),BROWNISH BROWN,SLICKENSIDED                    47

164.00   1.50    .80   164.00   1.50    .80   SANDSTONE(MED. TO C. GRAINED),BROWNISH BROWN                             22

165.50   3.00   2.00   165.50   3.00   2.00   SANDSTONE(MEDIUM GRAINED),BROWNISH BROWN                                 61

168.50   1.50   1.00   168.50   1.50   1.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH BROWN                             43

170.00   3.00   2.60   170.00   3.00   2.60   SANDSTONE(MEDIUM GRAINED),BROWNISH RED                                   75

173.00   3.00   2.50   173.00   3.00   2.50   SANDSTONE(MEDIUM GRAINED),BROWNISH RED                                   67

176.00   3.00   2.60   176.00   3.00   2.60   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED,                          76

179.00   3.00   2.50   179.00   3.00   2.50   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        66

182.00   3.00   2.00   182.00   3.00   2.00   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        20

185.00   3.00   2.99   185.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           87

188.00   3.00   2.99   188.00   3.00   2.99   SANDSTONE(MEDIUM GRAINED),BROWNISH BROWN                                 39

191.00   3.00   2.99   191.00   3.00   2.99   SANDSTONE(MEDIUM GRAINED),BROWNISH BROWN                                 70

194.00   1.00    .97   194.00   1.00    .97   SANDSTONE(MEDIUM GRAINED),BROWNISH RED                                   51

195.00   2.00   1.99   195.00   2.00   1.99   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               68

197.00   3.00   2.99   197.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               62

200.00   3.00   2.99   200.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED,FRACTURE                      47

203.00   3.00   2.98   203.00   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               94

206.00   3.00   2.98   206.00   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               91

209.00   3.00   2.98   209.00   3.00   2.98   SANDSTONE(MEDIUM GRAINED),BROWNISH RED                                   64

212.00   3.00   2.99   212.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               84

215.00   3.00   2.80   215.00   3.00   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH RED                           74

218.00   3.00   2.95   218.00   3.00   2.95   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               83

221.00   3.00   2.95   221.00   3.00   2.95   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               75

224.00   3.00   2.97   224.00   3.00   2.97   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               90

227.00   3.00   2.95   227.00   3.00   2.95   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               92

230.00   3.00   2.90   230.00   3.00   2.90   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            91

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

233.00   3.00   2.76   233.00   3.00   2.76   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            91

236.00   3.00   2.90   236.00   3.00   2.90   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            83

239.00   3.00   2.90   239.00   3.00   2.90   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            84

242.00   3.00   2.95   242.00   3.00   2.95   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            89

245.00   3.00   2.98   245.00   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            98

248.00   3.00   2.70   248.00   3.00   2.70   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            73

251.00   3.00   2.97   251.00   3.00   2.97   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            79

254.00   3.00   2.00   254.00   3.00   2.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            62

257.00   3.00   2.50   257.00   3.00   2.50   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            63

260.00   3.00   1.85   260.00   3.00   1.85   SANDSTONE(FINE GRAINED),BROWNISH RED                                     42

263.00    .41    .41   263.00    .41    .41   SANDSTONE(FINE GRAINED),BROWNISH BROWN                                  100

263.41    .80    .80   263.41    .80    .80   CLAYSTONE,BROWNISH BROWN                                                 84

264.21    .79    .79   264.21    .79    .79   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH BROWN                         99

265.00   1.00    .80   265.00   1.00    .80   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH BROWN                         53

266.00   2.00   1.98   266.00   2.00   1.98   SANDSTONE(MED. TO C. GRAINED),BROWNISH BROWN                             99

268.00   2.12   1.72   268.00   2.12   1.72   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                               67

270.12    .88    .46   270.12    .88    .46   CLAYSTONE,BROWNISH BROWN                                                 86

271.00   1.00    .97   271.00   1.00    .97   CLAYSTONE,BROWNISH BROWN,SLICKENSIDED                                    30

272.00   1.50   1.48   272.00   1.50   1.48   CLAYSTONE,BROWNISH BROWN,SLICKENSIDED                                    23

273.50   1.50   1.46   273.50   1.50   1.46   CLAYSTONE,BROWNISH RED                                                   23

275.00   2.00    .80   275.00   2.00    .80   MUDSTONE

277.00   1.00    .90   277.00   1.00    .90   MUDSTONE,GREY

278.00   3.00   2.90   278.00   3.00   2.90   MUDSTONE,VARIEGATE

281.00   1.50   1.25   281.00   1.50   1.25   MUDSTONE,VARIEGATE

282.50    .50    .45   282.50    .50    .45   MUDSTONE,GREY

283.00   1.50   1.20   283.00   1.50   1.20   MUDSTONE,GREY

284.50   3.00   2.88   284.50   3.00   2.88   MUDSTONE,GREY

287.50   1.50   1.45   287.50   1.50   1.45   MUDSTONE,GREY                                                            37

289.00   1.50   1.35   289.00   1.50   1.35   MUDSTONE,BROWNISH GREY

290.50    .50    .35   290.50    .50    .35   MUDSTONE,BROWNISH GREY

291.00    .67    .55   291.00    .67    .55   MUDSTONE,BROWNISH GREY

291.67   1.33   1.23   291.67   1.33   1.23   SHALE,BROWNISH GREY

293.00   3.00   2.95   293.00   3.00   2.95   SHALE,GREY,SLICKENSIDED

296.00   3.00   2.85   296.00   3.00   2.85   SHALE,GREY                                                               11

299.00   2.00   1.63   299.00   2.00   1.63   SHALE,GREY

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

301.00   2.00    .81   301.00   2.00    .81   SHALE,GREY

303.00   2.00    .98   303.00   2.00    .98   SHALE,GREY

305.00   3.00   1.00   305.00   3.00   1.00   SHALE,GREY

308.00   1.50    .79   308.00   1.50    .79   SHALE,GREY                                                               28

309.50   1.50    .84   309.50   1.50    .84   SHALE,GREY                                                               33

311.00   3.00   2.95   311.00   3.00   2.95   SHALE,GREY                                                               80

314.00   1.50   1.44   314.00   1.50   1.44   MUDSTONE,GREY

315.50   1.50    .60   315.50   1.50    .60   MUDSTONE,GREY

317.00   3.00   2.63   317.00   3.00   2.63   SHALE,GREY

320.00    .40    .30   320.00    .40    .30   SHALE,GREY

320.40   2.60   2.50   320.40   2.60   2.50   MUDSTONE,GREY

323.00   1.50   1.45   323.00   1.50   1.45   SHALE,GREY

324.50   1.50   1.44   324.50   1.50   1.44   SHALE,GREY                                                               29

326.00   1.00    .93   326.00   1.00    .93   SHALE,GREY                                                               10

327.00   2.00   1.96   327.00   2.00   1.96   SHALE,GREY

329.00   3.00   2.40   329.00   3.00   2.40   SHALE,GREY                                                               21

332.00   1.00   1.00   332.00   1.00   1.00   SANDY SHALE,GREY,SLICKENSIDED

333.00   3.00   2.97   333.00   3.00   2.97   SANDY SHALE,GREY                                                         38

336.00    .50    .42   336.00    .50    .42   SANDY SHALE,GREY                                                         82

336.50   1.50   1.40   336.50   1.50   1.40   INTERCALATIONS,GREY                                                      51

338.00    .70    .65   338.00    .70    .65   SANDY SHALE,GREY                                                         29

338.70   1.00    .95   338.70   1.00    .95   SANDSTONE(MED. TO C. GRAINED),GREY                                       50

339.70    .30    .30   339.70    .30    .30   SANDY SHALE,GREY

340.00   1.00    .90   340.00   1.00    .90   SANDY SHALE,GREY

341.00   1.50    .50   341.00   1.50    .50   SANDY SHALE,GREY

342.50   1.50   1.40   342.50   1.50   1.40   SILTSTONE,GREY,SHALY,SLICKENSIDED

344.00   1.40   1.35   344.00   1.40   1.35   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   35

345.40   1.60   1.55   345.40   1.60   1.55   MUDSTONE,GREYISH BROWN

347.00   2.00   1.90   347.00   2.00   1.90   MUDSTONE,GREY

349.00   1.00    .95   349.00   1.00    .95   MUDSTONE,BROWNISH GREY

350.00   2.00   1.95   350.00   2.00   1.95   SHALE,GREY                                                               14

352.00   2.00   1.93   352.00   2.00   1.93   SHALE,GREY                                                               20

354.00   1.00    .98   354.00   1.00    .98   SHALE,GREY                                                               71

355.00   1.50   1.36   355.00   1.50   1.36   SHALE,GREY                                                               75

356.50   1.50   1.47   356.50   1.50   1.47   SHALE,BROWNISH GREY                                                      91

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

358.00   2.00   1.78   358.00   2.00   1.78   SHALE,BROWNISH GREY                                                      60

360.00   2.00   1.99   360.00   2.00   1.99   SHALE,BROWNISH GREY                                                      54

362.00   1.00    .98   362.00   1.00    .98   SHALE,GREY                                                               83

363.00   3.00   2.74   363.00   3.00   2.74   SHALE,GREYISH BROWN                                                      15

366.00    .70    .70   366.00    .70    .70   SHALE,GREYISH BROWN                                                      60

366.70   1.00    .98   366.70   1.00    .98   SHALE,GREYISH BROWN                                                      16

367.70   1.30   1.26   367.70   1.30   1.26   SHALE,GREY                                                               35

369.00   3.00   1.90   369.00   3.00   1.90   SANDY SHALE,GREY                                                         21

372.00   1.42   1.40   372.00   1.42   1.40   SANDY SHALE,GREY                                                         51

373.42    .40    .35   373.42    .40    .35   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

373.82   1.18   1.13   373.82   1.18   1.13   SANDY SHALE,GREY                                                         19

375.00   3.00   1.90   375.00   3.00   1.90   SANDY SHALE,GREY                                                         13

378.00   3.00   1.80   378.00   3.00   1.80   SANDY SHALE,GREY                                                         36

381.00   3.00    .70   381.00   3.00    .70   SHALE,GREY

384.00   1.12    .82   384.00   1.12    .82   SHALE,GREY

385.12   1.88   1.70   385.12   1.88   1.70   CLAYSTONE,CHOCOLATEBROWN                                                 13

387.00   2.00   1.45   387.00   2.00   1.45   CLAYSTONE,CHOCOLATEBROWN                                                 39

389.00    .19    .18   389.00    .19    .18   CLAYSTONE,CHOCOLATEBROWN

389.19    .61    .61   389.19    .61    .61   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   28

389.80   1.20   1.20   389.80   1.20   1.20   CLAYSTONE,GREY

391.00   2.00    .96   391.00   2.00    .96   CLAYSTONE,GREY

393.00   1.46   1.45   393.00   1.46   1.45   CLAYSTONE,GREY

394.46    .54    .53   394.46    .54    .53   SHALE,GREY

395.00   1.00    .98   395.00   1.00    .98   MUDSTONE,DARK GREY

396.00   3.00   2.50   396.00   3.00   2.50   MUDSTONE,DARK GREY                                                       12

399.00   3.00   2.97   399.00   3.00   2.97   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                   7.7

402.00   3.00   2.52   402.00   3.00   2.52   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    14

405.00   1.00    .97   405.00   1.00    .97   CLAYSTONE,CHOCOLATEBROWN

406.00   2.00   1.97   406.00   2.00   1.97   CLAYSTONE,CHOCOLATEBROWN

408.00   1.00    .98   408.00   1.00    .98   CLAYSTONE,CHOCOLATEBROWN                                                 13

409.00   3.00    .98   409.00   3.00    .98   CLAYSTONE,GREYISH BROWN                                                  14

412.00   1.00    .30   412.00   1.00    .30   CLAYSTONE,GREYISH BROWN

413.00   1.50    .40   413.00   1.50    .40   CLAYSTONE,GREYISH BROWN

414.50   3.00    .90   414.50   3.00    .90   CLAYSTONE,GREYISH BROWN                                                  16

417.50    .50    .29   417.50    .50    .29   CLAYSTONE,GREYISH BROWN                                                  58

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

418.00   2.00    .30   418.00   2.00    .30   CLAYSTONE,VARIEGATE                                                      15

420.00   3.00   2.95   420.00   3.00   2.95   CLAYSTONE,GREYISH BROWN                                                  17

423.00   3.00   2.97   423.00   3.00   2.97   CLAYSTONE,GREYISH BROWN                                                  84

426.00   1.00    .90   426.00   1.00    .90   CLAYSTONE,BROWNISH GREY

427.00   3.00   1.90   427.00   3.00   1.90   CLAYSTONE,CHOCOLATEBROWN                                                 19

430.00   3.00   2.60   430.00   3.00   2.60   CLAYSTONE,CHOCOLATEBROWN                                                9.3

433.00   3.00   2.75   433.00   3.00   2.75   CLAYSTONE,CHOCOLATEBROWN                                                4.7

436.00   1.00    .85   436.00   1.00    .85   CLAYSTONE,CHOCOLATEBROWN                                                 24

437.00   3.00   2.99   437.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 37

440.00   3.00   2.98   440.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    21

443.00   3.00   2.98   443.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN

446.00    .45    .40   446.00    .45    .40   CLAYSTONE,CHOCOLATEBROWN                                                 47

446.45    .22    .17   446.45    .22    .17   SANDSTONE(COARSE GRAINED),GREYISH BROWN

446.67    .48    .43   446.67    .48    .43   SANDY SHALE,GREYISH BROWN

447.15   1.85   1.80   447.15   1.85   1.80   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              89

449.00    .35    .17   449.00    .35    .17   SANDSTONE(COARSE GRAINED),GREY                                           49

449.35   2.65   2.55   449.35   2.65   2.55   CLAYSTONE,GREYISH BROWN                                                  29

452.00   1.50   1.50   452.00   1.50   1.50   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

453.50   1.50   1.48   453.50   1.50   1.48   CLAYSTONE,CHOCOLATEBROWN                                                 28

455.00   2.00   1.98   455.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN

457.00   1.00   1.00   457.00   1.00   1.00   CLAYSTONE,GREY

458.00   3.00   2.98   458.00   3.00   2.98   CLAYSTONE,GREYISH BROWN

461.00   3.00   2.99   461.00   3.00   2.99   CLAYSTONE,GREYISH BROWN

464.00   3.00   2.97   464.00   3.00   2.97   CLAYSTONE,CHOCOLATEBROWN                                                 44

467.00   3.00   2.97   467.00   3.00   2.97   CLAYSTONE,CHOCOLATEBROWN

470.00   3.00   2.97   470.00   3.00   2.97   CLAYSTONE,CHOCOLATEBROWN

473.00   3.00   2.99   473.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN

476.00   2.00   1.94   476.00   2.00   1.94   CLAYSTONE,CHOCOLATEBROWN                                                 46

478.00   3.00   2.98   478.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 27

481.00   1.00   1.00   481.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN

482.00   2.00   1.99   482.00   2.00   1.99   CLAYSTONE,CHOCOLATEBROWN                                                7.5

484.00    .40    .40   484.00    .40    .40   CLAYSTONE,CHOCOLATEBROWN

484.40    .42    .42   484.40    .42    .42   SANDY SHALE,GREY

484.82    .18    .18   484.82    .18    .18   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

485.00    .47    .47   485.00    .47    .47   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

485.47    .53    .50   485.47    .53    .50   SHALE,GREY                                                               72

486.00   2.00   2.00   486.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN

488.00    .80    .80   488.00    .80    .80   CLAYSTONE,CHOCOLATEBROWN

488.80    .70    .70   488.80    .70    .70   SANDSTONE(MED. TO C. GRAINED),GREY                                       47

489.50   1.07   1.05   489.50   1.07   1.05   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              90

490.57    .43    .40   490.57    .43    .40   CLAYSTONE,GREY

491.00   1.00    .98   491.00   1.00    .98   CLAYSTONE,GREY                                                           16

492.00   1.30   1.30   492.00   1.30   1.30   CLAYSTONE,CHOCOLATEBROWN                                                 32

493.30    .70    .70   493.30    .70    .70   SANDSTONE(COARSE GRAINED),BROWNISH GREY                                  31

494.00   1.00    .99   494.00   1.00    .99   SANDSTONE(COARSE GRAINED),BROWNISH GREY

495.00   2.00   2.00   495.00   2.00   2.00   SANDSTONE(COARSE GRAINED),BROWNISH GREY                                  50

497.00   1.00    .99   497.00   1.00    .99   CLAYSTONE,GREYISH BROWN                                                  25

498.00   3.00   2.99   498.00   3.00   2.99   CLAYSTONE,GREYISH BROWN,SLICKENSIDED                                      9

501.00   3.00   2.99   501.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN

504.00   1.70   1.68   504.00   1.70   1.68   CLAYSTONE,CHOCOLATEBROWN                                                 55

505.70   1.30   1.30   505.70   1.30   1.30   SANDSTONE(MED. TO C. GRAINED),GREY

507.00    .27    .27   507.00    .27    .27   SANDSTONE(MED. TO C. GRAINED),GREY

507.27   1.60   1.60   507.27   1.60   1.60   CLAYSTONE,CHOCOLATEBROWN

508.87    .13    .13   508.87    .13    .13   SANDSTONE(MEDIUM GRAINED),GREY

509.00   1.04   1.04   509.00   1.04   1.04   CLAYSTONE,CHOCOLATEBROWN

510.04    .80    .80   510.04    .80    .80   SANDSTONE(MED. TO C. GRAINED),GREY                                       73

510.84    .16    .16   510.84    .16    .16   CLAYSTONE,CHOCOLATEBROWN

511.00   1.50   1.48   511.00   1.50   1.48   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

512.50   1.50   1.49   512.50   1.50   1.49   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

514.00   1.00   1.00   514.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

515.00   1.00    .98   515.00   1.00    .98   CLAYSTONE,CHOCOLATEBROWN

516.00   2.00   1.98   516.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

518.00    .37    .30   518.00    .37    .30   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

518.37    .41    .34   518.37    .41    .34   SANDSTONE(MEDIUM GRAINED),GREY                                           27

518.78    .58    .50   518.78    .58    .50   CLAYSTONE,CHOCOLATEBROWN

519.36    .77    .70   519.36    .77    .70   SANDSTONE(MEDIUM GRAINED),GREY

520.13    .87    .80   520.13    .87    .80   SANDY SHALE,GREY,FRACTURE

521.00    .30    .30   521.00    .30    .30   SANDY SHALE,GREY,FRACTURE

521.30    .70    .69   521.30    .70    .69   SANDSTONE(MED. TO C. GRAINED),GREY                                       44

522.00    .53    .52   522.00    .53    .52   SANDSTONE(MED. TO C. GRAINED),GREY                                       98

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

522.53   2.47   2.46   522.53   2.47   2.46   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

525.00   3.00   2.99   525.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

528.00   3.00   2.98   528.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

531.00   2.00   1.98   531.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

533.00   2.00   1.99   533.00   2.00   1.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

535.00   1.00   1.00   535.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

536.00    .10    .10   536.00    .10    .10   CLAYSTONE,CHOCOLATEBROWN

536.10   1.40   1.40   536.10   1.40   1.40   SANDSTONE(MED. TO C. GRAINED),GREY                                       60

537.50    .10    .10   537.50    .10    .10   CLAYSTONE,CHOCOLATEBROWN

*****************************************************************************************************************************

BARAKAR

537.60   1.40   1.40   537.60   1.40   1.40   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              94

539.00   1.50   1.47   539.00   1.50   1.47   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,                             71

540.50   1.50   1.46   540.50   1.50   1.46   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   83

542.00   3.00   2.99   542.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREY,FRACTURE                              88

545.00   3.00   2.98   545.00   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              90

548.00   3.00   2.53   548.00   3.00   2.53   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              25

551.00   1.00   1.00   551.00   1.00   1.00   SANDSTONE(COARSE GRAINED),GREYISH GREEN                                  83

552.00   1.17   1.17   552.00   1.17   1.17   SANDY SHALE,GREYISH GREEN,FRACTURE

553.17   1.63   1.63   553.17   1.63   1.63   SHALE,DARK GREY,FRACTURE

554.80    .20    .20   554.80    .20    .20   SHALE,GREY

555.00    .20    .18   555.00    .20    .18   SHALE,GREY

555.20   1.80   1.80   555.20   1.80   1.80   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    76

557.00    .42    .42   557.00    .42    .42   SHALE,GREY

557.42   2.16   2.16   557.42   2.16   2.16   SANDY SHALE,GREY,SLICKENSIDED                                            67

559.58    .42    .42   559.58    .42    .42   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          67

560.00   3.00   2.98   560.00   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              83

563.00   2.35   2.30   563.00   2.35   2.30   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          82

565.35    .65    .50   565.35    .65    .50   SANDY SHALE,GREY,SLICKENSIDED

566.00    .40    .39   566.00    .40    .39   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    33

566.40    .78    .78   566.40    .78    .78   SANDY SHALE,GREY,SLICKENSIDED                                            37

567.18   1.82   1.82   567.18   1.82   1.82   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          99

569.00   3.00   2.99   569.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              28

572.00   3.00   2.99   572.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              86

575.00   3.00   2.99   575.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              83

578.00   3.00   2.99   578.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),WHITE                                      35

581.00   2.75   2.73   581.00   2.75   2.73   SANDSTONE(MED. TO C. GRAINED),WHITE                                      86

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

583.75    .25    .20   583.75    .25    .20   SHALY COAL,SLICKENSIDED                                         L1

584.00    .11    .10   584.00    .11    .10   SHALY COAL,SLICKENSIDED                                         L1

584.11   2.89   2.89   584.11   2.89   2.89   SANDSTONE(MED. TO C. GRAINED),WHITE,SLICKENSIDED                         73

587.00   3.00   2.99   587.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),WHITE                                      83

590.00    .12    .12   590.00    .12    .12   SANDSTONE(FINE GRAINED),WHITE

590.12    .90    .89   590.12    .90    .89   SHALE,DARK GREY

591.02    .18    .15   591.02    .18    .15   CARB SHALE HIGH

591.20    .24    .24   591.20    .24    .24   CARB SHALE HIGH,SLICKENSIDED

591.44    .24    .24   591.44    .24    .24   CARB SHALE HIGH

591.68    .25    .25   591.68    .25    .25   SHALY COAL                                                      IIA

591.93    .98    .98   591.93    .98    .98   COAL                                                            IIA

592.91    .99    .98   592.91    .99    .98   COAL                                                            IIA

593.90    .30    .30   593.90    .30    .30   COAL                                                            IIA

594.20    .10    .06   594.20    .10    .06   COAL                                                            IIA

594.30   2.30   2.30   594.30   2.30   2.30   SANDSTONE(FINE GRAINED),GREY                                             87

596.60   3.00   2.99   596.60   3.00   2.99   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    88

599.60   3.00   2.99   599.60   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              87

602.60   3.00   2.99   602.60   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              84

605.60   3.00   2.99   605.60   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              78

608.60   3.00   2.99   608.60   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              95

611.60   3.00   2.99   611.60   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              99

614.60   1.67   1.63   614.60   1.67   1.63   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          95

616.27    .20    .17   616.27    .20    .17   SHALY COAL                                                      L

616.47   1.13   1.10   616.47   1.13   1.10   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              99

617.60   3.00   2.99   617.60   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              24

620.60   1.02    .98   620.60   1.02    .98   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              95

621.62    .08    .06   621.62    .08    .06   SHALY COAL                                                      L

621.70   1.90   1.86   621.70   1.90   1.86   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,                             61

623.60   2.90   2.85   623.60   2.90   2.85   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         62

626.50   1.90   1.88   626.50   1.90   1.88   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          93

BOREHOLE CLOSED AT   628.40 M DEPTH

BOREHOLE NO    :CMWWB05              LATITUDE    : 2209720.133M                   DATE OF COMMENCEMENT: 28.02.2024

REDUCED LEVEL  : 265.2455M           DEPARTURE   : 266452.7581M                   DATE OF COMPLETION: 09.04.2024

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

.00   2.00    .15      .00   2.00    .15   SOIL,BLACK

SOIL

****************************************************************************************************************************

TRAP

2.00   3.00   1.20     2.00   3.00   1.20   BASALT,GREYISH BROWN

5.00   2.00   1.10     5.00   2.00   1.10   BASALT,GREYISH BROWN

7.00   1.00    .45     7.00   1.00    .45   BASALT,GREY                                                              39

8.00   2.00   1.94     8.00   2.00   1.94   BASALT,GREY,FRACTURE                                                     37

10.00   1.50   1.50    10.00   1.50   1.50   BASALT,GREY                                                              42

11.50    .50    .49    11.50    .50    .49   BASALT,GREY,FRACTURE                                                     56

12.00   3.00   2.99    12.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     62

15.00   1.50   1.49    15.00   1.50   1.49   BASALT,GREY,FRACTURE

16.50   1.50    .65    16.50   1.50    .65   BASALT,GREY                                                              32

18.00   1.00    .33    18.00   1.00    .33   BASALT,BROWNISH GREEN

**************************************************************************************************************************

MOTUR

19.00   3.00   1.00    19.00   3.00   1.00   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    13

22.00   2.00   1.00    22.00   2.00   1.00   SANDSTONE(MED. TO C. GRAINED),VARIEGATEGREY                              35

24.00   3.00    .43    24.00   3.00    .43   SANDSTONE(MED. TO C. GRAINED),YELLOWISHBROWN                            3.7

27.00   1.50    .27    27.00   1.50    .27   SANDSTONE(COARSE GRAINED),YELLOWISHBROWN

28.50   1.50    .80    28.50   1.50    .80   CLAYSTONE,REDDISH BROWN

30.00   1.00    .90    30.00   1.00    .90   CLAYSTONE,REDDISH BROWN

31.00   2.00   1.85    31.00   2.00   1.85   CLAYSTONE,REDDISH BROWN

33.00   3.00   2.30    33.00   3.00   2.30   CLAYSTONE,REDDISH BROWN

36.00   3.00   1.52    36.00   3.00   1.52   CLAYSTONE,REDDISH BROWN

39.00   1.00    .90    39.00   1.00    .90   CLAYSTONE,REDDISH BROWN

40.00   2.00   1.85    40.00   2.00   1.85   CLAYSTONE,BROWNISH RED                                                   32

42.00   3.00   2.88    42.00   3.00   2.88   SANDSTONE(FINE GRAINED),BROWNISH RED                                     89

45.00   3.00   2.00    45.00   3.00   2.00   SANDSTONE(FINE GRAINED),BROWNISH RED                                     31

48.00   2.00   1.65    48.00   2.00   1.65   SANDSTONE(MEDIUM GRAINED),BROWN                                          38

50.00   1.00    .30    50.00   1.00    .30   CLAYSTONE,BROWN

51.00   1.00    .90    51.00   1.00    .90   CLAYSTONE,BROWN,FRACTURE

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

52.00   2.00    .55    52.00   2.00    .55   CLAYSTONE,BROWNISH RED

54.00   3.00    .80    54.00   3.00    .80   CLAYSTONE,BROWNISH RED

57.00   3.00    .75    57.00   3.00    .75   CLAYSTONE,BROWNISH RED

60.00   3.00   1.00    60.00   3.00   1.00   CLAYSTONE,REDDISH BROWN

63.00   2.00    .46    63.00   2.00    .46   CLAYSTONE,REDDISH BROWN

65.00   1.00    .24    65.00   1.00    .24   SANDSTONE(FINE GRAINED),BROWNISH YELLOW

66.00   3.00    .70    66.00   3.00    .70   SANDSTONE(COARSE GRAINED),REDDISH BROWN

69.00   3.00    .12    69.00   3.00    .12   SANDSTONE(COARSE GRAINED),YELLOWISHBROWN

72.00   3.00    .15    72.00   3.00    .15   SANDSTONE(COARSE GRAINED),YELLOWISHBROWN

75.00   3.00    .14    75.00   3.00    .14   SANDSTONE(COARSE GRAINED),YELLOWISHBROWN

78.00   3.00    .12    78.00   3.00    .12   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

81.00   3.00    .15    81.00   3.00    .15   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

84.00   3.00    .20    84.00   3.00    .20   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW

87.00   3.00    .55    87.00   3.00    .55   SANDSTONE(MEDIUM GRAINED),BROWNISH RED                                   12

90.00   3.00    .32    90.00   3.00    .32   SANDSTONE(MEDIUM GRAINED),BROWNISH RED

93.00   3.00    .16    93.00   3.00    .16   CLAYSTONE,REDDISH BROWN

96.00   3.00    .17    96.00   3.00    .17   CLAYSTONE,REDDISH BROWN

99.00   3.00    .25    99.00   3.00    .25   SANDSTONE(FINE GRAINED),REDDISH BROWN

102.00   3.00   2.10   102.00   3.00   2.10   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    30

105.00   3.00    .68   105.00   3.00    .68   SANDSTONE(FINE GRAINED),REDDISH BROWN

108.00   3.00    .70   108.00   3.00    .70   CLAYSTONE,REDDISH BROWN

111.00   3.00   1.50   111.00   3.00   1.50   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    37

114.00   3.00    .80   114.00   3.00    .80   SANDSTONE(FINE GRAINED),REDDISH BROWN                                     4

117.00   3.00   1.70   117.00   3.00   1.70   SANDSTONE(FINE GRAINED),BROWNISH YELLOW                                  41

120.00   3.00   1.20   120.00   3.00   1.20   SANDSTONE(FINE GRAINED),BROWNISH YELLOW                                 6.3

123.00   3.00   1.00   123.00   3.00   1.00   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW                                 4

126.00   3.00    .90   126.00   3.00    .90   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW

129.00   3.00   1.10   129.00   3.00   1.10   SANDSTONE(FINE GRAINED),BROWNISH YELLOW

132.00   3.00   1.10   132.00   3.00   1.10   SANDSTONE(FINE GRAINED),BROWNISH YELLOW                                   8

135.00   3.00   2.80   135.00   3.00   2.80   SANDSTONE(FINE GRAINED),BROWNISH YELLOW                                  23

138.00   2.41   2.26   138.00   2.41   2.26   SANDSTONE(FINE GRAINED),BROWNISH YELLOW

140.41    .59    .44   140.41    .59    .44   CLAYSTONE,REDDISH BROWN

141.00   3.00   2.90   141.00   3.00   2.90   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        40

144.00   3.00   2.90   144.00   3.00   2.90   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        61

147.00   3.00   2.85   147.00   3.00   2.85   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN                         25

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

150.00   3.00    .85   150.00   3.00    .85   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN                         22

153.00   3.00    .90   153.00   3.00    .90   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN

156.00    .75    .70   156.00    .75    .70   CLAYSTONE,REDDISH BROWN                                                  76

156.75    .95    .90   156.75    .95    .90   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN                                 69

157.70   1.30   1.25   157.70   1.30   1.25   CLAYSTONE,REDDISH BROWN                                                  61

159.00   3.00   2.00   159.00   3.00   2.00   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    42

162.00   3.00   2.57   162.00   3.00   2.57   SANDSTONE(FINE GRAINED),REDDISH BROWN,FRACTURE                           35

165.00   3.00   2.00   165.00   3.00   2.00   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    17

168.00   3.00   1.21   168.00   3.00   1.21   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    20

171.00   3.00   1.00   171.00   3.00   1.00   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    16

174.00   3.00    .40   174.00   3.00    .40   SANDSTONE(FINE GRAINED),REDDISH BROWN                                     5

177.00   3.00    .25   177.00   3.00    .25   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW

180.00   3.00    .60   180.00   3.00    .60   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                                9

183.00   3.00    .36   183.00   3.00    .36   SANDSTONE(MED. TO C. GRAINED),BROWNISH RED                              4.7

186.00   3.00   1.50   186.00   3.00   1.50   SANDSTONE(FINE GRAINED),BROWNISH YELLOW                                 3.7

189.00   3.00    .90   189.00   3.00    .90   SANDSTONE(FINE GRAINED),BROWNISH YELLOW                                 6.7

192.00   3.00   1.00   192.00   3.00   1.00   SANDSTONE(FINE GRAINED),BROWNISH YELLOW                                   4

195.00   3.00   1.10   195.00   3.00   1.10   SANDSTONE(FINE GRAINED),BROWNISH YELLOW                                   4

198.00   3.00   1.50   198.00   3.00   1.50   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW

201.00   3.00   1.45   201.00   3.00   1.45   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW                               8.3

204.00   3.00    .50   204.00   3.00    .50   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW                                 5

207.00   3.00   1.10   207.00   3.00   1.10   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN                        5.3

210.00   3.00   1.00   210.00   3.00   1.00   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN

213.00   3.00   1.30   213.00   3.00   1.30   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                       4.7

216.00   3.00   1.90   216.00   3.00   1.90   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                       3.7

219.00   3.00   1.50   219.00   3.00   1.50   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        10

222.00   2.00    .66   222.00   2.00    .66   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN                                6.5

224.00   1.00    .80   224.00   1.00    .80   CLAYSTONE,REDDISH BROWN                                                  63

225.00   3.00   1.25   225.00   3.00   1.25   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    18

228.00   3.00   1.30   228.00   3.00   1.30   CLAYSTONE,REDDISH BROWN,FRACTURE                                         22

231.00   3.00   1.50   231.00   3.00   1.50   CLAYSTONE,REDDISH BROWN,FRACTURE                                        3.3

234.00   3.00   1.00   234.00   3.00   1.00   CLAYSTONE,REDDISH BROWN,FRACTURE

237.00   1.00    .40   237.00   1.00    .40   CLAYSTONE,REDDISH BROWN,FRACTURE

238.00   2.00   1.76   238.00   2.00   1.76   CLAYSTONE,REDDISH BROWN,FRACTURE                                         25

240.00   2.00   2.00   240.00   2.00   2.00   MUDSTONE,GREY                                                            24

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

242.00   1.00   1.00   242.00   1.00   1.00   MUDSTONE,GREY                                                            28

243.00   1.00    .90   243.00   1.00    .90   MUDSTONE,GREY

244.00   3.00   2.80   244.00   3.00   2.80   MUDSTONE,GREY,FRACTURE

247.00   3.00   2.98   247.00   3.00   2.98   MUDSTONE,GREY,FRACTURE

250.00   2.00   2.00   250.00   2.00   2.00   MUDSTONE,GREY,FRACTURE                                                   19

252.00   3.00   2.95   252.00   3.00   2.95   MUDSTONE,GREY                                                           9.7

255.00   2.00   1.90   255.00   2.00   1.90   MUDSTONE,VARIEGATE

257.00   1.00    .95   257.00   1.00    .95   MUDSTONE,GREY                                                            61

258.00   3.00   2.90   258.00   3.00   2.90   MUDSTONE,GREYISH BROWN,FRACTURE                                         4.3

261.00   3.00   2.99   261.00   3.00   2.99   MUDSTONE,GREYISH BROWN,FRACTURE

264.00   1.00    .70   264.00   1.00    .70   MUDSTONE,GREY,FRACTURE

265.00   2.00   1.27   265.00   2.00   1.27   MUDSTONE,GREY,FRACTURE                                                  6.5

267.00   3.00   2.70   267.00   3.00   2.70   CLAYSTONE,GREYISH BROWN,FRACTURE                                         13

270.00   1.00    .54   270.00   1.00    .54   MUDSTONE,GREY,FRACTURE

271.00   2.00   1.17   271.00   2.00   1.17   CLAYSTONE,GREYISH BROWN,FRACTURE

273.00   3.00   1.60   273.00   3.00   1.60   CLAYSTONE,GREYISH BROWN,FRACTURE

276.00   3.00   1.80   276.00   3.00   1.80   CLAYSTONE,GREY,FRACTURE

279.00   2.00   1.70   279.00   2.00   1.70   CLAYSTONE,BROWN,FRACTURE

281.00   1.00    .98   281.00   1.00    .98   SANDSTONE(FINE GRAINED),GREYISH BROWN,FRACTURE                           20

282.00   3.00   2.98   282.00   3.00   2.98   MUDSTONE,GREYISH BROWN

285.00   3.00   2.98   285.00   3.00   2.98   MUDSTONE,GREYISH BROWN                                                   13

288.00   3.00   2.99   288.00   3.00   2.99   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  71

291.00   3.00   2.95   291.00   3.00   2.95   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                              46

294.00   1.24   1.23   294.00   1.24   1.23   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                              60

295.24   1.76   1.75   295.24   1.76   1.75   CLAYSTONE,GREYISH BROWN,FRACTURE                                         31

297.00    .62    .62   297.00    .62    .62   CLAYSTONE,GREYISH BROWN,FRACTURE                                         65

297.62   1.00   1.00   297.62   1.00   1.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN                          59

298.62    .45    .44   298.62    .45    .44   CLAYSTONE,BROWN

299.07    .93    .93   299.07    .93    .93   SANDSTONE(MED. TO C. GRAINED),BROWN                                      25

300.00    .51    .41   300.00    .51    .41   SANDSTONE(MED. TO C. GRAINED),BROWN                                      27

300.51    .88    .68   300.51    .88    .68   CLAYSTONE,BROWN                                                          18

301.39    .38    .28   301.39    .38    .28   SANDSTONE(FINE GRAINED),GREY                                             50

301.77   1.23   1.08   301.77   1.23   1.08   CLAYSTONE,GREYISH BROWN                                                  75

303.00   3.00   2.98   303.00   3.00   2.98   SANDY SHALE,GREY                                                         79

306.00    .75    .72   306.00    .75    .72   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              84

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

306.75    .75    .73   306.75    .75    .73   MUDSTONE,GREY

307.50    .46    .45   307.50    .46    .45   MUDSTONE,GREY

307.96   1.04   1.03   307.96   1.04   1.03   MUDSTONE,VARIEGATE

309.00   3.00   2.99   309.00   3.00   2.99   MUDSTONE,BROWN                                                           37

312.00   3.00   2.95   312.00   3.00   2.95   MUDSTONE,BROWN                                                           70

315.00   2.62   2.60   315.00   2.62   2.60   CLAYSTONE,BROWN                                                          70

317.62    .38    .35   317.62    .38    .35   SANDSTONE(MEDIUM GRAINED),GREY                                           71

318.00   1.00    .98   318.00   1.00    .98   SANDSTONE(MED. TO C. GRAINED),GREY                                       50

319.00   3.00   2.99   319.00   3.00   2.99   CLAYSTONE,GREY                                                           97

322.00   3.00   2.99   322.00   3.00   2.99   CLAYSTONE,GREY                                                           93

325.00   1.50   1.48   325.00   1.50   1.48   CLAYSTONE,GREYISH BROWN,SLICKENSIDED                                     98

326.50   3.00   2.98   326.50   3.00   2.98   CLAYSTONE,GREYISH BROWN,SLICKENSIDED                                     71

329.50   3.00   2.99   329.50   3.00   2.99   ALTERNATING SHALE AND SST,BROWNISH GREY                                  47

332.50   3.00   2.98   332.50   3.00   2.98   ALTERNATING SHALE AND SST,BROWNISH GREY                                  32

335.50   3.00   2.98   335.50   3.00   2.98   ALTERNATING SHALE AND SST,BROWNISH GREY                                  84

338.50   3.00   2.98   338.50   3.00   2.98   CLAYSTONE,BROWNISH GREY                                                  62

341.50   3.00   2.98   341.50   3.00   2.98   CLAYSTONE,GREY

344.50   3.00   2.95   344.50   3.00   2.95   MUDSTONE,GREY,FRACTURE                                                   30

347.50    .45    .30   347.50    .45    .30   MUDSTONE,GREY

347.95   1.65   1.55   347.95   1.65   1.55   CLAYSTONE,BROWN                                                          40

349.60    .90    .80   349.60    .90    .80   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   69

350.50    .90    .39   350.50    .90    .39   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  23

351.40   2.10   1.58   351.40   2.10   1.58   CLAYSTONE,VARIEGATE                                                      31

353.50   3.00   2.55   353.50   3.00   2.55   CLAYSTONE,VARIEGATE                                                      48

356.50   3.00   2.90   356.50   3.00   2.90   CLAYSTONE,BROWNISH GREY,SLICKENSIDED

359.50   1.50   1.45   359.50   1.50   1.45   MUDSTONE,GREY                                                            15

361.00   1.50   1.49   361.00   1.50   1.49   MUDSTONE,GREY

362.50   2.50   2.48   362.50   2.50   2.48   CLAYSTONE,VARIEGAT,FRACTURE                                               6

365.00   3.00   2.99   365.00   3.00   2.99   CLAYSTONE,BROWNISH GREY,FRACTURE

368.00   3.00   2.99   368.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                  23

371.00   3.00   2.98   371.00   3.00   2.98   CLAYSTONE,BROWN,FRACTURE

374.00   1.42   1.42   374.00   1.42   1.42   CLAYSTONE,BROWN                                                          17

375.42    .23    .23   375.42    .23    .23   MUDSTONE,GREY

375.65    .28    .28   375.65    .28    .28   SANDSTONE(FINE GRAINED),GREY                                             89

375.93   1.07   1.07   375.93   1.07   1.07   CLAYSTONE,BROWN,FRACTURE

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

377.00   3.00   2.99   377.00   3.00   2.99   CLAYSTONE,BROWN,FRACTURE                                                 14

380.00   3.00   2.99   380.00   3.00   2.99   CLAYSTONE,BROWN,FRACTURE                                                 51

383.00   3.00   2.99   383.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                  69

386.00   3.00   2.99   386.00   3.00   2.99   CLAYSTONE,BROWNISH GREY,FRACTURE                                         42

389.00   3.00   2.99   389.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                  65

392.00   3.00   2.99   392.00   3.00   2.99   MUDSTONE,GREYISH BROWN                                                   36

395.00   3.00   2.95   395.00   3.00   2.95   CLAYSTONE,GREYISH BROWN,FRACTURE                                         23

398.00   3.00   2.99   398.00   3.00   2.99   CLAYSTONE,BROWN,SLICKENSIDED                                              9

401.00    .52    .39   401.00    .52    .39   CLAYSTONE,BROWN,SLICKENSIDED

401.52    .42    .29   401.52    .42    .29   SANDSTONE(MEDIUM GRAINED),GREY                                           64

401.94   2.06   1.94   401.94   2.06   1.94   CLAYSTONE,GREYISH BROWN

404.00   3.00   2.57   404.00   3.00   2.57   CLAYSTONE,GREYISH BROWN                                                  51

407.00   3.00   2.30   407.00   3.00   2.30   CLAYSTONE,CHOCOLATEBROWN

410.00   3.00   2.98   410.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        21

413.00   3.00   2.99   413.00   3.00   2.99   CLAYSTONE,VARIEGAT,FRACTURE                                              29

416.00   3.00   2.99   416.00   3.00   2.99   CLAYSTONE,VARIEGAT,FRACTURE                                              32

419.00   3.00   2.97   419.00   3.00   2.97   CLAYSTONE,GREYISH BROWN,FRACTURE                                         24

422.00   3.00   2.97   422.00   3.00   2.97   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                       9.3

425.00   3.00   2.98   425.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

428.00   3.00   2.95   428.00   3.00   2.95   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

431.00   2.32   2.32   431.00   2.32   2.32   CLAYSTONE,GREYISH BROWN,FRACTURE

433.32    .46    .46   433.32    .46    .46   SANDSTONE(MEDIUM GRAINED),GREY                                           91

433.78    .22    .22   433.78    .22    .22   CLAYSTONE,GREY

434.00    .17    .13   434.00    .17    .13   CLAYSTONE,CHOCOLATEBROWN

434.17    .27    .24   434.17    .27    .24   SHALE,GREY,FRACTURE

434.44   1.78   1.75   434.44   1.78   1.75   SANDSTONE(MED. TO C. GRAINED),VIOLET WHITE                               69

436.22    .78    .75   436.22    .78    .75   MUDSTONE,GREYISH BROWN

437.00    .27    .22   437.00    .27    .22   CLAYSTONE,GREYISH BROWN,FRACTURE

437.27    .30    .25   437.27    .30    .25   SANDSTONE(FINE GRAINED),GREY

437.57   2.43   2.38   437.57   2.43   2.38   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

440.00    .41    .41   440.00    .41    .41   SANDSTONE(FINE GRAINED),GREY                                             98

440.41   1.60   1.60   440.41   1.60   1.60   MUDSTONE,GREY,SLICKENSIDED

442.01    .70    .70   442.01    .70    .70   SANDSTONE(FINE GRAINED),GREY,FRACTURE                                    30

442.71    .29    .29   442.71    .29    .29   CLAYSTONE,GREYISH BROWN,FRACTURE

443.00   3.00   2.90   443.00   3.00   2.90   CLAYSTONE,BROWN,FRACTURE

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

446.00   1.00   1.00   446.00   1.00   1.00   CLAYSTONE,GREYISH BROWN,FRACTURE

447.00   2.00   1.95   447.00   2.00   1.95   CLAYSTONE,GREYISH BROWN

449.00   1.48   1.48   449.00   1.48   1.48   CLAYSTONE,BROWN

450.48    .62    .62   450.48    .62    .62   SANDSTONE(FINE GRAINED),GREYISH GREEN                                    39

451.10    .90    .90   451.10    .90    .90   CLAYSTONE,BROWNISH GREY,SLICKENSIDED

452.00   3.00   2.98   452.00   3.00   2.98   CLAYSTONE,GREYISH BROWN

455.00   1.60   1.60   455.00   1.60   1.60   MUDSTONE,GREY                                                           6.9

456.60   1.40   1.38   456.60   1.40   1.38   CLAYSTONE,BROWN                                                          14

458.00   2.50   2.44   458.00   2.50   2.44   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          59

460.50   3.00   2.99   460.50   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN                          70

463.50    .20    .17   463.50    .20    .17   SANDSTONE(MEDIUM GRAINED),GREYISH BROWN                                  60

463.70   2.80   2.80   463.70   2.80   2.80   CLAYSTONE,BROWNISH GREY,FRACTURE

466.50   1.50   1.48   466.50   1.50   1.48   CLAYSTONE,BROWNISH GREY,FRACTURE                                         10

468.00   1.50   1.45   468.00   1.50   1.45   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   36

469.50   2.00   1.70   469.50   2.00   1.70   SANDSTONE(MEDIUM GRAINED),GREYISH GREEN                                  59

471.50   1.00    .75   471.50   1.00    .75   MUDSTONE,GREY,SLICKENSIDED

472.50   1.09   1.06   472.50   1.09   1.06   CLAYSTONE,BROWN                                                          29

473.59   1.05   1.02   473.59   1.05   1.02   SANDSTONE(FINE GRAINED),GREYISH BROWN                                    14

474.64    .36    .32   474.64    .36    .32   CLAYSTONE,BROWNISH GREY

475.00    .26    .22   475.00    .26    .22   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  85

475.26    .24    .20   475.26    .24    .20   CLAYSTONE,DARK BROWN

475.50    .50    .48   475.50    .50    .48   CLAYSTONE,DARK BROWN

476.00   3.00   2.95   476.00   3.00   2.95   CLAYSTONE,DARK BROWN,FRACTURE                                             6

479.00    .20    .20   479.00    .20    .20   CLAYSTONE,CHOCOLATEBROWN

479.20    .18    .18   479.20    .18    .18   MUDSTONE,GREY

479.38   2.12   2.12   479.38   2.12   2.12   SANDSTONE(FINE GRAINED),GREY                                             92

481.50    .50    .50   481.50    .50    .50   MUDSTONE,GREY,FRACTURE

482.00   3.00   2.98   482.00   3.00   2.98   MUDSTONE,GREYISH GREEN,FRACTURE

485.00   3.00   2.99   485.00   3.00   2.99   MUDSTONE,VARIEGAT,FRACTURE

488.00   3.00   2.99   488.00   3.00   2.99   CLAYSTONE,BROWN,FRACTURE                                                 14

491.00   1.50   1.50   491.00   1.50   1.50   SANDSTONE(FINE GRAINED),BROWN                                            74

492.50   1.00   1.00   492.50   1.00   1.00   SANDSTONE(FINE GRAINED),BROWN                                            42

493.50    .50    .47   493.50    .50    .47   CLAYSTONE,BROWN,FRACTURE

494.00   3.00   2.98   494.00   3.00   2.98   CLAYSTONE,BROWN                                                          45

497.00   3.00   2.99   497.00   3.00   2.99   CLAYSTONE,BROWN                                                          87

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

500.00   1.20   1.19   500.00   1.20   1.19   CLAYSTONE,BROWN                                                          93

501.20   1.80   1.79   501.20   1.80   1.79   SANDSTONE(FINE GRAINED),BROWN                                            92

503.00   2.40   2.36   503.00   2.40   2.36   SANDSTONE(MED. TO C. GRAINED),BROWN                                      76

505.40    .60    .54   505.40    .60    .54   CLAYSTONE,BROWN,FRACTURE                                                 33

506.00   3.00   2.98   506.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN

509.00   3.00   2.98   509.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN

512.00   3.00   2.99   512.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN

515.00   2.40   2.38   515.00   2.40   2.38   CLAYSTONE,CHOCOLATEBROWN

517.40    .60    .60   517.40    .60    .60   SANDSTONE(COARSE GRAINED),BROWNISH GREY                                  78

518.00    .20    .18   518.00    .20    .18   SANDSTONE(MED. TO C. GRAINED),GREY

518.20   2.80   2.80   518.20   2.80   2.80   CLAYSTONE,CHOCOLATEBROWN

521.00   3.00   2.99   521.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN

524.00   3.00   2.99   524.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 11

527.00   3.00   2.98   527.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 45

530.00   3.00   2.99   530.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 72

533.00   2.00   1.99   533.00   2.00   1.99   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    74

535.00   1.38   1.34   535.00   1.38   1.34   CLAYSTONE,CHOCOLATEBROWN                                                 57

536.38    .44    .40   536.38    .44    .40   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  86

536.82   1.18   1.14   536.82   1.18   1.14   CLAYSTONE,CHOCOLATEBROWN                                                 75

538.00   3.00   1.58   538.00   3.00   1.58   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

541.00   2.00    .45   541.00   2.00    .45   CLAYSTONE,BROWNISH GREY,SLICKENSIDED

543.00   3.00   1.90   543.00   3.00   1.90   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

546.00   3.00   2.70   546.00   3.00   2.70   CLAYSTONE,CHOCOLATEBROWN

549.00   1.00    .99   549.00   1.00    .99   CLAYSTONE,CHOCOLATEBROWN

550.00   2.00   1.70   550.00   2.00   1.70   CLAYSTONE,CHOCOLATEBROWN

552.00   3.00   2.50   552.00   3.00   2.50   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

555.00    .49    .47   555.00    .49    .47   CLAYSTONE,CHOCOLATEBROWN

555.49    .34    .32   555.49    .34    .32   SANDSTONE(FINE GRAINED),CHOCOLATEBROWN                                   97

555.83   1.06   1.04   555.83   1.06   1.04   CLAYSTONE,CHOCOLATEBROWN                                                 58

556.89    .47    .45   556.89    .47    .45   SANDSTONE(MEDIUM GRAINED),CHOCOLATEBROWN                                 70

557.36    .64    .62   557.36    .64    .62   CLAYSTONE,CHOCOLATEBROWN                                                 61

558.00    .45    .44   558.00    .45    .44   CLAYSTONE,CHOCOLATEBROWN                                                 33

558.45    .28    .26   558.45    .28    .26   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  93

558.73    .22    .20   558.73    .22    .20   CLAYSTONE,CHOCOLATEBROWN

558.95    .66    .65   558.95    .66    .65   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                              50

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

559.61   1.39   1.38   559.61   1.39   1.38   CLAYSTONE,CHOCOLATEBROWN                                                 29

561.00    .38    .38   561.00    .38    .38   CLAYSTONE,CHOCOLATEBROWN

561.38    .57    .57   561.38    .57    .57   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  74

561.95   2.05   2.05   561.95   2.05   2.05   CLAYSTONE,CHOCOLATEBROWN                                                 30

564.00   1.00   1.00   564.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN

565.00   2.00   1.48   565.00   2.00   1.48   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

567.00   3.00   2.98   567.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        29

570.00   3.00   2.99   570.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        60

573.00   3.00   2.99   573.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        42

576.00   3.00   2.98   576.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        10

579.00   3.00   2.99   579.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

582.00   3.00   2.99   582.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

585.00   3.00   2.99   585.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

588.00   3.00   2.98   588.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        24

591.00   1.64   1.60   591.00   1.64   1.60   CLAYSTONE,CHOCOLATEBROWN                                                 35

592.64    .36    .32   592.64    .36    .32   SANDSTONE(MEDIUM GRAINED),GREYISH BROWN                                  28

593.00    .18    .13   593.00    .18    .13   CLAYSTONE,CHOCOLATEBROWN

593.18    .47    .42   593.18    .47    .42   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              60

593.65    .35    .30   593.65    .35    .30   CLAYSTONE,CHOCOLATEBROWN                                                 37

594.00    .87    .40   594.00    .87    .40   CLAYSTONE,CHOCOLATEBROWN                                                 37

594.87    .73    .26   594.87    .73    .26   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  33

595.60   1.40   1.00   595.60   1.40   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 51

597.00   3.00   1.50   597.00   3.00   1.50   CLAYSTONE,CHOCOLATEBROWN                                                 35

600.00   3.00   2.99   600.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                7.7

603.00   1.00   1.00   603.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 12

604.00   3.00   1.50   604.00   3.00   1.50   CLAYSTONE,CHOCOLATEBROWN

607.00   3.00   2.99   607.00   3.00   2.99   CLAYSTONE,VARIEGATE                                                      83

610.00   3.00   2.98   610.00   3.00   2.98   CLAYSTONE,VARIEGATE                                                      39

613.00   3.00   2.90   613.00   3.00   2.90   CLAYSTONE,VARIEGATE                                                     8.7

616.00   3.00   2.99   616.00   3.00   2.99   CLAYSTONE,VARIEGATE                                                      31

619.00   2.20   2.20   619.00   2.20   2.20   CLAYSTONE,VARIEGATE

621.20    .20    .20   621.20    .20    .20   SHALE,GREYISH BROWN                                                      75

621.40    .60    .60   621.40    .60    .60   CLAYSTONE,VARIEGATE                                                      33

622.00   1.50   1.48   622.00   1.50   1.48   CLAYSTONE,VARIEGATE                                                      37

623.50   1.50   1.50   623.50   1.50   1.50   CLAYSTONE,VARIEGAT,SLICKENSIDED                                          54

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

625.00    .59    .55   625.00    .59    .55   CLAYSTONE,VARIEGAT,SLICKENSIDED

625.59    .28    .24   625.59    .28    .24   SHALE,BROWNISH GREY                                                      86

625.87   2.13   2.10   625.87   2.13   2.10   CLAYSTONE,CHOCOLATEBROWN

628.00   2.00   1.90   628.00   2.00   1.90   CLAYSTONE,CHOCOLATEBROWN                                                 28

630.00    .30    .30   630.00    .30    .30   SHALE,BROWNISH GREY                                                      40

630.30   1.20   1.20   630.30   1.20   1.20   CLAYSTONE,CHOCOLATEBROWN

631.50   2.50   2.48   631.50   2.50   2.48   CLAYSTONE,CHOCOLATEBROWN

634.00   3.00   2.98   634.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN

637.00    .57    .57   637.00    .57    .57   CLAYSTONE,CHOCOLATEBROWN

637.57   2.43    .92   637.57   2.43    .92   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              22

640.00   3.00   2.98   640.00   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              79

643.00   3.00   2.99   643.00   3.00   2.99   SANDSTONE(COARSE GRAINED),BROWNISH GREY                                  62

646.00    .57    .55   646.00    .57    .55   SANDSTONE(COARSE GRAINED),BROWNISH GREY                                  26

646.57   2.43   2.42   646.57   2.43   2.42   SANDSTONE(COARSE GRAINED),YELLOWISHBROWN,FRACTURE                       4.1

*******************************************************************************************************************************

BARAKAR

649.00    .22    .22   649.00    .22    .22   SANDY SHALE,GREY

649.22   2.78   2.78   649.22   2.78   2.78   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          86

652.00   3.00   2.99   652.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              92

655.00   2.00   1.99   655.00   2.00   1.99   SANDSTONE(MED. TO V.C. GRAINED),GREYISH GREEN                            68

657.00   3.00   2.99   657.00   3.00   2.99   SANDSTONE(MED. TO V.C. GRAINED),GREYISH GREEN                            76

660.00   3.00   2.99   660.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              92

663.00   1.40   1.40   663.00   1.40   1.40   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              70

664.40   1.30   1.30   664.40   1.30   1.30   MUDSTONE,GREY,FRACTURE

665.70    .30    .28   665.70    .30    .28   SANDY SHALE,GREY

666.00    .54    .48   666.00    .54    .48   SHALE,GREY,FRACTURE

666.54    .91    .85   666.54    .91    .85   SANDY SHALE,GREY,FRACTURE

667.45   1.55   1.50   667.45   1.55   1.50   SANDSTONE(FINE GRAINED),GREY                                             67

669.00   3.00   2.99   669.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              89

672.00   3.00   2.99   672.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              80

675.00   3.00   2.99   675.00   3.00   2.99   SANDSTONE(COARSE GRAINED),WHITE                                          77

678.00   3.00   2.99   678.00   3.00   2.99   SANDSTONE(COARSE GRAINED),WHITE                                          65

681.00   3.00   2.99   681.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          57

684.00   3.00   2.99   684.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  88

687.00   2.76   2.76   687.00   2.76   2.76   SANDSTONE(COARSE GRAINED),GREYISH WHITE                                  64

689.76    .24    .23   689.76    .24    .23   SANDY SHALE,GREY                                                         54

690.00    .11    .11   690.00    .11    .11   SANDY SHALE,GREY

S/W DEVELOPED BY :

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

690.11   2.89   2.89   690.11   2.89   2.89   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          73

693.00   3.00   3.00   693.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              87

696.00   3.00   3.00   696.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              95

699.00   1.00    .90   699.00   1.00    .90   SANDSTONE(FINE TO COARSE GRAINED),GREYISH WHITE                          52

700.00   3.50   3.50   700.00   3.50   3.50   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              75

703.50   2.00   2.00   703.50   2.00   2.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              83

705.50   3.00    .76   705.50   3.00    .76   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                             5.7

708.50   3.00   1.46   708.50   3.00   1.46   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              39

711.50   2.70   1.40   711.50   2.70   1.40   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              20

714.20   2.90   1.30   714.20   2.90   1.30   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              37

717.10   2.90   2.10   717.10   2.90   2.10   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              25

720.00   2.90   1.80   720.00   2.90   1.80   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              13

722.90   2.90   1.90   722.90   2.90   1.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              36

725.80    .38    .30   725.80    .38    .30   SHALY COAL                                                      IIA

726.18    .37    .30   726.18    .37    .30   COAL                                                            IIA

726.55    .38    .30   726.55    .38    .30   COAL                                                            IIA

726.93    .37    .30   726.93    .37    .30   COAL                                                            IIA

727.30   1.50    .70   727.30   1.50    .70   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          14

728.80   3.00    .90   728.80   3.00    .90   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          13

731.80    .75    .35   731.80    .75    .35   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE

732.55    .95    .60   732.55    .95    .60   INTERCALATIONS,GREY

733.50    .55    .48   733.50    .55    .48   CARB SHALE LOW                                                  L1

734.05   1.95   1.77   734.05   1.95   1.77   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

736.00   2.00   1.95   736.00   2.00   1.95   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

738.00    .10    .10   738.00    .10    .10   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

738.10    .80    .80   738.10    .80    .80   COAL                                                            IIB

738.90    .81    .80   738.90    .81    .80   COAL                                                            IIB

739.71    .23    .23   739.71    .23    .23   CARB SHALE LOW                                                  IIB

739.94    .38    .37   739.94    .38    .37   SHALY COAL                                                      IIB

740.32    .73    .72   740.32    .73    .72   SANDY SHALE

741.05    .85    .84   741.05    .85    .84   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

741.90   3.00   2.90   741.90   3.00   2.90   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

744.90   3.00   2.00   744.90   3.00   2.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

747.90   3.00   2.90   747.90   3.00   2.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE

750.90   3.00   2.95   750.90   3.00   2.95   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,

S/W DEVELOPED BY :

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

753.90   3.00   2.96   753.90   3.00   2.96   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,

756.90   3.00   2.90   756.90   3.00   2.90   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE,FRACTURE

759.90   3.00   2.95   759.90   3.00   2.95   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

762.90   3.00   2.88   762.90   3.00   2.88   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE

765.90   3.00   2.94   765.90   3.00   2.94   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE

768.90   3.00   2.98   768.90   3.00   2.98   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE,FRIABLE

771.90   2.30   2.30   771.90   2.30   2.30   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE

BOREHOLE CLOSED AT   774.20 M DEPTH

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

---------------------------------------------------------------------------

BOREHOLE NO    :CMWWB06              LATITUDE    : 2206377.896M                   DATE OF COMMENCEMENT: 20.03.2024

REDUCED LEVEL  : 258.7976M           DEPARTURE   : 269717.3688M                    DATE OF COMPLETION: 10.05.2024

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

.00   3.00   1.00      .00   3.00   1.00   SOIL,BLACK

SOIL

***************************************************************************************************************************

TRAP

3.00   3.00   1.00     3.00   3.00   1.00   BASALT,GREYISH BROWN

6.00   3.00   1.30     6.00   3.00   1.30   BASALT,GREYISH BROWN

9.00   3.00   1.10     9.00   3.00   1.10   BASALT,GREY

12.00   2.50    .70    12.00   2.50    .70   BASALT,GREY                                                              13

***************************************************************************************************************************

MOTUR

14.50   1.50    .60    14.50   1.50    .60   CLAY,YELLOWISHBROWN,SILTY

16.00   2.00    .90    16.00   2.00    .90   CLAY,YELLOWISHBROWN,SILTY

18.00   3.00    .63    18.00   3.00    .63   SANDSTONE(MED. TO C. GRAINED),GREY                                      6.7

21.00   3.00    .64    21.00   3.00    .64   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              16

24.00   2.00   1.05    24.00   2.00   1.05   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              23

26.00   1.00    .32    26.00   1.00    .32   CLAYSTONE,CHOCOLATEBROWN

27.00   3.00   2.70    27.00   3.00   2.70   SANDSTONE(FINE TO COARSE GRAINED),GREYISH WHITE                          42

30.00   3.00   2.00    30.00   3.00   2.00   CLAYSTONE,CHOCOLATEBROWN                                                 47

33.00   3.00   1.40    33.00   3.00   1.40   CLAYSTONE,CHOCOLATEBROWN                                                 37

36.00   1.00    .20    36.00   1.00    .20   CLAYSTONE,CHOCOLATEBROWN

37.00   2.00    .40    37.00   2.00    .40   SANDSTONE(MEDIUM GRAINED),YELLOW,FRACTURE                                 6

39.00   3.00    .50    39.00   3.00    .50   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN

42.00   3.00    .35    42.00   3.00    .35   SANDSTONE(MED. TO C. GRAINED),REDDISH BROWN                             6.7

45.00   3.00    .30    45.00   3.00    .30   CLAYSTONE,REDDISH BROWN

48.00   3.00    .35    48.00   3.00    .35   SANDSTONE(COARSE GRAINED),REDDISH BROWN

51.00   3.00    .46    51.00   3.00    .46   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN

54.00   3.00    .60    54.00   3.00    .60   CLAYSTONE,REDDISH BROWN                                                 7.3

57.00   3.00    .60    57.00   3.00    .60   CLAYSTONE,REDDISH BROWN                                                  20

60.00   3.00    .40    60.00   3.00    .40   CLAYSTONE,REDDISH BROWN

63.00   3.00    .40    63.00   3.00    .40   SANDSTONE(MED. TO C. GRAINED),REDDISH BROWN

66.00   3.00    .70    66.00   3.00    .70   CLAYSTONE,REDDISH BROWN                                                 9.7

69.00   3.00   1.10    69.00   3.00   1.10   CLAYSTONE,REDDISH BROWN                                                  16

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

72.00   3.00    .70    72.00   3.00    .70   CLAYSTONE,REDDISH BROWN

75.00   1.50   1.05    75.00   1.50   1.05   CLAYSTONE,REDDISH BROWN

76.50    .75    .55    76.50    .75    .55   CLAYSTONE,REDDISH BROWN

77.25    .75    .26    77.25    .75    .26   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN                                  13

78.00   3.00    .76    78.00   3.00    .76   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN                                  10

81.00   1.50    .31    81.00   1.50    .31   CLAYSTONE,REDDISH BROWN                                                  11

82.50   1.50    .25    82.50   1.50    .25   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN                                  11

84.00   3.00    .52    84.00   3.00    .52   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN                                   8

87.00   1.00    .50    87.00   1.00    .50   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN                                  13

88.00   2.00    .56    88.00   2.00    .56   SANDSTONE(FINE TO MEDIUM GRAINED),REDDISH BROWN

90.00   3.00    .60    90.00   3.00    .60   SANDSTONE(FINE TO MEDIUM GRAINED),REDDISH BROWN                           4

93.00   3.00    .70    93.00   3.00    .70   CLAYSTONE,REDDISH BROWN                                                 8.3

96.00   3.00    .74    96.00   3.00    .74   CLAYSTONE,REDDISH BROWN                                                  19

99.00   3.00    .79    99.00   3.00    .79   SANDSTONE(MED. TO C. GRAINED),REDDISH BROWN                             4.3

102.00   1.00    .28   102.00   1.00    .28   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    23

103.00   2.00    .65   103.00   2.00    .65   CLAYSTONE,REDDISH BROWN                                                  18

105.00   3.00    .80   105.00   3.00    .80   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN                                  12

108.00   3.00    .62   108.00   3.00    .62   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN                                  12

111.00   3.00    .70   111.00   3.00    .70   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN                                  11

114.00   3.00    .63   114.00   3.00    .63   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN                                  14

117.00   3.00    .80   117.00   3.00    .80   SANDSTONE(MEDIUM GRAINED),REDDISH BROWN                                 9.3

120.00   3.00    .75   120.00   3.00    .75   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

123.00   3.00    .82   123.00   3.00    .82   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            12

126.00   3.00   1.10   126.00   3.00   1.10   SANDSTONE(MED. TO C. GRAINED),YELLOWISHBROWN                             31

129.00   3.00   1.12   129.00   3.00   1.12   SANDSTONE(MED. TO C. GRAINED),YELLOWISHBROWN                             29

132.00   3.00    .98   132.00   3.00    .98   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            28

135.00   3.00    .90   135.00   3.00    .90   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            16

138.00   3.00   1.30   138.00   3.00   1.30   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            33

141.00   3.00   1.25   141.00   3.00   1.25   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            26

144.00   3.00    .82   144.00   3.00    .82   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            20

147.00   3.00    .90   147.00   3.00    .90   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            20

150.00   3.00   1.00   150.00   3.00   1.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            21

153.00   3.00    .85   153.00   3.00    .85   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                24

156.00   3.00   1.05   156.00   3.00   1.05   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                 9

159.00   3.00    .42   159.00   3.00    .42   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH YELLOW

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

162.00   3.00    .85   162.00   3.00    .85   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                25

165.00   3.00    .70   165.00   3.00    .70   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                23

168.00   3.00   1.00   168.00   3.00   1.00   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                27

171.00   3.00   1.05   171.00   3.00   1.05   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                31

174.00   3.00   1.60   174.00   3.00   1.60   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH YELLOW                         43

177.00   3.00   1.15   177.00   3.00   1.15   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH YELLOW                         36

180.00   3.00   1.30   180.00   3.00   1.30   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                34

183.00   3.00   2.60   183.00   3.00   2.60   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                66

186.00   3.00   1.95   186.00   3.00   1.95   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                47

189.00   3.00   1.20   189.00   3.00   1.20   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                33

192.00   3.00   2.13   192.00   3.00   2.13   CLAYSTONE,REDDISH BROWN                                                  57

195.00   3.00   1.86   195.00   3.00   1.86   CLAYSTONE,REDDISH BROWN                                                  56

198.00   3.00   2.35   198.00   3.00   2.35   CLAYSTONE,REDDISH BROWN                                                  41

201.00   3.00   1.90   201.00   3.00   1.90   CLAYSTONE,REDDISH BROWN                                                  21

204.00   3.00   1.45   204.00   3.00   1.45   CLAYSTONE,REDDISH BROWN                                                  25

207.00   3.00   1.55   207.00   3.00   1.55   CLAYSTONE,BROWNISH GREY                                                  36

210.00   3.00   1.20   210.00   3.00   1.20   CLAYSTONE,VARIEGATE                                                      17

213.00   1.00   1.00   213.00   1.00   1.00   CLAYSTONE,VARIEGATE                                                      89

214.00   2.00   1.00   214.00   2.00   1.00   CLAYSTONE,VARIEGATE                                                      15

216.00   3.00   1.95   216.00   3.00   1.95   CLAYSTONE,VARIEGATE                                                      25

219.00   2.00   1.00   219.00   2.00   1.00   CLAYSTONE,VARIEGATE                                                      16

221.00   1.00   1.00   221.00   1.00   1.00   MUDSTONE,GREY

222.00   3.00   2.05   222.00   3.00   2.05   MUDSTONE,GREY                                                             7

225.00   3.00   2.90   225.00   3.00   2.90   MUDSTONE,GREY                                                            30

228.00   3.00   2.40   228.00   3.00   2.40   MUDSTONE,GREY                                                            52

231.00   3.00   2.80   231.00   3.00   2.80   MUDSTONE,GREY,FRACTURE

234.00   1.00    .92   234.00   1.00    .92   MUDSTONE,GREY,FRACTURE

235.00   2.00   1.90   235.00   2.00   1.90   MUDSTONE,GREY,FRACTURE                                                   31

237.00   2.00   1.90   237.00   2.00   1.90   MUDSTONE,GREY,FRACTURE

239.00   3.00   2.90   239.00   3.00   2.90   MUDSTONE,GREYISH BROWN,FRACTURE

242.00   3.00   2.90   242.00   3.00   2.90   MUDSTONE,GREYISH BROWN,FRACTURE

245.00   3.00   2.98   245.00   3.00   2.98   MUDSTONE,VARIEGAT,SLICKENSIDED                                           66

248.00   1.00    .98   248.00   1.00    .98   MUDSTONE,GREY                                                            68

249.00   3.00   2.28   249.00   3.00   2.28   MUDSTONE,GREYISH BROWN                                                   20

252.00   3.00   2.90   252.00   3.00   2.90   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      20

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

255.00   2.00   2.00   255.00   2.00   2.00   CLAYSTONE,REDDISH BROWN,SLICKENSIDED

257.00   1.00   1.00   257.00   1.00   1.00   MUDSTONE,GREYISH BROWN

258.00   3.00   2.96   258.00   3.00   2.96   MUDSTONE,GREYISH BROWN                                                   55

261.00   1.50    .90   261.00   1.50    .90   MUDSTONE,VARIEGAT,FRACTURE                                               47

262.50   3.00   2.00   262.50   3.00   2.00   MUDSTONE,VARIEGATE                                                       14

265.50   3.00   2.40   265.50   3.00   2.40   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      72

268.50   1.00   1.00   268.50   1.00   1.00   MUDSTONE,GREYISH BROWN                                                   84

269.50   2.00   2.00   269.50   2.00   2.00   MUDSTONE,GREYISH BROWN,SLICKENSIDED

271.50   2.00   2.00   271.50   2.00   2.00   MUDSTONE,GREYISH BROWN,SLICKENSIDED

273.50    .44    .44   273.50    .44    .44   CLAYSTONE,REDDISH BROWN,SLICKENSIDED

273.94    .56    .56   273.94    .56    .56   MUDSTONE,GREYISH BROWN,SLICKENSIDED

274.50   1.50   1.50   274.50   1.50   1.50   MUDSTONE,GREYISH BROWN                                                   31

276.00   1.50   1.50   276.00   1.50   1.50   MUDSTONE,GREYISH BROWN                                                   94

277.50   1.28   1.28   277.50   1.28   1.28   MUDSTONE,GREYISH BROWN                                                   88

278.78   1.72   1.72   278.78   1.72   1.72   CLAYSTONE,VARIEGATE                                                      94

280.50   3.00   2.98   280.50   3.00   2.98   MUDSTONE,VARIEGATE                                                       79

283.50   2.00   2.00   283.50   2.00   2.00   MUDSTONE,GREYISH BROWN                                                   58

285.50   1.00    .99   285.50   1.00    .99   CLAYSTONE,REDDISH BROWN                                                  64

286.50   3.00   2.94   286.50   3.00   2.94   MUDSTONE,GREYISH BROWN                                                   83

289.50   3.00   3.00   289.50   3.00   3.00   MUDSTONE,VARIEGATE                                                       74

292.50    .80    .80   292.50    .80    .80   MUDSTONE,VARIEGATE                                                       98

293.30   2.20   2.20   293.30   2.20   2.20   CLAYSTONE,REDDISH BROWN                                                  50

295.50   2.00   2.00   295.50   2.00   2.00   CLAYSTONE,VARIEGATE                                                      84

297.50   1.00   1.00   297.50   1.00   1.00   CLAYSTONE,REDDISH BROWN                                                  30

298.50   3.00   3.00   298.50   3.00   3.00   MUDSTONE,GREYISH BROWN                                                   68

301.50   3.00   2.94   301.50   3.00   2.94   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      64

304.50   3.00   3.00   304.50   3.00   3.00   MUDSTONE,GREYISH BROWN                                                   77

307.50   3.00   3.00   307.50   3.00   3.00   MUDSTONE,GREYISH BROWN                                                   62

310.50   3.00   2.95   310.50   3.00   2.95   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      20

313.50   2.33   2.33   313.50   2.33   2.33   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      17

315.83    .67    .66   315.83    .67    .66   CLAYSTONE,REDDISH BROWN                                                  96

316.50   3.00   3.00   316.50   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  75

319.50   1.90   1.90   319.50   1.90   1.90   CLAYSTONE,REDDISH BROWN                                                  97

321.40   1.10   1.10   321.40   1.10   1.10   MUDSTONE,GREYISH BROWN                                                   74

322.50   3.00   3.00   322.50   3.00   3.00   MUDSTONE,GREYISH BROWN                                                   76

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

325.50   3.00   2.95   325.50   3.00   2.95   MUDSTONE,VARIEGAT,SLICKENSIDED                                           55

328.50   3.00   3.00   328.50   3.00   3.00   MUDSTONE,GREYISH BROWN                                                   27

331.50   3.00   2.34   331.50   3.00   2.34   MUDSTONE,VARIEGAT,FRACTURE                                               52

334.50   2.00   2.00   334.50   2.00   2.00   CLAYSTONE,REDDISH BROWN                                                  88

336.50   1.00    .80   336.50   1.00    .80   CLAYSTONE,REDDISH BROWN

337.50   3.00   2.55   337.50   3.00   2.55   CLAYSTONE,REDDISH BROWN                                                  49

340.50   3.00   3.00   340.50   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  88

343.50   3.00   3.00   343.50   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  87

346.50   3.00   3.00   346.50   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  73

349.50   3.00   3.00   349.50   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  86

352.50   3.00   3.00   352.50   3.00   3.00   MUDSTONE,GREYISH BROWN                                                   91

355.50   2.15   2.12   355.50   2.15   2.12   CLAYSTONE,REDDISH BROWN                                                  33

357.65    .85    .82   357.65    .85    .82   MUDSTONE,GREYISH BROWN

358.50   3.00   3.00   358.50   3.00   3.00   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     34

361.50   3.00   2.95   361.50   3.00   2.95   CLAYSTONE,VARIEGATE                                                      49

364.50   1.00    .96   364.50   1.00    .96   CLAYSTONE,REDDISH BROWN                                                  60

365.50   2.00   1.95   365.50   2.00   1.95   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     60

367.50   3.00   2.98   367.50   3.00   2.98   CLAYSTONE,VARIEGAT,SLICKENSIDED                                          31

370.50   2.00   1.93   370.50   2.00   1.93   CLAYSTONE,VARIEGATE                                                      29

372.50   1.00   1.00   372.50   1.00   1.00   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      53

373.50   3.00   3.00   373.50   3.00   3.00   MUDSTONE,VARIEGATE                                                       31

376.50   2.00   1.94   376.50   2.00   1.94   CLAYSTONE,REDDISH BROWN                                                  45

378.50   1.00    .98   378.50   1.00    .98   CLAYSTONE,REDDISH BROWN                                                  33

379.50   3.00   2.96   379.50   3.00   2.96   CLAYSTONE,VARIEGATE                                                      65

382.50   3.00   3.00   382.50   3.00   3.00   MUDSTONE,GREYISH BROWN                                                   48

385.50   1.00    .98   385.50   1.00    .98   MUDSTONE,GREYISH BROWN

386.50   2.00   1.94   386.50   2.00   1.94   MUDSTONE,GREYISH BROWN                                                   15

388.50   1.65   1.64   388.50   1.65   1.64   MUDSTONE,GREYISH BROWN                                                   24

390.15   1.35   1.34   390.15   1.35   1.34   CLAYSTONE,REDDISH BROWN                                                  19

391.50   2.00   1.95   391.50   2.00   1.95   CLAYSTONE,REDDISH BROWN                                                  38

393.50   1.00    .96   393.50   1.00    .96   CLAYSTONE,REDDISH BROWN                                                  43

394.50   3.00   2.95   394.50   3.00   2.95   CLAYSTONE,VARIEGATE                                                      55

397.50    .97    .95   397.50    .97    .95   MUDSTONE,GREYISH BROWN                                                   63

398.47   2.03   2.02   398.47   2.03   2.02   CLAYSTONE,VARIEGATE                                                      71

400.50   1.00    .95   400.50   1.00    .95   MUDSTONE,GREYISH BROWN,SLICKENSIDED

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

401.50    .80    .78   401.50    .80    .78   MUDSTONE,GREYISH BROWN,FRACTURE

402.30   1.20   1.15   402.30   1.20   1.15   CLAYSTONE,REDDISH BROWN                                                  19

403.50   3.00   3.00   403.50   3.00   3.00   CLAYSTONE,VARIEGATE                                                      18

406.50   2.00   2.00   406.50   2.00   2.00   MUDSTONE,VARIEGATE                                                       36

408.50   1.00    .98   408.50   1.00    .98   MUDSTONE,BROWNISH GREY                                                   53

409.50   2.00   2.00   409.50   2.00   2.00   CLAYSTONE,REDDISH BROWN                                                  46

411.50   2.00   1.93   411.50   2.00   1.93   CLAYSTONE,REDDISH BROWN                                                  58

413.50   1.00    .98   413.50   1.00    .98   CLAYSTONE,REDDISH BROWN                                                  63

414.50   1.00    .97   414.50   1.00    .97   CLAYSTONE,REDDISH BROWN                                                  59

415.50   3.00   2.99   415.50   3.00   2.99   CLAYSTONE,REDDISH BROWN                                                  25

418.50   3.00   2.98   418.50   3.00   2.98   CLAYSTONE,REDDISH BROWN                                                  20

421.50   3.00   2.95   421.50   3.00   2.95   CLAYSTONE,REDDISH BROWN                                                  34

424.50   3.00   2.99   424.50   3.00   2.99   CLAYSTONE,VARIEGATE                                                      72

427.50   1.00   1.00   427.50   1.00   1.00   CLAYSTONE,VARIEGATE                                                      63

428.50   2.00   2.00   428.50   2.00   2.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                         33

430.50   3.00   2.98   430.50   3.00   2.98   CLAYSTONE,VARIEGATE                                                      57

433.50   2.00   1.98   433.50   2.00   1.98   CLAYSTONE,VARIEGATE                                                      50

435.50   1.00   1.00   435.50   1.00   1.00   CLAYSTONE,REDDISH BROWN                                                  62

436.50   3.00   3.00   436.50   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  75

439.50   1.00   1.00   439.50   1.00   1.00   CLAYSTONE,VARIEGATE                                                      83

440.50   2.00   1.98   440.50   2.00   1.98   CLAYSTONE,VARIEGATE                                                      38

442.50   1.00    .98   442.50   1.00    .98   CLAYSTONE,REDDISH BROWN                                                  37

443.50    .14    .13   443.50    .14    .13   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE

443.64   1.86   1.84   443.64   1.86   1.84   CLAYSTONE,REDDISH BROWN                                                  71

445.50   1.10   1.10   445.50   1.10   1.10   CLAYSTONE,REDDISH BROWN                                                  73

446.60    .92    .92   446.60    .92    .92   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  88

447.52    .98    .98   447.52    .98    .98   CLAYSTONE,REDDISH BROWN

448.50   3.00   3.00   448.50   3.00   3.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                         39

451.50    .50    .50   451.50    .50    .50   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     52

452.00   2.50   2.50   452.00   2.50   2.50   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     51

454.50   1.60   1.60   454.50   1.60   1.60   MUDSTONE,GREYISH BROWN                                                   33

456.10    .80    .80   456.10    .80    .80   CLAYSTONE,REDDISH BROWN

456.90    .60    .60   456.90    .60    .60   MUDSTONE,GREYISH BROWN                                                   67

457.50    .78    .76   457.50    .78    .76   MUDSTONE,GREYISH BROWN                                                   64

458.28   2.22   2.20   458.28   2.22   2.20   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     33

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

460.50   1.40   1.38   460.50   1.40   1.38   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      22

461.90   1.60   1.54   461.90   1.60   1.54   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     34

463.50   3.00   2.15   463.50   3.00   2.15   CLAYSTONE,REDDISH BROWN                                                  26

466.50   1.50   1.40   466.50   1.50   1.40   CLAYSTONE,GREYISH BROWN,FRACTURE

468.00   3.00   2.05   468.00   3.00   2.05   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  56

471.00   3.00   2.60   471.00   3.00   2.60   CLAYSTONE,REDDISH BROWN                                                  27

474.00   1.50   1.50   474.00   1.50   1.50   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    58

475.50   1.50   1.50   475.50   1.50   1.50   CLAYSTONE,REDDISH BROWN                                                  77

477.00   1.50   1.50   477.00   1.50   1.50   CLAYSTONE,REDDISH BROWN                                                  45

478.50   1.50   1.50   478.50   1.50   1.50   CLAYSTONE,REDDISH BROWN                                                  65

480.00    .50    .48   480.00    .50    .48   SANDSTONE(MEDIUM GRAINED),BROWNISH PINK                                  90

480.50   1.00    .98   480.50   1.00    .98   CLAYSTONE,BROWNISH RED                                                   28

481.50   1.50   1.49   481.50   1.50   1.49   CLAYSTONE,BROWNISH RED                                                   59

483.00   1.50   1.45   483.00   1.50   1.45   CLAYSTONE,BROWNISH RED                                                   59

484.50    .13    .13   484.50    .13    .13   SANDSTONE(FINE GRAINED),BROWNISH PINK

484.63   1.00   1.00   484.63   1.00   1.00   CLAYSTONE,REDDISH BROWN                                                  48

485.63    .37    .37   485.63    .37    .37   SANDSTONE(FINE GRAINED),BROWNISH PINK                                    92

486.00   1.50   1.50   486.00   1.50   1.50   CLAYSTONE,VARIEGATE                                                      30

487.50   1.50   1.50   487.50   1.50   1.50   CLAYSTONE,REDDISH BROWN                                                  28

489.00   3.00   2.75   489.00   3.00   2.75   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     44

492.00   3.00   3.00   492.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  45

495.00   3.00   2.87   495.00   3.00   2.87   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     29

498.00   1.50   1.45   498.00   1.50   1.45   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     33

499.50   3.00   2.10   499.50   3.00   2.10   CLAYSTONE,REDDISH BROWN,FRACTURE

502.50   1.50   1.34   502.50   1.50   1.34   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     37

504.00   3.00   2.98   504.00   3.00   2.98   CLAYSTONE,REDDISH BROWN                                                  63

507.00   3.00   3.00   507.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  59

510.00   3.00   3.00   510.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  81

513.00   3.00   3.00   513.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                         43

516.00   3.00   3.00   516.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  61

519.00   3.00   3.00   519.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     30

522.00   3.00   3.00   522.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                         65

525.00   2.00   2.00   525.00   2.00   2.00   CLAYSTONE,REDDISH BROWN,FRACTURE HIGH ANGLE                              71

527.00   1.00   1.00   527.00   1.00   1.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                         18

528.00   3.00   2.98   528.00   3.00   2.98   CLAYSTONE,REDDISH BROWN                                                  41

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

531.00   3.00   2.94   531.00   3.00   2.94   CLAYSTONE,REDDISH BROWN,FRACTURE                                         65

534.00   3.00   2.99   534.00   3.00   2.99   CLAYSTONE,REDDISH BROWN                                                 4.7

537.00   1.70   1.70   537.00   1.70   1.70   CLAYSTONE,REDDISH BROWN                                                  14

538.70   1.30   1.30   538.70   1.30   1.30   MUDSTONE,GREYISH BROWN                                                   18

540.00   3.00   3.00   540.00   3.00   3.00   CLAYSTONE,VARIEGATE                                                      12

543.00   3.00   3.00   543.00   3.00   3.00   MUDSTONE,VARIEGAT,SLICKENSIDED                                           17

546.00   3.00   3.00   546.00   3.00   3.00   CLAYSTONE,VARIEGATE                                                      48

549.00   3.00   3.00   549.00   3.00   3.00   CLAYSTONE,VARIEGATE                                                      65

552.00   2.00   2.00   552.00   2.00   2.00   CLAYSTONE,VARIEGAT,SLICKENSIDED                                          19

554.00   1.00   1.00   554.00   1.00   1.00   CLAYSTONE,VARIEGATE                                                      48

555.00   3.00   2.99   555.00   3.00   2.99   CLAYSTONE,VARIEGATE                                                      40

558.00   3.00   3.00   558.00   3.00   3.00   CLAYSTONE,VARIEGAT,FRACTURE                                              37

561.00   1.50   1.39   561.00   1.50   1.39   CLAYSTONE,VARIEGAT,SLICKENSIDED                                          23

562.50   1.50   1.45   562.50   1.50   1.45   CLAYSTONE,VARIEGATE                                                      32

564.00   3.00   2.99   564.00   3.00   2.99   CLAYSTONE,REDDISH BROWN,FRACTURE                                         25

567.00   3.00   2.97   567.00   3.00   2.97   CLAYSTONE,REDDISH BROWN,FRACTURE                                         17

570.00   3.00   2.98   570.00   3.00   2.98   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     13

573.00   1.28   1.25   573.00   1.28   1.25   CLAYSTONE,REDDISH BROWN,FRACTURE

**************************************************************************************************************************

BARAKAR

574.28   1.72   1.70   574.28   1.72   1.70   SANDSTONE(MEDIUM GRAINED),MEDIUM CRYSTALLINE                             51

576.00   3.00   3.00   576.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          70

579.00   3.00   2.90   579.00   3.00   2.90   SANDSTONE(MED. TO C. GRAINED),AGGLOMERATE SIZE                           88

582.00   3.00   2.98   582.00   3.00   2.98   SANDSTONE(MEDIUM GRAINED),GREYISH GREEN                                  43

585.00   3.00   2.97   585.00   3.00   2.97   SANDSTONE(MEDIUM GRAINED),GREYISH GREEN                                  80

588.00   2.50   2.49   588.00   2.50   2.49   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          60

590.50   1.50   1.46   590.50   1.50   1.46   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          67

592.00   2.00   1.90   592.00   2.00   1.90   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN,                         51

594.00   3.00   2.92   594.00   3.00   2.92   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN,                         51

597.00   3.00   2.95   597.00   3.00   2.95   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              50

600.00   3.00   2.96   600.00   3.00   2.96   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              17

603.00    .68    .68   603.00    .68    .68   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN

603.68    .48    .48   603.68    .48    .48   SHALE,BROWNISH GREY

604.16   1.06   1.06   604.16   1.06   1.06   SHALE,GREY,SLICKENSIDED

605.22    .06    .06   605.22    .06    .06   CARB SHALE LOW                                                  L

605.28    .72    .72   605.28    .72    .72   CLAY,BROWNISH GREY

606.00    .66    .64   606.00    .66    .64   CLAY,BROWNISH GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

606.66    .49    .48   606.66    .49    .48   SANDY SHALE,GREY

607.15    .82    .80   607.15    .82    .80   SHALE,GREY,SLICKENSIDED

607.97    .82    .80   607.97    .82    .80   SANDSTONE(FINE TO COARSE GRAINED),GREYISH WHITE                         0.7

608.79    .21    .20   608.79    .21    .20   SHALE,GREY,FRACTURE

609.00   3.00   2.96   609.00   3.00   2.96   SANDSTONE(MED. TO C. GRAINED),BROWN,FRACTURE                             46

612.00    .12    .09   612.00    .12    .09   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE,FRACTURE

612.12   1.00    .85   612.12   1.00    .85   CLAY,GREYISH BROWN,FRACTURE

613.12   1.40   1.20   613.12   1.40   1.20   SHALE,GREY,FRACTURE                                                     2.8

614.52    .48    .45   614.52    .48    .45   CLAY,GREYISH BROWN,FRACTURE

615.00    .77    .72   615.00    .77    .72   SHALE,GREY,FRACTURE

615.77    .10    .08   615.77    .10    .08   CARB SHALE LOW                                                  L

615.87    .35    .28   615.87    .35    .28   SHALE,GREY                                                               80

616.22   1.78   1.60   616.22   1.78   1.60   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                 5.6

618.00   3.00   2.65   618.00   3.00   2.65   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  34

621.00   1.50   1.40   621.00   1.50   1.40   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              56

622.50   1.10   1.00   622.50   1.10   1.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              37

623.60   3.00   2.80   623.60   3.00   2.80   SANDSTONE(COARSE GRAINED),GREYISH WHITE,FRACTURE                         59

626.60   3.00   2.98   626.60   3.00   2.98   SANDSTONE(COARSE GRAINED),GREYISH WHITE                                  71

629.60   3.00   2.99   629.60   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     68

632.60   2.51   2.50   632.60   2.51   2.50   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     75

635.11    .49    .48   635.11    .49    .48   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              53

635.60   3.00   2.95   635.60   3.00   2.95   SANDSTONE(FINE TO COARSE GRAINED),GREYISH WHITE,                         59

638.60   2.90   2.88   638.60   2.90   2.88   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          67

641.50   2.90   2.85   641.50   2.90   2.85   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  26

644.40   2.90   2.80   644.40   2.90   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                         9.7

647.30   2.21   2.20   647.30   2.21   2.20   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  26

649.51    .42    .40   649.51    .42    .40   COAL,FRACTURE                                                   IIA

649.93    .27    .26   649.93    .27    .26   SHALY COAL                                                      IIA

650.20    .20    .20   650.20    .20    .20   SHALY COAL                                                      IIA

650.40    .80    .80   650.40    .80    .80   COAL,FRACTURE                                                   IIA

651.20    .77    .77   651.20    .77    .77   COAL                                                            IIA

651.97    .53    .53   651.97    .53    .53   SANDSTONE(FINE GRAINED),GREYISH WHITE

652.50   1.08   1.06   652.50   1.08   1.06   SANDSTONE(FINE GRAINED),GREYISH WHITE

653.58    .80    .78   653.58    .80    .78   INTERCALATIONS,GREY,FRACTURE                                             76

654.38    .35    .34   654.38    .35    .34   SHALE,GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

654.73    .77    .75   654.73    .77    .75   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  19

655.50   3.00   2.98   655.50   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     48

658.50   3.00   2.99   658.50   3.00   2.99   SANDSTONE(FINE TO COARSE GRAINED),GREYISH WHITE,                         60

661.50   2.50   2.50   661.50   2.50   2.50   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              43

664.00    .50    .50   664.00    .50    .50   COAL                                                            IIB

664.50    .25    .25   664.50    .25    .25   COAL                                                            IIB

664.75    .25    .25   664.75    .25    .25   SHALY COAL                                                      IIB

665.00    .21    .21   665.00    .21    .21   SANDY SHALE,GREY

665.21   2.29   2.28   665.21   2.29   2.28   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              90

667.50   3.00   2.98   667.50   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              88

670.50   2.90   2.88   670.50   2.90   2.88   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              85

673.40   2.90   2.90   673.40   2.90   2.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              81

676.30    .80    .80   676.30    .80    .80   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              90

677.10    .30    .30   677.10    .30    .30   COAL                                                            I

677.40    .22    .22   677.40    .22    .22   COAL                                                            I

677.62   1.03   1.03   677.62   1.03   1.03   INTERCALATIONS,GREY                                                      46

678.65    .55    .55   678.65    .55    .55   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    76

679.20   2.90   2.90   679.20   2.90   2.90   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    78

682.10   1.50   1.50   682.10   1.50   1.50   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    83

BOREHOLE CLOSED AT   683.60 M DEPTH

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

BOREHOLE NO    :CMWWB07              LATITUDE    : 2212725.505M                    DATE OF COMMENCEMENT: 18.04.2024

REDUCED LEVEL  : 290.59M             DEPARTURE   : 264820.0445M                    DATE OF COMPLETION: 03.06.2024

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

.00   2.00    .10      .00   2.00    .10   SOIL,BROWN

2.00   1.00    .12     2.00   1.00    .12   SOIL,BROWNISH BLACK,SANDY,FRIABLE

3.00   3.00    .15     3.00   3.00    .15   SOIL,BROWNISH GREY,FRACTURE

SOIL

TRAP

6.00    .25    .25     6.00    .25    .25   BASALT,BROWNISH GREY

6.25    .50    .50     6.25    .50    .50   BASALT,BROWNISH GREY,FRACTURE

6.75   1.75   1.73     6.75   1.75   1.73   BASALT,GREY,FRACTURE                                                     57

8.50   3.00   2.95     8.50   3.00   2.95   BASALT,GREY,FRACTURE                                                     66

11.50   3.00   2.96    11.50   3.00   2.96   BASALT,GREY,FRACTURE                                                     52

14.50   3.00   3.00    14.50   3.00   3.00   BASALT,GREY,FRACTURE                                                     60

17.50   3.00   3.00    17.50   3.00   3.00   BASALT,GREY,FRACTURE                                                     84

20.50   3.00   3.00    20.50   3.00   3.00   BASALT,GREY,FRACTURE                                                     87

23.50   3.00   3.00    23.50   3.00   3.00   BASALT,GREY                                                              95

26.50   3.00   3.00    26.50   3.00   3.00   BASALT,GREY                                                              98

29.50   3.00   3.00    29.50   3.00   3.00   BASALT,GREY,FRACTURE                                                     89

32.50   3.00   3.00    32.50   3.00   3.00   BASALT,GREY,FRACTURE                                                     67

35.50   1.50   1.50    35.50   1.50   1.50   BASALT,GREY,FRACTURE                                                     50

37.00    .60    .60    37.00    .60    .60   BASALT,GREYISH GREEN                                                     23

37.60    .50    .50    37.60    .50    .50   CLAY,REDDISH GREEN

38.10    .90    .90    38.10    .90    .90   BASALT,GREYISH GREEN

39.00   3.00   3.00    39.00   3.00   3.00   BASALT,GREY                                                              80

42.00   3.00   3.00    42.00   3.00   3.00   BASALT,GREY,FRACTURE                                                     41

45.00    .88    .80    45.00    .88    .80   BASALT,GREY,FRACTURE                                                     90

45.88   2.12   1.95    45.88   2.12   1.95   BASALT,REDDISH BROWN

****************************************************************************************************************************

MOTU

48.00   3.00   2.98    48.00   3.00   2.98   CLAYSTONE,REDDISH BROWN,FRACTURE

51.00   3.00   2.70    51.00   3.00   2.70   CLAYSTONE,REDDISH BROWN,FRIABLE

54.00   3.00   2.10    54.00   3.00   2.10   CLAYSTONE,RED

57.00   3.00   2.95    57.00   3.00   2.95   CLAYSTONE,REDDISH BROWN

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

60.00   3.00   1.65    60.00   3.00   1.65   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    16

63.00   3.00   1.25    63.00   3.00   1.25   SANDSTONE(FINE TO MEDIUM GRAINED),REDDISH BROWN,                         30

66.00   3.00   1.00    66.00   3.00   1.00   CLAYSTONE,REDDISH BROWN,FRIABLE

69.00   3.00    .41    69.00   3.00    .41   SANDSTONE(FINE GRAINED),REDDISH BROWN,FRACTURE

72.00   3.00    .36    72.00   3.00    .36   SANDSTONE(FINE GRAINED),REDDISH BROWN

75.00   3.00    .35    75.00   3.00    .35   SANDSTONE(FINE GRAINED),REDDISH BROWN

78.00   3.00    .45    78.00   3.00    .45   SANDSTONE(FINE GRAINED),REDDISH BROWN,FRIABLE

81.00   3.00    .35    81.00   3.00    .35   CLAYSTONE,REDDISH BROWN,FRIABLE

84.00   3.00    .30    84.00   3.00    .30   CLAYSTONE,REDDISH BROWN,FRIABLE

87.00   3.00    .30    87.00   3.00    .30   CLAYSTONE,REDDISH BROWN

90.00   3.00    .20    90.00   3.00    .20   CLAYSTONE,REDDISH BROWN

93.00   3.00    .50    93.00   3.00    .50   CLAYSTONE,REDDISH BROWN

96.00   3.00    .65    96.00   3.00    .65   SANDSTONE(FINE GRAINED),GREY,FRIABLE

99.00   1.20    .90    99.00   1.20    .90   CLAYSTONE,REDDISH BROWN                                                  44

100.20   1.80   1.40   100.20   1.80   1.40   SANDSTONE(FINE GRAINED),YELLOWISHGREY,FRIABLE                            10

102.00    .78    .77   102.00    .78    .77   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOW

102.78   2.22   2.20   102.78   2.22   2.20   CLAYSTONE,REDDISH BROWN,FRACTURE                                         59

105.00    .60    .30   105.00    .60    .30   CLAYSTONE,REDDISH BROWN

105.60   2.40   1.00   105.60   2.40   1.00   SANDSTONE(FINE GRAINED),YELLOW,FRIABLE

108.00   3.00   3.00   108.00   3.00   3.00   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW,FRIABLE                        41

111.00   3.00   2.10   111.00   3.00   2.10   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY,FRACTURE                     41

114.00   3.00   3.00   114.00   3.00   3.00   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                36

117.00   3.00   3.00   117.00   3.00   3.00   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW                                78

120.00   3.00   3.00   120.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH PINK                          87

123.00   3.00   3.00   123.00   3.00   3.00   SANDSTONE(FINE GRAINED),BROWN                                            71

126.00   3.00   2.98   126.00   3.00   2.98   SANDSTONE(FINE GRAINED),BROWNISH GREY                                    25

129.00   3.00   2.95   129.00   3.00   2.95   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          43

132.00   3.00   2.65   132.00   3.00   2.65   SANDSTONE(FINE GRAINED),BROWNISH PINK,FRACTURE                           55

135.00   3.00   2.45   135.00   3.00   2.45   SANDSTONE(FINE GRAINED),BROWNISH GREY,SLICKENSIDED                       52

138.00   3.00   2.98   138.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY,                         45

141.00   3.00   2.78   141.00   3.00   2.78   SANDSTONE(MED. TO C. GRAINED),BROWN                                      27

144.00   3.00   2.50   144.00   3.00   2.50   SANDSTONE(COARSE GRAINED),VARIEGATE                                      44

147.00    .70    .60   147.00    .70    .60   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                71

147.70    .88    .75   147.70    .88    .75   CLAYSTONE,REDDISH BROWN                                                  60

148.58   1.42   1.30   148.58   1.42   1.30   SANDSTONE(COARSE GRAINED),BROWN                                          68

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

150.00   2.00    .80   150.00   2.00    .80   SANDSTONE(COARSE GRAINED),BROWNISH GREY,FRACTURE

152.00   2.45   2.40   152.00   2.45   2.40   SANDSTONE(COARSE GRAINED),BROWNISH GREY,FRIABLE                          30

154.45    .55    .50   154.45    .55    .50   CLAYSTONE,REDDISH BROWN

155.00   3.00   3.00   155.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),BROWN,FRACTURE                         34

158.00   3.00   3.00   158.00   3.00   3.00   SANDSTONE(FINE GRAINED),BROWN,FRACTURE                                   55

161.00   1.00   1.00   161.00   1.00   1.00   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY,                         20

162.00   1.70   1.68   162.00   1.70   1.68   SANDSTONE(FINE GRAINED),BROWNISH BUFF,FRACTURE                           62

163.70   1.30   1.28   163.70   1.30   1.28   CLAYSTONE,REDDISH BROWN                                                  70

165.00   3.00   2.95   165.00   3.00   2.95   SANDSTONE(FINE GRAINED),REDDISH PINK,FRACTURE                            19

168.00    .50    .40   168.00    .50    .40   SANDSTONE(FINE GRAINED),BROWN

168.50   2.50   1.86   168.50   2.50   1.86   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              22

171.00   3.00   2.95   171.00   3.00   2.95   SANDSTONE(MED. TO C. GRAINED),BROWNISH PINK,FRIABLE

174.00   3.00   1.00   174.00   3.00   1.00   SANDSTONE(FINE GRAINED),BROWNISH GREY,FRIABLE

177.00   3.00    .65   177.00   3.00    .65   SANDSTONE(FINE GRAINED),BROWNISH GREY,FRIABLE

180.00   3.00   1.00   180.00   3.00   1.00   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          24

183.00   3.00   1.27   183.00   3.00   1.27   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          26

186.00   3.00   1.35   186.00   3.00   1.35   SANDSTONE(FINE GRAINED),BROWNISH GREY,FRIABLE

189.00   3.00   1.65   189.00   3.00   1.65   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          45

192.00   3.00   1.40   192.00   3.00   1.40   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  42

195.00   3.00   3.00   195.00   3.00   3.00   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  88

198.00   3.00   3.00   198.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              80

201.00   3.00   3.00   201.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              73

204.00   3.00   3.00   204.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              48

207.00   3.00   2.95   207.00   3.00   2.95   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          69

210.00   3.00   3.00   210.00   3.00   3.00   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  78

213.00   3.00   1.40   213.00   3.00   1.40   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  29

216.00    .90    .80   216.00    .90    .80   CLAYSTONE,REDDISH BROWN                                                  67

216.90   2.10   2.00   216.90   2.10   2.00   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE,FRACTURE                         62

219.00   3.00   2.95   219.00   3.00   2.95   CLAYSTONE,REDDISH BROWN                                                  64

222.00   3.00   2.80   222.00   3.00   2.80   CLAYSTONE,REDDISH BROWN                                                  69

225.00   3.00   2.60   225.00   3.00   2.60   CLAYSTONE,REDDISH BROWN,FRACTURE                                         21

228.00   1.10    .95   228.00   1.10    .95   CLAYSTONE,REDDISH BROWN,FRACTURE                                         62

229.10   1.90   1.60   229.10   1.90   1.60   MUDSTONE,GREYISH BROWN                                                   34

231.00   3.00   2.60   231.00   3.00   2.60   MUDSTONE,GREYISH BROWN                                                   22

234.00   3.00   2.62   234.00   3.00   2.62   MUDSTONE,GREYISH BROWN                                                   19

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

237.00   3.00   1.80   237.00   3.00   1.80   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      15

240.00    .35    .27   240.00    .35    .27   MUDSTONE,GREYISH BROWN

240.35   2.65   2.40   240.35   2.65   2.40   SANDSTONE(FINE GRAINED),GREYISH BROWN                                    62

243.00   2.00   1.00   243.00   2.00   1.00   MUDSTONE,GREYISH BROWN,SLICKENSIDED

245.00   1.00    .88   245.00   1.00    .88   MUDSTONE,GREYISH BROWN

246.00   3.00   1.88   246.00   3.00   1.88   CLAYSTONE,VARIEGATE                                                      51

249.00   3.00   1.12   249.00   3.00   1.12   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    32

252.00   3.00   1.80   252.00   3.00   1.80   MUDSTONE,GREYISH BROWN,FRACTURE

255.00   1.00   1.00   255.00   1.00   1.00   MUDSTONE,GREYISH BROWN                                                   35

256.00   2.00   2.00   256.00   2.00   2.00   MUDSTONE,GREYISH BROWN                                                   45

258.00   3.00   3.00   258.00   3.00   3.00   MUDSTONE,GREYISH BROWN,FRACTURE

261.00   3.00   2.95   261.00   3.00   2.95   MUDSTONE,VARIEGATE                                                       89

264.00   2.50   2.45   264.00   2.50   2.45   MUDSTONE,GREYISH BROWN,FRACTURE                                          27

266.50   3.00   3.00   266.50   3.00   3.00   MUDSTONE,GREYISH BROWN                                                   66

269.50   3.00   2.99   269.50   3.00   2.99   MUDSTONE,GREYISH BROWN                                                   17

272.50   1.50   1.45   272.50   1.50   1.45   MUDSTONE,GREYISH BROWN,FRACTURE

274.00   1.00   1.00   274.00   1.00   1.00   MUDSTONE,GREYISH BROWN,FRACTURE

275.00   2.28   2.23   275.00   2.28   2.23   MUDSTONE,GREYISH BROWN,SLICKENSIDED

277.28    .22    .15   277.28    .22    .15   SANDSTONE(FINE GRAINED),GREYISH WHITE

277.50    .50    .50   277.50    .50    .50   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    92

278.00   2.00   1.98   278.00   2.00   1.98   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    94

280.00   3.00   3.00   280.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              81

283.00   3.00   3.00   283.00   3.00   3.00   MUDSTONE,GREYISH BROWN                                                   78

286.00   1.50   1.40   286.00   1.50   1.40   MUDSTONE,GREYISH BROWN                                                   15

287.50    .50    .45   287.50    .50    .45   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      30

288.00   3.00   3.00   288.00   3.00   3.00   MUDSTONE,GREYISH BROWN,SLICKENSIDED

291.00   3.00   2.95   291.00   3.00   2.95   MUDSTONE,GREYISH BROWN,SLICKENSIDED

294.00   1.59   1.57   294.00   1.59   1.57   MUDSTONE,GREYISH BROWN                                                   64

295.59   1.41   1.40   295.59   1.41   1.40   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  73

297.00    .40    .40   297.00    .40    .40   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  65

297.40    .26    .25   297.40    .26    .25   MUDSTONE,GREYISH BROWN

297.66   2.34   2.34   297.66   2.34   2.34   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          68

300.00   3.00   2.85   300.00   3.00   2.85   MUDSTONE,GREYISH BROWN,FRACTURE                                          21

303.00   2.00   2.00   303.00   2.00   2.00   MUDSTONE,GREYISH BROWN                                                   25

305.00   1.00    .98   305.00   1.00    .98   MUDSTONE,GREYISH BROWN

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

306.00   2.09   2.08   306.00   2.09   2.08   MUDSTONE,GREYISH BROWN                                                   43

308.09    .91    .90   308.09    .91    .90   CLAYSTONE,REDDISH BROWN,FRACTURE                                         20

309.00   2.00   2.00   309.00   2.00   2.00   CLAYSTONE,REDDISH BROWN                                                  33

311.00   3.00   2.95   311.00   3.00   2.95   MUDSTONE,GREYISH BROWN                                                   67

314.00   1.50   1.45   314.00   1.50   1.45   MUDSTONE,GREYISH BROWN                                                   82

315.50   3.00   2.99   315.50   3.00   2.99   MUDSTONE,GREYISH BROWN                                                   36

318.50   1.00   1.00   318.50   1.00   1.00   MUDSTONE,GREYISH BROWN                                                   50

319.50   2.00   2.00   319.50   2.00   2.00   MUDSTONE,GREY                                                            26

321.50   3.00   2.45   321.50   3.00   2.45   MUDSTONE,GREYISH BROWN,FRACTURE                                         8.3

324.50   3.00   3.00   324.50   3.00   3.00   MUDSTONE,GREYISH BROWN                                                   36

327.50   3.00   2.98   327.50   3.00   2.98   MUDSTONE,GREYISH BROWN                                                   20

330.50   3.00   2.99   330.50   3.00   2.99   MUDSTONE,GREYISH BROWN,FRACTURE                                          41

333.50    .50    .50   333.50    .50    .50   MUDSTONE,GREYISH BROWN,FRACTURE                                          30

334.00   2.50   2.48   334.00   2.50   2.48   MUDSTONE,GREYISH BROWN                                                   44

336.50   1.50   1.50   336.50   1.50   1.50   MUDSTONE,GREYISH BROWN                                                   53

338.00   3.00   3.00   338.00   3.00   3.00   MUDSTONE,VARIEGAT,FRACTURE                                              5.7

341.00   2.00   2.00   341.00   2.00   2.00   MUDSTONE,VARIEGATE                                                       18

343.00   1.38   1.36   343.00   1.38   1.36   MUDSTONE,GREYISH BROWN

344.38   1.62   1.60   344.38   1.62   1.60   CLAYSTONE,REDDISH BROWN

346.00    .38    .37   346.00    .38    .37   MUDSTONE,REDDISH BROWN

346.38    .76    .74   346.38    .76    .74   SANDSTONE(FINE GRAINED),GREY                                             78

347.14   1.62   1.60   347.14   1.62   1.60   CLAYSTONE,REDDISH BROWN                                                  57

348.76    .24    .23   348.76    .24    .23   SANDSTONE(FINE GRAINED),GREY                                             96

349.00   2.00   2.00   349.00   2.00   2.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   56

351.00   1.00   1.00   351.00   1.00   1.00   MUDSTONE,GREYISH BROWN                                                   38

352.00    .50    .46   352.00    .50    .46   MUDSTONE,GREYISH BROWN

352.50   2.50   2.26   352.50   2.50   2.26   MUDSTONE,GREYISH BROWN                                                   52

355.00   3.00   3.00   355.00   3.00   3.00   MUDSTONE,GREYISH BROWN                                                   34

358.00   2.84   2.84   358.00   2.84   2.84   MUDSTONE,VARIEGATE

360.84    .16    .16   360.84    .16    .16   CLAYSTONE,REDDISH BROWN

361.00   1.20    .64   361.00   1.20    .64   MUDSTONE,GREYISH BROWN                                                   28

362.20    .80    .75   362.20    .80    .75   MUDSTONE,GREYISH BROWN                                                   63

363.00   3.00   2.95   363.00   3.00   2.95   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      58

366.00   3.00   3.00   366.00   3.00   3.00   MUDSTONE,VARIEGATE                                                       62

369.00   2.00   1.88   369.00   2.00   1.88   CLAYSTONE,GREYISH BROWN                                                  38

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

371.00   3.00   3.00   371.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  18

374.00   1.00    .86   374.00   1.00    .86   CLAYSTONE,REDDISH BROWN,FRACTURE                                         49

375.00   3.00   3.00   375.00   3.00   3.00   CLAYSTONE,VARIEGAT,FRACTURE                                              62

378.00   1.00    .98   378.00   1.00    .98   CLAYSTONE,GREYISH BROWN

379.00   3.00   3.00   379.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  48

382.00   3.00   3.00   382.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                         34

385.00   2.00   2.00   385.00   2.00   2.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                         43

387.00   1.00   1.00   387.00   1.00   1.00   CLAYSTONE,REDDISH BROWN                                                  18

388.00   1.00    .98   388.00   1.00    .98   MUDSTONE,GREYISH BROWN                                                   57

389.00   1.50   1.50   389.00   1.50   1.50   CLAYSTONE,REDDISH BROWN                                                  71

390.50   2.50   2.50   390.50   2.50   2.50   MUDSTONE,VARIEGATE                                                       50

393.00   2.10   2.10   393.00   2.10   2.10   MUDSTONE,VARIEGATE                                                       61

395.10    .90    .90   395.10    .90    .90   SANDSTONE(FINE GRAINED),GREY                                             90

396.00   3.00   3.00   396.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  79

399.00   3.00   3.00   399.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  83

402.00   3.00   3.00   402.00   3.00   3.00   CLAYSTONE,VARIEGATE                                                      91

405.00   3.00   3.00   405.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  55

408.00   3.00   3.00   408.00   3.00   3.00   MUDSTONE,VARIEGAT,SLICKENSIDED                                           36

411.00   3.00   3.00   411.00   3.00   3.00   MUDSTONE,VARIEGATE                                                       51

414.00    .86    .86   414.00    .86    .86   CLAYSTONE,GREYISH BROWN                                                  47

414.86    .17    .16   414.86    .17    .16   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  94

415.03   1.97   1.96   415.03   1.97   1.96   MUDSTONE,VARIEGATE                                                       93

417.00   2.00   2.00   417.00   2.00   2.00   MUDSTONE,GREYISH BROWN                                                   78

419.00   1.00   1.00   419.00   1.00   1.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                         73

420.00   3.00   3.00   420.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  64

423.00    .14    .14   423.00    .14    .14   CLAYSTONE,REDDISH BROWN

423.14    .42    .42   423.14    .42    .42   SANDSTONE(FINE GRAINED),GREY                                             48

423.56    .68    .68   423.56    .68    .68   MUDSTONE,GREYISH BROWN                                                   40

424.24   1.76   1.76   424.24   1.76   1.76   CLAYSTONE,REDDISH BROWN,FRACTURE                                         44

426.00   3.00   3.00   426.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                         39

429.00   3.00   3.00   429.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  59

432.00   1.14   1.14   432.00   1.14   1.14   CLAYSTONE,REDDISH BROWN                                                  25

433.14    .75    .75   433.14    .75    .75   SANDSTONE(FINE GRAINED),GREY                                             84

433.89   1.11   1.11   433.89   1.11   1.11   CLAYSTONE,REDDISH GREY                                                   53

435.00   2.00   2.00   435.00   2.00   2.00   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     47

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

437.00    .48    .48   437.00    .48    .48   CLAYSTONE,REDDISH BROWN                                                  56

437.48    .52    .52   437.48    .52    .52   MUDSTONE,REDDISH BROWN                                                   90

438.00    .60    .52   438.00    .60    .52   MUDSTONE,REDDISH BROWN                                                   80

438.60    .80    .75   438.60    .80    .75   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   90

439.40    .50    .46   439.40    .50    .46   MUDSTONE,GREYISH BROWN                                                   30

439.90   1.10    .95   439.90   1.10    .95   CLAYSTONE,REDDISH BROWN                                                  35

441.00   3.00   2.70   441.00   3.00   2.70   CLAYSTONE,REDDISH BROWN                                                  32

444.00   3.00   2.90   444.00   3.00   2.90   CLAYSTONE,REDDISH BROWN                                                  39

447.00    .88    .85   447.00    .88    .85   CLAYSTONE,REDDISH BROWN                                                  82

447.88    .46    .45   447.88    .46    .45   SANDSTONE(FINE GRAINED),GREY                                             87

448.34    .16    .15   448.34    .16    .15   CLAYSTONE,REDDISH BROWN                                                  81

448.50    .24    .24   448.50    .24    .24   CLAYSTONE,REDDISH BROWN

448.74    .35    .35   448.74    .35    .35   SANDSTONE(MEDIUM GRAINED),BROWN

449.09    .91    .90   449.09    .91    .90   MUDSTONE,GREYISH BROWN                                                   63

450.00   3.00   3.00   450.00   3.00   3.00   MUDSTONE,GREYISH BROWN,FRACTURE                                          22

453.00   3.00   3.00   453.00   3.00   3.00   MUDSTONE,VARIEGAT,FRACTURE                                               21

456.00    .56    .55   456.00    .56    .55   MUDSTONE,GREYISH BROWN                                                   75

456.56   1.63   1.60   456.56   1.63   1.60   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   80

458.19    .81    .80   458.19    .81    .80   CLAYSTONE,REDDISH BROWN,FRACTURE                                         30

459.00    .65    .65   459.00    .65    .65   CLAYSTONE,REDDISH BROWN                                                  40

459.65    .50    .50   459.65    .50    .50   SANDSTONE(FINE GRAINED),GREY                                             34

460.15    .35    .35   460.15    .35    .35   CLAYSTONE,REDDISH BROWN

460.50   1.50   1.50   460.50   1.50   1.50   CLAYSTONE,REDDISH BROWN                                                  47

462.00   2.65   2.65   462.00   2.65   2.65   CLAYSTONE,REDDISH BROWN                                                  83

464.65    .35    .35   464.65    .35    .35   MUDSTONE,GREYISH BROWN                                                   83

465.00   1.70   1.70   465.00   1.70   1.70   MUDSTONE,GREYISH BROWN,FRACTURE                                          31

466.70    .60    .60   466.70    .60    .60   SANDSTONE(FINE GRAINED),GREY                                             95

467.30    .70    .70   467.30    .70    .70   MUDSTONE,GREYISH BROWN                                                   61

468.00   1.60   1.60   468.00   1.60   1.60   CLAYSTONE,REDDISH BROWN                                                  46

469.60   1.40   1.40   469.60   1.40   1.40   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   43

471.00   2.30   2.30   471.00   2.30   2.30   CLAYSTONE,VARIEGATE                                                      79

473.30    .70    .70   473.30    .70    .70   SANDSTONE(MEDIUM GRAINED),GREY                                           99

474.00   2.00   2.00   474.00   2.00   2.00   SANDSTONE(MEDIUM GRAINED),GREY                                           84

476.00   1.00   1.00   476.00   1.00   1.00   CLAYSTONE,GREYISH BROWN                                                  90

477.00   1.80   1.80   477.00   1.80   1.80   CLAYSTONE,VARIEGATE                                                      86

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

478.80    .80    .80   478.80    .80    .80   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   89

479.60    .40    .40   479.60    .40    .40   MUDSTONE,GREYISH BROWN                                                   70

480.00   3.00   3.00   480.00   3.00   3.00   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      55

483.00   3.00   3.00   483.00   3.00   3.00   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      11

486.00   3.00   3.00   486.00   3.00   3.00   MUDSTONE,VARIEGAT,SLICKENSIDED                                          9.7

489.00   3.00   2.97   489.00   3.00   2.97   MUDSTONE,VARIEGAT,SLICKENSIDED

492.00   3.00   3.00   492.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  40

495.00   1.68   1.68   495.00   1.68   1.68   CLAYSTONE,REDDISH BROWN                                                  76

496.68    .32    .32   496.68    .32    .32   MUDSTONE,GREYISH BROWN                                                   47

497.00   1.00   1.00   497.00   1.00   1.00   SANDSTONE(FINE GRAINED),GREY                                             66

498.00    .40    .40   498.00    .40    .40   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   45

498.40    .30    .30   498.40    .30    .30   MUDSTONE,GREYISH BROWN                                                   57

498.70   2.30   2.30   498.70   2.30   2.30   CLAYSTONE,REDDISH BROWN                                                  74

501.00   3.00   3.00   501.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,DISTINCT TOP                                     61

504.00   3.00   2.97   504.00   3.00   2.97   CLAYSTONE,REDDISH BROWN                                                  63

507.00   3.00   3.00   507.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,DISTINCT TOP                                     69

510.00   1.50   1.48   510.00   1.50   1.48   CLAYSTONE,REDDISH BROWN                                                  61

511.50   1.50   1.48   511.50   1.50   1.48   CLAYSTONE,REDDISH BROWN                                                  70

513.00   3.00   3.00   513.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  70

516.00   3.00   2.98   516.00   3.00   2.98   CLAYSTONE,REDDISH BROWN                                                  73

519.00   3.00   3.00   519.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  58

522.00   3.00   3.00   522.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  42

525.00   3.00   3.00   525.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  66

528.00   3.00   3.00   528.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,DISTINCT TOP                                     59

531.00   3.00   3.00   531.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  70

534.00   3.00   3.00   534.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  73

537.00   1.82   1.82   537.00   1.82   1.82   CLAYSTONE,REDDISH BROWN                                                  78

538.82   1.18   1.18   538.82   1.18   1.18   SANDSTONE(FINE GRAINED),GREY                                             86

540.00   3.00   3.00   540.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  53

543.00   3.00   3.00   543.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,SLICKENSIDED

546.00   3.00   3.00   546.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     11

549.00   3.00   3.00   549.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  33

552.00   2.20   2.20   552.00   2.20   2.20   CLAYSTONE,REDDISH BROWN                                                  50

554.20    .80    .80   554.20    .80    .80   SANDSTONE(FINE GRAINED),GREY                                             48

555.00    .48    .48   555.00    .48    .48   CLAYSTONE,REDDISH BROWN                                                  29

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

555.48    .20    .20   555.48    .20    .20   SANDSTONE(FINE GRAINED),GREY

555.68   2.32   2.32   555.68   2.32   2.32   CLAYSTONE,REDDISH BROWN                                                  53

558.00   1.00   1.00   558.00   1.00   1.00   CLAYSTONE,REDDISH BROWN                                                  38

559.00   2.00   2.00   559.00   2.00   2.00   CLAYSTONE,REDDISH BROWN                                                  70

561.00   3.00   3.00   561.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  69

564.00   3.00   3.00   564.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  67

567.00   3.00   3.00   567.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  60

570.00   1.00   1.00   570.00   1.00   1.00   CLAYSTONE,REDDISH BROWN                                                  31

571.00   1.00   1.00   571.00   1.00   1.00   CLAYSTONE,REDDISH BROWN                                                  65

572.00    .40    .40   572.00    .40    .40   SANDSTONE(FINE GRAINED),GREY                                             63

572.40    .60    .60   572.40    .60    .60   CLAYSTONE,GREYISH BROWN                                                  23

573.00   3.00   2.86   573.00   3.00   2.86   CLAYSTONE,REDDISH BROWN                                                  50

576.00    .50    .50   576.00    .50    .50   CLAYSTONE,REDDISH BROWN                                                  50

576.50   2.50   2.50   576.50   2.50   2.50   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          72

579.00   3.00   3.00   579.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     30

582.00   2.00   2.00   582.00   2.00   2.00   CLAYSTONE,REDDISH BROWN                                                  56

584.00   1.00   1.00   584.00   1.00   1.00   CLAYSTONE,REDDISH BROWN                                                  89

585.00   3.00   3.00   585.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  74

588.00   3.00   3.00   588.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  20

591.00   3.00   3.00   591.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                 8.3

594.00   3.00   3.00   594.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  36

597.00   3.00   3.00   597.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  82

600.00   3.00   3.00   600.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  55

603.00   1.00   1.00   603.00   1.00   1.00   CLAYSTONE,REDDISH BROWN                                                  25

604.00   2.00   2.00   604.00   2.00   2.00   CLAYSTONE,REDDISH BROWN                                                  51

606.00   3.00   3.00   606.00   3.00   3.00   CLAYSTONE,REDDISH BROWN                                                  48

609.00   3.00   2.80   609.00   3.00   2.80   MUDSTONE,VARIEGAT,FRACTURE                                               32

612.00   3.00   2.75   612.00   3.00   2.75   MUDSTONE,VARIEGAT,FRACTURE

615.00   3.00   2.92   615.00   3.00   2.92   MUDSTONE,GREYISH BROWN                                                   36

618.00   3.00   3.00   618.00   3.00   3.00   MUDSTONE,VARIEGATE                                                       55

621.00   3.00   3.00   621.00   3.00   3.00   MUDSTONE,VARIEGAT,SLICKENSIDED                                           42

624.00   2.00   2.00   624.00   2.00   2.00   MUDSTONE,VARIEGAT,SLICKENSIDED                                           42

626.00   1.00   1.00   626.00   1.00   1.00   MUDSTONE,VARIEGAT,SLICKENSIDED                                           27

627.00   3.00   3.00   627.00   3.00   3.00   MUDSTONE,VARIEGAT,FRACTURE                                               45

630.00   2.00   2.00   630.00   2.00   2.00   MUDSTONE,VARIEGAT,SLICKENSIDED                                           19

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

632.00   1.00   1.00   632.00   1.00   1.00   MUDSTONE,VARIEGAT,FRACTURE

633.00   3.00   3.00   633.00   3.00   3.00   CLAYSTONE,VARIEGAT,FRACTURE                                               9

636.00   3.00   3.00   636.00   3.00   3.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                        8.3

639.00   2.80   2.80   639.00   2.80   2.80   CLAYSTONE,REDDISH BROWN,FRACTURE                                         13

641.80    .20    .20   641.80    .20    .20   SANDSTONE(FINE GRAINED),GREY

642.00   3.00   3.00   642.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          94

645.00   1.50   1.50   645.00   1.50   1.50   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              85

646.50   1.50   1.50   646.50   1.50   1.50   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              89

648.00   2.70   2.70   648.00   2.70   2.70   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              80

650.70    .30    .30   650.70    .30    .30   SHALE,GREYISH GREEN                                                      77

651.00    .66    .66   651.00    .66    .66   SHALE,GREY

651.66   2.34   2.34   651.66   2.34   2.34   SANDSTONE(FINE GRAINED),GREYISH GREEN,SLICKENSIDED                       87

****************************************************************************************************************************

BARAKAR

654.00   3.00   3.00   654.00   3.00   3.00   SANDSTONE(FINE GRAINED),GREY                                             86

657.00   3.00   3.00   657.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREY                                       84

660.00   1.50   1.50   660.00   1.50   1.50   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              79

661.50   1.50   1.50   661.50   1.50   1.50   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              68

663.00   1.24   1.21   663.00   1.24   1.21   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              86

664.24    .31    .30   664.24    .31    .30   SHALE,GREY

664.55    .56    .54   664.55    .56    .54   CARB SHALE LOW                                                  L1

665.11    .89    .88   665.11    .89    .88   SHALE,GREY

666.00    .70    .70   666.00    .70    .70   SANDY SHALE,GREY                                                         60

666.70    .79    .78   666.70    .79    .78   SHALE,GREY,SLICKENSIDED                                                  23

667.49   1.51   1.50   667.49   1.51   1.50   INTERCALATIONS,GREY,FRACTURE

669.00   1.66   1.66   669.00   1.66   1.66   INTERCALATIONS,GREY                                                      20

670.66   1.34   1.33   670.66   1.34   1.33   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    99

672.00   1.00   1.00   672.00   1.00   1.00   SANDSTONE(FINE GRAINED),WHITE                                            73

673.00   1.90   1.90   673.00   1.90   1.90   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  24

674.90    .10    .10   674.90    .10    .10   INTERCALATIONS,WHITE

675.00    .80    .80   675.00    .80    .80   SANDY SHALE,GREY,SLICKENSIDED                                            28

675.80    .59    .58   675.80    .59    .58   SHALE,GREY

676.39   1.61   1.60   676.39   1.61   1.60   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    91

678.00   3.00   3.00   678.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          89

681.00   3.00   3.00   681.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              81

684.00   3.00   3.00   684.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              84

687.00   3.00   2.50   687.00   3.00   2.50   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  67

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

690.00   3.00   2.70   690.00   3.00   2.70   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          63

693.00   3.00   2.96   693.00   3.00   2.96   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         82

696.00   3.00   2.98   696.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          72

699.00   3.00   2.90   699.00   3.00   2.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              31

702.00   3.00   3.00   702.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          48

705.00   3.00   2.99   705.00   3.00   2.99   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  55

708.00   1.20   1.20   708.00   1.20   1.20   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                 100

709.20    .60    .60   709.20    .60    .60   COAL                                                            IIA

709.80    .60    .60   709.80    .60    .60   COAL,FRACTURE                                                   IIA

710.40    .60    .60   710.40    .60    .60   COAL                                                            IIA

711.00    .60    .60   711.00    .60    .60   COAL                                                            IIA

711.60    .60    .60   711.60    .60    .60   COAL                                                            IIA

712.20   1.40   1.40   712.20   1.40   1.40   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    61

713.60    .40    .40   713.60    .40    .40   INTERCALATIONS,GREY                                                      80

714.00    .11    .11   714.00    .11    .11   SANDSTONE(FINE GRAINED),GREYISH WHITE

714.11   1.26   1.26   714.11   1.26   1.26   SHALE,GREY                                                               84

715.37    .73    .73   715.37    .73    .73   INTERCALATIONS,GREY                                                      97

716.10    .90    .90   716.10    .90    .90   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  74

717.00   3.00   3.00   717.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          45

720.00   1.70   1.70   720.00   1.70   1.70   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  85

721.70    .65    .65   721.70    .65    .65   COAL                                                            IIB

722.35    .65    .65   722.35    .65    .65   COAL                                                            IIB

723.00    .25    .25   723.00    .25    .25   COAL                                                            IIB

723.25    .05    .05   723.25    .05    .05   SHALE                                                           IIB

723.30    .31    .31   723.30    .31    .31   COAL                                                            IIB

723.61    .30    .30   723.61    .30    .30   COAL                                                            IIB

723.91    .31    .31   723.91    .31    .31   COAL                                                            IIB

724.22    .23    .23   724.22    .23    .23   COAL                                                            IIB

724.45   1.55   1.55   724.45   1.55   1.55   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    61

726.00    .95    .95   726.00    .95    .95   SANDSTONE(FINE GRAINED),GREYISH WHITE                                   100

726.95   2.05   2.05   726.95   2.05   2.05   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  81

729.00   3.00   3.00   729.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          94

732.00    .19    .19   732.00    .19    .19   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE

732.19    .51    .51   732.19    .51    .51   COAL,FRACTURE                                                   I

732.70   2.30   2.30   732.70   2.30   2.30   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    82

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

735.00   3.00   3.00   735.00   3.00   3.00   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    83

738.00   1.50   1.50   738.00   1.50   1.50   SANDSTONE(FINE GRAINED),GREYISH WHITE

BOREHOLE CLOSED AT   739.50 M DEPTH

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

---------------------------------------------------------------------------

BOREHOLE NO    :CMWWB08              LATITUDE    : 2211118.941M                      DATE OF COMMENCEMENT: 27.05.2024

REDUCED LEVEL  : 288.1883M           DEPARTURE   : 264890.2356M                      DATE OF COMPLETION: 25.09.2024

**********************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

.00    .25    .10      .00    .25    .10   SOIL,BLACK

SOIL

**************************************************************************************************************************

TRAP

.25    .75    .52      .25    .75    .52   BASALT,DARK GREY,FRACTURE

1.00   2.40   1.36     1.00   2.40   1.36   BASALT,GREYISH GREEN,FRACTURE                                            27

3.40   3.00   1.88     3.40   3.00   1.88   BASALT,GREYISH GREEN                                                     26

6.40   1.50   1.00     6.40   1.50   1.00   BASALT,GREYISH GREEN                                                     37

7.90   1.50   1.47     7.90   1.50   1.47   BASALT,GREYISH GREEN,FRACTURE                                            17

9.40   1.50   1.48     9.40   1.50   1.48   BASALT,GREYISH BROWN                                                     67

10.90   1.35   1.35    10.90   1.35   1.35   BASALT,GREY,FRACTURE                                                     49

12.25   2.00   2.00    12.25   2.00   2.00   BASALT,GREY                                                             100

14.25   1.10   1.10    14.25   1.10   1.10   BASALT,GREY,FRACTURE                                                     61

15.35   3.00   3.00    15.35   3.00   3.00   BASALT,GREY,FRACTURE                                                     53

18.35   2.90   2.90    18.35   2.90   2.90   BASALT,GREY

21.25    .10    .10    21.25    .10    .10   CLAY,GREYISH GREEN,FRACTURE

21.35   1.05   1.05    21.35   1.05   1.05   BASALT,GREYISH GREEN,FRACTURE

22.40   1.85   1.85    22.40   1.85   1.85   BASALT,GREY,FRACTURE                                                     99

24.25   1.00   1.00    24.25   1.00   1.00   BASALT,GREY,FRACTURE                                                     85

25.25   2.00   2.00    25.25   2.00   2.00   BASALT,GREY,FRACTURE                                                     93

27.25   3.00   3.00    27.25   3.00   3.00   BASALT,GREY,FRACTURE                                                     85

30.25   3.00   2.98    30.25   3.00   2.98   BASALT,GREY,FRACTURE                                                     88

33.25   3.00   3.00    33.25   3.00   3.00   BASALT,GREY,FRACTURE                                                     56

36.25   2.00   2.00    36.25   2.00   2.00   BASALT,GREY                                                              84

38.25   1.00    .90    38.25   1.00    .90   BASALT,GREY                                                              90

39.25   1.28   1.25    39.25   1.28   1.25   CLAY,REDDISH BROWN

40.53    .81    .80    40.53    .81    .80   BASALT,GREYISH GREEN                                                     48

41.34    .91    .90    41.34    .91    .90   BASALT,GREYISH GREEN                                                     85

42.25   3.00   3.00    42.25   3.00   3.00   BASALT,GREY,FRACTURE                                                     69

45.25   3.00   3.00    45.25   3.00   3.00   BASALT,GREY                                                              69

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

48.25   3.00   2.92    48.25   3.00   2.92   BASALT,GREY,FRACTURE                                                     59

51.25   2.00   2.00    51.25   2.00   2.00   BASALT,GREY,FRACTURE                                                     73

53.25   1.00   1.00    53.25   1.00   1.00   BASALT,GREY,FRACTURE                                                     37

54.25   3.00   3.00    54.25   3.00   3.00   BASALT,GREY,FRACTURE                                                     67

57.25   3.00   3.00    57.25   3.00   3.00   BASALT,GREY                                                              73

60.25   1.30   1.20    60.25   1.30   1.20   BASALT,GREYISH BROWN,SLICKENSIDED                                        39

*****************************************************************************************************************************

MOTUR

61.55   1.70   1.56    61.55   1.70   1.56   MUDSTONE,GREYISH BROWN,FRACTURE

63.25   2.00   1.66    63.25   2.00   1.66   CLAYSTONE,REDDISH BROWN,FRACTURE

65.25   1.00    .98    65.25   1.00    .98   CLAYSTONE,REDDISH BROWN,SLICKENSIDED

66.25   3.00   2.98    66.25   3.00   2.98   CLAYSTONE,REDDISH BROWN,SLICKENSIDED

69.25   3.00   3.00    69.25   3.00   3.00   CLAYSTONE,REDDISH BROWN

72.25   3.00   1.20    72.25   3.00   1.20   CLAYSTONE,REDDISH BROWN

75.25   3.00   1.80    75.25   3.00   1.80   CLAYSTONE,REDDISH BROWN

78.25   1.50    .80    78.25   1.50    .80   CLAYSTONE,REDDISH BROWN,FRACTURE                                         57

79.75   1.50   1.50    79.75   1.50   1.50   CLAYSTONE,REDDISH BROWN

81.25   3.00   3.00    81.25   3.00   3.00   SANDSTONE(FINE GRAINED),GREY                                             49

84.25    .84    .84    84.25    .84    .84   MUDSTONE,BROWN                                                           73

85.09   2.16   2.16    85.09   2.16   2.16   CLAYSTONE,REDDISH BROWN                                                  37

87.25   1.00   1.00    87.25   1.00   1.00   SANDSTONE(FINE GRAINED),REDDISH BROWN,FRACTURE

88.25    .30    .30    88.25    .30    .30   SANDSTONE(FINE GRAINED),GREYISH BROWN                                    54

88.55   1.70   1.70    88.55   1.70   1.70   CLAYSTONE,REDDISH BROWN

90.25   1.05    .90    90.25   1.05    .90   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    21

91.30   1.95   1.80    91.30   1.95   1.80   SANDSTONE(FINE GRAINED),REDDISH BROWN

93.25    .50    .34    93.25    .50    .34   MUDSTONE,GREYISH BROWN

93.75   2.50   2.20    93.75   2.50   2.20   MUDSTONE,GREYISH BROWN

96.25    .35    .24    96.25    .35    .24   SANDSTONE(FINE GRAINED),REDDISH BROWN

96.60   2.65   2.55    96.60   2.65   2.55   SANDSTONE(COARSE GRAINED),REDDISH BROWN                                  55

99.25   3.00   2.70    99.25   3.00   2.70   SANDSTONE(COARSE GRAINED),REDDISH BROWN

102.25   3.00   2.00   102.25   3.00   2.00   SANDSTONE(COARSE TO V.C.GRAINED),REDDISH BROWN                           26

105.25   3.00   2.50   105.25   3.00   2.50   SANDSTONE(COARSE TO V.C.GRAINED),REDDISH BROWN

108.25   1.50    .72   108.25   1.50    .72   CLAYSTONE,REDDISH BROWN

109.75   1.50    .38   109.75   1.50    .38   CLAYSTONE,REDDISH BROWN                                                  20

111.25   3.00   1.45   111.25   3.00   1.45   SANDSTONE(COARSE GRAINED),GREY                                           37

114.25   3.00   2.40   114.25   3.00   2.40   SANDSTONE(COARSE TO V.C.GRAINED),GREYISH BROWN                           17

117.25   1.80   1.60   117.25   1.80   1.60   CLAYSTONE,REDDISH BROWN,FRACTURE

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

119.05   1.20   1.00   119.05   1.20   1.00   CLAYSTONE,REDDISH BROWN,FRACTURE

120.25   3.00   2.50   120.25   3.00   2.50   CLAYSTONE,REDDISH BROWN                                                  23

123.25   3.00   2.80   123.25   3.00   2.80   CLAYSTONE,REDDISH BROWN

126.25   1.50   1.30   126.25   1.50   1.30   MUDSTONE,GREYISH BROWN

127.75   1.50   1.29   127.75   1.50   1.29   MUDSTONE,GREYISH BROWN,FRACTURE

129.25   1.00    .55   129.25   1.00    .55   CLAYSTONE,REDDISH BROWN

130.25   2.00    .95   130.25   2.00    .95   MUDSTONE,VARIEGAT,FRACTURE

132.25   3.00   1.60   132.25   3.00   1.60   MUDSTONE,VARIEGATE

135.25   1.00    .65   135.25   1.00    .65   SANDSTONE(FINE GRAINED),GREYISH BROWN

136.25    .90    .55   136.25    .90    .55   MUDSTONE,GREYISH BROWN,FRACTURE

137.15   1.10    .77   137.15   1.10    .77   MUDSTONE,GREYISH BROWN,FRACTURE

138.25   1.95   1.20   138.25   1.95   1.20   SANDSTONE(FINE GRAINED),GREY

140.20   1.05    .46   140.20   1.05    .46   MUDSTONE,GREYISH BROWN

141.25   1.00   1.00   141.25   1.00   1.00   CLAYSTONE,REDDISH BROWN,FRACTURE

142.25   1.00   1.00   142.25   1.00   1.00   CLAYSTONE,REDDISH BROWN,FRACTURE

143.25    .70    .70   143.25    .70    .70   SANDSTONE(FINE GRAINED),GREY

143.95    .15    .15   143.95    .15    .15   CLAYSTONE,REDDISH BROWN

144.10    .15    .15   144.10    .15    .15   CLAYSTONE,REDDISH BROWN,FRACTURE

144.25   3.00   2.55   144.25   3.00   2.55   CLAYSTONE,REDDISH BROWN,FRACTURE

147.25   3.00   2.20   147.25   3.00   2.20   CLAYSTONE,REDDISH BROWN,FRACTURE

150.25   3.00   2.70   150.25   3.00   2.70   CLAYSTONE,REDDISH BROWN

153.25   1.50   1.25   153.25   1.50   1.25   MUDSTONE,GREYISH BROWN

154.75   1.50   1.26   154.75   1.50   1.26   MUDSTONE,GREYISH BROWN,FRACTURE

156.25   3.00   1.70   156.25   3.00   1.70   MUDSTONE,GREYISH BROWN

159.25   1.95   1.95   159.25   1.95   1.95   SANDSTONE(FINE GRAINED),GREY

161.20   1.05   1.04   161.20   1.05   1.04   SANDSTONE(FINE GRAINED),GREY

162.25    .90    .55   162.25    .90    .55   CLAYSTONE,GREYISH BROWN

163.15   2.10   1.45   163.15   2.10   1.45   MUDSTONE,BROWN

165.25   2.00   1.95   165.25   2.00   1.95   MUDSTONE,BROWN,FRACTURE

167.25   2.00   1.85   167.25   2.00   1.85   MUDSTONE,GREYISH BROWN

169.25   1.00    .92   169.25   1.00    .92   MUDSTONE,GREYISH BROWN

170.25   3.00   1.80   170.25   3.00   1.80   MUDSTONE,GREYISH BROWN

173.25   3.00   2.10   173.25   3.00   2.10   MUDSTONE,REDDISH GREY

176.25   3.00   2.55   176.25   3.00   2.55   SANDSTONE(FINE GRAINED),YELLOWISHGREY,FRIABLE

179.25   1.00    .80   179.25   1.00    .80   CLAYSTONE,REDDISH BROWN

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

180.25   3.00    .92   180.25   3.00    .92   CLAYSTONE,REDDISH BROWN

183.25   3.00   1.25   183.25   3.00   1.25   CLAYSTONE,REDDISH BROWN

186.25   3.00   1.16   186.25   3.00   1.16   CLAYSTONE,REDDISH BROWN

189.25   3.00   1.94   189.25   3.00   1.94   CLAYSTONE,REDDISH BROWN

192.25   3.00    .64   192.25   3.00    .64   CLAYSTONE,REDDISH BROWN

195.25   3.00    .65   195.25   3.00    .65   CLAYSTONE,REDDISH BROWN

198.25   3.00    .88   198.25   3.00    .88   CLAYSTONE,REDDISH BROWN

201.25   3.00    .90   201.25   3.00    .90   CLAYSTONE,REDDISH BROWN

204.25   3.00    .47   204.25   3.00    .47   CLAYSTONE,REDDISH BROWN,FRACTURE

207.25   3.00    .60   207.25   3.00    .60   CLAYSTONE,REDDISH BROWN

210.25   3.00   1.54   210.25   3.00   1.54   SANDSTONE(FINE GRAINED),REDDISH BROWN                                   8.7

213.25   3.00   1.40   213.25   3.00   1.40   SANDSTONE(FINE GRAINED),REDDISH BROWN,FRIABLE

216.25   3.00   2.00   216.25   3.00   2.00   SANDSTONE(FINE GRAINED),REDDISH BROWN,FRIABLE

219.25   3.00   1.30   219.25   3.00   1.30   SANDSTONE(FINE GRAINED),REDDISH BROWN,FRIABLE

222.25   3.00   1.33   222.25   3.00   1.33   CLAYSTONE,GREYISH BROWN                                                  63

225.25   3.00   2.35   225.25   3.00   2.35   SANDSTONE(FINE GRAINED),REDDISH BROWN,FRIABLE

228.25   3.00   2.05   228.25   3.00   2.05   SANDSTONE(FINE GRAINED),GREY,FRIABLE                                     14

231.25   3.00   2.50   231.25   3.00   2.50   CLAYSTONE,REDDISH BROWN

234.25   3.00   2.00   234.25   3.00   2.00   CLAYSTONE,REDDISH BROWN

237.25   1.00    .72   237.25   1.00    .72   SANDSTONE(FINE GRAINED),GREY

238.25   2.00   1.64   238.25   2.00   1.64   SANDSTONE(FINE GRAINED),GREY,FRIABLE

240.25   1.50   1.48   240.25   1.50   1.48   SANDSTONE(FINE GRAINED),GREY,FRIABLE

241.75   1.50   1.46   241.75   1.50   1.46   SANDSTONE(FINE GRAINED),GREY,FRIABLE

243.25   3.00   2.58   243.25   3.00   2.58   SANDSTONE(FINE GRAINED),GREY,FRIABLE

246.25   2.00   2.00   246.25   2.00   2.00   SANDSTONE(FINE GRAINED),GREY,FRIABLE

248.25   1.00    .37   248.25   1.00    .37   SANDSTONE(FINE GRAINED),GREY,FRIABLE

249.25   3.00   2.75   249.25   3.00   2.75   SANDSTONE(FINE GRAINED),GREY,FRIABLE

252.25   3.00   2.92   252.25   3.00   2.92   SANDSTONE(FINE GRAINED),BROWN,FRIABLE

255.25   3.00   2.95   255.25   3.00   2.95   SANDSTONE(FINE GRAINED),YELLOWISHGREY,FRIABLE

258.25   3.00   2.75   258.25   3.00   2.75   SANDSTONE(FINE GRAINED),GREYISH WHITE,FRIABLE

261.25   3.00   2.48   261.25   3.00   2.48   SANDSTONE(FINE GRAINED),YELLOWIS,FRIABLE

264.25   3.00   1.84   264.25   3.00   1.84   SANDSTONE(FINE GRAINED),YELLOW

267.25   1.00    .45   267.25   1.00    .45   CLAYSTONE,REDDISH BROWN,FRACTURE

268.25   1.00    .64   268.25   1.00    .64   SANDSTONE(FINE GRAINED),GREYISH BROWN                                    85

269.25   1.00    .70   269.25   1.00    .70   SANDSTONE(FINE GRAINED),GREYISH BROWN

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

270.25   3.00   1.80   270.25   3.00   1.80   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   14

273.25   3.00   2.28   273.25   3.00   2.28   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

276.25   3.00   1.90   276.25   3.00   1.90   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   39

279.25   2.60   2.34   279.25   2.60   2.34   CLAYSTONE,REDDISH BROWN                                                  60

281.85    .40    .29   281.85    .40    .29   CLAYSTONE,REDDISH BROWN                                                  43

282.25    .65    .50   282.25    .65    .50   SANDSTONE(FINE GRAINED),GREYISH GREY                                     18

282.90   2.35   2.17   282.90   2.35   2.17   SANDSTONE(FINE GRAINED),GREYISH GREY                                     23

285.25   3.00   2.34   285.25   3.00   2.34   SANDSTONE(FINE GRAINED),GREYISH GREY                                     13

288.25   2.50   1.90   288.25   2.50   1.90   CLAYSTONE,REDDISH BROWN

290.75    .50    .46   290.75    .50    .46   CLAYSTONE,REDDISH BROWN

291.25    .40    .20   291.25    .40    .20   SANDSTONE(FINE GRAINED),GREYISH BROWN,FRACTURE

291.65   2.60   2.32   291.65   2.60   2.32   SANDSTONE(FINE GRAINED),GREY

294.25   1.00   1.00   294.25   1.00   1.00   SANDSTONE(FINE GRAINED),GREY

295.25   1.00   1.00   295.25   1.00   1.00   SANDSTONE(FINE GRAINED),GREY

296.25   1.00    .97   296.25   1.00    .97   CLAYSTONE,GREY,SLICKENSIDED                                              43

297.25   1.00   1.00   297.25   1.00   1.00   SANDSTONE(FINE GRAINED),RED

298.25   2.00    .32   298.25   2.00    .32   SANDSTONE(FINE GRAINED),RED

300.25   3.00    .25   300.25   3.00    .25   SANDSTONE(FINE GRAINED),RED

303.25   3.00    .55   303.25   3.00    .55   SANDSTONE(FINE GRAINED),YELLOW

306.25   3.00    .80   306.25   3.00    .80   SANDSTONE(FINE GRAINED),YELLOW,FAULTED

309.25   3.00    .87   309.25   3.00    .87   SANDSTONE(FINE GRAINED),YELLOW,FAULTED

312.25   3.00   1.10   312.25   3.00   1.10   SANDSTONE(FINE GRAINED),GREY,FAULTED

315.25   3.00    .60   315.25   3.00    .60   SANDSTONE(FINE GRAINED),GREY,FAULTED

318.25   3.00   1.20   318.25   3.00   1.20   SANDSTONE(FINE GRAINED),GREY,FAULTED

321.25   3.00   1.20   321.25   3.00   1.20   SANDSTONE(FINE GRAINED),GREY,FAULTED

324.25   3.00   1.00   324.25   3.00   1.00   SANDSTONE(FINE GRAINED),GREY,FAULTED

327.25   3.00   1.38   327.25   3.00   1.38   SANDSTONE(FINE GRAINED),GREY,FAULTED

330.25   1.00   1.00   330.25   1.00   1.00   SANDSTONE(FINE GRAINED),GREY,FAULTED

331.25   2.00   2.00   331.25   2.00   2.00   SANDSTONE(FINE GRAINED),GREY,FAULTED

333.25   3.00   2.95   333.25   3.00   2.95   SANDSTONE(FINE GRAINED),GREY,DISTINCT TOP

336.25   3.00   2.96   336.25   3.00   2.96   SANDSTONE(FINE GRAINED),GREY

339.25   3.00   2.95   339.25   3.00   2.95   SANDSTONE(FINE GRAINED),GREY,FAULTED                                     26

342.25   3.00   2.97   342.25   3.00   2.97   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   25

345.25   3.00   2.97   345.25   3.00   2.97   SANDSTONE(FINE GRAINED),GREY,FAULTED

348.25   3.00   2.96   348.25   3.00   2.96   SANDSTONE(FINE GRAINED),GREY                                             44

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

351.25   3.00   2.95   351.25   3.00   2.95   SANDSTONE(FINE GRAINED),GREY,FAULTED                                     51

354.25   3.00   2.93   354.25   3.00   2.93   CLAYSTONE,REDDISH BROWN,DISTINCT TOP                                     35

357.25   3.00   2.98   357.25   3.00   2.98   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    68

360.25   3.00   2.98   360.25   3.00   2.98   SANDSTONE(FINE GRAINED),GREYISH GREY                                     25

363.25   3.00   2.99   363.25   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREY                           58

366.25   1.80   1.80   366.25   1.80   1.80   CLAYSTONE,REDDISH BROWN,SLICKENSIDED                                     20

368.05   1.20   1.20   368.05   1.20   1.20   MUDSTONE,GREYISH BROWN,FRACTURE

369.25   1.00   1.00   369.25   1.00   1.00   MUDSTONE,GREYISH BROWN

370.25   3.00   2.99   370.25   3.00   2.99   MUDSTONE,GREYISH BROWN

373.25   1.00   1.00   373.25   1.00   1.00   MUDSTONE,GREYISH BROWN

374.25   1.00    .85   374.25   1.00    .85   MUDSTONE,GREYISH BROWN,DISTINCT TOP

375.25   3.00   1.31   375.25   3.00   1.31   MUDSTONE,VARIEGAT,DISTINCT TOP                                           21

378.25   3.00   3.00   378.25   3.00   3.00   MUDSTONE,VARIEGAT,SLICKENSIDED

381.25   3.00   2.98   381.25   3.00   2.98   MUDSTONE,GREY,DISTINCT TOP

384.25    .44    .44   384.25    .44    .44   SANDSTONE(FINE GRAINED),GREY,                                            53

INTERFINGERED WITH LOWER UNIT

384.69   1.71   1.71   384.69   1.71   1.71   MUDSTONE,GREYISH BROWN

386.40    .40    .40   386.40    .40    .40   SANDSTONE(FINE GRAINED),GREY

386.80    .45    .45   386.80    .45    .45   SANDSTONE(FINE GRAINED),GREY

387.25    .40    .40   387.25    .40    .40   MUDSTONE,GREYISH BROWN                                                   35

387.65   1.10   1.10   387.65   1.10   1.10   MUDSTONE,GREYISH BROWN

388.75    .20    .20   388.75    .20    .20   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   80

388.95    .41    .40   388.95    .41    .40   MUDSTONE,GREYISH BROWN,FRACTURE                                          18

389.36   2.39   2.38   389.36   2.39   2.38   MUDSTONE,VARIEGAT,FRACTURE

391.75   3.00   2.95   391.75   3.00   2.95   MUDSTONE,GREYISH BROWN,FRACTURE

394.75   2.50   2.48   394.75   2.50   2.48   MUDSTONE,GREYISH BROWN

397.25   1.00    .58   397.25   1.00    .58   SANDSTONE(MEDIUM GRAINED),GREY

398.25   2.00   1.46   398.25   2.00   1.46   MUDSTONE,GREYISH BROWN,FRACTURE

400.25   3.00   1.80   400.25   3.00   1.80   MUDSTONE,GREYISH BROWN,FRACTURE                                          22

403.25   3.00   2.96   403.25   3.00   2.96   MUDSTONE,GREYISH BROWN,SLICKENSIDED                                      32

406.25   2.00   2.00   406.25   2.00   2.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                         14

408.25   1.00    .98   408.25   1.00    .98   CLAYSTONE,GREYISH BROWN,FRACTURE

409.25   1.08   1.06   409.25   1.08   1.06   SANDSTONE(FINE GRAINED),GREY

410.33    .36    .34   410.33    .36    .34   MUDSTONE,GREY

410.69   1.56   1.54   410.69   1.56   1.54   MUDSTONE,GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

412.25   1.00    .98   412.25   1.00    .98   CLAYSTONE,GREYISH BROWN

413.25   3.00   2.98   413.25   3.00   2.98   CLAYSTONE,GREYISH BROWN,FRACTURE

416.25   3.00   2.98   416.25   3.00   2.98   CLAYSTONE,GREYISH BROWN,FRACTURE

419.25   1.00   1.00   419.25   1.00   1.00   MUDSTONE,GREY                                                            41

420.25   1.50   1.50   420.25   1.50   1.50   MUDSTONE,GREY

421.75   1.50   1.50   421.75   1.50   1.50   MUDSTONE,GREYISH BROWN

423.25   3.00   2.80   423.25   3.00   2.80   MUDSTONE,GREYISH BROWN

426.25    .50    .50   426.25    .50    .50   CLAYSTONE,GREYISH BROWN

426.75   2.50   2.50   426.75   2.50   2.50   CLAYSTONE,GREYISH BROWN

429.25   3.00   2.98   429.25   3.00   2.98   CLAYSTONE,GREYISH BROWN

432.25    .50    .48   432.25    .50    .48   MUDSTONE,GREY                                                            66

432.75   1.65   1.65   432.75   1.65   1.65   SANDSTONE(MEDIUM GRAINED),GREY

434.40    .10    .10   434.40    .10    .10   SANDSTONE(MEDIUM GRAINED),GREY                                           71

434.50   1.00   1.00   434.50   1.00   1.00   SANDSTONE(MED. TO C. GRAINED),GREY                                       93

435.50    .70    .70   435.50    .70    .70   MUDSTONE,GREY                                                            71

436.20    .34    .32   436.20    .34    .32   SANDSTONE(MED. TO C. GRAINED),GREY                                       67

436.54   1.46   1.46   436.54   1.46   1.46   SANDSTONE(COARSE TO V.C.GRAINED),GREY                                    88

438.00    .33    .32   438.00    .33    .32   MUDSTONE,GREY,FRACTURE

438.33   2.36   2.35   438.33   2.36   2.35   CLAYSTONE,CHOCOLATEPINK

440.69    .31    .30   440.69    .31    .30   CLAYSTONE,CHOCOLATEPINK

441.00    .50    .10   441.00    .50    .10   MUDSTONE,GREY                                                            22

441.50   1.00    .50   441.50   1.00    .50   MUDSTONE,GREY,FRACTURE

442.50   3.00    .95   442.50   3.00    .95   MUDSTONE,GREY,FRACTURE

445.50   1.00    .25   445.50   1.00    .25   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   45

446.50   1.50   1.05   446.50   1.50   1.05   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN                          56

448.00   3.00   1.62   448.00   3.00   1.62   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                              64

451.00   1.00    .95   451.00   1.00    .95   CLAYSTONE,GREYISH BROWN,FRACTURE

452.00    .56    .56   452.00    .56    .56   SANDSTONE(MEDIUM GRAINED),GREY                                           79

452.56    .19    .17   452.56    .19    .17   MUDSTONE,DARK GREY

452.75    .25    .25   452.75    .25    .25   SANDSTONE(MEDIUM GRAINED),GREY,FRACTURE                                  38

453.00    .65    .35   453.00    .65    .35   SANDSTONE(MEDIUM GRAINED),GREY                                           48

453.65    .67    .37   453.65    .67    .37   MUDSTONE,GREY

454.32   1.68   1.65   454.32   1.68   1.65   MUDSTONE,GREY

456.00   1.50   1.45   456.00   1.50   1.45   MUDSTONE,GREY

457.50    .70    .47   457.50    .70    .47   SANDSTONE(MED. TO C. GRAINED),GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

458.20    .80    .65   458.20    .80    .65   CLAYSTONE,DARK GREY

459.00   1.50   1.20   459.00   1.50   1.20   CLAYSTONE,DARK GREY

460.50   1.50   1.35   460.50   1.50   1.35   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED                                     20

462.00   2.00   1.90   462.00   2.00   1.90   CLAYSTONE,CHOCOLATEPINK

464.00   1.00    .66   464.00   1.00    .66   CLAYSTONE,CHOCOLATEPINK

465.00   1.00    .80   465.00   1.00    .80   CLAYSTONE,DARK BROWN,FRACTURE

466.00   1.50   1.30   466.00   1.50   1.30   CLAYSTONE,CHOCOLATEPINK                                                  17

467.50   1.50   1.45   467.50   1.50   1.45   CLAYSTONE,GREYISH BROWN                                                  46

469.00    .54    .53   469.00    .54    .53   SANDSTONE(MED. TO C. GRAINED),GREY                                       62

469.54    .45    .45   469.54    .45    .45   MUDSTONE,GREY

469.99    .23    .23   469.99    .23    .23   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         23

470.22   1.78   1.78   470.22   1.78   1.78   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED                                     35

472.00   1.50   1.45   472.00   1.50   1.45   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED                                     53

473.50   2.00   1.98   473.50   2.00   1.98   CLAYSTONE,CHOCOLATEPINK                                                  54

475.50   2.00   1.99   475.50   2.00   1.99   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         59

477.50   3.00   2.95   477.50   3.00   2.95   CLAYSTONE,CHOCOLATEPINK

480.50   2.50   2.50   480.50   2.50   2.50   CLAYSTONE,CHOCOLATEPINK                                                  33

483.00   3.00   2.98   483.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK

486.00   3.00   2.95   486.00   3.00   2.95   CLAYSTONE,CHOCOLATEPINK                                                 5.6

489.00   2.50   2.00   489.00   2.50   2.00   CLAYSTONE,CHOCOLATEPINK,FRACTURE

491.50   1.50   1.00   491.50   1.50   1.00   CLAYSTONE,CHOCOLATEPINK,FRACTURE

493.00   3.00   2.98   493.00   3.00   2.98   CLAYSTONE,GREYISH BROWN

496.00   2.00   1.50   496.00   2.00   1.50   CLAYSTONE,GREYISH BROWN,FRACTURE                                         90

498.00   2.00   1.80   498.00   2.00   1.80   CLAYSTONE,CHOCOLATEPINK                                                  12

500.00   1.00    .95   500.00   1.00    .95   CLAYSTONE,GREYISH BROWN                                                  52

501.00   3.00   2.82   501.00   3.00   2.82   CLAYSTONE,GREYISH BROWN                                                  15

504.00   2.50   2.36   504.00   2.50   2.36   CLAYSTONE,GREYISH BROWN                                                  15

506.50   1.00    .97   506.50   1.00    .97   CLAYSTONE,GREYISH BROWN,FRACTURE                                        5.2

507.50   2.50   2.50   507.50   2.50   2.50   CLAYSTONE,GREYISH BROWN,FRACTURE

510.00   3.00   2.90   510.00   3.00   2.90   CLAYSTONE,GREYISH BROWN                                                  14

513.00   2.00   1.96   513.00   2.00   1.96   CLAYSTONE,GREYISH BROWN,FRACTURE

515.00   3.00   2.90   515.00   3.00   2.90   CLAYSTONE,GREYISH BROWN,FRACTURE                                         19

518.00    .98    .98   518.00    .98    .98   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   53

518.98    .85    .81   518.98    .85    .81   CLAYSTONE,GREYISH BROWN,FRACTURE

519.83    .87    .87   519.83    .87    .87   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

520.70    .30    .25   520.70    .30    .25   CLAYSTONE,CHOCOLATEPINK                                                  23

521.00   3.00   2.98   521.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK                                                   7

524.00   3.00   2.99   524.00   3.00   2.99   CLAYSTONE,GREYISH BROWN,FRACTURE                                         32

527.00   2.00   1.95   527.00   2.00   1.95   CLAYSTONE,GREYISH BROWN                                                  30

529.00   2.00   1.90   529.00   2.00   1.90   CLAYSTONE,GREYISH BROWN,FRACTURE

531.00   3.00   2.95   531.00   3.00   2.95   SANDSTONE(FINE TO MEDIUM GRAINED),GREY,FRACTURE                         7.7

534.00   3.00   2.90   534.00   3.00   2.90   SANDSTONE(FINE TO MEDIUM GRAINED),GREY,FRACTURE

537.00    .41    .41   537.00    .41    .41   CLAYSTONE,CHOCOLATEPINK,FRACTURE

537.41   2.59   2.55   537.41   2.59   2.55   CLAYSTONE,CHOCOLATEPINK,FRACTURE

540.00   3.00   2.99   540.00   3.00   2.99   CLAYSTONE,CHOCOLATEPINK,FRACTURE

543.00   3.00   2.98   543.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         11

546.00   3.00   2.99   546.00   3.00   2.99   CLAYSTONE,GREYISH BROWN,FRACTURE

549.00   2.00   1.98   549.00   2.00   1.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE

551.00   2.80   2.78   551.00   2.80   2.78   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   65

553.80    .20    .20   553.80    .20    .20   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

554.00    .15    .15   554.00    .15    .15   MUDSTONE,GREYISH BROWN

554.15    .65    .60   554.15    .65    .60   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   86

554.80    .21    .21   554.80    .21    .21   MUDSTONE,GREYISH BROWN

555.01    .12    .12   555.01    .12    .12   SANDSTONE(MED. TO C. GRAINED),GREY                                       50

555.13    .68    .68   555.13    .68    .68   MUDSTONE,GREY

555.81    .18    .18   555.81    .18    .18   CLAYSTONE,GREYISH BROWN,FRACTURE

555.99    .35    .28   555.99    .35    .28   SANDSTONE(FINE TO MEDIUM GRAINED),GREY,FRACTURE

556.34    .35    .35   556.34    .35    .35   CLAYSTONE,CHOCOLATEPINK

556.69    .31    .28   556.69    .31    .28   CLAYSTONE,CHOCOLATEPINK

557.00   1.06    .99   557.00   1.06    .99   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   82

558.06    .83    .83   558.06    .83    .83   CLAYSTONE,CHOCOLATEPINK                                                  55

558.89   1.11   1.11   558.89   1.11   1.11   CLAYSTONE,CHOCOLATEPINK,FRACTURE

560.00   1.50   1.50   560.00   1.50   1.50   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

561.50    .50    .48   561.50    .50    .48   CLAYSTONE,GREYISH BROWN                                                  18

562.00   1.00   1.00   562.00   1.00   1.00   MUDSTONE,GREYISH BROWN                                                  100

563.00    .28    .28   563.00    .28    .28   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          80

563.28   1.04   1.04   563.28   1.04   1.04   MUDSTONE,GREY                                                            64

564.32   1.18   1.18   564.32   1.18   1.18   MUDSTONE,GREYISH BROWN,FRACTURE                                          19

565.50   1.40   1.40   565.50   1.40   1.40   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED

566.90   1.60   1.58   566.90   1.60   1.58   CLAYSTONE,CHOCOLATEPINK

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

568.50   1.19   1.15   568.50   1.19   1.15   MUDSTONE,GREY,FRACTURE

569.69    .20    .12   569.69    .20    .12   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              66

569.89   1.61   1.61   569.89   1.61   1.61   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

571.50    .26    .26   571.50    .26    .26   MUDSTONE,GREYISH BROWN

571.76    .24    .24   571.76    .24    .24   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          50

572.00   1.20   1.20   572.00   1.20   1.20   MUDSTONE,GREY                                                            30

573.20   1.17   1.15   573.20   1.17   1.15   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          85

574.37    .13    .13   574.37    .13    .13   MUDSTONE,GREY

574.50    .55    .55   574.50    .55    .55   CLAYSTONE,GREYISH BROWN,FRACTURE

575.05   2.45   2.43   575.05   2.45   2.43   CLAYSTONE,CHOCOLATEPINK                                                  17

577.50   3.00   2.98   577.50   3.00   2.98   CLAYSTONE,CHOCOLATEPINK                                                  38

580.50   3.00   2.98   580.50   3.00   2.98   MUDSTONE,GREYISH BROWN                                                   36

583.50   1.00   1.00   583.50   1.00   1.00   CLAYSTONE,CHOCOLATEPINK                                                  27

584.50   1.40   1.40   584.50   1.40   1.40   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                  100

585.90    .20    .20   585.90    .20    .20   MUDSTONE,GREY

586.10    .17    .17   586.10    .17    .17   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   87

586.27    .23    .23   586.27    .23    .23   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

586.50    .17    .17   586.50    .17    .17   MUDSTONE,GREY

586.67    .27    .25   586.67    .27    .25   CLAYSTONE,CHOCOLATEPINK                                                  42

586.94   2.56   2.56   586.94   2.56   2.56   CLAYSTONE,CHOCOLATEPINK                                                 9.7

589.50   3.00   2.99   589.50   3.00   2.99   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         27

592.50   3.00   2.98   592.50   3.00   2.98   CLAYSTONE,CHOCOLATEPINK

595.50   2.07   2.06   595.50   2.07   2.06   CLAYSTONE,GREYISH BROWN                                                  49

597.57    .45    .45   597.57    .45    .45   CLAYSTONE,CHOCOLATEPINK,FRACTURE

598.02    .48    .48   598.02    .48    .48   CLAYSTONE,GREYISH BROWN,FRACTURE                                         20

598.50   3.00   2.98   598.50   3.00   2.98   CLAYSTONE,GREYISH BROWN,FRACTURE                                          4

601.50   3.00   2.98   601.50   3.00   2.98   CLAYSTONE,CHOCOLATEPINK                                                  27

604.50   3.00   2.99   604.50   3.00   2.99   CLAYSTONE,CHOCOLATEPINK                                                  40

607.50   3.00   2.99   607.50   3.00   2.99   CLAYSTONE,CHOCOLATEPINK                                                  58

610.50   1.50   1.48   610.50   1.50   1.48   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         35

612.00   3.00   2.98   612.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         20

615.00   3.00   2.98   615.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK                                                  23

618.00   1.13   1.08   618.00   1.13   1.08   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              54

619.13    .76    .76   619.13    .76    .76   CLAYSTONE,CHOCOLATEPINK

619.89   1.11   1.06   619.89   1.11   1.06   CLAYSTONE,CHOCOLATEPINK                                                  42

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

621.00   1.25   1.25   621.00   1.25   1.25   SANDSTONE(MED. TO C. GRAINED),GREY                                       81

622.25    .43    .43   622.25    .43    .43   MUDSTONE,CHOCOLATEPINK

622.68    .32    .30   622.68    .32    .30   CLAYSTONE,CHOCOLATEPINK                                                 6.7

623.00   3.00   2.96   623.00   3.00   2.96   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED

626.00   3.00   2.90   626.00   3.00   2.90   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         11

629.00   3.00   2.96   629.00   3.00   2.96   CLAYSTONE,CHOCOLATEPINK                                                  66

632.00   3.00   2.99   632.00   3.00   2.99   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         31

635.00   3.00   2.98   635.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK                                                  34

638.00   3.00   2.99   638.00   3.00   2.99   CLAYSTONE,GREYISH BROWN                                                  33

641.00   3.00   2.99   641.00   3.00   2.99   CLAYSTONE,GREYISH BROWN                                                  24

644.00   1.45   1.44   644.00   1.45   1.44   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          53

645.45    .45    .45   645.45    .45    .45   MUDSTONE,GREY,FRACTURE

645.90    .20    .18   645.90    .20    .18   CLAYSTONE,CHOCOLATEPINK,FRACTURE

646.10    .90    .85   646.10    .90    .85   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED

647.00   3.00   2.98   647.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED

650.00   3.00   2.96   650.00   3.00   2.96   CLAYSTONE,CHOCOLATEPINK                                                  40

653.00   2.50   2.45   653.00   2.50   2.45   CLAYSTONE,CHOCOLATEPINK

655.50    .63    .62   655.50    .63    .62   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

656.13    .15    .13   656.13    .15    .13   CLAYSTONE,CHOCOLATEPINK

656.28    .20    .20   656.28    .20    .20   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

656.48    .32    .32   656.48    .32    .32   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED

656.80   1.70   1.70   656.80   1.70   1.70   CLAYSTONE,GREYISH BROWN,FRACTURE                                        3.7

658.50   3.00   2.98   658.50   3.00   2.98   CLAYSTONE,GREYISH BROWN,FRACTURE

661.50   1.00    .99   661.50   1.00    .99   CLAYSTONE,GREYISH BROWN,FRACTURE

662.50   2.00   1.98   662.50   2.00   1.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE

664.50   2.00   1.98   664.50   2.00   1.98   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN

666.50    .40    .38   666.50    .40    .38   CLAYSTONE,GREYISH BROWN

666.90    .60    .60   666.90    .60    .60   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         11

667.50   3.00   2.98   667.50   3.00   2.98   CLAYSTONE,GREYISH BROWN                                                  74

670.50   1.60   1.56   670.50   1.60   1.56   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   73

672.10    .55    .54   672.10    .55    .54   CLAYSTONE,CHOCOLATEPINK                                                  81

672.65    .85    .83   672.65    .85    .83   CLAYSTONE,CHOCOLATEPINK                                                  52

673.50   2.05   2.05   673.50   2.05   2.05   SANDSTONE(MED. TO C. GRAINED),GREY                                       89

675.55    .62    .62   675.55    .62    .62   CLAYSTONE,CHOCOLATEPINK                                                  55

676.17    .33    .32   676.17    .33    .32   CLAYSTONE,CHOCOLATEPINK                                                  45

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

676.50    .55    .50   676.50    .55    .50   MUDSTONE,GREY

677.05    .15    .11   677.05    .15    .11   SANDSTONE(MED. TO C. GRAINED),GREY

677.20    .40    .39   677.20    .40    .39   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                        7.9

677.60   1.90   1.88   677.60   1.90   1.88   CLAYSTONE,CHOCOLATEPINK,FRACTURE

679.50   1.00    .87   679.50   1.00    .87   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED                                     19

680.50   3.00   2.98   680.50   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         10

683.50   3.00   2.97   683.50   3.00   2.97   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED                                     12

686.50   3.00   2.98   686.50   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE

689.50   3.00   2.98   689.50   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED

692.50   3.00   2.96   692.50   3.00   2.96   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         12

695.50   3.00   2.99   695.50   3.00   2.99   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         36

698.50   1.00    .99   698.50   1.00    .99   CLAYSTONE,CHOCOLATEPINK                                                  41

699.50   2.00   1.98   699.50   2.00   1.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         16

701.50   3.00   2.98   701.50   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         24

704.50   3.00   2.99   704.50   3.00   2.99   CLAYSTONE,CHOCOLATEPINK                                                  48

707.50    .50    .50   707.50    .50    .50   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED                                     36

708.00   3.00   2.98   708.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                        6.7

711.00   3.00   2.98   711.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK                                                  29

714.00   3.00   2.99   714.00   3.00   2.99   CLAYSTONE,CHOCOLATEPINK                                                 3.7

717.00   3.00   2.98   717.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED                                    3.7

720.00   3.00   2.98   720.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         42

723.00   3.00   3.00   723.00   3.00   3.00   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                         14

726.00   3.00   2.97   726.00   3.00   2.97   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED                                     27

729.00   3.00   2.98   729.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE                                          5

732.00   3.00   2.99   732.00   3.00   2.99   CLAYSTONE,CHOCOLATEPINK,FRACTURE

735.00   3.00   2.99   735.00   3.00   2.99   CLAYSTONE,CHOCOLATEPINK,FRACTURE

738.00   3.00   2.98   738.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,FRACTURE

741.00    .32    .32   741.00    .32    .32   CLAYSTONE,GREY                                                           65

741.32    .37    .37   741.32    .37    .37   SANDSTONE(MED. TO C. GRAINED),CHOCOLATEPINK,FRACTURE

741.69   1.26   1.26   741.69   1.26   1.26   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   51

742.95   1.05   1.05   742.95   1.05   1.05   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              66

744.00   3.00   2.98   744.00   3.00   2.98   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED

747.00   1.50   1.48   747.00   1.50   1.48   CLAYSTONE,CHOCOLATEPINK,SLICKENSIDED

748.50   1.03   1.02   748.50   1.03   1.02   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   88

****************************************************************************************************************************

BARAKAR

749.53   1.97   1.97   749.53   1.97   1.97   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          60

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

751.50   2.00   1.35   751.50   2.00   1.35   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          21

753.50   1.50    .80   753.50   1.50    .80   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          15

755.00   1.00    .96   755.00   1.00    .96   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN,                             25

756.00   2.90   2.45   756.00   2.90   2.45   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              62

758.90    .50    .31   758.90    .50    .31   SANDSTONE(MED. TO C. GRAINED),GREY,FRACTURE                             6.3

759.40   1.75   1.62   759.40   1.75   1.62   SANDY SHALE,GREYISH WHITE                                                71

761.15    .65    .51   761.15    .65    .51   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         44

761.80   2.80   2.79   761.80   2.80   2.79   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN

764.60    .10    .10   764.60    .10    .10   SANDSTONE(FINE TO MEDIUM GRAINED)

764.70    .15    .12   764.70    .15    .12   SHALE,DARK GREY

764.85   1.05    .99   764.85   1.05    .99   SHALE,GREY,SLICKENSIDED

765.90   1.70   1.65   765.90   1.70   1.65   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   83

767.60   3.00   2.98   767.60   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              49

770.60   2.90   2.89   770.60   2.90   2.89   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              30

773.50   2.90   2.90   773.50   2.90   2.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              74

776.40   2.25   2.25   776.40   2.25   2.25   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     12

778.65    .65    .62   778.65    .65    .62   SHALE,GREY,SLICKENSIDED

779.30    .30    .30   779.30    .30    .30   SANDY SHALE,GREY,SLICKENSIDED

779.60   1.45   1.43   779.60   1.45   1.43   INTERCALATIONS,GREY                                                      29

781.05   1.15   1.14   781.05   1.15   1.14   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         58

782.20   2.35   2.35   782.20   2.35   2.35   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          88

784.55    .55    .53   784.55    .55    .53   SHALE,GREY,FRACTURE

785.10    .40    .40   785.10    .40    .40   SHALE,GREY

785.50    .87    .87   785.50    .87    .87   SHALE,GREY,SLICKENSIDED

786.37    .63    .63   786.37    .63    .63   INTERCALATIONS,GREY,SLICKENSIDED                                         24

787.00   1.34   1.32   787.00   1.34   1.32   INTERCALATIONS,GREY,SLICKENSIDED                                         30

788.34   1.56   1.56   788.34   1.56   1.56   SHALE,GREY,SLICKENSIDED

789.90   3.00   2.98   789.90   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,                             47

792.90   2.90   2.89   792.90   2.90   2.89   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              30

795.80   1.70   1.69   795.80   1.70   1.69   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              39

797.50   2.10   2.10   797.50   2.10   2.10   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              59

799.60   2.90   2.89   799.60   2.90   2.89   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,                             73

802.50   2.90   2.88   802.50   2.90   2.88   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              32

805.40   2.90   2.85   805.40   2.90   2.85   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                    6.2

808.30   1.20   1.20   808.30   1.20   1.20   SANDSTONE(FINE TO COARSE GRAINED),GREYISH WHITE                          87

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

809.50   1.70   1.69   809.50   1.70   1.69   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     71

811.20   2.90   2.89   811.20   2.90   2.89   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              67

814.10   2.90   2.89   814.10   2.90   2.89   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,                             83

817.00   2.90   2.88   817.00   2.90   2.88   SANDSTONE(FINE TO COARSE GRAINED),GREYISH WHITE                          65

819.90   1.90   1.90   819.90   1.90   1.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              87

821.80   2.90   2.90   821.80   2.90   2.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              93

824.70   1.85   1.84   824.70   1.85   1.84   SANDSTONE(MED. TO C. GRAINED),GREY                                       79

826.55   1.05   1.02   826.55   1.05   1.02   INTERCALATIONS,GREY,FRACTURE                                             65

827.60    .32    .32   827.60    .32    .32   SANDSTONE(FINE TO COARSE GRAINED),GREY                                   44

827.92    .68    .68   827.92    .68    .68   COAL,SLICKENSIDED                                               IIA

828.60    .55    .52   828.60    .55    .52   COAL,FRACTURE                                                   IIA

829.15   1.00    .97   829.15   1.00    .97   SANDSTONE(FINE TO COARSE GRAINED),GREYISH WHITE                          53

830.15   1.35   1.35   830.15   1.35   1.35   INTERCALATIONS,GREY,SLICKENSIDED                                         33

831.50    .51    .51   831.50    .51    .51   INTERCALATIONS,GREY                                                      22

832.01    .46    .45   832.01    .46    .45   CARB SHALE LOW                                                  L2

832.47   1.93   1.93   832.47   1.93   1.93   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     42

834.40    .24    .24   834.40    .24    .24   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     46

834.64    .50    .48   834.64    .50    .48   COAL,FRACTURE                                                   IIB

835.14    .45    .43   835.14    .45    .43   COAL                                                            IIB

835.59    .70    .70   835.59    .70    .70   COAL                                                            IIB

836.29    .43    .42   836.29    .43    .42   COAL                                                            IIB

836.72    .23    .23   836.72    .23    .23   COAL                                                            IIB

836.95    .18    .17   836.95    .18    .17   COAL                                                            IIB

837.13    .70    .70   837.13    .70    .70   COAL                                                            IIB

837.83    .62    .60   837.83    .62    .60   SANDY SHALE,GREY,SLICKENSIDED

838.45   1.35   1.32   838.45   1.35   1.32   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   52

839.80   2.20   2.00   839.80   2.20   2.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     57

842.00   1.10    .95   842.00   1.10    .95   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE

843.10   2.90   2.50   843.10   2.90   2.50   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     47

846.00   1.70   1.50   846.00   1.70   1.50   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     50

847.70    .83    .83   847.70    .83    .83   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

848.53    .17    .17   848.53    .17    .17   COAL                                                            L3

848.70    .30    .30   848.70    .30    .30   SANDSTONE(FINE TO COARSE GRAINED)

849.00   2.90   2.80   849.00   2.90   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,

851.90   2.90   2.90   851.90   2.90   2.90   SANDSTONE(MED. TO C. GRAINED),WHITE                                      80

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

854.80   2.90   2.88   854.80   2.90   2.88   SANDSTONE(MED. TO C. GRAINED),WHITE                                      89

857.70   2.90   2.89   857.70   2.90   2.89   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     81

860.60   2.90   2.89   860.60   2.90   2.89   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              32

863.50   2.90   2.90   863.50   2.90   2.90   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  89

866.40   2.90   2.90   866.40   2.90   2.90   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE,SLICKENSIDED                     74

869.30   2.70   2.70   869.30   2.70   2.70   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE,FRACTURE                         88

BOREHOLE CLOSED AT   872.00 M DEPTH

---------------------------------------------------------------------------

BOREHOLE NO    :CMWWB09              LATITUDE    : 2208274.308M                     DATE OF COMMENCEMENT: 14.06.2024

REDUCED LEVEL  : 257.0697M           DEPARTURE   : 267804.9796M                      DATE OF COMPLETION: 28.07.2024

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

************************************************************************************************************************

.00    .30    .10      .00    .30    .10   SOIL,BLACK

SOIL

**************************************************************************************************************************

TRAP

.30   2.70   2.60      .30   2.70   2.60   BASALT,GREY,FRACTURE

3.00   1.00   1.00     3.00   1.00   1.00   BASALT,GREY

4.00   3.00   2.80     4.00   3.00   2.80   BASALT,GREYISH BROWN,FRACTURE

7.00   3.00   1.62     7.00   3.00   1.62   BASALT,GREY,FRACTURE

**************************************************************************************************************************

MOTUR

10.00   3.00   1.30    10.00   3.00   1.30   MUDSTONE,GREYISH BROWN

13.00   3.00   1.25    13.00   3.00   1.25   MUDSTONE,GREYISH BROWN

16.00   3.00   1.60    16.00   3.00   1.60   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH PINK                           49

19.00   3.00   1.14    19.00   3.00   1.14   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH PINK                           13

22.00   3.00   1.38    22.00   3.00   1.38   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN,

25.00   3.00   1.73    25.00   3.00   1.73   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN,

28.00   3.00    .84    28.00   3.00    .84   SANDSTONE(FINE TO MEDIUM GRAINED),RED,FRIABLE

31.00   3.00    .78    31.00   3.00    .78   SANDSTONE(FINE TO MEDIUM GRAINED),RED,FRIABLE

34.00   3.00   1.00    34.00   3.00   1.00   SANDSTONE(FINE TO MEDIUM GRAINED),RED,FRIABLE

37.00   3.00   1.37    37.00   3.00   1.37   SANDSTONE(FINE TO MEDIUM GRAINED),RED,FRIABLE

40.00   3.00    .77    40.00   3.00    .77   SANDSTONE(FINE TO MEDIUM GRAINED),REDDISH BROWN,

43.00   3.00    .71    43.00   3.00    .71   CLAYSTONE,REDDISH BROWN

46.00   3.00    .99    46.00   3.00    .99   SANDSTONE(MED. TO C. GRAINED),REDDISH BROWN                              13

49.00   2.00   1.10    49.00   2.00   1.10   SANDSTONE(MED. TO C. GRAINED),REDDISH BROWN                              18

51.00   3.00    .52    51.00   3.00    .52   SANDSTONE(MEDIUM GRAINED),BROWNISH RED

54.00   3.00   1.10    54.00   3.00   1.10   SANDSTONE(FINE TO MEDIUM GRAINED),BROWN                                  22

57.00   1.00    .17    57.00   1.00    .17   SANDSTONE(FINE TO MEDIUM GRAINED),BROWN

58.00   2.00    .68    58.00   2.00    .68   CLAYSTONE,REDDISH BROWN

60.00   1.00    .12    60.00   1.00    .12   SANDSTONE(FINE GRAINED),BROWN

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

61.00   2.00    .88    61.00   2.00    .88   CLAYSTONE,REDDISH BROWN

63.00   3.00   1.00    63.00   3.00   1.00   CLAYSTONE,REDDISH BROWN,FRACTURE                                         13

66.00   3.00    .90    66.00   3.00    .90   CLAYSTONE,REDDISH BROWN                                                  20

69.00   3.00   1.08    69.00   3.00   1.08   SANDSTONE(MEDIUM GRAINED),GREY,FRACTURE                                  13

72.00   3.00   1.00    72.00   3.00   1.00   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN,FRACTURE                     13

75.00   3.00   1.28    75.00   3.00   1.28   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN,FRIABLE

78.00   3.00   1.00    78.00   3.00   1.00   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN,FRIABLE

81.00   3.00   1.20    81.00   3.00   1.20   SANDSTONE(MED. TO C. GRAINED),GREY,FRIABLE

84.00   3.00    .91    84.00   3.00    .91   SANDSTONE(MED. TO C. GRAINED),GREY,FRIABLE

87.00   3.00   1.03    87.00   3.00   1.03   SANDSTONE(MEDIUM GRAINED),GREY,FRIABLE

90.00   3.00   1.50    90.00   3.00   1.50   SANDSTONE(MEDIUM GRAINED),GREY

93.00   1.00    .44    93.00   1.00    .44   SANDSTONE(MEDIUM GRAINED),GREY                                           31

94.00    .90    .22    94.00    .90    .22   CLAYSTONE,REDDISH BROWN

94.90   1.10    .65    94.90   1.10    .65   SANDSTONE(MEDIUM GRAINED),GREY                                           32

96.00   3.00   2.87    96.00   3.00   2.87   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   77

99.00   3.00   2.02    99.00   3.00   2.02   SANDSTONE(FINE TO COARSE GRAINED),GREY,FRACTURE                          24

102.00   3.00    .38   102.00   3.00    .38   SANDSTONE(FINE GRAINED),GREY,FRACTURE

105.00   3.00    .25   105.00   3.00    .25   SANDSTONE(FINE GRAINED),GREY

108.00   1.00    .20   108.00   1.00    .20   CLAYSTONE,REDDISH BROWN

109.00   2.00    .18   109.00   2.00    .18   SANDSTONE(FINE GRAINED),GREY

111.00   3.00    .50   111.00   3.00    .50   CLAYSTONE,REDDISH BROWN                                                  10

114.00   3.00   2.72   114.00   3.00   2.72   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN                          26

117.00   3.00   1.50   117.00   3.00   1.50   SANDSTONE(FINE GRAINED),GREY,FRIABLE

120.00   3.00   2.47   120.00   3.00   2.47   SANDSTONE(FINE GRAINED),GREY                                             64

123.00   3.00   2.80   123.00   3.00   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   67

126.00   3.00   2.67   126.00   3.00   2.67   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN                          72

129.00    .65    .49   129.00    .65    .49   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   72

129.65   2.35   2.16   129.65   2.35   2.16   CLAYSTONE,REDDISH BROWN                                                  54

132.00   3.00   2.85   132.00   3.00   2.85   CLAYSTONE,REDDISH BROWN,FRACTURE                                         43

135.00   3.00   2.92   135.00   3.00   2.92   SANDSTONE(FINE GRAINED),RED,FRACTURE                                     43

138.00   3.00   1.94   138.00   3.00   1.94   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    47

141.00   3.00   1.34   141.00   3.00   1.34   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   36

144.00   3.00   1.50   144.00   3.00   1.50   SANDSTONE(FINE GRAINED),GREY,FRIABLE                                     14

147.00   3.00   2.96   147.00   3.00   2.96   SANDSTONE(FINE TO MEDIUM GRAINED),GREY,FRACTURE                          38

150.00   3.00   1.73   150.00   3.00   1.73   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN                          19

153.00   3.00   2.45   153.00   3.00   2.45   SANDSTONE(FINE GRAINED),GREYISH BROWN,FRIABLE                            13

156.00   3.00   1.83   156.00   3.00   1.83   SANDSTONE(FINE GRAINED),GREY                                             21

159.00   3.00   2.94   159.00   3.00   2.94   SANDSTONE(FINE GRAINED),GREYISH BROWN                                    51

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

162.00   3.00   2.98   162.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN                          55

165.00   3.00   3.00   165.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   64

168.00   3.00   3.00   168.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   62

171.00   3.00   2.85   171.00   3.00   2.85   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN,                         51

174.00   3.00   2.95   174.00   3.00   2.95   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN                          25

177.00   3.00   2.80   177.00   3.00   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN                          63

180.00   3.00   2.96   180.00   3.00   2.96   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN                          63

183.00   3.00   2.99   183.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN,                         20

186.00   3.00   3.00   186.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   34

189.00   3.00   1.90   189.00   3.00   1.90   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN,                         12

192.00   3.00   1.80   192.00   3.00   1.80   SANDSTONE(FINE GRAINED),GREY,FRIABLE                                    8.3

195.00   3.00   1.70   195.00   3.00   1.70   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW                               7.3

198.00   3.00   2.50   198.00   3.00   2.50   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW                                40

201.00   3.00   2.60   201.00   3.00   2.60   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW                                37

204.00   1.40   1.33   204.00   1.40   1.33   SANDSTONE(MED. TO V.C. GRAINED),BROWNISH YELLOW                          75

205.40   1.60   1.50   205.40   1.60   1.50   CLAYSTONE,REDDISH BROWN,FRACTURE                                         48

207.00   3.00   2.80   207.00   3.00   2.80   CLAYSTONE,REDDISH BROWN

210.00   2.50   1.30   210.00   2.50   1.30   CLAYSTONE,REDDISH BROWN

212.50   1.50   1.10   212.50   1.50   1.10   CLAYSTONE,REDDISH BROWN

214.00   1.50    .95   214.00   1.50    .95   MUDSTONE,GREY

215.50   3.00   1.50   215.50   3.00   1.50   MUDSTONE,GREY

218.50   1.50   1.40   218.50   1.50   1.40   MUDSTONE,GREY

220.00   3.00   2.90   220.00   3.00   2.90   MUDSTONE,GREY

223.00   2.00   1.95   223.00   2.00   1.95   MUDSTONE,GREY,FRACTURE HIGH ANGLE

225.00   3.00   2.95   225.00   3.00   2.95   MUDSTONE,GREY,FRACTURE HIGH ANGLE

228.00   3.00   2.98   228.00   3.00   2.98   MUDSTONE,BROWNISH GREY,FRACTURE HIGH ANGLE

231.00   3.00   2.98   231.00   3.00   2.98   MUDSTONE,BROWNISH GREY,FRACTURE HIGH ANGLE

234.00   2.00    .40   234.00   2.00    .40   MUDSTONE,GREY,FRACTURE HIGH ANGLE

236.00   1.00    .30   236.00   1.00    .30   MUDSTONE,GREY,FRACTURE HIGH ANGLE

237.00   3.00   1.00   237.00   3.00   1.00   MUDSTONE,GREY,FRACTURE HIGH ANGLE

240.00   2.00   1.65   240.00   2.00   1.65   MUDSTONE,GREY

242.00   3.00   2.60   242.00   3.00   2.60   MUDSTONE,GREY

245.00   2.00   1.50   245.00   2.00   1.50   MUDSTONE,GREY                                                            19

247.00   2.00   1.45   247.00   2.00   1.45   MUDSTONE,GREY,FRACTURE HIGH ANGLE

249.00   2.00   1.50   249.00   2.00   1.50   CLAYSTONE,GREY,FRACTURE HIGH ANGLE

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

251.00   3.00   2.98   251.00   3.00   2.98   CLAYSTONE,GREY,FRACTURE HIGH ANGLE

254.00   2.00   1.70   254.00   2.00   1.70   CLAYSTONE,GREY

256.00    .96    .76   256.00    .96    .76   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              57

256.96   1.04    .84   256.96   1.04    .84   CLAYSTONE,GREY

258.00   2.00   1.95   258.00   2.00   1.95   MUDSTONE,GREY

260.00   1.50    .50   260.00   1.50    .50   SANDSTONE(FINE GRAINED),GREY                                             23

261.50   1.50    .88   261.50   1.50    .88   CLAYSTONE,GREY

263.00   1.00    .90   263.00   1.00    .90   CLAYSTONE,GREY

264.00   3.00   2.80   264.00   3.00   2.80   MUDSTONE,GREY                                                            47

267.00   2.00    .80   267.00   2.00    .80   MUDSTONE,GREY                                                            30

269.00   1.00    .70   269.00   1.00    .70   MUDSTONE,GREY

270.00   3.00   1.50   270.00   3.00   1.50   MUDSTONE,BROWNISH GREY                                                   26

273.00   3.00   2.70   273.00   3.00   2.70   MUDSTONE,BROWNISH GREY

276.00   1.50   1.30   276.00   1.50   1.30   CLAYSTONE,BROWNISH GREY                                                  48

277.50   1.50   1.48   277.50   1.50   1.48   CLAYSTONE,BROWNISH GREY                                                  45

279.00   3.00   2.77   279.00   3.00   2.77   CLAYSTONE,GREYISH BROWN                                                  48

282.00   3.00   2.90   282.00   3.00   2.90   CLAYSTONE,GREY,FRACTURE                                                  40

285.00   2.00    .90   285.00   2.00    .90   MUDSTONE,GREY

287.00   1.00    .54   287.00   1.00    .54   MUDSTONE,GREY

288.00   3.00   1.47   288.00   3.00   1.47   CLAYSTONE,GREYISH BROWN                                                  11

291.00   2.50   1.65   291.00   2.50   1.65   CLAYSTONE,GREYISH BROWN

293.50   3.00   2.50   293.50   3.00   2.50   CLAYSTONE,GREYISH BROWN                                                  18

296.50   2.00   1.60   296.50   2.00   1.60   CLAYSTONE,GREYISH BROWN                                                  57

298.50   2.50   2.45   298.50   2.50   2.45   CLAYSTONE,GREYISH BROWN

301.00   3.00   2.98   301.00   3.00   2.98   CLAYSTONE,BROWNISH GREY                                                  60

304.00   2.00   1.85   304.00   2.00   1.85   CLAYSTONE,BROWNISH GREY                                                  13

306.00   1.50   1.50   306.00   1.50   1.50   CLAYSTONE,BROWNISH GREY

307.50   3.00   2.45   307.50   3.00   2.45   CLAYSTONE,BROWNISH GREY

310.50   2.50   2.47   310.50   2.50   2.47   CLAYSTONE,BROWN

313.00   2.00   1.99   313.00   2.00   1.99   CLAYSTONE,BROWN

315.00   3.00   2.99   315.00   3.00   2.99   CLAYSTONE,GREYISH BROWN

318.00   3.00   2.98   318.00   3.00   2.98   CLAYSTONE,GREYISH BROWN

321.00    .81    .73   321.00    .81    .73   CLAYSTONE,GREY

321.81    .39    .32   321.81    .39    .32   SANDSTONE(FINE GRAINED),GREY

322.20    .80    .72   322.20    .80    .72   CLAYSTONE,GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

323.00   1.00    .99   323.00   1.00    .99   MUDSTONE,GREY

324.00   3.00   2.99   324.00   3.00   2.99   MUDSTONE,GREYISH BROWN,SLICKENSIDED

327.00   3.00   2.98   327.00   3.00   2.98   MUDSTONE,GREY,SLICKENSIDED

330.00   2.00   1.97   330.00   2.00   1.97   CLAYSTONE,BROWNISH RED

332.00   1.00    .99   332.00   1.00    .99   CLAYSTONE,BROWNISH RED

333.00   3.00   2.99   333.00   3.00   2.99   CLAYSTONE,BROWNISH RED

336.00   3.00   2.99   336.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

339.00   3.00   2.99   339.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

342.00   3.00   2.96   342.00   3.00   2.96   CLAYSTONE,BROWNISH GREY

345.00   3.00   2.98   345.00   3.00   2.98   CLAYSTONE,BROWNISH GREY

348.00   3.00   2.99   348.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

351.00   3.00   2.99   351.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

354.00   3.00   2.99   354.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

357.00   2.22   2.20   357.00   2.22   2.20   CLAYSTONE,BROWNISH GREY

359.22    .78    .76   359.22    .78    .76   SANDSTONE(MED. TO C. GRAINED),BROWN

360.00    .20    .20   360.00    .20    .20   SANDSTONE(MED. TO C. GRAINED),GREY

360.20   2.80   2.79   360.20   2.80   2.79   CLAYSTONE,BROWNISH RED

363.00   3.00   2.99   363.00   3.00   2.99   CLAYSTONE,BROWNISH RED

366.00   3.00   2.99   366.00   3.00   2.99   CLAYSTONE,BROWNISH RED,SLICKENSIDED

369.00   3.00   2.98   369.00   3.00   2.98   CLAYSTONE,BROWNISH GREY

372.00   2.00   1.98   372.00   2.00   1.98   CLAYSTONE,BROWNISH GREY

374.00   1.00    .98   374.00   1.00    .98   CLAYSTONE,GREY

375.00   3.00   2.99   375.00   3.00   2.99   MUDSTONE,GREYISH BROWN,SLICKENSIDED

378.00   3.00   2.98   378.00   3.00   2.98   CLAYSTONE,BROWNISH RED,SLICKENSIDED

381.00   3.00   2.99   381.00   3.00   2.99   CLAYSTONE,BROWNISH RED

384.00   3.00   2.98   384.00   3.00   2.98   CLAYSTONE,BROWNISH RED

387.00   1.60   1.60   387.00   1.60   1.60   CLAYSTONE,BROWNISH RED

388.60    .70    .70   388.60    .70    .70   SANDSTONE(MED. TO C. GRAINED),BROWN

389.30    .70    .70   389.30    .70    .70   CLAYSTONE,BROWNISH RED

390.00   3.00   2.98   390.00   3.00   2.98   CLAYSTONE,BROWNISH RED,SLICKENSIDED

393.00   3.00   2.98   393.00   3.00   2.98   CLAYSTONE,BROWNISH GREY,SLICKENSIDED

396.00   3.00   2.99   396.00   3.00   2.99   CLAYSTONE,BROWNISH GREY,SLICKENSIDED

399.00   3.00   2.97   399.00   3.00   2.97   CLAYSTONE,BROWNISH GREY,SLICKENSIDED

402.00   3.00   2.98   402.00   3.00   2.98   CLAYSTONE,BROWNISH GREY,SLICKENSIDED

405.00   1.28   1.28   405.00   1.28   1.28   CLAYSTONE,BROWNISH GREY,SLICKENSIDED

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

406.28   1.72   1.72   406.28   1.72   1.72   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

408.00   1.35   1.35   408.00   1.35   1.35   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

409.35    .65    .62   409.35    .65    .62   MUDSTONE,GREY

410.00   3.00   2.99   410.00   3.00   2.99   CLAYSTONE,BROWNISH RED

413.00   3.00   2.99   413.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

416.00   3.00   2.99   416.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

419.00   3.00   2.99   419.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

422.00   1.00   1.00   422.00   1.00   1.00   CLAYSTONE,BROWNISH RED

423.00   2.32   2.30   423.00   2.32   2.30   CLAYSTONE,BROWNISH RED

425.32    .31    .29   425.32    .31    .29   SANDSTONE(MEDIUM GRAINED),GREY

425.63    .37    .35   425.63    .37    .35   MUDSTONE,GREY

426.00    .50    .50   426.00    .50    .50   SANDSTONE(MED. TO C. GRAINED),GREY

426.50   2.50   2.48   426.50   2.50   2.48   CLAYSTONE,BROWNISH RED

429.00    .20    .19   429.00    .20    .19   CLAYSTONE,BROWNISH RED

429.20    .95    .93   429.20    .95    .93   SANDSTONE(MED. TO C. GRAINED),BROWN

430.15   1.85   1.83   430.15   1.85   1.83   CLAYSTONE,VARIEGATE

432.00   2.00   1.99   432.00   2.00   1.99   CLAYSTONE,VARIEGATE

434.00   3.00   2.99   434.00   3.00   2.99   CLAYSTONE,VARIEGATE

437.00   3.00   2.99   437.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

440.00    .80    .78   440.00    .80    .78   CLAYSTONE,BROWNISH GREY

440.80    .50    .47   440.80    .50    .47   SANDSTONE(MEDIUM GRAINED),GREY

441.30    .90    .88   441.30    .90    .88   CLAYSTONE,BROWN

442.20    .80    .78   442.20    .80    .78   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY

443.00   1.00   1.00   443.00   1.00   1.00   SANDSTONE(MEDIUM GRAINED),VARIEGATE

444.00    .30    .28   444.00    .30    .28   SANDSTONE(MED. TO C. GRAINED),BROWN

444.30    .70    .70   444.30    .70    .70   CLAYSTONE,BROWNISH GREY

445.00   1.35   1.33   445.00   1.35   1.33   SANDSTONE(MED. TO C. GRAINED),GREY

446.35    .45    .43   446.35    .45    .43   SHALE,GREY

446.80    .20    .20   446.80    .20    .20   CLAYSTONE,BROWN

447.00    .55    .55   447.00    .55    .55   CLAYSTONE,BROWN,SLICKENSIDED

447.55    .30    .30   447.55    .30    .30   SANDY SHALE,GREY

447.85   2.15   2.12   447.85   2.15   2.12   CLAYSTONE,GREYISH BROWN

450.00    .74    .72   450.00    .74    .72   CLAYSTONE,BROWN

450.74    .60    .58   450.74    .60    .58   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY

451.34   1.66   1.64   451.34   1.66   1.64   CLAYSTONE,BROWNISH GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

453.00    .25    .22   453.00    .25    .22   SANDSTONE(MEDIUM GRAINED),GREY

453.25   2.75   2.75   453.25   2.75   2.75   CLAYSTONE,GREY

456.00   3.00   2.99   456.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

459.00    .15    .12   459.00    .15    .12   CLAYSTONE,BROWNISH GREY

459.15   2.85   2.85   459.15   2.85   2.85   SANDSTONE(MED. TO C. GRAINED),GREY

462.00   2.00   1.90   462.00   2.00   1.90   SANDSTONE(COARSE TO V.C.GRAINED),GREY

464.00   1.00    .90   464.00   1.00    .90   CLAYSTONE,GREYISH BROWN

465.00   2.00   1.99   465.00   2.00   1.99   CLAYSTONE,BROWNISH GREY

467.00   1.00   1.00   467.00   1.00   1.00   CLAYSTONE,BROWNISH GREY

468.00   1.56   1.54   468.00   1.56   1.54   CLAYSTONE,BROWNISH GREY

469.56    .62    .60   469.56    .62    .60   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

470.18    .82    .80   470.18    .82    .80   CLAYSTONE,GREYISH BROWN

471.00   3.00   2.97   471.00   3.00   2.97   CLAYSTONE,BROWNISH RED

474.00   1.58   1.58   474.00   1.58   1.58   CLAYSTONE,BROWNISH RED

475.58    .35    .34   475.58    .35    .34   SANDSTONE(FINE GRAINED),GREY

475.93   1.07   1.07   475.93   1.07   1.07   CLAYSTONE,BROWNISH RED

477.00   1.00   1.00   477.00   1.00   1.00   CLAYSTONE,BROWNISH RED,SLICKENSIDED

478.00   2.00   1.95   478.00   2.00   1.95   CLAYSTONE,BROWNISH RED,SLICKENSIDED

480.00   3.00   2.99   480.00   3.00   2.99   CLAYSTONE,BROWNISH RED

483.00   3.00   2.98   483.00   3.00   2.98   CLAYSTONE,BROWNISH RED

486.00    .81    .80   486.00    .81    .80   CLAYSTONE,BROWNISH RED

486.81   2.19   2.18   486.81   2.19   2.18   SANDSTONE(MED. TO C. GRAINED),GREY

489.00    .74    .73   489.00    .74    .73   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN

489.74   2.26   2.26   489.74   2.26   2.26   CLAYSTONE,BROWNISH RED

492.00   2.30   2.30   492.00   2.30   2.30   CLAYSTONE,BROWNISH RED

494.30    .70    .70   494.30    .70    .70   SANDSTONE(MEDIUM GRAINED),GREYISH BROWN

495.00    .30    .30   495.00    .30    .30   CLAYSTONE,BROWN

495.30   2.28   2.27   495.30   2.28   2.27   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN

497.58    .42    .41   497.58    .42    .41   CLAYSTONE,BROWN,SLICKENSIDED

498.00    .30    .30   498.00    .30    .30   CLAYSTONE,BROWN

498.30    .30    .30   498.30    .30    .30   SANDSTONE(COARSE GRAINED),BROWN

498.60   1.40   1.40   498.60   1.40   1.40   CLAYSTONE,BROWN

500.00   1.00   1.00   500.00   1.00   1.00   CLAYSTONE,BROWN

501.00   3.00   2.99   501.00   3.00   2.99   CLAYSTONE,BROWNISH RED

504.00   3.00   2.97   504.00   3.00   2.97   CLAYSTONE,BROWNISH RED,FRACTURE

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

507.00   2.00   1.96   507.00   2.00   1.96   CLAYSTONE,BROWNISH RED

509.00   1.00   1.00   509.00   1.00   1.00   CLAYSTONE,BROWNISH RED

510.00   3.00   2.98   510.00   3.00   2.98   CLAYSTONE,BROWNISH RED

513.00    .40    .39   513.00    .40    .39   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY

513.40   2.60   2.60   513.40   2.60   2.60   CLAYSTONE,BROWNISH RED,FRACTURE

516.00   3.00   2.90   516.00   3.00   2.90   CLAYSTONE,BROWNISH RED

519.00   2.00   2.00   519.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN

521.00   1.00   1.00   521.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN

522.00   2.00    .67   522.00   2.00    .67   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN

524.00   3.00   2.98   524.00   3.00   2.98   CLAYSTONE,BROWN

527.00   1.00   1.00   527.00   1.00   1.00   CLAYSTONE,BROWN

528.00    .97    .97   528.00    .97    .97   CLAYSTONE,BROWN

528.97    .43    .43   528.97    .43    .43   SANDSTONE(FINE GRAINED),GREYISH BROWN

529.40    .60    .60   529.40    .60    .60   CLAYSTONE,BROWN

530.00   3.00   2.99   530.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN

533.00   3.00   2.99   533.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN

536.00   1.00    .97   536.00   1.00    .97   CLAYSTONE,CHOCOLATEBROWN

537.00   2.00   1.94   537.00   2.00   1.94   CLAYSTONE,CHOCOLATEBROWN

539.00   3.00   2.96   539.00   3.00   2.96   CLAYSTONE,CHOCOLATEBROWN

542.00   3.00   1.99   542.00   3.00   1.99   CLAYSTONE,CHOCOLATEBROWN

545.00   1.00    .97   545.00   1.00    .97   CLAYSTONE,CHOCOLATEBROWN

546.00   2.00   1.95   546.00   2.00   1.95   CLAYSTONE,CHOCOLATEBROWN

548.00   3.00   1.56   548.00   3.00   1.56   CLAYSTONE,CHOCOLATEBROWN

551.00   1.00    .98   551.00   1.00    .98   CLAYSTONE,BROWNISH RED

552.00   2.00   2.00   552.00   2.00   2.00   CLAYSTONE,BROWNISH RED

554.00   3.00   2.99   554.00   3.00   2.99   CLAYSTONE,VARIEGATE

557.00   2.00   1.90   557.00   2.00   1.90   CLAYSTONE,VARIEGATE

559.00   1.00    .96   559.00   1.00    .96   CLAYSTONE,VARIEGATE

560.00   2.00   1.99   560.00   2.00   1.99   CLAYSTONE,VARIEGATE

562.00   3.00   2.99   562.00   3.00   2.99   CLAYSTONE,VARIEGATE

565.00   3.00   2.97   565.00   3.00   2.97   CLAYSTONE,VARIEGATE

568.00   1.00    .98   568.00   1.00    .98   CLAYSTONE,VARIEGATE

569.00   1.00    .98   569.00   1.00    .98   CLAYSTONE,VARIEGAT,SLICKENSIDED

570.00   3.00   2.99   570.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN

573.00   3.00   2.99   573.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

576.00   3.00   2.98   576.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN

579.00   3.00   2.96   579.00   3.00   2.96   CLAYSTONE,CHOCOLATEBROWN

582.00   2.00   1.95   582.00   2.00   1.95   CLAYSTONE,CHOCOLATEBROWN

584.00   1.00    .99   584.00   1.00    .99   CLAYSTONE,CHOCOLATEBROWN

585.00   3.00   2.94   585.00   3.00   2.94   CLAYSTONE,BROWNISH GREY

588.00   1.49   1.48   588.00   1.49   1.48   SANDSTONE(MEDIUM GRAINED),GREYISH BROWN

589.49    .46    .45   589.49    .46    .45   CLAYSTONE,CHOCOLATEBROWN

589.95   1.05   1.04   589.95   1.05   1.04   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

591.00   3.00   2.98   591.00   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN

594.00   1.00    .95   594.00   1.00    .95   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN

595.00   3.00   1.88   595.00   3.00   1.88   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN

598.00   3.00   1.90   598.00   3.00   1.90   SANDSTONE(COARSE GRAINED),BROWN

601.00   3.00   1.50   601.00   3.00   1.50   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH GREY

604.00   3.00   1.38   604.00   3.00   1.38   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY

607.00   3.00   1.95   607.00   3.00   1.95   SANDSTONE(COARSE GRAINED),BROWNISH GREY

610.00   2.00   1.90   610.00   2.00   1.90   SANDSTONE(FINE TO COARSE GRAINED),BROWNISH GREY

612.00   1.50    .40   612.00   1.50    .40   SANDSTONE(COARSE TO V.C.GRAINED),GREYISH GREEN

613.50   1.50   1.05   613.50   1.50   1.05   MUDSTONE,GREY,SLICKENSIDED

615.00   1.58   1.53   615.00   1.58   1.53   MUDSTONE,GREYISH GREEN

*******************************************************************************************************************************

BARAKAR

616.58   1.42   1.37   616.58   1.42   1.37   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN

618.00   3.00   2.96   618.00   3.00   2.96   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

621.00   3.00   2.96   621.00   3.00   2.96   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN

624.00   1.96   1.96   624.00   1.96   1.96   SANDSTONE(COARSE GRAINED),GREY

625.96    .57    .56   625.96    .57    .56   SANDY SHALE,GREY

626.53    .47    .47   626.53    .47    .47   SANDSTONE(MEDIUM GRAINED),GREY

627.00    .27    .24   627.00    .27    .24   SANDSTONE(MED. TO C. GRAINED),GREY

627.27    .43    .40   627.27    .43    .40   SANDY SHALE,GREY

627.70   2.30   2.27   627.70   2.30   2.27   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

630.00   3.00   2.95   630.00   3.00   2.95   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

633.00   1.00    .65   633.00   1.00    .65   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

634.00   2.00   2.00   634.00   2.00   2.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

636.00   3.00   2.99   636.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

639.00   3.00   2.99   639.00   3.00   2.99   SANDSTONE(COARSE TO V.C.GRAINED),GREYISH WHITE

642.00   3.00   1.40   642.00   3.00   1.40   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

645.00   3.00   1.00   645.00   3.00   1.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

648.00   1.00    .98   648.00   1.00    .98   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

649.00   2.00   1.98   649.00   2.00   1.98   SANDSTONE(FINE GRAINED),GREYISH WHITE

651.00   3.00   2.99   651.00   3.00   2.99   SANDSTONE(FINE GRAINED),GREYISH WHITE

654.00   3.00   2.99   654.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

657.00   3.00   2.97   657.00   3.00   2.97   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,

660.00   3.00   2.99   660.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

663.00   3.00   2.98   663.00   3.00   2.98   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE

666.00   3.00   2.97   666.00   3.00   2.97   SANDSTONE(COARSE TO V.C.GRAINED),GREYISH WHITE

669.00   1.02   1.00   669.00   1.02   1.00   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE

670.02    .99    .96   670.02    .99    .96   COAL                                                            IIA

671.01    .99    .97   671.01    .99    .97   SHALY COAL                                                      IIA

672.00    .43    .42   672.00    .43    .42   SANDSTONE(FINE GRAINED),GREY

672.43   2.42   2.40   672.43   2.42   2.40   INTERCALATIONS,GREY

674.85    .15    .10   674.85    .15    .10   SHALY COAL                                                      L

675.00    .50    .48   675.00    .50    .48   INTERCALATIONS,GREY

675.50    .20    .20   675.50    .20    .20   CARB SHALE LOW                                                  L

675.70   2.30   2.28   675.70   2.30   2.28   SANDSTONE(MED. TO C. GRAINED),GREY

678.00   3.00   2.99   678.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

681.00   1.84   1.84   681.00   1.84   1.84   SANDSTONE(COARSE TO V.C.GRAINED),GREYISH WHITE

682.84    .42    .40   682.84    .42    .40   COAL                                                            IIB

683.26    .37    .37   683.26    .37    .37   COAL                                                            IIB

683.63    .37    .37   683.63    .37    .37   COAL                                                            IIB

684.00    .45    .45   684.00    .45    .45   COAL                                                            IIB

684.45    .35    .35   684.45    .35    .35   COAL                                                            IIB

684.80    .46    .46   684.80    .46    .46   COAL                                                            IIB

685.26    .44    .44   685.26    .44    .44   SANDY SHALE,GREY

685.70   1.30   1.30   685.70   1.30   1.30   SANDSTONE(FINE GRAINED),GREYISH WHITE

687.00   3.00   2.99   687.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

690.00   3.00   2.98   690.00   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

693.00   3.00   2.99   693.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

696.00   2.50   2.50   696.00   2.50   2.50   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

698.50    .25    .25   698.50    .25    .25   COAL                                                            I

698.75    .25    .25   698.75    .25    .25   COAL                                                            I

699.00    .38    .37   699.00    .38    .37   SHALE,GREY                                                               14

699.38   2.62   2.62   699.38   2.62   2.62   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

702.00   3.00   2.99   702.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

BOREHOLE CLOSED AT   705.00 M DEPTH

S/W DEVELOPED BY :

GEOSYSTEM , EXPLORATION ,CMPDI(HQ) ,RANCHI

---------------------------------------------------------------------------

BOREHOLE NO    :CMWWB10              LATITUDE    : 2206241.004M                 DATE OF COMMENCEMENT : 18.08.2024

REDUCED LEVEL  : 305.6964M           DEPARTURE   : 266086.0171M                 DATE OF COMPLETION: 18.09.2024

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

.00    .70    .20      .00    .70    .20   SOIL,BLACK

.70   2.30    .55      .70   2.30    .55   SOIL,BLACK

SOIL

******************************************************************************************************************************

TRAP

3.00   1.00   1.00     3.00   1.00   1.00   BASALT,GREYISH BROWN,FRACTURE

4.00   2.00   1.98     4.00   2.00   1.98   BASALT,GREYISH BROWN,FRACTURE                                            44

6.00   3.00   2.38     6.00   3.00   2.38   BASALT,GREY                                                              53

9.00   2.00   1.95     9.00   2.00   1.95   BASALT,GREY                                                              82

11.00   1.00   1.00    11.00   1.00   1.00   BASALT,GREY                                                              83

12.00   3.00   3.00    12.00   3.00   3.00   BASALT,GREY                                                              80

15.00   3.00   2.99    15.00   3.00   2.99   BASALT,GREY                                                              72

18.00   3.00   3.00    18.00   3.00   3.00   BASALT,GREY,FRACTURE                                                     64

21.00   3.00   2.99    21.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     69

24.00   3.00   3.00    24.00   3.00   3.00   BASALT,GREY                                                              64

27.00   3.00   3.00    27.00   3.00   3.00   BASALT,GREY,FRACTURE                                                     74

30.00   3.00   3.00    30.00   3.00   3.00   BASALT,GREY                                                              61

33.00   3.00   2.90    33.00   3.00   2.90   BASALT,GREY                                                              91

36.00   3.00   2.99    36.00   3.00   2.99   BASALT,GREY                                                              47

39.00   3.00   3.00    39.00   3.00   3.00   BASALT,GREY                                                              77

42.00   3.00   2.98    42.00   3.00   2.98   BASALT,GREY                                                              77

45.00   3.00   2.96    45.00   3.00   2.96   BASALT,GREY                                                              62

48.00   3.00   2.99    48.00   3.00   2.99   BASALT,GREY                                                              62

51.00   3.00   2.98    51.00   3.00   2.98   BASALT,GREY                                                              75

54.00   3.00   2.99    54.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     53

57.00   3.00   2.95    57.00   3.00   2.95   BASALT,GREYISH GREEN                                                     63

60.00   3.00   2.90    60.00   3.00   2.90   BASALT,GREY                                                              84

63.00   1.50   1.48    63.00   1.50   1.48   BASALT,GREY                                                              77

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

64.50   1.50   1.49    64.50   1.50   1.49   BASALT,GREY                                                              72

66.00   3.00   3.00    66.00   3.00   3.00   BASALT,GREY                                                              79

69.00   2.60   2.60    69.00   2.60   2.60   BASALT,GREY                                                              60

71.60    .40    .40    71.60    .40    .40   CLAYSTONE,REDDISH BROWN

72.00    .35    .30    72.00    .35    .30   CLAYSTONE,REDDISH BROWN

72.35   2.65   2.65    72.35   2.65   2.65   BASALT,REDDISH BROWN                                                     56

75.00   3.00   2.99    75.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     63

78.00   3.00   2.99    78.00   3.00   2.99   BASALT,GREY                                                              86

81.00   3.00   2.99    81.00   3.00   2.99   BASALT,GREY,FRACTURE                                                     54

84.00   3.00   2.98    84.00   3.00   2.98   BASALT,GREY,FRACTURE                                                     68

87.00   3.00   2.98    87.00   3.00   2.98   BASALT,GREY                                                              80

90.00   3.00   3.00    90.00   3.00   3.00   BASALT,GREY                                                              60

**************************************************************************************************************************

MOTUR

93.00   3.00   2.90    93.00   3.00   2.90   CLAYSTONE,GREYISH BROWN

96.00   3.00   1.60    96.00   3.00   1.60   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN

99.00   3.00   1.20    99.00   3.00   1.20   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                             6.3

102.00   1.00   1.00   102.00   1.00   1.00   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                              79

103.00   2.00   1.88   103.00   2.00   1.88   CLAYSTONE,REDDISH BROWN                                                   6

105.00   3.00   2.90   105.00   3.00   2.90   CLAYSTONE,REDDISH BROWN

108.00   3.00   2.99   108.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),REDDISH BROWN                              36

111.00   3.00   2.99   111.00   3.00   2.99   SANDSTONE(VERY COARSE GRAINED),YELLOWISHBROWN                            62

114.00   3.00   3.00   114.00   3.00   3.00   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHBROWN                          54

117.00   3.00   2.99   117.00   3.00   2.99   SANDSTONE(VERY COARSE GRAINED),YELLOWISHBROWN                            32

120.00   1.00   1.00   120.00   1.00   1.00   CLAYSTONE,GREYISH BROWN

121.00   2.00   2.00   121.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN                                                 85

123.00   3.00   3.00   123.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 19

126.00   1.65   1.60   126.00   1.65   1.60   SANDSTONE(COARSE TO V.C.GRAINED),CHOCOLATEBROWN                          23

127.65   1.35   1.30   127.65   1.35   1.30   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHBROWN                          27

129.00   3.00   2.90   129.00   3.00   2.90   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHBROWN                          38

132.00   2.20   2.20   132.00   2.20   2.20   SANDSTONE(MED. TO C. GRAINED),CHOCOLATEBROWN                             41

134.20    .80    .80   134.20    .80    .80   SANDSTONE(COARSE GRAINED),YELLOWISHBROWN                                 54

135.00    .60    .30   135.00    .60    .30   CLAYSTONE,YELLOWISHBROWN

135.60   1.60   1.50   135.60   1.60   1.50   SANDSTONE(MEDIUM GRAINED),CHOCOLATEBROWN                                 53

137.20    .80    .74   137.20    .80    .74   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN

138.00   1.00    .85   138.00   1.00    .85   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN                                 71

139.00   2.00   1.90   139.00   2.00   1.90   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

141.00   3.00   2.20   141.00   3.00   2.20   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

144.00   2.00   1.50   144.00   2.00   1.50   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    25

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

146.00   1.00    .80   146.00   1.00    .80   CLAYSTONE,CHOCOLATEBROWN

147.00   3.00   1.80   147.00   3.00   1.80   CLAYSTONE,CHOCOLATEBROWN

150.00   3.00   2.99   150.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),CHOCOLATEBROWN                         71

153.00   3.00   2.97   153.00   3.00   2.97   SANDSTONE(FINE TO MEDIUM GRAINED),CHOCOLATEBROWN                         85

156.00   3.00   2.95   156.00   3.00   2.95   CLAYSTONE,CHOCOLATEBROWN                                                 76

159.00   3.00   2.97   159.00   3.00   2.97   SANDSTONE(FINE TO MEDIUM GRAINED),CHOCOLATEBROWN                         41

162.00   3.00   2.98   162.00   3.00   2.98   SANDSTONE(COARSE TO V.C.GRAINED),CHOCOLATEBROWN,                         64

165.00   3.00   2.99   165.00   3.00   2.99   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH YELLOW                         74

168.00   3.00   2.99   168.00   3.00   2.99   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH YELLOW                         83

171.00   3.00   2.95   171.00   3.00   2.95   SANDSTONE(MED. TO C. GRAINED),BROWN                                      56

174.00   3.00   1.00   174.00   3.00   1.00   SANDSTONE(MED. TO C. GRAINED),BROWN                                      26

177.00   3.00   1.10   177.00   3.00   1.10   SANDSTONE(COARSE TO V.C.GRAINED),BROWN                                    4

180.00    .92    .28   180.00    .92    .28   SANDSTONE(COARSE TO V.C.GRAINED),BROWN

180.92    .95    .30   180.92    .95    .30   SANDSTONE(MED. TO C. GRAINED),CHOCOLATEBROWN

181.87   1.13    .43   181.87   1.13    .43   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            38

183.00   3.00   2.20   183.00   3.00   2.20   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            31

186.00   3.00   1.90   186.00   3.00   1.90   SANDSTONE(COARSE TO V.C.GRAINED),BROWN                                   33

189.00   3.00   1.90   189.00   3.00   1.90   SANDSTONE(COARSE TO V.C.GRAINED),BROWN                                   20

192.00   3.00   2.00   192.00   3.00   2.00   SANDSTONE(COARSE TO V.C.GRAINED),CHOCOLATEBROWN

195.00   3.00   1.50   195.00   3.00   1.50   SANDSTONE(COARSE TO V.C.GRAINED),CHOCOLATEBROWN                          47

198.00   1.00    .52   198.00   1.00    .52   SANDSTONE(COARSE TO V.C.GRAINED),CHOCOLATEBROWN                          50

199.00    .90    .35   199.00    .90    .35   SANDSTONE(VERY COARSE GRAINED),CHOCOLATEBROWN

199.90   1.10    .37   199.90   1.10    .37   SANDSTONE(COARSE GRAINED),BROWNISH YELLOW                                18

201.00    .60    .19   201.00    .60    .19   SANDSTONE(COARSE GRAINED),YELLOWISHBROWN                                 20

201.60   1.55   1.25   201.60   1.55   1.25   SANDSTONE(MEDIUM GRAINED),CHOCOLATEBROWN                                 34

203.15    .85    .48   203.15    .85    .48   SANDSTONE(MEDIUM GRAINED),BROWNISH YELLOW                                35

204.00   3.00   2.80   204.00   3.00   2.80   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            53

207.00   3.00   2.90   207.00   3.00   2.90   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW,                           63

210.00    .79    .73   210.00    .79    .73   SANDSTONE(MED. TO C. GRAINED),YELLOWISHBROWN                             32

210.79    .75    .68   210.79    .75    .68   SANDSTONE(MEDIUM GRAINED),CHOCOLATEBROWN                                 77

211.54   1.46   1.40   211.54   1.46   1.40   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN                                 93

213.00   2.00   1.99   213.00   2.00   1.99   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN                                 92

215.00   1.00    .80   215.00   1.00    .80   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN                                 73

216.00   3.00   2.90   216.00   3.00   2.90   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN                                 85

219.00   3.00   2.95   219.00   3.00   2.95   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN,FRACTURE                        32

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

222.00   3.00   2.99   222.00   3.00   2.99   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN,FRACTURE                        58

225.00   2.00   2.00   225.00   2.00   2.00   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN                                 66

227.00   1.00   1.00   227.00   1.00   1.00   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN                                 71

228.00   3.00   2.99   228.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN,                        47

231.00   3.00   2.98   231.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN,                        69

234.00   3.00   2.98   234.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN,                        28

237.00   3.00   3.00   237.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN                         68

240.00    .37    .36   240.00    .37    .36   SANDSTONE(FINE TO MEDIUM GRAINED),CHOCOLATEBROWN                         92

240.37   2.63   2.62   240.37   2.63   2.62   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN,                        17

243.00   3.00   2.98   243.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN                         41

246.00   3.00   2.98   246.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        76

249.00   3.00   2.99   249.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        87

252.00   3.00   2.99   252.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW,                       38

255.00   3.00   2.98   255.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        91

258.00   3.00   2.97   258.00   3.00   2.97   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        63

261.00   3.00   2.99   261.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        62

264.00   3.00   2.99   264.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        75

267.00   3.00   2.99   267.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        46

270.00   3.00   1.50   270.00   3.00   1.50   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        13

273.00   3.00   2.99   273.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        79

276.00   3.00   2.99   276.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        55

279.00   3.00   1.40   279.00   3.00   1.40   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW,                       14

282.00   3.00    .60   282.00   3.00    .60   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW

285.00   3.00    .80   285.00   3.00    .80   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

288.00   3.00    .65   288.00   3.00    .65   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

291.00   3.00    .35   291.00   3.00    .35   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

294.00   3.00    .50   294.00   3.00    .50   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

297.00   3.00    .80   297.00   3.00    .80   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

300.00   3.00    .30   300.00   3.00    .30   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                             4

303.00   3.00    .80   303.00   3.00    .80   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH YELLOW                         17

306.00   3.00   1.05   306.00   3.00   1.05   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH YELLOW                         32

309.00   2.00    .85   309.00   2.00    .85   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH YELLOW                         39

311.00   1.00    .20   311.00   1.00    .20   CLAYSTONE,CHOCOLATEBROWN

312.00   3.00   2.95   312.00   3.00   2.95   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    75

315.00   3.00   2.98   315.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 40

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

318.00   3.00   2.99   318.00   3.00   2.99   MUDSTONE,GREYISH BROWN,FRACTURE                                          25

321.00   3.00   2.98   321.00   3.00   2.98   MUDSTONE,GREY,FRACTURE

324.00   3.00   2.95   324.00   3.00   2.95   MUDSTONE,GREY,FRACTURE

327.00   2.00   1.95   327.00   2.00   1.95   MUDSTONE,GREYISH BROWN,FRACTURE

329.00   1.00   1.00   329.00   1.00   1.00   MUDSTONE,GREYISH BROWN,FRACTURE

330.00   3.00   2.90   330.00   3.00   2.90   MUDSTONE,GREY,FRACTURE

333.00   3.00   2.99   333.00   3.00   2.99   MUDSTONE,GREYISH BROWN,FRACTURE

336.00   3.00   2.99   336.00   3.00   2.99   MUDSTONE,BROWNISH GREY                                                   21

339.00   3.00   2.98   339.00   3.00   2.98   MUDSTONE,GREY                                                            87

342.00   3.00   2.99   342.00   3.00   2.99   MUDSTONE,GREY                                                            49

345.00   3.00   2.98   345.00   3.00   2.98   MUDSTONE,GREY

348.00   2.00   1.70   348.00   2.00   1.70   MUDSTONE,GREY

350.00   1.00    .99   350.00   1.00    .99   MUDSTONE,GREYISH BROWN

351.00    .70    .70   351.00    .70    .70   CLAYSTONE,GREYISH BROWN,FRACTURE

351.70    .30    .30   351.70    .30    .30   SANDSTONE(FINE GRAINED),GREYISH BROWN                                    70

352.00   2.00   1.99   352.00   2.00   1.99   CLAYSTONE,BROWNISH GREY

354.00   3.00   2.99   354.00   3.00   2.99   MUDSTONE,GREY

357.00   1.43   1.40   357.00   1.43   1.40   SANDSTONE(MED. TO C. GRAINED),GREY                                       64

358.43   1.57   1.55   358.43   1.57   1.55   MUDSTONE,GREY,FRACTURE                                                   22

360.00   3.00   2.98   360.00   3.00   2.98   MUDSTONE,GREY                                                            37

363.00   3.00   2.99   363.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

366.00   3.00   2.99   366.00   3.00   2.99   CLAYSTONE,BROWNISH GREY

369.00   3.00   2.98   369.00   3.00   2.98   CLAYSTONE,BROWNISH GREY

372.00   1.65   1.55   372.00   1.65   1.55   CLAYSTONE,BROWN

373.65    .56    .46   373.65    .56    .46   MUDSTONE,GREY                                                            23

374.21    .79    .70   374.21    .79    .70   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              73

375.00   1.00   1.00   375.00   1.00   1.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              87

376.00   2.00   1.00   376.00   2.00   1.00   MUDSTONE,GREY,FRACTURE

378.00   3.00    .85   378.00   3.00    .85   MUDSTONE,GREY,FRACTURE

381.00   1.50    .70   381.00   1.50    .70   MUDSTONE,GREY,FRACTURE

382.50   1.50   1.20   382.50   1.50   1.20   CLAYSTONE,GREYISH BROWN,FRACTURE

384.00   3.00   2.40   384.00   3.00   2.40   CLAYSTONE,GREYISH BROWN,FRACTURE

387.00   3.00   2.00   387.00   3.00   2.00   CLAYSTONE,BROWNISH GREY                                                 5.7

390.00   3.00   2.00   390.00   3.00   2.00   CLAYSTONE,BROWNISH GREY                                                  48

393.00   3.00   1.50   393.00   3.00   1.50   CLAYSTONE,BROWNISH GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

396.00   3.00   2.45   396.00   3.00   2.45   CLAYSTONE,CHOCOLATEBROWN

399.00   3.00   2.95   399.00   3.00   2.95   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

402.00   1.50   1.50   402.00   1.50   1.50   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

403.50   1.50   1.50   403.50   1.50   1.50   CLAYSTONE,BROWNISH GREY,FRACTURE

405.00   3.00   2.95   405.00   3.00   2.95   CLAYSTONE,BROWNISH GREY                                                  25

408.00   3.00   2.99   408.00   3.00   2.99   MUDSTONE,GREYISH BROWN                                                   45

411.00   3.00   2.98   411.00   3.00   2.98   CLAYSTONE,BROWN                                                          19

414.00   3.00   2.95   414.00   3.00   2.95   CLAYSTONE,BROWN

417.00   3.00   2.90   417.00   3.00   2.90   CLAYSTONE,CHOCOLATEBROWN

420.00   3.00   2.99   420.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 32

423.00   3.00   2.90   423.00   3.00   2.90   CLAYSTONE,CHOCOLATEBROWN                                                 19

426.00   1.50   1.50   426.00   1.50   1.50   CLAYSTONE,CHOCOLATEBROWN                                                 28

427.50   1.50   1.45   427.50   1.50   1.45   CLAYSTONE,CHOCOLATEBROWN

429.00   3.00   2.95   429.00   3.00   2.95   CLAYSTONE,CHOCOLATEBROWN

432.00   3.00   2.90   432.00   3.00   2.90   CLAYSTONE,CHOCOLATEBROWN

435.00   3.00   2.98   435.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 11

438.00   3.00   2.95   438.00   3.00   2.95   CLAYSTONE,CHOCOLATEBROWN                                                 11

441.00   3.00   2.98   441.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN

444.00   3.00   2.98   444.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN

447.00   3.00   2.98   447.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN

450.00   2.00   2.00   450.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN

452.00   1.00   1.00   452.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN

453.00   3.00   2.90   453.00   3.00   2.90   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                         9

456.00   3.00   2.95   456.00   3.00   2.95   CLAYSTONE,CHOCOLATEBROWN                                                 34

459.00   3.00   2.90   459.00   3.00   2.90   CLAYSTONE,CHOCOLATEBROWN                                                 25

462.00   3.00   2.99   462.00   3.00   2.99   CLAYSTONE,GREYISH BROWN                                                  22

465.00   3.00   2.95   465.00   3.00   2.95   CLAYSTONE,GREYISH BROWN

468.00   1.00    .90   468.00   1.00    .90   CLAYSTONE,GREYISH BROWN                                                  21

469.00   2.00   1.99   469.00   2.00   1.99   CLAYSTONE,GREYISH BROWN,FRACTURE

471.00   1.60   1.57   471.00   1.60   1.57   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              20

472.60    .58    .55   472.60    .58    .55   MUDSTONE,GREY

473.18    .82    .78   473.18    .82    .78   SANDSTONE(MED. TO C. GRAINED),GREY                                       66

474.00   1.77   1.75   474.00   1.77   1.75   SANDSTONE(MED. TO C. GRAINED),GREY                                       32

475.77   1.23   1.20   475.77   1.23   1.20   CLAYSTONE,GREYISH BROWN                                                  37

477.00   3.00   2.98   477.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 27

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

480.00   3.00   2.99   480.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 38

483.00   3.00   2.95   483.00   3.00   2.95   CLAYSTONE,CHOCOLATEBROWN

486.00   3.00   2.99   486.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

489.00   3.00   2.99   489.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                   4.7

492.00   3.00   2.50   492.00   3.00   2.50   CLAYSTONE,CHOCOLATEBROWN                                                 59

495.00   1.00   1.00   495.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 74

496.00   1.00   1.00   496.00   1.00   1.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              78

497.00   1.00    .98   497.00   1.00    .98   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          57

498.00    .80    .76   498.00    .80    .76   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   50

498.80   2.20   2.20   498.80   2.20   2.20   CLAYSTONE,CHOCOLATEBROWN                                                 50

501.00   2.25   2.25   501.00   2.25   2.25   CLAYSTONE,CHOCOLATEBROWN                                                 33

503.25    .30    .22   503.25    .30    .22   SANDSTONE(FINE TO MEDIUM GRAINED),CHOCOLATEBROWN,                        63

SLICKENSIDED NEAR HORIZONTAL

503.55    .45    .36   503.55    .45    .36   CLAYSTONE,BROWNISH GREY,SLICKENSIDED NEAR HORIZONTAL

504.00    .60    .55   504.00    .60    .55   SANDSTONE(FINE TO MEDIUM GRAINED),GREY,

SLICKENSIDED NEAR HORIZONTAL

504.60   2.40   2.40   504.60   2.40   2.40   CLAYSTONE,CHOCOLATEBROWN                                                 21

507.00   3.00   2.98   507.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 22

510.00   2.00   1.98   510.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN                                                 28

512.00    .65    .65   512.00    .65    .65   CLAYSTONE,CHOCOLATEBROWN

512.65    .35    .33   512.65    .35    .33   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          89

513.00   3.00   2.98   513.00   3.00   2.98   CLAYSTONE,BROWNISH GREY,FRACTURE                                        4.3

516.00   3.00   2.98   516.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

519.00   3.00   2.99   519.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

522.00   1.00   1.00   522.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN

523.00   2.00   1.49   523.00   2.00   1.49   CLAYSTONE,CHOCOLATEBROWN

525.00   3.00   2.99   525.00   3.00   2.99   CLAYSTONE,BROWNISH GREY                                                  21

528.00    .14    .13   528.00    .14    .13   CLAYSTONE,BROWNISH GREY

528.14    .60    .60   528.14    .60    .60   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          70

528.74   2.26   2.26   528.74   2.26   2.26   MUDSTONE,BROWNISH GREY                                                   18

531.00   3.00   2.99   531.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   61

534.00   1.85   1.85   534.00   1.85   1.85   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN                          51

535.85   1.15   1.14   535.85   1.15   1.14   MUDSTONE,BROWNISH GREY,FRACTURE

537.00   3.00   2.99   537.00   3.00   2.99   MUDSTONE,BROWNISH GREY,FRACTURE

540.00   3.00   2.95   540.00   3.00   2.95   MUDSTONE,BROWNISH GREY,FRACTURE

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

543.00   1.00    .98   543.00   1.00    .98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

544.00   2.00   1.98   544.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN                                                 29

546.00   3.00   2.99   546.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 45

549.00   3.00   2.99   549.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 49

552.00   1.00    .98   552.00   1.00    .98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        62

553.00   2.00   2.00   553.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN                                                 84

555.00   1.30   1.30   555.00   1.30   1.30   CLAYSTONE,CHOCOLATEBROWN                                                 83

556.30   1.50   1.50   556.30   1.50   1.50   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              90

557.80    .20    .20   557.80    .20    .20   CLAYSTONE,GREY,FRACTURE

558.00   3.00   2.99   558.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                  8

561.00   2.00   1.98   561.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN                                                 10

563.00   1.00    .99   563.00   1.00    .99   CLAYSTONE,CHOCOLATEBROWN                                                 12

564.00   3.00   2.98   564.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN

567.00   1.10   1.06   567.00   1.10   1.06   CLAYSTONE,CHOCOLATEBROWN

568.10    .38    .38   568.10    .38    .38   SANDSTONE(MED. TO C. GRAINED),GREY

568.48   1.52   1.52   568.48   1.52   1.52   CLAYSTONE,CHOCOLATEBROWN                                                9.9

570.00   2.00   1.98   570.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN                                                 23

572.00   1.00    .99   572.00   1.00    .99   CLAYSTONE,CHOCOLATEBROWN                                                 77

573.00   3.00   2.98   573.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                       8.7

576.00   3.00   2.99   576.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 17

579.00   3.00   2.98   579.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                9.3

582.00    .50    .50   582.00    .50    .50   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          84

582.50    .50    .48   582.50    .50    .48   MUDSTONE,CHOCOLATEBROWN,FRACTURE                                         22

583.00   1.25   1.25   583.00   1.25   1.25   CLAYSTONE,CHOCOLATEBROWN                                                 14

584.25    .75    .73   584.25    .75    .73   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          15

585.00   1.45   1.45   585.00   1.45   1.45   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY

586.45   1.55   1.55   586.45   1.55   1.55   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              68

588.00   3.00   2.99   588.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

591.00   2.00   1.99   591.00   2.00   1.99   CLAYSTONE,CHOCOLATEBROWN                                                 67

593.00   1.00   1.00   593.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 70

594.00   3.00   2.98   594.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 30

597.00   3.00   2.99   597.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 32

600.00   1.60   1.60   600.00   1.60   1.60   CLAYSTONE,CHOCOLATEBROWN                                                 61

601.60   1.40   1.40   601.60   1.40   1.40   SANDSTONE(MED. TO C. GRAINED),GREY                                       67

603.00    .55    .55   603.00    .55    .55   SANDSTONE(MED. TO C. GRAINED),GREY                                       78

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

603.55   2.45   2.44   603.55   2.45   2.44   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

606.00    .30    .28   606.00    .30    .28   CLAYSTONE,CHOCOLATEBROWN

606.30   2.70   2.70   606.30   2.70   2.70   SANDSTONE(MED. TO C. GRAINED),GREY                                       59

609.00   1.05   1.05   609.00   1.05   1.05   SANDSTONE(MED. TO C. GRAINED),GREY                                       85

610.05    .85    .79   610.05    .85    .79   CLAYSTONE,CHOCOLATEBROWN                                                 15

610.90    .40    .40   610.90    .40    .40   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                              83

611.30    .70    .65   611.30    .70    .65   CLAYSTONE,CHOCOLATEBROWN                                                 21

612.00   1.00    .98   612.00   1.00    .98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        11

613.00   2.00   1.98   613.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        27

615.00    .65    .65   615.00    .65    .65   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

615.65    .70    .70   615.65    .70    .70   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                  100

616.35   1.65   1.65   616.35   1.65   1.65   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

618.00   3.00   2.99   618.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

621.00   2.00   1.98   621.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN

623.00   1.00    .99   623.00   1.00    .99   CLAYSTONE,CHOCOLATEBROWN

624.00   3.00   2.98   624.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        26

627.00   2.00   2.00   627.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN                                                 69

629.00    .21    .21   629.00    .21    .21   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

629.21    .19    .18   629.21    .19    .18   CLAYSTONE,CHOCOLATEBROWN

629.40    .22    .22   629.40    .22    .22   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                  100

629.62    .15    .15   629.62    .15    .15   CLAYSTONE,BROWNISH GREY                                                 100

629.77    .23    .23   629.77    .23    .23   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   57

630.00    .20    .20   630.00    .20    .20   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   90

630.20    .80    .78   630.20    .80    .78   CLAYSTONE,CHOCOLATEBROWN

631.00   2.00   2.00   631.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN                                                 55

633.00   3.00   2.99   633.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 23

636.00   3.00   2.97   636.00   3.00   2.97   CLAYSTONE,CHOCOLATEBROWN                                                 36

639.00   3.00   2.98   639.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 48

642.00   3.00   2.98   642.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 29

645.00   3.00   2.98   645.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        52

648.00   3.00   2.98   648.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        53

651.00   3.00   2.97   651.00   3.00   2.97   CLAYSTONE,CHOCOLATEBROWN                                                 27

654.00   2.00   1.98   654.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE HIGH ANGLE                            9.5

656.00   1.00    .96   656.00   1.00    .96   CLAYSTONE,CHOCOLATEBROWN,FRACTURE HIGH ANGLE                             17

657.00   3.00   2.98   657.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE HIGH ANGLE                             28

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

660.00   3.00   2.99   660.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,FRACTURE HIGH ANGLE

663.00   2.00   1.98   663.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE HIGH ANGLE                             39

665.00   1.00    .98   665.00   1.00    .98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE HIGH ANGLE

666.00   3.00   2.95   666.00   3.00   2.95   CLAYSTONE,CHOCOLATEBROWN,FRACTURE HIGH ANGLE                             11

669.00   3.00   2.98   669.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN                                                 56

672.00   2.00   1.98   672.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN

674.00   1.00    .99   674.00   1.00    .99   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

675.00   3.00   2.98   675.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

678.00   2.25   2.24   678.00   2.25   2.24   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

680.25    .45    .45   680.25    .45    .45   MUDSTONE,BROWNISH GREY,FRACTURE

680.70    .30    .30   680.70    .30    .30   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              80

681.00    .15    .13   681.00    .15    .13   MUDSTONE,BROWNISH GREY

681.15    .60    .60   681.15    .60    .60   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                  100

681.75    .55    .45   681.75    .55    .45   MUDSTONE,BROWNISH GREY,FRACTURE

********************************************************************************************************************************

BARAKAR

682.30   1.70   1.70   682.30   1.70   1.70   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          72

684.00   3.00   3.00   684.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              92

687.00   3.00   2.99   687.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              94

690.00   3.00   2.99   690.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              86

693.00   3.00   2.99   693.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              71

696.00   3.00   3.00   696.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              81

699.00    .85    .82   699.00    .85    .82   SHALE,DARK GREY,FRACTURE

699.85   1.45   1.42   699.85   1.45   1.42   SHALE,GREY,FRACTURE

701.30    .70    .70   701.30    .70    .70   SANDY SHALE,GREY,FRACTURE

702.00   1.57   1.57   702.00   1.57   1.57   INTERCALATIONS,GREY,SLICKENSIDED                                        8.3

703.57    .36    .36   703.57    .36    .36   SHALE,DARK GREY,FRACTURE

703.93    .60    .60   703.93    .60    .60   SANDY SHALE,GREY,FAULTED

704.53    .35    .35   704.53    .35    .35   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          60

704.88    .12    .10   704.88    .12    .10   SHALE,DARK GREY,FAULTED

705.00   2.57   2.57   705.00   2.57   2.57   SHALE,DARK GREY,SLICKENSIDED

707.57    .43    .43   707.57    .43    .43   SANDY SHALE,GREY                                                         28

708.00    .55    .53   708.00    .55    .53   INTERCALATIONS,GREY                                                      36

708.55   2.45   2.42   708.55   2.45   2.42   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,                             46

711.00    .15    .14   711.00    .15    .14   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

711.15    .70    .68   711.15    .70    .68   SANDY SHALE,GREY,SLICKENSIDED                                            73

711.85   2.15   2.11   711.85   2.15   2.11   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,                             39

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

714.00   3.00   2.98   714.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          87

717.00   3.00   2.99   717.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          78

720.00   3.00   2.99   720.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     68

723.00   3.00   2.98   723.00   3.00   2.98   SANDSTONE(MED. TO C. GRAINED),WHITE                                      79

726.00   3.00   2.99   726.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),WHITE                                      74

729.00   3.00   2.99   729.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),WHITE                                      79

732.00   1.40   1.25   732.00   1.40   1.25   SANDSTONE(MED. TO C. GRAINED),WHITE                                      74

733.40   1.60   1.47   733.40   1.60   1.47   SANDY SHALE,GREY,FRACTURE                                                18

735.00   1.45   1.22   735.00   1.45   1.22   SANDY SHALE,GREY,SLICKENSIDED                                           8.3

736.45   1.55   1.40   736.45   1.55   1.40   INTERCALATIONS,GREY,SLICKENSIDED                                         71

738.00   3.00   2.60   738.00   3.00   2.60   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  73

741.00   3.00   2.87   741.00   3.00   2.87   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  88

744.00   3.00   2.90   744.00   3.00   2.90   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              84

747.00   3.00   2.95   747.00   3.00   2.95   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE,FRACTURE                     42

750.00   3.00   2.99   750.00   3.00   2.99   SANDSTONE(MED. TO C. GRAINED),WHITE                                      96

753.00   1.40   1.38   753.00   1.40   1.38   SANDSTONE(MED. TO C. GRAINED),WHITE                                      91

754.40    .53    .50   754.40    .53    .50   COAL,SLICKENSIDED                                               IIA

754.93    .26    .26   754.93    .26    .26   SHALY COAL,SLICKENSIDED                                         IIA

755.19    .35    .35   755.19    .35    .35   COAL,SLICKENSIDED                                               IIA

755.54    .32    .32   755.54    .32    .32   SHALY COAL                                                      IIA

755.86    .50    .50   755.86    .50    .50   COAL                                                            IIA

756.36    .60    .60   756.36    .60    .60   COAL                                                            IIA

756.96   1.68   1.68   756.96   1.68   1.68   INTERCALATIONS,GREY                                                     100

758.64    .47    .46   758.64    .47    .46   COAL                                                            IIB

759.11    .60    .60   759.11    .60    .60   COAL                                                            IIB

759.71    .42    .42   759.71    .42    .42   COAL                                                            IIB

760.13    .94    .94   760.13    .94    .94   COAL                                                            IIB

761.07    .15    .14   761.07    .15    .14   COAL                                                            IIB

761.22    .65    .63   761.22    .65    .63   COAL                                                            IIB

761.87    .43    .40   761.87    .43    .40   COAL                                                            IIB

762.30    .20    .17   762.30    .20    .17   CARB SHALE HIGH                                                 IIB

762.50   1.70   1.66   762.50   1.70   1.66   SANDY SHALE,GREY                                                         98

764.20    .80    .78   764.20    .80    .78   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              65

765.00    .60    .57   765.00    .60    .57   SANDY SHALE,GREY                                                         95

765.60   2.40   2.40   765.60   2.40   2.40   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              83

768.00   3.00   2.96   768.00   3.00   2.96   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              87

771.00   1.56   1.56   771.00   1.56   1.56   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              93

772.56    .25    .22   772.56    .25    .22   SHALY COAL                                                      I

772.81    .30    .28   772.81    .30    .28   COAL                                                            I

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

773.11    .46    .46   773.11    .46    .46   SHALY COAL                                                      I

773.57    .30    .27   773.57    .30    .27   CARB SHALE LOW,SLICKENSIDED                                     I

773.87    .13    .13   773.87    .13    .13   SANDY SHALE,GREY

774.00   3.00   2.75   774.00   3.00   2.75   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         49

777.00   3.00   3.00   777.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         80

BOREHOLE CLOSED AT   780.00 M DEPTH

---------------------------------------------------------------------------

BOREHOLE NO    :CMWWB11              LATITUDE    : 268487.491M                         DATE OF COMMENCEMENT:05.10.2024

REDUCED LEVEL  : 263.00M            DEPARTURE   : 2204842.3532M                       DATE OF COMPLETION: 20.01.2025

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

.00   3.00    .20      .00   3.00    .20   SOIL,BLACK

3.00   3.00    .25     3.00   3.00    .25   SOIL,BLACK

SOIL

*****************************************************************************************************************************

TRAP

6.00   2.00    .35     6.00   2.00    .35   BASALT,GREYISH BROWN,STRONG ROCKS

8.00   1.50   1.50     8.00   1.50   1.50   BASALT,GREY,STRONG ROCKS                                                 53

9.50   1.50   1.50     9.50   1.50   1.50   BASALT,GREY,STRONG ROCKS                                                 71

11.00   1.50   1.50    11.00   1.50   1.50   BASALT,GREY,STRONG ROCKS                                                 60

12.50   1.50   1.50    12.50   1.50   1.50   BASALT,GREY,STRONG ROCKS                                                 81

14.00   1.50   1.50    14.00   1.50   1.50   BASALT,GREY,STRONG ROCKS                                                 69

15.50   1.50   1.50    15.50   1.50   1.50   BASALT,GREYISH GREEN,STRONG ROCKS                                        27

17.00   2.00   2.00    17.00   2.00   2.00   BASALT,GREY,STRONG ROCKS                                                 36

19.00   2.00   2.00    19.00   2.00   2.00   BASALT,GREY,STRONG ROCKS                                                 39

21.00   3.00   2.99    21.00   3.00   2.99   BASALT,GREY,STRONG ROCKS                                                 33

24.00   3.00   3.00    24.00   3.00   3.00   BASALT,GREY,STRONG ROCKS                                                 56

27.00   3.00   3.00    27.00   3.00   3.00   BASALT,GREY,STRONG ROCKS                                                 81

30.00   3.00   3.00    30.00   3.00   3.00   BASALT,GREY,STRONG ROCKS                                                 90

33.00   3.00   3.00    33.00   3.00   3.00   BASALT,GREY,STRONG ROCKS                                                 78

36.00   2.00   2.00    36.00   2.00   2.00   BASALT,GREY,STRONG ROCKS

38.00   1.00   1.00    38.00   1.00   1.00   BASALT,GREY,STRONG ROCKS

39.00   3.00   3.00    39.00   3.00   3.00   BASALT,GREYISH GREEN,STRONG ROCKS

42.00    .80    .80    42.00    .80    .80   BASALT,GREYISH GREEN,STRONG ROCKS

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

44.60    .40    .40    44.60    .40    .40   MUDSTONE,GREY,STRONG ROCKS                                               38

45.00   2.50   2.50    45.00   2.50   2.50   BASALT,GREY,STRONG ROCKS                                                 32

47.50    .50    .50    47.50    .50    .50   BASALT,GREY,STRONG ROCKS

48.00   3.00   2.99    48.00   3.00   2.99   BASALT,GREY,STRONG ROCKS                                                 16

51.00   3.00   2.99    51.00   3.00   2.99   BASALT,GREY,STRONG ROCKS                                                 51

54.00   3.00   3.00    54.00   3.00   3.00   BASALT,GREY,STRONG ROCKS                                                 61

******************************************************************************************************************************

MOTUR

57.00   2.75   2.74    57.00   2.75   2.74   CLAYSTONE,GREY

59.75    .25    .15    59.75    .25    .15   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

60.00   3.00   1.50    60.00   3.00   1.50   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                             8.3

63.00    .50    .22    63.00    .50    .22   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

63.50    .50    .20    63.50    .50    .20   MUDSTONE,GREY

64.00   2.00   1.08    64.00   2.00   1.08   CLAYSTONE,GREY                                                           24

66.00   3.00   2.00    66.00   3.00   2.00   CLAYSTONE,GREY,EXTREMELY STRONG                                          12

69.00   3.00   2.08    69.00   3.00   2.08   MUDSTONE,GREY,EXTREMELY STRONG

72.00    .50    .28    72.00    .50    .28   MUDSTONE,GREY

72.50   2.50   2.00    72.50   2.50   2.00   CLAYSTONE,CHOCOLATEBROWN

75.00    .50    .50    75.00    .50    .50   SANDSTONE(MED. TO C. GRAINED),GREY

75.50    .50    .49    75.50    .50    .49   CLAYSTONE,CHOCOLATEBROWN

76.00   1.00    .60    76.00   1.00    .60   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN

77.00   1.00    .60    77.00   1.00    .60   CLAYSTONE,REDDISH BROWN

78.00   3.00   2.10    78.00   3.00   2.10   CLAYSTONE,REDDISH BROWN

81.00    .90    .70    81.00    .90    .70   CLAYSTONE,REDDISH BROWN

81.90   1.10    .66    81.90   1.10    .66   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

83.00   1.00    .64    83.00   1.00    .64   MUDSTONE,GREY

84.00   3.00   2.50    84.00   3.00   2.50   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

87.00   1.50   1.40    87.00   1.50   1.40   CLAYSTONE,CHOCOLATEBROWN

88.50   1.50    .90    88.50   1.50    .90   SANDSTONE(MED. TO C. GRAINED),GREY                                       13

90.00   2.50   2.00    90.00   2.50   2.00   SANDSTONE(MED. TO C. GRAINED),GREY                                       22

92.50    .20    .19    92.50    .20    .19   SANDSTONE(MED. TO C. GRAINED),GREY

92.70    .30    .30    92.70    .30    .30   MUDSTONE,GREY

93.00   1.00   1.00    93.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 23

94.00   2.00   2.00    94.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN                                                 29

96.00   1.44   1.43    96.00   1.44   1.43   CLAYSTONE,CHOCOLATEBROWN                                                 21

97.44   1.56   1.56    97.44   1.56   1.56   MUDSTONE,GREY,STRONG ROCKS                                               10

99.00   3.00   3.00    99.00   3.00   3.00   CLAYSTONE,GREYISH BROWN,STRONG ROCKS                                     23

102.00   3.00   2.99   102.00   3.00   2.99   CLAYSTONE,GREYISH BROWN,STRONG ROCKS

105.00   1.55   1.55   105.00   1.55   1.55   MUDSTONE,GREY,HARD

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

106.55   1.45   1.45   106.55   1.45   1.45   SANDSTONE(MED. TO C. GRAINED),YELLOWISHBROWN,HARD                        70

108.00    .50    .50   108.00    .50    .50   SANDSTONE(MED. TO C. GRAINED),YELLOWISHBROWN

108.50   1.37   1.37   108.50   1.37   1.37   MUDSTONE,GREY,STRONG ROCKS                                               37

109.87   1.13   1.13   109.87   1.13   1.13   CLAYSTONE,CHOCOLATEBROWN

111.00   1.72   1.72   111.00   1.72   1.72   CLAYSTONE,CHOCOLATEBROWN

112.72   1.28   1.27   112.72   1.28   1.27   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            38

114.00    .35    .28   114.00    .35    .28   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

114.35   1.45   1.34   114.35   1.45   1.34   MUDSTONE,GREY                                                            63

115.80   1.20    .98   115.80   1.20    .98   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            56

117.00   3.00   1.50   117.00   3.00   1.50   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            34

120.00   1.40    .64   120.00   1.40    .64   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            12

121.40   1.60    .86   121.40   1.60    .86   MUDSTONE,GREY,STRONG ROCKS                                               48

123.00   2.59   2.59   123.00   2.59   2.59   MUDSTONE,GREY,STRONG ROCKS                                               28

125.59    .41    .41   125.59    .41    .41   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS                                    73

126.00   2.23   2.23   126.00   2.23   2.23   CLAYSTONE,CHOCOLATEBROWN                                                 67

128.23    .77    .77   128.23    .77    .77   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

129.00   3.00   3.00   129.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,HARD                                            33

132.00   3.00   3.00   132.00   3.00   3.00   MUDSTONE,GREY,HARD                                                       62

135.00   2.00   2.00   135.00   2.00   2.00   MUDSTONE,GREY,EXTREMELY STRONG                                           22

137.00    .32    .32   137.00    .32    .32   MUDSTONE,GREY

137.32    .68    .68   137.32    .68    .68   CLAYSTONE,CHOCOLATEBROWN

138.00   3.00   2.99   138.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS                                    29

141.00   3.00   3.00   141.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS                                    59

144.00   3.00   3.00   144.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS                                    56

147.00   3.00   2.99   147.00   3.00   2.99   MUDSTONE,GREYISH BROWN,STRONG ROCKS                                      38

150.00   3.00   3.00   150.00   3.00   3.00   MUDSTONE,GREYISH BROWN,STRONG ROCKS                                      64

153.00    .95    .95   153.00    .95    .95   MUDSTONE,GREYISH BROWN,STRONG ROCKS                                      84

153.95   2.05   2.05   153.95   2.05   2.05   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH BROWN,                         74

156.00   3.00   3.00   156.00   3.00   3.00   MUDSTONE,GREY,STRONG ROCKS                                              8.7

159.00   3.00   2.99   159.00   3.00   2.99   MUDSTONE,GREY,STRONG ROCKS                                               38

162.00   1.15   1.15   162.00   1.15   1.15   MUDSTONE,GREY,STRONG ROCKS                                               71

163.15   1.85   1.85   163.15   1.85   1.85   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          59

165.00   3.00   3.00   165.00   3.00   3.00   SANDSTONE(COARSE TO V.C.GRAINED),GREYISH WHITE                           60

168.00    .42    .42   168.00    .42    .42   MUDSTONE,GREY

168.42   2.58   2.57   168.42   2.58   2.57   CLAYSTONE,CHOCOLATEBROWN,EXTREMELY STRONG

171.00   3.00   2.99   171.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,EXTREMELY STRONG

174.00   2.00   2.00   174.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN,EXTREMELY STRONG

176.00   1.00   1.00   176.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 19

177.00   3.00   3.00   177.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

180.00   3.00   3.00   180.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN

183.00   3.00   3.00   183.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN

186.00   2.50   2.50   186.00   2.50   2.50   MUDSTONE,GREYISH BROWN

188.50   3.00   3.00   188.50   3.00   3.00   MUDSTONE,GREYISH BROWN,EXTREMELY STRONG                                  30

191.50   1.00    .70   191.50   1.00    .70   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN,                        42

192.50   2.50   1.49   192.50   2.50   1.49   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHBROWN                         25

195.00   1.30    .70   195.00   1.30    .70   CLAYSTONE,CHOCOLATEBROWN

196.30   1.70   1.40   196.30   1.70   1.40   SANDSTONE(MED. TO C. GRAINED),REDDISH GREY                               22

198.00   1.50   1.20   198.00   1.50   1.20   MUDSTONE,GREY                                                            25

199.50    .40    .30   199.50    .40    .30   CLAYSTONE,CHOCOLATEBROWN

199.90    .65    .62   199.90    .65    .62   SANDSTONE(MED. TO C. GRAINED),GREY                                       42

200.55    .60    .53   200.55    .60    .53   MUDSTONE,GREY,HARD                                                       32

201.15    .35    .33   201.15    .35    .33   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        57

201.50    .30    .30   201.50    .30    .30   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW,                       63

201.80   2.20   2.20   201.80   2.20   2.20   MUDSTONE,GREYISH BROWN,STRONG ROCKS                                      51

204.00    .95    .95   204.00    .95    .95   MUDSTONE,GREYISH BROWN,STRONG ROCKS                                      51

204.95   2.05   2.05   204.95   2.05   2.05   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         63

207.00    .46    .46   207.00    .46    .46   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          43

207.46    .52    .50   207.46    .52    .50   MUDSTONE,BROWNISH YELLOW

207.98   1.05   1.05   207.98   1.05   1.05   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        63

209.03    .65    .65   209.03    .65    .65   MUDSTONE,BROWNISH GREY

209.68    .32    .32   209.68    .32    .32   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        88

210.00   2.10   2.10   210.00   2.10   2.10   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        52

212.10    .90    .89   212.10    .90    .89   MUDSTONE,BROWNISH GREY,STRONG ROCKS

213.00   3.00   3.00   213.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,HARD                                            51

216.00   3.00   3.00   216.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 48

219.00   2.00   2.00   219.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN

221.00   1.00   1.00   221.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN

222.00   3.00   2.85   222.00   3.00   2.85   CLAYSTONE,CHOCOLATEBROWN

225.00   2.00   1.96   225.00   2.00   1.96   CLAYSTONE,CHOCOLATEBROWN

227.00   1.00    .90   227.00   1.00    .90   CLAYSTONE,CHOCOLATEBROWN

228.00   3.00   2.90   228.00   3.00   2.90   CLAYSTONE,CHOCOLATEBROWN,HARD

231.00   3.00   2.89   231.00   3.00   2.89   CLAYSTONE,CHOCOLATEBROWN

234.00   1.00    .97   234.00   1.00    .97   CLAYSTONE,CHOCOLATEBROWN                                                 29

235.00   2.00   2.00   235.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN                                                 14

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

237.00   3.00   3.00   237.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,EXTREMELY STRONG                                60

240.00   2.50   2.49   240.00   2.50   2.49   CLAYSTONE,BROWNISH GREY                                                  47

242.50    .50    .50   242.50    .50    .50   CLAYSTONE,BROWNISH GREY                                                  30

243.00   1.45   1.45   243.00   1.45   1.45   CLAYSTONE,BROWNISH GREY                                                  54

244.45   1.55   1.53   244.45   1.55   1.53   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                             9.7

246.00   3.00   3.00   246.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            62

249.00   3.00   2.50   249.00   3.00   2.50   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            34

252.00   2.50   1.00   252.00   2.50   1.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            56

254.50   3.00   2.70   254.50   3.00   2.70   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            45

257.50   3.00   2.80   257.50   3.00   2.80   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                             7

260.50   1.00    .80   260.50   1.00    .80   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

261.50   3.00   1.20   261.50   3.00   1.20   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            19

264.50   3.00    .80   264.50   3.00    .80   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            19

267.50   3.00   1.00   267.50   3.00   1.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                           7.7

270.50   3.00   1.50   270.50   3.00   1.50   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW                            33

273.50   2.50   1.00   273.50   2.50   1.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH YELLOW

276.00   3.00    .60   276.00   3.00    .60   MUDSTONE,CHOCOLATEBROWN                                                  26

279.00   3.00   1.65   279.00   3.00   1.65   MUDSTONE,BROWNISH GREY,STRONG ROCKS                                      20

282.00   1.45   1.05   282.00   1.45   1.05   MUDSTONE,BROWNISH GREY,STRONG ROCKS                                       9

283.45    .55    .50   283.45    .55    .50   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          76

284.00   1.00   1.00   284.00   1.00   1.00   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        75

285.00   3.00   2.80   285.00   3.00   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW,                       57

288.00   3.00   3.00   288.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        76

291.00   3.00   3.00   291.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        58

294.00   1.00   1.00   294.00   1.00   1.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              50

295.00   2.00   2.00   295.00   2.00   2.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              46

297.00   2.55   2.55   297.00   2.55   2.55   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY,                            5.9

299.55    .45    .45   299.55    .45    .45   MUDSTONE,GREYISH BROWN,STRONG ROCKS

300.00   3.00   2.98   300.00   3.00   2.98   CLAYSTONE,BROWN,STRONG ROCKS

303.00    .79    .78   303.00    .79    .78   CLAYSTONE,BROWN                                                          70

303.79   2.21   2.21   303.79   2.21   2.21   SANDSTONE(MED. TO C. GRAINED),GREY,STRONG ROCKS                          41

306.00   3.00   2.60   306.00   3.00   2.60   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          41

309.00   1.00    .97   309.00   1.00    .97   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          53

310.00   1.15    .98   310.00   1.15    .98   MUDSTONE,CHOCOLATEBROWN,HARD                                             29

311.15    .85    .70   311.15    .85    .70   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY,                             41

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

312.00    .45    .35   312.00    .45    .35   SANDSTONE(MEDIUM GRAINED),YELLOWISHGREY                                  78

312.45   1.55   1.45   312.45   1.55   1.45   MUDSTONE,BROWNISH GREY                                                   55

314.00   1.00    .75   314.00   1.00    .75   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY

315.00   3.00   2.65   315.00   3.00   2.65   SANDSTONE(MEDIUM GRAINED),BROWNISH GREY                                  54

318.00   3.00   2.50   318.00   3.00   2.50   SANDSTONE(MEDIUM GRAINED),YELLOWISHGREY,STRONG ROCKS                     27

321.00   1.00    .72   321.00   1.00    .72   SANDSTONE(MEDIUM GRAINED),YELLOWISHGREY,STRONG ROCKS                     27

322.00   2.00   1.50   322.00   2.00   1.50   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY,                         53

324.00    .80    .60   324.00    .80    .60   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          46

324.80    .50    .25   324.80    .50    .25   CLAYSTONE,CHOCOLATEBROWN                                                 54

325.30    .40    .18   325.30    .40    .18   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        45

325.70    .55    .42   325.70    .55    .42   CLAYSTONE,CHOCOLATEBROWN                                                 75

326.25    .75    .55   326.25    .75    .55   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          64

327.00   3.00   2.50   327.00   3.00   2.50   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         54

330.00   2.00   1.50   330.00   2.00   1.50   SANDSTONE(MEDIUM GRAINED),YELLOWISHGREY,STRONG ROCKS                     36

332.00   3.00   2.40   332.00   3.00   2.40   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY,                         35

335.00   3.00   2.80   335.00   3.00   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE,STRONG ROCKS                     56

338.00   3.00   2.76   338.00   3.00   2.76   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  54

341.00    .55    .52   341.00    .55    .52   CLAYSTONE,REDDISH BROWN                                                  91

341.55    .45    .34   341.55    .45    .34   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          40

342.00   2.50   2.00   342.00   2.50   2.00   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          21

344.50   1.50   1.40   344.50   1.50   1.40   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          18

346.00   1.50   1.50   346.00   1.50   1.50   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  44

347.50   2.00   1.99   347.50   2.00   1.99   SANDSTONE(MED. TO C. GRAINED),WHITE                                      41

349.50   1.50   1.40   349.50   1.50   1.40   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  51

351.00   2.00   1.90   351.00   2.00   1.90   SANDSTONE(MED. TO C. GRAINED),GREYISH BROWN                              74

353.00   1.00    .90   353.00   1.00    .90   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY,

354.00   1.10    .57   354.00   1.10    .57   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY,

355.10    .90    .43   355.10    .90    .43   CLAYSTONE,GREYISH BROWN                                                  49

356.00   1.00    .90   356.00   1.00    .90   CLAYSTONE,CHOCOLATEBROWN                                                 52

357.00   2.00   1.20   357.00   2.00   1.20   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY,                             13

359.00   2.00   1.23   359.00   2.00   1.23   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY,                             13

361.00   1.00    .17   361.00   1.00    .17   MUDSTONE,BROWNISH GREY                                                   17

362.00   3.00   2.60   362.00   3.00   2.60   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY,                             48

365.00   1.50    .91   365.00   1.50    .91   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE                                  13

366.50    .50    .39   366.50    .50    .39   MUDSTONE,BROWNISH GREY,HARD                                              60

S/W DEVELOPED BY :

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

367.00    .20    .20   367.00    .20    .20   MUDSTONE,BROWNISH GREY                                                   55

367.20    .80    .80   367.20    .80    .80   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          36

368.00   3.00   1.89   368.00   3.00   1.89   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                             9.7

371.00   3.00   1.85   371.00   3.00   1.85   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              31

374.00   3.00   2.40   374.00   3.00   2.40   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              40

377.00   1.00    .70   377.00   1.00    .70   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              36

378.00   2.00   1.50   378.00   2.00   1.50   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY,                             29

380.00   3.00   1.60   380.00   3.00   1.60   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHGREY,                          18

383.00   3.00   1.05   383.00   3.00   1.05   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHGREY,                         3.7

386.00   3.00    .90   386.00   3.00    .90   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHGREY,

389.00   3.00   1.25   389.00   3.00   1.25   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHGREY,

392.00   3.00   1.00   392.00   3.00   1.00   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY

395.00   3.00   1.25   395.00   3.00   1.25   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY

398.00   3.00   1.30   398.00   3.00   1.30   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          15

401.00   1.00    .50   401.00   1.00    .50   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          15

402.00   2.00   1.10   402.00   2.00   1.10   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          18

404.00   3.00   1.95   404.00   3.00   1.95   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                         4.3

407.00   3.00   1.15   407.00   3.00   1.15   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                         8.7

410.00   1.00    .80   410.00   1.00    .80   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          48

411.00   1.00    .85   411.00   1.00    .85   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          33

412.00   1.50   1.00   412.00   1.50   1.00   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY

413.50   2.00   1.35   413.50   2.00   1.35   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY,                             43

415.50   3.00   1.50   415.50   3.00   1.50   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              29

418.50   3.00   1.90   418.50   3.00   1.90   SANDSTONE(MED. TO C. GRAINED),DARK GREY                                  30

421.50   2.00   1.10   421.50   2.00   1.10   SANDSTONE(MED. TO C. GRAINED),DARK GREY                                  10

423.50   1.50    .90   423.50   1.50    .90   SANDSTONE(MED. TO C. GRAINED),DARK GREY                                  40

425.00   3.00   2.30   425.00   3.00   2.30   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                         5.3

428.00   3.00   1.90   428.00   3.00   1.90   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          25

431.00    .50    .25   431.00    .50    .25   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          26

431.50   2.50   2.15   431.50   2.50   2.15   MUDSTONE,BROWNISH GREY,STRONG ROCKS                                      30

434.00   1.20    .90   434.00   1.20    .90   SANDSTONE(FINE TO MEDIUM GRAINED),GREY,STRONG ROCKS                      37

435.20    .30    .12   435.20    .30    .12   MUDSTONE,GREYISH BROWN,STRONG ROCKS

435.50   3.00   2.90   435.50   3.00   2.90   MUDSTONE,BROWNISH GREY                                                   18

438.50   3.00   2.99   438.50   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 39

441.50   1.50   1.50   441.50   1.50   1.50   CLAYSTONE,BROWNISH GREY,STRONG ROCKS                                     22

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

443.00   3.00   3.00   443.00   3.00   3.00   CLAYSTONE,BROWNISH GREY,HARD

446.00   3.00   2.89   446.00   3.00   2.89   CLAYSTONE,CHOCOLATEBROWN                                                6.3

449.00   3.00   2.99   449.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN

452.00   1.00   1.00   452.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,HARD

453.00   2.00   2.00   453.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

455.00   3.00   3.00   455.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

458.00   3.00   3.00   458.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

461.00   2.00   2.00   461.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

463.00    .33    .33   463.00    .33    .33   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

463.33    .67    .67   463.33    .67    .67   MUDSTONE,GREY,STRONG ROCKS

464.00   3.00   2.99   464.00   3.00   2.99   MUDSTONE,GREY,STRONG ROCKS

467.00   1.25   1.20   467.00   1.25   1.20   MUDSTONE,GREY,STRONG ROCKS

468.25   2.00   1.60   468.25   2.00   1.60   MUDSTONE,GREY,STRONG ROCKS

470.25   1.75   1.67   470.25   1.75   1.67   MUDSTONE,GREY

472.00   1.00    .73   472.00   1.00    .73   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

473.00    .25    .25   473.00    .25    .25   MUDSTONE,GREY

473.25    .40    .33   473.25    .40    .33   MUDSTONE,GREY

473.65   1.60   1.54   473.65   1.60   1.54   CLAYSTONE,CHOCOLATEBROWN,HARD

475.25   2.00   1.50   475.25   2.00   1.50   CLAYSTONE,BROWNISH GREY,STRONG ROCKS

477.25   2.50   2.48   477.25   2.50   2.48   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

479.75   3.00   2.99   479.75   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

482.75   1.60   1.60   482.75   1.60   1.60   MUDSTONE,GREY,STRONG ROCKS

484.35   1.40   1.40   484.35   1.40   1.40   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

485.75   3.00   2.50   485.75   3.00   2.50   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

488.75   3.00   2.35   488.75   3.00   2.35   MUDSTONE,GREY,STRONG ROCKS

491.75    .75    .70   491.75    .75    .70   MUDSTONE,GREY,STRONG ROCKS

492.50   2.50   2.50   492.50   2.50   2.50   MUDSTONE,GREY,STRONG ROCKS                                              4.4

495.00   3.00   3.00   495.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS                                   3.7

498.00   2.00   1.99   498.00   2.00   1.99   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

500.00   1.00   1.00   500.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

501.00   2.40   2.40   501.00   2.40   2.40   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

503.40    .60    .60   503.40    .60    .60   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN

504.00    .75    .75   504.00    .75    .75   SANDSTONE(MED. TO C. GRAINED),GREYISH GREEN                              43

504.75   2.25   2.24   504.75   2.25   2.24   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

507.00   3.00   2.95   507.00   3.00   2.95   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS                                    14

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

510.00   3.00   3.00   510.00   3.00   3.00   CLAYSTONE,BROWNISH GREY,STRONG ROCKS                                     28

513.00   1.00   1.00   513.00   1.00   1.00   MUDSTONE,GREYISH BROWN,HARD                                              20

514.00    .59    .59   514.00    .59    .59   MUDSTONE,GREYISH BROWN

514.59    .35    .35   514.59    .35    .35   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          40

514.94    .32    .32   514.94    .32    .32   MUDSTONE,GREY

515.26    .74    .74   515.26    .74    .74   CLAYSTONE,BROWNISH GREY

516.00   3.00   2.99   516.00   3.00   2.99   CLAYSTONE,BROWNISH GREY,STRONG ROCKS

519.00    .77    .77   519.00    .77    .77   CLAYSTONE,BROWNISH GREY,STRONG ROCKS

519.77    .36    .36   519.77    .36    .36   MUDSTONE,GREY,STRONG ROCKS

520.13   1.87   1.87   520.13   1.87   1.87   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

522.00   1.00    .98   522.00   1.00    .98   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

523.00   2.00   1.30   523.00   2.00   1.30   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

525.00   2.00   1.98   525.00   2.00   1.98   CLAYSTONE,BROWNISH GREY,STRONG ROCKS                                      9

527.00   3.00   3.00   527.00   3.00   3.00   CLAYSTONE,BROWNISH GREY,STRONG ROCKS                                     13

530.00   2.00   1.99   530.00   2.00   1.99   CLAYSTONE,GREYISH BROWN,STRONG ROCKS                                    7.5

532.00   3.00   2.94   532.00   3.00   2.94   MUDSTONE,GREY,STRONG ROCKS

535.00   3.00   3.00   535.00   3.00   3.00   MUDSTONE,GREY,STRONG ROCKS                                               13

538.00   3.00   2.95   538.00   3.00   2.95   CLAYSTONE,BROWNISH GREY,STRONG ROCKS                                    4.7

541.00   1.00   1.00   541.00   1.00   1.00   CLAYSTONE,BROWNISH GREY,STRONG ROCKS

542.00   2.00   1.85   542.00   2.00   1.85   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS                                   8.5

544.00   2.50   2.43   544.00   2.50   2.43   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS                                   5.2

546.50    .50    .45   546.50    .50    .45   SANDSTONE(FINE GRAINED),BROWNISH GREY,STRONG ROCKS                       22

547.00    .55    .53   547.00    .55    .53   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   53

547.55   1.00   1.00   547.55   1.00   1.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              27

548.55    .45    .32   548.55    .45    .32   CLAYSTONE,BROWNISH GREY

549.00   1.00    .85   549.00   1.00    .85   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE

550.00    .25    .20   550.00    .25    .20   CLAYSTONE,CHOCOLATEBROWN

550.25   1.75   1.70   550.25   1.75   1.70   SILTSTONE,DARK GREY                                                      59

552.00   1.40   1.40   552.00   1.40   1.40   CLAYSTONE,GREYISH BROWN

553.40    .80    .80   553.40    .80    .80   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY

554.20    .80    .80   554.20    .80    .80   CLAYSTONE,CHOCOLATEBROWN

555.00   1.00   1.00   555.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,HARD

556.00   3.00   2.10   556.00   3.00   2.10   CLAYSTONE,CHOCOLATEBROWN,HARD

559.00   2.50   1.40   559.00   2.50   1.40   CLAYSTONE,CHOCOLATEBROWN,HARD

561.50   1.50   1.00   561.50   1.50   1.00   CLAYSTONE,CHOCOLATEBROWN,HARD

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

563.00   2.00   1.55   563.00   2.00   1.55   CLAYSTONE,CHOCOLATEBROWN,HARD

565.00   3.00   2.46   565.00   3.00   2.46   CLAYSTONE,CHOCOLATEBROWN,HARD

568.00   3.00   2.50   568.00   3.00   2.50   CLAYSTONE,CHOCOLATEBROWN,HARD

571.00   2.50   1.70   571.00   2.50   1.70   CLAYSTONE,GREYISH BROWN,HARD

573.50   2.00   1.31   573.50   2.00   1.31   CLAYSTONE,GREYISH BROWN,HARD

575.50   1.50   1.50   575.50   1.50   1.50   CLAYSTONE,CHOCOLATEBROWN,HARD

577.00   3.00   3.00   577.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,HARD

580.00   2.00   2.00   580.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN,HARD

582.00   3.00   3.00   582.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,HARD

585.00   2.00   2.00   585.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN,HARD

587.00   2.00   2.00   587.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN,HARD

589.00   1.00   1.00   589.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,HARD

590.00   2.00   2.00   590.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN,HARD

592.00   2.50   2.49   592.00   2.50   2.49   CLAYSTONE,CHOCOLATEBROWN,HARD

594.50   1.00   1.00   594.50   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,HARD

595.50   2.50   2.30   595.50   2.50   2.30   CLAYSTONE,CHOCOLATEBROWN,HARD

598.00   2.00   2.00   598.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN

600.00   2.50   2.50   600.00   2.50   2.50   CLAYSTONE,CHOCOLATEBROWN

602.50   2.00   2.00   602.50   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN,HARD

604.50   2.00   1.99   604.50   2.00   1.99   CLAYSTONE,CHOCOLATEBROWN,HARD

606.50   2.50   2.48   606.50   2.50   2.48   CLAYSTONE,CHOCOLATEBROWN,HARD

609.00   1.00   1.00   609.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,HARD

610.00   1.00   1.00   610.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,HARD

611.00   1.00   1.00   611.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,HARD

612.00   1.00    .98   612.00   1.00    .98   CLAYSTONE,CHOCOLATEBROWN,HARD

613.00   2.00   1.99   613.00   2.00   1.99   CLAYSTONE,CHOCOLATEBROWN,HARD

615.00   3.00   3.00   615.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,HARD

618.00   1.00   1.00   618.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,HARD

619.00   1.00   1.00   619.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,HARD

620.00   3.00   3.00   620.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,HARD

623.00   2.00   2.00   623.00   2.00   2.00   CLAYSTONE,GREYISH GREY,HARD

625.00   2.00   2.00   625.00   2.00   2.00   CLAYSTONE,GREYISH GREY,HARD

627.00   3.00   2.99   627.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,HARD

BOREHOLE CLOSED AT   630.00 M DEPTH

---------------------------------------------------------------------------

BOREHOLE NO    :CMWWB12              LATITUDE    : 271317.9331M                   DATE OF COMMENCMENT: 06.12.2024

REDUCED LEVEL  : 252.50M            DEPARTURE   : 2204740.6516M                  DATE OF COMPLEYION: 29.01.2025

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

.00   2.00    .90      .00   2.00    .90   SOIL,BLACK

SOIL

********************************************************************************************************************************

TRAP

2.00   1.00    .18     2.00   1.00    .18   BASALT,GREY                                                              18

3.00   3.00   1.10     3.00   3.00   1.10   BASALT,GREY,EXTREMELY STRONG

6.00   3.00   1.02     6.00   3.00   1.02   BASALT,GREY,EXTREMELY STRONG                                            4.3

9.00   3.00   1.40     9.00   3.00   1.40   BASALT,GREY,EXTREMELY STRONG                                            6.7

12.00   3.00   1.50    12.00   3.00   1.50   BASALT,GREY,EXTREMELY STRONG                                             10

****************************************************************************************************************************

MOTUR

15.00   3.00   1.45    15.00   3.00   1.45   SANDSTONE(FINE TO MEDIUM GRAINED),GREY

18.00   3.00   1.70    18.00   3.00   1.70   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY

21.00   1.65    .80    21.00   1.65    .80   SANDSTONE(MED. TO C. GRAINED),REDDISH BROWN

22.65   1.35    .65    22.65   1.35    .65   MUDSTONE,GREY,HARD

24.00   2.40   1.60    24.00   2.40   1.60   MUDSTONE,GREY,STRONG ROCKS                                               28

26.40    .60    .19    26.40    .60    .19   SANDSTONE(MED. TO C. GRAINED),REDDISH BROWN                              32

27.00    .50    .20    27.00    .50    .20   SANDSTONE(MED. TO C. GRAINED),REDDISH BROWN,

27.50   1.90   1.59    27.50   1.90   1.59   MUDSTONE,REDDISH BROWN

29.40    .60    .26    29.40    .60    .26   SANDSTONE(COARSE GRAINED),REDDISH GREY                                   28

30.00   3.00   2.40    30.00   3.00   2.40   SANDSTONE(COARSE GRAINED),REDDISH GREY,STRONG ROCKS                      56

33.00   3.00   2.98    33.00   3.00   2.98   MUDSTONE,GREY,STRONG ROCKS

36.00   3.00   2.99    36.00   3.00   2.99   MUDSTONE,GREY                                                            61

39.00   3.00   3.00    39.00   3.00   3.00   MUDSTONE,GREY,HARD                                                       75

42.00    .65    .62    42.00    .65    .62   MUDSTONE,GREY,HARD                                                       31

42.65   2.35   2.35    42.65   2.35   2.35   CLAYSTONE,REDDISH BROWN,HARD                                             48

45.00   3.00   2.93    45.00   3.00   2.93   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

48.00   2.60   2.50    48.00   2.60   2.50   CLAYSTONE,CHOCOLATEBROWN,STRONG ROCKS                                    15

50.60    .40    .37    50.60    .40    .37   MUDSTONE,GREY

51.00   1.40    .98    51.00   1.40    .98   MUDSTONE,GREY

52.40   1.60   1.20    52.40   1.60   1.20   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              43

54.00   3.00   2.90    54.00   3.00   2.90   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH GREY                           46

57.00   3.00   1.77    57.00   3.00   1.77   SANDSTONE(COARSE TO V.C.GRAINED),REDDISH BROWN                           55

60.00   3.00   2.70    60.00   3.00   2.70   CLAYSTONE,REDDISH BROWN                                                  24

63.00   1.10    .72    63.00   1.10    .72   CLAYSTONE,REDDISH BROWN

64.10   1.90   1.62    64.10   1.90   1.62   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        49

66.00   2.00    .30    66.00   2.00    .30   SANDSTONE(COARSE GRAINED),REDDISH BROWN

68.00   1.00    .45    68.00   1.00    .45   SANDSTONE(COARSE GRAINED),REDDISH BROWN

69.00   3.00   1.08    69.00   3.00   1.08   SANDSTONE(MEDIUM GRAINED),YELLOWISHBROWN

72.00   3.00    .93    72.00   3.00    .93   SANDSTONE(MEDIUM GRAINED),YELLOWISHGREY

75.00   3.00   1.12    75.00   3.00   1.12   SANDSTONE(MEDIUM GRAINED),YELLOWISHGREY

78.00   3.00   1.04    78.00   3.00   1.04   SANDSTONE(MEDIUM GRAINED),YELLOWISHGREY

81.00   3.00   1.18    81.00   3.00   1.18   SANDSTONE(MEDIUM GRAINED),YELLOWISHGREY

84.00   3.00    .70    84.00   3.00    .70   SANDSTONE(MEDIUM GRAINED),YELLOWISHGREY

87.00   3.00    .85    87.00   3.00    .85   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY

90.00   3.00   1.25    90.00   3.00   1.25   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY

93.00    .50    .30    93.00    .50    .30   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY

93.50    .50    .25    93.50    .50    .25   SANDSTONE(COARSE GRAINED),REDDISH BROWN

94.00    .40    .25    94.00    .40    .25   CLAYSTONE,REDDISH BROWN                                                  50

94.40    .45    .40    94.40    .45    .40   SANDSTONE(FINE GRAINED),YELLOWISHGREY                                    67

94.85    .25    .22    94.85    .25    .22   CLAYSTONE,REDDISH BROWN

95.10    .90    .90    95.10    .90    .90   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW                        63

96.00   3.00    .53    96.00   3.00    .53   SANDSTONE(COARSE GRAINED),REDDISH BROWN

99.00   3.00    .85    99.00   3.00    .85   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH YELLOW

102.00   2.50   2.20   102.00   2.50   2.20   CLAYSTONE,REDDISH BROWN,SLICKENSIDED

104.50    .50    .27   104.50    .50    .27   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

105.00    .45    .40   105.00    .45    .40   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE

105.45   2.55   2.40   105.45   2.55   2.40   CLAYSTONE,BROWNISH GREY

108.00   2.75   2.42   108.00   2.75   2.42   CLAYSTONE,BROWNISH GREY                                                  17

110.75    .25    .15   110.75    .25    .15   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY

111.00    .40    .35   111.00    .40    .35   CLAYSTONE,REDDISH BROWN

111.40   2.60   2.40   111.40   2.60   2.40   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY,                         26

114.00   2.00   1.65   114.00   2.00   1.65   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          58

116.00   1.00    .40   116.00   1.00    .40   MUDSTONE,BROWNISH GREY

117.00   3.00    .96   117.00   3.00    .96   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              15

120.00   1.00    .86   120.00   1.00    .86   SANDSTONE(COARSE GRAINED),YELLOWISHGREY,SLICKENSIDED

121.00    .30    .12   121.00    .30    .12   CLAYSTONE,REDDISH BROWN,FAULTED

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

121.30   1.70   1.40   121.30   1.70   1.40   SANDSTONE(FINE GRAINED),WHITE,FRIABLE

123.00   1.25   1.15   123.00   1.25   1.15   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY,

124.25    .20    .18   124.25    .20    .18   CLAYSTONE,REDDISH BROWN                                                  90

124.45    .45    .36   124.45    .45    .36   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY

124.90    .52    .51   124.90    .52    .51   CLAYSTONE,REDDISH BROWN                                                  71

125.42    .58    .57   125.42    .58    .57   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          66

126.00   3.00   2.80   126.00   3.00   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY

129.00    .40    .37   129.00    .40    .37   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY

129.40    .35    .32   129.40    .35    .32   CLAYSTONE,REDDISH BROWN                                                  46

129.75   2.25   2.05   129.75   2.25   2.05   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          18

132.00   3.00   2.95   132.00   3.00   2.95   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY,                         43

135.00   3.00   2.85   135.00   3.00   2.85   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          37

138.00   3.00   2.65   138.00   3.00   2.65   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          46

141.00   3.00   2.60   141.00   3.00   2.60   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          36

144.00    .70    .38   144.00    .70    .38   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

144.70   1.15    .95   144.70   1.15    .95   CLAYSTONE,BROWNISH GREY,FRACTURE

145.85   1.15   1.00   145.85   1.15   1.00   SANDSTONE(MEDIUM GRAINED),WHITE                                          83

147.00   3.00   1.25   147.00   3.00   1.25   SANDSTONE(MEDIUM GRAINED),WHITE                                          32

150.00   1.10    .80   150.00   1.10    .80   SANDSTONE(MEDIUM GRAINED),YELLOWISHGREY                                  63

151.10   1.30    .77   151.10   1.30    .77   CLAYSTONE,BROWNISH GREY                                                  42

152.40    .60    .37   152.40    .60    .37   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY,                         42

153.00   1.30    .36   153.00   1.30    .36   CLAYSTONE,REDDISH BROWN                                                  23

154.30   1.70   1.50   154.30   1.70   1.50   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY,                         68

156.00   3.00   2.60   156.00   3.00   2.60   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY,FRACTURE                     61

159.00    .50    .35   159.00    .50    .35   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              56

159.50    .65    .42   159.50    .65    .42   CLAYSTONE,BROWNISH GREY                                                  57

160.15    .70    .56   160.15    .70    .56   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          79

160.85    .50    .40   160.85    .50    .40   CLAYSTONE,CHOCOLATEBROWN                                                 68

161.35    .65    .52   161.35    .65    .52   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          74

162.00    .50    .37   162.00    .50    .37   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY,FRACTURE

162.50    .70    .42   162.50    .70    .42   CLAYSTONE,BROWNISH GREY                                                  49

163.20   1.80   1.57   163.20   1.80   1.57   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              74

165.00   3.00   1.77   165.00   3.00   1.77   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY,FRACTURE                     24

168.00    .75    .62   168.00    .75    .62   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH GREY                           67

168.75   2.25   2.18   168.75   2.25   2.18   CLAYSTONE,REDDISH BROWN                                                  57

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

171.00   3.00   2.80   171.00   3.00   2.80   SANDSTONE(FINE GRAINED),BROWNISH GREY,FRACTURE                           69

174.00   3.00   2.62   174.00   3.00   2.62   SANDSTONE(FINE GRAINED),BROWNISH GREY,FRACTURE                           34

177.00   3.00   2.35   177.00   3.00   2.35   SANDSTONE(FINE GRAINED),BROWNISH GREY,FRACTURE                           44

180.00   3.00   2.30   180.00   3.00   2.30   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY,                         21

183.00   3.00   1.30   183.00   3.00   1.30   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY,                         24

186.00   3.00   2.25   186.00   3.00   2.25   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY,                         30

189.00   3.00   2.50   189.00   3.00   2.50   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          28

192.00   3.00   2.41   192.00   3.00   2.41   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          54

195.00   3.00   1.00   195.00   3.00   1.00   SANDSTONE(FINE TO MEDIUM GRAINED),YELLOWISHGREY                          11

198.00   3.00   2.60   198.00   3.00   2.60   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              60

201.00   3.00   2.10   201.00   3.00   2.10   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH GREY                           65

204.00   3.00   2.55   204.00   3.00   2.55   SANDSTONE(COARSE TO V.C.GRAINED),BROWNISH GREY                           60

207.00   3.00   2.30   207.00   3.00   2.30   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHGREY,                          43

210.00   3.00   3.00   210.00   3.00   3.00   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHGREY,                          89

213.00   3.00   3.00   213.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              94

216.00   3.00   3.00   216.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              99

219.00   3.00   3.00   219.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              98

222.00   3.00   3.00   222.00   3.00   3.00   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHGREY                          100

225.00   3.00   2.99   225.00   3.00   2.99   SANDSTONE(COARSE TO V.C.GRAINED),YELLOWISHGREY,                          89

228.00   3.00   3.00   228.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY                              97

231.00   3.00   3.00   231.00   3.00   3.00   SANDSTONE(COARSE GRAINED),YELLOWISHGREY                                  94

234.00   3.00   2.50   234.00   3.00   2.50   SANDSTONE(COARSE GRAINED),YELLOWISHGREY                                  56

237.00   3.00   1.70   237.00   3.00   1.70   SANDSTONE(MED. TO C. GRAINED),YELLOWISHGREY,FRACTURE                     30

240.00   3.00   1.90   240.00   3.00   1.90   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              31

243.00   3.00   3.00   243.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          28

246.00   3.00   3.00   246.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          84

249.00   3.00   2.98   249.00   3.00   2.98   SANDSTONE(FINE GRAINED),BROWNISH GREY                                    82

252.00   1.98   1.98   252.00   1.98   1.98   CLAYSTONE,GREYISH BROWN                                                  76

253.98   1.02   1.02   253.98   1.02   1.02   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    93

255.00   3.00   3.00   255.00   3.00   3.00   CLAYSTONE,GREYISH BROWN,SLICKENSIDED                                     67

258.00   1.00   1.00   258.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        70

259.00   3.00   2.80   259.00   3.00   2.80   CLAYSTONE,REDDISH BROWN,FRACTURE                                         49

262.00   3.00   2.90   262.00   3.00   2.90   MUDSTONE,BROWNISH GREY,FRACTURE                                          20

265.00   1.00    .80   265.00   1.00    .80   MUDSTONE,BROWNISH GREY                                                   20

266.00   3.00   2.80   266.00   3.00   2.80   MUDSTONE,GREY                                                            23

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

269.00   3.00   2.90   269.00   3.00   2.90   MUDSTONE,GREY,SLICKENSIDED                                               18

272.00   3.00   2.80   272.00   3.00   2.80   MUDSTONE,GREY,SLICKENSIDED

275.00   3.00   2.90   275.00   3.00   2.90   MUDSTONE,GREY,SLICKENSIDED                                               23

278.00   3.00   2.70   278.00   3.00   2.70   MUDSTONE,GREY,SLICKENSIDED                                                7

281.00   3.00   2.85   281.00   3.00   2.85   MUDSTONE,GREY,SLICKENSIDED                                               66

284.00   3.00   2.98   284.00   3.00   2.98   MUDSTONE,BROWNISH GREY,SLICKENSIDED                                      21

287.00   3.00   2.99   287.00   3.00   2.99   MUDSTONE,GREY,SLICKENSIDED                                              6.3

290.00   1.00   1.00   290.00   1.00   1.00   MUDSTONE,GREY,SLICKENSIDED                                               25

291.00   3.00   3.00   291.00   3.00   3.00   MUDSTONE,GREY,SLICKENSIDED                                               16

294.00   3.00   3.00   294.00   3.00   3.00   MUDSTONE,GREY,SLICKENSIDED                                                7

297.00   3.00   3.00   297.00   3.00   3.00   MUDSTONE,BROWNISH GREY,SLICKENSIDED

300.00   3.00   3.00   300.00   3.00   3.00   MUDSTONE,BROWNISH GREY,SLICKENSIDED                                      21

303.00    .55    .55   303.00    .55    .55   SANDSTONE(FINE GRAINED),GREY                                             98

303.55   2.45   2.45   303.55   2.45   2.45   MUDSTONE,GREY                                                            49

306.00   3.00   2.99   306.00   3.00   2.99   MUDSTONE,GREY,SLICKENSIDED                                               59

309.00   3.00   3.00   309.00   3.00   3.00   MUDSTONE,GREY,SLICKENSIDED                                               27

312.00   3.00   2.99   312.00   3.00   2.99   MUDSTONE,GREY,SLICKENSIDED                                              8.7

315.00   3.00   3.00   315.00   3.00   3.00   MUDSTONE,GREY,SLICKENSIDED                                               77

318.00   2.00   2.00   318.00   2.00   2.00   CLAYSTONE,GREYISH BROWN                                                  21

320.00   1.00    .72   320.00   1.00    .72   MUDSTONE,BROWNISH GREY

321.00   2.00   1.68   321.00   2.00   1.68   MUDSTONE,GREY,SLICKENSIDED                                               69

323.00    .30    .30   323.00    .30    .30   MUDSTONE,GREY                                                            70

323.30    .70    .70   323.30    .70    .70   SANDSTONE(FINE GRAINED),REDDISH BROWN                                    99

324.00   3.00   2.50   324.00   3.00   2.50   MUDSTONE,GREY                                                            35

327.00    .36    .36   327.00    .36    .36   MUDSTONE,GREY

327.36    .34    .34   327.36    .34    .34   SANDSTONE(FINE GRAINED),BROWNISH GREY                                    41

327.70   1.30   1.27   327.70   1.30   1.27   MUDSTONE,GREY,SLICKENSIDED

329.00   1.00   1.00   329.00   1.00   1.00   MUDSTONE,GREY,SLICKENSIDED

330.00   3.00   3.00   330.00   3.00   3.00   CLAYSTONE,GREYISH BROWN,SLICKENSIDED                                     34

333.00   3.00   2.98   333.00   3.00   2.98   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

336.00   3.00   3.00   336.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 20

339.00    .98    .98   339.00    .98    .98   SANDSTONE(FINE TO MEDIUM GRAINED),WHITE,FRACTURE                         57

339.98    .92    .92   339.98    .92    .92   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              12

340.90   1.10   1.10   340.90   1.10   1.10   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        39

342.00   3.00   2.99   342.00   3.00   2.99   MUDSTONE,GREY,FRACTURE

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

345.00    .35    .35   345.00    .35    .35   SANDSTONE(FINE GRAINED),GREYISH WHITE

345.35   2.65   2.65   345.35   2.65   2.65   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              74

348.00   2.30   2.30   348.00   2.30   2.30   SANDSTONE(COARSE GRAINED),GREYISH GREEN                                  64

350.30    .70    .70   350.30    .70    .70   SHALE,DARK GREY,SLICKENSIDED

351.00    .37    .37   351.00    .37    .37   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          38

351.37   2.63   2.63   351.37   2.63   2.63   CLAYSTONE,BROWNISH GREY,SLICKENSIDED                                     27

354.00   3.00   3.00   354.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        54

357.00   3.00   3.00   357.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        71

360.00   2.35   2.35   360.00   2.35   2.35   CLAYSTONE,GREYISH BROWN                                                  30

362.35    .65    .63   362.35    .65    .63   SANDSTONE(FINE GRAINED),BROWNISH GREY                                    62

363.00   3.00   3.00   363.00   3.00   3.00   CLAYSTONE,BROWNISH GREY,FRACTURE                                         45

366.00   2.00   2.00   366.00   2.00   2.00   MUDSTONE,GREY,SLICKENSIDED                                               59

368.00   3.00   3.00   368.00   3.00   3.00   CLAYSTONE,BROWNISH GREY,SLICKENSIDED                                     43

371.00   3.00   3.00   371.00   3.00   3.00   CLAYSTONE,GREYISH BROWN,SLICKENSIDED                                     83

374.00   3.00   3.00   374.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        59

377.00   3.00   3.00   377.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        26

380.00   3.00   3.00   380.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    38

383.00   3.00   3.00   383.00   3.00   3.00   CLAYSTONE,GREYISH BROWN                                                  26

386.00   1.00    .98   386.00   1.00    .98   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

387.00   3.00   3.00   387.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

390.00   3.00   3.00   390.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

393.00   1.24   1.24   393.00   1.24   1.24   CLAYSTONE,GREYISH BROWN                                                  47

394.24    .49    .49   394.24    .49    .49   MUDSTONE,GREY                                                            22

394.73   1.10   1.10   394.73   1.10   1.10   SANDSTONE(COARSE GRAINED),GREYISH WHITE                                  91

395.83    .17    .17   395.83    .17    .17   MUDSTONE,GREY                                                           100

396.00    .20    .20   396.00    .20    .20   MUDSTONE,GREY

396.20    .80    .80   396.20    .80    .80   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

397.00   3.00   3.00   397.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 58

400.00   3.00   3.00   400.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    55

403.00   3.00   3.00   403.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                   4.3

406.00   3.00   2.99   406.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN                                                 21

409.00   3.00   3.00   409.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 16

412.00   3.00   3.00   412.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                         5

415.00   3.00   3.00   415.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 26

418.00   3.00   3.00   418.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        40

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

421.00   3.00   3.00   421.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN

424.00   3.00   3.00   424.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 16

427.00   3.00   3.00   427.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 15

430.00   3.00   3.00   430.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 64

433.00   1.00   1.00   433.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 49

434.00   2.00   1.98   434.00   2.00   1.98   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        82

436.00   3.00   3.00   436.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 67

439.00   3.00   3.00   439.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 20

442.00   1.00    .98   442.00   1.00    .98   CLAYSTONE,CHOCOLATEBROWN

443.00   3.00   3.00   443.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN

446.00   3.00   3.00   446.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                   4.7

449.00   3.00   3.00   449.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 12

452.00   1.00   1.00   452.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

453.00   3.00   3.00   453.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 18

456.00   3.00   3.00   456.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    27

459.00   1.00   1.00   459.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 94

460.00    .22    .22   460.00    .22    .22   CLAYSTONE,CHOCOLATEBROWN

460.22    .60    .60   460.22    .60    .60   SANDSTONE(MED. TO C. GRAINED),WHITE                                      18

460.82   1.18   1.18   460.82   1.18   1.18   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        10

462.00   3.00   3.00   462.00   3.00   3.00   CLAYSTONE,GREYISH BROWN,SLICKENSIDED

465.00   3.00   3.00   465.00   3.00   3.00   CLAYSTONE,BROWNISH GREY,SLICKENSIDED

468.00   3.00   3.00   468.00   3.00   3.00   CLAYSTONE,BROWNISH GREY,SLICKENSIDED                                     24

471.00   1.00   1.00   471.00   1.00   1.00   MUDSTONE,GREY,FRACTURE                                                   12

472.00   3.00   3.00   472.00   3.00   3.00   CLAYSTONE,GREYISH BROWN,FRACTURE                                        7.3

475.00   3.00   3.00   475.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    27

478.00   3.00   3.00   478.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    13

481.00   3.00   3.00   481.00   3.00   3.00   CLAYSTONE,BROWNISH GREY,FRACTURE                                         29

484.00   3.00   3.00   484.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

487.00   3.00   3.00   487.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

490.00   3.00   3.00   490.00   3.00   3.00   CLAYSTONE,MEDIUM CRYSTALLINE                                             19

493.00   3.00   3.00   493.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 11

496.00   3.00   3.00   496.00   3.00   3.00   CLAYSTONE,BROWNISH GREY                                                 8.3

499.00   3.00   3.00   499.00   3.00   3.00   CLAYSTONE,BROWNISH GREY                                                 4.7

502.00    .60    .60   502.00    .60    .60   CLAYSTONE,CHOCOLATEBROWN

502.60    .50    .50   502.60    .50    .50   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              86

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

503.10    .33    .33   503.10    .33    .33   MUDSTONE,GREY

503.43   1.57   1.57   503.43   1.57   1.57   CLAYSTONE,CHOCOLATEBROWN                                                8.3

505.00   3.00   3.00   505.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

508.00   3.00   3.00   508.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    13

511.00   3.00   3.00   511.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                8.3

514.00    .60    .60   514.00    .60    .60   CLAYSTONE,CHOCOLATEBROWN

514.60    .70    .70   514.60    .70    .70   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          96

515.30   1.33   1.33   515.30   1.33   1.33   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

516.63    .37    .37   516.63    .37    .37   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              73

517.00   3.00   3.00   517.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              77

520.00    .15    .15   520.00    .15    .15   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                             100

520.15    .13    .13   520.15    .13    .13   MUDSTONE,GREY

520.28    .23    .23   520.28    .23    .23   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                             100

520.51   2.49   2.49   520.51   2.49   2.49   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    17

523.00   3.00   3.00   523.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        12

526.00   3.00   3.00   526.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 40

529.00   3.00   3.00   529.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    39

532.00   3.00   3.00   532.00   3.00   3.00   CLAYSTONE,GREYISH BROWN,FRACTURE                                         27

535.00    .63    .63   535.00    .63    .63   CLAYSTONE,BROWNISH GREY                                                  19

535.63    .55    .55   535.63    .55    .55   MUDSTONE,GREY

536.18   1.82   1.82   536.18   1.82   1.82   CLAYSTONE,CHOCOLATEBROWN                                                 12

538.00   3.00   3.00   538.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

541.00   3.00   2.99   541.00   3.00   2.99   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED                                    32

544.00   3.00   3.00   544.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

547.00    .28    .28   547.00    .28    .28   MUDSTONE,GREY,SLICKENSIDED

547.28    .65    .64   547.28    .65    .64   SANDSTONE(MED. TO C. GRAINED),GREY,FRACTURE                              66

547.93    .51    .49   547.93    .51    .49   MUDSTONE,GREY,SLICKENSIDED

548.44    .56    .54   548.44    .56    .54   CLAYSTONE,CHOCOLATEBROWN,SLICKENSIDED

549.00   1.00   1.00   549.00   1.00   1.00   CLAYSTONE,GREYISH BROWN,SLICKENSIDED

550.00   3.00   2.98   550.00   3.00   2.98   CLAYSTONE,GREYISH BROWN,FRACTURE                                        8.3

553.00    .33    .33   553.00    .33    .33   MUDSTONE,GREY,FRACTURE

553.33    .68    .68   553.33    .68    .68   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              69

554.01    .64    .62   554.01    .64    .62   CLAYSTONE,BROWNISH GREY,FRACTURE

554.65    .20    .17   554.65    .20    .17   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                             100

554.85   1.15   1.10   554.85   1.15   1.10   MUDSTONE,GREY                                                            36

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

556.00    .34    .34   556.00    .34    .34   SANDSTONE(FINE GRAINED),GREY                                             50

556.34   2.66   2.66   556.34   2.66   2.66   CLAYSTONE,GREYISH BROWN,SLICKENSIDED                                     28

559.00   3.00   3.00   559.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 34

562.00   3.00   3.00   562.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        19

565.00   1.00   1.00   565.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN                                                 52

566.00    .12    .12   566.00    .12    .12   CLAYSTONE,CHOCOLATEBROWN

566.12    .93    .93   566.12    .93    .93   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         74

567.05    .95    .94   567.05    .95    .94   CLAYSTONE,GREYISH BROWN,FRACTURE

568.00    .50    .48   568.00    .50    .48   CLAYSTONE,GREYISH BROWN

568.50    .85    .83   568.50    .85    .83   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          82

569.35   1.65   1.60   569.35   1.65   1.60   CLAYSTONE,BROWNISH GREY

571.00   2.30   2.30   571.00   2.30   2.30   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          53

573.30    .70    .70   573.30    .70    .70   CLAYSTONE,BROWNISH GREY,FRACTURE

574.00   3.00   3.00   574.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        25

577.00   3.00   3.00   577.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE                                        19

580.00   3.00   3.00   580.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

583.00   3.00   3.00   583.00   3.00   3.00   CLAYSTONE,CHOCOLATEBROWN                                                 14

586.00   2.00   2.00   586.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN                                                 30

588.00   1.00    .99   588.00   1.00    .99   CLAYSTONE,CHOCOLATEBROWN                                                 42

589.00   3.00   3.00   589.00   3.00   3.00   CLAYSTONE,GREYISH BROWN,FRACTURE                                         42

592.00   3.00   3.00   592.00   3.00   3.00   CLAYSTONE,GREYISH BROWN,FRACTURE                                        5.7

595.00   1.00   1.00   595.00   1.00   1.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

596.00    .80    .80   596.00    .80    .80   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

596.80   1.50   1.50   596.80   1.50   1.50   MUDSTONE,GREY,FRACTURE

598.30    .70    .70   598.30    .70    .70   SILTSTONE,GREY,FRACTURE

599.00    .20    .20   599.00    .20    .20   SILTSTONE,GREY,SLICKENSIDED

599.20   2.80   2.79   599.20   2.80   2.79   MUDSTONE,GREY                                                            20

602.00    .16    .16   602.00    .16    .16   MUDSTONE,GREY

602.16    .41    .41   602.16    .41    .41   CLAYSTONE,GREYISH BROWN,SLICKENSIDED

602.57    .15    .15   602.57    .15    .15   MUDSTONE,GREY                                                           100

602.72    .43    .43   602.72    .43    .43   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                         100

603.15   1.85   1.85   603.15   1.85   1.85   MUDSTONE,BROWNISH GREY                                                   26

605.00   3.00   3.00   605.00   3.00   3.00   MUDSTONE,GREY,FRACTURE                                                  7.7

608.00   1.00   1.00   608.00   1.00   1.00   MUDSTONE,GREY,FRACTURE

609.00   2.60   2.60   609.00   2.60   2.60   MUDSTONE,GREY,FRACTURE

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

611.60    .26    .26   611.60    .26    .26   SILTSTONE,GREY

611.86    .14    .14   611.86    .14    .14   MUDSTONE,GREY,FRACTURE

612.00   3.00   3.00   612.00   3.00   3.00   MUDSTONE,GREY,FRACTURE                                                   29

615.00   1.00    .97   615.00   1.00    .97   MUDSTONE,GREY,FRACTURE                                                   38

616.00   2.00   2.00   616.00   2.00   2.00   MUDSTONE,GREY,FRACTURE                                                   35

618.00   1.30   1.30   618.00   1.30   1.30   MUDSTONE,GREY,FRACTURE                                                   45

619.30    .20    .20   619.30    .20    .20   SANDSTONE(FINE GRAINED),GREYISH GREEN                                    65

619.50    .21    .21   619.50    .21    .21   MUDSTONE,GREY

619.71    .18    .18   619.71    .18    .18   SANDSTONE(FINE GRAINED),GREYISH GREEN                                   100

619.89    .60    .60   619.89    .60    .60   SHALE,DARK GREY,SLICKENSIDED

620.49    .51    .51   620.49    .51    .51   MUDSTONE,GREY

621.00   3.00   3.00   621.00   3.00   3.00   MUDSTONE,GREY,SLICKENSIDED

624.00   1.86   1.86   624.00   1.86   1.86   MUDSTONE,GREY,SLICKENSIDED

625.86    .51    .51   625.86    .51    .51   CLAYSTONE,GREYISH BROWN,SLICKENSIDED

626.37    .63    .63   626.37    .63    .63   MUDSTONE,GREY,SLICKENSIDED

627.00   3.00   3.00   627.00   3.00   3.00   CLAYSTONE,BROWNISH GREY                                                  37

630.00   1.15   1.15   630.00   1.15   1.15   CLAYSTONE,GREYISH BROWN,HARD                                             16

631.15    .83    .83   631.15    .83    .83   MUDSTONE,GREY,HARD                                                       28

631.98    .46    .42   631.98    .46    .42   SHALE,GREY,SLICKENSIDED

632.44    .56    .56   632.44    .56    .56   CLAYSTONE,CHOCOLATEBROWN

633.00   1.00    .76   633.00   1.00    .76   MUDSTONE,GREY

634.00    .30    .28   634.00    .30    .28   CLAYSTONE,GREYISH BROWN

634.30   1.00   1.00   634.30   1.00   1.00   MUDSTONE,GREY,SLICKENSIDED

635.30    .50    .47   635.30    .50    .47   MUDSTONE,GREY,SLICKENSIDED                                               22

635.80    .20    .18   635.80    .20    .18   CLAYSTONE,GREYISH BROWN                                                  90

636.00   2.00   1.66   636.00   2.00   1.66   CLAYSTONE,GREYISH BROWN,FRACTURE                                         15

638.00   1.00    .98   638.00   1.00    .98   CLAYSTONE,BROWNISH GREY,FRACTURE                                         20

639.00   1.59   1.56   639.00   1.59   1.56   SILTSTONE,GREY                                                           41

640.59    .40    .38   640.59    .40    .38   CLAYSTONE,BROWNISH GREY,SLICKENSIDED

640.99    .90    .89   640.99    .90    .89   SHALE,GREY

641.89    .11    .11   641.89    .11    .11   SILTSTONE,GREY,SLICKENSIDED

642.00    .40    .37   642.00    .40    .37   SILTSTONE,GREY,SLICKENSIDED                                              38

642.40    .65    .63   642.40    .65    .63   SHALE,GREY,SLICKENSIDED                                                  26

643.05   1.95   1.92   643.05   1.95   1.92   MUDSTONE,GREY                                                           7.2

645.00    .22    .22   645.00    .22    .22   MUDSTONE,GREY

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

645.22    .38    .38   645.22    .38    .38   CLAYSTONE,GREYISH BROWN,FRACTURE                                         32

645.60    .40    .40   645.60    .40    .40   MUDSTONE,GREY,FRACTURE                                                   33

646.00   2.00   1.98   646.00   2.00   1.98   MUDSTONE,GREY,FRACTURE                                                   39

648.00    .28    .27   648.00    .28    .27   MUDSTONE,GREY

648.28    .72    .72   648.28    .72    .72   CLAYSTONE,BROWNISH GREY

649.00   2.00   2.00   649.00   2.00   2.00   CLAYSTONE,GREYISH BROWN,FRACTURE

651.00   3.00   3.00   651.00   3.00   3.00   CLAYSTONE,GREYISH BROWN,FRACTURE

654.00   3.00   3.00   654.00   3.00   3.00   CLAYSTONE,GREYISH BROWN,FRACTURE

657.00   2.00   2.00   657.00   2.00   2.00   CLAYSTONE,CHOCOLATEBROWN,FRACTURE

659.00   3.00   3.00   659.00   3.00   3.00   CLAYSTONE,BROWNISH GREY                                                 5.7

662.00   3.00   3.00   662.00   3.00   3.00   CLAYSTONE,BROWNISH GREY                                                  13

665.00   1.00   1.00   665.00   1.00   1.00   CLAYSTONE,GREYISH BROWN

666.00    .37    .37   666.00    .37    .37   CLAYSTONE,GREYISH BROWN

666.37   2.63   2.63   666.37   2.63   2.63   MUDSTONE,GREY,SLICKENSIDED NEAR HORIZONTAL

669.00   2.33   2.33   669.00   2.33   2.33   MUDSTONE,GREY,SLICKENSIDED NEAR HORIZONTAL

671.33    .67    .65   671.33    .67    .65   CLAYSTONE,GREYISH BROWN,SLICKENSIDED NEAR HORIZONTAL

672.00   3.00   2.98   672.00   3.00   2.98   CLAYSTONE,GREYISH BROWN,SLICKENSIDED NEAR HORIZONTAL

675.00   2.00   1.95   675.00   2.00   1.95   CLAYSTONE,GREYISH BROWN                                                  30

677.00   1.00    .98   677.00   1.00    .98   SANDSTONE(FINE GRAINED),BROWNISH GREY                                    89

678.00    .79    .79   678.00    .79    .79   SANDSTONE(FINE GRAINED),BROWNISH GREY                                    47

678.79   2.21   2.21   678.79   2.21   2.21   CLAYSTONE,GREYISH BROWN,SLICKENSIDED                                    6.3

681.00   1.45   1.42   681.00   1.45   1.42   CLAYSTONE,GREYISH BROWN,SLICKENSIDED

682.45   1.55   1.55   682.45   1.55   1.55   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          77

684.00   1.32   1.32   684.00   1.32   1.32   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              84

685.32    .16    .16   685.32    .16    .16   CLAYSTONE,YELLOW

685.48    .54    .54   685.48    .54    .54   CLAYSTONE,VARIEGATE

686.02    .17    .17   686.02    .17    .17   CLAYSTONE,VARIEGAT,SLICKENSIDED

686.19    .33    .33   686.19    .33    .33   CLAYSTONE,VARIEGATE

686.52    .23    .23   686.52    .23    .23   CLAYSTONE,VARIEGAT,SLICKENSIDED

686.75    .25    .25   686.75    .25    .25   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          92

687.00    .58    .56   687.00    .58    .56   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          55

687.58    .20    .19   687.58    .20    .19   CLAYSTONE,VARIEGATE

687.78    .15    .15   687.78    .15    .15   MUDSTONE,YELLOW

687.93   2.07   2.07   687.93   2.07   2.07   SANDSTONE(FINE GRAINED),BROWNISH GREY                                    93

690.00   3.00   2.99   690.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          77

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

693.00   3.00   3.00   693.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),BROWNISH GREY                              87

696.00   3.00   3.00   696.00   3.00   3.00   SANDSTONE(MED. TO C. GRAINED),GREYISH YELLOW                             95

699.00   1.40   1.21   699.00   1.40   1.21   SANDSTONE(MED. TO C. GRAINED),GREYISH YELLOW                             62

700.40    .40    .28   700.40    .40    .28   CLAYSTONE,YELLOW

700.80   1.20    .96   700.80   1.20    .96   SANDSTONE(FINE GRAINED),GREYISH YELLOW                                   78

702.00   3.00   2.60   702.00   3.00   2.60   SANDSTONE(FINE TO MEDIUM GRAINED),BROWNISH GREY                          65

705.00   3.00   2.83   705.00   3.00   2.83   SANDSTONE(FINE TO MEDIUM GRAINED),GREY                                   78

708.00   3.00   2.60   708.00   3.00   2.60   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          72

711.00   3.00   2.00   711.00   3.00   2.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          35

***************************************************************************************************************************

BARAKAR

714.00   3.00   2.32   714.00   3.00   2.32   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          59

717.00   3.00   2.80   717.00   3.00   2.80   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          53

720.00   3.00   3.00   720.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          54

723.00   3.00   2.98   723.00   3.00   2.98   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          90

726.00   3.00   2.75   726.00   3.00   2.75   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH GREEN                          76

729.00   3.00   2.60   729.00   3.00   2.60   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          32

732.00   3.00   2.65   732.00   3.00   2.65   SANDSTONE(MEDIUM GRAINED),GREYISH WHITE                                  54

735.00    .14    .14   735.00    .14    .14   SANDSTONE(MEDIUM GRAINED),GREYISH GREEN

735.14    .40    .26   735.14    .40    .26   CLAY,GREY,FRACTURE

735.54   1.00    .99   735.54   1.00    .99   SANDY SHALE,GREY,SLICKENSIDED

736.54    .31    .31   736.54    .31    .31   SHALE,GREY

736.85   1.15   1.00   736.85   1.15   1.00   CLAY,GREY,FRACTURE

738.00    .35    .33   738.00    .35    .33   SANDY SHALE,GREY

738.35    .55    .30   738.35    .55    .30   CLAY,GREY,FRACTURE

738.90    .60    .57   738.90    .60    .57   SHALE,GREY

739.50    .78    .75   739.50    .78    .75   SANDY SHALE,GREY,FRACTURE

740.28   1.20   1.20   740.28   1.20   1.20   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    71

741.48    .21    .21   741.48    .21    .21   INTERCALATIONS,GREY

741.69    .30    .30   741.69    .30    .30   SHALE,GREY,SLICKENSIDED

741.99    .20    .20   741.99    .20    .20   CARB SHALE LOW,SLICKENSIDED

742.19    .11    .11   742.19    .11    .11   COAL

742.30    .20    .20   742.30    .20    .20   SHALE,GREY

742.50    .47    .45   742.50    .47    .45   SANDY SHALE,GREY                                                         34

742.97    .60    .60   742.97    .60    .60   SHALY SANDSTONE,GREY                                                     27

743.57   1.02   1.02   743.57   1.02   1.02   INTERCALATIONS,GREY                                                      62

744.59    .91    .91   744.59    .91    .91   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    41

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

745.50   1.50   1.35   745.50   1.50   1.35   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          63

747.00   3.00   2.99   747.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          76

750.00    .28    .28   750.00    .28    .28   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          82

750.28    .58    .56   750.28    .58    .56   SHALE,GREY                                                               33

750.86    .17    .17   750.86    .17    .17   CARB SHALE LOW

751.03   1.97   1.97   751.03   1.97   1.97   SANDY SHALE,GREY                                                         42

753.00    .50    .48   753.00    .50    .48   SANDY SHALE,GREY                                                         94

753.50   2.50   2.49   753.50   2.50   2.49   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    97

756.00   2.35   2.35   756.00   2.35   2.35   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          75

758.35    .35    .34   758.35    .35    .34   CLAY,GREY,SLICKENSIDED

758.70    .30    .30   758.70    .30    .30   SANDSTONE(FINE GRAINED),GREYISH WHITE                                   100

759.00   3.00   2.99   759.00   3.00   2.99   SANDSTONE(FINE GRAINED),GREYISH WHITE,DISTINCT TOP                       93

762.00   3.00   3.00   762.00   3.00   3.00   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    94

765.00   3.00   2.99   765.00   3.00   2.99   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    93

768.00   1.75   1.75   768.00   1.75   1.75   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    85

769.75    .70    .70   769.75    .70    .70   SHALY SANDSTONE,GREY,SLICKENSIDED                                        99

770.45    .55    .53   770.45    .55    .53   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    85

771.00   3.00   2.98   771.00   3.00   2.98   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    92

774.00   3.00   2.99   774.00   3.00   2.99   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    96

777.00   3.00   2.98   777.00   3.00   2.98   SANDSTONE(FINE GRAINED),GREYISH WHITE,SLICKENSIDED                       92

780.00   3.00   2.98   780.00   3.00   2.98   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    82

783.00   1.76   1.76   783.00   1.76   1.76   SANDSTONE(MED. TO C. GRAINED),GREYISH WHITE                              93

784.76   1.24   1.24   784.76   1.24   1.24   SANDSTONE(FINE GRAINED),WHITE                                            97

786.00   3.00   2.99   786.00   3.00   2.99   SANDSTONE(FINE GRAINED),WHITE                                            97

789.00    .72    .72   789.00    .72    .72   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    94

789.72   1.04   1.04   789.72   1.04   1.04   COAL,DISTINCT TOP                                               IIA

790.76    .29    .29   790.76    .29    .29   SHALY COAL                                                      IIA

791.05    .95    .95   791.05    .95    .95   COAL,DISTINCT TOP                                               IIA

792.00    .18    .18   792.00    .18    .18   SANDSTONE(FINE GRAINED),GREYISH WHITE                                   100

792.18   1.87   1.87   792.18   1.87   1.87   INTERCALATIONS,GREY,DISTINCT TOP                                         99

794.05    .95    .95   794.05    .95    .95   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    91

795.00   3.00   3.00   795.00   3.00   3.00   SANDSTONE(FINE GRAINED),GREYISH WHITE                                    90

798.00   3.00   3.00   798.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          95

801.00   3.00   2.99   801.00   3.00   2.99   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          91

804.00    .07    .07   804.00    .07    .07   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

*************************************************************************************************************************

RUN DEPTH             LITHOLOGY DEPTH                    LITHOLOGY DESCRIPTION                     CORE   SEAM    RQD

FROM  THICK   RECO     FROM   THICK  RECO                                                            DIP    NAME

NESS    VERY            NESS   VERY

(M)   (M)              (M)    (M)

*************************************************************************************************************************

804.07    .17    .17   804.07    .17    .17   COAL,DISTINCT TOP                                               IIB

804.24    .37    .37   804.24    .37    .37   SHALY COAL,DISTINCT TOP                                         IIB

804.61    .44    .43   804.61    .44    .43   COAL,DISTINCT TOP                                               IIB

805.05    .15    .15   805.05    .15    .15   SHALY COAL,DISTINCT TOP                                         IIB

805.20    .50    .50   805.20    .50    .50   COAL,DISTINCT TOP                                               IIB

805.70    .37    .37   805.70    .37    .37   SHALY COAL,DISTINCT TOP                                         IIB

806.07    .21    .21   806.07    .21    .21   COAL,DISTINCT TOP                                               IIB

806.28    .11    .11   806.28    .11    .11   CARB SHALE LOW                                                  IIB

806.39    .61    .61   806.39    .61    .61   INTERCALATIONS,GREY,SLICKENSIDED                                         64

807.00   3.00   3.00   807.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          98

810.00   3.00   3.00   810.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          98

813.00   3.00   3.00   813.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                         100

816.00   2.35   2.35   816.00   2.35   2.35   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE                          99

818.35    .32    .32   818.35    .32    .32   COAL,DISTINCT TOP                                               I

818.67    .33    .33   818.67    .33    .33   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         70

819.00   3.00   3.00   819.00   3.00   3.00   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE,                         81

822.00   3.00   2.95   822.00   3.00   2.95   SANDSTONE(FINE TO MEDIUM GRAINED),GREYISH WHITE

BOREHOLE CLOSED AT   825.00 M DEPTH