ANNEXURE - VA

DESCRIPTIVE BAND-BY-BAND ANALYTICAL DATA OF BOREHOLES DRILLED IN

------------------------------------------------------------------------------------------

BLOCK: WEST OF BORDA & GHONSA PARSODA, COAL FIELD: WARDHA VALLEY       STATE: MAHARASHTRA

------------------------------------------------------------------------------------------

BOREHOLE NO   :  CMWWB01                                        LATITUDE    : 2209645.917

REDUCED LEVEL(M): 282.27                                          DEPARTURE   : 264858.8494

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

IIA  815.60    .37    .37  COAL                                         601   6.4 20.3

IIA  815.97    .65    .62  SHALY COAL                                  1051   4.9 42.8

IIA  816.62    .60    .59  COAL                                         810   8.1 14.5

IIA  817.22    .19    .19  COAL                                         300   6.6 20.2

IIA  817.41    .79    .77  COAL                                        1020  10.6 11.2

IIA  818.20    .25    .24  COAL                                         479   4.9 34.7

818.45   1.00   2.52  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

*   SEAM ( IIA       ) OF THICKNESS   2.85m  FROM DEPTH 815.60m  TO  818.45m

819.75   1.25   2.52  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

======================================================================================================

ANALYSIS CONT..,

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB01                                    LATITUDE    : 2209645.917

REDUCED LEVEL(M):  282.27                                    DEPARTURE   : 264858.8494

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

821.00    .75    .70  INTERCALATIONS                               NOT ANALYSED

821.75   1.00    .99  SANDY SHALE                                  NOT ANALYSED

IIB  822.75    .19    .16  SHALY COAL                                   356   4.5  49.5

IIB  822.94    .46    .46  COAL                                         685   9.0  17.3

IIB  823.40    .28    .27  COAL                                         446   7.2  15.9

IIB  823.68    .32    .32  COAL                                         550   6.6  23.2

IIB  824.00    .74    .74  COAL                                        1219   6.4  25.8

IIB  824.74    .35    .35  COAL                                         535   8.9  18.9

IIB  825.09    .65    .65  COAL                                         923  10.1  13.4

IIB  825.74    .16    .16  COAL                                         309   8.8  18.0

IIB  825.90    .46    .46  COAL                                         782   6.4  30.4

IIB  826.36    .24    .23  COAL                                         402   9.0  16.8

826.60    .40    .39  SANDY SHALE                                  NOT ANALYSED

827.00    .60   3.00  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

*   SEAM ( IIB       ) OF THICKNESS   3.85m  FROM DEPTH 822.75m  TO  826.60m

833.57   2.43   2.99  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

======================================================================================================

ANALYSIS CONT..,

-----------------------------------------------------------------------------------------------------

BOREHOLE NO   :  CMWWB01                                        LATITUDE    : 2209645.917

REDUCED LEVEL(M):  282.2669                                        DEPARTURE   : 264858.8494

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

836.00    .57    .57  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

I    836.57    .36    .35  SHALY COAL                                   420   3.6  49.6

826.60    .40    .39  SANDY SHALE                                  NOT ANALYSED

827.00    .60   3.00  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

833.57   2.43   2.99  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

836.00    .57    .57  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( I         ) OF THICKNESS    .36m  FROM DEPTH 836.57m  TO  836.93m

======================================================================================================

BOREHOLE NO   :  CMWWB03                                        LATITUDE    : 2208132.208

REDUCED LEVEL(M): 307.95                                          DEPARTURE   : 264852.0102

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

L1   859.97    .33    .28  SHALY COAL                                   579   5.2  47.7

L1   860.30    .27    .22  SHALY COAL                                   451   5.2  47.7

860.57    .43    .38  SHALE                                        NOT ANALYSED

861.00    .57    .62  SHALE                                        NOT ANALYSED

*   SEAM ( L1        ) OF THICKNESS    .60m  FROM DEPTH 859.97m  TO  860.57m

890.12    .88   1.00  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

891.00   2.12   2.12  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

L    893.12    .25    .25  CARB SHALE LOW                               481   4.9  57.7

893.37    .15    .15  SHALE                                        NOT ANALYSED

893.52    .13    .13  ALTERNATING SHALE AND SST                    NOT ANALYSED

893.65    .35    .35  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

894.00    .37   2.92  SANDSTONE(MEDIUM GRAINED)                    NOT ANALYSED

898.94   1.06   1.90  SANDSTONE(FINE TO COARSE GRAINED)            NOT ANALYSED

900.00   1.94   1.90  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

L    901.94    .06    .05  COAL                                         NOT ANALYSED

902.00    .90    .86  SANDSTONE(MEDIUM GRAINED)                    NOT ANALYSED

======================================================================================================

ANALYSIS CONT..,

------------------------------------------------------------------------------------------------------

BOREHOLE NO   :  CMWWB03                                        LATITUDE    : 2208132.208

REDUCED LEVEL(M): 307.95                                          DEPARTURE   : 264852.0102

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

902.90    .10   2.90  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( L         ) OF THICKNESS   8.88m  FROM DEPTH 893.12m  TO  902.00m

907.62   1.08   2.90  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

908.70   1.92   1.85  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

IIA  910.62    .98    .90  COAL                                        1214   5.2  15.4

IIA  911.60    .12    .10  COAL                                               5.2  15.4

911.72   1.59   1.55  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

913.31   1.19   1.15  INTERCALATIONS                               NOT ANALYSED

914.50    .20    .20  SHALE                                        NOT ANALYSED

914.70    .26    .26  CARB SHALE LOW                               638   3.5  60.9

914.96    .27    .27  CARB SHALE HIGH                              579   3.4  68.8

*   SEAM ( IIA       ) OF THICKNESS   1.10m  FROM DEPTH 910.62m  TO  911.72m

915.23    .27    .27  CARB SHALE HIGH                              626   3.5  67.5

IIB  915.50    .27    .27  SHALY COAL                                  1679   5.0  42.9

======================================================================================================

ANALYSIS CONT..,

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB03                                        LATITUDE    : 2208132.208

REDUCED LEVEL(M): 307.95                                          DEPARTURE   : 264852.0102

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

IIB  915.77    .43    .42  SHALY COAL                                         5.0  42.9

IIB  916.20    .30    .30  SHALY COAL                                         5.0  42.9

IIB  916.50    .30    .30  COAL                                         490   5.9  20.9

IIB  916.80    .30    .30  COAL                                         447   5.7  26.6

IIB  917.10    .30    .30  SHALY COAL                                   547   4.7  41.7

IIB  917.40    .60    .60  COAL                                         957   5.2  25.3

IIB  918.00    .75    .75  COAL                                        1010   6.1  14.3

IIB  918.75    .75    .75  COAL                                        1233   5.2  30.3

919.50    .80    .80  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

920.30    .20   2.90  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( IIB       ) OF THICKNESS   4.00m  FROM DEPTH 915.50m  TO  919.50m

926.08    .02   2.89  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

926.10   2.90   2.90  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

929.00    .08    .08  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

I    929.08    .28    .28  SHALY COAL                                   467   4.0  40.3

I    929.36    .28    .28  SHALY COAL                                   557   4.2  41.8

======================================================================================================

ANALYSIS CONT..,

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB03                                        LATITUDE    : 2208132.208

REDUCED LEVEL(M):  307.95                                         DEPARTURE   : 264852.0102

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

919.50    .80    .80  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

920.30    .20   2.90  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

926.08    .02   2.89  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

926.10   2.90   2.90  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

929.00    .08    .08  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

*   SEAM ( I         ) OF THICKNESS    .56m  FROM DEPTH 929.08m  TO  929.64m

======================================================================================================

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB04                                        LATITUDE    : 2203174.803

REDUCED LEVEL(M): 261.63                                          DEPARTURE   : 268127.9779

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

L1   583.75    .25    .20  CARB SHALE LOW                               441   4.0  56.4

L1   584.00    .11    .10  CARB SHALE LOW                                     4.0  56.4

584.11   1.00   2.89  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( L1        ) OF THICKNESS    .36m  FROM DEPTH 583.75m  TO  584.11m

588.68   1.32   2.99  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

590.00    .12    .12  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

590.12    .90    .89  SHALE                                        NOT ANALYSED

591.02    .18    .15  CARB SHALE HIGH                              749   3.2  70.7

591.20    .24    .24  CARB SHALE HIGH                                    3.2  70.7

591.44    .24    .24  CARB SHALE HIGH                              525   3.1  69.0

IIA  591.68    .25    .25  SHALY COAL                                   454   5.7  38.2

IIA  591.93    .98    .98  COAL                                        1290   5.5  17.2

IIA  592.91    .99    .98  COAL                                        1135   6.1  23.3

IIA  593.90    .30    .30  COAL                                         507   5.8  37.1

IIA  594.20    .10    .06  COAL                                         507   5.8  37.1

594.30   1.00   2.30  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

======================================================================================================

ANALYSIS CONT..,

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB04                                        LATITUDE    :2203174.803

REDUCED LEVEL(M): 261.63                                          DEPARTURE   :268127.9779

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

*   SEAM ( IIA       ) OF THICKNESS   2.62m  FROM DEPTH 591.68m  TO  594.30m

613.27   1.33   2.99  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

614.60   1.67   1.63  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

L    616.27    .20    .17  SHALY COAL                                   323   5.6  35.5

594.30   1.00   2.30  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

613.27   1.33   2.99  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

614.60   1.67   1.63  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

*   SEAM ( L         ) OF THICKNESS    .20m  FROM DEPTH 616.27m  TO  616.47m

======================================================================================================

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB05                                        LATITUDE    : 2209720.133

REDUCED LEVEL(M): 265.25                                          DEPARTURE   :  266452.7581

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

IIA  725.80    .38    .30  SHALY COAL                                   361   4.5  41.4

IIA  726.18    .37    .30  COAL                                         339   6.5  26.7

IIA  726.55    .38    .30  COAL                                         225   6.4  20.3

IIA  726.93    .37    .30  COAL                                         243   4.4  28.6

727.30   1.00    .70  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

*   SEAM ( IIA       ) OF THICKNESS   1.50m  FROM DEPTH 725.80m  TO  727.30m

730.50   1.30    .90  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

731.80    .75    .35  SANDSTONE(MEDIUM GRAINED)                    NOT ANALYSED

732.55    .95    .60  INTERCALATIONS                               NOT ANALYSED

L2   733.50    .55    .48  CARB SHALE HIGH                              943   3.2  68.1

734.05   1.00   1.77  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( L2        ) OF THICKNESS    .55m  FROM DEPTH 733.50m  TO  734.05m

735.10    .90   1.77  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

736.00   2.00   1.95  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

======================================================================================================

ANALYSIS CONT..,

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB05                                        LATITUDE    :2209720.133

REDUCED LEVEL(M):   265.25                                        DEPARTURE   :266452.7581

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

738.00    .10    .10  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

IIB  738.10    .80    .80  COAL                                        1051   6.1  24.8

IIB  738.90    .81    .80  COAL                                        1113   5.4  30.3

IIB  739.71    .23    .23  CARB SHALE HIGH                              574   3.1  69.2

IIB  739.94    .38    .37  SHALY COAL                                   596   4.8  42.7

734.05   1.00   1.77  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

735.10    .90   1.77  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

736.00   2.00   1.95  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

738.00    .10    .10  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( IIB       ) OF THICKNESS   2.22m  FROM DEPTH 738.10m  TO  740.32m

======================================================================================================

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB06                                        LATITUDE    : 2206377.896

REDUCED LEVEL(M): 258.80                                          DEPARTURE   : 269717.3688

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

IIA  649.51    .42    .40  COAL                                         450   5.2  23.6

IIA  649.93    .27    .26  SHALY COAL                                   468   4.8  48.8

IIA  650.20    .20    .20  SHALY COAL                                   354   4.4  39.4

IIA  650.40    .80    .80  COAL                                        1036  10.5  17.8

IIA  651.20    .77    .77  COAL                                         868   7.8  20.7

651.97    .53    .53  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

652.50    .47   1.06  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

*   SEAM ( IIA       ) OF THICKNESS   2.46m  FROM DEPTH 649.51m  TO  651.97m

661.00    .50   2.99  SANDSTONE(FINE TO COARSE GRAINED)            NOT ANALYSED

661.50   2.50   2.50  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

IIB  664.00    .50    .50  COAL                                         655   4.5  15.3

IIB  664.50    .25    .25  COAL                                         323   6.2  21.1

IIB  664.75    .25    .25  SHALY COAL                                   428   4.8  36.8

665.00    .21    .21  SANDY SHALE                                  NOT ANALYSED

665.21    .79   2.28  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( IIB       ) OF THICKNESS   1.00m  FROM DEPTH 664.00m  TO  665.00m

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB06                                        LATITUDE    : 2206377.896

REDUCED LEVEL(M): 258.80                                          DEPARTURE   :  269717.3688

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

674.10   2.20   2.90  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

676.30    .80    .80  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

I    677.10    .30    .30  COAL                                         381   5.2  11.4

I    677.40    .22    .22  COAL                                         320   5.8  19.7

665.00    .21    .21  SANDY SHALE                                  NOT ANALYSED

665.21    .79   2.28  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

674.10   2.20   2.90  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

676.30    .80    .80  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( I         ) OF THICKNESS    .52m  FROM DEPTH 677.10m  TO  677.62m

======================================================================================================

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB07                                        LATITUDE    : 2212725.505

REDUCED LEVEL(M): 290.59                                          DEPARTURE   : 264820.0445

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

L1   664.55    .56    .54  CARB SHALE HIGH                              813   5.3  64.1

665.11    .89    .88  SHALE                                        NOT ANALYSED

666.00    .11    .70  SANDY SHALE                                  NOT ANALYSED

*   SEAM ( L1        ) OF THICKNESS    .56m  FROM DEPTH 664.55m  TO  665.11m

706.20   1.80   2.99  SANDSTONE(MEDIUM GRAINED)                    NOT ANALYSED

708.00   1.20   1.20  SANDSTONE(MEDIUM GRAINED)                    NOT ANALYSED

IIA  709.20    .60    .60  COAL                                        1097   6.6  27.0

IIA  709.80    .60    .60  COAL                                         685   5.7  24.2

IIA  710.40    .60    .60  COAL                                        1076   8.2  20.5

IIA  711.00    .60    .60  COAL                                        1121   7.7  13.3

IIA  711.60    .60    .60  COAL                                         885   4.7  31.1

712.20   1.00   1.40  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

*   SEAM ( IIA       ) OF THICKNESS   3.00m  FROM DEPTH 709.20m  TO  712.20m

718.70   1.30   3.00  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

======================================================================================================

ANALYSIS CONT..,

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB07                                        LATITUDE    :  2212725.505

REDUCED LEVEL(M):  290.59                                         DEPARTURE   : 264820.0445

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

720.00   1.70   1.70  SANDSTONE(MEDIUM GRAINED)                    NOT ANALYSED

IIB  721.70    .65    .65  COAL                                         813   8.1  18.2

IIB  722.35    .65    .65  COAL                                        1469   8.4  24.1

IIB  723.00    .25    .25  COAL                                         704   9.1  14.8

IIB  723.25    .05    .05  SHALE                                        NOT ANALYSED

IIB  723.30    .31    .31  COAL                                         556   7.8  22.2

IIB  723.61    .30    .30  COAL                                         676   6.3  25.4

IIB  723.91    .31    .31  COAL                                         823   6.5  19.7

IIB  724.22    .23    .23  COAL                                         629   5.2  34.4

724.45   1.00   1.55  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

*   SEAM ( IIB       ) OF THICKNESS   2.75m  FROM DEPTH 721.70m  TO  724.45m

729.19   2.81   3.00  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

732.00    .19    .19  SANDSTONE(MEDIUM GRAINED)                    NOT ANALYSED

I    732.19    .51    .51  COAL                                        1260   8.6  33.6

724.45   1.00   1.55  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

729.19   2.81   3.00  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

732.00    .19    .19  SANDSTONE(MEDIUM GRAINED)                    NOT ANALYSED

*   SEAM ( I         ) OF THICKNESS    .51m  FROM DEPTH 732.19m  TO  732.70m

======================================================================================================

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB08                                        LATITUDE    : 2211118.941

REDUCED LEVEL(M):  288.19                                         DEPARTURE   : 264890.2356

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

IIA  827.92    .35    .34  COAL                                         409   4.5  21.9

IIA  828.27    .88    .86  COAL                                         906   4.3  16.7

829.15   1.00    .97  SANDSTONE(FINE TO COARSE GRAINED)            NOT ANALYSED

*   SEAM ( IIA       ) OF THICKNESS   1.23m  FROM DEPTH 827.92m  TO  829.15m

830.15   1.35   1.35  INTERCALATIONS                               NOT ANALYSED

831.50    .51    .51  INTERCALATIONS                               NOT ANALYSED

L2   832.01    .46    .45  CARB SHALE HIGH                              979   2.9  64.5

832.47   1.93   1.93  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( L2        ) OF THICKNESS    .46m  FROM DEPTH 832.01m  TO  832.47m

834.40    .24    .24  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

IIB  834.64    .50    .48  COAL                                         519   5.4  20.7

IIB  835.14    .45    .43  COAL                                         808   5.2  27.0

IIB  835.59    .70    .70  COAL                                        1107   5.7  25.8

IIB  836.29    .43    .42  COAL                                         625   4.9  20.7

======================================================================================================

ANALYSIS CONT..,

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB08                                        LATITUDE    :2211118.941

REDUCED LEVEL(M):   288.19                                        DEPARTURE   :264890.2356

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

IIB  836.72    .23    .23  COAL                                         431   4.5  24.9

IIB  836.95    .18    .17  COAL                                         285   5.5  15.8

IIB  837.13    .70    .70  COAL                                        1032   5.7  27.7

837.83    .62    .60  SANDY SHALE                                  NOT ANALYSED

838.45    .38   1.32  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

*   SEAM ( IIB       ) OF THICKNESS   3.19m  FROM DEPTH 834.64m  TO  837.83m

845.53    .47   2.50  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

846.00   1.70   1.50  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

847.70    .83    .83  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

L3   848.53    .17    .17  SHALY COAL                                   343   4.0  47.2

837.83    .62    .60  SANDY SHALE                                  NOT ANALYSED

838.45    .38   1.32  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

845.53    .47   2.50  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

846.00   1.70   1.50  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

847.70    .83    .83  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( L3        ) OF THICKNESS    .17m  FROM DEPTH 848.53m  TO  848.70m

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB09                                        LATITUDE    : 2208274.308

REDUCED LEVEL(M):   257.07                                        DEPARTURE   : 267804.9796

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

IIA  670.02    .99    .96  COAL                                        1736   9.0  27.8

IIA  671.01    .99    .97  SHALY COAL                                  2308   8.7  31.7

672.00    .43    .42  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

672.43    .57   2.40  INTERCALATIONS                               NOT ANALYSED

*   SEAM ( IIA       ) OF THICKNESS   1.98m  FROM DEPTH 670.02m  TO  672.00m

679.84   1.16   2.99  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

681.00   1.84   1.84  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

IIB  682.84    .42    .40  COAL                                         936  10.2  21.8

IIB  683.26    .37    .37  COAL                                         999   8.9  29.8

IIB  683.63    .37    .37  COAL                                         887  10.5  17.7

IIB  684.00    .45    .45  COAL                                        1011  12.8  14.8

IIB  684.45    .35    .35  COAL                                         758   8.3  16.8

IIB  684.80    .46    .46  COAL                                        1306  11.0  21.5

685.26    .44    .44  SANDY SHALE                                  NOT ANALYSED

685.70    .56   1.30  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

*   SEAM ( IIB       ) OF THICKNESS   2.42m  FROM DEPTH 682.84m  TO  685.26m

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB09                                        LATITUDE    : 2208274.308

REDUCED LEVEL(M):   257.07                                        DEPARTURE   :267804.9796

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

695.50    .50   2.99  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

696.00   2.50   2.50  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

I    698.50    .25    .25  COAL                                         637  10.8  20.0

I    698.75    .25    .25  COAL                                         651   6.5  25.2

685.26    .44    .44  SANDY SHALE                                  NOT ANALYSED

685.70    .56   1.30  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

695.50    .50   2.99  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

696.00   2.50   2.50  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( I         ) OF THICKNESS    .50m  FROM DEPTH 698.50m  TO  699.00m

======================================================================================================

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB10                                        LATITUDE    : 2206241.004

REDUCED LEVEL(M): 305.70 M                                      DEPARTURE   : 266086.0171

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

IIA  754.40    .53    .50  COAL                                         643   4.5  32.0

IIA  754.93    .26    .26  SHALY COAL                                   678   3.7  46.3

IIA  755.19    .35    .35  COAL                                         785   6.7  21.1

IIA  755.54    .32    .32  SHALY COAL                                   836   4.8  35.7

IIA  755.86    .50    .50  COAL                                        1043   7.0  14.6

IIA  756.36    .60    .60  COAL                                        1520   5.8  31.5

*   SEAM ( IIA       ) OF THICKNESS   2.56m  FROM DEPTH 754.40m  TO  756.96m

756.96   1.68   1.68                                               NOT ANALYSED

IIB  758.64    .47    .46  COAL                                        1061   7.8  15.9

IIB  759.11    .60    .60  COAL                                        1507   6.5  23.0

IIB  759.71    .42    .42  COAL                                        1096   6.4  27.0

IIB  760.13    .94    .94  COAL                                        2342   6.7  25.5

IIB  761.07    .15    .14  COAL                                         378   5.3  20.0

IIB  761.22    .65    .63  COAL                                        1221   5.3  19.2

IIB  761.87    .43    .40  COAL                                         982   5.2  22.5

762.30    .20    .17  CARB SHALE HIGH                              588   2.8  66.1

======================================================================================================

ANALYSIS CONT..,

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB10                                        LATITUDE    :2206241.004

REDUCED LEVEL(M): 305.70M                                       DEPARTURE   : 266086.0171

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

762.50    .80   1.66  SANDY SHALE                                  NOT ANALYSED

*   SEAM ( IIB       ) OF THICKNESS   3.66m  FROM DEPTH 758.64m  TO  762.30m

769.56   1.44   2.96  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

771.00   1.56   1.56  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

I    772.56    .25    .22  COAL                                         545   5.6  21.3

I    772.81    .30    .28  COAL                                         672   4.4  26.2

I    773.11    .46    .46  COAL                                        1149   5.5  22.1

I    773.57    .30    .27  SHALY COAL                                   858   3.6  46.5

762.30    .20    .17  CARB SHALE HIGH                              588   2.8  66.1

762.50    .80   1.66  SANDY SHALE                                  NOT ANALYSED

769.56   1.44   2.96  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

771.00   1.56   1.56  SANDSTONE(MED. TO C. GRAINED)                NOT ANALYSED

*   SEAM ( I         ) OF THICKNESS   1.31m  FROM DEPTH 772.56m  TO  773.87m

======================================================================================================

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB12                                             LATITUDE    :2204740.652

REDUCED LEVEL :  252.5                                               DEPARTURE   :271317.9331

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

IIA  789.72   1.04   1.04  COAL                                        2251   10.0  14.8

IIA  790.76    .29    .29  CARB SHALE LOW                               913   5.9   49.7

IIA  791.05    .95    .95  COAL                                        2209   10.7  18.6

792.00    .18    .18  SANDSTONE(FINE GRAINED)                      NOT ANALYSED

792.18    .82   1.87  INTERCALATIONS                               NOT ANALYSED

*   SEAM ( IIA       ) OF THICKNESS   2.28m  FROM DEPTH 789.72m  TO  792.00m

801.07   2.93   2.99  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

804.00    .07    .07  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

IIB  804.07    .17    .17  COAL                                         387   7.6   13.6

IIB  804.24    .37    .37  COAL                                         940   7.3   22.7

IIB  804.61    .44    .43  COAL                                        1022   8.8   21.7

IIB  805.05    .15    .15  COAL                                         423   6.9   23.5

IIB  805.20    .50    .50  COAL                                        1250   8.2   18.9

IIB  805.70    .37    .37  COAL                                        1014   5.5   31.1

IIB  806.07    .21    .21  COAL                                         542   9.2   18.3

806.28    .11    .11  SHALE                                        479   3.5   72.1

======================================================================================================

ANALYSIS CONT..,

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.

BOREHOLE NO   :  CMWWB12                                        LATITUDE    :2204740.652

REDUCED LEVEL : 252.5                                           DEPARTURE   : 271317.9331

______________________________________________________________________________________________________

DEPTH  THICK   RECO            DESCRIPTIVE LITHOLOGY      P R O X I M A T E   A N A L Y S I S

SEAM  FROM   NESS    VERY              MODIFIER/                SAMPLE WEIGHT MOIS  ASH   VM    FC

*     (M)    (M)    (M)               INDICATOR                  NO.  (gms.)  (%)  (%)   (%)   (%)

______________________________________________________________________________________________________

806.39    .61    .61  INTERCALATIONS                               NOT ANALYSED

*   SEAM ( IIB       ) OF THICKNESS   2.21m  FROM DEPTH 804.07m  TO  806.28m

807    3.00    3.00  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

810    3.00    3.00  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

813    3.00    3.00  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

816    2.35    2.35  SANDSTONE(FINE TO MEDIUM GRAINED)            NOT ANALYSED

I    818.35    .32    .32  COAL                                         769   7.4 21.6

*   SEAM ( I         ) OF THICKNESS    .32m  FROM DEPTH 818.35m  TO  818.67m

======================================================================================================

ANALYSIS CONT..,

------------------------------------------------------------------------------------------------------

SOFTWARE DEVELOPED BY  GEOSYSTEM UNIT, EXPLORATION DIVISION, CMPDI(HQ), RANCHI.