GOVERNMENT OF ODISHA

DEPARTMENT OF STEEL AND MINES

DIRECTORATE OF MINES & GEOLOGY

GEOLOGICAL REPORT ON

PRELIMINARY EXPLORATION FOR GRAPHITE

AROUND NARINGPANGA (SOUTH), RAYAGADA DISTRICT, ODISHA (G-3)

[Under section-8 of Minerals (Evidence of Mineral Contents)

Amendment Rules, 2021 Schedule-I (d), PART IV- A]

TOPOSHEET NO-E44F10

NATIONAL MINERAL EXPLORATION TRUST

BY

Ramesh Behera, Geologist & EIC

Ananta Kumar Das, Geologist

M.K. Oram, DDG & TSO

Dr. P. C. Mishra, JDG

CONTENTS

| Sl. No. | DESCRIPTION |  | PAGE NO. |
| --- | --- | --- | --- |
| CHAPTER- 1 |  | SUMMARY | I-XII |
| 1.1 |  | खनिज ब्लॉक सारांश | I-II |
|  |  | Mineral Block Summary | III-IV |
| 1.2 |  | ### कार्यकारी सारांश | V-VII |
|  |  | Executive Summary | VIII-XI |
| CHAPTER-2 | INTRODUCTION |  | Page No. 1-6 |
| 2.1 | Details of the project |  | 1 |
| 2.2 | Investigating Agency |  | 2 |
| 2.3 | Objectives of Investigation |  | 2 |
| 2.4 | Basis for taking up investigation |  | 3 |
| 2.5 | ## Details and nature and quantum of work proposed vs achievement (Table) |  | ## 4 |
| 2.6 | Personnel involved |  | 5 |
| 2.7 | Mode of operation of different work components & associated agencies |  | 6 |
| CHAPTER-3 | PROPERTY DESCRIPTION |  | Page No. 7-13 |
| 3.1 | Details of the area |  | 7 |
| 3.2 | Physiography & Environment |  | 9 |
| 3.3 | Infrastructure |  | 13 |
| CHAPTER-4 | PREVIOUS WORK |  | Page No. 14-15 |
| 4.1 | Previous exploration/previous history of mining in adjoining area |  | 14 |
| 4.2 | Details of previous exploration carried out by other agencies/parties |  | 15 |
| CHAPTER-5 | GEOLOGY OF THE AREA |  | Page No. 16-26 |
| 5.1 | Aerial reconnaissance |  | 16 |
| 5.2 | Regional geological set up of the area with stratigraphy, structure and metamorphism |  | 17 |
| 5.3 | surface indication of mineralisation |  | 25 |
| CHAPTER-6 | GEOSCIENCE INVESTIGATION |  | Page No. 27-65 |
| 6.1 | Geological mapping |  | 27-54 |
| 6.1.1 | Detailed Geological mapping |  | 28 |
| 6.1.2 | Description of different litho-units |  | 28 |
| 6.1.3 | Detailed structural analysis |  | 37 |
| 6.1.4 | Metamorphism |  | 40 |
| 6.1.5 | Mineralogy of the ore zones and ore textures |  | 41 |
| 6.1.6 | Pitting and trenching |  | 43 |
| 6.1.7 | Sampling |  | 45 |
| 6.1.8 | Discussion |  | 50 |
| 6.1.8 | Ore Zones |  | 54 |
| 6.2 | Geophysical exploration |  | 54-62 |
| 6.2.1 | Geophysical Survey |  | 54 |
| 6.2.2 | Methodology |  | 55 |
| 6.3 | Geochemical exploration |  | 62-65 |
| 6.3.1 | REE & RM |  | 62 |
| CHAPTER-7 | INTEGRATION OF GEOLOGY, GEOPHYSICS AND GEOCHEMICAL EXPLORATION DATA & THE INTERPRETATION |  | Page No. 66-67 |
| CHAPTER-8 | MINERAL PROSPECT |  | Page No. 68-70 |
| 8.1 | Surface indication |  | 68 |
| 8.2 | Mode of occurrence |  | 68 |
| 8.3 | Strike length and width of anomalies |  | 69 |
| 8.4 | Alteration zones |  | 69 |
| 8.5 | Genesis of mineralisation. |  | 69 |
| CHAPTER-9 | EXPLORATION SYSTEMATIC DRILLING |  | Page No. 71-85 |
| 9.1 | Methodology of drilling with details of type of drilling |  | ## 71 |
| 9.2 | Borehole planning (spacing of boreholes, level of intersection), co-ordinates, RL of collar, borehole logging, cover recovery percentage) |  | 72 |
| 9.3 | Mineralogy of ore zone |  | 79 |
| 9.4 | Methodology of ore zone sampling and sample preparation |  | 79 |
| 9.5 | Chemical analysis and laboratory procedures |  | 80 |
| 9.6 | Details of intersected ore zones of the boreholes drilled and their correlation |  | 82 |
| 9.7 | Depth of Ground water condition |  | 84 |
| CHAPTER-10 | GEOTECHNICAL STUDIES ON BOREHOLE CORE SAMPLES OF MINERALISED ZONE, HANGING WALL AND FOOTWALL |  | Page No. 86 |
| 10.1 | Bulk density |  | 86 |
| CHAPTER-11 | RESOURCE ESTIMATION |  | Page No. 87-101 |
| 11.1 | Detailed description of ore zones |  | 87 |
| 11.2 | Cut-off grade and minimum stoping width consideration |  | 88 |
| 11.3 | Description and correlation of lodes |  | 89 |
| 11.4 | Preparation of LV section and Level plan |  | 89 |
| 11.5 | Specific gravity/bulk density calculation |  | 90 |
| 11.6 | Assumption for resource estimation |  | 91 |
| 11.7 | Resource estimation by cross section and longitudinal vertical section methods |  | 91 |
| 11.8 | Grade Estimation |  | 96 |
| 11.9 | Category of resources as per MEMC, 2015 along with UNFC classification |  | 100 |
| CHAPTER-12 | CORE PRESERVATION |  | Page No. 102 |
| 12.1 | Core preservation |  | 102 |
| CHAPTER-13 | CONCLUSION AND RECOMMENDATION |  | Page No. 103-105 |
| CHAPTER-14 | EXPENDITURE |  | Page No. 106 |
| CHAPTER-15 | REFERENCES |  | Page No. 107-108 |
| CHAPTER-16 | LOCALITY INDEX |  | Page No. 109 |
| Abbreviations Used |  |  | Page No. 110-111 |
| LISTS OF ANNEXURES |  |  |  |
| Annexure-I | Locality Index |  | 112 |
| Annexure-II | Litho-log & Chemical Assay result of core samples |  | 113 |
| Annexure-III | Weighted average of the samples |  | 125 |
| Annexure-IV | Section wise average grade calculation |  | 132 |
| Annexure-V | External check sample analysis |  | 133 |
| Annexure-VI | Internal Check sample analysis |  | 144 |
| Annexure-VII | Composite sample analysis result |  | 147 |
| Annexure-VIII | Cross section wise Borehole data |  | 150 |
| Annexure-IX | Resource calculation by cross section method |  | 151 |
| Annexure-X | Summary of Resource calculation |  | 152 |
| Annexure-XI | Section wise overburden calculation |  | 153 |
| Annexure-XII | Chemical analysis of core sample by Research Lab, Bhubaneswar |  | 154 |
| Annexure-XIII | ICPMS analysis result by MECL, Nagpur |  | 168 |
| Annexure-XIV | BH details of Naringpanga Graphite block |  | 172 |
| Annexure-XV | Chemical analysis of Grab & Pit samples |  | 173 |
| Annexure-XVI | Petrographic, Mineralographic study and XRD study report |  | 175 |
| Annexure-XVII | DGPS Survey and Topographical Survey |  | 217 |
| Annexure-XVIII | Bulk density analysis result of Naringpanga (S) Graphite Block, Rayagada district, Odisha |  | 220 |
| Annexure-XIX | Self-potential anomaly Data of Naringpanga (S) Graphite Block, Rayagada district, Odisha |  | 225 |
| Annexure-XX | TCC and EC meeting Minutes of Naringpanga (S) Graphite Block, Rayagada district, Odisha |  | 256 |
| LISTS OF PLATES |  |  |  |
| PLATE-I | Index Map of Naringpanga (S) Graphite Block (1:25,000) |  |  |
| PLATE-II | Geological Map along with BHs. location plan of Naringpanga (S) Graphite Block, Rayagada district, Odisha (1:2000) |  |  |
| PLATE-III | Interpreted geological map of Naringpanga (S) Graphite Block, Rayagada,Dist (1:2000) |  |  |
| PLATE-IV | Borehole Litholog of Naringpanga (S) Graphite Block, Rayagada district, Odisha (1:200) |  |  |
| PLATE-V | Interpreted Litho-structural Map of Naringpanga area, Rayagada district, Odisha |  |  |
| PLATE-VI | Topographic Map of Naringpanga (S) Graphite Block, Rayagada district, Odisha (1:2000) |  |  |
| PLATE-VII | Mineralised and Non- Mineralised zone of Naringpanga (S) Graphite Block, Rayagada District, Odisha |  |  |
| PLATE-VIII | Geological Cross-Section of Naringpanga (S) Graphite Block, Rayagada district, Odisha (A-A’)1:1000 |  |  |
| PLATE-IX | Geological Cross-Section of Naringpanga (S) Graphite Block, Rayagada district, Odisha (B-B’)1:1000 |  |  |
| PLATE-X | Geological Cross-Section of Naringpanga (S) Graphite Block, Rayagada district, Odisha (C-C’)1:1000 |  |  |
| PLATE-XI | Geological Cross-Section of Naringpanga (S) Graphite Block, Rayagada district, Odisha (D-D’)1:1000 |  |  |
| PLATE-XII | Geological Cross-Section of Naringpanga (S) Graphite Block, Rayagada district, Odisha (E-E’)1:1000 |  |  |
| PLATE-XIII | Self-Potential map superimposed over Geological Map, Naringpanga (S) Graphite Block, Rayagada district, Odisha |  |  |
| PLATE-XIV | Pit wall Sections of Naringpanga (S) Graphite Block, Rayagada district, Odisha (1:200) |  |  |
| PLATE-XV | Resource Estimation of Graphite at 2% cutoff by LV Method Section X-X’ (1:1000) |  |  |
| PLATE-XVI | Resource Estimation of Graphite at 2% cutoff by LV Method Section Y-Y’ (1:1000) |  |  |
| PLATE-XVII | Resource Estimation of Graphite at 2% cutoff by LV Method Section Z-Z’ (1:1000) |  |  |

PRELIMINARY INVESTIGATION (G-3) FOR GRAPHITE IN NARINGPANGA (SOUTH) BLOCK, DISTRICT: RAYAGADA,

STATE: ODISHA

CHAPTER-1

### सारांश

### 1.1 खनिज ब्लॉक सारांश

भाग A – खनिज ब्लॉक के बारे में सामान्य जानकारी
(ई-नीलामी प्लेटफॉर्म की वेबसाइट पर उपलब्ध कराया जाएगा)

|  | विशेषता | विवरण |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | स्थान |  |  |  |  |  |  |
|  | खनिज ब्लॉक | नारींगपंगा (दक्षिण) ग्रेफाइट ब्लॉक |  |  |  |  |  |
|  | अक्षांश (Latitude) |  |  |  |  |  |  |
|  | 1 | 19043’28.79616” N |  |  |  |  |  |
|  | 2 | 19043’33.03300” N |  |  |  |  |  |
|  | 3 | 19043’28.77816” N |  |  |  |  |  |
|  | 4 | 19043’31.61568” N |  |  |  |  |  |
|  | 5 | 19043’22.03716” N |  |  |  |  |  |
|  | 6 | 19043’15.12624” N |  |  |  |  |  |
|  | 7 | 19043’28.35480” N |  |  |  |  |  |
|  | 8 | 19043’26.98248” N |  |  |  |  |  |
|  | देशांतर (Longitude) |  |  |  |  |  |  |
|  | 1 | 83033’12.89664” E |  |  |  |  |  |
|  | 2 | 83033’18.40104” E |  |  |  |  |  |
|  | 3 | 83033’22.53888” E |  |  |  |  |  |
|  | 4 | 83033’25.87356” E |  |  |  |  |  |
|  | 5 | 83033’33.40656” E |  |  |  |  |  |
|  | 6 | 83033’25.23672” E |  |  |  |  |  |
|  | 7 | 83033’16.44804” E |  |  |  |  |  |
|  | 8 | 83033’14.31180” E |  |  |  |  |  |
|  | गाँव | नारींगपंगा |  |  |  |  |  |
|  | तहसील/तालुका | मुनिगुड़ा |  |  |  |  |  |
|  | जिला | रायगढ़ा |  |  |  |  |  |
|  | राज्य | ओडिशा |  |  |  |  |  |
| 2. | क्षेत्रफल (हेक्टेयर/वर्ग किमी) |  |  |  |  |  |  |
|  | कुल क्षेत्रफल | 0.143 वर्ग किमी |  |  |  |  |  |
|  | खनिजीकृत क्षेत्र | 0.078 वर्ग किमी |  |  |  |  |  |
|  | गैर-खनिजीकृत क्षेत्र | 0.065 वर्ग किमी |  |  |  |  |  |
| 3. | अन्वेषण |  |  |  |  |  |  |
|  | स्थिति (G2/G3/G4 आदि) | G3 (प्रारंभिक अन्वेषण) ¦ |  |  |  |  |  |
|  | अन्वेषण एजेंसी | खान एवं भूविज्ञान निदेशालय, ओडिशा |  |  |  |  |  |
|  | कुल बोरहोल संख्या व मीटरज | 6 बोरहोल, कुल 398 मीटर |  |  |  |  |  |
|  | बोरहोल दूरी (घनत्व) | 6 BHs, 0.143 वर्ग किमी में |  |  |  |  |  |
| 4. | खनिज की मात्रा (ग्रेड अनुसार) |  |  |  |  |  |  |
|  | खनिज | ग्रेफाइट (1) |  |  |  |  |  |
|  | कुल भूवैज्ञानिक संसाधन | F.C 2% से कम से 5% तक |  |  |  |  |  |
|  |  | कुल अनुमानित संसाधन |  | 0.413 मिलियन टन |  |  |  |
|  | खनिज क्षेत्रों की संख्या | 01 (एक) |  |  |  |  |  |
|  | प्रवृत्ति (स्ट्राइक और डिप) | स्ट्राइक: N40º -60º W-S40º -60º E, डिप: 55º - 70º दक्षिण-पश्चिमी दिशा में |  |  |  |  |  |
| 5. | ग्रेड ¦ |  | F.C (%) |  | SiO2 (%) | Al2O3 (%) | LOI (%) |
|  |  | F.C content 2% to < 5% | 3.01 |  | 66.23 | 12.77 | 8.70 |
|  | खनिज | 1. ग्रेफाइट (F.C -3.01%) |  |  |  |  |  |
| 6. | पहुंच |  |  |  |  |  |  |
|  | निकटतम रेलवे स्टेशन | मुनिगुड़ा (पूर्वी तट रेलवे पर), नारींगपंगा से 22 किमी |  |  |  |  |  |
|  | सड़क | जिला मुख्यालय रायगढ़ा से दूरी लगभग 64 किमी |  |  |  |  |  |
|  | हवाई अड्डा | जेयपोर (210 किमी), निकटतम बंदरगाह: विशाखापत्तनम (260 किमी) |  |  |  |  |  |
| 7. | जलग्रहण क्षेत्र |  |  |  |  |  |  |
|  | सतही जल निकासी पैटर्न | सतही जल निकासी पैटर्न ¦ बारहा नाला द्वारा नियंत्रित, NW से SE दिशा में बहती है ¦ |  |  |  |  |  |
|  | नदियाँ/नाले | बारहा नाला |  |  |  |  |  |
| 8. | जलवायु |  |  |  |  |  |  |
|  | औसत वार्षिक वर्षा | 1288 मिमी |  |  |  |  |  |
|  | तापमान (दिसंबर) | 140-180 C |  |  |  |  |  |
|  | तापमान (मई) | 270-460C |  |  |  |  |  |
| 9. | स्थलाकृति |  |  |  |  |  |  |
|  | टोपोशीट नंबर | E44F10 (65M/10). |  |  |  |  |  |
|  | स्थलाकृति विवरण | समतल क्षेत्र जिसमें पूर्वी और पश्चिमी किनारों पर छोटे-छोटे पहाड़ियाँ हैं |  |  |  |  |  |

SUMMARY

1.1 MINERAL BLOCK SUMMARY

[To be made available on the website of e - auction platform]

PART A – GENERAL INFORMATION ABOUT MINERAL BLOCK

|  | Features | Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Location |  |  |  |  |  |  |
|  | Mineral Block | Naringpanga (South) Graphite Block |  |  |  |  |  |
|  | Latitude |  |  |  |  |  |  |
|  | 1 | 19043’28.79616” N |  |  |  |  |  |
|  | 2 | 19043’33.03300” N |  |  |  |  |  |
|  | 3 | 19043’28.77816” N |  |  |  |  |  |
|  | 4 | 19043’31.61568” N |  |  |  |  |  |
|  | 5 | 19043’22.03716” N |  |  |  |  |  |
|  | 6 | 19043’15.12624” N |  |  |  |  |  |
|  | 7 | 19043’28.35480” N |  |  |  |  |  |
|  | 8 | 19043’26.98248” N |  |  |  |  |  |
|  | Longitude |  |  |  |  |  |  |
|  | 1 | 83033’12.89664” E |  |  |  |  |  |
|  | 2 | 83033’18.40104” E |  |  |  |  |  |
|  | 3 | 83033’22.53888” E |  |  |  |  |  |
|  | 4 | 83033’25.87356” E |  |  |  |  |  |
|  | 5 | 83033’33.40656” E |  |  |  |  |  |
|  | 6 | 83033’25.23672” E |  |  |  |  |  |
|  | 7 | 83033’16.44804” E |  |  |  |  |  |
|  | 8 | 83033’14.31180” E |  |  |  |  |  |
|  | Village | Naringpanga |  |  |  |  |  |
|  | Tahasil/Taluka | Muniguda |  |  |  |  |  |
|  | District | Rayagada |  |  |  |  |  |
|  | State | Odisha |  |  |  |  |  |
| 2. | Area (Hectares/square km) |  |  |  |  |  |  |
|  | Total area | 0.143 Sq. km |  |  |  |  |  |
|  | Mineralised area | 0.078 Sq. km |  |  |  |  |  |
|  | Non-mineralised area | 0.065 Sq. km |  |  |  |  |  |
| 3. | Exploration |  |  |  |  |  |  |
|  | Status (G2/G3/G4 etc.) | G 3 (Preliminary Exploration) |  |  |  |  |  |
|  | Exploration Agency | Directorate of Mines & Geology, Odisha |  |  |  |  |  |
|  | Total No. of BHs with meterage | 6 boreholes, 398 m |  |  |  |  |  |
|  | Borehole spacing (Density) | 6 BHs in 0.143 sq. km |  |  |  |  |  |
| 4. | Quantity of Minerals (Grade wise) |  |  |  |  |  |  |
|  | Mineral | Graphite (1) |  |  |  |  |  |
|  | Total Geological Resources (Resource) | F.C content 2% to < 5% |  | 0.413 mt |  |  |  |
|  |  | Grand Total Inferred Geological Resource |  | 0.413 mt |  |  |  |
|  | Number of mineral zones | 01(one) |  |  |  |  |  |
|  | Trend (Dip and strike) | Strike: N40º -60º W-S40º -60º E dipping at 550 to 700 south westerly |  |  |  |  |  |
| 5. | Grade |  | F.C (%) |  | SiO2 (%) | Al2O3 (%) | LOI (%) |
|  |  | F.C content 2% to < 5% | 3.01 |  | 66.23 | 12.77 | 8.70 |
|  | Mineral | 1. Graphite (F.C -3.01%) |  |  |  |  |  |
| 6. | Accessibility |  |  |  |  |  |  |
|  | Nearest Rail head | Muniguda which is on East Coast Railway 22 km from Naringpanga. |  |  |  |  |  |
|  | Road | The distance from the district headquarter Rayagada is about 64km. |  |  |  |  |  |
|  | Airport | Jeypore 210, Visakhapatnam is the nearest port is at 260 km. |  |  |  |  |  |
| 7. | Hydrography |  |  |  |  |  |  |
|  | Local Surface Drainage Pattern (Channels) | The drainage system of the area is controlled by Barha Nala flowing from the NW to SE with large meanders and it ephemeral streamlets. |  |  |  |  |  |
|  | River/Streams | Barha Nala |  |  |  |  |  |
| 8. | Climate |  |  |  |  |  |  |
|  | Mean Annual Rainfall | 1288 mm |  |  |  |  |  |
|  | Temperature (December) | 140-180 C |  |  |  |  |  |
|  | Temperature (May) | 270-460C |  |  |  |  |  |
| 9. | Topography |  |  |  |  |  |  |
|  | Topo Sheet Number | E44F10 (65M/10). |  |  |  |  |  |
|  | Morphology of the area | The area in general represents a plain country with minor undulations. The eastern and western parts of the terrain are occupied by small hillocks. |  |  |  |  |  |

### कार्यकारी सारांश

ग्रेफाइट का अन्वेषण G-3 स्तर पर खान एवं भूविज्ञान निदेशालय (Directorate of Mines & Geology) द्वारा NMET की स्वीकृति के अनुसार किया गया, जैसा कि पत्र क्रमांक NMET/302 दिनांक 30.01.2023 में उल्लेखित है। इसके अनुसार, निदेशालय का 2023-24 का फील्ड सीजन कार्यक्रम (आदेश संख्या 13888/DoMG, दिनांक 18/11/2023) के अंतर्गत रायगड़ा जिले के नारिंगपंगा क्षेत्र में ग्रेफाइट का अन्वेषण शामिल किया गया था, जो 18 जनवरी 2024 को प्रारंभ होकर 18 अगस्त 2024 को पूरा हुआ।

यह अन्वेषण कार्य लिथोलॉजिकल इकाइयों और ग्रेफाइट खनिजीकरण की घटनाओं को सीमांकित करने और उनका आकलन करने के लिए किया गया, क्योंकि यह पहले से नीलाम किए गए एक ब्लॉक से सटा हुआ क्षेत्र है। G-3 स्तर के अन्वेषण के लिए स्वीकृत कार्यों में 1:2,000 स्केल पर 14.3 हेक्टेयर क्षेत्र में भूवैज्ञानिक मानचित्रण, स्थलाकृतिक सर्वेक्षण, सेल्फ-पोटेंशियल (SP) सर्वेक्षण, 6 बोरहोल की ड्रिलिंग (कुल 398 मीटर ड्रिलिंग मीटरज) और सैंपलिंग शामिल थे।

यह ब्लॉक भारत के सर्वेक्षण विभाग के टोपोशीट नंबर E44F10 (65M/10) में दर्शित है। अध्ययन क्षेत्र नारिंगपंगा गांव से 750 मीटर उत्तर-पूर्व में, मुनिगुड़ा ब्लॉक एवं तहसील मुख्यालय, रायगड़ा जिले में स्थित है। जिला मुख्यालय रायगड़ा, अध्ययन क्षेत्र से 64 किमी दूर स्थित है और राज्य राजमार्ग क्रमांक 5 के माध्यम से सुगम रूप से जुड़ा है। निकटतम रेलवे स्टेशन मुनीगुड़ा है, जो ईस्ट कोस्ट रेलवे पर स्थित है और अन्वेषण स्थल से लगभग 22 किमी दूर है।

अन्वेषण क्षेत्र पूर्वी घाट मोबाइल बेल्ट का हिस्सा है जिसमें मिश्रित खोंडालाइट, क्वार्ट्जो-फेल्डस्पैथिक ग्नाइस, लेप्टिनाइट, एपिडोट युक्त खोंडालाइट और मायलोनाइट शामिल हैं, जो मिट्टी एवं एलुवियम द्वारा आच्छादित हैं। लिथोइकाइयों में फोलिएशन की प्रवृत्ति N40°–60°W से S40°–60°E तक पाई जाती है, जो दक्षिण-पश्चिम दिशा में 55° से 75° तक डिप करती हैं।

उपग्रह चित्रों के विश्लेषण से ज्ञात हुआ कि अन्वेषण क्षेत्र प्रमुख शीयर ज़ोन के समीप स्थित है, जो ग्रेफाइट खनिजीकरण को प्रभावित करते हैं। यह क्षेत्र वंशधारा शीयर ज़ोन की उप-शीयर, टुमुडिबंधा शीयर ज़ोन से लगभग 12 किमी पश्चिम में स्थित है। इसके अतिरिक्त, एक सहानुभूतिशील शीयर ज़ोन भी क्षेत्र को पार करता है। इन प्रमुख और गौण शीयर ज़ोन की परस्पर क्रिया ने ग्रेफाइट के पुनः संकेंद्रण की संभावनाओं को बढ़ाया है। ये संरचनात्मक विशेषताएँ—फोल्डिंग, फॉल्टिंग और शीयरिंग—खनिजीकरण के स्थानीयकरण, संचलन और संकेंद्रण को नियंत्रित करती हैं।

उच्च तापमान पर उत्पन्न खनिज संघटन, जैसे सिलिमेनाइट, गार्नेट, एपिडोट आदि, ग्रैनुलाइट फेशियल रूपांतरण को दर्शाते हैं, जबकि गार्नेट की दरारों में बायोटाइट का विकास और फेल्स्पार का सौसेरिटाइजेशन प्रतिगामी रूपांतरण का संकेत देते हैं।

नारिंगपंगा ब्लॉक में संभावित ग्रेफाइट खनिजीकरण क्षेत्रों को चिन्हित करने हेतु एकीकृत अन्वेषण पद्धति अपनाई गई, जिसमें विस्तृत भूवैज्ञानिक मानचित्रण, स्थलाकृतिक सर्वेक्षण और SP सर्वेक्षण शामिल था। कुल 0.143 वर्ग किमी क्षेत्र में SP सर्वेक्षण 25m × 10m ग्रिड पर किया गया, जिससे 13.5 लाइन किलोमीटर में 1,335 मापन बिंदु प्राप्त हुए। नकारात्मक विसंगतियाँ -0.04 mV से -154.6 mV तक और सकारात्मक विसंगतियाँ 0.06 mV से 80.2 mV तक पाई गईं।

डेटा की व्याख्या आइसो-एनोमली नक्शों के माध्यम से की गई, जिससे केंद्रीय और पूर्वी क्षेत्र में संभावित खनिजीकरण संकेत मिले। SP मानचित्र में विसंगतियाँ NW-SE दिशा में पाई गईं, जो क्षेत्रीय भूगर्भीय प्रवृत्तियों के अनुरूप है।

ब्लॉक में ग्रेफाइट अयस्क बैंड, पॉकेट, लेन्स और बिखरे हुए रूप में मिगमैटाइट खोंडालाइट और क्वार्ट्जो-फेल्डस्पैथिक ग्नाइस में पाया गया है। यह खनिजीकरण संरचनात्मक रूप से नियंत्रित होता है, जो अक्सर फोलिएशन के अनुरूप संरेखित होता है। कुछ क्षेत्रों में होस्ट चट्टानों में मायलोनिटाइजेशन की प्रक्रिया पाई गई है।

SP विसंगतियों और सतही अनावरण के आधार पर, 45° झुकाव पर 6 बोरहोल ड्रिल किए गए, जिनकी कुल गहराई 398.00 मीटर थी। इनके अतिरिक्त, दक्षिणी भाग में 4 परीक्षण गड्ढों (100.5 क्यूबिक मीटर) की खुदाई की गई, जिनमें से दो में ग्रेफाइट खनिजीकरण पाया गया।

सभी 6 बोरहोल में ग्रेफाइट खनिजीकरण अवरोधित हुआ, जिसमें कुल 84.30 मीटर खनिजीकृत परत पाई गई। औसतन, खनिजीकृत परत की मोटाई लगभग 14.05 मीटर है। सबसे गहरे बोरहोल NPBH-5 की गहराई 78.80 मीटर (झुकाव सहित) थी, जिससे 5.50 मीटर ग्रेफाइट खनिजीकरण अवरोधित हुआ। सबसे उथले बोरहोल NPBH-2 की गहराई 55.00 मीटर (झुकाव सहित) थी, जिससे 9.65 मीटर ग्रेफाइट खनिजीकृत परत मिली।

कुल 160 कोर सैंपल, 8 ग्रैब सैंपल, 14 पिट सैंपल और 11 मिश्रित सैंपल का विश्लेषण भूविज्ञान निदेशालय की लैब, भुवनेश्वर में वेट केमिकल पद्धति से किया गया। 20 आंतरिक और 10 बाहरी जांच सैंपल भी विश्लेषित किए गए। थोक घनत्व विश्लेषण 5 सैंपलों पर NABL अनुमोदित प्रयोगशाला में किया गया। खनिज विज्ञान और भू-रासायनिक विश्लेषण के तहत 14 XRD और 8 पेट्रोग्राफिक सैंपल DoMG में, जबकि 5 थिन सेक्शन और 20 ICP-MS सैंपल MECL, नागपुर में विश्लेषित किए गए।

160 कोर सैंपलों के रासायनिक विश्लेषण में ग्रेफाइट में फिक्स्ड कार्बन (FC) 2% से 10.03% तक पाया गया। क्रॉस सेक्शनल एरिया पद्धति से अनुमानित कुल भूवैज्ञानिक संसाधन 0.413 मिलियन टन (औसत FC 3.01%) आंका गया।

यह ब्लॉक संयुक्त राष्ट्र खनिज वर्गीकरण प्रणाली (UNFC) की 333 श्रेणी में आता है और खनिज (खनिज सामग्री के साक्ष्य) नियम, 2015 के अनुरूप है।

कुल 0.143 वर्ग किमी क्षेत्र की मैपिंग में 0.078 वर्ग किमी को खनिजीकृत और 0.065 वर्ग किमी को अखनिजीकृत पाया गया।

प्रारंभिक निष्कर्षों से ब्लॉक की संभाव्यता स्पष्ट होती है, जिसे संसाधन अनुमान को सुदृढ़ करने और खनिजीकरण को अधिक सटीक रूप से सीमांकित करने हेतु G-2 स्तर के विस्तृत अन्वेषण की आवश्यकता है।

Bottom of Form

EXECUTIVE SUMMARY

Exploration graphite was undertaken at G-3 stage by the Directorate of Mines & Geology as per approval of NMET, communicated vide L. No. NMET/302 Dt. 30.01.2023. Accordingly, the Field Season Programme of the Directorate of Mines & Geology, Odisha for 2023-24 (Order no. 13888/DoMG, Dt. 18/11/2023) included the exploration for Graphite around Naringpanga of Rayagada district which commenced on 18th January 2024 and field work was completed on 18th August 2024. The exploration was taken up to delineate the litho-assemblages and incidences of graphite mineralisation and assessment thereof, as it is the adjacent block of an already auctioned block.  The approved components of exploration at G-3 stage included Geological mapping on scale 1:2,000 over 14.3 Ha area, topographic survey, Self-Potential (SP) Survey and drilling of 6 bore holes with a drilling meterage of 398 m and sampling.

The block is featured in Survey of India Toposheet No: E44F10 (65M/10). The study area lies 750m northeast of Naringpanga village in Muniguda Block & Tehsil Headquarters of Rayagada district. The district headquarters, Rayagada, is located 64 km from the study area and is accessible via State Highway No. 5. The nearest Railway Station is Muniguda, situated on the East Coast Railway, approximately 22 km away from the explored block.

The area of investigation forms a part of Eastern Ghats Mobile Belt comprising migmatised khondalite, quartzo-feldspathic gneiss, leptynite, epidote-bearing khondalite, and mylonite which are masked by soil & alluvium.  The litho units exhibit foliation trends ranging from N400-600 W to S400-600 E, dipping from 550 to 750 in a south westerly direction. Satellite imagery analysis of the Naringpanga exploration area indicates its proximity to key shear zones that significantly influence the graphite mineralization. The area is located about 12 km west of the Tumudibandha Shear Zone, a sub shear of Vanshadhara shear zone (Fig. 5), which runs in a north-south direction. Additionally, a sympathetic shear zone, also north-south oriented, crosses through the area. The interaction of these shears with minor asymmetric shears from the Tel Shear (ENE-WSW) creates a favourable environment for the emplacement of granitic melts. This geological setting leads to the remobilization of graphite along structural weak planes, specifically the foliation in the rock mass, enhancing its potential for mineralization. The intersection of these lineaments offers a structural avenue for the concentration and mobilisation of graphite. These structural features folding, faulting, and shearing play a critical role in controlling the localization, mobilization, and concentration of graphite mineralization. High-grade metamorphism coupled with intense deformation has redistributed graphite along zones of structural weakness, underscoring the structural control of mineralization in the region. High-temperature mineral assemblages of the involved litho-units with index minerals like sillimanite, garnet, epidotes etc. indicates a high temperature-pressure granulitic facies metamorphism while retrograde features like the occurrence of biotite within the fractures of garnet and saussuritization of feldspar attests retrogression.

To delineate the potential graphite mineralization zones in the Naringpanga block, an integrated exploration approach was undertaken, including detailed geological mapping, topographic survey, and Self-Potential (SP) survey over an area of 0.143 sq. km followed by drilling to ascertain the subsurface geometry. The objective of SP survey was to identify anomalous zones that could indicate the buried mineralised zones of graphite. The SP survey was conducted on a 25m X 10m grid pattern over a total area of 29.22 ha in the initial stage to identify a potential target zone, covering 13.5-line km, which resulted in 1,335 SP measurements. The SP data, with negative anomalies ranging from -0.04mV to -154.6mV and positive anomalies between 0.06mV and 80.2mV, were used to interpret possible graphite occurrences. The data interpretation was carried out using isoanomalous contour maps, revealing a significant range of anomalies, especially in the central and eastern sections of the survey area. The base point for the survey was located to the southwest of the project area, near a known graphite mineralization zone, which influenced the identification of potential graphite-bearing zones. These findings suggest that the area exhibits promising indications of graphite and possible sulphide ore bodies, with high negative SP anomalies correlating with potential mineralization.

The SP contour map further highlights a mineralization trend that follows a NW-SE direction, which aligns with the regional geological trend and the foliation trend of host rock. This alignment, along with the observed SP data, indicates that the central part of the survey area holds the highest potential for hosting significant graphite mineralization. This targeted exploration approach, utilizing the Self-Potential (SP) survey method, demonstrates its effectiveness in identifying potential graphite deposits.

Graphite ore bodies in the Naringpanga block occur as bands, pockets, lenses and disseminations within migmatized khondalite and quartzo-feldspathic gneiss. The mineralization is structurally controlled, often aligning with the foliation or layering of the host rocks. Khondalite, a high-grade metamorphic rock, and quartzo-feldspathic gneiss, metamorphosed granite, provide favourable loci for graphite enrichment synchronised with the bounding structural weak planes. In some areas, the host rocks have undergone mylonitization, where intense brecciation, shearing, and fracturing might have facilitated pathways for enrichment. The graphite in the region typically exhibits flaky nature, characteristic of metamorphic terrains. Its appearance is typically black to dark grey, often with a metallic sheen on fresh surfaces.

Based on surface exposures and self-potential (SP) anomalies, six inclined boreholes were drilled at a 45° inclination as per the approved borehole plan by TCC-NMET. These boreholes were located in the anomalous zones interpreted through SP Survey, covering a cumulative drilling depth of 398.00 meters. Besides, trial excavation to the tune of 100.5 cubic meters was done in 4 trial pits in the southern part of the prospect where there was no approval for additional boreholes by TCC. Out of the 4 pits, graphite has been encountered in two pits.

Graphite mineralization was intercepted in all six boreholes, with a cumulative thickness of 84.30 meters of ore-bearing formations intersected in the drill cores. The average thickness of the mineralized zones is approximately 14.05 meters. The deepest borehole, NPBH-5, reached an inclined depth of 78.80 meters, corresponding to a vertical depth of 55.72 meters, and revealed a graphite mineralized zone measuring 5.50 meters. The shallowest borehole, NPBH-2, achieved an inclined depth of 55.00 meters, corresponding to a vertical depth of 38.89 meters, and intercepted a 9.65-meter graphite mineralized zone.

A total of 160 core samples, 8 grab samples, 14 pit samples, and 11 composite samples were analysed at the Directorate of Mines and Geology (DoMG) Chemical Laboratory, Bhubaneswar by wet chemical analysis in dry state. Additionally, 20 internal check samples were analysed at DoMG, while 10 external check samples were processed at the New Green Pvt. Ltd. NABL-accredited Laboratory, Bhubaneswar. Bulk density determination was carried out for 5 samples in a NABL-accredited laboratory. Advanced mineralogical and geochemical studies included XRD analysis of 14 samples at the DoMG XRD Laboratory and petrographic examination of 8 samples at DoMG, Bhubaneswar. At the MECL Laboratory, Nagpur, 5 thin sections and 5 petrographic samples were also analysed, while 20 samples have been subjected to geochemical analysis by ICP-MS in MECL laboratories.

The chemical analysis of 160 core samples from the exploration area reveals that the fixed carbon (FC) content in graphite ranges from 2% to 10.03%. Using the cross-sectional area method, the total geological resource of graphite has been estimated at 0.413 million tonnes with an average fixed carbon content of 3.01%.

These findings classify the block under the 333 category of the United Nations Framework Classification (UNFC) and as per the Minerals (Evidence of Mineral Contents) Rules, 2015 (MEMC 2015).

During the exploration of the Naringpanga area, a total of 0.143 sq. km was surveyed, of which 0.078 sq. km was identified as mineralized and 0.065 sq. km as non-mineralized.

The preliminary results highlight the potential of the block, meriting further detailed exploration at the G-2 level to refine resource estimates and delineate the mineralization more precisely.

CHAPTER – 2

INTRODUCTION

#### 2.1 Details of the Project

Exploration graphite was undertaken at G-3 stage by the Directorate of Mines & Geology as per approval of NMET. The Project proposal was discussed in 48th TCC of NMET held on 22.12. 2022 and subsequently approved in the 27th Executive Committee (EC) meeting of NMET held on 10.01.2023. The approval was communicated vide L. No. NMET/302 Dt. 30.01.2023. Accordingly, the Field Season Programme of the Directorate of Mines & Geology, Odisha for fsp 2023-24 (Order no. 13888/DoMG, Dt. 18/11/2023) included the exploration for Graphite around Naringpanga (South) of Rayagada district which commenced on 18th January 2024 and field work was completed on 18th August 2024.

The exploration was taken up to delineate the litho-assemblages and incidences of graphite mineralisation and assessment thereof, as it is close to a block already auctioned.  The approved components of exploration at G-3 stage included Geological mapping on scale 1:2,000 over 14.3 Ha area, topographic survey, Self-Potential (SP) Survey and drilling of 6 bore holes with a drilling meterage of 398 m and sampling.

The project area lies in the southern part of the Naringpanga Graphite Block. It is featured in Survey of India Toposheet No: E44F10 (65M/10). This block is situated approximately 750 m northeast of Naringpanga village in Muniguda Block, Rayagada. To delineate the potential graphite mineralization zones in the Naringpanga block, an integrated exploration approach was undertaken, including detailed geological mapping, topographic survey, and Self-Potential (SP) survey over an area of 0.143 sq. km followed by drilling to ascertain the subsurface geometry. Graphite ore bodies in the Naringpanga block occur as bands, pockets, lenses and disseminations within migmatized khondalite and quartzo-feldspathic gneiss. During the exploration of the Naringpanga area, a total of 0.143 sq. km was surveyed, of which 0.078 sq. km was identified as mineralized and 0.065 sq. km as non-mineralized.

The preliminary results highlight the potential of the block, meriting further detailed exploration at higher level.

2.2 Investigating Agency

| a | Name of the explorer/ Mining or prospecting rights holder: | Directorate of Mines & Geology, Odisha, |
| --- | --- | --- |
| b | Address: | Directorate of Mines & Geology, Odisha, Bhubaneswar, Odisha, 751001 |
| c | Telephone No: | 0674 2392374 |
| d | E-Mail id: | dir.mines@orissaminerals.gov.in |

#### 2.3 Objectives & Scope of the Investigation

The objective of the present investigation was to assess the resource and grade of graphite occurrences of the block through topographic survey, detailed geological mapping of the block in 1:2000 scales, drilling, SP Survey and systematic sampling at 1m interval of the drilled core samples as per G3 stage of exploration of MEMC Rules-2015.

- Geological Mapping:

- To conduct detailed geological mapping on a 1:2000 scale to delineate the litho-assemblages and incidences of graphite mineralisation.

- To record and analyze structural features such as foliations, lineations, shear fractures & shear criterias including structural controls influencing graphite mineralisation.

- To define the potential mineralised area from auction point of view for detailed assessment.

- Geophysical Survey:

- In order to decipher the potential graphite bearing anomalous zone in hostile para-metamorphites of pelitic & psammo- pelitic gneisses schists employing self-potential geophysical survey to detect and delineate concealed graphite ore bodies within the block.

- Trial Excavation:

- In order to expose the shallow level in-situ graphite bearing gneisses, excavations to the tune of 100 cu. m in trial pits/trenches in the areas of moderate to shallow soil cover.

- Exploratory Drilling:

- In order to delineate the sub-surface geometrical disposition of graphitic gneisses & schists by penetrating six inclined boreholes to establish the incidence of graphite ore bodies in geo-physically interpreted anomalous zones.

- Laboratory studies:

- For characterisation of the lithological entities and ascertain the grade of the ore, various samples have been collected from bed rocks, drill cores, pit sections to ascertain the grade of graphite, petrography, mineralogy, trace elements, bulk density etc. and analysed.

- Resource & Grade Estimation:

- To Estimate preliminary graphite resources (category 333) following the UNFC (United Nations Framework Classification) standards and the guidelines of the Minerals (Evidence of Mineral Contents) Rules-2015.

#### 2.4 Basis for Taking up Investigation

Occurrence of Graphite within pelitic schists and gneisses of EGMB is an established fact. The Naringpanga block is located in the Western Khondalite Zone of the Eastern Ghats. Historical records by DG (O), GSI, and others suggested the high grade granulitic terrains as potential for significant graphite occurrences.  Five lithotectonic domains having distinct disposition patterns of mineralisation have been made The area under exploration forms part of the Eastern Ghats Supergroup and comprises khondalite suite of rocks, charnockite, quartzo-feldspathic gneiss, and pegmatoids with associated silica veinlets. The Khondalite group consists of garnet - sillimanite schist, Garnet-sillimanite-graphite gneisses, sillimanite - graphite gneiss, micaceous/ garnetiferous/ sillimanite bearing quartzites and garnet – sillimanite gneiss.  S. N. Parida & D. N. Pani (1983-84) of DG (O) conducted geological mapping near Naringpanga block and reported seven occurrences of graphite in the region. Later, a detailed survey by the Directorate of Geology during F.S. 2016-17 over an area of 0.36 Sq. km in the adjacent northern part of the present block at G-2 level carved out an auctionable block which was successfully auctioned by Govt. The present study area shares geological continuity with the auctioned blockA part of the block was granted for PL in 2003 to M/s T. P. Minerals Pvt. Ltd. But,  could not be executed as ML.

This block lies adjacent to a previously auctioned graphite block with established resource potentials. Thus, detailed exploration was warranted to assess the mineral potential.

2.5 Details and nature and quantum of work proposed vs achievement

| SL. No. | Item of work | Approved Target | Achievement |
| --- | --- | --- | --- |
| 1 | Detailed Geological mapping (1:2000) and topographic survey (contouring at 2m intervals) | 0.143 sq.km | 0.143 sq. km |
| 2 | Drilling | 400m | 398 m (in 6 angle B.Hs) |
| 3 | Pitting | 100 cum | 100.5 Cu.m (4 PIT) |
| 4 | Self-Potential Survey | 12-line km | 13.5-line km |
| 5 | Sampling |  | Chemical Analysis |
|  |  | Primary sample- 160 | (B.H samples) - 160 Grab sample -8 |
|  |  | XRD-studies-10 | XRD-studies-14 |
|  |  | Composite sample: 10 | Composite sample-11 Nos. |
|  |  | Bulk density-5 Nos. | Bulk density-5 Nos. |
|  |  | Preparation of Thin section study-5 | Preparation of Thin section study-5 |
|  |  | Petrographic study- 05. | Petrographic study- 13 Nos. |
|  |  | ICPMS (34 elements) – 20 Nos. | ICPMS (34 elements) -20 Nos. |
|  |  | Internal check: | 10% Internal Check sample- 20 Nos. |
|  |  | External Check: | 10% External Check sample- 10 Nos. |

#### 2.6 Personnel involved

(To be provided separately for all the qualified persons signing off the report)

| A | Name: | - Dr. P C Mishra, Joint Director Geology - R. Behera, Geologist & Exploration In-Charge - A. K. Das, Geologist |
| --- | --- | --- |
| B | Address: | Directorate of Mines & Geology, Odisha, Bhubaneswar |
| C | Contact Mobile No: | 0674 2392374 |
| D | E-Mail id: | dir.mines@orissaminerals.gov.in |
| E | Qualification | Sl. 1: M.Sc. in Geology, Ph.D. Sl. 2 : M.Sc. in Geology Sl. 3: M.Sc. in Applied Geology |
| F | Experience | Sl. No. 1: 34 years of experience Sl. No. 2: 11 years of experience Sl. No. 3: 1 year of experience |
| G | Affiliation to any organization/ company, if yes, specify the name of the organisation or company: | Directorate of Mines & Geology, Government of Odisha. |

ii	Details of qualification and experience of persons associated with various aspects of exploration assessment of resources and reserves.

| a | Name: | Dr. P.C. Mishra, Joint Director Geology | Ramesh Behera, Geologist and EIC | Ananta Kumar Das, Geologist |
| --- | --- | --- | --- | --- |
| b | Address: | Directorate of Mines & Geology, Bhubaneswar, Odisha, 751001 |  |  |
| c | Contact Mobile No: | 9861465695 | 7978513153 | 7008174291 |
| d | E-Mail id: | pcmishra66@rediffmail.com | Rameshbehera8@gmail.com | ananta071994@gmail.com |
| e | Qualification: | M.Sc., PhD (Geology) | M.Sc., (Geology) | M.Sc. (Applied Geology) |
| f | Experience: | 34 years | 11 years | 1 years |
| g | Affiliation to any organization, if yes, specify the name of the organization or company: | Directorate of Mines & Geology, Bhubaneswar, Odisha |  |  |

2.7   Mode of operation of different work components & associated agency:

The exploration components approved by TCC-NMET & EC was executed by efficient geo-scientists of the Directorate of Mines & Geology, Bhubaneswar, Odisha. The field officers from the Office of the Joint director of Geology, Koraput were deployed during the entire project period to execute the field components and Report preparation works. The drilling and Topographic survey components and geo-physical survey were outsourced. Besides, external check samples, ICPMS studies and bulk density determination studies were carried out in the NABL Laboratories.

CHAPTER-3

### Property Description

#### 3.1 Details of the area

- Village: Naringpanga

- Tahasil: Muniguda

- District: Rayagada

- State: Odisha

- Toposheet No: E44F10 (65M/10)

- DGPS and Global positioning System (GPS) coordinates of all corner points of the area. Handheld DGPS and GPS (GARMIN) were employed for fixation of Geo-Coordinates of corner points. The location map of the working area is shown in Fig.1.

| CP | EASTING | NORTHING | LONGITUDE | LATITUDE |
| --- | --- | --- | --- | --- |
| 1 | 767648.325 | 2183027.174 | 83033’12.89664” E | 19043’28.79616” N |
| 2 | 767806.703 | 2183159.921 | 83033’18.40104” E | 19043’33.03300” N |
| 3 | 767929.205 | 2183030.846 | 83033’22.53888” E | 19043’28.77816” N |
| 4 | 768025.026 | 2183119.601 | 83033’25.87356” E | 19043’31.61568” N |
| 5 | 768248.905 | 2182828.258 | 83033’33.40656” E | 19043’22.03716” N |
| 6 | 768014.12 | 2182612.074 | 83033’25.23672” E | 19043’15.12624” N |
| 7 | 767751.977 | 2183015.153 | 83033’16.44804” E | 19043’28.35480” N |
| 8 | 767690.39 | 2182971.999 | 83033’14.31180” E | 19043’26.98248” N |

- Borehole points in latitude and longitude (Degree Minutes Second) format WGS-84 Datum

| BH No | Northing | Easting | RL(m) | Longitude | Latitude |
| --- | --- | --- | --- | --- | --- |
| NPBH-01 | 767689.2503 | 2183040.786 | 331.40 | 83033’14.310” E | 19043’29.218” N |
| NPBH-02 | 767778.4465 | 2183058.278 | 333.31 | 83033’17.377” E | 19043’29.744” N |
| NPBH-03 | 767854.1705 | 2182910.55 | 331.02 | 83033’19.901” E | 19043’24.906” N |
| NPBH-04 | 767887.8965 | 2182985.00 | 335.52 | 83033’21.096” E | 19043’27.307” N |
| NPBH-05 | 767910.4273 | 2182829.996 | 329.86 | 83033’21.791” E | 19043’22.260” N |
| NPBH-06 | 767975.6609 | 2182906.237 | 334.14 | 83033’24.069” E | 19043’24.704” N |

- Cadastral details- Cadastral details of the area with land use, area under forest with type of forest. In case the cadastral details are not available an indicative data of breakup of government, private and forest land.

Government Land/ Private land/ Forest land:

(To be inserted after receipt of authenticated Land schedule)

Total Area: 0.143 ha

Mineral(s) under investigation: Graphite

Fig. 1: Location Map of Naringpanga Graphite Block, Rayagada district, Odisha

  - Physiography and Environment

- Relief of the area

Regionally, eastern and northern parts of the area consist of hill ranges and thick forests. The area exhibits a dendritic pattern of drainage. The general slope as indicated from the drainage is from northwest to southeast. Upper reaches of Vamsadhara River and its tributaries constitute the drainage system of the area. The hill ranges and the drainage in general have been controlled by the lithology and the structure of the formation. An analysis of the structural data has brought out that the long chain of hills represents synformal structures with intervening broad antiformal valleys indicative of attainment of geomorphological maturity of the terrain.

Locally, the area presents a predominantly plain landscape with minor undulations. The eastern and western region of the block is dotted with small hillocks. Elevation ranges from a minimum of 328 meters above mean sea level (MSL) in the central-west portion to a maximum of 352 meters msl in the northeastern corner of the block.

The Barha Nala, a tributary of River Vamshadhara, is a perennial Nala that dominates the drainage system of the area, is litho-structurally controlled and flows from northwest to southeast with pronounced meanders. This drainage is supported by smaller ephemeral streams and exhibits a sub-dendritic pattern influenced by the underlying lithological and structural framework. The water table in the region fluctuates between 2.00 and 11.70 meters.

- Accessibility:

Naringpanga is well connected by a metal road and is located approximately 20 km northeast of Muniguda, the Block & Tehsil Headquarters of Rayagada district. The study area lies 750 m northeast of Naringpanga village. The district headquarters, Rayagada, is located 64 km from the study area and is accessible via State Highway No. 5. The nearest railway station is Muniguda, situated on the East Coast Railway, approximately 22 km away. The nearest port is Vishakhapatnam, at a distance of 260 km. Healthcare infrastructure in the region includes a government CHC hospital in Muniguda block, a district hospital in Rayagada, and a sub-divisional hospital in Gunupur, along with several private healthcare facilities across the district. Financial services are well-represented, with 60 commercial bank branches and 24 rural bank branches. As far as the mining industry in the area is concerned. Industrial development is highlighted by the presence of Vedanta's alumina refinery at Lanjigarh. Hindalco's Utkal Alumina International Limited (UAIL) is already operating a 2.12 MTPA alumina refinery in the district. The study area falls under Muniguda Tehsil in Rayagada district.

- Host population (local tribes), Human settlements within and nearby the area

Naringpanga is a small village located in Muniguda Block of Rayagada district, Odisha. According to the 2011 Census, The village has a population of 233 people, among these, 107 are males and 126 are females, resulting in a sex ratio of 1,178 females per 1,000 males, which is significantly higher than the Odisha state average of 979. The village consists of 55 households, with most residents belonging to Scheduled Tribes (71.24%) and a smaller proportion to Scheduled Castes (19.74%).

- Socio Demographic profile of the area and nearby

Socio-demographic profile of Naringpanga village has been assessed as follows:

| Particulars | Total | Male | Female |
| --- | --- | --- | --- |
| Total No. of Houses | 55 | - | - |
| Children (0 to 6) | 27 | 12 | 15 |
| Population | 233 | 107 | 126 |
| ST | 166 | 75 | 91 |
| SC | 46 | 22 | 24 |
| Literacy | 24.76% | 36.84% | 14.41 % |
| Literate | 51 | 35 | 16 |
| Illiterate | 182 | 72 | 110 |
| Total worker | 130 | 63 | 67 |
| Main worker | 3 | - | - |
| Marginal worker | 127 | 62 | 65 |

- Historical sites and archaeological monuments, places of worship, public utilities etc. within or nearby

In the Muniguda block, some of the notable temples include the Jagannath Temple, which is significant within the region, and the Kumudabali Shiva Temple. Additionally, the Siva Temple at Chatikona is mentioned as an important place of worship. While no historical sites are specifically listed nearby Naringpanga itself, these temples contribute to the cultural and spiritual landscape of the area. The local schools also serve as key public infrastructure in the region.

- Forests, sanctuaries, national park and wildlife sanctuaries; grazing land and gochar land within or nearby the area with distance from periphery of the area explored.

No Wildlife Sanctuary is lying within a radial distance of 5 Km.

- Flora and Fauna within and nearby

Rayagada district spans a total geographical area of 7,073 square kilometers, of which 2,812.33 square kilometers amounting to 39.76% is covered by forest. The Naringpanga study area reflects this broader ecological profile, being sparsely vegetated with a mix of bushy forests and species such as Sal (Shorea robusta), Neem (Azadirachta indica), Mango, and Kendu (Diospyros melanoxylon). Occasional teak plantations are also observed. These forests play a significant ecological role and support both biodiversity and the livelihoods of local communities.

While the district is known to host wildlife like bears, jackals, and wild boars, the specific study area near Naringpanga lacks significant faunal presence. It is characterized by sub-tropical climatic conditions, which facilitate a rich floral composition but do not support large populations of wild animals. Additionally, there are no national parks, wildlife sanctuaries, or other protected areas in close proximity to the study site.

- Water bodies:

The area is situated in the eastern flanks of Bangei nala which is ephemeral. However, the area of investigation exhibits a plain land with small amplitude soil ridges manifested due to morpho-structures. The general slope of the terrain is from North to South. Seasonal rain cuts drain the area to Bangei nala, tributary of the Vansadhara River. Few deep rain cuts are also noticed in the western part of the area.

- Climatic conditions:

The district experiences a subtropical climate, with heavy monsoon rainfall concentrated between June and September, averaging around 1,288 mm annually. Summers are typically hot, with temperatures reaching up to 46°C, while winters are cool, with lows between 7°C and 8°C. This climatic variability supports the diverse vegetation and soil conditions found in the region, aligning with the physiographic and hydrological features of the Eastern Ghats.

- Any other physiographic, social and environmental factor having potential to affect the viability of the project and assessment of resource:

Local resistance for viability of the project and assessment of resources cannot be ruled out.

  - Infrastructure

| Local infrastructure with roads, railways, port facilities, electricity, water etc. With distance from the area | Roads | The block is accessible from the district Headquarters Rayagada covering 61 km SH-64. |
| --- | --- | --- |
|  | Railways | The nearest railway station / siding is at Muniguda Railway link of East-Coast Railway at a distance of 22 km from the block. |
|  | Port Facilities | Visakhapatnam is the nearest port on the eastern coast of Andhra Pradesh located at 260 km from the block. |
|  | Electricity | Available in the village Naringpanga |
|  | Water | Ground water can be made available from the bore well put in the Block for drinking & domestic purposes. |
|  | Airport | Jeypore, 210 km and Visakhapatnam 260 km from Muniguda. |
| Details of nearby industries in the area which may use the mineral commodity likely to be mined | Old graphite beneficiation plants are located in Rayagada district. |  |

CHAPTER – 4

PREVIOUS WORK

  - Previous exploration/previous history of mining in adjoining area:

- Mohanty, B.C. & Vajani, P.C. (1982-83) of Directorate of Geology, Odisha, first reported occurrences of graphite around villages such as Sollagudi, Naringpanga, Kallupadar, Mudra, Bongna, Birli, and Sabinala in Rayagada district. Preliminary investigations revealed graphite occurrences with fixed carbon (F.C.) content ranging from 7% to 49%, highlighting variability in ore quality. Graphite mineralisation is hosted by migmatised khondalite and termed as stratiform in type and mostly foliation controlled.

- S. N. Parida & D. N. Pani (1983-84) of DG (O) conducted geological mapping near Naringpanga block and reported seven occurrences of graphite in the region. Among these, the graphite deposit near Bandhamandi was noteworthy, although of low grade, with fixed carbon (F.C.) content ranging from 4.56% to 21.38% and is under active mining.

- Pasayat, S.; Jena, B.K.; Adhikari, K.N. and Satpathy, U.N. of GSI mapped the area in 1985-86 and reported 4 Graphite occurs in form of four parallel, narrow, conformable bands rich in flaky and amorphous graphite within argillaceous members (UE 10547) of khondalite group of rocks. These bands are traceable discontinuously for over a strike length of about 500 m and has considerable depth persistence along dip direction.

- Later, Nanda & Rauta of DG (O) carried out an investigation at G-4 level around Panchubai village Rayagada district in 2015-16 and estimated a reconnaissance resource of 0.032 million tonnes of graphite for 10 sporadic graphite pockets in the area. Among which, the Naringpanga occurrence was interpreted to be a potential area with graphite resource of 0.029 mt (G-4 level).

- With the above backdrop, Officers of the then Directorate of Geology carried out detailed exploration around Naringpanga, Rayagada District, (Sahoo, & Rauta, (2016-17)), the block is subsequently auctioned by State Govt. successfully.

4.2	 Details of previous exploration carried out by other agencies/parties

A part of the block was granted for PL in 2003 to M/s T. P. Minerals Pvt. Ltd. But could not be executed as ML. The PL holder also had not taken up any detailed exploration activities over the PL area.  In 2021, with the recommendation of the Technical Committee, the block was taken over by the Government.

Delineation of graphite through detailed geological mapping in 1:2,000 scale was undertaken by the Directorate of Geology during F.S. 2016-17 over an area of 0.36 Sq. Km through a topographic survey. Out of which, an area of 0.0549 Sq. Km, the adjacent northern part of the present block was explored at G-2 level through drilling of 404.45 meters through 19 vertical boreholes, spaced at 50m x 50m grid intervals and meter-wise sampling. The maximum explored depth in the block was 26.15 meters. A total of 204 meter-wise core samples were collected from the boreholes, along with five grab BRS samples.

Out of the 19 boreholes, 13 BHs encountered graphite disseminations, while the remaining boreholes were barren.

The resource and grade of the block was estimated by cross sectional area method and borehole influence method. The total indicated geological resource of the Naringpanga graphite block is estimated to be 0.142 million tonnes with UNFC 332 which constitute 0.132 million tonnes of graphite with F.C. content ranging from 2 to < 5% and 0.01 million tonnes of graphite with more than 5% F.C. The average grade of the block with F.C. content 2 to < 5% is LOI- 9.617%, SiO2-52.482 %, Al2O3- 17.355 % and F.C– 2.615% and for graphite with F.C. content ≥ 5% is LOI- 7.965%, SiO2- 52.107%, Al2O3-18.495% and F.C.- 5.551% (Sahoo et. al. 2016-17). The block has been successfully auctioned at 7.65% to M/s Maa Kudargarhi Steel Pvt. Ltd.  in the year 2022 by the State Government.

The present block shares the geological continuity with the auctioned block

CHAPTER -5

GEOLOGY OF THE AREA

  - Aerial Reconnaissance

The study area around naringpanga was interpreted with a buffer zone of about 30 km was interpreted from LISS II, band II &III satellite data over an area of 0.143 sq km to interpret the regional structure and local structural control of lithological geometry to decipher the possible control of mineralization. Satellite imagery interpretations of the Naringpanga area (fig.2) indicates its proximity to the Tumudibandha sub shear of Vanshadhara-Nagavalli shear zones that controls the geo-spatial distribution of the high grade granulites and might have facilitated the graphite mineralization in the sympathetic weak planes of host lithologies. The area is located about 12 km west of the Tumudibandha Shear, a sub shear of Vanshadhara Shear Zone (Fig. 2), which runs in a north-south direction. Additionally, a sympathetic shear zone, north-south oriented, passes through the area. The manifestations of these shears with minor asymmetric shears of the westerly lying Tel Shear (ENE-WSW) might have offered a favorable environment for the emplacement of granitic melts. The interpreted aerial maps have been subjected to ground truthing during field work.

Fig 2: Satellite image of the study area illustrating major shear fractures and folding patterns.

  - Regional Geological set up of the area with stratigraphy, structure & Metamorphism

5.2.1 Regional Geology

The area of exploration forms a part of Eastern Ghat Mobile Belt which skirts the eastern fringe of Dharwar and Bastar Cratons and the southern fringe of Singhbhum Craton. This belt has been sliced off along the eastern continental margin of India. In western and northern part, this belt has been thrusted over the respective Cratons. The Eastern Ghats Mobile Belt is also cut across by Gondwana grabens of the Mahanadi and the Godavari. It consists of a typical litho-assemblages of charnockites, supracrustals of khondalite group (dominated by garnet-sillimanite gneiss with sub-ordinate quartzite, marble and calc-silicate), migmatised gneisses (leptynite and orthogneiss) and granitoids. All have been metamorphosed in upper amphibolite to granulite facies. Younger intrusions of anorthosites, alkaline rocks and granites are also conspicuous in this belt (Ramakrishnan et.al. 1998). Retrograde metamorphism has been reported in this belt (Sen et. al. 1995). In general, the Eastern Ghat Mobile Belt is tightly folded into isoclinal and recline folds, the axial plane of which plunges south-easterly due to strong forces of compression directed from south-east (Chetty et. al. 1998). Nanda (1995) divided the Eastern Ghat Mobile Belt into four longitudinal litho zones depending on the predominance of litho types.  These are western Charnockite Zone (WCZ), Western Khondalite Zone (WKZ), Central Migmatite Zone (CMZ) and Eastern Khondalite Zone (EKZ). The present area of exploration lies in the Western Khondalite Zone.

Mahalik (1998) has divided Odisha into three geological sectors. Rocks of the region have been affected by two typical tectonic trends; the NE-SW trend, commonly known as the Eastern Ghats trend (oldest) and NW-SE trend, known as Nagavalli- Vansadhara trend. The NW-SE trend has been off-set by two major faults, the Eastern Ghats Boundary Fault and Tel River Shear. The NW-SE trend between Koraput and Bhawanipatna gets deflected to north-south orientation and continues up to Gandhamardan hill where it again deflected to NE-SW trend forming an arcuate belt. Faults and shear zones of south Odisha sector have genetically related either to the Eastern Ghats trend (NE-SW) or Nagavalli-Vansadhara trend (NW-SE).Khondalite and charnockite, part of the high-grade granulitic facies, show a predominant NE-SW trend, although local irregularities exist (Rao, A. T. 1977; Das, A. Sriram 1979). The folds in this area are mostly overturned, with the main axial plunge (F1) directed to the NE, influenced by superimposed folding along the NW-SE axis (F2), which is associated with the emplacement of charnockite. Secondary folding, associated with shearing, led to the rotation of fold axes and the formation of isoclinal folds. The third deformation episode is marked by upright open folds with horizontal axes, a result of migmatization (Bhattacharya, 1994). The fourth phase is characterized by shearing and fracturing, with fault directions trending WNW and NNE (Sarkar et al., 1981).

Fig. 3: Regional Geological map of Eastern Ghats Mobile Belt (Ramakrishnan et al. (1998).

Several tectonic models have been proposed by researchers to classify the Eastern Ghats Mobile Belt (EGMB) into various lithozones, provinces, and domains. These classifications are based on dominant lithology, tectonic fabric, and isotopic characteristics. Misra et al. (2015) provided a comprehensive review, discussing the merits and demerits of different classification schemes, offering insights into the complexity and diversity of the region. One of the notable classifications, as depicted in Fig. 3, is the litho-zonation proposed by Ramakrishnan et al. (1998). This model expanded the earlier divisions suggested by Nanda and Pati (1989), offering a refined understanding of the geological characteristics of the belt.

Ramakrishnan et al. (1998) identified four distinct longitudinal lithologic zones within the EGMB, arranged from west to east:

- Western Charnockite Zone (WCZ): This zone predominantly consists of charnockitic rocks and represents the westernmost part of the EGMB.

- Western Khondalite Zone (WKZ): Situated to the east of the WCZ, this zone is characterized by khondalite rocks, which include garnetiferous sillimanite gneisses, quartzites, and leptynites.

- Central Charnockite-Migmatite Zone (CMZ): This zone is marked by a complex mix of charnockitic and migmatitic rocks, indicating extensive high-grade metamorphism and partial melting.

- Eastern Khondalite Zone (EKZ): This is the easternmost zone, predominantly consisting of khondalite rocks, similar to the WKZ, but with variations in mineral assemblages and structural characteristics.

The merit of this classification is that it gives a clear understanding of the lithological distribution and variation across the belt.

Fig. 4: Litho-zones of EGMB (Ramakrishna et. al. 1998)

Similar to the litho-zonation, Rickers et al. (2001) proposed a classification based on available isotopic data and divided the EGMB into four crustal domains as mentioned in figure 4 which can be compared to the lithological subdivisions proposed by Ramakrishnan et al. (1998) with additional inputs from the shear zones identified by Chetty and Murthy (1994) which serve as boundaries for these domains. Chetty (2001, 2010, 2014) and Chetty et al. (2003) based on the study of satellite images attempted to delineate different tectonic domains in the EGMB. Chetty (2001) divided the EGMB into at least nine distinct heterogeneously deformed terranes based on the presence of shear zones, different fold styles, and axial planes of early formed folds, distinct signatures of alkaline, anorthositic and granitic magmatism and tectonic interactions that have shaped the EGMB.

Fig.5: Domains of EGMB, Rickers, et.al. 2001 (left) & Provinces & Domains of EGMB in the EGMB, Dobmeier, & Raith (2003) (source; Nana et al. 2002)

However, accurately defining the boundaries and determining the exact nature of terrains can be challenging due to complexities in the geological record, insufficient detailed structural mapping in the granulite terrain and limited geochronological data on different domains. Figure 5 illustrates the classification by Dobmeier and Raith (2003), providing insights into broader geological processes. In 2024, Behera & Mohakul et al., based on the contiguity of mineralised zones and their disruption due to structural discontinuities in EGMB, prepared a lithotectonic domain map with interpretation of prospective graphite mineralization provinces.

Five lithotectonic domains having distinct disposition patterns of mineralisation have been made which are (i) the southernmost, Vizianagaram - Rajahmahendravaram Domain bounded on its east by the N-S trending Nagavalli Shear Zone (NSZ), (ii) the Nagavalli - Vamsadhara Domain lying between NSZ and Vamsadhara Shear Zone (VSZ), (iii) the central Daspalla Phulbani Domain bounded by VSZ in the south and the Mahanadi Shear Zone (MSZ) in the north, (iv) the northernmost Angul Redhakhol Domain located north of the MSZ and (v) the western Balangir Domain between the eastern margin of the Bastar Craton and the Tel Shear Zone (TSZ) (Fig.6).

The broad stratigraphic framework of Eastern Ghats Super Group postulated by Ramakrishnan et al (1998) is as follows.

| 800-900 Ma | Alkaline rocks, granitoids |
| --- | --- |
|  | -------Eastern Ghats Orogeny------- |
| 1000-1100 Ma | Development of Eastern Ghats ‘Front’ of upgraded cratonic rocks Incipient and massive charnockite and garnet-hornblende-biotite gneisses and garnetiferous gneiss with biotite and sillimanite |
| 1300-1400 Ma | Emplacements of alkaline rocks (feldspathoidal gneisses) and anorthosites |
| Khondalite Group (WKZ, CMZ & EKZ) | Garnet-sillimanite-graphite gneisses Cordierite-sapphirine-spinel rocks Calc-silicate rocks and rare marbles Quartzite rich in garnet (and some sillimanite) (The whole suite interleaved with charnockites) |
| 1600-1800 Ma | Evolution of Purana basins |
| 2600-2800Ma | Charnockites with enclaves of basic granulites, high grade (WCZ) schists including BIF, and layered basic complexes, representing original cratonic basement (?) in ‘migmatitic’ amphibolite facies |
| 3000 Ma | Vestigial events (?) |

Fig.6: Regional Litho-tectonic domain map of EGMB after Behera & Mohakul, 2024

5.2.2 Local Geology and Stratigraphy:

The area under exploration forms part of the Eastern Ghats Supergroup and comprises khondalite, charnockite, quartzo-feldspathic gneiss, and pegmatoids with associated silica veinlets. These rock units are overlain by recent soil and alluvium. Pegmatoids occur as intrusive bodies within the migmatized khondalite, typically emplaced along the foliation planes (Plate-II).

Detail geological mapping over the area revealed that area is mostly soil covered with scanty exposures of the litho units like khondalite and its variants, quartzo-feldspathic gneiss, charnockite and pegmatite which are only exposed along the Barha Nala course adjacent to the west of study area, The Khondalite group consists of garnet - sillimanite schist, sillimanite - graphite gneiss, micaceous/ garnetiferous/ sillimanite bearing quartzites and garnet – sillimanite gneiss. All these litho-units are migmatised and altered to variable extent. Except minor quartz and pegmatoid veins, the area is significantly devoid of any intrusive rocks. Members of Khondalite group namely quartz-garnet-sillimanite schist with or without graphite (Khondalite sensu-stricto), sillimanite-graphite schist and quartzites are interbanded with each other or exhibit variations along strike from argillaceous to arenaceous facies and vice versa. Similarly, khondalite grades along strike to migmatite from un-migmatised to thoroughly migmatised rock without any relict mineralogy of the parent rock. This type of gradation is essentially due to increasing abundance of quartzo-feldspathic material in the original rock. Arenaceous and argillaceous members of khondalite show strike ward gradation to quartzo-feldspathic gneiss as recorded in the borehole cuttings indicate that they are para-gneisses. On close examination, the interbedded arenaceous bands at some places within khondalite are seen to be leptynites in composition indicating them originally to represent arkoses in the sedimentary basin. Khondalite & quartzofelspathic gneiss form more or less banded assemblage with variable degree of alteration. Based on their mutual field relation relationships, the following stratigraphic succession is suggested in order of their increasing antiquity.

| Recent to sub recent | Soil/Alluvium |
| --- | --- |
| Intrusive | Pegmatoids & silica veinlets |
|  | Quartzo-feldspathic gneiss (± Graphite & Garnet) |
| Archaean | Charnockite (migmatised) |
|  | Khondalite (migmatised/kaolinised/±graphite) |

### Regional Structure

The disposition of the mineralised zones is unique in each domain, primarily controlled by the second and third generation folding often modified by high strain zones.  The study area lies within the Domain IV, of the litho-structural domain of EGMB (Behera & Mohakul, 2024) (Fig: 7). The Nagavalli-Vamsadhara domain is a narrow NNW-SSE trending belt bounded by Nagavalli Shear Zone (NSZ) in the west and VSZ in the east. It constitutes parts of WKZ and CMZ of Ramakrishnan et al. (1998). The shear zones (NSZ and VSZ) join the western boundary shear zone in the north and in the south, they merge together to end abruptly against the east coast (Chetty, 2001). The authors proposed fold-thrust tectonics associated with westward thrusting in this domain. This domain trends at an angle (NNW-SSE) to the regional foliation (NE - SW) of the rocks of the EGMB. This domain becomes narrow towards the south. The domain constitutes tight folds with N-S axial traces (F2). The trend of the foliation near VSZ and NSZ are parallel to both shear zones. The foliation near VSZ steep westerly dip and is parallel to VSZ whereas the foliation in the vicinity of NSZ has steep easterly (Fig: 7). The steeply easterly dips of NSZ and merger with other westerly dipping shear zones at depth indicate the region’s tectonic complexity. In this domain, the F2 folds are appressed with axial traces that are parallel to the NSZ and VSZ.

#### Fig.7: Litho-tectonic setup of domain IV sandwiched by NSZ & VSZ & structurally controlled Graphite bearing horizons.        (After Behera & Mohakul, 2024)

5.2.3 Metamorphism:

Regionally, the EGMB has undergone complex polyphase metamorphism, characterized by granulite facies metamorphic conditions. The metapelitic rocks, particularly khondalites, exhibit mineral assemblages such as garnet + sillimanite + quartz + K-feldspar + biotite ± spinel, are suggestive of high-temperature and moderate-pressure conditions. Metamorphic reactions in these rocks include the prograde transformation of muscovite and biotite into garnet and sillimanite, such as:

Chlorite + Muscovite + Quartz→Garnet + Biotite + Sillimanite + H2​O

During peak metamorphism, dehydration melting reactions occurred, for instance:

Garnet + Sillimanite + Quartz↔Cordierite + K-feldspar

Which produces granitic melts and migmatitic textures. In charnockitic rocks, orthopyroxene is a diagnostic mineral formed through the dehydration of biotite, via:

Biotite + Quartz→Orthopyroxene + K-feldspar + H2​O

These reactions suggest the removal of water and stabilization of anhydrous minerals under high temperature conditions. In some areas, spinel + quartz + garnet assemblages are observed, indicating ultra high temperature metamorphism (>950°C).

Metamorphism in the EGMB is interpreted as a result of Proterozoic continent–continent collision, followed by prolonged crustal residence and exhumation. The mineral assemblages encountered in the study area suggest different metamorphic episodes based on the presence of specific minerals.

  - Surface indication of mineralization:

During geological mapping by the officers of DoMG, Odisha in F.S.P. 2016-17, around Naringpanga, exposures of graphite bearing quartzo-felspathic gneiss and migmatised graphite-sillimanite bearing gneisses of khondalite group of rocks were delineated along nala bed  and walls of the Bengei Nala flowing in the western vicinity of the present target area (Fig.8). Here, disseminated flaky graphite is found to occur as lenses and pockets along the folial plane of the politic gneisses and quartzo-felspathic gneisses with varying concentrations which shows sub vertical dip towards east with NNW-SSE foliation. Both quartzo-felspathic gneiss and migmatised graphite-sillimanite bearing gneisses exhibit more or less banded nature and conformable. The dip of litho units are extrapolable to the present area which led to intercept the graphite lodes in the boreholes given at 450 across the dip.

| Fig. 8. Altered migmatised khondalite in the Bangei Nala bed & eastern wall section. |  |
| --- | --- |

CHAPTER-6

GEOSCIENCE INVESTIGATION

6.1 Geological Mapping:

The exploration over the Naringpanga South Graphite block was taken up to assess the economic potentiality for graphite through detailed Geological mapping, SP survey, Pitting, Core drilling and Sampling followed by analysis to assess the inferred resource as per approval of exploration components of NMET. The achievements made under different components during exploration are given below.

Table 6.1: Exploration Components

| SL. No. | Item of work | Approved Target | Achievement |
| --- | --- | --- | --- |
| 1 | Detailed Geological mapping (1:2000) and topographic survey (contouring at 2m intervals) | 0.143 sq.km | 0.143 sq. km |
| 2 | Drilling | 400m | 398 m (in 6 angle B.Hs) |
| 3 | Pitting | 100 cum | 100.5 Cu.m (4 PIT) |
| 4 | Self-Potential Survey | 12-line km | 13.5-line km |
| 5 | Sampling |  | Chemical Analysis |
|  |  | Primary sample- 160 | (B.H samples) - 160 Grab sample -8 |
|  |  | XRD-studies-10 | XRD-studies-14 |
|  |  | Composite sample: 10 | Composite sample-11 Nos. |
|  |  | Bulk density-5 Nos. | Bulk density-5 Nos. |
|  |  | Preparation of Thin section study-5 | Preparation of Thin section study-5 |
|  |  | Petrographic study- 05. | Petrographic study- 13 Nos. |
|  |  | ICPMS (34 elements) – 20 Nos. | ICPMS (34 elements) -20 Nos. |
|  |  | Internal check: | 10% Internal Check sample- 20 Nos. |
|  |  | External Check: | 10% External Check sample- 10 Nos. |

6.1.1 Detailed Geological Mapping:

In order to facilitate geological mapping in 1:2000 scale, the entire area was surveyed using Total Station in 1:2000 scale by Total Station.  A 550 m baseline was established in the N 40° W direction, aligning with the litho-unit trend, using a Topcon Total Station (Model IM-55). Offset lines were laid out in the west–east direction. Coordinates of the block's corner points and borehole (BH) locations were recorded using a DGPS Trimble R8 Internal GNSS Dual Frequency system. The reduced levels (RL) of topographic control points across the area were also recorded, and a topographic map was prepared with a 2 m contour interval (Plate-VI).  Borehole locations were located as per SP anomalous zones and approved by NMET for exploration and collar height & coordinates are recorded.

The block has been surveyed using 2 - 4 DGPS points for accurate geo-referencing and ground control points. The DGPS points, established using the RTK (Real-Time Kinematic) method, ensured precise geo-referencing of the data. The survey data was extrapolated to the UTM grid system with the WGS-84 datum. The boundaries of the block were mapped using drone imagery, and critical points, such as borehole collar locations, were verified and fixed using DGPS. The ground control points and BH locations were recorded by Total station.

Detailed geological mapping over an area of 0.143 sq. km was conducted at a 1:2000 scale using a Brunton compass, Garmin GPS, and measuring tape to delineate litho-structural units. Traverses were spaced at 20 m intervals, and litho-structural elements were systematically recorded with respect to grid points and depicted in the geological map (Plate-II). The geometric disposition of litho-units beyond the study area was also recorded, as a significant portion of the working area is soil-covered, to aid in interpreting the spatial correlation of lithologies intersected in core samples. Based on surface and subsurface data, it is interpreted that 0.078 sq. km is mineralized, while 0.065 sq. km is non-mineralized. The study also included surface topographic mapping with contouring at 2 m intervals. Reduced Levels (RLs) of key points were recorded using a Total Station for precision. A separate topographic contour plan is provided in Plate-VI.

6.1.2 Description of different litho-units:

A brief note on the different litho-units of the area is discussed below:

(i) Khondalite:

Khondalite is the oldest rock unit occurring in the area which are migmatised and altered to variable extent. It is exposed in the Nala cuttings & wall sections only and also intercepted in boreholes. It forms a banded assemblage with the quartzo-feldspathic gneiss & often silicified. It is well foliated, shows compositional bending. The mineral present are quartz, feldspar, sillimanite, garnet, biotite and a few grains of graphite. In thin section it shows prisms and needles of sillimanite, porphyroblasts of garnet and recrystallised mosaic of quartz and feldspar. The accessory minerals are biotite, graphite and opaques. The rock shows well developed schistosity defined by the oriented needles of sillimanite, stretched grains of quartz and rod like forms of graphite. Because of close proximity to Bangei Nala, the lithounits have undergone higher degree of alteration and often places kaolinised. It is reddish brown to buff in colour, medium grained, and inequigranular consisting essentially of quartz, plagioclase, sillimanite, garnet and opaque minerals which are scattered throughout the rock mass. Quartz and orthoclase are abundant; plagioclase, sillimanite are common and garnet is noticed as an accessory. Secondary growth is noticed in most of the quartz grains. Under microscope plagioclases are highly saussuritised. Iron solution leached out from the garnet imparts a reddish look to the rock mass. Flakes of graphite occur as dissemination within the khondalite.

| Fig. 9. (a) Altered migmatised khondalite in the Bangei Nala bed & eastern wall section. |  |  |
| --- | --- | --- |

Fig . 9. (b) Micro-Photograph of graphite bearing migmatised khondalite. (Bt: Biotite, Gr Graphite, Gt: Garnet, Sil: Sillimanite, Qtz: Quartz)

The trend of foliation in khondalite varies from N400-600W to S400-S600 E dipping 550 to 700 southwesterly. Intense weathering and alteration of the constituent minerals rendered the lithounits moderately fissile at places with kaolinisation of the plagioclage porphyroblasts. It is a medium to coarse grained rock predominantly composed of quartz and feldspar showing crude foliation characteristics. The quartz grains are often flattened along the plane of foliation. Sillimanites are found as slender crystals with transverse fracture. The quartz grains exhibit sutured boundaries and granulation indicating a strained condition. Secondary overgrowth in the shape of rims is noticed in some quartz grains. Orthoclases are perthitic in nature. Orthoclase feldspars are altered and impart a dusky appearance. The garnets are isotropic, and it is likely that iron has been leached from them, giving the rock a reddish colour.

XRD studies of khondalite core samples (fig.10) exhibits mineral peaks for quartz, orthoclase, microcline, almandine, sillimanite, graphite, biotite, kaolinite, montmorillonite, titanite and epidote of variable intensity.

Fig. 10: XRD peaks of graphite bearing migmatised khondalite.

Hard and fresh khondalite are rare. Graphite flakes of variable dimensions are dimensionally oriented along the foliations defining gneissosity & schistocity of graphitized Khondalite.

(ii) Charnockite:

Charnockites, although not encountered within the block, are exposed as isolated patches in the western vicinity within the nala section, nevertheless, the dark coloured weathered & altered earthy mass encountered in shallow pit-4 (fig.11) is believed to be the altered product of charnockite.  Megascopically, it is altered, earthy gray coloured, mesocratic and medium grained rock showing a relict gneissic trend and exfoliation weathering features. Crude gneissosity is developed due to dimensional orientation of quartz and feldspar along a definite direction. Granulated prisms of hypersthenes and flakes of biotite constitute the melanocratic bands whereas felsic bands are represented by quartz and feldspar. Mineralogically, it is composed of plagioclase, orthoclase, hypersthenes and quartz as major and biotite are found as accessory minerals. Equidimensional quartz and feldspar form a mosaic texture. Under the microscope the rock shows inequigranular, xenomorphic, grano-blastic texture. The accessory constituents of the rock are apatite, zircon and opaques. K-feldspar which is the major constituent of the rock occurs as large plates and is mostly orthoclase. Microcline is rare, and occurs as small grains with irregular outline. Very often plagioclase replaces orthoclase in the margins as well as in central parts resulting in, myrmeckite. Alteration from hypersthene to biotite and garnet is also seen along grain boundaries and cleavage planes.

Fig. 11: Altered charnockite rock in the pit walls (Pit No.4).

(iii) Quartzo-Feldspathic gneiss:

Quartzo-feldspathic gneiss is the second most predominant unit exposed in the Nala section and invariably forms a banded assemblage with the migmatised khondalite in the sub-surface. The rock is medium-grained and highly altered rock, (in the nala bed & walls beyond the working area) and closely associated and interbanded in the contact with migmatised khondalite exhibiting foliation and exfoliation weathering at places. It is a medium grained highly altered and migmatised rock showing a distinct gneissic trend and gradational contact with khondalite. It is essentially composed of quartz, orthoclase, plagioclase and graphite and garnets are found as accessory minerals. Opaque are scattered throughout the rock mass. Graphite flakes occur as disseminations along the foliation of quartzo-feldspathic gneiss. Petrographic analysis of the both quartz and feldspar show dimensional orientation along the folial plane. Garnets are not uniform & highly altered with oxidation of ferromagnesian components. Secondary over-growth in most of the quartz grains indicates a sedimentary parentage. Graphite disseminations are also noticed in conformity to the folial plane defining gneissosity (fig. 12).

| NPGS -2 PPL 5X | Gt Garnet Orthoclase Sphene Quartz Bt NPGS -2 XPL 5X |
| --- | --- |

Fig. 12: Photo micrographs of quartzo-feldspathic gneiss (Gt: Garnet, Bt: Biotite, Qtz: quartz, Sph: Sphene, Or: Orthoclase).

Flaky or platy graphite & platy mineral biotites occur along the planes of foliation and parallel to the alignment of quartz grains. Darker bands called melanosomes are formed by the combination of garnet, biotite, and graphite, while lighter bands known as leucosomes are formed by quartz, plagioclase, and orthoclase, a characteristic feature of gneissosity. Orthoclase minerals are sericitized, and plagioclase minerals are saussuritized. Garnets are highly fractured, and some mineral grains exhibit signs of granulation and even mylonitised at depth. In certain areas, biotite and garnet occur close together, suggesting that biotite may have formed from the alteration of garnet.

XRD studies of quartzo-feldspathic gneiss core samples (fig.13) exhibits mineral peaks for quartz, orthoclase, microcline, andesine, almandine, graphite, biotite, kaolinite, titanite, pyrite and epidote of variable intensity.

Fig. 13: XRD peaks of graphite bearing Quartzo-Feldspathic gneiss core samples.

| (a) | (b) | (c) |
| --- | --- | --- |

Fig. 14: a. Litho core of sheared migmatised quartzo-feldspathic gneiss with melanosomes & neosomes (BH No. 2) b. Migmatised quartzo-feldspathic gneiss in Bangei Nala section.

Figure 14, a & b depicts the sheared migmatized quartzofeldspathic gneiss (BH No. 2), featuring alternating dark melanosomes and light neosomes formed by partial melting and recrystallization. Shearing is evident in the deformed banding and aligned mineral grains, reflecting tectonic stresses.

Fig. 15 (a): Spider plot of Trace elements.

Figure 15 represents (a) a spider plot and (b) tectonic discrimination plots (Pearce et al., 1984) to interpret the tectonic setting of the quartzo-feldspathic gneiss and its variants. The spider plot illustrates trace element dispersion patterns of 20 samples, showing enrichment in thorium, barium, and rubidium, along with depletion in niobium. These geochemical characteristics are indicative of granitic rocks formed in specific tectonic environments.

The tectonic discrimination plots (Fig. 15. b) utilize trace element ratios such as Y/Nb, Ta+Yb/Rb, Yb/Ta, and Y+Nb/Rb to classify granitic rocks into ocean ridge granites (ORG), volcanic arc granites (VAG), within-plate granites (WPG), and collision granites (COLG). The Y+Nb vs. Rb plot reveals most samples clustering in the WPG field, with minor extensions toward the syn-COLG field, suggesting a dominant within-plate tectonic setting. The Y vs. Nb plot similarly places most points in the WPG field, with a few near the VAG and syn-COLG boundary, indicating mixed tectonic influences. The Ta+Yb vs. Rb and Yb vs. Ta plots consistently show a strong alignment within the WPG field, reinforcing the within-plate origin of these rocks. These geochemical patterns indicate a primarily within-plate granitic magma origin with minor volcanic arc contributions, reflecting a complex magmatic evolution. The spider and tectonic discrimination plots provide key insights into the tectonic setting and regional geological evolution of the quartzofeldspathic gneiss and its variants.

Fig. 15 (b): Discrimination plot of Y+Nb/Rb, Y/Nb, Ta+Yb/Rb, & Yb/Ta.

(iv)  Pegmatite & vein quartz:

Small veinlets of pegmatoids/ silica within the host migmatites of khondalites & quartzofelspathic gneiss are exposed on the Nala section both in northern and southern part of the area and also noticed in the borehole cuttings.  These are mainly composed of megacrysts of quartz and feldspar, where mica constitutes a very less percentage showing concordant relationship with the country rock. The permeations are mostly foliation guided.

(v) Soil and alluvium:

The entire part of the area is covered with soil and alluvium with average thickness of 4m. Residual soil is formed by weathering of the in-situ country rocks. The alluvium is encountered in the narrow tracts which support the vegetation and agriculture of the area. The soil is grayish in apperance and plastic. Thickness of soil cover varies from 2 to 6 m.

6.1.3 Detailed Structural Analysis

Structural details of the area:

Both diastrophic and non-diastrophic structures are found within the litho assemblages. The diastrophic structures include foliation, shears, folds and faults and non-diastrophic structures include joint and impersistent mineral lineation. As revealed from the surface and subsurface data, graphite mineralization is litho structurally controlled as it is confined to the foliation plane of quartzo-feldspathic gneiss and migmatised khondalite.  The area is affected by granitic activity as a result of which meta-sediments are sheared. Drags and kinks are well marked in the migmatised khondalite. Secondary structural features are noticed in the litho units that include kinks and joints.

Satellite image interpretations of the area (fig. 2) deciphers the the structural complexities and controls of the litho-structural framework for the area where Vanshadhara-Nagavalli shear zones that merge with the Tumudibandh sub shear might have acted as a conduit to facilitate granitic invasion regionally which facilitated the graphite mineralization in the sympathetic weak planes of host lithologies in the domain. The area is located about 12 km west of the Tumudibandha Shear Zone, (Fig. 2) that runs in a north-south direction. Additionally, a sympathetic north-south oriented shear, passes through the present study area. The manifestations of these shears with minor asymmetric shears of the westerly lying Tel Shear (ENE-WSW) might have offered a favorable environment for the emplacement of granitic melts. This geological setting led to the remobilization of graphite along structural weak planes, specifically the foliation in the rock mass, enhancing its potential for mineralization. The intersection of these lineaments offers a structural avenue for the concentration and mobilisation of graphite, and distribution of the mineral in this region.

- Faults/Shear Zones

The study area, although masked by a rim of alluvium & soil but imprints of deformational signatures like brecciation, mylonitization, silicification, fracturing, and kaolinization are noticed in the drilled core cores (Fig.16) are noteworthy shear criterias. The presence of slickensides and mylonites within the involved lithounits attests to mobility & ductility of assemblages during faulting and shearing activity.

Fig. 16: Sheared migmatised khondalite & mylonitised quartzo-feldspathic gneiss in NPBH-2.

These structural features - folding, faulting, and shearing might be the controlling factors for the localization, mobilization, and concentration of graphite mineralization. High-grade metamorphism coupled with intense deformation has redistributed graphite along zones of favourable structural weak planes. These processes resulted in the pulverization of litho-units and might have offered secondary avenues for the redistribution of minerals. Mylonitisation has been observed in boreholes NPBH-3 and NPBH-5, at depths from 61.00m to 69.00m in NPBH-3 and from 60.50m to 65.00m in NPBH-5. Shearing served as conduits for migration, facilitating the emplacement of graphite along structural weak planes, particularly within fold hinges and foliations. The structural complexity of the area is further accentuated by the intersection of major lineaments, the Tumudibandha shear zone and the Tel shear and their sympathetic shears. These intersections might serve as loci for granitic melts and the remobilization of graphite, excited by intense tectonic forces.

#### (ii) Folding

The litho-units in the study area exhibit tight folds, kinks, and well-developed foliations, with structural trends ranging from N40°W–S40°E to N60°W–S60°E, dipping 50°–70° southwest and part of the regional F1 fold system. The F2 fold patterns (fig:13) are more or less isoclinal and reclined folds attributed to compressional forces and that played a role in localizing graphite mineralization, especially in the hinge zones of the folds, highlighting the structural control over the mineralization process. The F3 deformational signatures includes drag folds, parasitic folds, kink folds, ‘Z’ & ‘M’ type folds and ptygmatic crenulations. Drag and parasitic folds reflect the localized strain within major fold systems, which likely influenced the mobility of mineralizing melts. Kink folds, characterized by sharp angles, and ptygmatic folds, with bulbous shapes, indicate the intense deformation and pressure conditions conducive to mineral concentration.

(iii) Foliation:

The foliation in the khondalite and quartzo-feldspathic gneiss units is well-developed in the exposed Lithounits encountered only in nala sections to the west of the block, with its general trend varying between N40ºW-S40ºE and N60ºW-S60ºE. The dip of the foliation ranges from 55º to 70º towards the southwest, reflecting its genetic correlation to the adjacent Tel shear in the west & VSZ to the east and Tumudibandha sub-shear that have shaped the region's geological structure. This foliation plane plays its role in facilitating the distribution of graphite mineralization along the structural planes.

(iv)  Lineation:

Two distinct types of lineation are observed in the study area:

- Slickensides: These are prominently noted in quartzo-feldspathic gneiss and khondalite rocks, particularly in borehole cuttings. Slickensides are indicative of fault movements and represent striated, polished surfaces formed due to shearing.

- Mineral Lineation: This is characterized by the alignment of minerals such as graphite, biotite, and garnet as streaks. The mineral lineation provides evidence of deformation and metamorphism, showcasing the directional movement and alignment of mineral grains under tectonic forces and it makes a low to moderate plunge with foliation.

(v) Joints:

Two sets of joints are prominent in the khondalite and quartzo-feldspathic gneiss of the area in conformity to the foliation. The first set trends N45°E-S45°W with a dip of 74° northwestward, while the second set aligns parallel to the foliation trend.

6.1.4. Metamorphism:

a. (Quartz + Orthoclase + Plagioclase + Sillimanite+ Epidote + Garnet + Graphite)

This mineral assemblage indicates high-grade metamorphism. Sillimanite and garnet are characteristic minerals that form under high temperatures and pressures. Quartz, orthoclase, plagioclase, and graphite are also present. The presence of graphite suggests high-grade metamorphism.

b. (Quartz + Orthoclase + Plagioclase + Hypersthene + Garnet + Biotite)

This mineral assemblage suggests a medium to high-grade metamorphic episode. Hypersthene and biotite are minerals commonly found in metamorphic rocks. The presence of quartz, orthoclase, and plagioclase indicates the involvement of felsic minerals in the metamorphic process.

c. (Quartz + Orthoclase + Plagioclase + Garnet + Sillimanite +Graphite + Sphene)

This mineral assemblage suggests a high-grade metamorphic episode. Sillimanite & graphite are minerals commonly found in metamorphic rocks formed under high temperatures and pressures under granulitic facies conditions.

The given mineral assemblages indicate different degrees of metamorphism, ranging from high-grade to medium to high-grade. Each assemblage represents different combinations of minerals formed under specific temperature and pressure conditions. Based on the mineral assemblages and their textural characteristics, along with the occurrence of retrograde features, it seems that the metamorphic episodes in the area are indicative of the granulite facies of regional metamorphism. The granulite facies are associated with high-temperature and high-pressure conditions, typically found in the deeper crust. The mineral assemblages include minerals like garnet, sillimanite, hypersthene, and orthoclase, are commonly associated with the granulite facies. Incidences like saussuritisation of plagioclases alteration of hypersthene to biotite and garnet & biotite and biotite in the fractures of garnet, suggest retrogression. Retrogression refers to the process in which a rock that has experienced high-grade metamorphism subsequently undergoes changes at lower temperatures and pressures, leading to the growth of minerals of lower grade. Additionally, the sausseritisation and kaolinisation of feldspar, which refers to the alteration of feldspar minerals into a fine-grained aggregate, is another retrograde feature. This process occurs due to the breakdown of feldspar minerals under lower temperature and pressure conditions.

Therefore, the presence of high-temperature minerals and retrograde features like the occurrence of biotite and sausseritisation of feldspar supports the interpretation that the metamorphic episodes in the area are characteristic of the granulite facies of regional metamorphism.

6.1.5 Mineralogy of the ore zones and ore textures:

The integrated analytical studies of various studies of the host rocks and graphite bearing lito-units reveals the mineralogical characteristics of the ore zones in the study area. As discussed earlier, graphite occurs as flaky varieties mostly in disseminated form either as independent entities or tufts within the foliation plane of migmatised khondalites and quartzo-felspathic gneisses occurring in the area.

The litho-units in the study area primarily comprise altered khondalite, graphite-bearing migmatized khondalite, and quartzo-feldspathic gneiss, which form a banded assemblage. These rocks are of meta-sedimentary origin and exhibit strong foliation. The lithology includes khondalite ± graphite schists and gneisses, both of which have undergone high grade granulite facies metamorphism. Within this lithological framework, specific units contain graphite mineralization, particularly in the migmatised pelites, which are metamorphosed rocks made up of sand-sized psammite and fine-grained pelite minerals that have experienced partial melting and recrystallization. The quartzo-feldspathic invasions, which involve the intrusion of quartz and feldspar-rich materials, granitoids, are also present within these units. The graphite mineralization is concentrated along the foliation planes of migmatised khondalite, often exhibiting variations in concentration, with swelling and pinching of the mineralised zones along the structural weaks. The mineralisation is concordant with the host rock and is concealed under thick alluvial soil cover. Graphite occurs as disseminations & micro veinlets or tufts, with individual flakes ranging from a few millimetres up to 5 millimetres in size. The graphite is primarily flaky and disseminated along the foliation planes of schists and gneisses, reflecting a structural control over its emplacement.

Mig. Khondalite with opaque (graphite flakes) as thin veinlets & fillings of micro fractures.

Other lithological units encountered in the area include migmatised khondalite, quartzo-feldspathic gneiss, leptynite, epidote - bearing khondalite, and mylonite. The mineralization appears to be governed by both the lithological characteristics and structural features of the rocks, indicating a litho-structural control on the distribution of graphite. In terms of structural features, joints observed in outcrops are sub-vertical, contributing to the permeability and mineralisation controls. Secondary mineralisation of pyrite, epidote, secondary quartz, zoisite, and garnet has been identified in the core samples, further adding to the complexity of the processes in the area.

However, mineralogical studies indicate that the graphite occurs as disseminated flakes and fibers along the foliation planes of migmatised khondalite and quartzo-feldspathic gneiss, with an erratic distribution. The deleterious elements present in these mineralised zones include quartz, plagioclase, orthoclase, and opaques. Quartz grains, exhibiting secondary overgrowths, appear as oval to lensoidal shapes with sutured outlines. Orthoclase is mostly perthitic and significantly altered to kaolin, while plagioclase occurs as platy crystals suffering from sausseritisation. The mineralization is believed to be a remobilized product of carbon during contact regional metamorphism, facilitated by the emplacement of quartzo-feldspathic gneiss.

6.1.6: Pitting and Trenching:

To ensure geometrical disposition graphite lode exposed in the nala bed, 4 pits are given in the hanging wall side. This approach provided crucial insights into the structural disposition and mineral potential of the graphite-bearing gneisses.

The pit locations are located in the southern part of the block, along the up-dip direction of the graphite gneisses exposed in the nala section.

- The selection of pit locations was guided by the fact that the southern part of the area was not covered by the geophysical survey, and devoid of any exposure.

- In order to intercept the lode at appropriate depths with respect to the exposed outcrops in the Nala section. The pit’s location is chosen in the hanging wall sides, which proved to be useful to decipher the litho-structural disposition & delineate the mineralised zones.

- To overcome these limitations and ensure geometrical disposition of graphite lodes exposed in the nala bed, the pits are given in the hanging wall side in the up-dip direction.

- Out of the four pits, graphite was encountered only in the pit no. 2 while others exposed the thick alluvial cover and highly altered country rocks of khondalite & charnockite.

Table 6.2: Physical and Chemical Assay Data from Pit Samples, Naringpanga (S) Graphite Block.

| Physical & Chemical Assay Log of Naringpanga Graphite Block, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pit No | 01 |  |  |  |  | Date of Commencement |  |  |  | 27.06.2024 |  |
| Location ( DD) | 19.72455556 |  |  | 83.55719444 |  | Date of Completion |  |  |  | 27.06.2024 |  |
| (DMS) | 19° 43' 28.4" N |  |  | 83° 33' 25.9" E |  | Total Graphite Thickness |  |  |  | 0 m |  |
| Depth | 4.00 |  |  |  |  |  |  |  |  |  |  |
| R.L | 339 |  |  |  |  |  |  |  |  |  |  |
| Sl. NO | Sample ID | From | To | ASH % | M% | LOI% | SiO2 | Al2O3% | F.C% | Length (in m) | Lithology |
| 1 |  | 0.00 | 0.40 |  |  |  |  |  |  | 0.40 | Soil |
| 2 | NGPT 1/1 | 0.40 | 1.40 | 90.38 | 3.13 | 9.62 | 60.34 | 11.36 | 0.62 | 1.00 | Weath. Kh. |
| 3 | NGPT 1/2 | 1.40 | 2.40 | 91.34 | 2.96 | 8.66 | 59.24 | 17.66 | 0.50 | 1.00 |  |
| 4 | NGPT 1/3 | 2.40 | 3.40 | 89.70 | 3.71 | 10.30 | 55.66 | 16.44 | 0.59 | 1.00 |  |
| 5 | NGPT 1/4 | 3.40 | 4.00 | 93.04 | 2.29 | 6.96 | 60.48 | 22.68 | 0.55 | 0.60 |  |
| 6 | NGPT 1/5 | Horizontal channel in the pit floor |  | 90.92 | 2.70 | 9.08 | 54.74 | 20.12 | 0.82 |  |  |
| 7 | NGPT 1C | Composite |  | 90.80 | 2.46 | 9.20 | 55.86 | 19.42 | 1.20 |  | Mig. Kh. |

| Physical & Chemical Assay Log of Naringpanga Graphite Block, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIT No | 02 |  |  |  |  | Date of Commencement |  |  | 28.06.2024 |  |  |  |
| Location | 19.72258333 |  |  | 83.55847222 |  | Date of Completion |  |  | 28.06.2024 |  |  |  |
| (DD) | 19° 43' 21.3" N |  |  | 83° 33' 30.5" E |  | Total Graphite Thickness |  |  | 0.3m |  |  |  |
| (DMS) | 4.00 m |  |  |  |  |  |  |  |  |  |  |  |
| Depth | 336 m |  |  |  |  |  |  |  |  |  |  |  |
| R.L | Sample ID | From | To | ASH % | M% | LOI% | SiO2 | Al2O3% |  | F.C% | Length (in m) | Lithology |
| Sl. NO |  | 0.00 | 0.70 |  |  |  |  |  |  |  | 0.70 | Soil |
| 1 | NGPT 2/1 | 0.70 | 1.70 | 90.04 | 3.63 | 9.96 | 59.78 | 15.44 |  | 0.85 | 1.00 | Weath. Kh. |
| 2 | NGPT 2/2 | 1.70 | 2.60 | 90.36 | 4.20 | 9.64 | 62.86 | 13.87 |  | 0.28 | 0.90 |  |
|  |  | 2.60 | 2.70 |  |  |  |  |  |  |  | 0.10 |  |
| 3 | NGPT 2/3 | 2.70 | 3.70 | 95.70 | 1.16 | 4.30 | 75.48 | 9.74 |  | 0.29 | 1.00 |  |
| 4 | NGPT 2/4 | 3.70 | 4.00 | 87.40 | 1.96 | 12.60 | 50.20 | 19.44 |  | 2.82 | 0.30 | Graphite + Khondalite |
| 5 | NGPT 2/5 | Horizontal channel in the pit floor |  | 91.36 | 0.69 | 8.64 | 53.92 | 20.76 |  | 1.89 |  | Weath. Kh. |
| 6 | NGPT 2C | Composite |  | 90.90 | 1.56 | 9.10 | 59.20 | 15.34 |  | 1.81 |  | Mig. Khond. |

| Physical & Chemical Assay Log of Naringpanga Graphite Block, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIT No- | 03 |  |  |  |  | Date of Commencement |  |  |  | 28.06.2024 |  |
| Location( DD) | 19.72252778 |  |  | 83.55727778 |  | Date of Completion |  |  |  | 28.06.2024 |  |
| (DMS) | 19° 43' 21.1" N |  |  | 83° 33' 26.2" E |  | Total Graphite Thickness |  |  |  | 0m |  |
| Pit depth | 4.25 |  |  |  |  |  |  |  |  |  |  |
| R.L | 334 |  |  |  |  |  |  |  |  |  |  |
| Sl. NO | Sample ID | From | To | ASH % | M% | LOI% | SiO2 % | Al2O3% | F.C% | Length (in m) | Lithology |
| 1 |  | 0.00 | 0.60 |  |  |  |  |  |  | 0.60 | Soil |
| 2 |  | 0.60 | 3.10 |  |  |  |  |  |  | 2.50 | Weath. Kh. |
| 3 |  | 3.10 | 4.25 |  |  |  |  |  |  | 1.15 |  |
| 4 | NGPT 3/5 | Horizontal channel in the pit floor |  | 85.20 | 6.78 | 14.80 | 49.64 | 14.11 | 0.86 |  |  |

| Physical & Chemical Assay Log of Naringpanga Graphite Block,F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIT No- | 04 |  |  |  |  | Date of Commencement |  |  |  | 29.06.2024 |  |
| Location( DD) | 19.72163889 |  |  | 83.55722222 |  | Date of Completion |  |  |  | 29.06.2024 |  |
| (DMS) | 19° 43' 17.9" N |  |  | 83° 33' 26.0" E |  | Total Graphite Thickness |  |  |  | 0m |  |
| Pit depth | 4.50 |  |  |  |  |  |  |  |  |  |  |
| R.L | 326 |  |  |  |  |  |  |  |  |  |  |
| Sl. NO | Sample ID | From | To | ASH % | M% | LOI% | SiO2 | Al2O3% | F.C% | Length (in m) | Lithology |
| 1 |  | 0.00 | 0.75 |  |  |  |  |  |  | 0.75 | Soil |
| 2 |  | 0.75 | 4.05 |  |  |  |  |  |  | 3.30 | Weath. Kh. |
| 3 |  | 4.05 | 4.50 |  |  |  |  |  |  | 0.45 |  |
| 4 | NGPT 4/5 | Horizontal channel in the pit floor |  | 89.48 | 4.30 | 10.52 | 60.60 | 12.98 | 0.94 |  |  |

The graphite lode encountered in the pit-2 encompasses a distinct band in N-S direction and its extrapolation is done.

6.1.7 Sampling:

In order to ascertain the mineralogical composition and qualitative potential of graphite in the explored block, 160 core samples have been collected from the borehole cores at an appropriate length of 1 m or wherever there is abrupt change of the involved litho units. The graphite cores having longitudinal length of less than 0.5m were accounted for in the preceding samples.  While the samples lengths of +0.5 to 1m were drawn as one sample when recorded.  After extraction of core from core barrel, the drilled cores are spread systematically in the designated core boxes. The samples thus collected were reduced to desired quantity by coning and quartering were powdered to 200 mesh ASTM size. About 50 g of each sample was collected in duplicate by coning and quartering method and packed with proper leveling tags. All the core samples were analysed at the Research Laboratory Bhubaneswar, by wet chemical analysis method. In Naringpanga block, Al2O3, SiO2, F.C, Ash and LOI has been analysed as per approval of NMET & the assaying and laboratory procedures adopted are as per the BIS standard.

Besides, 13 Nos. core samples have been analysed for petrographic, and 14 Nos. core samples are subjected to XRD studies each at Research Laboratory, Bhubaneswar. For check sample analysis, the samples are tested at New Green Environmental Services Lab Ltd. (NABL accredited), Bhubaneswar. Further 20 Nos. of internal check samples are also analysed in wet chemical method in the Departmental Laboratory of DoMG (O) which shows positive correlation (Annexure-VI).

Another 20 samples have been analysed at MECL Laboratory, Nagpur for 34 trace REE elements (Method used- acid digestion followed by ICPMS) through  Agilent make ICPMS Model- 7800 (Report annexed in Annexure -XIII).

Samples have been properly coded after preparation by indexing with references of the investigation area, borehole number and sample serial number and maintained in a register. The quality and appropriateness of the assaying and laboratory procedures used, and the technique are considered as total.

To ensure the accuracy of analytical results, the pulp duplicate of core samples were introduced as check samples, one in nearly every 10 samples. Out of 160 primary samples, 20 samples were chosen and considered as check samples.

Analysis of Samples

The assaying and laboratory procedures adopted are as per the BIS standards. After extraction of core from core barrel, the drilled cores are spread systematically in the designated core boxes. During sampling, meter wise longitudinal half of the core has splitted and was crushed manually by iron mortar and pestle and the other half is preserved for future reference. In case of sludge cores, samples were reduced by standard coning and quartering at -200 mesh size.

Samples have been properly coded after preparation by indexing with references of the investigation area, borehole number and sample serial number and maintained in a register. The quality and appropriateness of the assaying and laboratory procedures used, and the technique are considered as total. Utmost care is taken to avoid any contamination of the sample and following standard procedures for core collection from drill holes and core boxes, marked core length by using metal tags.

The sample sizes are based on the lithology and its homogeneity. Any contrast in the lithology etc. has been taken into consideration for sample size. However, in core drilling maximum one meter length is considered and final samples reduced and sieved to 200 mesh size for analysis.

The Naringpanga Graphite exploration Project was conducted at G3 level. The tonnage has been estimated as an in-situ body with natural moisture content. To ensure the accuracy of analytical results, the pulp duplicate of core samples were introduced as check samples, one in nearly every 10 samples. Out of 160 primary samples, 20 samples were chosen and considered as check samples. The analytical results of primary and check samples showed very good correlation.

# Trace Elements Studies:

A total of 20 composite samples from graphite-mineralized zones were analysed using ICP-MS (Inductively Coupled Plasma Mass Spectrometry) at MECL laboratory. These analysis targeted minor and trace elements, covering 34 elements, including Li, Be, Sc, V, Cr, Co, Ni, Cu, U, Zn, Ga, As, Se, Rb, Sr, Y, Zr, Ge, Nb, Mo, Ag, Cd, In, Sn, Sb, Te, Cs, Ba, Hf, Ta, W, Pb, Bi, and Th at MECL Laboratory, Nagpur, the results of which are furnished in Annexure- XIII.

The ICP-MS studies show that Cr, V, Zn, Zr, Ba and Sr show good concentration. The ranges of trace elements detected are given below.

Table 6.3: Trace Element Concentration range (ppm).

| # Elements | # Li | # Be | # Sc | # V | # Cr | # Co | # Ni | # Cu | # U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Min | # 2.81 | # 0.61 | # 3.69 | # 30.83 | # 42.37 | # 4.68 | # 15.99 | # 32.22 | # 0.35 |
| # Max | # 28.68 | # 4.23 | # 20.23 | # 184.49 | # 139.91 | # 25.67 | # 70.84 | # 156.82 | # 7.28 |
| # Elements | # Zn | # Ga | # As | # Se | # Rb | # Sr | # Y | # Zr | # Ge |
| # Min | # 31.54 | # 7.72 | # 32.94 | # 0.30 | # 14.20 | # 53.39 | # 21.59 | # 116.23 | # 0.65 |
| # Max | # 230.34 | # 22.86 | # 41.97 | # 1.33 | # 115.86 | # 379.94 | # 48.92 | # 355.83 | # 2.98 |
| # Elements | # Nb | # Mo | # Ag | # Cd | # In | # Sn | # Sb | # Te |  |
| # Min | # 0.56 | # 0.82 | # 0.13 | # 0.24 | # 0.10 | # 0.89 | # 0.97 | # 0.12 |  |
| # Max | # 2.78 | # 5.99 | # 0.58 | # 0.57 | # 0.10 | # 6.31 | # 1.63 | # 0.40 |  |
| # Elements | # Cs | # Ba | # Hf | # Ta | # W | # Pb | # Bi | # Th |  |
| # Min | # 0.19 | # 164.14 | # 3.17 | # 0.11 | # 0.16 | # 10.82 | # 0.19 | # 2.19 |  |
| # Max | # 3.09 | # 853.62 | # 9.43 | # 1.02 | # 1.33 | # 31.49 | # 0.58 | # 31.59 |  |

X-Ray Diffraction Studies:

A total 14 no of samples have been analysed by X-ray diffraction method at DOMG’s Physical Laboratory to determine the proportions of major and minor constituents. The methodologies and results are given in Annexure-XVI(C).

The XRD studies identified the presence of the following constituent minerals: Quartz, Graphite, Biotite, Kaolinite, Orthoclase, Andesine, Almandine, Pyrite, Titanite, Montmorillonite, Sillimanite, Clinochlore, Microcline and Epidote are the constituent minerals which have been verified with the petrographic studies.

# Chemical Analysis:

Laboratory Procedure: The chemical analysis of primary samples has been carried out in Chemical Research Laboratory Bhubaneswar for determining 8 radicals i.e Al2O3, moisture, SiO2, FC, V.M, Ash, V2O5 and LOI.

Graphite Analysis:

Sample preparation: Crush and grind the sample to a fine powder.

Acid digestion: Digestion of the sample in a mixture of acids (e.g., sulfuric acid, nitric acid, and hydrofluoric acid) to break down the graphite.

Filtration: Filtration of the solution to separate the graphite from the acid-soluble impurities.

Ignition: Heating the graphite at high temperature (e.g., 1000°C) to remove volatile matter and moisture.

Gravimeter: Determination of the graphite content by weighing the residue.

The Wet Chemical Method is a traditional and reliable technique for analyzing the graphite, but it may be time-consuming and require specialized equipment and expertise.

Primary Sample Analysis: A total of 160 primary samples were analyzed at the Chemical Research Laboratory, Bhubaneswar, to determine eight radicals: Al₂O₃, Moisture, SiO₂, fixed carbon (FC), volatile matter (VM), ash, V₂O₅, and loss on ignition (LOI). The data were processed based on the results of these analyses. Considering the low grade of graphite in the area, interpretation of graphite zones was carried out using a cut-off grade of 2% FC for flaky graphite. Based on the borehole analytical data, mineralized zones were delineated as per the aforementioned cut-off and are presented in Annexure-II. These zones are also plotted along the boreholes in all the cross-sections of the Naringpanga (G-3) blocks and are shown in Plates VIII to XII.

Check Sample Analysis: As per provision made in the proposal for QA/QC, chemical analysis of check samples and its comparison have been carried out in order to assess the bias and inaccuracies in assaying. A total of 20 Nos. (Around 10%) internal check samples and 10 Nos. (Around 10%) external check samples have been analysed in the Chemical Laboratory of DOMG and New green environmental services (NABL accredited) respectively. The results and comparison of check sample analyses with primary sample analysis (External) are presented in Annexure-V and Annexure-VI respectively. During survey, cross verification was done after each day’s DGPS/Total station. Again, the RL values are correlated with the S.I.T.S RL values. The sampling was done with utmost care meticulously. The core being soft and moderately hard are broken with a polythene covering and during crushing & sieving. Sieve covers are used to ensure zero contamination and loss of fines. Zipped polythene are also used to avoid breaking and contamination.

Composite Sample Analysis:

After delineation of mineralised zones of graphite at the required cut-off grade of graphite FC% i.e., 2.00 % FC (Threshold Value for flaky graphite) composite samples were prepared by mixing the primary samples from the graphite mineralized zones in length proportion. A total of 11 no of composite samples have been prepared from the boreholes and pit and analysed for nine radicals in the Chemical Laboratory of the DOMG. The results of the chemical analysis of composite samples are given in Annexure No-VII. The composite sample analysis results are furnished below.

Table 6.4: Analytical Results of Composite Samples from Boreholes

| SI No. | Identity Mark | Lab No. | Ash% | M% | V.M% | F.C% | LOI% | SiO2 % | Al2O3% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NPC1/1C | 865-P/24 | 91.66 | 2.06 | 3.32 | 2.96 | 8.34 | 69.93 | 11.29 |
| 2 | NPC1/2C | 866-P/24 | 93.34 | 1.69 | 3.08 | 1.89 | 6.66 | 67.99 | 12.93 |
|  | NPBH-1 | Average | 92.50 | 1.88 | 3.20 | 2.43 | 7.50 | 68.96 | 12.11 |
| 3 | NPC2/1C | 867-P/24 | 93.80 | 1.50 | 3.26 | 1.44 | 6.20 | 70.54 | 12.52 |
| 4 | NPC2/2C | 868-P/24 | 93.00 | 1.29 | 4.50 | 1.21 | 7.00 | 64.06 | 16.21 |
|  | NPBH-2 | Average | 93.40 | 1.40 | 3.88 | 1.33 | 6.60 | 67.30 | 14.37 |
| 5 | NPC3/1C | 869-P/24 | 93.56 | 1.33 | 3.20 | 1.91 | 6.44 | 65.70 | 15.86 |
| 6 | NPC3/2C | 870-P/24 | 93.16 | 1.00 | 3.25 | 2.59 | 6.84 | 67.90 | 14.09 |
|  | NPBH-3 | Average | 93.36 | 1.17 | 3.23 | 2.25 | 6.64 | 66.80 | 14.98 |
| 7 | NPC4/1C | 871-P/24 | 91.98 | 2.04 | 4.45 | 1.53 | 8.02 | 66.05 | 14.59 |
| 8 | NPC4/2C | 872-P/24 | 90.16 | 1.40 | 3.45 | 4.99 | 9.84 | 72.70 | 9.94 |
|  | NPBH-4 | Average | 91.07 | 1.72 | 3.95 | 3.26 | 8.93 | 69.37 | 12.26 |
| 9 | NPC5/1C | 873-P/24 | 95.26 | 1.03 | 2.35 | 1.36 | 4.74 | 69.76 | 14.19 |
| 10 | NPC5/2C | 874-P/24 | 94.96 | 1.06 | 2.51 | 1.47 | 5.04 | 70.20 | 12.97 |
|  | NPBH-5 | Average | 95.11 | 1.05 | 2.43 | 1.42 | 4.89 | 69.98 | 13.58 |
| 11 | NPBH-6 | 875-P/24 | 87.14 | 2.82 | 4.65 | 5.39 | 12.86 | 63.66 | 16.12 |

6.1.8 Discussion:

The results of primary and check sample analysis show negligible variability and can be considered as valid and acceptable. The details of primary (internal) and external check samples are provided in Annexures VI and V, respectively, with a comparative analysis of Fe₂O₃, SiO₂, and Al₂O₃ (%) presented in Table 6.5. The scatter plots generated by plotting the primary and   check sample data sets are presented in fig. 17 to 19.

Table 6.5: Comparative Analysis of F.C. %, SiO₂%, and Al₂O₃ (%) in Primary and External Check Samples.

| Sl. No | Sample code | PRIMARY |  |  | EXTERNAL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | F.C. % | SiO2 % | Al2O3% | F.C. % | SiO2 % | Al2O3% |
| 1 | NPBH-1/1 | 7.84 | 80.78 | 2.58 | 7.79 | 40.91 | 0.21 |
| 2 | NPBH-1/28 | 1.31 | 65.94 | 13.76 | 0.31 | 59.04 | 0.26 |
| 3 | NPBH-2/12 | 2.39 | 64.76 | 13.94 | 1.06 | 40.28 | 0.15 |
| 4 | NPBH-2/22 | 2.00 | 64.30 | 12.02 | 0.93 | 52.53 | 0.16 |
| 5 | NPBH-3/3 | 3.93 | 55.06 | 14.32 | 1.72 | 44.54 | 0.26 |
| 6 | NPBH-3/30 | 2.07 | 66.26 | 15.78 | 0.75 | 43.55 | 0.54 |
| 7 | NPBH-4/26 | 10.03 | 78.62 | 3.00 | 9.03 | 40.67 | 0.11 |
| 8 | NPBH-4/28 | 9.26 | 80.72 | 0.60 | 8.48 | 39.86 | 0.19 |
| 9 | NPBH-5/20 | 2.50 | 66.12 | 12.56 | 1.68 | 41.26 | 0.62 |
| 10 | NPBH-6/8 | 3.38 | 57.56 | 16.18 | 0.87 | 50.78 | 0.57 |

Fig. 17: Scatter plots showing the F.C. % comparison between the primary samples and external check samples.

Fig. 18: Scatter Plot Showing Correlation Between Primary and External Al2O3% Samples

# Fig. 19: Scatter Plot Depicting Correlation Between Primary SiO₂ % and External Check Sample SiO₂ %.

Mean, Mode Median, Standard deviation and variogram

Mean, mode, median, standard deviation, and variogram of Fixed Carbon percentage have been calculated from the chemical analysis data (160 samples)

Table 6.6: Statistical Summary (Mean, Median, Mode, Standard Deviation, and Variance) of 160 Borehole Samples.

| Mean: | 2.30 |
| --- | --- |
| Median: | 1.89 |
| Mode: | 1.26 |
| Standard Deviation: | 1.79 |
| Variance | 3.20 |

Fig.20: Histogram Showing the Distribution of Fixed Carbon (F.C. %) in 160 Borehole Samples.

Fig. 25: Experimental Variogram Showing Spatial Variability of F.C. % in 160 Borehole Samples.

6.1.9 Ore Zones:

The Naringpanga graphite block is explored by way of penetrating 6 boreholes in the northern part while the southern part was examined by 4 pits. Boreholes NPBH-1, NPBH-2, and NPBH-4 encountered mineral zones at shallow levels, with mineralized zones intercepted at depths of 6-7 meters below the surface. The deepest borehole in this study, NPBH-5, attained an inclined depth of 78.80m, (55.71m vertical depth) and intercepted graphite mineralization of 5.50m thickness. While the minimum inclined depth of 55.00m (38.88m vertical depth) was achieved in the borehole, NPBH-2, in which 9.65m of graphite mineralization has been intercepted. The overall thickness of the mineralization varies in response to the litho-structural control, but at least 3 levels of mineralized zones of variable thickness are evident in most boreholes. The average thickness of graphite mineralization intercepted in the boreholes is 9.93m.

The grade of graphite ranges from 2.0% to 10.03% FC. Borehole NPBH-1 exhibits the highest concentration of F.C (Fixed Carbon) values, recorded between 12.00m and 14.00m depth levels, conversely, the lowest F.C. value of 2.20% is found in NPBH-5. In terms of mineralization thickness, the highest graphite thickness was observed in NPBH-1, followed by NPBH-3 and NPBH-4. Besides, Pit 1 & 4 have also intercepted graphitised gneisses at about 2.5 m depth levels.

6.2 Geophysical Exploration

6.2.1: Geophysical Survey:

As per approval of 27th EC held on 10th January 2023, Self-Potential geophysical survey was undertaken in Total Area: 0.143 Sq. Km of Naringpanga block involving 13.5 line km surveys. Initially the target area was covered by SP survey over 14.3 Ha. But due to local resistance the initially proposed area could not be taken up for exploration. With due approval of TCC of NMET the co-ordinates of the block were revised to the further south of the earlier proposed area. Thus, a total of 29.22 Ha area was geophysically surveyed out of which geophysical interpretation was done in the present block over 14.3 Ha. The detailed Geophysical Report on Self Potential (641 stations out of 1335 stations) for graphite mineralisation in the Naringpanga block is given in Annexure-XIX.

# Objective and Scope of Work:

# The main objective of the geophysical survey was to delineate graphite ore zone and other associated minerals in Naringpanga block. The survey revealed significant self-potential (SP) anomalies characterized by negative SP high values. These anomalies are most likely associated with graphite mineralization, indicating the presence of subsurface graphite deposits.

Scope of the Survey

- Self-potential survey over 1335 data points in 13.5 line km grid pattern in the entire block.

- Preparation of a self-potential survey Report covering the above aspects.

# 6.2.2 Methodology

# Exploration Parameters:

The coordinates of the corner points of the block boundary of the Naringpanga block are given in Chapter No. 3. The corner points of the block have been demarcated for Geophysical Survey in the field.

i) Instrument Details and theory:

The Self-Potential (SP) survey was performed using a Resistivity/SP Meter, Porous Pot Electrodes, and a Cable Reel. This method measures natural potential differences in the ground with a resolution in millivolts (mV). The SP survey detects subsurface electrical potentials caused by electrochemical, electro kinetic, and geothermal processes. These natural potentials arise from the movement of fluids, chemical reactions, or conductive materials such as graphite or sulphide minerals.

Due to the association of conductive materials like graphite with specific geological formations, SP surveys enable the delineation of mineralized zones, identification of groundwater pathways, and mapping of subsurface structures. This technique is particularly effective in locating structurally controlled deposits and understanding the subsurface conditions in areas with complex geology.

Data Acquisition:

The self-potential (SP) survey in the project area was conducted using a high-impedance potentiometer, two non-polarizing electrodes, and cables. The electrodes consisted of copper (Cu) metal immersed in a copper sulphate (CuSO₄) solution, housed in a porous pot. To ensure accuracy, a uniform batch of saturated salt solution was prepared, and the porous pots were filled accordingly. These were then short-circuited in a bath containing the same saturated salt solution and left overnight. Prior to commencing fieldwork, the potential difference between each pair of porous pots was measured, and only those with zero potential difference were selected for the survey.

At the end of each day, the porous pots and copper rods were thoroughly cleaned and recharged with fresh salt solution. They were then short-circuited in the same bath and kept overnight to prepare them for the following day's work. This rigorous process was consistently repeated throughout the duration of the survey to maintain the reliability and precision of the measurements.

| Fig.21: Instruments used for Self-potential survey. |  |
| --- | --- |

The self-potential (SP) data were measured at multiple survey stations along traverses laid in the area of interest, oriented perpendicular to the strike of the ore body or geological formations. A baseline was established along the NW-SE direction of the project area, and SP data were collected using a grid pattern of 25 m x 10 m spacing. This grid comprised 32 lines, all laid perpendicular to the baseline to ensure comprehensive coverage. A reference point was selected near the Nala section, located to the west of the project area. SP data collection began with respect to this primary reference point and continued to the end of the wire. Upon completion of the initial wire, a secondary reference base was chosen, and subsequent SP data were collected relative to this secondary reference point. To ensure consistency and accuracy, all secondary base data were ultimately reduced and recalibrated with respect to the primary base point.

.

Fig.22: Self-potential data acquisition arrangement.

Data Processing:

The recorded SP data were systematically entered into a notebook computer in a structured format, which included the station number; UTM coordinates, and corrected SP values. This dataset served as the foundation for subsequent analysis and visualization. The data were processed and utilized in Surfer, a specialized software tool, to generate various graphical outputs and SP contour maps. These visual representations provided valuable insights into the spatial distribution of SP anomalies and their correlation with subsurface features.

1. Pot Correction:

The difference in pot readings arises from subtle discrepancies in construction, leading to varying levels of porosity between pots. This discrepancy usually remains constant and does not impede the Self-Potential (SP) survey. Nevertheless, an abrupt shift in pot readings may stem from factors such as cracks, contact between the porous pot and metal sulphides, or an under saturation of copper sulphate in one or both pots' solutions. Ahead of the SP data acquisition, the pot difference was computed between two porous pots, and corresponding corrections have been applied to the SP data at each station point.

2. Reference Correction:

The readings for the last five station points were recorded in relation to the secondary reference station, employing the opposite terminal convention (where the stationary base pot connects to the positive terminal and the roving pot to the negative terminal of the potentiometer). These readings were then combined with their respective values measured in reference to the primary base. The average of these five combined readings served as the reference correction for subsequent measurements taken in reference to the secondary 9 reference point. This systematic approach was applied to reduce the entire Self-Potential (SP) dataset in relation to the primary reference point.

3. Drift Correction:

While measuring an SP line, the potential difference between the base pot and roving pot undergoes changes owing to fluctuations in chemical composition and temperature, leading to what is known as pot drift. To mitigate this drift, precautions were taken, such as placing the pots in the shade. Additionally, external factors like sunlight, magnetic storms, and high contact resistance can introduce further drift into the SP data acquisition process.

Following the reduction of SP data to the primary reference, corrections for drift were applied to the dataset. This correction process is essential to enhance the interpretability of SP variations, enabling a more accurate understanding of lithological variations, topographical influences, and the identification of mineralized zones of interest.

SP Data Interpretation:

Qualitative interpretation of self-potential (SP) data has been conducted through profile plots and contour maps. To enhance the correlation of local trends, lithological contacts, and historical workings, the geological map may be overlaid onto the SP contour map. While the SP anomaly minimum is generally assumed to coincide directly with the anomalous body, adjustments are made for steep topography where the anomaly minimum might be displaced downhill.

Fig.23: Integrated Geological and Geophysical map of Naringpanga Graphite Block, Rayagada district, Odisha

The isoanomali contour provides a pictorial view of the mineralization zone. The elliptical form of the SP contours indicates that the major axis aligns with the direction of elongation of the poles. The contour map unveils an elliptical structure with a NW-SE orientation, consistent with the geological strike direction. Within the project area, SP values range from 80.2mV to -154.6mV. The anomalous zone, highlighted by a deep red coloration in the central region, extends beyond the project area, as suggested by the open contours on the map. Analysis reveals an up-dip orientation in the southwest and a down-dip orientation in the northeast of the anomalous zone. Consequently, the deep red-colored portion on the contour map is indicative of a probable graphite-bearing zone.

Profile lines reveal a consistent trend of decreasing SP values from the southwest (SW) to the central region and then gently increasing towards the northwest (NW) Notably, the primary anomalous zone identified in the central part of the project area along the profile lines L20, L21, L22, L23, and L24. Striking details emerge when connecting station points corresponding to peak positive and peak negative SP values, offering insights into the anomalous zone's strike directions.

Fig.24: Composite Self-Potential (SP) Profile Plot across Multiple Survey Lines in the Naringpanga block.

Fig. 25: Isopotential Map illustrating SP Anomaly Zones in the Naringpanga block.

Location of data points:

All borehole collars were placed and fixed based on SP anomalies and graphite exposures observed along the Nala section, rather than adhering to a uniform grid pattern. Boreholes were inclined and drilled to a maximum depth of 78.80 m to intersect the mineralized zones. Additionally, four pits were excavated in the mineralized zones for resource estimation purposes.

The superimposed geological and geophysical map, along with borehole (BH) locations, is annexed as Plate No. XIII.

6.3 Geochemical Exploration:

REE & RM

REEs (Rare Earth Elements) include light REEs (LREEs) such as scandium, lanthanum, cerium, praseodymium, neodymium, promethium, samarium, and europium, as well as heavy REEs (HREEs) such as gadolinium, terbium, dysprosium, holmium, erbium, thulium, ytterbium, lutetium, and yttrium. Based on REE and RM (Rare Metal) analyses of 34 major and minor elements, the chondrite-normalized REE plots (Taylor & Mcmilan, Pierce, et. al. 1984 indicates elevated concentrations of LREEs compared to their crustal abundance (Fig.26, a, b, c, d).

Table 6.7: Trace element abundance in migmatised khondalite & quartzo-feldspathic gneiss.

| # Elements | # Li | # Be | # Sc | # V | # Cr | # Co | # Ni | # Cu | # U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Min | # 2.81 | # 0.61 | # 3.69 | # 30.83 | # 42.37 | # 4.68 | # 15.99 | # 32.22 | # 0.35 |
| # Max | # 28.68 | # 4.23 | # 20.23 | # 184.49 | # 139.91 | # 25.67 | # 70.84 | # 156.82 | # 7.28 |
| # Elements | # Zn | # Ga | # As | # Se | # Rb | # Sr | # Y | # Zr | # Ge |
| # Min | # 31.54 | # 7.72 | # 32.94 | # 0.30 | # 14.20 | # 53.39 | # 21.59 | # 116.23 | # 0.65 |
| # Max | # 230.34 | # 22.86 | # 41.97 | # 1.33 | # 115.86 | # 379.94 | # 48.92 | # 355.83 | # 2.98 |
| # Elements | # Nb | # Mo | # Ag | # Cd | # In | # Sn | # Sb | # Te |  |
| # Min | # 0.56 | # 0.82 | # 0.13 | # 0.24 | # 0.10 | # 0.89 | # 0.97 | # 0.12 |  |
| # Max | # 2.78 | # 5.99 | # 0.58 | # 0.57 | # 0.10 | # 6.31 | # 1.63 | # 0.40 |  |
| # Elements | # Cs | # Ba | # Hf | # Ta | # W | # Pb | # Bi | # Th |  |
| # Min | # 0.19 | # 164.14 | # 3.17 | # 0.11 | # 0.16 | # 10.82 | # 0.19 | # 2.19 |  |
| # Max | # 3.09 | # 853.62 | # 9.43 | # 1.02 | # 1.33 | # 31.49 | # 0.58 | # 31.59 |  |

The geochemical data from the study area provide insights into the elemental composition and its geological implications. The elemental data indicate that light rare earth elements (LREEs) such as lanthanum, cerium, along with heavy rare earth elements (HREEs) including scandium, and yttrium, are well-represented. Chondrite normalized REE plots display higher concentrations of LREEs relative to their crustal abundance, suggesting enrichment processes by dynamo-thermal & regional contact metamorphism.

(a)

(b)

Sample Index Nos.

Fig. 26: Chondrite normalized plots of Migmatised Khondalite & Quartzo-feldspathic Gneisses. (d) (c)

Additionally, the study highlights elevated concentrations of several elements like vanadium (V), strontium (Sr), zirconium (Zr), barium (Ba), and rubidium (Rb) are notable. These elements reflect distinct geochemical processes and the area's potential for mineral resources.

Vanadium exhibits a maximum concentration of 184.49 ppm, indicative of its association with mafic minerals or possible hydrothermal alteration. The presence of Sr (up to 379.94 ppm) and Ba (up to 853.62 ppm) suggests their enrichment in feldspathic phases, pointing to a magmatic or metamorphic origin, potentially involving pegmatite or granitic processes. Similarly, Zr, with a maximum value of 355.83 ppm, highlights zircon enrichment, often linked to high-grade metamorphism or felsic magmatic activities. Transition metals such as chromium (Cr), cobalt (Co), nickel (Ni), and copper (Cu) exhibit moderate concentrations, reflecting the contribution of mafic-ultramafic components or fluid-mediated transport during tectono-metamorphic processes. Additionally, thorium (Th) and uranium (U), with respective maximum values of 31.59 ppm and 7.28 ppm, point toward a potential for radioactive mineralization, often associated with granitic and metamorphic terrains. Rare metals like hafnium (Hf) and tantalum (Ta) show moderate enrichment, suggesting their association with accessory minerals such as zircon or columbite in high-grade metamorphic rocks. Secondary processes like silicification and mylonitization may also play a role in redistributing these elements along structural features.

The overall geochemical signature underscores the influence of tectonic and metamorphic processes, including partial melting, recrystallization, and fluid mobilization, in shaping the elemental distribution.

Fig.27: Histogram of REE concentrations across 34 parameters from 20 samples.

CHAPTER-7

INTEGRATION OF GEOLOGY, GEOPHYSICS AND GEOCHEMICAL EXPLORATION DATA & INTERPRETATION

The integration of the field geological data, outcrop disposition of graphitized lithologies in the vicinity of the study area and their extrapolation provided a guiding tool to fix the borehole target points to study the geo-spatial continuity of the ore bearing formations in the sub surface. The geological map has been superimposed over the self-potential (SP) contour map generated through mapping and SP survey in a GIS platform to interpret the potential target of mineralisation. Accordingly, the borehole points are fixed on anomalous zones in the up dip direction of the exposed graphitic gneisses and schists and borehole (BH) locations are presented as Plate No. XIII (Fig. 28). The map clearly demonstrates a spatial correlation between SP anomaly zones and geological formations. Boreholes were placed in low SP zones and extrapolated graphite-bearing outcrops based on geological reasoning. Notably, borehole NPBH-1, situated within an SP anomaly measuring -110 mV, confirmed the presence of graphite mineralization. Similar graphite intercepts were recorded in other boreholes located within low SP zones. This consistent alignment between SP anomalies and borehole results highlights a strong geophysical – geological correlation, underscoring the effectiveness of SP surveys in delineating subsurface graphite-rich zones.

Fig.28: Overlay of Self-Potential (SP) Anomalies on the Geological Map of the Study area.

CHAPTER-8

MINERAL PROSPECT

8.1: Surface indication

During geological mapping, over 0.143 sq km, exposures of highly altered and kaolinised pelitic gnessies and schists and also altered to soil exposed in the deep gullies. However, isolated exposures of migmatised quartzo-feldspathic gneisses and migmatised khondalites are noticed in the nala bed and walls in which graphite occurs as disseminations beyond the mapped area.

8.2: Mode of occurrence

Here, disseminated flaky graphite is found to occur as small lenses and pockets. Varying concentrations of graphite occur as disseminated flakes within the pelitic gneisses and quartzo-felspathic gneisses show NNW-SSE foliation and high angle dips towards west. Both quartzo-felspathic gneiss and migmatised graphite-sillimanite bearing gneisses exhibit more or less banded nature and conformable. The dip of litho units are extrapolatable to the present area which led to intercept the graphite lodes in the bore holes given at 450 across the dip.

| Fig. 29. Altered migmatised khondalite in the Bangei Nala bed & eastern wall section. |  |
| --- | --- |

8.3: Strike length and width of anomalies

Geological mapping and trial exaction in the area coupled with ground geophysical reveals that the area is a potential area for graphite mineralisation. Integration of surface geological data and geophysical data indicate the possible extent of mineralised zones along the axial length of the block. It has been deciphered that the graphite bearing host rocks extends for a strike length of 550 m and average width of 142 meter cumulatively.

8.4: Alteration zones

The host rock of graphite in the study area has undergone intense weathering and alterations. Feldspars are mostly kaolinised and garnets are oxidized to variable extent. The weathering and alteration are so intense that insitu exposures have altered to soi to variable depth as observed in the nala walls. The pit walls also exposed the altered rocks with relict foliations (Fig.30)

Fig. 30. Altered migmatised khondalite in the pit wall wall section.

8.5: Genesis of mineralisation.

Graphite is a high grade mineral representing high temperature and  pressure conditions. The litho-units in the study area primarily comprise altered khondalite, graphite-bearing migmatized khondalite, and quartzo-feldspathic gneiss, which form a banded assemblage. These rocks are of meta-sedimentary origin and exhibit strong foliation. Mineralogical studies indicate that the graphite occurs as disseminated flakes and fibers along the foliation planes of migmatised khondalite and quartzo-feldspathic gneiss, with an erratic distribution. The deleterious elements present in these mineralised zones include quartz, plagioclase, orthoclase, and opaques. The lithology includes khondalite ± graphite schists and gneisses, both of which have undergone high grade granulite facies metamorphism. Within this lithological framework, specific units contain graphite mineralization, particularly in the migmatised pelites, which are metamorphosed rocks made up of sand-sized psammite and fine-grained pelite minerals that have experienced partial melting and recrystallization. The quartzo-feldspathic invasions, which involve the intrusion of quartz and feldspar-rich materials, granitoids, are also present within these units. These geochemical patterns indicate a primarily within-plate granitic magma origin with minor volcanic arc contributions, reflecting a complex magmatic evolution. Secondary overgrowth in most of the quartz grains indicates a sedimentary parentage.

CHAPTER-9

EXPLORATION

SYSTEMATIC DRILLING

# 9.1 Methodology of drilling with details of type of drilling

In order to ascertain the subsurface geometry, grade and resource of graphite in the block, 6 inclined boreholes are given to intercept the ore zones in the first level depth as approved by NMET. The boreholes were planned to reach depths of 30 m vertical levels and 60 m levels to explore the subsurface effectively and continued upto interception of a non-mineralised zone.  Thus a cumulative drilling meterage of 398 m was achieved. The drilling work was outsourced to M/s Geo Environmental Services, Bhubaneswar. Diamond core drilling in six inclined boreholes were drilled at the borehole points interpreted from the SP anomalous zones, so as to intercept the graphite lode in the up-dip direction in compliance to the geological disposition of the ore body exposed in the nala section.  The inclined BHs are given at an angle of 450 with respect to the horizontal as the litho-assemblages in the terrain exhibit dip angles varying from 250-700.

The agency has deployed Volt-180 and Volt-90 rigs for the operation (Fig.31). The drilling employed a combination of dry and wet techniques. Dry drilling was conducted in soft formations and secured with HW casings up to inclined depth from 3 m to 6 m in length. For hard formations, wet drilling was carried out using a double-tube barrel system to ensure optimal core recovery. The cumulative drilling meterage achieved in 6 BHs is 398.00 m.

Fig. 31: Field photograph capturing drilling activities BH No. NPBH -1.

9.2 Borehole planning

Initially, the G3 stage exploration proposal included approximately 10 boreholes in a regular grid pattern. However, after a review by the TCC, it was advised to undertake only six boreholes with a total drilling meterage of 400m.  Borehole locations were selected based on anomalous self-potential (SP) zones and extrapolated outcrop trends within the Nala section (Plate- XIII). Borehole NPBH-1 was placed over a high-potential anomaly zone, exhibiting an SP value of -110 mV. The BH loci are logically fixed with respect to exposure map & SP anomalous zone, without any bias. All borehole collars were placed and fixed based on SP anomalies and extrapolated graphite exposures observed along the Nala section, rather than adhering to a uniform grid pattern with approval of TCC. Boreholes were inclined and drilled to a maximum depth of 78.80 m to intersect the mineralized zones. Additionally, four pits were excavated in the mineralized zones for resource estimation purposes

9.2.1 Spacing of Boreholes:

The borehole locations were selected based on the anomalous SP zones and extrapolated outcrops in the Nala section. NPBH-1 was positioned in a high-potential anomaly zone (SP value of -110mV).

- Spacing of NPBH-1 and NPBH-2- 81.00 m X 42.00 m (NPBH-2 is located to the southeast of NPBH-1)

- Spacing of NPBH-3 and NPBH-4 -  73.5 m X 36.00 m (NPBH-4 is located to the Northwest of NPBH-3)

- Spacing of NPBH-5 and NPBH-6 -99.0 m X 18.00 m (NPBH-6 is located to the Northwest of NPBH-5). This pair is located approximately 46.5 meters from the exposed graphite body in the Nala section.

# In order to ensure optimum core recovery within the fragile ore zone, Drilling was done by deploying triple tube barrels and the core recovery was satisfactory. The core recovery percentage for the graphite zone in individual boreholes provided in Table 9.4.

9.2.2 Level of Intersection.

All boreholes yielded positive results, reinforcing the area’s potential for graphite mineralization. Detailed information regarding the borehole locations, depths, graphite thickness, and sample generation is provided in Table 9.1.

Table. 9.1. Depth of Boreholes and thickness of mineralised Zones in the block

| Bore Hole | Inclined Depth(m) | Vertical Depth(m) | R.L(m) | Apparent Thickness of Graphite(m) | True Thickness of Graphite(m) | Samples Generated (Nos.) |
| --- | --- | --- | --- | --- | --- | --- |
| NPBH-1 | 70.00 | 49.50 | 331.40 | 24.55 | 17.36 | 31 |
| NPBH-2 | 55.00 | 38.89 | 333.31 | 9.65 | 6.82 | 24 |
| NPBH-3 | 74.50 | 52.68 | 331.02 | 19.55 | 13.82 | 31 |
| NPBH-4 | 56.50 | 39.95 | 335.52 | 18.55 | 13.11 | 31 |
| NPBH-5 | 78.80 | 55.72 | 329.86 | 5.50 | 3.89 | 28 |
| NPBH-6 | 63.20 | 44.69 | 334.14 | 6.50 | 4.59 | 15 |
| Total | 398.00 | 281.43 |  | 84.30 | 59.60 | 160 |

Interpretation of the intercepted ore horizons indicates that there exist two to four levels of ore zones whose dispositions are structurally controlled synchronous with the F2 & F3 foldings as revealed from the borehole cuttings of first level boreholes. As the area is litho-structurally controlled, with steep dipping sub surface litho assemblages, more levels of mineralisation can be proved in the second level and third level boreholes. The depth of intersection of graphite mineralisation in the six drilled holes is furnished below.

Table 9.2 Level of interception of ore zones in different bore holes.

| SL.NO | BHs. No | Lode No | From (m) | To (m) |
| --- | --- | --- | --- | --- |
| 1 | NPBH-1 | 1 | 12.00 | 14.00 |
| 2 |  | 2 | 21.00 | 34.00 |
| 3 |  | 3 | 43.50 | 44.30 |
| 4 |  | 4 | 52.10 | 69.50 |
| 5 | NPBH-2 | 1 | 12.00 | 13.00 |
| 6 |  | 2 | 15.60 | 20.50 |
| 7 |  | 3 | 26.50 | 27.00 |
| 8 |  | 4 | 41.45 | 45.45 |
| 9 |  | 5 | 50.45 | 51.70 |
| 10 | NPBH-3 | 1 | 20.50 | 21.10 |
| 11 |  | 2 | 48.60 | 71.95 |
| 12 | NPBH-4 | 1 | 11.00 | 13.05 |
| 13 |  | 2 | 21.00 | 23.00 |
| 14 |  | 3 | 31.00 | 49.00 |
| 15 | NPBH-5 | 1 | 60.50 | 66.00 |
| 16 |  | 2 | 73.50 | 75.00 |
| 17 | NPBH-6 | 1 | 25.00 | 31.00 |
| 18 |  | 2 | 51.20 | 51.70 |

9.2.3 Co-ordinates & RL of Collar

The depths of the boreholes varied from 55.0 m (NPBH-02) to 78.80 m (NPBH-05). Graphite occurrences were intercepted in all boreholes, confirming mineralization across the drilled locations. Cumulatively, a total thickness of 84.30 m of graphite was intercepted. The maximum graphite thickness of 24.55 m was encountered in NPBH-01, while the minimum thickness of 5.50 m was noted in NPBH-05. To recover high-quality core samples, short drilling runs were performed as needed, ensuring 95% core recovery in the mineralized zones, which is considered satisfactory. Some minor deviations occurred, but overall, the quality of drilling and sampling was well-maintained. Graphite was found associated with quartzo-feldspathic gneiss and migmatised khondalite at varying depths, further validating the geological model. After completion of drilling, all boreholes were properly sealed and plugged with cement pillars to ensure environmental and operational safety. The boreholes drilled are given in the following Table 9.3

Table 9.3   Borehole Positions with Corresponding Depths and Reduced Levels.

| BH No | RL(m) | Longitude | Latitude | Inclined Depth(m) |
| --- | --- | --- | --- | --- |
| NPBH-01 | 331.402 | 83033’14.310” E | 19043’29.218” N | 70.00 |
| NPBH-02 | 333.308 | 83033’17.377” E | 19043’29.744” N | 55.00 |
| NPBH-03 | 331.021 | 83033’19.901” E | 19043’24.906” N | 74.50 |
| NPBH-04 | 335.519 | 83033’21.096” E | 19043’27.307” N | 56.50 |
| NPBH-05 | 329.859 | 83033’21.790” E | 19043’22.260” N | 78.80 |
| NPBH-06 | 334.140 | 83033’24.069” E | 19043’24.704” N | 63.20 |

The overburden, consisting primarily of soil and weathered lithologies, ranges in thickness from 11 m to 20 m (Inclined depth)..

A mylonitic horizon was intercepted in boreholes NPBH-03 and NPBH-05, at depths ranging from 61.0 m to 69.0 m and 60.50 m to 65.00 m, respectively, indicating significant structural deformation zones. Detailed lithologs, capturing these geological features, are included in Annexure-II, while graphic lithologs are presented in Plate-IV for visual representation and interpretation.

# 9.2.4 Borehole Logging

# Core Logging:

The cores recovered from the boreholes, drilled using both dry and wet techniques, were systematically logged to identify and delineate various litho-units. The average core recovery across all boreholes was approximately 95%, indicating a high-quality sampling process. The logging process involved run-wise examination of the core as well as the cuttings from the boreholes. This helped in discerning the physical characteristics such as the colour, shape, and size of lithologies, the nature of graphite mineralization, and other textural and structural features. Key structural features like joints, fractures, and foliations, along with their orientation relative to the core axis, drags, kinks, silicifications, slickensides etc, were also meticulously recorded.

In addition to the physical observations, qualitative analytical data were used to delineate ore-bearing and non-ore litho-units. Since the litho-units showed minimal variation along the down-hole direction and the run lengths were relatively short, a consolidated and summarized litholog was prepared for all boreholes. This litholog provides a graphical representation of the litho-units along geological cross-sections, offering a clear visualization of subsurface geology.

The litho-units identified during the logging process were classified into distinct groups: soil, migmatised khondalite, quartzo-feldspathic gneiss, khondalite + graphite, quartzo-feldspathic gneiss + graphite, garnetiferous quartzo-feldspathic gneiss, and granulite. These groupings reflect the diversity of lithologies encountered in the boreholes. Detailed lithologs and summarized lithologs, providing comprehensive data on the lithological sequences and distributions, are included in Annexure-II for reference and have been utilised in borehole correlations.

Data spacing for reporting of exploration results

Graphite is observed to occur in multiple forms, including disseminated, flaky, veinlet bands, and irregular lensoidal bodies. It is closely associated with quartzo-feldspathic gneiss and khondalite lithologies. The graphite mineralization is believed to have been emplaced along shear planes and within hinge areas of folds present in the quartzo-feldspathic gneiss. BHs are planned on the geophysical anomalous zones so as to intercept the ore body and BH location plan was reviewed & approved by the TCC-NMET.

Exploration at the G3 level was planned based on surface exposures observed in nala sections. Drilling was carried out to investigate the subsurface distribution of graphite, with borehole samples collected at one-meter intervals for grade evaluation. Notably, no borehole was terminated, ensuring comprehensive data acquisition across the drilled sections.

The spacing and distribution of data collected for this G3-level reporting are deemed sufficient to establish the degree of geological and grade continuity necessary for reliable mineral resource estimation and classification procedures.

Drilling techniques and drill sampling employed:

Both dry and wet drilling methods have been adopted to puncture the boreholes. The drilling operation commenced with dry drilling using a TC (Tungsten Carbide) bit to penetrate the soft formations. Loose formations were secured with NQ casings of varying lengths, ranging from 3m to 6m. Following this, diamond core wet drilling was performed. Unlike a uniform grid pattern, the drilling plan was designed to target the ore zone by inclined boreholes. These were positioned based on field observations of graphite exposures along the Nala section and anomalies identified through the SP survey. This systematic approach resulted in a total cumulative drilling meterage of 398.00m, meeting G3-level exploration standards.

The drilled depths ranged from a minimum of 55.0m (NPBH-2) to a maximum of 78.8m (NPBH-5), using HW (66.67mm) and HQ (53.97mm) core barrels. Volt-180 and Volt-90 rigs were employed to execute the drilling. Wet drilling techniques were adopted to maximize sample recovery. Friable cores were carefully collected from the return water after sediment settling in a pond, followed by sun drying to ensure sample integrity and avoid contamination.

Data from drilling revealed no correlation between core recoveries and grade. The average core recovery of all stratas was approximately 95%, with losses attributed to voids, weathering, and alteration in quartzo-feldspathic gneiss and altered khondalite. Weathered formations, such as weathered khondalite and weathered quartzo-feldspathic gneiss, experienced preferential core loss, recovered as sludge. Sludge samples were meticulously collected at the casing mouths using polythene bags to minimize material loss and contamination.

9.2.5 Core Recovery Percentage:

Geological logging of borehole cores carried out to identify the lithology/rock type. The logging data have been recorded and preserved. As per the format of the record of the core length and percentage of the recovery recorded and result assessed properly. The core recovery is measured as retrieved from the core barrels Vis -a –Vis the drilling meterage run i.e., recovery is determined by the formula:  The average core recovery during the drilling operation was calculated to be 95%, influenced by the degree of intense weathering and alteration in quartzo-feldspathic gneiss, altered and weathered khondalite.

Length of core recovered

Recovery=		__________________________         X 100

Meterage of drill run

Standard tube conventional core drilling rigs were used during core drilling operations for maximum core recovery purpose. Utmost care is taken to avoid any contamination of the samples. Overall average recovery of core was 95% while that for the graphite zone in individual boreholes provided in the following table.

Table 9.4: Core Recovery Summary from Graphite Zones in Individual Boreholes.

| Core Recovery of Graphite zone in individual Boreholes |  |  |
| --- | --- | --- |
| SL No. | Borehole no | Recovery (%) |
| 1 | NPBH-1 | 95% |
| 2 | NPBH-2 | 95% |
| 3 | NPBH-3 | 92% |
| 4 | NPBH-4 | 91% |
| 5 | NPBH-5 | 95% |
| 6 | NPBH-6 | 92% |
|  | Average core recovery. | 93.33% |

All drill cores were logged meter wise visually in the field for both qualitative and quantitative. The logs were later modified with the laboratory results. The detailed core logs along with sample analysis results are appended at Annexure-II. All samples generated through core drilling have been analysed in the Research Laboratory, Bhubaneswar by wet chemical method.

The detailed core logging was done for each meter run visually in the field with pocket lens. The logs were later modified with the laboratory results.  In Naringpanga graphite block have been analysed for the core samples by standard procedure of wet chemical analysis and ICP-MS.  The bulk density was also determined to calculate the resource by following the IS: 5842-1986 (Reaffirmed 2008) procedure.

# 9.3  Mineralogy of Ore Zone:

Graphite mineralization in the Naringpanga block is primarily localised along the foliation planes of migmatized khondalites & quartzo-feldspathic gneiss. It occurs as disseminations, with graphite flakes ranging from a few millimeters to 5 millimeters. The mineralization exhibits variations in concentration, such as swelling and pinching, reflecting the structural complexity of the area. The graphite is primarily flaky, indicating low-grade metamorphism, and is deposited along the foliation planes of both khondalite and quartzo-feldspathic gneiss rocks, suggesting structural control over its emplacement.

The mineralized zones are concordant with the host rock, and the graphitised lithologies are masked beneath a thick alluvial soil cover. This soil may influence the distribution of graphite, potentially indicating the mineralization extends over a larger area than present area. The combination of structural controls, disseminated flaky graphite, and the alluvial soil cover makes this deposit unique and potentially extensive.

9.4 Methodology of Ore Zone Sampling and sample preparation

After extraction of core from core barrel, the drilled cores are spread systematically in the designated core boxes. During sampling, meter wise longitudinal half of the core has splitted and was crushed manually by iron mortar and pestle and the other half is preserved for future reference. In case of sludge cores, samples were reduced by standard coning and quartering.

The length of each sample was in general kept around one meter for the   purposes of the investigation. The graphite cores having longitudinal length of less than 0.5m were accounted for in the preceding samples.  While the sample lengths of +0.5 to 1m were drawn as one sample when recorded.  After extraction of core from core barrel, the drilled cores are spread systematically in the designated core boxes. The samples thus collected were reduced to the desired quantity by coning and quartering were powdered to 200 mesh ASTM size. About 50 g of each sample was collected in duplicate by coning and quartering method and packed with proper leveling tags. All the core samples were analysed at the Research Laboratory Bhubaneswar, by wet chemical analysis method. In Naringpanga block, Al2O3, SiO2, F.C, Ash and LOI have been analysed as per approval of NABL.

Based on the chemical assay result a comprehensive lithology of drill holes has been prepared for the individual ore zones of each bore hole.

Utmost care is taken to avoid any contamination of the sample and following standard procedures for core collection from drill holes and core boxes, marked core length by using metal tags.

The sample sizes are based on the lithology and its homogeneity. Any contrast in the lithology etc. has been taken into consideration for sample size. However, in core drilling maximum one meter length is considered and final samples reduced and sieved to 200 mesh size for analysis.

9.5 Chemical Analysis of Samples and Laboratory Procedures

The assaying and laboratory procedures adopted are as per the BIS standard. All the core samples were analysed at the Research laboratory, Bhubaneswar in wet chemical analysis method. Again 13 Nos. core samples have been analysed for petrographic, and 14 Nos. core sample XRD study each at Research Laboratory, Bhubaneswar. For check sample analysis, the samples will be tested at New Green Environmental Services Lab Ltd. (NABL accredited), Bhubaneswar. Further 20 Nos. of internal check samples are also analysed in wet chemical methods in the Departmental Laboratory of DoMG (O) which shows some positive correlation (Annexure-VI).

Another 20 samples have been analysed at MECL Laboratory, Nagpur for 34 REE elements (Method used- acid digestion followed by ICPMS) Instrument used Agilent make ICPMS Model- 7800 (Report annexed in Annexure -XIII).

Samples have been properly coded after preparation by indexing with references of the investigation area, borehole number and sample serial number and maintained in a register. The quality and appropriateness of the assaying and laboratory procedures used, and the technique are considered as total.

The Naringpanga Graphite exploration Project was conducted at G3 level. The tonnage has been estimated as an in-situ body with natural moisture content. To ensure the accuracy of analytical results, the pulp duplicate of core samples were introduced as check samples, one in nearly every 10 samples. Out of 160 primary samples, 20 samples were chosen and considered as check samples.

# Chemical Analysis:

Laboratory Procedure: The chemical analysis of primary samples has been carried out in Chemical Research Laboratory Bhubaneswar for determining 8 radicals i.e Al2O3, moisture, SiO2, FC, V.M, Ash, V2O5 and LOI.

Graphite Analysis:

Sample preparation: Crush and grind the sample to a fine powder.

Acid digestion: Digestion of the sample in a mixture of acids (e.g., sulfuric acid, nitric acid, and hydrofluoric acid) to break down the graphite.

Filtration: Filtration of the solution to separate the graphite from the acid-soluble impurities.

Ignition: Heating the graphite at high temperature (e.g., 1000°C) to remove volatile matter and moisture.

Gravimeter: Determination of the graphite content by weighing the residue.

The Wet Chemical Method is a traditional and reliable technique for analyzing the graphite, but it may be time-consuming and require specialized equipment and expertise.

# Trace Elements Studies:

A total of 20 composite samples from graphite-mineralized zones were analysed using ICP-MS (Inductively Coupled Plasma Mass Spectrometry) at MECL laboratory. These analysis targeted minor and trace elements, covering 34 elements, including Li, Be, Sc, V, Cr, Co, Ni, Cu, U, Zn, Ga, As, Se, Rb, Sr, Y, Zr, Ge, Nb, Mo, Ag, Cd, In, Sn, Sb, Te, Cs, Ba, Hf, Ta, W, Pb, Bi, and Th at MECL Laboratory, Nagpur, the results of which are furnished in Annexure- XIII.

The ICP-MS studies show that Cr, V, Zn, Zr, Ba and Sr show good concentration. The ranges of trace elements detected are given below.

X-Ray Diffraction Studies:

A total 14 no of samples have been analysed by X-ray diffraction method at DOMG’s Physical Laboratory to determine the proportions of major and minor constituents. The methodologies and results are given in Annexure-XVI(C).

The XRD studies identified the presence of the following constituent minerals: Quartz, Graphite, Biotite, Kaolinite, Orthoclase, Andesine, Almandine, Pyrite, Titanite, Montmorillonite, Sillimanite, Clinochlore, Microcline and Epidote are the constituent minerals which have been verified with the petrographic studies.

Beneficiation study

Beneficiation study has not been undertaken for Naringpanga graphite block as the exploration is at G-3 Stage.

9.6 Details of intersected ore zones of the boreholes drilled and their correlation.

Borehole core sample analysis for FC content indicates that mineralisation of graphite has been encountered at minimum inclined level of 11 m level in NPBH-4   and maximum intercepted inclined depth of interception is 75 m in BH-5. Deatailed analysis and plotting of analytical assay of FC values indicates 3 intersected zones in NPBH-1 with a minor inter calation of 80 cm between the second and third level; 3 zones in NPBH-2 with an intercalated graphite zone of 50 cm between first and second level;   3 zones in NPBH-3; 5 zones in NPBH-4;  3 zones in NPBH- 5  but the mineralisation is deep seated at an inclined depth of 60 m & 2 zones in NPBH - 6. The details are furnished in the table below.

Table No. 9.5  Depth of interception of graphite mineralisation in the boreholes.

| SL.No | FROM | TO | LENGTH | NPBH-1 | NPBH-2 | NPBH-3 | NPBH-4 | NPBH-5 | NPBH-6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 11.00 | 12.00 | 1.00 |  |  |  | F.C-2.03% |  |  |
| 2 | 12.00 | 13.00 | 1.00 | F.C-7.2% | F.C-2.04% |  |  |  |  |
| 3 | 13.00 | 13.05 | 0.05 |  |  |  |  |  |  |
| 4 | 13.05 | 14.00 | 0.95 |  |  |  |  |  |  |
| 5 | 14.00 | 15.60 | 1.60 |  |  |  |  |  |  |
| 6 | 15.60 | 16.50 | 0.90 |  | F.C-2.66% |  |  |  |  |
| 7 | 16.50 | 17.50 | 1.00 |  |  |  |  |  |  |
| 8 | 17.50 | 18.00 | 0.50 |  | F.C-2.14% |  |  |  |  |
| 9 | 18.00 | 19.00 | 1.00 |  |  |  |  |  |  |
| 10 | 19.00 | 20.00 | 1.00 |  | F.C-2.01% |  |  |  |  |
| 11 | 20.00 | 20.50 | 0.50 |  |  |  |  |  |  |
| 12 | 20.50 | 21.00 | 0.50 |  |  | F.C- 3.93% |  |  |  |
| 13 | 21.00 | 21.10 | 0.10 | F.C-3.89% |  |  |  |  |  |
| 14 | 21.10 | 22.00 | 0.90 |  |  |  | F.C-2.15% |  |  |
| 15 | 22.00 | 23.00 | 1.00 |  |  |  |  |  |  |
| 16 | 23.00 | 24.00 | 1.00 |  |  |  |  |  |  |
| 17 | 24.00 | 25.00 | 1.00 |  |  |  |  |  |  |
| 18 | 25.00 | 26.50 | 1.50 |  |  |  |  |  | F.C-2.86% |
| 19 | 26.50 | 27.00 | 0.50 |  | F.C-2.30% |  |  |  |  |
| 20 | 27.00 | 28.00 | 1.00 |  |  |  |  |  |  |
| 21 | 28.00 | 29.00 | 1.00 |  |  |  |  |  |  |
| 22 | 29.00 | 30.00 | 1.00 |  |  |  |  |  |  |
| 23 | 30.00 | 31.00 | 1.00 |  |  |  |  |  |  |
| 24 | 31.00 | 32.00 | 1.00 |  |  |  | F.C-2.03% |  |  |
| 25 | 32.00 | 33.00 | 1.00 |  |  |  |  |  |  |
| 26 | 33.00 | 34.00 | 1.00 |  |  |  |  |  |  |
| 27 | 34.00 | 35.00 | 1.00 |  |  |  |  |  |  |
| 28 | 35.00 | 36.00 | 1.00 |  |  |  |  |  |  |
| 29 | 36.00 | 37.00 | 1.00 |  |  |  | F.C-4.11% |  |  |
| 30 | 37.00 | 38.00 | 1.00 |  |  |  |  |  |  |
| 31 | 38.00 | 39.00 | 1.00 |  |  |  |  |  |  |
| 32 | 39.00 | 40.00 | 1.00 |  |  |  |  |  |  |
| 33 | 40.00 | 40.50 | 0.50 |  |  |  |  |  |  |
| 34 | 40.50 | 41.45 | 0.95 |  |  |  | F.C-6.39% |  |  |
| 35 | 41.45 | 42.00 | 0.55 |  | F.C-2.00% |  |  |  |  |
| 36 | 42.00 | 43.50 | 1.50 |  |  |  |  |  |  |
| 37 | 43.50 | 44.30 | 0.80 | F.C-2.19% |  |  |  |  |  |
| 38 | 44.30 | 45.45 | 1.15 |  |  |  |  |  |  |
| 39 | 45.45 | 46.00 | 0.55 |  |  |  |  |  |  |
| 40 | 46.00 | 47.00 | 1.00 |  |  |  |  |  |  |
| 41 | 47.00 | 48.00 | 1.00 |  |  |  |  |  |  |
| 42 | 48.00 | 48.60 | 0.60 |  |  |  |  |  |  |
| 43 | 48.60 | 49.00 | 0.40 |  |  | F.C-2.20% |  |  |  |
| 44 | 49.00 | 50.45 | 1.45 |  |  |  |  |  |  |
| 45 | 50.45 | 51.20 | 0.75 |  | F.C-2.15% |  |  |  |  |
| 46 | 51.20 | 51.70 | 0.50 |  |  |  |  |  | F.C-2.4% |
| 47 | 51.70 | 52.10 | 0.40 |  |  |  |  |  |  |
| 48 | 52.10 | 53.10 | 1.00 | F.C-2.0% |  |  |  |  |  |
| 49 | 53.10 | 53.60 | 0.50 |  |  |  |  |  |  |
| 50 | 53.60 | 54.10 | 0.50 |  |  |  |  |  |  |
| 51 | 54.10 | 55.10 | 1.00 | F.C-2.07% | BH closed |  |  |  |  |
| 52 | 55.10 | 55.60 | 0.50 |  |  |  |  |  |  |
| 53 | 55.60 | 56.35 | 0.75 |  |  | F.C- 2.23% | BH closed |  |  |
| 54 | 56.35 | 58.30 | 1.95 | F.C-2.34% |  |  |  |  |  |
| 55 | 58.30 | 58.60 | 0.30 |  |  |  |  |  |  |
| 56 | 58.60 | 60.00 | 1.40 |  |  |  |  |  |  |
| 57 | 60.00 | 60.50 | 0.50 | F.C-2.37% |  |  |  |  |  |
| 58 | 60.50 | 61.00 | 0.50 |  |  |  |  | F.C-2.94% |  |
| 59 | 61.00 | 61.50 | 0.50 |  |  | F.C- 2.39% |  |  |  |
| 60 | 61.50 | 62.00 | 0.50 |  |  |  |  |  |  |
| 61 | 62.00 | 63.00 | 1.00 |  |  |  |  |  |  |
| 62 | 63.00 | 63.30 | 0.30 |  |  |  |  | F.C-2.26% | BH closed |
| 63 | 63.30 | 64.00 | 0.70 |  |  |  |  |  |  |
| 64 | 64.00 | 65.00 | 1.00 |  |  |  |  |  |  |
| 65 | 65.00 | 66.00 | 1.00 |  |  |  |  |  |  |
| 66 | 66.00 | 67.00 | 1.00 |  |  |  |  |  |  |
| 67 | 67.00 | 68.00 | 1.00 |  |  |  |  |  |  |
| 68 | 68.00 | 69.50 | 1.50 | F.C-3.09% |  |  |  |  |  |
| 69 | 69.50 | 70.00 | 0.50 | BH closed |  |  |  |  |  |
| 70 | 70.00 | 71.00 | 1.00 |  |  |  |  |  |  |
| 71 | 71.00 | 72.00 | 1.00 |  |  |  |  |  |  |
| 72 | 72.00 | 73.50 | 1.50 |  |  |  |  |  |  |
| 73 | 73.50 | 74.50 | 1.00 |  |  | BH closed |  | F.C-2.35% |  |
| 74 | 74.50 | 75.00 | 0.50 |  |  |  |  |  |  |
| 75 | 75.00 | 78.80 | 3.80 |  |  |  |  | BH closed |  |
|  | Note: - Values are rounded off as per convention. |  |  |  |  |  |  |  |  |

9.7 Depth of the Ground Water Condition

Groundwater was encountered between 10.6 m and 12.7 m vertical depth across the inclined boreholes (drilled at 45°), indicating a moderately shallow to moderately deep water table. This variation is governed by the area's lithology, predominantly khondalite and quartzo-feldspathic gneiss, which exhibit variable degrees of weathering and fracturing. These hydrogeological conditions significantly affect drilling operations, particularly in terms of core recovery issues in graphitic zones, drilling fluid losses, and borehole instability in sheared sections. Such factors should be carefully considered in future exploration programs, drilling strategies, and mine planning.

CHAPTER-10

10.1 Bulk density

Bulk density is a critical parameter in the estimation of mineral resources, influencing both the tonnage calculations and the subsequent economic evaluation of mineral commodities. To assess the bulk density of graphite ore in the study area, five representative samples exhibiting varying physical characteristics were collected and analysed.

The analyses were conducted at New Green Environmental Labs Pvt. Ltd., Bhubaneswar, (NABL Accredited)(Annexure - XVIII) following the guidelines prescribed in IS: 5842-1986 (Reaffirmed 2008). The bulk density of graphite ore of different grades varies between 1.48 and 1.70, with an average bulk density of 1.606.

CHAPTER-11

RESOURCE ESTIMATION

11. 1 Detailed description of ore zones

The Naringpanga block hosts graphite zones that vary in size, with consistent lateral and vertical grade continuity. Given the area's qualitative and quantitative graphite potential, an estimation of the resource and grade has been attempted by integrating the average values through the cross-sectional area method and LV panel area method. The chemical analysis of the samples reveals that most exhibit an FC content ranging from 2% to 10.03%. Grades were determined based on this range, with samples containing less than the threshold FC being classified as altered varieties and excluded from the grade calculations. Only samples with an FC content of ± 2% or higher were considered for grade determination. The dissemminated graphite bands occur either regular lenses, streaks or pockets in conformity to the S2 and F3 foliation. Graphite has been encountered at minimum inclined level of 11 m  level in NPBH-4 and maximum intercepted inclined depth of interception is 75 m in BH-5. Deatailed analysis and plotting of analytical assay of FC values indicates 3 intersected zones in NPBH-1 with a minor intercalation of 80 cm between the second and third level; 3 zones in NPBH-2 with an intercalated graphite zone of 50 cm between first and second level;   3 zones in NPBH-3; 5 zones in NPBH-4; 3 zones in NPBH- 5 but the mineralisation is deep seated at an inclined depth of 60 m & 2 zones in NPBH - 6.

The resource estimation extends up to a maximum vertical depth of 55.72 meters from the ground profile, corresponding to an elevation of 270.0 m RL. The assessment has been conducted using the cross-sectional area method, which involves integrating the areas of delineated ore zones across cross-sections. To ensure realistic resource estimation, a 70% recovery factor has been applied. This factor accounts for the presence of intercalated waste material, potential voids within the ore body, sampling errors, and other geological uncertainties. These considerations ensure that the estimate is robust and reflective of the actual recoverable graphite.

Characteristics of High-Silica Graphite Ore:

The graphite in the Naringpanga Block is characterized as low-grade and high-silica ore, with silica (SiO₂) as a significant impurity. The elevated silica content presents challenges for processing and beneficiation, particularly in applications requiring high-purity graphite. Such ore types demand advanced techniques to separate silica and enhance the Fixed Carbon content, which is crucial for industrial applications like refractories, batteries, and lubricants.

11.2 Cut-off grade and minimum stoping width consideration

To classify the graphite into appropriate grades, the threshold limits and specifications provided by the IBM in Circular No. C-284/3/CMG/2017 were adhered. As per these guidelines, graphite from the Naringpanga block has been categorized as low grade, FC (2-5%). The ore zones less than 15 cm are not taken into account during sampling and hence not considered in estimation. However, the partings of 20 to 30 cm within the ore zones are taken into account during sampling and resource estimation.

According to IBM vide Circular No. C-284/3/CMG/2017

| Sl. No. | MINERAL | THRESHOLD VALUE |
| --- | --- | --- |
| 1. | Graphite | - For flaky variety- 2% Fixed Carbon (F.C) (Min.) - For amorphous variety - 10% Fixed Carbon (F.C) (Min.) |

“Threshold value of minerals” will have meaning as defined in Minerals (Evidence of Mineral Contents) Rule, 2015.

“Cut-off grade” means the minimum economic assay grade of the mineral for a deposit below which the mining operations become unviable in the present market dynamics or end use quality. It may vary from deposit to deposit depending upon the market conditions.

The significantly thick weathered quartzo-feldspathic gneiss, weathered khondalite altered to kaolinite, and weathered brecciated zones encountered in boreholes have not been included in the resource estimation. Graphite bearing gneisses, with respect to the grade (FC 2-5%), were used to identify ore horizons and assess vertical and lateral continuity.

Graphite in Naringpanga is of flaky variety and resource estimation is done with respect to the threshold values.

11.3 Description and correlation of lodes

The mineral resource has been explored at the G3 level of the MEMC Rule, 2015, ensuring a better confidence level. The tonnage has been estimated as an in-situ body.  Boreholes that intersected altered and weathered quartzo-feldspathic gneiss and barren khondalites were excluded from resource estimation but considered in delineating mineralized zones with appreciable graphite content.  Out of the six boreholes drilled, all were found positive, and their data were fully utilized for evaluating the resource and grade of graphite ores.

To estimate the graphite resources in the Naringpanga block, one surface method was applied for Pit-2 and five cross-sections were constructed perpendicular to the general foliation trend of the country rocks at a 1:2000 scale, providing a detailed representation of smaller graphite bands. These cross-sections, designated as A–A′, B–B′, C–C′, D–D′, and E–E′ (Annexure-X), were positioned at irregular intervals, guided by anomalous SP zones and observed surface exposures (Plate-II). The assay logs of six positive boreholes (BHs), along with their physical characteristics and intercepted lithologies, were plotted to establish sectional correlations. These correlations, with respect to the grade (FC 2-5%), were used to identify ore horizons and assess vertical continuity. The geometry of the graphite bodies is predominantly concordant with the host lithologies, namely quartzo-feldspathic gneiss and migmatized Khondalite. Altered khondalite, Void spaces, and weathered Quartzo-feldspathic gneiss areas were excluded from the cross-sectional area computation. The ore zones encountered in the bore holes were extrapolated on either side proportionately in compliance to the length of influence prescribed in M (EMC) Rules. The ore zones are joined as per core angle geometry and structural signatures recorded in the recovered litho cores.

11.4 Preparation of LV section and Level plan

The resources of graphite in the explored strike length of the block have been calculated by the LV panel method. The strike influence of the ore zone was taken at 50 % of the spacing between two positive bore holes and 25 %, when the adjacent bore hole is negative or if there is neither any borehole nor any surface evidence to substantiate the physical continuity of the ore zone. The third level borehole was generally drilled at more or less 200.0 m strike interval to check the depth persistence of the mineralization. When there is no borehole on the either side of a borehole the strike influence was extended to 50.0 m on either side considering the positive intersection of the shallower boreholes on the either side of the borehole. The RL of Lode intersection along with up dip and down dip RL for the respective lodes taken from the cross section, and were plotted on a Longitudinal- Vertical section on 1:1000 scale. The length of the panel of lodes of a borehole was prepared based on the strike influence as discussed above whereas the breadth of the panel was constructed based on the up dip and down dip RL of the respective lodes. The dip influence of the ore zone was taken at 50 % of the difference in the centre of intersection of the ore body at two positive levels of two boreholes along the same profile line, however, this was not strictly followed when there is a variation of thickness along different levels and rule of gradual change was applied. If there is a single borehole along a profile, the dip length was extended only up to 1/4th of the intersection considering the surface profile. These parameters were not followed, if the dip influence is falling within the zone itself, and the dip length was simply extended up to the bottom RL of the intersected ore zone.

11.5 Specific Gravity/Bulk density calculation:

In order to estimate the resource of different grades of graphite associated with migmatised khondalite and quartzo-feldspathic gneiss, 5 samples have been subjected to physical analysis in a NABL Laboratory. The range of BD varies between 1.48 and 1.70, with an average bulk density of 1.606. This range corresponds to variations in the physical grades of the ore, reflective of their mineralogical and compositional differences. The detailed bulk density results are given below. (Annexure- XVIII)

Table 11.1:  Bulk density result of the Block

| Name of the Block | Sample mark | Test Result in kg/lt | Average Bulk density |
| --- | --- | --- | --- |
| Naringpanga Graphite Block | NBD1 | 1.62 | 1.606 |
|  | NBD2 | 1.63 |  |
|  | NBD3 | 1.70 |  |
|  | NBD4 | 1.48 |  |
|  | NBD5 | 1.60 |  |

11.6   Assumption for Resource Estimation

### Parameters Considered for Resource Computation:

- Depth Level: Resource has been estimated up to a depth level of 270.00 mRL (as the highest borehole drilled was 78.80 m, corresponding to a vertical depth of 55.72 m) to 335.52 mRL. The up dip and & down dip is taken at 50m on either side w.r.t the thickness of ore band/zone

- Grade Selection: For resource calculation, graphite samples with ≥ 2% FC were considered for grade estimation, adhering to the threshold limits prescribed by IBM.

- Recovery Factor: In the absence of beneficiation studies and time-series production data for the Taken over Lease, and the parameters of estimations in the adjacent auctioned block, an average recovery factor of 70% was assumed for graphite. This accounts for its distinct localization with sharp to gradational contacts, sampling errors, altered/weathered zones, geo-scientific factors, miniature silica veinlets, and thin host rock intercalations.

- Bulk Density: The average bulk density for graphite in the block was calculated as 1.606, derived from five representative graphite samples of different grades.

- Length of Influence: The halfway distance between two consecutive section lines was taken as the length of influence for resource estimation, with actual measurements used for peripheral section lines. In case of isolated BHs, an influence of 25-50 m was considered keeping in view the thickness of ore zone.

- Thin Intercalations and Sub-Grade Ore: In cases of thin intercalations and/or sub-grade ore within a high-grade/sub-grade ore band, the average assay value of the entire similar-quality ore zone in the borehole was used for consolidated grade correlation.

11.7: Resource Estimation

11.7.1 Resource estimation by Cross Sectional Area method:

The volume of the ore body was calculated using the cross-sectional area of each section and its respective length of influence. The length of influence for each section was determined based on the proximity to the block boundary and the spacing between adjacent sections. For Section A-A', the length of influence was 17.5 m on one side and 21.5 m on the other. Similarly, for Section B-B', the lengths of influence were 21.5 m on one side and 63.5 m on the other. Moving to Section C-C', the lengths of influence were calculated as 63.5 m on one side and 18.5 m on the other. For Section D-D', the corresponding lengths were 18.5 m and 47.5 m, respectively. In the case of Section E-E', the lengths of influence were measured as 48.0 m on one side and 50.0 m on the other. In addition to the cross-sectional analysis, Pit-2 was considered separately, with a uniform length of influence of 25.0 m on either side. The total volume for each section was then obtained by multiplying its cross-sectional area by the sum of its respective lengths of influence. The lithological units in the cross-sections were extrapolated laterally, proportionate to the thicknesses observed in the boreholes.

The resource estimation has been done by the following formula.

Resource (R) = V (Cross Sectional Area X Length of Influence) x BD x RF

Where R= Resource in mt, V = volume in M3,

BD= Bulk density (Determined), RF= Recovery factor 70% (Assumed)

Table 11.2: Section wise resource Calculation of Naringpanga (S) Graphite Block, Rayagada District. (A)

| Ore category | Section area( m2) | Length of influence(m) | Volume(m3) | Recovery Factor (%) | Bulk density | Resource(t) |
| --- | --- | --- | --- | --- | --- | --- |
|  | A-A' |  |  |  |  |  |
| FC (2-5%) | 2172.95 | 38 | 82572.1 | 70 | 1.606 | 92827.55 |
|  | B-B' |  |  |  |  |  |
| FC (2-5%) | 560.25 | 85 | 47621.25 | 70 | 1.606 | 53535.81 |
|  | C-C' |  |  |  |  |  |
| FC (2-5%) | 1211.37 | 82 | 99332.34 | 70 | 1.606 | 111669.40 |
|  | D-D' |  |  |  |  |  |
| FC (2-5%) | 1212.81 | 66 | 80045.46 | 70 | 1.606 | 89987.11 |
|  | E-E' |  |  |  |  |  |
| FC (2-5%) | 586.05 | 98 | 57432.9 | 70 | 1.606 | 64566.07 |
|  |  |  | 367004.05 | Total (A) |  | 412585.95 |

During excavation, graphite incidences have been encountered in Pit-2 for about 30 cm in the pit floor. So an influence of about 25 m was taken on all sides of the pit to calculate the inferred resource by surface area method.

Resource calculation by Surface area method for the panel around pit-2. (B)

| Ore category | Surface area( m2) | Thickness(m) | Volume(m3) | Recovery Factor (%) | Bulk density | Resource(t) |
| --- | --- | --- | --- | --- | --- | --- |
| FC (2-5%) | 2500 | 0.3 | 750 | 70 | 1.606 | 843.15 |
| Total resource of the Block (In metric tonnes): (A+B) in Metric Tonnes |  |  |  |  |  | 413429 |
| Total resource of the Block (In Million tonnes) in mt |  |  |  |  |  | 0.41343 |

Thus, geological resource estimation (G3) has been carried out for the Naringpanga block to evaluate the graphite resources. The estimation reveals a net geological resource of 0.41343 million tonnes of graphite of inferred category. This calculation accounts for the recoverable quantity after considering various limiting factors.

11.7.2 Resource estimation by Longitudinal-Vertical Panel method

The resources of graphite in the explored block have also been estimated by the Longitudinal-Vertical section method for better confidence level.  The LV panel method is discussed as follows.

(i) Strike influence:

The strike influence of the ore zone was taken at 50 % of the spacing between two positive bore holes and 25 %, when the adjacent bore hole is negative or if there is neither any borehole nor any surface evidence to substantiate the physical continuity of the ore zone. The third level borehole was generally drilled at more or less 200.0 m strike interval to check the depth persistence of the mineralization. When there is no borehole on  either side of a borehole the strike influence was extended to 50.0 m on either side considering the positive intersection of the shallower boreholes on the either side of the borehole.

(ii) Dip influence:

The dip influence of the ore zone was taken at 50 % of the difference in the centre of intersection of the ore body at two positive levels of two boreholes along the same profile line, however, this was not strictly followed when there is a variation of thickness along different levels and rule of gradual change was applied. If there is a single borehole along a profile, the dip length was extended only up to 1/4th of the intersection considering the surface profile. These parameters were not followed, if the dip influence is falling within the zone itself, and the dip length was simply extended up to the bottom RL of the intersected ore zone. When there is a single 1st level borehole in between two 1st and 2nd level boreholes the dip length for the 1st level boreholes was extended to a RL which is average between the two. The down dip length of the 1st level boreholes become the up dip length of the 2nd level boreholes on the either side of the 1st level. This was done in order to avoid overlapping of resources.

(iii) Preparation of Panel and calculation of Panel area:

The RL of Lode intersection along with up dip and down dip RL for the respective lodes taken from the cross section, was plotted on a Longitudinal- vertical section on an mm square graph sheet on 1:1000 scale. The length of the panel of lodes of a borehole was prepared based on the strike influence as discussed above whereas the breadth of the panel was constructed based on the up dip and down dip RL of the respective lodes. The panel area was calculated and multiplied with the horizontal width to get the volume of the ore lode of the borehole. The up dip RL of the 1st level boreholes have been kept at 290.0 mRL. The down dip RL of the 1st level boreholes is the up dip RL of the 2nd level boreholes. The horizontal width of the respective lodes was calculated by using the formula HW=TW / Sin of Ore dip. The following formulae is used for resource estimation:

Tonnage= Panel Area X Horizontal width X Tonnage factor (Bulk density)

Panel wise ore resources as computed are shown in the table (11.3)

| SL.NO | BHs. No | Lode No | From (m) | To (m) | CI (β) | Sin(β) | App. Width(m) | T.W | Lode Top RL(m) | Lode Bottom RL(m) | Up dip RL (m) | Down Dip RL(m) | Ind_Area | ∑(ind_Area) | V(total) | AREA OF LVP | ∑(Overlap_Area) | T.W (overlap) | V(overlap) | Vpanel | BD | Resource in tonnes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NPBH-1 | 1 | 12.00 | 14.00 | 55 | 0.82 | 2.00 | 1.64 | 322.91 | 321.54 | 327.10 | 320.40 | 495.41 | 6909.58 | 82574.25 | 3910.59 | 2998.99 | 11.95 | 35840.00 | 46734.25 | 1.606 | 75055.21 |
| 2 |  | 2 | 21.00 | 34.00 | 65 | 0.91 | 13.00 | 11.78 | 316.55 | 307.36 | 327.10 | 304.76 | 1653.11 |  |  |  |  |  |  |  |  |  |
| 3 |  | 3 | 43.50 | 44.30 | 65 | 0.91 | 0.80 | 0.73 | 300.64 | 300.07 | 309.49 | 298.00 | 850.47 |  |  |  |  |  |  |  |  |  |
| 4 |  | 4 | 52.10 | 69.50 | 65 | 0.91 | 17.40 | 15.77 | 294.55 | 282.26 | 327.10 | 274.25 | 3910.59 |  |  |  |  |  |  |  |  |  |
| 5 | NPBH-2 | 1 | 12.00 | 13.00 | 70 | 0.94 | 1.00 | 0.94 | 324.65 | 323.83 | 330.20 | 322.42 | 778.59 | 10122.42 | 26096.12 | 4311.62 | 5810.80 | 2.58 | 14980.5451 | 11115.58 | 1.606 | 17851.61 |
| 6 |  | 2 | 15.60 | 20.50 | 70 | 0.94 | 4.90 | 4.60 | 322.00 | 318.53 | 330.20 | 316.49 | 1792.65 |  |  |  |  |  |  |  |  |  |
| 7 |  | 3 | 26.50 | 27.00 | 70 | 0.94 | 0.50 | 0.47 | 314.29 | 313.93 | 324.65 | 311.37 | 1328.57 |  |  |  |  |  |  |  |  |  |
| 8 |  | 4 | 41.45 | 45.45 | 70 | 0.94 | 4.00 | 3.76 | 303.72 | 300.89 | 329.65 | 294.14 | 3551.19 |  |  |  |  |  |  |  |  |  |
| 9 |  | 5 | 50.45 | 51.70 | 70 | 0.94 | 1.25 | 1.17 | 297.35 | 296.47 | 318.01 | 291.30 | 2671.42 |  |  |  |  |  |  |  |  |  |
| 10 | NPBH-3 | 1 | 20.50 | 21.10 | 55 | 0.82 | 0.60 | 0.49 | 316.51 | 316.08 | 328.01 | 313.33 | 1894.92 | 9089.77 | 158799.59 | 7194.07 | 1895.70 | 17.47 | 33118.13391 | 125681.45 | 1.606 | 201844.41 |
| 11 |  | 2 | 48.60 | 71.95 | 70 | 0.94 | 23.35 | 21.94 | 296.71 | 280.13 | 328.01 | 272.29 | 7194.86 |  |  |  |  |  |  |  |  |  |
| 12 | NPBH-4 | 1 | 11.00 | 13.05 | 65 | 0.91 | 2.05 | 1.86 | 327.83 | 326.37 | 331.42 | 325.49 | 592.71 | 5794.12 | 55364.80 | 3622.57 | 2171.55 | 9.56 | 20749.87703 | 34614.93 | 1.606 | 55591.57 |
| 13 |  | 2 | 21.00 | 23.00 | 55 | 0.82 | 2.00 | 1.64 | 320.76 | 319.34 | 331.42 | 315.63 | 1711.35 |  |  |  |  |  |  |  |  |  |
| 14 |  | 3 | 31.00 | 49.00 | 55 | 0.82 | 18.00 | 14.74 | 313.68 | 300.95 | 331.42 | 296.52 | 3490.05 |  |  |  |  |  |  |  |  |  |
| 15 | NPBH-5 | 1 | 60.50 | 66.00 | 65 | 0.91 | 5.50 | 4.98 | 287.15 | 283.26 | 304.34 | 279.01 | 2503.87 | 4192.21 | 14555.53 | 3029.82 | 1162.39 | 3.47 | 4035.868116 | 10519.67 | 1.606 | 16894.58 |
| 16 |  | 2 | 73.50 | 75.00 | 55 | 0.82 | 1.50 | 1.23 | 277.96 | 276.89 | 290.77 | 273.69 | 1688.34 |  |  |  |  |  |  |  |  |  |
| 17 | NPBH-6 | 1 | 25.00 | 31.00 | 65 | 0.91 | 6.00 | 5.44 | 316.54 | 312.30 | 331.54 | 308.58 | 2509.22 | 4269.41 | 14442.37 | 3940.55 | 328.86 | 3.38 | 1112.43933 | 13329.93 | 1.606 | 21407.86 |
| 18 |  | 2 | 51.20 | 51.70 | 65 | 0.91 | 0.50 | 0.45 | 298.01 | 297.66 | 311.87 | 294.27 | 1760.19 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Total Resource in Metric Tonnes(t): |  |  |  | 388645.25 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Total Resource in Million Tonnes (mt): |  |  |  | 0.388645255 |

Table 11.3: Graphite Resource Estimation by LV Panel Method (2% Cut-off Grade).

A comparison of the resources estimated using the LV panel method and the cross-sectional area method reveals a difference of 0.0244 million tonnes which may be attributed to inhomogeneity of graphite mineralisation. In the present estimate, the cross sectional area method is accepted without any bias.

Cross checking has been done by the L-V panel method & the resource & grade has been compared.

11.8: Grade Estimation:

The average grade of graphite in the Naringpanga Block has been determined by analyzing all 160 core samples, of which 71 samples yielded positive analysis results (≥ 2%). These samples were selected due to their high graphite purity. The grade estimation process involves multiple steps to ensure accuracy and reliability.

Initially, the weighted average of key chemical constituents viz: including Alumina, M, SiO₂, Ash, Fixed Carbon (FC), and Loss on Ignition (L.O.I) was calculated for each core sample. This was achieved by multiplying the sample length by the percentage of each constituent within the sample.

Subsequently, the average grade for each borehole was computed using the weighted averages of all samples from the borehole. To extrapolate these results across the deposit, the average grade of each cross-section was calculated by weighing the borehole averages of all boreholes situated within that cross-section against the corresponding cross-sectional area.

Finally, the block-wise weighted average grade was determined by integrating the average grades from all cross-sections. This approach ensures that the grade estimation reflects the geological variability and distribution of graphite within the block.

Based on the content of Fixed Carbon (FC), the graphite in this block has been categorized into a single grade that is graphite with  FC range of 2–5%.

The summation of the tonnage for all the section lines for each ore type indicates the total tonnage of the entire block. The average grade for that tonnage is calculated initially by calculating the weighted average grade for individual borehole and again for total number of boreholes falling in a particular cross section, then the average grade for the total tonnage of the block is obtained by calculating the weighted average grade of all sections to represent the grade of graphite. The final resource has been calculated separately and represented under a single grade, namely F.C. (2-5%), for this block.

- In similar fashion, average grade of that particular ore type is to be calculated for other boreholes lying in the same cross section.

- Thus, weighted average for a particular grade strip lying in the same cross section, Gw = (G1T1 + G2T2 + …… + GnTn) / (T1 + T2 + …… + Tn); where, G1-Average grade of a borehole lying in a cross section, T1 -Total thickness of the corresponding borehole.

- Then the tonnage is calculated by multiplying the strike influence w.r.t. the cross-sectional area of strip and bulk density. i.e., Tonnage or mass, M = A*S*d; where, A - cross sectional area of a particular strip, S - strike influence of the corresponding strip, and d -bulk density.

- Now the weighted average grade for the whole block of a particular ore type, Ga= (Gw1*M1 + Gw2*M2 + …… + Gwn*Mn) / (M1 + M2 + …… + Mn) ; where Gw1= weighted average grade of an individual cross section strip, M1 - Tonnage or mass of the respective strip of that section.

- The lateral extension has been considered for resource assessment to a maximum influence of 50% of grid spacing of the probe point depending on geological considerations supplemented by geological continuity by mapping.

Section wise grade calculation of different categories of graphite of Naringpanga Block is furnished below:

Table 11.4: Section wise Graphite average FC content

| Section wise Graphite grade average |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Borehole | Average (%) |  |  |  |  |  |  |  |
|  |  | M | Al2O3 | SiO2 | Ash | LOI | F.C | F.C weighted avg. |  |
| Section-AA’ | NPBH-1 | 1.81 | 11.06 | 67.74 | 89.35 | 8.41 | 3.7 | 3.21 |  |
|  | Section average | 1.81 | 11.06 | 67.74 | 89.35 | 8.41 | 3.7 | 3.21 |  |
| Section-B-B’ | NPBH -2 | 1.32 | 13.31 | 67.8 | 95.87 | 7.76 | 2.21 | 2.19 |  |
|  | Section average | 1.32 | 13.31 | 67.8 | 95.87 | 7.76 | 2.21 | 2.19 |  |
| Section-C-C’ | NPBH -4 | 1.51 | 8.93 | 71.37 | 92.02 | 10.40 | 4.58 | 3.34 |  |
|  | Section average | 1.51 | 8.93 | 71.37 | 92.02 | 10.40 | 4.58 | 3.34 |  |
| Section-D-D’ | NPBH -3 | 1.18 | 14.77 | 62.21 | 90.00 | 7.08 | 2.35 | 2.27 |  |
|  | Section average | 1.18 | 14.77 | 62.21 | 90.00 | 7.08 | 2.35 | 2.27 |  |
| Section-E-E’ | NPBH -5 | 1.19 | 12.08 | 60.12 | 84.87 | 6.04 | 2.20 | 2.52 |  |
|  | NPBH -6 | 4.42 | 16.48 | 68.12 | 95.20 | 12.49 | 3.01 | 2.63 |  |
|  | Section average | 14.28 | 64.12 | 90.04 | 9.26 | 2.60 | 2.80 | 2.57 |  |
| Graphite Average of PIT-2 |  |  |  |  |  |  |  |  |  |
|  |  | M | Al2O3 | SiO2 | Ash | LOI | F.C | F.C weighted avg. |  |
| PIT-2 | Average | 1.96 | 19.44 | 50.20 | 87.40 | 12.60 | 2.82 | 2.82 |  |

Resource and Grade

Table 11.5: Summary of Resource Calculation of Naringpanga Block

| Cross Section method(A) | Graphite in metric tonnes (t) |
| --- | --- |
| A-A' | 92827.55 |
| B-B' | 53535.81 |
| C-C' | 111669.40 |
| D-D' | 89987.11 |
| E-E' | 64566.07 |
| Total (A) | 412585.95 |
| Surface area method(B) |  |
| PIT-2 | 843.15 |
| Grand Total Resource in Metric Tonnes(t): (A+B) | 413429.1 |
| Grand Total Resource in Million Tonnes (mt) | 0.41343 |

Table 11.6 : Average Grade & Resource of graphite of Naringpanga block

| Ore | Resource in (t) | Average Grade |
| --- | --- | --- |
| Graphite F.C (2-5%) | 0.41343 | 3.01% |

So, the total estimated resource of graphite across all grades in the Naringpanga Block is 0.41343 million tonnes, with an average grade of 3.01% Fixed Carbon (FC). This estimation reflects the block's graphite resource potential, with variations in quantity and quality observed across different sections. The resource distribution highlights that section C-C’ holds the maximum resource of 111669.40 t, indicating a concentration of graphite mineralization in this area. In contrast, Pit-2 has the minimum resource of 843.15 t, reflecting limited graphite occurrence in this part of the block.

Overburden:

Overburden has been calculated from the cross-sections. The overburdens comprise khondalite, quartzite, felspathic gneiss, soil and alluvium. The thickness of overburden varies from 3.70 m to 24.00 m. Ore overburden ratio calculated is 1:0.546.

Implications for Resource Utilization:

The low-grade, high-silica nature of the ore limits its direct industrial utility, necessitating cost-intensive processing methods. Moreover, silica-rich ores can pose environmental concerns, as mining and beneficiation may result in tailings disposal issues and potential pollution from silica and other associated impurities infiltrating the local ecosystem.

Efforts to sustainably utilize the resources from the Naringpanga Block must focus on improving beneficiation techniques, optimizing resource recovery, and mitigating environmental impacts. By addressing these challenges, the potential of graphite resources in the block can be maximized for industrial and commercial purposes.

11. 9 Category of resources as per MEMC, 2015 along with UNFC classification

The exploration over the Naringpanga block was carried out as per Part III of MEMC and resource has been reported as per Part-IV A of MEMC rules, 2015.

Economic axis (E): The quantity of Graphite in tonnes with average grade has been estimated by preliminary Exploration. The resource identified is of intrinsically economic (E3) interest and economic viability of the resource is to be further ascertained through a pre-feasibility study by application of appropriate modifying factors. Therefore, the feasibility stage can be of E3 category.

Feasibility Axis (F): A preliminary and general geological study involving delineation of lithological units, mineralogical and chemical data and its interpretation and assessment of mineral resource with quantity and grade have been undertaken during the exploration. The data related to topographical setting, nature of land and infrastructure roads, railways, electricity, water etc. have been acquired. The mineralization was defined based on the IBM’s threshold value of graphite. No feasibility studies, encompassing mining, Environmental Impact Assessment (EIA), infrastructures, costing of mining, investment decision of the deposit were worked out.  Therefore, the feasibility stage can be of F3 category. Hence, the block is covered under F3 coding in the Feasibility Axis.

Geological axis (G): General exploration was conducted in Naringpanga block following the exploration norms and technological inputs comprising surface geological mapping and topographical survey at 1:2000 scales, geophysical prospecting, core drilling at located BH points, systematic core sampling at 1m interval, chemical analysis, petrographic and mineralogical studies etc., Trace element analysis to decipher the extent of mineralization and to evaluate mineral quantity and quality. Hence the geological axis is in compliance to the G3 level of exploration.

The estimated mineral resource has been assigned as inferred mineral resource (333) of as per Mineral (Evidence and Mineral Content) Rules 2015 & (Amendment) 2021.

CHAPTER-12

CORE PRESERVATION

G-3 level of exploration in Naringpanga Graphite block achieved a drilling meterage of 398 m in six inclined boreholes. The recovered cores are preserved in GI core boxes with partitions. All core boxes are properly leveled with BH No., Length of cores preserved with an arrow mark from top to bottom. (Fig. 32)

Fig. 32:  Preservation of drilled cores in GI Core box

Thus, all the litho cores are systematically preserved in 83 GI core boxes of 3m x 1m size with inner partitions. All the core boxes are now available in the Zonal Office, the Directorate at Koraput. Details of the core boxes are given in Table 12.1.

Table: 12.1-: Deatails of Cores and core boxes preserved

| Sl No | BH No | Used No of Core Boxes | Total Cores (in m) | Core box dimension (L X W) |
| --- | --- | --- | --- | --- |
| 1 | NPBH-1 | 14 | 70.00m | 1.00 m X 0.30m |
| 2 | NPBH-2 | 12 | 55.00m | 1.00 m X 0.30m |
| 3 | NPBH-3 | 16 | 74.50m | 1.00 m X 0.30m |
| 4 | NPBH-4 | 12 | 56.50m | 1.00 m X 0.30m |
| 5 | NPBH-5 | 16 | 78.80m | 1.00 m X 0.30m |
| 6 | NPBH-6 | 13 | 63.20m | 1.00 m X 0.30m |
|  | Total | 83 | 398.00 m |  |

CHAPTER-13

CONCLUSION AND RECOMENDATIONS

Conclusion:

The Naringpanga graphite block, featured T.S. No.  E44F10 (65 M/10), has been covered at G3-level exploration over an area of 0.143 sq. km with approval of NMET through topographic survey, detailed geological mapping, limited pitting & trenching at 4 locations, self-potential geophysical survey, drilling of six angle boreholes and sampling of cores followed by laboratory analysis. Detailed geological mapping of the area on 1:2000 scale, revealed that the area is mostly soil covered and graphite bearing gneisses & schists are exposed only in the Nala bed flowing in the western margin of the block.

The Naringpanga area lies within the Eastern Ghats Mobile Belt, a region known for its complex geological and tectono-metamorphic history. The lithological framework comprises migmatized khondalite, quartzo-feldspathic gneiss, leptynite, and mylonite, which host the graphite mineralization. The mineralization is litho-structurally controlled, occurring along the foliation planes of the host rocks as well as in conformity to the sympathetic lineaments of the Tumudibandha major shear. Secondary silicification, mylonitisations, dragging of foliations, slickensides and quartz overgrowths further highlight the complex post-metamorphic history of the area. The occurrence of secondary graphite enrichments and associated mineral assemblages’ points to multi-phase deformations & mineralization events.

Surface geophysical investigations, Self-Potential (SP) surveys, detected distinct negative anomalies ranging from -0.04 mV to -154.6 mV. These anomalies correlate with areas of high graphite potential, particularly in the central and eastern parts of the block, which align with the regional NW-SE trend of the geological formations. Drilling of six inclined boreholes totaling 398 meters and trial excavations in 4 locations with interception of graphite in two trial pits provided critical insights into the block’s geological and mineralogical characteristics. The investigations revealed 84.60 meters of mineralized zones, with an average thickness of 14.05 meters. Notably, the shallowest borehole (NPBH-2) intersected 9.65 meters of graphite mineralization, while the deepest (NPBH-5) revealed 5.5 meters, confirming the presence of amorphous graphite as fine-grained, flaky disseminations. The fixed carbon content in the mineralized zones ranged between 2% and 10.03%, establishing the deposit's potential for commercial exploitation.

Mineralogical assemblages revealed retrograde metamorphic features indicative of granulite facies metamorphism followed by retrogression. Observations included biotite growth along garnet fractures, feldspar saussuritization, and the development of secondary minerals like pyrite, epidote, and garnet. These features point to a dynamic tectono-metamorphic environment marked by partial melting, recrystallization, and fluid activity. Such processes have significantly influenced the distribution and concentration of graphite within the host rocks. Geochemical analysis further enriched the understanding of the area’s mineralization. The mineralization occurs as bands, pockets, and disseminations within the host rock, lying beneath a thick alluvial cover. Structural complexities such as mylonitization and foliation patterns have played a significant role in localizing the graphite mineralization, with variability in thickness and grade observed across the boreholes. This heterogeneity underscores the need for targeted exploration at a finer scale to delineate the mineralization more accurately. The study revealed rare earth element (REE) enrichment, with higher concentrations of light REEs (LREEs) such as cerium and lanthanum, alongside trace elements like vanadium, strontium, zirconium, and rubidium. These geochemical signatures reflect the effects of partial melting and recrystallization associated with the tectonic and metamorphic processes in the region. The presence of rare metals, including hafnium and tantalum, as well as uranium and thorium, are of concern and their potentialities can be ascertained after beneficiation during higher stages of exploration.

The estimated geological resource of the block by cross sectional methods is about 0.403 million tonnes with average fixed carbon content of 3.01%, which is of inferred category

Based on the findings, the Naringpanga graphite ore block demonstrates substantial potential for commercial graphite extraction, subject to further refinement through G2-level exploration. This next phase of exploration will involve more detailed geological, geophysical, and geochemical studies, as well as additional drilling to confirm the continuity and grade of the mineralization. It will also facilitate the preparation of a comprehensive mining plan that aligns with sustainable practices and economic viability.

The current exploration activities in the block reveals that out of the total block area of 14.3 Ha (0.143 Sq.Km), 0.078 sq. km is identified as mineralized, while the remaining 0.065 sq. km consists of non-mineralized zones at the peripheral areas of the block. These non-mineralized regions offer potential for mining infrastructure and other developmental activities, including facilities for ore processing, waste management, and ancillary activities. Proper utilization of these areas can significantly enhance the operational efficiency and economic feasibility of mining in the region.  The resource thus estimated can be classified under UNFC 333 and MEMC 2015& 2021.

Recommendations:

The geological investigation of the Naringpanga graphite block has successfully identified a promising graphite resource within the Eastern Ghats Mobile Belt. The integrated geological, geophysical, and geochemical studies offer a clear understanding of the deposit's characteristics and potential. These findings emphasize the need for advanced exploration to fully realize the resource's potential, ensuring sustainable and viable extraction. The study underscores Naringpanga's potentiality and it is recommended to upgrade the resource and grade estimation upto G 2 level for higher confidence level.

CHAPTER-14

EXPENDITURE

The Naringpanga graphite Project was approved in the  27th  EC of NMET with an estimated cost of  Rs 120.93 lakhs. The expenditure incurred by the Department so far is ₹61,78,802/- , which doesnot include the Salary components of the Officials engaged in the execution of the Project and In-house laboratory Charges. The Drilling, Topographic Survey and geo-physical survey components have been Out-Sourced.

The details of expenditure incurred are furnished below-:

Total approved cost: RS 1,20,92,849 /-

Total amount received: Rs 96,74,280/-

Total Expenditure of Naringapanga Graphite Project, Rayagada Dist

| In house expenses Field expenses | ₹6,34,161/- |
| --- | --- |
| Laboratory expenses | ₹3,05,839/- |
| The outsourcing agency, M/s Geo Environmental Services has been paid against the Topographical Survey Work | ₹2,01,780/- |
| Against the Drilling of 06 no's of Bore Holes | ₹44,85,062/- |
| Against the Geo Physical Work | ₹5,56,960 /- |
| Total amount against the outsourcing agency | ₹52,43,802/- |
| Total expenditure | ₹61,78,802/- |

CHAPTER-15

REFERENCES

Behera, K. K. & Mohakul, J. P. (2024). Delineating the Lithotectonic Domains in the Eastern Ghats Mobile Belt: Implication for the Regional exploration of the Metasediment hosted Graphite and Manganese ore horizons; Jour. Geol. Soc. India, Vol. 100(12) pp. 1741-1754

Chakraborty, A.K. & Lal, N. Report on the geophysical investigation for regional assessment of graphite resources, Koraput district, GSI, Odisha.

Chetty, T.R.K. (2001). The Eastern Ghats Mobile Belt, India: A collage of juxtaposed terranes (?). Gondwana Research, 4(3), pp. 319–328.

Chetty, T.R.K. & Murthy, D.S.N. (1998). Regional tectonic framework of the Eastern Ghats mobile belt: A new interpretation. GSI Special Publication No. 44, pp. 39–50.

Dobmeier, C. & Raith, M.M. (2003). Crustal architecture and evolution of the Eastern Ghats Belt and adjacent regions of India. In: M. Yoshida, B.F. Windley, and S. Dasgupta (Eds.), Proterozoic East Gondwana: Supercontinent Assembly and Breakup, Geological Society, London, Special Publication, Vol. 206, pp. 145–168.

Geology and Mineral Resources of Orissa (2006). pp. 277–292.

Mahalik, N.K. (1995). Geology of the contact between Eastern Ghats Belt and North Orissa Craton, India. Journal of the Geological Society of India, Vol. 44, pp. 41–52.

Parida, S. N., Mishra, P. C.   (1991-92) Unpublished Report on investigation for graphite around Bhaliapadar area of Koraput district, Directorate of Mines & Geology, Government of Odisha, pp. 1–10

Mohanty, B.C. & Vajani, P.C. (1982–83). Unpublished Report on investigation for graphite around Solagudi and Naringpanga area of Koraput district, Directorate of Mines & Geology, Government of Odisha, pp. 1–10.

Mohanty, B.C. & Vajani, P.C. (1983–85). Unpublished Report on investigation for graphite around Ambadola-Jagdalpur, Gunupur Sub-Division of Koraput district, Directorate of of Mines 7 Geology, Government of Odisha, pp. 9–15.

Nanda, J.K. (1995). Boundary relation of Eastern Ghats Mobile Belt. Symposium on India and Antarctica during the Precambrian and crustal processes in East Gondwana, Visakhapatnam, pp. 34–35 (abstract).

Nanda, J.K. & Pati, U.C. (1989). Field relations and petrochemistry of the granulites and associated rocks in the Ganjam-Koraput sector of the Eastern Ghats belt. Indian Minerals, Vol. 43, pp. 247–264.

Nash, C.R., Rankin, L.R., Leeming, P.M., & Harris, L.B. (1996). Delineation of lithostructural domains in northern Orissa (India) from Landsat Thematic Mapper imagery. Tectonophysics, 260(3–4), 245–257.

Parida, S.N. & Pani, D.N. (1983–84). Unpublished Report on investigation for graphite around Sikarpai, Bandhamandi & Minakhunti in Rayagada Sub-Division of Koraput District, Directorate of Mines & Geology, Government of Odisha, pp. 3–6.

Pasayat, S. & Adhikari, K.N. (1986–87). Report on the assessment of graphite resources in parts of Koraput and Phulbani districts, GSI, Odisha.

Patnaik, K. & Mishra, P.C. (1990–91). Unpublished Report on investigation for graphite around Maguni in Gunupur Sub-Division of Rayagada District, Directorate of Mines & Geology, Government of Odisha, pp. 11–16.

Ramakrishnan, M., Nanda, J.K., & Augustine, P.F. (1998). Geological evolution of the Proterozoic and Eastern Ghats Mobile Belt. GSI Special Publication No. 44, pp. 1–2.

Rickers, K., Mezger, K., & Raith, M.M. (2001). Evolution of the continental crust in the Proterozoic Eastern Ghats Belt, and new constraints for Rodinia reconstruction: implications from Sm–Nd, Rb–Sr and Pb–Pb isotopes. Precambrian Research, Vol. 112, pp. 183–212.

Sahoo, D.K. & Rauta, B.S. (2016–2017). Unpublished Report of detailed exploration around Naringpanga, Rayagada District, Directorate of Geology, Government of Odisha.

Sen, S.K., Bhattacharya, S., & Acharyya, A. (1995). A multistage pressure-temperature record in the Chilka Lake granulites: the epitome of metamorphic evolution of the Eastern Ghats, India. Journal of Metamorphic Geology, Vol. 13, pp. 287–298.

SGPB Agenda Volume (1987 and 1989).

Sinha, R.K. & Sharma, N.L. Mineral Economics, pp. 198–222.

CHAPTER-16

LOCALITY INDEX

| Sl. No. | Name of the Village | Latitude | Longitude |
| --- | --- | --- | --- |
| 1 | Naringpanga | 19o43'26.33''N | 83o32'46.68''E |
| 2 | Solagudi | 19o43'51.64''N | 83o32'01.63''E |
| 3 | Kalupadar | 19o42'10.55"N | 83o32'09.67"E |
| 4 | Muniguda | 19°35'21.47"N | 83°30'25.88"E |
| 5 | Sano Mudra | 19°43'00.32" N | 83°34' 15.00"E |
| 6 | Jagdalpur | 19°45'49.30"N | 83°32'56.30"E |
| 7 | Kumdabali | 19°40'06.08" N | 83°32'10.56" E |

ABBREVIATIONS USED

| SL. No. | Abbreviation | Full form |
| --- | --- | --- |
| 1 | m | Meter |
| 2 | Cu m | Cubic Meter |
| 3 | mt | Million Tonnes |
| 4 | t | Metric tonne |
| 5 | RL | Reduced Level |
| 6 | mRL | Meter Reduced Level |
| 7 | MSL | Mean Sea Level |
| 8 | UNFC | United Nations Frame work Classification |
| 9 | IBM | Indian Bureau of Mines |
| 10 | GSI | Geological Survey of India |
| 11 | BF | Blast Furnace |
| 12 | NMET | National Mineral Exploration Trust |
| 13 | TCC | Technical cum Cost Committee |
| 14 | EC | Executive Committee |
| 15 | MMDR | Mines & Minerals (Development and Regulation) |
| 16 | MEMC | Minerals (Evidence of Mineral Contents) |
| 17 | DoMG | Directorate of Mines and Geology, Odisha |
| 18 | NABL | National Accreditation Board for Testing and Calibration Laboratories |
| 19 | MECL | Mineral Exploration and Consultancy Limited |
| 20 | QA/QC | Quality Assessment/Quality Checks |
| 21 | WGS-84 | World Geodetic System-84 |
| 22 | DMS | Degree Minute Second |
| 23 | UTM | Universal Transverse Mercator |
| 24 | F.S.P. | Field Season Programme |
| 25 | DGPS | Differential Global Positioning System |
| 26 | GPS | Global Positioning System |
| 27 | T.S. No. | Toposheet Number |
| 28 | ppm | Parts per million |
| 29 | F.C. | Fixed Carbon |
| 30 | ML | Mining Lease |
| 31 | TSO | Technical Supervising officer |
| 32 | SP | Self-Potential |
| 33 | BRS | Bed Rock Sample |
| 34 | LOI | Loss of Ignition |
| 35 | EGMB | Eastern Ghats Mobile Belt |
| 36 | XRF | X-ray Fluorescence |
| 37 | XRD | X-ray Diffraction |
| 38 | ICP-MS | Inductively Coupled Plasma Mass Spectrometry |
| 39 | E.I.C | Exploration In-Charge |
| 40 | BHs. | Bore holes |
| 41 | ORG | Ocean Ridge Granites |
| 42 | VAG | Volcanic Arc Granites |
| 43 | WPG | Within-Plate Granites |
| 44 | COLG | Collision Granites |
| 45 | TC | Tungsten Carbide |
| 46 | RM | Rare Metal |
| 47 | HREE | Heavy Rare Earth Element |
| 48 | LREE | Light Rare Earth Element |
| 49 | REE | Rare Earth Element |

ANNEXURE-I

LOCALITY INDEX

| Sl. No. | Name of the Village | Latitude | Longitude |
| --- | --- | --- | --- |
| 1 | Naringpanga | 19o43'26.33''N | 83o32'46.68''E |
| 2 | Solagudi | 19o43'51.64''N | 83o32'01.63''E |
| 3 | Kalupadar | 19o42'10.55"N | 83o32'09.67"E |
| 4 | Muniguda | 19°35'21.47"N | 83°30'25.88"E |
| 5 | Sano Mudra | 19°43'00.32" N | 83°34' 15.00"E |
| 6 | Jagdalpur | 19°45'49.30"N | 83°32'56.30"E |
| 7 | Kumdabali | 19°40'06.08" N | 83°32'10.56" E |

| ANNEXURE -II |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical & Chemical Assay log of NPBH-1 (Naringpanga Graphite Block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bore Hole No- | NPBH-1 |  |  |  |  |  |  |  | Date of Commencement |  |  |  |  | 13.07.2024 |  |
| Location( DGPS) | 767689.2503 |  |  |  |  | 2183040.786 |  |  | Date of Completion |  |  |  |  | 30.07.2024 |  |
| (DMS) | 83°33'14.31000" E |  |  |  |  | 19°43'29.21880" N |  |  | Total Graphite Thickness |  |  |  |  | 24.55 m |  |
| Hole depth | 70.00 m |  |  |  |  |  |  |  | Total Recovery of Core |  |  |  |  | 67.28 m |  |
| R.L | 331.40 m |  |  |  |  |  |  |  | Percentage of Core Recovery |  |  |  |  | 96.11% |  |
| Water table depth | 15.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sl. NO | Sample ID |  | From |  | To | ASH % | M% | V.M% | LOI% | SiO2 | Al2O3% | V2O5% | F.C% | Length | Lithology |
| 1 |  |  | 0.00 |  | 6.00 |  |  |  |  |  |  |  |  | 6.00 | Soil |
| 2 |  |  | 6.00 |  | 12.00 |  |  |  |  |  |  |  |  | 6.00 | Quartzofeldspathic Gneiss ± Garnet |
| 3 | NPG-1/1 |  | 12.00 |  | 13.00 | 88.63 | 1.15 | 2.38 | 11.37 | 80.78 | 2.58 | 0.00 | 7.84 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 4 | NPG-1/2 |  | 13.00 |  | 14.00 | 89.77 | 0.71 | 1.93 | 10.23 | 82.87 | 2.72 | 0.00 | 7.59 | 1.00 |  |
| 5 |  |  | 14.00 |  | 21.00 |  |  |  |  |  |  |  |  | 7.00 | Quartzofeldspathic Gneiss ± Garnet |
| 6 | NPG-1/3 |  | 21.00 |  | 22.00 | 86.59 | 2.97 | 3.68 | 13.41 | 68.94 | 5.58 | 0.00 | 6.76 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 7 | NPG-1/4 |  | 22.00 |  | 23.00 | 89.08 | 2.70 | 4.31 | 10.93 | 72.80 | 5.32 | 0.00 | 3.92 | 1.00 |  |
| 8 | NPG-1/5 |  | 23.00 |  | 24.00 | 91.38 | 2.11 | 3.99 | 8.63 | 75.94 | 6.68 | 0.01 | 2.53 | 1.00 |  |
| 9 | NPG-1/6 |  | 24.00 |  | 25.00 | 90.32 | 1.24 | 3.66 | 9.69 | 74.92 | 7.09 | 0.01 | 4.79 | 1.00 |  |
| 10 | NPG-1/7 |  | 25.00 |  | 26.00 | 85.49 | 4.68 | 5.73 | 14.50 | 54.57 | 15.80 | 0.01 | 4.10 | 1.00 |  |
| 11 | NPG-1/8 |  | 26.00 |  | 27.00 | 85.55 | 1.79 | 5.41 | 14.45 | 61.66 | 12.77 | 0.00 | 7.25 | 1.00 |  |
| 12 | NPG-1/9 |  | 27.00 |  | 28.00 | 93.91 | 1.65 | 1.01 | 6.10 | 74.90 | 8.42 | 0.00 | 3.44 | 1.00 |  |
| 13 |  |  | 28.00 |  | 28.30 |  |  |  |  |  |  |  |  | 0.30 |  |
| 14 | NPG-1/10 |  | 28.30 |  | 29.00 | 94.29 | 1.38 | 1.06 | 5.72 | 68.95 | 14.09 | 0.00 | 3.28 | 0.70 |  |
| 15 | NPG-1/11 |  | 29.00 |  | 30.00 | 94.43 | 1.01 | 1.09 | 5.58 | 71.40 | 14.42 | 0.00 | 3.48 | 1.00 |  |
| 16 | NPG-1/12 |  | 30.00 |  | 31.00 | 94.10 | 1.42 | 0.98 | 5.91 | 70.28 | 13.82 | 0.00 | 3.51 | 1.00 |  |
| 17 |  |  | 31.00 |  | 31.85 |  |  |  |  |  |  |  |  | 0.85 |  |
| 18 | NPG-1/13 |  | 31.85 |  | 32.85 | 91.12 | 2.40 | 4.08 | 8.88 | 64.76 | 13.31 | 0.00 | 2.40 | 1.00 |  |
| 19 | NPG-1/14 |  | 32.85 |  | 34.00 | 91.18 | 2.33 | 1.12 | 8.82 | 66.62 | 13.75 | 0.00 | 5.37 | 1.15 |  |
| 20 |  |  | 34.00 |  | 42.50 |  |  |  |  |  |  |  |  | 8.50 | Quartzofeldspathic Gneiss ± Garnet |
| 21 | NPG-1/15 |  | 42.50 |  | 43.50 | 94.56 | 1.79 | 2.42 | 5.44 | 64.52 | 16.20 | 0.00 | 1.23 | 1.00 |  |
| 22 | NPG-1/16 |  | 43.50 |  | 44.30 | 92.46 | 2.31 | 3.04 | 7.54 | 63.27 | 17.76 | 0.00 | 2.19 | 0.80 | Quartzofeldspathic Gneiss + Graphite |
| 23 |  |  | 44.30 |  | 45.15 |  |  |  |  |  |  |  |  | 0.85 | Quartzofeldspathic Gneiss ± Garnet |
| 24 | NPG-1/17 |  | 45.15 |  | 45.65 | 95.44 | 1.12 | 2.33 | 4.56 | 69.05 | 13.16 | 0.00 | 1.11 | 0.50 |  |
| 25 |  |  | 45.65 |  | 47.50 |  |  |  |  |  |  |  |  | 1.85 |  |
| 26 |  |  | 47.50 |  | 52.10 |  |  |  |  |  |  |  |  | 4.60 |  |
| 27 | NPG-1/18 |  | 52.10 |  | 53.10 | 92.30 | 1.75 | 3.95 | 7.70 | 63.24 | 17.09 | 0.00 | 2.00 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 28 | NPG-1/19 |  | 53.10 |  | 54.10 | 94.64 | 1.67 | 2.37 | 5.36 | 66.04 | 16.18 | 0.00 | 1.32 | 1.00 | Quartzofeldspathic Gneiss ± Garnet |
| 29 | NPG-1/20 |  | 54.10 |  | 55.10 | 92.40 | 2.13 | 3.40 | 7.60 | 67.35 | 13.56 | 0.00 | 2.07 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 30 | NPG-1/21 |  | 55.10 |  | 56.35 | 93.94 | 1.18 | 3.31 | 6.06 | 72.53 | 9.74 | 0.00 | 1.57 | 1.25 | Quartzofeldspathic Gneiss ± Garnet |
| 31 | NPG-1/22 |  | 56.35 |  | 57.35 | 92.98 | 1.48 | 2.95 | 7.02 | 68.21 | 12.90 | 0.00 | 2.59 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 32 | NPG-1/23 |  | 57.35 |  | 58.30 | 94.10 | 1.43 | 2.39 | 5.90 | 72.24 | 10.41 | 0.00 | 2.08 | 0.95 |  |
| 33 |  |  | 58.30 |  | 59.00 |  |  |  |  |  |  |  |  | 0.70 | Quartzofeldspathic Gneiss ± Garnet |
| 34 | NPG-1/24 |  | 59.00 |  | 60.00 | 93.28 | 1.90 | 2.89 | 6.72 | 68.30 | 12.69 | 0.00 | 1.93 | 1.00 |  |
| 35 | NPG-1/25 |  | 60.00 |  | 61.00 | 92.98 | 1.66 | 3.18 | 7.02 | 67.34 | 12.88 | 0.00 | 2.18 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 36 | NPG-1/26 |  | 61.00 |  | 62.00 | 93.08 | 1.72 | 3.29 | 6.92 | 69.16 | 11.01 | 0.00 | 1.91 | 1.00 |  |
| 37 | NPG-1/27 |  | 62.00 |  | 63.30 | 90.40 | 2.48 | 4.25 | 9.60 | 61.94 | 13.89 | 0.00 | 2.87 | 1.30 |  |
| 38 |  |  | 63.30 |  | 64.00 |  |  |  |  |  |  |  |  | 0.70 | Granulite |
| 39 |  |  | 64.00 |  | 65.00 |  |  |  |  |  |  |  |  | 1.00 |  |
| 40 | NPG-1/28 |  | 65.00 |  | 65.80 | 93.02 | 2.26 | 0.00 | 6.98 | 65.94 | 13.76 | 0.00 | 1.31 | 0.80 | Quartzofeldspathic Gneiss ± Garnet |
| 41 |  |  | 65.80 |  | 66.00 |  |  |  |  |  |  |  |  | 0.20 |  |
| 42 |  |  | 66.00 |  | 67.00 |  |  |  |  |  |  |  |  | 1.00 |  |
| 43 | NPG-1/29 |  | 67.00 |  | 68.00 |  |  |  |  |  |  |  |  | 1.00 |  |
| 44 | NPG-1/30 |  | 68.00 |  | 69.00 | 94.50 | 0.98 | 1.95 | 5.50 | 67.06 | 14.21 | 0.00 | 2.57 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 45 | NPG-1/31 |  | 69.00 |  | 69.50 | 92.52 | 1.04 | 2.30 | 7.48 | 73.73 | 11.50 | 0.00 | 4.14 | 0.50 |  |
| 46 |  |  | 69.50 |  | 70.00 |  |  |  |  |  |  |  |  | 0.50 | Granulite+Garnet |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70.00 |  |

| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bore Hole No- | NPBH-2 |  |  |  |  | Date of Commencement |  |  |  |  | 03.05.2024 |
| Location( DGPS) | 767778.4465 |  |  | 2183058.278 |  | Date of Completion |  |  |  |  | 19.05.2024 |
| (DMS) | 83°33'17.37720" E |  |  | 19°43'29.74440" N |  | Total Graphite Thickness |  |  |  |  | 9.65 m |
| Hole depth | 55.00 m |  |  |  |  | Total Recovery of Core |  |  |  |  | 50.61 m |
| R.L | 333.31 m |  |  |  |  | Percentage of Core Recovery |  |  |  |  | 92.02% |
| Water table depth | 15.00 m |  |  |  |  |  |  |  |  |  |  |
| Sl. NO | Sample ID | From | To | ASH % | M% | LOI% | SiO2 | Al2O3% | F.C% | Length | Lithology |
| 1 |  | 0.00 | 4.00 |  |  |  |  |  |  | 4.00 | Soil |
| 2 |  | 4.00 | 6.20 |  |  |  |  |  |  | 2.20 | Khondalite |
| 3 | NPG - 2/1 | 6.20 | 7.20 | 93.16 | 2.10 | 6.84 | 67.86 | 9.54 | 0.79 | 1.00 |  |
| 4 | NPG - 2/2 | 7.20 | 8.20 | 90.44 | 3.51 | 9.06 | 62.86 | 11.28 | 0.96 | 1.00 |  |
| 5 | NPG - 2/3 | 8.20 | 9.50 | 92.64 | 2.08 | 7.36 | 65.58 | 8.74 | 1.20 | 1.30 |  |
| 6 |  | 9.50 | 12.00 |  |  |  |  |  |  | 2.50 |  |
| 7 | NPG - 2/4 | 12.00 | 13.00 | 91.80 | 1.55 | 8.20 | 63.02 | 14.44 | 2.04 | 1.00 | Khondalite+ Graphite |
| 8 | NPG - 2/5 | 13.00 | 13.50 | 93.92 | 1.27 | 6.08 | 66.18 | 12.42 | 1.20 | 0.50 | Quartzofeldspathic Gneiss ± Garnet |
| 9 |  | 13.50 | 15.60 |  |  |  |  |  |  | 2.10 |  |
| 10 | NPG - 2/6 | 15.60 | 16.50 | 91.30 | 1.91 | 8.70 | 64.90 | 12.94 | 2.66 | 0.90 | Khondalite+ Graphite |
| 11 |  | 16.50 | 17.50 |  |  |  |  |  |  | 1.00 | Khondalite |
| 12 | NPG - 2/7 | 17.50 | 18.00 | 92.30 | 1.80 | 7.70 | 67.20 | 10.72 | 2.14 | 0.50 | Khondalite+ Graphite |
| 13 |  | 18.00 | 19.00 |  |  |  |  |  |  | 1.00 | Khondalite |
| 14 | NPG - 2/8 | 19.00 | 20.50 | 94.40 | 1.17 | 5.60 | 70.02 | 6.70 | 2.01 | 1.50 | Khondalite+ Graphite |
| 15 |  | 20.50 | 21.90 |  |  |  |  |  |  | 1.40 | Khondalite |
| 16 | NPG - 2/9 | 21.90 | 22.00 | 96.18 | 0.63 | 3.82 | 74.68 | 13.50 | 1.50 | 0.10 |  |
| 17 | NPG - 2/10 | 22.00 | 23.90 | 96.46 | 0.99 | 3.54 | 70.74 | 12.50 | 0.98 | 1.90 |  |
| 18 |  | 23.90 | 24.50 |  |  |  |  |  |  | 0.60 |  |
| 19 | NPG - 2/11 | 24.50 | 25.30 | 95.26 | 1.01 | 4.74 | 72.46 | 9.70 | 1.36 | 0.80 |  |
| 20 |  | 25.30 | 26.50 |  |  |  |  |  |  | 1.20 |  |
| 21 | NPG - 2/12 | 26.50 | 27.00 | 92.20 | 1.56 | 7.80 | 64.76 | 13.94 | 2.39 | 0.50 | Khondalite+ Graphite |
| 22 |  | 27.00 | 29.50 |  |  |  |  |  |  | 2.50 | Khondalite |
| 23 |  | 29.50 | 41.45 |  |  |  |  |  |  | 11.95 | Quartzofeldspathic Gneiss ± Garnet |
| 24 | NPG - 2/13 | 41.45 | 42.45 | 92.88 | 0.91 | 7.12 | 63.54 | 15.26 | 2.09 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 25 | NPG - 2/14 | 42.45 | 43.45 | 93.18 | 0.40 | 6.82 | 68.34 | 10.94 | 1.80 | 1.00 |  |
| 26 | NPG - 2/15 | 43.45 | 44.45 | 91.88 | 1.04 | 8.12 | 60.50 | 17.76 | 2.24 | 1.00 |  |
| 27 | NPG - 2/16 | 44.45 | 45.45 | 93.34 | 1.06 | 6.66 | 67.72 | 13.72 | 1.88 | 1.00 |  |
| 28 | NPG - 2/17 | 45.45 | 46.45 | 93.90 | 0.76 | 6.10 | 67.00 | 11.38 | 1.61 | 1.00 | Quartzofeldspathic Gneiss ± Garnet |
| 29 | NPG - 2/18 | 46.45 | 47.45 | 92.96 | 0.82 | 7.04 | 66.66 | 11.02 | 1.63 | 1.00 |  |
| 30 | NPG - 2/19 | 47.45 | 48.45 | 93.00 | 0.93 | 7.00 | 63.66 | 15.46 | 1.98 | 1.00 |  |
| 31 | NPG - 2/20 | 48.45 | 49.45 | 92.44 | 1.50 | 7.56 | 61.20 | 17.56 | 1.47 | 1.00 |  |
| 32 | NPG - 2/21 | 49.45 | 50.45 | 94.40 | 1.23 | 5.60 | 66.48 | 12.38 | 0.58 | 1.00 |  |
| 33 | NPG - 2/22 | 50.45 | 51.70 | 91.88 | 1.33 | 8.12 | 64.30 | 12.02 | 2.00 | 1.25 | Khondalite+ Graphite |
| 34 |  | 51.70 | 53.00 |  |  |  |  |  |  | 1.30 | Quartzofeldspathic Gneiss ± Garnet |
| 35 | NPG - 2/23 | 53.00 | 54.00 | 92.04 | 2.06 | 7.96 | 61.14 | 19.68 | 0.48 | 1.00 |  |
| 36 |  | 54.00 | 54.50 |  |  |  |  |  |  | 0.50 |  |
| 37 | NPG - 2/24 | 54.50 | 55.00 | 94.58 | 1.52 | 5.42 | 58.94 | 18.02 | 0.53 | 0.50 |  |
|  |  |  |  |  |  |  |  |  |  | 55.00 |  |

| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bore Hole No- | NPBH-3 |  |  |  |  | Date of Commencement |  |  |  | 22.05.2024 |  |
| Location( DGPS) | 767854.1705 |  |  | 2182910.55 |  | Date of Completion |  |  |  | 03.06.2024 |  |
| (DMS) | 83°33'19.90080" E |  |  | 19°43'24.90600" N |  | Total Graphite Thickness |  |  |  | 19.55 m |  |
| Hole depth | 74.50 m |  |  |  |  | Total Recovery of Core |  |  |  | 72.38 m |  |
| R.L | 331.02 m |  |  |  |  | Percentage of Core Recovery |  |  |  | 97.15% |  |
| Water table depth | 17.00 m |  |  |  |  |  |  |  |  |  |  |
| Sl. NO | Sample ID | From | To | ASH % | M% | LOI% | SiO2 | Al2O3% | F.C% | Length | Lithology |
| 1 |  | 0.00 | 5.50 |  |  |  |  |  |  | 5.50 | Soil |
| 2 |  | 5.50 | 15.50 |  |  |  |  |  |  | 10.00 | Quartzofeldspathic Gneiss ± Garnet |
| 3 | NPG - 3/1 | 15.50 | 16.30 | 93.48 | 2.49 | 6.52 | 64.34 | 15.58 | 1.39 | 0.80 | Khondalite |
| 4 |  | 16.30 | 18.50 |  |  |  |  |  |  | 2.20 |  |
| 5 | NPG - 3/2 | 18.50 | 19.50 | 96.30 | 0.78 | 3.70 | 72.08 | 15.16 | 0.98 | 1.00 | Quartzofeldspathic Gneiss ± Garnet |
| 6 |  | 19.50 | 20.50 |  |  |  |  |  |  | 1.00 | Khondalite |
| 7 | NPG - 3/3 | 20.50 | 21.10 | 86.00 | 5.14 | 12.00 | 55.06 | 14.32 | 3.93 | 0.60 | Quartzofeldspathic Gneiss + Graphite |
| 8 |  | 21.10 | 36.50 |  |  |  |  |  |  | 15.40 | Quartzofeldspathic Gneiss ± Garnet |
| 9 |  | 36.50 | 37.50 |  |  |  |  |  |  | 1.00 | Khondalite |
| 10 |  | 37.50 | 41.60 |  |  |  |  |  |  | 4.10 | Quartzofeldspathic Gneiss ± Garnet |
| 11 | NPG - 3/4 | 41.60 | 42.60 | 95.94 | 0.80 | 4.06 | 67.48 | 12.74 | 1.18 | 1.00 |  |
| 12 | NPG - 3/5 | 42.60 | 43.60 | 94.78 | 0.85 | 5.22 | 63.28 | 18.44 | 0.93 | 1.00 |  |
| 13 | NPG - 3/6 | 43.60 | 44.60 | 93.34 | 1.55 | 6.66 | 64.50 | 19.20 | 1.26 | 1.00 |  |
| 14 | NPG - 3/7 | 44.60 | 45.60 | 93.20 | 1.04 | 6.80 | 64.46 | 17.74 | 1.55 | 1.00 |  |
| 15 | NPG - 3/8 | 45.60 | 46.60 | 93.86 | 1.35 | 6.14 | 65.90 | 16.44 | 1.02 | 1.00 |  |
| 16 | NPG - 3/9 | 46.60 | 47.60 | 94.18 | 0.85 | 5.82 | 65.80 | 16.22 | 1.33 | 1.00 |  |
| 17 | NPG - 3/10 | 47.60 | 48.60 | 94.42 | 0.78 | 5.58 | 62.58 | 19.78 | 1.27 | 1.00 |  |
| 18 | NPG - 3/11 | 48.60 | 49.60 | 94.28 | 0.85 | 5.72 | 65.40 | 16.30 | 1.65 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 19 | NPG - 3/12 | 49.60 | 50.60 | 93.36 | 0.84 | 6.64 | 63.12 | 17.96 | 2.37 | 1.00 |  |
| 20 | NPG - 3/13 | 50.60 | 51.60 | 93.06 | 0.93 | 6.94 | 59.74 | 16.42 | 1.96 | 1.00 |  |
| 21 | NPG - 3/14 | 51.60 | 52.60 | 92.78 | 0.87 | 7.22 | 63.58 | 16.48 | 2.57 | 1.00 |  |
| 22 | NPG - 3/15 | 52.60 | 53.60 | 93.00 | 0.93 | 7.00 | 63.10 | 16.06 | 2.43 | 1.00 |  |
| 23 | NPG - 3/16 | 53.60 | 54.60 | 94.78 | 1.04 | 5.22 | 63.62 | 18.30 | 1.21 | 1.00 | Quartzofeldspathic Gneiss ± Garnet |
| 24 | NPG - 3/17 | 54.60 | 55.60 | 94.02 | 0.64 | 5.98 | 65.92 | 17.84 | 1.74 | 1.00 |  |
| 25 | NPG - 3/18 | 55.60 | 56.60 | 93.08 | 0.81 | 6.92 | 66.34 | 12.08 | 2.64 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 26 | NPG - 3/19 | 56.60 | 57.60 | 92.90 | 0.95 | 7.10 | 65.22 | 16.34 | 2.73 | 1.00 |  |
| 27 | NPG - 3/20 | 57.60 | 58.60 | 95.04 | 0.95 | 4.96 | 65.56 | 16.08 | 1.33 | 1.00 |  |
| 28 |  | 58.60 | 61.00 |  |  |  |  |  |  | 2.40 | Quartzofeldspathic Gneiss ± Garnet |
| 29 | NPG - 3/21 | 61.00 | 62.00 | 93.30 | 0.84 | 6.70 | 65.60 | 16.30 | 2.08 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 30 | NPG - 3/22 | 62.00 | 63.00 | 92.12 | 0.96 | 7.80 | 60.78 | 11.78 | 2.82 | 1.00 |  |
| 31 | NPG - 3/23 | 63.00 | 64.00 | 92.80 | 0.86 | 7.20 | 65.00 | 9.54 | 2.81 | 1.00 |  |
| 32 | NPG - 3/24 | 64.00 | 65.00 | 92.60 | 0.99 | 7.40 | 62.08 | 17.66 | 2.79 | 1.00 |  |
| 33 | NPG - 3/25 | 65.00 | 66.00 | 92.42 | 0.71 | 7.58 | 64.54 | 16.42 | 2.94 | 1.00 |  |
| 34 | NPG - 3/26 | 66.00 | 67.00 | 92.88 | 0.94 | 7.12 | 66.04 | 13.64 | 1.85 | 1.00 |  |
| 35 | NPG - 3/27 | 67.00 | 68.00 | 89.96 | 1.78 | 10.04 | 61.60 | 17.34 | 2.94 | 1.00 |  |
| 36 | NPG - 3/28 | 68.00 | 69.00 | 92.04 | 1.38 | 7.96 | 67.04 | 15.48 | 2.24 | 1.00 |  |
| 37 | NPG - 3/29 | 69.00 | 71.00 | 93.84 | 1.24 | 6.16 | 70.16 | 12.76 | 1.84 | 2.00 |  |
| 38 | NPG - 3/30 | 71.00 | 71.95 | 93.96 | 1.14 | 6.04 | 66.26 | 15.78 | 2.07 | 0.95 |  |
| 39 |  | 71.95 | 72.50 |  |  |  |  |  |  | 0.55 | Quartzofeldspathic Gneiss ± Garnet |
| 40 | NPG - 3/31 | 72.50 | 73.40 | 95.16 | 1.09 | 4.84 | 71.96 | 13.58 | 1.50 | 0.90 |  |
| 41 |  | 73.40 | 74.50 |  |  |  |  |  |  | 1.10 | Khondalite |
|  |  |  |  |  |  |  |  |  |  | 74.50 |  |

| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bore Hole No- | NPBH-4 |  |  |  |  | Date of Commencement |  |  |  | 07.06.2024 |  |
| Location( DGPS) | 767887.8965 |  |  | 2182985 |  | Date of Completion |  |  |  | 28.06.2024 |  |
| (DMS) | 83°33'21.09600" E |  |  | 19°43'27.30720" N |  | Total Graphite Thickness |  |  |  | 18.55 m |  |
| Hole depth | 56.50 m |  |  |  |  | Total Recovery of Core |  |  |  | 54.32 m |  |
| R.L | 335.52 m |  |  |  |  | Percentage of Core Recovery |  |  |  | 96.14% |  |
| Water table depth | 18.00 m |  |  |  |  |  |  |  |  |  |  |
| Sl. No. | Sample ID | From | To | ASH % | M% | LOI% | SiO2 | Al2O3% | FC% | Length | Lithology |
| 1 |  | 0.00 | 6.00 |  |  |  |  |  |  | 6.00 | Soil |
| 2 |  | 6.00 | 11.00 |  |  |  |  |  |  | 5.00 | Quartzofeldspathic Gneiss ± Garnet |
| 3 | NPG - 4/1 | 11.00 | 12.00 | 88.86 | 3.58 | 11.14 | 59.14 | 14.44 | 2.12 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 4 | NPG - 4/2 | 12.00 | 13.05 | 90.76 | 3.11 | 9.24 | 66.22 | 12.30 | 1.95 | 1.05 |  |
| 5 |  | 13.05 | 15.80 |  |  |  |  |  |  | 2.75 | Quartzofeldspathic Gneiss ± Garnet |
| 6 |  | 15.80 | 17.00 |  |  |  |  |  |  | 1.20 | Khondalite |
| 7 | NPG - 4/3 | 17.00 | 18.00 | 93.24 | 1.68 | 6.76 | 66.84 | 11.28 | 0.91 | 1.00 |  |
| 8 | NPG - 4/4 | 18.00 | 19.00 | 90.56 | 2.08 | 9.44 | 60.82 | 17.14 | 1.52 | 1.00 |  |
| 9 | NPG - 4/5 | 19.00 | 20.00 | 90.34 | 1.82 | 9.66 | 58.32 | 19.86 | 1.89 | 1.00 |  |
| 10 | NPG - 4/6 | 20.00 | 21.00 | 91.46 | 1.93 | 8.54 | 57.04 | 12.94 | 0.91 | 1.00 |  |
| 11 | NPG - 4/7 | 21.00 | 22.00 | 89.82 | 2.45 | 10.18 | 65.58 | 9.72 | 3.11 | 1.00 | Khondalite + Graphite |
| 12 | NPG - 4/8 | 22.00 | 23.00 | 93.34 | 1.78 | 6.66 | 66.30 | 12.42 | 1.19 | 1.00 |  |
| 13 | NPG - 4/9 | 23.00 | 24.00 | 89.10 | 3.11 | 10.90 | 53.42 | 14.58 | 1.67 | 1.00 | Khondalite |
| 14 | NPG - 4/10 | 24.00 | 25.00 | 91.32 | 1.76 | 8.68 | 64.24 | 10.10 | 1.92 | 1.00 |  |
| 15 | NPG - 4/11 | 25.00 | 26.00 | 94.30 | 1.48 | 5.70 | 65.40 | 10.20 | 0.56 | 1.00 |  |
| 16 | NPG - 4/12 | 26.00 | 27.00 | 93.54 | 1.45 | 6.46 | 60.72 | 14.16 | 0.66 | 1.00 |  |
| 17 |  | 27.00 | 28.00 |  |  |  |  |  |  | 1.00 |  |
| 18 | NPG - 4/13 | 28.00 | 29.00 | 92.32 | 1.83 | 7.68 | 67.60 | 10.94 | 1.80 | 1.00 |  |
| 19 | NPG - 4/14 | 29.00 | 30.00 | 92.24 | 1.88 | 7.76 | 64.14 | 13.46 | 1.60 | 1.00 |  |
| 20 |  | 30.00 | 31.00 |  |  |  |  |  |  | 1.00 |  |
| 21 | NPG - 4/15 | 31.00 | 32.00 | 90.18 | 2.31 | 9.82 | 66.02 | 11.80 | 2.33 | 1.00 | Khondalite + Graphite |
| 22 | NPG - 4/16 | 32.00 | 33.00 | 90.62 | 1.00 | 9.38 | 67.68 | 11.56 | 2.38 | 1.00 |  |
| 23 |  | 33.00 | 33.50 |  |  |  |  |  |  | 0.50 |  |
| 24 | NPG - 4/17 | 33.50 | 34.00 | 91.42 | 1.21 | 8.58 | 72.46 | 10.30 | 2.74 | 0.50 |  |
| 25 | NPG - 4/18 | 34.00 | 35.00 | 91.50 | 1.38 | 8.50 | 67.36 | 10.28 | 1.92 | 1.00 | Khondalite |
| 26 | NPG - 4/19 | 35.00 | 36.00 | 91.66 | 1.50 | 8.34 | 68.52 | 7.58 | 1.78 | 1.00 |  |
| 27 | NPG - 4/20 | 36.00 | 37.00 | 91.44 | 1.15 | 8.56 | 68.32 | 14.90 | 2.25 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 28 | NPG - 4/21 | 37.00 | 38.00 | 89.50 | 0.97 | 10.50 | 70.78 | 8.50 | 4.60 | 1.00 |  |
| 29 | NPG - 4/22 | 38.00 | 39.00 | 87.72 | 2.48 | 12.28 | 66.42 | 7.68 | 5.47 | 1.00 |  |
| 30 |  | 39.00 | 40.50 |  |  |  |  |  |  | 1.50 | Quartzofeldspathic Gneiss ± Garnet |
| 31 | NPG - 4/23 | 40.50 | 41.50 | 85.24 | 3.89 | 14.76 | 54.36 | 8.50 | 4.15 | 1.00 | Khondalite + Graphite |
| 32 | NPG - 4/24 | 41.50 | 42.50 | 89.48 | 0.20 | 10.52 | 81.44 | 0.50 | 8.16 | 1.00 |  |
| 33 | NPG - 4/25 | 42.50 | 43.50 | 89.60 | 0.21 | 10.40 | 82.70 | 1.80 | 8.06 | 1.00 |  |
| 34 | NPG - 4/26 | 43.50 | 44.50 | 87.48 | 0.30 | 12.52 | 78.62 | 3.00 | 10.03 | 1.00 |  |
| 35 | NPG - 4/27 | 44.50 | 45.50 | 87.62 | 0.35 | 12.38 | 80.34 | 1.00 | 9.84 | 1.00 |  |
| 36 | NPG - 4/28 | 45.50 | 46.30 | 88.60 | 0.19 | 11.40 | 80.72 | 0.60 | 9.26 | 0.80 |  |
| 37 | NPG - 4/29 | 46.30 | 47.00 | 91.74 | 0.69 | 8.26 | 62.38 | 13.48 | 2.36 | 0.70 |  |
| 38 | NPG - 4/30 | 47.00 | 48.00 | 90.80 | 0.67 | 9.20 | 65.66 | 11.26 | 3.10 | 1.00 |  |
| 39 | NPG - 4/31 | 48.00 | 49.00 | 92.84 | 1.42 | 7.16 | 68.82 | 11.86 | 1.95 | 1.00 |  |
| 40 |  | 49.00 | 54.15 |  |  |  |  |  |  | 5.15 | Khondalite |
| 41 |  | 54.15 | 56.50 |  |  |  |  |  |  | 2.35 | Quartzofeldspathic Gneiss ± Garnet |
|  |  |  |  |  |  |  |  |  |  | 56.50 |  |

| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bore Hole No- | NPBH-5 |  |  |  |  | Date of Commencement |  |  |  | 09.06.2024 |  |
| Location( DGPS) | 76910.4273 |  |  | 2182829.996 |  | Date of Completion |  |  |  | 10.07.2024 |  |
| (DMS) | 83°33'21.79080" E |  |  | 19°43'22.26000" N |  | Total Graphite Thickness |  |  |  | 5.50 m |  |
| Hole depth | 78.80 m |  |  |  |  | Total Recovery of Core |  |  |  | 75.98 m |  |
| R.L | 329.86 m |  |  |  |  | Percentage of Core Recovery |  |  |  | 96.42% |  |
| Water table depth | 18.00 m |  |  |  |  |  |  |  |  |  |  |
| Sl. NO | Sample ID | From | To | ASH % | M% | LOI% | SiO2 | Al2O3% | FC% | Length | Lithology |
| 1 |  | 0.00 | 5.00 |  |  |  |  |  |  | 5.00 | Soil |
| 2 |  | 5.00 | 13.70 |  |  |  |  |  |  | 8.70 | Quartzofeldspathic Gneiss ± Garnet |
| 3 |  | 13.70 | 19.00 |  |  |  |  |  |  | 5.30 | Khondalite |
| 4 |  | 19.00 | 21.00 |  |  |  |  |  |  | 2.00 | Quartzofeldspathic Gneiss ± Garnet |
| 5 |  | 21.00 | 23.00 |  |  |  |  |  |  | 2.00 | Khondalite |
| 6 |  | 23.00 | 31.00 |  |  |  |  |  |  | 8.00 | Quartzofeldspathic Gneiss ± Garnet |
| 7 | NPG - 5/1 | 31.00 | 32.00 | 95.38 | 1.08 | 4.62 | 69.14 | 13.26 | 1.57 | 1.00 | Khondalite |
| 8 |  | 32.00 | 34.00 |  |  |  |  |  |  | 2.00 | Quartzofeldspathic Gneiss ± Garnet |
| 9 | NPG - 5/2 | 34.00 | 35.00 | 95.36 | 0.83 | 4.64 | 65.34 | 15.46 | 0.58 | 1.00 | Khondalite |
| 10 | NPG - 5/3 | 35.00 | 36.00 | 94.66 | 1.52 | 5.34 | 65.38 | 12.06 | 1.09 | 1.00 |  |
| 11 |  | 36.00 | 40.00 |  |  |  |  |  |  | 4.00 | Quartzofeldspathic Gneiss ± Garnet |
| 12 | NPG - 5/4 | 40.00 | 41.00 | 98.16 | 0.37 | 1.84 | 64.16 | 17.18 | 0.41 | 1.00 | Khondalite |
| 13 |  | 41.00 | 44.00 |  |  |  |  |  |  | 3.00 | Quartzofeldspathic Gneiss ± Garnet |
| 14 | NPG - 5/5 | 44.00 | 45.00 | 95.66 | 0.87 | 4.34 | 65.34 | 15.62 | 1.03 | 1.00 |  |
| 15 | NPG - 5/6 | 45.00 | 46.00 | 94.88 | 0.90 | 5.12 | 66.92 | 12.52 | 1.39 | 1.00 |  |
| 16 | NPG - 5/7 | 46.00 | 46.50 | 96.76 | 0.55 | 3.24 | 70.48 | 12.38 | 1.01 | 0.50 |  |
| 17 |  | 46.50 | 52.30 |  |  |  |  |  |  | 5.80 |  |
| 18 | NPG - 5/8 | 52.30 | 53.00 | 95.24 | 0.78 | 4.76 | 68.20 | 14.48 | 1.70 | 0.70 |  |
| 19 |  | 53.00 | 53.50 |  |  |  |  |  |  | 0.50 |  |
| 20 | NPG - 5/9 | 53.50 | 54.00 | 95.42 | 0.88 | 4.58 | 63.70 | 15.66 | 1.60 | 0.50 |  |
| 21 |  | 54.00 | 54.50 |  |  |  |  |  |  | 0.50 |  |
| 22 | NPG - 5/10 | 54.50 | 55.50 | 94.94 | 0.81 | 5.06 | 61.38 | 17.88 | 1.26 | 1.00 |  |
| 23 | NPG - 5/11 | 55.50 | 56.50 | 95.28 | 0.94 | 4.72 | 66.28 | 18.16 | 0.88 | 1.00 |  |
| 24 | NPG - 5/12 | 56.50 | 57.50 | 94.36 | 1.14 | 5.64 | 63.84 | 16.06 | 1.15 | 1.00 |  |
| 25 | NPG - 5/13 | 57.50 | 58.50 | 94.76 | 1.19 | 5.24 | 67.68 | 14.22 | 1.01 | 1.00 |  |
| 26 | NPG - 5/14 | 58.50 | 59.50 | 93.30 | 1.09 | 6.70 | 63.86 | 14.06 | 1.96 | 1.00 |  |
| 27 | NPG - 5/15 | 59.50 | 60.50 | 93.14 | 1.02 | 6.86 | 63.42 | 17.64 | 1.85 | 1.00 |  |
| 28 | NPG - 5/16 | 60.50 | 61.50 | 91.76 | 1.22 | 8.26 | 62.70 | 14.02 | 2.94 | 1.00 | Khondalite + Graphite |
| 29 | NPG - 5/17 | 61.50 | 62.30 | 93.58 | 1.20 | 6.42 | 56.92 | 16.44 | 1.61 | 0.80 | Khondalite |
| 30 |  | 62.30 | 63.00 |  |  |  |  |  |  | 0.70 |  |
| 31 | NPG - 5/18 | 63.00 | 64.00 | 93.60 | 1.03 | 6.40 | 66.86 | 13.82 | 2.53 | 1.00 | Khondalite + Graphite |
| 32 | NPG - 5/19 | 64.00 | 65.00 | 94.52 | 1.08 | 5.48 | 67.84 | 12.86 | 1.76 | 1.00 |  |
| 33 | NPG - 5/20 | 65.00 | 66.00 | 93.66 | 1.31 | 6.34 | 66.12 | 12.56 | 2.50 | 1.00 |  |
| 34 | NPG - 5/21 | 66.00 | 67.00 | 95.76 | 0.89 | 4.24 | 69.72 | 12.16 | 1.34 | 1.00 | Quartzofeldspathic Gneiss ± Garnet |
| 35 | NPG - 5/22 | 67.00 | 68.00 | 96.04 | 1.10 | 3.96 | 70.56 | 10.92 | 1.14 | 1.00 |  |
| 36 | NPG - 5/23 | 68.00 | 69.00 | 94.58 | 1.51 | 5.42 | 68.18 | 9.82 | 1.54 | 1.00 |  |
| 37 | NPG - 5/24 | 69.00 | 70.00 | 95.52 | 1.43 | 4.48 | 67.20 | 13.62 | 0.64 | 1.00 |  |
| 38 | NPG - 5/25 | 70.00 | 71.00 | 95.70 | 1.01 | 4.30 | 75.66 | 4.16 | 1.26 | 1.00 |  |
| 39 | NPG - 5/26 | 71.00 | 72.00 | 96,96 | 1.60 | 3.04 | 80.98 | 4.92 | 0.90 | 1.00 |  |
| 40 | NPG - 5/27 | 72.00 | 73.50 | 97.44 | 0.61 | 2.56 | 68.92 | 13.92 | 0.36 | 1.50 |  |
| 41 | NPG - 5/28 | 73.50 | 75.00 | 93.24 | 1.90 | 6.76 | 67.16 | 13.16 | 2.35 | 1.50 | Quartzofeldspathic Gneiss + Graphite |
| 42 |  | 75.00 | 78.80 |  |  |  |  |  |  | 3.80 | Quartzofeldspathic Gneiss ± Garnet |
|  |  |  |  |  |  |  |  |  |  | 78.80 |  |

| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bore Hole No- | NPBH-6 |  |  |  |  |  |  |  |  | Date of Commencement |  |  |  |  | 05.07.2024 |  |
| Location( DGPS) | 767975.6609 |  |  |  |  | 2182906.237 |  |  |  | Date of Completion |  |  |  |  | 27.07.2024 |  |
| (DMS) | 83°33'24.06960" E |  |  |  |  | 19°43'24.70440" N |  |  |  | Total Graphite Thickness |  |  |  |  | 6.50 m |  |
| Hole depth | 63.20 m |  |  |  |  |  |  |  |  | Total Recovery of Core |  |  |  |  | 60.5 m |  |
| R.L | 334.14 m |  |  |  |  |  |  |  |  | Percentage of Core Recovery |  |  |  |  | 95.72% |  |
| Water table depth | 16.00m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sl. NO | Sample ID |  | From | To |  | ASH % |  | M% | V.M% | LOI% | SiO2 | Al2O3% | V2O5% | FC% | Length | Lithology |
| 1 |  |  | 0.00 | 5.00 |  |  |  |  |  |  |  |  |  |  | 5.00 | Soil |
| 2 |  |  | 5.00 | 8.00 |  |  |  |  |  |  |  |  |  |  | 3.00 | Quartzofeldspathic Gneiss ± Garnet |
| 3 |  |  | 8.00 | 17.80 |  |  |  |  |  |  |  |  |  |  | 9.80 | Khondalite |
| 4 | NPG 6/1 |  | 17.80 | 19.00 |  | 90.64 |  | 3.28 | 4.33 | 9.37 | 66.99 | 14.47 | 0.00 | 1.76 | 1.20 |  |
| 5 |  |  | 19.00 | 21.00 |  |  |  |  |  |  |  |  |  |  | 2.00 |  |
| 6 | NPG 6/2 |  | 21.00 | 21.50 |  | 89.30 |  | 3.56 | 6.26 | 10.71 | 66.21 | 15.59 | 0.00 | 0.89 | 0.50 |  |
| 7 |  |  | 21.50 | 25.00 |  |  |  |  |  |  |  |  |  |  | 3.50 |  |
| 8 | NPG 6/3 |  | 25.00 | 26.00 |  | 90.00 |  | 3.49 | 4.69 | 10.00 | 61.20 | 17.10 | 0.00 | 1.82 | 1.00 | Khondalite + Graphite |
| 9 | NPG 6/4 |  | 26.00 | 27.00 |  | 86.62 |  | 4.81 | 4.66 | 13.38 | 62.02 | 16.05 | 0.00 | 3.91 | 1.00 |  |
| 10 | NPG 6/5 |  | 27.00 | 28.00 |  | 90.20 |  | 3.74 | 3.46 | 9.81 | 69.86 | 12.16 | 0.00 | 2.61 | 1.00 |  |
| 11 | NPG 6/6 |  | 28.00 | 29.00 |  | 88.33 |  | 4.66 | 4.42 | 11.68 | 66.72 | 13.74 | 0.00 | 2.60 | 1.00 |  |
| 12 | NPG 6/7 |  | 29.00 | 30.00 |  | 86.84 |  | 5.19 | 5.09 | 13.17 | 61.94 | 16.16 | 0.00 | 2.89 | 1.00 |  |
| 13 | NPG 6/8 |  | 30.00 | 31.00 |  | 85.56 |  | 5.31 | 5.76 | 14.45 | 57.56 | 16.18 | 0.00 | 3.38 | 1.00 |  |
| 14 |  |  | 31.00 | 37.00 |  |  |  |  |  |  |  |  |  |  | 6.00 | Khondalite |
| 15 |  |  | 37.00 | 46.50 |  |  |  |  |  |  |  |  |  |  | 9.50 | Quartzofeldspathic Gneiss ± Garnet |
| 18 | NPG 6/9 |  | 46.50 | 47.50 |  | 94.78 |  | 1.00 | 3.61 | 5.23 | 64.63 | 16.22 | 0.00 | 0.62 | 1.00 |  |
| 19 | NPG 6/10 |  | 47.50 | 48.50 |  | 95.16 |  | 1.04 | 3.77 | 3.09 | 67.79 | 15.73 | 0.00 | 0.03 | 1.00 |  |
| 20 | NPG 6/11 |  | 48.50 | 49.00 |  | 91.33 |  | 1.69 | 5.10 | 8.67 | 61.69 | 17.48 | 0.00 | 1.88 | 0.50 |  |
| 21 |  |  | 49.00 | 51.20 |  |  |  |  |  |  |  |  |  |  | 2.20 |  |
| 22 | NPG 6/12 |  | 51.20 | 51.70 |  | 91.31 |  | 1.51 | 4.79 | 8.70 | 63.38 | 15.72 | 0.00 | 2.40 | 0.50 | Quartzofeldspathic Gneiss + Graphite |
| 23 |  |  | 51.70 | 52.05 |  |  |  |  |  |  |  |  |  |  | 0.35 | Quartzofeldspathic Gneiss ± Garnet |
| 24 | NPG 6/13 |  | 52.05 | 53.05 |  | 98.03 |  | 0.70 | 1.19 | 0.27 | 70.06 | 17.06 | 0.00 | 0.08 | 1.00 |  |
| 25 | NPG 6/14 |  | 53.05 | 53.60 |  | 94.64 |  | 0.98 | 3.62 | 5.36 | 65.25 | 17.69 | 0.00 | 0.76 | 0.55 |  |
| 26 |  |  | 53.60 | 56.00 |  |  |  |  |  |  |  |  |  |  | 2.40 |  |
| 27 | NPG 6/15 |  | 56.00 | 57.10 |  | 91.24 |  | 1.56 | 5.46 | 8.77 | 64.44 | 15.59 | 0.00 | 1.75 | 1.10 |  |
| 28 |  |  | 57.10 | 63.20 |  |  |  |  |  |  |  |  |  |  | 6.10 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 63.20 |  |

| ANNEXURE –III |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HOLE WISE WEIGHT AND AVERAGE OF THE BLOCK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bore Hole No. | NPBH-1 |  |  |  |  |  |  |  |  | Date of Commencement |  |  |  |  |  | 13.07.2024 |  |
| Location ( DGPS) | 767689.2503 |  |  |  | 2183040.786 |  |  |  |  | Date of Completion |  |  |  |  |  | 30.07.2024 |  |
| (DMS) | 83°33'14.31000" E |  |  |  | 19°43'29.21880" N |  |  |  |  | Total Graphite Thickness |  |  |  |  |  | 24.55 m |  |
| Hole depth | 70.00 m |  |  |  |  |  |  |  |  | Total Recovery of Core |  |  |  |  |  | 67.28 m |  |
| R.L | 331.40 m |  |  |  |  |  |  |  |  | Percentage of Core Recovery |  |  |  |  |  | 96.11% |  |
| Water table depth | 15.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sl. No. | Sample ID | From |  | To | ASH % |  | M% |  | V.M% | LOI% | SiO2 | Al2O3% | V₂O₅ (%) | F.C% | F.C weight avg. | Length | Lithology |
| 1 | NPG-1/1 | 12.00 |  | 13.00 | 88.63 |  | 1.15 |  | 2.38 | 11.37 | 80.78 | 2.58 | 0.00 | 7.84 | 7.72 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 2 | NPG-1/2 | 13.00 |  | 14.00 | 89.77 |  | 0.71 |  | 1.93 | 10.23 | 82.87 | 2.72 | 0.00 | 7.59 |  | 1.00 |  |
| 3 | NPG-1/3 | 21.00 |  | 22.00 | 86.59 |  | 2.97 |  | 3.68 | 13.41 | 68.94 | 5.58 | 0.00 | 6.76 | 3.89 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 4 | NPG-1/4 | 22.00 |  | 23.00 | 89.08 |  | 2.70 |  | 4.31 | 10.93 | 72.80 | 5.32 | 0.00 | 3.92 |  | 1.00 |  |
| 5 | NPG-1/5 | 23.00 |  | 24.00 | 91.38 |  | 2.11 |  | 3.99 | 8.63 | 75.94 | 6.68 | 0.01 | 2.53 |  | 1.00 |  |
| 6 | NPG-1/6 | 24.00 |  | 25.00 | 90.32 |  | 1.24 |  | 3.66 | 9.69 | 74.92 | 7.09 | 0.01 | 4.79 |  | 1.00 |  |
| 7 | NPG-1/7 | 25.00 |  | 26.00 | 85.49 |  | 4.68 |  | 5.73 | 14.50 | 54.57 | 15.80 | 0.01 | 4.10 |  | 1.00 |  |
| 8 | NPG-1/8 | 26.00 |  | 27.00 | 85.55 |  | 1.79 |  | 5.41 | 14.45 | 61.66 | 12.77 | 0.00 | 7.25 |  | 1.00 |  |
| 9 |  | 28.00 |  | 28.30 | 0.00 |  | 0.00 |  | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |  | 0.30 |  |
| 10 | NPG-1/9 | 27.00 |  | 28.00 | 93.91 |  | 1.65 |  | 1.01 | 6.10 | 74.90 | 8.42 | 0.00 | 3.44 |  | 1.00 |  |
| 11 | NPG-1/10 | 28.30 |  | 29.00 | 94.29 |  | 1.38 |  | 1.06 | 5.72 | 68.95 | 14.09 | 0.00 | 3.28 |  | 0.70 |  |
| 12 | NPG-1/11 | 29.00 |  | 30.00 | 94.43 |  | 1.01 |  | 1.09 | 5.58 | 71.40 | 14.42 | 0.00 | 3.48 |  | 1.00 |  |
| 13 | NPG-1/12 | 30.00 |  | 31.00 | 94.10 |  | 1.42 |  | 0.98 | 5.91 | 70.28 | 13.82 | 0.00 | 3.51 |  | 1.00 |  |
| 14 |  | 31.00 |  | 31.85 | 0.00 |  | 0.00 |  | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |  | 0.85 |  |
| 15 | NPG-1/13 | 31.85 |  | 32.85 | 91.12 |  | 2.40 |  | 4.08 | 8.88 | 64.76 | 13.31 | 0.00 | 2.40 |  | 1.00 |  |
| 16 | NPG-1/14 | 32.85 |  | 34.00 | 91.18 |  | 2.33 |  | 1.12 | 8.82 | 66.62 | 13.75 | 0.00 | 5.37 |  | 1.15 |  |
| 17 | NPG-1/16 | 43.50 |  | 44.30 | 92.46 |  | 2.31 |  | 3.04 | 7.54 | 63.27 | 17.76 | 0.00 | 2.19 | 2.19 | 0.80 | Quartzofeldspathic Gneiss + Graphite |
| 18 | NPG-1/18 | 52.10 |  | 53.10 | 92.30 |  | 1.75 |  | 3.95 | 7.70 | 63.24 | 17.09 | 0.00 | 2.00 | 2.00 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 19 | NPG-1/20 | 54.10 |  | 55.10 | 92.40 |  | 2.13 |  | 3.40 | 7.60 | 67.35 | 13.56 | 0.00 | 2.07 | 2.07 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 20 | NPG-1/22 | 56.35 |  | 57.35 | 92.98 |  | 1.48 |  | 2.95 | 7.02 | 68.21 | 12.90 | 0.00 | 2.59 | 2.34 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 21 | NPG-1/23 | 57.35 |  | 58.30 | 94.10 |  | 1.43 |  | 2.39 | 5.90 | 72.24 | 10.41 | 0.00 | 2.08 |  | 0.95 |  |
| 22 | NPG-1/25 | 60.00 |  | 61.00 | 92.98 |  | 1.66 |  | 3.18 | 7.02 | 67.34 | 12.88 | 0.00 | 2.18 | 2.37 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 23 | NPG-1/26 | 61.00 |  | 62.00 | 93.08 |  | 1.72 |  | 3.29 | 6.92 | 69.16 | 11.01 | 0.00 | 1.91 |  | 1.00 |  |
| 24 | NPG-1/27 | 62.00 |  | 63.30 | 90.40 |  | 2.48 |  | 4.25 | 9.60 | 61.94 | 13.89 | 0.00 | 2.87 |  | 1.30 |  |
| 25 | NPG-1/30 | 68.00 |  | 69.00 | 94.50 |  | 0.98 |  | 1.95 | 5.50 | 67.06 | 14.21 | 0.00 | 2.57 | 3.09 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 26 | NPG-1/31 | 69.00 |  | 69.50 | 92.52 |  | 1.04 |  | 2.30 | 7.48 | 73.73 | 11.50 | 0.00 | 4.14 |  | 0.50 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 24.55 |  |
|  |  | AVERAGE |  |  | 89.35 |  | 1.81 |  | 2.90 | 8.41 | 67.74 | 11.06 | 0.00 | 3.70 | 3.21 |  |  |

| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bore Hole No- | NPBH-2 |  |  |  |  |  |  | Date of Commencement |  |  |  |  |  | 03.05.2024 |  |
| Location( DGPS) | 767778.4465 |  |  |  |  | 2183058.278 |  | Date of Completion |  |  |  |  |  | 19.05.2024 |  |
| (DMS) | 83°33'17.37720" E |  |  |  |  | 19°43'29.74440" N |  | Total Graphite Thickness |  |  |  |  |  | 9.65 m |  |
| Hole depth | 55.00 m |  |  |  |  |  |  | Total Recovery of Core |  |  |  |  |  | 50.61 m |  |
| R.L | 333.31 m |  |  |  |  |  |  | Percentage of Core Recovery |  |  |  |  |  | 92.02% |  |
| Water table depth | 15.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sl. NO | Sample ID |  | From | To |  | ASH % | M% | LOI% | SiO2 | Al2O3% | FC% |  |  | Length | Lithology |
| 1 | NPG - 2/4 |  | 12.00 | 13.00 |  | 91.80 | 1.55 | 8.20 | 63.02 | 14.44 | 2.04 | 2.04 |  | 1.00 | Khondalite+ Graphite |
| 2 | NPG - 2/6 |  | 15.60 | 16.50 |  | 91.30 | 1.91 | 8.70 | 64.90 | 12.94 | 2.66 | 2.66 |  | 0.90 | Khondalite+ Graphite |
| 3 | NPG - 2/7 |  | 17.50 | 18.00 |  | 92.30 | 1.80 | 7.70 | 67.20 | 10.72 | 2.14 | 2.14 |  | 0.50 | Khondalite+ Graphite |
| 4 | NPG - 2/8 |  | 19.00 | 20.50 |  | 94.40 | 1.17 | 5.60 | 70.02 | 6.70 | 2.01 | 2.01 |  | 1.50 | Khondalite+ Graphite |
| 5 | NPG - 2/12 |  | 26.50 | 27.00 |  | 92.20 | 1.56 | 7.80 | 64.76 | 13.94 | 2.39 | 2.39 |  | 0.50 | Khondalite+ Graphite |
| 6 | NPG - 2/13 |  | 41.45 | 42.45 |  | 92.88 | 0.91 | 7.12 | 63.54 | 15.26 | 2.09 | 2.00 |  | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 7 | NPG - 2/14 |  | 42.45 | 43.45 |  | 93.18 | 0.40 | 6.82 | 68.34 | 10.94 | 1.80 |  |  | 1.00 |  |
| 8 | NPG - 2/15 |  | 43.45 | 44.45 |  | 91.88 | 1.04 | 8.12 | 60.50 | 17.76 | 2.24 |  |  | 1.00 |  |
| 9 | NPG - 2/16 |  | 44.45 | 45.45 |  | 93.34 | 1.06 | 6.66 | 67.72 | 13.72 | 1.88 |  |  | 1.00 |  |
| 10 | NPG - 2/22 |  | 50.45 | 51.70 |  | 91.88 | 1.33 | 8.12 | 64.30 | 12.02 | 2.00 | 2.00 |  | 1.25 | Khondalite+ Graphite |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.65 |  |
|  |  |  | AVERAGE |  |  | 95.87 | 1.32 | 7.76 | 67.80 | 13.31 | 2.20 | 2.18 |  |  |  |

| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bore Hole No- | NPBH-3 |  |  |  |  | Date of Commencement |  |  |  | 22.05.2024 |  |  |
| Location( DGPS) | 767854.1705 |  |  | 2182910.55 |  | Date of Completion |  |  |  | 03.06.2024 |  |  |
| (DMS) | 83°33'19.90080" E |  |  | 19°43'24.90600" N |  | Total Graphite Thickness |  |  |  | 19.55 m |  |  |
| Hole depth | 74.50 m |  |  |  |  | Total Recovery of Core |  |  |  | 72.38 m |  |  |
| R.L | 331.02 m |  |  |  |  | Percentage of Core Recovery |  |  |  | 97.15% |  |  |
| Water table depth | 17.00 m |  |  |  |  |  |  |  |  |  |  |  |
| SL.NO | Sample ID | From | To | Ash % | M % | LOI % | SiO₂ % | AI₂O₃ % | F.C % | F.C weight avg. | Length | Lithology |
| 1 | NPG - 3/3 | 20.50 | 21.10 | 86.00 | 5.14 | 12.00 | 55.06 | 14.32 | 3.93 | 3.93 | 0.60 | Quartzofeldspathic Gneiss + Graphite |
| 2 | NPG - 3/11 | 48.60 | 49.60 | 94.28 | 0.85 | 5.72 | 65.40 | 16.30 | 1.65 | 2.20 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 3 | NPG - 3/12 | 49.60 | 50.60 | 93.36 | 0.84 | 6.64 | 63.12 | 17.96 | 2.37 |  | 1.00 |  |
| 4 | NPG - 3/13 | 50.60 | 51.60 | 93.06 | 0.93 | 6.94 | 59.74 | 16.42 | 1.96 |  | 1.00 |  |
| 5 | NPG - 3/14 | 51.60 | 52.60 | 92.78 | 0.87 | 7.22 | 63.58 | 16.48 | 2.57 |  | 1.00 |  |
| 6 | NPG - 3/15 | 52.60 | 53.60 | 93.00 | 0.93 | 7.00 | 63.10 | 16.06 | 2.43 |  | 1.00 |  |
| 7 | NPG - 3/18 | 55.60 | 56.60 | 93.08 | 0.81 | 6.92 | 66.34 | 12.08 | 2.64 | 2.23 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 8 | NPG - 3/19 | 56.60 | 57.60 | 92.90 | 0.95 | 7.10 | 65.22 | 16.34 | 2.73 |  | 1.00 |  |
| 9 | NPG - 3/20 | 57.60 | 58.60 | 95.04 | 0.95 | 4.96 | 65.56 | 16.08 | 1.33 |  | 1.00 |  |
| 10 | NPG - 3/21 | 61.00 | 62.00 | 93.30 | 0.84 | 6.70 | 65.60 | 16.30 | 2.08 | 2.39 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
| 11 | NPG - 3/22 | 62.00 | 63.00 | 92.12 | 0.96 | 7.80 | 60.78 | 11.78 | 2.82 |  | 1.00 |  |
| 12 | NPG - 3/23 | 63.00 | 64.00 | 92.80 | 0.86 | 7.20 | 65.00 | 9.54 | 2.81 |  | 1.00 |  |
| 13 | NPG - 3/24 | 64.00 | 65.00 | 92.60 | 0.99 | 7.40 | 62.08 | 17.66 | 2.79 |  | 1.00 |  |
| 14 | NPG - 3/25 | 65.00 | 66.00 | 92.42 | 0.71 | 7.58 | 64.54 | 16.42 | 2.94 |  | 1.00 |  |
| 15 | NPG - 3/26 | 66.00 | 67.00 | 92.88 | 0.94 | 7.12 | 66.04 | 13.64 | 1.85 |  | 1.00 |  |
| 16 | NPG - 3/27 | 67.00 | 68.00 | 89.96 | 1.78 | 10.04 | 61.60 | 17.34 | 2.94 |  | 1.00 |  |
| 17 | NPG - 3/28 | 68.00 | 69.00 | 92.04 | 1.38 | 7.96 | 67.04 | 15.48 | 2.24 |  | 1.00 |  |
| 18 | NPG - 3/29 | 69.00 | 71.00 | 93.84 | 1.24 | 6.16 | 70.16 | 12.76 | 1.84 |  | 2.00 |  |
| 19 | NPG - 3/30 | 71.00 | 71.95 | 93.96 | 1.14 | 6.04 | 66.26 | 15.78 | 2.07 |  | 0.95 |  |
|  |  |  |  |  |  |  |  |  |  |  | 19.55 |  |
|  |  | AVERAGE |  | 90.00 | 1.18 | 7.08 | 62.21 | 14.77 | 2.35 | 2.27 |  |  |

| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bore Hole No- | NPBH-4 |  |  |  |  |  |  |  |  | Date of Commencement |  |  |  |  |  | 07.06.2024 |
| Location( DGPS) | 767887.8965 |  |  |  |  |  | 2182985 |  |  | Date of Completion |  |  |  |  |  | 28.06.2024 |
| (DMS) | 83°33'21.09600" E |  |  |  |  | 19°43'27.30720" N |  |  |  | Total Graphite Thickness |  |  |  |  |  | 18.55 m |
| Hole depth | 56.50 m |  |  |  |  |  |  |  |  | Total Recovery of Core |  |  |  |  |  | 54.32 m |
| R.L | 335.52 m |  |  |  |  |  |  |  |  | Percentage of Core Recovery |  |  |  |  |  | 96.14% |
| Water table depth | 18.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SL.NO | Sample ID | From |  | To |  |  | Ash % | M % |  | LOI % | SiO₂ % | Al₂O₃ % | F.C % | F.C weight avg. | Length | Lithology |
|  | NPG - 4/1 | 11.00 |  | 12.00 |  |  | 88.86 | 3.58 |  | 11.14 | 59.14 | 14.44 | 2.12 | 2.03 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
|  | NPG - 4/2 | 12.00 |  | 13.05 |  |  | 90.76 | 3.11 |  | 9.24 | 66.22 | 12.30 | 1.95 |  | 1.05 |  |
|  | NPG - 4/7 | 21.00 |  | 22.00 |  |  | 89.82 | 2.45 |  | 10.18 | 65.58 | 9.72 | 3.11 | 2.15 | 1.00 | Khondalite + Graphite |
|  | NPG - 4/8 | 22.00 |  | 23.00 |  |  | 93.34 | 1.78 |  | 6.66 | 66.30 | 12.42 | 1.19 |  | 1.00 |  |
|  | NPG - 4/15 | 31.00 |  | 32.00 |  |  | 90.18 | 2.31 |  | 9.82 | 66.02 | 11.80 | 2.33 | 2.03 | 1.00 | Khondalite + Graphite |
|  | NPG - 4/16 | 32.00 |  | 33.00 |  |  | 90.62 | 1.00 |  | 9.38 | 67.68 | 11.56 | 2.38 |  | 1.00 |  |
|  |  | 33.00 |  | 33.50 |  |  | 0.00 | 0.00 |  | 0.00 | 0.00 | 0.00 | 0.00 |  | 0.50 |  |
|  | NPG - 4/17 | 33.50 |  | 34.00 |  |  | 91.42 | 1.21 |  | 8.58 | 72.46 | 10.30 | 2.74 |  | 0.50 |  |
|  | NPG - 4/20 | 36.00 |  | 37.00 |  |  | 91.44 | 1.15 |  | 8.56 | 68.32 | 14.90 | 2.25 | 4.11 | 1.00 | Quartzofeldspathic Gneiss + Graphite |
|  | NPG - 4/21 | 37.00 |  | 38.00 |  |  | 89.50 | 0.97 |  | 10.50 | 70.78 | 8.50 | 4.60 |  | 1.00 |  |
|  | NPG - 4/22 | 38.00 |  | 39.00 |  |  | 87.72 | 2.48 |  | 12.28 | 66.42 | 7.68 | 5.47 |  | 1.00 |  |
|  | NPG - 4/23 | 40.50 |  | 41.50 |  |  | 85.24 | 3.89 |  | 14.76 | 54.36 | 8.50 | 4.15 | 6.39 | 1.00 | Khondalite + Graphite |
|  | NPG - 4/24 | 41.50 |  | 42.50 |  |  | 89.48 | 0.20 |  | 10.52 | 81.44 | 0.50 | 8.16 |  | 1.00 |  |
|  | NPG - 4/25 | 42.50 |  | 43.50 |  |  | 89.60 | 0.21 |  | 10.40 | 82.70 | 1.80 | 8.06 |  | 1.00 |  |
|  | NPG - 4/26 | 43.50 |  | 44.50 |  |  | 87.48 | 0.30 |  | 12.52 | 78.62 | 3.00 | 10.03 |  | 1.00 |  |
|  | NPG - 4/27 | 44.50 |  | 45.50 |  |  | 87.62 | 0.35 |  | 12.38 | 80.34 | 1.00 | 9.84 |  | 1.00 |  |
|  | NPG - 4/28 | 45.50 |  | 46.30 |  |  | 88.60 | 0.19 |  | 11.40 | 80.72 | 0.60 | 9.26 |  | 0.80 |  |
|  | NPG - 4/29 | 46.30 |  | 47.00 |  |  | 91.74 | 0.69 |  | 8.26 | 62.38 | 13.48 | 2.36 |  | 0.70 |  |
|  | NPG - 4/30 | 47.00 |  | 48.00 |  |  | 90.80 | 0.67 |  | 9.20 | 65.66 | 11.26 | 3.10 |  | 1.00 |  |
|  | NPG - 4/31 | 48.00 |  | 49.00 |  |  | 92.84 | 1.42 |  | 7.16 | 68.82 | 11.86 | 1.95 |  | 1.00 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 18.55 |  |
|  |  | AVERAGE |  |  |  |  | 92.02 | 1.51 |  | 10.40 | 71.37 | 8.93 | 4.58 | 3.34 |  |  |

| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bore Hole No- | NPBH-5 |  |  |  |  | Date of Commencement |  |  |  |  |  | 09.06.2024 |
| Location( DGPS) | 76910.4273 |  |  | 2182829.996 |  | Date of Completion |  |  |  |  |  | 10.07.2024 |
| (DMS) | 83°33'21.79080" E |  |  | 19°43'22.26000" N |  | Total Graphite Thickness |  |  |  |  |  | 5.50 m |
| Hole depth | 78.80 m |  |  |  |  | Total Recovery of Core |  |  |  |  |  | 75.98 m |
| R.L | 329.86 m |  |  |  |  | Percentage of Core Recovery |  |  |  |  |  | 96.42% |
| Water table depth | 18.00 m |  |  |  |  |  |  |  |  |  |  |  |
| SL.NO | Sample ID | From | To | Ash % | M % | LOI % | SiO₂ % | AI₂O₃ % | F.C % | F.C weight avg. | Length | Lithology |
| 1 | NPG - 5/16 | 60.50 | 61.50 | 91.76 | 1.22 | 8.26 | 62.70 | 14.02 | 2.94 | 2.94 | 1.00 | Khondalite + Graphite |
| 2 | NPG - 5/18 | 63.00 | 64.00 | 93.60 | 1.03 | 6.40 | 66.86 | 13.82 | 2.53 | 2.26 | 1.00 | Khondalite + Graphite |
| 3 | NPG - 5/19 | 64.00 | 65.00 | 94.52 | 1.08 | 5.48 | 67.84 | 12.86 | 1.76 |  | 1.00 |  |
| 4 | NPG - 5/20 | 65.00 | 66.00 | 93.66 | 1.31 | 6.34 | 66.12 | 12.56 | 2.50 |  | 1.00 |  |
| 5 | NPG - 5/28 | 73.50 | 75.00 | 93.24 | 1.90 | 6.76 | 67.16 | 13.16 | 2.35 | 2.35 | 1.50 | Quartzofeldspathic Gneiss + Graphite |
|  |  |  |  |  |  |  |  |  |  |  | 5.50 |  |
|  |  | AVERAGE |  | 84.87 | 1.19 | 6.04 | 60.12 | 12.08 | 2.20 | 2.52 |  |  |

| Physical & Chemical Assay Log of Naringpanga Graphite Project, F.S.2023-24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bore Hole No- | NPBH-6 |  |  |  |  |  |  |  |  |  |  | Date of Commencement |  |  |  |  |  |  | 05.07.2024 |
| Location( DGPS) | 767975.6609 |  |  |  |  | 2182906.237 |  |  |  |  |  | Date of Completion |  |  |  |  |  |  | 27.07.2024 |
| (DMS) | 83°33'24.06960" E |  |  |  |  | 19°43'24.70440" N |  |  |  |  |  | Total Graphite Thickness |  |  |  |  |  |  | 6.50 m |
| Hole depth | 63.20 m |  |  |  |  |  |  |  |  |  |  | Total Recovery of Core |  |  |  |  |  |  | 60.5 m |
| R.L | 334.14 m |  |  |  |  |  |  |  |  |  |  | Percentage of Core Recovery |  |  |  |  |  |  | 95.72% |
| Water table depth | 16.00m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SL.NO | Sample ID | From |  | To |  | Ash % |  | M % |  | V.M % |  | LOI % | SiO₂ % | AI₂O₃ % | V₂O₅(%) | F.C % | F.C weight avg. | Length | Lithology |
| 1 | NPG 6/3 | 25.00 |  | 26.00 |  | 90.00 |  | 3.49 |  | 4.69 |  | 10.00 | 61.20 | 17.10 | 0.00 | 1.82 | 2.87 | 1.00 | Khondalite + Graphite |
| 2 | NPG 6/4 | 26.00 |  | 27.00 |  | 86.62 |  | 4.81 |  | 4.66 |  | 13.38 | 62.02 | 16.05 | 0.00 | 3.91 |  | 1.00 |  |
| 3 | NPG 6/5 | 27.00 |  | 28.00 |  | 90.20 |  | 3.74 |  | 3.46 |  | 9.81 | 69.86 | 12.16 | 0.00 | 2.61 |  | 1.00 |  |
| 4 | NPG 6/6 | 28.00 |  | 29.00 |  | 88.33 |  | 4.66 |  | 4.42 |  | 11.68 | 66.72 | 13.74 | 0.00 | 2.60 |  | 1.00 |  |
| 5 | NPG 6/7 | 29.00 |  | 30.00 |  | 86.84 |  | 5.19 |  | 5.09 |  | 13.17 | 61.94 | 16.16 | 0.00 | 2.89 |  | 1.00 |  |
| 6 | NPG 6/8 | 30.00 |  | 31.00 |  | 85.56 |  | 5.31 |  | 5.76 |  | 14.45 | 57.56 | 16.18 | 0.00 | 3.38 |  | 1.00 |  |
| 7 | NPG 6/12 | 51.20 |  | 51.70 |  | 91.31 |  | 1.51 |  | 4.79 |  | 8.70 | 63.38 | 15.72 | 0.00 | 2.40 | 2.40 | 0.50 | Quartzofeldspathic Gneiss + Graphite |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.50 |  |
|  |  | AVERAGE |  |  |  | 95.21 |  | 4.42 |  | 5.06 |  | 12.49 | 68.11 | 16.48 | 0.00 | 3.01 | 2.63 |  |  |

| Naringpanga Graphite Block, Rayagada, F.S.2023-24 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section wise Graphite average |  |  |  |  |  |  |  |  |  |
|  | Borehole | Average (%) |  |  |  |  |  |  |  |
|  |  | M | Al2O3 | SiO2 | Ash | LOI | F.C | F.C weighted avg. |  |
| Section-AA’ | NPBH-1 | 1.81 | 11.06 | 67.74 | 89.35 | 8.41 | 3.7 | 3.21 |  |
|  | Section average | 1.81 | 11.06 | 67.74 | 89.35 | 8.41 | 3.7 | 3.21 |  |
| Section-B-B’ | NPBH -2 | 1.32 | 13.31 | 67.8 | 95.87 | 7.76 | 2.21 | 2.19 |  |
|  | Section average | 1.32 | 13.31 | 67.8 | 95.87 | 7.76 | 2.21 | 2.19 |  |
| Section-C-C’ | NPBH -4 | 1.51 | 8.93 | 71.37 | 92.02 | 10.40 | 4.58 | 3.34 |  |
|  | Section average | 1.51 | 8.93 | 71.37 | 92.02 | 10.40 | 4.58 | 3.34 |  |
| Section-D-D’ | NPBH -3 | 1.18 | 14.77 | 62.21 | 90.00 | 7.08 | 2.35 | 2.27 |  |
|  | Section average | 1.18 | 14.77 | 62.21 | 90.00 | 7.08 | 2.35 | 2.27 |  |
| Section-E-E’ | NPBH -5 | 1.19 | 12.08 | 60.12 | 84.87 | 6.04 | 2.20 | 2.52 |  |
|  | NPBH -6 | 4.42 | 16.48 | 68.12 | 95.20 | 12.49 | 3.01 | 2.63 |  |
|  | Section average | 14.28 | 64.12 | 90.04 | 9.26 | 2.60 | 2.80 | 2.57 |  |
| Graphite Average of PIT-2 |  |  |  |  |  |  |  |  |  |
| PIT-2 | Average | 1.96 | 19.44 | 50.20 | 87.40 | 12.60 | 2.82 | 2.82 |  |

ANNEXURE-IV

ANNEXURE-V

External Check Sample Analysis results of NARINGPANGA Graphite Block, Rayagada District

N

| SI. No. | Sample Identity No. | BH No. | Ash % | M % | F.C. % | LOI % | SiO2 % | Al2O3% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NCG - 1 | NPBH-1/1 | 88.63 | 0.32 | 7.79 | 11.99 | 40.91 | 0.21 |
| 2 | NCG - 4 | NPBH-1/28 | 93.15 | 1.16 | 0.31 | 7.49 | 59.04 | 0.26 |
| 3 | NCG - 6 | NPBH-2/12 | 93.00 | 0.53 | 1.06 | 7.82 | 40.28 | 0.15 |
| 4 | NCG - 7 | NPBH-2/22 | 91.86 | 0.59 | 0.93 | 8.53 | 52.53 | 0.16 |
| 5 | NCG - 8 | NPBH-3/3 | 87.14 | 1.59 | 1.72 | 13.84 | 44.54 | 0.26 |
| 6 | NCG - 10 | NPBH-3/30 | 94.52 | 0.58 | 0.75 | 6.13 | 43.55 | 0.54 |
| 7 | NCG - 13 | NPBH-4/26 | 87.97 | 0.37 | 9.03 | 12.35 | 40.67 | 0.11 |
| 8 | NCG - 14 | NPBH-4/28 | 89.02 | 0.30 | 8.48 | 11.27 | 39.86 | 0.19 |
| 9 | NCG - 16 | NPBH-5/20 | 94.08 | 0.51 | 1.68 | 6.34 | 41.26 | 0.62 |
| 10 | NCG - 19 | NPBH-6/8 | 86.93 | 3.30 | 0.87 | 12.35 | 50.78 | 0.57 |

ANNEXURE-VI

Details of Internal Check Sample Analysis results of Naringpanga Graphite Block,  Rayagada District.

N

| SI. No. | Sample Identity No. | BH No. | Ash % | M % | F.C. % | LOI % | SiO2 % | Al2O3% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NCG - 1 | NPBH-1/1 | 88.56 | 1.00 | 8.06 | 11.43 | 80.85 | 2.49 |
| 2 | NCG - 2 | NPBH-1/14 | 91.06 | 2.23 | 1.04 | 8.94 | 66.83 | 13.69 |
| 3 | NCG - 3 | NPBH-1/18 | 92.18 | 2.43 | 2.05 | 7.82 | 63.02 | 17.10 |
| 4 | NCG - 4 | NPBH-1/28 | 93.02 | 2.26 | 1.31 | 6.98 | 65.94 | 13.76 |
| 5 | NCG - 5 | NPBH-2/6 | 91.06 | 1.99 | 2.96 | 8.94 | 65.64 | 13.68 |
| 6 | NCG - 6 | NPBH-2/12 | 92.40 | 1.52 | 2.30 | 7.60 | 65.59 | 15.64 |
| 7 | NCG - 7 | NPBH-2/22 | 91.56 | 1.48 | 2.15 | 8.43 | 64.05 | 15.52 |
| 8 | NCG - 8 | NPBH-3/3 | 86.34 | 4.88 | 3.86 | 13.65 | 54.74 | 14.60 |
| 9 | NCG - 9 | NPBH-3/18 | 93.44 | 0.85 | 2.5 | 6.56 | 69.27 | 13.25 |
| 10 | NCG - 10 | NPBH-3/30 | 94.00 | 1.23 | 1.51 | 6.00 | 68.58 | 13.34 |
| 11 | NCG - 11 | NPBH-4/16 | 90.50 | 2.65 | 2.10 | 9.49 | 66.53 | 13.58 |
| 12 | NCG - 12 | NPBH-4/21 | 89.22 | 1.70 | 4.29 | 10.78 | 70.94 | 12.33 |
| 13 | NCG - 13 | NPBH-4/26 | 87.74 | 0.69 | 9.63 | 12.25 | 81.42 | 2.76 |
| 14 | NCG - 14 | NPBH-4/28 | 88.82 | 0.63 | 9.02 | 11.18 | 83.00 | 2.74 |
| 15 | NCG - 15 | NPBH-5/16 | 93.03 | 1.35 | 2.64 | 6.96 | 62.90 | 17.73 |
| 16 | NCG - 16 | NPBH-5/20 | 93.70 | 1.24 | 2.46 | 6.29 | 67.81 | 13.94 |
| 17 | NCG - 17 | NPBH-5/28 | 93.48 | 1.57 | 2.28 | 6.52 | 68.69 | 13.76 |
| 18 | NCG - 18 | NPBH-6/4 | 87.49 | 4.39 | 3.17 | 12.51 | 62.23 | 16.66 |
| 19 | NCG - 19 | NPBH-6/8 | 85.34 | 5.15 | 3.45 | 14.65 | 58.23 | 15.49 |
| 20 | NCG - 20 | NPBH-6/12 | 91.46 | 1.99 | 2.17 | 8.53 | 63.22 | 15.38 |

ANNEXURE-VII

| SI No. | Identity Mark | Lab No. | Ash% | M% | V.M% | F.C% | LOI% | SiO2 % | Al2O3% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NPC1/1C | 865-P/24 | 91.66 | 2.06 | 3.32 | 2.96 | 8.34 | 69.93 | 11.29 |
| 2 | NPC1/2C | 866-P/24 | 93.34 | 1.69 | 3.08 | 1.89 | 6.66 | 67.99 | 12.93 |
|  | NPBH-1 | Average | 92.50 | 1.88 | 3.20 | 2.43 | 7.50 | 68.96 | 12.11 |
| 3 | NPC2/1C | 867-P/24 | 93.80 | 1.50 | 3.26 | 1.44 | 6.20 | 70.54 | 12.52 |
| 4 | NPC2/2C | 868-P/24 | 93.00 | 1.29 | 4.50 | 1.21 | 7.00 | 64.06 | 16.21 |
|  | NPBH-2 | Average | 93.40 | 1.40 | 3.88 | 1.33 | 6.60 | 67.30 | 14.37 |
| 5 | NPC3/1C | 869-P/24 | 93.56 | 1.33 | 3.20 | 1.91 | 6.44 | 65.70 | 15.86 |
| 6 | NPC3/2C | 870-P/24 | 93.16 | 1.00 | 3.25 | 2.59 | 6.84 | 67.90 | 14.09 |
|  | NPBH-3 | Average | 93.36 | 1.17 | 3.23 | 2.25 | 6.64 | 66.80 | 14.98 |
| 7 | NPC4/1C | 871-P/24 | 91.98 | 2.04 | 4.45 | 1.53 | 8.02 | 66.05 | 14.59 |
| 8 | NPC4/2C | 872-P/24 | 90.16 | 1.40 | 3.45 | 4.99 | 9.84 | 72.70 | 9.94 |
|  | NPBH-4 | Average | 91.07 | 1.72 | 3.95 | 3.26 | 8.93 | 69.37 | 12.26 |
| 9 | NPC5/1C | 873-P/24 | 95.26 | 1.03 | 2.35 | 1.36 | 4.74 | 69.76 | 14.19 |
| 10 | NPC5/2C | 874-P/24 | 94.96 | 1.06 | 2.51 | 1.47 | 5.04 | 70.20 | 12.97 |
|  | NPBH-5 | Average | 95.11 | 1.05 | 2.43 | 1.42 | 4.89 | 69.98 | 13.58 |
| 11 | NPBH-6 | 875-P/24 | 87.14 | 2.82 | 4.65 | 5.39 | 12.86 | 63.66 | 16.12 |

COMPOSITE SAMPLE RESULT OF NARINGPANGA (S) GRAPHITE BLOCK

ANNEXURE-VIII

CROSS SECTION WISE BOREHOLE DETAILS OF NARINGPANGA BLOCK

| NARINGPANGA GRAPHITE BLOCK |  |  |
| --- | --- | --- |
| Section | Boreholes |  |
| A-A' | NPBH-1 |  |
| B-B' | NPBH-2 |  |
| C-C' | NPBH-4 |  |
| D-D' | NPBH-3 |  |
| E-E' | NPBH-5 | NPBH-6 |

ANNEXURE-IX

RESOURCE CALCULATION BY CROSS-SECTIONAL METHOD

| Ore category | Section area (m2) | Length of influence (m) | Volume (m3) | Recovery Factor (%) | Bulk density | Resource(t) |
| --- | --- | --- | --- | --- | --- | --- |
|  | A-A' |  |  |  |  |  |
| FC (2-5%) | 2172.95 | 38 | 82572.1 | 70 | 1.606 | 92827.55 |
|  | B-B' |  |  |  |  |  |
| FC (2-5%) | 560.25 | 85 | 47621.25 | 70 | 1.606 | 53535.81 |
|  | C-C' |  |  |  |  |  |
| FC (2-5%) | 1211.37 | 82 | 99332.34 | 70 | 1.606 | 111669.40 |
|  | D-D' |  |  |  |  |  |
| FC (2-5%) | 1212.81 | 66 | 80045.46 | 70 | 1.606 | 89987.11 |
|  | E-E' |  |  |  |  |  |
| FC (2-5%) | 586.05 | 98 | 57432.9 | 70 | 1.606 | 64566.07 |
|  |  |  | 367004.05 | Total (A) |  | 412585.95 |

PIT-2 (Surface method)

| Ore category | Surface area ( m2) | Thickness(m) | Volume (m3) | Recovery Factor (%) | Bulk density | Resource (t) |
| --- | --- | --- | --- | --- | --- | --- |
| FC (2-5%) | 2500 | 0.3 | 750 | 70 | 1.606 | 843.15 |
| Total (B) |  |  |  |  |  | 843.15 |
| Grand Total resource of the Block (In metric tonnes): (A+B) in t |  |  |  |  |  | 413429 |
| Grand Total resource of the Block (In Million tonnes) in mt |  |  |  |  |  | 0.41343 |

| Section | Graphite F.C (2-5%) in metric tonnes (t) |
| --- | --- |
| A-A' | 92827.55 |
| B-B' | 53535.81 |
| C-C' | 111669.40 |
| D-D' | 89987.11 |
| E-E' | 64566.07 |
| PIT-2 | 843.15 |
| Total Resource in Metric Tonnes (t) | 413429.1 |
| Total Resource in Million Tonnes (mt) | 0.41343 |

ANNEXURE-X

SUMMARY OF RESOURCE CALCULATION

ANNEXURE-XI

CALCULATION OF ORE: OB OF NARINGPANGA GRAPHITE BLOCK

| CALCULATION OF ORE:OB OF NARINGPANGA GRAPHITE BLOCK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Section | Sectional area of ore in m2 | Over burden Sectional area in m2 | Strike Influence in m | Volume of Ore in m3 | volume of over burden in m3 | Ore : OB |
| AA' | 2172.95 | 391.61 | 38 | 82572.1 | 14881.18 | 0.180 |
| BB' | 560.25 | 348.33 | 85 | 47621.25 | 29608.05 | 0.622 |
| CC' | 1211.37 | 609.54 | 82 | 99332.34 | 49982.28 | 0.503 |
| DD' | 1212.81 | 404.82 | 66 | 80045.46 | 26718.12 | 0.334 |
| EE’ | 586.05 | 719.25 | 98 | 57432.9 | 70486.5 | 1.227 |
|  | Sectional area of ore in m2 | Over burden Sectional area in m2 | Strike Influence in (m) | Volume of Ore in m3 | Volume of over burden in m3 | Ore : OB |
| PIT-2 | 15 | 185 | 50 | 750 | 9250 | 12.333 |
| TOTAL |  |  |  | 367754.05 | 200926.13 | 0.546 |
| ORE:OB |  |  |  |  |  | 1:0.546 |

ANNEXURE-XII

CHEMICAL ANALYSIS OF 160 GRAPHITE POWDERED SAMPLE OF NARINGPANGA (S) BLOCK, RAYAGADA DISTRICT, ODISHA

|  | NPBH-1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SL. No. | POWDERED SAMPLE | FROM | TO | LENGTH | R.L. No. | Ash % | M % | V.M % | F.C % | LOI % | SiO2 % | AI2O3 % | V2O5 (%) |
| 1 | NPG-1/1 | 12.00 | 13.00 | 1.00 | 834-P/24 | 88.63 | 1.15 | 2.38 | 7.84 | 11.37 | 80.78 | 2.58 | 0.00 |
| 2 | NPG-1/2 | 13.00 | 14.00 | 1.00 | 835-P/24 | 89.77 | 0.71 | 1.93 | 7.59 | 10.23 | 82.87 | 2.72 | 0.00 |
| 3 | NPG-1/3 | 21.00 | 22.00 | 1.00 | 836-P/24 | 86.59 | 2.97 | 3.68 | 6.76 | 13.41 | 68.94 | 5.58 | 0.00 |
| 4 | NPG-1/4 | 22.00 | 23.00 | 1.00 | 837-P/24 | 89.08 | 2.70 | 4.31 | 3.92 | 10.93 | 72.80 | 5.32 | 0.00 |
| 5 | NPG-1/5 | 23.00 | 24.00 | 1.00 | 838-P/24 | 91.38 | 2.11 | 3.99 | 2.53 | 8.63 | 75.94 | 6.68 | 0.01 |
| 6 | NPG-1/6 | 24.00 | 25.00 | 1.00 | 839-P/24 | 90.32 | 1.24 | 3.66 | 4.79 | 9.69 | 74.92 | 7.09 | 0.01 |
| 7 | NPG-1/7 | 25.00 | 26.00 | 1.00 | 840-P/24 | 85.49 | 4.68 | 5.73 | 4.10 | 14.50 | 54.57 | 15.80 | 0.01 |
| 8 | NPG-1/8 | 26.00 | 27.00 | 1.00 | 841-P/24 | 85.55 | 1.79 | 5.41 | 7.25 | 14.45 | 61.66 | 12.77 | 0.00 |
| 9 | NPG-1/9 | 27.00 | 28.00 | 1.00 | 842-P/24 | 93.91 | 1.65 | 1.01 | 3.44 | 6.10 | 74.90 | 8.42 | 0.00 |
| 10 | NPG-1/10 | 28.30 | 29.30 | 1.00 | 843-P/24 | 94.29 | 1.38 | 1.06 | 3.28 | 5.72 | 68.95 | 14.09 | 0.00 |
| 11 | NPG-1/11 | 29.30 | 30.30 | 1.00 | 844-P/24 | 94.43 | 1.01 | 1.09 | 3.48 | 5.58 | 71.40 | 14.42 | 0.00 |
| 12 | NPG-1/12 | 30.30 | 31.00 | 0.70 | 845-P/24 | 94.10 | 1.42 | 0.98 | 3.51 | 5.91 | 70.28 | 13.82 | 0.00 |
| 13 | NPG-1/13 | 31.85 | 32.85 | 1.00 | 846-P/24 | 91.12 | 2.40 | 4.08 | 2.40 | 8.88 | 64.76 | 13.31 | 0.00 |
| 14 | NPG-1/14 | 32.85 | 34.00 | 1.15 | 847-P/24 | 91.18 | 2.33 | 1.12 | 5.37 | 8.82 | 66.62 | 13.75 | 0.00 |
| 15 | NPG-1/15 | 42.50 | 43.50 | 1.00 | 848-P/24 | 94.56 | 1.79 | 2.42 | 1.23 | 5.44 | 64.52 | 16.20 | 0.00 |
| 16 | NPG-1/16 | 43.50 | 44.30 | 0.80 | 849-P/24 | 92.46 | 2.31 | 3.04 | 2.19 | 7.54 | 63.27 | 17.76 | 0.00 |
| 17 | NPG-1/17 | 45.15 | 45.65 | 0.50 | 850-P/24 | 95.44 | 1.12 | 2.33 | 1.11 | 4.56 | 69.05 | 13.16 | 0.00 |
| 18 | NPG-1/18 | 52.10 | 53.10 | 1.00 | 851-P/24 | 92.30 | 1.75 | 3.95 | 2.00 | 7.70 | 63.24 | 17.09 | 0.00 |
| 19 | NPG-1/19 | 53.10 | 54.10 | 1.00 | 852-P/24 | 94.64 | 1.67 | 2.37 | 1.32 | 5.36 | 66.04 | 16.18 | 0.00 |
| 20 | NPG-1/20 | 54.10 | 55.10 | 1.00 | 853-P/24 | 92.40 | 2.13 | 3.40 | 2.07 | 7.60 | 67.35 | 13.56 | 0.00 |
| 21 | NPG-1/21 | 55.10 | 55.70 | 0.60 | 854-P/24 | 93.94 | 1.18 | 3.31 | 1.57 | 6.06 | 72.53 | 9.74 | 0.00 |
| 22 | NPG-1/22 | 56.35 | 57.35 | 1.00 | 855-P/24 | 92.98 | 1.48 | 2.95 | 2.59 | 7.02 | 68.21 | 12.90 | 0.00 |
| 23 | NPG-1/23 | 57.35 | 58.30 | 0.95 | 856-P/24 | 94.10 | 1.43 | 2.39 | 2.08 | 5.90 | 72.24 | 10.41 | 0.00 |
| 24 | NPG-1/24 | 59.00 | 60.00 | 1.00 | 857-P/24 | 93.28 | 1.90 | 2.89 | 1.93 | 6.72 | 68.30 | 12.69 | 0.00 |
| 25 | NPG-1/25 | 60.00 | 61.00 | 1.00 | 858-P/24 | 92.98 | 1.66 | 3.18 | 2.18 | 7.02 | 67.34 | 12.88 | 0.00 |
| 26 | NPG-1/26 | 61.00 | 62.00 | 1.00 | 859-P/24 | 93.08 | 1.72 | 3.29 | 1.91 | 6.92 | 69.16 | 11.01 | 0.00 |
| 27 | NPG-1/27 | 62.00 | 63.30 | 1.30 | 860-P/24 | 90.40 | 2.48 | 4.25 | 2.87 | 9.60 | 61.94 | 13.89 | 0.00 |
| 28 | NPG-1/28 | 65.00 | 65.80 | 0.80 | 861-P/24 | 93.02 | 2.26 | 3.33 | 1.31 | 6.98 | 65.94 | 13.76 | 0.00 |
| 29 | NPG-1/29 | 67.00 | 68.00 | 1.00 | 862-P/24 | 92.64 | 1.75 | 4.06 | 1.55 | 7.36 | 67.37 | 13.20 | 0.00 |
| 30 | NPG-1/30 | 68.00 | 69.00 | 1.00 | 863-P/24 | 94.50 | 0.98 | 1.95 | 2.57 | 5.50 | 67.06 | 14.21 | 0.00 |
| 31 | NPG-1/31 | 69.00 | 69.50 | 0.50 | 864-P/24 | 92.52 | 1.04 | 2.30 | 4.14 | 7.48 | 73.73 | 11.50 | 0.00 |

|  | NPBH NO-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Sample No. | R.L. No. | FROM | TO | Ash % | M % | F.C % | LOI % | SiO2 % | AI2O3 % |
| 1 | NPG - 2/1 | 518-P/24 | 6.20 | 7.20 | 93.16 | 2.10 | 0.79 | 6.84 | 67.86 | 9.54 |
| 2 | NPG - 2/2 | 519-P/24 | 7.20 | 8.20 | 90.44 | 3.51 | 0.96 | 9.06 | 62.86 | 11.28 |
| 3 | NPG - 2/3 | 520-P/24 | 8.20 | 9.50 | 92.64 | 2.08 | 1.20 | 7.36 | 65.58 | 8.74 |
| 4 | NPG - 2/4 | 521-P/24 | 12.00 | 13.00 | 91.80 | 1.55 | 2.04 | 8.20 | 63.02 | 14.44 |
| 5 | NPG - 2/5 | 522-P/24 | 13.00 | 13.50 | 93.92 | 1.27 | 1.20 | 6.08 | 66.18 | 12.42 |
| 6 | NPG - 2/6 | 523-P/24 | 15.60 | 16.50 | 91.30 | 1.91 | 2.66 | 8.70 | 64.90 | 12.94 |
| 7 | NPG - 2/7 | 524-P/24 | 17.50 | 18.00 | 92.30 | 1.80 | 2.14 | 7.70 | 67.20 | 10.72 |
| 8 | NPG - 2/8 | 525-P/24 | 19.00 | 20.50 | 94.40 | 1.17 | 2.01 | 5.60 | 70.02 | 6.70 |
| 9 | NPG - 2/9 | 526-P/24 | 21.90 | 22.90 | 96.18 | 0.63 | 1.50 | 3.82 | 74.68 | 13.50 |
| 10 | NPG - 2/10 | 527-P/24 | 22.90 | 23.90 | 96.46 | 0.99 | 0.98 | 3.54 | 70.74 | 12.50 |
| 11 | NPG - 2/11 | 528-P/24 | 24.50 | 25.30 | 95.26 | 1.01 | 1.36 | 4.74 | 72.46 | 9.70 |
| 12 | NPG - 2/12 | 529-P/24 | 26.50 | 27.00 | 92.20 | 1.56 | 2.39 | 7.80 | 64.76 | 13.94 |
| 13 | NPG - 2/13 | 530-P/24 | 41.45 | 42.45 | 92.88 | 0.91 | 2.09 | 7.12 | 63.54 | 15.26 |
| 14 | NPG - 2/14 | 531-P/24 | 42.45 | 43.45 | 93.18 | 0.40 | 1.80 | 6.82 | 68.34 | 10.94 |
| 15 | NPG - 2/15 | 532-P/24 | 43.45 | 44.45 | 91.88 | 1.04 | 2.24 | 8.12 | 60.50 | 17.76 |
| 16 | NPG - 2/16 | 533-P/24 | 44.45 | 45.45 | 93.34 | 1.06 | 1.88 | 6.66 | 67.72 | 13.72 |
| 17 | NPG - 2/17 | 534-P/24 | 45.45 | 46.45 | 93.90 | 0.76 | 1.61 | 6.10 | 67.00 | 11.38 |
| 18 | NPG - 2/18 | 535-P/24 | 46.45 | 47.45 | 92.96 | 0.82 | 1.63 | 7.04 | 66.66 | 11.02 |
| 19 | NPG - 2/19 | 536-P/24 | 47.45 | 48.45 | 93.00 | 0.93 | 1.98 | 7.00 | 63.66 | 15.46 |
| 20 | NPG - 2/20 | 537-P/24 | 48.45 | 49.45 | 92.44 | 1.50 | 1.47 | 7.56 | 61.20 | 17.56 |
| 21 | NPG - 2/21 | 538-P/24 | 49.45 | 50.45 | 94.40 | 1.23 | 0.58 | 5.60 | 66.48 | 12.38 |
| 22 | NPG - 2/22 | 539-P/24 | 50.45 | 51.70 | 91.88 | 1.33 | 2.00 | 8.12 | 64.30 | 12.02 |
| 23 | NPG - 2/23 | 540-P/24 | 53.00 | 54.00 | 92.04 | 2.06 | 0.48 | 7.96 | 61.14 | 19.68 |
| 24 | NPG - 2/24 | 541-P/24 | 54.50 | 55.00 | 94.58 | 1.52 | 0.53 | 5.42 | 58.94 | 18.02 |

|  | NPBH NO-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Sample No. | R.L. No. | FROM | TO | Ash % | M % | F.C % | LOI % | SiO2 % | AI2O3 % |
| 1 | NPG - 3/1 | 542-P/24 | 15.50 | 16.30 | 93.48 | 2.49 | 1.39 | 6.52 | 64.34 | 15.58 |
| 2 | NPG - 3/2 | 543-P/24 | 18.50 | 19.50 | 96.30 | 0.78 | 0.98 | 3.70 | 72.08 | 15.16 |
| 3 | NPG - 3/3 | 544-P/24 | 20.50 | 21.10 | 86.00 | 5.14 | 3.93 | 12.00 | 55.06 | 14.32 |
| 4 | NPG - 3/4 | 545-P/24 | 41.60 | 42.60 | 95.94 | 0.80 | 1.18 | 4.06 | 67.48 | 12.74 |
| 5 | NPG - 3/5 | 546-P/24 | 42.60 | 43.60 | 94.78 | 0.85 | 0.93 | 5.22 | 63.28 | 18.44 |
| 6 | NPG - 3/6 | 547-P/24 | 43.60 | 44.60 | 93.34 | 1.55 | 1.26 | 6.66 | 64.50 | 19.20 |
| 7 | NPG - 3/7 | 548-P/24 | 44.60 | 45.60 | 93.20 | 1.04 | 1.55 | 6.80 | 64.46 | 17.74 |
| 8 | NPG - 3/8 | 549-P/24 | 45.60 | 46.60 | 93.86 | 1.35 | 1.02 | 6.14 | 65.90 | 16.44 |
| 9 | NPG - 3/9 | 550-P/24 | 46.60 | 47.60 | 94.18 | 0.85 | 1.33 | 5.82 | 65.80 | 16.22 |
| 10 | NPG - 3/10 | 551-P/24 | 47.60 | 48.60 | 94.42 | 0.78 | 1.27 | 5.58 | 62.58 | 19.78 |
| 11 | NPG - 3/11 | 552-P/24 | 48.60 | 49.60 | 94.28 | 0.85 | 1.65 | 5.72 | 65.40 | 16.30 |
| 12 | NPG - 3/12 | 553-P/24 | 49.60 | 50.60 | 93.36 | 0.84 | 2.37 | 6.64 | 63.12 | 17.96 |
| 13 | NPG - 3/13 | 554-P/24 | 50.60 | 51.60 | 93.06 | 0.93 | 1.96 | 6.94 | 59.74 | 16.42 |
| 14 | NPG - 3/14 | 555-P/24 | 51.60 | 52.60 | 92.78 | 0.87 | 2.57 | 7.22 | 63.58 | 16.48 |
| 15 | NPG - 3/15 | 556-P/24 | 52.60 | 53.60 | 93.00 | 0.93 | 2.43 | 7.00 | 63.10 | 16.06 |
| 16 | NPG - 3/16 | 557-P/24 | 53.60 | 54.60 | 94.78 | 1.04 | 1.21 | 5.22 | 63.62 | 18.30 |
| 17 | NPG - 3/17 | 558-P/24 | 54.60 | 55.60 | 94.02 | 0.64 | 1.74 | 5.98 | 65.92 | 17.84 |
| 18 | NPG - 3/18 | 559-P/24 | 55.60 | 56.60 | 93.08 | 0.81 | 2.64 | 6.92 | 66.34 | 12.08 |
| 19 | NPG - 3/19 | 560-P/24 | 56.60 | 57.60 | 92.90 | 0.95 | 2.73 | 7.10 | 65.22 | 16.34 |
| 20 | NPG - 3/20 | 561-P/24 | 57.60 | 58.60 | 95.04 | 0.95 | 1.33 | 4.96 | 65.56 | 16.08 |
| 21 | NPG - 3/21 | 562-P/24 | 61.00 | 62.00 | 93.30 | 0.84 | 2.08 | 6.70 | 65.60 | 16.30 |
| 22 | NPG - 3/22 | 563-P/24 | 62.00 | 63.00 | 92.12 | 0.96 | 2.82 | 7.80 | 60.78 | 11.78 |
| 23 | NPG - 3/23 | 564-P/24 | 63.00 | 64.00 | 92.80 | 0.86 | 2.81 | 7.20 | 65.00 | 9.54 |
| 24 | NPG - 3/24 | 565-P/24 | 64.00 | 65.00 | 92.60 | 0.99 | 2.79 | 7.40 | 62.08 | 17.66 |
| 25 | NPG - 3/25 | 566-P/24 | 65.00 | 66.00 | 92.42 | 0.71 | 2.94 | 7.58 | 64.54 | 16.42 |
| 26 | NPG - 3/26 | 567-P/24 | 66.00 | 67.00 | 92.88 | 0.94 | 1.85 | 7.12 | 66.04 | 13.64 |
| 27 | NPG - 3/27 | 568-P/24 | 67.00 | 68.00 | 89.96 | 1.78 | 2.94 | 10.04 | 61.60 | 17.34 |
| 28 | NPG - 3/28 | 569-P/24 | 68.00 | 69.00 | 92.04 | 1.38 | 2.24 | 7.96 | 67.04 | 15.48 |
| 29 | NPG - 3/29 | 570-P/24 | 69.00 | 71.00 | 93.84 | 1.24 | 1.84 | 6.16 | 70.16 | 12.76 |
| 30 | NPG - 3/30 | 571-P/24 | 71.00 | 71.95 | 93.96 | 1.14 | 2.07 | 6.04 | 66.26 | 15.78 |
| 31 | NPG - 3/31 | 572-P/24 | 72.50 | 73.40 | 95.16 | 1.09 | 1.50 | 4.84 | 71.96 | 13.58 |

|  | NPBH-4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Sample No. | R.L. No. | FROM | TO | Ash % | M % | F.C % | LOI % | SiO2 % | AI2O3 % |
| 1 | NPG - 4/1 | 573-P/24 | 11.00 | 12.00 | 88.86 | 3.58 | 2.12 | 11.14 | 59.14 | 14.44 |
| 2 | NPG - 4/2 | 574-P/24 | 12.00 | 13.05 | 90.76 | 3.11 | 1.95 | 9.24 | 66.22 | 12.30 |
| 3 | NPG - 4/3 | 575-P/24 | 17.00 | 18.00 | 93.24 | 1.68 | 0.91 | 6.76 | 66.84 | 11.28 |
| 4 | NPG - 4/4 | 576-P/24 | 18.00 | 19.00 | 90.56 | 2.08 | 1.52 | 9.44 | 60.82 | 17.14 |
| 5 | NPG - 4/5 | 577-P/24 | 19.00 | 20.00 | 90.34 | 1.82 | 1.89 | 9.66 | 58.32 | 19.86 |
| 6 | NPG - 4/6 | 578-P/24 | 20.00 | 21.00 | 91.46 | 1.93 | 0.91 | 8.54 | 57.04 | 12.94 |
| 7 | NPG - 4/7 | 579-P/24 | 21.00 | 22.00 | 89.82 | 2.45 | 3.11 | 10.18 | 65.58 | 9.72 |
| 8 | NPG - 4/8 | 580-P/24 | 22.00 | 23.00 | 93.34 | 1.78 | 1.19 | 6.66 | 66.30 | 12.42 |
| 9 | NPG - 4/9 | 581-P/24 | 23.00 | 24.00 | 89.10 | 3.11 | 1.67 | 10.90 | 53.42 | 14.58 |
| 10 | NPG - 4/10 | 582-P/24 | 24.00 | 25.00 | 91.32 | 1.76 | 1.92 | 8.68 | 64.24 | 10.10 |
| 11 | NPG - 4/11 | 583-P/24 | 25.00 | 26.00 | 94.30 | 1.48 | 0.56 | 5.70 | 65.40 | 10.20 |
| 12 | NPG - 4/12 | 584-P/24 | 26.00 | 27.00 | 93.54 | 1.45 | 0.66 | 6.46 | 60.72 | 14.16 |
| 13 | NPG - 4/13 | 585-P/24 | 28.00 | 29.00 | 92.32 | 1.83 | 1.80 | 7.68 | 67.60 | 10.94 |
| 14 | NPG - 4/14 | 586-P/24 | 29.00 | 30.00 | 92.24 | 1.88 | 1.60 | 7.76 | 64.14 | 13.46 |
| 15 | NPG - 4/15 | 587-P/24 | 31.00 | 32.00 | 90.18 | 2.31 | 2.33 | 9.82 | 66.02 | 11.80 |
| 16 | NPG - 4/16 | 749-P/24 | 32.00 | 33.00 | 90.62 | 1.00 | 2.38 | 9.38 | 67.68 | 11.56 |
| 17 | NPG - 4/17 | 750-P/24 | 33.50 | 34.00 | 91.42 | 1.21 | 2.74 | 8.58 | 72.46 | 10.30 |
| 18 | NPG - 4/18 | 751-P/24 | 34.00 | 35.00 | 91.50 | 1.38 | 1.92 | 8.50 | 67.36 | 10.28 |
| 19 | NPG - 4/19 | 752-P/24 | 35.00 | 36.00 | 91.66 | 1.50 | 1.78 | 8.34 | 68.52 | 7.58 |
| 20 | NPG - 4/20 | 753-P/24 | 36.00 | 37.00 | 91.44 | 1.15 | 2.25 | 8.56 | 68.32 | 14.90 |
| 21 | NPG - 4/21 | 754-P/24 | 37.00 | 38.00 | 89.50 | 0.97 | 4.60 | 10.50 | 70.78 | 8.50 |
| 22 | NPG - 4/22 | 755-P/24 | 38.00 | 39.00 | 87.72 | 2.48 | 5.47 | 12.28 | 66.42 | 7.68 |
| 23 | NPG - 4/23 | 756-P/24 | 40.50 | 41.50 | 85.24 | 3.89 | 4.15 | 14.76 | 54.36 | 8.50 |
| 24 | NPG - 4/24 | 757-P/24 | 41.50 | 42.50 | 89.48 | 0.20 | 8.16 | 10.52 | 81.44 | 0.50 |
| 25 | NPG - 4/25 | 758-P/24 | 42.50 | 43.50 | 89.60 | 0.21 | 8.06 | 10.40 | 82.70 | 1.80 |
| 26 | NPG - 4/26 | 759-P/24 | 43.50 | 44.50 | 87.48 | 0.30 | 10.03 | 12.52 | 78.62 | 3.00 |
| 27 | NPG - 4/27 | 760-P/24 | 44.50 | 45.50 | 87.62 | 0.35 | 9.84 | 12.38 | 80.34 | 1.00 |
| 28 | NPG - 4/28 | 761-P/24 | 45.50 | 46.30 | 88.60 | 0.19 | 9.26 | 11.40 | 80.72 | 0.60 |
| 29 | NPG - 4/29 | 762-P/24 | 46.30 | 47.00 | 91.74 | 0.69 | 2.36 | 8.26 | 62.38 | 13.48 |
| 30 | NPG - 4/30 | 763-P/24 | 47.00 | 48.00 | 90.80 | 0.67 | 3.10 | 9.20 | 65.66 | 11.26 |
| 31 | NPG - 4/31 | 764-P/24 | 48.00 | 49.00 | 92.84 | 1.42 | 1.95 | 7.16 | 68.82 | 11.86 |

|  | NPBH NO-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Sample No. | R.L. No. | FROM | TO | Ash % | M % | F.C % | LOI % | Si02 % | AI203 % |
| 1 | NPG - 5/1 | 765-P/24 | 31.00 | 32.00 | 95.38 | 1.08 | 1.57 | 4.62 | 69.14 | 13.26 |
| 2 | NPG - 5/2 | 766-P/24 | 34.00 | 35.00 | 95.36 | 0.83 | 0.58 | 4.64 | 65.34 | 15.46 |
| 3 | NPG - 5/3 | 767-P/24 | 35.00 | 36.00 | 94.66 | 1.52 | 1.09 | 5.34 | 65.38 | 12.06 |
| 4 | NPG - 5/4 | 768-P/24 | 40.00 | 41.00 | 98.16 | 0.37 | 0.41 | 1.84 | 64.16 | 17.18 |
| 5 | NPG - 5/5 | 769-P/24 | 44.00 | 45.00 | 95.66 | 0.87 | 1.03 | 4.34 | 65.34 | 15.62 |
| 6 | NPG - 5/6 | 770-P/24 | 45.00 | 46.00 | 94.88 | 0.90 | 1.39 | 5.12 | 66.92 | 12.52 |
| 7 | NPG - 5/7 | 771-P/24 | 46.00 | 46.50 | 96.76 | 0.55 | 1.01 | 3.24 | 70.48 | 12.38 |
| 8 | NPG - 5/8 | 772-P/24 | 52.30 | 53.00 | 95.24 | 0.78 | 1.70 | 4.76 | 68.20 | 14.48 |
| 9 | NPG - 5/9 | 773-P/24 | 53.50 | 54.00 | 95.42 | 0.88 | 1.60 | 4.58 | 63.70 | 15.66 |
| 10 | NPG - 5/10 | 774-P/24 | 54.50 | 55.50 | 94.94 | 0.81 | 1.26 | 5.06 | 61.38 | 17.88 |
| 11 | NPG - 5/11 | 775-P/24 | 55.50 | 56.50 | 95.28 | 0.94 | 0.88 | 4.72 | 66.28 | 18.16 |
| 12 | NPG - 5/12 | 776-P/24 | 56.50 | 57.50 | 94.36 | 1.14 | 1.15 | 5.64 | 63.84 | 16.06 |
| 13 | NPG - 5/13 | 777-P/24 | 57.50 | 58.50 | 94.76 | 1.19 | 1.01 | 5.24 | 67.68 | 14.22 |
| 14 | NPG - 5/14 | 778-P/24 | 58.50 | 59.50 | 93.30 | 1.09 | 1.96 | 6.70 | 63.86 | 14.06 |
| 15 | NPG - 5/15 | 779-P/24 | 59.50 | 60.50 | 93.14 | 1.02 | 1.85 | 6.86 | 63.42 | 17.64 |
| 16 | NPG - 5/16 | 780-P/24 | 60.50 | 61.50 | 91.76 | 1.22 | 2.94 | 8.26 | 62.70 | 14.02 |
| 17 | NPG - 5/17 | 781-P/24 | 61.50 | 62.30 | 93.58 | 1.20 | 1.61 | 6.42 | 56.92 | 16.44 |
| 18 | NPG - 5/18 | 782-P/24 | 63.00 | 64.00 | 93.60 | 1.03 | 2.53 | 6.40 | 66.86 | 13.82 |
| 19 | NPG - 5/19 | 783-P/24 | 64.00 | 65.00 | 94.52 | 1.08 | 1.76 | 5.48 | 67.84 | 12.86 |
| 20 | NPG - 5/20 | 784-P/24 | 65.00 | 66.00 | 93.66 | 1.31 | 2.50 | 6.34 | 66.12 | 12.56 |
| 21 | NPG - 5/21 | 785-P/24 | 66.00 | 67.00 | 95.76 | 0.89 | 1.34 | 4.24 | 69.72 | 12.16 |
| 22 | NPG - 5/22 | 786-P/24 | 67.00 | 68.00 | 96.04 | 1.10 | 1.14 | 3.96 | 70.56 | 10.92 |
| 23 | NPG - 5/23 | 787-P/24 | 68.00 | 69.00 | 94.58 | 1.51 | 1.54 | 5.42 | 68.18 | 9.82 |
| 24 | NPG - 5/24 | 788-P/24 | 69.00 | 70.00 | 95.52 | 1.43 | 0.64 | 4.48 | 67.20 | 13.62 |
| 25 | NPG - 5/25 | 789-P/24 | 70.00 | 71.00 | 95.70 | 1.01 | 1.26 | 4.30 | 75.66 | 4.16 |
| 26 | NPG - 5/26 | 790-P/24 | 71.00 | 72.00 | 96,96 | 1.60 | 0.90 | 3.04 | 80.98 | 4.92 |
| 27 | NPG - 5/27 | 791-P/24 | 72.00 | 73.50 | 97.44 | 0.61 | 0.36 | 2.56 | 68.92 | 13.92 |
| 28 | NPG - 5/28 | 792-P/24 | 73.50 | 75.00 | 93.24 | 1.90 | 2.35 | 6.76 | 67.16 | 13.16 |

|  | NPBH NO-6 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | SAMPLE NO. | FROM | TO | R.L. No. | Ash % | M % | V.M % | F.C % | LOI % | SiO2 % | AI2O3 % | V2O5 (%) |
| 1 | NPG 6/1 | 17.80 | 19.00 | 819-P/24 | 90.64 | 3.28 | 4.33 | 1.76 | 9.37 | 66.99 | 14.47 | 0.00 |
| 2 | NPG 6/2 | 21.00 | 21.50 | 820-P/24 | 89.30 | 3.56 | 6.26 | 0.89 | 10.71 | 66.21 | 15.59 | 0.00 |
| 3 | NPG 6/3 | 25.00 | 26.00 | 821-P/24 | 90.00 | 3.49 | 4.69 | 1.82 | 10.00 | 61.20 | 17.10 | 0.00 |
| 4 | NPG 6/4 | 26.00 | 27.00 | 822-P/24 | 86.62 | 4.81 | 4.66 | 3.91 | 13.38 | 62.02 | 16.05 | 0.00 |
| 5 | NPG 6/5 | 27.00 | 28.00 | 823-P/24 | 90.20 | 3.74 | 3.46 | 2.61 | 9.81 | 69.86 | 12.16 | 0.00 |
| 6 | NPG 6/6 | 28.00 | 29.00 | 824-P/24 | 88.33 | 4.66 | 4.42 | 2.60 | 11.68 | 66.72 | 13.74 | 0.00 |
| 7 | NPG 6/7 | 29.00 | 30.00 | 825-P/24 | 86.84 | 5.19 | 5.09 | 2.89 | 13.17 | 61.94 | 16.16 | 0.00 |
| 8 | NPG 6/8 | 30.00 | 31.00 | 826-P/24 | 85.56 | 5.31 | 5.76 | 3.38 | 14.45 | 57.56 | 16.18 | 0.00 |
| 9 | NPG 6/9 | 46.50 | 47.50 | 827-P/24 | 94.78 | 1.00 | 3.61 | 0.62 | 5.23 | 64.63 | 16.22 | 0.00 |
| 10 | NPG 6/10 | 47.50 | 48.50 | 828-P/24 | 95.16 | 1.04 | 3.77 | 0.03 | 3.09 | 67.79 | 15.73 | 0.00 |
| 11 | NPG 6/11 | 48.50 | 49.00 | 829-P/24 | 91.33 | 1.69 | 5.10 | 1.88 | 8.67 | 61.69 | 17.48 | 0.00 |
| 12 | NPG 6/12 | 51.20 | 51.70 | 830-P/24 | 91.31 | 1.51 | 4.79 | 2.40 | 8.70 | 63.38 | 15.72 | 0.00 |
| 13 | NPG 6/13 | 52.05 | 53.05 | 831-P/24 | 98.03 | 0.70 | 1.19 | 0.08 | 0.27 | 70.06 | 17.06 | 0.00 |
| 14 | NPG 6/14 | 53.05 | 53.60 | 832-P/24 | 94.64 | 0.98 | 3.62 | 0.76 | 5.36 | 65.25 | 17.69 | 0.00 |
| 15 | NPG 6/15 | 56.00 | 57.10 | 833-P/24 | 91.24 | 1.56 | 5.46 | 1.75 | 8.77 | 64.44 | 15.59 | 0.00 |

ANNEXURE-XIII

ICPMS ANALYSIS RESULTS

| SL.No. | LAB NO. | Sender No. | Li | Be | Sc | V | Cr | Co | Ni | Cu | Zn | Ga | Ge | As | Se | Rb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PPM (LOD=0.1PPM) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | MNC-1 | NCPV-1 | 6.08 | 2.15 | 5.37 | 184.49 | 59.98 | 7.94 | 24.21 | 125.72 | 141.98 | 12.97 | 1.06 | 34.18 | 1.02 | 62.37 |
| 2 | MNC-2 | NCPV-2 | 16.43 | 2.51 | 10.02 | 110.50 | 81.14 | 11.91 | 34.05 | 81.66 | 185.98 | 14.70 | 1.28 | 33.01 | 0.48 | 69.05 |
| 3 | MNC-3 | NCPV-3 | 23.87 | 2.13 | 9.23 | 89.67 | 70.14 | 11.81 | 33.81 | 63.40 | 189.14 | 15.73 | 1.31 | 35.20 | 0.46 | 62.64 |
| 4 | MNC-4 | NCPV-4 | 12.33 | 0.95 | 10.84 | 78.56 | 77.96 | 4.68 | 24.45 | 48.64 | 113.51 | 12.82 | 1.20 | 40.31 | 0.77 | 91.81 |
| 5 | MNC-5 | NCPV-5 | 23.50 | 2.58 | 12.05 | 96.49 | 84.52 | 11.64 | 37.35 | 49.38 | 128.11 | 18.42 | 1.06 | 34.70 | 0.58 | 106.09 |
| 6 | MNC-6 | NCPV-6 | 12.93 | 2.35 | 12.61 | 96.49 | 79.10 | 11.51 | 35.01 | 50.68 | 230.34 | 19.81 | 1.17 | 33.97 | 0.70 | 108.49 |
| 7 | MNC-7 | NCPV-7 | 19.84 | 2.92 | 12.33 | 102.69 | 83.78 | 9.19 | 29.17 | 39.40 | 103.08 | 20.72 | 1.10 | 39.00 | 0.59 | 115.86 |
| 8 | MNC-8 | NCPV-8 | 24.25 | 2.56 | 11.74 | 114.31 | 74.53 | 19.44 | 70.84 | 142.59 | 94.21 | 16.62 | 1.27 | 34.16 | 0.59 | 105.56 |
| 9 | MNC-9 | NCPV-9 | 22.38 | 2.92 | 9.23 | 65.74 | 63.83 | 11.13 | 39.27 | 47.68 | 148.67 | 11.18 | 1.16 | 34.46 | 0.50 | 70.17 |
| 10 | MNC-10 | NCPV-10 | 25.14 | 2.40 | 11.34 | 135.44 | 102.05 | 21.15 | 27.50 | 58.76 | 149.91 | 21.18 | 1.37 | 32.94 | 0.64 | 108.14 |
| 11 | MNC-11 | NCPV-11 | 9.51 | 1.74 | 6.82 | 90.93 | 85.11 | 6.69 | 23.38 | 93.00 | 31.54 | 22.86 | 0.96 | 37.36 | 1.33 | 56.84 |
| 12 | MNC-12 | NCPV-12 | 2.81 | 0.61 | 3.69 | 66.20 | 42.37 | 6.26 | 29.29 | 89.48 | 127.59 | 8.47 | 0.65 | 35.36 | 0.87 | 14.20 |
| 13 | MNC-13 | NCPV-13 | 28.68 | 2.76 | 11.61 | 85.35 | 79.53 | 11.29 | 34.65 | 49.33 | 118.73 | 17.03 | 1.09 | 37.98 | 0.62 | 88.89 |
| 14 | MNC-14 | NCPV-14 | 12.59 | 4.23 | 9.40 | 82.81 | 74.15 | 7.73 | 27.01 | 78.70 | 85.03 | 14.75 | 1.04 | 36.44 | 0.50 | 68.96 |
| 15 | MNC-15 | NCPV-15 | 13.51 | 1.56 | 7.31 | 57.66 | 69.75 | 8.74 | 33.20 | 56.54 | 73.19 | 7.72 | 0.89 | 34.50 | 0.39 | 44.63 |
| 16 | MNC-16 | NCPV-16 | 19.07 | 1.83 | 11.82 | 120.74 | 97.90 | 18.11 | 45.93 | 133.62 | 65.84 | 21.86 | 1.20 | 41.97 | 0.91 | 114.41 |
| 17 | MNC-17 | NCPV-17 | 9.49 | 2.78 | 9.45 | 166.62 | 87.99 | 17.16 | 23.29 | 156.82 | 46.07 | 19.25 | 2.98 | 33.25 | 0.51 | 97.06 |
| 18 | MNC-18 | NCPV-18 | 14.99 | 2.24 | 13.80 | 126.41 | 79.94 | 10.57 | 36.99 | 43.39 | 83.78 | 20.15 | 1.08 | 36.66 | 0.74 | 61.71 |
| 19 | MNC-19 | NCPV-19 | 26.71 | 2.22 | 7.11 | 30.83 | 71.16 | 6.23 | 15.99 | 33.22 | 58.70 | 13.67 | 0.67 | 38.20 | 0.30 | 66.64 |
| 20 | MNC-20 | NCPV-20 | 25.95 | 3.55 | 20.23 | 171.23 | 139.91 | 25.67 | 51.46 | 72.02 | 214.29 | 19.18 | 1.61 | 38.87 | 0.71 | 110.38 |

| Sl. No. | LAB NO. | Sender No. | Sr | Y | Zr | Nb | Mo | Ag | Cd | In | Sn | Sb | Te | Cs | Ba | Hf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PPM(LOD=0.1PPM) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | MNC-1 | NCPV-1 | 379.94 | 43.96 | 179.18 | 1.81 | 3.58 | 0.58 | 0.55 | 0.10 | 2.17 | 1.07 | 0.38 | 0.83 | 492.97 | 4.66 |
| 2 | MNC-2 | NCPV-2 | 197.39 | 33.43 | 191.07 | 1.89 | 2.09 | 0.19 | 0.31 | BDL | 2.06 | 1.15 | BDL | 1.30 | 530.31 | 5.22 |
| 3 | MNC-3 | NCPV-3 | 66.89 | 32.60 | 179.35 | 1.12 | 1.76 | 0.22 | 0.39 | BDL | 1.68 | 1.07 | BDL | 0.88 | 164.14 | 4.79 |
| 4 | MNC-4 | NCPV-4 | 81.95 | 37.13 | 288.02 | 1.83 | 1.60 | 0.13 | 0.38 | BDL | 2.19 | 1.21 | BDL | 2.27 | 528.29 | 7.66 |
| 5 | MNC-5 | NCPV-5 | 80.14 | 35.18 | 232.21 | 2.25 | 1.40 | 0.17 | 0.31 | BDL | 1.98 | 1.09 | BDL | 1.27 | 655.96 | 6.21 |
| 6 | MNC-6 | NCPV-6 | 53.39 | 42.85 | 251.82 | 1.87 | 1.35 | 0.20 | 0.28 | BDL | 1.92 | 1.04 | BDL | 1.23 | 518.86 | 6.62 |
| 7 | MNC-7 | NCPV-7 | 106.72 | 37.11 | 236.47 | 1.30 | 1.39 | 0.17 | 0.25 | BDL | 2.18 | 1.18 | BDL | 1.53 | 604.71 | 6.48 |
| 8 | MNC-8 | NCPV-8 | 53.94 | 48.92 | 121.65 | 1.06 | 2.70 | 0.20 | 0.24 | BDL | 1.91 | 1.11 | 0.21 | 1.84 | 449.34 | 3.33 |
| 9 | MNC-9 | NCPV-9 | 63.32 | 25.09 | 175.75 | 2.13 | 1.50 | 0.16 | 0.25 | BDL | 1.90 | 0.99 | BDL | 2.25 | 378.81 | 4.79 |
| 10 | MNC-10 | NCPV-10 | 62.45 | 21.59 | 355.83 | 0.98 | 1.70 | 0.26 | 0.57 | BDL | 1.90 | 1.03 | BDL | 3.09 | 588.55 | 9.43 |
| 11 | MNC-11 | NCPV-11 | 76.52 | 39.68 | 251.82 | 1.37 | 2.13 | 0.39 | 0.30 | BDL | 1.50 | 1.63 | 0.24 | 0.68 | 427.57 | 6.79 |
| 12 | MNC-12 | NCPV-12 | 79.99 | 36.20 | 210.98 | 2.24 | 5.99 | 0.32 | 0.39 | BDL | 0.99 | 1.02 | 0.40 | 0.19 | 270.39 | 5.51 |
| 13 | MNC-13 | NCPV-13 | 153.14 | 43.06 | 319.09 | 1.41 | 1.64 | 0.24 | 0.37 | BDL | 3.05 | 1.22 | BDL | 1.52 | 609.86 | 8.67 |
| 14 | MNC-14 | NCPV-14 | 280.47 | 39.02 | 116.23 | 1.01 | 1.48 | 0.16 | 0.24 | BDL | 6.31 | 1.15 | BDL | 1.80 | 637.74 | 3.17 |
| 15 | MNC-15 | NCPV-15 | 73.51 | 33.60 | 280.21 | 0.79 | 1.44 | 0.20 | 0.24 | BDL | 1.46 | 1.11 | BDL | 0.96 | 247.91 | 7.40 |
| 16 | MNC-16 | NCPV-16 | 94.94 | 34.47 | 325.01 | 1.75 | 1.40 | 0.31 | 0.27 | BDL | 1.64 | 1.30 | 0.28 | 1.78 | 793.62 | 8.65 |
| 17 | MNC-17 | NCPV-17 | 189.34 | 23.69 | 227.26 | 1.09 | 1.72 | 0.16 | 0.25 | BDL | 1.32 | 0.97 | 0.14 | 0.82 | 777.98 | 5.90 |
| 18 | MNC-18 | NCPV-18 | 105.00 | 46.15 | 124.63 | 1.05 | 1.31 | BDL | 0.25 | BDL | 0.89 | 1.14 | 0.12 | 1.02 | 750.46 | 3.37 |
| 19 | MNC-19 | NCPV-19 | 135.48 | 33.64 | 293.03 | 0.56 | 0.82 | 0.21 | 0.29 | BDL | 1.02 | 1.18 | BDL | 0.29 | 607.06 | 7.67 |
| 20 | MNC-20 | NCPV-20 | 64.78 | 45.64 | 238.26 | 2.78 | 3.04 | 0.18 | 0.39 | BDL | 1.22 | 1.22 | BDL | 1.98 | 853.62 | 6.26 |

| Sl. No. | LAB NO. | Sender No. | Ta | W | Pb | Bi | Th | U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PPM(LOD=0.1PPM) |  |
| 1 | MNC-1 | NCPV-1 | 0.48 | 0.89 | 24.33 | 0.58 | 8.35 | 5.17 |
| 2 | MNC-2 | NCPV-2 | 0.69 | 0.77 | 18.15 | BDL | 8.17 | 1.00 |
| 3 | MNC-3 | NCPV-3 | 0.18 | 0.17 | 19.08 | BDL | 10.10 | 1.44 |
| 4 | MNC-4 | NCPV-4 | 0.26 | 0.30 | 17.55 | BDL | 29.96 | 1.86 |
| 5 | MNC-5 | NCPV-5 | 1.02 | 0.70 | 24.03 | BDL | 15.33 | 1.11 |
| 6 | MNC-6 | NCPV-6 | 0.73 | 1.33 | 22.94 | BDL | 20.50 | 1.26 |
| 7 | MNC-7 | NCPV-7 | 0.43 | 0.24 | 17.50 | BDL | 12.69 | 1.06 |
| 8 | MNC-8 | NCPV-8 | 0.37 | 0.27 | 19.20 | 0.28 | 13.13 | 0.99 |
| 9 | MNC-9 | NCPV-9 | 0.30 | 0.16 | 13.05 | BDL | 10.17 | 0.88 |
| 10 | MNC-10 | NCPV-10 | 0.23 | 0.27 | 21.48 | BDL | 15.92 | 1.42 |
| 11 | MNC-11 | NCPV-11 | 0.48 | 0.52 | 26.48 | 0.27 | 31.59 | 3.90 |
| 12 | MNC-12 | NCPV-12 | 0.38 | 0.71 | 15.10 | 0.52 | 10.99 | 7.28 |
| 13 | MNC-13 | NCPV-13 | 0.58 | 0.42 | 21.58 | BDL | 20.39 | 1.33 |
| 14 | MNC-14 | NCPV-14 | 0.58 | 0.21 | 22.04 | BDL | 2.19 | 0.35 |
| 15 | MNC-15 | NCPV-15 | 0.21 | 0.17 | 10.82 | BDL | 6.34 | 1.05 |
| 16 | MNC-16 | NCPV-16 | 0.94 | 0.30 | 27.37 | BDL | 21.45 | 2.13 |
| 17 | MNC-17 | NCPV-17 | 0.22 | 0.17 | 31.49 | 0.19 | 11.28 | 1.66 |
| 18 | MNC-18 | NCPV-18 | 0.37 | BDL | 18.27 | BDL | 20.72 | 1.65 |
| 19 | MNC-19 | NCPV-19 | 0.11 | BDL | 27.64 | BDL | 20.93 | 1.16 |
| 20 | MNC-20 | NCPV-20 | 0.36 | 0.40 | 20.32 | BDL | 21.84 | 1.99 |

ANNEXURE-XIV

DETAILS OF BOREHOLES OF NARINGPANGA BLOCK

| Bore Hole | Inclined Depth(m) | Vertical Depth(m) | R.L(m) | Thickness of Graphite(m) | Samples Generated(Nos.) |
| --- | --- | --- | --- | --- | --- |
| NPBH-1 | 70.00 | 49.50 | 331.40 | 24.55 | 31 |
| NPBH-2 | 55.00 | 38.89 | 333.31 | 9.65 | 24 |
| NPBH-3 | 74.50 | 52.68 | 331.02 | 19.55 | 31 |
| NPBH-4 | 56.50 | 39.95 | 335.52 | 18.55 | 31 |
| NPBH-5 | 78.80 | 55.72 | 329.86 | 5.50 | 28 |
| NPBH-6 | 63.20 | 44.69 | 334.14 | 6.50 | 15 |
| Total | 398.00 | 281.43 |  | 84.30 | 160 |

ANNEXURE-XV

CHEMICAL ANALYSIS OF GRAB AND PIT SAMPLES OF NARINGPANGA (S) GRAPHITE BLOCK, RAYAGADA DISTRICT, ODISHA

N.B- NGS: GRAB SAMPLE, NGPT- PIT SAMPLE

Annexure-XVI

Petrographic, Mineralographic study and XRD study report

By DoMG Laboratory and MECL Laboratory, Nagpur

|  | NTS-2 |
| --- | --- |
|  | NTS-3 |
|  | NTS-5 |

ANNEXURE-XVII

DGPS SURVEY AND TOPOGRAPHICAL SURVEY

ANNEXURE-XVIII

BULK DENSITY ANALYSIS RESULT OF NARINGPANGA (S) GRAPHITE BLOCK BY NEW GREEN ENVIRONMENTAL PVT. LTD., BHUBANESWAR (NABL ACCREDIATED LAB)

ANNEXURE-XIX

Self-potential anomaly Data of Naringpanga Graphite Block

| SL NO | NORTHING | EASTING | SP VALUE |
| --- | --- | --- | --- |
| 1 | 2182669.811 | 767688.1354 | 70.19 |
| 2 | 2182676.28 | 767695.7618 | 70.21 |
| 3 | 2182682.748 | 767703.3883 | 70.32 |
| 4 | 2182688.878 | 767671.965 | 71.65 |
| 5 | 2182689.216 | 767711.0148 | 70.45 |
| 6 | 2182695.346 | 767679.5914 | 71.35 |
| 7 | 2182695.684 | 767718.6412 | 70.48 |
| 8 | 2182701.814 | 767687.2179 | 71.09 |
| 9 | 2182702.153 | 767726.2677 | 70.34 |
| 10 | 2182707.944 | 767655.7946 | 72.56 |
| 11 | 2182708.282 | 767694.8443 | 70.78 |
| 12 | 2182708.621 | 767733.8941 | 69.18 |
| 13 | 2182714.412 | 767663.421 | 72.03 |
| 14 | 2182714.75 | 767702.4708 | 70.27 |
| 15 | 2182715.089 | 767741.5206 | 65.55 |
| 16 | 2182720.881 | 767671.0474 | 71.28 |
| 17 | 2182721.218 | 767710.0973 | 69.41 |
| 18 | 2182721.557 | 767749.1471 | 60.61 |
| 19 | 2182727.01 | 767639.6242 | 74.64 |
| 20 | 2182727.349 | 767678.674 | 70.11 |
| 21 | 2182727.687 | 767717.7237 | 67.70 |
| 22 | 2182728.025 | 767756.7735 | 53.43 |
| 23 | 2182733.478 | 767647.2506 | 73.84 |
| 24 | 2182733.817 | 767686.3004 | 69.10 |
| 25 | 2182734.155 | 767725.3502 | 64.84 |
| 26 | 2182734.494 | 767764.4 | 43.79 |
| 27 | 2182739.946 | 767654.8771 | 72.42 |
| 28 | 2182740.285 | 767693.9268 | 68.05 |
| 29 | 2182740.623 | 767732.9766 | 60.74 |
| 30 | 2182740.962 | 767772.0264 | 33.98 |
| 31 | 2182746.077 | 767623.4538 | 77.43 |
| 32 | 2182746.414 | 767662.5035 | 70.84 |
| 33 | 2182746.753 | 767701.5534 | 66.41 |
| 34 | 2182747.091 | 767740.6031 | 55.66 |
| 35 | 2182747.43 | 767779.6528 | 25.36 |
| 36 | 2182752.545 | 767631.0802 | 76.33 |
| 37 | 2182752.883 | 767670.13 | 69.19 |
| 38 | 2182753.221 | 767709.1798 | 64.41 |
| 39 | 2182753.559 | 767748.2296 | 49.71 |
| 40 | 2182753.897 | 767787.2794 | 18.56 |
| 41 | 2182759.012 | 767638.7066 | 74.74 |
| 42 | 2182759.351 | 767677.7565 | 67.94 |
| 43 | 2182759.689 | 767716.8063 | 61.60 |
| 44 | 2182760.027 | 767755.856 | 43.13 |
| 45 | 2182760.366 | 767794.9059 | 15.32 |
| 46 | 2182765.142 | 767607.2833 | 80.12 |
| 47 | 2182765.48 | 767646.3332 | 72.81 |
| 48 | 2182765.819 | 767685.3829 | 66.78 |
| 49 | 2182766.157 | 767724.4327 | 58.65 |
| 50 | 2182766.496 | 767763.4825 | 36.23 |
| 51 | 2182766.834 | 767802.5323 | 11.77 |
| 52 | 2182771.61 | 767614.9098 | 79.15 |
| 53 | 2182771.949 | 767653.9596 | 70.82 |
| 54 | 2182772.287 | 767693.0094 | 65.16 |
| 55 | 2182772.625 | 767732.0592 | 55.57 |
| 56 | 2182772.964 | 767771.1089 | 29.97 |
| 57 | 2182773.302 | 767810.1587 | 7.59 |
| 58 | 2182778.079 | 767622.5363 | 77.44 |
| 59 | 2182778.417 | 767661.5861 | 68.75 |
| 60 | 2182778.755 | 767700.6358 | 63.17 |
| 61 | 2182779.094 | 767739.6856 | 52.00 |
| 62 | 2182779.432 | 767778.7354 | 24.88 |
| 63 | 2182779.77 | 767817.7852 | 3.43 |
| 64 | 2182784.208 | 767591.113 | 78.73 |
| 65 | 2182784.547 | 767630.1627 | 75.19 |
| 66 | 2182784.885 | 767669.2125 | 66.79 |
| 67 | 2182785.223 | 767708.2623 | 60.42 |
| 68 | 2182785.562 | 767747.3121 | 46.55 |
| 69 | 2182785.9 | 767786.3619 | 20.71 |
| 70 | 2182786.239 | 767825.4116 | 0.44 |
| 71 | 2182790.677 | 767598.7394 | 77.58 |
| 72 | 2182791.015 | 767637.7892 | 72.39 |
| 73 | 2182791.353 | 767676.8389 | 65.06 |
| 74 | 2182791.692 | 767715.8887 | 56.67 |
| 75 | 2182792.03 | 767754.9385 | 40.03 |
| 76 | 2182792.368 | 767793.9883 | 16.09 |
| 77 | 2182792.707 | 767833.038 | 0.06 |
| 78 | 2182797.145 | 767606.3659 | 75.53 |
| 79 | 2182797.483 | 767645.4156 | 68.89 |
| 80 | 2182797.822 | 767684.4655 | 63.41 |
| 81 | 2182798.16 | 767723.5152 | 52.06 |
| 82 | 2182798.498 | 767762.565 | 33.26 |
| 83 | 2182798.836 | 767801.6148 | 10.19 |
| 84 | 2182799.175 | 767840.6646 | 0.31 |
| 85 | 2182803.274 | 767574.9425 | 66.87 |
| 86 | 2182803.613 | 767613.9923 | 73.75 |
| 87 | 2182803.951 | 767653.0421 | 64.79 |
| 88 | 2182804.29 | 767692.0919 | 61.54 |
| 89 | 2182804.628 | 767731.1417 | 46.71 |
| 90 | 2182804.967 | 767770.1915 | 27.89 |
| 91 | 2182805.305 | 767809.2412 | 3.82 |
| 92 | 2182805.643 | 767848.291 | -3.08 |
| 93 | 2182809.743 | 767582.569 | 65.32 |
| 94 | 2182810.081 | 767621.6187 | 72.18 |
| 95 | 2182810.419 | 767660.6686 | 60.90 |
| 96 | 2182810.758 | 767699.7183 | 56.51 |
| 97 | 2182811.096 | 767738.7681 | 40.79 |
| 98 | 2182811.435 | 767777.8179 | 24.30 |
| 99 | 2182811.773 | 767816.8677 | 0.56 |
| 100 | 2182812.112 | 767855.9175 | -14.44 |
| 101 | 2182816.211 | 767590.1955 | 63.31 |
| 102 | 2182816.549 | 767629.2452 | 66.92 |
| 103 | 2182816.887 | 767668.295 | 58.06 |
| 104 | 2182817.226 | 767707.3448 | 50.07 |
| 105 | 2182817.564 | 767746.3946 | 34.56 |
| 106 | 2182817.903 | 767785.4443 | 18.60 |
| 107 | 2182818.241 | 767824.4941 | 0.83 |
| 108 | 2182818.58 | 767863.5439 | -19.99 |
| 109 | 2182822.341 | 767558.7721 | 56.87 |
| 110 | 2182822.679 | 767597.8219 | 60.65 |
| 111 | 2182823.018 | 767636.8717 | 58.08 |
| 112 | 2182823.356 | 767675.9215 | 55.81 |
| 113 | 2182823.694 | 767714.9713 | 45.06 |
| 114 | 2182824.032 | 767754.021 | 28.80 |
| 115 | 2182824.371 | 767793.0708 | 8.46 |
| 116 | 2182824.709 | 767832.1206 | -5.00 |
| 117 | 2182825.048 | 767871.1704 | -20.61 |
| 118 | 2182828.809 | 767566.3986 | 54.33 |
| 119 | 2182829.147 | 767605.4484 | 57.21 |
| 120 | 2182829.486 | 767644.4982 | 52.53 |
| 121 | 2182829.824 | 767683.5479 | 52.88 |
| 122 | 2182830.163 | 767722.5977 | 40.06 |
| 123 | 2182830.5 | 767761.6475 | 23.43 |
| 124 | 2182830.84 | 767800.6973 | 1.01 |
| 125 | 2182831.177 | 767839.7471 | -17.25 |
| 126 | 2182831.515 | 767878.7968 | -23.70 |
| 127 | 2182835.277 | 767574.0251 | 51.40 |
| 128 | 2182835.615 | 767613.0748 | 52.78 |
| 129 | 2182835.954 | 767652.1246 | 49.39 |
| 130 | 2182836.292 | 767691.1744 | 49.20 |
| 131 | 2182836.631 | 767730.2241 | 33.62 |
| 132 | 2182836.969 | 767769.274 | 13.42 |
| 133 | 2182837.307 | 767808.3238 | 1.39 |
| 134 | 2182837.645 | 767847.3735 | -21.08 |
| 135 | 2182837.984 | 767886.4233 | -27.70 |
| 136 | 2182841.407 | 767542.6017 | 53.78 |
| 137 | 2182841.746 | 767581.6515 | 48.04 |
| 138 | 2182842.083 | 767620.7013 | 47.76 |
| 139 | 2182842.422 | 767659.7511 | 48.01 |
| 140 | 2182842.76 | 767698.8009 | 45.28 |
| 141 | 2182843.099 | 767737.8506 | 27.85 |
| 142 | 2182843.437 | 767776.9004 | 2.81 |
| 143 | 2182843.775 | 767815.9502 | -7.13 |
| 144 | 2182844.113 | 767855 | -20.49 |
| 145 | 2182844.452 | 767894.0498 | -29.92 |
| 146 | 2182847.875 | 767550.2282 | 52.06 |
| 147 | 2182848.214 | 767589.278 | 44.04 |
| 148 | 2182848.552 | 767628.3277 | 43.06 |
| 149 | 2182848.891 | 767667.3775 | 46.86 |
| 150 | 2182849.228 | 767706.4273 | 41.06 |
| 151 | 2182849.566 | 767745.4771 | 21.42 |
| 152 | 2182849.905 | 767784.5269 | 0.00 |
| 153 | 2182850.243 | 767823.5766 | -19.08 |
| 154 | 2182850.582 | 767862.6264 | -22.76 |
| 155 | 2182850.92 | 767901.6763 | -30.33 |
| 156 | 2182854.343 | 767557.8546 | 49.88 |
| 157 | 2182854.682 | 767596.9044 | 39.88 |
| 158 | 2182855.02 | 767635.9542 | 39.44 |
| 159 | 2182855.359 | 767675.0039 | 45.31 |
| 160 | 2182855.696 | 767714.0538 | 36.23 |
| 161 | 2182856.035 | 767753.1035 | 8.02 |
| 162 | 2182856.373 | 767792.1533 | -2.51 |
| 163 | 2182856.712 | 767831.2032 | -20.35 |
| 164 | 2182857.05 | 767870.2529 | -26.44 |
| 165 | 2182857.388 | 767909.3027 | -30.17 |
| 166 | 2182860.473 | 767526.4313 | 51.91 |
| 167 | 2182860.811 | 767565.4811 | 46.73 |
| 168 | 2182861.15 | 767604.5309 | 37.13 |
| 169 | 2182861.488 | 767643.5807 | 36.98 |
| 170 | 2182861.826 | 767682.6305 | 43.44 |
| 171 | 2182862.165 | 767721.6802 | 30.95 |
| 172 | 2182862.503 | 767760.73 | -1.17 |
| 173 | 2182862.841 | 767799.7798 | -14.66 |
| 174 | 2182863.18 | 767838.8296 | -21.32 |
| 175 | 2182863.518 | 767877.8794 | -29.47 |
| 176 | 2182863.857 | 767916.9291 | -29.29 |
| 177 | 2182866.942 | 767534.0578 | 51.32 |
| 178 | 2182867.279 | 767573.1076 | 42.75 |
| 179 | 2182867.618 | 767612.1573 | 33.74 |
| 180 | 2182867.956 | 767651.2071 | 36.03 |
| 181 | 2182868.294 | 767690.2569 | 40.86 |
| 182 | 2182868.633 | 767729.3067 | 25.46 |
| 183 | 2182868.971 | 767768.3565 | -1.06 |
| 184 | 2182869.309 | 767807.4063 | -20.51 |
| 185 | 2182869.648 | 767846.456 | -23.24 |
| 186 | 2182869.986 | 767885.5058 | -30.61 |
| 187 | 2182870.325 | 767924.5556 | -27.73 |
| 188 | 2182873.41 | 767541.6842 | 49.97 |
| 189 | 2182873.748 | 767580.734 | 38.68 |
| 190 | 2182874.086 | 767619.7838 | 30.42 |
| 191 | 2182874.424 | 767658.8336 | 35.47 |
| 192 | 2182874.763 | 767697.8834 | 36.39 |
| 193 | 2182875.101 | 767736.9331 | 16.29 |
| 194 | 2182875.439 | 767775.9829 | -6.78 |
| 195 | 2182875.778 | 767815.0327 | -20.62 |
| 196 | 2182876.116 | 767854.0825 | -25.58 |
| 197 | 2182876.454 | 767893.1323 | -30.79 |
| 198 | 2182876.793 | 767932.182 | -25.75 |
| 199 | 2182879.539 | 767510.2609 | 45.09 |
| 200 | 2182879.878 | 767549.3107 | 47.95 |
| 201 | 2182880.216 | 767588.3605 | 35.04 |
| 202 | 2182880.554 | 767627.4102 | 29.02 |
| 203 | 2182880.892 | 767666.46 | 34.04 |
| 204 | 2182881.231 | 767705.5098 | 31.15 |
| 205 | 2182881.569 | 767744.5596 | 3.08 |
| 206 | 2182881.908 | 767783.6094 | -15.56 |
| 207 | 2182882.246 | 767822.6592 | -21.94 |
| 208 | 2182882.585 | 767861.7089 | -27.76 |
| 209 | 2182882.922 | 767900.7587 | -30.45 |
| 210 | 2182883.261 | 767939.8085 | -23.47 |
| 211 | 2182886.007 | 767517.8874 | 44.38 |
| 212 | 2182886.346 | 767556.9371 | 42.94 |
| 213 | 2182886.684 | 767595.9869 | 32.40 |
| 214 | 2182887.022 | 767635.0367 | 29.40 |
| 215 | 2182887.361 | 767674.0865 | 31.32 |
| 216 | 2182887.699 | 767713.1362 | 26.77 |
| 217 | 2182888.037 | 767752.1861 | -0.21 |
| 218 | 2182888.376 | 767791.2358 | -19.92 |
| 219 | 2182888.714 | 767830.2856 | -24.24 |
| 220 | 2182889.053 | 767869.3354 | -29.10 |
| 221 | 2182889.391 | 767908.3852 | -29.23 |
| 222 | 2182889.73 | 767947.4349 | -20.66 |
| 223 | 2182892.475 | 767525.5138 | 42.58 |
| 224 | 2182892.814 | 767564.5636 | 36.91 |
| 225 | 2182893.152 | 767603.6134 | 30.84 |
| 226 | 2182893.49 | 767642.6631 | 29.96 |
| 227 | 2182893.829 | 767681.713 | 26.47 |
| 228 | 2182894.167 | 767720.7628 | 18.22 |
| 229 | 2182894.505 | 767759.8125 | -2.77 |
| 230 | 2182894.844 | 767798.8623 | -20.69 |
| 231 | 2182895.182 | 767837.9121 | -26.77 |
| 232 | 2182895.521 | 767876.9619 | -29.77 |
| 233 | 2182895.859 | 767916.0117 | -26.08 |
| 234 | 2182896.197 | 767955.0614 | -11.06 |
| 235 | 2182898.606 | 767494.0905 | 35.74 |
| 236 | 2182898.944 | 767533.1403 | 39.44 |
| 237 | 2182899.282 | 767572.19 | 32.37 |
| 238 | 2182899.62 | 767611.2399 | 30.23 |
| 239 | 2182899.959 | 767650.2897 | 29.65 |
| 240 | 2182900.297 | 767689.3394 | 17.28 |
| 241 | 2182900.636 | 767728.3892 | 5.63 |
| 242 | 2182900.974 | 767767.439 | -13.36 |
| 243 | 2182901.312 | 767806.4887 | -22.32 |
| 244 | 2182901.65 | 767845.5385 | -28.97 |
| 245 | 2182901.989 | 767884.5883 | -30.25 |
| 246 | 2182902.327 | 767923.6381 | -21.90 |
| 247 | 2182902.665 | 767962.6878 | -9.94 |
| 248 | 2182905.074 | 767501.7169 | 34.70 |
| 249 | 2182905.412 | 767540.7667 | 34.95 |
| 250 | 2182905.75 | 767579.8165 | 30.41 |
| 251 | 2182906.088 | 767618.8663 | 30.08 |
| 252 | 2182906.427 | 767657.9161 | 28.33 |
| 253 | 2182906.765 | 767696.9659 | 3.81 |
| 254 | 2182907.104 | 767736.0156 | -0.06 |
| 255 | 2182907.442 | 767775.0654 | -20.06 |
| 256 | 2182907.781 | 767814.1152 | -24.61 |
| 257 | 2182908.118 | 767853.165 | -30.16 |
| 258 | 2182908.457 | 767892.2148 | -29.78 |
| 259 | 2182908.795 | 767931.2646 | -17.15 |
| 260 | 2182911.541 | 767509.3434 | 33.57 |
| 261 | 2182911.88 | 767548.3932 | 29.64 |
| 262 | 2182912.218 | 767587.443 | 30.83 |
| 263 | 2182912.557 | 767626.4928 | 29.59 |
| 264 | 2182912.895 | 767665.5426 | 19.14 |
| 265 | 2182913.233 | 767704.5923 | 0.53 |
| 266 | 2182913.572 | 767743.6421 | -5.93 |
| 267 | 2182913.91 | 767782.6919 | -20.39 |
| 268 | 2182914.249 | 767821.7416 | -27.22 |
| 269 | 2182914.587 | 767860.7915 | -30.77 |
| 270 | 2182914.925 | 767899.8412 | -27.09 |
| 271 | 2182915.263 | 767938.891 | -12.41 |
| 272 | 2182917.671 | 767477.9201 | 29.87 |
| 273 | 2182918.01 | 767516.9699 | 31.77 |
| 274 | 2182918.348 | 767556.0197 | 25.05 |
| 275 | 2182918.686 | 767595.0694 | 31.39 |
| 276 | 2182919.025 | 767634.1192 | 28.09 |
| 277 | 2182919.363 | 767673.169 | 1.63 |
| 278 | 2182919.701 | 767712.2188 | -6.37 |
| 279 | 2182920.04 | 767751.2685 | -15.68 |
| 280 | 2182920.378 | 767790.3184 | -20.55 |
| 281 | 2182920.716 | 767829.3681 | -29.65 |
| 282 | 2182921.055 | 767868.4179 | -31.13 |
| 283 | 2182921.393 | 767907.4676 | -22.94 |
| 284 | 2182921.731 | 767946.5175 | -9.56 |
| 285 | 2182924.14 | 767485.5466 | 28.92 |
| 286 | 2182924.478 | 767524.5964 | 29.03 |
| 287 | 2182924.816 | 767563.6461 | 22.26 |
| 288 | 2182925.155 | 767602.6959 | 30.99 |
| 289 | 2182925.493 | 767641.7457 | 24.00 |
| 290 | 2182925.832 | 767680.7955 | -9.84 |
| 291 | 2182926.17 | 767719.8453 | -14.93 |
| 292 | 2182926.509 | 767758.895 | -19.84 |
| 293 | 2182926.846 | 767797.9448 | -22.12 |
| 294 | 2182927.185 | 767836.9946 | -30.83 |
| 295 | 2182927.523 | 767876.0444 | -31.03 |
| 296 | 2182927.861 | 767915.0942 | -18.79 |
| 297 | 2182930.608 | 767493.173 | 28.43 |
| 298 | 2182930.946 | 767532.2227 | 20.84 |
| 299 | 2182931.284 | 767571.2726 | 21.44 |
| 300 | 2182931.623 | 767610.3223 | 29.39 |
| 301 | 2182931.961 | 767649.3721 | 13.10 |
| 302 | 2182932.3 | 767688.4219 | -21.80 |
| 303 | 2182932.638 | 767727.4717 | -20.68 |
| 304 | 2182932.976 | 767766.5215 | -19.98 |
| 305 | 2182933.314 | 767805.5713 | -24.17 |
| 306 | 2182933.653 | 767844.6211 | -31.26 |
| 307 | 2182933.991 | 767883.6708 | -29.93 |
| 308 | 2182934.33 | 767922.7206 | -15.70 |
| 309 | 2182936.737 | 767461.7496 | 29.35 |
| 310 | 2182937.076 | 767500.7995 | 27.31 |
| 311 | 2182937.414 | 767539.8492 | 9.13 |
| 312 | 2182937.753 | 767578.899 | 20.88 |
| 313 | 2182938.091 | 767617.9488 | 26.56 |
| 314 | 2182938.429 | 767656.9986 | 1.66 |
| 315 | 2182938.768 | 767696.0483 | -21.24 |
| 316 | 2182939.106 | 767735.0982 | -20.64 |
| 317 | 2182939.444 | 767774.1479 | -20.21 |
| 318 | 2182939.783 | 767813.1977 | -26.21 |
| 319 | 2182940.121 | 767852.2475 | -31.42 |
| 320 | 2182940.459 | 767891.2973 | -27.05 |
| 321 | 2182940.798 | 767930.3471 | -13.85 |
| 322 | 2182943.205 | 767469.3761 | 28.87 |
| 323 | 2182943.544 | 767508.4259 | 24.87 |
| 324 | 2182943.882 | 767547.4757 | -0.46 |
| 325 | 2182944.221 | 767586.5255 | 19.22 |
| 326 | 2182944.559 | 767625.5752 | 22.66 |
| 327 | 2182944.897 | 767664.6251 | -2.42 |
| 328 | 2182945.236 | 767703.6748 | -22.95 |
| 329 | 2182945.574 | 767742.7246 | -22.82 |
| 330 | 2182945.912 | 767781.7744 | -22.22 |
| 331 | 2182946.251 | 767820.8242 | -27.98 |
| 332 | 2182946.589 | 767859.8739 | -31.28 |
| 333 | 2182946.927 | 767898.9238 | -23.09 |
| 334 | 2182949.674 | 767477.0026 | 28.56 |
| 335 | 2182950.012 | 767516.0524 | 19.57 |
| 336 | 2182950.351 | 767555.1021 | -2.79 |
| 337 | 2182950.689 | 767594.152 | 15.60 |
| 338 | 2182951.028 | 767633.2017 | 15.90 |
| 339 | 2182951.366 | 767672.2515 | -15.36 |
| 340 | 2182951.704 | 767711.3013 | -32.33 |
| 341 | 2182952.042 | 767750.3511 | -26.52 |
| 342 | 2182952.381 | 767789.4009 | -24.16 |
| 343 | 2182952.719 | 767828.4507 | -29.40 |
| 344 | 2182953.058 | 767867.5004 | -30.90 |
| 345 | 2182953.396 | 767906.5502 | -19.80 |
| 346 | 2182955.804 | 767445.5793 | 30.95 |
| 347 | 2182956.142 | 767484.6291 | 28.21 |
| 348 | 2182956.48 | 767523.6789 | 10.46 |
| 349 | 2182956.819 | 767562.7286 | -1.83 |
| 350 | 2182957.157 | 767601.7784 | 9.96 |
| 351 | 2182957.496 | 767640.8281 | 6.07 |
| 352 | 2182957.834 | 767679.878 | -20.44 |
| 353 | 2182958.172 | 767718.9278 | -45.70 |
| 354 | 2182958.51 | 767757.9775 | -32.87 |
| 355 | 2182958.849 | 767797.0273 | -25.49 |
| 356 | 2182959.187 | 767836.0771 | -30.26 |
| 357 | 2182959.526 | 767875.1269 | -29.77 |
| 358 | 2182959.864 | 767914.1767 | -18.25 |
| 359 | 2182962.272 | 767453.2057 | 30.23 |
| 360 | 2182962.61 | 767492.2555 | 26.71 |
| 361 | 2182962.948 | 767531.3053 | 1.22 |
| 362 | 2182963.287 | 767570.3551 | -1.21 |
| 363 | 2182963.625 | 767609.4049 | 4.70 |
| 364 | 2182963.964 | 767648.4546 | 1.63 |
| 365 | 2182964.302 | 767687.5044 | -25.77 |
| 366 | 2182964.64 | 767726.5542 | -51.63 |
| 367 | 2182964.979 | 767765.604 | -41.01 |
| 368 | 2182965.317 | 767804.6538 | -25.96 |
| 369 | 2182965.655 | 767843.7036 | -30.41 |
| 370 | 2182965.994 | 767882.7533 | -28.11 |
| 371 | 2182968.74 | 767460.8322 | 29.90 |
| 372 | 2182969.078 | 767499.8819 | 23.44 |
| 373 | 2182969.417 | 767538.9318 | -2.93 |
| 374 | 2182969.755 | 767577.9815 | -1.16 |
| 375 | 2182970.093 | 767617.0313 | 1.72 |
| 376 | 2182970.431 | 767656.0811 | -10.61 |
| 377 | 2182970.77 | 767695.1309 | -40.39 |
| 378 | 2182971.108 | 767734.1807 | -50.95 |
| 379 | 2182971.447 | 767773.2305 | -46.68 |
| 380 | 2182971.785 | 767812.2802 | -26.69 |
| 381 | 2182972.123 | 767851.33 | -30.08 |
| 382 | 2182972.462 | 767890.3797 | -27.86 |
| 383 | 2182974.87 | 767429.4089 | 39.22 |
| 384 | 2182975.208 | 767468.4587 | 29.39 |
| 385 | 2182975.547 | 767507.5084 | 16.24 |
| 386 | 2182975.885 | 767546.5582 | -3.43 |
| 387 | 2182976.224 | 767585.608 | -0.75 |
| 388 | 2182976.561 | 767624.6578 | 0.70 |
| 389 | 2182976.9 | 767663.7076 | -21.39 |
| 390 | 2182977.238 | 767702.7574 | -55.23 |
| 391 | 2182977.577 | 767741.8071 | -58.62 |
| 392 | 2182977.915 | 767780.8569 | -49.65 |
| 393 | 2182978.254 | 767819.9066 | -29.23 |
| 394 | 2182978.592 | 767858.9565 | -29.01 |
| 395 | 2182978.93 | 767898.0063 | -29.12 |
| 396 | 2182981.338 | 767437.0354 | 38.20 |
| 397 | 2182981.676 | 767476.0851 | 28.03 |
| 398 | 2182982.015 | 767515.1349 | 5.16 |
| 399 | 2182982.353 | 767554.1847 | -1.97 |
| 400 | 2182982.692 | 767593.2345 | 0.09 |
| 401 | 2182983.03 | 767632.2843 | -1.87 |
| 402 | 2182983.368 | 767671.3341 | -30.11 |
| 403 | 2182983.706 | 767710.3838 | -65.61 |
| 404 | 2182984.045 | 767749.4336 | -66.30 |
| 405 | 2182984.383 | 767788.4834 | -51.57 |
| 406 | 2182984.722 | 767827.5332 | -32.87 |
| 407 | 2182985.06 | 767866.5829 | -28.59 |
| 408 | 2182987.806 | 767444.6617 | 36.29 |
| 409 | 2182988.144 | 767483.7116 | 25.23 |
| 410 | 2182988.483 | 767522.7613 | -1.76 |
| 411 | 2182988.821 | 767561.8111 | -0.22 |
| 412 | 2182989.16 | 767600.8609 | -0.63 |
| 413 | 2182989.498 | 767639.9107 | -16.81 |
| 414 | 2182989.836 | 767678.9605 | -43.92 |
| 415 | 2182990.175 | 767718.0103 | -75.42 |
| 416 | 2182990.513 | 767757.06 | -69.68 |
| 417 | 2182990.851 | 767796.1098 | -53.43 |
| 418 | 2182991.19 | 767835.1596 | -33.73 |
| 419 | 2182991.528 | 767874.2094 | -30.38 |
| 420 | 2182993.936 | 767413.2385 | 42.51 |
| 421 | 2182994.274 | 767452.2882 | 34.04 |
| 422 | 2182994.613 | 767491.338 | 20.15 |
| 423 | 2182994.951 | 767530.3878 | -2.21 |
| 424 | 2182995.289 | 767569.4376 | -0.84 |
| 425 | 2182995.627 | 767608.4874 | -7.50 |
| 426 | 2182995.966 | 767647.5372 | -22.94 |
| 427 | 2182996.304 | 767686.587 | -60.76 |
| 428 | 2182996.643 | 767725.6367 | -83.25 |
| 429 | 2182996.981 | 767764.6864 | -71.10 |
| 430 | 2182997.319 | 767803.7363 | -54.99 |
| 431 | 2182997.658 | 767842.786 | -31.46 |
| 432 | 2182997.996 | 767881.8359 | -37.07 |
| 433 | 2183000.404 | 767420.8649 | 42.04 |
| 434 | 2183000.743 | 767459.9147 | 31.27 |
| 435 | 2183001.081 | 767498.9645 | 11.72 |
| 436 | 2183001.42 | 767538.0143 | -0.78 |
| 437 | 2183001.757 | 767577.0641 | -5.19 |
| 438 | 2183002.096 | 767616.1138 | -16.91 |
| 439 | 2183002.434 | 767655.1636 | -42.43 |
| 440 | 2183002.773 | 767694.2134 | -79.99 |
| 441 | 2183003.111 | 767733.2632 | -87.69 |
| 442 | 2183003.45 | 767772.3129 | -72.68 |
| 443 | 2183003.788 | 767811.3628 | -55.87 |
| 444 | 2183004.126 | 767850.4126 | -32.58 |
| 445 | 2183006.872 | 767428.4914 | 40.66 |
| 446 | 2183007.211 | 767467.5411 | 27.90 |
| 447 | 2183007.549 | 767506.591 | 2.94 |
| 448 | 2183007.888 | 767545.6406 | 0.50 |
| 449 | 2183008.226 | 767584.6905 | -11.81 |
| 450 | 2183008.564 | 767623.7403 | -21.43 |
| 451 | 2183008.902 | 767662.7901 | -64.38 |
| 452 | 2183009.241 | 767701.8399 | -99.87 |
| 453 | 2183009.579 | 767740.8897 | -89.21 |
| 454 | 2183009.918 | 767779.9394 | -75.08 |
| 455 | 2183010.256 | 767818.9892 | -54.44 |
| 456 | 2183010.594 | 767858.039 | -35.46 |
| 457 | 2183013.002 | 767397.068 | 42.72 |
| 458 | 2183013.34 | 767436.1179 | 37.81 |
| 459 | 2183013.679 | 767475.1676 | 23.28 |
| 460 | 2183014.017 | 767514.2174 | -1.29 |
| 461 | 2183014.356 | 767553.2671 | -1.14 |
| 462 | 2183014.694 | 767592.317 | -19.45 |
| 463 | 2183015.032 | 767631.3668 | -24.82 |
| 464 | 2183015.37 | 767670.4165 | -84.41 |
| 465 | 2183015.709 | 767709.4663 | -118.77 |
| 466 | 2183016.047 | 767748.5161 | -89.57 |
| 467 | 2183016.385 | 767787.5659 | -77.79 |
| 468 | 2183016.724 | 767826.6157 | -48.73 |
| 469 | 2183017.062 | 767865.6654 | -31.49 |
| 470 | 2183017.062 | 767865.6654 | -31.49 |
| 471 | 2183019.47 | 767404.6945 | 42.55 |
| 472 | 2183019.809 | 767443.7443 | 33.67 |
| 473 | 2183020.147 | 767482.794 | 16.49 |
| 474 | 2183020.485 | 767521.8439 | -1.01 |
| 475 | 2183020.823 | 767560.8937 | -6.91 |
| 476 | 2183021.5 | 767638.9932 | -47.21 |
| 477 | 2183021.839 | 767678.043 | -104.62 |
| 478 | 2183022.178 | 767717.0928 | -115.68 |
| 479 | 2183022.515 | 767756.1426 | -89.90 |
| 480 | 2183022.854 | 767795.1923 | -79.26 |
| 481 | 2183023.192 | 767834.2421 | -40.58 |
| 482 | 2183023.531 | 767873.2918 | -29.89 |
| 483 | 2183025.938 | 767412.3209 | 41.83 |
| 484 | 2183026.277 | 767451.3708 | 29.04 |
| 485 | 2183026.616 | 767490.4205 | 7.65 |
| 486 | 2183026.953 | 767529.4703 | 0.01 |
| 487 | 2183027.292 | 767568.5201 | -14.03 |
| 488 | 2183027.969 | 767646.6197 | -74.96 |
| 489 | 2183028.307 | 767685.6695 | -122.28 |
| 490 | 2183028.646 | 767724.7192 | -108.33 |
| 491 | 2183028.983 | 767763.769 | -90.94 |
| 492 | 2183029.322 | 767802.8187 | -79.86 |
| 493 | 2183029.66 | 767841.8686 | -34.77 |
| 494 | 2183029.999 | 767880.9183 | -30.12 |
| 495 | 2183032.068 | 767380.8977 | 40.15 |
| 496 | 2183032.407 | 767419.9474 | 39.49 |
| 497 | 2183032.745 | 767458.9972 | 24.44 |
| 498 | 2183033.084 | 767498.047 | -0.33 |
| 499 | 2183033.422 | 767537.0968 | -1.36 |
| 500 | 2183033.76 | 767576.1465 | -21.26 |
| 501 | 2183034.437 | 767654.2461 | -98.76 |
| 502 | 2183034.775 | 767693.2959 | -133.10 |
| 503 | 2183035.114 | 767732.3457 | -103.05 |
| 504 | 2183035.452 | 767771.3955 | -92.50 |
| 505 | 2183035.79 | 767810.4452 | -79.36 |
| 506 | 2183036.128 | 767849.4951 | -31.19 |
| 507 | 2183036.129 | 767849.495 | -31.19 |
| 508 | 2183036.467 | 767888.5448 | -29.93 |
| 509 | 2183038.536 | 767388.5241 | 39.15 |
| 510 | 2183038.875 | 767427.5739 | 34.09 |
| 511 | 2183039.213 | 767466.6237 | 19.91 |
| 512 | 2183039.552 | 767505.6734 | -1.95 |
| 513 | 2183039.89 | 767544.7233 | -6.84 |
| 514 | 2183040.905 | 767661.8726 | -122.65 |
| 515 | 2183041.243 | 767700.9224 | -134.43 |
| 516 | 2183041.581 | 767739.9721 | -101.13 |
| 517 | 2183041.92 | 767779.0219 | -93.97 |
| 518 | 2183042.258 | 767818.0717 | -79.09 |
| 519 | 2183042.597 | 767857.1214 | -29.73 |
| 520 | 2183042.935 | 767896.1712 | -29.80 |
| 521 | 2183045.005 | 767396.1506 | 37.58 |
| 522 | 2183045.343 | 767435.2003 | 28.27 |
| 523 | 2183045.681 | 767474.2501 | 15.79 |
| 524 | 2183046.019 | 767513.2999 | -0.91 |
| 525 | 2183046.358 | 767552.3497 | -14.32 |
| 526 | 2183047.373 | 767669.499 | -141.43 |
| 527 | 2183047.711 | 767708.5488 | -127.56 |
| 528 | 2183048.05 | 767747.5986 | -100.52 |
| 529 | 2183048.388 | 767786.6484 | -94.87 |
| 530 | 2183048.727 | 767825.6982 | -77.52 |
| 531 | 2183049.065 | 767864.7479 | -29.78 |
| 532 | 2183049.404 | 767903.7977 | -26.59 |
| 533 | 2183051.134 | 767364.7272 | 39.42 |
| 534 | 2183051.473 | 767403.777 | 34.67 |
| 535 | 2183051.811 | 767442.8268 | 22.67 |
| 536 | 2183052.149 | 767481.8766 | 12.62 |
| 537 | 2183052.488 | 767520.9263 | -0.51 |
| 538 | 2183052.826 | 767559.9762 | -21.77 |
| 539 | 2183053.842 | 767677.1254 | -152.60 |
| 540 | 2183054.179 | 767716.1753 | -119.26 |
| 541 | 2183054.518 | 767755.225 | -99.47 |
| 542 | 2183054.856 | 767794.2749 | -94.65 |
| 543 | 2183055.195 | 767833.3246 | -65.16 |
| 544 | 2183055.195 | 767833.3246 | -65.16 |
| 545 | 2183055.533 | 767872.3743 | -29.04 |
| 546 | 2183055.872 | 767911.4241 | -21.03 |
| 547 | 2183057.603 | 767372.3537 | 37.59 |
| 548 | 2183057.941 | 767411.4035 | 30.52 |
| 549 | 2183058.28 | 767450.4532 | 18.68 |
| 550 | 2183058.618 | 767489.5031 | 8.07 |
| 551 | 2183058.956 | 767528.5528 | -3.40 |
| 552 | 2183060.309 | 767684.752 | -154.57 |
| 553 | 2183060.648 | 767723.8018 | -113.01 |
| 554 | 2183060.986 | 767762.8515 | -98.63 |
| 555 | 2183061.324 | 767801.9013 | -93.10 |
| 556 | 2183061.663 | 767840.951 | -45.51 |
| 557 | 2183062.001 | 767880.0008 | -28.28 |
| 558 | 2183062.34 | 767919.0506 | -20.50 |
| 559 | 2183064.071 | 767379.9801 | 35.15 |
| 560 | 2183064.409 | 767419.03 | 25.86 |
| 561 | 2183064.748 | 767458.0796 | 16.72 |
| 562 | 2183065.086 | 767497.1295 | 4.58 |
| 563 | 2183065.424 | 767536.1793 | -8.73 |
| 564 | 2183066.777 | 767692.3784 | -131.81 |
| 565 | 2183067.116 | 767731.4282 | -108.98 |
| 566 | 2183067.454 | 767770.4779 | -98.26 |
| 567 | 2183067.792 | 767809.5278 | -88.98 |
| 568 | 2183068.132 | 767848.5774 | -34.89 |
| 569 | 2183068.469 | 767887.6273 | -27.85 |
| 570 | 2183068.808 | 767926.6771 | -23.30 |
| 571 | 2183070.201 | 767348.5569 | 39.63 |
| 572 | 2183070.539 | 767387.6066 | 31.95 |
| 573 | 2183070.877 | 767426.6564 | 21.65 |
| 574 | 2183071.216 | 767465.7061 | 15.31 |
| 575 | 2183071.554 | 767504.756 | 3.62 |
| 576 | 2183071.892 | 767543.8057 | -14.39 |
| 577 | 2183073.246 | 767700.0049 | -119.88 |
| 578 | 2183073.584 | 767739.0547 | -104.83 |
| 579 | 2183073.923 | 767778.1045 | -97.39 |
| 580 | 2183074.261 | 767817.1542 | -76.18 |
| 581 | 2183074.261 | 767817.1542 | -76.18 |
| 582 | 2183074.6 | 767856.204 | -30.95 |
| 583 | 2183074.938 | 767895.2537 | -27.92 |
| 584 | 2183075.276 | 767934.3035 | -29.01 |
| 585 | 2183076.669 | 767356.1833 | 38.45 |
| 586 | 2183077.007 | 767395.2331 | 27.99 |
| 587 | 2183077.345 | 767434.2829 | 18.48 |
| 588 | 2183077.683 | 767473.3326 | 14.12 |
| 589 | 2183078.022 | 767512.3824 | 1.01 |
| 590 | 2183079.714 | 767707.6313 | -113.19 |
| 591 | 2183080.052 | 767746.6811 | -101.13 |
| 592 | 2183080.391 | 767785.7309 | -95.48 |
| 593 | 2183080.729 | 767824.7806 | -59.04 |
| 594 | 2183081.068 | 767863.8304 | -29.04 |
| 595 | 2183081.406 | 767902.8802 | -29.11 |
| 596 | 2183081.745 | 767941.93 | -29.94 |
| 597 | 2183083.137 | 767363.8098 | 36.49 |
| 598 | 2183083.476 | 767402.8595 | 23.39 |
| 599 | 2183083.814 | 767441.9093 | 16.65 |
| 600 | 2183084.152 | 767480.9591 | 12.67 |
| 601 | 2183084.49 | 767520.0089 | -4.02 |
| 602 | 2183086.182 | 767715.2578 | -108.33 |
| 603 | 2183086.52 | 767754.3076 | -98.88 |
| 604 | 2183086.859 | 767793.3573 | -90.95 |
| 605 | 2183087.197 | 767832.4071 | -47.38 |
| 606 | 2183087.536 | 767871.4568 | -28.68 |
| 607 | 2183087.874 | 767910.5066 | -30.11 |
| 608 | 2183088.213 | 767949.5564 | -29.84 |
| 609 | 2183089.267 | 767332.3864 | 40.09 |
| 610 | 2183089.605 | 767371.4362 | 33.81 |
| 611 | 2183089.944 | 767410.486 | 19.30 |
| 612 | 2183090.282 | 767449.5358 | 15.72 |
| 613 | 2183090.62 | 767488.5856 | 9.49 |
| 614 | 2183090.958 | 767527.6354 | -9.43 |
| 615 | 2183092.65 | 767722.8843 | -104.10 |
| 616 | 2183092.988 | 767761.9341 | -97.05 |
| 617 | 2183093.328 | 767800.9838 | -80.35 |
| 618 | 2183093.328 | 767800.9838 | -80.35 |
| 619 | 2183093.665 | 767840.0335 | -39.77 |
| 620 | 2183094.004 | 767879.0834 | -29.21 |
| 621 | 2183094.342 | 767918.1331 | -30.25 |
| 622 | 2183094.681 | 767957.1828 | -30.29 |
| 623 | 2183095.735 | 767340.0129 | 39.87 |
| 624 | 2183096.073 | 767379.0627 | 30.83 |
| 625 | 2183096.412 | 767418.1124 | 16.70 |
| 626 | 2183096.75 | 767457.1623 | 14.54 |
| 627 | 2183097.088 | 767496.212 | 3.10 |
| 628 | 2183099.119 | 767730.5107 | -100.49 |
| 629 | 2183099.457 | 767769.5605 | -93.77 |
| 630 | 2183099.796 | 767808.6102 | -66.08 |
| 631 | 2183100.134 | 767847.66 | -34.53 |
| 632 | 2183100.472 | 767886.7098 | -29.95 |
| 633 | 2183100.81 | 767925.7596 | -29.55 |
| 634 | 2183101.148 | 767964.8094 | -33.68 |
| 635 | 2183102.203 | 767347.6394 | 38.10 |
| 636 | 2183102.541 | 767386.6891 | 26.74 |
| 637 | 2183102.88 | 767425.7389 | 14.82 |
| 638 | 2183103.218 | 767464.7887 | 12.61 |
| 639 | 2183103.556 | 767503.8385 | -3.09 |
| 640 | 2183105.587 | 767738.1372 | -96.16 |
| 641 | 2183105.925 | 767777.187 | -87.09 |
| 642 | 2183106.263 | 767816.2366 | -53.38 |
| 643 | 2183106.602 | 767855.2865 | -31.06 |
| 644 | 2183106.94 | 767894.3363 | -30.06 |
| 645 | 2183107.278 | 767933.386 | -28.76 |
| 646 | 2183107.617 | 767972.4359 | -39.27 |
| 647 | 2183108.671 | 767355.2658 | 31.89 |
| 648 | 2183109.01 | 767394.3156 | 21.62 |
| 649 | 2183109.349 | 767433.3653 | 13.56 |
| 650 | 2183109.686 | 767472.4152 | 9.29 |
| 651 | 2183110.025 | 767511.4649 | -8.09 |
| 652 | 2183112.055 | 767745.7636 | -91.52 |
| 653 | 2183112.393 | 767784.8134 | -76.89 |
| 654 | 2183112.393 | 767784.8133 | -76.89 |
| 655 | 2183112.731 | 767823.8632 | -44.85 |
| 656 | 2183113.07 | 767862.9129 | -29.59 |
| 657 | 2183113.408 | 767901.9627 | -29.40 |
| 658 | 2183113.747 | 767941.0125 | -28.78 |
| 659 | 2183114.085 | 767980.0623 | -37.76 |
| 660 | 2183115.14 | 767362.8923 | 30.21 |
| 661 | 2183115.478 | 767401.9421 | 16.32 |
| 662 | 2183115.816 | 767440.9918 | 12.38 |
| 663 | 2183116.154 | 767480.0416 | 4.11 |
| 664 | 2183118.523 | 767753.3901 | -87.29 |
| 665 | 2183118.861 | 767792.4398 | -65.42 |
| 666 | 2183119.2 | 767831.4896 | -38.76 |
| 667 | 2183119.538 | 767870.5394 | -29.55 |
| 668 | 2183119.876 | 767909.5892 | -28.64 |
| 669 | 2183120.215 | 767948.6389 | -31.62 |
| 670 | 2183120.553 | 767987.6887 | -34.25 |
| 671 | 2183121.608 | 767370.5187 | 27.23 |
| 672 | 2183121.946 | 767409.5685 | 11.28 |
| 673 | 2183122.284 | 767448.6182 | 10.53 |
| 674 | 2183122.623 | 767487.6681 | -2.53 |
| 675 | 2183124.991 | 767761.0165 | -81.19 |
| 676 | 2183125.33 | 767800.0663 | -53.27 |
| 677 | 2183125.668 | 767839.1161 | -33.59 |
| 678 | 2183126.006 | 767878.1658 | -29.73 |
| 679 | 2183126.345 | 767917.2156 | -28.36 |
| 680 | 2183126.683 | 767956.2654 | -37.55 |
| 681 | 2183127.021 | 767995.3152 | -30.26 |
| 682 | 2183128.076 | 767378.1452 | 21.95 |
| 683 | 2183128.414 | 767417.195 | 7.14 |
| 684 | 2183128.752 | 767456.2447 | 6.00 |
| 685 | 2183129.091 | 767495.2945 | -9.15 |
| 686 | 2183131.459 | 767768.643 | -68.85 |
| 687 | 2183131.459 | 767768.643 | -68.85 |
| 688 | 2183131.798 | 767807.6927 | -42.73 |
| 689 | 2183132.136 | 767846.7425 | -30.45 |
| 690 | 2183132.474 | 767885.7923 | -29.18 |
| 691 | 2183132.813 | 767924.8421 | -29.74 |
| 692 | 2183133.151 | 767963.8919 | -39.05 |
| 693 | 2183133.49 | 768002.9416 | -26.29 |
| 694 | 2183134.544 | 767385.7716 | 9.72 |
| 695 | 2183134.882 | 767424.8214 | 3.72 |
| 696 | 2183135.221 | 767463.8711 | 0.42 |
| 697 | 2183137.928 | 767776.2694 | -54.32 |
| 698 | 2183138.266 | 767815.3192 | -36.13 |
| 699 | 2183138.604 | 767854.3689 | -29.75 |
| 700 | 2183138.943 | 767893.4187 | -29.34 |
| 701 | 2183139.281 | 767932.4685 | -34.10 |
| 702 | 2183139.619 | 767971.5183 | -33.44 |
| 703 | 2183139.958 | 768010.568 | -23.21 |
| 704 | 2183141.013 | 767393.3981 | 0.64 |
| 705 | 2183141.35 | 767432.4479 | 0.56 |
| 706 | 2183141.689 | 767471.4976 | -2.87 |
| 707 | 2183144.396 | 767783.8959 | -40.14 |
| 708 | 2183144.734 | 767822.9456 | -32.13 |
| 709 | 2183145.073 | 767861.9955 | -29.80 |
| 710 | 2183145.411 | 767901.0452 | -31.89 |
| 711 | 2183145.749 | 767940.095 | -38.51 |
| 712 | 2183146.087 | 767979.1448 | -26.71 |
| 713 | 2183146.426 | 768018.1946 | -21.30 |
| 714 | 2183147.481 | 767401.0245 | -4.30 |
| 715 | 2183147.818 | 767440.0744 | -1.99 |
| 716 | 2183148.157 | 767479.1241 | -8.95 |
| 717 | 2183150.525 | 767752.4725 | -35.85 |
| 718 | 2183150.864 | 767791.5223 | -32.58 |
| 719 | 2183151.202 | 767830.5721 | -30.24 |
| 720 | 2183151.541 | 767869.6219 | -30.70 |
| 721 | 2183151.879 | 767908.6717 | -36.65 |
| 722 | 2183152.218 | 767947.7215 | -37.90 |
| 723 | 2183152.556 | 767986.7712 | -21.56 |
| 724 | 2183152.894 | 768025.821 | -22.45 |
| 725 | 2183153.948 | 767408.651 | -3.66 |
| 726 | 2183154.287 | 767447.7008 | -4.55 |
| 727 | 2183156.994 | 767760.099 | -30.44 |
| 728 | 2183157.332 | 767799.1487 | -30.00 |
| 729 | 2183157.67 | 767838.1986 | -29.69 |
| 730 | 2183158.009 | 767877.2483 | -34.12 |
| 731 | 2183158.347 | 767916.2981 | -39.51 |
| 732 | 2183158.686 | 767955.3479 | -30.45 |
| 733 | 2183159.024 | 767994.3977 | -19.61 |
| 734 | 2183159.363 | 768033.4475 | -29.32 |
| 735 | 2183160.416 | 767416.2775 | -2.37 |
| 736 | 2183160.755 | 767455.3273 | -7.17 |
| 737 | 2183163.462 | 767767.7255 | -26.36 |
| 738 | 2183163.8 | 767806.7752 | -29.66 |
| 739 | 2183164.138 | 767845.825 | -29.22 |
| 740 | 2183164.477 | 767884.8748 | -38.20 |
| 741 | 2183164.815 | 767923.9246 | -37.65 |
| 742 | 2183165.154 | 767962.9743 | -22.56 |
| 743 | 2183165.492 | 768002.0241 | -20.40 |
| 744 | 2183165.831 | 768041.0739 | -29.68 |
| 745 | 2183166.885 | 767423.9039 | -2.59 |
| 746 | 2183167.223 | 767462.9537 | -9.59 |
| 747 | 2183169.592 | 767736.3021 | -10.04 |
| 748 | 2183169.93 | 767775.3519 | -24.45 |
| 749 | 2183170.269 | 767814.4017 | -29.24 |
| 750 | 2183170.607 | 767853.4515 | -30.80 |
| 751 | 2183170.945 | 767892.5013 | -38.52 |
| 752 | 2183171.283 | 767931.551 | -31.93 |
| 753 | 2183171.4 | 767389.3706 | 12.25 |
| 754 | 2183171.622 | 767970.6008 | -19.36 |
| 755 | 2183171.96 | 768009.6506 | -22.45 |
| 756 | 2183172.299 | 768048.7004 | -29.82 |
| 757 | 2183173.353 | 767431.5304 | -3.91 |
| 758 | 2183176.06 | 767743.9286 | -2.63 |
| 759 | 2183176.398 | 767782.9784 | -20.72 |
| 760 | 2183176.737 | 767822.0282 | -28.79 |
| 761 | 2183177.075 | 767861.0779 | -36.50 |
| 762 | 2183177.414 | 767900.1277 | -34.43 |
| 763 | 2183177.751 | 767939.1775 | -24.77 |
| 764 | 2183177.868 | 767396.997 | 11.92 |
| 765 | 2183178.091 | 767978.2273 | -20.00 |
| 766 | 2183178.428 | 768017.2771 | -25.03 |
| 767 | 2183178.766 | 768056.3268 | -32.52 |
| 768 | 2183179.821 | 767439.1568 | -5.90 |
| 769 | 2183182.528 | 767751.5551 | 0.35 |
| 770 | 2183182.866 | 767790.6048 | -20.66 |
| 771 | 2183183.205 | 767829.6546 | -29.77 |
| 772 | 2183183.543 | 767868.7044 | -38.74 |
| 773 | 2183183.882 | 767907.7541 | -28.50 |
| 774 | 2183184.22 | 767946.804 | -20.31 |
| 775 | 2183184.336 | 767404.6235 | 9.06 |
| 776 | 2183184.558 | 767985.8538 | -21.96 |
| 777 | 2183184.896 | 768024.9035 | -27.09 |
| 778 | 2183185.235 | 768063.9533 | -38.94 |
| 779 | 2183186.289 | 767446.7833 | -8.75 |
| 780 | 2183188.658 | 767720.1317 | -3.08 |
| 781 | 2183188.997 | 767759.1815 | -2.14 |
| 782 | 2183189.334 | 767798.2313 | -23.08 |
| 783 | 2183189.673 | 767837.2811 | -34.44 |
| 784 | 2183190.011 | 767876.3309 | -33.14 |
| 785 | 2183190.35 | 767915.3806 | -23.24 |
| 786 | 2183190.688 | 767954.4304 | -20.67 |
| 787 | 2183190.804 | 767412.2499 | 5.38 |
| 788 | 2183191.026 | 767993.4802 | -24.59 |
| 789 | 2183191.364 | 768032.53 | -28.71 |
| 790 | 2183191.703 | 768071.5798 | -40.24 |
| 791 | 2183195.126 | 767727.7582 | 0.77 |
| 792 | 2183195.465 | 767766.808 | -9.44 |
| 793 | 2183195.803 | 767805.8577 | -27.28 |
| 794 | 2183196.142 | 767844.9075 | -38.77 |
| 795 | 2183196.479 | 767883.9573 | -25.38 |
| 796 | 2183196.817 | 767923.0071 | -20.55 |
| 797 | 2183196.935 | 767380.8266 | 6.50 |
| 798 | 2183197.156 | 767962.0569 | -22.81 |
| 799 | 2183197.272 | 767419.8764 | 2.88 |
| 800 | 2183197.494 | 768001.1066 | -27.33 |
| 801 | 2183197.833 | 768040.1564 | -30.96 |
| 802 | 2183198.171 | 768079.2063 | -40.09 |
| 803 | 2183201.594 | 767735.3846 | 1.05 |
| 804 | 2183201.933 | 767774.4344 | -13.57 |
| 805 | 2183202.271 | 767813.4842 | -33.67 |
| 806 | 2183202.61 | 767852.5339 | -35.58 |
| 807 | 2183202.947 | 767891.5838 | -19.87 |
| 808 | 2183203.286 | 767930.6335 | -21.09 |
| 809 | 2183203.403 | 767388.4531 | 4.62 |
| 810 | 2183203.624 | 767969.6833 | -25.52 |
| 811 | 2183203.741 | 767427.5028 | 0.85 |
| 812 | 2183203.963 | 768008.7332 | -29.68 |
| 813 | 2183204.301 | 768047.7829 | -34.75 |
| 814 | 2183204.639 | 768086.8327 |  |
| 815 | 2183207.724 | 767703.9613 | -1.50 |
| 816 | 2183208.062 | 767743.0111 | -1.52 |
| 817 | 2183208.401 | 767782.0609 | -16.62 |
| 818 | 2183208.739 | 767821.1107 | -39.12 |
| 819 | 2183209.077 | 767860.1605 | -25.98 |
| 820 | 2183209.416 | 767899.2102 | -19.21 |
| 821 | 2183209.754 | 767938.26 | -23.92 |
| 822 | 2183209.871 | 767396.0795 | 2.58 |
| 823 | 2183210.092 | 767977.3098 | -28.31 |
| 824 | 2183210.209 | 767435.1293 | -1.86 |
| 825 | 2183210.431 | 768016.3596 | -31.36 |
| 826 | 2183210.769 | 768055.4094 | -37.58 |
| 827 | 2183214.193 | 767711.5878 | -0.05 |
| 828 | 2183214.53 | 767750.6376 | -6.30 |
| 829 | 2183214.869 | 767789.6873 | -23.69 |
| 830 | 2183215.207 | 767828.7371 | -38.10 |
| 831 | 2183215.545 | 767867.7869 | -19.93 |
| 832 | 2183215.884 | 767906.8367 | -20.68 |
| 833 | 2183216 | 767364.6562 | -6.71 |
| 834 | 2183216.222 | 767945.8865 | -27.58 |
| 835 | 2183216.339 | 767403.706 | 2.72 |
| 836 | 2183216.56 | 767984.9363 | -30.32 |
| 837 | 2183216.677 | 767442.7558 | -5.22 |
| 838 | 2183216.899 | 768023.986 | -33.08 |
| 839 | 2183217.237 | 768063.0358 | -39.01 |
| 840 | 2183220.661 | 767719.2142 | -0.80 |
| 841 | 2183220.999 | 767758.264 | -11.93 |
| 842 | 2183221.337 | 767797.3138 | -35.57 |
| 843 | 2183221.675 | 767836.3636 | -30.89 |
| 844 | 2183222.014 | 767875.4134 | -19.23 |
| 845 | 2183222.352 | 767914.4631 | -22.81 |
| 846 | 2183222.468 | 767372.2827 | -10.12 |
| 847 | 2183222.69 | 767953.5129 | -29.43 |
| 848 | 2183222.807 | 767411.3324 | 2.77 |
| 849 | 2183223.029 | 767992.5627 | -31.73 |
| 850 | 2183223.145 | 767450.3822 | -8.68 |
| 851 | 2183223.367 | 768031.6125 | -35.14 |
| 852 | 2183223.705 | 768070.6623 | -39.98 |
| 853 | 2183226.79 | 767687.7909 | -0.06 |
| 854 | 2183227.129 | 767726.8407 | -3.89 |
| 855 | 2183227.467 | 767765.8905 | -18.14 |
| 856 | 2183227.805 | 767804.9402 | -40.81 |
| 857 | 2183228.143 | 767843.99 | -22.18 |
| 858 | 2183228.482 | 767883.0398 | -20.85 |
| 859 | 2183228.82 | 767922.0896 | -25.54 |
| 860 | 2183228.937 | 767379.9091 | -11.60 |
| 861 | 2183229.159 | 767961.1394 | -30.17 |
| 862 | 2183229.275 | 767418.9589 | 1.81 |
| 863 | 2183229.497 | 768000.1892 | -33.26 |
| 864 | 2183229.613 | 767458.0087 | -10.27 |
| 865 | 2183229.836 | 768039.2389 | -36.81 |
| 866 | 2183233.258 | 767695.4174 | 0.51 |
| 867 | 2183233.597 | 767734.4671 | -8.85 |
| 868 | 2183233.935 | 767773.5169 | -25.95 |
| 869 | 2183234.273 | 767812.5667 | -35.56 |
| 870 | 2183234.612 | 767851.6165 | -20.73 |
| 871 | 2183234.95 | 767890.6662 | -23.19 |
| 872 | 2183235.288 | 767929.7161 | -28.50 |
| 873 | 2183235.405 | 767387.5356 | -5.32 |
| 874 | 2183235.627 | 767968.7658 | -31.55 |
| 875 | 2183235.743 | 767426.5853 | -0.71 |
| 876 | 2183235.965 | 768007.8156 | -34.83 |
| 877 | 2183236.081 | 767465.6351 | -10.08 |
| 878 | 2183236.304 | 768046.8654 | -38.27 |
| 879 | 2183239.726 | 767703.0438 | -1.34 |
| 880 | 2183240.065 | 767742.0936 | -14.36 |
| 881 | 2183240.403 | 767781.1434 | -34.78 |
| 882 | 2183240.741 | 767820.1931 | -27.74 |
| 883 | 2183241.08 | 767859.243 | -24.01 |
| 884 | 2183241.418 | 767898.2928 | -25.86 |
| 885 | 2183241.535 | 767356.1122 | -9.97 |
| 886 | 2183241.756 | 767937.3425 | -30.70 |
| 887 | 2183241.873 | 767395.162 | 1.44 |
| 888 | 2183242.095 | 767976.3923 | -33.45 |
| 889 | 2183242.211 | 767434.2118 | -4.56 |
| 890 | 2183242.433 | 768015.4421 | -36.41 |
| 891 | 2183242.55 | 767473.2616 | -9.67 |
| 892 | 2183242.772 | 768054.4919 | -39.54 |
| 893 | 2183245.857 | 767671.6205 | 1.54 |
| 894 | 2183245.857 | 767671.6205 | 1.54 |
| 895 | 2183246.195 | 767710.6703 | -5.71 |
| 896 | 2183246.533 | 767749.72 | -20.15 |
| 897 | 2183246.871 | 767788.7699 | -39.28 |
| 898 | 2183247.21 | 767827.8197 | -21.17 |
| 899 | 2183247.548 | 767866.8694 | -27.21 |
| 900 | 2183247.887 | 767905.9192 | -28.28 |
| 901 | 2183248.003 | 767363.7387 | -9.54 |
| 902 | 2183248.225 | 767944.969 | -32.37 |
| 903 | 2183248.341 | 767402.7885 | 0.58 |
| 904 | 2183248.563 | 767984.0187 | -35.01 |
| 905 | 2183248.679 | 767441.8382 | -7.89 |
| 906 | 2183248.901 | 768023.0685 | -38.20 |
| 907 | 2183249.018 | 767480.8881 | -9.72 |
| 908 | 2183252.325 | 767679.2469 | 1.07 |
| 909 | 2183252.326 | 767679.2469 | 1.07 |
| 910 | 2183252.663 | 767718.2967 | -11.39 |
| 911 | 2183253.001 | 767757.3465 | -25.89 |
| 912 | 2183253.339 | 767796.3963 | -36.08 |
| 913 | 2183253.678 | 767835.4461 | -25.64 |
| 914 | 2183254.016 | 767874.4959 | -29.67 |
| 915 | 2183254.355 | 767913.5456 | -30.60 |
| 916 | 2183254.471 | 767371.3651 | -7.86 |
| 917 | 2183254.693 | 767952.5954 | -34.12 |
| 918 | 2183254.809 | 767410.415 | -1.76 |
| 919 | 2183255.032 | 767991.6452 | -36.51 |
| 920 | 2183255.148 | 767449.4648 | -10.16 |
| 921 | 2183255.369 | 768030.695 | -39.61 |
| 922 | 2183255.486 | 767488.5145 | -10.15 |
| 923 | 2183258.792 | 767686.8734 | -3.15 |
| 924 | 2183258.794 | 767686.8734 | -3.15 |
| 925 | 2183259.131 | 767725.9232 | -17.28 |
| 926 | 2183259.469 | 767764.973 | -31.97 |
| 927 | 2183259.808 | 767804.0228 | -28.56 |
| 928 | 2183260.146 | 767843.0726 | -31.64 |
| 929 | 2183260.484 | 767882.1223 | -31.23 |
| 930 | 2183260.601 | 767339.9418 | -9.65 |
| 931 | 2183260.823 | 767921.1721 | -32.95 |
| 932 | 2183260.939 | 767378.9916 | -6.05 |
| 933 | 2183261.161 | 767960.2219 | -35.86 |
| 934 | 2183261.277 | 767418.0414 | -4.47 |
| 935 | 2183261.5 | 767999.2716 | -38.11 |
| 936 | 2183261.616 | 767457.0912 | -12.82 |
| 937 | 2183261.838 | 768038.3215 | -40.52 |
| 938 | 2183261.954 | 767496.141 | -10.24 |
| 939 | 2183264.924 | 767655.4501 | -2.32 |
| 940 | 2183265.261 | 767694.4999 | -9.95 |
| 941 | 2183265.262 | 767694.4999 | -9.95 |
| 942 | 2183265.599 | 767733.5497 | -22.96 |
| 943 | 2183265.937 | 767772.5994 | -37.63 |
| 944 | 2183266.276 | 767811.6492 | -21.60 |
| 945 | 2183266.614 | 767850.699 | -33.04 |
| 946 | 2183266.952 | 767889.7488 | -32.75 |
| 947 | 2183267.069 | 767347.5683 | -9.98 |
| 948 | 2183267.291 | 767928.7985 | -35.32 |
| 949 | 2183267.407 | 767386.6181 | -5.79 |
| 950 | 2183267.629 | 767967.8484 | -37.11 |
| 951 | 2183267.746 | 767425.6679 | -6.94 |
| 952 | 2183267.967 | 768006.8981 | -39.66 |
| 953 | 2183268.084 | 767464.7177 | -16.85 |
| 954 | 2183268.422 | 767503.7674 | -10.02 |
| 955 | 2183271.392 | 767663.0765 | -4.64 |
| 956 | 2183271.729 | 767702.1264 | -16.95 |
| 957 | 2183271.73 | 767702.1263 | -16.95 |
| 958 | 2183272.067 | 767741.1761 | -28.05 |
| 959 | 2183272.406 | 767780.2259 | -38.54 |
| 960 | 2183272.744 | 767819.2757 | -21.28 |
| 961 | 2183273.083 | 767858.3255 | -33.70 |
| 962 | 2183273.421 | 767897.3753 | -34.62 |
| 963 | 2183273.537 | 767355.1948 | -10.01 |
| 964 | 2183273.76 | 767936.425 | -37.67 |
| 965 | 2183273.875 | 767394.2445 | -6.91 |
| 966 | 2183274.097 | 767975.4748 | -38.33 |
| 967 | 2183274.214 | 767433.2943 | -9.07 |
| 968 | 2183274.436 | 768014.5246 | -40.41 |
| 969 | 2183274.552 | 767472.3441 | -17.72 |
| 970 | 2183277.86 | 767670.703 | -8.80 |
| 971 | 2183278.197 | 767709.7527 | -23.86 |
| 972 | 2183278.199 | 767709.7528 | -23.86 |
| 973 | 2183278.535 | 767748.8026 | -33.15 |
| 974 | 2183278.874 | 767787.8523 | -32.25 |
| 975 | 2183279.212 | 767826.9021 | -25.80 |
| 976 | 2183279.551 | 767865.9519 | -34.65 |
| 977 | 2183279.889 | 767905.0017 | -36.93 |
| 978 | 2183280.005 | 767362.8212 | -9.92 |
| 979 | 2183280.227 | 767944.0515 | -39.77 |
| 980 | 2183280.344 | 767401.871 | -8.35 |
| 981 | 2183280.565 | 767983.1013 | -39.32 |
| 982 | 2183280.682 | 767440.9208 | -11.18 |
| 983 | 2183280.904 | 768022.1511 | -40.61 |
| 984 | 2183281.021 | 767479.9706 | -15.75 |
| 985 | 2183283.99 | 767639.2797 | -9.00 |
| 986 | 2183284.328 | 767678.3294 | -14.03 |
| 987 | 2183284.665 | 767717.3792 | -30.02 |
| 988 | 2183284.667 | 767717.3792 | -30.03 |
| 989 | 2183285.004 | 767756.429 | -38.14 |
| 990 | 2183285.342 | 767795.4788 | -23.96 |
| 991 | 2183285.68 | 767834.5286 | -29.57 |
| 992 | 2183286.019 | 767873.5783 | -35.98 |
| 993 | 2183286.135 | 767331.3978 | -9.70 |
| 994 | 2183286.137 | 767331.3979 | -9.70 |
| 995 | 2183286.357 | 767912.6282 | -39.23 |
| 996 | 2183286.473 | 767370.4477 | -9.85 |
| 997 | 2183286.695 | 767951.6779 | -41.21 |
| 998 | 2183286.812 | 767409.4974 | -9.38 |
| 999 | 2183287.034 | 767990.7277 | -39.91 |
| 1000 | 2183287.15 | 767448.5472 | -18.93 |
| 1001 | 2183287.489 | 767487.597 | -12.22 |
| 1002 | 2183290.458 | 767646.9061 | -11.34 |
| 1003 | 2183290.796 | 767685.9559 | -19.61 |
| 1004 | 2183291.133 | 767725.0057 | -34.51 |
| 1005 | 2183291.135 | 767725.0057 | -34.51 |
| 1006 | 2183291.472 | 767764.0555 | -39.36 |
| 1007 | 2183291.81 | 767803.1052 | -19.76 |
| 1008 | 2183292.148 | 767842.1551 | -31.79 |

| 1009 | 2183292.487 | 767881.2048 | -37.61 |
| --- | --- | --- | --- |
| 1010 | 2183292.603 | 767339.0243 | -9.75 |
| 1011 | 2183292.605 | 767339.0243 | -9.75 |
| 1012 | 2183292.825 | 767920.2546 | -41.37 |
| 1013 | 2183292.942 | 767378.0741 | -10.13 |
| 1014 | 2183293.163 | 767959.3044 | -42.02 |
| 1015 | 2183293.28 | 767417.1239 | -10.23 |
| 1016 | 2183293.502 | 767998.3542 | -40.01 |
| 1017 | 2183293.618 | 767456.1737 | -22.31 |
| 1018 | 2183296.927 | 767654.5325 | -14.57 |
| 1019 | 2183297.264 | 767693.5824 | -25.13 |
| 1020 | 2183297.602 | 767732.6321 | -37.91 |
| 1021 | 2183297.603 | 767732.6322 | -37.91 |
| 1022 | 2183297.94 | 767771.682 | -34.17 |
| 1023 | 2183298.279 | 767810.7317 | -21.13 |
| 1024 | 2183298.617 | 767849.7815 | -33.52 |
| 1025 | 2183298.955 | 767888.8313 | -39.50 |
| 1026 | 2183299.071 | 767346.6508 | -10.58 |
| 1027 | 2183299.073 | 767346.6508 | -10.58 |
| 1028 | 2183299.293 | 767927.8811 | -44.15 |
| 1029 | 2183299.41 | 767385.7006 | -11.15 |
| 1030 | 2183299.632 | 767966.9309 | -41.93 |
| 1031 | 2183299.748 | 767424.7503 | -10.98 |
| 1032 | 2183299.97 | 768005.9807 | -39.93 |
| 1033 | 2183300.086 | 767463.8002 | -20.42 |
| 1034 | 2183303.056 | 767623.1093 | -14.43 |
| 1035 | 2183303.395 | 767662.1591 | -18.50 |
| 1036 | 2183303.733 | 767701.2088 | -30.33 |
| 1037 | 2183304.07 | 767740.2586 | -39.78 |
| 1038 | 2183304.071 | 767740.2586 | -39.78 |
| 1039 | 2183304.408 | 767779.3084 | -26.09 |
| 1040 | 2183304.747 | 767818.3581 | -24.36 |
| 1041 | 2183305.085 | 767857.408 | -35.31 |
| 1042 | 2183305.202 | 767315.2275 | -9.07 |
| 1043 | 2183305.423 | 767896.4578 | -41.40 |
| 1044 | 2183305.54 | 767354.2772 | -11.88 |
| 1045 | 2183305.541 | 767354.2773 | -11.88 |
| 1046 | 2183305.761 | 767935.5075 | -46.70 |
| 1047 | 2183305.878 | 767393.3271 | -12.80 |
| 1048 | 2183306.1 | 767974.5573 | -40.80 |
| 1049 | 2183306.217 | 767432.3768 | -11.90 |
| 1050 | 2183306.555 | 767471.4266 | -15.28 |
| 1051 | 2183309.524 | 767630.7357 | -16.44 |
| 1052 | 2183309.863 | 767669.7855 | -22.49 |
| 1053 | 2183310.201 | 767708.8353 | -35.22 |
| 1054 | 2183310.538 | 767747.8851 | -37.45 |
| 1055 | 2183310.54 | 767747.8851 | -37.45 |
| 1056 | 2183310.876 | 767786.9349 | -20.33 |
| 1057 | 2183311.215 | 767825.9846 | -27.73 |
| 1058 | 2183311.553 | 767865.0344 | -37.20 |
| 1059 | 2183311.671 | 767322.8539 | -9.16 |
| 1060 | 2183311.891 | 767904.0842 | -43.26 |
| 1061 | 2183312.008 | 767361.9037 | -11.50 |
| 1062 | 2183312.009 | 767361.9037 | -11.50 |
| 1063 | 2183312.23 | 767943.134 | -48.85 |
| 1064 | 2183312.346 | 767400.9535 | -15.44 |
| 1065 | 2183312.568 | 767982.1838 | -39.43 |
| 1066 | 2183312.685 | 767440.0032 | -12.55 |
| 1067 | 2183313.023 | 767479.0531 | -11.97 |
| 1068 | 2183315.992 | 767638.3622 | -18.98 |
| 1069 | 2183316.331 | 767677.4119 | -26.30 |
| 1070 | 2183316.669 | 767716.4617 | -38.78 |
| 1071 | 2183317.006 | 767755.5115 | -31.90 |
| 1072 | 2183317.008 | 767755.5115 | -31.90 |
| 1073 | 2183317.344 | 767794.5613 | -20.23 |
| 1074 | 2183317.682 | 767833.6111 | -30.64 |
| 1075 | 2183318.021 | 767872.6609 | -39.16 |
| 1076 | 2183318.139 | 767330.4804 | -10.58 |
| 1077 | 2183318.359 | 767911.7107 | -45.88 |
| 1078 | 2183318.476 | 767369.5302 | -10.26 |
| 1079 | 2183318.478 | 767369.5302 | -10.26 |
| 1080 | 2183318.698 | 767950.7605 | -49.09 |
| 1081 | 2183318.814 | 767408.58 | -19.16 |
| 1082 | 2183319.036 | 767989.8102 | -38.31 |
| 1083 | 2183319.153 | 767447.6297 | -15.92 |
| 1084 | 2183322.123 | 767606.9389 | -19.12 |
| 1085 | 2183322.46 | 767645.9886 | -22.13 |
| 1086 | 2183322.799 | 767685.0385 | -29.88 |
| 1087 | 2183323.137 | 767724.0882 | -39.13 |
| 1088 | 2183323.475 | 767763.138 | -25.48 |
| 1089 | 2183323.476 | 767763.1379 | -25.48 |
| 1090 | 2183323.812 | 767802.1878 | -22.79 |
| 1091 | 2183324.151 | 767841.2376 | -33.02 |
| 1092 | 2183324.269 | 767299.057 | -7.85 |
| 1093 | 2183324.489 | 767880.2874 | -41.03 |
| 1094 | 2183324.607 | 767338.1068 | -15.94 |
| 1095 | 2183324.828 | 767919.3371 | -48.15 |
| 1096 | 2183324.944 | 767377.1566 | -12.32 |
| 1097 | 2183324.946 | 767377.1566 | -12.32 |
| 1098 | 2183325.166 | 767958.3869 | -45.60 |
| 1099 | 2183325.282 | 767416.2064 | -24.04 |
| 1100 | 2183325.62 | 767455.2562 | -19.35 |
| 1101 | 2183328.591 | 767614.5653 | -20.10 |
| 1102 | 2183328.929 | 767653.6151 | -25.58 |
| 1103 | 2183329.267 | 767692.6649 | -34.73 |
| 1104 | 2183329.605 | 767731.7147 | -35.16 |
| 1105 | 2183329.943 | 767770.7645 | -20.81 |
| 1106 | 2183329.943 | 767770.7645 | -20.81 |
| 1107 | 2183330.281 | 767809.8143 | -26.19 |
| 1108 | 2183330.619 | 767848.8641 | -35.08 |
| 1109 | 2183330.737 | 767306.6836 | -8.78 |
| 1110 | 2183330.957 | 767887.9138 | -42.82 |
| 1111 | 2183331.075 | 767345.7333 | -19.01 |
| 1112 | 2183331.296 | 767926.9636 | -50.03 |
| 1113 | 2183331.413 | 767384.7831 | -16.71 |
| 1114 | 2183331.414 | 767384.783 | -16.71 |
| 1115 | 2183331.634 | 767966.0134 | -39.74 |
| 1116 | 2183331.75 | 767423.8329 | -29.25 |
| 1117 | 2183332.089 | 767462.8827 | -20.74 |
| 1118 | 2183335.058 | 767622.1917 | -22.40 |
| 1119 | 2183335.397 | 767661.2416 | -29.16 |
| 1120 | 2183335.735 | 767700.2914 | -39.09 |
| 1121 | 2183336.073 | 767739.3411 | -28.85 |
| 1122 | 2183336.411 | 767778.3909 | -20.58 |
| 1123 | 2183336.412 | 767778.391 | -20.58 |
| 1124 | 2183336.749 | 767817.4407 | -29.47 |
| 1125 | 2183337.087 | 767856.4905 | -37.03 |
| 1126 | 2183337.205 | 767314.31 | -9.46 |
| 1127 | 2183337.426 | 767895.5403 | -45.19 |
| 1128 | 2183337.543 | 767353.3598 | -12.79 |
| 1129 | 2183337.764 | 767934.59 | -51.66 |
| 1130 | 2183337.881 | 767392.4096 | -21.74 |
| 1131 | 2183337.881 | 767392.4096 | -21.74 |
| 1132 | 2183338.102 | 767973.6398 | -36.47 |
| 1133 | 2183338.219 | 767431.4594 | -32.35 |
| 1134 | 2183341.526 | 767629.8183 | -25.15 |
| 1135 | 2183341.865 | 767668.868 | -33.53 |
| 1136 | 2183342.203 | 767707.9178 | -37.75 |
| 1137 | 2183342.542 | 767746.9676 | -22.98 |
| 1138 | 2183342.878 | 767786.0174 | -22.98 |
| 1139 | 2183342.88 | 767786.0174 | -22.98 |
| 1140 | 2183343.217 | 767825.0672 | -31.72 |
| 1141 | 2183343.555 | 767864.117 | -39.14 |
| 1142 | 2183343.673 | 767321.9365 | -9.78 |
| 1143 | 2183343.894 | 767903.1667 | -47.34 |
| 1144 | 2183344.011 | 767360.9862 | -11.16 |
| 1145 | 2183344.232 | 767942.2164 | -48.33 |
| 1146 | 2183344.349 | 767400.036 | -26.42 |
| 1147 | 2183344.35 | 767400.0361 | -26.42 |
| 1148 | 2183344.687 | 767439.0858 | -32.61 |
| 1149 | 2183347.995 | 767637.4447 | -28.22 |
| 1150 | 2183348.333 | 767676.4945 | -37.78 |
| 1151 | 2183348.671 | 767715.5443 | -31.53 |
| 1152 | 2183349.01 | 767754.594 | -20.35 |
| 1153 | 2183349.347 | 767793.6438 | -26.05 |
| 1154 | 2183349.348 | 767793.6438 | -26.05 |
| 1155 | 2183349.685 | 767832.6936 | -33.62 |
| 1156 | 2183349.803 | 767290.5131 | 0.87 |
| 1157 | 2183350.024 | 767871.7434 | -41.16 |
| 1158 | 2183350.141 | 767329.5629 | -9.59 |
| 1159 | 2183350.362 | 767910.7932 | -48.95 |
| 1160 | 2183350.48 | 767368.6127 | -15.69 |
| 1161 | 2183350.701 | 767949.8429 | -39.26 |
| 1162 | 2183350.816 | 767407.6625 | -29.22 |
| 1163 | 2183350.818 | 767407.6625 | -29.22 |
| 1164 | 2183351.155 | 767446.7123 | -31.44 |
| 1165 | 2183354.463 | 767645.0712 | -31.60 |
| 1166 | 2183354.801 | 767684.1209 | -39.99 |
| 1167 | 2183355.14 | 767723.1707 | -24.59 |
| 1168 | 2183355.478 | 767762.2205 | -21.76 |
| 1169 | 2183355.815 | 767801.2703 | -28.59 |
| 1170 | 2183355.816 | 767801.2703 | -28.59 |
| 1171 | 2183356.153 | 767840.3201 | -35.51 |
| 1172 | 2183356.271 | 767298.1396 | 1.55 |
| 1173 | 2183356.492 | 767879.3699 | -42.14 |
| 1174 | 2183356.609 | 767337.1894 | -9.65 |
| 1175 | 2183356.83 | 767918.4197 | -48.98 |
| 1176 | 2183356.948 | 767376.2391 | -22.01 |
| 1177 | 2183357.169 | 767957.4694 | -34.91 |
| 1178 | 2183357.285 | 767415.2889 | -30.08 |
| 1179 | 2183357.286 | 767415.2889 | -30.08 |
| 1180 | 2183357.623 | 767454.3387 | -29.90 |
| 1181 | 2183360.931 | 767652.6976 | -34.89 |
| 1182 | 2183361.269 | 767691.7474 | -35.39 |
| 1183 | 2183361.608 | 767730.7972 | -20.40 |
| 1184 | 2183361.946 | 767769.847 | -25.03 |
| 1185 | 2183362.283 | 767808.8968 | -30.46 |
| 1186 | 2183362.285 | 767808.8967 | -30.46 |
| 1187 | 2183362.621 | 767847.9465 | -37.34 |
| 1188 | 2183362.739 | 767305.766 | 1.86 |
| 1189 | 2183362.96 | 767886.9963 | -43.12 |
| 1190 | 2183363.078 | 767344.8158 | -9.90 |
| 1191 | 2183363.298 | 767926.0461 | -45.57 |
| 1192 | 2183363.416 | 767383.8656 | -28.21 |
| 1193 | 2183363.753 | 767422.9154 | -30.01 |
| 1194 | 2183363.754 | 767422.9154 | -30.01 |
| 1195 | 2183367.399 | 767660.324 | -37.79 |
| 1196 | 2183367.738 | 767699.3738 | -26.52 |
| 1197 | 2183368.076 | 767738.4236 | -21.19 |
| 1198 | 2183368.414 | 767777.4734 | -28.70 |
| 1199 | 2183368.751 | 767816.5232 | -32.77 |
| 1200 | 2183368.753 | 767816.5231 | -32.77 |
| 1201 | 2183369.09 | 767855.573 | -38.49 |
| 1202 | 2183369.207 | 767313.3925 | 2.07 |
| 1203 | 2183369.429 | 767894.6228 | -43.44 |
| 1204 | 2183369.546 | 767352.4423 | -10.30 |
| 1205 | 2183369.766 | 767933.6726 | -39.95 |
| 1206 | 2183369.884 | 767391.4921 | -29.89 |
| 1207 | 2183370.221 | 767430.5419 | -29.92 |
| 1208 | 2183370.223 | 767430.5418 | -29.92 |
| 1209 | 2183373.868 | 767667.9506 | -39.09 |
| 1210 | 2183374.206 | 767707.0003 | -20.67 |
| 1211 | 2183374.544 | 767746.0501 | -24.03 |
| 1212 | 2183374.882 | 767785.0999 | -31.22 |
| 1213 | 2183375.22 | 767824.1497 | -34.91 |
| 1214 | 2183375.221 | 767824.1497 | -34.91 |
| 1215 | 2183375.558 | 767863.1995 | -39.66 |
| 1216 | 2183375.676 | 767321.0189 | 1.05 |
| 1217 | 2183375.897 | 767902.2492 | -42.87 |
| 1218 | 2183376.014 | 767360.0687 | -13.20 |
| 1219 | 2183376.234 | 767941.299 | -36.64 |
| 1220 | 2183376.352 | 767399.1185 | -30.01 |
| 1221 | 2183376.689 | 767438.1683 | -29.95 |
| 1222 | 2183376.691 | 767438.1682 | -29.95 |
| 1223 | 2183380.336 | 767675.577 | -35.20 |
| 1224 | 2183380.674 | 767714.6268 | -21.17 |
| 1225 | 2183381.013 | 767753.6766 | -27.47 |
| 1226 | 2183381.351 | 767792.7263 | -32.92 |
| 1227 | 2183381.688 | 767831.7761 | -36.56 |
| 1228 | 2183381.689 | 767831.7761 | -36.56 |
| 1229 | 2183382.026 | 767870.8259 | -40.46 |
| 1230 | 2183382.144 | 767328.6454 | -0.75 |
| 1231 | 2183382.365 | 767909.8757 | -41.38 |
| 1232 | 2183382.482 | 767367.6952 | -18.06 |
| 1233 | 2183382.82 | 767406.745 | -30.07 |
| 1234 | 2183386.804 | 767683.2034 | -24.05 |
| 1235 | 2183387.142 | 767722.2532 | -24.92 |
| 1236 | 2183387.481 | 767761.303 | -30.56 |
| 1237 | 2183387.819 | 767800.3528 | -34.62 |
| 1238 | 2183388.156 | 767839.4026 | -38.05 |
| 1239 | 2183388.158 | 767839.4026 | -38.06 |
| 1240 | 2183388.494 | 767878.4524 | -40.81 |
| 1241 | 2183388.612 | 767336.2719 | -2.40 |
| 1242 | 2183388.832 | 767917.5021 | -39.57 |
| 1243 | 2183388.951 | 767375.3217 | -23.10 |
| 1244 | 2183389.289 | 767414.3714 | -30.22 |
| 1245 | 2183393.272 | 767690.8299 | -21.95 |
| 1246 | 2183393.61 | 767729.8797 | -28.61 |
| 1247 | 2183393.949 | 767768.9294 | -32.75 |
| 1248 | 2183394.287 | 767807.9792 | -36.26 |
| 1249 | 2183394.624 | 767847.029 | -39.36 |
| 1250 | 2183394.626 | 767847.029 | -39.36 |
| 1251 | 2183394.962 | 767886.0788 | -40.77 |
| 1252 | 2183395.08 | 767343.8983 | -5.35 |
| 1253 | 2183395.301 | 767925.1286 | -38.14 |
| 1254 | 2183395.419 | 767382.9481 | -26.93 |
| 1255 | 2183395.757 | 767421.9979 | -30.36 |
| 1256 | 2183399.74 | 767698.4564 | -24.97 |
| 1257 | 2183400.078 | 767737.5061 | -30.99 |
| 1258 | 2183400.417 | 767776.5559 | -34.72 |
| 1259 | 2183400.755 | 767815.6057 | -37.70 |
| 1260 | 2183401.093 | 767854.6554 | -40.00 |
| 1261 | 2183401.094 | 767854.6555 | -40.00 |
| 1262 | 2183401.43 | 767893.7053 | -40.41 |
| 1263 | 2183401.548 | 767351.5248 | -10.20 |
| 1264 | 2183401.887 | 767390.5745 | -29.16 |
| 1265 | 2183402.225 | 767429.6243 | -30.22 |
| 1266 | 2183406.209 | 767706.0828 | -29.27 |
| 1267 | 2183406.546 | 767745.1326 | -32.81 |
| 1268 | 2183406.886 | 767784.1824 | -36.49 |
| 1269 | 2183407.223 | 767823.2322 | -39.01 |
| 1270 | 2183407.56 | 767862.282 | -40.02 |
| 1271 | 2183407.561 | 767862.2819 | -40.02 |
| 1272 | 2183407.899 | 767901.3318 | -39.89 |
| 1273 | 2183408.016 | 767359.1512 | -18.94 |
| 1274 | 2183408.355 | 767398.201 | -29.94 |
| 1275 | 2183412.677 | 767713.7092 | -31.73 |
| 1276 | 2183413.015 | 767752.7591 | -34.57 |
| 1277 | 2183413.353 | 767791.8089 | -38.10 |
| 1278 | 2183413.691 | 767830.8586 | -39.93 |
| 1279 | 2183414.028 | 767869.9084 | -40.06 |
| 1280 | 2183414.03 | 767869.9084 | -40.06 |
| 1281 | 2183414.367 | 767908.9582 | -39.22 |
| 1282 | 2183414.484 | 767366.7777 | -28.27 |
| 1283 | 2183414.824 | 767405.8275 | -30.10 |
| 1284 | 2183419.145 | 767721.3357 | -33.20 |
| 1285 | 2183419.483 | 767760.3855 | -36.26 |
| 1286 | 2183419.821 | 767799.4353 | -39.46 |
| 1287 | 2183420.159 | 767838.4851 | -40.18 |
| 1288 | 2183420.497 | 767877.5349 | -40.07 |
| 1289 | 2183420.498 | 767877.5349 | -40.07 |
| 1290 | 2183420.953 | 767374.4042 | -30.21 |
| 1291 | 2183421.291 | 767413.454 | -30.29 |
| 1292 | 2183425.612 | 767728.9622 | -34.48 |
| 1293 | 2183425.951 | 767768.012 | -37.86 |
| 1294 | 2183426.289 | 767807.0617 | -40.29 |
| 1295 | 2183426.628 | 767846.1115 | -40.16 |
| 1296 | 2183426.965 | 767885.1613 | -39.98 |
| 1297 | 2183426.966 | 767885.1614 | -39.98 |
| 1298 | 2183427.421 | 767382.0306 | -29.70 |
| 1299 | 2183432.081 | 767736.5886 | -35.73 |
| 1300 | 2183432.419 | 767775.6384 | -39.33 |
| 1301 | 2183432.758 | 767814.6883 | -40.71 |
| 1302 | 2183433.096 | 767853.738 | -40.04 |
| 1303 | 2183433.433 | 767892.7878 | -39.64 |
| 1304 | 2183433.434 | 767892.7878 | -39.64 |
| 1305 | 2183433.889 | 767389.6571 | -27.94 |
| 1306 | 2183438.549 | 767744.2151 | -36.80 |
| 1307 | 2183438.887 | 767783.2649 | -40.40 |
| 1308 | 2183439.226 | 767822.3147 | -40.83 |
| 1309 | 2183439.564 | 767861.3645 | -40.02 |
| 1310 | 2183445.017 | 767751.8416 | -37.99 |
| 1311 | 2183445.355 | 767790.8914 | -40.86 |
| 1312 | 2183445.694 | 767829.9411 | -40.74 |
| 1313 | 2183446.032 | 767868.9909 | -40.00 |
| 1314 | 2183451.485 | 767759.468 | -39.17 |
| 1315 | 2183451.824 | 767798.5178 | -40.99 |
| 1316 | 2183452.162 | 767837.5676 | -40.50 |
| 1317 | 2183452.5 | 767876.6174 | -39.75 |
| 1318 | 2183457.954 | 767767.0945 | -39.98 |
| 1319 | 2183458.292 | 767806.1443 | -40.95 |
| 1320 | 2183458.631 | 767845.194 | -40.00 |
| 1321 | 2183464.422 | 767774.7209 | -40.38 |
| 1322 | 2183464.76 | 767813.7707 | -40.86 |
| 1323 | 2183465.099 | 767852.8205 | -40.00 |
| 1324 | 2183470.89 | 767782.3474 | -40.55 |
| 1325 | 2183471.228 | 767821.3972 | -40.71 |
| 1326 | 2183471.567 | 767860.447 | -40.00 |
| 1327 | 2183477.358 | 767789.9738 | -40.59 |
| 1328 | 2183477.696 | 767829.0236 | -40.52 |
| 1329 | 2183483.827 | 767797.6003 | -40.55 |
| 1330 | 2183484.164 | 767836.6501 | -40.43 |
| 1331 | 2183490.295 | 767805.2267 | -40.43 |
| 1332 | 2183490.633 | 767844.2766 | -40.03 |
| 1333 | 2183496.762 | 767812.8532 | -40.27 |
| 1334 | 2183503.231 | 767820.4797 | -40.12 |
| 1335 | 2183509.699 | 767828.1062 | -40.01 |

ANNEXURE-XX

TCC & EC MEETING MINUTES OF NARINGPANGA (S) GRAPHITE BLOCK, RAYAGADA DISTRICT, ODISHA